Program:	DASM.EXE V1.1

Release date:	December 13, 1997

Author:		John Wilson <wilson@dbit.com>

Description:	DASM is a simple disassembler for Z-80, 8051, 8080/8085, or

8086 binary files.  The file base address may be anywhere in 64

KB so ROM images may be disassembled as easily as .COM files.

The disassembler performs an execution trace before generating

output to put labels in the right places as much as possible,

and it tries to distinguish code from data, but it is possible

for it to fool itself by making symbol table entries for

addresses that are really indices.  A mechanism is provided for

feeding the disassembler a file containing a list of entry

points at which to begin the execution trace, so after each

disassembly you can inspect the output and add addresses from

any newly discovered jump tables, dispatch tables, computed

jumps etc. to the file so the next disassembly will be closer

to complete.

Ideally the disassembler would figure out these things itself

but that would require very sophisticated tracing of register

contents which would make the disassembler several orders of

magnitude more complicated.  For now it simply performs a "dry

run" disassembly starting at each entry point specified, adding

each jump (or call) target to the symbol table, recursing for

both paths when it reaches a CALL or conditional jump

instruction, and stopping when it reaches a halt or

unconditional return instruction, or a label that it's been to

before (so it won't get caught in loops).  Then a linear

disassembly is done starting at the specified starting address,

producing labels and code or data depending on the type of

access that created the symbol table entry for the most recent

label.

Command line:	DASM [filename[.COM]] [/switches]

The "filename" parameter is the name of the binary file, with a

default extension of ".COM".  The output file has the same name

but with the extension changed to ".ASM" (which better not have

been the input file's extension!).  If no filename is

specified, standard input and output are used.

The switches are all optional and must start with the current

DOS SWITCHAR, which is "/" by default.  They are:

/S:hhhh

Sets the address where disassembly starts (beginning of file by

default).

/E:hhhh

Sets the address where disassembly ends (end of file by

default).

/F:hhhh

Sets the file's load address (0100 by default), i.e. defines

the relationship between file addresses and memory addresses.

/T:hhhh

Sets the transfer address where the trace begins (0100 by

default).

/T:@file

Gives the name of a file containing a list of hex entry

addresses separated by white space and/or <CRLF> pairs.  If an

address has a '#' suffix then it is entered in the symbol table

as a "data" address.  Otherwise it is entered as a "code"

address (i.e. jump target) and an execution trace is begun at

that address.

/C:xxxx

Specifies which CPU's mnemonics are to be used for disassembly:

/C:8051 Intel MCS-51 mnemonics

/C:8080	Intel MCS-80/MCS-85 mnemonics (syn. /I)

/C:8086	Intel iAPX-86 (etc.) mnemonics (syn. /N)

/C:Z80	Zilog Z-80 mnemonics

Copyright:	This program is copyright (C) 1995, 1997 by John Wilson.  All

rights reserved.  The program may be freely distributed

provided that no charge is made for distribution, and that

proper credit is given to John Wilson in all copies, whether

modified or not.

All trademarks referenced are used for identification purposes

only and are the property of their respective owners.