DECmate II CP/M Technical Notes

DISCLAIMER

This document file was created by scanning  the

original  document and then editing the scanned

text.  As much as possible, the  original  text

format  was restored.  Some format changes were

made to insure this  document  would  print  on

current laser printers using 60 lines per page.

The  original  spelling  and  grammar  has been

preserved.

30-Mar-1996

30-June-83

DECmate II CP/M Technical Note Index

_________________________________________________________________

CP/M

No  Title				     Revision     Release	__  _____				     ________     _______

1  Z80 Memory Map			     0, 16-Sep-82   1.0

2  RX50 Diskette Format		     3, 22-Dec-82   1.0

3  8BIOS Interface Codes		     7, 07-Oct-82   1.0

4     (withdrawn)

5  CP/M Support for RD51D		     1, 17-Mar-83   2.x

6  RX50 Diskette Format for CP/M Release 2  0, 11-Feb-83   2.x

7  8BIOS Interface Codes for Release 2	     4, 20-Jun-83   2.x

8     (withdrawn)

9  System Bootstrap Operation		     1, 20-Jun-83   2.x

10  Z80 memory Map for Release 2	     0, 19-Apr-83   2.x

11  Console Standard Modes for Release 2     0, 20-Jun-83   2.x

12  Format of Hard Disk Backup Diskettes     0, 27-Jun-83

13  Loading and Using Extensions to 8BIOS    0, 30-Jun-83   2.x

R.F.Morse

MLO3-2/E41

DTN 223-6720

Rev 0, 16-Sep-82

DECmate II CP/M Technical Note 1:

Z80 Memory Map

_________________________________________________________________

This note describes the allocation of the 65536. (64K) bytes of

memory in the Z80 APU when operating under CP/M-80 version 2.2.

Usage		   Addresses	      Length		 _____		   _________	      ______

System parms	   0000..00FF	    100    256.

TPA		   0100..DFFF	   DF00  57088.

CCP		   E000..E7FF	    800   2048.

BDOS		   E800..F5FF	    E00   3584.

BIOS code	   F600..FBFF	    600   1536.

BIOS work area    FC00..FFFF 	    400   1024.

When generating a CP/M image with the MOVCPM program, this is

described as a 63K system.		       __

Application programs occupy the transient program area (TPA).

Large application programs usually overlay the CCP (console

command processor), so the limit on their size is 59136. bytes.

This map is for the initial release of the system which supports

RX50 and RX01/RX02 diskettes.  The release which supports the

RD50 Winchester disk will probably require additional space for

the BIOS code and work areas, which will reduce the TPA size by

1024. (1K) bytes.

None of the storage in the 6120 side of the system is directly

available to a CP/M application program.  (Extensive amounts of

this storage are used by the 8BIOS program which supports ZBIOS.)

Rev 3, 22-Dec-82

DECmate II CP/M Technical Note 2:

RX50 Diskette Format

_________________________________________________________________

This note describes the format and contents of the RX50 diskettes

used by the DECmate II version of the CP/M-80 operating system.

The description of the size, location and contents of the

CPMIMAGE.SYS file on a system disk applies to version 1.0 for

RX50-based systems.

The RX50 diskette is a 5 1/4", soft-sectored, single-sided,

double-density diskette with a track spacing of 96 tracks per

inch.  The arrangement of address marks, sector headers, data

fields and gaps is that used by the 1793 LSI controller chip.  A

diskette contains 80 tracks, numbered 0 through 79.  Each track

contains 10 512-byte sectors, numbered 1 through 10.  This gives

a capacity of 5,120 (5K) bytes per track and 409,600 (400K) bytes

per diskette.

When used for CP/M, these diskettes may be either system or data							  ______    ____

disks.  A system disk contains the CP/M operating system and the

necessary supporting files to allow it to be bootstrap loaded

into an idle machine.  A data disk contains only CP/M files and

cannot be bootstrap loaded.  The first 12 bytes of track 1,

sector 1 are coded to mark these as CP/M disks and to distinguish

system from data disks.  The DECmate II hardware bootstrap loader

tests these bytes and will not attempt to load a non-system disk.

Track usage	___________

Tracks on a diskette are used as follows:

Tracks  Use	     ______  ___

0     On a system disk:  data for the SETUP screen,

keyboard map, and special user-defined display

character sets.  On a data disk:  sector 1 is

zero-filled to prevent any attempt to store SETUP

data on the disk and sectors 2..10 are undefined.

1     On a system disk:  bootstrap program to load the

system into an idle machine.  On a data disk:

sector 1 is coded to mark it as a CP/M data disk and

sectors 2..10 are undefined.

2..79   CP/M directory and data area.  These 78 tracks are

controlled by CP/M's file system.  They begin with a

128-entry CP/M directory and contain program and

data files.  On a system disk, the first directory

entry describes a protected file named CPMIMAGE.SYS

RX50 Diskette Format					  Page 2

_________________________________________________________________

which occupies tracks 75..79 and contains the CP/M

system image and the extended DECmate firmware.  On

a data disk, all 78 tracks are usable for CP/M

files.

Record addressing	_________________

The CP/M file system always transfers data to and from its

callers in 128-byte records.  Four such records are recorded in			    _______

each 512-byte sector on an RX50 diskette.  The blocking and

deblocking of records is performed by the Z80 portion of the BIOS

(called ZBIOS; its companion on the 6120 part of the machine is

called 8BIOS).  From the CP/M point of view, a diskette contains

78 tracks numbered 2 through 79 and each track contains 40

records numbered 0 through 39.  These two parameters--track

[2..79] and record[0..39]-- are passed by BDOS to the BIOS SETTRK

and SETSEC routines, respectively.  The ZBIOS code divides the

CP/M record number by four and adds one to get the (l-origin)

physical sector number.  The remainder of this division is

multiplied by 128 to get the offset to the start of the record

within the sector.

Since the BIOS and disk controller cannot access physically

consecutive sectors without waiting for an extra revolution of

the disk, an interleave scheme is used to map logical sectors

into non-consecutive physical ones.  An every-other-sector (2:1)

interleave is used, as shown in the following table:

Logical      Physical

sector	    sector		      _______	   ________

1	       1

2	       3

3	       5

4	       7

5	       9

6             2

7	       4

8	       6

9	       8

10	      10

Since there are four CP/M records in each sector, records 0..3

are placed in physical sector 1, records 4..7 in sector 3, and so

on.  The actual sector translation table in ZBIOS (whose address

is in the DPH and is used by the SECTRAN routine in the BIOS)

operates on record numbers.  It contains 40 entries to convert a

0-origin record number into a 0-origin interleaved record number.

Thus its first eight entries are as follows:

RX50 Diskette Format					  Page 3

_________________________________________________________________

Logical      Interleaved

record	      record		      _______	   ___________

0		 0

1		 1

2		 2

3		 3

4		 8

5		 9

6		10

7	 	11

8		16

9		17

etc.

There is no track-to-track skew in this system; physical sector 1

is the first logical sector on every CP/M track

Space allocation	________________

Space in the CP/M area is subdivided into allocation blocks which

contain 2K bytes (16 records) each.  The 78 tracks of the area

hold 390K bytes, so there are 195 allocation blocks, numbered

0..194.  Thus every file occupies space whose size is a multiple

of 2K.

File directory	______________

The CP/M file directory has 128 entry positions of 32 bytes

each.  These occupy allocation blocks 0 and 1, which are the

first 32 records (8 sectors) of track 2.

System image file	_________________

Each CP/M system disk contains a file named CPMIMAGE.SYS which

contains the CP/M system, ZBIOS, 8BIOS, and extended DECmate II

firmware.  This file is always represented by the first entry in

the directory.  To protect it from accidental damage and to

prevent if from disrupting wild-card operations such as PIP *.*,

it has the R/O and SYS attributes and it is assigned to user 15.

The directory entry describes the file as occupying the last 12

allocation blocks (numbers 183..194), or 24K bytes.  Physically,

this comprises the last 8 logical sectors of track 75 and all of

tracks 76..79.

RX50 Diskette Format					  Page 4

_________________________________________________________________

The CPMIMAGE.SYS file is divided into three parts:

The CP/M-80 system image.  This comprises the CCP (2048	     ________________________

bytes), BDOS (3584 bytes), and ZBIOS (1536 bytes).  These 7K

bytes correspond to the standard image described in Digital

Research's "CP/M 2 Alteration Guide".  This data is stored

in standard interleaved file format and can be read and

written by any CP/M program (after conquering the R/O and

user 15 restrictions).

Note:  The first record of this file is the beginning

of the CCP.  This contains the command input buffer,

which can be patched with a command to be executed

automatically after bootstrap loading.  Be sure,

however, to rewrite the file IN PLACE--the boot loader

code expects to find the system image in the exact

sectors described above.

The 8BIOS image.  This is the code executed by the 6120	     _______________

processor.  All calls to ZBIOS for I/O services are passed

to 8BIOS, which has access the the DECmate's I/O devices.

It occupies the last 4 logical sectors of track 76 and all

of track 77.

DECmate firmware.  This code supports the DECmate II screen	     ________________

and keyboard handling.  It occupies all of tracks 78 and 79

and is loaded into the 6120's panel memory during bootstrap

loading.

Diskette initialization	_______________________

The CP/M utility program DISKINIT is used to initialize a

diskette as either a system or data disk.  For system disks, it

copies tracks 0, 1, and 75..79 from another system disk and writes

a CP/M directory on track 2 whose first entry describes

CPMIMAGE.SYS and whose remaining entries are unused,  For data

disks, it writes dummy information in sector 1 of tracks 0 and 1

and writes a CP/M directory on track 2 with all its entries

unused.

Note that any diskette to be used with the DECmate II must be

physically formatted by its vendor with 10 512-byte sectors on

each of 80 tracks.  The DECmate II disk controller cannot

rewrite a diskette's format data.

RX50 Diskette Format					  Page 5

_________________________________________________________________

Interchange with other DEC systems	__________________________________

The CP/M parameters used for the DECmate II RX50 diskettes are

the same as those used by the Rainbow for its CP/M-80/86 and by

the Professional for its CP/M-80.  To summarize, these parameters

are:

1)  File area in tracks 2..79;

2)  Physical sector interleave sequence of

1,3,5,7,9,2,4,6,8,10;

3)  Allocation block size of 2K;

4)  Directory containing 128 entries.

Any DECmate II CP/M disk--system or data--can be used as a data

disk on a Rainbow or Professional CP/M system.  A DECmate II

system disk cannot, of course, be bootstrap loaded as a system

disk on either of those systems.

RX50 Diskette Format					  Page 6

_________________________________________________________________

Appendix A:  Contents of System Image File

A CP/M system disk has a bootstrap loader and system code stored	       ______

in tracks 1 and 75 through 80.  The contents of each sector in

those tracks is described below.  All references are to physical

tracks (0..79) and 512-byte sectors (1..10).

Secondary boot	______________

The secondary boot code occupies one sector (track 1, sector 1).

It is written in 12-bit mode and occupies 256 (octal 400) words.

This is the code which is brought in by the firmware loader

during a power-on boot, and its first few bytes contain the DEC

standard boot-block code which marks a CP/M system disk.  The

secondary boot code source file is called BOOT2.

Tertiary boot	_____________

The tertiary boot code occupies two sectors (track 1, sectors 2

and 3).  It is written in 12-bit mode and occupies 512 (octal

100) words.  This code is loaded into field 7 by the secondary

boot, and it reads the actual 8BIOS code into fields 0 and 1.

The tertiary boot code source file is called BOOT3.

8BIOS	_____

The 8BIOS code occupies 18 sectors on parts of tracks 1, 77 and

76.  It is written in packed mode (two words in each three bytes)

and is 6144 (octal 14000) words long.  This code is loaded into

fields 0 and 1 by the tertiary boot.  The tertiary boot contains

the following table which determines which sectors are read and

in what order:

Track     Sector			   _____     ______

1		 4    Start field 0

1		 6

1		 8

1		10

1		 5

1		 7

1		 9

77		 2

77		 4

77		 6

77		 8

77		10

77		 3    Start of field 1

77		 5

77		 7

77		 9

77		 1

76		 6

RX50 Diskette Format					  Page 7

_________________________________________________________________

CP/M system image	_________________

The CPM system image occupies 14 sectors on tracks 75 and 76 and

is written in 8-bit mode.  It is 7168 bytes long and is

subdivided into three parts:  CCP (4 sectors, 2048 bytes), BDOS

(7 sectors, 3584 bytes), and ZBIOS (3 sectors, 1536 bytes).  The

sequence of sectors for this image must correspond to the first

14 physical sectors (56 CP/M logical records) of the CPMIMAGE.SYS

file, since the CP/M image is installed by using ordinary BDOS

file I/O calls.

Track     Sector			   _____     ______

75		 5    CCP, 2048 bytes

75		 7

75		 9

75		 2

75		 4    BDOS, 3584 bytes

75		 6

75		 8

75		10

76		 1

76		 3

76		 5

76		 7    ZBIOS, 1536 bytes

76		 9

76		 2

DECmate firmware	________________

The extended firmware which is loaded into 6120 panel memory

occupies all of tracks 78 and 79.

Uncommitted sectors	___________________

There are three uncommitted sectors on track 76.  Sector 4 can be

used to add 512 bytes to ZBIOS and sectors 8 and 10 can be used

to add 682 (octal 1252) words to 8BIOS, or all three sectors can

be used to add 1024 (octal 2000) words to 8BIOS.

Track     Sector			_____     ______

76	      4

76	      8

76	     10

In order to use these to expand 8BIOS, the sector tables in the

tertiary boot BOOT3 and the utility program PUT234 which writes

the code on a system diskette, must be modified accordingly.  In

order to expand ZBIOS, routines RDCPM and LODCPM in 8BIOS must be

modified.

R.F.Morse

DTN 223-6720

MLO3-2/E41

Rev 7, 07-Oct-82

DECmate II CP/M Technical Note 3:

8BIOS Interface Codes

_________________________________________________________________

This technical note describes the interface codes used for

commands sent from the Z80 CP/M BIOS to the 6120 8BIOS.  Each

command consists of a 4-bit command code and an 8-bit data byte.				    ____________	      _________

In a Z80 program, a command is sent by calling the OUTBYT routine

in the BIOS.  Most commands cause 8BIOS to generate a status or

data byte for return to the Z80, which is received by calling the

INBYT routine.  Two additional routines, BLKRD and BLKWR,

transmit blocks of data (such as disk sectors) from or to 8BIOS.

Four special call entry points have been added to the CP/M BIOS

entry vector to permit a CP/M program to send a command to 8BIOS

and to receive a byte from 8BIOS.  They are:

OUTBYT  sends a command to 8BIOS.  Entry is at offset 003Fh

from the BIOS vector base (whose address is in memory

at 0001..0002h).  Call with the command code in the

low 4 bits of the C register and the data byte in the

B register.  Returns after sending the command; the A

and F registers are destroyed.

INBYT   receives a byte from 8BIOS.  Entry is at offset 003Ch

from the BIOS vector base.  There are no call

arguments.  Returns with the data from 8BIOS in the A

register with the Z and S flags set to reflect its

contents.  Alters no other registers.

BLKRD   receives a block of data from 8BIOS.  Entry is at

offset 0042h from the BIOS vector base.  Call with

the HL register addressing the first byte of the area

to receive the block.  On return, HL will address the

last byte filled, plus one.  The amount of data

transferred is controlled by 8BIOS (e.g. 512 bytes

for an RX50 disk sector or 128 bytes for an RX01

sector) and can be determined from the value returned

in HL.  Destroys registers A, B, C and F.

BLKWR   sends a block of data to 8BIOS.  Entry is at offset

0045h from the BIOS vector base.  Call with the HL

register addressing the first byte to be sent.  On

return, HL will address the last byte sent, plus one.

The amount of data transferred is controlled by

8BIOS, as described above for BLKRD.  Destroys

registers A, B, C and F.

8BIOS Interface Codes					  Page 2

_________________________________________________________________

Commands	________

The following table lists the command codes and their associated

data bytes in numerical sequence.  Using an undefined code or

code/data combination should result in no operation, but is not

recommended!

Code  Data  Description	____  ____  ___________

00    any  Signals that a Z80 "HALT" instruction has been

executed.  This code is generated by hardware on the

Z80 board; it should not be used by software.

01    00   CP/M "cold boot" function.  Abandons the current

state of the Z80 and re-reads the CP/M system image

from the disk in drive A.  This replaces the stored

CP/M image (CCP, BIOS, and ZBIOS) for the Z80, but

does not reload the 8BIOS code.  A more complete			 ___

restart which reloads everything can be performed by

invoking function 02 0B to simulate a power-on reset.

01    01   CP/M "warm boot" function.  Must be followed by a

call to BLKRD to read all of CCP and BIOS into Z80

storage.

01    02   Get keyboard status.  The following INBYT call will

return FFh if a character is available from the

keyboard, or 00h if not.

01    03   Get printer output status.  The following INBYT call

will return FFh if the printer can accept an output

character, or 00h if not.

01    04   Get communication input.  The following INBYT call

will return the next incoming character from the

communication port, waiting if necessary until one is

available.

01    05   Get keyboard input.  The following INBYT call will

return the next character from the keyboard, waiting

if necessary until one is available.

01    06   Disk "home" request.  Sets the track number to 0 for

the next disk operation and purges any outstanding

read-ahead operation.  (Does not perform any physical

disk operation.)

01    07   Read disk sector.  Reads one sector from the

previously-selected drive, track and sector.  The

following INBYT will return 00h if the read was

successful, or a non-zero value if not.  If

successful, a call to BLKRD must follow to transfer

the sector image into Z80 storage.  (BLKRD must not								    ___

8BIOS Interface Codes					  Page 3

_________________________________________________________________

be called if the read was unsuccessful!)

01    08   Unused.

01    09   Write disk sector.  Writes one sector to the

previously-selected drive, track and sector.  Must be

followed by a call to BLKWR to transfer the sector

image to the 6120.  After the BLKWR call, INBYT must

be called.  It will return 00h if the write was

successful, or a non-zero value if not.

01    0A   Get disk type.  The following INBYT call will return

a code which indicates the format of the disk in the

previously-selected disk drive, as follows:

00  RX01--8", single-density, 128-byte sectors,

26 sectors/track, 77 tracks;

01  RX50--5", double-density, 512-byte sectors,

10 sectors/track, 80 tracks;

02  VT180--5", double-density, 512-byte sectors,

9 sectors/track, 40 tracks;

03  RX02--8", double-density, 256-byte sectors,

26 sectors/track, 77 tracks;

04  Other--5", double-density, not RX50 or VT180

format (use function 02 10 to determine its

characteristics);

FF  Drive not installed.

01    0B   Get screen output status.  Following INBYT call will

return FFh if ready to accept a character, or 00h if

not.

01    0C   Get printer input.  Following INBYT call will return

the next character from the printer input port,

waiting if necessary until one is available.

01    0D   Get printer input status.  Following INBYT call will

return FFh if a character is available from the

printer input port, or 00h if not.

01    0E   Get communication output status.  Following INBYT

call will return FFh if the communication port can

accept an output character, or 00h if not.

01    0F   Get communication input status.  Following INBYT call

will return FFh if a character is available from the

communication input port, or 00h if not.

02    00   Enable communication XON/XOFF pacing.  XON and XOFF

8BIOS Interface Codes					  Page 4

_________________________________________________________________

characters received at the communication input port

will control communication output and will not be

available to a caller of function 01 04.  (This is

the default setting.)

02    01   Disable communication XON/XOFF pacing.  Received XON

and XOFF characters will not control communication

output but will instead be passed to a caller of

function 01 04.

02    02   Get modem status from communication port.  Following

INBYT call will return a byte whose low-order 4 bits

reflect the modem status, as follows ("one" indicates

a "true" value):

bit 7--Set if any of bits 3..0 has changed since

the last request for modem status.

bit 3--Ring indicator (CCITT 125)

bit 2--Data carrier detect (CCITT 109)

bit 1--Data set ready (CCITT 107)

bit 0--Clear to send (CCITT 106)

02    03   Enable printer XON/XOFF pacing.  XON and XOFF

characters received at the printer input port will

control printer output and will not be available to a

caller of function 01 0C.  (This is the default

setting.)

02    04   Disable printer XON/XOFF pacing.  XON and XOFF

characters received at the printer input port will

not control printer output but will instead be passed

to a caller of function 01 0C.

02    05   Disable recognition of the SETUP key.  (The key will

be re-enabled at the next CP/M warm-start or by

function 02 0A.)

02    06   Send BREAK signal to communication output port.  A

second call to OUTBYT (with a command code of 0Ah)

sends a data byte whose value (0..255) specifies the

length of the break in units of 10 milliseconds (i.e.

a value of 1Ch gives a duration of 280 milliseconds).

02    07   Set modem control lines of communication port.  A

second call to OUTBYT (with a command code of 0Ah)

sends a data byte which is interpreted as follows:

bit 4--One asserts request to send (CCITT 105)			     ___

bit 3--One asserts data terminal ready (CCITT			     ___

108.2)

bit 2--One asserts signal rate select (CCITT 111)			     ___

02    08   Set communication baud rate.  A second call to OUTBYT

(with a command code of 0Ah) sends a data byte whose

8BIOS Interface Codes					  Page 5

_________________________________________________________________

low-order 4 bits are interpreted as follows:

00     50 bps	 08    1800

01     75		 09    2000

02    110		 0A    2400

03    134.5		 0B    3600

04    150		 0C    4800

05    300		 0D    7200

06    600		 0E    9600

07   1200		 0F   19200

Note:  The default value is set from the SETUP data

stored on the system disk.  If the rate is changed by

using function 02 08, that change cannot be detected

by SETUP, and any subsequent use of the SETUP menu

will change the baud rate back to that shown in the

menu.

02    09   Set printer port baud rate.  A second call to OUTBYT

(with a command code of 0Ah) sends a value as

described above for function 02 08.  The note

concerning the use if SETUP also applies to the

printer port.

02    0A   Enable SETUP key recognition.  This reverses the

effect of function 02 05.

02    0B   Perform power-on reset.  Abandons the current state

of the machine and begins execution in the bootstrap

ROM as if power had just been turned on.

02    0C   Reset real-time clock.  Sets the time counter to

zero.

02    0D   Read real-time clock.  The real-time clock is a

12-bit register which is incremented every 10

milliseconds and wraps around modulo 4096.  The value

is read in two INBYT calls.  The first returns the

high-order 4 bits of the counter in bits 3..0 (bits

7..4 are zero); the second returns the low-order 8

bits of the counter.

02    0E   Get screen width.  The following INBYT call will

return a value of 00h if the screen width is 80

columns or 01h if it is 132 columns.

02    0F   Set communication character type.  A second call to

OUTBYT (with a command code of 0Ah) sends a data byte

whose bits are interpreted as follows:

bits 7..6  Number of data bits (exclusive of

parity bit):

00 -- 5 bits

01 -- 7 bits

8BIOS Interface Codes					  Page 6

_________________________________________________________________

10 -- 6 bits

11 -- 8 bits

bits 5..4  Unused

bits 3..2  Number of stop bits transmitted:

01 -- 1 stop bit

10 -- 1.5 stop bits

11 -- 2 stop bits

bit 1      Parity sense (ignored if parity not

enabled):

0 -- ODD parity

1 -- EVEN parity

Bit 0	   Parity enable:

0 -- disable

1 -- enable

Using this function reset the communication channel

and discards any pending input and output data.  The

mode after a system reset is 8 data bits, no parity

and 1 stop bit; this is not affected by the use of the

SETUP key.  This function does not alter the

communication channel's baud rate; function 02 08 does

that.

02    10   Get 5" diskette characteristics.  The next 4 INBYT

calls will return bytes which describe the physical

characteristics of the 5" disk selected by the most

recent call to function 06 (do not use for external

8" drives--no useful information can be returned).

byte 1:  Sectors per track, a binary number in

the range 1..32 (01h..20h)

byte 2:  Sector size code:

00h --  128 bytes

01h --  256 bytes

02h --  512 bytes

03h -- 1024 bytes

byte 3:  Track density code:

00h -- 48 tpi

01h -- 96 tpi

byte 4:  Two-sided flag:

00h -- single-sided

01h -- double-sided

02    11   Perform a print-screen operation.  Issuing this call

has the same effect as the operator's pressing the

"Print Screen" key.

02    12   Perform a setup operation.  Issuing this call has the

same effect as the operator's pressing the "Setup"

key.  This call will be ignored if SETUP recognition

has been disabled by function 02 05.

03   data  Send data character to the printer output port.

8BIOS Interface Codes					  Page 7

_________________________________________________________________

04   data  Send data character to the video screen.

05   data  Send data character to the communication output port.

06   data  Select disk drive.  "Data" is 00..03 for internal

RX50 drives and 04..07 for external RX01/RX02 drives.

A subsequent INBYT call will return 00h if the drive

is installed, or FFh if not.

07   data  Select disk track.  Two successive calls must be made

to send the 16-bit track number; the first call sends

the high-order byte.

08   data  Select disk sector.  Two successive calls must be

made to send the 16-bit sector number; the first call

sends the high-order byte.

09   data  Select disk head.  (Not presently used.)

0A   data  Block data output.  Used by the BLKWR routine and for

functions (such as 02 07) which require a data byte.

The remaining command codes 0B..0F are not presently defined.

Rev 1, 17-Mar-83

DECmate II CP/M Technical Note 5:

CP/M Support for RD51D Hard Disk

_________________________________________________________________

This technical note describes modifications to the 8BIOS and

ZBIOS modules of CP/M to accommodate the RD51D hard disk.  It

should be read in conjunction with the following documents:

1.  Programming the DECmate II for the RD51D Disk Subsystem		_______________________________________________________

2.  DTN #7:  8BIOS Interface Codes for Release 2		____________________________________________

3.  CP/M 2 Alteration Guide (by Digital Research)		_____________________________________________

CP/M Volumes on an RD51D	________________________

A CP/M system may access up to four hard disk logical volumes at

once, using disk drive letters "E" through "H".  The 8BIOS

select-disk-drive function (command 06 xx) receives hex values

04..07 for hard disk volumes and converts these to references to

RD51D devices 0..3.  (devices 4 through 15 are not used by CP/M

and should have no volumes mounted in them during CP/M

operation.)

CP/M views each volume as being composed of tracks and sectors.

Each track contains 64 logical sectors of 128 bytes, or 16

physical sectors of 512 bytes.  Both logical and physical sectors

are numbered beginning from 0.  The 8BIOS select-disk-sector

function (command 08 xx) accepts physical sector numbers in the

range 0..15; the select-disk-track function (command 07 xx)

accepts track numbers in the range 0 through the volume size

minus one.  (The volume size is found in the volume's directory

entry in the two bytes beginning at offset 14.  Since this number

represents units of 16 blocks, it is already scaled to CP/M track

units.)  The 8BIOS select-disk-head function (command 09 xx) is

not used.

No sector interleave table is used by CP/M for hard disk volumes.

The size of a CP/M volume may vary from 1 track (scarcely useful)

to 1024 tracks (8 megabytes).  The size of the directory and the

size of allocation blocks for the volume are computed as a

function of the volume size; this algorithm is described below.

There are two types of CP/M volumes on an RD51D:  system and							  ______

data.  These are distinguished by the byte at offset 17 in the	____

CP/M Support for RD51D Hard Disk			  Page 2

_________________________________________________________________

volume's directory entry:

40h  Data volume.  The directory starts in block 0 and the		  ____

entire volume is usable for CP/M files.

C0h  System volume.  The first four tracks (blocks 0..63)		  ______

contain a bootable CP/M system, as follows:

Blocks  Contents		      ______  ________

0  System boot loader, written in 12-bit mode.

1..47  8BIOS code in compressed 8-bit format (this

area holds 16042 words, or just under four

fields).

48..51  CP/M's CCP (2048 bytes).

52..58  CP/M's BDOS (3584 bytes).

59..63  ZBIOS (up to 2560 bytes).

The directory begins at block 64, which is the first

sector of track 4 from CP/M's point of view.

Directory and allocation block sizes	____________________________________

CP/M allocates the space on a volume to files in units called

"allocation blocks" (ALB's).  The size of an allocation block

varies, depending upon the size of the volume and the space

available in the bit map area in the ZBIOS which is used by BDOS

when the volume is logged on.  The ZBIOS contains a 1024-bit

(128-byte) area (called the "allocation vector" or ALV) for each

of the four devices assigned to the RD51D.  Therefore, the ALB

size for a volume is chosen such that there will be no more than

1024 ALB's on it.

The CP/M directory occupies one or more consecutive ALB's

beginning with ALB number 0.  Each entry in the directory

describes a file (or an extent of a large file) and contains the

numbers of the ALB's assigned to the file.  A directory entry

occupies 32 bytes.

The following table shows the ALB and directory sizes for the

various ranges of volume sizes (note--these are the sizes after

subtracting 32KB if the volume is a system volume):

Volume size   ALB size    Directory   ALB's for   ALB's per

(kilobytes)  (kilobytes)   entries    directory     entry	    ___________  ___________  _________   _________   _________

16.. 256        1		  96	      3		  16

257.. 512	      2		 128	      2		  16

513..2048	      2		 256	      4		   8

2049..4096	      4		 384	      3		   8

4097..8192	      8		 512	      2		   8

CP/M Support for RD51D Hard Disk			  Page 3

_________________________________________________________________

BIOS Data Tables	________________

Two data tables in ZBIOS describe the characteristics of each

CP/M disk device:  the "disk parameter header" (DPH) and the

"disk parameter block" (DPB).  There is one DPH for each disk

drive supported by the BIOS and there is one DPB for each

different supported disk type and size.  Each DPH contains a

pointer to the DPB which describes the disk currently mounted on

the DPH's drive.  The contents of these are described in chapter

10 of the CP/M 2 Alteration Guide.		  _______________________

The DECmate II BIOS adds a 12-byte prefix to each DPH to describe

some of the physical attribute of its device and to support

CP/M's logical blocking/deblocking algorithm.  When a device is

used to access an RD51D volume, the contents of these two tables

must be adjusted to reflect the size of the mounted volume.  Data

for the variable areas of these two blocks are set up by the

8BIOS get-hard-disk-characteristics function (command 01 08) and

passed to ZBIOS in two block transfers--a "D" record for the DPH

prefix and a "P" record for the DPB.

The D record contains 10 bytes which are moved into the prefix to	    ________

the standard DPH.  These bytes are shown below (two-byte numeric

values are stored low-order byte first):

Byte  Name    Contents	      ____  ____    ________

1     D_FLAG  Volume type:  81h if a CP/M volume; 00h

otherwise

2     D_TYP   Device type:  88h (constant)

3..4  D_BPS   Bytes per physical sector:  512 (constant)

5..6  D_SPT   Physical sectors per track:  16 (constant)

7..8  D_TPD   Number of tracks in the volume:  from the

volume's directory entry  (this value is

taken straight from the directory; it is not

adjusted for a "system" volume.)

9     D_SAB   Physical sectors per CP/M allocation block:

computed by the formula (ALB_SIZE * 2); this

value is meaningless for non CP/M volumes

10    D_1PS   Number of first physical sector on a track:

0 (constant)

The P record contains 15 bytes which constitute a complete DPB.	    ________

Several of these are derived from ALB_SIZE, which is the CP/M

allocation block size expressed as a number of kilobytes.

ALB_SIZE has one of the five values 1, 2, 4, 8 or 16.  The bytes

in a P record are shown below (two-byte numeric values are stored

low_order byte first):

CP/M Support for RD51D Hard Disk			  Page 4

_________________________________________________________________

Byte  Name    Contents	      ____  ____    ________

1..2  P_SPT   Logical sectors per track:  64 (constant)

3     P_BSH   Block shift factor:  the number of bits in

P_BLM

4     P_BLM   Allocation block mask:  (ALB_SIZE * 8) - 1

This is the number of 128-byte records in an

allocation block, minus 1.

5     P_EXM   Extent mask:  depends upon the allocation

block size and the number of allocation

blocks in the volume, thus:

if P_DSM < 256

then P_EXM - P_BLM shr 3

else P_EXM - P_BLM shr 4

where "shr" is a right-shift operation which

fills the vacated high-order bits with zeros.

6..7  P_DSM   Number of allocation blocks in the volume,

minus one (i.e. the highest ALB number)

8..9  P_DRM   Number of directory entries, minus one.  This

must agree with the size implied by the

number of ALB's assigned to the directory in

P_ALD, below.  There are 32 entries per

kilobyte.

10..11  P_ALD   Sixteen-bit map to reserve allocation blocks

for the directory.  Bits are set, beginning

with the high-order bit of byte 10 and

proceeding toward the low-order bit of byte

11, one for each ALB used for the directory.

For example, a volume with ALB_SIZE = 2K and

256 directory entries uses the first 4 ALB's

for the directory, so the bytes of P_ALD are

F0h, 00h and P_DRM is 255.

12..13  P_CKS   Directory checksum area size.  Not used for

RD51D volumes; always 0.

14..15  P-OFF   Offset to the track upon which the first ALB

starts.  0 for an RD51D data volume; 4 for a

system volume.

Non-CP/M Volume Access	______________________

Non-CP/M volumes can be accessed by a CP/M program by using

direct calls to ZBIOS which address physical sectors and transfer

data in 512-byte blocks.  such a volume cannot be accessed via

normal BDOS calls.  The 8-megabyte volume size limit does not

apply to non-CP/M volumes.

CP/M Support for RD51D Hard Disk			  Page 5

_________________________________________________________________

8BIOS Functions	_______________

The following 8BIOS functions are used for both diskettes and

hard disk volumes:

06  xx  Select disk drive

07  xx  Select disk track

08  xx  Select disk sector

01  06  Home (selects track 0)

01  07  Read disk sector

01  09  Write disk sector

01  0A  Get disk type code (code 88h has been added for

hard disk devices)

Function 02 10, get diskette characteristics, may be used only

for 5 1/4" diskettes.  Attempting to use it for 8" diskettes or a

hard disk device will return useless bytes.

The following new functions have been added to support the RD51D:

01  08  Get hard disk characteristics.  Returns DPH and			 _____________________________

DPB data to fill in tables in ZBIOS to reflect

the dimensions of the volume on the currently

selected device.  After sending this function

code to 8BIOS, the caller must issue two BLDRD

calls:  the first returns a 10-byte D record and

the second returns a 15-byte P record.

02  1F  Execute RD51D command.  This is a general			 _____________________

function to allow a Z80 program to execute any

RD51D command.  After sending this function

code, the program must issue one OUTBYT call to

send the right 8 bits of the controller command

code (the high 4 bits are always 0).  The

program must then call either BLDRD or BLDWRT to

transfer the data required by the command.  The

direction of the transfer and the number of

bytes transferred depend upon the command.

Following the block transfer, the program must

issue one INBYT call to receive the error code

from the RD51D (0 indicates no error).

Note:  This call does not properly handle SET

block and SET PHYSICAL ADDRESS commands; these

require 12-bit values.  This problem is to be

resolved.

R.F.Morse

DTN 223-6720

MLO3-2/E41

Rev 0, 11-Feb-83

DECmate II CP/M Technical Note 6:

RX50 Diskette Format for CP/M Release 2

_________________________________________________________________

This note describes the format and contents of the RX50 diskettes

used by the DECmate II version of the CP/M-80 operating system.

The description of the size, location and contents of the

CPMIMAGE.SYS file on a system disk applies to CP/M Release 2 for

RX50-based systems.

The RX50 diskette is a 5 1/4", soft-sectored, single-sided,

double-density diskette with a track spacing of 96 tracks per

inch.  The arrangement of address marks, sector headers, data

fields and gaps is that used by the 1793 LSI controller chip.  A

diskette contains 80 tracks, numbered 0 through 79.  Each track

contains 10 512-byte sectors, numbered 1 through 10.  This gives

a capacity of 5,120 (5K) bytes per track and 409,600 (400K) bytes

per diskette.

When used for CP/M, these diskettes may be either system or data							  ______    ____

disks.  A system disk contains the CP/M operating system and the

necessary supporting files to allow it to be bootstrap loaded

into an idle machine.  A data disk contains only CP/M files and

cannot be bootstrap loaded.  The first 12 bytes of track 1,

sector 1 are coded to mark these as CP/M disks and to distinguish

system from data disks.  The DECmate II hardware bootstrap loader

tests these bytes and will not attempt to load a non-system disk.

Track usage	___________

Tracks on a diskette are used as follows:

Tracks  Use	     ______  ___

0     On a system disk:  data for the SETUP screen,

keyboard map, and special user-defined display

character sets.  On a data disk:  sector 1 is

zero-filled to prevent any attempt to store SETUP

data on the disk and sectors 2..10 are undefined.

1     On a system disk:  bootstrap program to load the

system into an idle machine.  On a data disk:

sector 1 is coded to mark it as a CP/M data disk and

sectors 2..10 are undefined.

2..79   CP/M directory and data area.  These 78 tracks are

controlled by CP/M's file system.  They begin with a

128-entry CP/M directory and contain program and

data files.  On a system disk, the first directory

entry describes a protected file named CPMIMAGE.SYS

RX50 Diskette Format for CP/M Release 2			  Page 2

_________________________________________________________________

which occupies tracks 74..79 and contains the CP/M

system image and the extended DECmate firmware.  On

a data disk, all 78 tracks are usable for CP/M

files.

Record addressing	_________________

The CP/M file system always transfers data to and from its

callers in 128-byte records.  Four such records are recorded in			    _______

each 512-byte sector on an RX50 diskette.  The blocking and

deblocking of records is performed by the Z80 portion of the BIOS

(called ZBIOS; its companion on the 6120 part of the machine is

called 8BIOS).  From the CP/M point of view, a diskette contains

78 tracks numbered 2 through 79 and each track contains 40

records numbered 0 through 39.  These two parameters--track

[2..79] and record[0..39]--are passed by BDOS to the BIOS SETTRK

and SETSEC routines, respectively.  The ZBIOS codes divides the

CP/M record number by four and adds one to get the (l-origin)

physical sector number.  The remainder of this division is

multiplied by 128 to get the offset to the start of the record

within a sector.

Since the BIOS and disk controller cannot access physically

consecutive sectors without waiting for an extra revolution of

the disk, an interleave scheme is used to map logical sectors

into non-consecutive physical ones.  An every-other-sector (2:1)

interleave is used, as shown in the following table:

Logical      Physical

sector	    sector		      ______	    ______

1	       1

2	       3

3	       5

4	       7

5	       9

6             2

7	       4

8	       6

9	       8

10	      10

Since there are four CP/M records in each sector, records 0..3

are placed in physical sector 1, records 4..7 in sector 3, and so

on.  The actual sector translation table in ZBIOS (whose address

is in the DPH and is used by the SECTRAN routine in the BIOS)

operates on record numbers.  It contains 40 entries to convert a

0-origin record number into a 0-origin interleaved record number.

Thus its first eight entries are as follows:

RX50 Diskette Format for CP/M Release 2			  Page 3

_________________________________________________________________

Logical      Interleaved

record	      record		      ______	   ___________

0		 0

1		 1

2		 2

3		 3

4		 8

5		 9

6		10

7	 	11

8		16

9		17

etc.

There is no track-to-track skew in this system; physical sector 1

is the first logical sector on every CP/M track

Space allocation	________________

Space in the CP/M area is subdivided into allocation blocks which

contain 2K bytes (16 records) each.  The 78 tracks of the area

hold 390K bytes, so there are 195 allocation blocks, numbered

0..194.  Thus every file occupies space whose size is a multiple

of 2K.

File directory	______________

The CP/M file directory has 128 entry positions of 32 bytes each.

These occupy allocation blocks 0 and 1, which are the first 32

records (8 sectors) of track 2.

System image file	_________________

Each CP/M system disk contains a file named CPMIMAGE.SYS which

contains the CP/M system, ZBIOS, 8BIOS, and extended DECmate II

firmware.  This file is always represented by the first entry in

the directory.  To protect it from accidental damage and to

prevent if from disrupting wild-card operations such as PIP *.*,

it has the R/O and SYS attributes and it is assigned to user 15.

The directory entry describes the file as occupying the last 15

allocation blocks (numbers 180..194), or 30K bytes.  Physically,

this comprises all of tracks 74 through 79.

RX50 Diskette Format for CP/M Release 2			  Page 4

_________________________________________________________________

The CPMIMAGE.SYS file is divided into three parts:

The CP/M-80 system image.  This comprises the CCP (2048	     ________________________

bytes), BDOS (3584 bytes), and ZBIOS (2048 bytes).  These

7.5K bytes correspond to the standard image described in

Digital Research's "CP/M 2 Alteration Guide".  This data is

stored in standard interleaved file format and can be read

and written by any CP/M program (after conquering the R/O

and user 15 restrictions).

Note:  The first record of this file is the beginning

of the CCP.  This contains the command input buffer,

which can be patched with a command to be executed

automatically after bootstrap loading.  Be sure,

however, to rewrite the file IN PLACE--the boot loader

code expects to find the system image in the exact

sectors described above.

The 8BIOS image.  This is the code executed by the 6120	     _______________

processor.  All calls to ZBIOS for I/O services are passed

to 8BIOS, which has access the the DECmate's I/O devices.

It occupies parts of tracks 1, 75, 76 and 77 (see Appendix

A).

DECmate firmware.  This code supports the DECmate II screen	     ________________

and keyboard handling.  It occupies all of tracks 78 and 79

and is loaded in to the 6120's panel memory during bootstrap

loading.

Diskette initialization	_______________________

The CP/M utility program DISKINIT is used to initialize a

diskette as either a system or data disk.  For system disks, it

copies tracks 0, 1 and 74..79 from another system disk and writes

a CP/M directory on track 2 whose first entry describes

CPMIMAGE.SYS and whose remaining entries are unused,  For data

disks, it writes dummy information in sector 1 of tracks 0 and 1

and writes a CP/M directory on track 2 with all its entries

unused.

Note that any diskette to be used with the DECmate II must be

physically formatted by its vendor with 10 512-byte sectors on

each of 80 tracks.  The DECmate II disk controller cannot rewrite

a diskette's format data.

RX50 Diskette Format for CP/M Release 2			  Page 5

_________________________________________________________________

Interchange with other DEC systems	__________________________________

The CP/M parameters used for the DECmate II RX50 diskettes are

the same as those used by the Rainbow for its CP/M-80/86 and by

the Professional for its CP/M-80.  To summarize, these parameters

are:

1)  File area in tracks 2..79;

2)  Physical sector interleave sequence of

1,3,5,7,9,2,4,6,8,10;

3)  Allocation block size of 2K;

4)  Directory containing 128 entries.

Any DECmate II CP/M disk--system or data--can be used as a data

disk on a Rainbow or Professional CP/M system.  A DECmate II

system disk cannot, of course, be bootstrap loaded as a system

disk on either of those systems.

RX50 Diskette Format for CP/M Release 2			  Page 6

_________________________________________________________________

Appendix A:  Contents of System Image File

A CP/M system disk has a bootstrap loader and system code stored	       ______

in tracks 1 and 74 through 80.  The contents of each sector in

those tracks is described below.  All references are to physical

tracks (0..79) and 512-byte sectors (1..10).

Secondary boot	______________

The secondary boot code occupies one sector (track 1, sector 1).

It is written in 12-bit mode and occupies 256 (octal 400) words.

This is the code which is brought in by the firmware loader

during a power-on boot, and its first few bytes contain the DEC

standard boot-block code which marks a CP/M system disk.  The

secondary boot code source file is called BOOT2.

Tertiary boot	_____________

The tertiary boot code occupies two sectors (track 1, sectors 2

and 3).  It is written in 12-bit mode and occupies 512 (octal

1000) words.  This code is loaded into field 7 by the secondary

boot, and it reads the actual 8BIOS code into fields 0, 1 and 2.

The tertiary boot code source file is called BOOT3.

8BIOS	_____

The 8BIOS code occupies 30 sectors on parts of tracks 1, 77, 76

and 75.  It is written in packed mode (two words in each three

bytes) and is 10240 (octal 24000) words long.  This code is

loaded into fields 0, 1 and 2 by the tertiary boot.  The tertiary

boot contains the following table which determines which sectors

are read and in what order:

RX50 Diskette Format for CP/M Release 2			  Page 7

_________________________________________________________________

Track     Sector			   _____     ______

1		 4    Start field 0

1		 6

1		 8

1		10

1		 5

1		 7

1		 9

77		 2

77		 4

77		 6

77		 8

77		10

77		 3    Start of field 1

77		 5

77		 7

77		 9

77		 1

76		 4

76		 6

76		 8

76		10

76		 2

76		 5

76		 7

76		 9    Start of field 2

76		 1

76		 3

75		 6

75		 8

75		10

Table A-1.  8BIOS Image Sector Map

CP/M system image	_________________

The CPM system image occupies 15 sectors on tracks 74 and 75 and

is written in 8-bit mode.  It is 7680 bytes long and is

subdivided into three parts:  CCP (4 sectors, 2048 bytes), BDOS

(7 sectors, 3584 bytes), and ZBIOS (4 sectors, 2048 bytes).  The

sequence of sectors for this image must correspond to the first

15 physical sectors (60 CP/M logical records) of the CPMIMAGE.SYS

file, since the CP/M image is installed by using ordinary BDOS

file I/O calls.

RX50 Diskette Format for CP/M Release 2			  Page 8

_________________________________________________________________

Track     Sector			   _____     ______

74		 1    CCP, 2048 bytes

74		 3

74		 5

74		 7

74		 9    BDOS, 3584 bytes

74		 2

74		 4

74		 6

74		 8

74		10

75		 1

75		 3    ZBIOS, 2048 bytes

75		 5

75		 7

75		 9

Table A-2.  ZBIOS Image Sector Map

DECmate firmware	________________

The extended firmware which is loaded into 6120 panel memory

occupies all of tracks 78 and 79.

Uncommitted sectors	___________________

There are two uncommitted sectors on track 75.  Sectors 2 and 4

can be used to add 682 (octal 1252) words to 8BIOS, or 1024 bytes

to ZBIOS or can be split between them.

Track     Sector			_____     ______

75	      2

75	      4

In order to use these to expand 8BIOS, the sector tables in the

tertiary boot BOOT3 and the utility program PUT234 which writes

the code on a system diskette, must be modified accordingly.  In

order to expand ZBIOS, routines RDCPM and LODCPM in 8BIOS must be

modified.

R.F.Morse

DTN 223-6720

MLO3-2/E41

Rev 7, 20-Jun-83

DECmate II CP/M Technical Note 7:

8BIOS Interface Codes for Release 2

_________________________________________________________________

This Tech Note has not been found -- yet.

Rev 1, 20-Jun-83

DECmate II CP/M Technical Note 9:

System Bootstrap Operation

_________________________________________________________________

This note describes the operation of the DECmate II system

bootstrap under three conditions:

-- At power-on,

-- when Remove and Do are pressed while the SETUP screen is

displayed,

-- When the panel request instruction PR3, 6 is issued.

This description applies to systems which have panel code ROM

chips at level 19 or greater.  These support the optional RD51D

hard disk system as well as the standard RX50 diskette drive.

Power-on	________

When system power is turned on, the ROM code first does basic

system checks (these result in the screen patterns full of "U"

and "a"), and then searches for a bootable system disk in either

RX50 drive 0 or (if installed) on the RD51 hard disk.  This

power-on boot search operates as follows:	____________________

REPEAT

IF RX50 drive 0 is ready

AND track 1 sector 1 has bootable-disk code

AND track 0 sector 1 has valid firmware

THEN attempt to load the system firmware and the

operating system from diskette

ELSE display the flashing diskette picture.

IF RD51 is installed and ready

AND block 0 indicates valid firmware

AND there is a volume marked with the

"startup" flag which is bootable

THEN load the system firmware, mount the startup

volume on device 0, and attempt to

load the operating system from it.

UNTIL a system is successfully loaded.

For a system which does not have an RD51, this simply attempts to

boot from RX50 drive 0 until a valid bootable disk is inserted.

For a system with both RX50 and RD51 drives, it will boot from

System Bootstrap Operation				  Page 2

_________________________________________________________________

whichever becomes ready first.  Thus, if the RD51 has a valid

startup volume and no diskette is in drive 0, the RD51 will be

booted.  If there is a bootable diskette in drive 0, it will be

used in preference to the RD51.  The normal spinup delay of 15 to

20 seconds for the RD51 gives ample time to insert a diskette and

close its door to preempt a hard-disk boot.

REMOVE option of SETUP	______________________

When the standard system SETUP screen is displayed, pressing the

Remove and Do keys initiates a power-on boot search operation				       ____________________

exactly as described above for power-on.  Thus if a user has been

running from the RD51 and wishes to change to a diskette-resident

system (e.g. WPS), he can do this by entering SETUP, inserting

the WPS system disk in diskette drive 0, and pressing Remove and

Do.  Conversely, if he wishes to go from a diskette-resident

system to the RD51, he need only be sure that diskette drive 0 is

not ready (door open) before pressing Remove and Do.

Panel Request 3	_______________

Function 6 of panel request 3 is invoked by the following 6120

instruction sequence:

PR3

0006

-1

The action performed depends upon the contents of the

accumulator, as follow:

AC   Action		  __   ______

0   Display the SETUP screen

-1   Display the 6120 HALT screen

-2   Attempt to boot from RX50 drive 0

-3   Attempt to boot from the RD51 volume presently

mounted on logical device 0.

If either boot attempt fails, the system then goes to the

power-on boot search process described above.

Warning:  This PRQ should not be used to change between	_______			  ___

diskette-loaded and hard-disk-loaded systems.  This is because

the panel code firmware is not reloaded when this PRQ is issued.

When the firmware was loaded, it noted which device--diskette or

hard disk--it was loaded from, and thereafter always refers to

System Bootstrap Operation				  Page 3

_________________________________________________________________

that device when the SETUP function is invoked.  Therefore, using

PR3, 6 to change from, say, a diskette-based operating system to

one loaded from the hard disk will not alter the firmware, which

will still attempt to use the diskette for a SETUP operation.

R.F.Morse

DTN 233-6720

MLO3-2/E41

Rev 0, 19-Apr-83

DECmate II CP/M Technical Note 10:

Z80 Memory Map for Release 2

_________________________________________________________________

This note describes the allocation of the 65536. (64K) bytes of

memory in the Z80 APU when operating under CP/M 2.2 release 2.

There are two maps:  one for diskette-only systems and one for

systems which support an RD51 hard Disk.

Diskette-only system (nominal size of 63K):	____________________

Usage		   Addresses	      Length		 _____		   _________	      ______

System parms	   0000..00FF	    100    256.

TPA		   0100..DFFF	   DF00  57088.

CCP		   E000..E7FF	    800   2048.

BDOS		   E800..F5FF	    E00   3548.

BIOS code	   F600..FBFF	    600   1536.

BIOS work area    FC00..FFFF	    400   1024.

Diskette and hard disk system (nominal size of 62K):	_____________________________

Usage		   Addresses	      Length		 _____		   _________	      ______

System parms	   0000..00FF	    100    256.

TPA		   0100..DBFF	   DB00  56064.

CCP		   DC00..E3FF	    800   2048.

BDOS		   E400..F1FF	    E00   3548.

BIOS code	   F200..F9FF	    800   2048.

BIOS work area    FA00..FFFF	    600   1536.

Application programs occupy the transient program area (TPA).

Large application programs usually overlay the CCP (console

command processor), so the limit on their size is the sum of the

sizes of the TPA and the CCP from the tables.

The area described as "BIOS code" is loaded from the CP/M system

image during a cold start.  It may actually have some work area

at its end if the assembled code is smaller than the maximum.

The top of the BIOS code cannot exceed address FD00 in any

system, since the monitor program which transfers data from the

6102 to the Z80 during a cold start begins there.

R.F.Morse

DTN 223-6720

MLO3-2/E41

Rev 0, 20-Jun-83

DECmate II CP/M Technical Note 11:

Console Standard Modes for Release 2

_________________________________________________________________

This note describes what CP/M does upon cold starts and warm

starts to set the console and keyboard to a standard state.  The

large number of programmable operating modes, display character

sets, and keyboard interpretations means that an application

program can leave the system in such a state that it is difficult

or impossible to enter a command to CP/M or to execute subsequent

applications.  Therefore, the CP/M BIOS takes the steps described

below whenever it receives control after initial loading (a "cold

start") and after every application program ends (a "warm

start").

CP/M establishes the following conditions for the display:

-- USASCII character set

-- ANSI level 2 control sequences

-- 7-bit mode (insensitive to the high-order bit of any

character sent to the screen.

-- full-screen scrolling region

-- character-replace mode

-- no auto-wrap at the end of a line

-- line-feed mode (not new-line)

-- normal character rendition

-- cursor visible

For the keyboard:

-- keyboard unlocked

-- top row function keys active

-- the three keys above the "+" key generate ESC, BS and LF

-- function keys emit the two-character CSI prefix "ESC ["

-- 7-bit mode

-- auto-repeat enabled

-- keypad in numeric mode

-- arrow keys generate cursor-positioning codes

The following things are not changed:

-- screen width

-- cursor style

-- keyclick

-- black or white screen background

-- scrolling speed

-- "caps lock" vs. "shift lock"

-- COMPOSE key activation

Console Standard Modes for Release 2			  Page 2

_________________________________________________________________

-- current case of the keyboard

Note that the preceding means that two of the items on the SETUP

screen are overridden by CP/M:  the cursor is made visible and the

terminal mode is made ANSI (shown as "VT100" on the setup menu).

Application programs may issue the appropriate control sequences

to alter the display and keyboard features to suit themselves.

In particular, a communication program may choose to change to

full level 2 mode with 8-bit characters in order to emulate a

VT2xx terminal, and a text processing program may use the full

DEC multinational character set and 8-bit character codes.

Warm start actions	__________________

The following actions take place (in the order shown) whenever

CP/M is initially loaded and whenever it reloads its console

command processor (CCP) following the execution of an application

program:

-- Enter ANSI mode (in case terminal is in VT52 mode)

ESC <

-- Terminate control string

ESC \

-- Select level 2 mode with 2-character CSI

ESC [ 6 2 ; 1 " p

-- Enable keyboard auto-repeat

ESC [ ? 8 h

-- Make cursor visible

ESC [ ? 2 5 h

-- Activate the ESC, BS, and LF keys

PR3, 0010, -1  with AC=0000

-- Select 7-bit host mode

PR3, 0014, -1  with AC=0000

(This sequence can be sent to the console at any time by calling

the BIOS OUTBYT routine to send command 02 23, as described in

DTN#7, 8BIOS Interface Codes.)	       _____________________

Invoking 8-bit mode	___________________

Application programs which wish to use the full 8-bit

capabilities of the terminal firmware must use an extended BIOS

command (see DTN#7) to execute a panel request in the 6120.  To

invoke 8-bit mode, send command 02 21 followed by the two

hexadecimal data bytes 0C and 01 (this executes the instruction

Console Standard Modes for Release 2			  Page 3

_________________________________________________________________

sequence PR3, 0014, -1  with 0001 in the AC).

The ANSI sequence  ESC [ 6 2 ; 1 " p  which invokes level 2

operation also sets all the top row function keys to generate

multicharacter control sequences.  If the one-character ESC, BS,

and LF codes are needed from the keys so labelled, the panel

request listed above under warm start actions must be issued

after sending the ANSI sequence.  This can be accomplished by	_____

sending extended BIOS command 01 21 followed by data bytes 08 and

00.

Compatibility with CP/M release 1	_________________________________

The standard console modes described above are generally the same

as those established by a warm start in CP/M release 1 for the

DECmate II.  Release 2 is more thorough:  it also sets the

character set to USASCII, turns off autowrap, and sets the

scrolling region to full screen and absolute origin mode.  Unlike

release 1, the case shift of the keyboard is not altered during a

cold start.  This means that it will be lower-case if the system

firmware was loaded as part of the cold start (as in loading from

a diskette), but may be either case if CP/M was invoked from some

other system (e.g. the master menu) in a Winchester environment.

R.F.Morse

DTN 223-6720

MLO3-2/E41

Rev 0, 27-Jun-83

DECmate II CP/M Technical Note 12:

Format of Hard Disk Backup Diskettes

_________________________________________________________________

This note describes the manner in which the contents of an RD51

hard disk volume are transcribed to one or more RX50 diskettes to

create a literal backup copy.  These backup diskettes are written

by the "HD" utility program (both the Master Menu and CP/M

versions) and are intended only for use by that program--they are

not compatible with any operating system's file format.

Each backup diskette contains up to 790 blocks from a hard disk

volume.  As many diskettes as required by the volume's size are

used, and all save the last diskette contain 790 blocks from the

volume.  The diskettes are numbered in sequence, starting from 1.

This sequence number and the number of diskettes used to hold the

volume are recorded in a label record, whose contents are

described below.

Volume data arrangement	_______________________

A volume is copied to diskettes by reading its blocks in

ascending order from block 0 through block n-1, where n is the

number of blocks in the volume.  Each group of 10 blocks is

written onto one track of a diskette.  To enhance performance,

the diskette sectors are written in an interleaved pattern which

enables writing (or reading) a track in two revolutions.  The

sequence of sector numbers is 1, 3, 5, 7, 9, 2, 4, 6, 8, 10.

Thus, for example, hard disk block 0 is written into diskette

sector 1, block 1 into sector 3, block 9 into sector 10, and

block 10 into sector 1 of the next diskette track.

Seventy-nine tracks of each diskette (except possibly the last)

are used in descending track number sequence from track 79 down

through track 2, and then track 0 (track 1 is reserved for the

label).  This apparently perverse ordering is intended to reduce

the chances that an operating system will recognize the first

diskette of a backup copy as an OS-format diskette.

The contents of the tracks of the last diskette of a backup set

which are not required to hold volume data are unspecified.

Likewise, the contents of the unused sectors in the last track of

the last diskette are also unspecified.

Format of Hard Disk Backup Diskettes			  Page 2

_________________________________________________________________

Diskette labels	_______________

A label record is written into sector 1 of track 1 of each backup

diskette.  (Sectors 2 through 10 of track 1 are not used; their

contents are unspecified.)  This label identifies the volume from

which the backup was made and contains the date and time of the

backup.  The label record is identical for every diskette of a

backup set, except for the sequence number.  When a backup copy

is read to restore it to a hard disk, the HD utility checks that

all the diskette labels are identical in order to prevent mixing

diskettes from backup copies made at different times.

The contents of the label are described below.  Byte positions

are given as decimal offsets from the beginning of the sector.

All unused bytes (0..255 and 366..511) are binary zero.

Bytes      Contents	    _________  _____________

256..263  Signature pattern:  the 7 ASCII characters

"RDBackup" followed by a NULL

264..287  24-byte volume directory entry, exactly as read

from the hard disk's directory, except

as noted:

264..271      volume name

272..275      passwords

276..277      starting block--set to 0

278..279      volume size in units of 8K (low-order byte

first)

280      volume flags--set to hex 10 ("valid volume"

value)

281      OS-type flags

282..287      Miscellaneous data

288  Diskette sequence number:  8-bit unsigned binary,

beginning with 1

289  Number of diskettes in backup set:  8-bit unsigned

binary

290..298  Date of backup:  8 ASCII characters "mm/dd/yy"

followed by a NULL

299..304  Time of backup:  5 ASCII characters "hh:mm"

followed by a NULL

305..365  User's remark:  0 to 60 ASCII characters, the last

followed by a NULL

R.F.Morse

DTN 223-6720

MLO3-2/E41

Rev 0, 7/1/83 Fri

DECmate II CP/M Technical Note 13:

Loading and Using Extensions to 8BIOS

_________________________________________________________________

This note describes the use of the 8BIOS reverse mini-monitor

(RMM) by CP/M-80 application programs.  The RMM features are a

part of release 2 of DECmate II CP/M 2.2.  The RMM is used to

load data to and from the 6120 program memory.  These data may be

stored in either 8 or 12 bit modes; the data content is

irrelevant to the RMM.

Uses for such data may be

o   Program overlay storage for the Z80

o   Data storage for the Z80

o   Additional BIOS functionality required by a CP/M

application and not presently implemented by Digital

For purposes of explanation, the reader is assumed to

be familiar with DTN #7 - 8BIOS Interface Codes for Release 2 of

DECmate II CP/M-80.  With reference to DTN #7 - there are nine

8BIOS functions available for implementation of the RMM.  These

are:

02 17	 Set block transfer address

02 18	 Set block transfer count

02 19	 Block transfer to 6120 in 12-bit mode

02 1A	 Block transfer to 6120 in 8-bit mode

02 1B	 Block transfer from 6120 in 12-bit mode

02 1C	 Block transfer from 6120 in 8-bit mode

02 1D	 Start 6120 execution at transfer address

02 1E	 Restore interrupt vector

0A xx	 Reserved for user functions

A thorough knowledge of PDP-8 architecture and programming is

assumed by the user of the RMM if it is desired to add 8BIOS

functionality by means of the RMM.  Guidelines for memory and

interrupt usage are set forth in this note.

Loading and Using Extensions to 8BIOS			  Page 2

_________________________________________________________________

Use of the RMM for data storage	_______________________________

Regardless of the user's choice of data length (8-bit or 12-bit)

the order of function calls remains the same.  To exchange a

block of data between the Z80 and 6120 main memory, use the

following procedure:

Set the 6120 transfer address - this is a 15-bit address

transferred in two bytes.

CAUTION -- the user is not protected from overwriting any				   ___

6120 memory area!  Consult the section in this note entitled

'6120 Memory Map'.

Set the 6120 transfer count - this is a positive 12-bit

number of bytes (or 12-bit words) to be transferred in this

block.  The minimum number of byte/words transferred per

block transfer allowed is one (1); maximum is 4096 decimal.

The transfer count function requires two data bytes to

describe the 12-bit count.  Note that a transfer count of

zero will result in a full field (4096) transfer.  Field

boundaries can not be crossed during a single block transfer

operation.  Attempts to do so will result in a wrap-around

within the field boundary described by the transfer address.

Request the block transfer function (either 8-bit or 12-bit

mode).  The next Z80 program function should be to perform a

BLKWRT (to move data from the Z80 to the 6120) or a BLKRD (to

move data from the 6120 to the Z80).  The 6120 counts the

number of transfers and terminates the block transfer.  Note

- the user program is responsible for initializing the Z80

for performing the BLKWRT or BLKRD (see DTN #7).

Loading and Using Extensions to 8BIOS			  Page 3

_________________________________________________________________

6120 Memory Map	_______________

Field    Octal Location(s)    Contents - Description	_________________________________________________________________

0	 0000-0200	      RESERVED - 8BIOS data storage

0201		      Address of communications interrupt

service routine (ISR)

0202		      Address of printer input ISR

0203		      Address of keyboard ISR

0204		      Address of printer output ISR

0205		      Address of modem ISR

0206		      Address of 100 Hz clock ISR

0207		      Address of function Axx CALL

0210		      Field of function Axx CALL

0211		      Interrupt Vector Terminator

0212-0221	      AVAILABLE for use by RMM for one				      _________			   ___

cross-field interrupt call

0222-7777	      RESERVED - 8BIOS program code and

stacks

1	 0000-7777	      RESERVED - 8BIOS program code

2	 0000-7777	      RESERVED - 8BIOS program code and

data storage

3	 0000-7777	      AVAILABLE for use by RMM				      _________

4	 0000-7777	      RESERVED - 8BIOS data storage

5	 0000-7777	      RESERVED - 8BIOS data storage

6	 0000-7777	      AVAILABLE for use by RMM				      _________

7	 0000-7777	      AVAILABLE for use by RMM				      _________

Use of the 8BIOS Interrupt Vector	_________________________________

There are eight fixed locations at the beginning of 8BIOS which

contain the interrupt vector locations.  For users desiring to

replace interrupt service code or to add additional BIOS

functions, the following discussion is of interest.

The interrupt vector resides in 6120 main memory field 0,

beginning at location 201.  As described in the '6120 Memory

Map', each location from 201 to 205 contains an address pointer

into the beginning of reserved interrupt service routines.  The

beginnings of each ISR must reside in field 0 of the 6120 memory.			       ____

Once execution of an ISR begins in field 0, the user is free to

transfer control into an AVAILABLE field for his/her own use as

an alternative ISR.

Loading and Using Extensions to 8BIOS			  Page 4

_________________________________________________________________

Room has been reserved from 0212-0221 for cross-field calls to a

user supplied ISR.  Since it is expected that user-supplied ISRs

will principally be used to augment communications support, only

one reserved area has been made available in field 0 which does

not require the user to overwrite a reserved portion of 8BIOS.

If a second type of ISR must be supplied by the user, then it is

at the user's discretion to overwrite a portion of 8BIOS code.

That code is non-recoverable except by re-booting CP/M.

Location 207 (octal) in the Interrupt Service Vector contains the

address that the user wishes to transfer control to for

extensions to 8BIOS through the use of Function Axx.  Location

210 is used to hold the field (3, 6 or 7) that the Axx parser

resides in.  The format for the field is 'X0' where 'X' is the

field number, ie: 30 in location 210 means that the ISR resides

in field 3.

With reference to DTN #7, note that Function 1xx is broken into a

main function (100 hex), and sixteen sub-functions (00 - 0F hex).

When the Z80 program requests any function from 100h to 10Fh,

8BIOS parses the two low order digits in the function call to

determine the sub-function; then executes that function.  If

additional data are required to be passed to or from the 6120 by

the function (ie: Function 10A - Get Disk Type Code), then those

data are sent and received as a part of the function, with

control returning to the 8BIOS IDLE routine.

Therefore, it is  up to the user to perform similar parsing for

Function Axx.  If only one additional BIOS function is desired by

the user, then the two low order digits passed (Axx) may be also							 __

used to pass a byte of data to the user function.

All Axx function parsing and processing; stack house-keeping and

non Digital-supplied interrupt service is the responsibility of

the user.

When the user wishes to return to basic 8BIOS functionality, and

the BIOS control path has not been disturbed (through use of

Function 21D - Execute 6120 code), Function 21E (Restore

Interrupt Vector) may be issued.  Function 21E replaces the

present contents of the Interrupt Service Vector with the

original contents for locations 0201-0210.  All control and ISR

structure is then returned to the Digital-supplied 8BIOS.

WARNING - no flags, variables or locations are guaranteed to be

preserved in any future releases of the DECmate II CP/M product

unless specified within this document.

Program Structure in 8BIOS	__________________________

The BIOS code supplied by Digital makes use of the two data

stacks available in the 6120.  Stack one is the CALL/RETURN

Loading and Using Extensions to 8BIOS			  Page 5

_________________________________________________________________

stack; stack two is a DATA stack.  An important distinction is

made between local routines and cross-field callable routines.		     ______________	_____________________________

Local routines are those which reside in a given field, and can

only be called from, and return control to that field.

Cross-field callable routines are identified in BIOS listings as

'ROUTINE NAME X0', where x0 is a field value (00, 10, 20).  These

routines can be called from any field, and automatically return

control to the proper PC in the calling field.

For both types of routines, stack one is used to preserve the

return address, and field if necessary.  In the following

examples, ROUNAM stands for a routine name; field 7 is used only

for example purposes.

Example of code to call a routine starting at location 0200, as a

local routine:	_____________

PPC1	   /form return address and save on stack 1

JMP I ROUNAM   /jump indirect to ROUNAM

Example of code to return to caller as a portion of a local							      _____

routine:	_______

RTN1	   /return to PC from stack 1

Example of code to call a cross-field routine, starting in field				  ___________________

7:

CLA	           /start with cleared AC

RIF		   /place present instruction field in AC

PAC1	   /place present instruction field on stack 1

CIF 70	   /change to called routine instruction field

PPC1	   /form return address and save on stack 1

JMP I ROUNAM   /jump indirect to ROUNAM

Example of code to return to caller as a portion of a cross-field							      ___________

routine:	_______

POP1	   /get return address from stack 1

DCA GOTO	   /save as the return address

POP1	   /get return instruction field from stack 1

TAD (CIF 00    /add a CIF 00 instruction to form full CIF

/needed

DCA .+1	   /and place the built CIF instruction in line

0		   /position holder for the CIF

JMP I GOTO	   /jmp indirect back to caller

It is important to understand the implications of Function Axx in

terms of CALL and RETURN.  The Axx function is defined to be a

cross-field callable routine.  As such, the Interrupt Vector has

Loading and Using Extensions to 8BIOS			  Page 6

_________________________________________________________________

a dummy routine address when BIOS is startup up.  This feature

protects the user from an inadvertent call to Function Axx before

the RMM has been used to place a new Axx vector in place.  The

dummy routine merely returns control to the BIOS IDLE routine.

Once the user describes, via the Interrupt Vector, the field and

address of the user's installed Function Axx, a call to that

function will be a cross-field call.  Therefore, stack 1 will

contain (at the top) the return address, and then one level down,

the return instruction field.  A return  from this routine is then

responsible to use the data on the stack to do a 'proper' return.

Furthermore, the active data field is not changed by calling

Function Axx, it is the responsibility of ALL cross-field called

routines to: 1) establish the present data field to be what the

routine requires; and 2) preserve the data field found at entry

into the routine so that when control is returned to the calling

code, the data field will be properly set for that code.

C.P.Gerstle

MLO3-2/E41

DTN 223-2703