RTS/8 DECNET/8

User's Guide

Order No. AA-5184A-TA

DISCLAIMER

This document file was created by scanning  the

original  document and then editing the scanned

text.  As much as possible, the  original  text

format  was restored.  Some format changes were

made to insure this  document  would  print  on

current laser printers using 60 lines per page.

The  original  spelling  and  grammar  has been

preserved.

1-Jun-1996

digital equipment corporation - maynard, massachusetts

First Printing, February 1977

The information in this document is subject to change  without  notice

and  should  not  be  construed  as  a commitment by Digital Equipment

Corporation. Digital Equipment Corporation assumes  no  responsibility

for any errors that may appear in this document.

The software described in this document is furnished under  a  license

and  may  be  used or copied only in accordance with the terms of such

license.

Digital Equipment Corporation assumes no responsibility  for  the  use

or  reliability  of  its software on equipment that is not supplied by

DIGITAL.

Copyright - 1977 by Digital Equipment Corporation

The postage prepaid READER'S COMMENTS form on the last  page  of  this

document  requests the user's critical evaluation to assist us in pre-

paring future documentation.

The following are trademarks of Digital Equipment Corporation:

DIGITAL		  DECsystem-10	     MASSBUS

DEC		  DECtape	     OMNIBUS

PDP		  DIBOL		     OS/8

DECUS		  EDUSYSTEM	     PHA

UNIBUS		  FLIP CHIP	     RSTS

COMPUTER LABS	  FOCAL		     RSX

COMTEX		  INDAC		     TYPESET-8

DDT		  LAB-8		     TYPESET-10

DECCOMM		  DECsystem-20	     TYPESET-11

2/77-14

CONTENTS

Page

PREFACE							       vii

CHAPTER  1		INTRODUCTION				       1-1

1.1	DEFINITION OF COMPUTER NETWORKS		       1-1

1.2	DEFINITION OF DECNET/8			       1-2

1.3	STRUCTURE OF DECNET/8			       1-2

1.4	USER CAPABILITIES			       1-5

1.4.1	  Terminal Communication Utility	       1-5

1.4.2	  Network Information Program		       1-6

1.4.3	  Communication Between User-Written Programs  1-6

CHAPTER  2		RTS/8 INTERTASK COMMUNICATION		       2-1

2.1	RTS/8 ASSEMBLY LANGUAGE FACILITIES OVERVIEW    2-1

2.1.1	  RTS/8 SEND Executive Request (ER)	       2-1

2.1.2	  Asynchronous System Trap (AST)	       2-3

2.1.3	  Connect Control Block (CCB)		       2-3

2.1.4	  Basic Procedure			       2-3

2.2	ISSUING DECNET/8 REQUESTS		       2-4

2.2.1	  DECNET/8 Function Calls		       2-5

2.2.2	  CONINI				       2-8

2.2.3	  CONCNF				       2-12

2 2 4	  CONREJ				       2-14

2 2 5	  DISCON				       2-16

2.2.6	  TRNSMT				       2-18

2.2.7	  INTRPT				       2-21

2.2.8	  RCVMSG				       2-23

2.2.9	  RCNMSG				       2-25

2.2.10	  RCVSM					       2-27

2.2.11	  TRNSM					       2-30

2.3	DECNET/8 AST ROUTINES			       2-31

2.4	CONNECT CONTROL BLOCK			       2-35

CHAPTER  3		TERMINAL COMMUNICATIONS UTILITY (TLK)	       3-1

3.1	INTRODUCTION				       3-1

3.2	INVOKING TLK				       3-1

3.3	EXAMPLES				       3-2

3.4	ERROR MESSAGES				       3-2

CHAPTER  4		NETWORK INFORMATION PROGRAM (NIP)	       4-1

4 1	INTRODUCTION				       4-1

4 2	PHYSICAL LINE STATUS			       4-1

4.3	LOGICAL LINK STATUS			       4-2

4 4	PHYSICAL LINE ERROR STATUS		       4-3

4 5	SAMPLE NIP PRINTOUT			       4-5

CHAPTER  5		RTS/8 DECNET/8 SYSTEM GENERATION	       5-1

5.1	NETWORK GENERATION OVERVIEW		       5-1

iii

CONTENTS (Cont.)

Page

5.2	DECNET/8 SYSTEM GENERATION PROCEDURES	       5-2

5.3	USER TASK PARAMETERS			       5-2

5.4	PARAMETER FILE				       5-4

5.4.1	  DECNET/8 Task Priorities		       5-6

5.4.2	  DDCMP and NSP Parameters		       5-7

5.4.3	  NIP Parameters			       5-9

5.4.4	  TLK and LSN Parameters		       5-11

5.5	NODE NAME TABLE				       5-11

5.6	INTERRUPT SERVICE ROUTINES		       5-13

5.7	LINE CONTROL FILE			       5-14

5.8	ASSEMBLY PROCESS			       5-16

5.9	LOAD PROCESS				       5-16

5.10	SAVE					       5-16

5.11	SAMPLE DECNET/8 PARAMETERS		       5-17

5.12	EXAMPLE OF A LINE CONTROL FILE		       5-25

CHAPTER  6		INTERNAL NETWORK DESCRIPTION		       6-1

6.1	NETWORK OVERVIEW			       6-1

6.1.1	  Protocol Fields			       6-1

6.2	DDCMP					       6-2

6.2.1	  Message Formats			       6-3

6.2.2	  DDCMP Executive Routine		       6-7

6.2.3	  Events				       6-8

6.2.4	  NSP to DDCMP Requests			       6-9

6.2.5	  Input					       6-11

6.2.6	  Output				       6-13

6.2.7	  Message Timing			       6-14

6.2.8	  Interrupt Service Routines and Buffer

Formats 			               6-15

6.2.9	  Line Control Blocks			       6-18

6.2.10	  CRC-16 Routine			       6-22

6.2.11	  Major DDCMP Support Subroutines	       6-23

6.3	NSP OVERVIEW				       6-24

6.3.1	  NSP Message Format			       6-25

6.3.1.1	  Extensible Binary and Extensible ASCII       6-27

6.3.2	  Overall Layout of the NSP Task	       6-28

6.3.3	  DDCMP Function Call Completion Processing    6-30

6.3.4	  DDCMP Events				       6-30

6.3.5	  User Function Calls			       6-31

6.3.6	  Major NSP Support Routines		       6-41

6.3.7	  NSP Tables				       6-43

CHAPTER  7		STAND-ALONE USE OF DDCMP		       7-1

7.1	STAND-ALONE USE OF THE DDCMP TASK	       7-1

7.1.1	  Bringing Up a Physical Link		       7-1

7.1.2	  Transmitting and Receiving Messages	       7-2

7.1.3	  Breaking the Physical Link		       7-2

7.2	ISSUING DDCMP FUNCTION CALLS		       7-2

7.2.1	  STARTUP				       7-3

7.2.2	  TRNSMT				       7-4

7.2.3	  HALT					       7-4

7.3	DDCMP EVENTS				       7-5

7.3.1	  RCVMSG				       7-5

iv

CONTENTS (Cont.)

Page

7.3.2	  LINDWN				       7-6

7.3.3	  STRT RECEIVED				       7-6

7.3.4	  Sample DDCMP Support Subroutines	       7-7

APPENDIX A	     GLOSSARY						A-1

INDEX							    Index-1

FIGURES

FIGURE  1-1     Computer Network Composed of Two Nodes

Connected by a Physical Link		       1-1

1-2     Functional Levels of DECNET/8		       1-2

1-3     Components of the Program Dialogue Level	       1-3

1-4     Sample Message Transmission from Node 1

to Node 2					       1-4

1-5     Terminal Communication Utility (TLK) Used

on Two PDP-8s				       1-6

2-1     User Task/NSP Relationship in DECNET/8	       2-1

2-2     Sample Sequence of I/O Requests		       2-4

5-1     Node Name Table				       5-12

6-1     DDCMP Field (DDCFLD)			       6-1

6-2     NSP Field (NSPFLD)				       6-2

6-3     Control Message Formats			       6-4

6-4     Example of the Uses of Control Messages	       6-5

6-5     Format of Data Messages			       6-6

6-6     Overall Organization of DDCMP		       6-8

6-7     NSP to DDCMP Function Call Format		       6-9

6-8     DDCMP Completion Queue			       6-11

6-9     DDCMP to NSP Function Call Format		       6-13

6-10    ISR to DDCMP Packet Format			       6-16

6-11    Layout of DDCMP's Input Queue for ISRs	       6-17

6-12    Data Ring Buffer				       6-18

6-13    LCBTAB Location				       6-18

6-14    General NSP Message Format			       6-25

6-15    Overall Layout of NSP			       6-29

6-16    CCBTAB					       6-43

6-17    LNKTAB					       6-44

6-18    NETTAB					       6-44

TABLES

TABLE   2-1     DECNET/8 Function Calls			       2-2

2-2     Destination Block Format			       2-10

4-1     Error Conditions Printed by NIP		       4-3

5-1     User Task Parameters			       5-3

5-2     NETGEN Parameters				       5-4

5-3     DDCMP and NSP Parameters			       5-8

5-4     NIP Parameters				       5-10

v

CONTENTS (Cont.)

Page

5-5     TLK and LSN Parameters			       5-11

5-6     Interrupt Service Routines Supported by

DIGITAL					       5-14

5-7     Line Control File Parameters		       5-14

6-1     Decision Table for Control Messages Received

by DDCMP					       6-12

6-2     Line Control Block Format			       6-19

vi

PREFACE

This manual describes the DECNET/8 facilities available  to  both  the

RTS-8  programmer  and  terminal user.  It is assumed that the user of

this manual is familiar with RTS-8.

The chapters in this document are as follows:

Chapter 1 provides a brief  overview  of  DECNET/8  and  explains  the

capabilities  offered  to terminal users.  This chapter should be read

by DECNET/RTS-8 operators as well as programmers and terminal users.

Chapter 2 is an overview of  the  DECNET/8  language  facilities.   It

introduces communications and network protocols.

Chapters 3 and 4 are the specific DECNET/RTS-8 utilities, such as  the

Terminal  Communications  Utility  (TLK)  and  the Network Information

Program (NIP).  These chapters can be used by operators,  programmers,

and terminal users.

Chapter 5 is directed to users who will  perform  system  and  network

generation.

Chapter 6 is an internal  network  description  that  should  be  used

preferably  by  the programmer.  It explains the implementation of the

protocols.

Chapter 7 describes how a protocol can be used as a  stand-alone  task

and it also provides guidelines for users writing their own protocols.

vii

CHAPTER 1

INTRODUCTION

1.1  DEFINITION OF COMPUTER NETWORKS

A computer network is a group of  two  or  more  independent  computer

systems interconnected to provide information exchange.  Each separate

computer thus becomes a local part (a node) of the larger  whole  (the

network).   Each  node  in a network is an independent computer system

that can operate alone or interact with another node.

Nodes need  not be identical.  Therefore, each node is not required to

maintain  a  full  complement  of  processing capabilities.  If a user

requires a processing or device capability not  available  locally,  a

network  can  meet  that need by providing access to another, possibly

distant, node that maintains the needed capabilities.

The connection between two adjacent nodes is called  a  physical  link

and can be any of several types of permanent or temporary connections.

A direct wire is an example  of  a  permanent  link  and  a  telephone

circuit is an example of a temporary link.  Figure 1-1 shows two nodes

connected by a satellite link.  A user located at Node 1 regards  that

node  as the local node and Node 2 as the remote node.  A user at Node

2 regards Node 2 as local and Node 1 as remote.

*******

*	    *

* Satellite *

*	    *

NODE		      .	  . ******* .   .		NODE

1		     . . .	     . . .		 2

.	.	      .   .

.			   .

--------			  --------

| Data |			  | Data |

........|  set |			  |  set |........

.	      --------			  --------  	 .

.							 .

------------					   ------------

| Computer |					   | Computer |

...|          |					   |          |...

.	 ------------					   ------------  .

.	 								 .

--------							   --------

| User |							   | User |

|  1   |			    Physical		           |  2   |

--------	       |<-------      Link	------->|	   --------

Figure 1-1  Computer Network Composed of Two Nodes Connected by

a Physical Link

1-1

INTRODUCTION

1.2  DEFINITION OF DECNET/8

DECNET is the collective name for software and hardware products  that

extend  the  facilities  of  various DIGITAL operating systems so that

they can  be  joined  to  form  computer networks.   DECNET/8  is  the

software  package  that  supplies  network  capabilities for the RTS/8

operating system.

DECNET/8 provides a general set of tools  that  allows  user  programs

operating  at  different nodes to communicate.  User programs can open

conversation paths (logical links) between  two  different  nodes  and

exchange messages.

1.3  STRUCTURE OF DECNET/8

DECNET/8 operations  are  distributed  over  four  functional  levels,

hierarchically  arranged  to  provide  an  orderly flow of information

between the user and the physical link (Figure 1-2).

1	   2	      3		    4

--------   -------   ---------   -------------

| User |   | NSP |   | DDCMP |   | Physical  |   Physical

| task |-->|     |-->|	  |-->|   Link    |...........

--------   -------   ---------   | Interface |         .	     To

|   (ISR)   |        ........> Remote

-------------     Link	     Node

Figure 1-2  Functional Levels of DECNET/8

Information passes from one level to another by  means  of  interfaces

that  are an integral part of each level.  In addition, each level can

communicate with its counterpart at a  distant  node  by  adhering  to

rules  of  operation  known as protocols.  Each level uses one or more

protocols to govern the format, control, and sequencing of information

exchange.

The four levels shown in Figure 1-2 are described in the following:

Level 1. Program Dialogue Level (User Task)

The  program  dialogue  level   consists   of   communicating

programs,  interfaces  to  the  user  and to the logical link

control level, and one or more  protocols  governing  message

exchange.  The basic components of this level are as follows:

a.  User Request - This interface is the user's entry to

DECNET.   It  is composed of DECNET utility names as

well as function calls that can be  incorporated  in

user-written  programs.   The  user  interface  also

allows  network   programs   to   access  peripheral

devices.

1-2

INTRODUCTION

b.  Communicating  Programs  -  DECNET/8  supplies   two

special-purpose     utility     programs.      Other

applications can be provided by incorporating

network function calls in user-written programs.

c.  Logical Link Control Interface - The dialogue  level

also includes a transparent interface to the logical

link control level.

Figure 1-3 shows the basic components of the dialogue  level  and  the

ways in which the user requests DECNET/8 services.

User Request	  Communicating		    Logical Link

Programs		  Control Interface

------------------	    -------------

Invoke		| Terminal       |	    |	        |

Utility  --------->| Communications |--------->|	        |

| Utility (TLK)  |	    |		|

------------------	    |		|

|	    	|

|	    	|

------------------	    |	    	|

Invoke		| Network	 |	    |	        |

Utility  --------->| Management	 |--------->|    NSP    |-----> To

| Utility (NIP)  |	    | Interface |       NSP

------------------	    |		|

|		|

|		|

------------------	    |		|

Enter 		| User-written   |	    |		|

Program  --------->| Network	 |--------->|		|

| Programs       |	    |		|

------------------	    -------------

Figure 1-3  Components of the Program Dialogue Level

Level 2. Logical Link Control Level (NSP)

A logical link is a conversation path between  two  programs.

The  Network  Services  Protocol  (NSP)  is  the logical link

protocol.  NSP functions include routing and sequence control

of  messages, diagnosis of information transfer problems, and

maintenance  of  error  statistics  for  logical  links.    A

principal  feature  of NSP is its ability to maintain several

simultaneous conversations (logical links)  between  programs

over a single physical link.

Level 3. Physical Link Control Level (DDCMP)

The  physical  link  control   level   is   responsible   for

maintaining  an  error-free physical data path between nodes.

1-3

INTRODUCTION

The DIGITAL Data Communications Message Protocol  (DDCMP)  is

the physical link control level protocol.

Level 4. Physical Link Interface Level (ISR)

This level transmits and receives messages  over  a  physical

link  and is usually implemented by a hardware device such as

a modem or data set.  The physical link interface is  handled

by an Interrupt Service Routine (ISR).

DECNET/8 supports communication  between  dialogue-level  programs  at

different  nodes  by routing user requests through the four functional

levels shown in Figure 1-2.  The interaction  between  levels  can  be

illustrated  by tracing a sample message transmission from one node to

another.  Figure 1-4 shows a user request passing from Node 1 to  Node

2.

NODE 1					    NODE 2

1						  4

-------------					 -------------

|  Sending  |					 | Receiving |

|  Program  |					 |  Program  |

-------------					 -------------

|					       ^

2	V					  3    |

-------------					 -------------

|    NSP    |					 |    NSP    |

| 	      |					 | 	     |

-------------					 -------------

|					       ^

3	V					  2    |

-------------					 -------------

|   DDCMP   |					 |   DDCMP   |

| 	      |					 | 	     |

-------------					 -------------

|					       ^

4	V					  1    |

-------------					 -------------

| Physical  |		   Physical		 | Physical  |

|   Link    |..................		 |   Link    |

| Interface |		      ..................>| Interface |

|   (ISR)   |		     Link		 |   (ISR)   |

-------------					 -------------

Figure 1-4  Sample Message Transmission from Node 1 to Node 2

The numbered steps in Figure 1-4 correspond to the following sequence

of events:

At Node 1:

1.  The  user  issues  a  network  service  request.   The   user

interface  level  receives  the  request,  translates it to a

1-4

INTRODUCTION

DECNET  message,  fetches  the  required  function  call   or

utility,  and  passes  the  request to NSP.  All this happens

under the control of the RTS/8 Executive.

2.  NSP determines where the message is to be sent, adds  routing

information, and passes it, along with the message, to DDCMP.

3.  DDCMP adds test characters to the message (for error checking

by  DDCMP  at  Node 2) and passes the message to the physical

link interface for transmission.

4.  The physical link interface (the Interrupt  Service  Routine)

transmits the message over the physical link.

At Node 2, the sequence of events is reversed:

1.  The physical link interface (the ISR)  receives  the  message

and passes it to DDCMP.

2.  DDCMP  examines  the  test  characters  to   ensure   message

integrity  and  acknowledges message receipt by returning the

test  result  to  DDCMP  at  Node  1.   If  the  message   is

error-free,  DDCMP  at Node 2 passes it to NSP.  If, however,

the message is not error-free,  DDCMP  at  Node  1  initiates

retransmission.

3.  NSP determines the destination of the message and  passes  it

to the specified user program or utility under the control of

the RTS/8 Executive.

4. The destination program receives the message.

This basic sequence of events is the foundation for all DECNET/8  user

capabilities.

1.4  USER CAPABILITIES

DECNET/8  provides  the  user  with  capabilities  that  include   the

following:

o Terminal Communication

o Network Information Utility

o Communication Between User-Written Programs

1.4.1  Terminal Communication Utility

The terminal user can communicate  with  any  other  terminal  in  the

network by invoking the DECNET/8 utility TLK.  For example, TLK can be

used to request that the console operator at a  remote  node  mount  a

specific  file-structured  device.  In order to receive a message sent

by TLK, the receiving node must have the Listen (LSN)  utility  or  an

1-5

INTRODUCTION

equivalent  active  task.   Figure  1-5 shows TLK being used to send a

message between two remote PDP-8s.  The numbered steps  correspond  to

those used in Figure 1-4.  TLK is explained in detail in Chapter 3.

NODE 1					    NODE 2

--------------------			     --------------------

|   TLK request:   |			     |   TLK Message:   |

| Please Mount DK1 |			     | Please Mount DK1 |

--------------------			     --------------------

|					       ^

|					       |

1    V					  4    |

-------------					 -------------

|    TLK    |					 |    LSN    |

|  	      |					 |  	     |

-------------					 -------------

|					       ^

2	V					  3    |

-------------					 -------------

|    NSP    |					 |    NSP    |

| 	      |					 | 	     |

-------------					 -------------

|					       ^

3	V					  2    |

-------------					 -------------

|   DDCMP   |					 |   DDCMP   |

| 	      |					 | 	     |

-------------					 -------------

|					       ^

4	V					  1    |

-------------					 -------------

| Physical  |		   Physical		 | Physical  |

|   Link    |..................		 |   Link    |

| Interface |		      ..................>| Interface |

-------------		     Link		 -------------

Figure 1-5  Terminal Communication Utility (TLK) Used on Two PDP-8s

1.4.2  Network Information Program

DECNET/8 provides a program  called  NIP  for  reporting  the  network

status.  NIP  (Network  Information  Program)  retrieves node and link

information such as error counters (maintained by NSP) and the  status

of each node that can be accessed by the local node.  NIP is described

in Chapter 4.

1.4.3  Communication Between User-Written Programs

Two programs in a network can use DECNET facilities to exchange  data.

An  RTS/8 program uses DECNET/8 Executive Requests to communicate with

other programs in the network.  Refer to Chapter 2.

1-6

CHAPTER 2

RTS/8 INTERTASK COMMUNICATION

2.1  RTS/8 ASSEMBLY LANGUAGE FACILITIES OVERVIEW

In DECNET/8, user tasks at different nodes communicate with each other

using  the  network  protocols  NSP  and  DDCMP.   NSP provides a user

interface to the network.  The user task  issues  a  request  to  NSP,

which  passes  the  request  to  DDCMP  and  then to the physical link

interface for transmission to a remote node.  This sequence  is  shown

in Figure 2-1.  A task communicates with NSP by means of the following

facilities:

1.  The RTS/8 SEND (or SENDW) Executive Request (ER), which  uses

the DECNET/8 function calls;

2.  The DECNET/8 Asynchronous System Trap (AST);

3.  A shared data area called the Connect Control Block (CCB).

Figure 2-1 shows the relationship of these facilities to  one  another

and  to  the overall system.  These facilities are discussed in detail

in this chapter.

User Task		    NSP		       DDCMP

----------		 ----------	     ----------

|	      |	 RTS/8	 |	  |  RTS/8   |	      |

|  Main  |--------->|	  |<-------->|	      |

|	Code  |	Messages |	  | Messages |	      |		 ISR

|	      |		 |	  |	     |	      |	      ----------

|--------|		 |        |	     |	      |	      |	       |

|  AST   |Interrupts|	  |	     |	      |	      |	       |

|	      |<---------|	  |	     |	      |<----->|	       |-->

|--------|		 |--------|	     |	      |	      |        |

|	CCB   |  Shared	 | CCBTAB |	     |	      |	      |	       |

|	      |<-------->|	  |	     |	      |	      |	       |

----------	  Data	 ----------	     ----------	      ----------

Subroutnie/Event

Flag Linkage

Figure 2-1  User Task/NSP Relationship in DECNET/8

2.1.1  RTS/8 SEND Executive Request (ER)

User-written communications operations are performed by sending  RTS/8

messages  to the NSP task using SEND and SENDW ERs.  The form used for

sending RTS/8 messages to NSP is explained in Section  2.2.1.   For  a

complete  description of these and other ERs, consult the RTS/8 User's							       ____________

Manual (DEC-08-ORTMA).     ______

2-1

RTS/8 INTERTASK COMMUNICATION

RTS/8 tasks communicate  with  other  tasks  in  a  network  by  using

DECNET/8  function calls with the RTS/8 SEND ER.  Using these function

calls the RTS/8 task can:

o Request  a  logical  link  connection  with   another   task,

optionally  passing up to 16 bytes of data to the target task

at the time the link is requested;

o Accept or reject a request from the source task for a logical

link  connection, optionally passing a maximum of 16 bytes of

data to  the  source  task  as  part  of  the  acceptance  or

rejection;

o Transmit messages to, and receive messages from, other  tasks

over established logical links;

o Transmit up to 16  bytes in  an  interrupt  message  for  the

immediate attention of the target task;

o Disconnect a logical link;

o Receive  and  transmit  single  messages  between  two  tasks

without a logical link.

The DECNET/8 function calls used with the RTS/8 SEND ER are listed  in

Table  2-1.   These  function calls are discussed in detail in Section

2.2.

Table 2-1

DECNET/8 Function Calls

----------------------------------------------------------------------

|  Function Call     |			 Function		  |

|  Symbol		  |						  |

----------------------------------------------------------------------

|			  |						  |

|  CONINI		  |	Initiate the creation of a logical link	  |

|			  |						  |

|	CONCNF		  |	Confirm the creation of a logical link	  |

|			  |						  |

|	CONREJ		  |	Reject the request for a logical link	  |

|			  |						  |

|	DISCON		  |	Disconnect a logical link		  |

|			  |						  |

|	TRNSMT		  |	Transmit a solicited data  message  to  a |

|			  |	partner task over a logical link	  |

|			  |						  |

|	INTRPT		  |	Interrupt a task at a remote node, passing|

|			  |	it a brief interrupt message		  |

|			  |						  |

|	RCVMSG		  |	Receive data from a task at a remote node |

|			  |						  |

|	RCNMSG		  |	Queue a receive request without soliciting|

|			  |	data with a link status message		  |

----------------------------------------------------------------------

2-2

RTS/8 INTERTASK COMMUNICATION

Table 2-1

DECNET/8 Function Calls (cont'd)

----------------------------------------------------------------------

|			  |						  |

|	RCVSM		  |	Receive a single message over a physical  |

|			  |	line					  |

|			  |						  |

|	TRNSM		  |	Transmit a single message over a physical |

|			  |	line					  |

----------------------------------------------------------------------

2.1.2  Asynchronous System Trap (AST)

A task must provide a special  routine,  called  an  AST  routine,  to

receive control when NSP detects an interrupt condition on the logical

link.  Interrupt conditions that can be detected by  NSP  include:  an

unsolicited  connect  from  a  remote  node;  receipt  of an interrupt

message over a logical link; breaking of the  logical  link  resulting

from  disruption  of  the physical link; the remote node disconnecting

from the logical link.  The  DECNET/8  AST  routine  is  discussed  in

detail in Section 2.3.

2.1.3  Connect Control Block (CCB)

The user task must provide a special data area, shared  with  NSP,  to

store  the  context  of  a connection with a remote task.  This shared

data area is called the CCB.  If the task is disk  resident,  the  CCB

must reside in the resident portion of the task.  There is one CCB for

each connection in the system, and generally only one  CCB  per  task.

The  CCB contains all the internal state information that NSP needs to

handle events related to that CCB.  The CCB is discussed in detail  in

Section 2.4.

2.1.4  Basic Procedure

The basic procedure for transmitting a message from an RTS/8  task  in

one node to an RTS/8 task in a remote node is:

o Create a logical link

The initiating task issues a DECNET/8 connect initiate to the

NSP  task  via  a SEND ER with the function code CONINI.  The

target task is  interrupted  and  is  passed  the  connection

request.   Based on the information (if any) accompanying the

interrupt, the target task can either accept  or  reject  the

connection.   A task running under RTS/8 accepts a connection

by issuing SEND ER with the function code CONCNF; it  rejects

a  connection  by  issuing  SEND  ER  with  the function code

CONREJ.

o Transmit messages

2-3

RTS/8 INTERTASK COMMUNICATION

If the target task accepts the connection, a logical link  is

created  and the two tasks can exchange messages. Either task

can then issue the SEND ER with the function call  TRNSMT  to

send  messages,  or  with the function call RCVMSG to receive

messages.

o Disconnect the logical link

Either task can disconnect the logical link between them.   A

task  running  under  RTS/8 disconnects the link by issuing a

SEND ER with the function call DISCON.

Figure 2-2 illustrates a sample sequence of I/O requests by  both  the

source task and the target task.

Source Task			  Target Task

--------------------

|      CONINI      |---------->(AST starts Running)

--------------------

--------------------

(Posted)      <----------|      CONCNF      |

--------------------

.		.

.		.

--------------------

|      TRNSMT      |---------->

--------------------

--------------------

<----------|      RCVMSG      |

--------------------

--------------------

|   DATA MESSAGE   |---------->	   (Posted)

--------------------

.

.

--------------------

(AST runs)     <----------|      DISCON	|

--------------------

--------------------

|Disconnect Confirm|---------->	   (Posted)

--------------------

Figure 2-2  Sample Sequence of I/O Requests

2.2  ISSUING DECNET/8 REQUESTS

User-written communications are performed by issuing DECNET/8 function

codes.  A DECNET/8 function code is issued by sending an RTS/8 message

2-4

RTS/8 INTERTASK COMMUNICATION

to the NSP task using the SEND or SENDW ER with the required  function

code as an argument.  The standard format of an RTS/8 message is:

RTSMSG,	      ZBLOCK 3		/THE STANDARD RTS/8 MESSAGE HEADER

STATUS

FUNCTION		/COMMON INFORMATION PRESENT IN ALL

CHANNL		/NSP FUNCTION CALLS

P1

P2

P3		/FUNCTION-DEPENDENT PARAMETERS

where:

STATUS	is the status word upon completion  of  the  function.  The

initial value of this word does not affect the command. NSP

sets the value of STATUS before posting the  message.  Zero

indicates  successful  completion.  Unsuccessful completion

leaves a nonzero completion  code  in  this  word.   Status

codes are defined symbolically in the source file CCB.PA.

FUNCTION	is one of the function  calls  listed  in  Table  2-1,  and

defined  in  the  source  file  CCB.PA.  This word is never

modified by NSP.

CHANNL	is a user-assigned channel number.  This word  is  used  by

NSP  to  associate the proper CCB with this function during

I/O.  This word is never modified by NSP.

P1,P2,P3..	The number of parameters  depends  on  the  function  being

requested.

No modifications should be made to any word in an  NSP  message  while

completion  is  pending.   Such  modifications  can  result  in system

failure.

2.2.1  DECNET/8 Function Calls

RTS/8 tasks request services from NSP by issuing function calls in the

form  of  RTS/8  messages.   The  following  groups  of  functions are

available in DECNET/8:

. CONNECT

. RECEIVE

. TRANSMIT

. INTERRUPT

. DISCONNECT

. RECEIVE SINGLE MESSAGE

. TRANSMIT SINGLE MESSAGE

CONNECT.  A connection is made when two tasks, a  source  task  and  a

target  task,  establish  a communications path (logical link) between

them.  The target task can accept or reject the  connection  requested

by the source task.  Accordingly, the following connect function calls

2-5

RTS/8 INTERTASK COMMUNICATION

are part of the DECNET/8 set of function calls:

. CONINI 		   Connect Initiate

. CONCNF 		   Connect Confirm

. CONREJ 		   Connect Reject

The function call CONINI allows a  task to  request  connection  to  a

remote  task  that  is specified by name in the request.  Under RTS/8,

the target task is interrupted by means of the  DECNET/8  AST  routine

(described  in Section 2.3).  The name and location of the source task

requesting the link are passed to the target task in the  shared  data

area, the CCB (see Section 2.4).

The target task must  then  accept  or  reject  the  connection.   The

decision  to  accept  or reject is based on the name of the source, or

the availability of a buffer or device.  If the connection is accepted

with a CONCNF, resources tentatively allocated when the connection was

requested (CONINI) are permanently allocated, and  the  connection  is

established.    Once   the  logical  link  has  been  confirmed  by  a

CONINI/CONCNF sequence, the dialog between tasks can  begin.   If  the

connection  is rejected with a CONREJ, the source task's function call

event flag is posted, and the resources are  reassigned  by  both  the

source task's NSP and the target task's NSP.

NSP also allows the user to add "optional data" to connect messages to

pass  preliminary  connection  information between nodes without using

separate data messages.  For example, a task sending  a  message  to a

remote  terminal  could  pass  the  number  of the remote terminal and

receive confirmation that the remote  terminal  is  available  in  the

connect  sequence.   The subsequent data could then be confined to the

terminal message itself.  Optional  data  is  limited  to  16  (8-bit)

bytes.

RECEIVE.  A task must request the reception of  data  by  reserving  a

buffer  to receive the requested data before the data can be sent over

the physical link.  Thus, the user  is  responsible  for  data  buffer

management.   The  following  receive  function calls are available to

request data from a remote node.

. RCVMSG 		   Receive message

. RCNMSG 		   Receive message without link status

RCVMSG sends the buffer address to NSP and instructs it to transmit  a

request  for  data  to the remote node.  This request is made using an

internal NSP message called the link status message.   The  local  NSP

transmits a link status message to the remote NSP to notify the remote

task that the user task  at  the  local  node  has  issued  a  receive

request.   RCNMSG passes the buffer address to a waiting queue, but no

link status message is sent.  Instead, a subsequent  RCVMSG  passes  a

cumulative  link status message to the remote node.  Thus, to transmit

the requests, a task can issue  n-1  (maximum  of  15)  RCNMSGs,  then

follow with a RCVMSG.

2-6

RTS/8 INTERTASK COMMUNICATION

TRANSMIT.  The TRNSMT (transmit) function call must be  used  to  send

data  to a remote task.  If a message request has been received by the

local task, TRNSMT sends data to the requesting remote task  at  once.

Otherwise,  the  data is queued and transmitted when a requesting link

status message RCVMSG is received.

INTERRUPT.  Frequently, a task must inform its remote  partner  of  an

urgent   condition   requiring   immediate   attention.    The  INTRPT

(interrupt) function call allows either task to bypass  the  buffering

scheme  and  to transmit an interrupt message.  This interrupt message

is transmitted to the remote NSP, which  interrupts  the  remote  task

using an AST routine and passes the interrupt message to its CCB.

DISCONNECT.  Tasks can disconnect from a logical  link  by  issuing  a

DISCON  (disconnect)  function call.  This function breaks the logical

link and interrupts the remote task to inform it of the disconnection.

Either  task  can  issue  a  DISCON  at  any  time without the other's

consent.

SINGLE MESSAGES.  The single-message function allows a message  to  be

sent  between  two  tasks  that  do not have a logical link connecting

them.   The  two  nodes  must  be  connected  by  a   physical   link.

Single-message  requests  reduce  the amount of nondata traffic on the

line because no logical link or NSP status messages are sent over  the

line.   Single-message  requests  are  suited  to  an inquiry/response

environment.  The function calls for single messages are:

. RCVSM 		   Receive single message

. TRNSM 		   Transmit single message

DECNET/8 does not guarantee the delivery of single  messages.   As  in

normal  receive  and transmit requests, a task must request to receive

data by reserving a buffer to store the message.  If a task receives a

single  message  without issuing a receive single-message request, the

message is discarded and no notification is sent to  the  transmitting

task.  Single-message functions should be used only in well-structured

environments.

The interaction of the  CCB,  the  AST,  and  the  function  calls  is

explained  further  in  the  descriptions  of  the individual function

calls.

In the examples in the subsequent sections, the following RTS/8 coding

conventions are observed:

CUR       is defined in the task as CURRENT FIELD multiplied by 10 (to

build a CDF instruction).

CHANNL    is equated to the task's CCB number; it is  NOT  an  address

of a word containing the number.

All symbolic function codes described in this chapter  are  predefined

at system generation.

2-7

RTS/8 INTERTASK COMMUNICATION

----------							 ----------

| CONINI |							 | CONINI |

----------							 ----------

2.2.2  CONINI

FUNCTION:

Initiate the creation of a logical link between the  source  task  and

the target task.

MESSAGE FORMAT:

ZBLOCK 3		   /RTS/8 HEADER

STATUS		   /RESERVED FOR COMPLETION STATUS

CONINI		   /FUNCTION CALL

CHANNL		   /CHANNEL NUMBER

P1

P2			   /PARAMETERS

P3

P4

PARAMETERS:

P1	       is a pointer to the 6-bit destination node  name.   As  with

all  DECNET/8  names, the node name must be null-padded, and

it has a maximum of six characters.  If  this  parameter  is

zero, the message is sent to the local node.

P2	       is a pointer to the destination block.  The format for  this

9-word block is shown in Table 2-2.  Refer to Usage Note 3.

P3        is a pointer to an optional data buffer  being  transmitted.

If  P3=zero, no optional data is sent.  Otherwise, a maximum

of 16 bytes is transmitted with the connection request.  The

first  negative word encountered delimits the data string in

the buffer.

P4        is a pointer  to  a  confirm/reject  optional  17-word  data

buffer  being  received.   On completion of the request, the

first word is set to a negative byte count, and the next  16

words  contain  any optional data sent by the remote task in

its connect-confirm or connect-reject call.  (There  is  one

byte per word.)

USAGE NOTES:

1.  The source task  uses  CONINI  to  request  a  logical  link

between itself and a remote task.

2.  Under RTS/8, a tentative logical link is  created  when  the

remote  node receives a CONINI from the source task.  If the

remote node rejects the request, the tentative logical  link

is  deassigned  at once.  There is no limit to the number of

2-8

RTS/8 INTERTASK COMMUNICATION

logical links that can be created.  Generally, one task  has

one  logical  link  but it is possible to have several tasks

assigned to the same logical link.

3.  Table 2-2 shows the format of the destination block.   Words

8  and  9  of this block contain a user identification code.

In multiuser environments, each user or group  of  users  is

assigned  a  user  identification code or UIC (also called a

programmer/project number or PPN).  The UIC is a combination

of  a  group  code and a user code, separated by a comma and

enclosed in square brackets ([,]).   Group  and  user  codes

must  be  in  the  range  of  1  through  7777 (octal).  For

example, [2,345] and [67,10] are both acceptable UICs.

In many operating systems, a UIC is associated with files or

tasks, and helps distinguish them.  Thus, more than one file

or task on the same device  can  have  the  same  name,  but

different  UICs.  Since a remote operating system might have

several tasks with the same  name,  the  user  may  have  to

specify the group and user codes to identify a specific file

or task.

If the group or user code for a file or task is not known, a

0 (zero) should be specified for either code.  NSP will then

use the default user  or  group  code.   For  DECNET/8,  the

default  user code is 1; the default group code is 2.  Thus,

a task expecting incoming connection requests should specify

[1,2]  as its UIC.  If another UIC is given, the remote task

can connect to the local  task  only  if  it  specifies  the

correct UIC.

If a task specifies its UIC as [0,0], a remote  task  cannot

connect to it.

STATUS CODES:

SUCST     Success.

DISST     A disconnect has  preempted  completion  of  the  connection

process.

DRJST     The remote NSP has issued a disconnect request.

CRJST     The remote task has rejected the connection.   Any  optional

data is stored in the user-assigned optional data buffer.

ILLST     Illegal connection request.   The  local  NSP  rejected  the

connection request because the CCB was not in an idle state.

This error might  indicate  that  a  connection  is  already

established.

NODST     The requested node name is not part of the  network.   Refer

to  the  SYSGEN  procedures  in Chapter 5 for information on

entering a node name into the DECNET/8 system.

2-9

RTS/8 INTERTASK COMMUNICATION

ERRST     An NSP error  occurred  that  preempted  the  source  task's

request.   (This  error  should  not occur.  ERRST indicates

either a software error or a  user  error  that  has  caused

system failure.)

LDNST     The physical link has broken, thus  preempting  the  pending

connection.

Table 2-2

Destination Block Format

----------------------------------------------------------------------

|	Word  |				  Contents			  |

|--------|-----------------------------------------------------------|

|	 1    |	The object type code for the target task.  This  code  is |

|	      |	zero for task-to-task communication.  Maximum value is    |

|	      | 127.							  |

|	      |								  |

|	 2-7  |	The 6-bit, null-padded symbolic name of the target  task. |

|	      |	This  can hold 12 characters.  This can be created with a |

|	      |	PAL/8 text pseudo-op.					  |

|	      |								  |

|	 8    |	The target task's  group  code  (for  compatibility  with |

|	      |	other  operating systems).  If zero, the remote node uses |

|	      | its default.  The default  group  code  is  specified  at |

|	      |	system generation.  Refer to Section 5.3.		  |

|	      |								  |

|	 9    |	The target task's user identification  code  (UIC).   The |

|	      | UIC  is  used  in various operating systems as a means of |

|	      |	identifying user files.  Refer to Usage Note 3.		  |

----------------------------------------------------------------------

EXAMPLE:

/SAMPLE CONNECT INITIATE (CONINI) CALL

.

.

CAL		 /MAINLINE CODE ISSUES

SENDW	 /CALL AND WAITS

NSP

CONIMS

.

.

CONIMS,	    ZBLOCK 3	 /RTS/8 MESSAGE HEADER

0		 /CURRENT STATUS

CONINI	 /FUNCTION CODE

CHANNL	 /CHANNEL CODE FOR CCB

P1,	    DNODE	 /POINTER TO DEST. NODE NAME

P2,	    DNAME	 /POINTER TO DEST. NAME BLOCK

P3,	    0		 /ZERO = NO OPTIONAL DATA SENT

P4,	    REPOPT	 /POINTER TO USERS REPLY OPT DATA BUF

DNODE,	    TEXT /MAYNRD//6 CHAR NODE NAME

*.-1 	 /ORIGIN OVER EXTRA ZERO PUT OUT

/BY TEXT STATEMENT

2-10

RTS/8 INTERTASK COMMUNICATION

DNAME,	    0 		 /OBJ TYPE=0 FOR TASK-TASK

TEXT /PARTNR//NAME OF MY PARTNER TASK

ZBLOCK 2	 /PADDING FOR SIX-WORD NAME

1,2		 /USE DEFAULT GROUP, USER CODES

REPOPT,	    0		 /WILL GET -# OPT BYTES SENT BACK

ZBLOCK 10	 /STORAGE AREA FOR REPLY OPT DATA

2-11

RTS/8 INTERTASK COMMUNICATION

----------							 ----------

| CONCNF |							 | CONCNF |

----------							 ----------

2.2.3  CONCNF

FUNCTION:

Confirm the creation of a logical link between the target task and the

source  task.   The  target  task must use CONCNF to confirm a logical

link creation.

MESSAGE FORMAT:

ZBLOCK 3		   /RTS/8 HEADER

STATUS		   /RESERVED FOR COMPLETION STATUS

CONCNF		   /FUNCTION CODE

CHANNL		   /CHANNEL NUMBER

P1			   /PARAMETER

PARAMETER:

P1	       is a pointer to an optional data buffer that is  transmitted

to  the task that issued the connection request.  The buffer

contains 8-bit bytes, one per word, terminated by a negative

word.   A  maximum  of  16 bytes can be returned to an RTS/8

task.  If P1 = 0, no optional data is sent.

USAGE NOTES:

1.  CONCNF instructs the local NSP to confirm the creation of  a

logical  link  between  the source task and the target task.

Upon receipt of the CONINI message from the source task, the

target  NSP  creates  a tentative logical link and schedules

the  target  task's  AST  routine.   When  the  source  task

receives  the  target  task's  CONCNF,  the  source task NSP

transmits a CONCNF message and marks the  tentative  logical

link  as waiting for a link status message.  Upon receipt of

the link status message, the source  task's  NSP  marks  the

tentative  logical  link  as "on-line" and posts the pending

CONCNF.  At this time a  logical  link  is  established  and

communication between tasks can begin.

2.  The most appropriate time to issue CONCNF is  from  the  AST

routine when a CONINI is received.

2-12

RTS/8 INTERTASK COMMUNICATION

EXAMPLE:

/SAMPLE CONNECT CONFIRM (CONCNF) CALL

.

.

.

CAL		/MAINLINE CODE ISSUES

SENDW	/CALL AND WAITS

NSP

CONFRM

.

.

.

CONFRM,	    ZBLOCK 3	/RTS/8 HEADER

0		/CURRENT STATUS

CONCNF	/FUNCTION CODE

CHANNL	/CHANNEL CODE FOR CCB

CNFOPT	/POINTER TO OPT DATA

CNFOPT,	    "O;"K;-1	/2 BYTES OF OPT DATA

/TERMINATED BY -1

STATUS CODES:

SUCST     Success.

ILLST     Illogical  request.   The  remote  task  did  not  issue   a

connection request.

ERRST     An NSP error  occurred  that  preempted  the  source  task's

request.   (This  error  should  not occur.  ERRST indicates

either a software error or a  user  error  that  has  caused

system failure.)

LDNST     The physical link  has  been  broken,  thus  preempting  the

pending connection.

2-13

RTS/8 INTERTASK COMMUNICATION

----------							 ----------

| CONREJ |							 | CONREJ |

----------							 ----------

2.2.4  CONREJ

FUNCTION:

Reject the creation of a logical link between the source task and  the

target  task.  The target task must issue CONREJ to reject the request

for a logical link.

MESSAGE FORMAT:

ZBLOCK 3		   /RTS/8 HEADER

STATUS		   /RESERVED FOR COMPLETION CODE

CONREJ		   /FUNCTION CODE

CHANNL		   /CHANNEL NUMBER

P1			   /PARAMETER

PARAMETER:

P1	       is a  pointer  to  an  optional  data  buffer.   The  buffer

contains 8-bit bytes, one per word, terminated by a negative

word.  A maximum of 16 bytes can be transmitted to an  RTS/8

task  with  this  request.   If  P1 = 0, no optional data is

sent.

USAGE NOTES:

1.  The target task can pass optional data to the  source  task.

For  example,  the  tasks can be coded to transmit rejection

reasons in the optional data buffer.

2.  When a CONREJ is  issued,  the  tentative  logical  link  is

immediately  deassigned,  and the function call is posted as

complete.

3.  The field is assumed to be the current field  and  the  byte

string is terminated by a delimiter.

4.  The most appropriate time to issue CONREJ is  from  the  AST

when a CONINI is received.

2-14

RTS/8 INTERTASK COMMUNICATION

EXAMPLE:

/SAMPLE CONNECT REJECT (CONREJ) CALL

.

.

.

CAL		/MAINLINE CODE ISSUES

SENDW	/CALL AND WAITS

NSP

REJMSG

.

.

.

REJMSG,	    ZBLOCK 3	/RTS/8 HEADER

0		/CURRENT STATUS

CONREJ	/FUNCTION CODE

CHANNL	/CHANNEL CODE FOR CCB

REJOPT	/OPT DATA PTR (P1)

REJOPT,	    0		/MUST BE SET TO THE USER'S REJECT CODE

/BEFORE ISSUING A FUNCTION CALL

-1		/DELIMITER

STATUS CODES:

SUCST     Success.

ILLST     Illogical  request.   The  remote  task  did  not   make   a

connection request.

ERRST     An NSP error  occurred  that  preempted  the  source  task's

request.   (This  error  should  not occur.  ERRST indicates

either a software error or a  user  error  that  has  caused

system failure.)

LDNST     The physical link  has  been  broken,  thus  preempting  the

connection.

2-15

RTS/8 INTERTASK COMMUNICATION

----------							 ----------

| DISCON |							 | DISCON |

----------							 ----------

2.2.5  DISCON

FUNCTION:

Disconnect the logical link between the source  task  and  the  target

task.

MESSAGE FORMAT:

ZBLOCK 3		   /RTS/8 HEADER

STATUS		   /RESERVED FOR COMPLETION CODE

DISCON		   /FUNCTION CODE

CHANNL		   /CHANNEL NUMBER

P1			   /PARAMETER

PARAMETER:

P1        is a  pointer  to  an  optional  data  buffer.   The  buffer

contains 8-bit bytes, one per word, terminated by a negative

word.  A maximum of 16 bytes can be transmitted to an  RTS/8

task with this request.  If P1=0, no optional data is sent.

USAGE NOTES:

1.  DISCON instructs NSP to break the logical link.  Either task

can  issue this function call at any time without consent of

the partner task.

2.  If a logical link existed, a DISCON is sent to  the  partner

task.   In  DECNET/8, the partner task is interrupted via an

AST routine and informed of  the  disconnect.   The  logical

link  is broken.  If there are any pending messages when the

DISCON is issued, the DISST status code is returned  to  the

task that originated the DISCON.

3.  If the logical link is tentative, the DISCON  is  not  sent.

Instead,  the  status  code  DISST  is  returned to the task

requesting the connection, and the CCB is made idle.

4.  If there is no logical link, the DISCON function call is  an

effective  no-opt The user can issue this call as part of an

error recovery routine to ensure that the logical  link  has

been broken.

2-16

RTS/8 INTERTASK COMMUNICATION

EXAMPLE:

/SAMPLE DISCONNECT (DISCON) CALL

.

.

.

CAL		 /MAINLINE CODE ISSUES

SENDW	 /CALL AND WAITS

NSP

DISMSG

.

.

.

DISMSG,	    ZBLOCK 3	 /RTS/8 HEADER

0		 /CURRENT STATUS

DISCON	 /FUNCTION CODE

CHANNL	 /CHANNEL CODE FOR CCB

DISOPT	 /POINTER TO OPTIONAL DATA

DISOPT,	    0		 /REASON FOR DISCON AS OPT DATA

-1		 /DELIMITER

STATUS CODES:

SUCST     Success.

DISST     A disconnect has preempted  completion  of  any  data  being

transmitted or received.

ILLST     Illegal disconnect  request.   The  disconnection  has  been

rejected because no logical link connection was established.

ERRST     An NSP error  occurred  that  preempted  the  source  task's

request.   (This  error  should  not occur.  ERRST indicates

either a software error or a  user  error  that  has  caused

system failure.)

LDNST     The physical link  has  been  broken,  thus  preempting  the

connection.

2-17

RTS/8 INTERTASK COMMUNICATION

----------							 ----------

| TRNSMT |							 | TRNSMT |

----------							 ----------

2.2.6  TRNSMT

FUNCTION:

Transmit a data message over the logical link.

MESSAGE FORMAT:

ZBLOCK 3		   /RTS/8 HEADER

STATUS		   /RESERVED FOR COMPLETION CODE

TRNSMT		   /FUNCTION CODE

CHANNL		   /CHANNEL NUMBER

P1

P2			   /PARAMETERS

P3

PARAMETERS:

P1        is a CDF to the message buffer.

P2        is the address of the first byte of the data buffer.

P3        is a negative byte count, indicating the number of bytes  to

be transmitted from the user data buffer.

USAGE NOTES:

1.  The CDF, ADDR, and -CNT form is used in the TRNSMT,  RCVMSG,

RCNMSG,  and  INTRPT function calls.  This form differs from

the form used in the CONINI,  CONCNF,  and  CONREJ  function

calls (in which the field is assumed to be the current field

and the byte string is terminated with a  delimiter  instead

of a byte count).

2.  TRNSMT causes NSP to transmit the data in the buffer to  the

target  task  when  the  target task  has  a receive request

pending.

3.  The data buffer  must  be  in  the  resident  portion  of  a

checkpointable task.

4.  For compatibility with other  operating  systems,  the  data

buffer must not be larger than 192 bytes in length.  The CDF

pointer pairs are not counted in the 192 bytes.

5.  If the target task's buffer is too small to accept  all  the

data sent to it, the task is warned that data will be lost.

6.  The data in the buffer  consists  of  8-bit  bytes,  usually

positive  (see  Example  1).  A negative byte indicates that

2-18

RTS/8 INTERTASK COMMUNICATION

the data is  continued  in  another  buffer.   The  negative

number  is  a  CDF  to  a  new  field;  the  next word is an

address-1 (for auto index register usage).  See  Example  2.

Data  buffers can be linked together in this manner to allow

users to insert their own message headers.

EXAMPLE 1:

/SAMPLE TRANSMIT (TRNSMT) CALL

BC=5	 /BYTE COUNT

.

.

CAL		 /MAINLINE CODE ISSUES

SENDW	 /CALL AND WAITS

NSP

SENDBF	 /SEND A "HELLO"

.

.

SENDBF,	    ZBLOCK 3	 /RTS/8 HEADER

0		 /CURRENT STATUS

TRNSMT	 /FUNCTION CODE

CHANNL	 /CHANNEL CODE FOR CCB

CDF CUR	 /FIELD OF THIS TASK

BUFFER	 /ADDRESS OF THE BUFFER

-BC		 /BYTE COUNT

BUFFER,	    "H;"E;"L;"L;"O

EXAMPLE 2:

BC=5	 /BYTE COUNT

.

.

CAL		 /MAINLINE CODE ISSUES

SENDW	 /CALL AND WAITS

NSP

SNDBF2	 /SEND A "HELLO"

.

.

SNDBF2,	    ZBLOCK 3	 /RTS/8 HEADER

0		 /CURRENT STATUS

TRNSMT	 /FUNCTION CODE

CHANNEL	 /CHANNEL CODE FOR CCB

CDF CUR	 /FIELD OF THIS TASK

BUF2	 /ADDRESS OF THE BUFFER

-BC		 /BYTE COUNT

BUF2,	    "H;"E;"L;CDF CUR;BUF3-1

.

.

BUF3,	    "L;"O

2-19

RTS/8 INTERTASK COMMUNICATION

STATUS CODES:

SUCST     Success.

DISST     A disconnect has preempted  completion  of  any  data  being

transmitted or received.

ILLST     Illegal  transmit  request.   There  is  no   logical   link

connection or the connect initiate message (CONINI) is still

pending.

ERRST     An error occurred that preempted the source task's  request.

(This  error  should  not  occur.   ERRST indicates either a

software error or  a  user  error  that  has  caused  system

failure.)

LDNST     The physical link  has  been  broken,  thus  preempting  the

request.

2-20

RTS/8 INTERTASK COMMUNICATION

----------							 ----------

| INTRPT |							 | INTRPT |

----------							 ----------

2.2.7  INTRPT

FUNCTION:

Interrupt a partner task and send it a short interrupt message.

MESSAGE FORMAT:

ZBLOCK 3		   /RTS/8 HEADER

STATUS		   /RESERVED FOR COMPLETION STATUS

INTRPT		   /FUNCTION CODE

CHANNL		   /CHANNEL NUMBERS

P1

P2			   /PARAMETERS

P3

PARAMETERS:

P1        is a CDF to the message buffer.

P2        is the address of the first byte of the data buffer.

P3         is a negative byte count indicating the number of bytes  to

be transmitted from the user data buffer.

USAGE NOTES:

1.  This function  bypasses  the  normal  synchronous  buffering

scheme  and  sends  a short interrupt message to the partner

task.  The partner task is  notified  of  the  interrupt  by

means  of  an  AST  routine;  for  DECNET/8,  the  interrupt

message is placed in the partner task's  AST  optional  data

buffer.

2.  The maximum interrupt message size is 16 bytes.

3.  Only one interrupt transmission request can  be  outstanding

at  a  time.  The interrupt is posted complete when data has

been transmitted and  acknowledged  by  DDCMP.   If  a  task

issues  another  interrupt message before the pending one is

finished, the second message is  rejected  with  an  illegal

interrupt request status code (ILLST).

4.  The data buffer  must  be  in  the  resident  portion  of  a

disk-resident task.

2-21

RTS/8 INTERTASK COMMUNICATION

EXAMPLE:

/SAMPLE INTERRUPT (INTRPT) CALL

.

.

.

CAL		 /MAINLINE CODE ISSUES

SENDW	 /CALL AND WAITS

NSP

WAKMSG	 /THIS MESSAGE RETURNS WHEN DONE

.

.

WAKMSG,	    ZBLOCK 3	 /RTS/8 HEADER

0		 /STATUS WORD

INTRPT	 /FUNCTION CODE

CHANNL	 /CHANNEL CODE FOR CCB

CDF CUR	 /CDF TO CURRENT RTS/8 TASK FIELD

WAKTXT	 /ADDRESS OF STRING

-10		 /-LENGTH OF STRING, 8 (DECIMAL) CHARACTERS

WAKTXT,	    "W;"A;"K;"E;" ;"U;"P;"!    /"WAKE UP!"

STATUS CODES:

SUCST     Success.

DISST     A disconnect has preempted  completion  of  any  data  being

transmitted or received.

ILLST     Illegal interrupt request; there is no  established  logical

link connection  or  the  line  has failed due to a hardware

error.  This status code is also returned  if  an  interrupt

message is already pending.

ERRST     An NSP error  occurred  that  preempted  the  source  task's

request.   (This  error  should  not occur.  ERRST indicates

either a software error or a  user  error  that  has  caused

system failure.)

LDNST     The physical link  has  been  broken,  thus  preempting  the

request.

2-22

RTS/8 INTERTASK COMMUNICATION

----------							 ----------

| RCVMSG |							 | RCVMSG |

----------							 ----------

2.2.8  RCVMSG

FUNCTION:

Receive data from a remote task.

MESSAGE FORMAT:

ZBLOCK 3		   /RTS/8 HEADER

STATUS		   /RESERVED FOR COMPLETION STATUS

RCVMSG		   /FUNCTION CODE

CHANNL		   /CHANNEL NUMBER

P1

P2			   /PARAMETERS

P3

PARAMETERS:

P1	       is a CDF to the message buffer.

P2        is the address of the first byte of the data buffer.

P3	       is a negative count  of  the  size  of  the  receive  buffer

excluding  the  CDF;  ADDR  pair.   This  word is altered to

contain the negative count of data bytes actually received.

USAGE NOTES:

1.  This function inserts the user's buffer into a waiting queue

and  then  transmits a link status message to the remote NSP

to inform it that a buffer  is  available.   When  a  TRNSMT

request  addressing the local task is issued, data is passed

to the buffer and the RCVMSG request is posted.

2.  The user should reinitialize P3 each time a RCVMSG has  been

issued  because P3 may change if fewer than P3 bytes of data

are received.

3.  The  buffer  must  be  in  the   resident   portion   of   a

checkpointable task that is issuing a RCVMSG.

4.  Words reserved for data bytes must contain positive  numbers

to distinguish them from CDF instructions.  For example, the

ZBLOCK pseudo-op can be used during assembly.  There  is  no

need  to  clear  the  buffer  before  issuing another RCVMSG

request.

5.  For compatibility with other operating systems,  the  number

of  data  bytes  in  the  buffer must not be larger than 192

bytes, excluding CDF;  ADDR pairs.

2-23

RTS/8 INTERTASK COMMUNICATION

EXAMPLE:

/SAMPLE RECEIVE MESSAGE (RCVMSG) CALL

/BUFFLD, BUFLOC, BUFSIZ ARE DEFINED BY USER

.

.

.

TAD (-BUFSIZ /INITIALIZE THE BUFFER SIZE

DCA SIZPRM 	 /LIMIT

CAL		 /MAINLINE CODE ISSUES

SENDW	 /CALL AND WAITS

NSP

GETMSG

.

.

.

GETMSG,	    ZBLOCK 3	 /RTS/8 HEADER

0		 /CURRENT STATUS

RCVMSG	 /FUNCTION CODE

CHANNL	 /CHANNEL CODE FOR CCB

CDF BUFFLD   /P1 = CDF TO BUFFER FIELD

BUFLOC 	 /P2 = ADDRESS OF BUFFER

SIZPRM,	    -BUFSIZ 	 /P3 = -BUFF SIZE UPON ISSUE

/AND P3 = -DATA COUNT UPON

/COMPLETION

.

.

.

FIELD BUFFLD%10 /CODE TO DEFINE THE BUFFER

*BUFLOC

ZBLOCK BUFSIZ

.

.

.

STATUS CODES:

SUCST     Success.

DISST     A disconnect has preempted  completion  of  any  data  being

transmitted or received.

ILLST     Illegal  receive  request.   There  is   no   logical   link

connection.

LSTST     Data lost.  The message was too long to fit  into  the  data

buffer.   Only  P3  bytes  of  data  were  transmitted;  the

remaining bytes in the message are lost.

ERRST     An NSP error  occurred  that  preempted  the  source  task's

request.   (This  error  should  not occur.  ERRST indicates

either a software error or a user error that  caused  system

failure.)

LDNST     The physical link  has  been  broken,  thus  preempting  the

request.

2-24

RTS/8 INTERTASK COMMUNICATION

----------							 ----------

| RCNMSG |							 | RCNMSG |

----------							 ----------

2.2.9  RCNMSG

FUNCTION:

Queue a receive request without soliciting data with a link status

message.

MESSAGE FORMAT:

ZBLOCK 3		   /RTS/8 HEADER

STATUS		   /RESERVED FOR COMPLETION STATUS

RCNMSG		   /FUNCTION CODE

CHANNL		   /CHANNEL NUMBER

P1

P2			   /PARAMETERS

P3

PARAMETERS:

P1	       is a CDF to the message buffer.

P2        is the address of the first byte of the data buffer.

P3	       is the negative length of the  buffer  in  bytes  (excluding

CDF;  ADDR  pairs).   This  word  is  altered to contain the

negative count of data bytes actually received.

USAGE NOTE:

The usage notes  for  RCVMSG  apply,  with  the  following  exception:

RCNMSG  only  queues  a buffer; it does not send a link status message

to the remote NSP.  Thus, the target task cannot receive data until  a

subsequent   RCVMSG   is   issued.    (A   subsequent  RCVMSG  request

automatically transmits a cumulative request count for all outstanding

RCNMSGs.) Therefore, to issue n requests for data, the user must issue

n-1 RCNMSG calls (n has a maximum value of 16), followed by an RCVMSG.

Each  receive request is posted when data is received in the specified

buffer.  This procedure saves line overhead by eliminating the need to

send individual link status messages with each request for data.  When

issuing an RCNMSG, the user  must  never  use  SENDW;   refer  to  the

following example.

2-25

RTS/8 INTERTASK COMMUNICATION

EXAMPLE:

/SAMPLE RECEIVE WITHOUT LINK STATUS (RCNMSG) CALL

/QUEUE 1 RECEIVE AND ISSUE ANOTHER WITH

/LINK STATUS FOR BOTH

.

.

.

CAL		 /QUEUE INITIAL RECEIVE ONLY

SEND	 /NEVER USE SENDW WITH RCNMSG!

NSP		 /THIS CALL IS NOT POSTED UNTIL

NOLKST	 /AN RCVMSG IS ISSUED

CAL

SENDW	 /QUEUE FINAL RECEIVE AND

NSP		 /WAIT FOR ALL OF THEM

FINRCV	 /LINK STATUS WILL BE SENT NOW

.

.

.

NOLKST,	    ZBLOCK 3	 /RTS/8 HEADER

0		 /CURRENT STATUS

RCNMSG	 /FUNCTION CODE

CHANNL	 /CHANNEL CODE FOR CCB

CDF BUFFLD 	 /CDF TO BUFFER FIELD

BUF1	 /BUFFER FOR INITIAL RECEIVE

-BUFSIZ	 /BUFFER SIZE

FINRCV,	    ZBLOCK 3	 /RTS/8 HEADER

0		 /CURRENT STATUS

RCVMSG	 /FUNCTION CODE (LS SENT)

CHANNL	 /CHANNEL CODE FOR CCB

CDF BUFFLD   /CDF TO BUFFER FIELD

BUF2 	 /BUFFER FOR FINAL RECEIVE

-BUFSIZ 	 /BUFFER SIZE

STATUS CODES:

SUCST     Success.

DISST     A disconnect has preempted  completion  of  any  data  being

transmitted or received.

ILLST     Illegal  receive  request.   There  is   no   logical   link

connection.

LSTST     Data lost.  The message was too long to fit into the  target

buffer.   Only P3 bytes of data have been transmitted to the

buffer.  The remaining bytes in the message are lost.

ERRST     An NSP error preempted the  source  task's  request.   (This

error  should  not occur.  ERRST indicates either a software

error or a user error that caused system failure.)

LDNST     The physical link  has  been  broken,  thus  preempting  the

request.

2-26

RTS/8 INTERTASK COMMUNICATION

----------							 ----------

| RCVSM  |							 | RCVSM  |

----------							 ----------

2.2.10  RCVSM

FUNCTION:

Receive a single message from a remote node.

MESSAGE FORMAT:

ZBLOCK 3		   /RTS/8 HEADER

STATUS		   /RESERVED FOR COMPLETION STATUS

RCVSM			   /FUNCTION CODE

CHANNL		   /CHANNEL NUMBER

P1

P2

P3			   /PARAMETERS

P4

P5

PARAMETERS:

P1	       is a pointer to the 3-word area reserved for the name of the

node sending the message.

P2	       is a pointer to the 9-word area reserved for the name of the

source task sending the message.  (Refer to Usage Note 3.).

P3	       is a CDF instruction to set the pointer to the buffer memory

field.

P4	       is the address of the first  byte  of  the  user  buffer  to

receive the data message.

P5	       is a negative word  count  of  the  maximum  length  of  the

message that can be received.

USAGE NOTES:

1.  If a remote task sends a  single  message  (TRNSM)  and  the

local task has not issued an RCVSM, the message is discarded

and no notification is sent to the issuing task.

2.  A task that issues a RCVSM must set the node name/task  name

buffers  to  zero.  When the single message is received, the

node name and task name buffers are set to the location  and

value of the sending task.

3.  The value of the CHANNL parameter is used to locate the  CCB

to be used for passing the request.

2-27

RTS/8 INTERTASK COMMUNICATION

This function reserves a 3-word area called NODNAM.  When  a

single  message is received, NODNAM is updated with the name

of the node from which  the  single  message  was  received.

A 17-word area is reserved to hold source  task  information

arranged  in the same format as a connect message.  Refer to

Table 2-2.   This  area  is  updated  when  the  message  is

received to contain the source task name.

4.  Enough space should be reserved to hold the  largest  single

message  that  the  task expects to receive.  The default is

192 words.  This value is stored in location BUFLEN.

EXAMPLE:

.

.

.

CAL	   /MAINLINE CODE ISSUES

SEND	   /SEND

NSP	   /NSP IS THE TASK THE MESSAGE GOES TO

RECVSM	   /ADDRESS OF THE AREA CONTAINING THE RECEIVE

/SINGLE-MESSAGE ARGUMENTS

.

.

.

RECVSM,        ZBLOCK 3	   /RTS/8 HEADER

RECVST,        0	   /STATUS INITIALLY SET TO ZERO

RCVSM	   /RECEIVE SINGLE-MESSAGE FUNCTION CODE

CHANNL

NODNAM	   /POINTER TO AREA RESERVED FOR NODE NAME

TSKBLK	   /POINTER TO 9-WORD AREA RESERVED FOR TASK

/INFORMATION

CDF BUFFLD  /CDF TO BUFFER MEMORY FIELD

RCVBUF	   /ADDRESS OF USER BUFFER TO HOLD DATA

BUFLEN	   /NEGATIVE WORD COUNT OF MAX. MESSAGE THAT CAN BE

/RECEIVED

.

.

NODNAM,        ZBLOCK 3	   /RTS/8 HEADER

TSRBLK,        0	   /OBJECT TYPE OF 0

TSRNAM,        ZBLOCK 6	   /6 WORDS FOR TASK NAME

GRPNUM,        0	   /GROUP NUMBER

USRNUM,        0	   /USER NUMBER

RCVBUF,        ZBLOCK 300  /RESERVES 192-WORD BUFFER AREA

BUFLEN,        -300 	   /NEGATIVE WORD COUNT OF MAXIMUM MESSAGE

STATUS CODES:

SUCST     Success.

ILLST     Too many RCVSM requests outstanding.  A task can  have  only

one RCVSM request outstanding at any time.

2-28

RTS/8 INTERTASK COMMUNICATION

NODST     The requested node name is not part of the  network.   Refer

to  the  SYSGEN  procedures  in Chapter 5 for information on

entering a node name into a DECNET/8 node.

ERRST     An NSP error  occurred  that  preempted  the  source  task's

request.   (This  error  should  not occur.  ERRST indicates

either a software error or a  user  error  that  has  caused

system failure.)

LDNST     The physical link  has  been  broken,  thus  preempting  the

pending connection.

2-29

RTS/8 INTERTASK COMMUNICATION

----------							 ----------

| TRNSM  |							 | TRNSM  |

----------							 ----------

2.2.11  TRNSM

FUNCTION:

Transmit a single message over a logical link.

MESSAGE FORMAT:

ZBLOCK 3		   /RTS/8 HEADER

STATUS		   /RESERVED FOR COMPLETION STATUS

TRNSM			   /FUNCTION CODE

CHANNL		   /CHANNEL NUMBER

P1

P2

P3			   /PARAMETERS

P4

P5

PARAMETERS:

P1	       is the address of the destination node name.

P2	       is the address of the  destination  task  block  (see  Table

2-2).

P3	       is a CDF instruction to the buffer memory field.

P4	       is the address of the buffer containing the  message  to  be

transmitted.

P5	       is a negative word count of the length of the message to  be

transmitted.

USAGE NOTES:

1.  The transmit single-message request not  only  contains  the

data  message  to  be  transmitted  (like  a normal transmit

message request) but also identifies the node  and  task  to

receive the single message (like a connect initiate).

2.  NSP uses the CHANNL value to locate the CCB to be used  when

sending the message.

3.  If the physical link used to transmit the single message  is

not "on-line", it is brought up when TRNSM is issued.

4.  If the task specified in the TRNSM request does  not  exist,

no notification is given to the user of the request failure.

2-30

RTS/8 INTERTASK COMMUNICATION

EXAMPLE:

.

.

.

CAL	   /MAINLINE CODE ISSUES

SEND	   /SEND

NSP	   /USE THE NSP TASK

XMITSM	   /ADDRESS OF THE SINGLE-MESSAGE BUFFER AREA

.

.

.

XMITSM,        ZBLOCK 3	   /RTS/8 HEADER

XSMSTA,        0	   /STATUS, SET TO 0 INITIALLY

TRNSM	   /FUNCTION CODE

CHANNL	   /CHANNEL CODE FOR CCB

NODNAM	   /ADDRESS OF DESTINATION NODE NAME

TSKNAM	   /ADDRESS OF DESTINATION TASK NAME

CDF BUFFLD  /CDF INSTRUCTION TO BUFFER FIELD

XMTBUF	   /ADDRESS OF BUFFER TO BE TRANSMITTED

XMTLEN	   /NEGATIVE WORD COUNT OF LENGTH OF MESSAGE TO BE

/SENT

.

.

.

NODNAM,        TEXT/NODE.C/  /DESTINATION NODE NAME

TSKNAM,        0 	     /DESTINATION OBJECT TYPE

TEXT /CHKTSK/ /DESTINATION TASK NAME

ZBLOCK 2      /PADDING FOR SIX-WORD NAME

1;2 	     /STANDARD GROUP AND USER VALUES

STATUS CODES:

SUCST     Success.

ILLST     Too many TRNSM requests outstanding.  A task can  have  only

one TRNSM request outstanding at any time.

NODST     The requested node name is not part of the  network.   Refer

to  the  SYSGEN  procedures  in Chapter 5 for information on

entering a node name into a DECNET/8 node.

ERRST     An NSP error  occurred  that  preempted  the  source  task's

request.   (This  error  should  not occur.  ERRST indicates

either a software error or a  user  error  that  has  caused

system failure.)

LDNST     The physical link  has  been  broken,  thus  preempting  the

pending connection.

2.3  DECNET/8 AST ROUTINES

An RTS/8 task using DECNET/8 facilities  must  have  an  AST  routine.

2-31

RTS/8 INTERTASK COMMUNICATION

This routine is scheduled asynchronously with the task when any of the

following interrupt conditions occur:

o  A remote task requests a connection with the local task.

o  A remote task sends an interrupt message to the local task.

o  An established logical link is broken by the  partner  task,

by the remote NSP (because of an abort of the partner task's

execution), or by the loss of contact with the remote node.

Unlike a normal RTS/8 derail subroutine, the  DECNET/8  AST  does  not

save  and  restore the state of the accumulator, the link, or the data

field.  In addition, AST execution can be requested while the  current

AST is running.  NSP queues the AST requests.  Additional requests are

honored when the current AST routine exits,  on  a  strict  first-come

first-served basis.

The AST routine is effectively "jumped"  to  at  the  address  of  the

symbol  USRAST  with  the  accumulator zero and the data field random.

The AST routine is exited by jumping to the symbol CCBRTN in the CCB.

Upon entry to an AST routine, a  task's  CCB  contains  the  following

information:

------------------------------------------------------------------

| Location in CCB.PA |			Contents		|

|--------------------|-------------------------------------------|

|   CCBRSN	    | A reason code for the interrupt  set  for |

|		    | each execution of the AST.		|

|		    |						|

|   CCBOPL	    | The negative byte count of  any  optional |

|		    | data  or  interrupt  data passed, up to a |

|		    | maximum of 16 bytes.  If the mainline code|

|		    | depends on any of this optional data, the |

|		    | AST should buffer the data as a safeguard |

|		    | against   loss.    If  a  subsequent  AST |

|		    | execution is requested, the optional data |

|		    | could be overwritten.			|

|		    |						|

|   CCBOPT	    | A 16-word optional data buffer.  This data|

|		    | is  also  vulnerable  if  another  AST is |

|		    | executed.  It should  be  buffered  as  a |

|		    | precaution against loss if it is important|

|		    | data.					|

|		    |						|

|   CCBDOT	    | The object  type  of  the  partner  task. |

|		    | Because these optional data locations are |

|		    | vulnerable, the AST should examine  these |

|		    | values first.				|

|		    |						|

|   CCBDN1-CCBDN6    | The 6-bit name of the partner task  (task |

|		    | name   is   limited   to  12  null-padded |

|		    | characters).  This name is changed only if|

|		    | a CONINI request is received.		|

|		    |						|

2-32

RTS/8 INTERTASK COMMUNICATION

|		    |						|

|   CCBDG,CCBDU	    | The group and owner codes from  the  user |

|		    | identification  code  used by the partner |

|		    | task  (for   compatibility   with   other |

|		    | operating systems).			|

------------------------------------------------------------------

Further information on these locations can be obtained by referring to

the CCB.PA source listings.

A task can examine this information (depending on the context  of  the

interrupt)   and   act   accordingly   (such   as,  accept  or  reject

connections).

The AST routine runs at the same priority as the user task.   When  an

interrupt  condition  occurs  on  the  logical link (such as a connect

request from a remote node or reception of an interrupt message),  NSP

saves  the  task's  PC,  AC, LINK, and DF in the task's CCB.  NSP then

changes the task's PC to point to the location USRAST.  Thus, the next

time  the  task is run, the task starts in the AST routine, and not at

the point where it was  when  it  was  interrupted.   (While  the  AST

routine is executing, the task is said to be at "AST level.")

If the task cannot be run, the AST routine does not run.  However, NSP

clears   the  task's  message  wait  bit  (MSGWT  and  EORMWT)  before

scheduling an AST routine.  If a task's mainline code is to  wait  for

completion  of  an  AST  routine,  the  task  can go to sleep with the

following code:

CAL

BLKARG

MSGWT

When the AST routine has completed execution, the user task leaves AST

level  and  returns to the mainline code by jumping to location CCBRTN

in the CCB.  The contents of CCBRTN determines if other  AST  requests

are  pending.   If  other AST requests are pending, the AST routine is

reentered  at  location  USRAST;  otherwise,  the  user  task  resumes

execution at the point at which it was interrupted.

If a task wants to wait for an event flag to be posted and also  allow

ASTs  to  run,  it  should wait for the event using the RTS/8 WAITX ER

rather than the WAITE ER.  If the task  issues  a  WAITE  and  an  AST

occurs, the AST routine does not run until the event is complete.  The

reason for this is, NSP does not clear the event flag wait bit  (EFWT)

in  a  task that has received an AST.  The WAITX ER places the task in

EORMWT.

The AST routine can determine where execution was interrupted  in  the

mainline code by examining the following locations in the CCB.

Location				 Contents	      ________				 ________

CCBPC	      The  next  address  to  be  executed  in  the

mainline code.

2-33

RTS/8 INTERTASK COMMUNICATION

CCBAC	      The contents of the task's AC.

CCBXDF	      A CDF to the data field of the task.  (A task

can  determine  if  it  is  at  AST  level by

examining the contents of this  location:  if

it is nonzero, the task is at AST level.)

The AST routine can change  the  contents  of  these  locations.   For

example,  the  AST  routine  can  set the value of location CCBPC to a

different address in the mainline code to abort the section  that  was

running.   When  the  AST  routine  exits,  control  transfers to that

address.

DECNET/8 tasks must have all executable code in a  single  field.   An

AST  routine  does not save and restore the task's MQ.  An AST routine

that uses the EAE must save and restore the MQ and, if  required,  the

step counter.

If a task wishes to send a message to  another  task  that  allows  an

event  flag  for  that message, or, to schedule AST, use the following

routine:

/ROUTINE TO SEND A MESSAGE TO A TASK

/ALLOWING EVENT OR MESSAGE WAIT TO

/UNBLOCK THAT TASK

/THE EVENT FLAG ON THE MESSAGE TO BE

/SENT MUST BE CLEAR ON ENTRY

/CALL+1 = DESTINATION TASK NUMBER

/CALL+2 = MESSAGE ADDRESS

/AC = 0

/DF = CUR

/EXIT TO CALL+3 WHEN MESSAGE IS POSTED

SENDWM, 0

TAD I   SENDWM  /GET DEST TASK NUMBER

ISZ     SENDWM

DCA     SNDTSK  /STORE IN LINE

TAD I   SENDWM  /GET MESSAGE ADDRESS

ISZ     SENDWM

DCA     SNDMSG  /STORE INLINE

CAL

SEND	     /NOW SEND IT WITHOUT WAITING

SNDTSK, 0		     /GETS TASK NUMBER

SNDMSG, 0		     /GETS MESSAGE ADDRESS

TAD     SNDMSG  /COPY EVENT FLAG ADDRESS

DCA     SNDEF

CAL	     /ISSUE RTS/8 WAITX DIRECTIVE TO

ALLOW ASTS TO RUN

WAITX

SNDEF,  0

JMP I   SENDWM  /RETURN TO CALLER

2-34

RTS/8 INTERTASK COMMUNICATION

2.4  CONNECT CONTROL BLOCK

An RTS/8 task using DECNET/8 must  provide  a  Connect  Control  Block

(CCB)  to store the context of any connection made with a remote task.

Each connection in the network must have its own CCB.  In addition  to

user  data,  the  CCB  contains queue listheads and internal NSP state

information.  The CCB is defined in the source file CCB.PA.  Refer  to

Chapter 5 for more information on the CCB parameters.

Before the CCB  can  be  used,  the  user  task  must  initialize  the

following locations:

Location			   Value	       ________			   _____

CCBDRL		The entry point of the task's AST routine.

CCBSOT		The object type of the local task (normally

zero).

CCBSN1-CCBSN6	Local task name.

CCBSG,CCBSU	Local task group and owner  codes  for  the

user identification code (for compatibility

with other operating systems).

The CCB (as well as all event flags and buffers) must  reside  in  the

resident  portion  of  nonresident RTS/8 tasks.  The CCB is not longer

than 120 (octal) words and must lie in a PDP-8 page of core.  (The CCB

contains  a  small  CCB exit  routine  at  location CCBRTN, which uses

current page references.)  Once  a  connection  is  in  progress,  the

contents  of  the  CCB  cannot  be modified dynamically.  Altering the

contents of the CCB dynamically could cause a system failure.

2-35

CHAPTER 3

TERMINAL COMMUNICATIONS UTILITY (TLK)

3.1  INTRODUCTION

The utility TLK consists of  two  tasks:  "TALK"  (TLK)  and  "LISTEN"

(LSN).   TLK  allows  the  user  to send messages from one terminal to

another terminal in the same node or in a different node.   The  tasks

TLK  and  LSN  both  require  a CCB.  TLK is invoked by an MCR request

followed by a single line of  input  containing  the  node  name,  the

terminal  designation,  and  the message.  At the target node, the LSN

task prints the name of the node and the terminal sending the message,

and  the  text  of  the message.  Both TLK and LSN respond with a TLK>

prompt and have the task name "TLK." For further information regarding

MCR requests, refer to the RTS/8 User's Manual (DEC-08-ORTMA).				___________________

3.2 INVOKING TLK

An MCR REQ command is used to invoke TLK;  TLK then prompts for input.

The format of the command is:

MCR>REQ TLK <ESC>

TLK>[dstnode_][dev:]['message]

where:

dstnode_	     is the name of the target node.  If the target node is

the  local node, this argument can be omitted.  If the

target node  is  specified,  the  node  name  must  be

terminated by an underscore or a backarrow.

dev:	     is the device  designation  of  the  terminal  at  the

target  node,  which will receive the message.  If the

terminal is the operator's console, this argument  can

be  omitted.   When  using the TLK utility, the device

designation usually has the form TTYn, in which  n  is

the  terminal number of the target terminal.  TLK uses

only the number n in determining  where  to  send  the

message.   For  example,  to  talk  to terminal 3, the

following  device  designations  are  all  acceptable:

TTY3:,  TT3:,  or  3:.  The console terminal is always

terminal zero.

'message	     is any ASCII  string  containing  the  message  to  be

transmitted.  The message must be preceded by a single

quote (') and cannot extend  beyond  the  end  of  the

current line.

3-1

TERMINAL COMMUNICATIONS UTILITY (TLK)

If the line is terminated with  RETURN,  TLK  prompts  for  additional

input.   If  the line is terminated with ESCAPE, TLK stops running and

enters run-wait mode.

The message is printed at the target terminal in the following form:

TLK>srcnode_TTYn: 'message

where:

srcnode_	     is the name of the node that originated the message.

TTYn:	     is the device designation of the source terminal.

'message	     is the message printed at the target terminal.

3.3  EXAMPLES

If the user on TTYO:   at NODE1 types the message:

TLK>NODE3_TTY2:'HELLO. PLEASE TURN LP ON-LINE <ESC>

then the following message is printed on TTY2:  at NODE3:

TLK>NODE1_TTY0:'HELLO. PLEASE TURN LP ON-LINE

The following command prints a message on the  operator's  console  at

the local node:

TLK>' PLEASE MOUNT DECTAPE PART60 ON UNIT DTA2

To send messages longer than one line, the user must  wait  until  TLK

has  transmitted  a  line  successfully (indicated by a prompt) before

entering another line.  For example:

TLK>NODE12_TTYl:'IF YOU WANT HOSE LISTINGS, TAPE ON DTA1 <RETURN>

TLK>NODE12_TTYl:'UIC 256,40 NAMES H20,H2S04,HCL.FT <ESC>

3.4  ERROR MESSAGES

If TLK cannot complete transmission, it prints one  of  the  following

messages on the source terminal:

CONNECT ABORTED		    The   network   has   terminated    the

connection abnormally.  Reissue the TLK

command.

DEST LSN TASK NOT INSTALLED    The connection could not be established

because  LSN, or a similar task, is not

installed at the target node.   Contact

the   operator   at   the  target  node

3-2

TERMINAL COMMUNICATIONS UTILITY (TLK)

personally.

NO SUCH NODE		    The node name specified for the  target

node is not in the network.  The target

node name might be misspelled.  Reissue

the  TLK  command with the correct node

name.

NONEXISTENT TERMINAL	    The operating system at the target node

cannot  access  the specified terminal.

The  terminal  might  be  dedicated  to

another  job,  or  no  terminal  at the

target node has the specified number.

SYNTAX ERROR		    A syntactical error occurred in the TLK

command.   Refer to Section 3.2 for the

format  of  the  TLK  command.   Common

sources  of  error  include omission of

the underscore (or backarrow) following

the  node  name  or  the  single  quote

preceding the message.

3-3

CHAPTER 4

NETWORK INFORMATION PROGRAM (NIP)

4.1  INTRODUCTION

The Network Information Program (NIP)  prints  status  and  diagnostic

information  about  the  local  node and its surroundings.  NIP can be

made nonresident, requiring only one page of  resident  code  and  not

requiring  a  CCB.  NIP can be invoked either from the MCR terminal or

by running under program control.

A NIP printout consists of three parts:  physical line status, logical

link  status,  and physical line error status.  The data for each part

is summarized in the following sections.  All numbers are octal unless

otherwise specified.

4.2  PHYSICAL LINE STATUS

LINE		  The line number of the physical  line  (refer  to

the  example in Section 4.5).  Line numbers begin

with zero.

NODNAM		  The name of the node adjacent to the  local  node

on the specified line.

(#)		  The number of the  node  adjacent  to  the  local

node.  Node numbers are absolute and are assigned

at network generation time.  Nodes recognize each

other by these numbers;  node names can vary from

node to node.

STATE		  The state of the local line.  The state is one of

the following:

ON-LINE

OFF-LINE

STARTING

A line can be "ON-LINE" even though  no  task  is

currently using it.

DRIVER		  The type of driver (e.g., KL8A and DKC8A).

VRSN		  The version number (decimal) of the driver  being

used.

UPSINCE		  If the line is "ON-LINE," the time and date when

the  line came up are listed under this heading.

The  time  is  in  decimal,  specifying   hours,

minutes, and seconds after midnight.

4-1

NETWORK INFORMATION PROGRAM (NIP)

4.3  LOGICAL LINK STATUS

CHAN		  The channel number of the logical link.   Channel

numbers  begin  at  one.   There  is  one channel

number for each CCB.

STATE		  The state of the logical link using the specified

channel.  The state is one of the following:

UNUSED

IN-USE

WAKING (In the process of creating a logical

link)

BOOT (Used in BOOT mode)

LINE		  The physical line  number  used  by  the  logical

link.

TASK		  The task number using (or waiting  to  use)  this

channel.   (The task's name can be determined via

the SYSTAT MCR command and may not be the same as

its source name.)

SRCE-NAME[PPN]  	  The DECNET name of the task using this channel on

the  local  node.   This name can differ from the

task's RTS/8  name.   The  PPN  is  of  the  form

[GRP,USER]  and  is not used by RTS/8.  Group and

user numbers are restricted to twelve bits.

TYPE		  The DECNET object type of the source  task.   The

object  type code is reserved for future releases

of DECNET/8 providing  special  functions.   This

code   should   be  set  to  0  for  task-to-task

communication.

LINK		  The logical link number of the local  task  using

this  channel.   This  is  an  internal reference

number used by  NSP  to  define  the  link.   The

logical  link  number  can  be different from the

channel number.

DEST-NAME[PPN]	  The DECNET name, group, and user  number  of  the

remote task using the logical link.

TYPE		  The object type of the remote task.

LINK		  The logical link number being used by the  remote

task.

4-2

NETWORK INFORMATION PROGRAM (NIP)

4.4  PHYSICAL LINE ERROR STATUS

This portion of  the  NIP  output  prints  the  internal  DDCMP  error

counters.   DDCMP maintains two different groups of error counters for

the ISR associated with  each  physical  line.   One  group  of  error

counters  deals  with  error conditions that have been detected by the

DDCMP module at the other end of the physical link;  these errors have

been  reported  in  a NAK message.  The other group of counters is for

error conditions detected by the local DDCMP task in messages  it  has

received.   The  same  type  of error occurs at both the local and the

remote DDCMP.

NIP prints the number of errors of each type for each line in decimal.

The  type  of  error  is  printed  in  a  short English phrase.  Error

counters set to zero are not printed.  The seven types  of  errors  in

each group are shown in Table 4-1.

Table 4-1

Error Conditions Printed by NIP

----------------------------------------------------------------------

|		   Remote DDCMP		|	    Local DDCMP		  |

|----------------------------------|---------------------------------|

| 1    NAK with Header CRC Error	| Message with Header CRC Error	  |

|					|				  |

| 2    NAK with Data CRC Error	| Message with Data CRC Error	  |

|					|				  |

| 3 a. NAK with Rep Response Error | b. Rep Response Time-out Error  |

|					|				  |

| 4    NAK with Data Overrun Error	| Message with Data Overrun Error |

|					|				  |

| 5    NAK with Buffer Unavailable	| Message with Buffer Unavailable |

|	    Error			| Error				  |

|					|				  |

| 6    NAK with Header Format Error| Message with Header Format Error|

|					|				  |

| 7    NAK with Message Too Long   | Message Too Long Error	  |

|	    Error			|				  |

----------------------------------------------------------------------

Error Condition				 Explanation     _______________				 ___________

1  Header CRC Error	    The CRC value generated for  the  DDCMP

message  header  at  the receiving node

does not match the CRC  value  received

in  the  message  header.   All  header

information for this message  has  been

discarded  and  the data portion of the

message has been ignored.

2  Data CRC Error		    The CRC value generated  for  the  data

portion  of  the message does not match

the CRC value received.  The  data  has

been discarded.

4-3

NETWORK INFORMATION PROGRAM (NIP)

3a Reply Response		    Sent by the receiving DDCMP  module  in

reply   to   a   REP   issued   by  the

transmitting DDCMP.  This  is  used  to

indicate  that  all  the  data messages

that the transmitting DDCMP  claims  to

have  sent,  have  not been received by

the receiving DDCMP.

3b Reply Response Time-out    When the transmitting DDCMP  tasks  the

receiving DDCMP for verification of the

sequence  number  of  the   last   data

message   received,   the  transmitting

DDCMP starts a timer.  If the receiving

DDCMP   has   not  responded  within  a

specified period, a timeout occurs.

4 Data Overrun		    This  condition  occurs  when  data  is

coming  in  faster  than  the receiving

DDCMP can accept it.  This happens when

the  line  interface  device  does  not

regain control fast  enough  to  accept

incoming characters.

5 Buffer Unavailable	    The DDCMP at the receiving node did not

have  enough  pool  space  available to

handle the incoming message.   It  does

not  imply  that anything is wrong with

the message.

6 Header Format Error	    The  DDCMP  message   header   contains

invalid  or illogical information.  The

entire message is ignored.

7 Message Too Long	    The incoming message is longer than the

maximum  size  allowed by DDCMP when it

was implemented.  Normally, the maximum

message size for any DECNET node is 192

characters including the  message  size

and NSP header.

4-4

NETWORK INFORMATION PROGRAM (NIP)

4.5  SAMPLE NIP PRINTOUT

The following is a typical NIP printout:

NETWORK INFORMATION PROGRAM V1

PHYSICAL LINE STATUS     [DDCMP V4B]

LINE  NODNAM (#)	  STATE      DRIVER VRSN  UP SINCE

0    ACTON  (3)	  ON-LINE    KL8J     3A   8:03:35  28-DEC-76

1    MAYNRD (2)	 OFF-LINE    DKC8     2B

2    CONCRD (5)	 STARTING    LOCAL   15A

LOGICAL LINK STATUS     [NSP VllE]

CHAN STATE LINE TASK SRCE-NAME[PPN]  TYPE LINK DEST-NAME[PPN] TYPE LINK

1  IN-USE    0   12  TARZAN[6,0]	    10	 4  JANE  [3,1]	      7	  6

2  WAKING    1    3  DONALD[2,2]	     3	 3

3  UNUSED    1    3  DUMMY [4,4]	    29	14

4   BOOT     2    6  MASS  [1,23]	     3	44

PHYSICAL LINE ERROR STATUS

LINE  0:

3  NAKS WITH HEADER CRC ERRORS

5  NAKS WITH DATA CRC ERRORS

83  NAKS WITH REP ERRORS

1  NAK WITH BUFFER UNAVAILABLE

2  NAKS WITH HEADER FORMAT ERRORS

192  NAKS WITH MESSAGES TOO LONG

100  MSGS WITH HEADER CRC ERRORS

1  MSG WITH REP ERROR

1  MSG WITH DATA OVERRUN

10  MSGS WITH HEADER FORMAT ERRORS

9  MSGS WITH MESSAGES TOO LONG

LINE  1:  NO ERRORS

LINE  2:

3  NAKS WITH DATA OVERRUNS

1  MSG WITH REP ERROR

4-5

CHAPTER 5

RTS/8 DECNET/8 SYSTEM GENERATION

5.1  NETWORK GENERATION OVERVIEW

DECNET/8 consists of a set of assembly  modules  that  run  under  the

RTS/8  V2B  Executive.   The RTS/8 system generation procedure has not

been modified by the addition of the DECNET/8 modules.  In order to do

a  DECNET/8  system generation, a current RTS/8 user requires only the

new parameter file that contains the DECNET/8 definitions.  Users  can

write tasks that use the DECNET/8 facilities and incorporate them into

their  RTS/8  system.   However,  the user must also load the DECNET/8

support tasks and any  Interrupt  Service  Routines  (ISRs)  that  are

needed.

The DECNET/8 support tasks include DDCMP and NSP,  as  well  as  three

optional tasks:  TLK, LSN, and NIP.

o  DDCMP  implements  the  DIGITAL  Data  Communications  Message

Protocol.   This  protocol  allows message transmission over a

communications line.

o  NSP implements the Network Services Protocol.   This  protocol

handles   network   management   functions  including  routing

messages between nodes and within a node.

o  TLK controls the Talk utility.  This utility allows a user  to

send a message to any terminal in the network.

o  LSN  controls  the  Listen  utility.   This  utility   accepts

messages from TLK and prints them on the appropriate terminal.

o  NIP controls the Network  Information  Program,  which  prints

status and diagnostic information about the network.

Before generating the RTS/8 DECNET/8 system, the user must:

1.  Create the user tasks and application programs  that  are  to

use the network facilities;

2.  Decide which tasks to  run  (e.g.,  TLK,  LSN,  and  NIP  are

optional);

3.  Assign priorities to each task;

4.  Decide where the tasks are to be loaded in core.

The user is then ready to start system generation.  System  generation

involves  physically  mounting  a  copy  of  the  distribution medium,

bootstrapping the development system, and editing the parameter  files

to build the system to suit a particular configuration.

5-1

RTS/8 DECNET/8 SYSTEM GENERATION

5.2  DECNET/8 SYSTEM GENERATION PROCEDURES

The general procedures for generating an RTS/8 DECNET/8 system are:

1.  Bootstrap the development system (OS/8, PTS, or CAPS/8).

2.  Create all the user tasks and application programs  that  are

to  use  the  network facilities.  These tasks and facilities

must be created before system generation because the names of

each  task,  their  priorities,  and  their locations must be

specified when PARAM.PA is edited.

Include the user task parameters,  CHANNL,  CCB,  and  USRAST

with each task (explained in Section 5.3).

3.  Obtain a listing of  the  file  PARAM.PA.   Under  OS/8,  the

following command can be used:

.LIST PARAM.PA

4.  Use an editor to establish the values of  the  parameters  in

the   parameter   file   (PARAM.PA).    The  RTS/8  Executive

parameters that must be specified are explained in detail  in

the  RTS/8  User's Manual.  The network parameters are listed

in Table 5-2 and described in detail in Section 5.4.

PARAM.PA should be read in as an input file, edited, and then

renamed  as  an  output  file;   this procedure maintains the

integrity of DIGITAL-supplied sources.

5.  Edit the Line Control File (LCF);   the  parameters  in  this

file are explained in Section 5.7.

6.  After all the required parameters are defined,  assemble  the

sources  with  the  parameter  file.  Appendix B of the RTS/8

User's Manual lists all the RTS/8  components,  their  sizes,

and  their default origins.  For networks, assemble DDCMP and

NSP.  Assemble the tasks  TLK,  LSN,  and  NIP  if  required.

Section  5.8  describes  the  assembly process, which creates

binary files from the sources.

7.  Load the binaries of the system  tasks,  network  tasks,  and

user tasks.  Section 5.9 describes the load process.

8.  Save the RTS/8 DECNET/8 system (see Section 5.10).

Section 5.11 illustrates a sample RTS/8 DECNET/8 system generation.

5.3  USER TASK PARAMETERS

Each user task must have three parameters defined in its source if  it

uses  DECNET/8 facilities.  These three parameters can be specified at

any point in the user task.  The parameters  are  described  in  Table

5-1.

5-2

RTS/8 DECNET/8 SYSTEM GENERATION

Table 5-1

User Task Parameters

----------------------------------------------------------------------

|  Parameter    |			     Contents			  |

|---------------|----------------------------------------------------|

|    CHANNL     | Channel number of the logical channel used by  the |

|		     | task.   This  channel  number  is  used  by NSP to |

|		     | associate logical links with this task.		  |

|		     |							  |

|    CCB	     | First memory location of the Connect Control Block |

|		     | (CCB)  area used by the task.  The CCB must reside |

|		     | in the same area as the task (i.e., field CUR) and |

|		     | must not cross a page boundary.			  |

|		     |							  |

|	  USRAST     | Address of the  user's  AST  routine.   Each  task |

|		     | using  DECNET/8 must have an AST routine.  The AST |

|		     | routine must reside in the same field as the  user |

|		     | task  (i.e., field CUR).  (This symbol is coded as |

|		     | a tag rather than an equate because the  user  may |

|		     | want  to  write code to handle any conditions that |

|		     | may trap to this location.)			  |

----------------------------------------------------------------------

Example:

CHANNL = 2      /USER TASK USES CHANNEL 2

CCB	  = 2023   /THE CONNECT CONTROL BLOCK STARTS AT

/LOCATION 2023 (OCTAL)

1000  USRAST,		   /THE USER AST ROUTINE BEGINS AT LOCATION

/1000

Each task using a Connect Control Block  must  insert  nine  words  of

initial data into the CCB.  The contents of the nine words are:

Word				       Contents       ____				       ________

1		 DECNET object type.  For  intertask  communication

between two DECNET/8 nodes, the object type should

be zero indicating task-to-task communication.

2-7		 DECNET name for this task.   The  name  should  be

alphanumeric, 12 bytes, zero-padded on the right.

8		 The number 1 to indicate a group code.

9		 The number 2 to indicate an owner code.

Words 8 and 9 contain the group and  owner  codes  that  form  a  user

identification   code.    Although  DECNET/8  does  not  support  user

identification codes (referred to as UIC or  PPN  in  other  operating

systems), it assumes [1,2] if a default request is issued from another

node.  User identification codes must be in the range 1 through  7777.

Refer  to  Usage Note 3 of the CONINI request (Section 2.2.2) for more

information on UICs.

5-3

RTS/8 DECNET/8 SYSTEM GENERATION

5.4  PARAMETER FILE

Before  running  RTS/8  or  DECNET/8,  the  user   must   modify   the

DIGITAL-supplied   parameter  file  (PARAM.PA).   The  parameter  file

contains several user-defined symbols.  These  symbols  determine  the

specific  configurations for a system.  The parameter file also allows

users to specify the locations of DECNET/8 tasks and their priorities.

The DECNET/8 tasks are:  DDCMP, NSP, NIP, TLK, and LSN.  NSP and DDCMP

parameters  are  described  in  Section  5.4.2.   NIP  parameters  are

described  in  Section 5.4.3.  TLK and LSN parameters are described in

Section 5.4.4.  LCF parameters are explained in  Section  5.7.   Table

5-2 summarizes these parameters.

All fields are expressed as the field number multiplied  by  10.   For

example, field 3 is represented by 30.

Table 5-2

NETGEN Parameters

----------------------------------------------------------------------

|Task/File|Parameter|	     Function				  |

|---------|---------|------------------------------------------------|

|  DDCMP  | MAXLIN  | Sets the maximum number of physical lines to be|

|	       |	 | used.					  |

|	       |         |						  |

|	DDCMP  | KG8E    | Indicates if the  system  includes  KG8E  (CRC |

|	       |	 | computation) hardware.			  |

|	       |         |						  |

|	DDCMP  | DDCFLD  | Sets the number of the  field  (times  10)  in |

|	       |	 | which the DDCMP task resides.		  |

|	       |         |						  |

|	DDCMP  | DDCLOC  | Specifies the initial memory  location  within |

|	       |	 | the DDCMP field.				  |

|	       |         |						  |

|	NSP    | MAXCCB  | Sets the maximum  number  of  Connect  Control |

|	       |	 | Blocks to be used at any one time. 		  |

|	       |         |						  |

|	NSP    | MAXNOD  | Sets the number of node names to appear in the |

|	       |	 | Node Name Table.				  |

|	       |         |						  |

|	NSP    | NSPFLD  | Specifies the initial location of the field in |

|	       |	 | which NSP resides.				  |

|	       |         |						  |

|	NSP    | NSPLOC  |Specifies the first memory location in the  NSP |

|	       |	 | field.					  |

|	       |         |						  |

|	NSP    | NODNUM  | Sets the number of  the  node  on  which  this |

|	       |	 | DECNET/8 system operates.			  |

|	       |         |						  |

|NSP/DDCMP| MAXPKT	 | Sets the number of 14-word (octal) I/O packets |

|	       |	 | in the node pool.				  |

----------------------------------------------------------------------

5-4

RTS/8 DECNET/8 SYSTEM GENERATION

Table 5-2 (Cont.)

NETGEN Parameters

----------------------------------------------------------------------

|Task/File|Parameter|	     Function				  |

|---------|---------|------------------------------------------------|

|	NIP    | NIPFLD  | Specifies the initial location of the field in |

|	       |	 | which NIP resides.				  |

|	       |         |						  |

|	NIP    | NIPLOC  | Specifies the first memory location within the |

|	       |	 | field for the NIP task.			  |

|	       |         |						  |

|	NIP    | NIPART  | Indicates if NIP is disk-resident.		  |

|	       |         |						  |

|	NIP    | SKIMP   | Indicates if the smaller version of NIP is  to |

|	       |	 | be used.					  |

|	       |         |						  |

|	TLK    | TLK     | Sets the task number assigned to the TLK task. |

|	       |         |						  |

|	TLK    | TLKFLD  | Specifies the field in which TLK resides. 	  |

|	       |         |						  |

|	TLK    | TLKLOC  | Specifies the starting address of TLK. 	  |

|	       |         |						  |

|	LSN    | LSN 	 | Sets the task number assigned to the LSN task. |

|	       |         |						  |

|	LSN    | LSNCHN  | Sets the channel number for the LSN task. 	  |

|	       |         |						  |

|	LSN    | LSNFLD  | Specifies  the  field  number  in  which   LSN |

|	       |	 | resides.					  |

|	       |         |						  |

|	LSN    | LSNLOC  | Specifies the starting address of LSN. 	  |

|	       |         |						  |

|	LCF    | LINE 	 | Associates a physical line number with the Line|

|	       |	 | Control File.				  |

|	       |         |						  |

|	LCF    | ADJNOD  | The node number of the  node  located  at  the |

|	       | 	 | other end of the physical line.		  |

|	       |         |						  |

|	LCF    | ISRFLD  | Specifies the field  in  which  the  Interrupt |

|	       |	 | Service Routine resides.			  |

|	       |         |						  |

|	LCF    | ISRLOC  | Specifies the first memory location within the |

|	       |	 | field for the Interrupt Service Routine. 	  |

|	       |         |						  |

|	LCF    | ISRDEV  | Specifies the device code used by the Interrupt|

|	       |	 | Service Routine. 				  |

|	       |         |						  |

|	LCF    | ISRBRK  | Specifies  the  address  of  the  data   break |

|	       |	 | location used by the DP8E. 			  |

|	       |         |						  |

|	LCF    | BUFFLD  | Sets the number of the field for the I/O buffer|

|	       |	 | in the Interrupt Service Routine to reside.	  |

----------------------------------------------------------------------

5-5

RTS/8 DECNET/8 SYSTEM GENERATION

Table 5-2 (Cont.)

NETGEN Parameters

----------------------------------------------------------------------

|Task/File|Parameter|	     Function				  |

|---------|---------|------------------------------------------------|

|	LCF    | BUFLOC  | Specifies the address of the start of the  I/O |

|	       |	 | buffer used by the Interrupt Service Routine.  |

|	       |         |						  |

|	LCF    | BUFLEN  | Sets the length of the I/O buffer used by  the |

|	       |	 | Interrupt Service Routine.			  |

|	       |         |						  |

|	LCF    | KLMDEV  | Set to the device  code  for  the  KL8M  modem |

|	       |	 | control option.				  |

|	       |         |						  |

|	LCF    | KL8AMD  | Set  to  1  for  a  KL8A  with  modem  control |

|	       |	 | features.  Left undefined otherwise. 	  |

|	       |         |						  |

|	LCF    | /TASK 	 | Set  if  the  DKC8A  parallel  ISR  is   being |

|	       | =DKC8A  | assembled.  The DKC8A requires a support task. |

|	       |         |						  |

|	LCF    | KL8HDX  | Set to 1 if KL8M or KL8A half-duplex  features |

|	       |	 | are required. 				  |

----------------------------------------------------------------------

5.4.1  DECNET/8 Task Priorities

The user must edit the parameter file  to  assign  priorities  to  the

DECNET/8  tasks.  The user must have two tasks to run DECNET/8:  DDCMP

and NSP.  Optionally, the user  can  have  NIP,  TLK,  and  LSN.   For

example,  to  have  DECNET/8 support with the option of using the NIP,

TLK, and LSN utilities, the  following  lines  in  PARAM.PA  could  be

edited as follows:

Original:

/DDCMP=	   /DDCMP TASK FOR DECNET

/NSP=	   /NETWORK SERVICES PROTOCOL TASK

/NIP=	   /NETWORK INFORMATION PROGRAM

/TLK=	   /TALK UTILITY

/LSN=	   /LISTEN UTILITY

Edited version:

DDCMP = 5  /DDCMP TASK FOR DECNET

NSP = 6    /NETWORK SERVICES PROTOCOL TASK

NIP = 32   /NETWORK INFORMATION PROGRAM

TLK = 35   /TALK UTILITY

LSN = 36   /LISTEN UTILITY

The example sets the DDCMP priority to five, the NSP priority to  six,

NIP  to  32,  TLK  to 35, and LSN to 36.  The priority of DDCMP should

always be higher than the priority of NSP.  The  priorities  of  DDCMP

and NSP must be higher than user tasks but lower than the line driver.

5-6

RTS/8 DECNET/8 SYSTEM GENERATION

If any of the DECNET/8 utilities (TLK, LSN, and NIP) are not required,

the user should leave the original statements undefined:

/NIP =     /NETWORK INFORMATION PROGRAM

TLK = 35   /TASK UTILITY

LSN = 36   /LISTEN UTILITY

In this example, NIP is not being used.

5.4.2  DDCMP and NSP Parameters

The DDCMP and NSP parameters, which must be defined  by  the  user  if

DECNET/8  is to run, are described in Table 5-3.  These parameters are

located in the parameter file under the conditional:

IFDEF NSP <

They should be edited to suit the system on which DECNET/8 will run.

5-7

RTS/8 DECNET/8 SYSTEM GENERATION

Table 5-3

DDCMP and NSP Parameters

----------------------------------------------------------------------

| Parameter     |	     		      Purpose			  |

|---------------|----------------------------------------------------|

|	MAXLIN	     | Sets the maximum number of physical  lines  to  be |

|		     | used.    The   lines  are  numbered  consecutively |

|		     | beginning with zero.				  |

|		     |							  |

|	MAXCCB	     | Sets the maximum number of Connect Control  Blocks |

|		     | (CCB)  to  be  used  at any one time.  CCBs do not |

|		     | have to be in use simultaneously.  Sections  2.1.3 |

|		     | and 2.4 describe the CCB.			  |

|		     |							  |

|	MAXNOD	     | Sets the number of node  names  appearing  in  the |

|		     | Node  Name  Table.  Section 5.5 describes the Node |

|		     | Name Table.  If no names are to be entered in  the |

|		     | Node  Name  Table, MAXNOD = 0 should be specified. |

|		     | MAXNOD = 0 should  be  used  only  when  intertask |

|		     | communication is not required.			  |

|		     |							  |

|	MAXPKT	     | Specifies how much pool space is made available to |

|		     | DDCMP,  NSP,  and  the  ISRs for communication and |

|		     | queueing.  The node pool is divided  into  packets |

|		     | of  14  words each.  Each page contains 10 decimal |

|		     | packets.  The node pool space is located  in  core |

|		     | directly  after the DDCMP code.  The default value |

|		     | is 20 (decimal).					  |

|		     |							  |

|	KG8E	     | Indicates  whether  the  system  uses   the   KG8E |

|		     | hardware.  KG8E should be set to 6xx0 (where xx is |

|		     | the KG8E device code) to cause DECNET/8 to use the |

|		     | hardware.   If  the  KG8E  is not to be used, this |

|		     | parameter should be left undefined.		  |

|		     |							  |

|	NSPFLD	     | Sets the number of the field (times 10  octal)  in |

|		     | which the NSP task resides.			  |

|		     |							  |

|	NSPLOC	     | Sets the first memory location  in  the  specified |

|		     | field  for  the  NSP  task.  This location must be |

|		     | less  than  or  equal  to  3200;   otherwise,   an |

|		     | assembly   error   is  flagged.   This  parameter, |

|		     | together with NETFLD, defines the field and memory |

|		     | location of the NSP task.			  |

|		     |							  |

|	DDCFLD	     | Sets the number of the field (times 10  octal)  in |

|		     | which  the DDCMP task resides.  This field must be |

|		     | different from NSPFLD.				  |

|		     |							  |

|	DDCLOC	     | Sets the first memory location  in  the  specified |

|		     | field  for  the  DDCMP  task.  This parameter, to- |

|		     | gether with DDCFLD, defines the field  and  memory |

|		     | location  of the DDCMP task.  The default for this |

|		     | parameter is 200.				  |

----------------------------------------------------------------------

5-8

RTS/8 DECNET/8 SYSTEM GENERATION

Table 5-3 (Cont.)

DDCMP and NSP Parameters

----------------------------------------------------------------------

| Parameter     |	     		      Purpose			  |

|---------------|----------------------------------------------------|

|	NODNUM	     | Sets the number of the DECNET/8 node on which this |

|		     | system  runs.  The system manager assigns the node |

|		     | number, which must be greater  than  one.   Though |

|		     | user  tasks  can  refer  to  individual  nodes  by |

|		     | symbolic names (see Section 5.5), all  nodes  must |

|		     | refer  (internally)  to  this number.  This number |

|		     | must be entered into the Node Name  Table  of  all |

|		     | other  nodes where communication with this node is |

|		     | required.  This  parameter  also  checks  received |

|		     | routing headers.					  |

----------------------------------------------------------------------

Example:

IFDEF NSP <

MAXLIN=3	      /NUMBER OF PHYSICAL LINES BEING USED

/E.G. 3 FOR 3 LINES

/THESE ARE NUMBERED 0,1,2

MAXCCB=3	      /NUMBER OF LOGICAL CHANNELS (CCB'S) BEING USED

/E.G. 3 FOR 3 CHANNELS

/THESE ARE NUMBERED 1,2,3

MAXNOD=4	      /NUMBER OF NODE NAMES IN NODE TABLE

MAXPKT= 24	      /SET TO NUMBER OF NODE POOL PACKETS

/THE NODE POOL EXISTS AT THE END OF DDCMP

/JUST BEFORE THE LCB TABLE (SIMILAR TO THE CLOCK

QUEUE)

/EACH PACKET REQUIRES 14 WORDS OCTAL. (ABOUT 10. PER

PAGE)

/(THE DEFAULT REQUIRES 2 PAGES CORE)

/KG8E=6110	      /SET TO IOT SKELETON IF KG8E IS PRESENT (E.G. 6110)

NSPFLD=20	      /FIELD OF NSP TASK AND MOST NETWORK TABLES

/(TABLES INCLUDE CCBTAB, LNKTAB, NODTAB AND NETTAB)

NSPLOC=3200	/ORIGIN OF NSP TASK. MUST BE .LE. 3200

/THE DEFAULT IS CURRENTLY 3200

DDCFLD=10	      /FIELD OF DDCMP TASK, LCBTAB AND 'NODE POOL'

/THIS FIELD MUST BE DIFFERENT FROM NSPFLD

DDCLOC=200	      /ORIGIN OF DDCMP TASK

/THE ABOVE MUST BE BELOW 5000-SIZE OF NODE POOL AND

LCBTAB

/THE DEFAULT IS CURRENTLY 200

NODNUM=ll	      /NODE NUMBER OF THIS NODE

LCBSIZ= 32	      /GLOBAL DEFINITION OF LCB SIZE (DO NOT ALTER)

PKSIZE= 14	      /GLOBAL DEFINITION OF PACKET SIZE (DO NOT ALTER)

5.4.3  NIP Parameters

The NIP parameters, which must be defined  if  NIP  is  required,  are

described  in  Table 5-4.  These parameters are located after the Node

Name Table under the conditional:

5-9

RTS/8 DECNET/8 SYSTEM GENERATION

IFDEF NIP <

They should be edited to suit the user's configuration.

Table 5-4

NIP Parameters

----------------------------------------------------------------------

| Parameter     |	     		      Purpose			  |

|---------------|----------------------------------------------------|

|	NIPFLD	     | Sets the number of the field (times 10  octal)  in |

|		     | which NIP resides.				  |

|		     |							  |

|	NIPLOC	     | Sets the first location in the specified field for |

|		     | NIP.    This   parameter,  together  with  NIPFLD, |

|		     | defines the field  and  memory  location  of  NIP. |

|		     |							  |

|	NIPLOG	     | Sets the number of a task to a line printer or TTY |

|		     | task.  For example, NIPLOG=TTY2.  IF NIPLOG is not |

|		     | specified it defaults to LPT.			  |

|		     |							  |

|	NIPRES	     | Sets the non-resident part of NIP  to  400  words. |

|		     |							  |

|	NIPART*	     | Indicates if NIP is disk-resident.  If  NIPART  is |

|		     | not  defined,  NIP is always resident.  For NIP to |

|		     | be  nonresident,  NIPART  must  be  set   to   the |

|		     | partition in which it is to reside.		  |

|		     |							  |

|	SKIMP**	     | Indicates that the 8-page version of NIP is to  be |

|		     | used  instead of the standard 12-page version.  If |

|		     | this option is used, an abbreviated NIP display is |

|		     | produced when NIP is run.  The full version of NIP |

|		     | is assumed by default.				  |

----------------------------------------------------------------------

* The first page of NIP is the resident portion.   The  remainder  can

be  made nonresident.  If part of NIP is to be nonresident, NIP should

start on a page boundary that is not a multiple of 400.  The partition

numbers and their locations are defined in the swapper.

** The abbreviated NIP display does not contain any dates,  times,  or

printouts  obtained by the full-sized NIP.  All DDCMP errors, however,

are still properly logged by the abbreviated form of NIP.

Example:

IFDEF NIP <

NIPFLD= 30		   /FIELD OF NIP

NIPLOC= 1000		   /LOCATION OF NIP

NIPART= 2		   /PARTITION FOR NIP

SKIMP=  1		   /SET TO 1 TO GET SHORT NIP

>

In this example, NIP resides in Field 3 at location 1000.  It runs  in

two  partitions and the shorter 8-page version rather than the 12-page

version is used.

5-10

RTS/8 DECNET/8 SYSTEM GENERATION

5.4.4  TLK and LSN Parameters

The TLK and LSN parameters, which must be defined if network  terminal

communications is required, are described in Table 5-5.

Table 5-5

TLK and LSN Parameters

----------------------------------------------------------------------

| Parameter     |	     		      Purpose			  |

|---------------|----------------------------------------------------|

|	TLKFLD	     | Sets the number of the field (times 10  octal)  in |

|		     | which TLK resides.				  |

|		     |							  |

|	TLKCHN	     | Specifies the channel number to be used by TLK.	  |

|		     |							  |

|	TLKLOC	     | Sets the first location in the specified field for |

|		     | TLK.    This   parameter,  together  with  TLKFLD, |

|		     | defines the field and memory location of TLK. 	  |

|		     |							  |

|	LSNCHN	     | Specifies the channel number to be used by LSN.	  |

|		     |							  |

|	LSNFLD	     | Sets the number of the field (times 10  octal)  in |

|		     | which LSN resides.				  |

|		     |							  |

|	LSNLOC	     | Sets the first location in the specified field for |

|		     | LSN.    This   parameter,  together  with  LSNFLD, |

|		     | defines the field and memory location of LSN.	  |

----------------------------------------------------------------------

Example:

IFDEF TLK <

TLKFLD = 30 	/TLK RESIDES IN FIELD 3

TLKLOC = 1000 	/TLK RESIDES IN LOCATION 1000

/OF FIELD 3

TLKCHN = 2 	/TLK USES CHANNEL 2

IFDEF LSN <

LSNFLD = 20 	/LSN RESIDES IN FIELD 2

LSNLOC = 1000 	/LSN RESIDES IN LOCATION 1000 OF

/FIELD 2

LSNCHN = 3 	/LSN USES CHANNEL 3

5.5  NODE NAME TABLE

During system generation, the correspondence between  node  names  and

node numbers is specified in the Node Name Table.  The network manager

assigns a node number to each node in a network.  Node numbers must be

in the range one to 177 (octal).  Each node number must be unique.  In

contrast, users assign  node  names  for  the  nodes  that  they  use.

Consequently,  a node in the network can have several names or none at

all, but each has a unique node number.

5-11

RTS/8 DECNET/8 SYSTEM GENERATION

The node name can be used by user tasks when referring to  this  node.

The node number is used by NSP to recognize messages intended for this

node.  Figure 5-1 illustrates the Node Name Table section of the

parameter file.

----------------------------------------------------------------------

|    /NODE TABLE ENTRIES						  |

|									  |

|    /EACH ENTRY HAS THE FORM					  |

|    /WORDS 1-3 	   NODE NAME (6-BIT, 0-PADDED)			  |

|    /WORD 4	   LINE NUMBER					  |

|    /WORD 5	   BIT 0=1 IF ADJACENT NODE			  |

|    /		   BITS 4-11 CONTAIN NODE NUMBER		  |

|									  |

|    	  IFDEF TASK <     IFZERO TASK-NSP <			  |

|    	  FIELD NETFLD%10					  |

|    	  *7500							  |

|									  |

|    NODTAB, TEXT    /NODE.Y/					  |

|    	  *. - 1						  |

|      	  0		   /LINE NUMBER				  |

|    	  4000		   /NODE NUMBER				  |

|    	  *NETTAB+4						  |

|    	  NODNUM	   /OUR NODE NUMBER			  |

|    	  TEXT /NODE.X/	   /OUR NODE NAME WITH ADJACENT FLAG	  |

|    			   /SET					  |

|    	  FIELD 0						  |

|    	  >>							  |

|    	  >							  |

----------------------------------------------------------------------

Figure 5-1  Node Name Table

The user modifies the line

NODNUM	   /OUR NODE NUMBER

to add the node number, and the line

TEXT /NODE.X/ /OUR NODE NAME

to add the node name.  In the following example, three nodes are

named.  (The network can consist of more than three nodes.) Node 2 has

the node name of NOD22 and is an adjacent node.  (The node is directly

connected to the user's system;  no route-through is needed.) Node 3

has the node name NOD506 and is an adjacent node.  Node 5 has the node

name X and is not adjacent (that is, it is not directly connected).

NODTAB,  TEXT/NOD22/

*.-1

0

4002

TEXT/NOD506/

*.-1

5-12

RTS/8 DECNET/8 SYSTEM GENERATION

1

4003

TEXT/X/;0;0

1

5

To send messages to other terminals on the  user's  system,  the  user

must  include  the  local  system's name in the Node Name Table.  If a

node number of an adjacent node is not known, the user should set  the

node  number  to  zero  in the Node Name Table.  This procedure can be

used only for adjacent nodes  (i.e.,  one  directly  connected  via  a

physical link).

The number of entries in the Node Name Table must be equivalent to the

parameter  MAXNOD  specified  earlier in the parameter file.  The user

must enter the exact number of entries  in  the  Node  Name  Table  as

indicated by MAXNOD.

5.6  INTERRUPT SERVICE ROUTINES

Once  information  on  NSP,  DDCMP,  NIP,  TLK,  and  LSN   has   been

incorporated  and  the  Node  Name Table set up, the user must specify

information on the system's physical lines.  For  each  physical  line

attached  to the system, DECNET/8 must have an associated line driver,

called an Interrupt Service  Routine  (ISR),  and  an  associated  I/O

buffer  for the ISR to use.  The user sets the size of the I/O buffer;

the buffer must be at least as long as the largest message that is  to

be  handled by the local node.  Each I/O buffer should be at least 193

locations.  However, an optimal buffer size  is  about  256  (decimal)

locations.

The user must allocate space for these ISRs (together with  their  I/O

buffers)  during system generation.  The ISRs are loaded with the user

tasks (see Section 5.9).  Although these files  are  not  tasks,  they

must  be  loaded into the core-image.  If any ISR is not loaded, DDCMP

halts at location ISRHLT during system startup.

The ISRs supplied by DIGITAL vary in sizes.  Table 5-6 lists the  ISRs

supported  by  DIGITAL  and their sizes.  A user can also write a line

driver (ISR)  to  support  a  communications  line  not  supported  by

DIGITAL.  The DKC8-A is the only ISR that runs as a task.

The KL8E, KL8J-A and KL8A (without modem control) support asynchronous

and full-duplex data transmission.  They do not provide modem control.

The KL8A  supports  four  lines,  which  can  be  used  for  different

purposes.    For   example,   two   could  be  used  by  DECNET/8  for

communications, one could be used by the MCR, and one by OS/8  in  the

background.   For  each  line  used for DECNET/8 communications, there

must be an Interrupt Service Routine and an I/O buffer.  If  the  KL8A

is  to  be used as a communications line, the following statement must

appear in the parameter file:  KL8A = 1.

5-13

RTS/8 DECNET/8 SYSTEM GENERATION

Table 5-6

Interrupt Service Routines Supported by DIGITAL

----------------------------------------------------------------------

|	Device   | Filename Containing ISR Code |  Size (length in pages) |

|-----------|------------------------------|-------------------------|

|	KL8E	 |	    KL8ISR.PA		|     4			  |

|	KL8J-A	 |	    KL8ISR.PA		|     4			  |

|	KL8A	 |	    KL8ISR.PA		|     4			  |

|	KL8M	 |	    KL8ISR.PA		|     4			  |

|	DP8E	 |	    DP8ISR.PA		|     6 + BUFSIZ/128	  |

|	DKC8-A	 |	    DKCISR.PA		|     4			  |

|	LOCAL	 |	    LCLISR.PA		|     3			  |

----------------------------------------------------------------------

The KL8M and KL8A (with modem control) support asynchronous  and  both

full-   and   half-duplex   data   transmission  with  modem  control.

The DP8E supports synchronous and full-duplex data  transmission  with

modem control.

The DKC8-A supports parallel and full-duplex data transmission.   This

ISR is used for hard-wired PDP-8 inter-processor communications only.

There is also a pseudo-line driver (LOCAL) that returns data to itself

(without a physical line).  Its code is in the file LCLISR.PA.

5.7  LINE CONTROL FILE

The Line Control File (LCF) is a short  source  file  similar  to  the

parameter  file.   It  contains  parameters that describe a particular

line driver (ISR).  Each physical line has its own  Interrupt  Service

Routine.   Consequently,  each  physical  line  or  ISR  must  have  a

corresponding line control file to describe specific information about

the  ISR,  such  as where the ISR is to reside and what IOTs it should

use.  Section 5.12 shows the source code for the  Line  Control  File.

Table  5-7  describes  each parameter in the Line Control File.  After

creating a line control file for each ISR, the user must assemble each

file;  Section 5.8 describes the assembly process.

Table 5-7

Line Control File Parameters

----------------------------------------------------------------------

| Parameter     |	     		      Purpose			  |

|---------------|----------------------------------------------------|

|	LINE	     | Associates a physical line number with  this  Line |

|		     | Control File.  Line numbers begin at 0. 		  |

|		     |							  |

|	ADJNOD	     | Specifies the node number of the node  located  at |

|		     | the other end of the physical line. 		  |

|		     |							  |

|	ISRFLD	     | Sets the number of the field (times 10  octal)  in |

|		     | which the Interrupt Service Routine resides. 	  |

----------------------------------------------------------------------

5-14

RTS/8 DECNET/8 SYSTEM GENERATION

Table 5-7 (Cont.)

Line Control File Parameters

----------------------------------------------------------------------

| Parameter     |	     		      Purpose			  |

|---------------|----------------------------------------------------|

|	ISRLOC	     | Specifies the memory  location  within  the  field |

|		     | where   the   ISR   resides.   This  parameter  in |

|		     | conjunction with  ISRFLD  defines  the  field  and |

|		     | memory location of this Interrupt Service Routine. |

|		     |							  |

|	ISRDEV	     | Specifies the device code used  by  the  Interrupt |

|		     | Service  Routine.  For KL8As, this is set to 100+n |

|		     | where n is the KL8A line number (e.g., 103).   For |

|		     | all other ISRs, this is of the form xx. 		  |

|		     |							  |

|	ISRBRK	     | Indicates whether the system uses  the  DP8E  ISR. |

|		     | ISRBRK  should  be  set to the data break location |

|		     | used by the DP8E to use this ISR.  If this ISR  is |

|		     | not  to  be used, it should be set to zero or left |

|		     | undefined. 					  |

|		     |							  |

|	BUFFLD	     | Sets the number of the field where the I/O  buffer |

|		     | for the ISR resides. 				  |

|		     |							  |

|	BUFLOC	     | Specifies the address of the first memory location |

|		     | for  the  I/O  buffer used by the ISR.  The buffer |

|		     | need not start on a page boundary. 		  |

|		     |							  |

|	BUFLEN	     | Sets the length of the I/O buffer used by the ISR. |

|		     | This   parameter   in   conjunction   with  BUFLOC |

|		     | determines the exact location of the I/O buffer in |

|		     | memory. 						  |

|		     |							  |

|	KLMDEV	     | Sets the device code for the  KL8M  modem  control |

|		     | option (e.g., 46).  This parameter is used only by |

|		     | KL8JA  service   routines   with   modem   control |

|		     | features. 					  |

|		     |							  |

|	KL8AMD	     | Set to 1 for a KL8A line 3  requiring  KL8A  modem |

|		     | control   features.    This   parameter   is  left |

|		     | undefined otherwise. 				  |

|		     |							  |

|	/TASK=DKC8A  | The DKC8A is the only ISR that runs as  a  support |

|		     | task.  This parameter must be defined if the DKC8A |

|		     | parallel ISR is being assembled.  DKC8A must  also |

|		     | be  defined  in  the  RTS/8  parameter  file.  The |

|		     | priority of this task should be close  to  DDCMP's |

|		     | priority.  A recommended priority is one less than |

|		     | DDCMP's priority. 				  |

|		     |							  |

|	KL8HDX	     | Set to 1 if KL8M or KL8A half-duplex features  are |

|		     | required.    This   parameter  is  left  undefined |

|		     | otherwise. 					  |

----------------------------------------------------------------------

5-15

RTS/8 DECNET/8 SYSTEM GENERATION

5.8  ASSEMBLY PROCESS

After creating the user tasks with the  DECNET/8  parameters  defined,

editing the parameter file to suit the specific configuration on which

RTS/8 DECNET/8 is to run, and creating a Line Control  File  for  each

line  driver,  the user is ready to assemble the system.  The assembly

process creates binary files that are ready to be loaded and run.  The

following commands illustrate the assembly process for RTS/8 DECNET/8.

.PAL PARAM-NB,TASK,CCB	/(USER TASK ASSEMBLY)

.PAL PARAM-NB,DDCMP		/(DECNET/8 MODULE ASSEMBLY)

.PAL PARAM-NB,NSP		/(DECNET/8 MODULE

.PAL PARAM-NB,NIP,CCB		/ ASSEMBLY)

.PAL PARAM-NB,TLK,CCB

.PAL PARAM-NB,LSN,CCB

.PAL DEV<PARAM,LCFN,FILENAME  /(LINE CONTROL FILE ASSEMBLY)

Each user task requesting network functions must be assembled with the

file  CCB.PA and the file PARAM.PA.  The CCL switch -NB indicates that

PARAM is not to be used as the name of the binary file being  created.

To  use  DECNET/8,  DDCMP, the line driver, and NSP must be assembled.

The utility programs NIP, TLK, and LSN  are  optional.   If  required,

they  must be assembled.  Each Line Control File that was created must

also be assembled.  For example, if three Line Control Files have been

created, they must each be assembled:

.PAL dev <PARAM,LCFl,filename

.PAL dev <PARAM,LCF2,filename

.PAL dev <PARAM,LCF3,filename

An example of the assembly process is given in Section 5.11.

5.9  LOAD PROCESS

After assembling all the required modules, the user is ready  to  load

the  system.   All  required system tasks, user tasks, DECNET/8 tasks,

and ISRs can be loaded at one time.  For example:

.R ABSLDR

*RTS8,CLOCK,TTY,MCR,NSP,DDCMP <return>

*TLK,LSN,NIP,USRTSK <esc>

After loading the system, the user can either run it or save  it.   If

the system is saved, the user will not have to rebuild it each time it

is needed.

5.10  SAVE

After the system is loaded, the  user  can  save  it  by  issuing  the

following command:

.SAVE SYS filename

5-16

RTS/8 DECNET/8 SYSTEM GENERATION

5.11  SAMPLE DECNET/8 PARAMETERS

/RTS8 V2 EXEC PARAMETERS - EDITED BY USER

PDP8E=	     /1 IF PDP 8/E, ELSE 0

/PDP12=	     /1 IF PDP-12, 0 OR UNDEFINED IF NOT PDP-12

/EAE=	     /1 IF SYSTEM SHOULD SAVE MQ

/PWRFAL=	     /1 IF POWER FAIL/RESTART ENABLED IN HARDWARE

KL8A=0	     /NUMBER OF KL8A'S IN USE

/I.E. 1 IF ONE KL8A (UP TO 4 LINES)

HGHFLD=	     /HIGHEST FIELD USED (KG 30 FOR A 16K MACHINE)

IFDEF CUR <

IFNZRO HGHFLD-CUR4000 <CURBIG,ERROR>> /FLAG WARNING IF

UNDEFINED FIELD SEEN

NTASKS=	     /NUMBER OF TASKS IN SYSTEM

CHECKP=0        /1 IF ANY NON-RESIDENT TASK IS CHECKPOINTABLE

PARTNS=0        /NUMBER OF MEMORY PARTITIONS DEFINED

/(THE N PARTITIONS ARE NUMBERED FROM 0 TO N-1)

/COMMON TASK NUMBERS - EDITED BY USER

/IT IS ADVISABLE TO DEFINE ALL TASKS HERE. NAMES GIVEN BELOW

/ARE USED BY SOME SYSTEM TASKS AND SHOULD REMAIN COMMENTED OUT

/IF THE CORRESPONDING TASK IS NOT INCLUDED IN THE SYSTEM

/PWRF=	     /POWER FAIL HANDLING TASK

/CLOCK=	     /CLOCK HANDLER - SHOULD BE HIGH PRIORITY

/SWAPPER=	     /NONRESIDENT TASK SWAPPER TASK

/TTY=	     /TELETYPE DRIVER TASK

/LPT=	     /LINE PRINTER DRIVER TASK

/MCR=	     /MONITOR CONSOLE ROUTINE

/DTA=	     /DECTAPE DRIVER TASK

/LTA=	     /LINCTAPE DRIVER TASK

/RK8=	     /RK8 OR RK8E DISK DRIVER TASK

/RF08=	     /RF08 DISK DRIVER TASK

/DF32=	     /DF32 DISK DRIVER TASK

/CSA=	     /CASSETTE DRIVER TASK

/CSAF=	     /CASSETTE FILE SUPPORT TASK

/UDC=	     /UNIVERSAL DIGITAL CONTROLLER TASK

/RX8A=	     /FIRST FLOPPY CONTROLLER

/RX8B=	     /SECOND FLOPPY CONTROLLER

/RX8C=	     /THIRD FLOPPY CONTROLLER

/RX8D=	     /FOURTH FLOPPY CONTROLLER

/OS8= NTASKS    /OS/8 SUPPORT - NORMALLY LOWEST PRIORITY

/OS8F=	     /OS/8 FILE SUPPORT

/DDCMP=	     /DDCMP TASK FOR DECNET

/NSP=	     /NETWORK SERVICES PROTOCOL TASK

/NIP=	     /NETWORK INFORMATION PROGRAM

/TLK=	     /NETWORK TERMINAL COMMUNICATIONS TASK TRANSMITTER

/LSN=	     /NETWORK TERMINAL COMMUNICATIONS TASK RECEIVER

/NULL8A=        /NULL JOB FOR PDP-8/A

/EXIT= 	     /EXIT TASK

/DKC8A= 	     /AUXILIARY DKC8A HANDLER

/SOFTWARE PARAMETERS - EDITED BY USER

XLIST   1

IFDEF   OS8 <

XLIST   LSTFLG

0SFLDS=	     /NUMBER OF FIELDS TO ALLOCATE TO OS/8 (KG 2)

0STTDV=	     /OS/8 TELEPRINTER DEVICE CODE (KG 04)

5-17

RTS/8 DECNET/8 SYSTEM GENERATION

/OSKBDV=	     /OS/8 KEYBOARD DEVICE CODE (KG 03 = CONSOLE)

/DEFAULT IS OSTTDV-1

0SSYSD=	     /OS/8 SYSTEM DEVICE DRIVER TASK (KG RK8)

/OSFILL=	     / FILLS NEEDED AFTER LINE FEED

/(EG 4 FOR 2400 BAUD VT05)

/OS80RG=	     /ORIGIN (IN FIELD O)

XLIST   1>

IFDEF   MCR     <

XLIST   LSTFLG

/MCRCLK=	     /0 IF MCR TOD FACILITIES TO BE OMITTED (DEFAULT = 1)

/MCRSYS=	     /0 IF MCR SYSTAT FACILITY NOT DESIRED

/1 IF DESIRED (DEFAULT)

/MCRFLD=	     /FIELD OF MCR

/MCRORG=	     /LOCATION OF MCR (DEFAULT IS END OF FIELD)

/MCRPRT=	     /PARTITION NUMBER OF MCR (IF NON-RESIDENT)

/MCRCDV=	     /CONSOLE TO BE USED BY MCR, E.G. TTY

/DEFAULT IS TTY

XLIST   1>

IFDEF   CLOCK <

XLIST   LSTFLG

CLKTYP=	     /O(DK8EA/DK8EC) OR l(KW12) OR 2(PDP8/A) OR 3(DK8EP)

CLKQLN=20	     /MINIMUM NUMBER OF ENTRY SLOTS IN THE CLOCK QUEUE

/MAY BE CHANGED BY USER

DECIMAL

HERTZ=	     /NUMBER OF HARDWARE TICKS PER SECOND

SHERTZ=	     /NUMBER OF SYSTEM TICKS PER SECOND

IFNZRO  CLKTYP1 <HERTZ=1750>	/FORCE DK8EP,KW12 TO 1 KHZ

OCTAL

XLIST   1>

IFDEF   LPT     <

/LPTLOC=

/LPTFLD=

>

IFDEF   DTA     <

/DTALOC=

/DTAFLD=

>

TBLLST= 0		     /SET TO 'LSTFLG' IF YOU DON'T DESIRE

/TO SEE TABLES WHEN PARAMETER FILE IS

/NOT LISTED.

/SYSTEM LOCATIONS:

MSGTBL= 1200-2		     /TASK MESSAGE TABLE

TSTABL= NTASKS+22+MSGTBL-4	     /TASK STATE TABLE - HOLDS

/TASK LINK,UM,DF,IF,PC,AC,MQ

TFTABL= NTASKS+24+TSTABL-1	     /TASK FLAGS TABLE - HOLDS

/TASK STATUS FLAGS

XLIST   1

IFDEF   SWAPPER <

XLIST   LSTFLG

SYS=	     /SWAP DEVICE DRIVER TASK (KG RK8)

/SUNIT=	     /SWAP DEVICE PHYSICAL DRIVE UNIT (KG O FOR RKAO)

IFNDEF  SUNIT <SUNIT= 0>	     /DEFAULT SWAP UNIT IS O

FIELD 0

/

5-18

RTS/8 DECNET/8 SYSTEM GENERATION

/PARTITION TABLE (PARTBL) ENTRIES:

/MUST BE INITIALIZED BY USER AS EXPLAINED IN THE COMMENTS

/DON'T FORGET TO REMOVE LEADING "/" FROM LINES USED

/

RESTBL= TFTABL+NTASKS+2		     /RESIDENCY TABLE

PARTBL= NTASKS-SWAPPER2+RESTBL+37774    /PARTITION TABLE

*PARTBL

XLIST   1

IFNZRO  PARTNS	    <

XLIST   TBLLST

MFLDOO=	     	     /MEMORY FIELD OF PARTITION 0 (E.G. 2 IF FIELD 2)

ADDROO=	     	     /LOWEST ADDRESS IN PARTITION 0

SIZE00=		     /SIZE OF PARTITION 0 (CORE PAGES)

SIZE0010+MFLD0010+4000

ADDR00

ZBLOCK  2

XLIST

IFNZRO  PARTNS-1	     <

XLIST   TBLLST

MFLD01=		     /MEMORY FIELD OF PARTITION 1

ADDR01=		     /LOWEST ADDRESS IN PARTITION 1

SIZE01=		     /SIZE OF PARTITION 1 (CORE PAGES)

SIZE0110+MFLD0110+4000

ADDR01

ZBLOCK  2

XLIST

IFNZRO  PARTNS-2	     <

XLIST   TBLLST

MFLD02=		     /MEMORY FIELD OF PARTITION 2

ADDR02=		     /LOWEST ADDRESS IN PARTITION 2

SIZE02=		     /SIZE OF PARTITION 2

SIZE0210+MFLD0210+4000

ADDR02

ZBLOCK  2

XLIST

IFNZRO  PARTNS-3	     <

XLIST   TBLLST

MFLD03=	          /PARTITION 3

ADDR03=

SIZE03=

SIZE0310+MFLD0310+4000

ADDR03

ZBLOCK  2

XLIST

IFNZRO  PARTNS-4      <

XLIST   TBLLST

MFLD04=	     /PARTITION 4

ADDR04=

SIZE04=

SIZE0410+MFLD0410+4000

ADDR04

ZBLOCK  2

XLIST

IFNZRO  PARTNS-5      <

5-19

RTS/8 DECNET/8 SYSTEM GENERATION

XLIST   TBLLST

MFLD05=	     /PARTITION 5

ADDR05=

SIZE05=

SIZE0510+MFLD0510+4000

ADDR05

ZBLOCK  2

XLIST

IFNZRO  PARTNS-6 <

XLIST   TBLLST

MFLD06=	     /PARTITION 6

ADDR06=

SIZE06=

SIZE0610+MFLD0610+4000

ADDR06

ZBLOCK  2

XLIST

IFNZRO  PARTNS-7      <

XLIST  TBLLST

MFLD07=	     /PARTITION 7

ADDR07=

SIZE07=

SIZE0710+MFLD0710+4000

ADDR07

ZBLOCK  2

XLIST 1>>>>>>>>

/

/ADDITIONAL PARTITIONS MAY BE DEFINED BY THE USER AS SHOWN ABOVE

/FURTHERMORE, THE PARTITION TABLE MAY RESIDE ANYWHERE IN FIELD ZERO

/

PRTEND=.	     /NOTE END OF PARTITION TABLE

XLIST 1>

IFDEF TTY	<

XLIST LSTFLG

/	     TTDEV=	     /PRINTER DEVICE CODE - DEFAULT IS 4

/	     KBDEV=	     /KEYBOARD DEVICE CODE - DEFAULT IS TTDEV-1

/	     CONSOL=	     /1 MEANS CONSOLE TTY (DEFAULT)

/	     VT50=	     /1 ENABLES CTRL/S AND CTRL/Q

/	     SCOPE=	     /1 MEANS TTY CAN DO A BACKSPACE

/	     FILL=	     /NUMBER OF FILL CHARACTERS, I.E. 4

/	     WIDTH=	     /TTY LINE WIDTH (0 MEANS INFINITE), DEFAULT

/				IS 120

/	     TAB=	     /1 IF TTY HAS HARDWARE TABS

/	     0LDTTY=	     /1 TO USE OLD 2-PAGE TTY HANDLER

/	     LSBOT=	     /1 LISTS BOTH HANDLERS (DEFAULT 0)

/	     TTFLD=	     /FIELD OF TTY TASK

/	     TTLOC=	     /LOCATION OF TTY TASK

XLIST   1>

IFNZRO  KL8A    <

XLIST   LSTFLG

/KL8ADV=		     /KL8A DEVICE CODE - DEFAULT IS 40

/KL8ACT=		     /KL8A CONNECT ROUTINE PAGE - DEFAULT IS 7400

XLIST   1>

IFDEF   EXIT    <

XLIST   LSTFLG

5-20

RTS/8 DECNET/8 SYSTEM GENERATION

/EXITFLD=       /FIELD OF EXIT TASK

/EXITLOC=       /LOCATION OF EXIT TASK

XLIST   1>

XLIST   LSTFLG

IFNDEF  PDP8E   <PDP8E=1>

IFNDEF  PDP12   <PDP12=0>

IFNDEF  EAE     <EAE=0>

IFNDEF  PWRFAL  <PWRFAL=0>

IFNDEF  KL8A    <KL8A=0>

XLIST   1

IFDEF   NSP <

XLIST   LSTFLG

MAXCCB=	     /NUMBER OF LOGICAL CHANNELS (CCB'S) BEING USED

/E.G. 3 FOR 3 CHANNELS

/THESE ARE NUMBERED 1,2,3

MAXNOD=	     /NUMBER OF NODE NAMES IN NODE TABLE

NSPFLD=	     /FIELD OF NSP TASK AND MOST NETWORK TABLES

/(TABLES INCLUDE CCBTAB, LNKTAB, NODTAB AND NETTAB)

NSPLOC= 2600    /ORIGIN OF NSP TASK. MUST BE .LE. 2600

/THE DEFAULT IS CURRENTLY 3200

NODNUM=	     /NODE NUMBER OF THIS NODE

/IMPORTANT RELATIVE ORIGINS WITHIN NETWORKS TASKS

DRLXIT= NSPLOC+4400 /ADDRESS OF AST DE-QUEUER

CCBTAB= DRLXIT+200 /ADDRESS OF CCB TABLE

NODTAB= CCBTAB+100 /ADDRESS OF NODE TABLE

NETTAB= NODTAB+60 /ADDRESS OF NETWORK 'INFORMATION' TABLE

/THE DEFAULT NETWORKS TASKS USE CORE AS FOLLOWS:

/DDCMP: PAGE 0, 0200-3577 (1 LINE, 2 PAGE NODE POOL)

/NSP:   PAGE 0, 3200-7577

/NETWORK TASKS USE PAGE 0 AS FOLLOWS:

/DDCFLD:	     10-12, 30-77

/NSPFLD: 	     15-17, 77-177

/NODE TABLE ENTRIES

/EACH ENTRY HAS THE FORM

/WORDS 1-3      NODE NAME (6-BIT, 0-PADDED)

/WORD 4	     LINE NUMBER

/WORD 5	     BIT 0=1 IF ADJACENT NODE

/		     BITS 4-11 CONTAIN NODE NUMBER

IFDEF TASK < IFZERO TASK-NSP <

FIELD NSPFLD%10

*NODTAB

NODTAB, TEXT    /NAME/

0 		     /LINE NUMBER

0 		     /NODE NUMBER

*NETTAB+4

NODNUM 	     /OUR NODE NUMBER

TEXT    /NAME/  /OUR NODE NAME

FIELD 0

>>

>

XLIST   1

IFDEF   DDCMP <

XLIST   LSTFLG

MAXLIN=	     /NUMBER OF PHYSICAL LINES BEING USED

/E.G. 3 FOR 3 LINES

5-21

RTS/8 DECNET/8 SYSTEM GENERATION

/THESE ARE NUMBERED 0,1,2

MAXPKT= 24	     /TO NUMBER OF NODE POOL PACKETS TO ALLOW

/THE NODE POOL EXISTS AT THE END OF DDCMP

/JUST BEFORE THE LCB TABLE (SIMILAR TO THE CLOCK QUEUE)

/EACH PACKET REQUIRES 14 WORDS OCTAL. (ABOUT 10. PER

/     PAGE)

/(THE DEFAULT REQUIRES 2 PAGES CORE)

/KG8E=	     /SET TO IOT SKELETON IF KG8E IS PRESENT (E.G. 6110)

DDCFLD=	     /FIELD OF DDCMP TASK,LCBTAB AND 'NODE POOL'

/THIS FIELD MUST BE DIFFERENT FROM NSPFLD

DDCLOC= 0200    /ORIGIN OF DDCMP TASK

/THE ABOVE MUST BE BELOW 5000-SIZE OF NODE POOL AND

/     LCBTAB

/THE DEFAULT IS CURRENTLY 200

LCBSIZ= 32	     /GLOBAL DEFINITION OF LCB SIZE (DO NOT ALTER)

PKSIZE= 14      /GLOBAL DEFINITION OF PACKET SIZE (DO NOT ALTER)

DDCFNC= DDCLOC 	      /ADDRESS OF DDCMP 'FUNCTION CALL'

/     ROUTINE

HEADPK= DDCLOC+3020      /ADDRESS OF START OF PACKET FREELIST

LCBTAB= MAXPKTPKSIZE+HEADPK      /ADDRESS OF LINE CONTROL

/    BLOCK TABLE

/IMPORTANT NETWORKS PAGE 0 GLOBALS

DDCEF= 46	     /DDCMP I/O EVENT FLAG

FREHD= 47	     /LOCATION OF I/O PACKET FREELIST HEAD

DDCTL= 50	     /POINTS TO TAIL OF DDCMP INPUT QUEUE

DDCHD= 51	     /POINTS TO HEAD OF DDCMP INPUT QUEUE

ATNINP= 52	     /POINTS TO TRANSMIT COMPLETE RING BUFFER

0HDR= 55	     /LOCATION OF HEADER BUFFER FOR TRANSMITS

0CRCL= 63	     /HEADER CRC FOR TRANSMITS

0DCRCL= 65	     /DATA CRC FOR TRANSMITS

0UTCDF= 67	     /DATA DESCRIPTOR FOR TRANSMITS

DDCUSR= NSP     /DEFAULT USER OF DDCMP TASK

XLIST   1>

XLIST   1

IFDEF   NIP     <

XLIST   LSTFLG

/NIPFLD=		     /FIELD OF NIP

/NIPLOC=		     /LOCATION OF NIP

/NIPART=		     /PARTITION FOR NIP

/SKIMP=		     /SET TO 1 TO GET SHORT NIP

/NIPLOG=		     /DEVICE NIP OUTPUTS TO

/DEFAULT IS LPT IF IT EXISTS (OTHERWISE TTY)

/NIPRES=		     /LOCATION FOR RESIDENT PORTION OF NIP

/REQUIRED ONLY IF NIPART DEFINED

/DEFAULT IS NIPLOC-200

XLIST   1       >

IFDEF   TLK     <

XLIST   LSTFLG

/TLKFLD=		     /FIELD OF TLK TASK

/TLKLOC=		     /START OF TLK TASK

TLKCHN=	     /CCB CHANNL TO ASSIGN TO TLK TASK

XLIST   1	     >

IFDEF   LSN     <

XLIST   LSTFLG

/LSNFLD=		     /FIELD OF LSN TASK

5-22

RTS/8 DECNET/8 SYSTEM GENERATION

/LSNLOC=		     /START OF LSN TASK

LSNCHN=	     /CCB CHANNL TO ASSIGN TO LSN TASK

XLIST   1	     >

XLIST   LSTFLG

/EQUIVALENCES:

AC7776= CLL STA RAL

AC7775= CLL STA RTL

AC4000= CLA STL RAR

AC3777= CLL STA RAR

AC2000= CLA STL RTR

AC0002= CLA STL RTL

/MONITOR CALL VALUES:

CAL=    JMS     20	     /CALL THE EXECUTIVE

POSTDS= JMP I   24	     /DISMISS AN INTERRUPT

WAITM=  JMS I   25      /WAIT FOR MULTIPLE EVENTS

/NOTE:  "**" MEANS CRITICAL VALUE MAY NOT

/BE CHANGED WITHOUT MODIFYING SYSTEM CODE!!

SEND=   0	     /SEND MESSAGE

RECEIV= 1	     /RECEIVE MESSAGE

WAITE=  2	     /WAIT FOR EVENT FLAG

RUN=    3	     /CONTINUE TASK EXECUTION

SUSPND= 4	     /SUSPEND TASK EXECUTION

POST=   5	     /POST AN EVENT FLAG

SKPINS= 6	     /INSERT CODE INTO INTERRUPT SKIP CHAIN

DERAIL= 7	     /INITIATE END-ACTION

BLKARG= 10	     /BLOCK TASK FOR REASON SPECIFIED IN ARG

SENDW=  11	     /SEND MESSAGE AND WAIT

UNBARG= 12	     /UNBLOCK TASK FOR REASON SPECIFIED IN ARG

RESCHD= 13	     /FORCE A RESCHEDULE

WAITX=  14	     /WAIT FOR EXACTLY THIS EVENT FLAG

FREE=   4000    /**FREE PARTITION

XLIST   1

IFDEF UDC       <

XLIST   LSTFLG

AO=0;DO=1;DI=2;GC=3;EC=4;RC=5

DC=6;ECT=7;CS=10;DCT=ll;AI=12

XLIST   1>

XLIST   LSTFLG

/TASK STATUS FLAGS:

NONRWT= 4000    /**NONRESIDENT TASK WAIT

EFWT=   2000    /EVENT FLAG WAIT

RUNWT=  1000    /SCHEDULE WAIT

SWPWT=  0400    /**SWAPPER WAIT

EORMWT= 0200    /EVENT FLAG OR MESSAGE WAIT

USERWT= 0100    /USER SPECIFIED WAIT

ENABWT= 0040    /ENABLE WAIT

MSGWT=  0020    /MESSAGE WAIT

NETWT=  0010    /NETWORK WAIT  (RESERVED FOR POSSIBLE FUTURE USE)

DNEWT=  0001    /**DOES NOT EXIST WAIT

IFNZRO  KL8A    <IFNDEF KL8ACT  <

KLUD=   KL8A-1/3200

KL8ACT= 7400-KLUD>>

TSWFLG= 35	     /TASK SW INHIBIT FLAG IN FIELD 0

TODL=   36	     /LOW ORDER TIME OF DAY IN FIELD 0

TODH=   37	     /HIGH ORDER TIME OF DAY IN FIELD 0

5-23

RTS/8 DECNET/8 SYSTEM GENERATION

DATE=   40	     /DATE IN OS8 FORMAT IN FIELD 0

MCREF=  41	     /MCR START EVENT FLAG IN FIELD 0

/SOME USEFUL EQUATES FOR TTY AND LPT MESSAGES:

NOPACK=4000     /TEXT IS NOT PACKED IN 6-BIT

NOCRLF=2000     /OUTPUT SHOULD NOT BE FOLLOWED BY CR/LF

IND=1000	     /OUTTXT PTS TO FIRST WORD OF TEXT

NOLINE=400	     /INPUT IS IN CHARACTER MODE

ASSGN=200	     /ASSIGNS DEVICE

KL8ALINE=100    /USED TO SPECIFY A LINE OF A KL8A

XLIST   1

IFDEF   CLOCK   <

XLIST   LSTFLG

/	     SOME USEFUL EQUATES FOR STANDARD CLOCK MESSAGES:

MARKTIME=       0	     /POST EVENT FLAG AFTER SPECIFIED

/    INTERVAL

SCHEDULE=	     1000    /RUN TASK AFTER SPECIFIED INTERVAL

TIMOUT= 	     2000    /DERAIL TASK AFTER SPECIFIED INTERVAL

PERIODICALLY=   2000    /USED AS MODIFIER TO 'SCHEDULE'

/RE-QUEUES RUN REQUEST AFTER SPECIFIED

/    INTERVAL

/E.G. 'SCHEDULE FOO PERIODICALLY'

CANCEL=	7000 	     /DELETE ALL REQUESTS FROM SPECIFIED

/    TASK FROM QUEUE

XLIST   1>

XLIST   1	     /FORCE LISTING OFF

IFDEF   TASK    <

XLIST

/TASK TABLE SETUP - "TASK", "CUR","INIWT", AND "START"

/MUST BE DEFINED BY TASK:

IFNDEF  INIWT   <INIWT=0>

IFNDEF  INIWT2  <INIWT2=0>

IFNDEF  INIWT3  <INIWT3=0>

*TASK2+MSGTBL

ZBLOCK 2	     /MESSAGE BUFFER INITIALLY CLEAR

*TASK4+TSTABL

CUR%10+CUR	     /INITIAL FLAGS

START

0	     	     /INITIAL AC 0

XLIST;  IFDEF   VERS    <

XLIST

VERS	     /INITIAL MQ

XLIST   >

XLIST

*TASK+TFTABL

INIWT

XLIST

>

IFDEF   TASK2   <

XLIST

*TASK22+MSGTBL

ZBLOCK  2       /MESSAGE BUFFER INITIALLY CLEAR

*TASK24+TSTABL

CUR2%10+CUR2    /INITIAL FLAGS2

START2

0		     /INITIAL AC 0

5-24

RTS/8 DECNET/8 SYSTEM GENERATION

XLIST;  IFDEF   VERS2   <

XLIST

VERS2	     /INITIAL MQ

XLIST   >

XLIST

*TASK2+TFTABL

INIWT2

XLIST

>

IFDEF   TASK3   <

XLIST

*TASK32+MSGTBL

ZBLOCK  2	     /MESSAGE BUFFER INITIALLY CLEAR

*TASK34+TSTABL

CUR3%10+CUR3    /INITIAL FLAGS3

START3

0		     /INITIAL AC 0

XLIST;  IFDEF   VERS3   <

XLIST

VERS3	     /INITIAL MQ

XLIST   >

XLIST

*TASK3+TFTABL

INIWT3

XLIST

>

IFDEF   TASK    <

IFDEF   PARTNO  <

XLIST

/RESIDENCY TABLE (RESTBL) ENTRY:

/INITIALIZED FOR NONRESIDENT TASKS ONLY

*TASK-SWAPPER-12+RESTBL

PARTNO4+PARTBL+CPABLE+CPABLE+WRITE

XLIST

IFNDEF  SWAPPER <NOSWAP,ERROR>  /SWAPPER MISSING

IFNZRO  TASK-SWAPPER4000 <SWPRIO,ERROR>/NON-RESIDENT TASK

/HAS PRIORITY HIGHER THAN SWAPPER

>>

IFDEF   PARTNO  <

IFNDEF  TASK    <NOTASK ERROR> /PARTITION BUT NO TASK

IFNDEF  SWAPPER <NOSWAP ERROR> /PARTITION BUT NO SWAPPER

IFNDEF  PARTNS  <NOPART,ERROR> /MISSING PARTITIONS

IFZERO  PARTNO-PARTNS4000 <PRTERR,ERROR> /PARTNO.GE.PARTNS

>

XLIST   0

5.12  EXAMPLE OF A LINE CONTROL FILE

TLK     TLK     Sets the task number assigned to the TLK task.

TLK     TLKCHN  Sets the CCB channel number for the TLK task.

IFDEF DDCMP <XLIST>

/       SKELETON LINE CONTROL FILE

/ONE OF THESE FILES IS NEEDED FOR EACH PHYSICAL LINE

5-25

RTS/8 DECNET/8 SYSTEM GENERATION

/WHICH IS TO BE USED WITH DECNET/8. THIS FILE SELECTS DEVICE OPTIONS

/AS WELL AS THE CORE LOCATIONS TO LOAD THE DEVICE DRIVER AND ITS

/BUFFERS.  THE PARAMETERS IN THIS FILE ARE EDITED BY THE USER.

/(ALTERNATIVELY, IF ONLY ONE COPY OF THE INTERRUPT

/SERVICE ROUTINE IS NEEDED, THIS FILE CAN BE OMITTED, AND THE

/PARAMETERS HEREIN CAN BE EDITED DIRECTLY INTO THE

/INTERRUPT SERVICE ROUTINE SOURCE.)

/ASSEMBLY INSTRUCTIONS:

/       .PAL PARAM,LCF-NB,ISR

/THE ABOVE CCL COMMAND WILL CREATE A BINARY FILE, ISR.BN, WHERE

/"ISR" IS ANY ONE OF THE DECNET/8 INTERRUPT SERVICE ROUTINE SOURCE FILES

/IF MORE THAN ONE ISR IS BEING ASSEMBLED YOU SHOULD CREATE SEPARATE

/LINE CONTROL FILES AND SEPARATE BINARIES, SUCH AS "LCF0.PA"

/AND "ISR0.BN" FOR LINE 0.

/THE ISR BINARY FILE WILL CONTAIN ALL THE NECESSARY INFORMATION

/TO LOAD THE ISR AND SETUP THE ASSOCIATED DDCMP LINE CONTROL BLOCKS.

/THE FOLLOWING ISR SOURCES ARE PROVIDED:

/KL8ISR.PA	     FULL DUPLEX ASYNCHRONOUS SERVICE FOR KL8E

/	KL8JA,       KL8A OR KL8JA WITH KL8M MODEM CONTROL

/		     HALF DUPLEX IS ALSO PROVIDED FOR KL8M AND KL8A

/DP8ISR.PA	     SERVICE FOR DP8E FULL DUPLEX SYNCHRONOUS DEVICE

/DKCISR.PA	     SERVICE ROUTINE FOR DKC8A FULL DUPLEX PARALLEL INTERFACE

/LCLISR.PA	     SERVICE FOR LOCAL TURNAROUND "PSEUDO ISR"

/		     THE LOCAL ISR DOES NOT NEED A PHYSICAL DEVICE OF ANY KIND

/		     AND CAN BE A VALUABLE DEVELOPMENT TOOL

/	     THE FOLLOWING 7 PARAMETERS ARE MANDATORY FOR ALL ISRS

LINE= 0	     /PHYSICAL LINE NUMBER. LINE NUMBERS BEGIN AT 0

/THE LINE NUMBER SETS THE SKIPCHAIN PRIORITY

/OF THE ISR.

ADJNOD= 3	     /NODE NUMBER OF NODE PHYSICALLY ON OTHER END

/     OF LINE

/NODE NUMBERS BEGIN AT 2 (SEE DECNET/8

/     PROGRAMMING MANUAL)

ISRFLD= 20	     /FIELD (TIMES 10) OF INTERRUPT SERVICE ROUTINE

/     (E.G. 30)

ISRLOC= 200     /ADDRESS OF START OF ISR

BUFFLD= 20	     /FIELD (TIMES 10) OF ISR BUFFER (E.G. 40)

BUFLOC= 1200    /LOCATION OF START OF ISR BUFFER

BUFLEN= 400     /LENGTH OF ISR BUFFER IN WORDS (NORMALLY 2

/     PAGES IS SUFFICIENT)

/THE FOLLOWING ISR CORE SIZES APPLY (NOT INCLUDING RING BUFFER SPACE):

/KL8ISR 	     4 PAGES

/DP8ISR 	     8 PAGES

/LCLISR 	     2 PAGES

/DKCISR 	     4 PAGES

/THE FOLLOWING PARAMETERS ARE DEVICE DEPENDANT

/THE ISR SOURCE FILES SHOULD BE CONSULTED FOR EXACT USAGE IN EACH CASE

/	ISRDEV= 30	/DEVICE CODE USED BY ISR

/NORMALLY OF THE FORM XY (E.G. 36) FOR DEVICES

/OTHER THAN THE KL8A

/FOR THE KL8A THIS MUST BE SET TO 100+N WHERE

/N IS THE KL8A LINE NUMBER (E.G. 103)

/	ISRBRK=		/DATA BREAK LOCATION USED BY ISR (E.C. 7550)

/USED ONLY BY DP8E SERVICE ROUTINES

/CAUTION: THESE LOCATIONS USUALLY DESTROY

5-26

RTS/8 DECNET/8 SYSTEM GENERATION

/PART OF THE OS/8 RESIDENT HANDLER.

/THE DP8E ISR WILL CALL THE "EXIT" TASK IF

/THE EXIT TASK IS DEFINED TO SET ITSELF TO RESTORE

/THOSE LOCATIONS WHEN THE MCR "EXIT" COMMAND IS TYPED.

/SEE THE RTS/8 USERS MANUAL FOR FURTHER DETAILS.

/	KLMDEV=		/DEVICE CODE FOR KL8M MODEM CONTROL OPTION (E.G. 46)

/USED ONLY BY KL8JA SERVICE ROUTINES

/WITH MODEM CONTROL FEATURES

/	KL8AMD=		/SET TO 1 IF THIS IS A KL8A LINE 3 NEEDING

/KL8A MODEM CONTROL FEATURES

/THIS PARAMETER MUST BE LEFT BE UNDEFINED OTHERWISE

/TASK= DKC8A	/THIS PARAMETER MUST BE DEFINED IF

/THE DKC8A PARALLEL ISR IS BEING ASSEMBLED

/THE DKC8A IS THE ONLY ISR THAT NEEDS

/A SUPPORT TASK. DKC8A MUST ALSO BE DEFINED IN

/THE RTS/8 PARAMETER FILE.

/KL8HDX=		/THIS PARAMETER MUST BE SET TO 1 IF KL8M OR KL8A

/HALF DUPLEX FEATURES ARE DESIRED.

/YOU MUST NOT DEFINE OTHERWISE

5-27

CHAPTER 6

INTERNAL NETWORK DESCRIPTION

6.1  NETWORK OVERVIEW

This chapter describes the DECNET/8 network support code.  It contains

an  internal  description  of  the operation of the network protocols,

DDCMP and NSP.  It explains how these protocols communicate with  each

other,  the  user  tasks, and the physical line interfaces provided by

the Interrupt Service Routines.

6.1.1  Protocol Fields

The protocols are implemented by two RTS/8 tasks called NSP and DDCMP.

Both  tasks must be located in different memory fields as they are too

large to fit into the same field.  DDCMP and NSP share a  common  area

of  dynamic  storage called the Node Pool.  This is located just after

the main DDCMP code and requires a minimum of two pages of memory.

The Node Pool Space is arranged in packets of  14  octal  words  long.

Data  is  passed  in  these packets in 8-bit ASCII.  The layout of the

memory field containing DDCMP and the Node  Pool  space  is  shown  in

Figure 6-1.

*30  -----------------

|    Page 0	|

*77  -----------------

*200  -----------------  DDCLOC

|		|

|		|

|		|

|		|

|     DDCMP	|

|     code	|

|		|

|		|

|		|

*3200	|---------------|

|   Node Pool	|

*3400	|---------------|

|    LCBTAB	|

-----------------

Figure 6-1  DDCMP Field (DDCFLD)

The DDCMP task can be set up in any memory field except page 0.  DDCMP

uses  11  pages  of core and 50 locations in page 0.  The beginning of

DDCMP code (DDCLOC) can be set up anywhere between  location  200  and

6-1

INTERNAL NETWORK DESCRIPTION

4200.   The Node Pool Space occupies at least 2 pages of core directly

beneath the main DDCMP code.

The NSP task can also be set up in any memory field except page  0  or

the  DDCMP  field.   NSP  and its tasks occupy 18 pages of core and 64

locations at the end of page 0.  The starting location of NSP (NSPLOC)

can be set on a page boundary from 200 to 3200.  The layout of the NSP

field (NSPFLD) is shown in Figure 6-2.

*100  -----------------

|    Page 0	|

*177  -----------------

*3200  -----------------  NSPLOC

|		|

|		|

|		|

|		|

|      NSP	|

|      code	|

|		|

|		|

|		|

*7577	|---------------|

|  NSP TABLES	|

-----------------

Figure 6-2  NSP Field (NSPFLD)

The Interrupt  Service  Routines  can  be  located  anywhere  in  core

including page 0.  The lengths of the individual routines are given in

Section 5.6.

6.2  DDCMP

In DECNET/8, the DDCMP protocol is handled  by  a  single  RTS/8  task

called  DDCMP.   This  task is divided up into several source files to

facilitate a  flexible  system  generation.   It  is  composed  of  an

Executive task that includes a CRC-16 Calculation Routine and a set of

Interrupt Service Routines.

DDCMP is responsible for creating a physical link  between  two  nodes

and  transferring  data  between  these  nodes.   It  guarantees  that

messages sent or received over the physical link are error free  using

its  CRC-16  Routine.  In DECNET/8, each physical line requires a line

driver routine called an Interrupt Service Routine.  This line  driver

interfaces the DDCMP module with the hardware interface.

Although DECNET/8 does not initially support multidrop lines,  it  can

create  multiple logical links over each point-to-point physical link.

The DDCMP task never runs compute  bound  for  very  long  -  when  it

completes  one  stage of a task it pauses for another request.  During

these pauses, it can service requests from  other  physical  lines  by

6-2

INTERNAL NETWORK DESCRIPTION

keeping  a record of the state of each physical line in a Line Control

Block (LCB).  DDCMP maintains a table of  LCBs  in  the  DDCMP  memory

field  directly  beneath  the  Node Pool Space.  This gives DDCMP easy

access to information on the state of each line in the network.

6.2.1  Message Formats

Data is passed in the  network  via  messages.   When  DDCMP  receives

either  a channel control request or a transmit data request from NSP,

it "envelops" the request with system information.  This "envelope" is

called  a  header  and  the request is now in message format ready for

transmission  to  an  adjacent node within the network.  Transmit data

requests  are  numbered;   control  messages  are  unnumbered.    Data

messages  are  numbered  to  ensure  they are received in the order in

which they were transmitted.

The last two words of the header contain  a  cyclic  redundancy  check

(CRC)  on  the  characters  in  the  header.  This is computed using a

CRC-16  Calculation  Routine  (described  in  Section  6.2.10).    The

transmitting  node performs a CRC calculation on the header characters

and transmits the result of the calculation in the last two  words  of

the  header.   The receiving node performs the same check and compares

the two results.  This reduces the possibility  of  undetected  errors

occurring in the headers during transmission.

There are five control messages.  The first word of a control  message

is  <ENQ>,  which  distinguishes  it from a data message.  The control

messages are:

STRT	    Initializes  a  physical  link.   It   resets   message

counters  to zero and clears the system of all messages

remaining from the last session.  It ensures  that  the

new  session starts with no outstanding messages in the

system.

STACK	    Acknowledges that a STRT  message  has  been  received.

This  message  is  issued from the receiving node only.

It informs the transmitting node that it  is  ready  to

receive messages.

ACK	    Acknowledges the receipt of messages across a  physical

link.   It  is  also  used to acknowledge a STACK.  For

example, if  a  properly  sequenced  message  has  been

received  with a good CRC, the receiving DDCMP responds

with an ACK.

NAK	    Negative acknowledge.  Used for error reporting in  the

system.   For  example, if a message is received with a

bad CRC, DDCMP sends a  NAK.   The  type  of  error  is

described in the subtype word of the header (see Figure

6-3).  Refer to Section 4.4 for a  list  of  the  error

messages.

REP	    Forces a response to a transmitted message.  A  REP  is

6-3

INTERNAL NETWORK DESCRIPTION

used  if DDCMP suspects that data has been lost because

no response has been received to a message sent.

The formats of the control messages are given in Figure 6-3.

STAT/STACK	     ACK	       NAK		 REP

--------------    --------------    --------------    --------------

|    ENQ	   |    |    ENQ     |    |    ENQ     |    |    ENQ     |

|------------|    |------------|    |------------|    |------------|

|	STAT/STACK |	|    ACK     |	  |    NAK     |    |	 REP	 |

|------------|    |------------|    |------------|    |------------|

|	  FILLS	   |	|   FILLS    |	  |  SUBTYPE   |    |   FILLS	 |

|------------|    |------------|    |------------|    |------------|

|	  FILLS    |	|    RES     |	  |    RES     |    |	FILLS	 |

|------------|    |------------|    |------------|    |------------|

|	  FILLS    |	|   FILLS    |	  |   FILLS    |    |	 NUM	 |

|------------|    |------------|    |------------|    |------------|

|STATION ADDR|	|STATION ADDR|	  |STATION ADDR|    |STATION ADDR|

|------------|    |------------|    |------------|    |------------|

|    CRC	   |	|    CRC     |	  |    CRC     |    |	 CRC	 |

|------------|    |------------|    |------------|    |------------|

|    CRC	   |	|    CRC     |	  |    CRC     |    |	 CRC	 |

--------------    --------------    --------------    --------------

Figure 6-3  Control Message Formats

The RES word in the ACK and NAK messages is the  number  of  the  last

good message received.

The NUM word in the REP message is the  number  of  the  last  message

transmitted.   The  receiving  DDCMP  checks  this  number against the

number of the last good message it received and responds with  either

an ACK or NAK.

The STATION ADDR equals 1 if multidrop is not being  used  or  STATION

ADDR equals the station address if multidrop is being used.  Multidrop

is not available in this release so this word  is  always  set  to  1.

Figure  6-4  is  an example of the uses of control messages on a noisy

channel.

Data messages are transmitted in two parts, the header  and  then  the

data.   Thus,  the  header can be checked before the data is received.

If the header has an error, it is not necessary  to  allocate  buffers

for the data.  The format for data messages is given in Figure 6-5.

6-4

INTERNAL NETWORK DESCRIPTION

STATE							  STATE

----------------

STRT SENT     |    STRT      |--------->		       DOWN

----------------

----------------

<---------|     STRT	    |  STRT SENT

----------------

----------------

STACK CONFIRM |    STACK     |--------->

----------------

----------------

<---------|    ACK #0    |  ON LINE

----------------

----------------

ON LINE	   | Data Message |--------->

|    #1, 0     |

----------------

----------------

<---------|    ACK #1    |

----------------

----------------  (NOISE)

| Data Message |--------->

|    #2, 0     |

----------------

-----------------

<---------| NAK (Bad CRC) |

|       #1      |

-----------------

----------------

RETRANSMIT	   | Data Message |--------->

#2	   |    #2, 0     |

----------------

----------------

<---------| Data Message |  DATA MESSAGE

|    #1, 2     |  WITH AN

----------------  IMPLIED ACK

----------------

|   ACK #1     |--------->

----------------

----------------  (LOST)

| Data Message |--------->

|    #3, 1     |

----------------

Figure 6-4  Example of the Uses of Control Messages

6-5

INTERNAL NETWORK DESCRIPTION

.

TIME OUT		   .

.

----------------

RETRANSMIT	   |   REP #3     |--------->

#3	   ----------------

----------------

<---------|    NAK #2    |  REP RESPONSE

----------------

----------------

| Data Message |--------->

|    #3, 1     |

----------------

----------------

<---------|    ACK #3    |

----------------

Figure 6-4  (cont.) Example of the Uses of Control Messages

----------------

|      SOH     |

|--------------|

|  LOW COUNT   |

|--------------|

|  HIGH COUNT  |

|--------------|

|      RES     |

|--------------|

|      NUM     |

|--------------|

| STATION ADDR |

|--------------|

|     CRCL     |

|--------------|

|     CRCH     |

----------------

.

.

.

----------------

|	       |

|     DATA     |

|	       |

|--------------|

|     CRC      |

|--------------|

|     CRC      |

----------------

Figure 6-5  Format of Data Messages

6-6

INTERNAL NETWORK DESCRIPTION

SOH		Start of header identifier

LOW COUNT  \	These two words contain a count of the characters

HIGH COUNT /	in the data part of the message excluding the two

CRC words.

RES		Number of last good message received.  This saves

the overhead from sending an ACK.

NUM		Number of the message being transmitted.

STATION ADDR	= 1 if multidrop not being used.

= Station address if multidrop being used.

Figure 6-5 (Cont.)  Format of Data Messages

6.2.2  DDCMP Executive Routine

DDCMP is a state and  event  driven  task.  Its  main  routine  is  an

Executive  Listen  Routine  called DDCLP.  This routine "sleeps" using

the RTS/8 WAITM directive for events caused by  time-outs,  interrupts

and  RTS/8  messages.   When  DDCLP  detects an event, it wakes up and

determines which of its five major routines is required to handle  the

event and passes control to that routine.  These routines are:

DDCIN	    Processes messages received from a remote line.

DISPAT    Builds a message header (if required) and calls an  ISR

to  start  transmitting  the message.  DDCLP calls this

routine when a message is ready for transmission.

OUTCMP    Starts a timeout when a transmission is  complete.   It

then passes control to DISPAT.

GOTTCK    Scans  Line  Control   Blocks   looking   for   expired

time-outs.   DDCLP  calls  this routine when a time-out

tick has occurred.

GETR8M    Handles RTS/8 messages received from NSP on  the  local

node.

These routines return control  to  DDCMP's  idle  loop  (DDCLP)  where

another  WAIT  directive  begins.   DDCMP becomes unrunnable only at a

WAITM directive.  Figure 6-6 shows the overall organization  of  DDCMP

and its main routines.

6-7

INTERNAL NETWORK DESCRIPTION

DDCLP

------------------------

|  WAIT FOR AN EVENT   |

------------------| AND DISPATCH CONTROL |-------------------

|		   |  TO ONE OF THE FIVE  |		     |

|		---|    MAJOR ROUTINES    |----		     |

|		|  ------------------------   |		     |

|		|	       |	      |		     |

|		|	       |	      |		     |

|		|	       |	      |		     |

V		V	       V	      V		     V

------------   ------------   ------------   ------------   ------------

| PROCESS  |   | BUILDS A |   | STARTS A |   |  SCANS   |   | HANDLES  |

| MESSAGES |   | MESSAGE  |   | TIME-OUT |   |   LCBs   |   |   RTS/8  |

| RECEIVED |   |   AND    |   |  WHEN A  |   | LOOKING  |   | MESSAGES |

|  FROM A  |   |  STARTS  |<--|  TRANS-  |   |   FOR    |   | RECEIVED |

|  REMOTE  |   |  TRANS-  |   | MISSION  |   | EXPIRED  |   |   FROM   |

|   LINE   |   | MISSION  |   |    IS    |   | TIMEOUTS |   |   NSP    |

------------   ------------   | COMPLETE |   ------------   ------------

------------

---------------	      ---------------

|   SUPPORT   |	      |  INTERRUPT  |

| SUBROUTINES |	      |   SERVICE   |

|		|	      |  ROUTINES   |

---------------	      ---------------

.			     .

.			     .

.			     .

Figure 6-6  Overall Organization of DDCMP

6.2.3  Events

Events cause DDCMP to wake up and start running.  Four conditions  can

cause an event:

1.  An entry in the RTS/8 message queue caused by a user  request

from NSP;

2.  An entry in the receiver interrupt service queue caused by  a

message received from a remote DDCMP;

3.  An entry placed by an ISR in the transmitter  attention  ring

buffer   after  it  has  finished  transmitting  the  current

message;

4.  A DDCMP clock tick.

6-8

INTERNAL NETWORK DESCRIPTION

6.2.4  NSP to DDCMP Requests

NSP requests service from DDCMP by sending a function request to DDCMP

using  the  RTS/8  SEND  directive.   Figure 6-7 shows the format of a

function call from NSP to DDCMP.  When NSP wants to send a message  to

DDCMP it goes through the following steps:

1.  Gets a free packet from the Node Pool (14 octal words long);

2.  Fills in the offsets with the function code parameters;

3.  Issues a SEND (via RTS/8 SEND/RECEIVE mechanism) to DDCMP;

4.  Goes into a RECEIVE WAIT state.

When DDCMP has completed an NSP request, it sends the packet  back  to

NSP  rather than posting an event flag.  This avoids the complexity of

multiple event flag waits.

----------------------------------------------------------------------

| Packet |			  |					  |

| Offset |      Name	  |		 Usage			  |

|--------|-------------------|---------------------------------------|

|	 0    | SEND RETURN ADDR  | NSP    continuation    address   when |

|	      |			  | complete				  |

|	      |			  |					  |

|	 1    |	CALL RETURN ADDR  | NSP   continuation    address    when |

|	      |			  | complete				  |

|	      |			  |					  |

|	 2    |	EF		  | RTS/8 Event Flag (unused)		  |

|	      |			  |					  |

|	 3    |	CDF		  | RTS/8 Message Queue CDF word  (unused |

|	      |			  | by DDCMP or NSP)			  |

|	      |			  |					  |

|	 4    |	ADDR		  | RTS/8 Message Address Word.  Used  by |

|	      |			  | DDCMP  to queue messages and function |

|	      |			  | calls (as a thread word).		  |

|	      |			  |					  |

|	 5    |	STATUS/LINE	  | Return status and line  number.   The |

|	      |			  | low-order  byte  contains  the  DDCMP |

|	      |			  | line  number.   The  high-order  byte |

|	      |			  | contains the status completion code.  |

|	      |			  |					  |

|	 6    | FUNCTION	  | Function  Code.    There   are   four |

|	      |			  | possible  function  codes (only three |

|	      |			  | are used for this release).		  |

|	      |			  |					  |

|	 7    | CHANNL		  | CCB Number (if required)		  |

|	      |			  |					  |

|	10    |	P1		  | Parameter 1				  |

|	      |			  |					  |

----------------------------------------------------------------------

Figure 6-7  NSP to DDCMP Function Call Format

6-9

INTERNAL NETWORK DESCRIPTION

----------------------------------------------------------------------

| Packet |			  |					  |

| Offset |      Name	  |		 Usage			  |

|--------|-------------------|---------------------------------------|

|	11    |	P2		  | Parameter 2				  |

|	      |			  |					  |

|	12    |	P3		  | Parameter 3				  |

|	      |			  |					  |

|	13    |			  | Unused				  |

|	      |			  |					  |

|	14    |			  | Unused				  |

----------------------------------------------------------------------

Figure 6-7 (COnt.)  NSP to DDCMP Function Call Format

FUNCTION CODES:

Code	Request		    Parameters

0		TRANSMIT	    P1 = CDF to data

P2 = ADDR-1 of first byte

P3 = Negative byte count

CHANNL = 0 if not related to a user's

CCB

= CCB number if related to a CCB

1		STARTUP		    No parameters

CHANNL = 0 (unused)

2		HALT LINE	    Unused by NSP (A stand-alone use of

DDCMP would use this code.)

3		SEND MAINT MESSAGE  Reserved for a future release.

It causes DDCMP to send a

maintenance message.

When DDCMP receives a message from NSP, it places  the  function  call

packet  for  the message in a completion queue.  This queue stores the

message packet until DDCMP is ready to service the request.  DDCMP has

a completion queue for each physical line in the network.

The pointers to the queue are contained in the Line Control Block  for

that  line.  When the routine GETR8M scans the LCB, it finds a message

awaiting transmission in DDCMP's queue if the WFTHD word  in  the  LCB

contains  an address.  The queue is used for storing TRANSMIT, STARTUP

and SEND MAINT MESSAGE messages.  The HALT LINE message is not queued.

The layout of the queue is shown in Figure 6-8.

The WFPHD pointer is used only for TRANSMIT  messages.   This  pointer

contains the address of the message descriptor of the next message for

which an ACK is expected.  STARTUP and SEND MAINT MESSAGE  are  posted

as  soon  as  they  are transmitted, not when an ACK is received.  The

completion queue is used mainly for  storing  TRANSMIT  messages  that

contain pointers to the data ready for transmission by DDCMP.

6-10

INTERNAL NETWORK DESCRIPTION

QUETL ---------------------------------------------------

|

WFTHD -------------------				     |

|				     |

WFPHD --		     |				     |

|		     |				     |

|		     |				     |

|   ----------   |  ----------      ----------   |  ----------

|   |        |   |  |        |      |        |   |  |        |

|   |--------|   V  |--------|      |--------|   V  |--------|

--->|--------|----->|--------|----->|--------|----->|--------|

|        |      |        |      |        |      |        |

|        |      |        |      |        |      |        |

----------      ----------      ----------      ----------

ALREADY SENT      SECOND TO			  LATEST

AWAITING ACK       BE SENT			  ENTRY

WFPHD   Pointer to the ADDR word in the packet (offset 4) of the first

message in the queue.

WFTHD   Pointer to the ADDR word of the second message in the queue.

QUETL   The tail of the DDCMP completion queue.  The ADDR word in this

packet will contain zero to denote the end of the queue.

Figure 6-8  DDCMP Completion Queue

The queue can contain messages of one type only;  it  cannot  store  a

mixture  of messages.  If STARTUP is put in the queue when the line is

already up, DDCMP returns all  outstanding  messages  to  NSP  unsent.

DDCMP  accesses  packets  from  the  queue  by  using a routine called

REMOVE.  If the queue is empty, the LCB  pointers  WFPHD,  WFTHD,  and

QUETL contain zero.

6.2.5  Input

When DDCMP receives a  message  from  a  remote  node,  it  dispatches

control  to  its  DDCIN routine.  DDCIN determines if the message is a

data or control message.  If the message  is  a  data  message,  DDCIN

dispatches  control  to the subroutine DDCSOH.  This subroutine checks

the CRC-16 calculation, ensures that the message is in  sequence,  and

then passes the data to NSP using the ISR's ring buffer.

If the message received is a control  message  (preceded  by  an  ENQ)

DDCMP  passes  control  to an appropriate subroutine, depending on the

message.

Message Received	    Subroutine	     ________________	    __________

ACK		     DDCACK

NAK		     DDCNAK

6-11

INTERNAL NETWORK DESCRIPTION

REP		     DDCREP

STRT		     DDCSTRT

STACK	     DDCSTCK

Table 6-1 describes how these subroutines handle the control  messages

depending on the state of the line.

Table 6-1

Decision Table for Control Messages Received by DDCMP

----------------------------------------------------------------------

|\ STATE OF | DOWN      | STRT SENT  | STACK	  | LIVE	  |

| \  LINE	 |  00 	     |    0l	  | CONFIRM	  |  ll		  |

|	\	 |	     |		  |   10	  |		  |

|   ---	 |	     |            |		  |		  |

|      \	 |	     |            |		  |		  |

|MESSAGE\	 |	     |            |		  |		  |

|RECEIVED\	 |	     |            |		  |		  |

|(ROUTINE)\ |	     |            |		  |		  |

|-----------|-----------|------------|---------------|---------------|

| STACK	 | IGNORE    | REQUEST	  | REQUEST ACK	  | REQUEST	  |

| (DDCSTAK) |	     | STACK	  | POST STRT REQ | ACK		  |

|           |	     |            |               | 		  |

|		 |	     | -> STACK   | -> LIVE	  |		  |

|		 |	     |    CONFIRM |		  |		  |

|-----------|-----------|------------|---------------|---------------|

| STRT	 | NOTIFY    | SEND	  | IGNORE	  | NOTIFY NSP	  |

| (DDCSTRT) | NSP	     | STACK	  |		  | PURGE ALL	  |

|		 |	     |		  |		  | MESSAGES	  |

|		 |	     | -> STACK   |		  |		  |

|		 |	     |    CONFIRM |		  | -> DOWN	  |

|-----------|-----------|------------|---------------|---------------|

| ACK	 | IGNORE    | IGNORE	  | POST STRT	  | CALL "CRACK"  |

| (DDCACK)  |	     |		  | REQUEST	  | TO POST ACKed |

|		 |	     |		  |		  | MESSAGES	  |

|		 |	     |		  | -> LIVE	  |		  |

|-----------|-----------|------------|---------------|---------------|

| REP	 | IGNORE    | IGNORE	  | POST STRT	  | SEND NAK IF   |

| (DDCREP)  |	     |		  | REQUEST	  | HAVE MESSAGE  |

|		 |	     |		  |		  | NUMBER	  |

|		 |	     |	 	  | -> LIVE	  | SEND ACK IF	  |

|		 |	     |		  |		  | HAVE IT	  |

|-----------|-----------|------------|---------------|---------------|

| NAK	 | IGNORE    | IGNORE	  | IGNORE	  | CALL "CRACK"  |

| (DDCNAK)  |	     |		  |		  | BUMP THRESHOLD|

|		 |	     |		  |		  | COUNT	  |

|		 |	     |		  |		  | RETRANSMIT	  |

|		 |	     |		  |		  | ANY NAKed	  |

|		 |	     |		  |		  | MESSAGES	  |

----------------------------------------------------------------------

6-12

INTERNAL NETWORK DESCRIPTION

NOTE

"NEW STATE" signifies a change of  state

(represented  by  front  arrows).   Only

state   changes   caused   by   messages

received  are  shown.   If  there  is no

change of STATE,  then  "NEW  STATE"  is

omitted.    NSP  function  requests  and

threshold count overflows can also cause

changes of state.

6.2.6  Output

Output from DDCMP  is  either  transmission  of  a  message  across  a

communication  line  to  a  remote node or communication with NSP.  To

send a message to a remote node, DDCMP uses the DISPAT routine.   This

routine  checks  whether  the message is a control or data message and

builds a header accordingly.  It also does a  CRC-16  calculation  and

then   calls   an  ISR  to  start  transmitting  the  message.   After

transmission, control is returned to the DDCMP Executive Idle Loop.

----------------------------------------------------------------------

| Packet |		      |						  |

| Offset |	  Name	      | Usage					  |

|--------|---------------|-------------------------------------------|

|	 0    |	  EF   \      |						  |

|	      |		\     |						  |

|	 1    |	  CDF 	 >    | RTS/8 message header			  |

|	      |		/     |						  |

|	 2    |	  ADDR /      |						  |

|	      |		      |						  |

|	 3    |	  STATUS/LINE | The line number is contained in the	  |

|	      |		      | low-order byte (high-order byte is unused)|

|	      |		      |						  |

|	 4    |	  FUNCTION    |	Function code				  |

|	      |		      |						  |

|	 5    |	  CHANNL      |	Always = 0 (unused)			  |

|	      |		      |						  |

|	 6    |   P1   \      |						  |

|	      |		\     |						  |

|	 7    |	  P2	 >    | Parameter words				  |

|	      |		/     |						  |

|	10    |   P3   /      |						  |

|	      |		      |						  |

|	11    |		      | Unused					  |

|	12    |		      | Unused					  |

|	13    |		      | Unused					  |

|	14    |		      | Unused					  |

----------------------------------------------------------------------

Figure 6-9  DDCMP to NSP Function Call Format

6-13

INTERNAL NETWORK DESCRIPTION

DDCMP communicates with  NSP  using  function  calls.   If  DDCMP  has

completed a request from NSP, it sends the packet back to NSP with the

completion code set to the STATUS word.  It is NSP's responsibility to

return the packet to the Node Pool.

If DDCMP is calling NSP (e.g., message received), it gets a packet (14

octal  words)  from  the Node Pool and fills in the parameters.  DDCMP

sends the message to NSP using an RTS/8 SEND  Executive  request.   It

then  goes  into  a RECEIVE WAIT state.  This allows NSP to accept all

work requests with the RECEIVE directive, and  avoids  using  multiple

event flag waits.

The format for a DDCMP to NSP function call is shown in Figure 6-9.

FUNCTION CODES:

Code		  Request	     Parameters

4	      MESSAGE RECEIVED	     P1 = CDF to BUFFLD

P2 = ADDR - 1 of the buffer

P3 = Negative byte count

5	     START MESSAGE RECEIVED  P1 - P3 unused

NSP must issue a START request

to bring the line up.

DDCMP only lets NSP know  that

a STRT was received.

If the  line  is  already  up,

DDCMP  returns all outstanding

messages to NSP unsent;  these

messages  must  belong  to the

previous connection.

6	     LINE WENT DOWN	     A      threshold       counter

overflowed.   The  line  is no

longer on-line.

7	     MAINT MESSAGE RECEIVED  Reserved for a future release.

6.2.7  Message Timing

After transmitting a message, DDCMP sets a message timer in  the  Line

Control  Block.  This timer is a safeguard against losing messages and

is initiated in the  DDCMP  transmission  completion  routine  at  the

location  OUTCMP.   A  time  limit is set for a response to a message,

such as an ACK or NAK in normal message mode or  a  STACK  in  STARTUP

mode.   If  no  definite response is received in the time limit, a REP

message is issued to the receiving node.  In STARTUP mode, a  STRT  or

STACK  is present to prompt a response.  If no response is received to

these prompts, DDCMP assumes after a threshold count that the line has

gone down.

6-14

INTERNAL NETWORK DESCRIPTION

The routine GOTTCK is scheduled to run once per second.  This  routine

scans  the  Line  Control  Block  Table  (LCBTAB)  looking for expired

time-outs, incrementing the message timers in the LCBs,  and  checking

for overflows.

If an overflow occurs and the count of consecutive time-outs is  below

threshold, the subroutine SCHOUT is called to schedule transmission of

the appropriate message prompt.  The message prompts required for  the

different line states are:

Line State		   Message Required	  __________		   ________________

0 = DOWN		   No message

1 = STRT SENT		   STRT

2 = STACK CONFIRM	   STACK

3 = LIVE		   REP

When the threshold count is exceeded, the subroutine DOWN is called to

break  the physical link.  All unacknowledged messages are returned to

NSP.

When GOTTCK has finished scanning all the LCBs,  it  calls  the  clock

handler,  which  sets  a  new 1-second timer.  Control then returns to

DDCLP.

6.2.8  Interrupt Service Routines and Buffer Formats

The Interrupt Service Routines (ISRs) are supplied as separate  source

files   to   facilitate   a  flexible  system  generation.   ISRs  are

responsible for  first-level  handling  of  received  and  transmitted

character  strings.   They ensure that received messages are correctly

aligned before passing them to DDCMP and separate headers and  control

messages  from user data.  The ISRs communicate with DDCMP using three

basic facilities:

1.  The DDCMP event flag DDCEF and its associated queues;

2.  A shared data area called the ring buffer;

3.  The DDCMP input queue.

An ISR scans characters as they are received looking for the start  of

a  header or control message.  This happens at interrupt level as soon

as a MESSAGE START is detected.  The ISR obtains  a  packet  from  the

freelist  of  packets  in  the  Node  Pool  and  stores the next seven

characters in this packet.  It then posts DDCEF to schedule  DDCMP  to

wake  up and start running.  If the message was a control message, the

ISR searches for other messages.  If it was a  SOH,  it  collects  the

data portion of the message in the ring buffer and passes a pointer to

this data to DDCMP in a second packet.  Figure 6-10 shows  the  format

6-15

INTERNAL NETWORK DESCRIPTION

of  these  packets  and  Figure 6-11 shows the layout of DDCMP's input

queue for ISRs.

-----------------------       -----------------------

| POINTER TO NEXT PKT |   0   | POINTER TO NEXT PKT | =0 if last packet

|---------------------|	   |---------------------|    in queue

|	STATUS  |   LINE   |	   |  STATUS  |   LINE   |

|---------------------|	   |---------------------|

|	     SOH/ENQ	   |	   |         CDF	 | = Start of data

|---------------------|	   |---------------------|   in ring buffer

|		.	   | 	   |        ADDR-1	 | = Start of data

|---------------------|	   |---------------------|   in ring buffer

|		.	   | 	   |        -CNT	 | = Negative count

|---------------------|	   |---------------------|   of user data

|		.	   | 	   |         CRC	 | = CRC on data

|---------------------|	   |---------------------|

|		.	   | 	   |         CRC	 | = CRC on data

|---------------------|	   |---------------------|

|		.	   | 	   |         CDF	 | = CDF to ring

|---------------------|	   |---------------------|   buffer

|		.	   | 	   |        ADDR-1	 | = Last data byte

|---------------------|	   |---------------------|   passed

|	     SOH/ENQ	   |	   |        UNUSED	 |

|---------------------|	   |---------------------|

|       UNUSED	   |	   |	    UNUSED	 |

|---------------------|	   |---------------------|

|       UNUSED	   |  14   |	    UNUSED	 |

-----------------------	   -----------------------

Figure 6-10  ISR to DDCMP Packet Format

When transmitting, the ISR sends characters across the line until  the

entire  message  has been sent.  It then posts DDCMP to inform it that

transmission is complete and that the transmitter is now  idle.   ISRs

act  as an asynchronous process with DDCMP even though they operate at

interrupt level.

Each ISR in the system has an associated ring buffer for storing data.

This   buffer  stores  only  data  from  data  messages;   the  header

characters and control messages are passed  directly  to  DDCMP.   The

size, address, and length of the ring buffer for each ISR is specified

at system generation by setting the  parameters  BUFLEN,  BUFLOC,  and

BUFFLD  in  the Line Control File.  Figure 6-12 shows the principle of

the data ring buffer.

The parameter BUFLEN indicates the length of the entire buffer.   This

length  includes  the  two  words  reserved  for  the  CDF  and ADDR-1

instructions;  these  instructions  could  be  used  to  link  several

buffers  together  across  fields.   (This  is  not  a standard SYSGEN

option.)

6-16

INTERNAL NETWORK DESCRIPTION

-------------------

DDCHD ------>|		     |-----    DDCHD = Pointer to the first

|-----------------|	  |	       message in the queue

|        |	2    |	  |

|-----------------|	  |

|	  ENQ	     |	  |

|	  ACK	     |	  |

|	   .	     |	  |

|	   .	     |	  |

-------------------	  |

|

/

/

------------------- /

|		     |-----

|-----------------|	  |

|        |	3    |	  |

|-----------------|	  |

|	  SOH	     |	  |

|	   .	     |	  |

|	   .	     |	  |

|	   .	     |	  |

-------------------	  |

|

/

/

------------------- /

DDCTL ------>|		     |-----    DDCTL = Pointer to the most

|-----------------|		       recent entry

|        |	3    |

|-----------------|

|	  CDF	     |

|	 ADDR-1	     |

|	   .	     |

|	   .	     |

-------------------

Figure 6-11  Layout of DDCMP's Input Queue for ISRs

Each ISR has a set  of  pointers  for  putting  information  into  the

buffer.   DDCMP  checks  the  data in the buffer and passes it to NSP.

The words SVDBPT and SVDBCD  in  the  Line  Control  Block  contain  a

pointer  and  a CDF instruction to the end of the last segment of good

data passed to NSP.  This is a safeguard should an  error  occur  when

the  data is being received by NSP.  If this happens, the ISR pointers

are reset to the address in SVDBPT by the  subroutine  PURGE.   It  is

NSP's  responsibility  to  clear  the ring buffer when it has finished

with the data.

Positive words in  the  ring  buffer  are  assumed  to  contain  8-bit

characters.   A  word  of  the  form 62xl indicates a CDF instruction.

Negative words are assumed to be empty buffer locations.   NSP  clears

the buffer by placing -1 in each unused word.

6-17

INTERNAL NETWORK DESCRIPTION

-----------------

--->|		|-----> First word in the ring buffer.

|	|---------------|

|	|		|--

|	|---------------|  |

|	|		|   >   Data checked and sent to NSP.

|	|---------------|  |    Waiting to be cleared by NSP.

|	|		|--

|	|---------------| \

|	|		|  \___ SVDBPT \ Pair of words marking the

|	|---------------|       SVDBCD / end of the segment of the

|	|		|		 last good data sent to NSP.

|	|---------------|

|	|		|--

|	|---------------|  |

|	|		|   >   New data

|	|---------------|  |	being entered.

|	|		|--

|	|---------------|

|	|		|

|	|---------------|

|	|  CDF  (62X1)	|

|	|---------------|

----|     ADDR-1	|<----- Pointer to the first word of

-----------------       the buffer (=ADDR-1 because

the address was automatically

incremeneted).

Figure 6-12  Data Ring Buffer

6.2.9  Line Control Blocks

Each physical line control to a node must have a  Line  Control  Block

(LCB) associated with it.  The LCB is used by the system for a variety

of reasons including keeping a record of  the  current  state  of  the

line.  Each LCB requires 32 (octal) words.  DDCMP maintains a table of

LCBs called LCBTAB, which is kept in memory after the Node Pool Space.

The Node Pool Space and the LCBTAB must be in the same memory field as

DDCMP (see Figure 6-13).

MEMORY FIELD x

|		       |

|----------------|

|     DDCMP      |

|----------------|

|	NODE POOL AREA |

|----------------|

|     LCBTAB     |

|----------------|

|		       |

Figure 6-13  LCBTAB Location

6-18

INTERNAL NETWORK DESCRIPTION

The amount of space allocated for LCBTAB is calculated by  the  system

by  multiplying  the  amount of space required by each LCB fixed at 32

(octal) words by the MAXLIN parameter (refer to Table 5-3).

Size of LCBTAB = 32(octal) x value of MAXLIN

The format of a Line Control Block is described in Table 6-2.

Table 6-2

Line Control Block Format

----------------------------------------------------------------------

| Offset |  Name	| 		     Function			  |

|--------|---------|-------------------------------------------------|

|   0    |	SUCFLD	| Contains a CDF instruction to the ISR for  this |

|	      |		| line.  (Makes the ISR's field independent.)	  |

|	      |		|						  |

|	 1    |	ERRTAB	| The address of the error table (ERRTAB),  which |

|	      |		| contains  the  cumulative  error  counters  and |

|	      |		| status words for this line.  ERRTAB resides in  |

|	      |		| the ISR.					  |

|	      |		|						  |

|	 2    | RCVINI	| The entry address of the "Receive Initiator" in |

|	      |		| the   ISR.    This   subroutine   is   used  to |

|	      |		| resynchronize the receiver when a CRC error  is |

|	      |		|  detected.					  |

|	      |		|						  |

|	 3    |	TRNINI	| The entry address of the  "Transmit  Initiator" |

|	      |		| subroutine  in  the  ISR.  When this routine is |

|	      |		| called,  the  transmitter  is   turned-on   and |

|	      |		| characters are transmitted asynchronously until |

|	      |		| the end of the message.			  |

|	      |		|						  |

|	 4    |	STATE	| This  important   word   contains   the   flags |

|	      |		| indicating  the  state of the line.  The format |

|	      |		| is:						  |

|	      |		|						  |

|	      |		| 0			    11			  |

|	      |		|  -------------------------			  |

|	      |		|  | | | | | | | | | | | | |			  |

|	      |		|  -------------------------			  |

|	      |		|   | | \_____________/ \ /			  |

|	      |		|   | |        |         V			  |

|	      |		|   | |        V	2-bit state count	  |

|	      |		|   | |     Reserved				  |

|	      |		|   | |				     = 00 idle 	  |

|	      |		|   | -> Flag set if transmitter     = 00 STRT    |

|	      |		|   |	 is on (message being 	   	  send    |

|	      |		|   |	 sent)			     = 10 STACK   |

|	      |		|   |					  confirm |

|	      |		|   -> Flag set if a STRT was 	     = 11 Live	  |

|	      |		|      received; this avoids 			  |

|	      |		|      this avoids notifying NSP 		  |

|	      |		|      more than once of a start 		  |

|	      |		|      condition.				  |

----------------------------------------------------------------------

6-19

INTERNAL NETWORK DESCRIPTION

Table 6-2 (Cont.)

Line Control Block Format

----------------------------------------------------------------------

| Offset |  Name	| 		     Function			  |

|--------|---------|-------------------------------------------------|

|	 5    |	LSTACK	| The message number of  the  last  ACK  received |

|	      |		| from  the  other  end  of the line.  This is an |

|	      |		| 8-bit (mod 256) count and  is  saved  to  range |

|	      |		| check  received ACK, SOH, or NAK RES fields in  |

|	      |		| the subroutine CKACK.				  |

|	      |		|						  |

|	 6    |	LSTXMT	| An 8-bit counter containing the number  of  the |

|	      |		| last  message  sent (mod 256).  It is also used |

|	      |		| to range check the RES field in CKACK.	  |

|	      |		|						  |

| 	 7    |	LSTRCV	| The message number of  the  last  message  that |

|	      |		| successfully received an ACK.			  |

|	      |		|						  |

|	10    |	LSTNUM	| Temporary 8-bit count used to hold  the  number |

|	      |		| field  of  the  data  message  currently  being |

|	      |		| received.  This count is used  in  the  routine |

|	      |		| DDCSOH.					  |

|	      |		|						  |

|	11    |	LSTSNT	| An 8-bit message number for the last message  a |

|	      |		| transmit was started on.			  |

|	      |		|						  |

|	12    |	MTIMER	| A timeout counter set at the  beginning  of  an |

|	      |		| interval  and  cleared  when  the  timed  event |

|	      |		| occurs. This word is decremented by  the  timer |

|	      |		| scan  routine  GOTTCK.   When  equal to zero, a |

|	      |		| time-out has occurred.			  |

|	      |		|						  |

|	13    |	MTMODE	| A mode for MTIMER.				  |

|	      |		|						  |

|	14    |	CSCREP	| A temporary counter used to count the number of |

|	      |		| consecutive REPs, STRTs, and STACKs.		  |

|	      |		|						  |

|	15    |	WFPHD	| The address of the message  descriptor  of  the |

|	      |		| next  message  for  which  an  ACK  is expected |

|	      |		| (waiting for posting).			  |

|	      |		|						  |

|	16    |	WFTHD	| The address of the message  descriptor  of  the |

|	      |		| next message to be transmitted.		  |

|	      |		|						  |

|	17    |	QUETL	| The tail of the above  DDCMP  completion  queue |

|	      |		| (see Section 6.2.4).				  |

|	      |		|						  |

|	20    |	OUTBTS	| A  priority-ordered  set  of  flags   for   all |

|	      |		| messages.   When  transmission  of a message is |

|	      |		| requested, the  relevant  bit  is  set  by  the |

|	      |		| subroutine  SCHOUT.   When  the  routine DISPAT |

|	      |		| starts a new transmission,  it  first  searches |

|	      |		| OUTBTS  starting  at  the  LSB.   The first bit |

|	      |		| found set indicates  which  message  should  be |

|	      |		| built and sent:				  |

----------------------------------------------------------------------

6-20

INTERNAL NETWORK DESCRIPTION

Table 6-2 (Cont.)

Line Control Block Format

----------------------------------------------------------------------

| Offset |  Name	| 		     Function			  |

|--------|---------|-------------------------------------------------|

|	      |		| 0			    11			  |

|	      |		|  -------------------------			  |

|	      |		|  | | | | | | | | | | | | |  			  |

|	      |		|  -------------------------   (Highest priority) |

|	      |		|   \_______/ | | | | | | |			  |

|	      |		|       |     | | | | | | --> MAINT message	  |

|	      |		|	V     | | | | | |	    (reserved)	  |

|	      |		|   Reserved  | | | | | |			  |

|	      |		|	      | | | | | --> STACK		  |

|	      |		|	      | | | | --> STRT			  |

|	      |		|	      | | | --> NAK			  |

|	      |		|	      | | --> REP			  |

|	      |		|	      | --> Numbered message		  |

|	      |		|	      --> ACK				  |

|	      |		|	          (Lowest priority)		  |

|	      |		|						  |

|	      |		| For example, if  both  a  NAK  and  a  numbered |

|	      |		| message  are  queued  for transmission, the NAK |

|	      |		| will be sent first.				  |

|	      |		| 						  |

|	21    |	RNAK	| The NAK  subtype  if  a  NAK  was  flagged  for |

|	      |		| transmission  (by calling SCHOUT).  If multiple |

|	      |		| NAK requests are made, only the highest priority|

|	      |		| NAK will be sent.  There is no need to send more|

|	      |		| than one NAK in succession.			  |

|	      |		| 						  |

|	      |		| The following codes are used, sorted  according |

|	      |		| to priority:					  |

|	      |		| 						  |

|	      |		| 01 = Checksum error on header or control 	  |

|	      |		|      characters				  |

|	      |		| 02 = Checksum error on data			  |

|	      |		| 03 = Replying with a NAK to a REP message	  |

|	      |		| 10 = No buffer space in the ISR for message	  |

|	      |		| 11 = Receiver overrun (hardware error)	  |

|	      |		| 20 = Count field of message too big;  message   |

|	      |		|      discarded				  |

|	      |		| 21 = Header format error - could indicate a	  |

|	      |		|      software error				  |

|	      |		| 						  |

|	22    |	SVDBPT	| This pair of words contains  a  pointer  and  a |

|	23    |	SVDBCD	| CDF instruction to the data ring buffer for this|

|	      |		| line.  It marks the end of the last segment  of |

|	      |		| good data passed to NSP.  If an error occurs in |

|	      |		| the data being received, the ISR's pointers are |

|	      |		| reset to this point by the subroutine PURGE.	  |

----------------------------------------------------------------------

6-21

INTERNAL NETWORK DESCRIPTION

Table 6-2 (Cont.)

Line Control Block Format

----------------------------------------------------------------------

| Offset |  Name	| 		     Function			  |

|--------|---------|-------------------------------------------------|

|	24    |	PLISTN	| The return address of the listen routine, which |

|	      |		| must be saved because the DDCIN routine operates|

|	      |		| in coroutine fashion with the  DDCLP  Executive |

|	      |		| loop.						  |

|	      |		| 						  |

|	25    |	OUTCH1	| The first character of the header of the message|

|	      |		| currently being sent over the line.		  |

|	      |		| 						  |

|	26    |	OUTCH2	| The second  character  of  the  header  of  the |

|	      |		| message currently being transmitted.		  |

|	      |		| 						  |

|	27    |	OUTNUM	| The number of the field of the message currently|

|	      |		| being  transmitted.  This information about the |

|	      |		| current message is used to determine what to do |

|	      |		| next when the transmit is complete in OUTCMP.   |

|	      |		| 						  |

|	30    |	THRCT1	| Received threshold error count.		  |

|	      |		| 						  |

|	31    |	THRCT2	| Transmitted threshold error count.		  |

----------------------------------------------------------------------

6.2.10  CRC-16 Routine

DDCMP uses a Cyclic Redundancy Check (CRC) to detect errors in message

transmission.   This  check is a polynomial division of the data using

modulo 2 arithmetic.  The remainder after division is transmitted with

the  message  to  the  receiving node as Block Check Characters (BCC).

The receiving node performs  the  same  computation  on  the  received

message,  determines  the  remainder after division (BCC) and compares

its result with the  result  sent  by  the  transmitting  node.   This

procedure checks for errors occurring during the transmission.

The CRC-16 routine can be set at system generation  to  use  either  a

table-driven software algorithm or the KG8-E hardware.

CRC-16 is the polynomial  used  by  DDCMP.   DDCMP  performs  two  CRC

computations  at  each  node;  the first for the header characters and

the second for the data portion of each DDCMP  data-carrying  message.

The CRC-16 polynomial is:

16   15   2			15

x   +x   +x  +1 = (x+1)  *  (x   +x+1)

The polynomial is a template for the division;  the actual division is

by  11000000000000101.  The remainder of the division performed on the

header characters is a 16-bit header BCC;   the  remainder  after  the

division  of  the  data  portion  is  16-bit  data  BCC.   Each BCC is

transmitted as two 8-bit characters.  The CRC Block Check is explained

6-22

INTERNAL NETWORK DESCRIPTION

in  detail  in  Chapter  6  of  the PDP/8E, PDP/8M Maintenance Manual,					 _________________________________

Volume 3 (DEC-8E-HMM3A).

The advantages of the CRC check are good error detection at relatively

low  cost.  CRC can be rapidly computed using exclusive-OR techniques.

It can detect the following error types:

1.  Errors that change single bits;

2.  Errors that change an odd number of bits;

3.  Errors that change two bits;

4.  Most occurrences of multiple and burst errors.

6.2.11  Major DDCMP Support Subroutines

This section lists the major DDCMP support subroutines in alphabetical

order.

CKACK     Checks the RES  field  of  received  DDCMP  messages.   This

routine is called if an SOH, ACK or NAK is received.  If the

RES is in the range of outstanding  messages,  the  messages

are  posted complete.  Otherwise, a NAK-format error is sent

back over the line.

CRC16     Calculates  the  CRC-16  checksum  on  a  character   string

(explained in Section 6.2.10).  This routine takes advantage

of the KG8-E hardware CRC  generator  if  available.   DDCMP

uses most of its compute time calculating this routine.

DDCFNC    Allows NSP to send RTS/8 messages constructed  in  the  Node

Pool  to  DDCMP.   (Normally,  an  RTS/8  task cannot send a

message in a different field.)

DOWN      Brings a link off-line.  It returns  all  messages  awaiting

transmission or acknowledgment to NSP and clears the LCB.

FREEBF    Frees a packet passed by an ISR by returning it to the Node

Pool.

GETLCB    Sets up indirect pointers to the LCB for the line  currently

being  used  by  DDCMP.   This  routine is called when DDCMP

wants to service a different line.

INCERR    Increments threshold and  cumulative  error  counters  in  a

line's  LCB.   It is called when an error is detected either

in a received message or in a time-out condition.

POSTR8    Passes an RTS/8 message back to  NSP  using  an  RTS/8  SEND

directive.

PURGE     Clears and resets input buffers if CRC errors  are  detected

on  a  line's  receiver.   The  AC  on  entry contains a NAK

6-23

INTERNAL NETWORK DESCRIPTION

message subtype code if one is required.

REMOVE    Removes an NSP message from the waiting list and posts it by

sending  it  back to NSP.  On entry, the AC is a status code

(e.g., a successful SEND).

SCHOUT    Schedules the transmission of a message.  CALL + 1  contains

the  bit  flagging the type of message.  If the message is a

NAK, the AC contains  the  subtype.   The  DDCMP  idle  loop

detects  when  SCHOUT  is  called  and passes control to the

transmission routine DISPAT.  SCHOUT sets a bit in  the  LCB

word OUTBTS to queue the message.

SNDNSP    Sends an RTS/8 message to NSP to inform it of some event.

TMRSUM    When DDCLP detects a  timer  scan  in  progress,  it  passes

control  to  this routine.  A timer scan occurs when a clock

tick occurs (once per second) and the routine TMRCNT is  set

to nonzero.  When the scan is complete, TMRCNT is cleared.

6.3  NSP OVERVIEW

The Network Services Protocol is implemented by one task  called  NSP.

This  task provides the user interface to the network facilities.  NSP

is called the logical link  protocol  because  it  allows  two  tasks,

irrespective of their locations or whether they have a direct physical

link, to communicate with each other.

NSP is responsible for both the  creation  of  logical  links  between

tasks  wishing  to communicate and the control of data flow over these

links.  The DECNET/8 NSP task is a subset of the standard  DECNET  NSP

task.  The standard NSP task provides two levels of implementations:

o   Level 1, which assumes that  a  network  exists  between  the

communicating  NSP  modules  and that this network guarantees

the sequential delivery of messages.

o   Level 2, which makes no assumptions of the network.

DECNET/8  implements  the  Level  1  NSP,  which  is  most  suited  to

point-to-point connections.  The difference between the levels is that

Level 2 NSP must perform functions  such  as  message  acknowledgment,

time-outs,  and  message  retransmission  to  ensure message delivery.

These functions are omitted in the Level 1  NSP  task  for  efficiency

because they are provided by DDCMP (see Section 6.2).

NSP can receive requests for service from three sources:

1.  From the user task via an RTS/8 Executive request;

2.  From DDCMP via the RTS/8 Executive when it  has  completed  a

task;

3.  From DDCMP when it has received a  request  from  the  remote

DDCMP.

6-24

INTERNAL NETWORK DESCRIPTION

The NSP task has three main routines to service these different tasks.

A message is formatted using the appropriate routine by adding control

information and then the data is passed either to DDCMP or to the user

task.

6.3.1  NSP Message Format

Two types of messages are exchanged by NSP modules:  data and  control

messages.   Data  messages  carry  the  user  or  dialogue information

between processes and control messages carry information  between  NSP

modules.   For  each  physical link, a maximum block size (BLKSIZE) is

agreed on by the NSP modules at each end.  All nodes must  also  agree

on  the  maximum  size  of an NSP data message (NSPSIZE).  This is the

size of a single NSP message including the NSP header and  user  data.

The NSP header for data messages is less than 10 bytes in length.  The

default for NSPSIZE is 192.

NSPSIZE = 10 + User Data

The general message format for data being passed by NSP  to  DDCMP  is

shown in Figure 6-14.

------------------

|	    COUNT      |

|----------------|

|	    RTHDR      |

|----------------|

|	    MSGFLG     |

|----------------|

|	    MSGDATA    |

.		       .

.		       .

.		       .

Figure 6-14  General NSP Message Format

MSGDATA contains message-specific data and the length of this  section

of  the  header  varies according to the type of message.  (The entire

header never exceeds 10 bytes for data messages.)

COUNT:    The COUNT byte is optional.  If the  low-order  bit  of  the

first  byte  of  an  NSP message is set, the byte is a COUNT

byte.  (The low-order bits in RTHDR  and  MSGFLG  are  never

set.)

6-25

INTERNAL NETWORK DESCRIPTION

---------------------------------

|   |   |   |   |   |   |   | 1 |

---------------------------------

\_________________________/  |

|		      --> Set to distinguish byte

|			    from RTHDR or MSGFLG

|

--> Size of the following message in bytes

(not including the count byte itself)

The seven high-order bits of  the  COUNT  byte  contain  the

number  of  bytes  in  the  message.   The COUNT byte allows

several messages to be packed into a single DDCMP  envelope.

If  the  COUNT  byte is omitted, the remaining DDCMP data is

treated as one message.

RTHDR:    This optional byte is a route  header.   It  is  flagged  by

setting the two low-order bits to 1,0.

Routing  headers  are   used   in   large   networks   where

non-adjacent  nodes  want  to  communicate.  DECNET/8 has no

route-through  facilities  but  RTHDR  can  be  used  if  an

adjacent  node  can  provide  route-through facilities.  The

local node would be a valid route header if it  had  routing

capabilities.

If the RTHDR byte is omitted, the local and  adjacent  nodes

on  the  physical link are assumed.  If RTHDR is used it has

three fields.

RTFLG:    This optional byte  is  a  route  header.   It  is

flagged by setting the two low-order bits to 1,0.

---------------------------------

| 0 | 0 | 0 | 0 | P | P | 1 | 0 |

---------------------------------

\_____________/ \_____/ \_____/

V	    |	    |

unused 	    |	    |

|	    --> Set to distinguish byte

|		  from COUNT or MSGFLG

|

--> Message priority bits

00 = Low-priority data

01 = Normal data

10 = Interrupt message

11 = Control message

DESTNODE:  The destination node  address  contained  in  one

byte.  The values are:

0  = Local node (used for diagnostics)

1  = Adjacent node

2-127  = General remote nodes

6-26

INTERNAL NETWORK DESCRIPTION

SRCNODE:   The source  node  address  in  extensible  binary

contained  in  one byte.  The values are the same

as DESTNODE except this byte must not  contain  0

or 1.  It must contain an actual node address.

MSGFLG:   The message flag byte is always present  and  specifies  the

type of message.  It is flagged by setting the two low-order

bits to 00.

---------------------------------

| 0 | M | M | M | T | T | 0 | 0 |

---------------------------------

\_________/ \_____/ \_____/

|	    |	    |

|	    |	    --> Set to distinguish byte

|	    |		  from COUNT or RTHDR

|	    |

|	    --> Type bits:

|		  00= Data message

|		  01 = Single message

|		  10 = Link status

|		  11 = Control message

|

--> Modifier bits set according to the type

of message

Message specific data follows the MSGFLG byte.  The modifier

gives the subtype of the message.

Message Type     Modifier Bits		  Subtype     ____________     _____________		  _______

00		     4		Interrupt message

5		Unused

6		End-of-message segment

01		     4		Unused

5		Single-message reply requested

6		Single-message reply

10		     4		Unused

5		Unused

6		Unused

11		     0		Unused

1		Connect

2		Disconnect

3		Startup message

4-7	Unused

6.3.1.1  Extensible Binary and Extensible ASCII  -  Extensible  binary

and  extensible  ASCII  fields  are  used  when passing message header

information within the NSP task.  In these fields, the high-order  bit

of  each  byte in the data stream is set if another data byte follows;

the high-order bit is clear in the last byte.

Extensible binary is packed with the low-order seven bits of  data  in

the first byte, the next seven bits in the next byte and so on for the

entire message.  The high-order bit of  each  byte  is  set  to  1  if

6-27

INTERNAL NETWORK DESCRIPTION

another  data  byte follows, or to 0 if it is the last byte.  The data

is right-justified in a binary register.

EXAMPLE:		     1371 in Extensible Binary is:

-->	 Last data byte	   -->	Indicates another

|			   |	data byte follows

|			   |

---	 000   001   01    ---	11   111   001

\______________/       \______________/

|    \_____________/   |	    |

|	   |	      |	    |

V	   V	      V     V

1	   3	      7     1   = 1371

The low-order numbers, which are the most significant, are read first.

Extensible 7-bit  ASCII  is  packed  into the seven high-order bits of

each  byte.  It is left-justified with the left byte transmitted first.

The  high-order  bit  indicates  the  length  of the data stream as in

extensible binary.

EXAMPLE:

TLK in Extensible ASCII is:

ASCII		   Extensible ASCII	        _____	           ________________

T   =    124		   11 010 100  =  224

L   =    114		   11 001 100  =  214

K   =    113		   01 001 011  =  113

6.3.2  Overall Layout of the NSP Task

NSP acts as an intermediate processor between user tasks and the  line

protocol  (DDCMP).   The  overall  layout  of the NSP task is shown in

Figure 6-15.  The task is  basically  tree-structured  with  an  RTS/8

RECEIVE directive at the apex.

The RTS/8 Executive maintains a queue of  intertask  messages.   Since

the  NSP  task is non-reentrant, it must be queued sequentially by the

user and DDCMP tasks.  The messages in the queue are acquired in order

of  priority  using  the  RTS/8  RECEIVE directive.  DDCMP is serviced

before any of the user tasks in the queue because it has been assigned

a  higher  priority.   This  is  done  using the SEND/RECEIVE priority

queueing in RTS/8.

NSP divides tasks into three  groups  and  has  separate  routines  to

handle each group of tasks.  The groups are:

1.  DDCMP functions that have been completed;

6-28

INTERNAL NETWORK DESCRIPTION

********

*          *

*    LOOP    *---------------------

*          *		       |

********		       |

V

-----------------

| ISSUE RECEIVE |

|   DIRECTIVE   |

-----------------

|

|

V

--------------------------

|  DISPATCH TO ROUTINE   |

| ACCORDING TO FUNCTION  |

| WORD IN  RTS/8 MESSAGE |

--------------------------

|	       |	 |

---------	       |	 ----------

V    	       V 	          V

-------------------      -------------------      -------------------

| COMPLETED DDCMP |      |	  DDCMP EVENTS	|      |  USER FUNCTION  |				  ____________

|    FUNCTIONS    |      |			|      |      CALLS	 |	  _________					      _____

|		       |      |     MESSAGE 	|      |		 |

|	  TRANSMITS    |      |	   RECEIVED **	|      |     CONINI	 |

|		       |      |			|      |     CONCNF	 |

|	   STARTUP     |      |  START MESSAGE 	|      |     CONREJ	 |

|		       |      |	   RECEIVED	|      |     DISCON	 |

|     HALT *      |      |			|      |     TRNSMT	 |

|		       |      |   LINE FAILED   |      |     INTRPT	 |

|     MAINT *     |      |			|      |     RCVMSG	 |

-------------------      |   MAINTENANCE 	|      |     RCNMSG	 |

|	    MESSAGE	|      |     RCVSM	 |

|	  RECEIVED *	|      |     TRNSM	 |

-------------------      -------------------

-----------------------------

| MISCELLANEOUS SUBROUTINES |

-----------------------------

.

.

----------

| TABLES |

----------

.

.

* = Unused - Reserved for future release

** = Breaks down further according to

type of message received

Figure 6-15  Overall Layout of NSP

6-29

INTERNAL NETWORK DESCRIPTION

2.  DDCMP events;  and

3.  User function calls.

When a task has been completed, NSP enters at LOOP  again.   If  there

are  no  messages  in  the  queue,  NSP  is  suspended  by the RECEIVE

directive.

6.3.3  DDCMP Function Call Completion Processing

When DDCMP completes  an  NSP  task,  it  notifies  NSP  of  the  task

completion.   Four  DDCMP  functions  notify  NSP on their completion.

They are:

o   TRANSMIT

o   STARTUP

o   HALT (Reserved for future release)

o   MAINT (Reserved for future release)

The interface between NSP and DDCMP is  described  in  Section  6.2.4.

NSP  requests  to  DDCMP  were described and how NSP waits for posting

from DDCMP after task completion.  The following sections describe the

user interface to NSP.  Rather than describe TRANSMIT and STARTUP now,

it is more appropriate to describe the use of these functions as  part

of the user function calls in Section 6.3.5.

6.3.4  DDCMP Events

DDCMP events occur asynchronously and include:

o   MESSAGE RECEIVED

o   STRT RECEIVED

o   LINE FAILED

o   MAINT MESSAGE RECEIVED (Reserved for future release)

MESSAGE RECEIVED is subdivided into the type of NSP message  received.

The  message  type  is  determined  by  parsing the message when DDCMP

receives it.  When NSP receives a message, it determines the  type  of

message  and dispatches control to a routine to handle the processing.

The type of message that can  be  accepted  is  explained  in  Section

6.3.1.  Handling individual messages is described in Section 6.3.5.

When a STRT RECEIVED is accepted, NSP responds by sending a request to

DDCMP to bring the link on-line.

LINE FAILED causes NSP to search the table  of  active  logical  links

(LNKTAB).   NSP  tables  are  described in Section 6.3.7.  LINE FAILED

6-30

INTERNAL NETWORK DESCRIPTION

breaks any logical link connections using the physical line for  which

the LINE FAILED was received.

6.3.5  User Function Calls

All communication from user  tasks  is  passed  directly  to  NSP  for

processing.  The user issues function calls (described in Section 2.2)

and NSP has a routine to handle these requests.  Each function call is

handled  differently  and  is  described individually in the following

sections.

When NSP is communicating back to the user task it posts  event  flags

and  schedules  the  user's AST routine.  The function calls and their

formats are described in this section.

CONNECT

CONNECT processing involves three user functions:   CONNECT  INITIATE,

CONNECT  CONFIRM, and CONNECT REJECT.  The connect process begins when

a user issues a CONINI request.   NSP  first  locates  the  node  name

specified  in the request, which sets up the physical link with DDCMP,

and then checks that the physical link is on-line.

A CONINI message is built using data from the CONINI call and data  in

the  CCB.   The  CONINI function call is held in location CCBCON.  The

CHNCIS bit is set in the CCB state word and the CONINI is transmitted.

CONNECT processing is suspended until a CONNECT CONFIRM (CC),  CONNECT

REJECT  (CR),  or DISCONNECT REJECT (DR) message is received.  If a CR

or DR is received, the user CONINI is posted unsuccessful.  If a CC is

received,  the  CONINI  is  posted  successful  and a null LINK STATUS

message is sent to confirm the link;  the connection is then on-line.

The header on the CONNECT message is the general message  format;   it

is  followed  by  the  message  specific  data MSGDATA.  The format of

MSGDATA for CONINI, CONCNF and CONREJ is:

6-31

INTERNAL NETWORK DESCRIPTION

-------------

|  CONFLGS  |

|-----------|

|  DESTADDR |

|-----------|

|  SRCADDR  |

|-----------|

|  LNKCHR   |

|-----------|

|  SEGSIZE  |

|-----------|

|  LNKACT   |

|-----------|

.  DESTNAME .

.-----------.

.  SRCNAME  .

.-----------.

.  OPTDATA  .

.-----------.

CONFLGS   :    The connect message flags.

---------------------------------

| 0 | 0 | 0 | L | L | C | C | C |

---------------------------------

\_________/ \_____/ \_________/

|	      |	        |

|	      |	        --> Connect Type:

|	      |		      000 = CONINI

|	      |		      001 = CONCNF

|	      |

|	      --> Fixed Link Priority = 01

|

--> Unused

DESTADDR  :    The destination logical link address contained  in  two

bytes.   For  CONINI,  this  is  set  to 0 to allow the

receiving NSP to assign an  address  dynamically.   For

CONCNF,  this  is  reserved for SRCADDR.  The values of

DESTADDR and SRCADDR define the logical link.

SRCADDR   :    The source logical link address contained in two bytes.

This  address  is  assigned  by the sending node and is

used by the destination node to  address  all  messages

for this link.  The value 0 is illegal.

LNKCHR     :    The logical link characteristics contained in one byte.

This  field defines the operational characteristics and

logical link options chosen by the user.

6-32

INTERNAL NETWORK DESCRIPTION

---------------------------------

| 0 | 0 | N | D | D | F | F | F |

---------------------------------

\_____/  |  \_____/ \_________/

|	|     |	     |

|	|     |	     --> Flow Control Options:

|	|     |	           000 = No flow control

|	|     |	           001 = Segment flow control

|	|     |	           010 = Message flow control

|	|     |	           011 = Segment and message

|	|     |	        	 flow control

|	|     |

|	|     --> Delivery Options:

|	|     	    00 = No delivery

|	|     	    01 = Guaranteed delivery

|	|

|	--> Network Characteristic Bit:

|	      0 = Level 2 NSP

|	      1 = Level 1 NSP

|

--> Unused

DECNET/8 assumes one segment per message.   It  assumes

message  flow  control  and  guaranteed  delivery.  The

network characteristic  bit  is  always  set  to  1  to

indicate a level 1 implementation of NSP.

SEGSIZE   :    The maximum size  of  the  user  data  message  segment

contained in two bytes.

LNKACT    :    The account field  for  charging  link  storage  in  an

image-mode  field.   This  field  consists of the COUNT

byte followed by the zero or more bytes containing  the

account   information.   This  field  is  not  used  by

DECNET/8.

DESTNAME  :    The name of the object on the destination node to which

the  connection  is  required  (used  for CONINI only).

DESTNAME has six subfields and has one of the following

formats:

1.  OBJTYPE

2.  OBJTYPE, FORMAT, and DSCRIP

OBJTYPE : The object type being referenced (1 byte long).

This number is an encoding of commonly referenced

objects on the network, e.g., processes, file

systems, or resource allocators.  This allows

systems to assign local names to objects in the

network and reference them by using these names.

For this release, only 0 is allowed, which

indicates a task.

6-33

INTERNAL NETWORK DESCRIPTION

---------------------------------

|   |   |   |   |   |   |   |   |

---------------------------------

|  \_________________________/

|	             |

|		     --> Object Type Number:

|			   0 = default for task

|

--> 0 = 0BJTYPE only

1 = FORMAT and DSCRIP follow

FORMAT :  The format of the object descriptor field (1  byte

long).  Present only if bit 7 of OBJTYPE is set.

---------------------------------

| 0 | 0 | 0 | F | F | F | F | F |

---------------------------------

\_________/ \_________________/

|		 |

|		 --> Format number:

|		       0 = GROUP(2), USER(2),

|			   NAME(O- 12)

|		       1 = NAME(1 - 16)

|

--> Unused

DECNET/8 uses the  GROUP(2),  USER(2),  NAME(0-12)

format, which has a minimum length of four bytes.

DSCRIP :  The object descriptor field.  The  format  depends

on  the value of FORMAT.  DSCRIP has the subfields

GROUP, USER, and NAME:

GROUP :   The  group  code  of   the   object

contained   in   two  bytes.   This

subfield and the USER subfield  are

not  used  by  DECNET/8.   They are

provided  for  compatibility   with

other systems using the network.

USER :    The  user  code   of   the   object

contained  in  two bytes (also used

for compatibility).

NAME :  	The name of the object in 12 bytes.

If 0BJTYPE = 0, this field contains

the   process   or    task    name.

Otherwise,  it  contains additional

information specific to each object

type.

SRCNAME   :    The name of the source requesting the  connection;   it

must  be in the same field as DESTNAME (used for CONINI

6-34

INTERNAL NETWORK DESCRIPTION

only).  SRCNAME also has the subfields OBJTYPE, FORMAT,

DSCRIP, GROUP, USER and NAME as described for DESTNAME.

OPTDATA   :    Any optional data the  source  wants  to  pass  to  the

destination.  The length of OPTDATA is restricted to 16

bytes.  The length of this field is determined  by  the

COUNT  value  (if  used)  or  the  message length.  The

length can have a value of zero.  It is the last  field

in the message.

DISCONNECT

A logical link can be disconnected as a result of  the  failure  of  a

physical  line or it can be requested by the task at either end of the

link.  When a task issues a DISCON function call, the  task's  CCB  is

marked as pending a disconnect and a disconnect message is sent to the

remote task.  The remote NSP interrupts its task using an AST  routine

(to  inform it of the disconnect) and confirms the disconnect initiate

with a disconnect confirm message.  The connection at the  remote  NSP

is  immediately  broken;   that  is,  its logical link address is made

available and the task can issue a new connect request or be connected

to.   When  the disconnect confirm is received at the originating NSP,

the task's DISCON is posted and the task is completely disconnected.

If a physical link fails, NSP searches the link table and  disconnects

any connection using that physical link.  A special AST reason code is

passed  to  the  task   to   explain   the   disconnection.    Unusual

disconnections, such as both tasks sending a DISCON simultaneously, or

one task disconnecting during a connection setup,  are  not  described

here.  These cases can, however, be handled by the NSP protocol.

NOTES

1.  Normal disconnection does not affect

single message transfers.

2.  Any unposted receives or transmits

are posted as incomplete during a

disconnect.

The form of MSGDATA for DISCON is:

-------------

|  DISFLGS  |

|-----------|

|  DESTADDR |

|-----------|

|  SRCADDR  |

|-----------|

|  OPTDATA  |

-------------

6-35

INTERNAL NETWORK DESCRIPTION

DISFLGS:	    The disconnect message flags.

---------------------------------

|   |   |   |   |   | D | D | D |

---------------------------------

\_________________/ \_________/

|     	    |

|     	    --> Disconnect Types:

|     	          000 = Disconnect initiate

|			or connect reject

|     	        	by user

|     	          001 = Disconnect initiate

|			by NSP

|     	          010 = Disconnect confirm

|     	          011 = Disconnect reject

|     	        	by NSP

|

--> Unused

DESTADDR  :    The destination logical link address contained  in  two

bytes.

SRCADDR   :    The source logical link address contained in two bytes.

If  this is a disconnect reject, this address is set to

0 because no address has been assigned.

DISDATA   :    Additional   disconnection   information.     If    the

disconnect  type  bits  are  set to 000 (user-generated

disconnect request), this field contains OPTDATA.   For

non-user-generated  disconnects, this field contains an

explanation of the disconnection (REASON).

OPTDATA : Any optional data the source wants to pass to

the  destination.   The  length of OPTDATA is

restricted to 16 bytes.  The length  of  this

field  is  determined  by the COUNT value (if

used) or the message length.

REASON :  The disconnect reason code (1 byte long).

Reason Code		    Reason		      ___________		    ______

1	   NSP resources error or lack of resources

(DI, DC)

2	   Node does not exist -  intercept  return

(DC)

3	   Node shutting down (DI, DC)

4	   Object unsupported  or  does  not  exist

(DC)

5	   Invalid format for object  specification

(source or destination ; DC)

6	   Mailbox queue full for  queuing  message

to object (DC)

6-36

INTERNAL NETWORK DESCRIPTION

32	   Too many connects to node (DC)

33	   Too many connects to object (DC)

34	   Access barred  (invalid  or  non-allowed

node or source object ; DC)

35	   Logical  link  characteristics  mismatch

(LNKCHR ; DI, DC)

36	   Invalid account information (DI, DC)

37	   Segment size too small (SEGSIZE; DI, DC)

38	   User abort/time-out/cancels link (DI)

39	   Unable to communicate with node (no path

- intercept ; DI)

40	   No buffer for received message on L - no

flow control (DI)

41	   Message  received  for  IDLE  state  (no

current logical link ; DC)

42	   Confirmation of disconnect initiate (DC)

TRNSMT

In NSP, data is not sent to a remote task unless the task has assigned

a  buffer  to receive the message.  NSP maintains a "request count" in

each CCB to keep a record of the number  of  buffers  available.   The

LINK STATUS message is used to declare new buffers becoming available.

When a TRNSMT function call is issued, NSP checks the current  request

count  and  if  it is nonzero, it decrements it and transmits the data

message.  If the request count is zero, NSP holds the  TRNSMT  request

in a queue.  A data message header is attached to the beginning of the

user data.  The type bits in  the  MSGFLG  field  are  set  to  00  to

indicate  a  data  message.  The modifier bit in MSGFLG is clear.  The

format for MSGDATA for a data message is:

--------------------

|     DESTADDR     |

|------------------|

|     SRCADDR	 |

|------------------|

|      		 |

|       USER     	 |

|       DATA	 |

| 		 |

DESTADDR  :    The logical link destination address contained  in  two

bytes.   This  is the address for a conversation and is

dynamically assigned by the CONINI message.

SRCADDR   :    The source logical link address contained in two bytes.

USERDATA  :    The data the user wants to send  or  receive  over  the

logical  link.   This  field is transparent and can use

all eight bits of each octet.

6-37

INTERNAL NETWORK DESCRIPTION

INTRPT

The flow control scheme used in the TRNSMT function call  is  used  to

relieve  NSP  of the responsibility of maintaining a large buffer pool

for storing data for tasks that have not issued receive requests.  The

INTRPT  request  bypasses  this  control scheme by forwarding the data

directly to the remote task using the AST  routine.   This  interrupts

the  remote  task  to  inform it of an urgent condition.  The USERDATA

field is limited to eight bytes.

INTRPT messages are sent in a data message envelope.  The type bits in

the  MSGFLG  field are set to 00 and the modifier bit is set.  For the

most part, the header is exactly the same as  for  TRNSMT;   the  only

difference is the limitation on the USERDATA field.

RCVMSG

When a task issues a RCVMSG function call, NSP  sends  a  LINK  STATUS

message  to  the  remote  NSP.  The request count at the remote NSP is

incremented and any pending data messages are returned.  This  message

is used for flow control and to solicit a RECEIVE message.  The format

for MSGDATA for RCVMSG is:

--------------------

|     DESTADDR     |

|------------------|

|     SRCADDR	 |

|------------------|

|     RSEGCNT	 |

|------------------|

|     RMSGCNT	 |

--------------------

DESTADDR  :    The destination logical link address contained  in  two

bytes.

SRCADDR   :    The source logical link address contained in two bytes.

RSEGCNT   :    The number of message segments requested by the source.

This  number  usually  corresponds  to  the  number  of

receives done by the user.  Since logical links can  be

made  in  two  directions,  each  end of the link has a

request count.  Messages are not  sent  across  logical

links   until  they  are  requested.   This  number  is

incremented each time a request is made for a message.

RMSGCNT   :    The number of messages requested by the  source.   This

number  is  equal  to  RSEGCNT  because  in  DECNET/8 a

message is equal to a segment.

RCNMSG

The RCNMSG function call is used to declare  several  buffers  with  a

single  LINK  STATUS  message.   The request is put in a queue and the

6-38

INTERNAL NETWORK DESCRIPTION

local request count is  incremented.   When  a  subsequent  RCVMSG  is

issued, this cumulative count is sent in the LINK STATUS message.  All

counts are cleared during the connection procedure.

RCVSM

The RCVSM request does not issue any  message  (unlike  RCVMSG,  which

issues  a  LINK  STATUS  message).   DECNET/8  does  not guarantee the

delivery of single messages.  The advantage of issuing these  messages

is  that  the  amount  of  nondata traffic on the line is reduced.  No

logical link connection messages or NSP status messages are sent.

When a task issues a RCVSM  request,  it  makes  itself  available  to

receive  single  messages.  It goes into the WAIT state in preparation

for receiving a TRNSM.

TRNSM

The format of MSGDATA for TRNSM is:

--------------------

|      SMFLGS	 |

|------------------|

|     DESTNAME     |

|------------------|

|      SRCNAME     |

|------------------|

|			 |

|       USER	 |

|       DATA	 |

|			 |

SMFLGS    :    Single-message flag (1 byte long).

---------------------------------

| 0 | 0 | 0 | R | R | R | R | R |

---------------------------------

\_________/ \_________________/

|     	         |

|     	         --> Reply reason

|

--> Unused

Reason Code			  Reason		    ___________			  ______

0	 Message delivered and put on queue

1	 NSP resources error or lack of resources

2	 Node does not exist;  intercept return

3	 Node shutting down

4	 Object unsupported or does not exist

5	 Invalid format  for  object  specification

6-39

INTERNAL NETWORK DESCRIPTION

(source or destination)

6	 Mailbox queue for messages being  sent  to

object is full

7	 Message too long

DESTNAME  :    The name of the object on the destination node to which

the  connection  is  required  (used  for CONINI only).

DESTNAME has six subfields and has one of the following

formats:

1. OBJTYPE

2. OBJTYPE, FORMAT, and DSCRIP

OBJTYPE : The object type being referenced  (1  byte  long).

This  number is an encoding of commonly referenced

objects on  the  network,  e.g.,  processes,  file

systems,  or  resource  allocators.   This  allows

systems to assign local names to  objects  in  the

network  and  reference them by using these names.

For  this  release,  only  0  is  allowed,   which

indicates a task.

---------------------------------

|   |   |   |   |   |   |   |   |

---------------------------------

|  \_________________________/

|     	     |

|     	     --> Object Type Number:

|			   0 = default for task

|

--> 0 = OBJTYPE only

1 = FORMAT and DSCRIP follow

FORMAT :  The format of the object descriptor field (1  byte

long).  Present only if bit 7 of OBJTYPE is set.

---------------------------------

| 0 | 0 | 0 | F | F | F | F | F |

---------------------------------

\_________/ \_________________/

|     	    	 |

|     	    	 --> Format Number:

|     	    	       0 = GROUP(2), USER(2),

|		  	   NAME(O - 12)

|     	    	       1 = NAME(1 - 16)

|

--> Unused

DECNET/8 uses the  GROUP(2),  USER(2),  NAME(0-12)

format, which has a minimum length of four bytes.

DSCRIP :  The object descriptor field.  The  format  depends

on  the value of FORMAT.  DSCRIP has the subfields

GROUP, USER, and NAME.

6-40

INTERNAL NETWORK DESCRIPTION

GROUP :   The  group  code  of   the   object

contained   in   two  bytes.   This

subfield and the USER subfield  are

not  used  by  DECNET/8.   They are

provided  for  compatibility   with

other systems using the network.

USER :  	The  user  code   of   the   object

contained  in  two bytes (also used

for compatibility).

NAME :  	The name of the object in 12 bytes.

If 0BJTYPE = 0, this field contains

the   process   or    task    name.

Otherwise,  it  contains additional

information specific to each object

type.

SRCNAME   :    The name of the source task requesting the  connection;

it  must be in the same field as DESTNAME.  SRCNAME has

the subfields DSCRIP, GROUP, and USER as described  for

DESTNAME.

USERDATA  :    The data being sent in the single message.

6.3.6  Major NSP Support Routines

This section lists the major NSP support subroutines  in  alphabetical

order.

CALDDC    Initiates DDCMP functions.   The  word  following  the  CALL

contains  the  DDCMP  function  code.   NSP  returns  to the

routine that  called  CALDDC  on  completion  of  the  DDCMP

function.

CHKDST    Called when a logical  link  address  must  be  checked  for

validity  during received message processing.  The CCB being

checked is brought into the scratch area on page 0.

DRLTSK    Sends a task's AST for execution.  This  routine  is  called

when  a  task  must  be interrupted, e.g., during a received

CONNECT INITIATE.

DRLXIT    Runs as an extension of the user's task.  This subroutine is

called  from  a  small  interface  in  the  user's  CCB with

interrupts off.  It inhibits task switching immediately  and

sets up the user's AST routine to run.

FLUSH     Flushes  any  unused  data from  received  messages  when  a

message has been processed or if an error is detected.

GETCCB    Allows NSP to search for CCBs by  taskname  when  processing

connect and single messages.

6-41

INTERNAL NETWORK DESCRIPTION

GETCH     Gets the next  byte  from  the  current  NSP  message  being

scanned.

PUTNAM    Converts a packed  6-bit  name  into  extensible  ASCII  for

transmission.

PUTNOD    This pair of subroutines gets and returns packets  from  the

GETNOD    freelist in DDCMP's Node Pool.

PUTNUM    Converts a 12-bit number into an extensible binary character

string.

PUTOPT    Passes user optional data to the current NSP  message  being

built.  This subroutine is used for connect messages.

SENDDC    Composes  an  NSP  message  and  passes  it  to  DDCMP   for

SENDDX    transmission.    When  the  transmission  is  complete,  NSP

returns to the routine that called  SENDDC.   SENDDX  is  an

alternative  entry  point that does not resume processing on

completion.  NSP uses the Node Pool for dynamic storage when

constructing headers.

SNDRTN    On completion of a DDCMP message transmission,  returns  all

packets used to contain the NSP message header.

SRCHCC    Searches the table of active logical links  and  breaks  any

links  using  the current physical link.  This subroutine is

called if a line fails (when all links must be broken).

GETLNK    This pair of subroutines assigns and deassigns logical  link

PUTLNK    addresses from the free addresses in the link table.

GETNAM    Parses the DESTNAM and SRCNAM fields of connect initiate and

single messages.

GETNOD    This pair of subroutines gets and returns packets  from  the

PUTNOD    freelist in DDCMP's Node Pool.

GETNUM    Converts an extensible binary character string into a 12-bit

number.

MOVCCB    When NSP runs, it copies the user's CCB into a page 0 buffer

for easy access.  Upon exit from NSP, this routine moves the

CCB back to the user's area.

MOVWDS    Moves data to and from the user's field and NSP's field.

NODOVT    Looks up a node name during processing  of  the  CONINI  and

TRNSM function calls.

POSTPK    Posts a user NSP function call.

PUTCH     Stores the next character in the NSP message.

PUTLNK    This pair of subroutines assigns and deassigns logical  link

GETLNK    addresses from the free addresses in the link table.

6-42

INTERNAL NETWORK DESCRIPTION

6.3.7  NSP Tables

NSP maintains four major tables for keeping records of  the  state  of

the logical links and other related information.

o   CCBTAB. A table mapping channel numbers to CCB addresses.

o   LNKTAB. A table of logical link addresses.

o   NETTAB. A table of global information about the network.

o   NODTAB. A table relating node names to node numbers.

The use of Connect Control Blocks  (CCBs)  is  described  in  Sections

2.1.3  and  2.4.   The CCB table is used to establish a correspondence

between channel numbers and CCB addresses.  The table has  a  two-word

entry for each channel.  The first word contains a CDF instruction and

the second word contains the address of the CCB for the channel.   The

length  of  the  table  is  determined  by  the  number of CCBs in the

network.  Figure 6-16 shows the CCB table.

--------------------

|	      CDF	 |  \  CHANNEL 1

|       ADDR	 |  /

|------------------|

|	      CDF	 |  \  CHANNEL 2

|       ADDR	 |  /

|------------------|

.			 .

.			 .

.			 .

.			 .

Figure 6-16  CCBTAB

LNKTAB is a table of one-word entries of logical link  addresses.   It

is  used  to  tell if the address is in use or if it is available.  If

the address is in use, its entry in LNKTAB contains the channel number

for  the connection.  If the address is unused, the entry in LNKTAB is

a thread word in a first-in first-out list of unused addresses.  LNKHD

is  a  pointer  to  the  beginning of the list of unused addresses and

LNKTL points to the end of that list.  LNKTAB is illustrated in Figure

6-17.

6-43

INTERNAL NETWORK DESCRIPTION

---------------

|   0  |  1   |     The high-order bytes of

|-------------|     the entries padded with

LNKHD	------>	|	      |     zeros.

|-------------|

|   0  |  4   |

|-------------|

|	      |

|-------------|

|	      |

|-------------|

|   0  |  2   |

|-------------|

LNKTL	------>	|      0      |

---------------

Figure 6-17  LNKTAB

An active address contains a channel number, CHANNL, in  the  range  0

through  77.  An inactive address contains an address greater than 77.

This makes it easy to differentiate between used and unused addresses.

NETTAB contains global information about the network.  It contains the

version  numbers  of  the protocols in use, and the user's node number

and node name.  Figure 6-18 shows the layout of NETTAB.

---------------------

|   DDCMP VERSION   |

|-------------------|

|	DDCMP SUBVERSION |  = Patch Level

|-------------------|

|	  NSP VERSION    |

|-------------------|

|   NSP SUBVERSION  |  = Patch Level

|-------------------|

| USER'S NODE NUMBER|

|-------------------|

|			 |

|---	      ---|

|	  6- CHARACTER	 |

|---	      ---|

|			 |

|---	      ---|

|	   NODE NAME	 |

|---	      ---|

|			 |

|---	      ---|

|			 |

---------------------

Figure 6-18  NETTAB

6-44

INTERNAL NETWORK DESCRIPTION

NODTAB relates node names to node numbers.  Node numbers are  assigned

by  the  system  manager and must be unique.  Node names, however, are

assigned by users and there is no restriction to the number  of  names

assigned  to  a node.  This table keeps a record of the names assigned

to each node.

The Node Name Table must be assembled at  system  generation;   it  is

described  in  Section  5.5.   Figure 5-1 shows the layout of the Node

Name Table.

6-45

CHAPTER 7

STAND-ALONE USE OF DDCMP

This chapter describes how  the  DDCMP  protocol  can  be  used  as  a

stand-alone  task  and provides guidelines for users writing their own

line protocols.

7.1  STAND-ALONE USE OF THE DDCMP TASK

Using  DDCMP  without  the  NSP  protocol  has  both  advantages   and

disadvantages.   Using  DDCMP  as a stand-alone task means there is no

implied compatibility with other  DECNET  systems  using  NSP.   Also,

DDCMP  can communicate with only one task at a time, and this task has

the overhead of providing certain ancillary functions that would  have

been  provided  by  NSP.   The main advantage is the core savings from

using only one protocol.  This  is  particularly  useful  in  a  small

network  configuration,  for example, an RTS/8 to RTS/8 link with only

one logical link between the tasks.

DDCMP is unlike normal RTS/8 device driver tasks.  First, it is linked

to  only  one  task  because  it was designed to service the NSP task.

Second, the user task must be prepared not only to send RTS/8 messages

but also to receive messages.

All the NSP parameters are grouped together in the parameter  file  so

that  they  can  be left undefined at system generation.  To use DDCMP

without NSP, the parameter DDCUSR in the RTS/8 parameter file must  be

set  to the user task number.  (This parameter is automatically set to

the NSP task number when NSP is being used.) The other differences are

that  DDCMP  conventions  for data storage must be observed.  The user

task is not limited to having  only  one  physical  link  between  two

stand-alone DDCMP tasks.

There  are  three  phases  to  the  use  of  DDCMP:    startup,   data

transmission,  and bringing the line down.  These phases are described

in the following sections.

7.1.1  Bringing Up a Physical Link

Before a physical link can be used to exchange  messages  between  two

nodes, the DDCMP tasks at both ends must be initialized.  The user can

initiate this process by issuing a DDCMP startup request to the  local

DDCMP, which in turn sends a startup request to the remote node on the

selected  line.   The  remote  DDCMP then informs its user task of the

received startup request.

The remote node responds with a startup request and the physical  link

between  the  two nodes is brought on line.  This procedure is similar

to the connect sequence used by NSP.

7-1

STAND-ALONE USE OF DDCMP

7.1.2  Transmitting and Receiving Messages

After the initialization procedure is complete and the  physical  link

is on line, the two nodes are ready to exchange messages.  Without the

flow control provided by NSP, the user task must be ready  to  receive

messages  at any time from the remote node.  If the user task requests

DDCMP to transmit a message, DDCMP services the request at  the  first

opportunity.   Messages are assured of correct, sequential delivery by

DDCMP.

7.1.3  Breaking the Physical Link

If the physical link between two connecting DDCMPs is "lost", DDCMP is

notified  of  the  line  failure  by  its timing routine (described in

Section 6.2.7).  A physical link  breakage  could  be  caused  if  the

remote  node  was  powered  down.   DDCMP's  timing  routine tests the

physical link periodically.

A HALT function is provided to allow  a  node  to  continue  operating

without  using its link to the remote node.  HALT causes DDCMP to post

the link as dormant until another STARTUP sequence is initiated.   The

connecting  nodes  should  establish  a convention on what action they

both want to take if the physical line goes down.

7.2  ISSUING DDCMP FUNCTION CALLS

The procedure for issuing a function call to  DDCMP  is  to  send  the

message  and then wait for DDCMP to send it back.  The data being sent

in the message must be located in the same  field  as  DDCMP  (DDCFLD)

because  DDCMP maintains its own internal queues of function calls and

messages associated with each line using packets from the  Node  Pool.

This can be accomplished in two ways.

1.  Build the task in the same field as DDCMP and use  the  RTS/8

SEND  Executive Request as normal.  The function call must be

assembled in the user task.

2.  Build the RTS/8 message dynamically in a  Node  Pool  packet.

(This method is used by NSP.) Use the DDCMP subroutine DDCFNC

to send the assembled  packet  to  DDCMP.   This  avoids  the

necessity  of  the  user  task  residing  in the DDCMP field.

DDCFNC is defined as a global symbol in the  parameter  file.

Note  that  the  RTS/8  SEND  ER  cannot  send a message in a

different field from the user task field.

Example:

.

.

.

DCA PACKET	      /STORE ADDRESS OF PACKET NODE

CDF CUR

CIF DDCFLD

7-2

STAND-ALONE USE OF DDCMP

JMS I (DDCFNC)

PACKET, 0

/RETURN HERE WITH DF = CUR

SENDW requests must not be used to pass DDCMP  function  calls.   When

DDCMP  has  completed  a  function call, it sends the call back to the

user task.  The user task must issue a RECEIVE ER to  be  prepared  to

accept notification of function completion.

When DDCMP needs to inform the user task of some  external  event,  it

sends  a  message  to the task.  DDCMP generates these messages in the

Node Pool.  A common receive request can be issued by the user task to

receive  both  types  of messages;  function codes issued by DDCMP and

DDCMP event codes are distinct  and  can  be  easily  identified  from

user-generated  function  calls.   The following sections describe the

DDCMP function calls and event messages.

7.2.1  STARTUP

FUNCTION:

Initiate the startup procedure on a specified line.

MESSAGE FORMAT:

ZBLOCK 3		/RTS/8 HEADER

STATUS/LINE		/STATUS CODE AND LINE NUMBER

1			/FUNCTION CODE FOR STARTUP

0			/PADDING

USAGE NOTES:

1.  STARTUP instructs DDCMP to initiate a STARTUP procedure.   It

is  used  to bring up a physical link or to respond to a STRT

received condition.

2.  DDCMP tries to establish contact with the  remote  DDCMP  ten

times  with  three-second intervals between each attempt.  If

the  remote  node  responds,  DDCMP  executes   the   STARTUP

procedure and returns the function call packet to the issuing

DDCMP with a successful status word.  If no contact is  made,

the status word indicates an error.

3.  If the line is already on-line, the request  is  returned  at

once  with  a  successful  status  word.   To  initiate a new

STARTUP, the line must first be HALTed, i.e., marked as idle.

4.  If a STARTUP function call is issued  while  DDCMP  is  still

completing  a  previous  STARTUP  call, the second request is

queued and  returned  to  the  user  task  when  the  STARTUP

procedure has completed.

5.  An error condition is not reported if a  STARTUP  request  is

issued when the line is already on line.

7-3

STAND-ALONE USE OF DDCMP

7.2.2  TRNSMT

FUNCTION:

Transmit a message over a physical link.

EXAMPLE:

ZBLOCK 3		   /RTS/8 HEADER

STATUS/LINE		   /STATUS CODE AND LINE NUMBER

0			   /FUNCTION CODE FOR TRNSMT

CDF BUFFLD		   /CDF INSTRUCTION

BUFLOC -1		   /BUFFER LOCATION

COUNT			   /NEGATIVE COUNT

USAGE NOTES:

1.  TRNSMT instructs DDCMP to transmit the user  data  stored  in

the buffer to the remote node.

2.  The function call is returned to the user task on completion.

If  the message was sucessfully acknowledged, the STATUS byte

of the STATUS/LINE word is set to  zero  and  the  LINE  byte

contains the line number.

If error thresholds are exceeded or  an  unexpected  STRT  is

received,  the  function  call  is  returned  with  an  error

condition.  The  function  call  packet  is  returned  first,

followed  by  an  error  packet  (see  LINE  FAILED  and STRT

RECEIVED).

3.  If DDCMP has not recorded the line as on-line (STATE  =  11),

the  user  function  call  packet is returned with an ILLEGAL

status.  This could be caused by an error in the software  or

timing;   that  is,  if  the  user task sent the request just

before DDCMP detected an error condition.

7.2.3  HALT

FUNCTION:

Puts a line in the idle state.

EXAMPLE:

ZBLOCK 3		   /RTS/8 HEADER

STATUS/LINE		   /STATUS CODE AND LINE NUMBER

2			   /FUNCTION CODE FOR HALT

0			   /PADDING

USAGE NOTES:

1.  HALT causes DDCMP to force a line to the idle state.

7-4

STAND-ALONE USE OF DDCMP

2.  The user packets are returned if  they  are  pending  in  the

queues  for  this line.  This function could be used to abort

operations on a line.

3.  Once the HALT request has  been  made,  no  messages  can  be

passed  to the user task using this line.  In the idle state,

only STRT requests can be passed to the user task.

4.  When DDCMP has notified its user task of a STRT received  but

has  not  received  a  STRT in response, it ignores any other

STRT messages.  HALT can  be  used  to  enable  further  STRT

messages to be sent.

7.3  DDCMP EVENTS

DDCMP notifies its user task when a DDCMP event occurs,  such  as  the

line  going  down,  a  message  received  from a remote node or a STRT

request.  It sends a Node Pool message packet to the  task  containing

the appropriate request.  This packet is 14 (octal) words in length.

7.3.1  RCVMSG

EVENT:

DDCMP has received a message from a remote node.

PACKET FORMAT:

ZBLOCK 3		    /RTS/8 HEADER

LINE			    /LINE NUMBER

4			    /FUNCTION CODE FOR RCVMSG

0			    /PADDING

CDF BUFFLD		    /BUFFER DESCRIPTOR

ADDR -1

COUNT

USAGE NOTES:

1.  The user task has the responsibility of removing the  message

from  the  ISR ring buffer and returning the Node Pool packet

to the freelist.

Sample subroutines for getting a node from the Node Pool  and

removing  data  from  the  ring  buffer  are given in Section

7.3.4.  These subroutines are used in NSP and  their  use  is

explained in the NSP listing.

2.  The user task does not have to post the event flag associated

with RCVMSG.

7-5

STAND-ALONE USE OF DDCMP

7.3.2  LINDWN

EVENT:

Brings the line down.

PACKET FORMAT:

ZBLOCK 3	      /RTS/8 HEADER

LINE		      /LINE NUMBER

6		      /FUNCTION CODE FOR LINDWN

0		      /PADDING

USAGE NOTES:

1.  Issued if DDCMP detects a threshold counter overflow.

2.  The user task must return the packet to the Node  Pool.   The

message event flag does not have to be posted.

3.  Any outstanding messages are returned to the user task before

this  message  is  sent;  this gives a warning of the loss of

the line.  LINDWN is always sent even if  the  user  task  is

already aware of the condition.

7.3.3  STRT RECEIVED

EVENT:

Notifies the user task that a STRT message has been  received  from  a

remote node.

PACKET FORMAT:

ZBLOCK 3	/RTS/8 HEADER

LINE	/LINE NUMBER

5	/FUNCTION CODE FOR STRT RECEIVED

0	/PADDING

USAGE NOTES:

1.  The user task must return the packet to  the  Node  Pool  but

does not need to pool the event flag.

2.  Any outstanding messages are returned (as  if  the  line  had

gone down) before this message is sent.

3.  DDCMP sets a flag when it notifies the user task  of  a  STRT

message.   This  flag  is  cleared  when the user task issues

either a STARTUP or a HALT function call.

4.  If a subsequent STRT is received on this line when  the  flag

is  set, DDCMP ignores the message.  This allows disabling of

the user task without using unnecessary  STRT  messages  that

would  queue  up  uselessly.   This  could happen if a remote

7-6

STAND-ALONE USE OF DDCMP

DDCMP keeps trying to start up a line.

7.3.4  Sample DDCMP Support Subroutines

This section lists two examples of subroutines used to implement DDCMP

events.   The  user  task  must provide subroutines to remove messages

from the ISR ring buffer and to obtain a node  pool  packet  from  the

freelist  of  packets  to  transport  the  message to its destination.

These facilities would  normally  be  provided  by  NSP  but  must  be

provided  by  the user task in the absence of NSP.  The examples below

are NSP  support  subroutines  and  illustrate  the  kind  of  support

subroutines required for the stand-alone use of DDCMP.

EXAMPLE #1:

Subroutine to get a node pool packet from the freelist of packets.

/GET A NODE FROM THE FREE LIST

/EXIT AC -> NODE

/ENTRY OF MAY BE RANDOM

/EXIT OF = CUR

GETNOD, 0

CDF     DDCFLD  /GET INTO PACKET FIELD

CIF     CUR     /DISABLE INTERRUPTS

TAD I   (FREHD  /LOOK AT THE FREELIST HEAD

SNA

JMP     NOSPACE /JMP TO USER SUPPLIED ERROR ROUTINE

DCA     CURNOD  /POINT AT NEXT AVAIL PACKET

TAD I   CURNOD  /UPDATE FREELIST TO NEXT PACKET

DCA I   (FREHD

DCA I   CURNOD  /ALWAYS CLEAR THE THREAD WORD BEFORE EXITING

TAD     CURNOD  /EXIT WITH AC = PACKET ADDR

CDF     CUR     /RESTORE DATA FIELD

JMP I   GETNOD  /RETURN TO CALLER AND RE ENABLE INTERRUPTS

/PUT A NODE IN THE FREE LIST

/ENTRY AC -> NODE

PUTNOD, 0

CDF     DDCFLD  /GET INTO PACKET FIELD

CIF     CUR     /DISABLE INTERRUPTS

DCA     CURNOD  /STORE ADDR OF PACKET IN TEMP

TAD I   (FREHD  /POINT ITS THREAD AT CURRENT LIST

DCA I   CURNOD

TAD     CURNOD  /NOW UPDATE FREELIST HEADER

DCA I   (FREHD

CDF     CUR     /RE ENTER OUR DF

JMP I   PUTNOD  /EXIT WITH CLEAR AC AND REENABLE INTERRUPTS

CURNOD, 0

EXAMPLE #2:

7-7

STAND-ALONE USE OF DDCMP

Subroutine to remove characters from the ring buffer.

/GET A CHARACTER FROM THE CURRENT BUFFER

/MARK LOCATION CLEAR (-1) WHEN DONE

/RETURN TO CALL+1 IF NO MORE DATA

/RETURN TO CALL+2 IF CHAR IN AC

GETCH, 0

TAD     GETCHC  /ANY?

SMA CLA

JMP I   GETCH   /TAKE ERROR RETURN

ISZ     GETCH   /TAKE NORMAL RETURN

GETCHF, 0		     /GETS FIELD OF DATA

ISZ     GETCHP  /PREINCREMENT CHAR POINTER

TAD I   GETCHP

SMA	     /SKP IF LINK

JMP     GOTCH   /ELSE GOT A CHAR

DCA     GETCHF  /SET THE FIELD INLINE

ISZ     GETCHP

TAD I   GETCHP  /GET THE NEW POINTER

DCA     GETCHP

JMP     GETCHF

GOTCH,  DCA CHAR	     /SAVE THE CURRENT CHAR

STA

DCA I   GETCHP  /MARK THAT LOCATION FREE

CDF     CUR

TAD     CHAR

ISZ     GETCHC

JMP I   GETCH

JMP I   GETCH   /PROTECT AGAINST OVERFLOW

GETCHP, 0		     /SET TO ADDR-1 OF NEXT BYTE IN BUFFER

GETCHC, 0		     /SET TO -COUNT OF DATA IN BUFFER

CHAR,   0		     /TEMPORARY TO HOLD CURRENT CHAR

7-8

APPENDIX A

GLOSSARY

ACK 0, ACK n

An acknowledgment message sent by a receiving node to  a  transmitting

node  to inform it that message n has been received.  ACK 0 is used in

the initialization sequence to inform the transmitting node  that  the

receiving node is ready to accept messages.

Asynchronous Transmission

Transmission in which time intervals  between  transmitted  characters

may be of unequal length.

Block

A group of digits transmitted as a unit.  A block  is  equal  to  4096

words.

Block Character Check (BCC)

The result of a transmission verification  algorithm  on  a  block  of

characters and appended to the end of the character string, i.e., CRC.

Channel

A path for electrical transmission between two points.  Also called  a

line, link, or path.

Computer Network

An   interconnection   of   computer   systems,   I/O   devices,   and

communications facilities.

Connection

Process that creates a logical link.

Cyclic Redundancy Check (CRC)

An error detection routine.   The  check  character  is  generated  by

taking the remainder after dividing all the serialized bits in a block

of data by a predetermined binary number.

A-1

GLOSSARY

Data Set

A modem.

Data Transmission

Sending data from one computer to another over  a  physical  link,  or

from one task to another over a logical link.

DDCMP

DIGITAL Data Communications Protocol.  A set of conventions  providing

error-free sequential transmission of data over physical lines.

Delay Distortions

Distortion resulting from the non-uniform transmission  speed  of  the

frequency components of a signal through a transmission medium.

Demodulation

Retrieving an original signal from a  modulated  carrier  wave.   This

technique   is  used  in  data  sets  to  make  communication  signals

compatible with computer signals.

Dial-Up Line

A communication line established by a switched circuit connection.

Disconnection

Destroying a logical link, preventing further communication over  that

link.

Down-Line Loading

Transmitting a system core image to a remote node.

Duplex

Simultaneous two-way  independent  transmission  in  both  directions.

Also referred to as full duplex.

A-2

GLOSSARY

Echo

A portion of the transmitted signal returned from the distant point to

the source with sufficient magnitude and delay to cause interference.

ENQ (Enquiry)

Used as a result for  response  to  obtain  identification  and/or  an

indication  of  status.   ENQ is also transmitted as the first word of

channel control messages.

Extensible ASCII

Packing ASCII characters into the high-order 8 bits of each word.  The

ASCII  characters  are  packed  into  7 bits, the high-order bit being

reserved as an indicator bit.  If this bit is  set,  another  byte  of

data  follows.   If  it is not set, it indicates the last data byte in

the string.

Extensible Binary

Packing a binary character string into  a  binary  register  in  8-bit

bytes.   The  high-order bit of each byte is used as an indicator bit.

If this bit is set, another byte of data follows.  If it is  not  set,

it indicates the last data byte in the string.

Full Duplex

See Duplex.

Half-Duplex

Transmission   in   either   direction   but   not   both   directions

simultaneously.

Header

The "envelope" containing the control information and  CRC  characters

at the beginning of a data message.

Image-Mode Field

A field consisting of a COUNT byte (0  or  greater)  followed  by  the

number of data bytes specified in COUNT.  This field has no formatting

restrictions; for example, it could be in extensible binary  or  8-bit

ASCII.

A-3

GLOSSARY

Information Bit

A bit generated by the data source and not used for error  control  by

the data transmission system.

Interrupt Message

A message transmitted by a task that interrupts  the  execution  of  a

target task.

Line

A permanent or temporary physical connection between two nodes in  the

network.  A direct wire joining two nodes is an example of a permanent

line and a telephone circuit is an example of a temporary  line.   See

also physical link.

Link

A communication path between two nodes.  The link could be a  physical

or logical link.

Link Status Message

An internal NSP message.   The  local  NSP  transmits  a  Link  Status

message to the remote NSP to notify the remote task that the user task

at the local node has issued a receive request.

Local Node

The node at which a user task is executing or at which a terminal user

is located.

Local Task

A task executing at a local node.

Logical Link

A virtual data path between two tasks in the network that allows  them

to communicate.

Modem

Modulator-demodulator.   A  device  that  modulates  and   demodulates

signals transmitted over communications facilities.

A-4

GLOSSARY

NAK

Negative acknowledgment.  A control message used to indicate that  the

previous message has not been received due to an error occuring either

during the transmission or in the message itself.  Also  used  in  the

initialization sequence to indicate "not ready."

Network

An interconnected or interrelated group of nodes.  See  also  computer

network.

Node

An end point of any branch of a network or a junction common to two or

more  branches  of  a network.  Synonymous with a computer system in a

network.

Node Name

A 1- to 6-character name uniquely identifying a node.  Node names  can

be  any combination of the characters A through Z, 0 through 9, .(dot)

and  $.   The  network  manager  assigns  node  names  during  network

generation.

NSP

Network Services Protocol.  A set  of  conventions  providing  network

management  functions  including  routing messages between systems and

within any given system.

Overhead Bit

A bit other than an information bit, for example, check bit.

Packet

A group of 14 words of data including control words transmitted  as  a

unit.  The words are arranged in a specified format.

Parallel Transmission

Method of data transfer in which all bits of a character or  byte  are

transmitted simultaneously either over separate communication lines or

on different carrier frequencies on the same communication line.

A-5

GLOSSARY

Parse

Break down a command string and separate  the  constituent  parts  for

analysis and interpretation.

Partner Task

The task located at a remote node communicating with the task  running

at the local node.

Path

A route for electrical transmission between two points.  Also called a

line, link or channel.

Physical Link

A communication path between two nodes.

Point-to-Point Connection

A connection established between two, and only two, nodes in a network

configuration.  The communication may include switching facilities.

Protocol

A formal set of conventions  governing  the  format,  sequencing,  and

relative timing of message exchange.

Redundancy

In a protocol, the portion of the total characters or bits that can be

eliminated without any loss of information.

Remote Node

A network node that is not the local node.

Remote Task

A task executing in a remote node.

Remote Task Loading

Transmitting task images to and from a node for execution.

A-6

GLOSSARY

Serial Transmission

A method of transmission in which each  bit  of  information  is  sent

sequentially  on  a  single  channel  rather than simultaneously as in

parallel transmission.

SOH

Start of header.  A communication control character used as the  first

word of a data message to distinguish it from a control message.

Source Node

During intertask communication, the node  at  which  a  connection  is

requested or from which a message is transmitted.

Source Task

A task executing at a source  node  which  requests  a  connection  or

transmits a message.

Synchronous Transmission

Transmission in which time intervals  between  transmitted  characters

are of equal length.

Target Node

The node at  which  the  request  for  a  connection  is  accepted  or

rejected, or to which a message is transmitted.

A-7

INDEX

12-page version of NIP, 5-10         Connect initiate, 2-5

8-page version of NIP, 5-10          Connect reject, 2-6

ACK, 6-3                             Connect request, 2-3

Acknowledge, 6-3                     Connection, 2-3

ADDR pairs, 2-23                     CONREJ, 2-3, 2-6, 2-14

Adjacent node, 5-12                  CR, 6-31

ASCII string, 3-1                    CRC, 6-3

AST, 2-1, 2-3, 2-7, 2-31             CRC block check, 6-22

AST execution, 2-32                  CRJST, 2-9

Asynchronous system trap,            CUR, 2-7

2-1                              CURRENT FIELD, 2-7

Cyclic redundancy check, 6-3

BCC, 6-22

BLKARG, 2-33                         Data message, 2-18

Block check characters,              Data stream, 6-27, 6-28

6-22                             DDCLP, 6-7

DDCMP, 1-3, 5-1, 6-2

DDCMP and NSP parameters,

5-7

CAL, 2-33                            DDCMP completion queue, 6-11

CC, 6-31                             DDCMP error counters, 4-3

CCB, 2-1, 2-3, 2-6, 2-7              DDCMP field, 6-1

CCB.PA, 2-33                         DDCMP support subroutines,

CCBAC, 2-34                              7-7

CCBDRL, 2-35                         DDCUSR, 7-1

CCBPC, 2-33                          DECNET/8 system generation,

CCBRTN, 2-32                             5-1

CCBSG, 2-35                          Destination block, 2-9

CCBSN, 2-35                          Device designation, 3-1

CCBSOT, 2-35                         DISCON, 2-7, 2-16, 6-35

CCBSU, 2-35                          Disconnect, 2-7, 2-9, 6-35

CCBXDF, 2-34                         DISST, 2-9, 2-17

CDF, 2-7                             DR, 6-31

CDF pointer pairs, 2-18              DRJST, 2-9

Channel control request,

6-3

Channel number, 2-5

CHANNL, 2-5, 2-7                     EAE, 2-31

Checkpointable task, 2-18, 2-23      Entry point, 2-32

Clock tick, 6-8                      ER, 2-1

Communicating programs, 1-3          Error recovery routine,

Communications path, 2-5                 2-16

Computer network, 1-1                ERRST, 2-10, 2-13, 2-15,

CONCNF, 2-3, 2-6, 2-12                   2-17

CONINI, 2-6, 2-8, 6-31               Executive idle loop, 6-13

Connect, 2-5, 6-31                   Executive listen routine,

Connect confirm, 2-6                     6-7

Connect control block, 2-1,          Executive request, 2-1

5-3                              Executive requests, 1-6

Index-1

INDEX (CONT.)

Extensible binary and ASCII, 6-27    Logical link control level,

1-3

Logical link number, 4-2

Logical link status, 4-2

Full-duplex data                     LSN, 1-5, 3-1, 5-1

transmission, 5-14

Function calls, 2-2, 2-5

Function codes, 2-5

MAXNOD, 5-13

Memory fields, 6-1

HALT, 7-4                            Message transmission, 1-3

HALT LINE message, 6-10              Message acknowledgement, 6-24

Message retransmission, 6-24

MQ, 2-34

MSGWT, 2-33

Illegal connection request,          Multidrop, 6-4

2-9

Illogical request, 2-13

ILLST, 2-9, 2-13, 2-15,              NAK, 6-3

2-17                             Negative acknowledge, 6-3

Initialize, 6-3                      NETTAB, 6-44

Inquiry/response, 2-7                Network Information program,

Interrupt, 2-7, 2-21                     1-6, 4-1, 5-1

Interrupt message, 2-32              Network Services Protocol,

Interrupt service routine,               5-1, 6-24

5-13                             NEW STATE, 6-13

Interrupt service routines,          NIP, 1-6, 4-1, 5-1

5-1                              NIP parameters, 5-9

INTRPT, 2-21, 6-38                   Node, 1-1

ISR, 1-4, 5-13                       Node Name Table, 5-11

Node pool space, 6-1

NODNAM, 2-28

NODST, 2-9

NODTAB, 6-45

LCBTAB, 6-15                         NSP, 1-3, 5-1

LCF, 5-14                            NSP error, 2-10, 2-13

LDNST, 2-10, 2-13, 2-15,             NSP field, 6-2

2-17                             NSP function call format,

LINDWN, 7-6                              6-13

Line control block table,            NSP status messages, 2-7

6-15                             NSP support subroutines,

Line control file, 5-14,                 6-41

6-17                             NSPFLD, 6-2

Link status message, 2-6             NUM word, 6-4

Listen utility, 1-5, 3-1,

5-1

LNKTAB, 6-43

Local task, 4-2                      Object type, 2-32, 2-35, 4-2

Local task group, 2-35               Optional data, 2-6

Local task name, 2-35                Owner codes, 2-33

Logical link, 2-5, 4-2

Logical link control

interface, 1-3

Index-2

INDEX (CONT.)

PARAM.PA, 5-2                        Status word, 2-5

Permanent link, 1-1                  STRT, 6-3

Physical line error status,          STRT RECEIVED, 7-6

4-1                              Success, 2-9, 2-13

Physical line number, 4-2            SUCST, 2-9, 2-13, 2-15,

Physical line status, 4-1                2-17

Physical link, 1-1, 7-1              Synchronous buffering, 2-21

Physical link control level,         SYSGEN, 2-9

1-3

Physical link interface

level, 1-4

PPN, 2-9                             Talk utility, 1-5, 3-1, 5-1

Program dialogue level, 1-2          Target node, 3-1

Programmer/project number, 2-9       Target task, 2-3

Protocols, 1-2                       Task number, 4-2

Pseudo-line driver, 5-14             Temporary connection, 1-1

Pseudo-op, 2-23                      Terminal communication

utility, 1-5

Threshold count, 6-15

Time-outs, 6-24

RCNMSG, 2-6, 2-25, 6-38              TLK, 1-5, 3-1, 5-1

RCVMSG, 2-3, 2-6, 2-23,              TLK and LSN parameters,

6-38, 7-5                            5-11

RCVSM, 2-7, 2-27, 6-39               Transmit, 2-6, 2-30

Receive, 2-6                         Transmit data request, 6-3

Receive messages, 2-3                Transmit single message,

Receive single message, 2-7              2-7

Reject, 2-14                         TRNSM, 2-7, 2-30, 6-39

REP, 6-3                             TRNSMT, 2-3, 2-6, 2-18,

Response, 6-3                            6-37, 7-4

Ring buffer, 6-16

Routing headers, 6-26

RTS/8 system generation,

5-1                              UIC, 2-9

User identification code,

2-9

User request, 1-2

SEND, 2-1                            User task, 1-2

Send messages, 2-3                   USRAST, 2-33

SENDW, 2-1

Single-message, 2-7

Source task, 2-3, 4-2

STACK, 6-3

STARTUP, 7-3                         WAITE, 2-33

STATION ADDR, 6-4                    WAITX, 2-33

Station address, 6-4

Index-3

RTS/8 DECNET/8

User's Guide

AA-5184A-TA

READER'S COMMENTS

NOTE:  This form is for document comments only. DIGITAL will

use comments submitted on this form at the company's

discretion. Problems with software should be reported

on a Software Performance Report (SPR) form. If you

require a written reply and are eligible to receive

one under SPR service, submit your comments on an SPR

form.

Did you find errors in this manual? If so, specify by page.

----------------------------------------------------------------------

----------------------------------------------------------------------

----------------------------------------------------------------------

Did you find this manual understandable, usable, and well-organized?

Please make suggestions for improvement.

----------------------------------------------------------------------

----------------------------------------------------------------------

----------------------------------------------------------------------

Is there sufficient documentation on associated system programs

required for use of the software described in this manual? If not,

what material is missing and where should it be placed?

----------------------------------------------------------------------

----------------------------------------------------------------------

----------------------------------------------------------------------

Please indicate the type of user/reader that you most nearly represent.

O Assembly language programmer

O Higher-level language programmer

O Occasional programmer (experienced)

O User with little programming experience

O Student programmer

O Non-programmer interested in computer concepts and capabilities

Name ___________________________________ Date _______________________

Organization ________________________________________________________

Street ______________________________________________________________

City ________________________ State __________ Zip Code _____________

or

Country