DECmate II  Specification

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

REV.    DATE 	    MAJOR CHANGES	    # OF PAGES

------------------------------------------------------------

|	 |	    |				   |	      |

|   0	 |16-Nov-81 | Originate                    |	      |

|	 |	    |				   |	      |

------------------------------------------------------------

|	 |	    |				   |	      |

|   1   |17-Nov-81 |Added CPU instruction list	   |	      |

|	 |	    |				   |	      |

------------------------------------------------------------

|	 |	    |Changed instruction timings to|	      |

|   2	 |30-Dec-81 |reflect 8/4 MHz. operation.   |	      |

|	 |	    |Added Z80 softcard to options.|   33     |

|	 |	    |Added Index, changed name.    |	      |

|	 |	    |				   |	      |

------------------------------------------------------------

|	 |	    |				   |	      |

|   3	 |10-Apr-82 |Added details of communication|	      |

|	 |	    |and user writeable characters |	      |

|	 |	    |Re-arranged sections.	   |	      |

|	 |	    |				   |	      |

------------------------------------------------------------

|	 |	    |				   |	      |

|	 |08-Aug-82 |Added environmental specns.,  |	      |

|	 |	    |dimensions and electrical	   |	      |

|	 |	    |requirements. Added example of|	      |

|	 |	    |programming the communications|   59     |

|	 |	    |port. Added to list of ESCape |	      |

|	 |	    |sequences supported.	   |	      |

|	 |	    |				   |	      |

------------------------------------------------------------

|	 |	    |				   |	      |

|   4	 |18-Jan-83 |Added ROM useage data, disk   |	      |

|	 |	    |useage for TRK 0 and 78, 79   |   71     |

|	 |	    |Updated ESCape sequence list  |	      |

|	 |	    |				   |	      |

------------------------------------------------------------

|	 |	    |				   |	      |

|	 |	    |				   |	      |

|	 |	    |				   |	      |

|	 |	    |				   |	      |

------------------------------------------------------------

JOHN KIRK

DECmate II  Specification

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

INDEX.

Section		   Contents				   Page

1.		Physical				    2

1.1		   Mechanical				    2

1.2		   Electrical				    2

1.3		   Environmental			    2

2.		System Components

2.1		   CPU					    3

2.2		   Memory				    3

2.3		   Disk					    4

2.4		   Display				    4

2.5		   Keyboard				    5

2.6		   Printer Controller			    5

2.7		   Clock				    5

2.8		   Communications Port			    6

3.		Options

3.1		   Graphics				    6

3.2		   5.25" Winchester Disk Sub-system	    6

3.3		   Z80 Softcard				    6

3.4		   RX01/02 Adaptor			    6

4.		Detailed Description - Basic Machine

4.1		   CPU					    7

4.2		   Display				    7

4.2.1	      Screen Format and Modes		    8

4.2.2	      Character Attributes		    8

4.2.3	      Character Types			    9

4.2.4	      Scrolling				   10

4.2.4.1	         Types of Scrolling		   10

4.2.4.2	         Scrolling Regions		   10

4.2.5	      Display Control - ANSI Mode	   10

4.2.5.1	         CPR				   10

4.2.5.2	         CUU				   10

4.2.5.3	         CUD				   11

4.2.5.4	         CUF				   11

4.2.5.5	         CUB				   11

4.2.5.6	         CUP				   12

4.2.5.7	         HVP				   12

4.2.5.8	         DSR				   12

4.2.5.9	         IND				   12

4.2.5.10	         NEL				   12

4.2.5.11	         RI				   12

4.2.5.12	         DA				   13

4.2.5.13	         ED				   13

DECmate II  Specification

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

4.2.5.14	         EL				   14

4.2.5.15	         DECKPNM			   14

4.2.5.16	         DECPAM				   14

4.2.5.17	         SM				   15

4.2.5.18	         RM				   15

4.2.5.19	         ANSI Specified SM/RM Parameters   15

4.2.5.20	         DEC Private SM/RM Parameters	   16

4.2.5.21	         SCS				   17

4.2.5.22	         SGR				   20

4.2.5.23	         DECDWL				   20

4.2.5.24	         DECDHL				   21

4.2.5.25	         DECSWL				   21

4.2.5.26	         DECSTBM			   21

4.2.5.27	         HTS				   22

4.2.5.28	         TBC				   22

4.2.5.29	         DECSC				   22

4.2.5.30	         DECRC				   22

4.2.5.31	         DECPRS				   22

4.2.5.32	         DECLL				   22

4.2.5.33	         DECALN				   22

4.2.5.34	         IL				   23

4.2.5.35	         DL				   23

4.2.5.36	         DCH				   23

4.2.6	      Display Control - VT52 Mode

4.2.6.1	         Cursor Up			   23

4.2.6.2	         Cursor Down			   24

4.2.6.3	         Cursor Right			   24

4.2.6.4	         Cursor Left			   24

4.2.6.5	         Enter Graphics Mode		   24

4.2.6.6	         Exit Graphics Mode		   24

4.2.6.7	         Cursor Home			   24

4.2.6.8	         Reverse Linefeed		   24

4.2.6.9	         Erase to End of Screen		   24

4.2.6.10	         Erase to End of Line		   24

4.2.6.11	         Direct Cursor Address		   25

4.2.6.12	         Identify 			   25

4.2.6.13	         Enter Alternate Keypad Mode	   25

4.2.6.14	         Exit Alternate Keypad Mode	   25

4.2.6.15	         Enter ANSI Mode		   25

4.3		   Keyboard				   25

4.3.1	      Instruction List			   26

4.4		   RX50 Disk Controller			   26

4.5		   Printer Port				   29

4.6		   Communications Port			   30

4.6.1	      Instruction List			   30

4.6.2	      Internal Registers		   32

4.6.2.1		 Control Register - R0		   32

DECmate II  Specification

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

4.6.2.2		 Control Register - R1		   33

4.6.2.3		 Control Register - R2		   33

4.6.2.4		 Control Register - R3		   33

4.6.2.5		 Control Register - R4		   34

4.6.2.6		 Control Register - R5		   34

4.6.2.7		 Control Register - R6		   35

4.6.2.8		 Control Register - R7		   35

4.6.2.9		 Status Register 0		   36

4.6.2.10		 Status Register 1		   36

4.6.3	      Data Transfer

4.6.3.1		 Input				   37

4.6.3.2		 Output				   37

4.7		   Clock				   37

5.		Detailed Description - Options

5.1		   Identification			   38

5.1.1	   Z80 APU				   38

5.2		   Graphics				   39

5.3		   Micro Winchester Disk Adaptor	   39

5.4		   RX01/02 Adaptor			   39

6.		Firmware Details			   39

6.1		   CRT Controller and Associated Logic	   40

6.1.1	      80/132 Column Mode		   41

6.1.2	      Video Enable			   41

6.1.3	      Screen Mode			   41

6.1.4	      Cursor Mode			   41

6.1.5	      Erase Control			   41

6.6.6	      Extended Character Mode		   42

6.1.7	      User Character Set Mode		   42

6.1.8	      CRT Controller Registers		   44

6.1.9	      Cursor Visibility			   46

6.2		   Panel Requests			   47

6.2.1	      PR0				   47

6.2.2	      PRO 				   47

6.2.3	      PR2				   47

6.2.4	      PR3				   47

6.3		   Terminal Input Output		   50

Appendix A.	Standard ROM Character Definitions	   51

Appendix B.	CPU Instruction Set			   52

Appendix C.	Example of Communications-Port Use	   56

Appendix D.	Memory Allocation			   62

Appendix E.	ROM Contents				   63

Appendix F.	Disk Format Track 0, Tracks 78 and 79	   68

Appendix G.	Boot Block Standard			   71

DECmate II  Specification

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

General

This document describes a single board computer system  having  a

single RX50 drive as the basic mass storage device. A second RX50

is optional.

Together with the LK201 keyboard  and  the  VR201  monitor,  this

forms a lower cost replacement for the existing DECmate I system.

Features of this new system are compared and contrasted to  those

of DECmate I.

Options include Z80 auxiliary processing  unit,  bitmap  graphics

card  and  interfaces  to  RX02(RX01)  disk drives and to a 5.25"

Winchester disk.

One stated goal is that existing software,  now  running  on  the

DECmate  I,  will  run  unaltered with the exceptions relating to

disk capacity differences and  the  programming  notes  given  in

Sections  4.6  and  4.7.  The new machine has additional features

over and above those of DECmate I which may be used  as  software

is  developed, these will not, however, cause improper operation

for those programs unaware of the new features.

This specification describes what the combination of hardware and

firmware   currently   do   (Firmware   Revision   0224).  Future

enhancements to the firmware may add to what is described here.

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

16-January-1983

Page 1      Rev 4

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

DECmate II  Specification

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

1. Physical

The single board computer system  module, the  RX50  floppy  disk

drive(s)  (RD50/51  Disk Drive) and  a power supply to supply the

module, disk(s), monitor and keyboard are housed in  a small  box

that  connects  to  the  AC  power  line  and to the monitor. The

keyboard connects to the  monitor.  Provision  is  made  for  the

addition  of one or more daughter boards to allow optional bitmap

graphics, Z80 APU, RX01/02 adaptor and an interface  to  a  5.25"

Winchester disk drive.

1.1 Mechanical

1.1.1 System Box

Dimensions		  6.5" (H) x 14.3" (D) x 19" (W)

Weight (single RX50)	  30 lbs.

(Dual RX50)	  32 lbs.

1.1.2 Monitor

Dimensions		  11.5" x13.75" x12.25"

Weight			  14 lbs.

1.1.3 Keyboard

Dimensions		  2" x6.75" x21"

Weight			  4.5 lbs.

1.2 Electrical

Input voltage		  95 - 128 VAC 1 phase or

190 - 256 VAC 1 phase

Frequency		  47 - 63 Hz.

Power Consumption	  218 watts (max.)

3A @ 120 VAC.

1.5A @ 240 VAC.

The AC input voltage selection is made by a switch at the rear of

the  system  box.  A  detachable linecord allows the system to be

configured easily for operation in most countries of the world.

1.3 Environmental

Operating :

temperature range	  59 - 90 F

humidity		  20 - 80 % non condensing with a

maximum  wet bulb of 77 F and a

minimum dewpoint of 6 F.

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

16-January-1983

Page 2      Rev 4

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

DECmate II  Specification

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

(Recommended operating range   65 - 75 F, 40 - 60% RH)

Altitude 		  0 - 10,000 feet.

2.0 System Components

The basic DECmate II configuration is as follows :

.	CPU  -  6120 High Speed CMOS Processor

.	32K words of user memory

.	32K words of control memory used for

Internal diagnostics

Floppy disk bootstrap

Character display level 1 terminal emulation

Display buffer memory

Space available to user for fast overlays etc.

.	Dual/quad 5.25" floppy disk controller

.	Video display controller

.	Keyboard controller

.	Serial printer controller

.	100 Hz. crystal controlled clock

.	Multi-protocol communications line controller

2.1 CPU

DECmate II	DECmate I

CPU chip		6120		6120

Speed			8/4 MHz.	5 MHz.

Interrupt Latency	< 250 usec.	< 1 msec.

DMA overhead		~10%		2.5%

2.2 Memory

DECmate II	DECmate I

User memory		32K words	32K words

Control memory		32K words	4K words

Startup & self-test	4K words	Included in control

memory number.

The startup ROM is loaded into control memory RAM  at  power  on.

This   code   performs   self-test,  initialization  of  the  I/O

controllers  and  CRT  controller.  When  this  is   successfully

completed  the  disk bootstrap code is executed. The main part of

control panel memory which defines the  terminal  characteristics

is  then  loaded from the disk. Additional or replacement control

memory code may be loaded from the disk as required to extend the

function of the machine, e.g. support for multiple display pages,

high speed overlays etc.

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

16-January-1983

Page 3      Rev 4

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

DECmate II  Specification

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

2.3 Disk

The DECmate II has as standard a single RX50 disk  drive  with  a

second  drive  optional.  Capacity differences from DECmate I are

given below. All numbers are in decimal.

2.3.1 		DECmate II		DECmate I

RX50 drive(s)	RX02

Basic

# of drives 	1 - 5.25"		2 - 8"

# of surfaces   2			2

Capacity/drive  408K 12-bit words	256K 12-bit words

816K bytes		512K bytes

Basic capacity  408K 12-bit words	512K 12-bit words

816K bytes		1.024M bytes

Expansion	Second drive within	Second RX02, requires

same system box.	additional box

Total capacity  816K 12-bit words	1.024M 12-bit words

1.632M bytes		2.048M bytes

2.3.2 Format

# tracks	 80			 77

# sectors/track  10			 26

Bytes/sector    512			256

Words/sector	256			128

Recording       MFM			MFM with FM headers

All the current  operating  systems  use  12-bit  mode  for  disk

storage,  thus  capacity numbers are given for both this mode and

for 8-bit mode (byte).

2.4 Display

DECmate II 		DECmate I

CRT		   12" diagonal, P4 phosphor

Format, normal  24 x 80 		24 x 80

Format, wide    24 x 132 		14 x 132

Character	7 x 9 matrix 	7 x 9 matrix

(within a 10 x 10 character cell)

Character set   DEC Multi-national	94 ASCII subset plus

set plus Katakana	special graphics

and DECmate I foreign.

128 user defined	128 fixed (ROM) option

for foreign language.

Character	Bold 			Bold

attributes	Blink 			Blink

Underline 		Underline

Reverse 		Reverse

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

16-January-1983

Page 4      Rev 4

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

DECmate II  Specification

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

Character type  Normal			Normal

Double Height		  -

Double Width		  -

Screen		Normal 			Normal

attributes	Reverse			  -

Scrolling	Jump or variable rate*  Jump only

smooth scrolling.

Scroll region   Selectable scroll 	Full page scroll region.

only

Windows		Multiple display pages  Single page only

horizontal scroll.**

Performance	Full screen update      Full screen update

< 0.1 seconds. 		< 2 seconds.

*  Current firmware implements fixed rate smooth scroll only.

**  Current  firmware  implements  single  page  only  with   no

horizontal scrolling.

The  CRT  controller  uses  a  buffer  area  in  control  memory,

accessing  data by DMA using a single row buffer. Sequential list

screen addressing is used, allowing all the features of the VT100

to  be  implemented.  In  addition, multiple display pages may be

used in some future revison of the  firmware,  giving  fast  page

swaps  or  horizontal  scrolling  for those applications programs

that  can  take  advantage  of  these  features.  (not  currently

implemented).

2.5 Keyboard

This is a version of the LK201 keyboard with keycaps  appropriate

for  the Word Processing Software. Initially only a subset of the

keyboard capabilities will be supported, enough  to  perform  the

functions  of  the  VT100/DECmate  I  keyboard.  The interface to

DECMate  II  is  by  a  full  duplex,  serial  asynchronous  line

operating at 4800 baud.

2.6 Printer controller

A full duplex serial asynchronous line with EIA signalling levels

and programmable speed, DECmate I compatible.

2.7 Clock

A 100  Hz.  clock,  crystal  controlled  as  on  DECmate  I,  the

instruction set is, however, different - see Section 4.7

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

16-January-1983

Page 5      Rev 4

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

DECmate II  Specification

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

2.8 Communications

DECmate II		DECmate I

Standard	Single line, async/	None

sync., byte and bit

protocols

Optional	None			Two lines, async/sync.

byte and bit protocols

I/O levels	EIA only		EIA only

The communications line  that  is  standard  on  the  DECmate  II

supports  both  bit  and byte oriented synchronous communications

protocols in addition to asynchronous operation.

3.0 Options

3.1 Graphics

An optional daughter board provides bitmap graphics capabilities.

Refer to the appropriate option description for full details:

Resolution	768 (H) x 240 (V)

# of planes	3 with a fourth optional

Grey scale      8 levels

Colour		R-G-B outputs  to  external  colour  monitor  via

colour  map,  16  colours  (with  optional fourth

plane).

Performance	Medium, programmed I/O device

3.2 Micro Winchester Disk System

A second option board allows connection  of  a  5.25"  Winchester

disk  drive, capacity either 5 or 10 MBytes. This drive is housed

in the system box, occupying the space normally reserved for  the

second RX50 floppy disk drive.

3.3 Z80 Auxiliary Processing Unit (APU)

A third option module allows a Z80 processor and memory system to

be  added  to  the DECmate II. Control code, executed by the 6120

processor, allows the Z80 to be selected as the active  processor

with the 6120 performing all the I/O operations for it.

3.4 RX01/02 Adaptor

Another option allows one or two RX01  or  RX02  8"  floppy  disk

drives  to be attached to DECmate II. This allows for simple file

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

16-January-1983

Page 6      Rev 4

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

DECmate II  Specification

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

interchange between existing 8"  diskettes  and  5.25"  diskettes.

This  option  card  and  the Micro Winchester Option are mutually

exclusive.

4.0 Detailed Description

4.1 CPU

The 6120 is a high speed, CMOS  processor  that  executes,  as  a

subset,   the   instruction   set  of  the  PDP  8/A.  Additional

instructions implement stack operations and provide  a  mechanism

for  using  the dual address space feature of the processor. Each

address space has  its  own  interrupt  mechanism,  allowing  the

single  CPU  to  be  used  to  execute  a  user program and, in a

completely  transparent  mode,  to  emulate  certain   peripheral

devices  -  the keyboard and display portions of the machine - to

allow program compatibility with older PDP 8 machines  where  the

terminal was a separate device connected via a serial line.

The emulation process is similar to that used with the DECmate  I

and  although  the  hardware is significantly different, software

compatibility is maintained.

4.2 Display

The DECmate I used a display buffer that could be switched to  be

a part of the CPU control memory address space when an update was

required. When the  CRT  controller  was  accessing  the  display

buffer  it  was  no longer a part of the CPU memory and there was

therefore no contention between the CRT controller and  the  CPU,

nor  any  CPU  bus overhead. In this way all the bandwidth of the

display buffer memory was available during the active portion  of

the  display.  The  limitation of this scheme is that the display

buffer is only available to the CPU for about 1.5 msec. per frame

(16.67  msec.).  This meant that display buffer updates had to be

performed  as  a  burst  operation,  during  this  1.5  msec.  In

addition, as this was a control memory operation, user interrupts

were locked out for the total duration of the update interval. It

was thus necessary to limit the update time to less-than  l-msec.

to  avoid  losing  data  coming in on the communications line, or

limit the communications line speed to something less  than  9600

baud.  The  result of these constraints was that the screen could

not be  changed  faster  than  an  effective  rate  of  9K  baud.

(effective  rate  compares  the  change to that of a conventional

terminal connected over a serial asynchronous line)

The DECmate II uses a  portion  of  the  control  memory  as  the

display  buffer memory together with a single row buffer which is

filled from control memory by DMA. The DMA filling process  is  a

burst operation and locks out the CPU for one line time, 64usec.,

every 640 usec. This is an overhead of about 10%, but distributed

over  the  entire frame rather than lumped as is the 1 msec. with

DECmate I. As the display buffer is available to the CPU  at  all

times  other  than  during the DMA cycles, the screen data may be

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

16-January-1983

Page 7      Rev 4

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

DECmate II  Specification

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

changed very quickly.

More memory space is available for the display  buffer  than  was

possible  with DECmate I, giving the ability to maintain multiple

display pages and switch between them very quickly or to use this

additional  space  to  implement  horizontal scrolling. The space

allocated for the display buffer allows for up to  three  display

pages  in 80 column mode and, subject to certain constraints, two

pages in 132 column mode.  An  added  advantage  of  the  display

buffer  structure  adopted  is  that  the  screen contents may be

preserved when changing from 80 column to  132  column  mode  and

vice versa.

4.2.1 Screen Format and Modes

The display has 24 rows of characters, selectable for 80  or  132

characters per row on a screen basis.

Characters may be  displayed  as  white  on  a  black  background

(Normal Mode) or black on a white background (Reverse Mode).

4.2.2 Character Attributes

For both formats and modes the following  attributes,  selectable

on a character basis are allowed :

Normal Screen Mode

BLINK - the selected character is  displayed  alternating

between two intensities, normal (as modified by attribute

BOLD) and lower than normal.

BOLD - the selected character is displayed  at  a  higher

than normal intensity.

UNDERLINE - the selected character has a continuous  line

added  below  the  lowest  character element for an upper

case character  or  through  the  first  element  of  the

descender for characters with descenders.

REVERSE -  the  selected  character  is  displayed  as  a

reverse  image,  i.e.  the  cell background is white, the

character information is black. This  attribute  modifies

the  action  of  attributes BLINK and BOLD such that they

operate  on  the  cell  background  rather  than  on  the

character  data,  e.g. REVERSE and BOLD results in a cell

background that is brighter than  normal,  the  character

itself remains black.

Reverse Screen Mode

BLINK - the cell background of the selected character  is

displayed alternating between two intensities, normal (as

modified by attribute BOLD) and lower than normal.

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

16-January-1983

Page 8      Rev 4

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

DECmate II  Specification

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

BOLD - the cell background of the selected  character  is

displayed at a higher than normal intensity.

UNDERLINE -  the  selected  character  has  a  continuous

(black) line added below the lowest character element for

an upper case character or through the first  element  of

the descender for characters with descenders.

REVERSE -  the  selected  character  is  displayed  as  a

reverse  image,  i.e.  the  cell background is black, the

character information is white.  This attribute  modifies

the action of attributes BLINK and BOLD in Reverse SCreen

Mode such that they operate on the character data  rather

than on the cell background e.g. REVERSE and BOLD results

in a character that is white and  brighter  than  normal,

the cell background being black.

4.2.3 Character Types

Character type may be selected on a row basis to be :

Normal - the character row is 80 or 132  characters  each

defined within a 10 dot x 10 dot cell as shown in Fig. 1

Double Width - the character row is 40 or  66  characters

each defined within a double width cell.

Double Height - the character row is 40 or 66  characters

each  defined  within  a  double  height and double width

cell, thus a double height  character  row  occupies  two

normal row positions.

Dot Position

(9)0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0

L 0      . . . . . . . . . . . . . . . . . . . . . .

i 1	 . * * * * * * * . . . * * * * * * * . . . *

n 2	 . * * * * * * * . . . * * * * * * * . . . *

e 3	 . * * * * * * * . . . * * * * * * * . . . *

4	 . * * * * * * * . . . * * * * * * * . . . *

N 5	 . * * * * * * * . . . * * * * * * * . . . *

u 6	 . * * * * * * * . . . * * * * * * * . . . *

m 7	 . * * * * * * * . . . * * * * * * * . . . *

b 8      # = = = = = = = # # # = = = = = = = # # # =

e 9	 . + + + + + + + . . . + + + + + + + . . . +

r 0	| |		    | |

| |<---Character----->|

| |       Cell      | |

|<-----Attribute--->|

|         Cell      |

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

16-January-1983

Page 9      Rev 4

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

DECmate II  Specification

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

* Upper case, lower case or Special Graphics

= Lower case, Underline or Special Graphics

+ Lower case or Special Graphics

. Special Graphics

FIGURE 1 - Character Cell Dot Placement

4.2.4 Scrolling

4.2.4.1 Types

Scrolling may be set to be Jump or Smooth.  In  Jump  scroll  the

display advances by a complete character row at a time, in Smooth

scroll it advances by one or more lines of a row at a time,  with

the default of two lines at a time. (every 16 msec.)

4.2.4.2 Regions

The normal mode of operation of the display is for the entire  24

rows  to be considered as a scrolling region. The display control

also allows the scroll region to be set to be only a part of  the

screen,  e.g.  rows 5 to 12 may be set to scroll with rows 1 to 4

and 13 to 24 remaining fixed.

4.2.5 Display Control

To control the features of the display portion  of  the  machine,

the following ESCAPE sequences are defined as the user interface.

The convention in the descriptions that follow is that the ESCAPE

sequence,  when  issued by the user, causes the effect indicated.

ESCAPE sequences generated  by  the  display  emulation  code  in

response  to  an  ESCAPE  sequence  received  from  the  user are

explicity noted.

4.2.5.1 CPR - Cursor Position Report

ESC [ Pv ; Ph R	033 133 ** 073 *** 122

Generated  by   the   display   emulation

firmware as a reply to the user generated

sequence  "DSR"  asking  for  a  position

report.

Pv	the  current  cursor  vertical  position,

expressed as a decimal string.

Ph	the current cursor  horizontal  position,

expressed as a decimal string.

A response with no parameters, or parameters  of  0;0,  indicates

that the cursor is at the HOME position.

4.2.5.2 CUU - Cursor Up

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

16-January-1983

Page 10     Rev 4

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

DECmate II  Specification

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

ESC [ Pn A 	033 133 ** 101

Moves the cursor up by the number of rows

specified by the parameter Pn, given as a

decimal  string.  No   parameter   or   a

parameter of 0 or 1 indicate a single row

move. If  an  attempt  is  made  to  move

cursor  above  the top margin, the cursor

is positioned  at  the  top  margin.  The

cursor   horizontal   position  does  not

change.

4.2.5.3 CUD - Cursor Down

ESC [ Pn B 	033 133 ** 102

Moves the cursor down by  the  number  of

rows specified by the parameter Pn, given

as a decimal string. No  parameter  or  a

parameter  of  0  or 1 indicates a single

row move. If an attempt is made  to  move

the  cursor  below the bottom margin, the

cursor  is  positioned  at   the   bottom

margin.  The  cursor  horizontal does not

change.

4.2.5.4 CUF - Cursor Forward

ESC [ Pn C 	033 133 ** 103

Moves the cursor right by the  number  of

columns  specified  by  the parameter Pn,

given as a decimal string.  No  parameter

or  a  parameter  of  0  or 1 indicates a

single column move. If an attempt is made

to  move  the  cursor  beyond  the  right

margin, the cursor is positioned  at  the

right   margin.   The   cursor   vertical

position does not change.

4.2.5.5 CUB - Cursor Backwards

ESC [ Pn D   	033 133 ** 104

Moves the cursor left by  the  number  of

columns  specified  by  the parameter Pn,

given as a decimal string.  No  parameter

or  a  parameter  of  0  or 1 indicates a

single column move. If an attempt is made

to   move  the  cursor  beyond  the  left

margin, the cursor is positioned  at  the

left margin. The cursor vertical position

does not change.

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

16-January-1983

Page 11     Rev 4

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

DECmate II  Specification

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

4.2.5.6 CUP - Cursor Position

ESC [ Pv ; Ph H	033 133 ** 073 *** 110

Moves  the   cursor   to   the   position

specified  by  the  parameters Pv and Ph.

Out of range parameters cause the  cursor

to be positioned at the respective screen

extremity. See "Origin Mode", 4.2.5.20

Pv - cursor vertical position,  expressed

as a decimal string.

Ph  -   cursor    horizontal    position,

expressed as a decimal string.

4.2.5.7 HVP - Horizontal and Vertical Position

ESC [ Pv ; Ph f	033 133 ** 073 *** 146

Same effect as CUP - see 4.2.5.6

4.2.5.8 DSR - Device Status Report

ESC [ 6 n 		033 133 66 156

A  request  to  the   display   emulation

firmware   to  report  back  the  current

cursor position using a CPR sequence, see

Section 4.2.5.1

4.2.5.9 IND - Index

ESC D 		033 104

Moves the cursor position down  one  row,

scrolling   if   necessary.   The  cursor

horizontal position is unchanged.

4.2.5.10 NEL - New Line

ESC E 		033 105

Moves the cursor to the first  column  of

the   next   row   down,   scrolling   if

necessary.

4.2.5.11 RI - Reverse Index

ESC M 		033 115

Moves the  cursor  up  one  row,  reverse

scrolling   if   necessary.   The  cursor

horizontal position is unchanged.

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

16-January-1983

Page 12     Rev 4

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

DECmate II  Specification

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

4.2.5.12 DA - Device Attributes

ESC [ Pn c		033 133 * 143

Pn - 0  or  not  present  is  used  as  a

sequence generated by the user program to

request  that   the   display   emulation

firmware    report    back   the   device

characteristics, using a DA  sequence  as

defined below.

ESC [ ? 61;1 c	033 133 077 066 061 073 061 143

Defines the  DECmate  II  as  a  Level  1

Terminal   with   132  column  extension.

Having been identified as such, the  user

program  may  request more information by

use of  the  Secondary  Device  Attribute

Request.

ESC [ > Pn c	033 133 076 * 143

Pn - 0 or not present is a  request  from

the user program for transmission back of

the Secondary Device Attribute  Response.

The  resposnse for a DECmate II operating

with its own internal  terminal  Firmware

is:

ESC [ > 3 c	033 133 076 63 143

4.2.5.13 ED - Erase in Display

ESC [ Pn J 	033 133 * 112

Pn  -  a  parameter  defining  the  erase

function.

Pn 		Erase Function

0 (060) or none    Erase from the current cursor position to

the end of the screen, inclusive.

1 (061) 		Erase from the start of the screen to the

current cursor position, inclusive.

2 (062) 		Erase the entire screen.

In all cases  the  cursor  position  is  unchanged  when  the  ED

function has been completed.

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

16-January-1983

Page 13     Rev 4

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

DECmate II  Specification

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

4.2.5.14 EL - Erase in Line

ESC [ Pn K		033 133 * 113

Pn  -  a  parameter  defining  the  erase

function.

Pn			Erase Function

0 (060) or none   Erase from the current cursor position to

the end of the current row, inclusive.

1 (061) 		Erase from the start of the  current  row

to the cursor, inclusive.

2 (062) 		Erase the entire current row.

"Current row" refers to that row on which the cursor is displayed

when the EL command is given. In all cases the cursor position is

unchanged when the EL function has been completed.

4.2.5.15 DECKPNM - Keypad Numeric Mode

ESC > 		033 076

In this mode  of  operation,  the  keypad

keys  transmit  codes  that correspond to

the symbols shown on the keys,  with  the

exception   of  PF1-4.  The  cursor  keys

transmit  ANSI  cursor  movement   ESCAPE

sequences - see 4.2.5.20

4.2.5.16 DECPAM - Keypad Application Mode

ESC = 		033 075

In this mode the codes transmitted by the

keypad keys change to the sequences shown

below.

Key		Keypad Numeric		Keypad Application

Mode			Mode

0		0	(060)		ESC O p (033 117 160)

1		1	(061)		ESC O q (033 117 161)

2		2	(062) 		ESC O r (033 117 162)

3		3	(063)		ESC O s (033 117 163)

4		4	(064)		ESC O t (033 117 164)

5		5	(065)		ESC O u (033 117 165)

6		6	(066)		ESC O v (033 117 166)

7		7 	(067)		ESC O w (033 117 167)

8		8	(070)		ESC O x (033 117 170)

9		9	(071)		ESC O y (033 117 171)

-		-	(055)		ESC O m (033-117 155)

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

16-January-1983

Page 14     Rev 4

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

DECmate II  Specification

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

,		,       (054)		ESC O 1 (033 117 154)

ENTER		RET     (015)		ESC O M (033 117 115)

PF1		ESC O P (033 117 120)   ESC O P (033 117 120)

PF2		ESC O Q (033 117 121)   ESC O Q (033 117 121)

PF3		ESC O R (033 117 122)   ESC O R (033 117 122)

PF4		ESC O S (033 117 123)   ESC O S (033 117 123)

4.2.5.17 SM - Set Mode

ESC [ PS ; Ps ; ..... h        033 133 * 073 * ..... 154

Set one or more operating parameters, specified by the  parameter

string  Ps....,  as  defined  in  Section  4.2.5.19.  DEC Private

parameters are specified by adding the character "?"  immediately

after the CSI introducer (ESC [).

i.e. the SM sequence for a DEC Private parameter is :

ESC [ ? ;Ps ; Ps ; ...... l

4.2.5.18 RM - Reset Mode

ESC [ Pr ; Pr ; ...... l       033 133 * 073 * 073 ... 150

Reset  one  or  more  operating  parameters,  specified  by   the

parameter  string  Pr....,  as  defined  in Section 4.2.5.19. DEC

Private parameters are specified  by  adding  the  character  "?"

immediately after the CSI introducer (ESC [).

i.e. the RM sequence for a DEC private parameter is :

ESC [ ? Pr ; Pr ; ..... l

4.2.5.19 ANSI Specified Parameters Applicable to SM and RM

Parameter		Function 		Mnemonic

2 (062)	SET - turns OFF the keyboard    KAM

RESET - enables the keyboard

4 (064)	SET-- selects insertion mode    IRM

RESET - selects replacment mode

12 (061 062)  SET - turns OFF local echo	SRM

RESET - turns ON local echo

20 (062 060)  SET - enables Newline mode	LNM

RESET - disable Newline mode

IRM		When  Insertion  Mode  is  set  to  Insert  (SM),

characters  sent  to  the  screen for display are

inserted  at  the  current  cursor  position  and

characters to the right of the cursor position

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

16-January-1983

Page 15     Rev 4

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

DECmate II  Specification

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

are pushed towards the  right  hand  end  of  the

current  row.  As  characters moved right in this

fashion move beyond the right  margin,  they  are

lost.  When  Insertion  Mode is set to Replace, a

character sent to the screen for display replaces

any character at the current cursor position.

SRM		When local echo is turned ON, characters typed on

the  keyboard  are  displayed  on  the screen and

transmitted to the application program. When this

mode  is  turned  OFF  (normal),  the application

program is responsible for echoing characters.

LNM		When this mode is  enabled  (SET),  pressing  the

RETURN  key  causes  both CR (015) and a LF (012)

codes to be passed to  the  application  program.

When  this mode is disabled (RESET), pressing the

RETURN  key  causes  a  CR  code   only   to   be

transmitted.

4.2.5.20 DEC Private Parameters Applicable to SM and RM

Parameter		Function 		Mnemonic

1 (061)	SET - enables Cursor Key Mode   DECCKM

RESET - disables "    "   "

2 (062)	SET - enables ANSI mode		DECANM

RESET - enables VT52 mode

3 (063)	SET - enables 132 column mode   DECCOLM

RESET - enables 80 column mode

4 (064)	SET - enables smooth scroll	DECSCLM

RESET - enables jump scroll

5 (065)	SET - gives white background	DECSCNM

RESET - gives black background

6 (066)	SET - enables Origin Mode	DECOM

RESET - disables Origin Mode

7 (067)      SET - enables auto-wrap 	DECAWM

RESET - disables auto-wrap

8 (070)	SET - keyboard keys auto-repeat DECARM

RESET - no auto-repeat

25 (062 065)  SET - the cursor is visible     DECCUR

RESET - the cursor is invisible

Cursor key mode - the  four  cursor  keys  may  be  selected  to

transmit  either  ANSI cursor key movement ESCAPE

sequences or Application ESCAPE sequences by this

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

16-January-1983

Page 16     Rev 4

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

DECmate II  Specification

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

SM/RM parameter if Keypad Application Mode is SET

-  see  4.2.5.16.  If  Keypad Application Mode is

SET, the  alternative  code  sequences  that  the

cursor keys can transmit are as follows :

Cursor Key 	Cursor Key Mode		Cursor Key Mode

RESET			SET

UP arrow	ESC [ A (033 133 101)   ESC O A (033 117 101)

DOWN arrow 	ESC [ B (033 133 102)   ESC O B (033 117 102)

RIGHT arrow	ESC [ C (033 133 103)   ESC O C (033 117 103)

LEFT arrow	ESC [ D (022 133 104)   ESC O D (033 117 104)

ANSI/VT52 Mode -If  ANSI  Mode  is  SET,  the  display  interface

responds  to  the  ANSI  set  of ESCAPE sequences

defined in Section 6.6

If ANSI  Mode  is  RESET,  i.e.  VT52  Mode,  the

display  interface  responds  to  the VT52 set of

ESCAPE sequences defined in Section 7.

Origin Mode -  	If this mode  is  RESET,  the  screen  origin  is

always  located  at  the upper left corner of the

screen and cursor addressing is absolute  wrt  to

screen  physical location. If Origin Mode is SET,

the screen origin is relative to the start of the

current scrolling region and the cursor cannot be

positioned outside of the scrolling region.  When

the  Mode  is changed, the cursor is repositioned

at the new origin.

Autowrap -	If this Mode is RESET,  characters  sent  to  the

display  when  the  cursor  is  positioned at the

rightmost column  of  a  row  overwrite  previous

data,  if  any, at that location. If this Mode is

SET, sending new characters at  this  point  will

cause an automatic NEWLINE to occur.

4.2.5.21 SCS - Select Character Set

The DECmate II  is  able  to  display  characters  and  character

graphics from data stored in a character generator ROM or in RAM.

The ROM provides the bit patterns for the  standard USASCII,  UK,

DEC  Multinational, Special Graphics and Katakana character sets.

The RAM may be loaded by the user to provide a  wide  variety  of

other  Foreign  Language  characters  or extensions to the normal

display capabilities, e.g. scientific symbols.

DECmate II can support both 7-bit and 8-bit codes for display. In

its  normal  mode  of  operation,  codes received for display are

assumed to be 7-bit and are masked to 7-bit prior to any  display

translation  occurring.  If  DEC 8-bit mode is selected, the full

8-bits of an input code have significance *.

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

16-January-1983

Page 17     Rev 4

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

DECmate II  Specification

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

In interpreting input codes  for  display,  there  are  two  code

groups  defined:-  GL, codes in the range 041 - 176 and GR, codes

in the range 241 - 376. At any time GL may be invoked to  be  any

one  of four sets G0, G1, G2 or G3 and GR to any one of G1, G2 or

G3. In turn G0 - G3 may be  designated  to  be  any  one  of  the

possible  character  reportoires. Character reportoires supported

by DECmate II at present are :

* Current firmware implements ONLY Terminal Level 1, any  attempt

to change to 8-bit display mode is ignored.

USACSII

LINE DRAWING (SPECIAL GRAPHICS)

DEC MULTINATIONAL SUPPLEMENTAL

These are the reportoires recommended for future use. UK  is  now

considered a part of DEC Multinational Supplemental.

UK

FRENCH

FRENCH-CANADIAN

DUTCH

GERMAN

These are included (together with  any  other  Foreign  Languages

that may be added) for compatibility with DECmate I and any other

software that uses the obsolete language mapping of the VTl00W.

JIS ROMAN

KATAKANA

These are extensions to the  DEC  8-bit  Multinational  character

representation (see DEC Std. 169)

ALTERNATE ROM A

ALTERNATE ROM SPECIAL GRAPHICS

USER DEFINED (RAM)

The  first  two  of  these  are  again  included  for   backwards

compatibility,  the  User  defined RAM is a new feature whose use

and designation is TBD.

For further details on the future use of character sets and those

to be supported in future DEC products, refer to DEC Std. 169.

Following self test, G0, G1 and G3 are set to USASCII and  G2  is

set  to  DEC  Multinational  Supplemental. The loadable character

generator RAM is cleared.

The G0 - G3 sets  may  be  designated  by  the  following  ESCape

sequences :

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

16-January-1983

Page 18     Rev 4

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

DECmate II  Specification

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

ESC ( Fx	033 050 Fx	Designate the G0 set

ESC ) Fx	033 051 Fx	Designate the G1 set

ESC * Fx	033 052 Fx	Designate the G2 set

ESC + Fx	033 053 Fx	Designate the G3 set

Where Fx is a parameter defining one of the  available  character

reportoires as follows :

Fx		Character reportoire

A (101)	  	UK set

B (102)		USASCII

I (111)		JIS KATAKANA

J (112)		JIS Roman Graphic

K (113)		German

R (122)		French

0 (060)		Special graphics

1 (061)		Alternate character ROM

2 (062)		Alternate character ROM, special graphics

3 (063)		French-Canadian

4 (064)		Dutch

< (074)		DEC 8-bit Multinational Supplemental

GL and GR may be invoked as G0 - G3 in  either  Single  Shift  or

Locking  Shift.  In  Single  Shift,  the  next  graphic character

received following the receipt of the Single Shift Code  will  be

displayed  using  the  newly  selected  character  reportoire, in

Locking Shift, all graphic characters received following  receipt

of  the  Locking  Shift code will be displayed using the selected

character reportoire.

Single Shift Invokation

SS2  -  ESC N   (033 116)       Invoke GL to G2

SS3  -  ESC O   (033 117)	Invoke GL to G3

There are no Single Shift invokation ESCape sequences for GR, nor

for  GL  to G0 or G1. In 8-bit mode GL may be invoked as a single

shift to G2 or G3 on receipt of the following :-

SS2		(216)		Invoke GL to G2

SS3 		(217)		Invoke GL to G3

Locking Shift Invokation

LS2  - ESC n    (033 156)	Invoke GL to G2

LS3  - ESC o    (033 157)	Invoke GL to G3

LS1R - ESC ~    (033 176)	Invoke GR to Gl

LS2R - ESC }    (033 175)	Invoke GR to G2

LS3R - ESC |    (033 174)	Invoke GR to G3

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

16-January-1983

Page 19     Rev 4

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

DECmate II  Specification

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

There is no ESCape sequence to Invoke GL to  G0  or  G1,  instead

this function is performed by Control Codes :

LS0  -  SI      (017) 		Invoke GL to G0

LS1  -  SO 	(016) 		Invoke GL to G1

Appendix A defines the standard character patterns.

4.2.5.22 SGR - Select Graphic Rendition

ESC [ Pn ; Pn ;..... m 	 	033 133 * 073 * 073 ... 155

Establishes  the  attributes  to  be   appended   to   subsequent

characters according to the parameter string Pn .., as follows :

Pn	Action

0 (060) All attributes OFF, normal character display

1 (061) Display at increased intensity

4 (064) Display with underline

5 (065) Display blinking

7 (067) Display as a negative (reverse) image

There are four display intensity levels for  a  given  brightness

setting,  these  are  0  or  no  display  and  1,  2,  3  - being

sequentially increasing  levels  of  intensity.  Combinations  of

attributes result in displays as defined below :

Pn	Character/Underscore	Background

0	Level 2			Level 0

1	Level 3			Level 0

4	Level 2	 * 		Level 0

5	Level 2/level 1		Level 0

1;4	Level 3	 * 		Level 0

1;5	Level 3/level 2  *	Level 0

4;5	Level 2/level 1  *	Level 0

1;4;5   Level 3/level 2		Level 0

7 	Level 0			Level 2

1;7 	Level 0			Level 3

4;7	Level 0			Level 2	 *

5;7	Level 0			Level 2/level 1

1;4;7   Level 0			Level 3	 *

1;5;7   Level 0			Level 3/level 2  *

4;5;7   Level 0			Level 2/level 1  *

1;4;5;7 Level 0			Level 3/level 2

* these display levels alternate at the character BLINK rate.

4.2.5.23 DECWDL - Double Width Row

ESC $ 6		033 043 066

This causes the  row  containing  the  cursor  to

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

16-January-1983

Page 20     Rev 4

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

DECmate II  Specification

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

become  a  double  width   character   row.   All

characters  to  the  right  of  the centre of the

screen are lost, the display becomes either 40 or

66 columns wide. The cursor position is unchanged

unless it was previously  to  the  right  of  the

screen  centre, in which case it is positioned at

the new right margin.

4.2.5.24 DECDHL - Double Height Row

ESC # 3		033 043 063

This causes the  row  containing  the  cursor  to

become  the  top  half of a double height, double

width character row.

ESC # 4		033 043 064

This causes the  row  containing  the  cursor  to

become the bottom half of a double height, double

width row.

These sequences must be used  in  pairs  and  the

same  character  data  must be placed in the same

positions on each of the  rows  to  form  correct

double height characters. An attempt to display a

standalone top half or bottom half  of  a  double

height   row  will  result  in  an  unpredictable

display. If the row was previously single  width,

all  characters to the right of the centre of the

screen are lost. The cursor position is unchanged

unless  it  was  previously  to  the right of the

screen centre, in which case it is positioned  at

the new right margin.

4.2.5.25 DECSWL - Single Width Row

ESC # 5		033 043 065

This causes the  row  containing  the  cursor  to

become  single  width,  single height. The cursor

position is unchanged, i.e.  it  remains  at  the

same column number.

4.2.5.26 DECSTBM - Set Top and Bottom Margins

ESC [ Pn ; Pm r 033 133 ** 073 ** 162

This sets the top and bottom  screen  margins  to

define   the  scrolling  region.  Parameters  are

expressed as decimal strings.

Pn - the number of the top row in  the  scrolling

region.

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

16-January-1983

Page 21     Rev 4

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

DECmate II  Specification

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

Pm  -  the  number  of  the  bottom  row  in  the

scrolling region.

The minimum scrolling region is two rows, the top row number must

be lower than the bottom, see Origin Mode - 4.2.5.20

4.2.5.27 HTS - Horizontal Tab Set

ESC H 		033 110

Set one horizontal stop at the current cursor position.

4.2.5.28 TBC - Tab Clear

ESC [ Pc g 	033 133 *** 147

Clear horizontal tab stop(s), defined by parameter Pc.

Pn	Action

0 (060) Clear  horizontal  tab  at  the  current   cursor

position, the default case with no parameter.

3 (063) Clear all horizontal tabs.

4.2.5.29 DECSC - Save Cursor (DEC Private)

ESC 7 		033 067

This sequence causes the current cursor position,

graphic  rendition and active character set to be

saved.

4.2.5.30 DECRC - Restore Cursor (DEC Private)

ESC 8 		033 070

This sequence causes the previously saved  cursor

position,  graphic rendition and character set to

be restored.

4.2.5.31 DECPRS - Print Screen (DEC Private)

ESC [ i 	033 133 151

This sequence is ignored.

4.2.5.32 DECLL - Load LEDS (DEC Private)

ESC [ Ps q 	This sequence has no effect on DECmate II.

4.2.5.33 DECALN - Screen Alignment Display (DEC Private)

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

16-January-1983

Page 22     Rev 4

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

DECmate II  Specification

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

ESC g 8 	033 043 070

This sequence fills the entire  screen  with  the

screen alignment pattern.

4.2.5.34 IL - Insert Line

ESC [ Pn L	033 133 *** 114

This sequence causes Pn (expressed as  a  decimal

string)  lines of blanks to be inserted below the

current line. Lines below the  current  line  are

scrolled down. This sequence has no effect if the

cursor is positioned outside the scrolling region.

4.2.5.35 DL - Delete Line

ESC [ Pn M 	033 133 *** 115

This sequence causes Pn (expressed as  a  decimal

string)  lines  to  be  deleted below the current

line. As lines are deleted, blank lines scroll in

from  the  botton. This sequence has no effect if

the cursor is  positioned  outside  the  scroiing

region.

4.2.5.36 DCH - Delete Character

ESC [ Pn P	033 133 *** 120

This sequence causes P characters  (expressed  as

a  decimal  string)  to te deleted, starting with

the character at the cursor position and deleting

towards  the right hand margin. As characters are

deleted, characters to the right  of  the  cursor

move to the left to fill the space created by the

deletion.   See  Set/Reset  Mode  for  Character

Insertion.

4.2.6 VT52 Mode ESCAPE sequences

When the DECmate II is placed into VT52 Mode - see  4.2.5.20  for

details  of  this  SM  parameter,  the  following  set  of ESCAPE

sequences become recognised, replacing those  listed  in  Section

4.2.5.*

4.2.6.1 Cursor Up

ESC A		The cursor is move up by one row. If  an  attempt

is  made  to move the cusor above the top margin,

it is positioned at the top  margin.  The  cursor

horizontal position does not change.

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

16-January-1983

Page 23      Rev 4

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

DECmate II  Specification

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

4.2.6.2 Cursor Down

ESC B 		The cursor is move down by one row. If an attempt

is  made  to  move  the  cursor  below the bottom

margin, it is positioned at  the  bottom  margin.

The cursor horizontal position does not change.

4.2.6.3 Cursor Right

ESC C		The cursor is moved one column to the  right.  If

an  attempt is made to move the cursor beyond the

right margin,  it  is  positioned  at  the  right

margin.  The  cursor  vertical  position does not

change.

4.2.6.4 Cursor Left

ESC D 		The cursor is moved one column to the left. If an

attempt is made to move the cursor byond the left

margin, it is positioned at the left margin.  The

cursor vertical position does not change.

4.2.6.5 Enter Graphics Mode

ESC F 		The  special  graphics  character  set   of   the

standard  character  ROM  are  selected - see the

ANSI SCS sequence ESC ( 0 - Section 4.2.5.20

4.2.6.6 Exit Graphics Mode

ESC G 		The character set reverts to the standard USASCII

-  see  ANSI  SCS  sequence  ESC  (  B  - Section

4.2.5.20

4.2.6.7 Cursor to Home

ESC H 		The cursor is positioned at HOME,  the  top  left

corner of the screen.

4.2.6.8 Reverse Line Feed

ESC I 		Moves the cursor up one row without changing  the

horizontal    position,   reverse   scrolling  if

necessary.

4.2.6.9 Erase to End of Screen

ESC J 		Erases all character positions from  the  current

cursor  position  to the end of screen inclusive.

The cursor position remains unchanged.

4.2.6.10 Erase to End of Line

ESC K 		Erases all character positions from  the  current

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

16-January-1983

Page 24     Rev 4

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

DECmate II  Specification

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

cursor position to the end of  the  current  row.

The cursor position remains unchanged.

4.2.6.11 Direct Cursor Address

ESC Y Pr Pc	The cursor is moved to the screen location  given

by  the  two  ASCII character codes following the

Direct  Cursor  Addressing   delimiter   (Y)   as

follows:

the row  and  column  parameters  are  formed  by

adding  037 to the actual row and column numbers,

and sending that  pair  of  ASCII  characters  so

formed  as  the next two characters following the

upper case Y, e.g.  to  position  the  cursor  at

screen  position  2;5  the sequence sent would be

ESC Y     !     $

(041=2 044=5)

4.2.6.12 Identify

ESC Z		This causes the  display  emulation  firmware  to

return  the  following identification sequence to

the user program :

ESC / Z

indicating  that  it  is  a  VT100  type   device

emulating a VT52.

4.2.6.13 Enter Alternate Keypad Mode

ESC = 		This causes the auxiliary keypad keys to transmit

ESCAPE   sequences   for   Application  Dependent

Purposes.

4.2.6.14 Exit Alternate Keypad Mode

ESC > 		This causes the auxiliary keypad to transmit  the

ASCII  codes  for  the  functions  or  characters

engraved on the keys.

4.2.6.15 Enter ANSI Mode

ESC < 		This causes the display emulation firmware to  go

into  ANSI  mode  and  interpret ESCAPE sequences

defined in Section 4.2.5.* (VT100 Mode).

4.3 Keyboard

The LK2XX keyboard implements the functions of keyboard scanning,

debouncing,  autorepeat  etc.  and  transmits  key  codes  to the

DECmate II to indicate keydown conditions. Following decoding  in

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

16-January-1983

Page 25     Rev 4

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

DECmate II  Specification

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

Control Panel Memory, the resulting keyboard  character  code  is

passed  to the user program. Data transmission to the keyboard is

provided to allow certain keyboard parameters to be set up,  e.g.

keyclick  loudness. The following list of instructions is for use

by the keyboard handling code in Panel Memory  and  not  for  the

user.  The  characteristics  of this interface are fixed with the

following parameters :

Speed 		4800 baud 	Character	8 data bits

4.3.1 Instruction List

Instruction	Description

6050		Set keyboard input flag.

6051	  	Skip on keyboard input flag, clear it if set.

6052		Clear the AC.

6053		NOP

6054		Inclusive OR keyboard input data  with  AC<4:11>,

result to AC<4:11>, AC<0:3> undefined.

6055		Set/clear keyboard input  interrupt  enable  with

AC<ll>. AC unchanged following this instruction.

6056	 	Clear  AC,  then  read  keyboard  input  data  to

AC<4:11>, AC<0:3> undefined.

6057		NOP

6110		Set keyboard output flag.

6111		Skip on keyboard output flag, clear it if set.

6112		NOP

6113		NOP

6114		AC<4:11> to the keyboard, AC unchanged.

6115		Set/clear keyboard output flag  interrupt  enable

with   AC<ll>.   AC   unchanged   following  this

instruction.

6116		AC<4:11> to the keyboard, AC cleared.

6117		NOP

4.4 Disk Interface

The disk storage integral to the DECmate  II  is  a  single  RX50

floppy  disk  drive  with a second RX50 optional, see Section 2.3

for capacity details.

The disk may be used in either 8 or 12 bit modes. In  both  modes

the  disk  format  is  the  same,  the  differences  are  in  the

utilization of the available capacity  and  the  format  of  data

transfer  between  the  user  program and the disk controller. In

8-bit mode the transfers are as 8-bit bytes to/from AC<4:ll>  and

the full disk capacity is available. In 12-bit mode the transfers

are full word transfers with an  effective  sector  size  of  256

12-bit words, the disk controller still reads and writes 512 byte

sectors, reformatting the 12-bit data as it is received from  the

user and creating 12-bit words prior to transferring read data to

the user. The unused bits in each sector are discarded.

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

16-January-1983

Page 26     Rev 4

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

DECmate II  Specification

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

Instruction     Function

SEL - 6750 	Select a disk pair.

If the RX02 Adaptor is not present (see section 5.4)

AC 11

0   Select RX50 drive pair A

1   Select RX50 drive pair B

If the RX02 Adaptor is present (see section 5.4)

AC 00   11

0   0/1	Select RX50 drive pair A/B

1    0	Select RX02 drive pair A/B

(See RX02 adaptor description for

details)

The  first  pair  of  RX50  drives  (pair  A)  is

selected  by  CAF and following self test. The AC

is unchanged by this IOT.

LCD - 6751	Load the Command Register.

This instruction must only be issued when DONE is

true.  The  contents of the AC are transferred to

the disk controller to be used as a command.  The

AC is cleared following this instruction.

AC(5)=0 selects 12-bit mode of operation.

AC (5)=1 selects 8-bit mode of operation.

12-bit  mode  is  selected  by  CAF,  RXINIT  and

following self test.

AC(7)=0 selects the left drive (drive 0 - RX50).

AC(7)=1 selects the right drive (drive 1 - RX50).

AC<8:10> define the command to be executed by the

disk controller as follows :

Command	        	AC 8  9  10

Read Sector		   0  1   1

Write Sector		   0  1   0

Fill Buffer		   0  0   0

Empty Buffer		   0  0   1

Read Status		   1  0   1

Read Error Code		   1  1   1

Write Sector with

Deleted Data mark	   1  1   0

No Operation 		   1  0   0

AC<O:4, 6, 11> have no effect.

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

16-January-1983

Page 27     Rev 4

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

DECmate II  Specification

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

XDR - 6752	Transfer Data.

This IOT performs data interchange between the AC

and  the  disk controller, the transfer direction

is controlled by the disk controller depending on

the  command  being  executed  which controls the

data direction line OUT, and  the  state  of  the

DONE interface signal.

1. DONE false and OUT Low (data from disk)

12-bit  Mode  :  the  next  word  of  data  is

transferred   from   the  disk  controller  to

AC<0:11>, the  AC  is  cleared  prior  to  the

transfer.

8-bit  Mode  :  the  next  byte  of  data   is

transferred   from  the  disk  controller,  an

inclusive OR is performed  with  AC<4:11>  and

the  result  placed  in  AC<4:11>. AC<0:3> are

unchanged.

2. DONE false and OUT High (transfer to disk)

12-bit Mode : AC<0:11> are transferred to  the

disk   controller.   The   AC   is  unchanged.

8-bit Mode : AC<4:11> are transferred  to  the

disk controller. The AC is unchanged.

3. DONE true

This IOT now performs a  transfer  to  the  AC

from   the   disk  controller.  8/12-bit  Mode

determines whether an 8  or  12  bit  transfer

occurs.

STR - 6753	Skip on Transfer Request transition.

If  the  Transfer  Request  interface  line   has

changed  from  false  to  true since this IOT was

last issued, the next sequential instruction will

be skipped. If the skip does occur the transition

detector will be reset. The AC  is  unchanged  by

this IOT.

SER - 6754	Skip on Error transition.

If the Error  interface  line  has  changed  from

false to true since this IOT was last issued, the

next sequential instruction will be  skipped.  If

the skip does occur, the transition detector will

be reset. The AC is unchanged by this IOT.

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

16-January-1983

Page 28     Rev 4

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

DECmate II  Specification

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

SDN - 6755	Skip on Done transition.

If the Done interface line has changed from false

to  true since this IOT was last issued, the next

sequential instruction will be  skipped.  If  the

skip  does occur, the transition detector will be

reset. The AC is unchanged by this IOT.

INTR - 6756	Set/clear Interrupt Enable.

If AC(11)=0, the Interrupt Enable  for  the  DONE

transition detector to cause an interrupt will be

cleared. If AC(ll)=l, it will be set. The  AC  is

unchanged by this IOT.

Power-on and CAF clear the Interrupt Enable.

RXINIT - 6757   Initialize.

The following are cleared :

Done transition flag

Transfer Request transition flag

Error transition flag

Done Interrupt Enable

Drive pair select is set to Unit A.

All drives are caused  to  initialize,  the  disk

controller is reset.

At the end  of  drive  initialization,  the  Done

transition   flag   will   be   set  to  indicate

completion.

4.5 Printer

The  printer  interface  is  a  standard,  full  duplex,   serial

asynchronous  port. The receive capability is provided to support

printers that use an XON-XOFF protocol to signal  when  they  are

able/unable to accept new data from the DECmate II.

Instruction	Description

6320		Set printer port input flag

6321		Skip on printer port input flag, clear it if set

6322		Clear the AC

6323		NOP

6324		Inclusive OR the printer  input  port  data  with

AC<4:11>,  place  the result in AC<4:11>. AC<0:3>

are unchanged.

6325		Set/clear printer  port  input  interrupt  enable

with AC(ll).

6326		Clear the AC then transfer the printer port input

data to AC<4:11>.

6327		NOP

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

16-January-1983

Page 29     Rev 4

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

DECmate II  Specification

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

6330		Set printer port output flag

6331		Skip on printer port output  flag,  clear  it  if

set.

6332		NOP

6333		Set the printer port baud rate for both input and

output as follows:

AC<8:11>	Baud Rate       AC<8:11>	Baud Rate

0 	    	    50	  	  10		  1800

1		    75	  	  11	 	  2000

2		   110		  12		  2400

3		   134.5	  13		  3600

4		   150		  14		  4800

5		   300		  15		  7200

6		   600		  16		  9600

7		  1200		  17		 19200

6334		Transfer AC<4:11> to the printer, leaving the  AC

unchanged.

6335		Set/clear interrupt enable for the  printer  port

output with AC(11). The AC is unchanged.

6336		Transfer AC<4:11> to the printer, then clear  the

AC.

6337		NOP

4.6 Communications

The communications controller  provides  a  single,  full  duplex

serial port  for either asynchronous or synchronous operation. In

the synchronous mode of operation either  bit  or  byte  oriented

protocols may be selected. Full modem control is provided.

4.6.1 Instruction List

Three device codes are used to control the  communications  port,

30,  31  and  36. 30 is associated with transferring receive data

from the communications controller chip to the AC, 31 with moving

data   from   the   AC   to  the  communications  controller  for

transmission and 36 for modem control functions.

The communications controller chip has several internal registers

which  must be initialised prior to using the communications port

in a particular mode. Following self-test, the  port  is  set  to

asynchronous mode, with parameters as shown below:

Baud Rate       -	1200

Data elemenets  -	8

Stop bit(s)     -	1

Parity	   	-	None

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

16-January-1983

Page 30     Rev 4

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

DECmate II  Specification

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

Instruction     Description

6360 	Set Modem Change Flag. 		Note 1.

6361 	Skip on Modem Change Flag, clear it if set.

6362 	Load Modem Control Register from AC<4,7:11>

AC Bit  Function 			Circuit #	Note 2

04	Enable Terminal Timing		103/116

05	Select Internal/External Timing 		Note 3

07	Set Request to Send		105

08	Set Data Terminal Ready		108.2

09	SR				111

10	Enable Local Loopback		141

11	Enable Remote Loopback

6363		Set Baud Rate from AC<4:11> as shown  in  4.5  in

the printer description (6333 definition)

6364		Read Modem Status to AC<7:11>

AC Bit  Modem Line			Circuit #. Note 2

07

08 					125

09 					109

10 					107

11 					106

6365		Set/clear Modem Change Flag Interrupt Enable with

AC<ll>. The AC is unchanged by this instruction.

6366		Access   Communications    Controller    Internal

Register

Following power-on or after a RESET,  this  instruction  is  used

together  with  the  Data Field to write the communicatiions chip

Control Register R0 and in so doing may specify another  internal

register  for  access,  Control  Registers <0:7>, which are write

only and Status Registers <0:1> which are  read  only.  Whichever

register is specified will be the register accessed the next time

that  this  instruction  is  issued,  following  access  to   the

specified register, access reverts to Control Register R0.

Note 1. This flag is set if any one of the following signals changes

state :

Signal		Circuit #

Carrier 	109

Ring 		125

Data Set Ready  107

Clear to Send   106

2. Circuit numbers refer to CCITT circuits.

3. Internal timing must always be selected for asynchronous

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

16-January-1983

Page 31     Rev 4

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

DECmate II  Specification

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

operation, send and receive baud rates are controlled by the  IOT

6363, as described above.

4.6.2 Internal Registers

4.6.2.1 Control Register R0

This register has three fields, used as follows :

Field A - Register Pointer.

AC<9:11>  specify  the  register  to  be  accessed  on  the  next

execution  of  this  instruction.  If the register specified is a

WRITE register, AC<00> must be set to 0 for this instruction.  If

the register is a READ register, AC<00> must be set to 1 for this

instruction.

Field B - Command

AC<6:8> specify one of a set of  commonly  used  commands  to  be

executed :

AC<6:8>		Command

000		No operation

001	  	Send ABORT - used when operating  in  SDLC  mode,

causes  the  SDLC  ABORT  code to be transmitted,

destroying any data  currently  in  the  transmit

buffer.  Following transmission of the ABORT, the

transmitter reverts to the IDLE phase.

010	 	Unused

011		Communications  Port  Reset.  The  communications

controller and Modem control lines are set to the

same initial state as following power-on.

100		Enable  Interrupt   on   next   character.   When

operating   in   Interrupt   on   First  Received

Character Mode,  this  command  may  be  used  to

re-enable  the  interrupt  for  the next received

character (generally at the end of amessage)

101		Reset Pending Transmitter Interrupt.  Clears  the

interrupt  condition  caused  by  the transmitter

buffer becoming empty.

110		Error Reset, used to reset an interrupt caused by

detection of a parity error or overrun error.

111		End  of  Interrupt,  resets  the   communications

controller  chip  internal  priority  network  to

allow for a lower priority interrupt.

Field C - CRC control commands

AC<4:5> specify the operating mode of the CRC generator/checker.

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

16-January-1983

Page 32     Rev 4

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

DECmate II  Specification

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

AC<4:5>		CRC Mode

0 0		No operation

0 1		Reset CRC receiver check logic

1 0		Reset CRC transmitter generator logic

1 1		Reset Idle/CRC latch, allows transmission of  the

CRC   field   when  transmitter  underrun  occurs,

followed by idle characters.

4.6.2.2 Control Register R1

This register is loaded from the AC as follows :

AC <4:6>	0

<7:8>

0 0		Disable Receive Data interrupts

0 1		Interrupt  only  on  the  first   data  character

received following this instruction

1 0		Interrupt on all data characters received and  on

errors

1 1		Interrupt on received data characters

AC <9>		0

AC <10>		Enable interrupts on  transmit  buffer  empty  or

when the transmitter enters the Idle phase.

AC <11>		Enable interrupts on the following :

SDLC ABORT detection or termination

Idle/CRC latch ebcoming set (CRC being sent)

Entering or leaving synchronous Hunt Phase break

detection or termination

4.6.2.3 Control Register 2

This register has no relevant bits for DECmate II

operation  of the communications controller chip,

if it is ever written, it must  be  written  with

all zeroes.

4.6.2.4 Control Register 3

Loaded from AC<4:11> as follows :

AC <4:5>	Select data bits per character for receive data

0 0		5 bits

0 1		7 bits

1 0		6 bits

1 1		8 bits

AC <6>		0

AC <7>		Enter Sync Hunt Phase

AC <8>		Enable receiver CRC. Used in  character  oriented

protocols    to   selectively   exclude   certain

characters from the CRC calculation.  There is  a

one  character  delay between a receive character

becoming available  for  reading  and  its  being

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

16-January-1983

Page 33     Rev 4

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

DECmate II  Specification

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

presented to  the  CRC  logic,  allowing  a  full

character time to decide whether to exclude it or

not.

AC <9>		Enable Address Search Mode. In SDLC Mode, setting

this  bit  inhibits  character assembly until the

8-bit   character   (secondary   address   field)

following  the starting flag of a message matches

either  the  address  programmed   into   Control

Register R6 or the global address 11111111.

AC <10>		Sync Character Load Inhibit. In synchronous mode,

setting  this  bit  prevents sync characters from

being loaded into the receiver buffer.

AC <11>		Receiver Enable, starts  the  receiver  following

all initialization.

4.6.2.5 Control Register 4

Loaded from AC<4:11> as follows :

AC <4>		0

AC <5>		1

AC <6:7>	Select operating mode for synchronous operation

0 0		8-bit    internal    synchronisation    character

(MONOSYNC)

0 1		16-bit   internal    synchronisation    character

(BISYNC)

1 0		SDLC

1 1		Illegal

AC <8:9>	Select synchronous/asynchronous mode

0 0		Synchronous mode, see AC <6:7>

0 1		Asynchronous mode, 1 stop bit

1 0		Asynchronous mode, 1.5 stop bits

1 1		Asynchronous mode, 2 stop bits

Note: the  above  stop  selections  are  for  the

transmitter  only, the receiver always checks for

a single stop bit.

AC <10:11>	Parity select

0 0		No parity check or generate.

O 1	 	 "    "     "    "    "

1 0		Check/generate odd parity

1 1		Check/generate even parity

4.6.2.6 Control Register 5

loaded from AC <4:11> as follows :

AC <4>		0

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

16-January-1983

Page 34     Rev 4

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

DECmate II  Specification

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

AC <5:6>	Select transmit data bits/character

0 0		5 bits or less - see Table II

0 1		7 bits

1 0		6 bits

1 1		8 bits

For 5 bits per character or less, the  format  of

the  data  loaded into the transmitter determines

the actual data word length as below :

AC 4 5 6 7 8 9 10 11	Data bits/character

1 1 1 1 0 0  0  D		1

1 1 1 0 0 0  D  D		2

1 1 0 0 0 D  D  D 		3

1 0 0 0 D D  D  D		4

0 0 0 D D D  D  D 		5

AC <7>		Set the transmit data line to the spacing  state,

used to send the BREAK condition.

AC <8>		Enable   the   transmitter,   set    after    all

initialization has been completed.

AC <9>		Select CRC polynomial

16    12    5

0		CRC-CCITT (x   + x   + x  + 1)

16    15    2

1		CRC-16 (X   + x   + x  + 1)

AC <10>		0

AC <11>		Enable/disable CRC  generation  on  the  transmit

data stream.

4.6.2.7 Control Register 6

loaded from AC <4:11> with SYNC byte 1, used as follows :

MONOSYNC	the 8-bit SYNC character that is  transmitted  in

the Idle phase

BISYNC		the least  significant  (first)  8  bits  of  the

16-bit   receive   and  transmit  synchronisation

field.

SDLC		the  secondary  address  value  matched  to   the

Secondary Address Field of the SDLC frame when in

Addresss Search Mode.

4.6.2.8 Control register 7

loaded from AC <4:11> with SYNC byte 2, used as follows :

MONOSYNC	8-bit SYNC character matched by the-receiver

BISYNC		the most  significant  (second)  8  bits  of  the

16-bit   receive   and  transmit  synchronisation

field.

SDLC		the AC value must be set to 01111110.

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

16-January-1983

Page 35     Rev 4

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

DECmate II  Specification

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

4.6.2.9 Status Register 0

The following information is loaded to AC <4:11> :

AC <4>		If operating in asynchronous mode, indicates that

a  BREAK  has  been  received.  If  operating  in

synchronous SDLC mode, that an ABORT sequence has

been received.

AC <5>		Indicates the state of the Idle/CRC latch used in

synchronous modes.

AC <6)		0

AC <7>		If operating in asynchronous mode - 0

If operating in synchronous mode a zero indiactes

that the receiver is in the receive data phase, a

one indicates the sync hunt phase.

AC <8>		0

AC <9>		Transmit buffer empty

AC <10>		Interrupt pending

AC <11>		Received character avalilable

AC <0:3 >	Undefined

4.6.2.10 Status Register 1

the following information is loaded into AC <4:11> :

AC <4> 		End of  SDLC  Frame;  when  in  SDLC  Mode,  this

indicates  that  the  End  OF Frame flag has been

received and that the CRC error flag and  residue

code  are  valid, This flag is reset on the first

character of the next message frame.

AC <5> 		In  asynchronous  mode,  this  flag  is  set   to

indicate  that  a framing error has been detected

on the received data. In synchronous modes,  this

bit indicates the current result of comparsion of

the computed CRC value and the appropriate  check

value. It is usually set to a one.

AC <6>		A receive overrun has occurred.

AC <7>		Parity check is  enabled  and  a  receive  parity

error has been detected.

AC <8:10> 	SDLC Residue Code. The data portion  of  an  SDLC

message   is   terminated  by  an  End  of  Frame

character. It may consist of any number  of  bits,

not  necessarily an integral number of characters,

thus  when  the  End  of   Frame   condition   is

indicated,  these  three  bits indicate where the

data field terminated.

AC <11> 	When operating in  asynchronous  mode,  this  bit

indicates  when  the  transmitter  buffer and the

output  shift  register  are   both   empty.   In

synchronous mode, it is always set to a one.

AC <0:3> 	Undefined

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

16-January-1983

Page 36     Rev 4

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

DECmate II  Specification

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

4.6.3 Data Transfer

4.6.3.1 Input

6300		Set this device code flag

6301		Skip on any  communications  flag  condition  and

clear the flag, if set. Flag conditions are :

Transmit buffer empty

Receive character available

Errors; parity, overrun, framing

Special synchronous conditions, if enabled

6302		NOP

6303		NOP

6304		Transfer the contents of the receive buffer to AC

<4:11>

6305		Enable interrupts for this device code, using  AC

<11>

6306		Transfer the contents of the receive buffer to AC

<4:11>

6307		NOP

4.6.3.2 Output

6310		Set this (dummy) flag

6311		Skip on dummy flag

6312		NOP

6313		NOP

6314		Transfer AC <4:11> to the transmit buffer

6315		Enable interrupts for this device code, using  AC

<11>

6316		Transfer AC <4:11> to the transmit buffer

6317		NOP

As can be seen from the set of  instructions  given  above,  this

communications   controller  has  a  quite  different  method  of

operation from that of the DP278. An example of how to initialize

and service this device for the asynchronous mode of operation is

given in Appendix C. For more details, refer to the DEC  Purchase

Specification for the controller chip (7201).

4.7 Clock

The clock provides a periodic interrupt at a 100  Hz.  rate.  The

clock  frequency  source  is  crystal controlled, thus cumulative

errors should be expected. As the basic accuracy of  the  crystal

is  0.01%,  these  errors  should not account to greater than +-3

seconds in an 8 hour period.

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

16-January-1983

Page 37     Rev 4

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

DECmate II  Specification

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

Instruction	Description

6130		Set clock flag.

6131		Skip on clock flag, clear it if set.

6132		NOP

6133		NOP

6134		NOP

6135		Set/clear clock interrupt enable with AC(ll).

6136		NOP

6137		NOP

The clock interrupt enable is cleared following self test.

Note that the clock instruction set is different from that of the

DECmate I and the VT78.

5. Detailed Description - Options

5.1 Identification

The LAS instruction allows a user to determine which options  are

present on DECmate II :

AC

08	0	-	APU present

1	-	No APU present

09	0	-	RX78 or RD51 controller present*

1	-	No additional storage adaptor

present

10	0	-	Graphics Controller present

1	-	No graphics Controller present

AC<00:07>, <11> -	undefined

* Which of these is present may be tested  by  issuing  the  RD51

controller instruction 6707 with the AC set non-zero. If the RD51

controller is present, the AC will be cleared.

5.1.1 Z80 Softcard

A daughter module may be added to the basic single board computer

module  to  allow the execution of applications written for a Z80

microprocessor. This added module has a Z80  processor  with  its

own 64 K bytes of memory and an interface to the 6120 CPU system.

The 6120  performs  all  the  actual  I/0  operations  for  disk,

keyboard, screen, printer and communications.

A sequence  (TBD)  will  allow  operation  to  be  switched  from

executing  applications programs using the PDP-8 CPU to using the

Z80 CPU, supporting CP/M(TM) file structure.

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

16-January-1983

Page 38     Rev 4

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

DECmate II  Specification

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

For more details refer  to  the  Z80  Auxiliary  Processing  Unit

Specification.

5.2 Graphics

Refer to the Option Engineering Specification.

5.3 Micro Winchester Disk Adaptor

Refer to the Option Engineering Specification.

5.4 RX01/RX02 Adaptor

This daughter module may be added to the base machine in place of

the  Micro Winchester Adaptor to allow one or two RX01 or RX02 8"

floppy disk units to be attached to the DECmate II. An  extension

to  the  SEL IOT allows the external 8" disk(s) to be selected in

place of the internal 5.25" disk(s) - see Section  4.4.  In  this

way  the  same IOTs may be used to control either the internal or

external disk unit(s).

The adaptor has a 37-pin subminiature D  connector  allowing  the

same  cable  to  be  used  to  connect to one or two RX01/02 disk

drives as is used with DECmate I.

The selection of the external adaptor using the SEL IOT has  some

limitations  in  the selection of the subsequently active pair of

RX01/02 drives.  As  the  external  adaptor  cannot  perform  any

operations  until  after  it  has been selected, the SEL IOT that

changes disk selection from one  of  the  internal  units  to  an

external unit cannot also change which pair of external drives is

active. Drive pair  "A"  of  the  external  drive  controller  is

selected  on  power-on  or  following  a  CAF/RXINIT instruction.

Subsequently issuing a SEL  IOT  with  AC=400X  will  select  the

external  drive  pair  "A"  regardless of AC<ll>. To select drive

pair "B",  a  second  SEL  must  be  issued  with  AC=4001.  When

switching back to the internal drives, the setting of AC<ll> will

determine  which  of  the  external  drive  pairs  will  be  left

"selected". This "selected" drive pair will be the pair activated

when the next SEL with AC=400X is issued.

6. Firmware Details

In general, the information given in this section, relates to how

Control  Panel Code, invisible to the user, accesses the hardware

to perform some function requested by  a  user  generated  Escape

Sequence.

The section on the User Defined Character Set which  is  included

gives  information on how a user may directly modify the contents

of the User Defined Character Generator Memory.

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

16-January-1983

Page 39     Rev 4

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

DECmate II  Specification

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

6.1 CRTC and Associated Logic Programming

The CRT Controller for the VT 278 (DECmate  I)  occupied  control

panel  memory  address space. As all control panel memory is used

with DECmate II, the CRT registers are now programmed by IOTs. It

is  necessary to set a register number with one IOT and then read

or write this register with another IOT. Note that registers  are

pre-defined  as  READ  or WRITE, thus two IOTs are needed, one to

Read a specified register and the other to Write. Issuing a  Read

IOT  to a WRITE register will give unpredictable results, issuing

a Write IOT to a READ register will result in  a  bus  contention

situation, again with unpredictable results.

It is also necessary  to  be  able  to  establish  the  following

parameters :

80/132 column mode 	        - Section 6.1.1

Video Enable 	        	- Section 6.1.2

Screen mode, normal or reverse  - Section 6.1.3

Cursor mode, block or underline - Section 6.1.4

Erase control bits 0 and 1 	- Section 6.1.5

Extended Character Set Mode     - Section 6.1.6

User Character Set Mode 	- Section 6.1.7

IOT list - Device Code 12

6121	Skip on End of Frame Interrupt, clear the flag if set and

the skip occurs.

6122	Load the CRT Controller register  number  from  AC<6:11>.

Load  the  cursor visibility enable with AC<01>, load the

cursor visibility disable with AC<00>. - Section

6123	If the DF is odd:

Load   User   Character   RAM   Character   number   from

AC<1:7>, Line number from AC<8:11>.

If the DF is even:

Load User Character RAM Data from AC<4:11> - Section

6124	Write selected CRT controller  register  from  AC<4:11> -

Section

6126	Load control register

AC  00  - Erase bit 0

01  - Erase bit 1

04  - 80/132 column mode (0 = 80 column)

05  - Video Enable (0 = disable)

08  - Extended Character Set Mode

09  - User Character Set Mode

10  - Cursor mode:  0 = block

1 = underline

11 - Screen Mode:   0 = white on black

1 = black on white

6127	Read selected CRT controller register, used to clear  End

of Frame Interrupt.

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

16-January-1983

Page 40     Rev 4

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

DECmate II  Specification

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

6.1.1 80/132 Column Mode

This bit is changed when the DEC Private ESCape Sequence  ESC [ 3

l/h is received, the SM final character (Lower Case "H") sets 132

column mode, the RM final character  (Lower  Case  "L")  sets  80

column  mode.  Changing  column  mode  leaves  the display in the

following condition :

Screen cleared - all memory locations in the visible  region  set

to contain 0000.

All display rows-set to single-width, single height.

Origin Mode cleared.

Scroll region set to 24 rows (default).

Scroll mode unchanged.

Cursor placed at the HOME position.

Cursor Mode unchanged.

Screen Mode unchanged.

Character sets for GO and G1 unchanged.

Character attributes cleared.

6.1.2 Video Enable

This bit is used to blank the  display  and  to  allow  the  User

Character Set RAM to be altered - see Section 6.1.7

6.1.3 Screen Mode

Changing Screen Mode (white characters on a black ground  =  mode

cleared,  black  characters on a white ground = mode set), alters

no other display parameters.

6.1.4 Cursor Mode

Changing cursor mode (block  cursor  =  mode  cleared,  underline

cursor = mode set), alters no other display parameters.

6.1.5 Erase Control

To minimize CPU overhead,  certain  long  duration  screen  erase

functions are executed by hardware, these are listed below :

EL0 - Erase in Line from current cursor position to end of line

EL1 - Erase in Line, entire line

ES0 - Erase in Screen from current cursor position  to  end  of

screen

ES1 - Erase in Screen, entire screen

The  particular  erase  function  is  initiated  by   loading   a

combination of Erase Bits 0 and 1 :

ERBIT 0		ERBIT 1

EL0, ES1 *	  1		  0

ES0, ES1 * 	  1		  1

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

16-January-1983

Page 41     Rev 4

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

DECmate II  Specification

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

*  for these functions, the  display  firmware  is  responsible  for

setting  the  cursor  to the left margin before issuing the erase

function request and then restoring the cursor when  the  display

hardware indicates that the erase is complete.

Erase functions are synchronised with the display timing  by  the

Control  Panel  code waiting for an End of Frame interrupt before

initiating an erase function and then waiting for the next End of

Erame interrupt to indicate that the erase has been completed.

6.1.6 Extended Character Set Mode

DECmate I has two 2K x 8 character generator ROMs each  of  which

allows  for  the display of up to 128 characters/symbols. One has

the US/UK character set plus the  special  graphics  set  of  the

VT100,  the  other  all  the  characters for the Foreign Language

DECmate  I  variations  plus  some  additional  special  graphics

symbols.  These  two  ROMs  together  provide the basic DEC 8-bit

multinational character set. (See DEC Std. 138). DECmate II  adds

to   this  the  ability  to  display  the  Katakana  set  of  the

multinational character set and a  User  Defined  Character  Set.

These  additional  characters/symbols  are accessed using a ninth

character code bit.

To be able to display  more  than  256  characters,  an  Extended

Character  Set  Mode is provided for DECmate II. In this mode the

internal representation of characters in the  display  buffer  is

nine  bits,  rather  than  the  eight  bits  of  DECmate  I.  The

additional bit is obtained by deleting  the  character  attribute

"BLINK".  Section  6.1.7  gives details of this mode and the User

Character Set Mode.

6.1.7 User Character Set Mode

To provide even greater flexibilty for displaying characters  and

symbols  that  may be needed by a particular user program, a User

Defined Character Generator is provided  with  DECmate  II.  This

allows  a  further  128  characters,  defined  by the user, to be

displayed. Display is controlled by the User Character  Set  Mode

Enable bit, set on a page basis.

6.1.7.1 Loading the User Defined Character Generator (UDCG)

To enable writing the UDCG the Video Enable Bit must  be  cleared

in  the  display  control word (AC<5>). This is done by issuing a

PR3 Panel Call with the AC zero, the UDCG may  then  be  changed.

The  entire  UDCG  may be altered or only specific locations, the

order of changing the UDCG does not  matter.  Programming  is  as

follows:

A. Clear the AC, issue PR3 with the  following  location  containing

(TBD)  -  the  function  number  of  the  Panel memory routine to

disable the display and enable changing the UDCG.

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

16-January-1983

Page 42     Rev 4

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

DECmate II  Specification

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

B. Load AC<1:7> with the 7-bit character code for the  character  to

be  altered  and AC<8:11> with the first line number (000). (Line

numbers run from 0000 to 1001 : 0 - 9 decimal).

C. Set the Data Field to an odd value (1, 3, 5, 7)

Issue the IOT 6123 (this does not clear the AC)

D. Set the Data Field to an even value (0, 2, 4, 6).

Load AC<4:11> with the character dot pattern  for  the  specified

line.  The  character  dot  pattern is entered as it will appear,

e.g. AC<4> is the leftmost dot of the character cell, see  Figure

1,  Section  4.2.3,  note  that  if  a  character is defined with

CD<7>=1, it will be a right filled character, i.e. dot  positions

8 and 9 in Figure 1. will also be present.

Issue the IOT 6123 (the AC is not cleared)

Repeat steps B, C, D for each line of the character definition.

Set the AC=7777, issue PRQ as in A. to re-enable the video.

6.1.7.2 Accessing the Extended Character Set and the User Defined

Character Set.

As noted earlier, the Extended Character Set and the User Defined

Character  Set are accessed by an Extended Character Set Mode bit

(ECSM) and a User Character Set Mode Enable (UCSME) bit. The  way

the  user  program  controls  these  bits is not yet defined. The

combination of these bits results  in  access  to  the  character

generators as shown below :

ECSM = 1          Character Code |  Character Code

Bit 9 = 0    |    Bit 9 = 1

------------------------------------------------

|	    |    	 |	     |		  |

|UCSME = 0 | UCSME = 1	 | UCSME = 0 |	UCSME = 1 |

|          |            |           |		  |

----------------------------------------------------------

Internal |          |		 |	     |		  |

Char.Code|  US/UK   | US/UK	 |   US/UK   |User defined|

Bit 8=0  | 	    |		 |   with    | Characters |

|	    |		 | Yen mark  |		  |

----------------------------------------------------------

Internal |	    |		 |	     |		  |

Character| Alternate| Alternate  |Katakana   |User defined|

Code Bit | (Foreign)| (Foreign)  |	     | Characters |

Bit 8=1  |	    |		 |	     |		  |

----------------------------------------------------------

|<-These are the only-->|

|possibilities when not |

| in Extended Mode 	 |

In this way arbitrary character substitutions may  be  made  from

the user defined character set when displaying US/UK, any Foreign

Language or Katakana.

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

16-January-1983

Page 43     Rev 4

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

DECmate II  Specification

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

6.1.8 Loading and Reading the CRT Controller Registers

The CRT Controller has one group of internal registers  that  are

generally   only   changed   when  it  is  initially  programmed,

parameters which affect the display format  and  a  second  group

that must be modified continually, cursor position etc.

6.1.8.1 R0 - Characters per horizontal period

This register is  loaded  from  AC  <4:11>  with  the  number  of

characters that make up one row of the display. This includes the

visible area, non-visible areas and retrace time.

6.1.8.2 R1 - Characters per data row

This register is loaded  from  AC  <4:11>  with  the  (number  of

displayed characters per row - 1).

6.1.8.3 R2 - Horizontal delay

This register is  loaded  from  AC  <6:11>  with  the  number  of

character  times  which represents the time from the leading edge

of horizontal sync to the trailing edge of horizontal blank.

6.1.8.4 R3 - Horizontal sync width

This register is  loaded  from  AC  <5:11>  with  the  number  of

character  times  representing  the  width of the horizontal sync

pulse.

6.1.8.5 R4 - Vertical sync width

This register is  loaded  from  AC  <5:11>  with  the  number  of

horizontal  periods  representing  the width of the vertical sync

pulse.

6.1.8.6 R5 - Vertical delay

This register is  loaded  from  AC  <4:11>  with  the  number  of

horizontal periods representing the time between the leading edge

of vertical sync and the trailing  edge  of  the  vertical  blank

signal.

6.1.8.7 R6 - Skew bits

This register is loaded from AC <6:11> with a number of character

times  that  define a skew between cursor and blank and the video

signal going to the monitor.

6.1.8.8 R7 - Visible data rows per frame

This register is loaded from AC <4:11> with the (number  of  data

rows  to  be  displayed - 1). For DECmate II this means 23(8) for

both 80 and 132 column modes of operation.

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

16-January-1983

Page 44     Rev 4

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

DECmate II  Specification

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

6.1.8.9 R8 - Scan sines per data row/Scan lines per frame (MSB)

This register is loaded from AC <7:11> with the (number  of  scan

lines  that  make  up one data row -1), for DECmate II this means

11(octal). AC <4:6> define the most significant three bits of the

number of scan lines per frame.

6.1.8.10 R9 - Scan lines per frame

This register is loaded from AC <4:11> with the least significant

8 bits that define the total number of scan lines per frame - see

R8.

6.1.8.11 ROA - DMA control register

This  register  is  loaded  from  AC  <4:11>  with  DMA   control

information as follows :

AC <4>		DMA disable,  disables  all  CRT  controller  DMA

accesses.  As the CRT controller on DECmate II is

also the source of  memory  refreshes,  this  bit

must  never  be set to zero unless loss of memory

data can be tolerated.

AC <5:7>	Define the delay between DMA bursts when the  CRT

controller  is  operating  in burst DMA mode. For

DECmate II DMA cycles for a given row occur as  a

single burst, so these three bits are set to 111.

AC <8:11>	Define the number of DMA cycles in any one burst,

when  the  CRT  controller  is  operating  in the

burst DMA mode. The  number  loaded  here  is  N,

where the number of DMA cycles in a burst is then

4*(N+1).  This  mode is not applicable to DECmate

II.

6.1.8.12 ROB - Control Register

This register is loaded from AC <4:11> with the following :

AC <4>		Unused

AC <5>		0 - smooth scroll. This causes the smooth  scroll

offset  register  (R17)  to  define the number of

lines that will be scrolled per  frame.  This  is

the  only  mode  used  for  DECmate II, thus this

bit must be set to 0.

AC <6:7>	These two bits define interlace mode. DECmate  II

uses no interlace. These two bits are both set to

0.

AC <8:10>	These three bits define the buffer  configuration

being  used  with  the CRT controller. DECmate II

uses a single row buffer, thus these  three  bits

must be set to 100.

AC <11>	0

Thus the programming word for ROB is X0 001 000 010(8).

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

16-January-1983

Page 45     Rev 4

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

DECmate II  Specification

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

6.1.8.13 ROC - Table start register, low byte

6.1.9 Cursor Visibility

The DECmate I attempted to "make the cursor do the correct thing"

by  suppressing  the  cursor  whenever new data was output to the

screen  at  greater  than  a  given  rate.  This  was   partially

successful  in  that the cursor was not visible when updating the

screen using Direct Cursor  Addressing  sequences,  unfortunately

the  cursor was also not visible when doing things like moving it

across a line when editing. DECmate II attempts  to  treat  these

two  events separately, when the firmware detects that the cursor

is being re-positioned by means of  a  Direct  Cursor  Addressing

Sequence, it will clear the cursor visibility bit when it updates

the cursor address register within the CRT controller. The effect

of  this  is  to suppress the cursor (make it invisible) for half

the cursor blink rate period (130 milliseconds), timed  from  the

update, regardless of the previous state of the cursor visiblity.

Then if another Direct Cursor Address command occurs within  this

period, the cursor will remain invisible.

If the cursor address register is being  updated  for  any  other

reason,  the  firmware  will  set the cursor visibility bit, this

causes the cursor to be visible for half the  cursor  blink  rate

period   (130   milliseconds),   timed  from  the  update,  again

regardless of the previous state of cursor visibility.  Thus,  in

the  cases  of  normal  output  to the screen by echoing keyboard

input and advancing through a piece  of  text,  the  cursor  will

remain visible.

Setting and clearing the cursor is done by loading one of two  AC

bits  when  a  CRT  controller  register  is selected. AC<00> = 1

clears the cursor visibility enable bit  and  would  be  selected

prior  to  updating  the  cursor as the result of a direct cursor

addressing sequence, AC<01> = 1 sets the  visibility  enable  and

would  be  selected prior to updating the cursor position for any

other reason. The effect of setting either of these bits (but not

both)  is  that  when the selected CRT register is later written,

the cursor will be set ON or OFF for the  time  specified  above.

Care  must be taken to ensure that both AC<0:1> are set to 0 when

no effect is intended on the normal alternating cursor display.

6.2 Panel Requests

There are four instructions which, when issued  in  user  memory,

cause  some action to be performed by the control memory code. To

maintain compatibility with DECmate I, the first three  of  these

operate in the same way as with DECmate I. The fourth is modified

and although the types of operations that may be performed  using

this  fourth  instruction  are  similar  there  is  no  backwards

compatibility.

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

16-January-1983

Page 46     Rev 4

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

DECmate II  Specification

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

6.2.1 PR0 - Panel Request 0

This instruction may be used to read the contents of a  specified

screen location. Its use is as follows :

PR1

Row Address	/ Absolute, 0 - 27

Column Address  / Absolute, 0 - 117 (0 - 203)

XXXX		/ Return made here with contents

/ of specified screen location in

/ the AC

6.2.2 PR1 - Panel Request 1

This instruction was used with DECmate I  to  allow  the  display

buffer  memory  to  be  updated  quickly.  It  is not needed with

DECmate II as the same screen update restrictions do not apply to

this  new machine, it is retained for compatibility reasons only.

To move data to  the  display  buffer  portion  of  memory,  this

instruction is used in the following way :

PR1

Row Address	/ Absolute, 0 - 27

Column Address  / Absolute, 0 - 117 (0 - 203)

Data		/ 12 bits, includes attributes

...		/ group of three entries

...		/ repeated for as many screen

...		/ locations as to be changed

7777		/ terminator

Attributes :

AC	00	-	Blink

01	-	Underline

02	-	Bold

03	-	Reverse

6.2.3 PR2 - Panel Request 2

For DECmate I this was equivalent to  entering  SET-UP  mode  and

depressing a key, specified as :

PR2

N 		/ The ASCII code for the  key  to

/ be "pressed"

This Panel Request has no action on DECmate II. If it  is  issued,

return will be made to CALL+2 for compatibility.

6.2.4 PR3 - Panel Request 3

This is a general call to execute a routine  in  control  memory.

With  DECmate I  the absolute address in control memory had to be

specified in the location following the PR3  instruction  and  no

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

16-January-1983

Page 47     Rev 4

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

DECmate II  Specification

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

validity checking was performed, calls to some arbitrary  address

could,  therefore,  cause unpredictable results. The operation of

this instruction for  DECmate  II  has  been  modified,  now  the

routine  to  be  executed  in  control  memory  is specified by a

function number in the location following the request. The format

is as follows :

PR3

FUNCT   /Function

ARG1    /First argument (optional)

ARG2    /Second argument (optional)

..

..

7777    /Terminator

If a return is to be made, it is made to the  location  following

the 7777 terminator.

The following functions are implemented in the  current  firmware

(Revision 216 or 224)

Function	Action				Argument(s)

0000		Print the character		None

in the accumulator.

0001		Select video enable		None

with AC<ll>.

0 = disable, 1 = enable

0002		Select extended character

Mode with AC<ll>.

0 = disable, 1 = enable

0003		Select User Charaacter Set	None

with AC<ll>.

0 = disable, 1 = enable

0004 		Select Cursor Style with AC<ll> None

0 = Block, 1 = Underline

0005		Execute Power-on Clear 		None

There is no return as self-test

is run.

0006		Execute SET-UP Menu 		None

0007		Request Display Status 		None

On return, AC = 0000 means the

display is not busy, AC =7777

it is busy.

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

16-January-1983

Page 48     Rev 4

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

DECmate II  Specification

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

Function	Action 				Argument(s)

0010		Select Function Key Mode with   None

AC<ll>. 0 = transmit ESC/BS/LF

1 = transmit ESCape sequences

for these three keys.

0011		Select Keyboard Case with	none

AC<11>. 0 = lower-case,

1 = upper-case.

4000+10*N+M	Load data from user memory to

panel memory.

N - CP memory destination field

M - user memory source field

Argument 1	User memory SA

Argument 2	CP memory SA

Argument 3	2's complement of

#   of  words  to

move

5000+10*N+M	Load data from panel memory to

user memory.

N - user memory destination field

M - control panel source field

Argument 1	CP memory SA

Argument 2	User memory SA

Argument 3	2's complement of

#   of  words  to

move

6000+10*N	Transfer control to CP memory

field N.

Argument 1	CP memory SA

7000		Send data to the keyboard

Argument(s)	data in bits

<4:11>. Bits

<0:3> must be

zero.

N.B. No check is made on the

data sent.

7100+10*N	Transfer Screen Control Data

to a user buffer in Field N.

Argument l	User Buffer SA

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

16-January-1983

Page 49     Rev 4

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

DECmate II  Specification

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

29  (decimal)  words   are   moved   to   the   specified

user buffer. They are :

Word 1 - Column Mode: 0 = 80, 7777 = 132

Word 2 - Origin Mode: 0 = absolute, 7777 = relative

Word 3 - The  number  of  the  top  row  of  the  current

scrolling region.

Word 4 - The number of the  bottom  row  of  the  current

scrolling region.

Word 5 - The control word for  the  CRT  Controller  Chip

(See Section 6.asdasdas)

Word 6 - High Word of the Row Address Table for Row 1

Word 7 -

....	  -

....   -

Word 29- High Word of the Row Address Table for Row 24.

6.3 Terminal Input Output

DECmate I had a physical register (TTI), through  which  keyboard

input  data  was  transmitted  from the keyboard scan routines in

Control Panel memory for the user  program  to  be  able  to  use

normal  keyboard  instrcutions,  KRB,  KRS KCC. DECmate II has no

such register, all keyboard data transfer  instructions  cause  a

single  Control Panel Interrupt, control panel code then inspects

the interrupting instruction and  modifies  the  AC  as  required

prior to returning control to the user program.

For output, TLS and other such instructions also cause a  Control

Panel  interrupt  in  the same way as with DECmate I, the data in

the AC is taken and interpreted by Control Panel Memory code.

Instructions that cause Control Panel Interrupts are :

6030, 6032, 6034, 6036

6040, 6042, 6044, 6046

Interrupts  from  any  of  these  user  issued  instructions  are

identified by the single instruction 6071.

The Skip and Interrupt Enable instructions,  (6031,  6035,  6041,

6045) do not cause Control Panel Memory interrupts.

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

16-January-1983

Page 50     Rev 4

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

DECmate II  Specification

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

Appendix A - Standard ROM Character Definitions

TBS

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

16-January-1983

Page 51     Rev 4

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

DECmate II  Specification

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

Appendix B - CPU Instruction Set

All times are for operation at 8 MHz. (I/O at 4 MHz.)

B1. Basic Instructions

Mnemonic   Octal   Description	        Execution Time

Direct  Indirect  Autoindex

AND	0XXX	Logical AND to AC	1.75	2.5	 3.25

TAD	lXXX	Two's complement ADD	1.75	2.5	 3.25

ISZ	2XXX	Increment & Skip if 0   2.26	3.00	 3.76

ISZ	2XXX	(When SKIP occurs)	2.76	3.51	 4.26

DCA	3XXX	Deposit & clear AC	1.75	2.5	 3.25

JMS	4XXX	Jump to subroutine	1.75	2.5	 3.25

JMP	5XXX	Jump 			1.00    1.75	 2.47

B2. Group 1 Operate Micro-instructions

1.5 uSec if no BSW or double rotate, 1.9 uSec otherwise.(#)

*

Mnemonic   Octal   Description 			  Sequence

NOP	7000	No operation 				-

CLA	7200	Clear AC 				1

CLL	7100    Clear Link 				1

CMA	7040	Complement AC 				2

CML	7020	Complement Link 			2

RAR	7010	Rotate AC & Link right 			4

RAL	7004	Rotate AC & Link left 			4

RTR#	7012	Rotate AC & Link two right 		4

RTL#	7006    Rotate AC & Link two left 		4

BSW#	7002	Swap AC<0:5> & AC<6:11> 		4

R3L	7014	Rotate AC ONLY three left 		4

IAC	7001	Increment AC, carry to Link 		3

B3. Group 2 Operate Micro-instructions - 1.75 uSec.

*

Mnemonic   Octal   Description 			  Sequence

SMA	7500	Skip on minus AC 			1

SZA	7440	Skip on zero AC 			1

SPA	7510	Skip on positive AC 			1

SNA	7450	Skip on non-zero AC 			1

SNL	7420	Skip on non-zero Link 			1

SZL	7430	Skip on zero Link 			1

SKP	7410	Unconditional Skip 			1

OSR+	7404	Inclusive OR status bits to AC 		3

HLT$	7402	Halt program in main memory 		3

CLA	7600	Clear AC 				2

+ Add 0.25 uSec. 		$ NOP in Panel Memory.

* Sequence indicates at which point in the instruction execution

the operation occurs. 1 is before 2, 2 before 3, etc.

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

16-January-1983

Page 52     Rev 4

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

DECmate II  Specification

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

B4. Group 3 Operate Micro-instructions - 1.5 uSec

Mnemonic   Octal   Description

NOP	7401    NO operation

CLA	7601    CLear AC

MQL	7421    Load AC to MQ then clear AC

MQA	7501    Inclusive  OR  MQ  with  AC,  result  to  AC,  MQ

unchanged

CAM	7621    Clear both AC and MQ

SWP	7521    Swap AC and MQ

ACL	7701    Load MQ to AC, MQ unchanged

CLA SWP 7721    Load MQ to AC then clear MQ

B5. Internal Control Instructions ( Panel Memory )

Mnemonic   Octal   Description		    Execution Time, uSec.

PRS	6000	Read Panel Status Word*			  2.01

ION	6001	Enable Interrupt System			  1.5

IOF	6002	Disable Interrupt System		  1.5

PGO	6003	Reset HLTFLG				  1.5

PEX	6004	Exit to main Memory after next

JMP or JMS				  1.5

RTF	6005	Restore fields and flags from AC	  2.01

SGT	6006	Skip on GT flag				  1.75

CAF	6007	Clear all flags				  1.75

WSR	6246	I/O write			 	  1.72

GCF	6256	Get current fields and flags		  2.26

CPD	6266	Clear Panel Data Flag			  1.5

SPD	6276	Set Panel Data Flag			  1.5

B6. Internal Control Instructions ( Main Memory )

Mnemonic   Octal   Description                 Execution Time, uSec.

SKON	6000	Skip if Interrupt System ON,

turn it OFF				  1.75

ION	6001	Turn ON Interrupt System		  1.5

IOF	6002	Turn OFF Interrupt System		  1.5

SRQ	6003	Skip on Interrupt Request		  1.75

GTF	6004	Get Interrupt Flags 			  2.26

RTF	6005	Restore Flags and Fields from AC	  2.00

SGT	6006	Skip on GT flag				  1.75

CAF	6007	Clear all flags				  1.75

WSR	6246	I/O write				  1.72

GCF	6256	Get current fields and flags		  2.26

PR0	6206	Make Panel Request			  1.5

PR1	6216	Make Panel Request			  1.5

PR2	6226	Make Panel Request			  1.5

PR3	6236	Make Panel request			  1.5

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

16-January-1983

Page 53     Rev 4

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

DECmate II  Specification

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

B7. Memory Extension Instructions

Mnemonic   Octal   Description                 Execution Time, uSec.

CDF     62N1	Change to Data Field N			  1.5

CIF     62N2	Change to Instruction Field N		  1.5

CDF CIF 62N3	Change both DF and IF to N		  1.5

RDF     6214	Read Data Field to AC<6:8>		  1.5

RIF     6224	Read Instruction Field to AC<6:8>	  1.5

RIB     6234	Read Interrupt Buffer to AC<6:11>	  2.25

RMF     6244	Restore Memory Fields		  	  1.5

B8. Stack Instructions

Mnemonic   Octal   Description                 Execution Time, uSec.

PPC1	6205	Push (PC+1) to stack 1			 2.25

PPC2	6245	Push (PC+1) to stack 2		  	 2.25

PAC1	6215	Push AC to stack 1			 2.25

PAC2	6255	Push AC to stack 2			 2.25

RTN1	6225	Pop top of stack 1 to PC		 2.25

RTN2	6265	Pop top of stack 2 to PC		 2.25

POP1	6235	Pop top of stack 1 to AC		 2.25

POP2	6275	Pop top of stack 2 to AC		 2.25

RSP1	6207	Read stack 1 pointer to AC		 1.25

RSP2	6227	Read stack 2 pointer to AC		 1.25

LSP1	6217	Load stack 1 pointer from AC		 1.25

LSP2	6237	Load stack 2 pointer from AC		 1.25

B9. IOT instructions - External

Of the form "6XYZ" where XY is not 00 nor in the range 20  -  27.

They  require 4.0 uSec for Write only, 4.5 for Write followed by

Read.

Terminal I/O Instructions

Input

Mnemonic   Octal   Description                 Execution Time, uSec.

KCF	6030    Set Keyboard Flag. *			  4.0

KSF	6031	Skip on keyboard flag, clear

it if set. * 			  	  4.0

KCC	6032    Clear the AC. *			  	  4.0

KRS	6034    "OR" keyboard buffer to AC.		  4.5

KIE	6035    Set/clear Keyboard Interrupt

Enable with ACll. * 			  4.0

KRB	6036    Load Keyboard buffer to AC.		  4.5

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

16-January-1983

Page 54     Rev 4

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

DECmate II  Specification

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

Output

Mnemonic   Octal   Description                 Execution Time, uSec.

SPF	6040	Set output flag.			  4.0

TSF	6041	Skip on output flag, clear

it if set. *				  4.0

TCF	6042	Clear the AC. *				  4.0

TIE	6045	Set/clear Output Interrupt

Enable with AC 11. *			  4.0

TLS	6046	Load AC<4:11> to the screen.		  4.0

* These instructions execute differently from the PDP 8/A or VT78.

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

16-January-1983

Page 55     Rev 4

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

DECmate II  Specification

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

Appendix C.

Communications Controller Programming

This example is  in  two  parts,  channel  initialization  and  a

suggested way to handle interrupt service.

C1. Communications Port Initialization

Cl.l Defaults

Following the completion of Self-test,  the  communications  port

will be left in the following state :

Mode			:	Asynchronous

Baud rate		:	1200

Character		:	8 data bits

Parity			:	NONE

Stop Bit(s)		:	1

Modem Lines

Data Terminal Ready	:	ON

Request To Send		:	OFF

Local loopback		:	OFF

Remote Loopback		:	OFF

Clock Mode		:	Internal

C1.2 Setting Communications Controller Chip Parameters

The communications controller chip has two sets of internal 8-bit

registers,  A  and B. Mostly the registers of set A are used, but

to identify the cause of interrupt a register from the B set must

be  read.  The  A set and B set are addressed using the same IOT,

the Data Field determines which will be accessed. With  the  Data

Field  set to an Even number (0, 2, 4, 6), register set A will be

accessed, with the Data Field set to an ODD number, register  set

B.  The  following shows how they must be set up for asynchronous

mode operation.

The general method of addressing a communications  chip  register

is  to  define a register number in AC<9:11> and whether it is to

be read or written, specified by AC<00>, issue the IOT 6366, then

read or write the selected register with this same IOT, i.e. this

IOT  sequentially  selects  a  register  then accesses it. If the

register selected is Control Register 0,  which  is  Write  Only,

then  this  sequence can be shortened to a single IOT with the AC

specifying  the  register  number  (0)  in  AC<9:11>   and   also

containing the data to be written into CR0 in AC<4:8>.

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

16-January-1983

Page 56     Rev 4

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

DECmate II  Specification

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

e.g. Write Control Register 4A

CDF	00	/or 20, 40, 60

TAD	(4	/select CR4 for a write

6366

CLA

TAD	(VAL	/data to be written into CR4

6366		/Load CR4

(CLA)

CDF	XX	/back to user required value

Read Status register 2B

CDF	10      /or 30, 50, 70

TAD	(4002   /select Status register 2 for

/READ, AC<00>=1 specifies READ

6366		/select register

CLA

6366		/Status Register 2 -> AC

AND	(177    /High order bits undefined

CDF	XX	/back to user required value

Write Control Register OA

CDF	00

TAD	(0XX0

6366		/load CR0 from AC<4:8>

(CLA)

CDF	XX	/user value

C1.3 Values to be used

The chip must be initialized in the following sequence any  time;

that any parameter involved is modified.

C1.3.1 Issue chip reset	6367

C1.3.2 Reset Channel

CDF	00

TAD	(30

6366		/Select CR0A, load with

CLA		/30 = RESET

C1.3.3 Initialize CR2A

CDF	00

TAD     (2	/select register CR2A

6366

CLA

TAD     (20	/Fixed number

6366 	/load into CR2A

CLA

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

16-January-1983

Page 57     Rev 4

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

DECmate II  Specification

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

C1.3.4 Initialize CR2B

CDF	10

TAD	(2	/Select CR2B

6366

CLA

6366		/load CR2B with 000

C1.3.5 Initialize CR4A

CDF	00

TAD	(4

6366		/select CR4A

CLA

TAD	CR4VAL

6366

CLA ....

CR4VAL is loaded with the following:

AC <4:5>	01 (Must not be-changed)

<6:7>	00 (Must not be changed)

<8:9>	Define number of STOP bits

00 - Illegal

01 - 1 STOP bit

10 - 1.5 STOP bits

11 - 2 STOP bits

<10>	Parity - ignored if AC<ll> = 0

0 - ODD parity

1 - EVEN parity

<11>	Parity Enable

0 - no parity generated or checked

1 - generate and check parity, sense set

by AC<10>.

e.g. 1 STOP bit, no parity, value loaded is 104

C1.3.6 Initialize CR1A

CDF	00

TAD	(1

6366		/select CR1A

CLA

TAD	(26	/fixed, must not change

6366

CLA ...

C1.3.7 Initialize CR2A

CDF     00

TAD	(2

6366

CLA

TAD	(26

CLA ....

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

16-January-1983

Page 58     Rev 4

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

DECmate II  Specification

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

C1.3.8 Initialize CR3A

CDF	00

TAD	(3

6366

CLA

TAD	CR3VAL

6366

CLA

CR3VAL is loaded with the following :

AC <4:5>	Number of data bits for receive

00 - 5 bits

01 - 7 bits

10 - 6 bits

11 - 8 bits

AC <6:10>	00000

AC <11>		1 - Enables Receiver

e.g. 8 data bits for receive, receiver enabled, value  loaded  is

301.

C1.3.9 Initialize CR5A

CDF	00

TAD	(5

6366

CLA

TAD CR5VAL

6366

CLA

CR5VAL is loaded with the following ;

AC <4>		0

AC <5:6>	Number of data bits for transmit

00 - 5 bits (or less)

01 - 7 bits

10 - 6 bits

11 - 8 bits

AC <7>		Send a BREAK

AC <8>		1 - enables transmitter

AC <9:11>	000

The chip is now ready for transmit and receive.

C1.3.10 Set Modem Status and clock mode

For the simplest case, the only Modem line to be  asserted  would

be  DTR.  In  all cases for asynchonous operation, internal clock

must be selected, thus  the  Modem  Control  Register  should  be

loaded with the value 0010 using IOT 6362.

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

16-January-1983

Page 59     Rev 4

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

DECmate II  Specification

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

C2. Interrupt Service

C2.1 Data Transfers

As there is only one flag for Send, Receive  and  Errors,  it  is

necessary  to  interrogate  the communications controller chip to

identify the cause of a communications interrupt. There are three

possible  interrupt  conditions,  Transmit  Buffer Empty, Receive

Data Available and Error (Overrun, Framing, Parity - if enabled).

6301		/Skip on Comm. port flag

JMP 	NOTCOM

IDENT,  CDF	10	/ODD to read Status Reg. 2B

TAD	(4002

6366

CLA

6366		/Perform read

AND	(7	/mask to three LS bits

the AC now contains the following :

AC	9	10	11	Condition

0	0	0	Hardware malfunction

0	0	1	Hardware malfunction

0	1	0	Hardware malfunction

0	1	1	Hardware malfunction

1	0	0	Transmitter Buffer Empty

1	0	1	Hardware malfunction

1	1	0	Receive Data Available

1	1	1	Parity, overrun or framing error

OR Hardware malfunction

To determine what caused the 111  result,  one  further  register

must be read, Status register 1A

CDF	00

TAD	(4001   /select SRlA for reading

6366

CLA

6366		/read selected register

AND	(377

the AC now contains the following :

AC <0:4>	Undefined

<5>  	Framing Error

<6> 		Overrun Error

<7> 		Parity Error

<8:11>	Undefined

In addition status register  1A  should  be  read  following  any

Receive Data interrupt to check for Errors.

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

16-January-1983

Page 60     Rev 4

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

DECmate II  Specification

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

When the cause of the  interrupt  has  been  determined  and  the

appropriate  action taken, Read Receiver, transmit next character

etc., the interrupt condition must be cleared. For a READ this is

done by writing to Control Register 0A with a data pattern of 70.

For a Transmit Buffer condition, the transmit done condition must

first  be  cleared by issuing a write to CR0A with a data pattern

of 50.

e.g. to dismiss the transmitter

CDF     00

TAD     (50	   /select CR0A

6366		   /and write 50 to it (dismisses

/transmitter)

and common exit for any interrupt condition

CDF	   00

TAD	   (70

6366 	   /select CR0A and write 70 to it

/(common End Of Interrupt Exit)

C2.2 Modem Interrupts

The second interrupt associated with the communications  port  is

the  Modem  Change  Flag. This may be identified by a normal flag

test,

6361    /Skip on Modem Change Flag, clear flag

/if found set

The Modem status may then be read by the Read Modem Status IOT

6364	   /Read Modem Status to AC<7:11>

AC <0:6>	   Undefined

<7>	   State of Analog Loopback

<8>	   State of Calling Indicator (Ring)

<9>	   State of Signal Detect

<10>	   State of Data Set Ready

<11>	   State of Clear to Send

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

16-January-1983

Page 61     Rev 4

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

DECmate II  Specification

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

Appendix D.

Memory Allocation

Of the 64K words of memory available with  DECmate  II,  32KW  is

allocated  to the Application Program at all times - Main Memory.

The second 32KW - Control Panel Memory, is used as follows :

Fields <P0:1>   Firmware,  loaded  from  disk   controlling   the

terminal emulation, keyboard handling etc.

Fields <P2:3>   Display Buffer

Fields <P4:7>   Not allocated, may be used  by  the  Applications

Program with certain limitations.

Additionally, at power-on time, Field 0 of Control  Panel  Memory

is  overlayed  by  ROM with program execution starting at address

P07777. ROM or RAM  may  be  selected  as  being  the  source  of

instructions  and indirect addresses or the source/destination of

operands in way similar to that in which the  IF  and  DF  fields

operate  with  the  normal memory extension. The ROM-RAM accesses

are controlled by the WSR (6246) instruction as follows :

Instruction Fetch (Defer)	Operand Access

AC<11>=0    Next cycle from ROM		No change

AC<ll>=l    Next cycle from RAM		No change

AC<00>=0 	No change		ROM

AC<00>=1	No change 		RAM

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

16-January-1983

Page 62     Rev 4

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

DECmate II  Specification

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

Appendix E.

ROM Useage.

DECmate II has, as noted in Appendix D, 4K of ROM which  overlays

Field  0  of  Control  Panel  Memory  at  power-on time. This ROM

performs several functions:

E1. Processor Initialization

Dynamic RAM memory is initialized.

The I/O Controller chips are initialized

The CRT Controller Chip is initialized

The Communications Controller Channel is initialized

E2. Self Test

A very basic RAM memory test is performed. The machine will  loop

forever if this test fails.

A simple loader is then moved from  ROM  to  RAM,  Control  Panel

Field  P0  and  executed  to  move  the  remainder of ROM to RAM,

Control Panel Field P7.

Execution of the loaded code begins  at  RAM  address  P70200  in

Control Panel Memory.

Fields P2 and P3 of Control Panel Memory are cleared.

The CRT Controller chip is checked to ensure that it  is  causing

interrupts.

Fields <MO:7> of Main Memory and Fields <P2:6> of  Control  Panel

Memory are then tested.

Code is moved to Main Memory to allow the normal interrupt system

to  be  checked  for the Real Time Clock and Printer Port. Checks

are also made to ensure that the HLT instruction, Panel  Requests

and Keyboard instructions cause Control Panel Interrupts.

The Keyboard serial data path is then checked in  local  loopback

mode.

If there is an RD51 present on the system, a delay of 15  seconds

is  initiated  to allow time for the RD51 motor current demand on

the power supply to drop to its normal operating level. When this

delay  is  over,  or  if  it  is determined that there is no RD51

present, the RX50 controller is allowed to start  by  issuing  an

initial LCD IOT. Data transfer to/from the RX50 controller buffer

is then checked.

Any errors detected during the self-test result in the display of

an error number at the completion of the test.

E3. Bootstrap Procedure

At the end of self-test,a further check is made for the  presence

of the RD51 disk.

. RD51 Present:

If the controller is found to be present,  a  further  10  second

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

16-January-1983

Page 63     Rev 4

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

DECmate II  Specification

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

delay is initiated to allow the disk to come fully up  to  speed.

During  this 10 second period, the RD Ready bit is tested. If the

bit does not come true before the 10 second period is  over,  the

timer  is  reset  and  the flag test made again. Once the flag is

found to be set, the disk error flag is checked. If there  is  an

error  condition  indicated by the RD51 controller, bootstrapping

continues using the RX50. If no  error  is  indicated,  the  RD51

Startup  Block is read into Control Panel Memory Field P7. At the

completion of the read, the block is checked  for  validity.  The

first 6 bytes must be as follows :

001	/Start of identifier sequence

122	/R

104	/D

065	/5

061	/1

002	/End of identifier sequence

If the Startup Block verifies good, it is then executed. Refer to

the  appropriate  RD51 controller specification(s) for details of

the Startup Block format.  If  the  block  is  invalid  (by  this

check), bootstrapping continues from the RX50.

. RD51 Not present:

A Read Status command is issued to the RX50 controller  and  a  4

second timer established. If the RX50 controller does not set the

Done flag within this period, the sequence is repeated. When  the

Done  flag  is finally found, the drive status is read. If Unit 0

(Drive A) is not ready, the DECMATE II logo  is  displayed  as  a

double height , double width image, centred on the screen. If the

drive is ready or when it does become ready (when a  diskette  is

inserted in drive 0), the screen is cleared and Track 1 Sector 0,

the Bootstrap Block for  all  operating  systems,  is  read  into

memory  in  8-bit  mode. If a disk error is indicated during this

read operation, a picture  of  a  diskette  is  displayed  as  an

indication  to  try another system disk. If no error is indicated

by the disk  controller,  the  data  read  is  then  checked  for

validity.  A  valid  Boot  block  must  conform to the Boot Block

Standard (See Appendix G).  If  the  validity  check  fails,  the

diskette picture is displayed and the read operation repeated. If

the Boot Block passes this check it is assumed good.

The next area of  the  diskette  to  be  validated  is  the  Main

Keyboard  Translation  Table. This is stored in sector 3 of Track

0. This area of the diskette  is  read  in,  in  8-bit  mode,  to

Control  Panel  Memory,  starting  at  address 15350. (The actual

keyboard translation table data begins at 15360, the first  eight

characters  read  which  are  used for validation are read into a

buffer area). If a disk controller error is indicated during this

read  operation,  the  diskette picture is displayed and the read

repeated. If no controller error is indicated, the data read  can

then be validated. The first bytes read must be :

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

16-January-1983

Page 64     Rev 4

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

DECmate II  Specification

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

001	/Start of identifier sequence

113	/K

102	/B

104	/D

102	/B

122	/R

104	/D

002	/End of identifier sequence

If the data does not pass this check,  it  is  assumed  that  the

keyboard  translation  table  is invalid, the diskette picture is

displayed and the read operation repeated. If the validity  check

is  passed, system initialization continues with reading the User

Character Generator RAM data from the diskette, Track 0,  Sectors

7  -  10,  into  memory. If no controller error is indicated, the

data read in is  validated  in  the  same  way  as  the  Keyboard

Translation Table. In this case the first words of data read must

be :

001	/Start of identifier sequence

125	/U

103	/C

107	/G

122	/R

101	/A

115	/M

002	/End of identifier sequence

If the validity check fails, or if there was  a  disk  controller

error  indicated  after  the  read  operation, the User Character

Generator RAM is loaded with zero (all blank characters). If  the

validity  check  is  passed,  the  data read from the diskette is

loaded into the UCG RAM.

This completes the loading of language  specific  data  from  the

diskette,  the actual firmware is now loaded. All Track 78 is now

read in 12-bit  mode  into  Control  Panel  Memory,  starting  at

address  P00000.  If  a  controller  error  is  indicated  at the

completion of the read, the diskette picture is displayed and the

entire  diskette  readin  operation  restarted.  If no controller

error is indicated, the remainder of Control Panel  Memory  Field

P0  is  loaded  from Track 79, Sectors 1 - 6, with the same check

made for controller error as with Track 78. Following  this,  the

remainder of Track 79 is read into Control Panel Memory Field P1,

starting at address P10000 with the same error check  made.  When

all  the  firmware  has been read in, it is checked for validity,

memory locations P00001 - P00003 must be :

P00001/6201

P00002/0033

P00003/0077

(memory location P00000 contains the current  firmware  revision)

If   the  validity  check  fails,  the  entire  diskette  read-in

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

16-January-1983

Page 65     Rev 4

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

DECmate II  Specification

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

operation is restarted.  If  the  check  is  good,  execution  of

firmware begins at Address P00200 in Control Panel Memory.

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

16-January-1983

Page 66     Rev 4

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

DECmate II  Specification

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

E4. SETUP

Code in ROM supports the "SETUP" feature of  the  DECmate  II  to

allow  certain  screen  features  such  as  column mode (80/132),

cursor style etc. to be changed by a user and  optionally  stored

in  the  firmware  area  of the system diskette to be used as new

defaults. SETUP is invoked by a  user  program  issuing  a  Panel

Request  to the firmware which in turn loads a section of the ROM

into Control Panel Memory Field P7 and then transfers control  to

the  loaded code at address P00177, by means of a subroutine call

to address P000176 with the AC = 0000. (See HLT processing)

The ROM code then reads Track 0, Sectors 1 and 2 in  8-bit  mode.

These two sectors contain the text file that results in the SETUP

Menu being displayed. The text  file  is  validated  after  being

read.  If there is an error on reading in these two sectors or if

the validation check fails, return is  made  immediately  to  the

firmware  and  hence  to the user program. The first bytes of the

text file must be:

001	/Start of identifier sequence

123	/S

105	/E

124	/T

125	/U

120	/P

002	/End of identifier sequence

The remainder of the file can contain any text  with  a  page  of

text  starting  with  a  control  code  002 (^B), using any 8-bit

character codes, but must refer in  sequence  to  the  selectable

parameters  as listed below. The end of a text field and start of

a parameter field is  defined  by  the  control  code  036  (^^),

sub-fields  within a parameter field are separated by the control

code 037 (^_). The end of the entire screen of text is defined by

the  control  code  003  (^C). Subsequent pages are prefixed with

another control code 002 (^B). See the  example  of  the  English

Language  SETUP  file below, in particular refer to the operating

instructions given below the row of "====".

In  this  example,  the  following  representations  of   control

characters are used:

. 	Space

^A 	001

^B 	002

^C 	003

^^ 	036

^_ 	037

All other codes are actual displayed characters.

The second "page" of text "Are you sure etc..." is  displayed  in

response  to  the  user  pressing  the  "Do"  key to save the new

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

16-January-1983

Page 67     Rev 4

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

DECmate II  Specification

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

Selections on the  firmware  area  of  the  system  diskette.  It

replaces  the  text  displayed below the row of "====". The third

"page" of text is displayed in response to the user pressing  the

"Re-move"  key  to  mount  a new system diskette. It replaces the

entire page.

English Language SETUP Text File Version 1.

^ASETUP^B......USER.SELECTIONS.MENU

Screen.Width..........^^80......^_132

Cursor.Style..........^^Block...^_Underline

Cursor.Visibility.....^^Visible.^_Invisible

Scrolling.............^^Fast....^_Slow

Screen.Mode...........^^Normal..^_Reverse

Keyboard.keyclick.....^^ON......^_OFF

Terminal.Mode.........^^VT100...^_VT52

Baud.rates

.....Printer..........^^300^_600^_1200^_2400^_4800^_9600

.....Communications...^^300^_600^_1200^_2400^_4800^_9600

=======================================================================

....Press.:

...........'->'.to.Advance '<-'.to.Backup,.'Select'.to.fix.new.selection

...........'Do'.to.fix.new.selections.and.leave.Selections.Menu

...........'Return'.to.use.new.selections.and.leave.Selections.Menu

...........'Remove'.to.load.new.System.Disk

^C^BAre.you.sure.?..Press.'Do'.to.confirm

^C^BInsert.new.System.Disk.and.press.'Do'

Any.other.key.will.return.to.User.Selections.Menu

^C

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

16-January-1983

Page 68     Rev 4

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

DECmate II  Specification

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

Appendix F.

RX50 Disk Format/Useage.

There are three tracks on a DECmate II floppy diskette  that  are

reserved  and  may not be used by any operating system, tracks 0,

78 and 79.

Track 0 :

Sectors 1 and 2 contain the text file  that  supports  the  SETUP

handler  part  of  ROM.  These  sectors are written (and read) in

8-bit mode.

Sector 3 contains the main keyboard translation table, (the  main

keyboard  comprises  all keys of the main array, but excludes the

cursor pad, the application pad  and  the  function  keys).  This

sector  is  written  (and  read in) in 8-bit mode and consists of

four bytes per key location on the main key array, defining  what

each key shall generate in the four possible modes :

Normal, Shifted, Caps (or Shift) Lock and Control.

The value stored in each location of  the  table  is  the  actual

8-bit  value  to  be  generated  for  that combination of key and

modifier. For any key and modifier that generates no  code,  that

table entry is set to contain a zero.

Example : Key address 194 is  the  key  labelled  "A",  the  four

entries for this key address are :

Normal  Shift   Caps Lock	Control

141 	101 	101		001

(a)     (A)     (A)		(Control code SOH)

The key sequence is as follows :

Key Address	Key definition for USASCII - WPS

188		DELETE

189		RETURN

190		TAB

191		GRAVE ACCENT- TILDE

192		1 !

193		Q

194		A

195		Z

196		NO KEY AT THIS POSITION

197		2 @

198		W

199		S

200		X

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

16-January-1983

Page 69     Rev 4

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

DECmate II  Specification

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

201		< >

202		NO KEY AT THIS POSITION

203		3 #

204		E

205		D

206		C

207		NO KEY AT THIS POSITION

208		4 $

209		R

210		F

211		V

212		"SPACE"

213		"SPACE"

214		5 %

215		T

216		G

217		B

218		NO KEY AT THIS POSITION

219		6 ^

220		Y

221		H

222		N

223		NO KEY AT THIS POSITION

224		7 &

225		U

226		J

227		M

228		NO KEY AT THIS POSITION

229		8 *

230		I

231		K

232		, ,

233		NO KEY AT THIS POSITION

234		9 (

235		O

236		L

237		. .

238		NO KEY AT THIS POSITION

239		0 )

240		P

241		NO KEY AT THIS POSITION

242		; :

243		/ ?

244		NO KEY AT THIS POSITION

245		= +

246		] }

247	 	\ |

248		NO KEY AT THIS POSITION

249		- _

250		[ {

251		' "

252		NO KEY AT TH IS POSITION

253 		NO KEY AT THIS POSITION

254		NO KEY AT THIS POSITION

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

16-January-1983

Page 70     Rev 4

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

DECmate II  Specification

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

255 		NO KEY AT THIS POSITION

For keys required to generate a NULL (000) code, the  appropriate

table  location  is  setup  to contain 377. (For the USASCII- WPS

keyboard, there are three entries of this kind, the  two  "SPACE"

key  addresses  and  the "2" key address, all three generate NULL

for the CNTRL condition). The ROM loading program then performs a

further  translation  on  this code to load a 4000 value into the

actual keyboard translation table.

Refer  to  LK201  Keyboard  Specification  for  Foreign  Language

Variations.

Sector 4 contains the text file that supports the HLT instruction

part  of the ROM code. This file is specified in a similar way to

the SETUP text file. When a HLT instruction is executed  in  user

memory,  the  HLT  processor  present  in  panel memory moves the

contents of ROM into control panel memory Field  7,  starting  at

location  70100  and then transfers control to the loaded code by

executing a JMS to location 70176. The HLT processing  code  then

reads  Track 0 Sector 4 and performs a validity check on the text

data. The first eight words read must be :

001	/Header

110	/H

114	/L

124	/T

106	/F

114	/L

107	/G

002	/Terminator

If an error is made on reading the  sector  or  if  the  validity

check  fails,  return  is made immediately to the firmware, which

will in turn return to the user program at the location following

the  HLT  instruction. If there is no read error and the validity

check is passed, the CRT controller Row Address Table is reset so

that  the  current  screen data may be preserved for the possible

return from the HLT processing and the text file displayed.  This

is  formatted  in  the same way as the SETUP text file but allows

only the options of "Do" to continue from the HLT and "SETUP"  to

enter  SETUP.  If the "Do" option is selected, the CRT Controller

RAT is reset to restore the original screen data.

Sectors 5 and 6 are currently unused.

Sectors 7 - 10 contain the  data  to  be  loaded  into  the  User

Defined  character  Generator RAM. This data is read in as a part

of the Bootstrap process. If a disk read error occurs or the disk

data  fails  a  validity  check, the User Charatcer RAM is filled

with nulls. If there is no read error and the validity  check  is

passed,  the  data read is loaded into the RAM. The data consists

of 16 bytes  per  character,  one  byte  for  each  possible  RAM

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

16-January-1983

Page 71     Rev 4

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

DECmate II  Specification

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

location allocated for a character. As only 10 lines are actually

used  to  display a character, only the first 10 bytes of each 16

have significance, the last 6 of a group of 16  should  be  zero.

The validation check for this data is :

001	/Header

125	/U

103	/C

107	/G

122	/R

101	/A

115	/M

002	/Terminator

Appendix G.

The section of the RX50 Bootstrap Block Standard that applies  to

DECmate  II  requires  that  The  Boot  Block (Track 1, Sector 1)

contain the following at its beginning :

Byte #		Contents

0		0

1		0

2		N - an   offset    to    additional    ID

information

3		1 - System Disk (A non-system disk  would

contain 0 here)

4		0

5		0

Additional ID words begin here

2*N+2		010 - PDP 8 Code on this diskette

2*N+3		010 - RX278 Controller

2*N+4		010 - WPS8 Diskette Structure

011 - OS8 Diskette Structure

012 - COS310 Diskette Structure

100 - CP/M Diskette Structure

2*N+5		347 - WPS8  )

346 - OS8   ) computed check value

345 - COS310) for above data

257 - CP/M  )

2*N+6		0   - always

2*N+7		1   - Revision of this standard +  single

sided diskette

For greater detail refer to the actual standard.

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

16-January-1983

Page 72     Rev 4

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -