-----------------------------------------------------------------

------------------------------

|    |   |   |   |   |   |   |			 ENGINEERING

|  D | I | G | I | T | A | L |			SPECIFICATION

|    |   |   |   |   |   |   |

------------------------------

-----------------------------------------------------------------

LK200 FUNCTIONAL SPECIFICATIONS

Copyright (c) May, 1982 	    Digital Equipment Corporation

Author L. Huntington

These designs and specifications are the sole property of Digital

Equipment Corporation and shall not be reproduced, copied or used

in whole or in part as a basis for the manufacture or sale of any

equipment  described  herein  without  written  permission of the

owner.

Table of Contents				_________________

Section 						   Page

1.0     INTRODUCTION 					     1

1.1       Scanning					     2

1.1.1       Timing Requirements				     2

1.1.2       Sneak Path Detection			     2

1.1.3       Rollover					     2

1.2       Transmission					     2

1.2.1       Transmission Buffer and speed		     2

1.2.2       Transmission Encoding			     2

1.3       Receive					     3

1.3.1       Receive Buffer and Speed			     3

1.3.2       Receive Encoding				     3

2.0     KEYBOARD FUNCTIONAL DIVISIONS 			     3

3.0     MODES 						     3

3.1       Down Only					     4

3.2       Down, Autorepeat				     4

3.3       Down/up				 	     4

3.4       Special Considerations Regarding Autorepeat	     4

3.5       Special Considerations Regarding Down/up mode	     6

3.6       Autorepeat Rates				     6

4.0     KEYBOARD PERIPHERALS 				     6

4.1       Audio						     6

4.2       LED's						     7

5.0     KEYBOARD-TO-TERMINAL PROTOCOL 			     7

5.1       Keycode Transmission				     7

5.2       Special Code Transmission			     7

5.3       Powerup Transmission				     9

6.0     TERMINAL-TO-KEYBOARD PROTOCOL 			     9

6.1       Commands					    10

6.2       Parameters					    10

6.3       Peripheral Commands				    11

6.3.1	    Flow control				    12

6.3.2	    LED's					    13

6.3.3	    Audio					    13

6.3.4	    Autorepeat					    13

6.3.5	    Other					    13

6.4       Mode Set Commands				    14

6.4.1       Division Mode Settings			    14

6.4.2       Autorepeat Rate Buffer Association		    15

6.4.3       Autorepeat Rate Buffer Values		    15

7.0     SPECIAL CONSIDERATIONS 				    16

7.1       Error Handling				    16

7.2       Keyboard Locked Condition			    17

7.3       Reserved code					    17

7.4       Test mode					    17

7.5       Future Expansion				    17

Page ii

Table of Contents				_________________

Section 						   Page

8.0     APPENDIX I    KEYCODE TRANSLATION TABLE 	    19

9.0     APPENDIX II   DEFAULTS 				    24

10.0    APPENDIX III  KEYBOARD MATRIX  			    25

11.0    APPENDIX IV   KEYSWITCH LAYOUT       		    27

Page 1

1.0 INTRODUCTION

This document is a specification of  the  firmware  functionality

available  in  the  LK200 keyboard.  Products using the LK200 may

use only a subset of the functionality available in the keyboard.

The firmware inside the keyboard  is  responsible  for  executing

three concurrent tasks.

The first task consists of detecting any changes in the state  of

the  keyboard  matrix, referred to as the process of scanning the

keyboard.

The second task consists  of  transmitting  the  results  of  the

keyboard scan to the terminal.

The  third  task  consists  of  receiving  information  from  the

terminal.  This  information  may  control  either the peripheral

hardware or the transmission parameters on the LK200.

Page 2

1.1 SCANNING

1.1.1 Timing Requirements

The LK200  firmware  is  responsible  for  handling  a  Keystroke

depressed  for  as little as 20 ms., during which time the switch

closure may be expected to bounce for 5 ms. Multiple keys will be

detected  in  correct  order  when interkeying times are not less

than 8.33 ms.

Given the susceptibility to high frequency noise, each  key  must

be verified as a legitimate event.

Scan rate is 120 hertz, or 8.33 ms.

1.1.2  Sneak Path Detection

A sneak path, or "ghost" key may occur  only  when  3  out  of  4

corners  of a rectangle in the keyboard matrix are closed.  Sneak

paths will be prevented by  carefully  laying  out  the  keyboard

matrix positions, so that the keys belonging to a  single fingers

zone are not found in the same row.  However, should a sneak path

occur,  firmware will not transmit the keys involved until one of

the keys causing the condition is  released.  This  will  prevent

the transmission of ghost keys entirely.

1.1.3  Rollover

With the exception  of  those  keys  involved  in  a  sneak  path

arrangement,  the  keyboard will transmit ALL keys depressed.  It

is, however, limited by transmission speed and the  size  of  its

output  buffer.  The  keyboard is capable of transmitting 4 bytes

per scan.  Rollover is determined by the amount of time the  keys

are  held  down.  If  they are held down for 17 ms., the keyboard

will scan them twice, and therefore be able to  send  8  keycodes

from its 4-byte fifo.  Actually, 17 ms. is considered to be below

the minimum time that the keys can be held closed.  If  the  keys

are  held down for another scan (25 ms.), then 12 keycodes can be

transmitted.  Of course, if there are sneak paths  involved,  not

all keys can be transmitted.

1.2  TRANSMISSION

1.2.1  Transmission Buffer and Speed

The LK200 has a transmission speed of 4800 bps and a fifo size of

4 bytes.

1.2.2  Transmission Encoding

Encoding of transmit and receive data is identical.  Each encoded

event  is  framed  into  10  single  elements, each element being

transmitted at equal time intervals.  The first element (0)  will

Page 3

be the start (spacing) element;  the next 8 elements will contain

data.  The last element will be the stop (1) element.

1.3  RECEIVE

1.3.1  Receive Buffer and Speed

The LK200 has a receive speed of 4800 bps and a fifo  size  of  4

bytes.  Commands  are  processed  within  1 ms.  after the entire

command is received.

1.3.2  Receive Encoding

Encoding of transmit and receive data is identical.  Each encoded

event  is  framed  into  10  single  elements, each element being

transmitted at equal time intervals.  The first element (0)  will

be the start (spacing) element;  the next 8 elements will contain

data.  The last element will be the stop (1) element.

2.0 KEYBOARD FUNCTIONAL DIVISIONS

The keyboard is divided into 14 functional divisions.

These fourteen functional divisions are:

Division	        ________

1.  48 graphic keys, Spacebar and B13, D13, D99  and  E99

on the typewriter mass

2.  Numeric keypad

3.  Delete Character(E12)

4.  Return(C13) and Tab(D00)

5.  Lock(C00), COMPOSE(A99) and A10

6.  Shift(B00 and B11) and CTRL (C99)

7.  Horizontal cursors(B16 and B18)

8.  Vertical cursors(B17 and C17) and A17

9.  Six keys directly above the cursor keys (D16-D18  and

E16-E18)

Function Keys

10. G99-G04 and H99-H04

11. G05-G09 and H05-H09

12. G10-G14 and H10-H14

13. G15-G16

14. G20-G23 and H20-H23

See Appendix IV for keyswitch designators.

3.0 MODES

Section 3 explains the overall  function  of  the  three  keycode

transmission   modes.  For  actual  mode  set  and  special  code

Page 4

transmission protocol, please see  sections  5.1,  6.3  and  6.4.

Any  one of the keyboard's functional divisions may be set to any

one of the following modes.

Mode			Function		____			________

Down only		Keyboard sends key code when  key

is first depressed.

Autorepeat, Down	Keyboard sends key code when  key

is  first  depressed,  then sends

"metronome"   code   and   sounds

keyclick   at  the  specified  or

default rate, until that  key  is

released.

Down UP			keyboard sends key code when  key

is  first  depressed,  and either

key code or "all ups"  code  when

key is released.

See Appendix II for division defaults.

3.1  Down only

"Down only" keys transmit a keycode when they are depressed.

3.2  Down, Autorepeat

"Autorepeat" keys transmit a keycode when they are depressed.  If

the key is held down past the specified or default timeout period

(usually 300 to 500  ms.),  then  a  fixed  "metronome"  code  is

transmitted.  "metronome"  codes  will  then  be  sent at regular

intervals until the key is released.

3.3 Down/up Mode

"Down up" keys transmit a keycode when they are depressed and  an

"up code" when they are released.  The "up code" will be either a

repeat of the keycode or an "all ups"  code.  If  there  are  any

other  down/up  keys down on the keyboard, it will be a repeat of

the keycode.  If there are no other  down/up  keys  down  on  the

keyboard, it will be an "all ups" code.

3.4  Special Considerations Regarding Autorepeat

If multiple autorepeating keys are held down,  "metronome"  codes

will  still  be  generated.  The  metronome  codes  apply  to the

keycode which was transmitted most recently.  If that  "last  key

down"  is  released,  and  other  key(s)  is(are) still down, the

keycode of one of the "down" keys will be retransmitted.

Page 5

Example:  Suppose the "a" key is being held down.

Transmission will look like this:

a metronome metronome

Now suppose the "b" key is depressed.

a metronome metronome b metronome metronome

Now the "b" key is released.

a metronome metronome b metronome metronome a met. met.

A non-autorepeating keycode or special code  may  be  transmitted

while  metronome  codes  are being generated for an autorepeating

key. In this case, the keyboard will transmit, in  the  place  of

the  next  metronome  code,  the  keycode  of  the  key  which is

supposed to  be  autorepeating.  This  way,  the  keycode  to  be

autorepeated is always the LAST BYTE TRANSMITTED.

Example: Suppose the "a" key is being held down.

Transmission will look like this:

a metronome metronome

Now suppose the SHIFT key is depressed.

a metronome metronome shift a metronome

Now the SHIFT key is released.

a metronome metronome shift a metronome allups a met...

If, for any reason, an autorepeating key  should  not  autorepeat

(such  as  a  CTRL "C", for example), the terminal should issue a

"temporary  inhibit  autorepeat"  command.  This  will  halt  the

transmission  of  any  metronome  codes or keyclicks FOR THAT KEY

ONLY.  Metronome codes will resume when another key is depressed.

The   command   must   be   issued  after  the  keycode  for  the

autorepeating key is received.

Autorepeat can be  enabled  and  disabled  independently  of  the

division  settings, using the enable/disable autorepeat commands.

This is an "across the  board"  enable/disable.  Internally,  the

keyboard  is  still  autorepeating characters while autorepeat is

disabled,  but  simply  does  not  transmit  metronome  codes  or

keyclicks.  Therefore,  when autorepeat  is enabled, the keyboard

will resume metronome transmission at the point where it would be

if  autorepeat  were not disabled in the first place. This may be

within EITHER the timeout or interval period, depending upon  the

time elapsed since key depression.

During the period that a  key  is  depressed,  the  keyboard  may

receive  a  request to change that division's mode to autorepeat.

This change will be implemented by the LK200  immediately.  After

the  regular timeout period, the keyboard will transmit metronome

codes for the key depressed.  In place  of  the  first  metronome

code, the Keyboard will transmit the keycode of the autorepeating

key.

It is possible to change the mode of all autorepeating  divisions

to  down  only with one command.  This and some other across-the-

board autorepeat  commands  are  grouped  with  the  "peripheral"

commands. (see section 6.3)

Page 6

3.5  Special Considerations Regarding Down/up Mode.

If two down/up keys are released simultaneously, (within the same

scan),  and there are no other down/up keys down on the keyboard,

only one "all ups" code will be generated.

3.6  Autorepeat rates

There are four buffers in the keyboard to store autorepeat rates.

They  are  numbered  O  through 3.  Two values are stored in each

buffer.  The first value is the "timeout" value.  The  second  is

the  "interval"  value.  The values in each buffer can be changed

by the terminal.

The "timeout" value reflects  the  amount  of  time  between  the

detection  of  a  key  down  and  the  transmission  of the first

metronome  code  (defaults  range  from  300  to  500  ms.)   The

"interval"  value  reflects  number of metronome codes per second

(defaults to 30).

Each division is associated with one of the four  buffers.  Rates

are  taken  from  the  associated buffer each time the autorepeat

timers are loaded.  This buffer-to-division  association  can  be

changed by the terminal or left to default.

4.0  KEYBOARD PERIPHERALS

Section 4 is an overall explanation of the peripherals  available

on  the  keyboard.  See  section 6.3 for information on  terminal

control of these peripherals.

Keyclick, bell and LED's are all considered "peripherals" on  the

keyboard.

4.1  Audio

Keyclick is a 2 ms. "beep".  Bell is a 125 ms.  "beep".  Bell  is

sounded  ONLY  upon  request from the terminal.  Keyclick (if not

disabled by the terminal) is sounded under three conditions:

1. When a Key is depressed

2. When a metronome code is sent.

3. When a "sound keyclick" command is received  from  the

terminal.

Keyclick will NOT be generated when Keys B11  or  B99  (the  left

and  right SHIFT keys on the LK201) are depressed.  Keyclick will

NOT be generated when key C99 (the CTRL  key  on  the  LK201)  is

depressed,  unless  a command is sent from the terminal to enable

keyclick on that Key. (see section 6.3, key positions illustrated

in Appendix IV)

Either keyclick or bell (or both) may be disabled.  When keyclick

or  bell  is  disabled,  it  will not sound, even if the terminal

Page 7

requests it. (see  section  6.3  for  enable  and  disable  audio

commands).

Both keyclick and bell may be set (independently)  to  one  of  8

volume levels. Volume levels are as follows:

000 - highest

001

010 - default

011

100

101

110

111 - lowest

4.2  LED's

The  terminal  is  responsible  for  transmitting   LED   control

commands, with the following exceptions:

1. The keyboard will turn OFF all LED's on powerup.

2. The keyboard will light the Keyboard Locked  LED  upon

receipt  of an "inhibit transmission" command and turn

it  off  upon  receipt  of  a  "resume   transmission"

command.

5.0  KEYBOARD-TO-TERMINAL PROTOCOL

5.1  Keycode Transmission

The  keyboard  is  responsible   for   transmitting   single-byte

keycodes  which  reflect  the  state of the keyboard matrix.  The

8-bit codes  above  64  are  used  for  keycodes.  Every  key  is

identified  by  its  unique  keycode.  There are no special codes

for shifted or control keys.

See Appendix I for the complete keycode matrix translation table.

5.2  Special Code Transmission

There are also 13 "special" codes, some  with  values  above  64,

some below.

Nine special codes have been defined  above  64,  (keycode  value

range):

1. ALL UPS		Keycode 179 Dec, B3 Hex

2. METRONOME CODE	Keycode 180 Dec, B4 Hex

3. OUTPUT ERROR		Keycode 181 Dec, B5 Hex

4. INPUT ERROR		keycode 182 Dec, B6 Hex

5. KBD LOCKED ACK	Keycode 183 Dec, B7 Hex

6. TEST MODE ACK	Keycode 184 Dec, B8 Hex

7. PREFIX TO KEYS DOWN  Keycode 185 Dec, B9 Hex

8. MODE CHANGE ACK 	Keycode 186 Dec, BA Hex

9. RESERVED 		Keycode 127 Dec, 7F Hex

Page 8

Four special codes have also been defined within the  "below  64"

value range.

1. KEYBOARD ID - FIRMWARE (01 Dec, 01 Hex)

2. KEYBOARD ID - HARDWARE (00)

3. KEY DOWN ON POWER UP ERROR CODE (61 Dec, 33 Hex)

4. POWERUP SELF-TEST ERROR CODE (62 Dec, 3E Hex)

METRONOME CODE indicates to the terminal  that  an  interval  has

passed,  a keyclick has been generated, and the last key received

by the terminal is still depressed.

ALL UPS indicates to the terminal that a  down/up  mode  key  was

just  released,  and that now there are no more down/up keys down

on the keyboard.

OUTPUT ERROR indicates to the terminal that the keyboard's output

buffer  overflowed  after receipt of a "keyboard inhibit" command

from the terminal, and that some keystrokes may have been lost.

INPUT ERROR CODE indicates to  the  terminal  that  the  keyboard

received a garbage command, or too many or too few parameters.

KEYBOARD LOCKED CONFIRMATION indicates to the terminal  that  the

keyboard received an "inhibit transmission" command. (see section

6.3)

TEST MODE ACKNOWLEDGE indicates that the keyboard has  gone  into

TEST  MODE.  This  is  a  special  mode inserted for a production

bed-of-nails tester.  If the terminal receives  this  acknowledge

unsolicited,  it  should send "80 HEX", which will terminate test

mode and jump to powerup.

MODE CHANGE ACKNOWLEDGE indicates that the keyboard has  received

and processed a mode change command. (see section 6.4)

PREFIX TO KEYS down indicates that the next byte is a keycode for

a  key  already  down  in  a  division  which has been changed to

down/up. (see Section 6.4)

Keycode 7F is reserved for internal use.

KEYBOARD ID is a two byte identification code, transmitted  after

the  powerup  self-test  is performed.  (see section 5.3) it will

also be sent upon request from the terminal (see section 6.3)

POWER UP SELF TEST ERROR CODE indicates to the terminal that  the

ROM or RAM self-test of the terminal failed. (see section 5.3)

The keyboard may be locked by command  from  the  terminal.  Upon

receiving  the  "inhibit transmission" command, the keyboard will

light the "Keyboard  locked"  LED.  It  will  then  transmit  the

KEYBOARD  LOCKED  ACKNOWLEDGE  special  code.   The keyboard will

attempt to buffer characters (up to four) until its output buffer

overflows.  Then  it  will  stop  scanning  its  matrix. When the

keyboard is  subsequently  unlocked  by  the  terminal,  it  will

transmit  all  bytes  collected  in  its  buffer, followed by the

"OUTPUT ERROR," special code if the buffer overflowed  while  the

keyboard was locked.

Page 9

5.3  Power Up transmission

Upon powerup, the LK200 will perform a self test and transmit the

results of that self-test to the terminal in 4 bytes.

BYTE 1:   KBID (firmware) - the  keyboard  ID  which   is

stored in firmware

BYTE 2:   KBID (hardware) - the keyboard ID which is read

from jumpers in the hardware

BYTE 3:   ERROR or O - 	    Two  error   codes   indicate

either  a)  failure of ROM or

RAM  self-test   within   the

processor (3E HEX) b) keydown

on powerup (3E HEX)

BYTE 4:   KEYCODE or O -    the keyboard does a  complete

scan   for   keys   down   on

powerup.   There  should   be

none.   If   there  are,  the

error code  (above)  will  be

set    and    first   keycode

detected will be sent as  the

fourth byte.

If  the  ROM  self-test  (checksum)  fails,  the  error  may   be

catastrophic. 	The  keyboard  may  be unable to send anything at

all.

If the keyboard finds a key down  on  the  first  scan,  it  will

continue to look for an "all key UP" condition.  If the offending

key(s) is  (are)  released,  then  the  keyboard  will  send  the

corrected  4-byte  powerup  sequence  (unsolicited).  This  is an

attempt  to  avoid  a  fatal  error   condition   when   a   user

inadvertently holds down a key while powering up.

The keyboard LED's are all lit during powerup. If  the  self-test

passes,  the  keyboard will turn all LED's off.  The terminal may

transmit a "sound bell" command to the  keyboard  if  a  bell  is

desired  upon  powerup.  This  should  not be done until the last

byte of the four-byte sequence is received by the terminal.  This

will test the serial line and terminal connection.

Powerup self test will not exceed 70 ms.

A jump to powerup can be requested at any time from the terminal.

Please  note  that  a jump to powerup will cause the LED's on the

keyboard to "blink" on and off once.

6.0  TERMINAL-TO-KEYBOARD PROTOCOL.

The terminal can control both the peripherals associated with the

Keyboard  and some of the keyboard transmit characteristics.  The

protocol for the transmission of commands and parameters from the

terminal to the Keyboard is as follows:

Page 10

6.1  Commands

There are two types of commands--  those  that  control  keyboard

transmission  characteristics  and  those  that  control keyboard

peripherals.  The LOW bit of the command is the "TYPE flag".   It

is  clear  if the command is a TRANSMISSION command. It is set if

the command is a PERIPHERAL command.

TRANSMISSION COMMANDS		PERIPHERAL COMMANDS    		_____________________		___________________

mode set			flow control

autorepeat rate set		led

audio

keyboard ID

reinitiate kbd

some autorepeat control

jump to test mode

reinstate defaults

The HIGH order bit of every command is the "PARAMS" flag.  It  is

clear  if  there ARE any parameters to follow. It is set if there

are NO parameters.

6.2 Parameters

The HIGH order bit of every parameter is the  "PARAMS  flag".  It

is clear if there are more parameters to follow. It is set on the

last parameter.  The remaining 7 bits of the  parameter  are  for

data.

COMMAND SUMMARY (with parameters)

7    6   5   4   3    2   1    0

-------------------------------------

COMMAND:	| O | representation  | mode or|type|

|   |		      | off/on |    |

-------------------------------------

PARAMETERS:	-------------------------------------

| O |	 	data	            |

-------------------------------------

-------------------------------------

| 1 |		data		    |

-------------------------------------

Page 11

6.3  Peripheral Commands

Command			  	Representation		_______			  	______________

Flow Control			     0001

LED				     0010

Keyclick			     0011

Bell				     0100

KBD ID				     0101

Reinitiate keyboard		     0110

to defaults (Jmp to PWRUP)

Keyclick for CTRL key		     0111

Temporarily inhibit autorepeat	     1000

Jump to test mode 		     1001

Table I

To send a "Peripheral" type command, set  TYPE  flag  (low  order

bit).  Bits 6 - 3 contain a COMMAND representation from the above

chart. Bits 2 and 1 specify on (01), off  (00),  or  sound  (11).

Bit 7 should be set if there are no parameters to follow.

Page 12

The following chart summarizes the peripheral commands in HEX.

-------------------------------------------------------

|   FUNCTION		 |   HEX   |   PARAMETERS   |

|============================|=========|================|

| Flow control 		 |	   |		    |

|----------------------------|---------|----------------|

| resume kbd xmission	 |   8B    |      NONE      |

| inhibit kbd xmission       |   89    |      NONE      |

|----------------------------|---------|----------------|

| LED's			 |	   |		    |

|----------------------------|---------|----------------|

| light LED(s)		 |   13    |  Bit pattern   |

| turn off LED(s)	         |   11    |  Bit pattern   |

|			         |	   |		    |

|----------------------------|---------|----------------|

| Audio		         |	   |		    |

|----------------------------|---------|----------------|

| disable keyclick	         |   99    |      NONE      |

| enable click, set vol      |   1B    |      volume    |

| disable CTRL keyclick      |   B9    |      NONE      |

| enable CTRL keyclick       |   BB    |      NONE      |

| sound keyclick	         |   9F    |      NONE      |

| disable bell	         |   A1    |      NONE      |

| enable bell, set vol       |   23    |      volume    |

| sound bell		 |   A7    |      NONE      |

|			         |	   |		    |

|----------------------------|---------|----------------|

| Autorepeat		 |	   |		    |

|----------------------------|---------|----------------|

| Temp. autorpt inhibit      |   C1    |      NONE      |

| Enable AR across kbd       |   E3    |      NONE      |

| Disable AR across kbd      |   E1    |      NONE      |

| Change all AR to down only |   D9    |      NONE      |

|                            |         |                |

|----------------------------|---------|----------------|

| Other                      |	   |                |

|----------------------------|---------|----------------|

| Request keyboard ID        |   AB    |      NONE      |

| Reinitiate keyboard        |   FD    |      NONE      |

| Jump to test mode          |   CB    |      NONE      |

| Reinstate defaults         |   D3    |      NONE      |

-------------------------------------------------------

Table II

6.3.1  FLOW CONTROL

The terminal can "lock" the keyboard with the  "inhibit  keyboard

transmission"   command.   When   the  keyboard  is  subsequently

unlocked, it will respond with an error code  if  any  keystrokes

may have been missed.  (see section 7.2)

Page 13

6.3.2  LED'S

The LED parameter can be summarized as follows:

---------------------------------------------------------------

|   1   |   0   |   0   |   0   | LED 4 | LED 3 | LED 2 | LED 1 |

---------------------------------------------------------------

The LED structure on the keyboard can be summarized as follows:

[O]     [O]     [0]     [0]

LED 4   LED 3   LED 2   LED 1

6.3.3  AUDIO

The audio volume  parameter can be summarized as follows:

---------------------------------------------------------------

|   1   |   0   |   0   |   0   |   0   |   3-bit volume        |

---------------------------------------------------------------

Volume levels are as follows:

000 - highest

001

010

011

100

101

110

111 - lowest

Either keyclick or bell (or both) may be disabled.  When keyclick

or  bell  is  disabled,  it  will not sound, even if the terminal

requests it.

6.3.4  AUTOREPEAT

TEMPORARY AUTOREPEAT INHIBIT - stop autorepeat for this key only.

Autorepeat   will   automatically  resume  when  another  key  is

depressed.

DISABLE/ENABLE   AUTOREPEAT   ACROSS   KEYBOARD   -    stop/start

transmission  of  metronome  codes  without  affecting autorepeat

timing or division settings.

CHANGE ALL AUTOREPEAT TO DOWN ONLY - change division settings for

all autorepeating divisions to down only.

6.3.5  OTHER

REQUEST KEYBOARD ID - keyboard  will  send  2-byte  keyboard  ID.

Keyboard will not jump to POWER UP.

Page 14

REINITIATE KEYBOARD - keyboard will jump to its POWERUP  routine.

Note:  terminal  should  not try to transmit anything to keyboard

until the last byte of the powerup sequence has been received.

JUMP TO TEST MODE - special test mode for production test

REINSTATE DEFAULTS - set the  following  functions  back  to  the

powerup default settings:

Division mode settings

Autorepeat interval and timeout rates

Autorepeat buffer selections

Audio volume

Control key keyclick

6.4  Mode Set Commands

6.4.1  Division Mode Setting

For an explanation of transmission modes and rates,  see  section

3.

Each DIVISION on the keyboard has a unique 4-bit  representation,

as follows:

Division			   Representation		________			   ______________

1. 48 graphic keys, Spacebar		0001

and B13, D13, D99 and E99 on

the typewriter mass

2. Numeric keypad			0010

3. Delete character			0011

4. Return(C13) and Tab(D00)		0100

5. Lock(C00), COMPOSE(A99) and A10	0101

6. Shift(B00 and B11) and CTRL (C99)	0110

7. Horizontal cursors(B16 and B18)	0111

8. Vertical cursors(B17 and C17)	1000

and A17

9. Six keys directly above the cursor	1001

keys(D16-D18 and E16-E18)

Function keys

10. G99-G04 and H99-H04			1010

11. G05-G09 and H05-H09			1011

12. G10-C14 and H10-H14			1100

13. G15-G16				1101

14. G20-G23 and H20-H23			1110

Table III

See Appendix IV for keyswitch designators.

Page 15

Each MODE has a unique 2-bit representation.

Modes			Representation		_____			______________

Down only		       00

Autorepeat, Down	       01

Down Up			       11

Table IV

To set key transmission mode on a particular  keyboard  division,

the  terminal must send the PARAMS flag, followed by the keyboard

division representation with the mode representation, followed by

the TYPE flag (cleared).

Example: (set main array to down/up)

|param|      division         |   mode    |type |

-----------------------------------------------

|  1  |  0     0     0     1  |  1     1  |  0  |

-----------------------------------------------

7     6     5     4     3     2     1     0

The PARAMS flag of the mode set command will be set to 1 if there

are  no  parameters.  The  params flag will be clear if there are

parameters.

6.4.2  Autorepeat Rate Buffer Association -

If the mode selected is autorepeat, the  terminal  may  elect  to

transmit  a  parameter  to  change  the buffer association of the

selected division.  (See appendix II for defaults)

Example: (set main array  to  autorepeat,  change  buffer

association to buffer 3)

|param|      division         |   mode    |type |

-----------------------------------------------

|  0  |  0     0     0     1  |  0     1  |  0  |

-----------------------------------------------

|  1  |  0     0     0     0     0     1  |  0  |

-----------------------------------------------

7     6     5     4     3     2     1     0

6.4.3  Autorepeat Rate Buffer values

At keyboard powerup time, the four autorepeat rate  buffers  will

contain  default  values.  (see section 3.6, defaults in appendix

II) The terminal may change these values, as shown below.

In the command byte, bit 7 (PARAMS flag) should  be  clear.  Bits

6-3 should be 1111 (to indicate that this is a RATE SET command).

Bits 2-1 should be the buffer number (O to 3).  Bit 0 (TYPE flag)

should  be  clear.  There  should be two parameters, carrying the

rate set data.

Page 16

Example: change rates in buffer 3

|param|  rate change command  | buffer #  |type |

-----------------------------------------------

|  0  |  1     1     1     1  |  1     1  |  0  |

-----------------------------------------------

|  0  |           Param 1  (timeout)            |

-----------------------------------------------

|  1  |           Param 2  (interval)           |

-----------------------------------------------

7     6     5     4     3     2     1     0

The first parameter should specify TIMEOUT to be  stored  in  the

selected buffer.  The second parameter should specify INTERVAL.

1.  TIMEOUT

This number represents the amount of  time  the  keyboard

will wait before autorepeating a character.

2.  INTERVAL

This number is the effective  rate  at  which  characters

should be transmitted, i.e. once very X ms.

For example, to set the autorepeat rate to be stored in buffer 1,

terminal  firmware  will transmit: 00000011 followed by two bytes

of numeric parameters, as follows:

The Autorepeat TIMEOUT will be the transmitted  number,  times  5

ms.  i.e.  In  order  to specify a rate of 5 ms. delay, the first

parameter received would be 00000001.  The maximum allowed is 630

ms.,  or 01111110.  THE TERMINAL MUST NOT SEND 631 - or 01111111.

This code is reserved for internal use of  the  keyboard.  00  is

also an illegal value.  Autorepeat TIMEOUT will be implemented as

a multiple of 8.33 ms., which is  the  keyboard's  internal  scan

rate.  For this reason, TIMEOUT rates can vary +/- 4.15 ms.

The Autorepeat INTERVAL will be the number of metronome codes per

second.  In  order  to  specify  a  speed of 16 hertz, the second

parameter received would be 10010000.  (note that the high  order

bit  is set because it is the last parameter).  The highest which

may be sent is 126, or 11111100.  THE TERMINAL MUST NOT SEND  127

-  or 11111101.  This code is the PWRUP command.  The lowest rate

which can be implemented by the keyboard is 12 hz.  Values as low

as 1 may be transmitted, but they will be translated to 12 hz.

7.0  SPECIAL CONSIDERATIONS

7.1  Error Handling.

There are 4 error codes.  The first 2 are sent at power  up  time

if self-test fails.  (see section 5.3)

Page 17

The other two are the INPUT error code and the OUTPUT error code.

OUTPUT  error  (B6  HEX)  is  sent ONLY upon receipt of a "resume

transmission" command, if the output buffer overflowed while  the

keyboard was locked.

INPUT error (B6 HEX) is sent whenever the Keyboard detects  noise

(unidentified command or parameter) on the line.  B6 is also sent

if the keyboard detects  a  delay  of  more  than  100  ms.  when

expecting a parameter.

7.2 Keyboard Locked Condition

When the keyboard receives a command to "inhibit transmission" it

will  transmit one more byte.  This is a special code to indicate

that the Keyboard is locked.  If the terminal receives this  code

unsolicited,  it  is  an  indication  that  noise on the line was

interpreted as  an  "inhibit  transmission"  command.  It  should

immediately  send the "resume transmission" command to unlock the

keyboard.

Due to limited RAM within the keyboard, the output FIFO is  small

(4 bytes).  When the Keyboard is locked it will attempt to buffer

characters received from  the  keyboard.  When  the  Keyboard  is

subsequently  unlocked it will transmit all 4 bytes in its output

buffer.  If any  keystrokes  have  been  missed,  due  to  buffer

overflow,  the  keyboard will transmit an error code as the fifth

byte.  Any keys which were not transmitted and  are  still  being

held  down  when  the keyboard is "unlocked" will be processed as

new  keys.  Therefore,  an  error  code  upon  keyboard  "unlock"

indicates to the terminal a POSSIBLE loss of keystrokes.

The keyboard stops scanning its matrix while its buffer is  full.

It does, however, process all incoming commands.

7.3  Reserved Code

The number 7F (hex) is reserved  for  the  internal  use  of  the

keyboard input and output buffers handling routines.

7.4  Test mode

The keyboard will jump into a  test  mode  by  command  from  the

QV/DIAGNOSE bed-of-nails tester used in production test.  It will

transmit a special code to the terminal to confirm test mode.  If

a  terminal  receives  this  code unsolicited, it should send the

byte 80(HEX) to resume.  (this will cause a jump to powerup).

Page 18

7.5 Future Expansion

In order to provide for expansion of functionality, the following

keycodes  have  been  reserved for future use as special codes or

keycodes, and should be ignored if received by  a  terminal  that

does not support them:

decimal		hex		decimal		hex		_______		___		_______		___

110		56E		173		AD

122		7A		187		BB

165		A5		218		DA

156		A6		228		E4

172		AC

Page 19

8.0  APPENDIX I -- KEYCODE TRANSLATION TABLE

Keycodes 00 thru 64 are reserved. keycodes 65 thru 85 are unused.

Please note that the legends given under "Keycap" are taken  from

the  keycap  legends  of the LK201AA (American), as known at this

time.  They are  provided  for  convenience  only,  and  are  not

necessarily  complete  or  up  to  date.  Those keys which do not

exist on the LK201 are labeled "unused".  Key  positions  may  be

referenced in Appendix IV.

-----------------------------------------------------------------

DIVISION  POSITION 	  KEYCAP              KEYCODE   KEYCODE

(see pg 30)	     		       (dec) 	 (hex)

-----------------------------------------------------------------

=================================================================

Function Keys

-----------------------------------------------------------------

10	    G99		HOLD SCREEN 		086	   56

G00		PRINT SCREEN 	        087	   57

G01 	BREAK			088	   58

G02		SETUP			089	   59

G03		F5			090	   5A

G04		unused			091	   5B

H99		unused			092	   5C

H00		unused			093	   5D

H01		unused			094	   5E

H02		unused			095	   5F

H03		unused			096	   60

H04		unused			097	   61

098	   62

-----------------------------------------------------------------

099	   63

11       G05		INTERRUPT		100	   64

G06		RESUME			101	   65

G07		CANCEL			102	   66

G08		MAIN SCREEN		103	   67

G09		EXIT			104	   68

H05		unused			105	   69

H06		unused			106	   6A

H07		unused			107	   6B

H08		unused			108	   6C

H09		unused			109	   6D

---    reserved for future use	110	   6E

-----------------------------------------------------------------

111	   6F

12	    G10 	unused			112	   70

G11 	F11 (ESC)		113	   71

G12 	F12 (BS)		114	   72

G13 	F13 (LF)		115	   73

G14		ADDITIONAL OPTIONS  	116	   74

H10 	unused			117	   75

H11 	unused			118	   76

H12 	unused			119	   77

H13 	unused			120	   78

H14 	unused			121	   79

---    reserved for future use	122	   7A

Page 20

-----------------------------------------------------------------

DIVISION  POSITION 	  KEYCAP              KEYCODE   KEYCODE

(see pg 30)	     		       (dec) 	 (hex)

-----------------------------------------------------------------

123 	   7B

13	    G15 	HELP 			124	   7C

G16 	DO 			125 	   7D

-----------------------------------------------------------------

126	   7E

---    RESERVED FOR INTERNAL USE 	127	   7F

14	    G20		F17			128	   80

G21		F18			129	   81

G22		F19			130	   82

G23		F20			131	   83

H20		unused 			132	   84

H21		unused			133	   85

H22		unused			134	   86

H23		unused			135	   87

=================================================================

6 basic editing keys

-----------------------------------------------------------------

136 	   88

137 	   89

9	    E16 	FIND 			138	   8A

E17 	INSERT HERE 		139	   8B

E18 	REMOVE 			140 	   8C

D16 	SELECT 			141 	   8D

D17 	PREV SCREEN 		142	   8E

D18 	NEXT SCREEN 		143	   8F

144	   90

=================================================================

Keypad

-----------------------------------------------------------------

145	   91

2	    A20		0			146	   92

A21		unused			147	   93

A22		.			148	   94

A23		ENTER			149	   95

B20 	1			150	   96

B21 	2 		 	151 	   97

B22 	3 			152	   98

C20 	4			153	   99

C21 	5 			154 	   9A

C22 	6 			155 	   9B

C23 	, 			156 	   9C

D20 	7 			157 	   9D

D21 	8 			158 	   9E

D22 	9 			159 	   9F

D23 	- 			160 	   A0

E20 	PF1 			161 	   A1

E21 	PF2 			162 	   A2

E22 	PF3 			163 	   A3

E23 	PF4 			164 	   A4

---    reserved for future use 	165 	   A5

Page 21

-----------------------------------------------------------------

DIVISION  POSITION 	  KEYCAP              KEYCODE   KEYCODE

(see pg 30)	     		       (dec) 	 (hex)

-----------------------------------------------------------------

=================================================================

Cursor Keys

-----------------------------------------------------------------

---    reserved for future use 	166 	   A6

7	    B16 	LEFT 			167 	   A7

B18 	RIGHT 			168 	   A8

-----------------------------------------------------------------

8	    B17		DOWN			169	   A9

C17		UP			170	   AA

A17		unused			171	   AB

---	   reserved for future use	172	   AC

=================================================================

Shift, Lock, CTRL, A99 and A10

-----------------------------------------------------------------

---    reserved for future use 	173 	   AD

6 	    B99,B11 	SHIFT	 		174 	   AE

C99 	CTRL 			175 	   AF

-----------------------------------------------------------------

5	    C00		LOCK			176	   B0

A99		COMPOSE			177	   B1

A10		unused			178	   B2

=================================================================

Special Codes

-----------------------------------------------------------------

---		all ups			179	   B3

---		metronome		180	   B4

---		output error		181	   B5

---		input error		182	   B6

---		KBD LOCKED acknowledge	183	   B7

---		TEST MODE acknowledge	184	   B8

---		PREFIX to keys down	186	   B9

---		MODE CHANGE acknowledge 185	   BA

---	   reserved for future use 	187	   BB

=================================================================

Delete

-----------------------------------------------------------------

3 	    E13 	DELETE(X) 		188 	   BC

=================================================================

Return and Tab

-----------------------------------------------------------------

4 	    C13 	RETURN 			189 	   BD

D00		TAB			190	   BE

Page 22

-----------------------------------------------------------------

DIVISION  POSITION 	  KEYCAP              KEYCODE   KEYCODE

(see pg 30)	     		       (dec) 	 (hex)

-----------------------------------------------------------------

=================================================================

48 graphics keys, space

B13, D13, D99 and E99 on

the main typewriter mass.

-----------------------------------------------------------------

1	    E00		TILDE `			191	   BF

E01		! 1			192	   C0

D01		Q			193	   C1

C01		A			194	   C2

B01		Z			195	   C3

D99		unused 			196	   C4

E02		@ 2			197	   C5

D02		W			198	   C6

C02		S			199	   C7

B02		X			200	   C8

B00		> <			201	   C9

E99		unused			202	   CA

E03		# 3 			203	   CB

D03		E			204	   CC

C03		D			205	   CD

B03		C			206	   CE

207	   CF

E04		$ 4			208	   D0

D04		R			209	   D1

C04		F			210	   D2

B04		V			211	   D3

A01-A09	SPACE			212	   D4

213	   D5

E05		% 5			214	   D6

D05		T			215	   D7

C05		G			216	   D8

B05		B			217	   D9

---    reserved for future use	218	   DA

E06		^ 6			219	   DB

D06		Y			220	   DC

C06		H			221	   DD

B06		N			222	   DE

unused			223	   DF

E07		& 7			224	   E0

D07		U			225	   E1

C07		J			226	   E2

B07		M			227	   E3

---    reserved for future use	228	   E4

E08		* 8			229	   E5

D08		I			230	   E6

C08		K			231	   E7

B08		,,			232	   E8

unused			233	   E9

E09		( 9			234	   EA

D09		O			235	   EB

C09		L			236	   EC

B09		..			237	   ED

unused			238	   EE

Page 23

-----------------------------------------------------------------

DIVISION  POSITION 	  KEYCAP              KEYCODE   KEYCODE

(see pg 30)	     		       (dec) 	 (hex)

-----------------------------------------------------------------

1	    E10		) 0			239	   EF

D10		P			240	   F0

D13		unused			241	   F1

C10		: ;			242	   F2

B10		? /			243	   F3

unused			244	   F4

E12		+ =			245	   F5

D12		} ]			246	   F6

C12		, \			247	   F7

unused			248	   F8

E11		_ -			249	   F9

D11		{ [			250	   FA

C11		" '			251	   FB

B13		unused			252	   FC

unused			253	   FD

unused			254	   FE

unused			255	   FF

Page 24

9.0  APPENDIX II - DEFAULTS

The  following  defaults  have  been  established  regarding  key

transmission modes:

Keyboard division	   Mode		AR buffer assoc		_________________	   ____		_______________

Main Array		autorepeat	      0

keypad			autorepeat	      0

Del			autorepeat	      1

Return and Tab		down only

Function kevs		down only

lock,compose,A10	down only

shift,ctrl		down/up

cursor keys		autorepeat	      1

6 Basic Editing Keys	down only

Table V

Effective Rates stored in Autorepeat buffers:

TIMEOUT	      INTERVAL				_______	      ________

Buffer 0:	530 ms.		30 hz

Buffer 1:	300 ms.		30 hz

Buffer 2:	500 ms.		40 nz

Buffer 3:	300 ms.		40 hz

Audio volume: Both keyclick and bell volumes will  be  2  decimal

(010 binary) by default.

The key in position C99 of the keyboard in Appendix IV (the  CTRL

key in the LK201) will NOT GENERATE a click unless enabled by the

terminal.  The keys in position B99 and B11 (SHIFT  keys  on  the

LK201) will NEVER generate a click.

Page 25

10.0  APPENDIX III - KEYBOARD MATRIX

This is the current matrix for the LK200  keyboard.  Keys  F1-F16

and  F35-F38  are  not physically possible on the CT100 keyboard.

For physical key locations see the bottom line in each  location.

Please  note that the legends given are from the LK201AA as known

at this time.  They are provided for convenience  only,  and  are

not  necessarily complete or up to date.  Key positions which are

non-existent on the LK201 are marked "unused".

COLUMN 			 ROW

|     7       6       5       4      3        2       1       0

v -----------------------------------------------------------------

17 |unused |  F19  |unused |  F20  |  PF4  |   N-  |   N,  | ENTER |

|       |       |       |       |       |       |       |       |

|  H23  |  G22  |  H22  |  G23  |  E23  |  D23  |  C23  |  A23  |

-----------------------------------------------------------------

16 |  F18  |  PF3  |unused |   N9  |   |   |   N6  |   N3  |   N.  |

|       |       |       |       |   v   |       |       |       |

|  G21  |  E22  |  H21  |  D22  |  B17  |  C22  |  B22  |  A22  |

-----------------------------------------------------------------

15 |  F17  |  PF2  |unused |   N8  |   N5  |  -->  |   N2  |   NO  |

|	|	|	|	|	|	|	| RIGHT |

|  G20  |  E21  |  H20  |  D21  |  C21  |  B18  |  B21  |  A21  |

-----------------------------------------------------------------

14 |  PF1  |next   |remove |   up  |   N7  |   N4  |   N1  |   NO  |

|	| screen|	|	|	|	|	|  LEFT |

|  E20  |  D18  |  E18  |  C17  |  D20  |  C20  |  B20  |  A20  |

-----------------------------------------------------------------

13 |insert | _ 	|  do 	|prev 	| { 	| "	|unused |unused |

|   here|     - |	| screen|     [	|     '	| 	|	|

|  E17 	|  E11 	|  G16  |  D17 	|  D11 	|   C11 |  B13 	|  A17  |

-----------------------------------------------------------------

12 | find  | +	| help  |select | } 	| RETURN|  <--  | |	|

|	|     =	|	|	|     ]	| (bot)	|	|     \	|

|  E16 	|  E12 	|  G15  |  D16 	|  D12  |  C13  |  B16  |  C12  |

-----------------------------------------------------------------

11 |addtnl | DELETE|unused | ) 	|   P 	| RETURN| : 	| ? 	|

|options|   (X) |	|     O |	| (top) |     ;	|     / |

|  G14  |  E13  |  H14  |  E10  |  D10  |  D13  |  C10  |  B10  |

-----------------------------------------------------------------

10 |unused |  F12  |  F14  |  F13  | ( 	| O 	| L 	| .	|

|	|	|	|	|     9	|	|	|     . |

|  H13  |  G12  |  H12  |  G13  |  E09  |  D09  |  C09  |  B09  |

-----------------------------------------------------------------

Page 26

COLUMN 			 ROW

|     7       6       5       4      3        2       1       0

v -----------------------------------------------------------------

9 |unused |  F11	|unused |unused | *	|   I 	|   K	| , 	|

|	|	|	|	|     8	|	|	|     ,	|

|  H11  |  G11	|  H10  |  G10  |  E08  |  D08  |  C08  |  B08	|

-----------------------------------------------------------------

8 |unused |main 	|unused | exit 	| & 	|   U   |   J 	|   M	|

|	| screen|	|	|     7	|	|	|	|

|  H09  |  G08 	|  H08 	|  G09 	|  E07 	|  D07	|  C07 	|  B07	|

-----------------------------------------------------------------

7 |unused |cancel |unused |resume | ^ 	|   Y	|   H 	|   N	|

|	|	|	|	|     6	|	|	|	|

|  H07  |  G07  |  H06  |  G06 	|  E06 	|  D06 	|  C06 	|  B06	|

-----------------------------------------------------------------

6 |unused |unused |unused |inter- | %	|   T	|   G	|   B   |

|	|	|	|  rrupt|     5	|	|	|	|

|  H05  |  G04 	|  H04 	|  G05 	|  E05 	|  D05 	|  C05  |  B05	|

-----------------------------------------------------------------

5 | setup	|  F5 	|unused | $ 	|   R 	|   F 	|   V 	| SPACE |

|	|	|	|     4	|	|	|	|	|

|  G02	|  G03 	|  H03 	|  E04 	|  D04 	|  C04	|  B04	|A01-A09|

-----------------------------------------------------------------

4 |unused |print 	|unused | break | # 	|   E 	|   D 	|   C	|

|	| screen| 	|	|     3 |	|	|	|

|  H02 	|  G00 	|  H01 	|  G01 	|  E03 	|  D03 	|  C03 	|  B03	|

-----------------------------------------------------------------

3 |hold 	| @	|unused |  TAB 	|   W 	|   S 	|   X 	| > 	|

| screen|     2 |	|	|	|	|	|     < |

|  G99 	|  E02  |  H00 	|  D00	|  D02 	|  C02 	|  B02 	|  B00	|

-----------------------------------------------------------------

2 |unused |unused |unused | ~ 	| ! 	|   Q 	|   A 	|   Z	|

| 	|	|	|     `	|     1 | 	|	|	|

|  E99 	|  D99 	|  H99  |  E00	|  E01 	|  D01  |  C01  |  B01 	|

-----------------------------------------------------------------

1 | CTRL 	|  LOCK |COMPOSE|unused |	|	|	|	|

|	|	|	|	|	|	|	|	|

|  C99 	|  C00	|  A99  |  A10 	|	|	|	|	|

-----------------------------------------------------------------

0 | SHIFT |		        DO NOT USE THIS AREA		|

|	|			(Phantom key avoidance)		|

|B99,B11|							|

-----------------------------------------------------------------

NO of the numeric keypad is layed out such that it can be  broken

into  two  keys.  Normally  only  the NO LEFT is implemented as a

double  size  key.  Thus;  that  is  the  code  which  should  be

considered  for  that  double key.  The interpretation of the new

key formed by splitting the double key has yet to be determined.

The RETURN key is also laid out so as to be able to split it into

two keys.  The one which is decoded as return is the RETURN (bot)

key.  If this key is divided the meaning of the  top  portion  is

TBD.

Page 27

11.0  APPENDIX IV - KEYSWITCH LAYOUT

Keyswitch Layout, page 1 of 4.

------------------------------------------------------------------>

|unused |unused |unused |unused |unused |unused |unused |unused |

|       |       |       |       |       |       |       |       |

|  H99  |  H00  |  H01  |  H02  |  H03  |  H04  |  H05  |  H06  |

------------------------------------------------------------------>

------------------------------------------------------------------>

| HOLD  | PRINT | SETUP |  F4   | BREAK |unused |  F6   |  F7   |

|SCREEN |SCREEN |       |       |       |       |       |       |

|  G99  |  G00  |  G01  |  G02  |  G03  |  G04  |  G05  |  G06  |

------------------------------------------------------------------>

------------------------------------------------------------------>

| un- |  ~ `  |  ! 1  |  @ 2  |  # 3  |  $ 4  |  % 5  |  ^ 6  |

| used|       |       |       |       |       |       |       |

| E99 |  E00  |  E01  |  E02  |  E03  |  E04  |  E05  |  E06  |

-------------------------------------------------------------------

| un- |    TAB    |   Q   |   W   |   E   |   R   |   T   |   y   |

| used|           |       |       |       |       |       |	 |

| D99 |    D00    |  D01  |  D02  |  D03  |  D04  |  D05  |  D06  |

-------------------------------------------------------------------

| CTRL  |    LOCK   |   A   |   S   |   D   |   F   |   G   |

|       |	   |       |       |       |       |       |

|  C99  |    C00    |  C01  |  C02  |  C03  |  C04  |  C05  |

------------------------------------------------------------------>

|     SHIFT     |  > <  |   Z   |   X   |   C   |   V   |   B   |

|               |       |       |       |       |       |       |

|      B99,     |  B00  |  B01  |  B02  |  B03  |  B04  |  B05  |

------------------------------------------------------------------>

|    COMPOSE    |			SPACE

|		 |

|      A99      |                    A01-A09

-------------------------------------------------------->

Page 28

Keyswitch Layout, page 2 of 4.

<--------------------------------------------------------------------

|unused |unused |unused |unused |unused |unused |unused |unused |

|       |       |       |       |	  |	  |	  |	  |

|  H07  |  H08  |  H09  |  H10  |  H11  |  H12  |  H13  |  H14  |

<--------------------------------------------------------------------

<--------------------------------------------------------------------

|  F8   |  F9   |  F10  |unused |  F11  |  F12  |  F13  |  F14  |

|       |       |	  |       |  ESC  |  BS   |  LF   |	  |

|  G07  |  G08  |  G09  |  G10  |  G11  |  G12  |  G13  |  G14  |

<--------------------------------------------------------------------

<--------------------------------------------------------------

|  & 7  |  * 8  |  ( 9  |  ) 0  |  _ -  |  + =  |  DELETE   |

|       |       |       |       |       |       |           |

|  E07  |  E08  |  E09  |  E10  |  E11  |  E12  |  E13      |

<----------------------------------------------------------------

|   U   |   I   |   O   |   P   |  { [  |  } ]  | RETURN  |

|       |       |       |       |       |       |	      |

|  D07  |  D08  |  D09  |  D10  |  D11  |  D12  |   D13   |

---------------------------------------------------------       |

|   H   |   J   |   K   |   L   |  : ;  |  " '  |  | \  |       |

|       |       |       |       |       |       |       |       |

|  C06  |  C07  |  C08  |  C09  |  C10  |  C11  |  C12  |  C13  |

-----------------------------------------------------------------

|   N   |   M   |  , ,  |  . .  |  ? /  |     SHIFT     |unu|

|       |       |       |       |       |               |sed|

|  B06  |  B07  |  B08  |  B09  |  B10  |      B11      |B13|

<----------------------------------------------------------------

SPACE		    |     unused      |

|                 |

A01-A09		    |       A10       |

<--------------------------------------------------------

Page 29

Keyswitch Layout, page 3 of 4.

-------------------------

| HELP  |      DO       |

|       |               |

|  G15  |      G16      |

-------------------------

-------------------------

| FIND  |INSERT |REMOVE |

|       | HERE  |       |

|  E16  |  E17  |  E18  |

-------------------------

|SELECT | PREV  | NEXT  |

|       | SCREEN| SCREEN|

|  D16  |  D17  |  D18  |

-------------------------

|  UP   |

|       |

|  C17  |

-------------------------

| LEFT  | DOWN  | RIGHT |

|       |       |       |

|  B16  |  B17  |  B18  |

-------------------------

|unused |

|       |

|  A17  |

---------

Page 30

Keyswitch Layout, page 4 of 4.

---------------------------------

|unused |unused |unused |unused |

|       |       |       |       |

|  H20  |  H21  |  H22  |  H23  |

---------------------------------

---------------------------------

|  F17  |  F18  |  F19  |  F20  |

|       |       |       |       |

|  G20  |  G21  |  G22  |  G23  |

---------------------------------

---------------------------------

|  PF1  |  PF2  |  PF3  |  PF4  |

|       |       |       |       |

|  E20  |  E21  |  E22  |  E23  |

---------------------------------

|   7   |   8   |   9   |   -   |

|       |       |       |       |

|  D20  |  D21  |  D22  |  D23  |

---------------------------------

|   4   |   5   |   6   |   ,   |

|       |       |       |       |

|  C20  |  C21  |  C22  |  C23  |

---------------------------------

|   1   |   2   |   3   | ENTER |

|       |       |       |       |

|  B20  |  B21  |  B22  |       |

-------------------------       |

|   0   |unused |   .   |       |

|       |       |       |       |

|  A20  |  A21  |  A22  |  A23  |

---------------------------------