MMV12.DOC

Master Menu Software Installation Guide

Version 1.2

DISCLAIMER

This document file was created by scanning  the

original  document and then editing the scanned

text.  As much as possible, the  original  text

format  was restored.  Some format changes were

made to insure this  document  would  print  on

current laser printers using 60 lines per page.

The  original  spelling  and  grammar  has been

preserved.

Figures of the  DECmate  hardware  showing  the

keyboard,   floppy   drives,   etc.  cannot  be

reproduced with the  ASCII  character  set  and

are not included in this document.

1-Oct-1996

DECmate (TM)

___________________________________________________________

Master Menu Software

Installation Guide			        Version 1.2

Master Menu Software Installation Guide

AA-HV22A-TV

May, 1986

The information in this document is subject to change without notice

and should not be construed as a commitment by Digital Equipment

Corporation.  Digital Equipment Corporation assumes no responsibility

for any errors that may appear in this document.

The software described in this document is furnished under a license

and may be used or copied only in accordance with the terms of such

license.

Digital Equipment Corporation assumes no responsibility for the use or

reliability of its software on equipment that is not supplied by

DIGITAL or its affiliated companies.

Copyright (C) 1986 by Digital Equipment Corporation. All Rights

Reserved.

The following are trademarks of Digital Equipment Corporation:

digital(TM)	  MASSBUS	  RSX

ALL-IN-1	  DIBOL		  RT

DEC		  PDP		  UNIBUS

DECmate	  P/OS		  VAX

DECspell	  Professional	  VMS

DECUS		  Rainbow	  VT

DECwriter	  RSTS		  Work Processor

CP/M is a trademark of Digital Research Inc.

MS is a trademark of Microsoft Corporation.

A NOTE ON PRODUCTION:  This document was prepared on a DECmate II

using DECmate/WPS version 2.2 software, and printed on the LN03 laser

printer.  Text was printed using the Compugraphic CG Times font

cartridge (LN03XX-CB).  Heading and titles were printed using the CG

Triumvirale font cartridge (LN03XX-CJ).  The book was sent for

publication from the LN03 without enhancement or alteration except for

the addition of line art.

DIGITAL EQUIPMENT CORPORATION

Merrimack, New Hampshire

ii

CONTENTS

______________________________________________________________________

Preface

New Features for Version 1.2  v

Intended Audience  v

How to Use This Book  v

Conventions  vi

Reference Manuals  vii

1  Introduction

Difference Between DECmate II and DECmate III Plus

Diskette Drive Operations  1-1

Conventions Used to Describe Diskette Drive Operations  1-2

Special Function Keys  1-3

Required Software  1-3

Handling Diskettes  1-4

2  Installing Master Menu

Installing Master Menu  2-1

3  Backup Diskette to Diskette With Master Menu

Backup Installation Software  3-1

D - Backup Diskette to Diskette  3-1

Figures

1-1   RX50 Diskette Drive Doors on DECmate II System  1-2

1-2   RX33 Diskette Drive Doors on DECmate III Plus System  1-2

1-3   CANCEL and MAIN SCREEN Keys on the DECmate Keyboard  1-3

2-1   Master Menu Install Menu  2-2

3-1   Hard Disk Services Menu  3-2

3-2   Fixed Menu Screen in Master Menu  3-4

iii

PREFACE

___________________________________________________________________

Product		DECmate Master Menu Install

Version		1.2

The Master Menu Installation Guide describes how to install Master

Menu on a DECmate system with a hard disk drive.

NOTE: Master Menu Install version 1.2 uses one diskette drive to

install Master Menu software.  The installation procedure

described in this book replaces the one described in chapter 3

of the Master Menu User's Guide under Section "INST - Install

Master Menu."

NOTE: If you have the DECmate III Plus system, you cannot use the COPY

option to make a backup copy of Master Menu Install diskettes A

and B.  Refer to Chapter 2 in this manual for instructions on

installing Master Menu.  Refer to Chapter 3 for instructions on

making backup copies using a single diskette drive.

The COPY option is described in the Master Menu User's Guide in

Chapter 2 and in Chapter 3 under the section "COPY - Make Copies

of the Installation Diskettes.  The COPY option requires two

diskette drives."

iv

PREFACE

___________________________________________________________________

New Features for Version 1.2

Master Menu version 1.2 has a new option in the Hard Disk Services

menu. The D (Backup diskette to diskette) option makes backup copies

of software in Master Menu using a single diskette drive.  This option

is described in Chapter 3, "Diskette to Diskette With Master Menu."

Intended Audience

This book is written for first-time users and experienced users who

want to install Master Menu on a DECmate system.  The book covers both

first-time installation and updates to existing Master Menu software.

How to Use This Book

This book is organized into the following chapters:

o  Chapter 1 describes the necessary hardware and software.

o  Chapter 2 explains how to install Master Menu.

o  Chapter 3 explains how to make backup copies using the Master

Menu D option (backup diskette to diskette).

v

PREFACE

___________________________________________________________________

Conventions

The following conventions are used throughout this book:

Press	   Means to press a specific key once. For examples, press

RETURN means press the RETURN key once.

Enter	   Means to enter or type text from the keyboard. For

example, enter the date and press RETURN.

ALL CAPS	   Indicates the function or edit key to press.

For example, press ADVANCE means you press the ADVANCE

key located on the Edit keypad.

Gold XXX	   Press the Gold key on the edit keypad, followed by

another key.  For example, press Gold MENU means press

the Gold key, followed by the M (MENU) key on the main

keyboard.

___

Rubout Key    Refer to the key marked with an < X | symbol:						    ___

The RUBOUT key, also marked WORD CHAR, erases text to

the left of the cursor.

< > key	   The key in the lower right corner of the Edit keypad,

labeled with < >, and also marked ENTER.

{     }	   The braces enclose areas of the screen with inverted

video.

-------	   A box is used to enclose the text displayed on a menu

|     |	   screen.

|     |

-------

vi

PREFACE

___________________________________________________________________

Reference Manuals

This book assumes your DECmate system is correctly set up and that you

have formatted your hard disk. Consult the following manuals for

specific information:

DECmate II

o  DECmate II Installation Guide

o  DECmate II Hard Disk Subsystem Documentation

o  DECmate II Owner's Manual

DECmate III Plus

o  DECmate III Plus System Kit

The DECmate Master Menu Software Installation Guide only describes how

to install Master Menu software on a hard disk.  You should refer to

the Master Menu User's Guide for instructions on how to use Master

Menu with your DECmate system.

vii

INTRODUCTION

______________________________________________________________________

The DECmate Master Menu Software installation Guide explains how to

install Master Menu software on a DECmate system with a hard disk.  The

instructions in this book can be used on the following DECmate

systems:

o  DECmate II

DECmate II system, including monitor, keyboard, system unit

with dual diskette drives, and the DECmate II hard disk

subsystem installed.

o  DECmate III Plus

DECmate III Plus system, including monitor, keyboard, and

system unit with one diskette drive and one hard disk.

Master Menu is installed using only one diskette drive.  This is the

default diskette drive on the DECmate system.  On a DECmate II this is

the uppermost drive;  on a DECmate III Plus this is the only diskette

drive.  Throughout this book, the default diskette drive is called

drive 0.

Other operating systems, such as CP/M and MS-DOS, use letters instead

of numbers for device assignments.  For example, the default diskette

drive is always assigned the letter A.  Other documentation may refer

to the default diskette drive as drive 0/A or just drive A.

Difference Between DECmate II and DECmate III Plus

Diskette Drive Operations

The main difference between DECmate II and DECmate III Plus hardware

is the diskette drive used on each system:

o  The DECmate II uses an RX50 dual diskette drive.  There are two

diskette drive doors.  The hard disk subsystem is optional.

o  The DECmate III Plus uses an RX33 diskette drive.  There is one

diskette drive with a lock.  The RD31 hard disk comes with the

system.

Figure 1-1 shows a DECmate II system with RX50 dual diskette drive and

the hard disk subsystem installed.  Note that the RX50 diskette drives

are opened by pressing on the top or bottom of the diskette drive

doors.

1-1

INTRODUCTION

______________________________________________________________________

Figure 1-1 not included in this document.

ASD-522

Figure 1-1  RX50 Diskette Drive Doors on DECmate II System

Figure 1-2 shows a DECmate III Plus system with the RX33 diskette

drive and the RD31 hard disk.  Note that the RX33 diskette drive is

opened by turning a lock.

Figure 1-2 not included in this document.

ASD-871

Figure 1-2  RX33 Diskette Drive Lock on DECmate III Plus System

Conventions Used to Describe Diskette Drive Operations

This book uses the term diskette drive door to refer to both the

diskette drive door on the RX50 and the lock on the RX33.  When you

are instructed to open the diskette drive door, follow the correct

procedure for your system:

DECmate II		Press the top of the diskette drive door. The door

springs open.

DECmate III Plus	Turn the diskette lock up.

When you are instructed to close the diskette drive door, follow the

correct procedure for your system:

DECmate II		Press the bottom of the diskette drive until it

closes.

DECmate III Plus	Turn the diskette lock down.  You must have a

diskette in the drive before you turn down the

lock.

1-2

INTRODUCTION

______________________________________________________________________

Special Function Keys

Figure 1-3 shows the DECmate keyboard with two function keys

highlighted.

Figure 1-3 not included in this document.

ASD-232

Figure 1-3  CANCEL and MAIN SCREEN Keys on the DECmate Keyboard

The two keys shaded on the DECmate keyboard (CANCEL and MAIN SCREEN)

in Figure 1-3 are keys Master Menu uses that may not be familiar to

you:

CANCEL	The CANCEL key (F8) lets you cancel an operation, function,

utility, or menu screen.

MAIN	The MAIN SCREEN key (F9) allows you to display the previous

SCREEN	menu screen after you complete an operation.

For a complete discussion of all the other keys on the DECmate

keyboard, refer to the DECmate Owner's Manual for your system.

Required Software

You need the following software to install Master Menu on a formatted

hard disk:

o  Master Menu Install diskette A

o  Master Menu Install diskette B

After you install Master Menu, on the hard disk, you can install the

following software:

o  DECmate WPS (Word Processing System). version 2.2 or higher.

Refer to the book Installation Guide for WPS and DECspell (1)

for instructions on installing DECmate WPS on the hard disk.

______________________________________________________________________

(1) DECmate/DECspell software uses the Houghton Mifflin dictionary,

copyrighted by Houghton Mifflin Company, Boston, Massachusetts.

1-3

INTRODUCTION

______________________________________________________________________

Handling Diskettes

Follow these precautions when you handle diskettes:

o  Handle diskettes carefully, holding them on the edges or

corners, much as you would a stereo record.

o  NEVER touch the read/write openings with your fingers.

o  Keep diskettes in their paper envelopes when they are not in

use.

o  Do not attempt to clean diskettes by wiping them with a cloth.

You will ruin the magnetic surface.

o  Write on labels before you affix them to a diskette. If you

must write on a label already on a diskette use a felt-tip pen.

Do not use ballpoint pens, and never use a pencil.

o  Keep diskettes away from direct sunlight or high temperatures.

Store them in a cool, dry place.

o  Keep diskettes away from the video monitor, magnets, tools,

phone, or electric motors.

1-4

2

______________________________________________________________________

INSTALLING MASTER MENU

Once you have formatted your hard disk, use the following procedure to

install Master Menu on the hard disk using Master Menu Install

diskettes A and B.  This procedure differs from the one described in

the Master Menu User's Guide because you must use the original

(master) copy of the Master Menu Install diskettes.  You cannot make

backup copies until Master Menu has been installed.

Refer to Chapter 3, "Backup Diskette to Diskette With Master Menu,"

for instructions on making backup copies of original (master) software

after you install Master Menu.

INSTALLING MASTER MENU

1  If necessary, turn the DECmate system OFF.  Open the diskette

drive door(s) and make sure all drive(s) are empty.

2  Press the system unit power switch to the ON (1) position.

3  Immediately insert the Master Menu Install diskette A into drive

0.  If your diskette drive has a colored bar on the drive, make

sure the arrow on the diskette lines up with the colored bar

Close the diskette drive door.

OR

If you already have Master Menu, you can start up the new

Master Menu Install diskette A from the existing Master Menu by

pressing SET-UP and then REMOVE.  Insert Master Menu Install

diskette A into drive 0, and close the door.  Press DO.

NOTE:  If you do not insert the DECmate Master Menu Install

diskette A immediately, the system will boot the startup

volume of Master Menu that is already established on

your hard disk (if one exists).  If this should happen,

either turn off the system and repeat from step 2, or

press SET-UP, press REMOVE, insert the Master Menu

Install diskette A into drive 0, close the drive door

and press DO.

The software takes a minute or two to load.  The Master Menu

Install Diskette Menu, similar to the one shown in Figure 2-1,

appears on the screen.  If you have a DECmate II, the COPY

option appears on the screen.

2-1

INSTALLING MASTER MENU

______________________________________________________________________

----------------------------------------------------------------------

|{	    	     DECmate Master Menu Install diskette		 }|

|	Version 1.2				Panel Code Rom Level: xxx |

|						Internal Data Level: xxxx |

|									  |

|									  |

|{ INST }	Install Master Menu					  |

|									  |

|	SMPW	Set Master password

|									  |

|	PSWD	List volume passwords

|									  |

|	MBB	Mark a bad block

|									  |

|	QUIT	Leave this program and go to SETUP to reboot

|									  |

|									  |

|									  |

|									  |

|									  |

|									  |

|	Select an item using arrow keys or type its tag, then press DO.	  |

|									  |

|									  |

----------------------------------------------------------------------

Figure 2-1  Master Menu Install Menu

4  Use the ^ (UP ARROW) or v (DOWN ARROW) keys to move the		    |		    |

highlight to the INST option (Install Master Menu). Press DO.

OR

Enter INST and press DO.

If you already have Master Menu on your system, the following

message appears:

---------------------------------------------------------------

|								  |

| Saving your old menu items.				  |

|								  |

---------------------------------------------------------------

Master Menu is installed with the volume name MENU.  If you

already have a volume named MENU on your hard disk, and if this

volume does not contain the Master Menu system (it is possible

that another application is named MENU), the following message

appears:

2-2

INSTALLING MASTER MENU

______________________________________________________________________

---------------------------------------------------------------

|								  |

| Volume named MENU is not a Master Menu volume.		  |

|								  |

| If you proceed, the contents of this volume will be erased. |

|								  |

| To proceed, press DO. To abort, press CANCEL.		  |

|								  |

---------------------------------------------------------------

If you want to continue with the installation (in other words,

you do not need the contents of the volume named MENU), press

DO. Proceed to the next step.

OR

If you do not want to continue with the installation (in other

words, you want to keep the contents of the volume named MENU),

press CANCEL and the Master Menu Install menu appears. Use ^ or								       |

v to highlight QUIT and press DO.  Press REMOVE, located on the	    |

Arrow Keypad.  Open drive door 0 and remove Master Menu

diskette A.  Press DO to restart the DECmate system.  If you

want to keep a volume currently named MENU, use the Rename

function (in CP/M or MS-DOS) change the volume name to another

name.  You must rename the volume before you install Master

Menu.

5  Follow the prompts on the screen and remove Master Menu Install

diskette A from drive 0.  Place the diskette in its protective

envelope.

6  Insert Master Menu Install diskette B into drive 0, and close

the door.  When you are ready, press DO.  The screen displays a

row of dots that turn into asterisks (*) during installation.

7  When the installation is complete, you are prompted to remove

Master Menu Install diskette B from drive 0.  Take the diskette

out, and place it in its protective envelope.

8  Replace Master Menu Install diskette A in drive 0, and press DO.

If you had menu items that were saved, the following message

appears:

--------------------------------------------------------------

|								  |

| Restoring your old menu items.				  |

|								  |

--------------------------------------------------------------

2-3

INSTALLING MASTER MENU

______________________________________________________________________

9  You are prompted to press the MAIN SCREEN key to redisplay the

Install Menu.  Press MAIN SCREEN.  This key is Function Key 9

(F9), located in the uppermost row of gray function keys.

10  When the Master Menu Install Diskette Menu appears,

installation is complete.  Do not select any other options

except QUIT until you make a backup copy of the Master Menu

Installation diskettes A and B, as described in the next

section.

11  Use the ^ or v keys to select QUIT and press DO.		    |    |

OR

Enter QUIT and press DO.

The DECmate Set Up Screen appears.

12  Press REMOVE, located on the Arrow Keypad.  Open drive door 0,

and remove Master Menu diskette A.  Press DO.  The DECmate

system starts itself and displays the Master Menu Date/Time

screen.

Once you have installed Master Menu on the hard disk, you should make

backup copies of the installation diskettes, as described in the next

chapter.  When you complete the backup, store the original (master)

copies and use the backup copies and repeat steps 1 to 3 to redisplay

the Master Menu Install Menu.  You can then select and use other menu

items (Set Master password, List volume passwords, Mark a bad block)

as described in the Master Menu User's Guide.

2-4

3

______________________________________________________________________

BACKUP DISKETTE TO DISKETTE WITH MASTER MENU

The D Option (Backup diskette to diskette) is a new Master Menu

utility that makes backup copies of diskettes.  The D option is found

in the Hard Disk Services menu, as shown in Figure 3-1.

The D option transfers a complete copy from one diskette to another

diskette using a single diskette drive (drive 0). The D option is

similar to backup in WPS, and can be used to make working copies of

any diskette, including WPS document diskettes.  Use the D option to

make backup copies of software installation diskettes, or to make

copies of other diskettes from Master Menu.

The other backup utility in Master Menu (B -- Backup Volume(s) to

diskettes) transfers the contents of a hard disk volume onto a

diskette in drive 0.  The B option is used primarily for backups of

hard disk volumes.  Unlike the D option, the B option does not make

workable copies.  For example, WPS cannot read data from diskettes

backed up with the B option;  you must use the R (Restore a volume

from diskette) Option in Master Menu before WPS can read the data.

Consult Chapter 7, "Hard Disk Services," in the Master Menu User's

Guide for instructions on using the B and R options in Master Menu.

Backup Installation Diskettes

After you install Master Menu you should make backup copies of the

original Master Menu Install diskettes A and B.  Once this procedure

is complete, do not use the original diskettes for installation:

always use backup copies. Store the original diskettes in a safe

place.

At the same time, you should also make backup copies of the WPS

Install, WPS System, and WPS Utility diskettes, and any other software

you plan to install on the hard disk.  Do this before you install the

software.  The original Master Menu Install diskettes A and B are the

only software that cannot be backed up before installation.

Use blank diskettes for backups, or use diskettes you are sure you do

not want to keep.  Remember that the contents of these diskettes will

be erased and replaced when you make backups.

D - Backup Diskette to Diskette

The D Option involves four basic steps:

3-1

BACKUP DISKETTE TO DISKETTE WITH MASTER MENU

______________________________________________________________________

o  Insert the diskette to be copied in drive 0.

o  Copy the contents of the diskette into a temporary storage area

on the hard disk.

o  Remove the diskette to be copied and insert a blank diskette to

receive the copy.

o  Copy the contents of the temporary storage area on the hard

disk onto the blank diskette.

You need at least one area of 400K bytes free space on your hard disk

for the temporary storage area.  The 400K bytes must be contiguous, or

one complete block of space.  If you have 400K bytes free space, but

the largest area is less than 400K, the space is not contiguous and

the D option will not work.  Compress space on the hard disk, or erase

a volume you do not want, to create enough contiguous free space and

retry the D option.

----------------------------------------------------------------------

|{			      Hard Disk Services			 }|

|	Screen 1 of 2        Monday 11/10/86 0:00:00	       Version 1.2|

|									  |

|		     {L}-- List directory of volumes			  |

|									  |

|		      A -- Allocate a new volume			  |

|									  |

|		      E -- Erase an existing volume			  |

|									  |

|		      B -- Backup Volume(s) to diskettes		  |

|									  |

|	  	      D -- Backup diskette to diskette			  |

|									  |

|		      R -- Restore a volume from diskettes		  |

|									  |

|		      Q -- Return to Master Menu			  |

|									  |

|									  |

|									  |

|									  |

|									  |

|	Select an item using arrow keys or type its tag; then press DO.   |

|	      Press NEXT SCREEN for more items.				  |

----------------------------------------------------------------------

Figure 3-1  The D Option in the Hard Disk Services Menu

3-2

BACKUP DISKETTE TO DISKETTE WITH MASTER MENU

______________________________________________________________________

The following procedure describes how to make backup copies from

diskette to diskette using the D option in Master Menu's Hard Disk

Services menu.

1  If necessary, turn the DECmate system OFF.  Open the diskette

drive door(s) and make sure all drive(s) are empty.  Press the

system unit power switch to the ON (|) position.  Master Menu

starts tip and the Date/Time screen appears.  Enter the date

and press RETURN.  Entering the time is optional.  Press RETURN

to skip entering the time.

OR

If the DECmate system is on, recall Master Menu.  From WPS Main

Menu press F (Finish);  from drive E in CP/M or MS-DOS, enter

MENU and press RETURN.

2  If you installed Master Menu for the first time the Fixed Menu

screen appears, similar to the one shown in Figure 3-2.

If you already have Master Menu installed, you also may have

other menu items.  Press NEXT SCREEN until the Fixed Menu

screen appears.

3  Use the ^ or v keys to highlight HD (Hard Disk Services) and		    |	 |

press DO or RETURN.

OR

Enter HD and press DO or RETURN.  The Hard Disk Services Menu

appears, similar to the one shown in Figure 3-1.

4  Use the ^ or v keys to select D (Backup diskette to diskette)		    |	 |

and press DO or RETURN.

OR

Enter D and press DO.  The following prompt appears:

---------------------------------------------------------------

|								  |

| Place the diskette to be backed up in drive 0.		  |

|								  |

| The contents of this diskette will be copied into a	  |

| temporary storage area on the hard disk.			  |

|								  |

| Press DO to proceed or CANCEL to quit.			  |

|								  |

---------------------------------------------------------------

3-3

BACKUP DISKETTE TO DISKETTE WITH MASTER MENU

______________________________________________________________________

----------------------------------------------------------------------

|{									 }|

|{  d i g i t a l	      DECmate Master Menu			 }|

|{									 }|

|Screen  2 of  2	        Monday 11/10/86 0:00:00     Version 1.2	  |

|--------------------------------------------------------------------|

|									  |

| {HD } DECmate Hard Disk Services				 	  |

|									  |

|	 MM   DECmate Menu Maintenance Utility				  |

|									  |

|	 DAT  Set System Date and Time					  |

|									  |

|	 SDF  Select New Date Format					  |

|									  |

|	 F    Finish Using the system					  |

|									  |

|									  |

|									  |

|--------------------------------------------------------------------|

|									  |

| Select an item using arrow keys or type its tag; then press DO.	  |

|	       Press NEXT SCREEN for more items.			  |

|									  |

----------------------------------------------------------------------

Figure 3-2  Fixed Menu Screen in Master Menu

Proceed to step 5.

OR

If your hard disk does not have enough free space for the

temporary storage area, a message similar to the one below

appears instead:

---------------------------------------------------------------

|								  |

| There is not enough contiguous free space on your hard disk.|

|								  |

| The minimum amount of free space required to perform a	  |

| diskette backup is 400K.					  |

|								  |

| Press CANCEL to return to Hard Disk Services.		  |

|								  |

---------------------------------------------------------------

3-4

BACKUP DISKETTE TO DISKETTE WITH MASTER MENU

______________________________________________________________________

Press CANCEL to recall the Hard Disk Services menu.  You must

create 400K bytes of contiguous free space before you can use

the D option.  Use the X (Compress) utility to compress space

on your hard disk, or E (Erase) to erase a volume you no longer

need. When you have enough free space, repeat steps 1 to 4.

5  Place the diskette you want to copy into drive O and close the

drive door.  Press DO.  While the diskette is being copied, a

row of dots turns to asterisks.

When the copy is complete, a second prompt appears:

---------------------------------------------------------------

|								  |

| Place the diskette to be OVERWRITTEN in drive 0.		  |

|								  |

| The contents of the previous diskette will be copied from	  |

| the hard disk onto this diskette.				  |

|								  |

| Press DO to proceed or CANCEL to quit.			  |

|								  |

---------------------------------------------------------------

6  Open the door on drive 0 and remove the diskette.  Insert a

blank diskette, or a diskette you no longer need, into drive O

and close the door.  Press DO.  While the diskette is being

copied, a row of dots turns to asterisks.

When the copy is complete, the last message appears:

---------------------------------------------------------------

|								  |

| The backup was successful.				  |

|								  |

| Remove the diskette from drive 0.				  |

|								  |

| Press RETURN to make another backup of this diskette.	  |

|								  |

| Press DO to backup a different diskette.			  |

|								  |

| Press CANCEL to quit.					  |

|								  |

---------------------------------------------------------------

7  You can make another copy of the same diskette by pressing

RETURN.  The prompt in step 5 appears.  Repeat step 6 as often

as required to make additional copies of the same diskette.

OR

3-5

BACKUP DISKETTE TO DISKETTE WITH MASTER MENU

______________________________________________________________________

You can make a backup copy of a new diskette by pressing DO.

Repeat steps 4 to 6.

OR

Press CANCEL.  This is the function key 8 (F8), located in the

uppermost row of gray function keys.  The Hard Disk Services

Menu appears.

8  Use the ^ or v keys to select Q (Return to Master Menu) and		    |	 |

press DO.

OR

Enter Q and press DO.

Master Menu displays either the Fixed Menu screen or Screen 1

of the menu selections.

3-6