OS8CKTIG.DOC

OS/8 Combined Kit

Information Guide

DISCLAIMER

This document file was created by scanning  the

original  document and then editing the scanned

text.  As much as possible, the  original  text

format  was restored.  Some format changes were

made to insure this  document  would  print  on

current laser printers using 60 lines per page.

The  original  spelling  and  grammar  has been

preserved.

1-Nov-1996

OS/8 COMBINED KIT

INFORMATION

GUIDE

Order No. AA-J016B-TA

January 1980

This Guide describes the contents of the OS/8 Combined Kit

and how to get the system on line.

SUPERSESSION/UPDATE INFORMATION:    This manual supersedes the

previous edition, Order No.

AA-J016A-TA.

OPERATING SYSTEM AND VERSION:       OS/8 V3D

---------------------------------------------------------------------

| To order additional copies of this document, contact the Software |

| Distribution Center, Digital Equipment Corporation, Maynard,      |

| Massachusetts 01754						 |

---------------------------------------------------------------------

digital equipment corporation - maynard. massachusetts

First Printing, August 1979

Revised:       January 1980

The information in this document is subject to change without notice

and should not be construed as a commitment by Digital Equipment

Corporation. Digital Equipment Corporation assumes no responsibility

for any errors that may appear in this document.

The software described in this document is furnished under a license

and may only be used or copied in accordance with the terms of such

license.

No responsibility is assumed for the use or reliability of software on

equipment that is not supplied by DIGITAL or its affiliated companies.

Copyright (c) 1979, 1980 by Digital Equipment Corporation

The postage-prepaid READER'S COMMENTS form on the last page of this

document requests the user's critical evaluation to assist us in pre-

paring future documentation.

The following are trademarks of Digital Equipment Corporation:

DIGITAL		  DECsystem-10	     MASSBUS

DEC		  DECtape	     OMNIBUS

PDP		  DIBOL		     OS/8

DECUS		  EDUSYSTEM	     PHA

UNIBUS		  FLIP CHIP	     RSTS

COMPUTER LABS	  FOCAL		     RSX

COMTEX		  INDAC		     TYPESET-8

DDT		  LAB-8		     TYPESET-11

DECCOMM		  DECSYSTEM-20	     TMS-11

ASSIST-MM		  RTS-8		     ITPS-10

VAX		  VMS		     SBI

DECnet		  IAS		     PDT

DATATRIEVE	  TRAX

CONTENTS

Page

1.0    INTRODUCTION					      1

2.0    OVERVIEW					      1

3.0    DOCUMENTATION				      1

4.0    DISTRIBUTION					      3

5.0    GETTING ON LINE				      3

5.1      General Bootstrap Procedures		      3

5.1.1    Operating a Hardware Bootstrap		      3

5.1.2    Entering a Bootstrap Manually		      4

5.2      RK05 Cartridge Disk Pack			      6

5.2.1    Loading Procedure				      6

5.2.2    Hardware Bootstrap Procedure		      6

5.2.3    Manual Bootstrap Procedure			      6

5.3      RL01 Cartridge Disk Pack			      7

5.3.1    Loading Procedure				      7

5.3.2    Hardware Bootstrap Procedure		      8

5.3.3    Manual Bootstrap Procedure			      8

5.4      RX01 and RX02 Diskettes			      9

5.4.1    Loading Procedure				      9

5.4.2    Hardware Bootstrap Procedure		     10

5.4.3    Manual Bootstrap Procedure			     11

5.5      DECtape (12K TD8E)				     12

5.5.1    Loading Procedure				     13

5.5.2    Hardware Bootstrap Procedure		     13

5.5.3    Manual Bootstrap Procedure			     13

5.6      Running the System				     14

6.0    VERSION CODES				     15

7.0    OBTAINING VERSION CODES FROM OS/8 MODULES	     17

FIGURES

Figure 1      Loading the RK05 Disk			      6

2      Loading the RL01 Disk Pack			      7

3      Loading the RX01 and RX02 Diskette		     10

TABLES

Table  1      Manual Bootstrap Procedure			      5

2      RK8E (RK05) Bootstrap Program		      6

3      RL01 Bootstrap Program			      9

4      RX01 Bootstrap Program			     11

5      RX02 Bootstrap Program			     12

iii

CONTENTS (Cont.)

TABLES (Cont.)

Page

Table  6      TD8E DECtape Bootstrap Program		     14

7      File and Media Duplication Utility Programs       15

8      OS/8 Combined Kit Modules and Version Codes       15

9      Obtaining OS/8 Module Version Codes		     17

iv

1.0  INTRODUCTION

This document briefly describes  the  OS/8  Combined  Kit,  lists  the

manuals  and  software contained in the kit, and provides instructions

for getting the system on line.

2.0  OVERVIEW

The OS/8 Combined Kit  is  an  operating  system/software  development

package  for  PDP-8  computers.   As  the  name  implies, the kit is a

combination of OS/8 software products.  It includes the  software  and

documentation contained in the following kits:

o  OS/8, Version 3D,  Operating  System  (described  in  Software

Product Description (S.P.D.  4.1.11)

o  OS/8, Version 3D, Extension (S.P.D.  4.5.8)

o  OS/8, Version 3D, Device Extensions for RL01, RX02,  and  KT8A

(S.P.D.  4.12.0)

o  OS/8, Version 3D, FORTRAN IV (S.P.D.  4.10.7)

In addition, the kit's modules  have  been  updated  with  the  binary

patches  described  in  the OS/8 Device Extensions User's Guide and in				 ___________________________________

the issues of the Digital Software News (through June 1979).		       _____________________

3.0  DOCUMENTATION

The documents contained in the OS/8 Combined Kit are as follows. The

titles are presented in alphabetical order.

OS/8 COMBINED KIT INFORMATION GUIDE - AA-J016B-TA

Describes the kit's contents and how to get the system

on line.

OS/8 COMBINED KIT - V3D SPD 4.4.2

Describes the  OS/8   Combined   Kit,   its   options,

licenses,   support   category   and  other  pertinent

information.

OS/8 DEVICE EXTENSION RELEASE NOTES - AA-H565A-TA

Contains supplemental information on the  features  of

the  OS/8  Device  Extensions Kit not discussed in the

DEVICE EXTENSIONS USER'S GUIDE.

OS/8 DEVICE EXTENSIONS USER'S GUIDE - AA-D319A-TA

Describes the RL01 and  RX02  disks,  their  bootstrap

procedures,  and  the  software  support  for the KT8A

(128K) Memory Management option.

1

OS/8 ERROR MESSAGES - AA-H610A-TA

Describes the error messages  generated  by  the  OS/8

Monitor and its system programs.

OS/8 FORTRAN IV SOFTWARE SUPPORT MANUAL - DEC-S8-LFSSA-A-D

Describes internal features of OS/8  FORTRAN  IV  that

may be useful to the advanced user.

OS/8 LANGUAGE REFERENCE MANUAL - AA-H609A-TA

Describes the languages  supported  by  OS/8:   BASIC,

FORTRAN IV, PAL8, FORTRAN II, FLAP/RALF, and SABR.

OS/8 MARK SENSE BATCH USER'S MANUAL - DEC-S8-OBUGA-A-D

Describes the use and  operation  of  the  Mark  Sense

Batch  (MSBATCH)  OS/8  system  program.   It includes

step-by-step procedures  for  preparing  programs  for

submission to MSBATCH.

OS/8 SOFTWARE SUPPORT MANUAL - DEC-S8-OSSMB-A-D

Describes internal system features that may be  useful

to the advanced user.

OS/8 SYSTEM GENERATION NOTES - AA-H606A-TA

Describes PDP-8/E bootstrap procedures  for  LINCtape,

DECtape  (TC08  and TD8E), and disk (DF32, RF08, RK08,

RK8E, and RX01), as well as  procedures  for  building

the system from paper tape and DECassette (TA8E).

OS/8 SYSTEM REFERENCE MANUAL - AA-H607A-TA

Describes  the  OS/8  system   conventions,   keyboard

commands and utility programs.

OS/8 TECO REFERENCE MANUAL - AA-H608A-TA

Describes the Text Editing and Correcting program  for

OS/8 users.

OS/8 V3D SYSTEM RELEASE NOTES - DEC-S8-OSRNA-B-D

Describes the differences between version V3D of  OS/8

and previous versions, as well as corrections to known

software problems.

TECO POCKET GUIDE - AV-D530A-TK

Contains  a  guick-reference  summary  of  OS/8   TECO

commands.

2

4.0  DISTRIBUTION

The Combined Kit is distributed on the following media:

o  One RK05 cartridge disk

o  One RL01 cartridge disk pack

o  Six RX01 (single density) diskettes

o  Three RX02 (double density) diskettes

o  Three DECtapes (12K TD8E system)

5.0  GETTING ON LINE

To use OS/8, you must load a bootstrap program into memory and run it.

The  bootstrap  program  loads  the OS/8 monitor from its distribution

medium and initializes its operation.  Each  type  of  storage  device

(for  example,  RL01  disk,  DECtape,  etc.)  has a separate bootstrap

program.

Your system may have either an internal hardware bootstrap loader or a

programmer's  console.   (Some hardware configurations have both.) The

hardware bootstrap  loader  loads  and  starts  an  internally  stored

bootstrap  program  under the control of a switch located on the front

of the processor.  The programmer's console has switches that let  you

manually enter the bootstrap program into memory and start it.

The following paragraphs tell you how to load the system medium and to

initialize  (bootstrap)  it.   You  will  also  find  these  bootstrap

procedures in the OS/8 System Generation Manual and in the OS/8 Device		       _____________________________		___________

Extensions User's Guide.     _______________________

5.1  General Bootstrap Procedures

This section tells you how to use the hardware  bootstrap  loader  and

how to manually enter and initialize a bootstrap from the programmer's

console.  The remaining sections of this document provide loading  and

bootstrap procedures for the OS/8 Combined Kit distribution media.

5.1.1  Operating a Hardware Bootstrap - If your system has a  hardware

bootstrap loader and the processor is a:

PDP-8/A   1.  Load the distribution medium  into  its  device  (see

Sections 5.2 through 5.5).

3

2.  Run the bootstrap loader:

o  Press the BOOT button twice  on  the  programmer's

console.

or

o  Raise and lower the BOOT switch on the  operator's

panel.

PDP-8/E   1.  Load the distribution medium into its device (see

Sections 5.2 through 5.5).

2.  Lower and raise the HALT switch.

3.  Lower and raise the SW switch.

This procedure will have no effect if there is no  hardware  bootstrap

loader  in  the system for the device that runs your OS/8 distribution

medium.

5.1.2  Entering a Bootstrap Manually - If your system has no  hardware

bootstrap loader for your distribution medium, you must load and start

the bootstrap, using the programmer's console as follows:

1.  Load the distribution medium into its  device  (see  Sections

5.2 through 5.5).

2.  Use the steps in Table 1 to manually load  and  initiate  the

appropriate bootstrap program (see Sections 5.2 through 5.5).

4

Table 1

Manual Bootstrap Procedure

----------------------------------------------------------------------

|      |		  |		 Processor			  |

| Step | Function    |-----------------------------------------------|

|      |		  |	 PDP-8/A      |		PDP-8/E		  |

|------|-------------|-------------------|---------------------------|

|  1   | Halt the    | Press HLT/SS      | Lower and raise the HALT  |

|      | processor   |		      | switch			  |

|      |		  |		      |				  |

|  2   | Display     | a. Press MD       | Rotate selector switch    |

|      | program     |		      | to MD			  |

|      | data to be  | b. Press DISP     |				  |

|      | entered     |		      |				  |

|      |		  |		      |				  |

|  3   | Select mem- | a. Press 0	      | a. Lower switches 6, 7, 8 |

|      | ory field 0 |		      |				  |

|      |		  | b.  Press LXA     | b. Press EXTD ADDR LOAD   |

|      |		  |		      |				  |

|  4   | Select      | Using the numeric | Using the numeric	  |

|      | bootstrap   | keys:	      | switches:		  |

|      | loading     |		      |				  |

|      | address     | a. Enter the      | a. Enter the binary equiv-|

|      |		  | first (lowest)    |    alent of the first     |

|      |		  | bootstrap	      |    (lowest) address	  |

|      |		  | address	      |				  |

|      |		  |		      | b. Press ADDR LOAD	  |

|      |		  | b. Press LA       |				  |

|      |		  |		      |				  |

|  5   | Enter each  | Using the numeric | Using the numeric	  |

|      | instruction | keys:	      | switches:		  |

|      | in the pro- |		      |				  |

|      | gram into   | a. Enter the      | a. Enter the binary equiv-|

|      | memory      |    first	      |    alent of the first     |

|      |		  |    instruction    |    instruction		  |

|      |		  |		      |				  |

|      |		  | b. Press D NEXT   | b. Raise and lower DEP    |

|      |		  |		      |				  |

|      |		  | Repeat above for  | Repeat 1 & 2 for suc-     |

|      |		  | succeeding	      | ceeding instructions      |

|      |		  | instructions      |				  |

|      |		  |		      |				  |

|  6   | Select      | Using the numeric | Using the numeric	  |

|      | bootstrap   | keys:	      | switches:		  |

|      | starting    |		      |				  |

|      | address     | a. Enter the      | a. Enter the binary equiv-|

|      |		  |    starting       |    alent of the starting  |

|      |		  |    address	      |    address		  |

|      |		  |		      |				  |

|      |		  | b. Press LA       | b. Press ADDR LOAD	  |

|      |		  |		      |				  |

|  7   | Begin       | a. Press INIT     | a. Press CLEAR		  |

|      | bootstrap   |		      |				  |

|      | operation   | b. Press RUN      | b. Press CONT		  |

----------------------------------------------------------------------

5

5.2  RK05 Cartridge Disk Pack

5.2.1  Loading Procedure - If your OS/8 Combined  Kit  medium  is  the

RK05 disk cartridge, insert the disk into the drive as shown in Figure

1.  DIGITAL suggests that you write-protect the distribution medium by

pressing the WTPROT switch.  If there is more than one RK05 disk drive

on your system, insert the disk pack into the drive marked "0".

----------------------------------------------------------------------

|									  |

|		RK05 picture not included here.				  |

|									  |

|									  |

| 1.  Switch power on.						  |

|									  |

| 2.  Set RUN/LOAD switch (3) to LOAD;LOAD indicator light (4) will  |

|     be lit.							  |

|									  |

| 3.  Open access door (1), insert RK05 disk cartridge (2); close    |

|     access door.							  |

|									  |

| 4.  Set RUN/LOAD switch to RUN; the LOAD indicator light will go   |

|     out.								  |

|									  |

| 5.  READY indicator light (5) will go on when it is ready for      |

|     access.							  |

----------------------------------------------------------------------

Figure 1  Loading the RK05 Disk

5.2.2  Hardware Bootstrap Procedure - If your system  has  a  hardware

bootstrap  loader  for  the  RK05  (RK8E controller) disk, perform the

steps in Section 5.1.1.  This procedure will have no effect  if  there

is no hardware bootstrap loader for the RK8E on your system.

5.2.3  Manual Bootstrap Procedure - If your system has a  programmer's

console,  use  its switches as described in Section 5.1.2 to enter and

initialize the RK8E bootstrap program listed in Table 2.

Table 2

RK8E (RK05) Bootstrap Program

Address      Instruction					     _______      ___________

Loading and starting address ------->  30	     6743

31	     5031

6

5.3  RL01 Cartridge Disk Pack

5.3.1  Loading Procedure - If your OS/8 Combined  Kit  medium  is  the

RL01  Cartridge  Disk  Pack,  load the disk into its drive as shown in

Figure 2.  DIGITAL suggests that you protect the  distribution  medium

by  pressing the WRITE PROTECT button (when illuminated, protect is in

effect).  If there is more than one RL01 drive on  your  system,  load

the pack into the drive marked "0".

----------------------------------------------------------------------

|									  |

| TO READY DRIVE:							  |

|									  |

| Raise cartridge access door, and load RL01K disk cartridge.	  |

|									  |

| Depress load switch.  Note that spindle motor starts turning.      |

|									  |

| After 30 seconds, UNIT SELECT/READY indicator should light,	  |

| indicating drive is ready to read or write.			  |

|									  |

| If write protection is desired, depress WRITE PROTECT switch       |

| (WRITE PROTECT indicator).					  |

|--------------------------------------------------------------------|

|									  |

|									  |

|									  |

|	  RL01 picture not included here				  |

|									  |

|									  |

----------------------------------------------------------------------

(continued on next page)

Figure 2  Loading the RL01 Disk Pack

7

----------------------------------------------------------------------

|					|				  |

| DRIVE INDICATORS:		| TO LOAD RL01K CARTRIDGE:	  |

|					|				  |

| LOAD:				| Support cartridge "A" with left |

| When lit, indicates that		| hand holding protection cover	  |

| cartridge may be loaded or that  | "B".				  |

| spindle is stopped.		|				  |

|					| Push handle slide "C" to left   |

| UNIT SELECT/READY:		| with thumb of right hand.       |

| Indicates logical drive address. |				  |

| When lit, indicates drive is     | Raise cover handle "D" to full  |

| ready to read, write or receive  | upright position, releasing     |

| controller commands.		| protection cover "B".		  |

|					|				  |

| FAULT:				| Lift cartridge "A" from	  |

| When lit, indicates a drive      | protection cover "B" and	  |

| error condition.  If this	| carefully seat it on drive      |

| condition persists, seek		| spindle with handle recess      |

| assistance.			| facing rear of drive.		  |

|					|				  |

| WRITE PROTECT:			| Carefully rotate top cover      |

| When lit, indicates that		| handle "D" a few degrees	  |

| cartridge currently mounted is   | clockwise and counterclockwise  |

| write protected.			| to ensure firm seating.	  |

|					|				  |

|					| Gently lower top cover handle   |

|					| "D" to horizontal position to   |

|					| engage cartridge on drive       |

|					| spindle.			  |

|					|				  |

|					| Place protection cover "B" on   |

|					| top of cartridge		  |

----------------------------------------------------------------------

Figure 2  Loading the RL01 Disk Pack (cont.)

5.3.2  Hardware Bootstrap Procedure - If your system  has  a  hardware

bootstrap  loader  for  the  RL01  disk,  perform the steps in Section

5.1.1.  This procedure will have no effect if  there  is  no  hardware

bootstrap loader for the RL01 on your system.

5.3.3  Manual Bootstrap Procedure - If your system has a  programmer's

console,  use  its switches as described in Section 5.1.2 to enter and

initialize the RL01 bootstrap program listed in Table 3.

8

Table 3

RL01 Bootstrap Program

Address		Instruction				       _______		___________

Loading and starting address--->0001		   6600

0002		   7201

0003		   4027

0004		   1004

0005		   4027

0006		   6615

0007		   7002

0010		   7012

0011		   6615

0012		   0025

0013		   7004

0014		   6603

0015		   7325

0016		   4027

0017		   7332

0020		   6605

0021		   1026

0022		   6607

0023		   7327

0024		   4027

0025		   0377

0026		   7600

0027		   0000

0030		   6604

0031		   6601

0032		   5031

0033		   6617

0034		   5427

0035		   5001

5.4  RX01 and RX02 Diskettes

5.4.1  Loading Procedure - If  your OS/8  Combined  Kit  medium  is  a

single-density  (RX01)  diskette  or a double-density (RX02) diskette,

refer to Figure 3 and perform the procedures listed below to load  the

diskette into the drive.

CAUTION

You can insert a single-density diskette

into  either  an  RX01 or an RX02 drive.

You can insert a double-density diskette

into an RX02 drive only.

9

RX02 picture not included here

Figure 3

Loading the RX01 and RX02 Diskette

Ordinarily you should load the diskette that contains the monitor  and

system programs (system diskette) into drive 0, which is the left-hand

slot.  You can use drive 1 (right-hand slot), if necessary, as long as

there is no diskette in drive 0 when you start the bootstrap program.

1.  Select a diskette from the kit.  If you have an RX01 kit, use

diskette 1, 2, or 3.  If you have an RX02 kit, use diskette 1

or  2.   (The  other  diskette(s)  in  the  kit  contain  the

remaining OS/8 modules.)

2.  Open the drive's door:

a.  Notice that the door latch is embedded in the  horizontal

bar that is part of the door assembly.

b.  Compress the latch between the thumb  and  index  finger.

The  door  is  spring-loaded and will open when the latch

disengages.

3.  Grasping the diskette by its top (label) edge, remove it from

its  protective  storage  jacket.   (This  is  not  the black

protective  cover  in  which  the  diskette  is   permanently

sealed.)

CAUTION

Do not touch the exposed portions of the

diskette  that show through the slots in

the protective cover.

4.  Insert  the  diskette  label-side  up  (the  seams   of   the

protective  cover  are  down) into the horizontal slot in the

drive.

5.  Close the door.  (It will automatically lock when you push it

down.)

5.4.2  Hardware Bootstrap Procedure - If your system  has  a  hardware

bootstrap  loader  for the RX01 or RX02 diskette, perform the steps in

Section 5.1.1.  This procedure will have no  effect  if  there  is  no

hardware bootstrap loader for the RX01 or RX02 on your system.

10

5.4.3  Manual Bootstrap Procedure - If your system has a  programmer's

console,  use  its switches as described in Section 5.1.2 to enter and

initialize the RX01 or RX02 bootstrap program listed in Tables  4  and

5, respectively.

Table 4

RX01 Bootstrap Program

Address		  Instruction			_______		  ___________

Loading address---->0024		     7126

0025		     1060

0026		     6751

0027		     7201

0030		     4053

0031		     4053

0032		     7104

Starting address--->0033		     6755

0034		     5054

0035		     6754

0036		     7450

0037		     7610

0040		     5046

0041		     1060

0042		     7041

0043		     1061

0044		     3060

0045		     5024

0046		     6751

0047		     4053

0050		     3002

0051		     2050

0052		     5047

0053		     0000

0054		     6753

0055		     5033

0056		     6752

0057		     5453

0060		     7024

0061		     6030

11

Table 5

RX02 Bootstrap Program

Address		  Instruction			_______		  ___________

Loading address --->0020		     1061

0021		     1046

0022		     0060

0023		     3061

0024		     7327

0025		     1061

0026		     6751

0027		     7301

0030		     4053

0031		     4053

0032		     7004

Starting address---->0033		     6755

0034		     5054

0035		     6754

0036		     7450

0037		     5020

0040		     1061

0041		     6751

0042		     1061

0043		     0046

0044		     1032

0045		     3060

0046		     0360

0047		     4053

0050		     3002

0051		     2050

0052		     5047

0053		     0000

0054		     6753

0055		     5033

0056		     6752

0057		     5453

0060		     0420

0061		     0020

5.5  DECtape (12K TD8E)

The DECtape bootstrap procedures described below apply  to  CPUs  that

have  a  TD8E Simple DECtape control and at least 12K words of memory.

The system on the DECtape distribution medium cannot  be  bootstrapped

in an 8K TD8E ROM environment.

12

5.5.1  Loading  Procedure - If  your  OS/8  Combined  Kit  medium   is

DECtape, mount the tape as follows:

1.  Select either tape 1  or  tape  2  from  the  kit.   (Tape  3

contains the remaining OS/8 modules.)

2.  Remove the tape from its protective canister.

3.  Remove the plastic seal from the end of the tape.

4.  Mount the tape (label-side out) on one of the left-hand  hubs

of  the  DECtape  transport.  You can use either the right or

the left drive.  (If your system has more  than  one  DECtape

transport,  you must use the transport that is wired for TD8E

units 0 and 1.)

5.  Unwrap a  few  turns  of  tape,  pass  it  over  the  drive's

read/write head (dull side toward the head), and wind it onto

the takeup reel on the right-hand hub.

6.  Wind several more turns of tape onto the takeup reel so  that

the tape is not slack.

7.  Press the REMOTE/LOCAL switch to LOCAL.

8.  Wind more tape onto the takeup reel by  momentarily  pressing

the  forward/reverse  switch  (marked  with  left  and  right

arrows) toward the right arrow.

9.  Rotate the numbered thumbwheel switch to 0.

10.  Press the REMOTE/LOCAL switch to REMOTE.

11.  Press the WRITE ENABLE/WRITE LOCK switch  to  WRITE  LOCK  to

protect the distribution medium.

5.5.2  Hardware Bootstrap Procedure - If your system  has  a  hardware

bootstrap  loader  for  the  TD8E, perform the steps in Section 5.1.1.

This procedure will have no effect if there is no  hardware  bootstrap

loader for the TD8E on your system.

5.5.3  Manual Bootstrap Procedure - If your system has a  programmer's

console,  use  its switches as described in Section 5.1.2 to enter and

initialize the TD8E bootstrap program listed in Table 6.

13

Table 6

TD8E DECtape Bootstrap Program

Address	     Instruction				    _______	     ___________

Loading and starting address--->7300		1312

7301		4312

7302		4312

7303		6773

7304		5303

7305		6777

7306		3726

7307		2326

7310		5303

7311		5732

7312		2000

7313		1300

7314		6774

7315		6771

7316		5315

7317		6776

7320		0331

7321		1327

7322		7640

7323		5315

7324		2321

7325		5712

7326		7354

7327		7756

7330		7747

7331		0077

7332		7400

5.6  Running the System

After you load and  run  a  bootstrap,  the  Keyboard  Monitor  should

identify itself by displaying its prompting symbol, a period (.).  The

system is waiting for you to enter one of the OS/8 commands  described

in the OS/8 System Reference Manual.  If there is no response from the	    ____________________________

system, repeat the bootstrap procedure.  If there is still no response

from the system, call your local DIGITAL sales office for assistance.

When you have determined that OS/8 is running, DIGITAL  suggests  that

you  make  one  or  more working copies of the distribution medium for

daily use.  You can make copies by using one of the file  manipulation

utility  programs  listed  in Table 7 and described in the OS/8 System								___________

Reference Manual.     ________________

14

Table 7

File and Media Duplication Utility Programs

----------------------------------------------------------------------

| Program | CCL Command |			  Use			  |

|---------|-------------|--------------------------------------------|

|  FOTP   |    COPY     | Copies   files   between   file-structured |

|	       |	     | devices.   (You  must  use  PIP  with /Z/Y |

|	       |	     | first to create a new directory and put  a |

|	       |	     | system head on the new media.)		  |

|	       |	     |						  |

|  PIP    |    ZERO     | Copies   files   between   file-structured |

|	       |	     | devices,  formats  a  directory  area on a |

|	       |	     | device (/Z option),  copies  system  heads |

|	       |	     | between    file-structured   devices   (/Y |

|	       |	     | option).  (You must use /Z/Y first  before |

|	       |	     | copying   the  system  files  to  the  new |

|	       |	     | media.)					  |

|	       |	     |						  |

|  RKCOPY |    None     | Duplicates RK05 cartridge disks		  |

|	       |	     |						  |

|  RXCOPY |  DUPLICATE  | Duplicates RX01 and RX02 floppy diskettes  |

----------------------------------------------------------------------

6.0  VERSION CODES

Many of the version codes for the OS/8  system  modules  have  changed

because  of the binary patches that have been added (see Section 2.0).

Table 8 lists the version code for each module in the kit.

Table 8

OS/8 Combined Kit Modules and Version Codes

----------------------------------------------------------------------

|	      Module	     | Version ||	 Module		| Version |

|-----------------------|---------||-----------------------|---------|

| Keyboard Monitor      |   3T    || FORTRAN II		|	  |

|		CCL overlay  |   3A    ||	 RWTAPE.RL      |   2     |

|		CD	     |   5A    ||	 SQRT.RL	|   4     |

|		ODT	     |   6     ||	 TRIG.RL	|   6     |

|		USR	     |   4A    ||	 UTILITY.RL	|   10    |

|			     |	       ||			|	  |

| Utilities	     |	       || FORTRAN IV		|	  |

|		ABSLDR.SV    |   6B    ||	 FRTS.SV	|   5A    |

|		BATCH.SV     |   8A    ||	 F4.SV		|   4B    |

|		BITMAP.SV    |   4A    ||	 PASS2.SV	|   4A    |

|		BOOT.SV      |   5A    ||	 PASS2O.SV	|   4A    |

|		BUILD.SV     |   7A    ||	 PASS3.SV	|   4A    |

|		CAMP.SV      |   5A    ||	 LIBRA.SV	|   24A   |

|		CCL.SV	     |   3A    ||	 LOAD.SV	|   24A   |

----------------------------------------------------------------------

(continued on next page)

15

Table 8 (Cont.)

OS/8 Combined Kit Modules and Version Codes

----------------------------------------------------------------------

|	      Module	     | Version ||	 Module		| Version |

|-----------------------|---------||-----------------------|---------|

| Utilities (cont.)     |	       || FORTRAN IV (cont.)    |	  |

|		CREF.SV      |   5B    ||	 RALF.SV	|   62A   |

|		DIRECT.SV    |   6A    ||			|	  |

|		EDIT.SV      |   12D   || Handlers		|	  |

|		EPIC.SV      |   5A    ||	 ASR33.BN	|   A     |

|		FUTIL.SV     |   8B    ||	 BAT.BN		|   B     |

|		FOTP.SV      |   9B    ||	 CR8E.BN	|   C     |

|		FOTP.SV      |   9B    ||	 CR8E.BN	|   C     |

|		HELP.SV      |   2A    ||	 CSA.BN		|   A     |

|		MCPIP.SV     |   6B    ||	 CSB.BN		|   A     |

|		MSBAT.SV     |   3B    ||	 CSC.BN		|   A     |

|		GENIOX.S     |   7A    ||	 CSD.BN		|   A     |

|		PAL8.SV      |   13A   ||	 DF32NS.BN	|   A     |

|		PIP.SV       |   14A   ||	 DF32SY.BN	|   B     |

|		PIP10.SV     |   3B    ||	 DUMP.BN	|   C     |

|		RESORC.SV    |   5A    ||	 KL8E.BN	|   E     |

|		SET.SV       |   2A    ||	 LINCNS.BN	|   A     |

|		SRCCOM.SV    |   4A    ||	 LINCSY.BN	|   B     |

|		TECO.SV      |   5.08  ||	 LPSV.BN	|   C     |

|		TDINIT.SV    |   7A    ||	 LQP.BN		|   A     |

|			     |	       ||	 LSPT.BN	|   A     |

| Formatters and Copiers|	       ||	 L645.BN	|   A     |

|		DTCOPY.SV    |   10A   ||	 PT8E.BN	|   A     |

|		DTFRMT.SV    |   4A    ||	 RF08NS.BN	|   A     |

|		RKLFMT.SV    |   D     ||	 RF08SY.BN	|   B     |

|		RLFRMT.SV    |   1A    ||	 RK08NS.BN	|   A     |

|		RXCOPY.SV    |   4B    ||	 RK08SY.BN	|   D     |

|		TDCOPY.SV    |   4A    ||	 RK8ENS.BN	|   A     |

|		TDFRMT.SV    |   4A    ||	 RK8ESY.BN	|   C     |

|			     |	       ||	 RLSY.BN	|   A     |

| Languages	     |	       ||	 RL0.BN		|   A     |

|		BASIC	     |	       ||	 RL1.BN		|   A     |

|		BASIC.SV     |   5B    ||	 RL2.BN		|   A     |

|		BCOMP.SV     |   5A    ||	 RL3.BN		|   A     |

|		BLOAD.SV     |   5B    ||	 RLC.BN		|   A     |

|		BRTS.SV      |   5C    ||	 ROMSY.BN	|   A     |

|		EABRTS.SV    |   5A    ||	 RXNS.BN	|   N     |

|			     |	       ||	 RXSY1.BN	|   M     |

| FORTRAN II	     |	       ||	 RXSY2.BN	|   M     |

|		FORT.SV      |   5A    ||	 RX78C.BN	|   N     |

|		LOADER.SV    |   4A    ||	 TC08NS.BN	|   A     |

|		LIBSET.SV    |   3A    ||	 TC08SY.BN	|   B     |

|		SABR.SV      |   18A   ||	 TD8EA.BN	|   D     |

|		LIB8.RL      |	       ||	 TD8EB.BN	|   D     |

|		ATAN.RL      |   11    ||	 TD8EC.BN	|   D     |

|		FLOAT.RL     |   5     ||	 TD8ED.BN	|   D     |

|		INTEGR.RL    |   5     ||	 TD8ESY.BN	|   A     |

----------------------------------------------------------------------

(continued on next page)

16

Table 8 (Cont.)

OS/8 Combined Kit Modules and Version Codes

----------------------------------------------------------------------

|	      Module	     | Version ||	 Module		| Version |

|-----------------------|---------||-----------------------|---------|

| FORTRAN II (cont.)    |	       || Handlers (cont.)      |	  |

|		IOH.RL       |   10    ||	 TM8E.BN	|   F     |

|		IOPEN.RL     |   21    ||	 VR12.BN	|   A     |

|		IPOWER.RL    |   2     ||	 VT50.BN	|   A     |

|		POWERS.RL    |   5     ||	 VXNS.BN	|   A     |

|			     |	       ||	 VXSY.BN	|   A     |

----------------------------------------------------------------------

7.0  OBTAINING VERSION CODES FROM OS/8 MODULES

You can obtain version codes from OS/8 system modules in several ways.

Many  of  the  programs  allow  you  to enter an option to the Command

Decoder.  Other programs display their version codes when  they  begin

operation.   To  get the version codes of device handlers, you can use

either the BUILD or ODT programs.

Table 9 lists the modules and the methods for obtaining their  version

codes.   Additional  information about some of the methods is given in

the notes that follow.

Table 9

Obtaining OS/8 Module Version Codes

----------------------------------------------------------------------

|	       Module	     |		       Method			  |

|-----------------------|--------------------------------------------|

| ABSLDR		     | Examine location 12200 with ODT (Note 1)   |

|			     |						  |

| BASIC.SV		     | Type .EXECUT prog.BA/V			  |

| BAT		     | Type .RESORC/E (Note 2)			  |

| BATCH		     | Type /V after BATCH's *			  |

| BCOMP		     | Type .EXECUT prog.BA/V			  |

| BITMAP		     | See heading in output listing		  |

| BLOAD		     | Type .EXECUTE prog.BA/V			  |

| BOOT		     | Type VERS after Boot's /			  |

| BRTS		     | Type .EXECUT prog.BA/V			  |

| BUILD		     | Type VE after BUILD's $			  |

|			     |						  |

| CAMP		     | Type VE after CAMP.s #			  |

| CCL		     | Type VER after Monitor's .		  |

| CD		     | FUTIL 51.0 (Note 3)			  |

| CREF		     | See heading in output listing		  |

----------------------------------------------------------------------

(continued on next page)

17

Table 9 (Cont.)

Obtaining OS/8 Module Version Codes

----------------------------------------------------------------------

|	       Module	     |			 Method			  |

|-----------------------|--------------------------------------------|

| DIRECT		     | Type /W after DIRECT's *			  |

| DTCOPY		     | See heading in output listing		  |

| DTFRMT		     | See heading in output listing		  |

|			     |						  |

| EABRTS		     | Examine locations 1115 and 1116 with ODT   |

|			     |   (Note 1)				  |

| EDIT		     | Type # after EDIT's #			  |

| EPIC		     | ODT locations 352 and 353 (Note 1)	  |

|			     |						  |

| FORT		     | Type /V after FORT's * or examine location |

|			     |   0342 with ODT (Note 1)			  |

| FOTP		     | Type /W after FOTP's *			  |

| FRTS		     | Type /V after FRTS's * or examine locations|

|			     |   5533 and 5534 with ODT (Note 1)	  |

| F4		     | See heading in output listing or examine	  |

|			     |   location 0113 with ODT (Note 1)	  |

| FUTIL		     | Type SH VERS after FUTIL's *		  |

|			     |						  |

| KBM (Keyboard Monitor)| Type VER after Monitor's prompt.		  |

| KL8E		     | Type RESORC/E (Note 2)			  |

|			     |						  |

| LIBRA		     | Examine locations 4404 and 4405 with ODT   |

|			     |   (Note 1)				  |

| LIBSET		     | Examine location 12753 with ODT (Note 1)   |

| LOADER		     | Examine location 17374 with ODT (Note 1)   |

| LOAD		     | Examine location 12073 and 17074 with ODT  |

|			     |   (Note 1)				  |

| LPSV		     | Type RESORC/E (Note 2)			  |

| LQP		     | Type RESORC/E (Note 2)			  |

| LINCSY		     | Examine location 207 with ODT (Note 4)     |

|			     |						  |

| MCPIP		     | Type /V after MCPIP			  |

| MSBAT		     | Type /V after MSBAT'			  |

|			     |						  |

| ODT		     | FUTIL 60.0 (Note 3)			  |

| PAL8		     | See heading in output listing		  |

| PASS2		     | See heading in F4 output listing		  |

| PASS2O		     | See heading in F4 output listing		  |

| PASS3		     | See heading in F4 output listing		  |

| PIP		     | Type /V after PIP's *			  |

| PIP10		     | See directory listing or examine location  |

|			     |   4320 with ODT (Note 1)			  |

|			     |						  |

| RALF		     | See heading in F4 output listing		  |

| RESORC		     | Type RESORC/V				  |

| RF08NS		     | Type RESORC/E (Note 2)			  |

| RF08SY		     | Examine location 207 with ODT (Note 4)     |

----------------------------------------------------------------------

(continued on next page)

18

Table 9 (Cont.)

Obtaining OS/8 Module Version Codes

----------------------------------------------------------------------

|	       Module	     |			 Method			  |

|-----------------------|--------------------------------------------|

| RK08NS		     | Type RESORC/E (Note 2)			  |

| RK08SY		     | Examine location 207 with ODT (Note 4)     |

| RK8ENS		     | Type RESORC/E (Note 2)			  |

| RK8ESY		     | Type RESORC/E (Note 2)			  |

| RKLFMT		     | Examine location 0000 with ODT (Note 1)    |

| RLFRMT		     | See startup dialog			  |

| RLSY		     | Type RESORC/E (Note 2)			  |

| RL0		     | Type RESORC/E (Note 2)			  |

| RL1		     | Type RESORC/E (Note 2)			  |

| RL2		     | Type RESORC/E (Note 2)			  |

| RL3		     | Type RESORC/E (Note 2)			  |

| RLC		     | Type RESORC/E (Note 2)			  |

| ROMMSY		     | Type RESORC/E (Note 2)			  |

| RXNS		     | Examine location 206 with ODT (Note 4)     |

| RXSY1		     | Examine location 206 with ODT (Note 4)     |

| RXSY2		     | Examine location 206 with ODT (Note 4)     |

| RX28C		     | Examine location 206 with ODT (Note 4)     |

| RX78C		     | Type RESORC/E (Note 2) or examine location |

|			     |   0572 with ODT (Note 4)			  |

| RXCOPY		     | Type /V after RXCOPY's *			  |

|			     |						  |

| SABR		     | Examine location 206 with ODT (Note 1)     |

| SET		     | Type VERS after SET's #			  |

| SRCCOM		     | See output listing			  |

|			     |						  |

| TC08SY		     | Examine location 206 with ODT (Note 4)     |

| TD8ESY		     | Type RESORC/E (Note 2) or examine location |

|			     |   0207 with ODT (Note 4)			  |

| TDCOPY		     | See startup dialog or examine locations    |

|			     |   0074 and 0075 with ODT (Note 1)	  |

| TDFRMT		     | Examine location 206 with ODT (Note 1)     |

| TDINIT		     | See startup dialog or examine locations    |

|			     |   1420 and 1421 with ODT (Note 1)	  |

| TDROM		     | None					  |

| TD12K		     | None					  |

| TD8EA		     | Type RESORC/E (Note 2)			  |

| TECO		     | Type EO= after TECO's $			  |

|			     |						  |

| VXNS		     | Type RESORC/E (Note 2)			  |

| VXSY		     | Type RESORC/E (Note 2)			  |

----------------------------------------------------------------------

19

NOTES:

1.  Load the program with  the  GET  command,  then  use  ODT  to

examine the contents of the specified location.

.GET SYS:prog.ex  <RETURN>

.ODT  <RETURN>

aaaaa/cccc  <RETURN>

where:

prog.ex    is the name of the program

aaaaa      is  the  memory  location   that   contains   the

version code

nnnn       is an  octal  number  that  is  the  6-bit  ASCII

equivalent  of  the  version  code.  The left two

digits represent the  first  character,  and  the

right    two    digits   represent   the   second

character.

Example:

To obtain the version code for the ABSLDR program, type:

.GET SYS: ABSLDR

.ODT

12200/6602

The value 6602 is the  version  code  in  6-bit  ASCII.   It

translates into the characters 6B as follows:

66 02			__ __

|   |

V   V

6   B

2.  Use the RESORC CCL command with the /E  option  to  list  the

active  handlers  in the current system head.  If the handler

is not in the system head, you must:

a.  Run the BUILD program  (see  OS/8  System  Reference

Manual).

o  Load it into BUILD with the LOAD command.

o  Make it active with the INSERT command

o  Insert it into the system head  with  the  BOOT

command.

b.  Use the RESORC/E command to get  a  listing  of  the

system's handlers.

20

3.  To obtain some program version codes, you must run the  FUTIL

program as follows:

.R FUTIL  <RETURN>

bbbb.ddd/nnnn  <RETURN>

where:

bbbb       is the block number on the system device

ddd	is the displacement into the block where  the

version code resides.

nnnn       is an octal number that is  the  6-bit  ASCII

equivalent of the version code.  The left two

digits represent the first character, and the

right   two   digits   represent  the  second

character.  (See the example in Note 1.)

4.  To obtain   some  handler  version  codes,  you  must  use  ODT  as

follows:

.LOAD filnam.BN  <RETURN>

.ODT  <RETURN>

aaaaa/nnnn  <RETURN>

where:

filnam.BN   is the filename of the handler

aaaaa       is  the  memory  location   that   contains   the

version code

nnnn	is an  octal  number  that  is  the  6-bit  ASCII

equivalent  of  the  version  code.  The left two

digits represent the  first  character,  and  the

right    two    digits   represent   the   second

character.  (See the example in Note 1.)

21

OS/8 Combined Kit

Information Guide

DEC-S8-OSRNA-B-D

READER'S COMMENTS

NOTE:  This form is for document comments only.  DIGITAL will

use comments submitted on this form at the company's

discretion. If you require a written reply and are

eligible to receive one under Software Performance

Report (SPR) service, submit your comments on an SPR

form.

Did you find this manual understandable, usable, and well-organized?

Please make suggestions for improvement.

______________________________________________________________________

______________________________________________________________________

______________________________________________________________________

______________________________________________________________________

______________________________________________________________________

______________________________________________________________________

______________________________________________________________________

Did you find errors in this manual?  If so, specify the error and the

page number.

______________________________________________________________________

______________________________________________________________________

______________________________________________________________________

______________________________________________________________________

______________________________________________________________________

______________________________________________________________________

______________________________________________________________________

Please indicate the type of user/reader that you most nearly represent.

[]  Assembly language programmer

[]  Higher-level language programmer

[]  Occasional programmer (experienced)

[]  User with little programming experience

[]  Student programmer

[]  Non-programmer interested in computer concepts and capabilities

Name______________________________________Date________________________

Organization__________________________________________________________

Street________________________________________________________________

City__________________________State_____________Zip Code______________

or

Country

+-+-+-+-+-+-+-+					  +---------------+

|d|i|g|i|t|a|l|					  |  FIRST CLASS  |

+-+-+-+-+-+-+-+					  | PERMlT NO. 33 |

| MAYNARD. MASS.|

+---------------+

========

========

------------------------------------------	   ========

|	  BUSINESS REPLY MAIL		 |	   ========

| FIRST CLASS PERMIT NO.33 MAYNARD MASS. |	   ========

------------------------------------------	   ========

========

POSTAGE WILL BE PAID BY ADDRESSEE		   ========

========

========

========

RT/C SOFTWARE PUBLICATIONS  ML 5-5/E45		   ========

DIGITAL EQUIPMNT CORPORATION			   ========

146 MAIN STREET					   ========

MAYNARD, MASSACHUSETTS  01754			   ========

-----------------Do Not Tear - Fold Here and Staple-------------------