OS8SGEN.DOC

OS/8

System Generation Notes

DISCLAIMER

This document file was created by scanning  the

original  document and then editing the scanned

text.  As much as possible, the  original  text

format  was restored.  Some format changes were

made to insure this  document  would  print  on

current laser printers using 60 lines per page.

The  original  spelling  and  grammar  has been

preserved.

1-Nov-1996

OS/8

System Generation Notes

Order No. AA-H606A-TA

March 1979

ABSTRACT

This document describes the procedures for getting on line with OS/8.

SUPERSESSION/UPDATE INFORMATION:    This manual supersedes and updates

information in the OS/8 Handbook

(DEC-S8-OSHBA-A-D) and the

OS/8 Handbook Update

(DEC-S8-OSHBA-A-DN4).

OPERATING SYSTEM AND VERSION:	 OS/8 V3D.

----------------------------------------------------------------------

| To order additional copies of this document, contact the Software  |

| Distribution Center, Digital Equipment Corporation, Maynard,       |

| Massachusetts 01754 						  |

----------------------------------------------------------------------

digital equipment corporation -  maynard, massachusetts

First Printing, March 1979

The information in this document is subject to change without notice

and should not be construed as a commitment by Digital Equipment

Corporation. Digital Equipment Corporation assumes no responsibility

for any errors that may appear in this document.

The software described in this document is furnished under a license

and may only be used or copied in accordance with the terms of such

license.

No responsibility is assumed for the use or reliability of software on

equipment that is not supplied by DIGITAL or its affiliated companies.

Copyright (c) 1979 by Digital Equipment Corporation

The postage-prepaid READER'S COMMENTS form on the last page of this

document requests the user's critical evaluation to assist us in pre-

paring future documentation.

The following are trademarks of Digital Equipment Corporation:

DIGITAL		  DECsystem-l0	      MASSBUS

DEC		  DECtape	      OMNIBUS

PDP		  DIBOL		      OS/8

DECUS		  EDUSYSTEM	      PHA

UNIBUS		  FLIP CHIP	      RSTS

COMPUTER LABS	  FOCAL		      RSX

COMTEX		  INDAC		      TYPESET-8

DDT		  LAB-8		      TYPESET-11

DECCOMM		  DECSYSTEM-20	      TMS-ll

ASSIST-11		  RTS-8		      ITPS-l0

VAX	  	  VMS		      SBI

DECnet		  IAS		      PDT

DATATRIEVE	  TRAX

CONTENTS

Page

DOCUMENTATION SET FOR OS/8					     v

1.0      GETTING ON LINE WITH OS/8				     1

2.0      DECTAPE SYSTEMS 					     1

2.1	The TC01/TC08 DECtape				     2

2.2	TD8E DECtapes 					     4

2.2.1	TD8E Initialization Program			     4

2.2.2	TD8E Initialization Error Messages 		     6

2.2.3	TD8E Bootstraps 				     7

2.2.3.1	8K ROM Bootstrap (PDP-8/E) 			     7

2.2.3.2	12K TD8E Bootstrap 				     7

2.3	LINCtape (PDP-12 Users) 			     8

3.0      CASSETTE SYSTEMS 					     9

3.1	Building OS/8 from Cassette 			     9

3.2	Loading System Programs from Cassette 		     14

4.0      PAPER TAPE SYSTEMS 				     15

4.1	Building from Paper Tape 			     16

4.2	Loading a Paper Tape Binary Kit 		     19

5.0      USING A DISK AS THE SYSTEM DEVICE 		     21

5.1	RF08 and DF32 Disks 				     22

5.2	The RK8E Disk 					     22

5.3	The RK8 Disk 					     23

5.4	The RX01 Diskette 				     25

6.0      RESTARTING OS/8 					     26

INDEX							Index-1

TABLES

TABLE 1  TC01/TC08 DECtape Bootstrap			     2

2  TD8E Initialization Error Messages		     6

3  12K TD8E DECtape Bootstrap		     	     8

4  Cassette Bootstrap			     	     10

5  System Devices			     		     12

6  System Devices					     17

7  RF08/DF32 Disk Bootstrap			     	     22

8  Single RK8E Disk Bootstrap			     23

9  Multiple RK8E Disk Bootstrap			     23

10 Single RK8 Disk Bootstrap			     	     24

11 Multiple RK8 Disk Bootstrap		     	     25

12 RX01 Floppy Disk Bootstrap		     	     26

iii

DOCUMENTATION SET FOR OS/8

OS/8 SYSTEM GENERATION NOTES (AA-H606A-TA)

The  System  Generation Notes provide the information you need

to get a new OS/8 system running.

OS/8 SYSTEM REFERENCE MANUAL (AA-H607A-TA)

The System Reference Manual describes OS/8 system conventions,

keyboard commands, and utility programs.

OS/8 TECO REFERENCE MANUAL (AA-H608A-TA)

The TECO Reference Manual describes the OS/8 version  of  this

character-oriented text editing and correcting program.

OS/8 LANGUAGE REFERENCE MANUAL (AA-H609A-TA)

The  Language  Reference  Manual   describes   all   languages

supported  by  OS/8, including BASIC, FORTRAN IV, and the PAL8

assembly language.

OS/8 ERROR MESSAGES (AA-H610A-TA)

This manual lists in alphabetical  order  all  error  messages

generated by OS/8 system programs and languages.

iv

OS/8 SYSTEM GENERATION NOTES

1.0  GETTING ON LINE WITH OS/8

OS/8 software is distributed in a form appropriate for your particular

hardware  configuration.   The  general  system categories are DECtape

(LINCtape), cassette,  and  paper  tape.   This  manual  provides  the

information  that  the  user of any of these types of systems needs to

start using OS/8.

This manual also describes the procedures  for  bootstrapping  a  disk

system and for restarting OS/8.

To get on-line when your system is installed,  refer  to  the  section

that deals with your medium of distribution.

2.0 DECTAPE SYSTEMS

OS/8 supports the following DECtape  systems:   TC01/TC08,  TD8E,  and

LINCtape (PDP-12).  Since the software is supplied on a system DECtape

(or LINCtape), it is not necessary to build an initial system,  as  it

is when using cassettes or paper tapes.

Two tapes are distributed with each DECtape (LINCtape) system.

System Tape #1 contains the  system  programs  and  all  OS/8  Monitor

functions.

System Tape #2 contains TDINIT.SV (used in TD8E system initialization)

and  two TD8E DECtape monitor images (8K ROM and 12K).  Other files on

this second tape contain the device handlers in a format suitable  for

the  OS/8  BUILD program.  Each file contains a handler for a specific

device type.  These files are to be used as input for the LOAD command

in  BUILD  and  are  described  in the BUILD section of Chapter 2.  In

addition to these files, the tape  also  contains  relocatable  binary

files  of  the FORTRAN II library subroutines.  LIBSET, the FORTRAN II

librarian, is used to create a FORTRAN II library as described in  the

OS/8  Language  Reference  Manual.  Finally, the tape contains several

OS/8 help files.  These help files are intended to  provide  the  user

with  a  quick  command  summary  for most OS/8 programs.  They can be

printed with either OS/8 PIP or the CCL command HELP.

1

OS/8 SYSTEM GENERATION NOTES

2.1 The TC01/TC08 DECtape

The following short procedure is used to start  OS/8  on  a  TC01/TC08

system:

1.  Mount the  system  DECtape  (AL-4711C-BA)  on  unit  0  (this

appears  as  unit 8 on some DECtape units), making certain to

wind at least 10 feet of tape onto the empty reel.   Set  the

tape unit switches to REMOTE and WRITE LOCK.

2.  Bootstrap the OS/8 DECtape by following one of  two  methods.

If the system includes an MI8-E hardware bootstrap option:

a.  Place the terminal on line.  Raise the SING  STEP  switch

on  the  PDP-8/E  console.   Press the CONT switch.  Then

lower and raise the HALT switch.  At  least  one  console

indicator lamp should light.

b.  Having mounted the OS/8 System  Tape  #1  on  unit  0  as

described  above,  lower  and  raise the SW switch on the

left side of the console.

If the system does not include  a  hardware  bootstrap,  this

procedure  will have no effect.  In this case, execute step 1

above, place the terminal  on  line,  and  then  perform  the

switch  manipulations shown in Table 1.  For each step in the

table, place each of  the  PDP-8/E  console  SWITCH  REGISTER

switches  numbered  0  to 11 either in the up position if the

corresponding table entry is a 1, or in the down position  if

the  corresponding  table entry is a 0.  When all 12 switches

have been set to correspond to a line in  the  table,  follow

the  instructions in the right hand column and proceed to the

next line.  In step 4, for example, place switches 2,  4,  7,

and  10 in the up position;  place switches 1, 3, 5, 6, 8, 9,

and 11 in the down  position;   lift  the  DEP  switch;   and

proceed  to  step 5.  The table also includes octal values of

the binary switch settings for the benefit of users  familiar

with octal numbers.

Table 1

TC01/TC08 DECtape Bootstrap

----------------------------------------------------------------------

|	STEP  | OCTAL  |    SWITCH REGISTER     |        AND THEN	  |

|	 #    | VALUES |	SETTING	        |			  |

|--------|--------|------------------------|-------------------------|

|	      |	       |  012  345  678  91011  |			  |

|	  1   |  0000  |  000  000  000  000    |  press EXTD ADDR LOAD   |

|	  2   |	 7613  |  111  110  001  011    |  press ADDR LOAD	  |

|	  3   |	 6774  |  110  111  111  100    |  lift DEP key		  |

|	  4   |	 1222  |  001  010  010	 010    |  lift DEP key		  |

----------------------------------------------------------------------

(continued on next page)

2

OS/8 SYSTEM GENERATION NOTES

Table 1 (Cont.)

TC01/TC08 DECtape Bootstrap

----------------------------------------------------------------------

|	STEP  | OCTAL  |    SWITCH REGISTER     |        AND THEN	  |

|	 #    | VALUES |	SETTING	        |			  |

|--------|--------|------------------------|-------------------------|

|	      |	       |  012  345  678  91011  |			  |

|	  5   |	 6766  |  110  111  110	 110    |  lift DEP key		  |

|	  6   |	 6771  |  110  111  111	 001    |  lift DEP key		  |

|	  7   |	 5216  |  101  010  001	 110	|  lift DEP key		  |

| 	  8   |	 1223  |  001  010  010	 011	|  lift DEP key		  |

| 	  9   |	 5215  |  101  010  001	 101	|  lift DEP key		  |

| 	 10   |	 0600  |  000  110  000	 000	|  lift DEP key		  |

|	 11   |	 0220  |  000  010  010	 000	|  lift DEP key		  |

|	 12   |	 7754  |  111  111  101	 100	|  press ADDR LOAD	  |

|	 13   |	 7577  |  111  101  111	 111	|  lift DEP key		  |

|	 14   |	 7577  |  111  101  111	 111	|  lift DEP key		  |

|	 15   |	 7613  |  111  110  001	 011	|  press ADDR LOAD and	  |

|	      |        |			|  press CLEAR and	  |

|	      |        |			|  press CONT		  |

----------------------------------------------------------------------

Either bootstrapping procedure first rewinds the  DECtape  on

unit  0  to  the  end  zone,  then  starts  it moving forward

reading block 0 into locations beginning at 7600 in field  0.

In  block  0  of  the  DECtape  is  a larger bootstrap.  This

bootstrap continues to read the tape and install the resident

Monitor  code,  finally  turning  control  over  to  the OS/8

Keyboard Monitor.

3.  DECtape unit 0 will  rock,  and  the  console  terminal  will

respond  by  printing  a dot (.) at the left margin.  At this

point, OS/8 is active;  DECtape unit 0 must be set  to  WRITE

ENABLE.

NOTE

If  the  terminal   does   not   respond

properly,  check  that the bootstrap was

loaded  correctly,  that   unit   0   is

selected  and  set  to  REMOTE, that the

correct tape is mounted,  and  that  the

terminal  is  set to REMOTE or LINE.  If

trouble  persists,  contact  the   local

Digital sales office.

3

OS/8 SYSTEM GENERATION NOTES

2.2 TD8E DECtapes

OS/8 supports TD8E  DECtape  hardware  in  two  configurations:   TD8E

DECtape  and  12K  or  more  core,  and  TD8E  DECtape and 8K core and

256-word Read-Only-Memory (ROM).

2.2.1 TD8E Initialization Program - TD8E  DECtape  users  must  run  a

special  initialization program before OS/8 can be used.  This program

need only be run once to create the proper configuration;  thereafter,

the  appropriate  TD8E  bootstrap  (discussed  shortly) can be used to

start OS/8.

Use the following procedures to initialize the TD8E DECtape system.

1.  Mount the binary DECtape (AL-4712C-BA)  on  DECtape  unit  0.

Set the tape unit switches to REMOTE and WRITE LOCK.

2.  Turn the console terminal to LINE or REMOTE.

3.  Execute one of the TD8E bootstraps (see Section 2.2.3).

4.  When the bootstrap is executed correctly, the message:

TD8E INITIALIZER PROGRAM VERSION 4		   __________________________________

is printed on the terminal.  Then, depending upon which  type

of  TD8E  configuration  is  present,  one  of  the following

messages is printed to indicate the system on which OS/8 will

be built.

a.  8K ROM SYSTEM		  _____________

is printed if the user has the 256-word ROM.

b.  12K SYSTEM		  __________

is printed if the user has no ROM but does  have  12K  or

more of core memory.

NOTE

If neither the ROM  nor  12K  of  memory

exists, the message:

NEED ROM OR l2K			 _______________

appears, and the  machine  halts.   This

indicates  that the configuration is not

suitable for running the TD8E version of

OS/8.

4

OS/8 SYSTEM GENERATION NOTES

5.  After the message specifying the hardware configuration (a or

b above), the following instructions to the user appear:

MOUNT A CERTIFIED DECTAPE ON UNIT 1  WRITE-ENABLED		   __________________________________________________

ALWAYS KEEP ORIGINAL SYSTEM DECTAPES WRITE-LOCKED		   _________________________________________________

STRIKE A CHARACTER TO CONTINUE		   ______________________________

Perform the specified operations.  At this point, the current

OS/8 Monitor is written onto a blank DECtape on unit 1.  Note

that the original tape (on unit 0) is not written upon.

6.  When  the  copy  operation   is   complete,   the   following

instructions are printed:

DISMOUNT TAPE #2 FROM UNIT 0 AND SAVE IT		   ________________________________________

MOUNT ORIGINAL SYSTEM TAPE #l ON UNIT 0		   _______________________________________

PREPARE TO COPY FILES OVER		   __________________________

STRIKE A CHARACTER TO CONTINUE		   ______________________________

The system programs will now be copied from System Tape #1

(AL-4711C-8A) to the tape being created.  Perform the

specified operations and type any character except CTRL/Z to

continue.  PREPARE TO COPY FILES OVER means to expect copying

to take place;  no additional preparation is implied.  The

following message is printed:

COPYING FILES FROM UNIT O UNIT 1		   ________________________________

and the system copies the files and updates the DECtape

directory.

NOTE

If  you  wish  to  perform   nonstandard

special processing, you can respond with

a CTRL/Z to the preceding dialogue.   If

CTRL/Z  is typed, the following messages

appear:

TYPE 1 TO COPY FILES FROM UNIT O TO UNIT 1			 __________________________________________

TYPE 2 TO ZERO THE DIRECTORY OF UNIT 1			 ______________________________________

TYPE 3 TO LEAVE THE DIRECTORY OF UNIT 1 ALONE			 _____________________________________________

STRIKE A CHARACTER TO CONTINUE			 ______________________________

Reply with either a 1, 2,  or  3  (which

will  not  echo) to indicate the desired

option.  Typing any character other than

those  indicated will repeat the request

message.    One   of    the    following

confirmatory  messages  will  appear, to

indicate  the  options  1,  2,   or   3,

respectively:

5

OS/8 SYSTEM GENERATION NOTES

COPYING FILES FROM UNIT O TO 1			 ______________________________

ZEROING THE DIRECTORY ON TAPE UNIT 1			 ____________________________________

DIRECTORY ON UNIT 1 PRESERVED			 _____________________________

7.  When the files have been copied, the  following  instructions

appear:

REMOVE AND SAVE TAPE ON UNIT O			 ______________________________

TAKE NEW TAPE (ON UNIT 1) WHICH WAS JUST CREATED			 ________________________________________________

AND PLACE IT ON UNIT O			 ______________________

IT IS YOUR NEW OS/8 SYSTEM TAPE			 _______________________________

STRIKE A CHARACTER TO CONTINUE			 ______________________________

Remove the original OS/8 tape and save it for later use. Set

DECtape  unit  0  to WRITE-ENABLE, and type any character to

continue.  The tape on unit 0 will be initialized to a  TD8E

configuration.

When the initialization is completed, a dot (.)  is  printed

at  the  left  margin  of the terminal.  OS/8 is active on a

TD8E-based system.

2.2.2  TD8E Initialization Error  Messages - The  messages  listed  in

Table 2 may appear during the TD8E initialization process.

Table 2

TD8E Initialization Error Messages

----------------------------------------------------------------------

|		   Message	     |		 Meaning		  |

|-------------------------------|------------------------------------|

|  FATAL IO ERROR		     |  Unable to read from newly  copied |

|				     |  system tape.			  |

|				     |					  |

|	MOUNT CORRECT TAPE ON UNIT 0 |	Cannot copy tape currently	  |

|				     |  mounted.			  |

|				     |					  |

|	NEED ROM OR 12K		     |	Improper hardware configuration.  |

|				     |					  |

|	NOT ORIGINAL OS/8 SYSTEM     |	The  tape  copied from was not an |

|	TAPE #2			     |  original OS/8  tape  supplied  by |

|				     |  Digital.			  |

|				     |					  |

|	STRIKE A CHARACTER TO	     |  An  I/O  error  occurred  on  the |

|	CONTINUE		     |  DECtape.  Type any  character  to |

|				     |  retry the operation.		  |

|				     |					  |

|	TYPE ANY OTHER CHARACTER TO  |	First  retry  failed.   Type  any |

|	RETRY THIS I/O OPERATION     |	other character to retry  another |

|				     |  time.				  |

|				     |					  |

|	TYPE A TO ABORT AND	     |  Return to Step 1.		  |

|	START OVER AGAIN	     |					  |

----------------------------------------------------------------------

6

OS/8 SYSTEM GENERATION NOTES

2.2.3  TD8E Bootstraps - The 8K ROM and 12K TD8E bootstraps both  read

record  0 of the system tape into memory and then start it at location

7400 in field 0.  The  code  that  is  read  into  7400  is  a  larger

bootstrap  which  installs  all resident tables and then turns control

over to the OS/8 Keyboard Monitor or the TD8E initialization  program.

(The  12K  system  must  move down to tape block 154 to accomplish the

full bootstrap.  This explains the extra tape motion.)

When the TD8E system (either 8K ROM or 12K) is initialized, only  TD8E

DECtapes 0 and 1 (DTA0, DTA1) are available on the system.  The others

(DTA2-DTA7) are not in the system.  To make  other  drives  available,

you  must  run  the  BUILD program.  See the BUILD chapter in the OS/8								       ____

System Reference Manual for details on how to reconfigure a system.     _______________________

2.2.3.1  8K ROM Bootstrap (PDP-8/E)

1.  Set the switch  register  on  the  PDP-8/E  console  to  7470

(octal), i.e., set switches 0, 1, 2, 3, 6, 7, and 8 in the up

position, and set switches 4, 5, 9, 10, and 11  in  the  down

position.

2.  Raise the SING STEP switch.  Lower and raise the HALT switch.

3.  Press  the  EXTD  ADDR  LOAD,  ADDR  LOAD,  CLEAR,  and  CONT

switches.   The tape bootstrap will be executed and a message

will be  printed  (if  initializing)  or  the  OS/8  Keyboard

Monitor  will  print a dot (.) to indicate that it is active.

If initializing, set DECtape unit 0 to WRITE-LOCK.   If  OS/8

is already active, set DECtape unit 0 to WRITE-ENABLE.

2.2.3.2  12K TD8E Bootstrap - The contents of the 12K  TD8E  bootstrap

are included in Table 3.

The tape bootstrap will be executed and a message will be printed  (if

initializing)  or  the  OS/8  Keyboard Monitor will print a dot (.) to

indicate that it is active.  If initializing, set DECtape  unit  0  to

WRITE-LOCK.   If  OS/8  is  already  active,  set  DECtape  unit  0 to

WRITE-ENABLE.

7

OS/8 SYSTEM GENERATION NOTES

Table 3

12K TD8E DECtape Bootstrap

----------------------------------------------------------------------

|	STEP  | OCTAL  |    SWITCH REGISTER     |        AND THEN	  |

|	 #    | VALUES |	SETTING	        |			  |

|--------|--------|------------------------|-------------------------|

|	      |	       |  012  345  678  91011  |			  |

|    1   |  7300  |  111  011  000  000	|  press ADDR LOAD and	  |

|	      |	       |			|  press EXTD ADDR LOAD	  |

|    2   |  1312  |  001  011  001  010	|  lift DEP key		  |

|    3   |  4312  |  100  011  001  010	|  lift DEP key		  |

|    4   |  4312  |  100  011  001	 010	|  lift DEP key		  |

|    5   |  6773  |  110  111  ll1	 011	|  lift DEP kev		  |

|    6   |  5303  |  101  011  000	 011	|  lift DEP key		  |

|    7   |  6777  |  110  111  111	 111	|  lift DEP key		  |

|    8   |  3726  |  011  111  010	 110	|  lift DEP key		  |

|    9   |  2326  |  010  011  010	 110	|  lift DEP key		  |

|   10   |  5303  |  101  011  000	 011	|  lift DEP key		  |

|   11   |  5732  |  101  111  011	 010	|  lift DEP key		  |

|   12   |  2000  |  010  000  000	 000	|  lift DEP key		  |

|   13   |  1300  |  001  011  000  000	|  lift DEP key		  |

|   14   |  6774  |  110  111  111	 100	|  lift DEP key		  |

|   15   |  6771  |  110  111  111  001	|  lift DEP key		  |

|   16   |  5315  |  101  011  001	 101	|  lift DEP key		  |

|   17   |  6776  |  110  111  111	 110	|  lift DEP key		  |

|   18   |  0331  |  000  011  011	 001	|  lift DEP key		  |

|   19   |  1327  |  001  011  010	 111	|  lift DEP key		  |

|   20   |  7640  |  111  110  100	 000	|  lift DEP key		  |

|   21   |  5315  |  101  011  001	 101	|  lift DEP key		  |

|   22   |  2321  |  010  011  010	 001	|  lift DEP key		  |

|   23   |  5712  |  101  111  001	 010	|  lift DEP key		  |

|   24   |  7354  |  111  011  101	 100	|  lift DEP key		  |

|   25   |  7756  |  111  111  101	 110	|  lift DEP key		  |

|   26   |  7747  |  111  111  100  111	|  lift DEP key		  |

|   27   |  0077  |  000  000  111	 111	|  lift DEP key		  |

|   28   |  7400  |  111  100  000	 000	|  lift DEP key		  |

|   29   |  7300  |  111  011  000	 000	|  press ADDR LOAD and    |

|        |	       |			|  press CLEAR and	  |

|	      |	       |			|  press CONT		  |

----------------------------------------------------------------------

2.3  LINCtape (PDP-12 Users)

The following is the bootstrap procedure for PDP-12 systems:

1.  Mount the system LINCtape (AL-3580C-BM) on LlNCtape  unit  0.

Set  the LINCtape switches to WRITE-LOCK and REMOTE.  Set the

terminal to LINE or to REMOTE.

2.  Set the left switches to 0700.  Set  the  right  switches  to

0000.  Set the MODE key to LINC.

8

OS/8 SYSTEM GENERATION NOTES

3.  Press I/O PRESET.

4.  Press DO.

The LINCtape bootstrap will be executed, causing  unit  0  to

move.   When  tape  movement  stops,  make  sure  that the AC

contains -1 (has all lights on).  If the AC does not  contain

-1, return to step 1 above.

5.  Press the START 20 key.

The LINCtape on unit 0 will move again, and a dot (.) will be

printed  at  the  left  margin  of the terminal.  OS/8 is now

active.

6.  Set LINCtape unit 0 to WRITE-ENABLE.

3.0  CASSETTE SYSTEMS

Use the following procedures to build and load  an  OS/8  system  from

cassettes.

3.1  Building OS/8 from Cassette

When OS/8 software is supplied on  cassettes,  use  the  BUILD  system

library  program  to  create  the  initial OS/8 system.  The following

procedures build OS/8 onto a mass storage device.

1.  The OS/8 cassette containing BUILD (AR-4585C-BA) supplied  by

DIGITAL,   is   write-protected   (lugged   red  tabs  expose

write-protect holes).  Open the locking bar on the right side

of  cassette  transport  unit  0  by pushing it to the right.

Hold the cassette so that  the  DIGITAL  trademark  in  large

letters  is  upright  and  to the front.  Insert the cassette

into transport unit 0, rotating it over the  drive  sprockets

without  forcing  it, so that the locking bar closes over the

back edge.

Press the rewind button on the cassette transport  unit  once

to  rewind  the  tape to the beginning of its leader,/trailer

segment.  When the unit stops moving, the tape is  positioned

for data transfer operations.

2.  Bootstrap the OS/8 cassette by following one of two  methods.

If the system includes an MI8-E hardware bootstrap option:

a.  Place the terminal on line.  Raise the SING  STEP  switch

on  the  PDP-8/E  console.   Press the CONT switch.  Then

lower and raise the HALT switch.  At  least  one  console

indicator lamp should light.

9

OS/8 SYSTEM GENERATION NOTES

b.  Having mounted the OS/8 system  cassette  on  unit  0  as

described  above,  lower  and  raise the SW switch on the

left side of the console.

If the system does not include  a  hardware  bootstrap,  this

procedure  will have no effect.  In this case, execute step 1

above and then perform the switch manipulations in  Table  4.

For each step in the table, place each of the PDP-8/E console

SWITCH REGISTER switches numbered 0 to 11 either  in  the  up

position  if  the corresponding table entry is a 1, or in the

down position if the corresponding table entry is a 0.   When

all  twelve switches have been set to correspond to a line in

the table, follow instructions in the right-hand  column  and

proceed  to  the  next  line.   In step 3, for example, place

switches 2, 4, 9, and 10 in the up position;  place  switches

0,  1,  3, 5, 6, 7, 8, and 11 in the down position;  lift the

DEP switch;  and proceed to step 4.  The table also  includes

octal values of the binary switch settings for the benefit of

users familiar with octal numbers.

Table 4

Cassette Bootstrap

----------------------------------------------------------------------

|	STEP  | OCTAL  |    SWITCH REGISTER     |        AND THEN	  |

|	 #    | VALUES |	SETTING	        |			  |

|--------|--------|------------------------|-------------------------|

|	      |	       |  012  345  678  91011  |			  |

|	  1   |  4000  |  100  000  000  000	|  press ADDR LOAD and	  |

|	      |	       |			|  press EXTD ADDR LOAD	  |

|	  2   |  1237  |  001  010  011	 111	|  lift DEP key		  |

|	  3   |  1206  |  001  010  000  110	|  lift DEP key		  |

|	  4   |  6704  |  110  111  000	 100	|  lift DEP key		  |

|	  5   |  6706  |  110  111  000  110	|  lift DEP key		  |

|	  6   |  6703  |  110  111  000	 011	|  lift DEP key		  |

|	  7   |  5204  |  101  010  000	 100	|  lift DEP key		  |

|	  8   |  7264  |  111  010  110  100	|  lift DEP key		  |

|	  9   |  6702  |  110  111  000  010	|  lift DEP key		  |

|	 10   |  7610  |  111  110  001  000	|  lift DEP key		  |

|	 11   |  3211  |  011  010  001  001	|  lift DEP key		  |

|	 12   |  3636  |  011  110  011  110	|  lift DEP key		  |

|	 13   |  1205  |  001  010  000  101	|  lift DEP key		  |

|	 14   |  6704  |  110  111  000  100	|  lift DEP key		  |

|	 15   |  6706  |  110  111  000  110	|  lift DEP key		  |

|	 16   |  6701  |  110  111  000  001	|  lift DEP key		  |

|	 17   |  5216  |  101  010  001  110	|  lift DEP key		  |

|	 18   |  7002  |  111  000  000  010	|  lift DEP key		  |

|	 19   |  7430  |  111  100  011	 000	|  lift DEP key		  |

|	 20   |  1636  |  001  110  011  110	|  lift DEP key		  |

|	 21   |  7022  |  111  000  010  010	|  lift DEP key		  |

|	 22   |  3636  |  011  110  011  110	|  lift DEP key		  |

----------------------------------------------------------------------

(continued on next page)

10

OS/8 SYSTEM GENERATION NOTES

Table 4 (Cont.)

Cassette Bootstrap

----------------------------------------------------------------------

|	STEP  | OCTAL  |    SWITCH REGISTER     |        AND THEN	  |

|	 #    | VALUES |	SETTING	        |			  |

|--------|--------|------------------------|-------------------------|

|	      |	       |  012  345  678  91011  |			  |

|	 23   |  7420  |  111  100  010	 000	|  lift DEP key		  |

|	 24   |  2236  |  010  010  011	 110	|  lift DEP key		  |

|	 25   |  2235  |  010  010  011  101 	|  lift DEP key		  |

|	 26   |  5215  |  101  010  001  101	|  lift DEP key		  |

|	 27   |  7346  |  111  011  100  110	|  lift DEP key		  |

|	 23   |  7002  |  111  000  000  010	|  lift DEP key		  |

|	 29   |  3235  |  011  010  011	 101	|  lift DEP key		  |

|	 30   |  5201  |  101  010  000  001	|  lift DEP key		  |

|	 31   |  7737  |  111  111  011  111	|  lift DEP key		  |

|	 32   |  3557  |  011  101  101  111	|  lift DEP key		  |

|	 33   |  7730  |  111  111  011  000	|  lift DEP key		  |

|	 34   |  4000  |  100  000  000  000	|  press ADDR LOAD key and|

|	      |	       |	 		|  press CLEAR and 	  |

|	      |        |			|  press CONT		  |

----------------------------------------------------------------------

Either  bootstrapping  procedure  should  cause  the   system

cassette to move and BUILD to print a $ at the left margin of

the console terminal.  If there is no  response,  check  that

the  system  cassette is properly mounted on transport unit 0

and repeat the  bootstrapping  procedure,  paying  particular

attention  to  the  switch  manipulations.  Be careful not to

bounce the DEP switch.

3.  When BUILD prints:

$		   _

respond with the system device on which OS/8 is to be  built.

(At  this  point, the usual command editing features of BUILD

are available;  for details, see the  BUILD  chapter  in  the

OS/8  System  Reference Manual.) This response must be in the

following form:

$SYS dev=n		   _

where "dev" represents one of the legal  replies  taken  from

Table  5.   The  "n"  is  optional  and  need only be used to

indicate the  number  of  physical  disk  platters  that  are

present  if  the system device is RF08 or DF32.  The possible

replies and the maximum value of n that can be used for  each

one are indicated below.

11

OS/8 SYSTEM GENERATION NOTES

Table 5

System Devices

---------------------------------------------

|         Device          |	Maximum n   |

|-------------------------|-----------------|

|    DF32 (DF32 disk)	  |	    4	    |

|    RF08 (RF08 disk)	  |	    4	    |

|    RK8 (RK8 disk)	  |	    1	    |

|    RK8E (RK8E disk)	  |	    1	    |

---------------------------------------------

n must be a digit in the range 1 to 4.  If no value for n  is

specified,  a value of 1 is assumed.  If you enter a response

that is not a digit, the message:

?SYNTAX		   _

is printed, and you must type the SYS command again.  If  you

specify  n  as  a  digit  that  is  too  large for the device

specified, you must retype the SYS command.  For example:

$SYS RF08=5		   _

?PLAT		   _____

$SYS RF08=4		   _

4.  When you have entered a SYS command correctly, e.g.,

$SYS RK8E		   _

BUILD prints another $.  At this  time,  insert  the  desired

devices  for  the  initial  system, which  must  include as a

minimum the terminal handler, the mass  storage  device,  and

the  cassette  handlers.   (See the BUILD chapter in the OS/8								       ____

Reference Manual for detailed information.)	      ________________

In response to the $ printed by BUILD,  type  the  following,

remembering  to  make  a  carriage  return at the end of each

command line.

$IN TA8A:CSA0-1 	(cassette unit 0, drives 0 and 1)		   _

$IN KL8E:TTY 	(terminal keyboard)		   _

5.  You should also specify the device that is to be the  default

mass  storage  device  by  entering  the  DSK  command.   For

example:

$DSK=SYS		   _

Any device other than SYS (or carriage return)  specified  in

the  DSK  command must be the permanent name of a device that

appeared in one of the INSERT commands.

12

OS/8 SYSTEM GENERATION NOTES

6.  When  you  have  entered  all  desired  devices  with  INSERT

commands, type the following in response to the S:

$BUILD		   _

BUILD responds by printing:

LOAD OS/8:		   __________

Type CSA0, followed by carriage return, in response  to  this

message, i.e.,

LOAD OS/8: CSA0		   __________

BUILD loads and writes the various parts  of  OS/8  onto  the

system  device.   If  a  SYS  ERR  message occurs at any time

during  the  load,  make  sure  that  the  system  device  is

write-enabled  and  press  the  CONT switch to retry.  If the

retry is unsuccessful, return to step 2.

7.  After writing OS/8, BUILD prints:

LOAD CD:		   ________

Respond with a carriage  return.   BUILD  loads  the  Command

Decoder  from  cassette  unit 0 and writes it onto the system

device.

8.  When BUILD responds with another $, type the following:

$BOOT		   _

to initiate the final system creation process.  BUILD creates

OS/8  on  the  system  device,  writes  ABSLDR  on the system

device, and prints:

SYS BUILT		   _________

.		   _

The  dot  indicates  that  the  OS/8  keyboard   Monitor   is

activated.  BUILD is still in memory at this time and must be

written onto the system device.  To save the  copy  of  BUILD

just used with the current date, type:

.DATE mm/dd/yy  ( mm=month, dd=day, yy=year )		   _

.SAVE SYS BUILD		   _

This copy of BUILD reflects the current configuration of  the

system.  It can be loaded and rerun with the command:

.RUN SYS BUILD		   _

13

OS/8 SYSTEM GENERATION NOTES

9.  To prepare for loading the OS/8 system  programs  from  their

respective    cassettes,    first    load   MCPIP   (Magnetic

Tape/Cassette  Peripheral  Interchange  Program).   Type  the

following commands to load MCPIP:

.GET SYS BUILD		   _

.START 17400		   _

.SAVE SYS MCPIP; 12000=6400		   _

3.2  Loading System Programs from Cassette

After creating an OS/8 system from cassettes, you  must  transfer  the

system  programs  from  cassette  to the system device.  This transfer

operation is performed with MCPIP, which you have saved on the  system

device.

NOTE

Users with  OS/8  software  supplied  on

DECtape  (LINCtape)  already  have  core

images of the  system  programs  on  the

system  device.   This  section concerns

only users  with  software  supplied  on

cassettes.

Each  cassette  supplied  with  OS/8  contains  several  OS/8   system

programs.   To  transfer  the programs to the system device, mount the

appropriate cassette on a cassette drive and type  MCPIP  commands  as

shown  below.   Use  the  following  procedures  to  load  the  system

programs.

1.  Mount the system cassette AR-4586C-BA on cassette drive 0.

2.  Mount the system cassette AR-4587C-BA on cassette drive 1.

3.  Type the following to call MCPIP from the system device:

.R MCPIP		   _

MCPIP responds with an asterisk, indicating that it is  ready

to receive a command line of I/O specifications.

4.  Respond as follows to the asterisks printed by MCPIP:

*SYS:CCL.SV<CSA0:CCL.SV		   _

*SYS:DIRECT.SV<CSA0:DIRECT.SV		   _

*SYS:FOTP.SV<CSA0:FOTP.SV		   _

*SYS:PIP.SV<CSA0:PIP.SV		   _

*SYS:LIB8.RL<CSA0:LIB8.RL		   _

*SYS:EDIT.SV<CSA0:EDIT.SV		   _

14

OS/8 SYSTEM GENERATION NOTES

*SYS:PAL8.SV<CSA0:PAL8.SV		   _

*SYS:CREF.SV<CSA0:CREF.SV		   _

*SYS:BITMAP.SV<CSA0:BITMAP.SV		   _

*SYS:BOOT.SV<CSA0:BOOT.SV		   _

*SYS:CAMP.SV<CSA0:CAMP.SV		   _

*SYS:FORT.SV<CSA1:FORT.SV		   _

*SYS:SABR.SV<CSA1:SABR.SV		   _

*SYS:LOADER.SV<CSA1:LOADER.SV		   _

*SYS:SRCCOM.SV<CSA1:SRCCOM.SV		   _

*SYS:EPIC.SV<CSA1:EPIC.SV		   _

*SYS:PIP10.SV<CSA1:PIP10.SV		   _

*SYS:RXCOPY.SV<CSA1:RXCOPY.SV		   _

*SYS:HELP.SV<CSA1:HELP.SV		   _

5.  To write SET.SV and HELP.HL files on the system device, mount

AR-4688C-BA  in drive 0 and AR-4689C-BA in drive 1.  Type the

following command line after the asterisk is printed  on  the

terminal:

*SYS: SET.SV<CSA0:SET.SV		   _

*SYS:HELP.SV<CSA1:HELP.SV		   _

By typing the command line:

.R CCL		   _

you can run your programs using CCL commands.

6.  The source file of CCL should  be  written  onto  the  system

device  if  you  wish to add your own CCL commands.  To write

this file on the system device,  mount  the  system  cassette

AR-4690C-B  on  cassette  drive 0.  Respond as follows to the

asterisk printed by MCPIP:

*SYS:CCL.SV<CSA0:CCL.PA 		   _

This completes the building of the OS/8 system.  If the OS/8 extension

cassette  is  available,  see  the  appropriate  chapters  for loading

instructions.  Additional device  handlers  may  be  loaded  and  made

active  using  BUILD.   See  the  BUILD  chapter  in  the  OS/8 System

Reference Manual for this procedure.

4.0  PAPER TAPE SYSTEMS

You can construct an OS/8 system initially on a  mass  storage  device

from the paper tapes supplied with each OS/8 kit.  The paper tapes can

be loaded from a low-speed reader on a Teletype or from  a  high-speed

reader.  This initial construction is only necessary when the software

is not supplied on DECtape or cassettes.

15

OS/8 SYSTEM GENERATION NOTES

4.l  Building from Paper Tape

The system library program BUILD is used to construct an  OS/8  system

from paper tapes in the following manner.

1.  Load the RIM and  Binary  loaders  into  field  0  (refer  to

Appendix B in the System Reference Manual for instructions on

loading programs manually and on paper tape).

2.  Using  the  Binary  Loader,  load  the  BUILD   binary   tape

(AK-4678C-BA) into memory.

3.  After you have loaded the entire BUILD binary  tape  with  no

checksum  errors (i.e., AC=0), set the switch register to 200

(octal), i.e., set switch 4 in the up position, set all other

switches  in the down position.  Press the ADDR LOAD and CONT

switches.  BUILD prints:

$		   _

(At this point, all the usual editing features of  BUILD  are

available.)  Respond with the system (mass storage) device on

which OS/8 is to be built.  This  response  must  be  in  the

following form:

$SYS dev=n		   _

where "dev" represents one of the legal  replies  taken  from

Table  6.   The  "=n"  is  optional  and need only be used to

indicate the  number  of  physical  disk  platters  that  are

present if the system device is a RF08 or DF32 disk.

The "n" must be a digit in the range 1 to 4.  If no value for

n  is  specified,  a  value  of 1 is assumed.  If you enter a

response that is not a digit, the message:

?SYNTAX		   _

is printed, and you must type the SYS command line again.  If

you  specify  n  as  a digit that is too large for the device

specified, you must retype the SYS command.  For example:

*SYS RF08=5		   _

?PLAT		   _____

*SYS RF08=4		   _

16

OS/8 SYSTEM GENERATION NOTES

Table 6

System Devices

---------------------------------------------

|         Device          |	Maximum n   |

|-------------------------|-----------------|

|    DF32 (DF32 disk)	  |	    4	    |

|    RF08 (RF08 disk)	  |	    4	    |

|    RK8 (RK8 disk)	  |	    1	    |

|    RK8E (RK8E disk)	  |	    1	    |

---------------------------------------------

4.  When you have entered a correct SYS command line, e.g.,

$SYS RK8E		   _

BUILD prints another $.  At this  time,  insert  the  desired

devices  for the initial system.  You must insert the devices

listed below for a minimum system with paper tape.  Type  the

following commands, followed by carriage returns, to insert a

low-speed paper tape configuration.

$IN KS33:PTR,PTR 	   (low-speed paper tape punch/reader)	      _

$IN KL8E:TTY 	   (terminal keyboard)	      _

Type the following commands, followed by carriage returns, to

insert a high-speed paper tape configuration.

$IN PT8E:PTP,PTR 	   (high-speed paper tape punch/reader)	      _

$IN KL8E:TTY 	   (terminal keyboard)	      _

5.  At this time, you must specify the device that is to  be  the

default mass storage device by entering the DSK command.  For

example:

$DSK=SYS		   _

Any device other than SYS (or carriage return)  specified  in

the  DSK command must be the permanent name of a mass storage

device that appeared in one of the INSERT commands.

6.  When you have entered all desired devices with  IN  commands,

type the following in response to the BUILD $.

$BUILD		   _

BUILD responds by printing:

LOAD OS/8:		   __________

At this point, load the OS/8 Keyboard Monitor tape

(AK-4679C-BA) in the proper reader and respond PTR followed

by a carriage return, i.e.,

17

OS/8 SYSTEM GENERATION NOTES

LOAD OS/8: PTR		   __________

BUILD loads and writes the various parts of the OS/8 Keyboard

Monitor  onto the system device.  If a SYS ERR message occurs

at any time during the load, make sure that the system device

is  write-enabled  and  press  the CONT switch on the PDP-8/E

console to retry.  If the retry is  unsuccessful,  return  to

step 2.

NOTE

When building from the low-speed  reader

(KS33),  after you enter PTR followed by

carriage  return,  the  system  responds

with  an  up-arrow;  you must respond by

typing any character on the terminal and

then  immediately turning on the reader.

If the reader is not turned on promptly,

the  system hangs.  Remember to turn off

the   reader   when   it   reaches   the

leader/trailer at the end of the tape.

7.  After successfully writing  the  Keyboard  Monitor  onto  the

system device, BUILD prints:

LOAD CD:		   ________

Place the Command Decoder binary tape  (AK-4680C-BA)  in  the

proper  paper  tape  reader  and  respond  PTR  followed by a

carriage return, i.e.,

LOAD CD: PTR		   ________

BUILD loads and writes the Command Decoder.

8.  When BUILD responds with another $, type the following:

$BOOT		   _

to initiate the final system creation process.  BUILD creates

OS/8  on  the  system  device,  writes  ABSLDR  on the system

device, and prints:

SYS BUILT		   _________

.		   _

The  dot  indicates  that  the  OS/8  Keyboard   Monitor   is

activated.

9.  At this time, BUILD is still in memory and you will  have  to

copy  it  onto  the system device.  To save the copy of BUILD

with the current date, type:

18

OS/8 SYSTEM GENERATION NOTES

.DATE mm/dd/yy     (mm=month, dd=day, yy=year)		   _

.SAVE SYS BUILD		   _

This copy of BUILD reflects the current configuration of  the

system.  You can load and rerun it with the command:

.RUN SYS BUILD		   _

See the BUILD chapter in the OS/8 Reference Manual for details on  how				  _____________________

to use BUILD effectively.

You must now use EPIC (which resides on the system device) to load the

various   system   programs.   Refer  to  the  following  section  for

instructions.

4.2  Loading a Paper Tape Binary Kit

Paper tape binary kits for OS/8 V3D are punched using EPIC.  This  use

of  EPIC  simplifies loading these tapes onto SYS:.  All tapes, except

those used to build a Monitor and a System Head and EPIC itself,  must

be loaded onto SYS:  using EPIC.  The procedure for loading paper tape

binary kits is described below.

NOTE

Skip step l if EPIC.SV exists in  system

directory.

1.  Place the EPIC binary tape (AK-4667C-BA) in  the  reader  and

type:

.R ABSLDR		   _

*PTR: ($)^		   _        _

Turn on reader and type any key on keyboard.

NOTE

($) is escape or  altmode;   strike  ESC

key.

EPIC will be read in by this procedure.  If  necessary,  turn

off the reader.  Save EPIC as a file by typing:

.SAVE SYS:EPIC.SV		   _

19

OS/8 SYSTEM GENERATION NOTES

2.  Type

.R EPIC		   _

3.  To load any paper tape onto SYS:, put the paper tape for that

file in the reader and type:

*SYS:</O/Y($)   for the high-speed reader or

*SYS:</O/Y/L($) for  the  low-speed  reader.   (Turn  on  the

low-speed   reader,   depress   CONT  on  the

operator's console after the  computer  halts

to  allow  loading  the  tape  in the reader.

After the tape  has  read  in,  the  computer

halts  again.   If there are no more tapes to

that file to be loaded, turn off  the  reader

and depress CONT.  If there are more tapes to

the file continue to step 4.

4.  If the file you are creating requires more than one  tape  to

be input, the message:

END OF TAPE ENTER NEXT		   ______________________

will be displayed on the console terminal  and  the  computer

will halt with 7777(8) in the AC.  Place the next tape of the

file in the reader, turn it on,  and  depress  CONT.   Repeat

step 4 until all tapes for the file are loaded.

If the tapes of a multiple tape file are read out of sequence

an error message:

Need nnnn FOUND mmmm		   ____________________

will be output on the console terminal.  Check the  tapes  of

the  file,  place  the proper tape in the reader, and depress

CONT on the operators console.

5.  Repeat steps 3 and 4 to load each tape or set of tapes into a

file on SYS:

See the OS/8 System Reference Manual for more information  on		      ____________________________

EPIC.

6.  If desired, you can load CCL.SV, EDIT,SV,  and  BATCH.SV  (if

you  have  the  OS/8  Extension  Kit) and then create a batch

stream to load the desired files onto SYS:

Create a batch file as follows:

20

OS/8 SYSTEM GENERATION NOTES

.CREATE LOAD.BI		   _

#A		   _

$JOB TO LOAD FILES USING EPIC

.R EPIC		   _

*SYS:</O/Y$	       $=dollar sign key (shift/4) - not		   _

*SYS:</O/Y$	       escape or altmode.  Add/L before		   _

.	       $  (see example in step 3 above)

.	       if reading from low-speed

.	       reader.

*SYS:</O/Y$	       Put in a few of the load commands;		   _

.SU LOAD.BI/T       the more you put in, the fewer times		   _

the job will re-submit itself.

^L		       (CTRL/FORM) means hold down		   _

#E		       CTRL key, depress "L" (form) key		   _

Then run the file, using the command:

.SU LOAD.BI/T		   _

Every time the computer stops, replace the tape in the reader

with a new one (or next in sequence) and depress CONT.

Ignore any

L/T ERROR		   _________

messages on console terminal caused by running off the end of

the paper tape.

5.0  USING A DISK AS THE SYSTEM DEVICE

If your OS/8 system device is a disk, you must build  an  OS/8  system

onto  the  disk  from  the  distribution media, i.e., cassettes, paper

tape, or DECtape (LINCtape).  The disks available  as  system  devices

are  RF08,  DF32,  RK8,  RK8E,  and  the  RX01 diskette.  Refer to the

appropriate part of this  section  for  the  cassette  or  paper  tape

building  procedure.   For  DECtape or LINCtape distribution, refer to

the BUILD chapter in the OS/8 System Reference Manual.

Once you have built an OS/8 system on a disk, it may  occasionally  be

necessary  to start (bootstrap) the system into operation when nothing

is in memory.  For example, whenever an RK8E disk cartridge is  placed

in  its  slot  for use, the system should be bootstrapped.  Also, if a

program error is encountered  such  that  the  contents  of  locations

7600-7777 in either field 0 or field 1 are in doubt, the system should

be bootstrapped.  The following sections detail the specific bootstrap

used for each type of disk.

21

OS/8 SYSTEM GENERATION NOTES

5.1  RF08 and DF32 Disks

If the OS/8 system device is a RF08 or DF32 disk, use the bootstrap

shown in Table 7.

Table 7

RF08/DF32 Disk Bootstrap

----------------------------------------------------------------------

|	STEP  | OCTAL  |    SWITCH REGISTER     |        AND THEN	  |

|	 #    | VALUES |	SETTING	        |			  |

|--------|--------|------------------------|-------------------------|

|	      |	       |  012  345  678  91011  |			  |

|	 1    |  0000  |  000  000  000  000	|  press EXTD ADDR LOAD	  |

|	 2    |  7750  |  111  111  101  000	|  press ADDR LOAD	  |

|	 3    |  7600  |  111  110  000  000	|  lift DEP key		  |

|	 4    |  6603  |  110  110  000  011	|  lift DEP key		  |

|	 5    |  6622  |  110  110  010  010	|  lift DEP key		  |

|	 6    |  5352  |  101  011  101  010	|  lift DEP key		  |

|	 7    |  5752  |  101  111  101  010	|  lift DEP key		  |

|	 8    |  7750  |  111  111  101  000	|  press ADDR LOAD and	  |

|	      |	       |	  		|  press CLEAR and	  |

|	      |	       |			|  press CONT		  |

----------------------------------------------------------------------

When you have loaded the bootstrap, the OS/8 Keyboard  Monitor  should

respond  with  a  dot  (.).   If  it  does  not,  repeat the bootstrap

procedure.  If an error persists,  consult  the  local  DIGITAL  sales

office.

5.2  The RK8E Disk

If only one RK8E disk unit is present on  the  OS/8  system,  use  the

bootstrap shown in Table 8.

NOTE

If you  are  using  a  PDP-12  computer,

execute  an  I/O PRESET in 8 mode before

performing step 5 of  the  bootstrap  in

Table 8.

22

OS/8 SYSTEM GENERATION NOTES

Table 8

Single RK8E Disk Bootstrap

----------------------------------------------------------------------

|	STEP  | OCTAL  |    SWITCH REGISTER     |        AND THEN	  |

|	 #    | VALUES |	SETTING	        |			  |

|--------|--------|------------------------|-------------------------|

|	      |	       |  012  345  678  91011  |			  |

|	 1    |	 0000  |  000  000  000  000	|  press EXTD ADDR LOAD   |

|	 2    |	 0030  |  000  000  011  000	|  press ADDR LOAD	  |

|	 3    |  6743  |  110  111  100  011	|  lift DEP key		  |

|	 4    |	 5031  |  101  000  011  001	|  lift DEP key		  |

|	 5    |	 0030  |  000  000  011  000	|  press ADDR LOAD and	  |

|	      |	       |			|  press CLEAR and	  |

|	      |	       |			|  press CONT		  |

----------------------------------------------------------------------

If more than one RK8E disk unit is present  on  the  system,  you  may

choose  which unit (0-3) you wish to be the system device.  To specify

the correct RK8E unit  as  the  system  device,  load  the  OS/8  disk

cartridge  in  the desired unit and enter the bootstrap shown in Table

9.

Table 9

Multiple RK8E Disk Bootstrap

----------------------------------------------------------------------

|	STEP  | OCTAL  |    SWITCH REGISTER     |        AND THEN	  |

|	 #    | VALUES |	SETTING	        |			  |

|--------|--------|------------------------|-------------------------|

|	      |	       |  012  345  678  91011  |			  |

|	 1    |	 0000  |  000  000  000	 000	|  press EXTD ADDR LOAD	  |

|	 2    |	 0025  |  000  000  010  101	|  press ADDR LOAD	  |

|	 3    |	 7604  |  111  110  000  100	|  lift DEP key		  |

|	 4    |	 6746  |  110  111  100  110	|  lift DEP key		  |

|	 5    |	 6743  |  110  111  100  011	|  lift DEP key		  |

|	 6    |	 7604  |  111  110  000  100	|  lift DEP key		  |

|	 7    |	 5031  |  101  000  011  001	|  lift DEP key		  |

|	 8    |	 0025  |  000  000  010	 101	|  press ADDR LOAD	  |

----------------------------------------------------------------------

Enter the desired unit number (0-3)  in  switch  register  settings  9

and 10 as follows:

unit 0 all switches down

unit 1 switch 10 up;  all others down

unit 2 switch 9 up;  all others down

unit 3 switches 9 and 10 up;  all others down

Press CLEAR and CONT.

23

OS/8 SYSTEM GENERATION NOTES

If you have loaded either of the bootstraps, the OS/8 Keyboard Monitor

should  respond  with a dot (.).  If it does not, repeat the bootstrap

procedure.  If an error persists,  consult  the  local  DIGITAL  sales

office.

5.3  The RK8 Disk

If you have only one RK8 disk unit on your OS/8 system, use the

bootstrap in Table 10 to start.

Table 10

Single RK8 Disk Bootstrap

----------------------------------------------------------------------

|	STEP  | OCTAL  |    SWITCH REGISTER     |        AND THEN	  |

|	 #    | VALUES |	SETTING	        |			  |

|--------|--------|------------------------|-------------------------|

|	      |	       |  012  345  678  91011  |			  |

|	 1    |	 0000  |  000  000  000  000	|  press EXTD ADDR LOAD   |

|	 2    |	 0030  |  000  000  011  000	|  press ADDR LOAD	  |

|	 3    |	 6733  |  110  111  011	 011	|  lift DEP key		  |

|	 4    |	 5031  |  101  000  011	 001	|  lift DEP key		  |

|   5    |  0030  |  000  000  011  000	|  press ADDR LOAD and	  |

|	      |	       |			|  press CLEAR and	  |

|	      |	       |			|  press CONT		  |

----------------------------------------------------------------------

NOTE

If you  are  using  a  PDP-12  computer,

execute  an  I/O PRESET in 8 mode before

performing   step   5   of   the   above

bootstrap.

If more than one RK8 disk unit is  present  on  the  system,  you  may

choose  which unit (0-3) you wish to be the system device.  To specify

the correct RK8  unit  as  the  system  device,  load  the  OS/8  disk

cartridge  in  the desired unit and enter the bootstrap shown in Table

11.

24

OS/8 SYSTEM GENERATION NOTES

Table 11

Multiple RK8 Disk Bootstrap

----------------------------------------------------------------------

|	STEP  | OCTAL  |    SWITCH REGISTER     |        AND THEN	  |

|	 #    | VALUES |	SETTING	        |			  |

|--------|--------|------------------------|-------------------------|

|	      |	       |  012  345  678  91011  |			  |

|	 1    |	 0000  |  000  000  000  000	|  press EXTD ADDR LOAD	  |

|	 2    |  0026  |  000  000  010  110	|  press ADDR LOAD	  |

|	 3    |  7604  |  111  110  000  100	|  lift DEP key		  |

|	 4    |	 6732  |  110  111  011  010	|  lift DEP key		  |

|	 5    |	 6733  |  110  111  011  011	|  lift DEP key		  |

|	 6    |  5031  |  101  000  011  001	|  lift DEP key		  |

|	 7    |  0026  |  000  000  010	 110	|  press ADDR LOAD	  |

----------------------------------------------------------------------

enter the desired unit number (0-3) in the switch register settings  9

and 10 as follows:

unit 0 all switches down

unit 1 switch 10 up;  all others down

unit 2 switch 9 up;  all others down

unit 3 switches 9 and 10 up;  all others down

Press CLEAR and CONT.

If you have loaded either of the above bootstraps, the  OS/8  Keyboard

Monitor  should  respond  with  a dot (.).  If it does not, repeat the

bootstrap procedure.  If an error persists, consult the local  Digital

sales office.

5.4  The RX01 Diskette

If the OS/8 system device is  an  RX01  diskette,  use  the  following

bootstrap procedure.

25

OS/8 SYSTEM GENERATION NOTES

Table 12

RX01 Floppy Disk Bootstrap

----------------------------------------------------------------------

|	STEP  | OCTAL  |    SWITCH REGISTER     |        AND THEN	  |

|	 #    | VALUES |	SETTING	        |			  |

|--------|--------|------------------------|-------------------------|

|	      |	       |  012  345  678  91011  |			  |

|	  1   |  0000  |  000  000  000  000	|  press EXTD ADDR LOAD	  |

|	  2   |  0024  |  000  000  010  100	|  press ADDR LOAD	  |

|	  3   |  7126  |  111  001  010  110	|  lift DEP key		  |

|	  4   |  1060  |  001  000  110  000	|  lift DEP key		  |

|	  5   |  6751  |  110  111  101  001	|  lift DEP key		  |

|	  6   |  7201  |  111  010  000  001	|  lift DEP key		  |

|	  7   |  4053  |  100  000  101  011	|  lift DEP key		  |

|	  8   |  4053  |  100  000  101  011	|  lift DEP key		  |

|	  9   |  7104  |  111  001  000  100	|  lift DEP key		  |

|	 10   |  6755  |  110  111  101	 101	|  lift DEP key		  |

|	 11   |  5054  |  101  000  101  100	|  lift DEP key		  |

|	 12   |  6754  |  110  111  101  100	|  lift DEP key		  |

|	 13   |  7450  |  111  100  101  000	|  lift DEP key		  |

|	 14   |  7610  |  111  110  001  000	|  lift DEP key		  |

|	 15   |  5046  |  101  000  100  110	|  lift DEP key		  |

|	 16   |  1060  |  001  000  110  000	|  lift DEP key		  |

|	 17   |  7041  |  111  000  100  001	|  lift DEP key		  |

|	 18   |  1061  |  001  000  110  001	|  lift DEP key		  |

|	 19   |  3060  |  011  000  110  000	|  lift DEP key		  |

|	 20   |  5024  |  101  000  010  100	|  lift DEP key		  |

|	 21   |  6751  |  110  111  101  001	|  lift DEP key		  |

|	 22   |  4053  |  100  000  101  011	|  lift DEP key		  |

|	 23   |  3002  |  011  000  000  010	|  lift DEP key		  |

|	 24   |  2050  |  010  000  101  000	|  lift DEP key		  |

|	 25   |  5047  |  101  000  100  111	|  lift DEP key		  |

|	 26   |  0000  |  000  000  000  000	|  lift DEP key		  |

|	 27   |  6753  |  110  111  101  011	|  lift DEP key		  |

|	 28   |  5033  |  101  000  011  011	|  lift DEP key		  |

|	 29   |  6752  |  110  111  101  010	|  lift DEP key		  |

|	 30   |  5453  |  101  100  101  011	|  lift DEP key		  |

|	 31   |  7024  |  111  000  010  100	|  lift DEP key		  |

|	 32   |  6030  |  110  000  011  000	|  lift DEP key		  |

|	 33   |  0033  |  000  000  011  011	|  press ADDR LOAD and	  |

|	      |	       |			|  press CLEAR and	  |

|	      |	       |			|  press CONT		  |

----------------------------------------------------------------------

6.0  RESTARTING OS/8

If you ever fail to receive apparent response from  the  OS/8  system,

you  can  restart  the computer by loading a restart address of either

7600 or 7605.  If you choose to start at location 7600, you  can  save

the  contents  of  locations  0-1777  on  the  system  device.   These

locations  are  then  available  when  the  Keyboard  Monitor  resumes

26

OS/8 SYSTEM GENERATION NOTES

operation.   Starting  at  7605  does not have the core locations, but

does save time on a DECtape configuration.

To load a restart address, set the console switches to 7600  or  7605;

press  the  HALT,  ADDR  LOAD, EXTD ADDR, CLEAR, and CONT switches.  A

period should appear on the terminal.  If there is no  response,  OS/8

is no longer in memory and must be bootstrapped in.

27

INDEX

Bootstraps,			   Initialization program, TD8E,

8K ROM (PDP8/E) for TD8E		       4-6

DECtape, 7

12K TD8E DEC tape, 7

cassette, 9

multiple RK8 disk, 24		   LINCtape, 8-9

multiple RK8E disk, 23

RF08/DF32 disk, 21

RX01 diskette, 25

single RK8 disk, 24		   Paper tape,

single RK8E disk, 22		     building OS/8 from, 15-19

TC01/TC08 DECtape, 2		     loading a binary kit from,

19-21

Cassette systems, 9-15

bootstrap, 9

loading system programs from,	   RF08/DF32 disk, 22

14-15				     bootstrap, 22

RK8 disk, 24-25

bootstrap for multiple RK8,

25

DECtape systems, 1-8		     bootstrap for single RK8, 24

bootstraps, 2, 3, 7		   RK8E disk, 22-24

TC01/TC08, 2-3			     bootstrap for multiple disk,

TD8E, 4-8			       23

Diskettes, RX01, 25		     bootstrap for single disk, 23

Disks,				   RX01 diskette, 25

bootstraps, 22, 23, 24, 25	     bootstrap, 26

RF08/DF32, 22

RK8, 24-25

RK8E, 22-24

TC01 DECtape, 2

bootstrap, 2

Error messages,			   TD8E DECtapes, 4-8

TD8E initialization program,	     bootstraps, 7, 8

6				     initialization program, 4-6

Index-1

READER'S COMMENTS

NOTE:  This form is for document comments only.   DIGITAL will

use comments submitted on this form at the company's

discretion.   If you require a written reply and are

eligible to receive one under Software Performance

Report (SPR) service, submit your comments on an SPR

form.

Did you find this manual understandable, usable, and well-organized?

Please make suggestions for improvement.

______________________________________________________________________

______________________________________________________________________

______________________________________________________________________

______________________________________________________________________

______________________________________________________________________

______________________________________________________________________

______________________________________________________________________

______________________________________________________________________

______________________________________________________________________

Did you find errors in this manual?  If so, specify the error and the

page number.

______________________________________________________________________

______________________________________________________________________

______________________________________________________________________

______________________________________________________________________

______________________________________________________________________

______________________________________________________________________

______________________________________________________________________

______________________________________________________________________

Please indicate the type of user/reader that you most nearly represent.

[]  Assembly language programmer

[]  Higher-level language programmer

[]  Occasional programmer (experienced)

[]  User with little programming experience

[]  Student programmer

[]  Other (please specify) _______________________________________

Name______________________________________Date________________________

Organization__________________________________________________________

Street________________________________________________________________

City__________________________State_____________Zip Code______________

or

Country

-------------------------------Fold Here------------------------------

-----------------Do Not Tear - Fold Here and Staple-------------------

+---------------+

|  FIRST CLASS  |

| PERMlT NO.  33 |

| MAYNARD.  MASS.|

+---------------+

BUSINESS REPLY MAIL					   ========

NO POSTAGE STAMP NECESSARY IF MAILED IN THE UNITED STATES  ========

======================================================================

Postage will be paid by:				   ========

========

+-+-+-+-+-+-+-+			   ========

|d|i|g|i|t|a|l|			   ========

+-+-+-+-+-+-+-+			   ========

========

Software Communications			   ========

146 Main Street  ML5-5/E39		   ========

Maynard, Massachusetts  01754		   ========