Programming the DECMATE II for the RD51D Disk Subsystem

Revision 1

16-March-83

John Elkins

Robert F. Morse

Table Of Contents

Section    						     Page

1.	INTRODUCTION					       1

Bad block mapping				       2

Error retries					       2

2.	SYSTEM CONTROL DATA ON DISK			       3

Block 1: Disk geometry and bad block map	       4

Blocks 13..15: Disk directory			       5

3.	6120 INSTRUCTIONS AND FLAGS			       6

Instructions					       6

Flags						       6

Interrupts					       7

Command execution				       7

Power-on reset				       8

4.	CONTROLLER COMMANDS				       9

Command errors				      10

Command error codes				      11

5.	COMMAND DETAILS

Mount volume					      12

Get volume data				      13

Update volume data				      14

Read disk directories				      15

Dismount volume				      16

Set block					      17

Fill buffer					      18

Write						      19

Read						      20

Empty buffer					      21

Get error					      22

Test error					      23

Get status					      24

Execute self-test				      25

Set retry-count				      26

Set normal mode				      27

Set special mode				      28

Set format sequence				      29

Set physical address				      30

Restore					      31

Format					      32

APPENDIX A.  OPERATING SYSTEM CONSIDERATIONS		      A1

APPENDIX B.  CHANGES TO RD51 PROGRAMMING SPECIFICATIONS       B1

-----------------------------------------------------------------

Programming the DECMATE II for the RD5lD Disk Subsystem

Revision 1, 16-March-83					  Page i

-----------------------------------------------------------------

Programming the DECMATE II for the RD51D Disk Subsystem	    _______________________________________________________

1. INTRODUCTION        _______________

The RD51D Disk Subsystem consists of a controller and one or  two					       __________

Winchester disk units.  Each unit has up to eight heads and up to			_____				  _____

4096 cylinders.  Each cylinder-head (track) has 16 sectors of 512	     _________			     _____	   _______

bytes each.

Each physical unit contains up to 60 logical volumes.  Disk  data						     _______

are  addressed in units of 512-byte (or 256-word) blocks relative							  ______

to the beginning of a volume (the first  block  is  numbered  0).

Each  block  is stored in one physical sector (the terms are used

interchangably in this document).

Volumes are created on a disk by entering their descriptions in a

disk  directory which is written in a fixed location on the disk.	______________

Each volume's entry  in  the  directory  has  an  eight-character

volume name, separate 16-bit passwords for read and write access,

a physical disk starting block, a block  count  which  determines

the  volume's  size,  a system format indicator (e.g., WPS, CP/M,

COS-310, OS/8), and additional operating-system dependent data.

The controller allows access to a volume  via  a  path  called  a

logical  device.  The  controller supports 16 such devices, which	_______________

are designated by numbers from  0  through  15.  To  access  disk

data  it  is  first necessary to associate a volume with a device

number by issuing  a  MOUNT  VOLUME  command.  The  eight  device

numbers  0  through  7  are  for  use  by  operating  systems and

application programs.  Devices 8  through  15  are  reserved  for

special functions.

Once a volume is mounted, its blocks can be read and written.  To

read  a  block,  use  the SET BLOCK command to specify the device

number and block number.  Then issue the READ  command  and  wait

for the "done" flag.  Then use an EMPTY BUFFER command to get the

block image from the controller's buffer.

To write disk data, issue SET BLOCK to specify device  and  block

number.  Then  use a FILL BUFFER command to send the block's data

to the controller's buffer and issue a WRITE command.

For all normal volumes, only those blocks within the  volume  can

be  accessed, and volume spaces do not overlap (i.e. a particular

physical block can be assigned to only  one  volume).  Access  to

-----------------------------------------------------------------

Programming the DECMATE II for the RD5lD Disk Subsystem

Revision 1, 16-March-83					  Page 1

-----------------------------------------------------------------

the entire  disk  for  volume  directory  maintenance  is  via  a

pseudo-volume  called  the "master volume", which maps the entire

physical disk as a volume such that  volume  block  0  refers  to

physical  block  0.  The  master  volumes  for  units 0 and 1 are

permanently mounted on devices 8 and 9, respectively.

A 6120 program uses six IOT instructions to send commands to  the

controller,  exchange  data  with it, and test its status.  These

commands are discussed in section 3, below.  Section 2  describes

the principal data structures on the disk and in the controller's

local storage which are necessary to understand the operation  of

the   various   controller  commands.  Section  4  describes  the

controller commands in detail.

Bad block mapping	_________________

The controller automatically resolves  references  to  physically

defective blocks.  The disk control block (permanently written on

block 1 of each disk) contains a  list  of  physically  defective

blocks  and  the  location  of  a replacement block for each one.

This list is read  into  the  controller's  storage  whenever  an

EXECUTE  SELF-TEST  command  is  executed  to create a "bad block

map".

There is a value in each sector header that indicates  whether  a

block  is  a  bad  block.  If a block is bad or if a block is not

found or if a sector header  contains  a  CRC  error,  the  block

number  is  looked  up in the bad-block map.  If the block is not

found in the bad-block map, retries will continue.  If the  block

is  found,  the block number of the corresponding good block will

be substituted for the bad block number and  the  read  or  write

will be retried.

Error retries	_____________

Retries consist of  the  following:  If  the  DAA  state  machine

detects  an  error in a block, the controller firmware determines

whether the error is  retryable.  There  are  two  categories  of

retryable errors:

Category 1 errors consist of data or header  CRC  errors  and

header  found but no data found.  These errors are retried by

staying on track and  continuing  to  attempt  the  operation

until  either the error goes away or the retry-count (see SET

RETRY-COUNT) has been satisfied.

Category 2 errors consist of header-not-found  errors.  These

errors  are  retried  by  restoring  the  unit  to  track  0,

reseeking, and trying again until either the error goes  away

or the retry-count has been satisfied.

-----------------------------------------------------------------

Programming the DECMATE II for the RD5lD Disk Subsystem

Revision 1, 16-March-83					  Page 2

-----------------------------------------------------------------

2.  SYSTEM CONTROL DATA ON DISK        _______________________________

The first 64 blocks of a disk contain  special  information  that

the disk controller firmware requires for operation.

Blocks	Usage             ______     _____

0		System boot block.  This block  is  read  by  the

DECmate   II  ROM  bootstrap  code  to  determine

whether there is valid firmware in blocks 16..47.

1		Disk geometry and bad block map.

2..12	    (Reserved)

13..15 	Disk directory.  Each block contains a header and

twenty 24-byte directory entries.

16..47 	DECmate II firmware.

48..63 	Alternates for bad sectors.

These blocks constitute a volume whose name is always "FIRMWARE".

This volume is created by the utility program which formats a new

disk unit, and can  be  mounted  by  programs  which  update  the

firmware   or  directory  contents.  (The  contents  of  the disk

directory can be read more easily  and  safely  by  a  READ  DISK

DIRECTORIES command.)

-----------------------------------------------------------------

Programming the DECMATE II for the RD5lD Disk Subsystem

Revision 1, 16-March-83					  Page 3

-----------------------------------------------------------------

Block 1:  Disk geometry and bad-block map	_________________________________________

This block contains information about the unit geometry  and  bad

block  mapping.  Its contents are read by the controller into its

local RAM storage during an EXECUTE SELF TEST command  (which  is

automatically performed upon a power-on reset).

Since the controller is designed to operate with a wide range  of

Winchester units, it must accomodate varying numbers of cylinders

and heads.  Therefore this geometry information is recorded in  a

"disk  control block" at a fixed location on the disk that may be

accessed without prior knowldege of  the  unit's  geometry.  (All

units are assumed to have 16 blocks of 512 bytes on each track).

Occasionally a unit will have bad spots that make  it  impossible

to  reliably  read  or  write some blocks.  When these blocks are

discovered, their numbers are recorded in a  "bad-block  map"  in

the  disk  control  block  and they are physically formatted with

"bad block" flags.  Subsequent references to such  a  block  will

cause  it  them  to  be  looked  up  in  the bad-block map, which

contains the location of an alternate  block  to  be  used  as  a

replacement for the original block.

The disk control block is physical block 1 and looks like this:

Bytes	Contents	    _____	________

0..7 	Literal ASCII value "DRIVEHDR"

8..15 	Disk name (ASCII)

16..19 	Disk password (binary)

20..31 	    Reserved

32..33 	16-bit number of  cylinders  on  the  unit  (low,

high).

34		Number of heads on the unit (1..8).

35..63	    Reserved

64..191	Bad block map.  Provision is made to map up to 16

bad  blocks.  The  map  contains  16 entries of 8

bytes each.  The first four  bytes  identify  the

bad block, as follows:

0:	low-order cylinder number

1:	high-order cylinder number

2:	head number

3:	sector number

The next four  bytes  identify  (using  the  same

format) the replacement block.

-----------------------------------------------------------------

Programming the DECMATE II for the RD5lD Disk Subsystem

Revision 1, 16-March-83					  Page 4

-----------------------------------------------------------------

Blocks 13..15:  Disk directory	______________________________

Each disk contains up to 60 logical volumes allocated by  a  user

through  the  use of a special disk utility program.  Each volume

has a name, read and write passwords, a number of  blocks  (which

is always a multiple of 16), and a physical starting point.

The disk directory occupies three blocks and each block  contains

directory  entries  for  20  volumes.  Each block is formatted as

follows:

Bytes	Contents	    _____	________

0..11	Literal ASCII value "DIRECTORY   "

12..31 	    (reserved)

32..511 	Twenty entries of 24 bytes each

Each directory entry contains information  as  described  in  the

following  table.  Note  that  bits  within a byte are numbered 7

through 0, left to right. Bit 0 is the least-significant bit.

Bytes	Contents	    _____	________

0..7 	Volume name (ASCII)

8..9 	Password for reading (binary)

10..11 	Password for writing (binary)

12..13	Physical starting  block,  divided  by  16  (low-

order byte first)

14..15	Number  of  blocks  in  volume,  divided  by   16

(low-order byte first)

16		<4> "1" = active volume entry; "0" = unused entry

<2> "1"  marks  the  "startup  volume"  which  is

mounted by the MOUNT STARTUP VOLUME command

<1>  "1"  indicates  that  the  volume  has  been

modified  by one or more WRITE commands (this bit

should be cleared by a utility which has  made  a

backup copy)

17..23 	7 bytes of operating system data (see Appendix A)

-----------------------------------------------------------------

Programming the DECMATE II for the RD5lD Disk Subsystem

Revision 1, 16-March-83					  Page 5

-----------------------------------------------------------------

3. 6120 INSTRUCTIONS AND FLAGS	______________________________

Instructions	____________

Six instructions are used to program the RD51D controller.  There

is one instruction to send a command (RDSC), one to transfer data

(RDTD), one to control interrupts (RDWE), and three to  test  and

reset  the  flags.  All  of  the  instructions clear the AC after

execution except for an RDTD when the direction of data  transfer

is  "in"  to  the 6120.  Table 1 lists the instructions and their

octal values.

_________________________________________________________________

Octal   Mnemonic	Description	    _____   ________    ___________

6700     --		     (reserved, effect TBS)

6701    RDSR	Skip on DATA REQUEST flag; clear flag.

6702    RDSC	Send command (in AC) to controller.

6703    RDSD	Skip on DONE flag; clear flag.

6704    RDTD	Transfer data to/from AC <0:11>.

6705    RDWE	Write interrupt-enable mask from AC <11>.

6706    RDSE	Skip on ERROR flag; clear flag.

6707     --		     (reserved, effect TBS)

Table 1. 6120 Instructions for RD51D

_________________________________________________________________

Flags	_____

Three flags are used to synchronize the 6120 and the  controller.

They are:

-- DONE, 	  which is set at the  completion  of  a  command

(and     causes    an    interrupt    if    the

interrupt-enable mask has been set).

-- ERROR, 	  which is set simultaneously with  DONE  if  the

command ended with an error condition.

-- DATA REQUEST,  which is set for each word to be transferred to

or  from the controller for commands which have

associated data transfers.

Each flag has an associated "skip" instruction  which  tests  the

flag  and,  if the flag is set, skips the next instruction in the

program and clears the flag.  A 6120  CAF  instruction  does  not								      ___

affect the flags.

-----------------------------------------------------------------

Programming the DECMATE II for the RD5lD Disk Subsystem

Revision 1, 16-March-83					  Page 6

-----------------------------------------------------------------

Interrupts	__________

The RDWE instruction stores the low-order-bit of the  AC  in  the

controller's  interrupt-enable  mask.  A "one" enables interrupts

so that the assertion of the DONE  flag  generates  an  interrupt

request.  This  mask  is cleared to "zero" uponpower-on reset.  A

6120 CAF instruction does not affect the interrupt mask.

Command execution	_________________

A program executes an RD51D command as follows:

-- Issue a command.

-- Transfer zero or more words of data.

-- Wait for done flag.

-- Test error flag.

To issue a command, the program loads the  AC  with  the  desired

command word and executes an RDSC instruction to send the command

to the controller.  The AC is cleared by this instruction.

Each command may  be  followed  by  a  number  of  data  transfer

requests  (which may  be  zero).  The  command  descriptions  (in

section 4) show for each command the number and direction of such

transfers.  If the number is not zero, the program transfers each

word by waiting until the DATA REQUEST flag  is  set  (using  the

RDSR  instruction)  and  then  issuing  an  RDTD  instruction  to

transfer one word of data.  If the  transfer  direction  is  "in"

from  the controller to the 6120, the RDTD clears the AC and then

loads it with the incoming word.  If the  transfer  direction  is

"out" to the controller from the 6120, the program must place the

outgoing data in the AC  before  issuing  the  RDTD.  The  AC  is

cleared by an RDTD for outgoing data.

The program uses the RDSD instruction to test the DONE flag which

indicates  the  completion  of a command.  If the RD51D interrupt

enable mask is set, assertion of the DONE flag will also  request

an  interrupt.  In  this  case,  the  interrupt service code must

issue an RDSD to clear  the  flag  and  turn  off  the  interrupt

request.

After the DONE flag has been detected and  cleared,  the  program

may  use  the  RDSE instruction to test and clear the ERROR flag.

The GET ERROR command can be issued to determine the nature of an

error.

Once a command has been issued to  the  controller,  the  program

must  monitor the DATA REQUEST and DONE flags and satisfy them as

described above before attempting another command.  There  is  no

way  to  prematurely  terminate  a  command  except by a power-on

reset.  Note that the CAF instruction has no effect on the  RD51D

controller.

-----------------------------------------------------------------

Programming the DECMATE II for the RD5lD Disk Subsystem

Revision 1, 16-March-83					  Page 7

-----------------------------------------------------------------

Power-on reset	______________

A power-on reset causes the controller to execute the  equivalent

of  an EXECUTE SELF-TEST command and to set the DONE and possibly

the ERROR flags to reflect the outcome of  this  operation.  Note

that this operation may require 20 seconds or more to complete if

the disk unit must spin up to speed from rest.

-----------------------------------------------------------------

Programming the DECMATE II for the RD5lD Disk Subsystem

Revision 1, 16-March-83					  Page 8

-----------------------------------------------------------------

4. CONTROLLER COMMANDS	______________________

This section describes the commands which  are  executed  by  the

controller.  These are listed below in Table 2.

_________________________________________________________________

Octal

Command Name		 In/out   Words	   code	    ____________		 ______   _____   _____

Normal commands		_______________

MOUNT VOLUME		 Out	      9   0000

GET VOLUME DATA		 In	     24   0030

UPDATE VOLUME DATA		 Out	     25   0006

READ DISK DIRECTORIES	 In	 n * 24   0033

DISMOUNT VOLUME		 Out	      1   0005

SET BLOCK			 Out	      3   0001

FILL BUFFER (8-bit)	 Out	    512   0102

FILL BUFFER (12-bit)	 Out	    256   0002

WRITE			 -	      -   0003

READ			 -	      -	  0004

EMPTY BUFFER (8-bit)	 In	    512   0125

EMPTY BUFFER (12-bit)	 In	    256   0025

GET ERROR		 	 In	      1	  0027

TEST ERROR			 -	      -   0021

GET STATUS			 In	      5   0026

EXECUTE SELF-TEST		 -	      -   0011

SET RETRY COUNT		 Out	      1   0013

SET NORMAL MODE		 -	      -   0020

SET SPECIAL MODE		 -	      -	  0007

SET FORMAT SEQUENCE	 Out	     16	  0015

Special commands		________________

SET PHYSICAL ADDRESS	 Out	      4   0014

RESTORE			 -	      -   0016

FORMAT			 -	      -   0017

Table 2.  Command Summary.

_________________________________________________________________

-----------------------------------------------------------------

Programming the DECMATE II for the RD5lD Disk Subsystem

Revision 1, 16-March-83					  Page 9

-----------------------------------------------------------------

There are two classes of commands:  normal  and  special.  Normal

commands  are  used by operating systems and application programs

to read and write the various volumes on a disk.  As long as  the

FIRMWARE  volume  is not mounted, none of the normal commands can

compromise the volume structure of the disk, and only  blocks  in

volumes  which are mounted can be read and written.  Only devices

0 through 7 may be used in normal mode.

System utility programs which create and delete volumes and  make

backup  copies  can  use  the master volumes which are mounted on

devices 8 and 9 to obtain write access to the  volume  directory.

These  devices can only be used when the controller is in special

mode.

Special commands are used only to physically format  a  disk  and

write   its  disk  control  block.  They  allow  device-dependent

addressing by physical unit, cylinder, head, and sector  and  for

execution of the FORMAT and RESTORE commands.  These commands can

only be executed after  a  SET  SPECIAL  MODE  command  has  been

issued.  The controller is set to normal mode by a power-on reset

and by SET NORMAL MODE and EXECUTE SELF-TEST commands.

Command errors	______________

Execution of a command sets or clears the ERROR flag and  puts  a

value  in  the error register in the controller which can be read

by a GET ERROR command.  The  detailed  command  descriptions  in

this  section list the error codes which can be generated by each

command.  There is a group of codes which can result from  actual

read  or write access to a disk; these are denoted by the summary

code DIO in the command  descriptions.  These  codes  (in  octal)

are:

0001  Invalid cylinder number (too high for disk)

0005  Data CRC error

0006  Header CRC error

0007  Header not found (format error, or bad block  with  no

alternate assigned in bad-block map)

0014  DAA failed to complete (controller hardware error)

0015  DAA failed to start (controller hardware error)

0016  Invalid head number (too high for disk)

0020  Invalid unit--not ready or not installed

0021  Seek incomplete (disk hardware error)

0032  No data found after header (format error)

An RDSC instruction which sends an invalid command  code  to  the

controller  yields  an error code of 0011.  Table 3 lists all the

error codes.

-----------------------------------------------------------------

Programming the DECMATE II for the RD5lD Disk Subsystem

Revision 1, 16-March-83					  Page 10

-----------------------------------------------------------------

_________________________________________________________________

Octal

code  Meaning	    _____  _______

0000   No error

0001   Invalid cylinder number

0002   Invalid block number

0003     (not used)

0004     (not used)

0005   Data CRC error

0006   ID (header) CRC error

0007   ID not found

0010   Write fault

0011   Illegal command code

0012   Unable to find cylinder 0

0013   (not used)

0014   DAA state machine failed to complete in 2 revolutions

0015   DAA state machine failed to respond to abort command

0016   Invalid head selected

0017   No units in system

0020   Invalid unit--not ready or not installed

0021   Seek incomplete

0022   Invalid device number

0023   Volume name not found in directory

0024   Device has no volume mounted

0025   Read or write access denied

0026   Special mode required

0027   RAM failure during self-test

0030   Bad format command

0031   No index found during FORMAT command

0032   Time-out (found header but no data)

0033   DAA failed self-diagnosis

0034   Invalid volume directory

0035   Invalid disk control block

0036     (not used)

0037     (not used)

Table 3. Command Error Codes.

_________________________________________________________________

-----------------------------------------------------------------

Programming the DECMATE II for the RD5lD Disk Subsystem

Revision 1, 16-March-83					  Page 11

-----------------------------------------------------------------

MOUNT VOLUME 					     MOUNT VOLUME	____________					     ____________

This command associates a volume with a device number so that its

contents can be read and written.  A MOUNT VOLUME command must be

issued once before the first SET BLOCK, READ or WRITE command for

that   device.   Each   volume  in  the  disk  directory  has  an

8-character name.  In addition, one volume may be marked  with  a

special  flag  as the "startup volume".  In this command, you may

specify the volume you want either by name or by asking  for  the

startup volume.

Command code:  0000

Data words:  9 out

Word	  Bits  Contents		 ____	  ____  ________

1    <4:4>  Read access allowed if "1"

<5:5>  Write access allowed if "1"

<6:6>  "0" for unit 0; "1" for unit 1

<7:7>  "0" for mount by name; "1" to  mount  the

startup volume

<8:11> 	Device number by which to refer  to  this

volume (0..15)

2..9   <4:11>  8-character volume name

Error codes:

0022  Invalid device number

0023  Volume name not found in directory

0026  Special mode required for devices 8..15

0034  Invalid volume directory

DIO   Disk read errors

Notes:

1.  Execution of this command sets the device number to  the

number  specified in word 1 and sets the block number to

0.

2.  This command destroys the contents of the block buffer.

3.  Do not attempt to mount a volume  on  devices  8  or  9;

these  are  reserved  for  the master volumes (which are

automatically mounted at the conclusion of  a  self-test

command).

-----------------------------------------------------------------

Programming the DECMATE II for the RD5lD Disk Subsystem

Revision 1, 16-March-83					  Page 12

-----------------------------------------------------------------

GET VOLUME DATA 				  GET VOLUME DATA	_______________					  _______________

This command returns the contents of the directory entry for  the

volume  mounted  on  the device referenced in the most recent SET

BLOCK command.

Command code:  0030

Data words:  24 in

Word	  Bits  Contents		 ____	  ____  ________

1..8   <4:11>  Volume name

9..10   <4:11>  Read password

11..12   <4:11>  Write password

13..14   <4:11>  Starting physical block  number,  divided

by 16  (low-order 8 bits in first

word)

15..16   <4:11>  Number of blocks in volume, divided by 16

(low-order 8 bits in first word)

17	   <4>  "1" indicates read access permitted

<5>  "1" indicates write access permitted

<6>  "0" for unit 0; "1" for unit 1

<7>  "1" if a volume is mounted on this device

<9>  "1" if this is the "startup" volume

<10>  "1" if this volume has been modified by a

WRITE command

18..24   <4:11>  OS-dependent data (see Appendix A)

Error codes:  none

Notes:

1.  If bit 7 of word 17 is "0", no volume is mounted on  the

device and the rest of the data returned by this command

is invalid.

-----------------------------------------------------------------

Programming the DECMATE II for the RD5lD Disk Subsystem

Revision 1, 16-March-83					  Page 13

-----------------------------------------------------------------

UPDATE VOLUME DATA 			       UPDATE VOLUME DATA	__________________			       __________________

This command replaces the contents of the directory entry for the

volume  mounted  on the specified device (except for the physical

size and location of the volume).  It can be used, in conjunction

with  GET  VOLUME DATA, to rename a volume, change its passwords,

alter the "startup" and "modified" flags, and change its OS data.

Command code:  0006

Data words:  25 out

Word     Bits  Contents

1   <8:11>  Device upon which the volume is presently

mounted (0..7)

2..9   <4:11>  Volume name

10..11   <4:11>  Read password

12..13   <4:11>  Write password

14..17 		  (ignored)

18	   <4>  "1" indicates read access permitted

<5>  "1" indicates write access permitted

<6:7>    (ignored)

<9>  "1" if this is the "startup" volume

<10>  "1" if this volume has been modified by a

WRITE command

19..25   <4:11>  OS-dependent data (see Appendix A)

Error codes:

0022  Invalid device number

0024  Device has no volume mounted

DIO   Disk I/O errors

Notes:

1.  The arrangement of the 24 words to  be  written  to  the

volume's  directory  entry is the same as those returned

by GET VOLUME DATA.  Thus  these  two  commands  can  be

conveniently  used  to selectively alter a volume's data

by getting it with GET VOLUME DATA, making  the  desired

alterations,  and  then  rewriting it with UPDATE VOLUME

DATA (remembering to precede the  data  for  the  latter

with the device number).

2.  The new data is  both  written  to  the  permanent  disk

directory  and  stored in the controller's device table,

making any changes immediately effective.  For  example,

if  the  update  alters  the  read or write access bits,

these will govern subsequent attempts to use the  volume

until it is dismounted and remounted (these bits are not

written in the disk directory).

3.  This command destroys the contents of the block buffer.

-----------------------------------------------------------------

Programming the DECMATE II for the RD5lD Disk Subsystem

Revision 1, 16-March-83					  Page 14

-----------------------------------------------------------------

READ DISK DIRECTORIES 			    READ DISK DIRECTORIES	_____________________			    _____________________

This command reads the directories of all the disks available  to

the controller and returns the contents of each 24-byte directory

entry which represents a volume on a  disk.  Unused  entries  are

not  returned.  The  number of transfer requests will be 24 times

the number of volumes defined in the available directories.

Command code:  0033

Data words:  n * 24 in

Word 	  Bits  Contents		 ____	  ____  ________

1..8   <4:11>  Volume name

9..10   <4:11>  Read password

11..12   <4:11>  Write password

13..14   <4:11>  Starting physical block  number,  divided

by  16  (low-order  8  bits in first

word)

15..16   <4:11>  Number of blocks in volume, divided by 16

(low-order  8  bits  in  first word)

17	   <6>	"0" for unit 0; "1" for unit 1

<7>  "1" if an active directory entry  (should

never be "0")

<9>  "1" if this is the "startup"

<10>  "1" if this volume has been modified by a

WRITE command

18..24   <4:11>  OS-dependent data (see Appendix A)

Error codes:

0034  Invalid volume directory

DIO   Disk read errors

Notes:

1.  This command destroys the contents of the block buffer.

-----------------------------------------------------------------

Programming the DECMATE II for the RD5lD Disk Subsystem

Revision 1, 16-March-83					  Page 15

-----------------------------------------------------------------

DISMOUNT VOLUME 				  DISMOUNT VOLUME	_______________					  _______________

This command reverses the effect of a  MOUNT  VOLUME  command  by

breaking  the  association  between a device number and a volume.

After execution of this command,  subsequent  references  to  the

device number will be rejected.

Command code:  0005

Data words:  1 out

Word     Bits  Contents		 ____	  ____  ________

1   <8:11>  Device number from which volume is to  be

dismounted (0..15)

Error codes:

0022  Invalid device number

0026  Special mode required for devices 8..15

Notes:

1.  Issuing DISMOUNT VOLUME invalidates  the  stored  device

and  block  numbers  in the controller.  A new SET BLOCK

must precede the next READ or WRITE.

2.  No error occurs if no volume was mounted on the device.

3.  The master volumes on devices 8 and 9  should  never  be

dismounted.  (An  EXECUTE  SELF-TEST  will remount them,

but will dismount the volumes on all other devices.)

-----------------------------------------------------------------

Programming the DECMATE II for the RD5lD Disk Subsystem

Revision 1, 16-March-83					  Page 16

-----------------------------------------------------------------

SET BLOCK 						SET BLOCK	_________						_________

This command establishes  the  device  and  block  number  for  a

subsequent READ or WRITE operation.

Command code:  0001

Data words:  3 out

Word     Bits  Contents		 ____	  ____  ________

1   <8:11>  Device number (0..15)

2   <0:11>  Low-order 12 bits of block number

3   <4:11>  High-order 8 bits of block number

Error codes:

0002  Invalid block number (too high for volume)

0022  Invalid device number

0024  Device has no volume mounted

0026  Special mode required for devices 8..15

-----------------------------------------------------------------

Programming the DECMATE II for the RD5lD Disk Subsystem

Revision 1, 16-March-83					  Page 17

-----------------------------------------------------------------

FILL BUFFER 					      FILL BUFFER	___________					      ___________

This command transfers a block of  data  from  the  6120  to  the

controller's  block buffer.  It is used prior to a WRITE command.

If the command qualifier is "1" (8-bit mode) then there  will  be

512 transfer requests, each for a byte of data from AC<4:11>.  If

the command qualifier is "0" (12-bit mode) then there will be 256

transfers, each for a word of data from AC<0:11>.

Command code:  0002 (12-bit)

0102 (8-bit)

Data words:  256 out (12-bit)

512 out (8-bit)

Error codes:  none

-----------------------------------------------------------------

Programming the DECMATE II for the RD5lD Disk Subsystem

Revision 1, 16-March-83					  Page 18

-----------------------------------------------------------------

WRITE 							    WRITE	_____							    _____

This command writes the contents of the block buffer to the disk.

This  command  normally  should  be  preceeded  by  a FILL BUFFER

command and a SET BLOCK  command  or  (in  special  mode)  a  SET

PHYSICAL ADDRESS command.

Command code:  0003

Data words:  none

Notes:

1.  If the "modified" flag in the volume's directory data is

"0",  execution  of a WRITE sets that bit to "1" in both

the device table and the on-disk  directory.  (This  bit

can be cleared by using the UPDATE VOLUME DATA command.)

Error codes:

0025  Write access denied  (i.e. the volume was mounted with

a  "0"  in  AC<5> of word 1 of the command's data

words).

DIO   Disk write errors

-----------------------------------------------------------------

Programming the DECMATE II for the RD5lD Disk Subsystem

Revision 1, 16-March-83					  Page 19

-----------------------------------------------------------------

READ 							     READ	____							     ____

This command reads the selected block  from  the  disk  into  the

controller's  block buffer.  A subsequent EMPTY BUFFER command is

needed to bring the data back  to  the  6120.  Note  that  before

issuing  this  command, the 6120 program should issue a SET BLOCK

command or (in special mode) a SET PHYSICAL ADDRESS command.

Command code:  0004

Data words:  none

Error codes:

0025  Read access denied (i.e. the volume was  mounted  with

a  "0"  in  AC<4> of word 1 of the command's data

words).

DIO   Disk read errors

-----------------------------------------------------------------

Programming the DECMATE II for the RD5lD Disk Subsystem

Revision 1, 16-March-83					  Page 20

-----------------------------------------------------------------

EMPTY BUFFER 					     EMPTY BUFFER	____________					     ____________

This command transfers the contents  of  the  controller's  block

buffer to the 6120.  If the command qualifier is "1" (8-bit mode)

then there will be 512 data transfers, each for a  byte  of  data

into  AC<4:11> (AC<0:3> is cleared).  If the command qualifier is

"0" (12-bit mode) then there will be 256 transfers,  each  for  a

word of data into AC<0:11>.

Command code:  0025 (12-bit)

0125 (8-bit)

Data words:  256 in (12-bit)

512 in (8-bit)

Error codes:  none

Notes:

1.  This  command  does  not  alter  the  contents  of   the

controller's block buffer.

-----------------------------------------------------------------

Programming the DECMATE II for the RD5lD Disk Subsystem

Revision 1, 16-March-83					  Page 21

-----------------------------------------------------------------

GET ERROR 						GET ERROR	_________						_________

This command returns the error code   set  by  the  last  command

executed  (other  than GET ERROR).  This command may be issued at

any time after the completion of another command,  regardless  of

whether  or not the error flag was set.  A returned value of zero

indicates that there was no  error.  Execution  of  this  command

does  not  clear  the error register.  Table 3 lists the possible

error code values.

Command code:  0027

Data words:  1 in

Word     Bits  Contents

1   <0:11>  Error code from previous command

Error codes:

(any; see description above)

-----------------------------------------------------------------

Programming the DECMATE II for the RD5lD Disk Subsystem

Revision 1, 16-March-83					  Page 22

-----------------------------------------------------------------

TEST ERROR 					       TEST ERROR	__________					       __________

This command sets or clears  the  ERROR  flag  according  to  the

current  value  in the controller's error register.  If the value

is not zero, the ERROR flag is  set;  otherwise  it  is  cleared.

Execution of this command does not alter the error register.

Command code:  0021

Data words:  none

Error codes:

(any; see description above)

-----------------------------------------------------------------

Programming the DECMATE II for the RD5lD Disk Subsystem

Revision 1, 16-March-83					  Page 23

-----------------------------------------------------------------

GET STATUS 					       GET STATUS	__________					       __________

This command returns the current mechanical status of  the  last-

selected  unit  and  the  physical cylinder, head and sector last

addressed.

Command code:  0026

Data words:  5 in

Word	  Bits  Contents

1    <4:4>  Index pulse (transient)

<5:5>  "1" if unit is at physical cylinder zero

<6:6>  "1" if a write fault occurred on the last

disk operation

<7:7>  "1" if selected unit is ready

<8:8>       (undefined)

<9:9>  "1" if seek complete (from unit)

<10:10>  "1" if unit 1  is  selected  (acknowledge

signal from unit)

<11:11>  "1" if unit 0  is  selected  (acknowledge

signal from unit)

2   <0:11>  Cylinder number

3   <9:11>  Head number

4   <8:11>  Sector number

5   <0:11>  Controller program version number

Error codes:  none

-----------------------------------------------------------------

Programming the DECMATE II for the RD5lD Disk Subsystem

Revision 1, 16-March-83					  Page 24

-----------------------------------------------------------------

EXECUTE SELF-TEST 				EXECUTE SELF-TEST	_________________				_________________

This command causes the controller to execute its self-test  code

and  reset  its  internal data.  It dismounts all mounted volumes

and sets the controller to "normal" command mode.  It  reads  the

disk  control  blocks  (block  1)  from  each  available  disk to

establish their geometries and store their  bad-block  maps,  and

mounts the master volumes on devices 8 and 9.

Command code:  0011

Data words:  none

Error codes:

0017  No units are attached to the controller

0027  RAM failure during self-test

0033  DAA failed its self-test

0035  Invalid device control block

DIO   Disk read error (on device control block)

Notes:

1.  Since self-test  dismounts  all  volumes,  no  reads  or

writes  can  be performed (except to the master volumes)

until one  of  the  MOUNT  commands  has  been  executed

successfully after this command.

2.  This command is executed automatically upon  a  power-on

reset.

-----------------------------------------------------------------

Programming the DECMATE II for the RD5lD Disk Subsystem

Revision 1, 16-March-83					  Page 25

-----------------------------------------------------------------

SET RETRY-COUNT 				  SET RETRY-COUNT	_______________					  _______________

The disk controller will retry read and write errors by restoring

seeking  to cylinder 0) and re-seeking and reading/writing again.

It will do  this  <retry-count>  times.  The  default  value  for

<retry-count>  is  1.  This   command   alters   the   value   of

<retry-count>; the  new value remains effective until another SET

RETRY-COUNT or an EXECUTE SELF-TEST command is executed.

Command code:  0013

Data words:  1 out

Word     Bits  Contents		 ____	  ____	________

1   <4.11>  Number of retries to attempt

Error codes:  none

-----------------------------------------------------------------

Programming the DECMATE II for the RD5lD Disk Subsystem

Revision 1, 16-March-83					  Page 26

-----------------------------------------------------------------

SET NORMAL MODE 				 SET NORMAL MODE	_______________					 _______________

This command  places  the  controller  in  "normal"  mode,  which

disallows  execution  of  all  of  the  "special  mode" commands.

Special mode can only be reestablished by the  SET  SPECIAL  MODE

command.

Command code:  0020

Data words:  none

Error codes:  none

-----------------------------------------------------------------

Programming the DECMATE II for the RD5lD Disk Subsystem

Revision 1, 16-March-83					  Page 27

-----------------------------------------------------------------

SET SPECIAL MODE 				 SET SPECIAL MODE	________________				 ________________

This command enables the execution  of  "special"  disk  commands

listed in Table 2 and the use of devices 8 through 15.

Command code:  0007

Data words:  none

Error codes:  none

-----------------------------------------------------------------

Programming the DECMATE II for the RD5lD Disk Subsystem

Revision 1, 16-March-83					  Page 28

-----------------------------------------------------------------

SET FORMAT SEQUENCE			      SET FORMAT SEQUENCE	___________________			      ___________________

This command specifies the order in which the sixteen sectors  on

a  track  are  numbered  when the track is formatted, and must be

issued  before  issuing  a  FORMAT  command.  Sixteen  words  are

transmitted to the controller, each containing a sector number in

bits  8:11.  Each  number from 0 through 15 must be sent once and

only once.  The sequence can be set up to effect an  interleaving

scheme  to  match  the  speed  of  the  disk  to the speed of the

programs which use it.

A physically defective sector is permanently marked as such by  a

value of 255 (decimal) in the word in the pattern list containing

its sector number.  The controller will record this in the sector

header  on  the  disk  track.  When  a  block  so marked is later

accessed, the controller  will  note  the  flag  and  search  the

bad-block map for the identity of a substitute block.

Command code:  0015

Data words:  16 out

Word     Bits  Contents		 ____	  ____	________

1..16   <4:11>  Sector number 0..15, or  255  for  a  bad

sector

Error codes:  none

-----------------------------------------------------------------

Programming the DECMATE II for the RD5lD Disk Subsystem

Revision 1, 16-March-83					  Page 29

-----------------------------------------------------------------

SET PHYSICAL ADDRESS 			     SET PHYSICAL ADDRESS	____________________			     ____________________

This command selects the unit, cylinder, head and  sector  to  be

used for subsequent READ, WRITE, RESTORE, or FORMAT commands.

Command code:  0014

Data words:  4 out

Word	   Bits  Contents		  ____	   ____	 ________

1  <11:11>  "0" = select unit 0

"1" = select unit 1

2	 <0:11>	 Cylinder number, 0 4095 (subject to  the

physical limits of the unit!)

3   <9:11>  Head number, 0..7

4   <8:11>  Sector number, 0..15

Error codes:

0026  Special mode required

Notes:

1.  This command merely stores the cylinder  value; it  does

not perform a physical disk seek.

2.  The cylinder number must be in the  range  zero  through

one  less  than  the number of physical cylinders in the

unit.  This is not checked in this command, but will  be

checked  when  an  actual disk operation attempts to use

the cylinder value.

3.  The head number must be in the range  zero  through  one

less  than the number of heads in the unit.  This is not

checked in this command, but will  be  checked  when  an

actual disk operation attempts to use the head value.

-----------------------------------------------------------------

Programming the DECMATE II for the RD5lD Disk Subsystem

Revision 1, 16-March-83					  Page 30

-----------------------------------------------------------------

RESTORE 						  RESTORE	_______							  _______

This command moves the heads of the previously selected unit (see

SET  PHYSICAL ADDRESS command) to cylinder 0 (the outside edge of

the disk).  The command will complete within 1.2 seconds.

Command code:  0016

Data words:  none

Error codes:

0012  Unable to find cylinder 0 (disk hardware error)

0020  Unit not installed

0026  Special mode required

-----------------------------------------------------------------

Programming the DECMATE II for the RD5lD Disk Subsystem

Revision 1, 16-March-83					  Page 31

-----------------------------------------------------------------

FORMAT 							   FORMAT	______							   ______

This command  writes  sector  format  information  on  the  track

designated  by  the  preceding SET PHYSICAL ADDRESS command.  The

sectors are numbered in the sequence specified by  the  preceding

SET FORMAT SEQUENCE command.

Command code:  0017

Data words:  none

Error codes:

0026  Special mode required

0030  Bad format command

0031  No index found

DIO   Disk I/O errors

Notes:

1.  Formatting destroys the contents  of  the  track.  After

formatting,  each  sector  contains  512 bytes of binary

zeros.

2.  A SET FORMAT SEQUENCE command must be issued before  the

first use of the FORMAT command, but that pattern is not

altered by  FORMAT  and  remains  valid  for  subsequent

FORMAT commands.

-----------------------------------------------------------------

Programming the DECMATE II for the RD5lD Disk Subsystem

Revision 1, 16-March-83					  Page 32

-----------------------------------------------------------------

Appendix A.  OPERATING SYSTEM CONSIDERATIONS	____________________________________________

The last seven bytes (offset 17  through  23)  of  each  volume's

24-byte  directory  entry  contain  data  specific  to  the  file

structure of the  volume.  The  first  byte  indicates  the  file

structure of the volume, as follows:

Bytes    Bits      Contents	     _____  _________	________

17  X... ....	Bootable-system  flag  A  "1"   marks   a

volume  whose  block  0  contains  an

executable bootstrap loader.

17  .XXX XXXX 	File structure (values in octal):

010  WPS-8

011  OS-8

012  COS 310

100  CP/M

Bytes 18..23 for the various file structures are TBS.

Operating system bootstrapping	______________________________

A logical volume may contain an operating  system  which  can  be

bootstrapped  into  an  empty  machine.  Such  a volume is marked

"bootable" (as shown above) and has a 256-word  bootstrap  loader

in  its  block  0.  By convention, such a system is loaded by the

following command sequence:

-- Execute self-test to dismount all volumes.

-- Mount the system volume on device 0.

-- Read block 0 from the volume

-- Empty the buffer in 12-bit mode (256 words) into 6120

storage starting at address 0200 in field 0.

-- Begin execution at address 0210.

A special case is the "startup" volume.  This is marked  so  that

it  can  be mounted by a MOUNT VOLUME command without knowing its

name.  The DECmate II firmware mounts and bootstraps this  volume

after  the firmware has been loaded during system initialization.

Operating systems which have completed operation  and  have  been

directed to "quit" should mount and bootstrap the startup volume.

<<< End of document >>>

-----------------------------------------------------------------

Programming the DECMATE II for the RD5lD Disk Subsystem

Revision 1, 16-March-83					  Page A1

-----------------------------------------------------------------

Changes to RD5lD Programming Specifications 		  Page B1

_________________________________________________________________

- - - - - - -

d i g i t a l				MEMORANDUM

- - - - - - -

From: Bob Morse

To: J. Elkins				Dept: Micro Systems Eng.

D. Rice					Loc : MLO3-2/E41

W. Keefe				DTN : 223-6720

cc: Interested parties			Date: 16-Mar-83 (rev 2)

Subject: Changes to RD51D Programming Specifications

_________________________________________________________________

This memo summarizes changes to  the  programming  specifications

for  the  RD51D  hard  disk controller for the DECmate II.  These

alter  the  controller  commands  described  in  Programming  the							 ________________

DECmate  II  for  the  RD51D  Disk Subsystem, revision 0, dated 7	____________________________________________

January  1983,  and  are  incorporated  in  revision  1  of  that

document.  The revised specifications are reflected in version 13

("D") and later of the 8751 controller ROM.

1.	The number of logical devices available to a  normal-mode

user  has  been  reduced from 16 to 8.  Only devices 0..7

should be used for normal program  and  operating  system

functions.  Devices  8..15  may be used only when special

mode is in effect.

2.	Devices 8 and 9 are automatically initialized  to  access

the "master volume" (which maps the entire physical unit)

for units 0 and 1, respectively.  These  devices  can  be

used  only in master mode.  Their volumes should never be

dismounted!

3.	The MOUNT MASTER VOLUME  command  (code  0006)  has  been

deleted.

4.	The MOUNT STARTUP VOLUME command  (code  0010)  has  been

deleted.  Its  function  is  now  performed  by the MOUNT

VOLUME command (see next item).

5	The MOUNT VOLUME command no longer checks  the  read  and

write  passwords;  this  is now the responsibility of the

operating system (error  code  0037  has  been  deleted).

This  reduces the number of data words from 13 to 9:  one

option byte followed by 8 bytes of volume name.

This command no longer checks for duplicate mounting of a

volume (error code 0036 has been deleted).  Such mounting

poses no problems to the controller, but programs  should

be  wary  of  mounting  the  same volume on more than one

device if they then believe the two  devices  to  contain

independent data!

Changes to RD5lD Programming Specifications 		  Page B2

_________________________________________________________________

Bit 4 (or AC<7>) of the flag byte is  "0"  for  a  normal

mount  by  name,  or "1" to mount the startup volume.  In

the latter case, the name supplied with  the  command  is

ignored.

6.	The GET  VOLUME  PARAMETERS  and  READ  DISK  DIRECTORIES

commands  no longer suppress the contents of the password

fields.  This data is now returned to the caller, who  is

now responsible for password checking.

7.	The GET CONTROLLER VERSION command (code 0031)  has  been

eliminated.  Instead,  the  version number is returned as

the fifth word of data returned by a GET  STATUS  command

(code 0026).

8.	The READ STARTUP SECTOR  command  (code  0032)  has  been

eliminated.  Instead,  read  block 0 of the master volume

(which is pre-mounted on device 8) and do a normal 12-bit

empty-buffer.

9.	The SET DRIVE, SET CYLINDER,  SET  HEAD  and  SET  SECTOR

commands  (codes  0040,  0041,  0042  and 0043) have been

replaced by a single command named SET  PHYSICAL  ADDRESS

(code   0014)   which   sends  four  data  words  to  the

controller.  These four words are identical to  the  four

sent   separately  by  the  four  former  commands.  This

command is for use only by  the  disk-formatting  utility				   ____

and diagnostic exerciser programs.

10.	Command  codes  for  the  following  commands  have  been

changed (to reduce the size of the command dispatch table

in the controller):

Command		    Old code   New code		    _______		    ________   ________

SET NORMAL MODE	      0047 	 0020

SET FORMAT SEQUENCE	      0050 	 0015

RESTORE		      0045 	 0016

FORMAT		      0046	 0017

11.	The information returned by the GET  STATUS  command  has

been  revised:  Bit  3  of  the  first byte (or AC<8>) no

longer reflects the "error"  status;  its  value  is  now

unspecified.  A  fifth  word  has  been  added; it is the

controller program version number.

12.  The SET  SPECIAL  MODE  COMMAND  (code  0007)  no  longer

requires a password and hence requires no data words.

13.	Electrical interface problems limit the  present  version

of  the  firmware  to  one  unit  (unit 0); a second unit

cannot  be  attached.  However,  the  controller  command

structure  remains in place to handle two units in future

versions.

Changes to RD51D Programming Specifications 		  Page B3

_________________________________________________________________

14.	A new command named UPDATE VOLUME DATA (code 0006) allows

any  of  the  data in a volume's directory entry, such as

name, OS type, and "modified" flag, to be revised without

disturbing  the  contents  of  the  volume.  The volume's

physical position and size are protected from  alteration

by this command.

15.	A new command named TEST ERROR (code 0021)  requests  the

controller   to   set  its  ERROR  flag  (again)  if  the

controller's error register  contains  a  non-zero  error

code.

16.	The following error codes are no longer (or  never  were)

generated by the controller:

0003  Self test failed in unit

0004  Self test failed in controller

0013  Data mark (in format) not found

0036  Volume already mounted on another device

0037  Incorrect read or write password

Error code 0027 is  newly  defined.  Its  description  is

"RAM failure in self test".

In order  to  reduce  confusion  with  various  operating  system

terminologies,  the  word "unit" has been substituted for "drive"

to refer to the physical hard disk mechanism which  is  connected

to one of the two electrical interfaces of the controller.

For the convenience of various system maintenance  programs,  the

64-block  area  at  the  beginning  of a unit (which contains its

device control block, volume directory, and system firmware) will

be  described  as  a  volume named FIRMWARE with a file structure

code of 0.  This will be the first entry in the volume directory,

and  is created by the FORMAT program which initializes the unit.

The controller does not depend upon the presence of  this  entry,

but  utility  programs may use it instead of the master volume to

reduce the chances of inadvertantly  damaging  data  outside  the

first 64 blocks.

The bootstrap block which is written in block  0  of  a  bootable

volume  now  has a standard execution entry point of 0210 instead

of 0200.  This leaves the first 16 bytes of  the  block  free  to

contain the DEC standard boot block identification sequence.

rfm