RTS8V1.DOC

DISCLAIMER

This document file was created by scanning  the

original  document and then editing the scanned

text.  As much as possible, the  original  text

format  was restored.  Some format changes were

made to insure this  document  would  print  on

current laser printers using 60 lines per page.

The  original  spelling  and  grammar  has been

preserved.

1-Apr-1996

DEC-08-ORTMA-A-D

RTS-8 USER'S MANUAL

Version 1

|-------------------------------------------------------|

|  Order additional copies as directed on the Software  |

|  Information page at the back of this document.       |

|-------------------------------------------------------|

digital equipment corporation - maynard massachusetts

First Printing June 1974

The information in this document is subject to change without notice

and should not be construed as a commitment by Digital Equipment

Corporation.  Digital Equipment Corporation assumes no responsibility

for any errors that may appear in this manual.

The software described in this document is furnished to the purchaser

under a license for use on a single computer system and can be copied

(with inclusion of DIGITAL's copyright notice) only for use in such

system, except as may otherwise be provided in writing by DIGITAL.

Digital Equipment Corporation assumes no responsibility for the use

or reliability of its software on equipment that is not supplied by

DIGITAL.

Copyright @ 1974 by Digital Equipment Corporation, Maynard, Mass.

The HOW TO OBTAIN SOFTWARE INFORMATION page, located at the back of

this document, explains the various services available to DIGITAL

software users.

The postage prepaid READER'S COMMENTS form on the last page of this

document requests the user's critical evaluation to assist us in

preparing future documentation.

The following are trademarks of Digital Equipment Corporation:

CDP		DIGITAL	  	 INDAC		  PS/8

COMPUTER LAB	DNC		 KA10		  QUICKPOINT

COMSYST		EDGRIN		 LAB-8		  RAD-8

COMTEX		EDUSYSTEM	 LAB-8/e	  RSTS

DDT		FLIP CHIP	 LAB-K		  RSX

DEC		FOCAL		 OMNIBUS	  RTM

DECCOMM		GLC-8		 OS/8		  RT-ll

DECTAPE		IDAC		 PDP		  SABR

DIBOL		IDACS		 PHA		  TYPESET 8

UNIBUS

CONTENTS

Page

PREFACE								 v

CHAPTER 1	       INTRODUCTION				       1-1

CHAPTER 2	       USING TASKS UNDER RTS-8			       2-1

2.1       THE STRUCTURE OF RTS-8 TASKS		       2-1

2.2       CONCEPTS OF TASK COMMUNICATION		       2-2

2.2.1     Task Synchronization and Event Flags	       2-2

2.2.2     Intertask Messages			       2-3

CHAPTER 3	       RTS-8 EXECUTIVE REQUESTS			       3-1

3.1       COMMUNICATION WITH THE RTS-8 EXECUTIVE	       3-1

3.2       ERs USED TO SEND AND RECEIVE MESSAGES	       3-1

3.2.1     SEND					       3-1

3.2.2     WAITE					       3-2

3.2.3     SENDW					       3-2

3.2.4     RECEIVE					       3-2

3.2.5     POST					       3-3

3.3       INTER-TASK CONTROL THROUGH TASK FLAGS	       3-5

TABLE ENTRY

3.4       ERs USED TO SET AND CLEAR WAIT BITS	       3-6

3.4.1     BLKARG					       3-6

3.4.2     UNBARG					       3-6

3.4.3     SUSPND					       3-6

3.4.4     RUN					       3-7

3.5       DERAILING THE EXECUTION OF A TASK	       3-8

3.5.1     DERAIL					       3-8

3.6       USING INTERRUPTS IN RTS-8		       3-9

3.6.1     SKPINS					       3-9

3.7       ADVANCED RTS-8 PROGRAMMING TECHNIQUES	       3-11

3.7.1     WAITM					       3-11

3.7.2     Direct References to System Tables	       3-12

CHAPTER 4	       RTS-8 SYSTEM TASKS			       4-1

4.1       CLOCK HANDLER				       4-1

4.1.1     Examples of Clock Handler Calls		       4-2

4.2       TERMINAL HANDLER				       4-3

4.2.1     Examples of Terminal Handler Messages	       4-5

4.3       LINE PRINTER HANDLER			       4-5

4.4       MASS STORAGE HANDLERS			       4-5

4.4.1     Example of Mass Storage Handler Call	       4-7

iii

Page

4.5       POWER FAIL TASK				       4-7

4.6       OS/8 SUPPORT TASK			       4-8

4.7       OS/8 FILE SUPPORT TASK			       4-9

4.8       UDC HANDLER				       4-10

4.8.1     Analog Output				       4-11

4.8.2     Analog Input				       4-11

4.8.3     Digital Output				       4-12

4.8.4     Digital Input				       4-12

4.8.5     Generic Code				       4-12

4.8.6     Enable Counter				       4-13

4.8.7     Read Counter				       4-14

4.8.8     Disable Counter				       4-14

4.8.9     Enable Contacts				       4-14

4.8.10    Change of State				       4-15

4.8.11    Disable Contacts				       4-15

4.8.12    UDC Assembly Parameters			       4-16

4.8.13    UDC Error Conditions			       4-16

4.9       CASSETTE HANDLER				       4-17

4.10      CASSETTE LABEL SUPPORT HANDLER		       4-18

CHAPTER 5	       MONITOR CONSOLE ROUTINE			       5-1

5.1       MCR COMMAND ARGUMENTS			       5-1

5.2       MCR COMMANDS				       5-2

5 2.1     Date					       5-2

5.2.2     Time					       5-2

5.2.3     Name					       5-2

5.2.4     Request					       5-3

5.2.5     Stop					       5-4

5.2.6     Disable					       5-4

5.2.7     Enable					       5-4

5.2.8     Cancel					       5-4

5.2.9     Systat					       5-4

5.2.10    Open Address				       5-5

5.2.11    Examine Address				       5-6

5.2.12    Deposit Address				       5-6

5.2.13    Post Address				       5-6

5.2.14    Exit					       5-6

5.3       MCR ERRORS				       5-6

CHAPTER 6	       ASSEMBLING AND LOADING TASKS FOR RTS-8	       6-1

CHAPTER 7	       DEMONSTRATION PROGRAM			       7-1

APPENDIX A	       RTS-8 DISTRIBUTED SOURCE FILES		       A-1

APPENDIX B        RTS-8 COMPONENT SIZES			       B-1

GLOSSARY						        GLOSSARY-1

INDEX						           INDEX-1

iv

PREFACE

This manual describes the PDP-8 Real-Time System (RTS-8) and  provides

information  and  examples necessary for its efficient use.  Knowledge

of PDP-8 machine language programming is essential;  in addition,  the

user should be familiar with real-time systems in general and with the

use and operation of the OS/8 system.

The  following  PDP-8  handbooks  will  be  helpful  for  review   and

reference:   INTRODUCTION TO PROGRAMMING, THE SMALL COMPUTER HANDBOOK,

the OS/8  HANDBOOK  (DEC-S8-OSHBA-A-D),  the  UDC8  UNIVERSAL  DIGITAL

CONTROL  SUBSYSTEM  MAINTENANCE MANUAL (DEC-08-HZDC-D), and the CAPS-8

USER'S MANUAL (DEC-8E-OCASA-A-D).

v

CHAPTER 1

INTRODUCTION

A real-time system is a software framework under which  the  resources

at hand (core, CPU time, peripheral devices) are divided among several

tasks.  The term "task" is difficult to define, as  it  overlaps  with

the definitions of "program" and "subroutine", but basically a task is

a  portion  of  machine  code  which  performs  a  specific  function;

sometimes  this  function is an end in itself, sometimes it is related

only to the needs of other tasks.  The  real-time  system  distributes

resources to tasks at any time depending on whether the tasks can make

use of them, and whether a  more  privileged  (higher  priority)  task

wants to pre-empt resources already in use.

Most real-time systems consist of a group of programs  or  tasks  that

run  at  varying  times  or  frequencies  and  alternate between being

compute  bound  and  I/O  bound.   To  efficiently  use  the   central

processor,  these  tasks  cannot  be  run  in series since the central

processor will be poorly utilized during the periods  that  tasks  are

I/O bound.   In  addition,  real-time  tasks are time-dependent in one

sense or another and in general cannot wait for a slow, less important

I/O  or  compute  bound  task  to  finish  before  starting execution.

Multiprogramming and some sort of priority scheme for  scheduling  the

central processor are thus required.

Multiprogramming allows many tasks to be in some  state  of  execution

simultaneously.   When  one  task cannot use all the available central

processor time because it must wait for an I/O operation or is blocked

by  some  other  condition,  the  central processor can be switched to

another task to make use of the available time.  It is  possible  even

in  a  multiprogramming environment that there may not be a task in an

executable  state  to  utilize  this  available  time.    While   this

condition  is  certainly  possible, it is less likely to occur than in

a uni-programming environment.

A priority scheme is needed to distinguish the relative importance  of

the various tasks in the system.  It must be possible to interrupt the

execution of a less important task to  execute  a  critical  real-time

program.   A  fixed  priority  scheme has been chosen.  Fixed priority

meets the above requirement, is simple and adequate, and requires  low

scheduling overhead.

RTS-8 is a small real-time system which uses any family-of-8 processor

(with  the exception of the PDP-8/S).  It allows up to 63 tasks to run

concurrently, competing  for  resources  on  a  fixed-priority  basis.

Included  in  the  RTS-8  system  are  system tasks which control most

standard Digital Equipment Corporation I/O devices (listed in  Chapter

4),  a  task  which allows interactive system control from the console

terminal, and a task which allows a single copy of  the  OS/8  monitor

system to run in the background.

1-1

CHAPTER 2

USING TASKS UNDER RTS-8

2.1  THE STRUCTURE OF RTS-8 TASKS

The controlling program in an RTS-8 System  is  the  RTS-8  Executive.

The  Executive  decides  which  task  should  be running (based on the

priorities of the tasks which are  able  to  run)  and  also  provides

services  to the tasks by means of Executive Requests, discussed later

in Chapter 3.

Each task in an RTS-8 system has  an  associated  unique  Task  Number

between 1 and 63, which serves the following purposes:

1.  The Task Number is used by the RTS-8 Executive as an index to

various system tables where information about tasks is kept.

2.  The Task Number is used by other  tasks  in  the  system  for

reference in Executive Requests.

3.  The Task Number determines the task's  priority;   the  lower

the Task Number, the higher the priority of the task.

The Executive uses three internal tables to maintain information about

the  tasks  in the system. Each task's Task Number is used as an index

into these tables to retrieve and update information  for  that  task.

The internal tables are:

1.  The Task State Table - a table of 4-word entries holding  the

most  recent  contents  of  CPU  registers  for  each task as

follows:

Word 1 - contains the link (bit 0)

the data field (bits 9-11)

the instruction field (bits 6-8)

Word 2 - contains the program counter (PC)

Word 3 - contains the accumulator (AC)

Word 4 - contains the multiplier quotient (MQ)

Whenever a task is executed, the contents of its  Task  State

Table  are  loaded  into the corresponding CPU registers, and

whenever a task stops executing, its Task State  Table  entry

is set to the new contents of these registers.

2.  The Task Flags Table - a  table  of  l-word  entries  holding

various  bits  for  each task to determine whether or not the

2-1

Using Tasks Under RTS-8

task is runnable (a detailed description  of  these  bits  is

provided in Section 3.3). A task is runnable only if its Task

Flags Table entry contains zero.

3.  The Task Input Message Queue Header  Table   -   a  table  of

2-word  entries  which  represent  the  "head"  of  the input

message queue for each task:

Word 1 - if zero, there are no messages  in  the  queue;

if  non-zero, the word is a CDF to the field of

the first message in the queue.

Word 2 - if word 1 was not zero, this word is a  pointer

to the first message in the queue.

Messages are discussed later in this chapter.

2.2 CONCEPTS OF TASK COMMUNICATION

There are two main  types  of  task  communication  in  RTS-8--message

sending  and  synchronization.   Since  synchronization  is  the  more

fundamental of the two, it will be discussed first.

2.2.1 Task Synchronization and Event Flags

Whenever two processes occur independently, there may be  a  need  for

one  to  wait  while  the  other  finishes execution.  The most common

example of this is the PDP-8 terminal interface.  The  terminal  waits

for  the  PDP-8  to produce a printable character.  When the PDP-8 has

one, it alerts the terminal by issuing  a  TLS  instruction.   If  the

PDP-8  then  has  another  character to print, it must first wait in a

TSF;JMP .-1 loop until the  terminal  signals  that  it  has  finished

printing  the  character  by  raising  the ready flag.  Then the PDP-8

exits from its loop and proceeds to print the next character.

Similarly, tasks in RTS-8 may synchronize with each other by means  of

Event Flags.  An Event Flag is a location which contains the status of

an Event.  An Event can be a  physical  occurrence  such  as  a  clock

ticking,  a valve closing, or a device ready flag raising, or an Event

can be conceptual, as for example, a result, an  inconsistency,  or  a

certain  string  of  characters  typed  by  the  operator.   Like  the

terminal ready flag, an Event Flag  has  the  states  PENDING(>0)  and

FINISHED(=0).   Thus a task can (via a message) direct another task to

perform some action, at which time it  will  set  a  (mutually  agreed

upon)  Event  Flag  to  the  PENDING  value.   When  the  second  task

finishes it sets the Event  Flag  to  the  FINISHED  value  (known  as

Posting  the  Event Flag).  If the first task has been waiting for the

Event Flag with the instructions:

TAD Event Flag; SZA CLA; JMP .-2

then the Posting of the Event Flag will cause the first task  to  exit

from its loop.

2-2

Using Tasks Under RTS-8

Unfortunately,  the  above  loop   ties   up   a   vitally   important

resource--the  PDP-8  processor.   Therefore, under RTS-8, Event Flags

have an additional state: WAITING(<0).  Thus in the example above, the

first  task  can  tell  the RTS-8 Executive it wants to WAIT until the

Event Flag is FINISHED.  The monitor saves the  task's  PC  and  marks

in  its Task Flags Table word that it is Waiting on an Event Flag; the

Event Flag is set to the proper  WAITING  state  (in  this  case,  the

proper  WAITING  state  is  octal  4000  plus  the waiting task's Task

Number).  When the Event Flag is POSTed (via an RTS-8 Executive  call)

the  task  which  is  WAITING  for  it  is  automatically  started  by

RTS-8. The mnemonic for the RTS-8 Executive Request which Waits for an

Event Flag is WAITE.

2.2.2 Intertask Messages

Just as Event Flags under  RTS-8  are  analogous  to  hardware  device

flags,   messages   are   analogous   to   data-sending  hardware  I/O

instructions (for example,  TLS).  That  is,  messages  start  a  task

(providing  the  task  is  WAITING for a message) and at the same time

pass information to the task.

A message under RTS-8 is a contiguous area  of  core  storage.   RTS-8

requires the first three words of the message (the Message Header) for

its use.  The first word of the Message Header is the Event  Flag  for

the  message.   When  the  message is sent, the Event Flag is set in a

PENDING state by the RTS-8 Executive; this signifies that the  message

has  been  sent  but  not yet completed. No action occurs to the Event

Flag upon receipt of the message by the receiving task; however  RTS-8

protocol requires that when the receiver has performed whatever action

is specified (or implied) by the message, it  POSTS  the  Event  Flag.

This  posting  serves two purposes: it informs the task which sent the

message (the "sender") that the requested action has  been  completed,

and it "frees" the message so that it can be sent again (see 2 below).

The second and third words  of  the  Message  Header  are  used  as  a

CDF-address  pair to link the message to the next queue entry.  If the

receiving task is not actively waiting for a message, the  message  is

placed on the receiving task's Input Message Queue.  Messages are then

queued in order of decreasing priority of the sender (increasing  task

number) and in the order issued if a task sends more than one message.

The rest of the message may contain any desired information.   Sending

a  message  does  not  physically  move the message information to the

receiving task, but provides the receiver with the field  and  address

of the first data word of the message.

Note again that the information in a message is not  copied  into  the

receiver's area; this has several implications:

1.  Data in a message should not be modified by the sender  while

the Event Flag for the message is PENDING.

2.  The same message cannot be sent  a  second  time  before  its

Event  Flag  is FINISHed the first time.  RTS-8 enforces this

by checking the Message Event Flag on a  SEND  operation  and

2-3

Using Tasks Under RTS-8

putting the sender into Event Flag Wait  if  the  message  is

still PENDING.

3.  It is legal for the receiving task to  store  information  in

the  body  of  the  message.   In this way an "answer" to the

message can be returned without the complications of  sending

a  return  message  back  to the sender.  For example, when a

task sends a message to the disk driver task requesting  I/O,

the  error  status  of the completed operation is placed in a

specific word in  the  message  by  the  driver  to  indicate

whether an error occurred.

2-4

CHAPTER 3

RTS-8 EXECUTIVE REQUESTS

3.1  COMMUNICATION WITH THE RTS-8 EXECUTIVE

RTS-8  tasks  communicate  with  the  RTS-8  Executive  via  Executive

Requests  (abbreviated  as  ERs).  In  order to make ERs simple across

field boundaries, RTS-8 uses locations  20-27  in  every  field  as  a

communication  region  for  ERs.  The  Executive  can be called in any

field via a JMS 20 instruction (hereafter referred to as a  CAL).  The

Data Field (DF) does not have to be any specific value when the CAL is

given, as the code in location 20 sets the DF to the Instruction Field

(IF),  sets  the  IF  to  0  and jumps to the RTS-8 Executive.  (Other

instruction definitions used throughout the manual  and  described  in

Chapter  3 are POSTDS which is defined as JMP I 24, and WAITM, defined

as JMS I 25.)

Because the architecture  of  the  PDP-8  does  not  permit  reentrant

programs,    the    RTS-8   Executive   runs   with   task   switching

inhibited, i.e., interrupts are on, but higher priority tasks may  not

request  the  CPU  until the Executive has finished its request.  When

the Executive finishes processing the request, it checks  whether  any

higher-priority  tasks  have  become runnable while task switching was

inhibited, and, if so, it runs the highest-priority task of these.

Also, because of the non-reentrant  architecture  of  the  PDP-8,  the

RTS-8  Executive  will  not  honor any request to switch tasks arising

from an interrupt if the interrupted task's Program Counter  (PC)  was

less  than  100.  This  is done to protect the RTS-8 Executive's entry

point (location 20 in each field) from  being  destroyed.  User  tasks

should  be  careful  NEVER  to execute any instructions below 100 in a

field.

3.2  ERs USED TO SEND AND RECEIVE MESSAGES

There are five ERs  associated  with  the  Messages  and  Event  Flags

described in Chapter 2. Descriptions of these follow and an example of

their usage is presented in Section 3.2.5.

3.2.1  SEND  Send Message

Format:   CAL

SEND

TSKNUM

MESSAG

3-1

RTS-8 Executive Requests

SEND sends the message located at MESSAGE in the current field to  the

task  whose  number  is  TSKNUM.  If the receiving task is of a higher

priority than the sender and is waiting for a message, the  sender  is

temporarily  suspended and the receiver runs.  In any case, the sender

is not put into any WAIT state once the message is sent.  However,  if

the  event  flag in location MESSAG is PENDING (non-zero), meaning the

message is still busy from a previous SEND, the  sender  will  be  put

into  Event Flag Wait on location MESSAG, and only when the Event Flag

becomes FINISHED (zero) will this SEND be performed.

3.2.2  WAITE  Wait on Event Flag

Format:   CAL

WAITE

EFLG

WAITE checks the status of  location  EFLG  and  if  it  is  FINISHED,

returns  control  to  the  caller.  If  EFLG  is PENDING, its state is

changed to WAITING and the calling task is put into Event  Flag  WAIT.

When location EFLG is POSTed by another task or interrupt routine, the

calling task becomes runnable again.

3.2.3  SENDW  Send and Wait

Format:   CAL

SENDW

TSKNUM

MESSAG

SENDW is exactly equivalent to the sequence:

CAL

SEND	 /SEND THE MESSAGE

TSKNUM

MESSAG

CAL

WAITE	 /WAIT FOR RECEIVER TO ACKNOWLEDGE

MESSAG

3.2.4  RECEIVE  Receive Message

Format:	     TAD RESTRICTION     /ONLY TO RESTRICT

CAL		 /SENDING TASKS

RECEIVE

MADDR, 0			 /MESSAGE ADDRESS STORED

/HERE; CDF TO MESSAGE

/FIELD IN AC ON RETURN

If the AC is zero when the RECEIVE ER is issued,  the  calling  task's

Input Message Queue is examined.  If there are messages in the calling

task's Input Message Queue, the first (i.e., highest-priority) message

is dequeued and the address of its first data word is placed in MADDR.

3-2

RTS-8 Executive Requests

A CDF to the field of the message is stored in the AC.  If  there  are

no  messages,  the  task  is placed in Message Wait until such time as

there are any.  There is no ER which will allow a task  to  check  for

messages and not be placed in Message Wait if there are none;  however

a task may examine its Input Message Queue Header in field 0 first  to

determine the state of the Input Message Queue.

If the AC is non-zero when the  RECEIVE  ER  is  issued,  the  calling

task's  Input  Message  Queue is searched for a message whose sender's

Task Number matches the contents of the AC.   If  such  a  message  is

found,  it  is  removed from the queue as specified above;  if none is

found the issuing task is placed in Message Wait.

3.2.5  POST  Post Event Flag

Format:	       TAD EFPTR   /POINTER TO EVENT FLAG

CAL

POST

EFCDF, CDF EFFLD   /FIELD OF EVENT FLAG

The Event Flag pointed to by the AC at the CAL, in the field specified

by  EFCDF, is set to the FINISHED (zero) state.  If its previous state

was WAITING, the task which was waiting for it is cleared of its Event

Flag  Wait.   This ER never sets the calling task in a WAIT state, but

if the task Waiting for the Event Flag is of a  higher  priority  than

the  calling task, the calling task is temporarily suspended while the

other is run.

This example illustrates the RTS-8 ERs dealing with Messages and Event

Flags.   Since  I/O and interrupts under RTS-8 have not been discussed

yet, this example is trivial;  i.e., there is no advantage to  keeping

the  functions  of the two tasks separate, and the entire send/receive

structure is being used here  as  an  elaborate  subroutine  call.   A

description follows the example.

Task A				     ______

A1	      ALOOP,  CAL

SEND		 /SEND TASK B A MESSAGE

B

MES1

A2		      CAL

SEND		 /AND ANOTHER

B

MES2

A3		      CAL

WAITE		 /WAIT FOR FIRST ONE

MES1

A4		      JMP ALOOP		 /LOOP

MES1,  ZBLOCK 3

15		 /RANDOM NUMBERS

37

23

MES2,  ZBLOCK 3

-1

4

3-3

RTS-8 Executive Requests

Task B    				     ______

B1	      BLOOP,  CAL

RECEIV		 /GET A MESSAGE

MADDR,  0

B2		      DCA EFCDF		 /SAVE CDF FOR POST

B3		      TAD EFCDF

B4		      DCA .+1		 /PUT CDF INLINE

B5		      HLT		 /CDF TO MESSAGE FIELD

B6		      TAD I MADDR	 /GET 1ST DATA WORD OF MESSAGE

B7		      CLA

B8		      STA CLL RTL	 /-3 IN AC

B9		      TAD MADDR		 /AC POINTS TO MESSAGE

B10	      CAL		 /EVENT FLAG

POST

EFCDF,  HLT		 /CDF TO MESSAGE FIELD HERE

Bll	      JMP BLOOP		 /LOOP

The flow of execution in this example depends  on  which  of  the  two

tasks  has  higher  priority.  Assuming that at some time both A and B

become runnable and task  A  has  higher  priority,  the  sequence  of

execution is as follows:

Instructions			   Why Executed	  ____________			   ____________

A1	      Task A has higher priority than task B.

A2	      Task A has higher priority than task B.

A3	      Task A has higher priority than task B.

B1	      Task A is now in Event Flag Wait  since  MES1

was PENDING; MES1 is now in WAITING state.

B2-B10	      Task A is still  WAITing--the  RECEIV  at  B1

received MES1

A4	      The POST at B10 posted MES1 and "woke up"  A,

which has higher priority than B.

A1	      A continues executing.

A2	      A tries to send MES2 again;  B  has  not  yet

processed it;  MES2 is PENDING.

Bll	      Therefore, A is put into Event Flag Wait  and

B is resumed; MES2 is now WAITING.

Bl-B10	      B now receives and posts MES2.

A2	      This brings A out of Event  Flag  Wait;   the

RTS8  Executive has modified task A's Program

Counter  so  that  it  will  re-execute   the

offending SEND.

A3	      A3 now waits for MES1 to be POSTed.

If task B has higher priority, the sequence of execution is:

Instructions			   Why Executed	  ____________			   ____________

B1	      Task B has higher priority than task A.

A1	      Task B is placed in Message Wait since  there

are no messages in its input queue.

A1	      Loops back to A1 and sends MES1 again.

3-4

RTS-8 Executive Requests

B2-B10	      Task A's message brings task B out of Message

Wait;   since  B  has  higher  priority, A is

stopped and B runs.

Bll	      The POST at B10 sets MES1 to FINISHED but has

no other effect.

B1	      Now task B tries to get another message.

A2	      There are none, so task B is put  in  Message

Wait and A is run.

B2-Bll	      Task A sends  MES2  which  "wakes  up"  B;  B

processes MES2 and

B1	      returns for more,

A3	      and is put in Message Wait.   Since  MES1  is

FINISHED

A4	      the WAITE at A3 has no effect and task A

A1	      loops back to A1 and sends MES1 again.

3.3  INTER-TASK CONTROL THROUGH TASK FLAGS TABLE ENTRY

A task's Task Flags Table entry is used by RTS-8 to determine  if  the

task  can  run.   A  zero  Task Flags Table entry indicates a runnable

task.  Each bit which is set in a non-zero word indicates a reason why

the  task  cannot run.  The currently defined bits and their meanings,

if set, are:

Octal	   Symbolic	          Meaning          _____    ________	          _______

0001	   DNEWT      Does Not Exist Wait - This  task  cannot

run because it is nonexistent.

4000	   MSGWT      Message Wait - This task is  waiting  to

be sent a message.

2000	   EFWT       Event Flag Wait - This task  is  waiting

for  an  Event  Flag  (which  contains a

WAITing  value  corresponding  to   this

task) to be POSTed.

0400	   SWPWT      Swap Wait - This task cannot run because

it  is not in core (this bit is reserved

for future RTS-8 releases).

1000	   RUNWT      Run Wait - This task is  waiting  for  a

RUN ER to be executed with its number in

the AC, or  for  the  operator  to  type

"REQUEST  task"  to  the Monitor Console

Routine (see Chapter 5).

0200	   EORMWT     Event or Message Wait  -  This  task  is

waiting for an Event Flag to be set or a

message  to  arrive,  whichever  happens

first.

0040	   ENABWT     Enable Wait - This task is waiting to be

Enabled.   Use of this bit is restricted

to the Monitor Console Routine  for  use

by  the "ENABLE task" and "DISABLE task"

commands.

3-5

RTS-8 Executive Requests

0100	   USERWT     User Wait - This bit is reserved for use

by  user-written  tasks.  RTS-8 does not

use this bit.

3.4  ERs USED TO SET AND CLEAR WAIT BITS

Several ERs exist which allow a task to explicitly set and clear  wait

bits in the Task Flags Table entry of another task, and to set bits in

its own table-entry.

3.4.1  BLKARG  Block Task for Specified Reason

Format:  TAD TASKNUM	     /OR 0 IF SELF

CAL

BLKARG

WAITBITS

TASKNUM contains the number of the task to be blocked (or not  allowed

to  run),  and  WAITBITS  specifies one or more bits to be set in that

task's Task Flags word.  Assuming  WAITBITS  is  non-zero,  this  will

cause  the specified task to become non-runnable.  If TASKNUM contains

0, the issuing task will be blocked on the specified wait  bits.   The

TASKNUM=0 form of this ER is the only legal way to specify the issuing

task as the task to be blocked;  if TASKNUM is equal  to  the  issuing

task number the action of this ER is undefined.

3.4.2  UNBARG  Unblock Task for Specified Reason

Format:	TAD TASKNUM

CAL

UNBARG

WAITBITS

TASKNUM contains the number of  the  task  to  unblock,  and  WAITBITS

specifies  one  or  more  bits to be cleared in that task's Task Flags

word.  If the Task Flags  word  becomes  zero  as  a  result  of  this

operation, the specified task becomes runnable;  if the specified task

has  higher  priority  than the issuing task and becomes runnable, the

issuing task is temporarily suspended while that task runs.

This ER is a no-op if issued with TASKNUM equal to the issuing  task's

number.

3.4.3  SUSPND  Suspend a Task's Execution

Format:	TAD TASKNUM	     /0 IF SELF

CAL

SUSPND

3-6

RTS-8 Executive Requests

This ER is identical in action to the following instructions:

TAD TASKNUM

CAL

BLKARG

RUNWT

3.4.4  RUN  Run a Task

Format:	TAD TASKNUM

CAL

RUN

This ER is identical in action to the following instructions:

TAD TASKNUM

CAL

UNBARG

RUNWT

The SUSPND and RUN ERs  exist  because  their  function  is  performed

sufficiently  often enough to warrant a shorthand version.  An example

of how they can be used follows.

A data collection task is to print a report  every  1000  data  points

without interrupting the data collection/reduction process.

Data Task				   _________

DLOOP,  TAD (-1750	 /1000 DECIMAL

DCA COUNT

DATALP, CAL

WAITE

DATAEF

.

.

.

.			 /GET A DATA POINT

ISZ COUNT		 /AND PROCESS IT

JMP DATALP	 /COUNT OFF 1000

TAD (REPORT	 /POINTS

CAL		 /RUN REPORT TASK

RUN

JMP DLOOP		 /KEEP COLLECTING

COUNT, 0

3-7

RTS-8 Executive Requests

Report Task	    			  ___________

RLOOP,  CAL		 /AC=0, SUSPEND

SUSPND		 /UNTIL NEEDED

JMS TITLE		 /HAS BEEN RUN

.

.			 /PRINT REPORT

.			 /WITH TITLE

JMP RLOOP		 /REPORT OVER-GO

/BACK AND WAIT

To eliminate interference with the data collection, REPORT should have

a lower priority than DATA.

3.5  DERAILING THE EXECUTION OF A TASK

There is one task control ER which does not cause any wait bits to get

set or cleared, but which terminates the execution of a specified task

and transfers control to a subroutine.

3.5.1  DERAIL  Derail a Task's Execution

Format:  TAD TASKNUM

CAL

DERAIL

ADDR

This ER causes a "JMS ADDR" to be simulated for the task whose  number

is  contained  in TASKNUM (the "derailed" task). ADDR is assumed to be

in the same field  in  which the  derailed  task  is  executing.   The

derailed  task's  PC  (from  its  Task State Table entry) is stored in

ADDR;  the PC entry in its Task State  Table  entry  is  then  set  to

ADDR+1.  Two  important  points  to  note  about  the operation of the

DERAIL ER are:

1.  The derailed task's AC, Link, and Data Field settings are not

saved  by  the  DERAIL  ER;  therefore they must be saved and

restored by the derail subroutine. In this  sense,  a  derail

subroutine is very much like an interrupt at the task level.

2.  The contents of the derailed task's Task Flags word  are  not

affected  by  the  DERAIL  ER.   If  the derailed task is not

runnable, the derail subroutine will not be executed until it

becomes runnable.

The DERAIL ER is generally used to signal an  emergency  condition  to

another  task.   For  example, in a process-control environment, it is

sometimes necessary to abort all operations if  the  room  temperature

exceeds  some  critical  value.   This  can be checked by a task which

measures room temperature every 10 seconds.   It  is  inefficient  and

unmodular  to  include  shutdown  code in this "watchdog task" for all

machinery being  controlled.   A  better  solution  is  to  provide  a

3-8

RTS-8 Executive Requests

location to which each equipment-controlling task can be  DERAILED  in

order  to  shut down its own piece of equipment.  The RTS-8 Power-Fail

task uses the DERAIL ER to provide a similar  facility  on  power-fail

recovery (see Section 4.5).

3.6  USING INTERRUPTS IN RTS-8

The RTS-8 Executive contains code to receive  and  dismiss  interrupts

and  to  perform  interrupt-initiated  task switching, but it does not

provide room for an interrupt skip chain.  Instead, the skip chain  is

literally a chain and is built up dynamically via the SKPINS ER.

3.6.1  SKPINS  Insert Code Into Skip Chain

Format:   CAL

SKPINS

MODULE

MODULE  is  the  address  (in  the  current  field)  of  an  interrupt

processing  module.   An interrupt processing module has the following

format:

MODULE, 0			 /THIS WORD GETS A POINTER

/TO THE NEXT MODULE

0			 /MODULE ENTERED HERE - CONTAINS

/CDF CIF NEXT MODULE FIELD

SKDR		 /SKIP ON DEVICE READY

(SKP)		 /(ONLY IF SKDR REALLY MEANS SKIP

/ON DEVICE NOT READY)

JMP I MODULE	 /NOT READY - GO TO NEXT MODULE IN

/CHAIN

CDF CIF CUR	 /THIS ONE IS MINE - SET DF AND IF

/CORRECTLY

.			 /INTERRUPT PROCESSING

.

.

CIF 0

POSTDS		 /DISMISS THE INTERRUPT, MAYBE POST

/AN EVENT FLAG

The RTS-8 interrupt skip chain initially consists of one  such  module

for  the  system  clock.   Whenever  a  task executes a SKPINS ER, the

interrupt chain is broken at the very end  and  the  user's  interrupt

module is inserted.  The last interrupt module points to the interrupt

dismiss routine as its "next module", so that RTS-8 will try to  avoid

superfluous  interrupts.  Notice that SKPINS always inserts at the end

of the skip chain.  This implies that the skips in the skip chain will

be  ordered roughly by priority of the task which inserted them, since

any SKPINS ERs in a task are usually executed  as  once-only  code  at

system startup time.

Once an interrupt module receives control  (by  having  its  I/O  skip

succeed) there are several restrictions on its execution:

3-9

RTS-8 Executive Requests

1.  The Data Field and Instruction Field are those  of  the  NEXT

interrupt module;  this must be corrected before any indirect

addressing or jumps are performed.

2.  An interrupt module may not issue any RTS-8 ERs.

3.  An interrupt module should  not  compute  excessively,  since

interrupts  are  off.  Typical execution time should be under

75 microseconds;  if considerably more computing than this is

needed,  a task should be scheduled to perform it rather than

computing excessively with interrupts off.

4.  Interrupt modules must not turn interrupts on, as  the  state

of  the  interrupted  task  may  be  destroyed  by  a  second

interrupt.

5.  On entry to the interrupt module,  the  AC,  Link,  and  Data

Field  have  already  been saved, but the Multiplier Quotient

(MQ) has not.  Therefore interrupt modules wishing to use the

MQ  should save it first and restore it before dismissing the

interrupt.

6.  Interrupt modules must dismiss the interrupt by  setting  the

Instruction  Field  to  0  and  issuing a POSTDS instruction.

POSTDS is currently defined as a JMP I  24  instruction.   An

Event  Flag  may be POSTed when the interrupt is dismissed by

setting the Data Field to the field of  the  Event  Flag  and

placing  the  location  of  the Event Flag in the AC prior to

issuing the POSTDS.  For example:

CDF CUR 	/DF = THIS FIELD

TAD (EVFLG 	/EVFLG MAY NOT BE AT LOC 0

CIF 0

POSTDS 	/DISMISS INTERRUPT AND POST EVFLG

For example, an RTS-8 Paper Tape  Punch  handler  task  might  contain

sections of code such as the following:

In the initialization code:

CAL		/LINK THE PUNCH

SKIP

SKPINS	/INTO THE SKIP CHAIN

PTPINT

.

.

.

3-10

RTS-8 Executive Requests

As a character punch subroutine:

PUNCH,	 0		/ENTER WITH CHAR IN AC

DCA TEMP	/SAVE CHAR

CAL		/WAIT UNTIL PUNCH READY

WAITE

PTPEF

/SET PUNCH EVENT FLAG

ISZ PTPEF	/PENDING

TAD TEMP

PLS		/PUNCH CHAR

CLA

JMP I PUNCH

.

.

.

Interrupt skip chain code:

PTPINT,   ZBLOCK 2	/USED TO CHAIN SKIPS

PSF		/CHECK PUNCH FLAG

JMP I PTPINT	/NOT READY

CDF CIF CUR	/SET CORRECT DF, IF

PCF		/CLEAR PUNCH FLAG

TAD (PTPEF

POSTDS		/DISMISS INTERRUPT,

/POSTING PTPEF

.

.

.

PTPEF,	 0		/PUNCH INITIALLY READY

TEMP,	 0

RTS-8 does not currently provide a mechanism for  dynamic  removal  of

entries from the interrupt skip chain.

3.7  ADVANCED RTS-8 PROGRAMMING TECHNIQUES

The information in this section is for the experienced RTS-8 user  and

should  be  ignored  until  the  user  has gained familiarity with the

system.

3.7.1  WAITM  Waiting For Multiple Event Flags

Sometimes it is desirable to wait for a logical  combination  (AND  or

OR) of Event Flags.  Waiting for the logical AND of two Event Flags is

quite simple.  The sequence:

CAL

WAITE

A	 /WAIT FOR EVENT FLAG A

CAL

WAITE

B	 /AND THEN WAIT FOR EVENT FLAG B

waits until both A and B have been POSTed.

3-11

RTS-8 Executive Requests

Waiting for the logical OR of several event flags  is  more  difficult

since there is a possible race condition between the various tests and

the interrupts  (or  task  executions)  which  POST  the  Event  Flags

involved.  The  key  to  waiting  for  an  OR  of  several Event Flags

successfully is not to allow  any  interrupts  to  occur  between  the

testing  of the first Event Flag and the placing of the task in a Wait

state if none of the flags were POSTed.

To accomplish this requires a special sequence of instructions  and  a

special  RTS-8  call  named  WAITM. WAITM is currently defined to be a

JMS I 25.  It must be executed with interrupts  off,  the  Instruction

Field  set  to  0  and the Data Field set to the current field, and it

must be followed by a word containing the blocking bit(s) to be set in

the Task Flags Table.  The action of WAITM is equivalent to the action

of the RTS-8 ER BLKARG except that  a  fast  path  through  the  RTS-8

Executive  is  taken and interrupts remain off until the blocking bits

are on in the Task Flags Table.

For an example of the use of WAITM, assume that a task "TASK" wants to

test  two  Event  Flags A and B.  If A is POSTed, control should go to

location ADONE, and if B is  POSTed  control  should  go  to  location

BDONE.   If neither is POSTed, the task must wait until one of them is

POSTed.  The code to perform this function is:

TESTAB, IOF 		/INTERRUPTS OFF - DELICATE CODE

TAD A

SNA CLA

JMP ADONE 		/ADONE MUST TURN INTERRUPTS ON

TAD (4000+TASK 		/SET A TO "WAITING' STATE

/INDICATING

DCA A			/THAT THIS TASK IS WAITING ON IT

TAD B

SNA CLA

JMP BDONE		/BDONE MUST TURN INTERRUPTS ON

TAD (4000+TASK 		/SET B TO "WAITING" STATE

/INDICATING

DCA B			/THAT THIS TASK IS WAITING ON IT

CIF 0

CDF CUR

WAITM

EFWT			/BLOCK TASK ON EVENT FLAG WAIT

JMP TESTAB		/WE'RE BACK - ONE OF THE TWO

/EVENT FLAGS HAS BEEN POSTED.

/GO BACK TO FIND OUT WHICH ONE

3.7.2  Direct References to System Tables

A task may directly refer to locations in the RTS-8  Executive  tables

to  obtain  information about itself or any other task, as long as two

restrictions are observed:

1.  Due to the interrupt-driven nature of the system, these table

entries  may change at any time; therefore, interrupts should

be inhibited while these entries are tested.

3-12

RTS-8 Executive Requests

2.  System table  entries  may  be  changed  only  through  RTS-8

Executive Requests.

Symbols have been defined  in  the  system  parameter  table  to  make

references into these tables symbolic.  The symbolic expressions which

yield the address of the system table entries for task N are:

N+TFTABL	      Task Flags Table Entry for task N;  if  zero,

task N is runnable.

N^2+MSGTBL     Input Message Queue Header  for  task  N;  if

zero, task N has no messages in its queue.

N^4+TSTABL     First Word of Task State Table Entry for task

N.

For example, for a task to determine whether it had  messages  in  its

input queue without issuing a RECEIV request, the code would be:

CDF 0 	     /EXEC TABLES IN FIELD 0

IOF 	     /TURN INTERRUPTS OFF

TAD I (TASK^2+MSGTBL

SNA CLA 	     /ANY MESSAGES?

JMP NONE 	     /NO

ION 	     /YES

The code at NONE must eventually turn interrupts back on.

3-13

CHAPTER 4

RTS-8 SYSTEM TASKS

The sources of the System Tasks are supplied with  the  RTS-8  system.

The  tasks  referred  to  as "handlers" are completely message-driven,

i.e., when idle they are in the Message Wait state.  Other tasks  send

these  handlers  I/O  request messages, and when the handler completes

the I/O operation it sets the Event Flag associated with  the  request

message and issues another RECEIVE ER.

4.1  CLOCK HANDLER

This task can be assembled to  handle  any  of  four  hardware  clocks

(DR8EA, DK8EC, DK8EP, KW12). It accepts requests (in the form of RTS-8

messages) to perform actions after a specified time has  elapsed.  The

clock  handler  uses  an  interval  queue  (whose  length  is fixed at

assembly time by the definition of the symbol CLKQLN in the  parameter

file) to keep track of its pending requests.

The format of a clock message is:

CLKMSG,  ZBLOCK 3	       /3 WORDS RESERVED FOR RTS-8

COMMAND+TASKNO  /TASKNO=0 MEANS TASKNO=SENDING TASK

TIMEHI

TIMELO

EXTRA1

EXTRA2

The words TIMEHI and TIMELO specify a time interval from  the  present

time  in  terms  of  "system  ticks".  The number of system ticks in a

second is specified by the user in the RTS-8 parameter  file  via  the

parameter  SHERTZ.   The  hardware  tick rate (in ticks per second) is

specified by the parameter HERTZ.  HERTZ must be an exact multiple  of

SHERTZ.   For example, the parameters for a line-frequency clock might

be:

DECIMAL

HERTZ= 60

SHERTZ= 10

indicating that only 1/10 second resolution is necessary in the  clock

handler.   Note  that  the  maximum interval which can be expressed in

TIMEHI and TIMELO is (2**24)/SHERTZ seconds.  This is approximately  3

days if SHERTZ=60.

The clock handler is referred to by other RTS-8 system tasks  via  the

symbol  CLOCK.   This  symbol should be defined in the RTS-8 parameter

4-1

RTS-8 System Tasks

file to be equal to the clock handler's Task  Number.   It  should  be

undefined if no clock handler is to be included in the system.

COMMAND is the type of request and has the following meanings:

0000 - POST the event flag CLKMSG after  the  specified  interval

elapses. TASKNO, EXTRA1, and EXTRA2 are ignored.

1000 - POST CLKMSG immediately. Execute a  RUN  ER  on  the  task

specified  by TASKNO after the specified interval elapses.

EXTRA1 and EXTRA2 are ignored.

2000 - POST CLKMSG immediately.  DERAIL  the  task  specified  by

TASKNO  into  a  subroutine  whose address is specified in

EXTRA1 after the specified interval  elapses.   EXTRA2  is

ignored.

3000 - POST CLKMSG immediately.  Execute a RUN  ER  on  the  task

specified  by TASKNO after the specified interval elapses,

and re-queue this command with the parameters  EXTRA1  and

EXTRA2 in place of TIMEHI and TIMELO.  This has the effect

of running the specified task periodically with  a  period

specified by EXTRA1 and EXTRA2.

7000 - Delete from the clock queue  all  requests  for  the  task

specified  by  TASKNO.  TIMEHI, TIMELO, EXTRA1, and EXTRA2

are ignored.  POST CLKMSG.

The clock handler also maintains the current  time-of-day  (in  system

ticks  until  midnight), in locations TODH (high-order) and TODL (low-

order) in Page 0 of Field  0.   When  this  time-of-day  reaches  zero

(i.e.,  at  midnight), it is reset to the quantity -(SHERTZ*86400) (24

hours until midnight) and an OS/8-format date word in location DATE in

Page 0 of Field 0 is incremented by one day.

Note that in order for the quantity SHERTZ*86400 to be contained in 24

bits,  SHERTZ must be less than 192.  If SHERTZ is larger, an assembly

error will result while assembling the clock handler.

4.1.1  Examples of Clock Handler Calls

CAL		/WITH A 50HZ SYSTEM TICK RATE,

SENDW		/THIS CAUSES THE CURRENT TASK

CLOCK		/TO "GO TO SLEEP" FOR 2 SECONDS.

SLEEPM

.

.

SLEEPM,  ZBLOCK 3	/RTS-8 MESSAGE OVERHEAD

0		/SET EVENT FLAG AFTER INTERVAL

0; 144		/INTERVAL IS 100 (DECIMAL) SYSTEM

/TICKS

4-2

RTS-8 System Tasks

Note that by changing the 144 to the assembler expression 2^SHERTZ the

above sequence becomes configuration-independent.

CAL		/RUN THE TASK REPORT ONCE

SEND		/EVERY HOUR, INDEFINITELY,

CLOCK		/ASSUMING A 50HZ SYSTEM TICK RATE

RUNMSG

.

.

RUNMSG,  ZBLOCK 3	/RTS-8 MESSAGE OVERHEAD

3000+REPORT	/RUN REPORT AFTER SPECIFIED

/INTERVAL AND PERIODICALLY

/THEREAFTER,

0;1		/FIRST RUN IS ALMOST IMMEDIATELY

/(1/50 SECOND)

53;7440	/PERIOD BETWEEN RUNS IS 180000

/(DECIMAL) SYSTEM TICKS = 3600

/SECONDS = 1 HOUR.

4.2  TERMINAL HANDLER

The RTS-8 terminal handler handles a single terminal in either line or

character mode.  Input in line mode is terminated by a carriage return

or an ALTMODE character and may be edited  using  the  RUBOUT  and  ^U

characters.   The  RUBOUT  character  deletes the last valid character

typed and prints a backslash;  the ^U  character  deletes  the  entire

line and returns the carriage.  Character mode input is not echoed and

is terminated by overflow of a specified character count.  If multiple

terminals  are to be handled, multiple copies of this terminal handler

must be assembled.  Assembly parameters in the  body  of  the  handler

specify  which  device  codes  the  handler  should  use to access its

terminal and whether the handler is to be a "console" terminal handler

(the  console  terminal handler invokes the MCR whenever a ^C is typed

on the keyboard;  non-console terminal handlers treat ^C as any  other

character).  The parameters edited into the distributed version of the

terminal handler assemble it to handle the PDP-8 console terminal as a

"console" device.

A message to the terminal handler performs a  (possibly  null)  output

operation  followed (optionally) by an input operation.  The format of

messages to the terminal handler is:

TTYMSG, ZBLOCK 3

PACK+CRLF+IND+LINE+ASSIGN+LENGTH

INBUF

OUTTXT

where:

PACK   =0     If output  text  is  in  6-bit  packed  ASCII

format.

=4000  If output text  is  in  unpacked  ASCII  (one

character per word, zero word terminates).

4-3

RTS-8 System Tasks

CRLF   =0     If output message should  be  followed  by  a

carriage return/line feed.

=2000  If output message should not be followed by a

carriage return/line feed.

IND    =0     If OUTTXT is the first  word  of  the  output

text.

=1000  If OUTTXT points to the  first  word  of  the

output text.

LINE   =0     If input  is  in  line  mode  (terminated  by

carriage return or ALTMODE).

=400   If input is  in  character  mode  (terminated

after  "LENGTH"  input  characters  have been

read).

ASSIGN =0     For normal I/O operations.

=200   "Assigns" the terminal handler  to  the  task

specified  by  the  LENGTH  field.  This will

cause the terminal  handler  to  only  accept

messages from the specified task.  If another

task tries to SEND a message to the  terminal

handler  while  it  is  assigned, the message

will be  placed  in  the  terminal  handler's

Message  Input  Queue but will not be removed

for processing by the terminal handler  until

the  assignment  is  released.   The  task to

which the terminal handler  is  assigned  can

release  the  assignment by sending a message

assigning the terminal handler to task number

0.   No  I/O  operation  is  performed  by an

assignment message.

LENGTH	      Is a  seven-bit  field  which  specifies  the

maximum  size of the input buffer if input is

in line mode, the  number  of  characters  to

input  if  input  is in character mode, and a

task number if the ASSIGN  bit  is  set.   If

input  is in line mode and there are LENGTH-1

characters in the  input  buffer,  characters

other  than  carriage return, ALTMODE, RUBOUT

and ^U will not be accepted or echoed.

INBUF	      Is a pointer to the input buffer;  if  it  is

zero, no input is taken.  The input buffer is

filled with input characters packed  one  per

word with the parity bit forced on.  If input

is in line mode, the last  character  of  the

line  is  followed  by  a  zero  word  (if  a

carriage return terminated  the  line)  or  a

negative   word   (if  an  ALTMODE  character

terminated the line).

OUTTXT	      Is either the first word of the  output  text

string  (if  IND=0) or a pointer to the first

word of the output string (if IND=1000).

4-4

RTS-8 System Tasks

4.2.1  Examples of Terminal Handler Messages

HIYA,   ZBLOCK 3      /RTS-8 MESSAGE OVERHEAD

0 	     /PACKED TEXT, END WITH CR/LF

0 	     /NO INPUT

TEXT /HELLO/  /TEXT TO BE OUTPUT

Sending the above message to the terminal handler prints HELLO on  the

terminal.

QUEST,  ZBLOCK 3	     /RTS-8 MESSAGE OVERHEAD

2060	     /PACKED TEXT, NO CR/LF,

/48-CHARACTER INPUT LIMIT

ANSWER	     /POINTER TO INPUT BUFFER

TEXT /TYPE THE ANSWER:/

Sending the above message to the  terminal  handler  prints  TYPE  THE

ANSWER:   on  the  terminal and inputs a reply without first returning

the carriage.  The answer obtained from the  above  message  could  be

printed on the terminal by sending the following message:

TYPANS, ZBLOCK 3      /THE USUAL OVERHEAD

5000	     /UNPACKED TEXT, INDIRECT, WITH CR/LF

0	     /NO INPUT

ANSWER	     /POINTER TO OUTPUT TEXT

4.3  LINE PRINTER HANDLER

The RTS-8 line printer handler outputs to an LE8,  LS8E  or  LV8  line

printer.   The  format  of  messages  to  the  line printer handler is

identical to the format of messages to the terminal handler,  but  the

INBUF word and the LINE bit are ignored (the INBUF word must, however,

be present).  Refer to Chapter 7  for  an  example  showing  the  line

printer handler message.

4.4  MASS STORAGE HANDLERS

This group of handlers all accept the same request message  format  to

read  or  write blocks on various mass storage devices.  Handlers have

been written for TC08 DECtape, DF32 and RF08 fixed-head disks, and RK8

and RK8E moving-head disks.

The format of messages to mass storage handlers is:

MSMESG, ZBLOCK 3

UNIT

RW + PAGES + FIELD

BUFADD

BLOKNO

STATUS

4-5

RTS-8 System Tasks

where:

UNIT    Is the number of the unit on which the operation  is

to be performed.

DF32 and RF08 disks consist of only one unit.

TC08 DECtape has  units  0-7  corresponding  to  its

physical units 0-7.

RK8 disk has units 0-3 corresponding to its physical

units 0-3.

RK8/E disk has units 0-7. Units  0-3  correspond  to

the  outer  (lower  track  number)  half of physical

units 0-3, and units 4-7  correspond  to  the  inner

(higher track number) half of physical units 0-3.

RW      Is  0  for  a  read  operation,  4000  for  a  write

operation.

PAGES   Specifies  the  number  of   (128-word)   pages   to

transfer,  times 100(8)(i.e., if PAGES=2000 it means

transfer 20(8) pages  or  2048  words;  if  PAGES=0,

40(8) pages or 4096 are transferred).

FIELD   Is the PDP-8  field  in  which  the  transfer  takes

place,  times  10(8)  i.e., if FIELD=30 the transfer

takes place in field 3).

BUFADD  Is  the  starting  address  of  the  buffer  to   be

transferred.

BLOKNO  Is the block number on the  device  from  which  the

transfer  will  begin.   All  devices are assumed to

have 256-word blocks.  On  DECtape,  the  first  128

words  of  each  of  an  even/odd  pair  of 129-word

DECtape records are considered to be a block.

STATUS  Is a word which is set by the handler on  completion

of  the  operation.   It  will contain a zero if the

operation was successful, otherwise it will  contain

a  non-zero  quantity  which  is  the  device status

register.  Tasks which use the mass storage handlers

should  test  this  word after the I/O operation has

been completed (i.e., after the Event Flag has  been

POSTed)  to  determine if any errors occurred during

the transfer.  All RTS-8 mass storage handlers retry

operations  three  times  if  errors are encountered

before setting the STATUS word non-zero.

Note that the middle three words  of  a  message  to  the  RTS-8  mass

storage  handlers  are  identical  to the arguments to an OS/8 handler

which performs the same operation.

4-6

RTS-8 System Tasks

4.4.1  Example of Mass Storage Handler Call

CAL

SENDW

DTA 	   /SEND A MESSAGE TO THE DECTAPE HANDLER

DTAMSG      /AND WAIT FOR COMPLETION

TAD STATUS  /CHECK THE STATUS OF THE OPERATION

SZA CLA

JMP ERR	   /BAD - GO TO ERROR ROUTINE

DTAMSG, ZBLOCK 3    /RTS-8 MESSAGE OVERHEAD

4	   /DECTAPE UNIT 4

4210	   /WRITE 256 WORDS FROM FIELD 1

BUFFER	   /LOCATION BUFFER

55	   /INTO BLOCK 55 (RECORDS 132 & 133)

STATUS, 0	   /STATUS OF OPERATION STORED HERE

4.5  POWER FAIL TASK

The Power Fail  Task  provides  the  mechanism  by  which  the  system

recovers  from power failure.  If the power-fail/auto-restart hardware

option is present and if the system parameter PWRFAL was equated to  a

non-zero value, the SPL (Skip on Power Low) instruction is included in

the interrupt skip chain.   If  a  power  low  condition  occurs,  the

processor  state  is  saved  and  the processor is halted.  When power

comes back, the processor state is  restored  and  an  Event  Flag  is

POSTed  which  wakes  up  the  Power  Fail  Task.  The Power Fail Task

restores the clock, console terminal and OS/8  terminal  if  they  are

present, and also performs an action for each task in the system based

on the contents of an internal table.  Each task has a one-word  entry

in this table, which contains:

0	  If nothing should be done for this task (default value)

-1	  If the EFWT (Event Flag Wait) bit should  be  cleared  in

the Task Flags Table entry for this task (i.e., this task

should be taken out of Event Flag Wait)

ADDR	  If the task should be DERAILed to location  ADDR  in  the

field in which it is executing as well as having its EFWT

bit cleared.

Each task in the system may alter its entry in the Power  Fail  Task's

table  by  sending a message to the Power Fail Task. The format of the

message is:

PWRMSG, ZBLOCK 3         /RTS-8 OVERHEAD

WORD

where:

WORD	       is the new contents of the Power Fail  Task's  table

entry for the sending task.

4-7

RTS-8 System Tasks

4.6  OS/8 SUPPORT TASK

This task supports the execution of the OS/8  operating  system  as  a

task  under RTS-8.  OS/8 is run in the top two (or more) memory fields

under control of the KT8/E (standard on PDP-8/E, /F, or /M with 8K  or

more  of core memory) or TSS-8 time sharing hardware option.  The OS/8

Support task is configured at  system  startup  time  to  establish  a

correspondence between OS/8 devices and RTS-8 handler tasks.  Terminal

input and output from OS/8 is ring-buffered by several  characters  to

minimize  input  loss  due to usurpation of the CPU by tasks of higher

priority.  Because of the large number of trapped CDF instructions  in

OS/8  and  its  CUSPs, response time is slower than a stand-alone OS/8

system but is still quite reasonable.

Several parameters in the system parameter file control  the  assembly

of the OS/8 support task.  They are:

OSFLDS  Defined as the number of fields to be  dedicated  to

OS/8.  OSFLDS=2 specifies 8K for OS/8.

OSKBDV  Set equal to the  keyboard  IOT  code  of  the  OS/8

terminal (OS/8 requires its own dedicated terminal).

Example:  OSKBDV=03 will use  the  console  terminal

keyboard for OS/8.

OSTTDV  Set equal to the teleprinter IOT code  of  the  OS/8

terminal.

Example:    OSTTDV=04   will   use    the    console

teleprinter for OS/8.

OSNULL  Specifies how many null  characters  must  follow  a

line-feed  character  on  the  OS/8  terminal.  This

allows high-speed VT05 terminals to be used as  OS/8

terminals.  For standard Teletypes and parallel LA30

terminals this parameter should be set to zero.

OSSYSD  Equated to the task number of the OS/8 system device

handler.

Example: OSSYSD=DTA would specify DTA0 as  the  OS/8

system device.

The OS/8 system which runs under the OS/8 Support  Task  may  run  all

OS/8 CUSPs except BUILD and FRTS, the FORTRAN IV run-time system.  All

references to  the  keyboard  and  teleprinter  are  diverted  to  the

specified  OS/8  keyboard  and teleprinter.  References in OS/8 to the

LE8, LS8E or LV8 line printers are diverted to the RTS-8 line  printer

handler  if  the  system parameter LPT is defined;  otherwise they are

executed directly by the Support Task.  References  to  the  following

OS/8  device names will be diverted to the corresponding RTS-8 handler

if one is defined:

DTA0-DTA7

RKA0-RKA3

RKB0-RKB3

___________________________

1) Teletype is a registered trademark of the Teletype Corporation.

4-8

RTS-8 System Tasks

In addition, the OS/8 handlers SYS and DSK are diverted to the handler

specified  by  the parameter OSSYSD. Other references to I/O under the

supported OS/8 system may cause the OS/8 support task  to  hang  in  a

loop.

4.7  OS/8 FILE SUPPORT TASK

The OS/8 File Support Task (hereafter  referred  to  as  OS8F)  allows

other  tasks  to lookup, create, and delete files in OS/8 directories.

It is included in the same source file as the OS/8 Support  Task,  but

may  be assembled independently of that task (depending on which tasks

were defined in the system parameter file).  The format of messages to

OS8F is:

OSFMSG, ZBLOCK 3 /RTS-8 OVERHEAD

DEVHND^10+UNIT+FUNCT

FILPTR

STATUS

BLOKNO

LENGTH

where:

DEVHND	       Is the task number of the  handler  for  the

desired device.

UNIT	       Is the unit number on which the operation is

to be performed.

FILPTR	       Is a pointer to a four-word  filename.   The

PAL8  pseudo-op  FILENAME  may  be  used  to

generate these filenames.

FUNCT 	       Represents the function to be performed.  It

may have the following values:

0     Look up the specified  file  name

and  return  its  starting  block

number   in    BLOKNO   and   its

(positive) length in LENGTH.

2000  Enter  the  specified  file  name

into the first empty space on the

device whose length is  equal  to

or  exceeds  the value in LENGTH.

Return the starting block  number

of  the new file in BLOKNO.  If a

file of the same name  previously

existed   on  the  device  it  is

deleted.  The value of LENGTH  is

unchanged.

4000  Delete the specified file name.

4-9

RTS-8 System Tasks

STATUS	       Describes the final status of the operation

as follows:

0	  Operation successful.

1	  File  not  found  on  Lookup   or

Delete.

2	  No room for file on Enter.

>2	  I/O error  occurred  -  value  is

hardware error status of device.

-1	  Invalid directory on device.

If both OS8F and the OS/8 Support Task are present  in  a  system,  an

interlock  is  set  up  to  prevent simultaneous updating of directory

blocks by both systems.  Because OS/8 tends to leave directory  blocks

in  core  for  long  periods  of  time,  this interlock scheme is very

restrictive.  Before a Delete or Enter operation  is  performed,  OS8F

waits until OS/8 is in a state in which:

1.  There  is  no  active  temporary  file  on  the  OS/8  device

corresponding to DEVHND and UNIT.

2.  OS/8 has just loaded the Keyboard Monitor,  Command  Decoder,

or USR into core.

Lookup operations are not interlocked since they  do  not  modify  the

directory.

4.8  UDC HANDLER

This handler gives the user the  capability  of  controlling  all  the

various  types  of  UDC functional devices.  This handler performs two

types of actions--immediate and deferred.  Immediate  actions  include

reading  and  sending  analog  and  digital  values to appropriate UDC

functional devices.  Deferred  actions  may  be  linked  to  specified

events  within  the  UDC (counters overflowing, switches being thrown)

and include:

1.  Run a specified task when the event occurs

2.  Set the Event Flag when the event occurs

3.  DERAIL a specified task when the event occurs

The number of deferred requests which may be pending simultaneously is

an assembly parameter of the handler.

The UDC handler permits the following operations:

1.  Analog Output - send a 10-bit value to an analog channel

2.  Analog Input - accept input from analog subchannel

3.  Digital Output - send a 12-bit value to a digital channel

4.  Digital Input - read a digital channel

4-10

RTS-8 System Tasks

5.  Get Generic Code - determine the generic code for a specified

channel

6.  Enable Counter   - permit interrupts from a counter channel

7.  Read Counter     - read current value of counter channel

8.  Disable Counter  - disable interrupts from a counter channel

9.  Enable Contacts  - permit interrupts from a contact channel

10.  Change Of State  - find the current COS value for  a  contact

channel

11.  Disable Contacts - ignore interrupts from a contact channel

Each operation is discussed in detail below, including the  format  of

the message for specifying the operation.  As with all RTS-8 messages,

a 3-word header is required for use by the Executive.  Word 4  of  all

UDC   messages  specifies  one  of  the  eleven  UDC  operations.   An

additional one to five words is also required in the  UDC  message  to

completely  specify  the  operation.   Word  10  of  all  UDC messages

contains the error state.

4.8.1  AO  Analog Output

Format:   AO channel number

subchannel & value

Channel number is the analog output channel. The subchannel and  value

word  is formed by the subchannel (0-3) in bits 0 and 1 and the 10-bit

value in bits 2-11.  For example,  a  message  for  an  analog  output

operation:

AOEX, ZBLOCK 3

AO		/ANALOG OUTPUT

23		/CHANNEL 23

4614	/SUBCHANNEL 2, VALUE 614

ZBLOCK 3

AOER, 0		/ERROR INDICATOR

4.8.2  AI  Analog Input

Format:   AI channel

subchannel & gain

answer

where channel is the analog input channel.  The  subchannel  and  gain

word need only specify the gain in bits 1-3 and the subchannel in bits

9-11.  The handler automatically sets bits 0 and 7 (enable  conversion

and read control register).  The result is put in answer.

4-11

RTS-8 System Tasks

AIEX, ZBLOCK 3

AI	/ANALOG INPUT

17	/CHANNEL 17

3		/SUBCHANNEL 3, GAIN 0

AIANS, 0		/RESULT HERE

ZBLOCK 2

AIERR, 0		/ERROR INDICATOR

It is the user's responsibility to ensure for each major channel  that

there  is  sufficient  time  (approximately 250 microseconds) for each

subchannel conversion to be completed before another is indicated.  In

general,  it may be helpful if all A/D conversions for a major channel

are initiated from the same task.

4.8.3  DO  Digital Output

Format:   DO

channel

value

Channel is a legal digital output channel and value is the  number  to

be output.  For example:

DOEX, ZBLOCK 3

DO		/DIGITAL OUTPUT

20		/CHANNEL 20

7777	/VALUE = 7777

ZBLOCK 3

DOER, 0		/ERROR INDICATOR

4.8.4  DI  Digital Input

Format:   DI

channel

result

Channel is the appropriate  digital  input  channel  and  result  will

contain the value of the channel when read.  For example:

DIEX,  ZBLOCK 3

DI	/DIGITAL INPUT

27	/CHANNEL 27

DIANS, 0		/VALUE OF CHANNEL 27 WILL BE PUT HERE

ZBLOCK 3

DIER,  0		/ERROR INDICATOR

4.8.5  GC  Generic Code

Format:   GC

channel

result

4-12

RTS-8 System Tasks

The generic code of the specified  channel  is  put  in  result.   For

example:

GCEX,  ZBLOCK 3

GC	/DETERMINES GENERIC CODE

27	/CHANNEL 27

GCANS, 0		/GENERIC CODE PUT HERE

ZBLOCK 3

GCER,  0		/ERROR INDICATOR

4.8.6  EC  Enable Counter

Format:   EC

channel

initial value

reload value

event action

address

Channel is the counter channel to be enabled,  initial  value  is  the

first  value  to  be loaded into that channel, and reload value is the

value with which to reload the channel  after  every  event.   If  the

reload  value is 0, the counter is not reloaded.  The event action and

address words specify what is to happen when the  counter  interrupts.

There are three mutually exclusive possibilities, indicated by setting

the appropriate bit in the event action word:

bit 0 = 1 =    set  event  flag  of  this   job;    continue

execution  of this job when the event occurs.

Address word not used.

bit 1 = 1 =    run a task; run task specified by  bits  4-11

of event action word.  Address word not used.

bit 2 = 1 =    DERAIL the task; the  address  word  is  only

used  by  the  DERAIL operation and specifies

the address of the DERAIL  subroutine.   That

subroutine  must  be in the same field as the

calling task.

Bit 3 indicates how many times the action is to occur:

bit 3 = 1 =    do action just once. If bit 3 = 0,  specified

action is performed after each interrupt.

Several enable counter examples follow:

ECEX1,  ZBLOCK 3

EC	/ENABLE COUNTER

4	/CHANNEL 4

7700	/INITIAL VALUE OF 7700

7710	/RESET TO 7710 AFTER EACH EVENT

4000	/POST EVENT FLAG ON EVENT

0	/UNUSED

ECER1,  0	/ERROR INDICATOR

4-13

RTS-8 System Tasks

ECEX2,  ZBLOCK 3

EC	/ENABLE COUNTER

4	/CHANNEL 4

1205	/INITIAL VALUE OF 1205

0	/DON'T RESET

2016	/RUN TASK 16 ON EVENT

0	/UNUSED

ECER2,  0	/ERROR INDICATOR

ECEX3,  ZBLOCK 3

EC	/ENABLE COUNTER

5	/CHANNEL 5

10	/INITIAL VALUE OF 10

7700	/RESET TO 7700

1015	/DERAIL TO TASK 15

5620	/AT LOCATION 5620

ECER3,  0	/ERROR INDICATOR

4.8.7  RC  Read Counter

Format:   RC

channel

result

where channel is the counter channel whose  current  value  is  to  be

read.  That value is placed in result. For example:

RCEX,  ZBLOCK 3

RC	/READ COUNTER

6		/CHANNEL 6

RCANS, 0		/VALUE OF CHANNEL 6 PUT HERE

ZBLOCK 3

RCER,  0		/ERROR INDICATOR

4.8.8  DC  Disable Counter

Format:   DC

channel

where channel is the counter channel from which interrupts are  to  be

ignored.  For example:

DCEX, ZBLOCK 3

DC		/DISABLE COUNTER

6		/CHANNEL 6

DCER, ZBLOCK 4

0		/ERROR INDICATOR

4.8.9  ECT  Enable Contacts

Format:   ECT

bit & channel

event action

address

4-14

RTS-8 System Tasks

where the bit & channel word specifies the bit on the contact  channel

from  which  to  enable interrupts.  Channel is specified in bits 4-11

and the contact bit is packed in bits  0-3  as  a  value  from  0 - 13

(octal).

Event action and address are specified in the same manner  as  in  the

enable counter function.  For example:

ECTEX1,  ZBLOCK 3

ECT	      /ENABLE CONTACTS

5401	      /FROM BIT 13(8) OF CHANNEL 1

2013	      /RUN TASK 13 AFTER AN EVENT OCCURS

ZBLOCK 3

ECTE1R,  0	      /ERROR INDICATOR

ECTEX2,  ZBLOCK 3

ECT	      /ENABLE CONTACT

1001	      /FROM BIT 2 OF CHANNEL 1

4000	      /ON 1ST OCCURRENCE OF EVENT, POST

/EVENT FLAG

ZBLOCK 3

ECTE2R,  0	      /ERROR INDICATOR

4.8.10  CS  Change of State

Format:   CS

channel

result

where channel is the contact channel whose  current  change  of  state

value is to be placed in result.  For example:

COSEX,   ZBLOCK 3

CS	      /READ COS

1	      /CHANNEL 1

COSANS,  0	      /RESULT HERE

ZBLOCK 3

COSER,   0	      /ERROR INDICATOR

4.8.11  DCT  Disable Contacts

Format:   DCT

bit & channel

where bit & channel is specified as in enable contact, that  is,  bits

0-3  specify the bit (0 - 13(octal)) and bits 4-11 specify the channel

to be disabled.  For example:

DCTEX,   ZBLOCK 3

DCT	      /DISABLE CONTACTS

5401	      /FROM CHANNEL 1, SUBCHANNEL 13(8)

ZBLOCK 4

DCTANS,  0	      /ERROR INDICATOR

4-15

RTS-8 System Tasks

4.8.12  UDC  Assembly Parameters

The UDC handler has several assembly parameters that must be specified

by the user to indicate his UDC configuration.  They are:

RINGBF	      Number of interrupts that can  be  stored  in

ring buffer.

NCNTR	      Number of counter channels.

NCNTC	      Number of contact channels.

NAD	      Number of analog input channels.

FCTR	      Address of first counter.  The counters  must

be at contiguous channels.

FCT	      Address of first contact.  Contacts must also

be at contiguous channels.

FAD	      Address of first A/D channel.   Analog  input

channels must be at contiguous channels.

These parameters are used mainly  to  specify  the  sizes  of  several

tables in the UDC handler, allocated as 30(8) words/contact channel, 3

words/counter channel, and 16(8) words/analog channel.  Note that  the

UDC  handler currently assumes that the handler and all its tables are

entirely within the same data field (although the  user  could  easily

reprogram this).

When considering the size of RINGBF, keep in mind that when the buffer

is  full,  UDC  interrupts  are  disabled  until  there is room in the

buffer. Each interrupt requires two  entries  in  the  buffer;   i.e.,

actual buffer size is 2 * RINGBF.

4.8.13  UDC  Error Conditions

To indicate error conditions, a value is placed in the tenth  word  of

the task's message by the UDC handler.  The values and meanings are:

1.  Illegal generic code for specified channel and operation

2.  Channel or subchannel value not valid

3.  Illegal function code

It is the user's responsibility to  initialize  and  check  the  error

word.  A no error condition puts a 0 in this location.

Only errors encountered at non-interrupt time  are  returned  in  this

manner,  thus  they may also indicate a faulty UDC hardware functional

device.  Generic codes of 0 or 1 encountered  at  interrupt  time  are

ignored.

4-16

RTS-8 System Tasks

4.9  CASSETTE HANDLER

This handler (CSA) allows the user to read and  write  variable-length

records  on  DECassettes, as well as perform various special functions

(rewind, write end-file, etc).

There are two general categories of cassette operation:

1.  Handler functions - read and write

2.  Utility functions - rewind, backspace file  gap,  write  file

gap, backspace block gap, and skip to file gap

It is up to the user to call these functions in a meaningful sequence.

For both functions, word 1 of the message defines the cassette unit in

bits 9-11 and function in bit 0 (where bit 0 = 0 means a utility  call

and bit 0 = 1 means a handler call). For a handler function, four more

words are required:

word 2  bit    0 = 0 - read

0 = 1 - write

bits 6-8     - field of buffer

bit   11	    - do not store data (applicable to read

only)

word 3		    - buffer address

word 4		    - record size in bits 4-11

word 5		    - status return

Cassette conventions specify a record size of 200 bytes, but the  user

may use any size up to 377, according to his requirements.  The buffer

specified by the message cannot cross field boundaries.   For  a  read

operation,  the  buffer  is optional (although its word in the message

must be included), according to bit  11  of  word  2.   The  non-store

capability  can  be  used  for  advancing through a long file.  Word 5

contains the contents of status register B, which is  defined  by  the

bit setting as follows:

Bit     Meaning			      ___     _______

4  = CRC /block error

5  = timing

6  = EOT/BOT

7  = EOF

8  = drive empty

9  = read/write

10 = write lockout

11 = ready

Word 5 should be examined by the user at  the  end  of  each  cassette

operation to check for errors encountered.

For example, to write 100 words from a buffer  starting  at  21200  to

cassette unit 3, the message is:

4-17

RTS-8 System Tasks

MSG1, ZBLOCK 3

4003	/HANDLER OPERATION ON UNIT 3

4020	/WRITE FROM FIELD 2 THE

1200	/BUFFER AT 1200 WHICH IS

0100	/100 BYTES LONG

0000	/STATUS RETURN

To read and not store 200 words from unit 2, the message is:

MSG2, ZBLOCK 3

4002	/HANDLER OPERATION ON UNIT 2

0001	/READ AND DON'T STORE

0000	/UNUSED

0200	/200 BYTES

0000	/STATUS RETURN

For a utility operation, the format of the rest of the message is:

word 2  (function in bits 6-8): 10 = Rewind

30 = Backspace file gap

40 = Write file gap

50 = Backspace block gap

70 = Skip to file gap

word 3 Status return

For example, to request a rewind on unit 1, the message is:

MSG3, ZBLOCK 3

0001      /UTILITY OPERATION ON UNIT 1

0010      /REWIND

0000      /STATUS RETURN

If an error is encountered, the operation is retried 3  times,  except

write lock out on a write operation or error while reading CRC.

The CAPS-8 USER'S MANUAL (DEC-8E-OCASA-A-D) is suggested  reading  for

users who are unsure of cassette conventions.

4.10  CASSETTE LABEL SUPPORT HANDLER

This handler supports the DEC standard cassette format and allows  the

calling  task  to  lookup and enter files on cassettes in that format.

This "handler" requires the cassette handler to perform the actual I/O

operations involved.

The cassette operations ENTER, LOOKUP and CLOSE are performed  by  the

task  CSAF  which in turn calls the cassette handler (CSA).  ENTER and

LOOKUP require the user to put appropriate  information  in  a  record

header  area with which CSAF performs the file operations.  The header

area must be  at  least  forty  words  long  and  cannot  cross  field

boundaries.  The format of a CSAF message is:

4-18

RTS-8 System Tasks

word 1 bit 0 = 1 = ENTER

bit 1 = 1 = LOOKUP

bit 2 = 1 = CLOSE

bits 9-11 = unit

word 2 Address of header for ENTER and LOOKUP

Status return for CLOSE

word 3 Field of header for ENTER and LOOKUP

word 4 Status return for ENTER and LOOKUP

In all cases, the status return is the contents of Status Register B.

For ENTER and LOOKUP, the format of the header area must conform  with

cassette  standards  (and  therefore  will be compatible with CAPS-8).

This format is:

Byte	       Use		    ____	       ___

0-5		   Filename

6-10	   Filename extension

11		   File type

1 = ASCII

0 = undefined

12-13	   File record length.  Currently  word  12

must be 0

14-15	   Unused

16-23	   Date (ASCII) specified as ddmmyy

24-37	   Unused

For an ENTER operation, if a file  with  the  name  specified  in  the

header area is found on the specified unit, it is deleted.

For a LOOKUP operation, the record  size  of  the  specified  file  is

returned in location header+13 (byte 13).  If the file is not found or

if an error occurs, this location contains 0.

The CLOSE operation is automatically followed by a REWIND.

Examples of messages follow.

MSG4, ZBLOCK 3

4000	/ENTER ON UNIT 0

6400	/INFORMATION IN HEADER STARTING AT 6400

0010	/OF FIELD 1

0000	/STATUS RETURN

MSG5, ZBLOCK 3

1003	/CLOSE ON UNIT 3

0000	/STATUS RETURN

4-19

CHAPTER 5

MONITOR CONSOLE ROUTINE

The Monitor Console Routine (MCR) provides functions  which  the  user

can  request  from  the console terminal in order to control, inspect,

and (to some extent) debug his system.

The MCR indicates that it is ready to  accept input  by  printing  the

prompting character ">" on the terminal.  An MCR command consists of a

command word (of which only the first 2  characters  are  significant)

followed by arguments and terminated by either a carriage return or an

altmode.  Commands may be a maximum of 40  characters  long.   If  the

command is terminated by a carriage return, the MCR will return to the

terminal  for  another command when it finishes processing the current

command;  if an altmode terminates the command line the MCR  will  put

itself  in  a  WAIT state when it finishes processing the command;  it

can be brought out of this wait state by typing ^C  (CTRL  C)  on  the

console  terminal.   When  the  MCR  has  prompted with its ">" and is

waiting for  input,  no  other  RTS-8  tasks  can  use  the  terminal;

therefore  if  the  terminal  is  used  for  something  other  than an

exclusive MCR terminal (for instance for error logging)  the  operator

should  type ^C, type his MCR command and terminate it with an altmode

character.  This way the MCR will not tie up the terminal.

5.1  MCR COMMAND ARGUMENTS

Certain syntactic constructions are used as arguments to  several  MCR

commands;  these are defined below:

,		 A single comma or  a  single  space  may  be  used

interchangeably   to  separate  arguments  to  MCR

commands.

Task-ID	 A Task-ID is either a number or a name.  If it  is

a  number  it  represents  the internal RTS-8 Task

Number, which is also  the  task  priority,  of  a

task.   If it is a name, the first 4 characters of

the name are looked up  in  the  MCR's  Task  Name

table to produce a Task Number.

Time-of-day A Time-of-day is  of  the  form  hh:mm,  where  hh

represents  hours  past midnight and mm represents

minutes past hh:00.

Address	 An Address is an octal number from 1 to  5  digits

in length which represents a PDP-8 memory address.

If the address is less than 5 digits long the high

order digits are assumed to be 0.

Word	 A Word is an octal number from 1 to 4 digits long.

5-1

Monitor Console Routines

A list of the  MCR  commands  follows.  In  the  header  line  of  the

description,    the   significant  portion  of  the  command  word  is

capitalized.  Optional arguments are enclosed in square brackets  ([])

and   choices  will  be  embedded  in  parentheses  and  separated  by

exclamation  points  (!).   Following  the  header  line  will  be   a

description  of  the  action  which the command causes and possibly an

example.

Commands which are preceded by asterisks (*) are not  present  if  the

symbol  CLOCK  is  not defined in the RTS-8 parameter file (indicating

that there is no clock in the system).

5.2  MCR COMMANDS

5.2.1  * DAte [mm/dd/yyyy [,Time-of-day]]

The date mm/dd/yyyy, if specified, becomes the system date.  Only  the

last digit of the year is significant;  all others are ignored and 197

assumed.  The  RTS-8  system  date  is  automatically  incremented  at

midnight,  but  all months are treated as being 31 days in length.  If

the second argument is specified, the system Time-of-day is set  equal

to  it.   If  no  arguments  are specified, the current system date is

printed on the console terminal in the form mm/dd/197y.

5.2.2  * TIme [Time-of-day]

If a Time-of-day is specified, it becomes the system time-of-day.   If

no argument is specified the current system time-of-day is printed out

on the console terminal in the form hh:mm.

5.2.3  NAme Task-ID,Newname

The character  string  Newname  becomes  the  new  name  of  the  task

specified  by  Task-ID.   The  old name of that task (if any) is lost.

Newname may be any length but only the first 4 characters are  stored.

Newname  should  not be the name of any other task or an error message

will result.

Examples:

>NAME 7 REPORT

Task number 7 is given the name REPO.

>NAME REPORT, FOO

Task number 7, which is known as REPO, is now known as FOO.

5-2

Monitor Console Routines

NOTE

The MCR name  table  is  initialized  at

assembly  time  to  contain the names of

any DEC-supplied tasks which are  listed

in  the  parameter  file  (e.g.,  if the

symbol CLOCK is defined in the parameter

file  as CLOCK=2, task number 2 will get

the name CLCK).  The MCR name table  may

be  modified  by the user to include his

own task names  permanently  by  editing

the file MCR.PA after the label NMTBL.

5.2.4  REquest Task-ID [,(@Time-of-day ! Interval)[,Interval]]

Where Interval is of the form:

nH 	     n hours

nM 	     n minutes

nS 	     n seconds

nT 	     n system ticks

This command requests a task to run immediately (if  only  Task-ID  is

specified),  at  a  given  Time-of-day,  or  after  a  given Interval.

Requesting a task clears the RUNWT bit in the Task Flags  Table  entry

for  that  task.  If the third argument is specified the task is rerun

periodically with the Interval given in  the  third  argument  as  the

period.   If  the  parameter  CLOCK in the RTS-8 parameter file is not

defined, the second and third arguments of this  command  are  ignored

and the given task is always run immediately.

Examples:

>REQUEST X

Will run task X immediately.

>RE FOO,@2:00

Will run task FOO at 2:00 A.M. (if it is after 2 A.M., FOO will be run

tomorrow at 2 A.M.).

>RE 5,lOM,5M

Will run task number 5  10  minutes  from  now  and  every  5  minutes

thereafter.

>REQ HIPR,1T,6T

On a machine with a 60 Hz clock, this command will run the  task  HIPR

immediately,  that  is, .016 seconds from now, and 10 times per second

thereafter.

5-3

Monitor Console Routines

NOTE

If, at the time the REQUEST  command  is

executed,  (which  may  be several hours

after it is typed in) the task specified

by  Task-ID  does not have the RUNWT bit

set in its Task Flags Table entry,  then

the   REQUEST   command   is   a  no-op.

Similarly, if the task  had  other  bits

set  besides  RUNWT  it  will not be run

upon execution of the  REQUEST  command,

but  will be run when the other blocking

bits are cleared.

5.2.5  STop Task-ID

Suspends execution of the task specified by  Task-ID  by  turning  the

RUNWT  bit  on  in  the  Task Flags Table entry for that task.  A task

which has been stopped may be resumed via the REQUEST MCR command  (in

this instance it is easier to think of it as the resume MCR command).

5.2.6  DIsable Task-ID

Disables future execution of the specified task by setting the  ENABWT

bit on in the Task Flags Table entry for that task.

5.2.7  ENable Task-ID

Clears the ENABWT bit in the Task Flags Table entry for the  specified

task,  thus  enabling  it  to run.  If the ENABWT bit was not set, the

command is a no-op.

5.2.8  * CAncel Task-ID

Cancels any clock queue entries involving the task specified by

Task-ID.  This includes any entries made by the MCR (from previous

timed REQUEST commands), entries involving the specified task made by

other tasks (e.g., a timed DERAIL) and entries made by the specified

task involving itself (e.g., a timed POST).  Note that in the case of

the timed POST the event flag will not be POSTed and the task may hang

up forever waiting for it.

5.2.9  SYstat [Task-ID]

If no argument is specified, the SYSTAT command prints a system status

report.   Each  line  of  the report describes an existent task in the

system.  The data printed for each task are its Task  Number/priority,

its  name  (if  it has one), and what blocking bits are on in its Task

Flags Table entry.  Each blocking bit is printed as a one-letter code,

preceded by a space.  The possible one letter codes and their meanings

are

5-4

Monitor Console Routines

E Waiting for event flag

M Waiting for a message

O Waiting for an event flag or a message

R Waiting to be REQUESTed or RUN

S Waiting to be swapped in (not in Phase 1)

D Disabled

U USERWT bit set

In addition, an asterisk is printed at the end of the line if the task

has any messages waiting in its input queue.

A more detailed status report on a single  task  may  be  obtained  by

specifying  the  Task-ID  of  that  task  as an argument to the SYSTAT

command.  The detailed report contains  all  the  information  in  the

general report, followed by five octal words:

WORD 1 The location of the Task State Table entry  containing

words 2-5.  This word is followed by a colon.

WORD 2 Task Link in sign bit, IF in bits 6-8, DF in bits 9-11.

WORD 3 Task PC

WORD 4 Task AC

WORD 5 Task MQ

Examples:

A general SYSTAT command might produce  the  following  sample  output

line:

13  CARD   E *

meaning task number 13, named "CARD", is in Event Flag  Wait  and  has

input messages pending.  The command:

>SYSTAT CARD

might produce the single line:

13  CARD   E *  1320: 0022  1741  0000  2525

indicating that CARD is stopped at location 21741 with its AC and Link

zero and 2525 in its MQ.

The SYSTAT command may be left out of the MCR assembly by setting  the

system  parameter  MCRSYS  equal  to zero in the RTS-8 parameter file,

saving one page of code.

5.2.10  OPen Address [,Count]

If Count is not specified it is  assumed  to  be  1.  Count  locations

starting  at Address are displayed in octal on the console terminal in

the form:

lllll/ cccc

The range of locations displayed may cross a field boundary.

5-5

Monitor Console Routines

5.2.11  EXamine Address [,Count ]

The EXAMINE command is identical to the OPEN command.

5.2.12  DEposit Address,Word [,Word] [,Word]....

The  second  through  nth  arguments  are  deposited  into   core   at

consecutive   locations  starting  at  Address.   As  in  OPEN,  field

boundaries may be crossed.  For example:

>DEPOSIT 26132 7700,7200,50

Deposits 7700 into location 26132, 7200 into location 26133, and  0050

into location 26134.

5.2.13  POst Address

POSTs the Event Flag at the specified address. This command is  useful

when  debugging  a  system  for  simulating the effects of nonexistent

tasks or devices.

5.2.14  EXIT

The EXIT command causes the MCR to terminate  RTS-8  execution  (after

pausing  approximately  1  second  for  pending  I/O  to complete) and

returns to the OS/8 monitor at location 07600. This  command  must  be

typed out completely or it will be regarded as an EXAMINE command.

5.3  MCR ERRORS

The MCR checks each command line for errors  and,  if  it  finds  one,

issues one of the following messages and reprompts on the terminal for

a corrected command:

BAD CHAR	 Character was not expected where it occurred

BAD NUMBER	 Number out of legal range or missing

BAD DELIM	 No space or comma where one was expected

BAD NAME	 Task name not in name table or duplicate

5-6

CHAPTER 6

ASSEMBLING AND LOADING TASKS FOR RTS-8

RTS-8 tasks are assembled with the OS/8 PAL8 assembler and the use  of

a  parameter file.  RTS-8 parameter files are all edited versions of a

master parameter file which is included  in  the  distributed  sources

(see  Appendix A).  All definitions in the master file which are to be

supplied by the user are left blank in the file;  i.e., a sample  line

in the file might be:

PDP8E= /SET TO 1 IF PDP8E OR PDP8A, ELSE 0

There should be a  unique  parameter  file  created  for  every  RTS-8

environment,  where  environment  is  a  combination  of the available

hardware and the set of tasks being run. The parameter file is divided

into 5 sections:

1.  Executive specifications - these specifications  control  the

assembly  of the RTS-8 Executive and therefore are essential;

specifications include whether the machine is a PDP-8/E,  how

much  core  is available, whether to save/restore the MQ, how

many tasks exist in the system, etc.

2.  Task definitions - this section defines  symbolic  names  for

the  various  system  tasks.   The  names of all system tasks

which are to be included in  the  system  should  be  defined

here,  and  any system task which is not included should have

the line which defines it deleted from this section.   It  is

recommended  that  this  deletion be performed by inserting a

slash (/) character at the beginning of the line, making  the

entire  line  a  comment.   The  user  is  encouraged  to add

symbolic definitions of his own tasks to this section.

3.  System task specifications - these specifications control the

assemblies  of various system tasks. They include the desired

clock rate, the number of fields to allocate  to  OS/8,  etc.

The  set  of  parameters  controlling  a task are all grouped

together and assembled conditionally only if the task name is

defined.

4.  System wide definitions - these include  the  definitions  of

the  symbols  used in this document to describe the Executive

Requests  and  Task  Flag  bits,  plus  many   other   useful

definitions.  This section should not be altered by users.

6-1

Assembling and Loading Tasks

5.  Task setup - this section uses four symbols  defined  in  the

body of the user's task to initialize the RTS-8 table entries

needed to put that task in  the  system.   The  four  symbols

needed are:

TASK	 Defined  to  be  the  task  number  of  the  task,

generally by a statement of the form:

TASK=symbol

where "symbol" is the symbolic name for  the  task

which  the user has defined in the RTS-8 parameter

file.

CUR	 Defined  to  contain  the  field  of  the   task's

starting address in bits 6-8 (e.g., CUR=10)

NOTE

It is the user's responsibility to place

the  task's  starting  code in the field

specified by CUR.  This is done  by  the

PAL8 assembler pseudo-op:

FIELD CUR%10

START	 Defined to be the  task's  starting  address  (not

necessarily the lowest address in the task)

INIWT	 Defines the initial wait bits in  the  Task  Flags

Table  entry for this task; INIWT=0 means the task

is runnable when the system starts up.

Up to three tasks may be defined in one assembly; see TASK above. (The

demonstration  program in Chapter 7 uses only one.)  The corresponding

symbols for the other tasks are TASK2 and TASK3, CUR2 and  CUR3,  etc.

The  task  setup  section  places  its  data  into the RTS-8 tables by

origining into them; no executable code is generated.

To create a real-time system using RTS-8, first lay out the tasks  you

will  need on paper.  Then create a parameter file with the parameters

for that system, and allocate core storage among  the  Executive,  the

system  tasks, and your own tasks.  Write your tasks and assemble each

task preceded by the parameter file.  Load the resulting binaries with

OS/8  ABSLDR,  placing  the RTS-8 binary first.  At this point you may

start your system, or save it as a core image file to be run later.

Each RTS-8 system task contains assembler code which causes it  to  be

assembled  for  loading  into  a  specific area of core.  The user may

assemble his own tasks to load into the areas of core not used by  the

system  tasks in his configuration, or he may edit the system tasks so

that they load wherever he wishes.  Page zero locations and auto-index

registers  used  by  system tasks can also be redefined by editing the

tasks involved.

6-2

Assembling and Loading Tasks

The OS/8 BITMAP program can be extremely useful in determining that no

two  RTS-8 tasks are loading into the same area or using the same Page

0 locations.  OS/8 BITMAP accepts a list of binary files as input  and

produces  as  output  a  map  of  core.   Each  location  of  core  is

represented in this map by a single  digit  which  has  the  following

meaning:

0    Nothing has been loaded into this location

1    Information has been loaded into this location

2    Information has been loaded into this location twice

3    Information has been loaded into this location three or

more times

There are only two places in core where twos are allowed to appear  in

the  bit map.  These areas are the RTS-8 Executive Tables (starting at

location 01200) and the MCR name table in the MCR.   Appearance  of  a

three or a two in an area (other than those just mentioned) in the bit

map is an indication that two or more tasks have loaded something into

the same location.

An RTS-8 system created as above has a starting address of 00200.   If

RTS-8  was  not  specifically  configured  for  a PDP 8/E it will halt

initially so that the operator  may  clear  any  stray  device  flags.

Press  START  to  resume  operation on a PDP 8, 8/I or 8/L;  press I/O

PRESET followed by CONTINUE on a PDP-12.

6-3

CHAPTER 7

DEMONSTRATION PROGRAM

This system will show OS/8 execution in the  background  with  several

foreground  task  executions, i.e., CLOCK, MCR and a line printer on a

16K PDP-8/E.  OS/8 will use a 2400 baud VT05 terminal (device codes 32

and  33)  and RK8E as the system device.  Below is a discussion of the

parameter file (lines 1-162) found in the program which follows  (note

that   more   tasks  than  necessary  have  been  defined  for  system

execution):

Line #		   Description	  ______		   ___________

48	    Running on a PDP-8/E or PDP-8/A

49	    No tasks use the MQ, so there is no need  to  save  and

restore it

50	    No powerfail hardware

51	    16K machine

52      Eleven (octal) tasks

*59	    System has a clock (DK8-EA), this task is priority #1

61	    Terminal module this task is priority #3

*62	    Line printer module, this task is priority #2

63	    MCR module, this task is priority #7

65	    RK8E module required for OS/8 as  system  device,  this

task is priority #4

71	    OS/8 background task, this task is priority #11

74	    User task #1, this task is priority #5 (this is  TICK),

see lines 163-213

75	    User task #2 (to be defined  by  user),  this  task  is

priority #10

80	    OS/8 requires two memory fields

81      OS/8 terminal input device code=32

82	    OS/8 terminal output device code=33

83	    OS/8 system device=RK8E

84	    VT05 at 2400 BAUD  (without  KL8-JA)  requires  4  fill

characters

87	    MCR SYSTAT facility should be included in MCR assembly

*90	    Clock is DK8EA or DK8-EC

91	    Twenty entries in clock queue

*93 	    Hardware clock runs at 120 ticks/second

*94	    Number of system ticks

________________

*  This demo will show a user task (UT1) as  printing  "TICK"  on  the

line  printer every half second.  If the user does not have a clock

or line printer do not execute any of the MCR commands shown before

those that deal with "TIME" or starting or stopping the task  named

"TICK". Also do not perform the assembly named TICKER.

7-1

Demonstration Program

.R PAL8

*RTS8.BN<DEMOPM,RTS8.PA	 \

ERRORS DETECTED: 0		  |

LINKS GENERATED: 0		  |

|

.R PAL8			  |

MCR.BN<DEMOPM,MCR.PA	  |

ERRORS DETECTED: 0		  |

LINKS GENERATED: 0		  |

|

.R PAL8			  |

*CLOCK.BN<DEMOPM,CLOCK.PA	  |

ERRORS DETECTED: 0		  |

LINKS GENERATED: 0		  |

|

.R PAL8			  |  Assembly of Digital-Supplied Modules

*TTY.BN<DEMOPM,TTY.PA	  |

ERRORS DETECTED: 0		  |

LINKS GENERATED: 0		  |  (DEMOPM is a Parameter File;

|  see following listing, lines 1-162)

.R PAL8			  |

*LPT.BN<DEMOPM,LPT.PA	  |

ERRORS DETECTED: 0		  |

LINKS GENERATED: 0		  |

|

.R PAL8			  |

*OS8SUP.BN<DEMOPM,OS8SUP.PA  |

ERRORS DETECTED: 0		  |

LINKS GENERATED: 0		  |

|

.R PAL8			  |

*RK8E.BN<DEMOPM,RK8E.PA	  |

ERRORS DETECTED: 0	  |

LINKS GENERATED: 0	 /

.R PAL8			 \

*TICKER.BN<DEMOPM,TICKER.PA  |  User Task (see listing, lines 163-213)

ERRORS DETECTED: 0	          |

LINKS GENERATED: 0		 /

7-2

Demonstration Program

.R ABSLDR

*RTS8.BN,MCR.BN,CLOCK.BN,TTY.BN,LPT.BN,OS8SUP.BN,RK8.BN,TICKER.BN/G <-

>DATE	      \  MCR prints ">"; Type "DATE"			  |

02/06/74	       | Current OS/8 Date is Printed			  |

>TIME 13:00       | Enter Time (1:00 PM)				  |

>TIME	       | Ask for Time		  ________________________|

13:00	      /				  |

|_/ Loading Binaries into

>SYS	      \  Ask for System Status	    \ Core with OS/8 ABSLDR.

01  CLCK 0	       |

02  LPT  M	       |

03  TTY  M	       |

04  RK8  M	       |

05       R	       |

07  MCR	       |

11  OS8  E	      /

>NAME 5 TICK     \  Call Task #5 "TICK"

>SYS	       | Ask for System Status

01  CLCK 0	       |

02  LPT  M	       |

03  TTY  M	       |

04  RK8  M	       |

05  TICK R	       |

07  MCR	       |

11  OS8  E	      /

>REQUEST TICK    \  Start TICK running; "TICK" is printed on LPT every

|   1/2 second.

>STOP TICK	       | Stop TICK

>TIME	       | Ask Time Again

13:02	      /

>REQ TICK,@l3:05 \	 Ask TICK to Run at 1:05

>TIME	       |

13:02	       | Time = 1:02

>TIME	       |

13:03	       | Time = 1:03

>TIME	       |

13:04	       | Time = 1:04

>TIME	       | TICK Starts Running Again

13:05	      /  Time Now 1:05

>STOP TICK       \  Stop TICK Task

>EXIT	       | Return to OS/8 Monitor

|

...	      /

7-3

Type this program with the OS/8 Editor and save it

on the  system device under the name of TICKER.PA.

/PARAMETERS FOR RTS-8 TASKS 		PAL8-V9B  05/22/74  PAGE 1

1		   /PARAMETERS FOR RTS-8 TASKS

2		   /

3		   /

4		   /

5		   /

7		   /

8		   /

9		   /

10	 	   /

11	 	   /COPYRIGHT	(C)   1974 BY DIGITAL EQUIPMENT CORPORATION

12		   /

13		   /

14		   /

15		   /

16		   /

17		   /

18		   /

19		   /

20		   /

21		   /

22		   /THE INFORMATION IN THIS DOCUMENT IS SUBJECT TO CHANGE

23		   /WITHOUT NOTICE AND SHOULD NOT BE CONSTRUED AS A

24		   /COMMITMENT BY DIGITAL EQUIPMENT CORPORATION.  DIGITAL

25		   /EQUIPMENT CORPORATION ASSUMES NO RESPONSIBILITY FOR

26		   /ANY ERRORS THAT MAY APPEAR IN THIS DOCUMENT.

26		   /

27		   /THE SOFTWARE DESCRIBED IN THIS DOCUMENT IS FURNISHED TO

28		   /THE PURCHASER UNDER A LICENSE FOR USE ON A SINGLE

29		   /COMPUTER SYSTEM AND CAN BE COPIED (WITH INCLUSION OF

30		   /DIGITAL'S COPYRIGHT NOTICE) ONLY FOR USE IN SUCH SYSTEM

31		   /EXCEPT AS MAY OTHERWISE BE PROVIDED IN WRITING BY

32		   /DIGITAL.

33		   /

34		   /DIGITAL EQUIPMENT CORPORATION ASSUMES NO RESPONSIBILITY

35		   /FOR THE USE OR RELIABILITY OF ITS SOFTWARE ON EQUIPMENT

36		   /THAT IS NOT SUPPLIED BY DIGITAL.

37		   /

38		   /

39		   /

40		   /

41		   /

42		   /

43		   /

44		   /

7-4

/PARAMETERS FOR RTS-8 TASKS 		PAL8-V9B  05/22/74  PAGE 2

45

46	           /RTS8 EXEC PARAMETERS - EDITED BY USER

47

48	     0001  PDP8E=  1	   /1 IF PDP 8/E

49	     0000  EAE=    0	   /1 IF SYSTEM SHOULD SAVE MQ

50	     0000  PWRFAL= 0	   /1 IF POWER FAIL/RESTART ENABLED IN HARDWARE

51	     0030  HGHFLD= 30      /HIGHEST FIELD USED (EG 30 FOR A 16K MACHINE)

52	     0011  NTASKS= 11      /NUMBER OF TASKS IN SYSTEM

53

54	           /COMMON TASK NUMBERS - EDITED BY USER

55	           /IT IS ADVISABLE TO DEFINE ALL TASKS HERE.  NAMES GIVEN

56    		   /BELOW ARE USED BY SOME SYSTEM TASKS AND SHOULD BE DELETED

57    		   /FROM THIS LIST IF THE CORRESPONDING TASK IS NOT INCLUDED

/IN THE SYSTEM

58

59	     0001  CLOCK=  1       /CLOCK HANDLER - SHOULD BE HIGH PRIORITY

60	           /PWRF=	   /POWER FAIL HANDLING TASK

61	     0003  TTY=    3       /TELETYPE DRIVER TASK

62	     0002  LPT=    2       /LINE PRINTER DRIVER TASK

63	     0007  MCR=    7       /MONITOR CONSOLE ROUTINE

64	           /DTA=	   /DECTAPE DRIVER TASK

65	     0004  RK8=    4       /RK8 OR RK8E DISK DRIVER TASK

66	           /RFO8=	   /RF08 DISK DRIVER TASK

67	           /DF32=          /DF32 DISK DRIVER TASK

68	           /CSA=	   /CASSETTE DRIVER TASK

69	           /CSAF=          /CASSETTE FILE SUPPORT TASK

70	           /UDC=	   /UNIVERSAL DIGITAL CONTROLLER TASK

71	     0011  OS8=    NTASKS  /OS/8 SUPPORT - NORMALLY LOWEST PRIORITY

72	           /OS8F=	   /OS/8 FILE SUPPORT

73

74	     0005  UT1=    5       /USER TASK 1

75	     0010  UT2=    10      /USER TASK 2

76

77	           /SOFTWARE PARAMETERS - EDITED BY USER

78

79		           IFDEF   OS8     <

80	     0002  OSFLDS= 2	   /NUMBER OF FIELDS TO ALLOCATE TO OS/8 (EG 2)

81	     0032  OSKBDV= 32      /OS/8 KEYBOARD DEVICE CODE (EG 03 = CONSOLE)

82	     0033  OSTTDV= 33      /OS/8 TELEPRINTER DEVICE CODE (EG 04)

83	     0004  OSSYSD= RK8     /OS/8 SYSTEM DEVICE DRIVER TASK (E.G. RK8)

84	     0004  OSFILL= 4       /# FILLS NEEDED AFTER LINE FEED

85		           >       /(EG 4 FOR 2400 BAUD VT05)

7-5

86		           IFDEF   MCR     <

87	     0001  MCRSYS= 1       /1 IF MCR SYSTAT FACILITY DESIRED

88		           >

89		           IFDEF   CLOCK  <

90   	     0000  CLKTYP= 0       /0(DK8EA/DK8EC) OR 1(KW12) OR 2(VT8E) OR

/3(DK8FP)

91	     0020  CLKQLN= 20      /MINIMUM NUMBER OF ENTRIES ON THE CLOCK

/OUEUE

92		           DECIMAL

93	     0170  HERTZ=  120     /NUMBER OF HARDWARE TICKS PER SECOND

94	     0074  SHERTZ= 60      /NUMBER OF SYSTEM TICKS PER SECOND

95		           OCTAL

96		           >

/PARAMETERS FOR RTS-8 TASKS		PAL8-V9B  05/22/74  PAGE 3

97	            /EQUIVALENCES:

98

99	     7344  AC7776= CLL STA RAL

100	     7346  AC7775= CLL STA RTL

101	     7330  AC4000= CLA STL RAR

102	     7350  AC3777= CLL STA RAR

103	     7332  AC2000= CLA STL RTR

104	     7326  AC0002= CLA STL RTL

105

106	           /MONITOR CALL VALUES:

107

108	     4020  CAL=    JMS     20	    /CALL THE EXECUTIVE

109	     5424  POSTDS= JMP I   24	    /DISMISS AN INTERRUPT

110	     4425  WAITM=  JMS I   25	    /WAIT FOR MULTIPLE EVENTS

111

112	     0000  SEND=   0	   /SEND MESSAGE

113	     0001  RECEIV= 1       /RECEIVE MESSAGE

114	     0002  WAITE=  2       /WAIT FOR EVENT FLAG

115	     0003  RUN=    3       /CONTINUE TASK EXECUTION

116	     0004  SUSPND= 4	   /SUSPEND TASK EXECUTION

117	     0005  POST=   5       /POST AN EVENT FLAG

118 	     0006  SKPINS= 6       /INSERT CODE INTO INTERRUPT SKIP CHAIN

119	     0007  DERAIL= 7       /INITIATE END-ACTION

120	     0010  BLKARG= 10      /BLOCK TASK FOR REASON SPECIFIED IN ARG

121	     0011  SENDW=  11      /SEND MESSAGE AND WAIT

122	     0012  UNBARG= 12      /UNBLOCK TASK FOR REASON SPECIFIED IN ARG

7-6

123

124		           IFDEF UDC	   <AO=0;DO=1;DI=2;GC=3;EC=4;RC=5

125				           DC=6;ECT=7;CS=10;DCT=11;AI=12>

126

127	           /TASK STATUS FLAGS:

128

129	     4000  MSGWT=  4000    /MESSAGE WAIT

130	     2000  EFWT=   2000    /EVENT FLAG WAIT

131	     1000  RUNWT=  1000    /SCHEDULE WAIT

132	     0400  SWPWT=  0400    /SWAPPER WAIT

133	     0200  EORMWT= 0200    /EVENT FLAG OR MESSAGE WAIT

134	     0100  USERWT= 0100    /USER SPECIFIED WAIT

135	     0040  ENABWT= 0040    /ENABLE WAIT

136	     0001  DNEWT=  0001    /DOES NOT EXIST WAIT

137

138		   /SYSTEM LOCATIONS

139

140	     1176  MSGTBL= 1200-2  		/TASK MESSAGE TABLE

141	     1220  TSTABL= NTASKS+2^2+MSGTBL-4  /TASK STATE TABLE - HOLDS

142					        /TASK LINK,UM,DF,IF,PC,AC,MQ

143	     1273  TFTABL= NTASKS+2^4+TSTABL-l  /TASK FLAGS TABLE - HOLDS

144					        /TASK STATUS FLAGS

145

146	     0035  TSWFLG= 35      /TASK SW INHIBIT FLAG   IN FIELD 0

147	     0036  TODL=   36      / LOW ORDER TIME OF DAY IN FIELD 0

148	     0037  TODH=   37      /HIGH ORDER TIME OF DAY IN FIELD 0

149	     0040  DATE=   40      /DATE IN OS8 FORMAT     IN FIELD 0

150	     0041  MCREF=  41      /MCR START EVENT FLAG   IN FIELD 0

151

/PARAMETERS FOR RTS-8 TASKS		PAL8-V9B  05/22/74  PAGE 4

152	           /TASK TABLE SETUP - "TASK", "CUR","INIWT" AND "START"

153	           /MUST BE DEFINED BY TASK:

154

155	     1210	   *TASK^2+MSGTBL

156   01210  0000	   ZBLOCK 2	   /MESSAGE BUFFER INITIALLY CLEAR

157	     1244	   *TASK^4+TSTABL

158   01244  0011	   CUR%10+CUR	   /INITIAL FLAGS

159   01245  5000	   START

160   01246  0000	   0		   /INITIAL AC 0

161	     1300	   *TASK+TFTABL

162   01300  1000	   INIWT

7-7

/TASK TO PRINT "TICK" ON THE LINE PRINT   PAL8-V9B  05/22/74  PAGE 5

163		  /TASK TO PRINT "TICK" ON THE LINE PRINTER TWICE A SECOND

164

165		  /DEFINITIONS OF TASK PARAMETERS FOR RTS8:

166

167	     0005  TASK=  UT1     /I HAVE DEFINED UT1 IN MY PARAMETER FILE!

168	     0010  CUR=	  10	  /THIS TASK STARTS IN FIELD 1

169	     1002  INIWT= RUNWT   /THIS TASK IS NOT RUNNABLE INITIALLY. IT

170    				  /IS IN "RUN WAIT". THE ONLY WAY TO MAKE

171    				  /THIS TASK RUNNABLE IS FOR ANOTHER TASK

172    				  /TO ISSUE A "RUN" EXECUTIVE REQUEST

173    				  /SPECIFYING THIS TASK, OR FOR THE OPERATOR

/TO TYPE A "REQUEST" COMMAND TO THE MCR

/SPECIFYING THIS TASK.

174

175	     0001	 FIELD CUR%10	/SET FIELD FOR ASSEMBLER

178	     5000	 *5000		/SET ORIGIN

177

178   15000  4020  START, CAL		/"START" IS STARTING ADDRESS OF TASK

179   15001  0011	  SENDW		/SEND A MESSAGE TO THE CLOCK HANDLER

180   15002  0001 	  CLOCK		/AND WAIT FOR THE MESSAGE EVENT FLAG

181   15003  5011	  SLPMSG	/THIS IS A POINTER TO THE MESSAGE

182

183		  /THE MESSAGE TO THE CLOCK HANDLER TELLS IT TO POST THE

184		  /MESSAGE EVENT FLAG IN HALF A SECOND

185

188   15004  4020  	  CAL		/NOW SEND A MESSAGE TO THE LINE

187   15005  0011	  SENDW		/PRINTER TASK AND WAIT FOR IT TO

188   15008  0001	  LPT		/COMPLETE.

189   15007  5017	  TCKMSG

190

191   15010  5200	  JMP	 START	/LOOP MINDLESSLY - THIS WILL ANNOY

192					/THE OPERATOR NO END, AS THE LINE

193					/PRINTER WILL GO "TICK...TICK...TICK"

194					/UNTIL HE STOPS THIS TASK WITH THE

/MCR "STOP" COMMAND.

195

196   15011  9000  SLPMSG,ZBLOCK 3	/THREE WORDS REQUIRED BY RTS8 FOR

/MESSAGE HEADER

197   15014  0000	  0		/0 HERE MEANS "SET MESSAGE EVENT FLAG

/AFTER INTERVAL"

198   15015  0000	  0;SHERTZ%2	/DOUBLEWORD QUANTITY = 1/2 SECOND

199   15016  0036			/SINCE

7-8

200

201					/"SHERTZ" IS THE NUMBER OF TICKS/

202					/SECOND.  LAST 2 PARAMETERS NOT

203					/NEEDED IN THIS TYPE OF MESSAGE.

204

205   15017  0000  TCKMSG, ZBLOCK 3	/MESSAGE HEADER

206   15022  0000	  0		/FLAG WORD - THIS COMBINATION OF FLAG

207					/BITS MEAN: PACKED ASCII MESSAGE CR/LF

208					/AFTER TEXT INCLUDED IN MESSAGE

209   15023  0000	  0		/UNUSED - THIS SPECIFIES INPUT BUFFER

210					/FOR TELETYPE MESSAGES.

211   15024  2411	  TEXT \TICK!\	/ACTUAL TEXT TO BE OUTPUT

212   15025  0313

213   15026  4100

214			  $

/TASK TO PRINT "TICK" ON THE LINE PRINT   PAL8-V9B  05/22/74  PAGE 6

AC0002 7326      TODL   0036

AC2000 7332      TSTABL 1220

AC3777 7350      TSWFLG 0035

AC4000 7330      TTY    0003

AC7775 7346      UNBARG 0012

AC7776 7344      USERWT 0100

BLKARG 0010      UT1    0005

CAL    4023      UT2    0010

CLKQLN 0020      WAITE  0002

CLKTYP 0000      WAITM  4425

CLOCK  0001

CUR    0010

DATE   0040

DERAIL 0007

DNEWT  0001

EAE    0000

EFWT   2000

ENABWT 0040

EORMWT 0200

HERTZ  0170

HGHFLD 0030

7-9

INIWT  1000

LPT    0002

MCR    0007

MCREF  0041

MCRSYS 0001

MSGTBL 1176

MSGWT  4000

NTASKS 0011

OSFILL 0004

OSFLDS 0002

OSKBDV 0032

OSSYSD 0004

OSTTDV 0033

OS8    0011

PDP8E  0001

POST   0005

POSTDS 5424

PWRFAL 0000

RECEIV 0001

RK8    0004

RUN    0003

RUNWT  1000

SEND   0000

SENDW  0011

SHERTZ 0074

SKPINS 0006

SLPMSG 5011

START  5000

SUSPND 0004

SWPWT  0400

TASK   0005

TCKMSG 5017

TFTABL 1273

TODH   0037

ERRORS DETECTED: 0

LINKS GENERATED: 0

7-10

AC0002     104#

AC2000     103#

AC3777     102#

AC4000     101#

AC7775     100#

AC7776      99#

AI         130#

AO         124#

BLKARG     120#

CAL        108#  176   188

CLKQLN      91#

CLKTYP      90#

CLOCK       59#   89   180

CS         127#

CT         125#

CUR        158    158  168#  175

DATE       149#

DCT        129#

DERAIL     119#

DNEWT      136#

EAE         49#

ENABWT     135#

EORMWT     133#

HERTZ       93#

HGHFLD      51#

INIWT      162   169#

LPT         62#  188

MCR         63#   86

MCREF      150#

MCRSYS      87#

MSGTBL     140#  141   155

NTASKS      52#   71   141   143

OSFILL      84#

OSFLDS      80#

OSKBDV      81#

OSSYSD      83#

OSTTDV      82#

OS8         71#  79

PDP8E       48#

POST       117#

POSTDS     109#

PWRFAL      50#

7-11

RECEIV      113#

RK8          65#   83

RUN         115#

RUNWT       131#  169

SEND        112#

SENDW       121#  179   187

SHERTZ       94#  199

SKPINS      118#

SLPMSG      181   196#

START       159   178#  191

SUSPND      116#

SWPWT       132#

TASK        155   157   161   167#

TCKMSG      189   205#

TFTABL      143#  161

TODH        148#

TODL        147#

TSTABL      141#  145   157

TSWFLG      146#

TTY          61#

UDC         124#

UNBARG      122#

USERWT      134#

UT1          74#  167

UT2          75#

WAITE       114#

WAITM       110#

V3

7-12

Notes for the bitmap below:

Locations		What is there     _________          _____________

00000-00002	RTS-8 Page 0 locations

00010

00017-00047

00200-01177	RTS-8 Task

01200-01377	Task Tables ( two's are legal; three's are not)

01400-01777	Clock Task

02000-02177	Clock Queue and Initializer

03000-03377	TTY Task

03400-03777	RK8E Task

06200-07577	OS/8 Support Task

10021-10025	Field 1 RTS-8 Resident

10100-10116	MCR Page 0 Locations

14600-14777	LPT Task

15000-15026	User "Foreground" Task

20021-20025	Field 2 RTS-8 Resident destroyed by OS/8 "field 1".

30021-30025	Field 3 RTS-8 Resident destroyed by OS/8 "field 0".

03400-34377	OS/8 Support unit code to set up fake OS/8 in

fields 3 and 2.

7-13

BITMAP V4    FIELD 0

00000 11100000 10000001 11111111 11111111 11111111 00000000 00000000 00000000

00100 00000000 00000000 00000000 00000000 00000000 00000000 00000011 11111111

00200 11111111 11111111 11111111 11111111 11111111 11111111 11111111 11111111

00300 11111111 11111100	00000000 00000000 00000000 00000000 00000000 00000111

00400 11111111 11111111 11111111 11111111 11111111 11111111 11111111 11111111

00500 11111111 11111111 11111111 11000000 00000000 00000000 00000000 01111111

00600 11111111 11111111 11111111 11111111 11111111 11111111 11111111 11111111

00700 11111111 11111111 11111111 11111111 11111111 11111111 11111000 00111111

01000 11111111 11111111 11111111 11111111 11111111 11111111 11111111 11111111

01100 11111111 11111111 11111111 11111111 11111111 11111111 11111001 11111111

01200 22222222 22112211 22222221 22212221 22212221 11112221 11112221 22212222

01300 21212200 00000000 00000000 00000000 00000000 00000000 00000000 00000000

01400 11111111 11111111 11111111 11111111 11111111 11111111 11111111 11111111

01500 11111111 11111111 11111111 11111111 11111111 11111111 11111111 11111111

01600 11111111 11111111 11111111 11111111 11111111 11111111 11111111 11111111

01700 11111111 11111111 11111111 11111111 11111111 11000000 00111111 11111111

02000 11111111 11111111 11111111 11111111 11111111 11111110 00000000 00000000

02100 00000000 00000000 00000000 00000000 00000000 00000000 00000000 00111111

02200

02300

02400

02500

02600

02700

03000 11111111 11111111 11111111 11111111 11111111 11111111 11111111 11111111

03100 11111111 11111111 11111111 11111111 11111111 11111111 11100011 11111111

03200 11111111 11111111 11111111 11111111 11111111 11111111 11111111 11111111

03300 11111111 11111111 11111111 11111111 00000000 00000111 11111111 11111111

03400 11111111 11111111 11111111 11111111 11111111 11111111 11111111 11111111

03500 11111111 11111111 11111111 11111111 11111111 11111111 01111111 11111111

03600 11111111 11111111 11111111 11111111 11111110 00000000 00000000 00000000

03700 00000000 00000000 00000000 00000000 00000000 00000000 00000000 11111111

7-14

BITMAP V4    FIELD 0

04000

04100

04200

04300

04400

04500

04600

04700

05000

05100

05200

05300

05400

05500

05600

05700

06000

06100

06200 11111111 11111111 11111111 11111111 11111111 11111111 11111111 11111111

06300 11111111 11111111 11111111 11111111 11111100 00000000 00000111 11111111

06400 11111111 11111111 11111111 11111111 11111111 11111111 11111111 11111111

06500 11111111 11111111 11111111 11111111 10000000 00000011 11111111 11111111

06600 11111111 11111111 11111111 11111111 11111111 11111111 11111111 11111111

06700 11111111 11111111 11111111 11111111 11111111 11111111 01111111 11111111

07000 11111111 11111111 11111111 11111111 11111111 11111111 11111111 11111111

07100 11111111 11111111 11111111 11111111 11111111 10000000 00111111 11111111

07200 11111111 11111111 11111111 11111111 11111111 11111111 11111111 11111111

07300 11111111 11111110 00000000 00000000 00000000 00000000 00000001 11111111

07400 11111111 11111111 11111111 11111111 11111111 11111111 11111111 11111111

07500 11111111 11111111 11111111 11111111 11111111 11111111 11111111 11111100

07600

07700

7-15

BITMAP V4    FIELD 1

10000 00000000 00000000 01111100 00000000 00000000 00000000 00000000 00000000

10100 11111111 11111110 00000000 00000000 00000000 00000000 00000000 00000000

10200

10300

10400

10500

10600

10700

11000

11100

11200

11300

11400

11500

11600

11700

12000

12100

12200

12300

12400

12500

12600

12700

13000

13100

13200

13300

13400

13500

13600

13700

7-16

BITMAP V4    FIELD 1

14000

14100

14200

14300

14400

14500

14600 11111111 11111111 11111111 11111111 11111111 11111111 11111111 11111111

14700 11111111 11111111 11111111 11111111 11111111 11111111 11110000 00011111

15000 11111111 11111111 11111110 00000000 00000000 00000000 00000000 00000000

15100

15200

15300

15400 11111111 11111111 11111111 11111111 11111111 11111111 11111111 11111111

15500 11111111 11111111 11111111 11111111 11111111 11111111 11110000 11111111

15600 11111111 11111111 11111111 11111111 11111111 11111111 11111111 11111111

15700 11111111 11111111 11111111 11111111 11111111 11000000 00000111 11111111

16000 11111111 11111111 11111111 11111111 11111111 11111111 11111111 11111111

16100 11111111 11111111 11111111 11111111 11111111 11111111 11100000 01111111

16200 22222222 11112211 22111111 11111111 11111111 11111111 11111111 11111111

16300 11111111 11100000 00000000 00000000 00000000 00000000 00011111 11111111

16400 11111111 11111111 11111111 11111111 11111111 11111111 11111111 11111111

16500 11111111 11111111 11111111 11111111 11111111 11110000 00111111 11111111

16600 11111111 11111111 11111111 11111111 11111111 11111111 11111111 11111111

16700 11111111 11111111 11111111 11111111 11111111 11111111 11110000 11111111

17000 11111111 11111111 11111111 11111111 11111111 11111111 11111111 11111111

17100 11111111 11111111 11111111 11111111 11111111 11111111 10111111 11111111

17200 11111111 11111111 11111111 11111111 11111111 11111111 11111111 11111111

17300 11111111 11111111 11111111 11111111 11100000 00000011 11111111 11111111

17400 11111111 11111111 11111111 11111111 11111111 11111111 11111111 11111111

17500 11111111 11111111 11111111 11111111 11111111 11111101 11111111 11111111

17600

17700

7-17

BITMAP V4    FIELD 2

20000 00000000 00000000 01111100 00000000 00000000 00000000 00000000 00000000

20100

20200

20300

20400

20500

20600

20700

21000

21100

21200

21300

21400

21500

21600

21700

22000

22100

22200

22300

22400

22500

22600

22700

23000

23100

23200

23300

23400

23500

23600

23700

7-18

BITMAP V4    FIELD 2

24000

24100

24200

24300

24400

24500

24600

24700

25000

25100

25200

25300

25400

25500

25600

25700

26000

26100

26200

26300

26400

26500

26600

26700

27000

27100

27200

27300

27400

27500

27600

27700

7-19

BITMAP V4    FIELD 3

30000 00000000 00000000 01111100 00000000 00000000 00000000 00000000 00000000

30100

30200

30300

30400

30500

30600

30700

31000

31100

31200

31300

31400

31500

31600

31700

32000

32100

32200

32300

32400

32500

32600

32700

33000

33100

33200

33300

33400

33500

33600

33700

7-20

BITMAP V4    FIELD 3

34000 11111111 11111111 11111111 11111111 11111111 11111111 11111111 11111111

34100 11111110 00000000 00000000 00000000 00000000 00000000 00000000 00011111

34200 11111111 11111111 11111111 11111111 11111111 11111111 11111111 11111111

34300 11111111 11100000 00000000 00000000 00000000 00000000 00000000 00011111

34400

34500

34600

34700

35000

35100

35200

35300

35400

35500

35600

35700

36000

36100

36200

36300

36400

36500

36600

36700

37000

37100

37200

37300

37400

37500

37600

37700

7-21

APPENDIX A

RTS-8 DISTRIBUTED SOURCE FILES

The RTS-8 source files included on the distribution tape are:

FILE	 TASK(S)		     COMMENTS	____     _______		     ________

PARAM.PA	 -		   System parameter file with  all  equates

blank.   Should  never  be  altered, but

should  be  edited  to  create  specific

parameter files

RTS8.PA	 - (null task)	   RTS-8 Executive

MCR.PA	 MCR		   Monitor Console Routine

OS8SUP.PA   OS8		   OS/8 Support Task

OS8F		   OS/8 File Support Task

PWRF.PA	 PWRF		   Power Fail Task

CLOCK.PA	 CLOCK		   Clock handler Task

TTY.PA	 TTY		   Terminal driver Task

LPT.PA	 LPT		   Line printer driver Task

DTA.PA	 DTA		   TC08 DECtape driver Task

RK8.PA	 RK8		   RK8 disk driver Task

RK8E.PA	 RK8		   RK8E disk driver Task

RF08.PA	 RF08/DF32	   RF08/DF32 fixed-head disk driver Task

CSA.PA	 CSA		   Cassette driver Task

CSAF.PA	 CSAF		   Cassette File Support Task

UDC.PA	 UDC		    Universal  Digital  Controller  handler

Task

A-1

APPENDIX B

RTS-8 COMPONENT SIZES

The following table gives the approximate size and default origins  of

each  component  of  the  RTS-8  system as of the first release of the

system.

NTASKS = Number of tasks in system

CLKQLN = Number of entries in clock queue

MCRSYS = 1 if MCR SYSTAT function desired, else 0

Any fractions from divides should be dropped.

Table B-1

RTS-8 Component Sizes

-------------------------------------------------------------------------

|		  |		  | NUMBER OF PAGES |			   |

|  SOFTWARE    |    DEFAULT    | REQUIRED (1 PAGE|			   |

|  COMPONENT   |    ORIGIN	  | =128 WORDS      |       COMMENTS       |

|--------------|---------------|-----------------|----------------------|

|		  |		  |		    |			   |

| RTS-8	  |    00200	  | 5+NTASKS/18     | Must be in locations |

| Executive    |		  |		    | 00200-01200. Uses    |

|		  |		  |		    | page 0 locations 0-3 |

|		  |		  |		    | and 20-47 and	   |

|		  |		  |		    | auto-index register  |

|		  |		  | 		    | 17.		   |

|		  |		  |		    |			   |

| Clock Module | 1st page	  | 3+CLKQLN/22     | Must be in field 0.  |

|		  | after end of  | 		    | Uses auto-index      |

|		  | of RTS-8 	  |		    | register 10.	   |

|		  | Executive	  |		    |		  	   |

|		  |		  |		    |			   |

| Console 	  |    03000	  | 2		    |			   |

| Terminal	  |		  |		    |			   |

| Module	  |		  |		    |			   |

|		  |		  |		    |			   |

| Non-Console  |     --        | 2		    |			   |

| Terminal	  |		  |		    |			   |

| Module	  |		  |		    |			   |

|		  |		  |		    |			   |

| Line Printer |    14600	  | 1		    |			   |

| Module	  |		  |		    |			   |

|		  |		  |		    |			   |

| TC08 DECtape |    15000	  | 2		    |			   |

| Module	  |		  |		    |			   |

|		  |		  |		    |			   |

-------------------------------------------------------------------------

B-1

RTS-8 Component Sizes

Table B-1 (Cont.)

RTS-8 Component Sizes

-------------------------------------------------------------------------

|		  |		  | NUMBER OF PAGES |			   |

|  SOFTWARE    |    DEFAULT    | REQUIRED (1 PAGE|			   |

|  COMPONENT   |    ORIGIN	  | =128 WORDS      |       COMMENTS       |

|--------------|---------------|-----------------|----------------------|

|		  |		  |		    |			   |

| RK8E Disk 	  |    03400	  | 2		    |			   |

| Module	  |		  |		    |			   |

|		  |		  |		    |			   |

| DF32/RF08 	  |    04000	  | 1		    |			   |

| Module	  |		  |		    |			   |

|		  |		  |		    |			   |

| RK8 Disk	  |    03400	  | 1		    |			   |

| Module	  |		  |		    |			   |

|		  |		  |		    |			   |

| Power Fail	  |    10200	  | 1+NTASKS/32	    |			   |

| Module	  |		  |		    |			   |

|		  |		  |		    |			   |

| UDC Module	  |    10600	  | 7-9 depending   | Uses page 0	   |

|		  |		  | on table space  | locations 130-144	   |

|		  |		  | desired.	    |			   |

|		  |		  |		    |			   |

| Cassette	  |    13200	  | 3		    |			   |

| Module	  |		  |		    |			   |

|		  |		  |		    |			   |

|Cassette Label|    12400	  | 3		    | Requires cassette	   |

|Support Module|		  |		    | handler.		   |

|		  |		  |		    |			   |

| OS/8 File    | 04600 if OS/8 | 6		    | Requires a mass	   |

|Support Module| support       |		    | storage handler.	   |

|		  | present,	  |		    | Must run in field 0. |

|		  |otherwise 06200|		    |			   |

|		  |		  |		    |			   |

| OS/8 Support |    06200	  | 6		    | Must run in field    |

| Task	  |		  |		    | 0 - requires 8K for  |

|		  |		  |		    | OS/8 plus a mass     |

|		  |		  |		    | storage handler.     |

|		  |		  |		    | Uses page	0	   |

|		  |		  |		    | locations 164-177.   |

|		  |		  |		    |			   |

| Monitor 	  | 17600 minus   | 5+3 (if clock   | Requires "console"   |

| Console	  | length (15200 | functions 	    | terminal handler	   |

| Routine Task | with all	  | desired) +	    | and page 0	   |

|		  | options)	  | MCRSYS +	    | locations 100-117    |

|		  |		  | (NTASKS+40)/64. |			   |

|		  |		  |		    |			   |

-------------------------------------------------------------------------

B-2

GLOSSARY

Accumulator:   the  register  in  which  arithmetic   operations   are

performed (abbreviated AC).

Analog:  representation of information by continuous variables.

Analog Channel:  a UDC functional device.

Argument:  a variable or constant which is given  in  the  call  of  a

subroutine   as information to it a variable upon whose value the

value of a function depends; the known reference factor necessary

to find an item in a table or array (i.e., the index).

Assembler:  a program which translates symbolic op-codes into  machine

language   and   assigns   memory  locations  for  variables  and

constants.

Auto-index Register: whenever one of the absolute locations from  0010

through  0017  in  any  memory field is addressed indirectly, the

contents of that location is incremented by one, rewritten in the

same  location,  and used as the effective address of the current

instruction.

Auto-restart:  the ability to start the CPU automatically on power-up.

Baud:  a unit of measure of data flow (one bit per second).

Bit:  a binary digit (each PDP-8 word is composed of 12 bits).

Bit Map:  a method of keeping track of used  and  unused  entities  by

assigning one bit in a table to each entity.

Block Gap:  the blank space between blocks on a recording medium.

Block:  a set of  consecutive  machine  words,  characters  or  digits

handled  as  a unit, particularly with reference to I/O;  also to

inhibit a process from continuing.

Blocking Bits:  bits in an RTS-8 Monitor table  which  specify  why  a

given task is blocked.

Byte:  a group of binary digits usually operated upon as a unit.

Cassette:  a magnetic tape device used for program and data storage.

Central Processing Unit:  the unit of a computing system that includes

the circuits controlling  the  interpretation  and  execution  of

instructions; also central processor (abbreviated CPU).

Clear:  to erase the contents of a storage location by  replacing  the

contents, normally with zeroes.

Clock:   a  time-keeping  or  frequency-measuring  device  within  the

computer system.

Communication Region:   locations  20-27  of  every  field,  used   to

simplify passing executive requests across field boundaries.

Compute Bound:  requiring extensive (or total) use of the CPU relative

to other hardware elements (such as I/O devices).

Configuration:  the number and types of hardware present on a system.

Contact Channel:  a UDC functional device.

Contiguous:  code which resides in core immediately adjacent to  other

sections of code.

Controller:  the circuitry which controls a device.

Counter Channel:  a UDC functional device.

GLOSSARY-1

Core Image File:  a file in core image format (i.e.,  a  'picture'  of

core).

Core Storage:  the main high-speed storage  of  a  computer  in  which

binary  data is represented by the switching polarity of magnetic

cores.

CPU Registers:   high-speed  circuitry  used  to   store   information

affecting the operation of the CPU (e.g., PC, IF, DF).

Data:  a general term used  to  denote  any  or  all  facts,  numbers,

letters  and  symbols.  It connotes basic elements of information

which can be processed or produced by computer.

Data Field:  a 3-bit register which determines the memory  field  from

which  operands  are  taken  in indirectly addressed instructions

(abbreviated DF).

Debug:  to detect, locate and correct mistakes in a program.

Deferred Actions:  actions which are considered low-priority  and  are

not performed until higher-priority actions are serviced.

Deferred Requests:  requests which are considered low-priority and are

serviced after high-priority requests.

Derail:  to transfer control or execution of a  specified  task  to  a

subroutine.

Device Codes:  numbers assigned to each device in the system.

Device Status Register:  a register which contains the current  status

of a device.

Digital:  representation of information by discrete units.

Digital Channel:  a UDC functional device.

Driver:  see Handler.

Dynamic:  pertaining to a quantity that is affected by some  condition

(such as time) and is therefore relative to the condition.

Entry Point:  the location in  a  routine  to  which  control  can  be

transferred and execution begun.

Event Flag:  the location which contains the status of an event (event

being either a result of some operation, or a physical

occurrence).

Executive:  the program which controls the execution of other programs

or routines.

Executive Requests:  means of  communication  between  tasks  and  the

RTS-8 Executive.

Field:  a division of memory on a PDP-8 computer  referring  to  a  4K

section of core.

File Gap:  a  fixed  length  of  blank  tape  separating  files  on  a

recording  medium;   generally  several times the size of a block

gap.

Functional Devices:  the devices  available  for  use  under  the  UDC

handler.

Gain:  an increase in signal power.

Generic Codes:  codes used to identify which type  of  UDC  functional

device  caused  an  interrupt  and  to  direct program control to

service routines.

Handler:  a routine which is designed to control the  operation  of  a

device.

GLOSSARY-2

I/O Bound:  a condition in which a process is performing much I/O  but

using very little CPU time.

Indirect  Address:  an  address  in  a  computer   instruction   which

indicates  a location where the address of the referenced operand

is to be found.

Initialization  Code:   code  which  sets  counters,   switches,   and

addresses to zero or other starting values at the beginning of or

at prescribed points in a computer routine.

Input Buffer:  a section of memory used for storage of input data.

Instruction Field:  a register which holds  the  contents  determining

from  which field the operand of a directly addressed instruction

should be taken (abbreviated IF).

Interactive:  highly responsive to real-world inputs.

Interface:  the common boundary between two devices or systems.

Interlock Scheme:  arranging the control  of  devices  so  that  their

operation is interdependent.

Interrupt:  a break  in  execution  caused  by  some  external  event;

execution is usually resumed at a later time.

Interrupt Processing Module:  a routine which acts upon  the  external

event which caused an interrupt.

Interval Queue:  a list of actions to perform, each accompanied by the

interval  of  time which is to elapse from the previous action to

the current one.

Line-frequency Clock:  a clock which determines when to tick according

to the state of the current flowing in a power-supplying wire.

Link:  a one-bit register in the PDP-8; an address  pointer  generated

automatically by PAL8 or MACRO-8 assemblers to indirectly address

an off-page symbol.

Logical OR:  a logical function of two inputs which is  true  whenever

either input is true.

Loop:  a sequence of instructions that is executed repeatedly until  a

terminal condition prevails.

Mass Storage Device:  a device such as disk or  DECtape  which  stores

large   amounts   of  data  readily  accessible  to  the  central

processing unit.

Master Parameter File:  a file included in the distributed sources  of

RTS-8  which the user can edit to indicate parameters specific to

his system configuration.

Memory Address:  a register which holds the  address  specified  by  a

memory reference instruction.

Message:  a contiguous area of memory which contains information about

execution of tasks.

Message-driven:  an RTS-8 task is called  message-driven  if  it  only

executes in response to messages received from other tasks.

Mnemonic:  alphabetic representation of a function  or  octal  machine

instruction.

Module:  a routine which handles a particular function.

Monitor Console Routine:  the Monitor routine which provides the  user

with functions which allow him to control, inspect, and debug his

system.

Multiplier Quotient:  a 12-bit register used in conjunction  with  the

accumulator to perform mathematical operations (abbreviated MQ).

Multi-programming:  two or more programs in  core  at  the  same  time

which  execute  alternately depending on the current state of the

system.

GLOSSARY-3

No-op:  no operation occurs; control proceeds to the next  instruction

in sequence.

Null Characters:  characters with ASCII code 000.

Overflow:  a condition  that  occurs  when  a  mathematical  operation

yields  a  result  whose magnitude is greater than the program is

capable of handling.

Pack:  to conserve memory by combining information.

Page:  a 128-word section of PDP-8 core memory beginning at an address

which is a multiple of 200.

Parameter file:  a file used to record arguments which may be assigned

different values.

Parity Bit:  a bit which indicates whether the total number of  binary

one digits in a word is even or odd.

Pointer:  a word containing the address of another word in memory.

Posted:  one of the states of an event flag, indicating that the event

is complete.

Power-fail:  an interruption of power to the computer.

Priority  Scheme:   a  scheme  by  which  certain  operations  or  the

execution of a set of instructions is given preference over other

operations.

Program Counter:  a register which contains the address  of  the  next

instruction to be executed (abbreviated PC).

Prompting Character:  a character which prints on the console terminal

and cues the user to perform some action.

Queue:  a  waiting  list  (e.g.,  a  queue  of  programs  waiting  for

processing time).

Ready Flag:  a bit which a device controller sets when it is ready  to

accept commands from the CPU.

Real-time System:  a system in which computation is performed while  a

related  physical process is occurring so that the results of the

computation can be used in guiding the physical process.

Receiver:  the task which has received a  message  from  another  task

(sender).

Record:  a collection of related items of data treated as a unit.

Record Header Area:  an area of at least  40  words  in  length  which

contains information necessary to perform cassette operations.

Ring-buffer:   a  storage  area  for  data  accessed  on  a  first-in,

first-out basis.

Scheduling:  the operation of sharing resources among computing tasks.

Sender:  the task which has sent a message to another task (receiver).

Sign Bit:  the bit which contains the sign of a number.

Simulate:  to represent the function of a device,  system  or  program

with another device, system or program.

Skip Chain:  an instruction sequence which determines the source of an

interrupt request on a PDP-8.

State:  a complete description of the condition of a piece of hardware

or of a task.

Status:  that portion of the state which other devices or tasks  might

be interested in.

Subchannel:  1 of the 4 channels of a UDC analog functional device.

Suspend:  to temporarily halt execution of a task while  another  task

of higher priority runs.

GLOSSARY-4

Symbols:  names which can be assigned values or which can be  used  to

indicate specific locations in a program.

Synchronization:  a means of processing tasks (through event flags) so

that one task executes while others wait.

System Ticks:  an RTS-8 convention designed to obviate  the  tasks  in

the system from knowing the frequency of the clock.

Task:  a routine which performs a specific function.

Task Flags Table:  a table of l-word entries in  the  RTS-8  Executive

whose contents determine whether or not a task is runnable.

Task Input Message Queue Header Table:   a  table  of  2-word  entries

which  represent  the  head  of  the input message queue for each

task.

Task Number:  a unique number between 1 and 63 assigned to  each  task

in an RTS-8 system.

Task State Table:  a table of 4-word entries which contains  the  most

recent contents of CPU registers for each task.

Task Switching:  the  act  of  stopping  execution  of  one  task  and

continuing execution of another from the point that it was last

stopped.

Task Synchronization:  logically ordering the execution of tasks.

Terminal:  a peripheral device in a  system  through  which  data  can

enter or leave the computer.

Utilities:  routines to perform non-Monitor related functions.

Word:  in the PDP-8, a 12-bit unit of data which may be stored in  one

addressable location.

GLOSSARY-5

INDEX

Address, 5-1 			   POst, 5-6

Advanced programming techniques,	   REquest, 5-3

3-11				   STop, 5-4

AI (Analog Input), 4-11		   SYstat, 5-4

ALTMODE character, 4-3, 5-1	   TIme, 5-2

Analog I/O, 4-11			Component sizes, B-1

> (angle bracket) MCR response,	Console terminal handler, 4-3

5-1				Console terminal module, B-1

AO (Analog Output), 4-11		Core storage, 2-3, 6-3

Assembler code, 6-2		Creating a real-time system, 6-2

Assembling and loading tasks, 6-1	CS (Change of State), 4-15

Assembly parameters, UDC, 4-16 	CTRL/C, 4-3, 5-1

* (asterisk), 			CTRL/U, 4-3

at end of line, 5-5 		CUR symbol, 6-2

Auto-index registers, 6-2

Auto-restart hardware option, 4-7

Data Field, 3-10

DAte command, 5-2

DC (Disable Counter), 4-14

BITMAP, 6-3			DCT (Disable Contacts), 4-15

BLKARG ER (Block Task for speci-	DECassettes, 4-17

fied reason), 3-6		Definitions, 6-1

Deletion of a line or character, 4-3

Demonstration program, 7-1

CAL, 3-1				DEposit command, 5-6

CAncel command, 5-4		DERAIL ER (Derail a Task's Execu-

Carriage return, 4-3, 5-1		   tion), 3-8

Cassette handler, 4-17		DF32 fixed-head disk, 4-5

Cassette Label Support handler,	DF32/RF08 module, B-1

4-18				DI (Digital Input), 4-12

Cassette Label Support module, B-2 Digital I/O, 4-12

Cassette module, B-2		Direct references to System Tables,

Character mode input, 4-3		   3-12

Character punch subroutine code, 	DIsable command, 5-4

3-11 				Distribution tape, A-1

Clear Wait bits, 3-6 		DO (Digital Output), 4-12

Clock Handler, 4-1, 4-2 		Does Not Exist Wait, (DNEWT), 3-5

Clock Handler calls, examples of,

4-2

Clock module, B-1 			EC (Enable Counter), 4-13

Clock queue entries cancelled, 5-4 ECT (Enable Contacts), 4-14

CLOSE, 4-18, 4-19			Emergency condition signals, 3-8

Comma, 5-1				ENable command, 5-4

Command arguments (MCR), 5-1	Enable Wait, (ENABWT), 3-5

Commands, MCR			ENTER, 4-18, 4-19

CAncel, 5-4			ER example, 3-3

DAte, 5-2			ER (Executive Request), 3-1

DEposit, 5-6			Error conditions, UDC, 4-16

DIsable, 5-4			Errors, MCR, 5-6

ENable, 5-4			Event Flag, 2-3

EXamine, 5-6			Event Flag Wait, (EFWT), 3-5

EXIT, 5-6			Event or Message Wait, (EORMWT), 3-5

NAme, 5-2			EXamine command, 5-6

OPen, 5-5

INDEX-1

Examples,				Logical OR, 3-12

Clock handler call, 4-2		LOOKUP, 4-18, 4-19

ER, 3-3

Mass Storage handler call, 4-7

SUSPND and RUN ERs, 3-7		Mass storage handlers, 4-5

Terminal handler messages, 4-5	   call, 4-7

Executive, 2-1, B-1		MCR command arguments, 5-1

Executive Requests (ERs), 3-1	MCR errors, 5-6

Executive specifications, 6-1	MCR name table, 5-3

EXIT command, 5-6			Memory address, 5-1

Message, cassette, 4-18

Message Header, 2-3

Fields, 3-10, 4-8			Message Wait, (MSGWT), 3-5

FINISHED value, 2-2		Messages, intertask, 2-3

Fixed priority, 1-1		Messages to terminal handler, 4-3

Flags Table, 2-1			Monitor Console Routine (MCR), 5-1

Monitor Console Routine task, B-2

Multiple terminals, 4-3

GC (Generic Code), 4-12		Multiplier Quotient (MQ), 3-10

Multiprogramming, 1-1

Handlers, 4-1

Cassette, 4-17			NAme command, 5-2

Cassette Label Support, 4-18	Non-console Terminal module, B-1

Clock, 4-1			Null characters, 4-8

Console terminal, 4-3

Line Printer, 4-5

Mass Storage, 4-5		OPen command, 5-5

Terminal, 4-3			OR of several Event Flags, 3-12

UDC, 4-10			OS/8 File Support module, B-2

Hardware clocks, 4-1		OS/8 File Support Task, 4-9

Header area, cassette, 4-18	OS/8 Support Task, 4-8

Initialization code, 3-10		Page zero locations, 6-2

INIWT symbol, 6-2			Parameter file, 6-1

I/O, analog and digital, 4-10	PENDING value, 2-2

Input Message Queue, 2-3, 3-2	POst command, 5-6

Input Message Queue Header, 3-3	POSTDS instruction, 3-10

Input Message Queue Header Table,	POST ER (Post Event Flag), 3-3

2-2				Posting, 2-3

Input mode, 4-3			Power Fail Task, 4-7

Instruction field, 3-10		Power Fail module, B-2

Interrupts, 3-9			Priorities, 1-1, 2-1

Interrupt skip chain code, 3-11	Program Counter (PC), 3-1

Intertask control, 3-5

Intertask messages, 2-3

Race condition, 3-12

RC (Read Counter), 4-14

Keyboard IOT code, 4-8		Real-time systems, 1-1

RECEIVE ER (Receive Message), 3-2

Record size of cassettes, 4-17

LA30 terminals, 4-8		Reentrant programs, 3-1

Line mode input, 4-3		REquest command, 5-3

Line Printer handler, 4-5		RF08 fixed-head disk, 4-5

Line printer module, B-1		RK8 disk module, B-2

Loading tasks, 6-1			RK8E disk module, B-1

INDEX-2

RK8 moving-head disks, 4-5		TSS-8 timesharing hardware option,

RK8E moving-head disk, 4-5		   4-8

RUBOUT, 4-3

RUN ER (Run a Task), 3-7

Run Wait, (RUNWT), 3-5		UDC assembly parameter, 4-16

UDC Error conditions, 4-16

UDC handler, 4-10

Scheduling tasks, 5-3		UDC assembly parameter, 4-16

SEND ER (Send Message), 3-1	UDC Error conditions, 4-16

SENDW ER (Send and Wait), 3-2	UDC handler, 4-10

Set Wait bits, 3-6			UDC module, B-2

Sizes of components, B-1		UNBARG ER (Unblock Task for

SKPINS ER (Insert Code into Skip	   Specified Reason), 3-6

Chain), 3-9			User Wait, (USERWT), 3-6

Slash (/) character, 6-1		Using Interrupts, 3-9

Source files, A-1			Utility function, cassettes, 4-17

Space, 5-1

Starting address, 6-3

START symbol, 6-2			VT05 terminals, 4-8

State Table, 2-1

STop command, 5-4

Structure of RTS-8 tasks, 2-1	Wait Bits, 3-6

SUSPND ER (Suspend a Task's	WAITE ER (wait on Event Flag), 3-2

Execution), 3-6, 3-7 		WAITING state, 2-3

Swap Wait, (SWPWT), 3-5 		WAITM (Waiting for Multiple Event

Symbolic names, 6-1		   Flags), 3-11

Synchronization of tasks, 2-2	Word, 5-1

SYstat command, 5-4

System parameter file, 4-8

system tables, direct references

to, 3-12

System tasks, 4-1

System task specifications, 6-1

System Wide definitions, 6-1

Task (definition of), 1-1

Task definitions, 6-1

Task Flags Table, 2-1

Task Flags Table entry, 3-5

Task-ID, 5-1

Task Input Message Queue Header

Table, 2-2

Task number, 2-1

Task setup, 6-2

Task State Table, 2-1

Task switching, 3-1

TASK symbol, 6-2

TC08 DECtape, 4-5

TC08 DECtape module, B-1

Teleprinter IOT code, 4-8

Terminals, 4-8

Terminal handler, 4-3

Ticks, 4-1

TIme command, 5-2

Time-of-day, 4-2, 5-1

INDEX-3

HOW TO OBTAIN SOFTWARE INFORMATION

SOFTWARE NEWSLETTERS, MAILING LIST     __________________________________

The Software Communications Group, located at corporate headquarters in

Maynard, publishes newsletters and Software Performance Summaries (SPS)

for the various Digital products.  Newsletters are published monthly,

and contain announcements of new and revised software, programming

notes, software problems and solutions, and documentation corrections.

Software Performance Summaries are a collection of existing problems

and solutions for a given software system, and are published periodi-

cally.  For information on the distribution of these documents and how

to get on the software newsletter mailing list, write to:

Software Communications

P. O. Box F

Maynard, Massachusetts 01754

SOFTWARE PROBLEMS     _________________

Questions or problems relating to Digital's software should be reported

to a Software Support Specialist.  A specialist is located in each

Digital Sales Office in the United States.  In Europe, software problem

reporting centers are in the following cities.

Reading, England 	    Milan, Italy

Paris, France 	    Solna, Sweden

The Hague, Holland      Geneva, Switzerland

Tel Aviv, Israel 	    Munich, West Germany

Software Problem Report (SPR) forms are available from the specialists

or from the Software Distribution Centers cited below.

PROGRAMS AND MANUALS     ____________________

Software and manuals should be ordered by title and order number.  In

the United States, send orders to the nearest distribution center.

Digital Equipment Corporation     Digital Equipment Corporation

Software Distribution Center      Software Distribution Center

146 Main Street 		    1400 Terra Bella

Maynard, Massachusetts 01754 	    Mountain View, California 94043

Outside of the United States, orders should be directed to the nearest

Digital Field Sales Office or representative.

USERS SOCIETY     _____________

DECUS, Digital Equipment Computer Users Society, maintains a user

exchange center for user-written programs and technical application

information.  A catalog of existing programs is available.  The society

publishes a periodical, DECUSCOPE, and holds technical seminars in the

United States, Canada, Europe, and Australia.  For information on the

society and membership application forms, write to:

DECUS			    DECUS

Digital Equipment Corporation    Digital Equipment, S.A.

146 Main Street		    81 Route de l'Aire

Maynard, Massachusetts 01754     1211 Geneva 26

Switzerland