RTS8V3.DOC

DISCLAIMER

This document file was created by scanning  the

original  document and then editing the scanned

text.  As much as possible, the  original  text

format  was restored.  Some format changes were

made to insure this  document  would  print  on

current laser printers using 60 lines per page.

The  original  spelling  and  grammar  has been

preserved.

1-Sep-1996

RTS/8

User's Manual

Order No. AA-0724D-TA

February 1979

ABSTRACT

This manual describes version 3 of RTS/8, the real-time

operating system for the PDP-8 which allows up to 127

tasks to run concurrently. Described are the Executive

Requests which permit intertask communication, the Monitor

Console Routine which permits the user to control task

execution from a terminal, and the message formats for

DIGITAL supplied tasks.

SUPERSESSION/UPDATE INFORMATION: This manual supersedes DEC-08-ORTMA-C-D.

OPERATING SYSTEM AND VERSION:    RTS/8, V3

--------------------------------------------------------

| To order additional copies of this document, contact |

|  the Software Distribution Center, Digital Equipment |

|  Corporation, Maynard, Massachusetts 01754           |

--------------------------------------------------------

digital equipment corporation - maynard, massachusetts

First Printing, February 1979

The information in this document is subject to change without notice

and should not be construed as a commitment by Digital Equipment

Corporation. Digital Equipment Corporation assumes no responsibility

for any errors that may appear in this document.

The software described in this document is furnished under a license

and may only be used or copied in accordance with the terms of such

license.

No responsibility is assumed for the use or reliability of software on

equipment that is not supplied by DIGITAL or its affiliated companies.

Copyright (c) 1979 by Digital Equipment Corporation

The postage-prepaid READER'S COMMENTS form on the last page of this

document requests the user's critical evaluation to assist us in

preparing future documentation.

The following are trademarks of Digital Equipment Corporation:

DIGITAL             DECsystem-10        MASSBUS

DEC                 DECtape             OMNIBUS

PDP                 DIBOL               OS/8

DECUS               EDUSYSTEM           PHA

UNIBUS              FLIP CHIP           RSTS

COMPUTER LABS       FOCAL               RSX

COMTEX              INDAC               TYPESET-8

DDT                 LAB-8               TYPESET-11

DECCOMM             DECSYSTEM-20        TMS-11

ASSIST-11           RTS-8               ITPS-10

VAX                 VMS                 SBI

DECnet              IAS                 PDT

DATATRIEVE          TRAX

3/79-15

CONTENTS

Page

PREFACE                                                           vii

CHAPTER  1         INTRODUCTION                                   1-1

1.1       RTS/8 DESCRIPTION                              1-1

1.2       REAL-TIME OPERATION                            1-3

CHAPTER  2         USING TASKS UNDER RTS/8                        2-1

2.1       RTS/8 TASKS                                    2-1

2.2       CONCEPTS OF TASK COMMUNICATION                 2-1

2.2.1       Task Synchronization                         2-2

2.2.2       Intertask Messages                           2-4

2.3       PRIORITY                                       2-5

2.4       TASK INTERRUPTS AND SCHEDULING                 2-6

2.5       CREATING RTS/8 TASKS                           2-6

2.6       RUNNING AN RTS/8 SYSTEM                        2-8

CHAPTER  3         RTS/8 EXECUTIVE REQUESTS                       3-1

3.1       COMMUNICATION WITH THE RTS/8 EXECUTIVE         3-1

3.2       EXECUTIVE REQUESTS                             3-4

3.3       EXAMPLE OF EXECUTIVE REQUESTS FOR MESSAGE

AND EVENT FLAGS                                3-28

3.4       EXECUTIVE REQUEST WAIT STATES                  3-30

CHAPTER  4         RTS/8 SYSTEM TASKS                             4-1

4.1       RTS/8 SYSTEM TASK SUMMARY                      4-1

4.2       CASSETTE HANDLER                               4-3

4.2.1       Handler Function                             4-3

4.2.2       Utility Function Handler                     4-6

4.3       CASSETTE FILE SUPPORT HANDLER                  4-8

4.4       CLOCK HANDLER                                  4-11

4.5       CONSOLE AND NONCONSOLE TERMINAL HANDLERS       4-16

4.5.1       EMT Task                                     4-21

4.6       EXIT TASK                                      4-23

4.7       FLOPPY DISK HANDLER                            4-24

4.8       LINCTAPE HANDLER                               4-29

4.9       LINE PRINTER HANDLER                           4-32

4.10      MASS STORAGE HANDLERS                          4-34

4.11      OS/8 FILE SUPPORT TASK                         4-38

4.12      OS/8 SUPPORT TASK                              4-40

4.12.1      OS/8-RTS/8 Communication (OS8COM)            4-41

4.12.2      Other Techniques                             4-43

4.13      PDP-8A NULL TASK                               4-44

4.14      POWER FAIL TASK                                4-45

4.15      UDC/ICS HANDLER                                4-46

4.15.1      AI (Analog Input)                            4-48

iii

CONTENTS (Cont.)

Page

4.15.2      AO (Analog Output)                           4-49

4.15.3      CS (Change of State)                         4-50

4.15.4      DI (Digital Input)                           4-51

4.15.5      DO (Digital Output)                          4-51

4.15.6      DCT (Disable Contacts)                       4-52

4.15.7      DC (Disable Counter)                         4-53

4.15.8      ECT (Enable Contacts)                        4-54

4.15.9      EC (Enable Counter)                          4-55

4.15.10     GC (Generic Code)                            4-56

4.15.11     RC (Read Counter)                            4-57

4.15.12     UDC/ICS Status/Error Conditions              4-58

CHAPTER  5         MONITOR CONSOLE ROUTINE                        5-1

5.1       MCR FUNCTIONS                                  5-1

5.2       MCR COMMAND SYNTAX                             5-1

5.3       MCR COMMANDS                                   5-2

5.3.1       CANCEL                                       5-3

5.3.2       DATE                                         5-3

5.3.3       DEPOSIT                                      5-4

5.3.4       DISABLE                                      5-4

5.3.5       ENABLE                                       5-5

5.3.6       EXAMINE                                      5-5

5.3.7       EXIT                                         5-5

5.3.8       NAME                                         5-5

5.3.9       OPEN                                         5-6

5.3.10      POST                                         5-6

5.3.11      REQUEST                                      5-7

5.3.12      STOP                                         5-8

5.3.13      SYSTAT                                       5-8

5.3.14      TIME                                         5-10

5.3.15      MCR Errors                                   5-10

CHAPTER  6         NONRESIDENT TASKS                              6-1

6.1       NONRESIDENT TASK DESCRIPTION                   6-1

6.1.1       Checkpointable Tasks                         6-2

6.1.2       Modified Tasks                               6-3

6.1.3       Interaction Between Tasks                    6-3

6.2       MEMORY PARTITIONS                              6-3

6.2.1       FREE Command                                 6-4

6.2.2       Writing Nonresident Tasks                    6-5

CHAPTER  7         PROGRAMMING TECHNIQUES                         7-1

7.1       OVERVIEW                                       7-1

7.2       PERFORMING A RESCHEDULE                        7-1

7.2.1       Writing Delicate Code                        7-1

7.2.2       Inhibiting Task Switching                    7-3

7.3       MULTIPLE EVENT FLAG WAITS                      7-5

7.4       EXECUTIVE INTERNAL TASK TABLES                 7-6

7.4.1       Task State Table (TSTABL)                    7-7

7.4.2       Task Flags Table (TFTABL)                    7-8

iv

CONTENTS (Cont.)

Page

7.4.3       Task Input Message Queue Header Table

(MSGTBL)                                     7-10

7.4.4       Residency Table (RESTBL)                     7-10

7.4.5       Partition Table (PARTBL)                     7-11

7.4.6       Task Table Structure                         7-11

7.4.7       Direct References to System Tables           7-14

7.5       KL8-A SUPPORT                                  7-15

APPENDIX A         RTS/8 DISTRIBUTED SOURCE FILES AND MNEMONICS   A-1

APPENDIX B         RTS/8 FLOWCHARTS                               B-1

APPENDIX C         GLOSSARY                                       C-1

INDEX                                                         Index-1

FIGURES

FIGURE   2-1       Message Format and Linking of Messages         2-4

4-1       Cassette Handler Unit Word Format              4-4

4-2       Cassette Handler Call Function Word Format     4-5

4-3       Cassette Handler Status Return Word Format     4-5

4-4       Cassette Handler Utility Call                  4-7

4-5       Cassette File Support Handler Unit Word

Format                                         4-10

4-6       Clock Handler Command Word Format              4-14

4-7       Terminal Handler I/O Mode Command and

Length Word Format                             4-20

4-8       Terminal Handler ASSGN Mode Command Word

Format                                         4-20

4-9       Floppy Disk Handler Unit Word Format           4-27

4-10      Floppy Disk Handler Status Word Format         4-27

4-11      LINCtape Handler Unit Word Format              4-31

4-12      Line Printer Handler I/O Mode Command and

Length Word Format                             4-33

4-13      Line Printer Handler ASSGN Mode Command

Word Format                                    4-33

4-14      Mass Storage Handlers Unit Word Format         4-36

4-15      Mass Storage Handlers Function Word Format     4-37

4-16      OS8F Call Function Word                        4-39

4-17      UDC/ICS Handler Subchannel and Gain Word

Format                                         4-48

4-18      UDC/ICS Handler Subchannel and Value Word

Format                                         4-49

6-1       Nonresident Task Implementation                6-2

7-1       Executive Internal Task Table Structure        7-12

TABLES

TABLE    1-1       RTS/8 System Tasks                             1-2

2-1       Summary of Event Flag States                   2-3

v

CONTENTS (Cont.)

Page

TABLES (Cont.)

TABLE    3-1       Summary of Executive Requests                  3-2

3-2       Symbolic Names for Specifying WAITBITS         3-5

3-3       Summary of Wait States Incurred by Executive

Requests                                       3-31

4-1       Cassette Handler Status Register Bits          4-4

4-2       Cassette Header Formats                        4-9

4-3       Clock Handler Commands                         4-12

4-4       Terminal Handler Commands                      4-18

4-5       Floppy Status Bit Descriptions                 4-26

4-6       OS/8 Floppy Disk Interleave Scheme             4-28

4-7       UDC/ICS Operations                             4-47

4-8       Generic Code Values                            4-57

4-9       UDC/ICS Messages                               4-58

5-1       MCR Command Summary                            5-2

7-1       Task Switch Flag (TSWFLG) States               7-5

7-2       Waitbit Descriptions                           7-8

vi

PREFACE

This manual describes the PDP-8 Real-Time  Operating  System  (RTS/8).

RTS/8  is a system that you generate using the RTS/8 system generation

procedures.  The system that you  create  is  individualized  to  your

needs.   Knowledge  of  PDP-8 assembly language programming (MACREL or

PAL8) is essential for a complete understanding of  this  manual.   In

addition, you should be familiar with real-time systems in general and

with the operation and use of the development system  for  the  PDP-8,

OS/8.

This manual only describes how you use  the  RTS/8  operating  system.

See  the  RTS/8  System  Generation  Manual  for information on how to	       _________________________________

generate this operating system.

Chapters 1 and 2 present introductory information.  If you have  never

used  RTS/8  before,  these chapters present sufficient information so

that you can place the information in the  remainder  of  this  manual

into  a  framework.  Also, you should carefully read Section 2.5 which

overviews the procedure which  you  should  use  when  creating  RTS/8

tasks.

Chapter 3 documents RTS/8 Executive Requests.   These  requests  serve

two  functions:  they  are  the  means by which tasks communicate with

each other and they are the means by which a  task  communicates  with

the RTS/8 Executive.

Chapter 4 discusses the message formats that user tasks must use  when

they request a DIGITAL-supplied handler to perform an I/O operation.

Chapter 5 examines the RTS/8 Monitor Console  Routine.   This  routine

allows you to communicate interactively with an RTS/8 system.

Chapter 6 describes non-resident tasks.  A non-resident task is a task

which the Executive brings into memory when it is executable.  You use

non-resident tasks when memory space is at a premium because more than

one non-resident task can share the same part of memory.

The information in Chapter 7,  "Programming  Techniques"  is  for  the

experienced  PDP-8  user.   You  should  read  it  only  after you are

familiar with RTS/8.

vii

The following PDP-8 manuals will be helpful for review and reference:

Introduction to Programming (DEC-08-XINPA-A-D)	  ___________________________

OS/8 Handbook (DEC-S8-OSHBA-A-D)	  _____________

MACREL/LINK User's Manual (AA-5664B-TA)	  _________________________

Small Computer Handbook (90P45)	  _______________________

PDP-8A Minicomputer Handbook (EB0621976)	  ____________________________

UDC8 Universal Digital Control Subsystem Maintenance Manual	  ___________________________________________________________

(46H745)

ICS8 Industrial Control Subsystem Maintenance Manual (EKOICS8MM)	  ____________________________________________________

This manual describes how you use an RTS/8 system if  it  was  created

using  the  RTS/8  system  generation procedures as this procedure was

distributed by DIGITAL.  Because it is possible to modify  the  System

Generation procedure, the RTS/8 system that is created may differ from

the generalized RTS/8 system that this manual  describes.   Therefore,

you must determine if there are any differences between the system you

are using and that which the DIGITAL System Generation  procedure  can

create.

viii

CHAPTER 1

INTRODUCTION

1.1  RTS/8 DESCRIPTION

RTS/8 is a compact Real-Time System for the PDP-8 family of processors

(except  for  the  PDP-8S).   This  system allows you to run up to 127

tasks concurrently, each competing for system resources.

As do other real-time systems, RTS/8 responds  to  both  internal  and

external  events.  These events permit timely execution and scheduling

of tasks.  Thus, you may use RTS/8 for monitoring  and  controlling  a

number of processes.

The RTS/8 Executive controls the  execution  and  interaction  of  all

tasks.   This  Executive  decides  which  tasks should run (basing its

decisions on the priorities of any  other  runnable  task  or  tasks),

services these tasks by means of Executive Requests (which are special

processing routines within the Executive), and administers interactive

communication.

RTS/8 includes system tasks that control most  standard  I/O  devices,

including  RK8,  RK8E,  and  RL01  moving-head  disks,  DF32  and RF08

fixed-head  disks,  TC08  DECtape,   RX8   floppy   disks,   LINCtape,

DECcassette,  and  LS8,  LS8E, LP8, and LV8 line printers.  (Note that

RTS/8 does not  support  TD8E  DECtape.)  These  tasks,  described  in

Chapter 4, are listed in Table 1-1.

The Monitor Console Routine (see Chapter  5),  provides  you  with  an

interface  between  the  console terminal and the system.  The Monitor

Console  Routine  accepts  a  series  of  interactive  commands  which

control,  inspect,  and,  to  some  extent, debug the system.  It also

allows you to schedule  and  execute  tasks  at  specified  intervals,

suspend task execution, and print system status information.

RTS/8 also contains a system task that allows a  single  copy  of  the

OS/8  operating system to run in the background.  Thus, you may create

a real-time  foreground-OS/8  background  system.  With  OS/8  in  the

background, you may assemble, debug, edit, and link programs.

1-1

INTRODUCTION

Table 1-1

RTS/8 System Tasks

----------------------------------------------------------------------

| Task Name | File Name |		  Task Function			  |

|-----------|-----------|--------------------------------------------|

| _       	 | RTS8.MA   | RTS/8 Executive 				  |

| CLOCK     | CLOCK.MA  | Clock Handler Task			  |

| CSA       | CSA.MA    | Cassette Driver Task			  |

| CSAF      | CSAF.MA   | Cassette File Support Task		  |

| DTA       | DTA.MA    | TC08 DECtape Driver Task			  |

| EXIT      | EXIT.MA   | Exit Task				  |

| LPT       | LPT.MA    | Line Printer Driver Task			  |

| LTA       | LTA.MA    | LINCtape Driver Task			  |

| MCR       | MCR.MA    | Monitor Console Routine			  |

| null task | MCR.MA    | Null Task (contained within MCR)		  |

| NULL8A  	 | NULL8A.MA | Null Task for PDP-8A			  |

| OS8       | OS8.MA    | OS/8 Support Task			  |

| OS8F      | OS8F.MA   | OS/8 File Support Task			  |

| PWRF      | PWRF.MA   | Power Fail Task				  |

| RF08/DF32 | RF08.MA   | RF08/DF32 Fixed-Head Driver Task		  |

| RK8       | RK8.MA    | RK8 Disk Driver Task			  |

| RK8E      | RK8E.MA   | RK8E Disk Driver Task			  |

| RL01      | RL01.MA   | RL01 Disk Driver Task			  |

| RX8A      | RX.MA     | Floppy Disk Handler (1st Controller)	  |

| RX8B      | RX.MA     | Floppy Disk Handler (2nd Controller)	  |

| RX8C      | RX.MA     | Floppy Disk Handler (3rd Controller)	  |

| RX8D      | RX.MA     | Floppy Disk Handler (4th Controller)	  |

| SWAP      | SWAP.MA   | Nonresident Task Swapper			  |

| TTY       | TTY.MA    | Terminal Driver Task (1st Driver)	  |

| TTYl      | TTY.MA    | Terminal Driver Task (2nd Driver)	  |

| TTY2      | TTY.MA    | Terminal Driver Task (3rd Driver)	  |

| TTY3      | TTY.MA    | Terminal Driver Task (4th Driver)	  |

| TTY4      | TTY.MA    | Terminal Driver Task (5th Driver)	  |

| UDC/ICS   | UDCICS.MA | Universal Digital Controller 		  |

|           |           | Industrial Controller Subsystem		  |

|           |           | Handler task 				  |

|--------------------------------------------------------------------|

|									  |

|                   	     NOTE				  |

|									  |

|		    TTYl-4  and  RX8A-D  are  not  on  the 		  |

| 		    distribution Medium.  They are files which		  |

|		    the RTS/8 System Generation Program			  |

|		    creates.						  |

----------------------------------------------------------------------

RTS/8 supports both resident and nonresident tasks.  (A resident  task

is  a  task that resides permanently in memory;  a nonresident task is

one in which a portion of the task resides on a mass storage  device.)

The  Executive loads a nonresident task into memory only as it becomes

executable.  By allowing several tasks  to  share  the  same  area  of

memory,  RTS/8 minimizes the amount of memory that it needs to execute

user tasks.

1-2

INTRODUCTION

1.2  REAL-TIME OPERATION

RTS/8 is a real-time  multiprogramming  system;   that  is,  it  is  a

software  framework  that allows more than one task to share available

resources (such as memory, CPU time, and peripheral  devices)  and  to

respond  to  events  in  real-time.   A  real-time system is a type of

multiprogramming system that  provides  immediate  or  near  immediate

response to external events.

Real-time multiprogramming operations require the precisely  organized

interaction of the following major elements:

o  Executive--the RTS/8 Executive is the kernel of the RTS/8

system;  it is the software that directs task execution.

o  Tasks--a task is a sequence of instructions that  directs  the

computer in manipulating data to achieve some goal.  Tasks are

either user-written or supplied by DIGITAL.

o  Memory--memory is the hardware storage medium in  which  tasks

actually reside when they execute.

Multiprogramming allows the execution of  many  tasks  simultaneously.

If  a  task is waiting for the completion of an I/O operation (or some

other condition is blocking it),  and  thus  cannot  use  the  central

processor,  RTS/8 switches control to another task so that you may use

available CPU time.  This increases system efficiency;  that  is,  you

get  more  work  done because no program puts the CPU on hold while it

waits for an event or information.

1-3

CHAPTER 2

USING TASKS UNDER RTS/8

2.1  RTS/8 TASKS

The RTS/8 Executive is the controlling program in  the  RTS/8  system.

One  of  its  major  functions is to decide which task should run.  It

bases its decision on the priorities of the runnable (or active) tasks

(those  tasks  not waiting for completion of any I/O or other events).

It also services tasks by means of Executive Requests.  (See Chapter 3

for a discussion of Executive Requests.)

Each task in an RTS/8 system is given a unique  task  priority  number

when  a  user generates the system.  Up to 127 (177 octal) tasks, each

with its own task priority, can run at one time.  A task number,  once

assigned,  cannot  change  during  the  execution  of the RTS/8 system

because RTS/8 uses a fixed priority system.

A task priority number serves the following purposes:

o  The RTS/8 Executive uses the number as  an  index  to  various

system tables which contain information about each task.

o  Other tasks use this number to  reference  a  particular  task

when  performing certain Executive Requests (such as sending a

message).

o  The task number determines the task's priority--the lower  the

number, the higher the priority of the task.

The executive uses five internal tables to maintain information  about

the tasks in the system.  Chapter 7 discusses these tables.

2.2  CONCEPTS OF TASK COMMUNICATION

RTS/8 is event driven;  that is, the  highest-priority  runnable  task

executes continuously until:

o  It completes its function, or

o  Some event or condition in the system suspends it.

Another change or condition can reactivate the task.  (Task scheduling

is discussed in Section 2.4.)

2-1

USING TASKS UNDER RTS/8

-------------------- NOTE --------------------

|					     |

|  A task is self-starting if  you  include  |

|  the  appropriate  instruction within the  |

|  task (see Section 2.5). A task may  also  |

|  start  another task, or you may start it  |

|  at the terminal  console  by  using  the  |

|  Monitor Console Routine.		     |

----------------------------------------------

RTS/8 performs two main types of task communication:

1.  Task synchronization through the use of event flags

2.  Intertask messages

2.2.1  Task Synchronization

Whenever two processes execute independently, one process may need  to

wait  until  the other completes executing.  For example, consider the

PDP-8 terminal interface.  The PDP-8, when it is ready to  generate  a

character,  alerts the terminal by issuing a Load Teleprinter Sequence

(TLS) instruction.  The PDP-8 must now wait in a loop if it wishes  to

do further I/O immediately.  For example,

TSF 	    /SKIP ON TELEPRINTER FLAG

JMP.-1

.

.

.

The processor cannot transmit the next character  until  the  terminal

raises  the  ready  flag  to  signal  that  it  has  printed the first

character.  When the terminal raises this flag,  the  processor  exits

from the wait loop and proceeds to load the next character.

Similarly, RTS/8 provides event flags as signalling  mechanisms.   (An

event  flag  is  simply  a location which you choose that contains the

status of an event.) These flags synchronize tasks  with  each  other.

The events are either:

1.  Physical processes  such  as  a  clock  ticking  or  a  valve

closing, or

2.  Conceptual occurrences such as a certain string of characters

which you type, or the scheduling of a task for execution.

Like device flags, an event flag can  denote  either  a  "busy"  or  a

"completed"  state (designated as PENDING and FINISHED, respectively).

Thus, a task can direct another task to perform  an  action.   At  the

time  of  the  request,  it  sets the event flag to the PENDING value.

When the  second  task  completes  the  action,  it  issues  the  POST

Executive  Request.  This  request sets the event flag to the FINISHED

value, a process known as "posting the event flag."

2-2

USING TASKS UNDER RTS/8

As a simple example, if the first task is waiting for the  event  flag

with the following instructions:

TAD event flag  	  /LOAD EVENT FLAG IN AC

SZA CLA 		  /IF EVENT FINISHED, SKIP

JMP .-2 		  /KEEP TRYING

.

.

.

then the posting of the event flag causes the first task to exit from

its loop only when the second task completes its processing.

Since this loop ties up the  PDP-8  processor,  event  flags  have  an

additional  state, WAITING.  In the above example, the addition of the

WAITING state now allows the first task to tell  the  RTS/8  Executive

that  it will wait until the event flag signifies that the second task

completes the event.

The WAIT code is as follows:

CAL

WAITE

event flag

(CAL is a system mnemonic meaning JMS 20 and WAITE is the mnemonic for

the RTS/8 Executive Request that waits for an event flag.)

The WAIT returns execution control to the task issuing  the  WAITE  if

the  event  flag  is  zero  (or FINISHED).  If the flag is PENDING (or

greater than zero), the Executive changes it to WAITING and  the  task

issuing  the WAITE becomes non-runnable (that is, its WAITBITS word is

now non- zero).

If the issuing task sends a message to a second  task,  the  Executive

automatically  makes  the event flag PENDING when the message is sent.

If the issuing task is  using  the  event  flag  to  synchronize  task

actions,  this task must change the flag to PENDING before issuing the

WAIT.  (It may set the flag with an ISZ command.)

Table 2-1 summarizes RTS/8 event flag states.

Table 2-1

Summary of Event Flag States

-----------------------------------------------

| Event Flag State  |           Value         |

|-------------------|-------------------------|

|     FINISHED 	    |   0		      |

|		    |			      |

|     PENDING 	    |   > 0		      |

|		    |			      |

|     WAITING 	    |   < 0 (4000 + Task No.) |

-----------------------------------------------

2-3

USING TASKS UNDER RTS/8

When an RTS/8 Executive Request posts the event flag, the task WAITING

for  it  is  automatically taken out of Event Flag Wait.  (If no other

blocking condition exists, the waiting task  is  again  runnable.   It

resumes execution when higher priority tasks cannot run.)

2.2.2  Intertask Messages

Just as event flags under RTS/8 are similar to hardware device  flags,

messages  are similar to hardware I/O instructions (for example, TLS).

That is, messages start a task (providing that the task is WAITING for

a message) and, at the same time, they pass information to the task.

All RTS/8 messages consist of  a  3-word  message  header  immediately

followed  by the information to be exchanged between the tasks.  (Only

the RTS/8 system uses the message  header.)  The  first  word  of  the

message header is the event flag for the message.

When a task sends a message, the RTS/8 Executive sets the  event  flag

to the PENDING state.  This signifies that a task has sent the message

but the task receiving the messages has not  finished  processing  the

event associated with the flag.

The  receiving  task  must  post  the  event  flag  when  it  finishes

performing  the  action signalled by the message.  This posting serves

two purposes.

1.  It informs the task that sent the message (the "sender") that

the receiver performed the action.

2.  It allows the task to send the message  again  (see  Item  2,

below).

If a task is waiting to receive more than one message, RTS/8 uses  the

second  and  third  words of the message header to link these messages

together (see Figure 2-1).  The second word  is  a  CDF  (Change  Data

Field)  instruction  to  the field of the next message.  If the second

word is 0, then this is the last message.

The third word of the message is a pointer to the next message.

If the receiving task is not actively waiting  for  a  message,  RTS/8

places  the message on the receiving task's "input message queue." The

Executive queues all messages  that  a  task  receives  in  decreasing

priority  order.   For  example, the Executive queues a message from a

priority 3 task ahead of any message sent by another task  except  the

tasks  with  priorities 1 and 2.  Also, RTS/8 queues messages from the

same task in the order in which that task issues them.

Sending a message does not physically move the message information  to

the receiving task;  instead a SEND operation establishes a pointer to

the location of the first data word of the message.

The implications of RTS/8 not moving a message to the receiving task's

work area are:

2-4

USING TASKS UNDER RTS/8

RTS/8

EXECUTIVE        MESSAGE	      MESSAGE	      MESSAGE

MSGTBL		 1		 2		 3

---------       ---------       ---------       ---------

|  CDF  |   --->| Event |   --->| Event |   --->| Event | <-

|-------|   |   | FLag  |   |   | Flag  |   |   | Flag  |  |

|  PTR  |----   |-------|   |   |-------|   |   |-------|  |  Message

---------       |  CDF  |   |   |  CDF  |   |   |   0   |  |  Header

|-------|   |   |-------|   |   |-------|  |

|  PTR  |----   |  PTR  |----   |       | <-

|-------|       |-------|       |-------|

|	     |	     |	     |	     |	     | <-

|	     |	     |	     |	     |	     |  |

|	     |	     |	     |	     |	     |  |  Message

|	     |	     |	     |	     |	     |  |  Content

---------	     |	     |	     |	     |  |

|	     |	     |	     | <-

|	     |	     ---------

---------

Figure 2-1 Message Format and Linking of Messages

1.  You should not have the sender modify data in a message while

the event flag for the message is PENDING.

2.  A task cannot send the same message a second time before  its

event  flag  is FINISHED the first time. (Note, however, that

the task can send other messages while a message is pending.)

RTS/8  enforces  this rule by checking the message event flag

on a SEND Executive Request;  it  puts  the  sender  into  an

Event Flag Wait State if the message is still PENDING.

3.  It is legal for the receiving task to  store  information  in

the  body  of  the  message.   In  this  way,  a receiver can

"answer"  without  the  complications  of  sending  a  return

message.   For  example,  when  a task sends a message to the

disk driver  task  requesting  I/O,  the  driver  places  the

operation's  status in a specific word in the message.  Thus,

this word indicates whether an error occurs.

2.3  PRIORITY

The priority of a task  is  established  when  a  user  generates  the

system.  This number is in the range 1 to 127 (decimal), where a lower

number indicates a higher priority.  The  highest-priority  task  that

has  access  to  all  the  resources  it  needs  controls  the central

processor.  When that  task  becomes  blocked  (to  wait  for  an  I/O

transfer  to  complete,  for example), the Executive looks for another

task to use the processor.  The chosen task is the one  that  has  the

highest priority, has access to all the resources it needs, and is not

waiting for any event to occur.

2-5

USING TASKS UNDER RTS/8

In an RTS/8 installation that mixes real-time applications  with  less

urgent  work,  the real-time tasks usually have a high priority.  This

arrangement ensures that the Executive gives CPU time to the real-time

tasks ahead of other, less urgent tasks.

2.4  TASK INTERRUPTS AND SCHEDULING

A task, once it is running, releases the  CPU  only  if  an  interrupt

occurs  or  if the task itself issues an Executive Request that places

it into a wait state.

An interrupt is a break in the normal operation of the task caused  by

a  signal  from outside the processor.  When the processor receives an

interrupt, it looks for the  appropriate  interrupt  service  routine.

The  SKPINS  Executive  Request  is  the means by which you link these

interrupt routines together into a skip chain.  Note  that  this  skip

chain  is  the  path  which  the  Executive takes when looking for the

interrupt service routine and is  independent  of  the  tasks  in  the

system.

When an interrupt occurs, the Executive follows the skip  chain  until

it  finds  the  appropriate  interrupt  service  routine.   After  the

interrupt routine services the request, control returns to either  the

interrupted  task  or  the  RTS/8 scheduler.  If control passes to the

scheduler, RTS/8 runs the highest priority task that  is  not  waiting

for an event or message.

RTS/8 has a table of reserved words in its Executive.  (Each  word  is

called a WAITBITS word.) A task is runnable only when this word equals

zero.  If any bit is nonzero, then the task is waiting for some  event

to  occur.   The  kinds of events are listed in Table 3-2.  Thus, when

the WAITBITS equals zero, the Executive can run a  task  because  that

task  is  not  waiting for an event or action.  This table of reserved

words is ordered by task priority.  When the RTS/8 scheduler  examines

this  table,  it  selects  the  highest  priority task that has a zero

WAITBITS entry.  Therefore, the only time that a higher-priority  task

can run after control passes to a lower priority task is when:

1.  A lower priority task finishes processing the event which  is

the  cause  of a higher-priority event being in a wait state,

or

2.  An interrupt service routine removes the  condition  blocking

the higher-priority task.

2.5  CREATING RTS/8 TASKS

You create RTS/8 tasks in the same manner  as  you  would  create  any

other  program  on  the PDP-8.  The only influences RTS/8 has on these

programs are:

1.  You must include the MACREL .TASK macro in the task (this  is

explained below).

2-6

USING TASKS UNDER RTS/8

2.  If the task is nonresident (see Chapter 6),

3.  If there is to be intertask communication (see Chapter 3), or

4.  If a task uses a DIGITAL-supplied task (see Chapter 4).

All RTS/8 tasks must include the MACREL .TASK macro.   This  macro  is

placed  in the parameter file during system generation.  Its source is

within the SCRIPT.MA  source  program  distributed  with  RTS/8.   The

format for calling this macro is:

.TASK TASK-NUMBER,TASK-NAME,STARTING-ADDRESS,INITIAL-WAIT,VERSION

where:

TASK-NUMBER	      is the  RTS/8  task  number  which  you  must

supply;   DIGITAL recommends that you use the

symbolic name that was or will be  used  when

the system is generated.

TASK-NAME	      is the symbolic  name  used  when  the  RTS/8

system  was  generated.   You  may  omit this

parameter.   If  you  omit  it,   the   macro

computes this string from TASK-NUMBER.

STARTING-ADDRESS    is  optional.   If  you  omit  this  symbolic

address,  it  defaults  to the START label in

the task. You would  provide  this  value  in

symbolic  form  so  that  MACREL may know the

proper field (as MACREL labels have  a  field

attribute).

INITIAL-WAIT	      is an optional value.  If you do not  include

this parameter, the task is runnable when you

start the RTS/8 system.  You  may  provide  a

symbolic  or  absolute  WAITBITS  value  (see

Section 3.2.1).   (In  previous  versions  of

RTS/8,  it  was obligatory for you to include

this value.)

VERSION 	      is an optional value.  If  you  include  this

value, it is the initial value of the MQ.  If

you omit this value, it defaults to the value

of the label VERS.

RTS/8 requires Version 2 of MACREL.

Examples:

.TASK CLOCK

This declaration uses all of the defaults.

.TASK TEST,,STEST,RUNWT

2-7

USING TASKS UNDER RTS/8

This declaration is for task TEST which starts  at  symbolic  location

STEST and is in RUNWT when the RTS/8 system begins running.

If a task wishes to communicate with another task, it should  use  one

of  the  RTS/8 Executive Requests.  Chapter 3 describes these requests

and Table 3-1 summarizes them.

If a task you write uses one of the DIGITAL-supplied  tasks,  it  must

send a message to that task.  Chapter 4 presents the formats that your

tasks must use when they send messages to  these  tasks.   (Chapter  3

describes the way in which you send these messages.)

Because you normally create an RTS/8 system that  uses  DIGITAL  tasks

and  sends  messages  using  Executive Requests, you should create the

tasks before you generate the RTS/8 system.  DIGITAL  recommends  that

you  write  and  add tasks to an RTS/8 system one at a time.  That is,

after you create one task, you should use the RTS/8 System  Generation

to  create  a minimum system that will support the task.  In this way,

you can effectively monitor the interaction between tasks as  you  are

building the RTS/8 system.  Experience has shown that this is the best

method of locating bugs in real-time systems.

2.6  RUNNING AN RTS/8 SYSTEM

In response to the period that the OS/8 monitor prints, type

R Sname

where:

S     is the default  character  supplied  by  the  RTS/8  System

Generation program.

name 	is the 1  to  5  character  name  supplied  by  the  person

building the system.

After you type this command, the RTS/8 system is running.

For more information, see the RTS/8 System Generation Manual.				   ______________________________

2-8

CHAPTER 3

RTS/8 EXECUTIVE REQUESTS

3.1  COMMUNICATION WITH THE RTS/8 EXECUTIVE

RTS/8 tasks communicate with the  RTS/8  Executive  through  Executive

Requests.  When an RTS/8 system is running, it normally uses locations

20-27 in a field as the communication region across field  boundaries.

(However,  if  a field contains OS/8, the Executive does not use these

locations after OS/8 is running.) You may call the  Executive  in  any

field via a JMS 20 (or CAL) instruction.

-------------------- NOTE --------------------

|					      |

|  When a user generates an  RTS/8  system,  |

|  it included the equate		      |

|					      |

|	 CAL = JMS 20			      |

|					      |

|  in its symbol table.   Consequently,  it  |

|  is not necessary for you to include this  |

|  equate in any task that you may write.    |

----------------------------------------------

The data field (DF) does not have to be any specific value when a task

issues a CAL because the code in location 20

o  Sets the DF to the instruction field (IF),

o  Sets the IF to 0, and

o  Jumps to the RTS/8 Executive.

Table 3-1 gives  a  summary  of  Executive  Requests.   Although  this

chapter  presents  all  Executive  Requests, full presentations of the

RESCHED and FREE Executive Requests appear in Chapter 7.

The RTS/8 Executive does not honor requests to switch tasks  that  come

from  an  interrupt  if  the interrupted task's program counter (PC) is

less than 100 (octal).  This prevents a second Executive  Request  from

destroying  the  RTS/8  Executive's  entry  point  (location 20 in each

field).  Therefore, no task may  have  instructions  that  the  central

processor  will  execute  below  location  100  in any field (except as

noted in Chapter 7).

3-1

RTS/8 EXECUTIVE REQUESTS

-------------------- NOTE --------------------

|					      |

|  You should not write executable code  in  |

|  ZSECTs  that  the linker can place below  |

|  location 100 (octal) in a  field.   This  |

|  is  usually  a  fatal  error.  Also, you  |

|  must not write executable code which has  |

|  an  ASECT  that may be linked below 1000  |

|  (octal) in  field  0.   These  locations  |

|  constitute the RTS/8 Executive.	      |

|					      |

|  If   you   must    include    executable  |

|  instructions in a ZSECT, you must insure  |

|  that this code executes  above  location  |

|  77 (octal) in every field.  Note that in  |

|  the   field   containing    the    RTS/8  |

|  Executive,  approximately  one  half  of  |

|  page  0  is  available  for  user  code.  |

|  Also, if your RTS/8 system has OS/8, you  |

|  have   five   auto-increment   registers  |

|  available.  If OS/8 is not in the  RTS/8  |

|  system, you have six available.	      |

----------------------------------------------

You use many of the Executive Requests  for  intertask  communication.

This,  however,  does  not imply that tasks must communicate with each

other.  A task can execute independently of all other tasks.

Table 3-1

Summary of Executive Requests

-----------------------------------------------------

|        |	Symbolic  |				 |

| Code   |   Name    |	      Description	 |

|--------|-----------|------------------------------|

|    0   |  SEND     |  Send a message to a task    |

|	      |		  |				 |

|    1   |  RECEIVE  |  Look for and/or  receive  a |

|	      |		  |  message from a task	 |

|	      |		  |				 |

|    2   |  WAITE    |  Wait for any event flag for |

|	      |		  |  a   specific   task  to  be |

|	      |		  |  posted			 |

|	      |		  |				 |

|    3   |  RUN      |  Run a task			 |

|	      |		  |				 |

|    4   |  SUSPND   |  Suspend execution of a task |

|	      |		  |				 |

|    5   |  POST     |  Post an event flag		 |

|	      |		  |				 |

|    6   |  SKPINS   |  Insert code into  interrupt |

|	      |		  |  skip chain			 |

-----------------------------------------------------

(continued on next page)

3-2

RTS/8 EXECUTIVE REQUESTS

Table 3-1 (Cont.)

Summary of Executive Requests

-----------------------------------------------------

|        |	Symbolic  |				 |

| Code   |   Name    |	      Description	 |

|--------|-----------|------------------------------|

|    7   |  DERAIL   |  Derail (that  is,  force  a |

|	      |		  |  task's  execution  to a new |

|	      |		  |  address)			 |

|	      |		  |				 |

|    8   |  BLKARG   |  Block a task  from  running |

|	      |		  |  for a specified reason 	 |

|	      |		  |				 |

|    9   |  SENDW    |  Send a message and wait for |

|	      |		  |  it to be received		 |

|	      |		  |				 |

|   10   |  UNBARG   |  Signal that a reason that a |

|	      |		  |  task    is   blocked   from |

|	      |		  |  running no  longer  exists. |

|	      |		  |  (A  task may be blocked for |

|	      |		  |  more than one reason)	 |

|	      |		  |				 |

|   11   |  RESCHD   |  Force the  RTS/8  Scheduler |

|	      |		  |  to run			 |

|	      |		  |				 |

|   12   |  WAITX    |  Wait for a particular event |

|	      |		  |  flag to be posted		 |

|	      |		  |				 |

| 4000   |  FREE     |  Allow another task to  take |

|	      |		  |  control of a partition	 |

-----------------------------------------------------

WAITM (which is discussed in Chapter 7) is a  special  RTS/8  mnemonic

you use for waiting for multiple event flags.

3-3

RTS/8 EXECUTIVE REQUESTS

3.2  EXECUTIVE REQUESTS

--------------					     --------------

|		  |					     |		  |

|   BLKARG   | 					     |   BLKARG   |

|		  --------------------------------------------		  |

|									  |

| Use the BLKARG Executive Request to prevent a task from running.	  |

|									  |

| Format:								  |

|	      TAD TSKNUM						  |

|	      CAL							  |

|	      BLKARG							  |

|	      WAITBITS							  |

|									  |

| where:								  |

|	     TSKNUM    is the number of the task to be blocked.		  |

|	     CAL       is an RTS/8 mnemonic for JMS 20.			  |

|	     BLKARG    is an Executive Request mnemonic.		  |

|	     WAITBITS  is one or more of the values defined in Table 3-2. |

|									  |

----------------------------------------------------------------------

You prevent a task from running (that is, you block it) by  issuing  a

BLKARG  Executive Request.  This Executive Request sets one or more of

the bits in a special, reserved location in a task's Task  Table  (see

Section  7.3)  called  the Task Flags Word.  The Executive checks this

location whenever one task calls another or whenever the task is in  a

runnable position.  Only when this word equals zero is a task runnable.

TSKNUM is a location or literal that contains the number of  the  task

to  be  blocked.   If  TSKNUM contains a zero, the issuing task blocks

itself.

------------------- NOTE -------------------

|					     |

| The  TSKNUM=0  form  of  this  Executive |

| Request is the only legal way for a task |

| to block itself.  If TSKNUM is equal  to |

| the issuing task's number, the action of |

| this Executive Request is undefined  and |

| unpredictable results will occur.	     |

--------------------------------------------

The WAITBITS specify one or more bits to be set in a task's Task Flags

Word.   (They  are called WAITBITS because the bits in the task's Task

Flags Word  indicate  the  reason  or  reasons  that  a  task  is  not

runnable.)  Note  that  a  task is runnable only when this word equals

zero.

Symbolic  names  for  specifying  the  conditions  for  blocking  (and

unblocking)  a  task in the WAITBITS word are given in Table 3-2.  The

symbolic names listed in the table are equates  established  when  the

system was generated.

3-4

RTS/8 EXECUTIVE REQUESTS

Table 3-2

Symbolic Names for Specifying WAITBITS

----------------------------------------------------------------------

| Symbolic  |	   |						  |

|   Name    |  Value  |		 	    Meaning		  |

|-----------|---------|----------------------------------------------|

|  NONRWT   |  4000   |  Nonresident  Wait--this  task  cannot  run  |

|		 |	   |  because it is not in memory.		  |

|		 |	   |						  |

|  EFWT     |  2000   |  Event Flag Wait--this task is  waiting  for |

|		 |	   |  an  event  flag  (which  contains a WAITING |

|		 |	   |  value corresponding to  this  task)  to  be |

|		 |	   |  posted.					  |

|		 |	   |						  |

|  RUNWT    |  1000   |  Run Wait--This task is waiting for a RUN ER |

|		 |	   |  to  be  executed with its number in the AC, |

|		 |	   |  or for the operator to type a REQUEST  task |

|		 |	   |  command to the Monitor Console Routine (see |

|		 |	   |  Chapter 5).				  |

|		 |	   |						  |

|  SWPWT    |  0400   |  Swap Wait--This task cannot run because  it |

|		 |	   |  is  in  the  process  of being brought into |

|		 |	   |  memory.					  |

|		 |	   |						  |

|  EORMWT   |  0200   |  Event or Message Wait--This task is waiting |

|		 |	   |  for an event flag to be posted or a message |

|		 |	   |  to arrive, whichever happens first.	  |

|		 |	   |						  |

|  USERWT   |  0100   |  User Wait--This bit is reserved for use  by |

|		 |	   |  user-written  tasks.   RTS/8  does  not use |

|		 |	   |  this bit.					  |

|		 |	   |						  |

|  ENABWT   |  0040   |  Enable Wait--This task  is  waiting  to  be |

|		 |	   |  enabled.   Used only by the Monitor Console |

|		 |	   |  Routine.  (See Chapter 5 for an explanation |

|		 |	   |  of the ENABLE and DISABLE commands.)	  |

|		 |	   |						  |

|  MSGWT    |  0020   |  Message Wait--This task is  waiting  to  be |

|		 |	   |  sent a message.				  |

|		 |	   |						  |

|           |  0010   |  Reserved for future expansion.		  |

|		 |	   |						  |

|           |  0004   |  Reserved for future expansion.		  |

|		 |	   |						  |

|           |  0002   |  Reserved for future expansion.		  |

|		 |	   |						  |

|  DNEWT    |  0001   |  Task does not exist.  A  user  task  should |

|		 |	   |  never set or clear this bit.		  |

----------------------------------------------------------------------

3-5

RTS/8 EXECUTIVE REQUESTS

Examples:

You place task 14 into User Wait by executing the following code.

TAD	  (14)

CAL

BLKARG

USERWT

Task PRINT is placed into Run Wait by executing the following code.

TAD (PRINT)

CAL

BLKARG

RUNWT

3-6

RTS/8 EXECUTIVE REQUESTS

--------------					     --------------

|		  |					     |		  |

|   DERAIL   |					     |   DERAIL	  |

|		  --------------------------------------------		  |

|									  |

| Use the DERAIL Executive Request to modify the execution of a task |

| by  transferring  control  to  a special subroutine when that task |

| becomes runnable.						  |

|									  |

| Format:								  |

|	      TAD TSKNUM						  |

|	      CAL							  |

|	      DERAIL							  |

|	      ADDR							  |

|									  |

| where:								  |

|	     TSKNUM    is the number of the task to be derailed.	  |

|	     CAL       is an RTS/8 mnemonic for JMS 20.			  |

|	     DERAIL    is an Executive Request mnemonic.		  |

|	     ADDR      is the actual or symbolic starting location of the |

|	      	       subroutine.     					  |

|									  |

----------------------------------------------------------------------

You use the DERAIL Executive Request to force a task's execution to  a

new  address.   This  address  is  normally  the  address of a special

subroutine of the task.  Note that this Executive Request does not set

or clear any of the WAITBITS.

The DERAIL Executive Request simulates a "JMS ADDR" for the task whose

number  is in location TSKNUM (which is the derailed task).  Note that

RTS/8 assumes that ADDR is in the same field  as  the  derailed  task.

The  Executive  stores  the  derailed task's program counter (from its

Task State Table entry) in location ADDR, and also  sets  the  program

counter entry in its Task State entry to ADDR + 1.

Two important points concerning the operation of the DERAIL  Executive

Request are:

1.  The DERAIL  Executive  Request does  not  save  the  derailed

task's   accumulator,   link,   and   data   field  settings;

therefore, the derail subroutine must save and restore  these

values.  In this sense, a derail subroutine is very much like

an interrupt at the task level.

2.  DERAIL does not alter the contents  of  the  derailed  task's

Task Flags word.  If the derailed task is not runnable, RTS/8

does not execute the derail subroutine until the task becomes

runnable.

Generally, a high priority task uses the DERAIL Executive  Request  to

signal  lower priority tasks that an emergency condition occurred.  As

an example, it is sometimes necessary to  abort  all  processes  in  a

process-control  environment  if  the  room  temperature  exceeds some

critical  value.   You  may  program  a  task  that  checks  for  this

3-7

RTS/8 EXECUTIVE REQUESTS

temperature  every ten seconds.  However, it is inefficient to include

shutdown code in this "watchdog  task"  for  all  machinery  that  the

processor  controls.   A   better solution is to provide a location to

which each equipment-controlling task can be derailed in order to shut

down its own piece of equipment.

Example:

An example of a derail subroutine is as follows:

DENTRY, 0       	      /DERAIL ROUTINE ENTRY POINT

DCA SAVAC           /SAVE AC

RAR

DCA SAVLNK          /SAVE LINK

RDF

TAD (CDF)

DCA SAVDF           /SAVE DATA FIELD

CDF .FLD

.       	      /HANDLE EMERGENCY CONDITION

.

.		      /BRANCH TO "RESUME" IF YOU WANT

.		      / TO RESUME WHERE YOU LEFT OFF

.       	      /BRANCH TO "NOGO" IF NOT

RESUME, TAD SAVLNK          /RESTORE LINK

CLL RAL

TAD SAVAC

SAVDF,  HLT      	      /RESTORE DF

JMP I DENTRY        /RESUME (SAME FIELD)

NOGO,   CLA CLL

CDF .FLD

JMP RESTRT          /RESTART TASK

SAVAC, 0

SAVLNK, 0

Dangers of DERAIL

A task can get into serious trouble if the Executive derails it  while

it  is  already  in  a  derail routine.  If this situation occurs, the

original processor state is lost.  There is no simple solution to this

problem.   You  cannot  simply  turn  off the interrupts in the derail

routine;  a second derail may occur before the Executive executes  the

first  derail  routine.  Consequently, you must ensure that a task can

only be derailed by one other task in the system.

Restrictions Using DERAIL

If a task is not runnable, derailing it does  not  make  it  runnable.

For  example,  if a task is in Event Flag Wait, it will remain in this

state until the event occurs.  When the task finally posts  the  event

flag,  the  receiving  task  wakes  up and begins to run in its derail

routine rather than in the mainline routine.  Thus, derailing  a  task

so  that  it can perform some important job immediately may not always

work because a task may be in a wait state and not be able to run  for

3-8

RTS/8 EXECUTIVE REQUESTS

some  time.  For example, if the task were in Message Wait at the time

of the DERAIL Executive Request, the  derail  routine  would  not  run

until  a  message  came  in for that task.  Furthermore, if some event

causes the routine sending the message to shut off, then the receiving

task will never run.

A partial solution to this restriction is  to  code  the  task  to  be

derailed  so  that  it  always  waits  for  events  by using the WAITX

Executive Request instead of WAITE.  If you subsequently  derail  this

task, it is first taken out of MSGWT or EORMWT and then derailed.

An example of the code for this situation is as follows:

TAD TSKNUM

CAL

DERAIL

DENTRY  		  /DERAIL ADDRESS

TAD TSKNUM

CAL

UNBARG  		  /UNBLOCK THE TASK FROM

MSGWT ! EORMWT  	  / MESSAGE-RELATED WAITS

.

.

.

CAL

WAITX

EFLAG

.

.

This works because both the RECEIVE and the WAITX  Executive  Requests

move  the program counter back to the location of the CAL before going

into a wait state.  Thus, no harm can occur if the Executive takes the

task  out  of  that wait state for an incorrect reason.  When the task

resumes running at that point, it  re-executes  the  CAL  (RECEIVE  or

WAITX)  and  goes  back into the wait state as necessary.  This method

does not work if the task was in  Event  Flag  Wait  due  to  a  WAITE

Executive Request because the task would resume running as if the task

posted the event flag when, in fact, it had not.  A way to  circumvent

this problem (other than WAITX) is for the task to do WAITM instead of

a WAITE, and then poll the event  flag  upon  waking  up.   (WAITM  is

discussed in Chapter 7.)

3-9

RTS/8 EXECUTIVE REQUESTS

--------------					     --------------

|		  |					     |		  |

|    FREE	  |					     |    FREE	  |

|		  --------------------------------------------		  |

|									  |

| Use the FREE Executive Request to indicate that the Executive  can |

| swap  a nonresident task out of a partition if another nonresident |

| task wishes to use it.						  |

|									  |

| Format:								  |

|	      CAL							  |

|	      COMMND+FREE     						  |

|									  |

| where:								  |

|	     CAL       is an RTS/8 mnemonic.				  |

|	     COMMND    is an Executive Request that can put a task into a |

|		       wait state.					  |

|	     FREE      is an Executive Request mnemonic.		  |

|									  |

----------------------------------------------------------------------

When a nonresident task is in a wait state, you should  allow  another

nonresident request to use its memory partition if it needs to.  Thus,

the FREE Request tells  the  Executive  that  it  can  swap  the  task

currently  in  a  portion  of memory to the swapping device if another

task is ready to execute in the same partition.

If no other nonresident task is ready to run, the Executive  does  not

swap out the task currently in memory.

If the Executive Request to which you append the FREE Request does not

put  the nonresident task into a wait state, the Executive ignores the

FREE request.

Section 6.2.1 discusses the FREE request.

3-10

RTS/8 EXECUTIVE REQUESTS

--------------					     --------------

|		  |					     |		  |

|    POST	  |					     |    POST	  |

|		  --------------------------------------------		  |

|									  |

| Use the POST Executive  Request  to  set  an  event  flag  to  the |

| FINISHED (zero) state.						  |

|									  |

| Format:								  |

| 	      TAD EFPTR   /POINTER TO EVENT FLAG			  |

| 	      CAL							  |

| 	      POST							  |

| 	      CDF EFFLD   /FIELD OF EVENT FLAG				  |

|									  |

| where:								  |

|	     EFPTR     is the location of the event flag.		  |

|	     CAL       is an RTS/8 mnemonic for JMS 20.			  |

|	     POST      is an Executive Request mnemonic.		  |

|	     EFFLD     is the field of the event flag.			  |

|									  |

----------------------------------------------------------------------

A task signals that it has received a message or completed an event by

using a POST Executive Request.

The POST Executive Request sets the  event  flag  pointed  to  by  the

accumulator  in  the field of the CDF to the FINISHED (or zero) state.

If its previous state was WAITING, the POST Executive  Request  clears

the  event  flag wait bit of the waiting task's WAITBIT.  (This is the

EORMWT bit.) It also sets the PENDING flag to  zero.   This  Executive

Request  never places the calling task into a wait state.  If the task

waiting for the event flag has a  higher  priority  than  the  calling

task,  the  Executive  transfers  control to the higher priority task.

Note that the lower priority task remains runnable;  that is, none  of

its  WAITBITS  are  set.   However, this lower-priority task runs only

when no other higher priority task is runnable.

See Section 3.3 for an example of the POST Executive Request.

3-11

RTS/8 EXECUTIVE REQUESTS

--------------					     --------------

|		  |					     |		  |

|   RECEIV	  |					     |   RECEIV	  |

|		  --------------------------------------------		  |

|									  |

| Use a RECEIV Executive Request to specify that a task is to  await |

| a message before proceeding.					  |

|									  |

| Format:								  |

| 		     TAD TSKNUM						  |

| 		     CAL						  |

| 		     RECEIV						  |

|        MADDR, 0							  |

|									  |

| where:								  |

| 	     TSKNUM    is the task from which  the  receiving  task  will |

|                 receive  a message; or, if TSKNUM equals zero, the |

|                 task can receive a message from any task.	  |

|         CAL     is an RTS/8 mnemonic for JMS 20.			  |

|         RECEIV  is an Executive Request mnemonic. 		  |

|         MADDR   is the address of the message.			  |

|									  |

----------------------------------------------------------------------

The RECEIV Executive Request is the means by which a  task  tells  the

Executive  that  it wishes to receive a message.  The Executive places

both the address of the message in MADDR and a CDF  to  the  message's

field in the accumulator.

If the accumulator is zero when a task  issues  the  RECEIV  Executive

Request, the RTS/8 Executive examines the calling task's (that is, the

task executing the RECEIV) input message queue.  If there are messages

in  the  calling task's input message queue, the Executive removes the

first (i.e., the highest priority) message from the queue  and  places

the  address of its first data word in MADDR.  It also stores a CDF to

the field of the message in the accumulator.

If the accumulator is nonzero when a task issues the RECEIV  Executive

Request,  the  Executive  searches  the  calling  task's input message

queue.  It looks for a message whose sender's task number is the  same

as  the  number  in  the  accumulator.  If it finds such a message, it

removes it from the queue as specified above;  if it does not  find  a

message, it places the issuing task in Message Wait.

------------------- NOTE -------------------

|					     |

| When a task is  in  Message  Wait  after |

| executing a  RECEIV  Executive  Request, |

| its  program  counter,  as stored in the |

| Task  State  Table  (see  Section  7.3), |

| points  back  to the location containing |

| the CAL.  Thus, when a message comes in, |

| the    task   re-executes   the   RECEIV |

| Executive  Request   and   accepts   the |

|					     |

3-12

RTS/8 EXECUTIVE REQUESTS

|					     |

| message.   This  mechanism  is  normally |

| transparent.  One implication is that no |

| harm  is  done  by  taking a task out of |

| Message  Wait  because,  once  the  task |

| starts  up again, it will re-execute the |

| RECEIV Executive  Request  and  go  back |

| into Message Wait. 			     |

--------------------------------------------

Normally, if there are no messages in the input message queue  when  a

task performs a RECEIV, the task is put into Message Wait.  However, a

1 in bit O of the accumulator (i.e., the accumulator is negative) when

a task issues a RECEIV indicates that the task is not willing to wait.

With  no messages in the input message queue (or none sent by the task

whose number is in the accumulator), the task will continue to run (at

location CAL + 3) with the accumulator equal to zero.   Thus,  a  zero

accumulator  is  the means by which the RTS/8 Executive informs a task

that there are no messages (of the desired type) PENDING.

A major restriction in using this Executive Request  is  that  a  task

cannot  send  the  same message twice.  If a task attempts to send the

same message a second time, the Executive prevents  this  action  from

occurring  by  placing  the  task  in  Event Flag Wait until the first

instance of the message has been processed  and  posted.   The  second

instance  of  the  message  is then sent and the Executive removes the

offending task from Event Flag Wait.  However,  if  the  same  message

(that  is,  the  same event flag) is sent by two separate tasks, RTS/8

may stop executing as this is probably a user error.  If this  occurs,

you must reload the RTS/8 system.

See Section 3.3 for an example of the RECEIV command.

3-13

RTS/8 EXECUTIVE REQUESTS

--------------					     --------------

|		  |					     |		  |

|   RESCHD	  |					     |   RESCHD	  |

|		  --------------------------------------------		  |

|									  |

| Use the RESCHD Executive Request to force the RTS/8  scheduler  to |

| run.								  |

|									  |

| Format:								  |

| 	      CAL							  |

|	      RESCHD							  |

|									  |

| where:								  |

|	     CAL       is an RTS/8 mnemonic for JMS 20.			  |

|	     RESCHD    is an Executive Request mnemonic.		  |

|									  |

----------------------------------------------------------------------

You use the RESCHD Executive Request in conjunction with writing  code

that  more than one task uses.  After a task finishes manipulating the

shared code, you must check certain conditions and go directly to  the

RTS/8  scheduler.   Checking  these  conditions is the function of the

RESCHD Executive Request.

Section 7.1.2 discusses the RESCHD request.

3-14

RTS/8 EXECUTIVE REQUESTS

--------------					     --------------

|		  |					     |		  |

|    RUN	  |					     |    RUN	  |

|		  --------------------------------------------		  |

|									  |

| Use the RUN Executive Request to run a task.			  |

|									  |

| Format:								  |

|	      TAD TSRNUM						  |

|	      CAL							  |

|	      RUN							  |

|									  |

| where:								  |

|	     TSKNUM    is the number of the task to be run.		  |

|	     CAL       is an RTS/8 mnemonic for JMS 20.			  |

|	     RUN       is an Executive Request mnemonic.		  |

|									  |

----------------------------------------------------------------------

The RUN Executive Request is equivalent to the following instructions:

TAD TSKNUM

CAL

UNBARG

RUNWT

This request exists because the Executive performs this function often

enough  to justify a shorthand version.  See the UNBARG discussion for

more information.

An example of the RUN Executive Request is included  with  the  SUSPND

discussion.

3-15

RTS/8 EXECUTIVE REQUESTS

--------------					     --------------

|		  |					     |		  |

|    SEND	  |					     |    SEND	  |

|		  --------------------------------------------		  |

|									  |

| Use the SEND Executive Request to send a message to a task.	  |

|									  |

| Format:								  |

| 	      CAL							  |

|	      SEND							  |

|	      TSKNUM							  |

|	      MESSAG							  |

|									  |

| where:								  |

|	     CAL       is an RTS/8 mnemonic for JMS 20.			  |

|	     SEND      is an Executive Request mnemonic.		  |

|	     TSKNUM    is the number of the task to which  a  message  is |

| 		       being sent.					  |

|	     MESSAG    is the location of the message being sent.	  |

|									  |

----------------------------------------------------------------------

The SEND Executive Request sends the location of the message  labelled

MESSAG in the field of the CAL instruction to the task whose number is

TSKNUM.  If the receiving task has a higher priority than  the  sender

and is waiting for a message, RTS/8 transfers control to the receiver.

If this occurs, the sending task is still runnable;  however, it  does

not  regain  control  of the processor until all other higher priority

tasks are in a wait state.

A SEND Executive Request places the event flag into a  PENDING  state.

If  the event flag in location MESSAG is already PENDING (that is, the

event flag is nonzero and the message is still busy  from  a  previous

SEND),  the sender is put into Event Flag Wait on location MESSAG, and

the Executive only performs the  SEND  when  the  event  flag  becomes

FINISHED (zero).  You should ensure that only one task sends a message

because the Executive remembers only the last request to send a "busy"

message;   the  first  task  could  then remain permanently dormant in

Event Wait because the Executive took the  wrong  task  out  of  Event

Wait.  Only after a task acknowledges receipt can the task (or another

task) send the same message.

See Section 3.3 for an example of the SEND command.

3-16

RTS/8 EXECUTIVE REQUESTS

--------------					     --------------

|		  |					     |		  |

|   SENDW	  |					     |    SENDW	  |

|		  --------------------------------------------		  |

|									  |

| Use the SENDW Executive Request to send a message and to wait  for |

| an acknowledging response.					  |

|									  |

| Format:								  |

|	      CAL							  |

|	      SENDW							  |

|	      TSKNUM							  |

|	      MESSAG							  |

|									  |

| where:								  |

|	     CAL       is an RTS/8 mnemonic for JMS 20.			  |

|	     SENDW     is an Executive Request mnemonic.		  |

|	     TSKNUM    is the number of the task that  will  receive  the |

|		       message.						  |

|	     MESSAG    is the address of the first word of the message.   |

|									  |

----------------------------------------------------------------------

The SENDW Executive Request is equivalent to a SEND Executive  Request

followed  immediatly by a WAITE Executive Request;  that is, the SENDW

Executive Request is equivalent to the following sequence:

CAL

SEND    	  /SEND THE MESSAGE

TSKNUM

MESSAG

CAL

WAITE   	  /WAIT FOR RECEIVER TO ACKNOWLEDGE

MESSAG

See the SEND and WAITE discussions for further information.

3-17

RTS/8 EXECUTIVE REQUESTS

--------------					     --------------

|		  |					     |		  |

|   SKPINS	  |					     |   SKPINS	  |

|		  --------------------------------------------		  |

|									  |

| Use the SKPINS Executive Request to insert an interrupt processing |

| routine into the interrupt skip chain.				  |

|									  |

| Format:								  |

|	      CAL							  |

|	      SKPINS							  |

|	      MODULE							  |

|									  |

| where:								  |

|	     CAL       is an RTS/8 mnemonic for JMS 20.			  |

|	     SKPINS    is an Executive Request mnemonic.		  |

|	     MODULE    is the  address  (in  the  current  field)  of  an |

|		       interrupt processing routine.			  |

|									  |

----------------------------------------------------------------------

The RTS/8 Executive is able to receive and dismiss hardware interrupts

and  to  perform  interrupt-initiated task switching. However, it does

not provide room for an interrupt skip chain.  Instead, the skip chain

is  literally a chain which the Executive dynamically builds at system

start-up time using SKPINS Executive Requests  which  you  include  in

tasks.

Whenever a task executes a SKPINS  Executive  Request,  the  Executive

inserts  the interrupt routine at the end of the interrupt skip chain.

(This is usually done by tasks at system start-up time only.) The last

interrupt  routine  which  the  Executive  includes  in the skip chain

points to the RTS/8 interrupt dismiss routine as its "next module." In

this  way,  you control the order in which RTS/8 links the skip chain.

This means that the Executive encounters most-used  interrupts  first.

For  example,  if  a  task  is  not  runnable at system start- up, the

Executive places its interrupt routine into the chain  only  when  the

task becomes runnable.

SKPINS always inserts the interrupt routine at the  end  of  the  skip

chain, and you usually execute a SKPINS Executive Request in a task as

"once-only" code at system start-up time.  Therefore, if all tasks are

runnable,  RTS/8  links  the routines in the skip chain by priority of

the task which inserts them.

Note that the order of  the  skip  chain  can  differ  from  the  task

priority  order.   If  a higher-priority task is not runnable when you

start up the system, then the Executive may link  a  task's  interrupt

routine  into  the  skip chain in a position that is farther away from

the beginning of the chain than that of a lower-priority request.   It

is important to remember that the function of the skip chain is not to

order the priorities but rather the path that the Executive takes when

it looks for an interrupt processing routine.

3-18

RTS/8 EXECUTIVE REQUESTS

If you wish to control the order of the skip chain, you should  create

an RTS/8 system that contains only one runnable task when you start-up

the system.  The function of this task  would  be to  make  the  tasks

runnable in the order in which interrupt routines should link into the

skip chain.  This task should then block itself permanently after  the

Executive  links  all  tasks.   For example, you may use the following

code to block a task permanently.

CLA

CAL

BLKARG

(-1)

After this code executes, this task blocks itself on all the  WAITBITS

and frees its partition.

Once  an  interrupt  routine  receives  control,  there  are   several

restrictions on its execution:

1.  The interrupt routine must  clear  the  interrupt's  hardware

flag.

2.  The interrupt routine is entered  with  the  Data  Field  and

Instruction  Field  of  the next interrupt routine;  you must

correct this--as shown in the  example  interrupt  processing

routine  example  following  this  list--before the processor

executes any indirect addressing.

3.  An interrupt  routine  can  not  issue  any  RTS/8  Executive

Requests.

4.  An  interrupt  routine  must  not  compute  excessively  when

interrupts  are  off.  Typical execution time should be under

75 microseconds.  If  you  need  considerably  more  computer

time,  you  should schedule a task to perform this processing

by posting an event flag.  You use a POSTDS instruction  (see

7 below) to wake up the task.

5.  Interrupt routines must not  turn  on  interrupts  because  a

second   interrupt   destroys  the  processor  state  of  the

interrupted task.

6.  Upon entering the interrupt routine, the Executive saves  the

processor  state.   However,  it may not save the contents of

the Multiplier Quotient, EAE, FPP, etc.  Therefore, interrupt

routines  requiring the use of these resources must save them

before processing the interrupt, and then restore them before

leaving the interrupt routine.

7.  Interrupt routines should dismiss the  interrupt  by  setting

the  Instruction  Field  to  RTS8 (which is a system mnemonic

that equals the field in which the RTS/8  Executive  resides)

and issuing a POSTDS instruction.  (POSTDS is simply an RTS/8

mnemonic for JMP I 24.)  When  the  Executive  dismisses  the

interrupt,  it  can  post  an  event flag by setting the Data

3-19

RTS/8 EXECUTIVE REQUESTS

Field to the field of the event flag and placing the location

of  the  event  flag  in the accumulator prior to issuing the

POSTDS.

For example,

CDF .FLD          /DF = THIS FIELD

TAD (EVFLG)       /EVFLG CAN NOT BE AT LOCATION 0

CIF RTS8	    /RTS8 = EXECUTIVE FIELD

POSTDS	    /DISMISS INTERRUPT AND POST EVFLG

If an interrupt routine will not  post  an  event  flag,  the  routine

should  clear the accumulator prior to issuing the POSTDS instruction.

In this case, the data field DF is irrelevant.

The following  example  illustrates  the  structure  of  an  interrupt

processing routine:

MODULE, 0		     /THIS WORD GETS A POINTER

/   TO THE NEXT MODULE

0		     /MODULE ENTERED HERE--CONTAINS

/   CDF CIF TO NEXT MODULE FIELD

SKDR		     /SKIP ON DEVICE READY

JMP I MODULE	     /NOT READY--GO TO NEXT MODULE

/   IN CHAIN

CDF CIF .FLD	     /THIS ONE IS MINE--SET DF AND IF

/   CORRECTLY

.

.		     /INTERRUPT PROCESSING

.

CIF RTS8	     /DISMISS THE INTERRUPT, MAYBE

POSTDS 	     /   POST AN EVENT FLAG DEPENDING

/   UPON THE CONTENTS OF THE AC

Using the above interrupt format, an RTS/8 Paper  Tape  Punch  handler

task might contain the following sections of code:

In the initialization code (contained in a task that  is  runnable  at

system start-up time):

START,  CAL		     /LINK THE PUNCH SKIP

SKPINS  	     /   INTO THE SKIP CHAIN

PTPINT

GETREQ, CAL

RECEIVE	     /WAIT FOR MESSAGE

.

.

.

As a character punch subroutine used by the main body of the task:

3-20

RTS/8 EXECUTIVE REQUESTS

PUNCH,  0		     /ENTER WITH CHAR IN AC

DCA CHAR	     /SAVE CHAR

CAL		     /WAIT UNTIL PUNCH IS READY

WAITE

PTPEF		     /SET PUNCH EVENT FLAG

ISZ PTPEF	     /   TO THE PENDING STATE

TAD CHAR

CIF RTS8	     /SET INSTRUCTION FIELD TO THAT

/   OF RTS8 EXECUTIVE

PLS		     /LOAD PUNCH BUFFER SEQUENCE

CLA

JMP I PUNCH	     /RETURN

.

.

.

In the interrupt skip chain code:

PTPINT, ZBLOCK 2	     /USED TO CHAIN SKIPS

PSF		     /CHECK PUNCH FLAG

JMP I PTPINT	     /NOT READY

CDF CIF .FLD	     /SET CORRECT DF, IF

PCF		     /CLEAR PUNCH FLAG -

TAD (PTPEF)

POSTDS	     /DISMISS INTERRUPT,

.		     /   POSTING PTPEF

.

.

PTPEF,  0		     /PUNCH INITIALLY READY

CHAR,   0

------------------- NOTE -------------------

|					     |

| RTS/8 does not provide a  mechanism  for |

| removal  of  entries  from the interrupt |

| skip chain.				     |

--------------------------------------------

3-21

RTS/8 EXECUTIVE REQUESTS

--------------					     --------------

|		  |					     |		  |

|   SUSPND	  |					     |   SUSPND	  |

|		  --------------------------------------------		  |

|									  |

| Use the SUSPND Executive Request to suspend  the  execution  of  a |

| task.								  |

|									  |

| Format:								  |

| 	      TAD TSKNUM						  |

|	      CAL							  |

|	      SUSPND							  |

|									  |

| where:								  |

|	     TSKNUM    is the number of  the  task  to  be  blocked  from |

|		       running.						  |

|	     CAL       is an RTS/8 mnemonic for JMS 20.			  |

|	     SUSPND    is an Executive Request mnemonic.		  |

|									  |

----------------------------------------------------------------------

The  SUSPND  Executive  Request  is  equivalent   to   the   following

instructions:

TAD TSKNUM

CAL

BLKARG

RUNWT

This Executive Request exists  because  the  Executive  performs  this

function  often enough to justify a shorthand version.  See the BLKARG

discussion for more information.

The following example shows how you use SUSPND in a task:

A data collection task is to print a report  every  1000  data  points

without interrupting the data collection/reduction process.  At system

start-up, the report generation task comes  up  running;   this  means

that the first report occurs when it first receives information.

(In this simplified example, the data upon which  the  report  program

operates may have been already updated for the next cycle before being

reported.  A full example  would  require  a  scheme  such  as  double

buffering to protect the data.)

3-22

RTS/8 EXECUTIVE REQUESTS

Data Control Task

DLOOP,     TAD (-1750)     /1000 DECIMAL

DCA COUNT

DATALP,    CAL

WAITE

DATAEF  	     /WAIT FOR DATA READY

.		     /   CODE TO STORE DATA

.		     /   POINT IN BUFFER

.		     /GET A DATA POINT

ISZ COUNT       /   AND PROCESS IT

JMP DATALP      /COUNT OFF 1000 POINTS

TAD (REPORT)

CAL	     /RUN REPORT TASK

RUN

JMP DLOOP       /KEEP COLLECTING

COUNT,     0

Report Generation Task

RLOOP,     CAL	     /AC=0, SUSPEND

SUSPND	     /   UNTIL NEEDED

JMS TITLE       /HAS BEEN RUN

.

.		     /PRINT REPORT

.		     /   WITH TITLE

JMP RLOOP	     /REPORT OVER; GO BACK

/   AND WAIT

To eliminate interference with the data collection, REPORT should have

a lower priority than DATA.

3-23

RTS/8 EXECUTIVE REQUESTS

--------------					     --------------

|		  |					     |		  |

|   UNBARG	  |					     |   UNBARG	  |

|		  --------------------------------------------		  |

|									  |

| Use the UNBARG Executive  Request  to  specify  that  a  condition |

| preventing a task from running no longer exists.			  |

|									  |

| Format:								  |

|	      TAD TSKNUM						  |

|        CAL							  |

|        UNBARG							  |

|        WAITBITS							  |

|									  |

| where:								  |

|	     TSKNUM    is the number of the task to be unblocked.	  |

|	     CAL       is an RTS/8 mnemonic for JMS 20.			  |

|       UNBARG    is an Executive Request mnemonic.		  |

|	     WAITBITS  is one or more of the values defined in Table 3-2. |

|									  |

----------------------------------------------------------------------

You use the UNBARG Executive Request to clear one or more  bits  in  a

task's  Task  Flags  Word.  These bits represent reasons why a task is

not runnable.  Note that a task may not be running because of  several

conditions;   executing  an UNBARG upon one condition may not make the

task runnable.  A task can be runnable only if all  of  the  bits  are

zero.

TSKNUM contains the number  of  the  task  to  unblock,  and  WAITBITS

specifies  one  or more bits to be cleared in task TSKNUM's Task Flags

word.  If the Task Flags  word  becomes  zero  as  a  result  of  this

operation,  TSKNUM becomes runnable.  Note that if TSKNUM has a higher

priority than the issuing task and  becomes  runnable,  the  Executive

transfers  control  to  the  higher priority task.  The lower priority

task remains runnable;   that  is,  none  of  its  WAITBITS  are  set.

However,  the  lower  priority task will run only when no other higher

priority task is runnable.

This Executive Request is a no-op (no operation) if it  has  a  TSKNUM

equal to the issuing task's number.

Example:

The Executive takes task 14 out of User  Wait  when  it  executes  the

following code.

TAD (14)

CAL

UNBARG

USERWT

However, if task 14 had been blocked with the following code

3-24

RTS/8 EXECUTIVE REQUESTS

TAD (14)

CAL

BLKARG

USERWT ! RUNWT

then the execution of the above UNBARG would  not  unblock  this  task

because the UNBARG does not clear the RUNWT bit.

3-25

RTS/8 EXECUTIVE REQUESTS

--------------					     --------------

|		  |					     |		  |

|   WAITE	  |					     |   WAITE	  |

|		  --------------------------------------------		  |

|									  |

| Use a WAITE Executive Request to wait for any event flag for  this |

| task to be posted.						  |

|									  |

| Format:								  |

| 	      CAL							  |

|	      WAITE							  |

|	      EFLAG							  |

|									  |

| where:								  |

|	     CAL       is an RTS/8 mnemonic for JMS 20.			  |

|	     WAITE     is an Executive Request mnemonic.		  |

|	     EFLAG     is the location of the event flag.		  |

|									  |

----------------------------------------------------------------------

The WAITE Executive Request prevents a task  from  running  until  the

Executive posts any event flag.

The WAITE Executive Request checks the status of location  EFLAG  and,

if  it  is  FINISHED,  returns  control to the caller.  If the task is

runnable and EFLAG is FINISHED, control returns immediately.  If EFLAG

is  PENDING,  the  Executive changes its state to WAITING and puts the

calling task into Event Flag Wait.  After another  task  or  interrupt

routine posts location EFLAG, the calling task again becomes runnable.

In most cases, you must initialize the event flag (that is, set it  to

1)  before  using  it;   this  is  particularly  true  when  a task is

initiating an event to  be  completed  by  another  task.   Also,  the

waiting  task  must reset the event flag before using it again because

the event flag does not reset itself.

------------------- NOTE -------------------

|					     |

| In some applications, you may be waiting |

| for multiple event flags.  In this case, |

| the task runs whenever a task  posts  an |

| event flags, and not necessarily the one |

| specified in the WAITE.  To ensure that  |

| a  task  posts  a particular event flag, |

| use the WAITX Executive Request. 	     |

--------------------------------------------

See Section 3.3 for an example of the WAITE command.

3-26

RTS/8 EXECUTIVE REQUESTS

--------------					     --------------

|		  |					     |		  |

|   WAITX	  |					     |   WAITX	  |

|		  --------------------------------------------		  |

|									  |

| Use the WAITX Executive Request to wait for a particular event  to |

| be posted.

|									  |

| Format:								  |

| 	      CAL							  |

|	      WAITX							  |

|	      EFLAG							  |

|									  |

| where:								  |

|	     CAL       is an RTS/8 mnemonic for JMS 20.			  |

|	     WAITX     is an Executive Request mnemonic.		  |

|	     EFLAG     is the location of the event flag to be posted.	  |

|									  |

----------------------------------------------------------------------

The WAITX Executive Request causes a task to wait for a specific event

posting, rather than any posting.

The WAITX Executive Request, although similar in most respects to  the

WAITE  Executive Request, differs from it in that if the event flag is

not FINISHED, the  task  goes  into  EORMWT,  Event  or  Message  Wait

(instead  of  EFWT);  moreover, the task's program counter points back

to the location containing the CAL of this  Executive  Request.   (The

program  counter  of  a  task executing a WAITE points to the location

following EFLAG.) Thus, when  the  task  resumes  execution,  it  will

re-execute the WAITX.

If you or the Executive clear the EORMWT bit  for  some  reason  other

than  the  posting of the event flag in question, the task immediately

goes back into EORMWT.  Consequently, control never  flows  past  this

Executive  Request  unless  a task actually posts that event flag.  If

you use a WAITE and if the task is waiting for multiple  event  flags,

then  the  task  conceivably  could start up after the WAITE Executive

Request.  This is because some  other  event  flag--one  that  you  no

longer  care  about--was  posted.   This situation cannot occur with a

WAITX.

See Section 3.3 for an example of the WAITX Executive Request.

3-27

RTS/8 EXECUTIVE REQUESTS

3.3  EXAMPLE OF EXECUTIVE REQUESTS FOR MESSAGE AND EVENT FLAGS

The following example illustrates the RTS/8 Executive Requests dealing

with  messages  and  event  flags.   A  description  of  the execution

sequence follows the example.

Task A

A1	     ALOOP,  CAL     __

SEND	     /SEND TASK B MESSAGE 1

B

MES1

CAL

SEND	     /SEND TASK B MESSAGE 2

B

MES2

A3      	     CAL     __

WAITX	     /WAIT ONLY FOR MESSAGE 1

MES1

A4		     JMP ALOOP       /LOOP     __

MES1,   ZBLOCK 3	     /MESSAGE 1

15		     /RANDOM NUMBERS

37

23

MES2,   ZBLOCK 3	     /MESSAGE 2

-1		     /RANDOM NUMBERS

4

Task B

B1      BLOOP,  CAL     __

RECEIVE	     /GET A MESSAGE

MADDR,  0

B2		     DCA EFCDF       /SAVE MESSAGE CDF FOR POST     __

TAD EFCDF

B4		     DCA .+1	     /PUT CDF INLINE     __

B5		     HLT	     /CDF TO MESSAGE FIELD     __

B6		     TAD I MADDR     /GET 1ST DATA WORD OF MESSAGE     __

/ (DO NOTHING WITH IT)

B7		     CLA     __

B8		     STA CLL RTL     /-3 IN AC     __

B9		     TAD MADDR       /AC POINTS TO MESSAGE EVENT FLAG     __

B10	     CAL     ___

POST	     /DECLARE MESSAGE RECEIVED

EFCDF, HLT      /CDF TO MESSAGE FIELD HERE

Bll	     JMP BLOOP       /LOOP     ___

3-28

RTS/8 EXECUTIVE REQUESTS

The flow of execution in this example depends  on  which  of  the  two

tasks  has  higher  priority.  Assuming that at some time both A and B

become runnable and that task A has higher priority, the  sequence  of

execution is as follows:

Sequence		   Reason for Execution	 ________		   ____________________

A1	    Task A has higher priority than Task B.	    __

A2	    Task A has higher priority than Task B.	    __

A3	    Task A has higher priority than Task B.	    __

B1	    Task A is  now  in  Event  Flag  Wait  since  MES1  was	    __

PENDING;  MES1 is now in waiting state.

B2-B10    Task A is still waiting;  the RECEIVE  at  B1  received	  ______

MES1.

A4	    The post at B10 posted MES1 and "woke up" A, which  has	    __

a higher priority than B.

A1      A continues executing.	    __

A2      A tries to send MES2 again;  B has  not  yet  processed	    __

it;  MES2 is pending.

Bll	    Therefore, A is put into  Event  Flag  Wait  and  B  is	    ___

resumed;  MES2 is not waiting.

Bl-B10    B now RECEIVes and posts MES2.	  ______

A2      This brings A  out  of  Event  Flag  Wait;   the  RTS/8	    __

Executive has modified task A's program counter so that

it will re-execute the offending SEND.

A3      A3 now waits for MES1 to be posted.	    __

If Task B has higher priority, the sequence of execution is:

B1      Task B has higher priority than Task A.	    __

A1      Task B is placed in Message Wait  since  there  are  no	    __

messages in its input queue.  Task A then sends MES1 to

Task B.

B2-B10    Task A's message brings task B  out  of  Message  Wait;	  ______

since B has a higher priority, A is stopped and B runs.

Bll     The post at B10 sets MES1 to FINISHED but has no  other	    ___

effect.

B1      Now Task B tries to get another message.	    __

3-29

RTS/8 EXECUTIVE REQUESTS

A2      There are no other messages;  therefore, Task B is  put	    __

into Message Wait, and A is run.

B2-Bll    Task A sends MES2 which wakes up B;  B processes MES2,	  ______

B1      Returns for more,	    __

A3      And is put into Message Wait.  Since MES1 is FINISHED,	    __

A4      The WAITE at A3 has no effect and Task A	    __

A1      Loops back to A1 and sends MES1 again.	    __

3.4  EXECUTIVE REQUEST WAIT STATES

Table 3-3 is a summary of Executive Request Wait states.

3-30

RTS/8 EXECUTIVE REQUESTS

Table 3-3

Summary of Wait States Incurred by Executive Requests

----------------------------------------------------------------------

|     ER	   | Wait State |      Condition	|  PC Suspended	  |

|		   |		|			|       at	  |

|-------------|------------|-----------------------|-----------------|

|		   |		|			|		  |

| BLKARG  	   | any (given | If task = self       	|    "CAL"+3	  |

|		   |  argument) |			|		  |

|		   |		|			|		  |

| DERAIL      | none    	|	_ 		|	_	  |

|		   |		|			|		  |

| POST	   | none	|	_		|	_	  |

|		   |		|			|		  |

| RECEIVE	   | MSGWT   	| If no messages 	|    "CAL"	  |

| (No wait    |		| in input queue	|		  |

| if AC=4000) |		| and AC positive	|		  |

|		   |		|			|		  |

| RESCHD	   | none	|	_		|	_	  |

|		   |		|			|		  |

| RUN	   | none	|	_		|	_	  |

|		   |		|			|		  |

| SEND	   | none	| EFWT for SEND if 	|    "CAL"	  |

|		   |		| message busy at "CAL"	|		  |

|		   |		|			|		  |

| SENDW	   | EFWT	| If message free 	|    "CAL"+3	  |

|		   |		| but event flag	|		  |

|		   |		| not FINISHED		|		  |

|		   |		|			|		  |

|		   | EFWT	| If message busy 	|    "CAL"	  |

|		   |		|			|		  |

| SKPINS	   | none	|	_		|	_	  |

|		   |		|			|		  |

| SUSPND	   | RUNWT	| If task = self 	|    "CAL"+2	  |

|		   |		|			|		  |

| UNBARG	   | none	|	_		|	_	  |

|		   |		|			|		  |

| WAITE	   | EFWT	| If event flag 	|    "CAL"+3	  |

| (No wait if |		|			|		  |

| EF "done")  |		|			|		  |

|		   |		|			|		  |

| WAITX	   |  EORMWT	| If specified event	|    "CAL"	  |

|		   |		| flag not FINISHED	|		  |

|____________________________________________________________________|

|									  |

| NOTE: (a) "CAL" denotes the address of the CAL instruction in  the |

|		 Executive Request.					  |

|	     (b) A message is said to be busy if its event flag  has  not |

|		 yet been posted by its previous user.			  |

----------------------------------------------------------------------

3-31

CHAPTER 4

RTS/8 SYSTEM TASKS

4.1  RTS/8 SYSTEM TASK SUMMARY

The RTS/8 system includes  system  tasks  that  control  most  of  the

standard  DIGITAL  PDP-8  I/O  devices.   Also included are tasks that

provide interactive system control from the console terminal and  that

allow  a  single  copy  of  the  OS/8  operating  system to run in the

background of a foreground/background system.

The system tasks in the RTS/8 system are as follows:

1.  Cassette Handler--allows you to read data on a tape cassette.              ________________

2.  Cassette File Support Handler--allows you to look up,  enter,              _____________________________

and delete files from a DECcassette in CAPS-8 format.

3.  Clock Handler--performs actions after a  specified  time  has              _____________

elapsed.

4.  Console and Non-console Terminal  Handlers--handle  a  single              __________________________________________

terminal in either line or character mode.

5.  Exit Task--permits special processing before making  an  exit              _________

from RTS/8.

6.  Floppy Disk Handler--provides support for the use  of  single              ___________________

and double density floppy disks.

7.  LINCtape  Handler--supports   both   OS/8   and   DIAL-format              _________________

LINCtapes.

8.  Line Printer Handler--supports LS8, LS8E, LP8,  or  LV8  line              ____________________

printer.

9.  Mass Storage Handlers--control  the  passing  of  information              _____________________

from  these devices to and from memory for the RK8, RK8E, and

RLO1 moving head disks, DF32 and RF08 fixed-head  disks,  and

TC08  DECtape  units.   These handlers read and write data in

the standard OS/8 block format.

10.  OS/8 Files Support Task--allows you to look up,  create,  and              _______________________

delete  files  in  OS/8  directories  from a foreground task.

This task, when you use it with the  mass  storage  handlers,

provides  the  capability to read or write OS/8 files on mass

storage devices.  The OS/8 Support Task is not necessary  for

using this task.

4-1

RTS/8 SYSTEM TASKS

11.  OS/8  Support  Task--supports  the  execution  of   an   OS/8              ___________________

operating  system  in the background.  The OS/8 Files Support

Task is not necessary for using this task.

12.  PDP-8A Null Task--counts in decimal and displays this  number              ________________

on the LED display accumulator of the PDP-8A.

13.  Power Fail Task--provides for an  orderly  shutdown  when  AC              _______________

power  is  lost  (if  you   have power fail hardware and core

memory).   Also  allows  a  programmed  restart  when   power

returns.

14.  UDC/ICS Handler--allows you to control the various  types  of              _______________

UDC/ICS modules.

The remainder of this chapter describes tasks supplied by DIGITAL.

------------------- NOTE -------------------

|                                          |

| The tasks actually in your RTS/8  system |

| are  dependent  upon  the  configuration |

| established   when   the   system    was |

| generated.                               |

--------------------------------------------

4-2

RTS/8 SYSTEM TASKS

4.2  CASSETTE HANDLER

The  Cassette  Handler  (CSA)   allows   you   to   read   and   write

variable-length records on DECcassettes, as well as to perform various

special functions (such as rewind and write end-file marks).  One copy

of the Cassette Handler can operate up to eight units.

There are two general categories of cassette operation:

1.  Handler functions--read and write.

2.  Utility functions--rewind, backspace  file  gap,  write  file

gap, backspace block gap, and skip to file gap.

4.2.1  Handler Function

----------------------------------------------------------------------

| Use the Cassette Handler to perform I/O on a tape cassette.        |

|                                                                    |

| Message Format:                                                    |

|                ZBLOCK 3                                            |

|                CALL+UNIT                                           |

|                RW+FIELD+NOSTORE                                    |

|                BUFADD                                              |

|                SIZE                                                |

|                STATUS                                              |

|                                                                    |

| where:                                                             |

|       CALL      specifies whether you  are  making  a  handler  or |

|                 utility call.                                      |

|       UNIT      is the device unit of the cassette.                |

|       RW        specifies whether you are  performing  a  read  or |

|                 write operation.                                   |

|       FIELD     is the field of the I/O  buffer  in  the  standard |

|                 mass storage handler format (see Section 4.10).    |

|       NOSTORE   specifies whether information will be stored.      |

|       BUFADD    is the address of the I/O buffer in field FIELD.   |

|       SIZE      is the record size.                                |

|       STATUS    is a location where the Handler places I/O  status |

|                 messages.                                          |

----------------------------------------------------------------------

You use the Cassette Handler to perform read and write operations on a

cassette.

Cassette conventions specify a record size of 200 (octal)  bytes,  but

you  can  use  any  size  up  to  377  (octal);   that is, the Handler

transfers 8 bits.  This record buffer cannot cross  field  boundaries.

For  a  read  operation,  the  buffer  is  optional (although you must

include its word in the message), according to bit 11 of Word 5.

You can use the NOSTORE capability for advancing through a long file.

4-3

RTS/8 SYSTEM TASKS

Word 8 contains the contents of status register B, which is defined by

the bit setting as follows:

Table 4-1

Cassette Handler Status Register Bits

-----------------------------

|  Bit  |        Meaning    |

|-------|-------------------|

|   4   |  CRC/block error  |

|   5   |  Timing           |

|   6   |  EOT/BOT          |

|   7   |  EOF              |

|   8   |  Drive empty      |

|   9   |  Read/write       |

|  10   |  Write lockout    |

|  11   |  Ready            |

-----------------------------

The following listing describes the words and meaningful bit positions

of the Message Format.

Word 4 bit  0 = 0        Utility Call

bit  0 = 1        Handler Call

bits 9-11         Cassette Unit

Word 5 bit  0 = 0        Read

0 = 1        Write

bits 6-8          Field of Buffer

bit 11            Do not store data

(applicable to read only)

Word 6                   Buffer Address

Word 7                   Record size in bits 4-11

Word 8                   Status Return

---------------------------------------------------

| 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |

---------------------------------------------------

^       ^                          \___________/

0:  Handler call \___|       |                                |

1:  Utility call /           |                                |

|                                |

RL01 Mode ___________________|                                |

|

Unit _________________________________________________________|

Figure 4-1 Cassette Handler Unit Word Format

4-4

RTS/8 SYSTEM TASKS

---------------------------------------------------

| 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |

---------------------------------------------------

^                      \__________________/   ^

0:  Read  \__________|                               |             |

1:  Write /                                          |             |

|             |

Field _______________________________________________|             |

|

0:  Read into memory \_____________________________________________|

1:  Check data       /

Figure 4-2 Cassette Handler Call Function Word Format

---------------------------------------------------

| 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |

---------------------------------------------------

^   ^   ^   ^   ^   ^    ^    ^

CRC/block error _____________________|   |   |   |   |   |    |    |

|   |   |   |   |    |    |

Timing __________________________________|   |   |   |   |    |    |

|   |   |   |    |    |

EOT/BOT _____________________________________|   |   |   |    |    |

|   |   |    |    |

EOF _____________________________________________|   |   |    |    |

|   |    |    |

Drive empty _________________________________________|   |    |    |

|    |    |

Read/write ______________________________________________|    |    |

|    |

Write lookout ________________________________________________|    |

|

Ready _____________________________________________________________|

Figure 4-3 Cassette Handler Status Return Word Format

At the end of each cassette operation, you should examine  Word  8  to

check for any errors that might occur during an I/O operation.

Examples:

A Cassette Handler message to write 100 bytes from a  buffer  starting

at 21200 to cassette unit 3 is as follows:

UNIT3=3                 /INITIALIZE VARIABLES TO BE

HANDLR=4000             /   USED IN MESSAGE

WRITE=4000

SIZE=100

.

MSG1,      ZBLOCK 3

HANDLR+UNIT3            /HANDLER OPERATION ON UNIT 3

<XEDF BUFFER>+WRITE     /WRITE FROM FIELD OF "BUFFER"

BUFFER

SIZE                    /100 BYTES LONG

MSGER1,    0000                    /STATUS RETURN

4-5

RTS/8 SYSTEM TASKS

To read and not store 200 bytes from unit 2, the message is:

READ=0                  /INITIALIZE VARIABLES USED

NOSTOR=1                /   USED IN MESSAGE

HANDLR=4000

UNIT2=2

SIZE=200

.

.

.

MSG2,      ZBLOCK 3

HANDLR+UNIT2            /HANDLER OPERATION ON UNIT 2

NOSTOR+READ             /READ AND DON'T STORE

0000                    /UNUSED

SIZE                    /200 BYTES

MSGER2,    0000                    /STATUS RETURN

4.2.2  Utility Function Handler

----------------------------------------------------------------------

| Use the Utility Handler to perform  auxiliary  operations  upon  a |

| CAPS-8 formatted cassette.                                         |

|                                                                    |

| Message Format:                                                    |

|                ZBLOCK 3                                            |

|                CALL+UNIT                                           |

|                FUNCTN                                              |

|                STATUS                                              |

|                                                                    |

| where:                                                             |

|       CALL      specifies if the operation is a utility or handler |

|                 call.                                              |

|       UNIT      is the  logical  device  upon  which  the  message |

|                 performs the action.                               |

|       FUNCTN    is the operation which the Handler performs.       |

|       STATUS    is a word indicating the status of the operation.  |

----------------------------------------------------------------------

You  use  the  Utility  Function  Handler  to  perform  the  following

operations:

1.  Rewind--place the cassette at its first record.

2.  Backspace File Gap--place the read/write  head  at  the  last

file gap marking.

3.  Write  File  Gap--write  a  file  gap  at  the  current  tape

position.

4.  Backspace  Block  Gap--place  the  read/write  head  at   the

previous record.

5.  Skip--place the read/write head  at  the  next  file  on  the

cassette.

4-6

RTS/8 SYSTEM TASKS

If the Handler encounters an error, it normally retries the  operation

three  times.   The exceptions occur when a write lock out occurs on a

write operation or when  an  error  appears  while  reading  a  cyclic

redundancy check.

The CAPS-8 User's Manual (DEC-8E-OCASA-A-D) is  suggested  reading  if

you are unsure of cassette conventions.

For a utility function, the words after the RTS/8 message  header  are

as follows:

Word 4 bit  0 = 0        Utility Call

bit  0 = 1        Handler Call

bits 9-11         Cassette Unit

Word 5 (function in bits 6-8):

10           Rewind

30           Backspace File Gap

40           Write File Gap

50           Backspace Block Gap

70           Skip to File Gap

Word 6 (see Figure 4-4)

---------------------------------------------------

| 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |

---------------------------------------------------

\________/

Function: _______________________________________|

1 Rewind

3 Backspace file gap

4 Write file gap

5 Backspace block gap

7 Skip to file gap

Figure 4-4 Cassette Handler Utility Call

For example, to request a rewind on unit 1, the message is:

UTILTY=0        /SET UP VARIABLES FOR USE IN

UNITl=1         /   MESSAGE FORMAT

REWIND=10

.

.

.

MSG3,      ZBLOCK 3

UTILTY+UNIT1    /UTILITY OPERATION ON UNIT 1

REWIND          /REWIND

MSGER3,    0000            /STATUS RETURN

4-7

RTS/8 SYSTEM TASKS

4.3  CASSETTE FILE SUPPORT HANDLER

----------------------------------------------------------------------

| Use the Cassette File Support  Handler  to  look  up,  enter,  and |

| delete files from a DECcassette in CAPS-8 format.                  |

|                                                                    |

| Message Format:                                                    |

|                ZBLOCK 3                                            |

|                COMMND+UNIT                                         |

|                ADRHED                                              |

|                ADRFLD                                              |

|                STATUS                                              |

|                                                                    |

| where:                                                             |

|       COMMND    can be ENTER, LOOKUP, or CLOSE.                    |

|       UNIT      is the device number.                              |

|       ADRHED    is the address of the header for ENTER and LOOKUP, |

|                 or the status return for a CLOSE operation.        |

|       ADRFLD    is the field of the header.                        |

|       STATUS    is the status return location for ENTER and LOOKUP |

|                 operations.                                        |

----------------------------------------------------------------------

The Cassette File Support  Handler  (CSAF)  supports  the  DECcassette

CAPS-8  format.   Thus,  it allows a calling task to look up and enter

files on cassettes in that format.  This Handler requires the Cassette

Handler Task (CSA) to perform the I/O operations.

The Cassette File Support  Handler  performs  the  following  cassette

operations:

o  ENTER--enters a file name into the cassette directory.

o  LOOKUP--finds a file name in the cassette directory.

o  CLOSE--closes the currently open file, enters its directory

information, and then rewinds the cassette.

ENTER and LOOKUP require you to put appropriate information  into  the

record  header  area  which  CSAF uses to perform the file operations.

The header area is at least 40 (octal) words  long  and  cannot  cross

field boundaries.

For ENTER and LOOKUP, the format of the header area must conform  with

cassette  standards  (and therefore be compatible with CAPS-8).  These

formats are listed in Table 4-2.

4-8

RTS/8 SYSTEM TASKS

Table 4-2

Cassette Header Formats

----------------------------------------------------

| Byte (octal)  |                Use               |

|---------------|----------------------------------|

| 0-5           |  Filename                        |

|               |                                  |

| 6-10          |  Filename extension              |

|               |                                  |

| 11            |  File Type                       |

|               |     1 = ASCII                    |

|               |     0 = undefined                |

|               |                                  |

| 12-13         |  File Record Length              |

|               |  (currently word 12 must equal 0)|

|               |                                  |

| 14-15         |  Unused                          |

|               |                                  |

| 16-23         |  Date (ASCII) specified as       |

|               |  ddmmyy                          |

|               |                                  |

| 24-37         |  Unused                          |

|--------------------------------------------------|

| Reference is to 8-bit bytes, one per word, right |

| justified.                                       |

----------------------------------------------------

For an ENTER operation, if the Handler finds a file with the name  you

specify in the header area on the unit, it deletes the file.

For a LOOKUP operation, the Handler returns the  record  size  of  the

file  in  location HEADER+13 (byte 13).  If it cannot find the file or

if an error occurs, this location contains a 0.

A REWIND operation automatically follows a CLOSE operation.

Word definitions for a CSAF message are as follows:

Word 4  bit  0 = 1       ENTER

bit  1 = 1       LOOKUP

bit  2 = 1       CLOSE

bits 9-11        UNIT

Word 5                   Address of header for ENTER and  LOOKUP;

status return for CLOSE

Word 6                   Field of header  for  ENTER  and  LOOKUP

(bits 6-8)

Word 7                   Status return for ENTER and LOOKUP

In all cases, the status return is the contents of status register  B.

For further information, see the CAPS-8 User's Manual.                                      ____________________

4-9

RTS/8 SYSTEM TASKS

---------------------------------------------------

| 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |

---------------------------------------------------

\_________/                         \___________/

|                                    |

Function: _________|                                    |

1 Close                                                 |

2 Lookup                                                |

4 Enter                                                 |

|

Unit ___________________________________________________|

Figure 4-5  Cassette File Support Handler Unit Word Format

Examples:

MSG4,      ZBLOCK 3

4000           /ENTER ON UNIT 0

6400           /INFORMATION IN HEADER

/   STARTING AT 6400

0010           /   OF FIELD 1

0000           /STATUS RETURN

MSG5,      ZBLOCK 3

1003           /CLOSE ON UNIT 3

0000           /STATUS RETURN

4-10

RTS/8 SYSTEM TASKS

4.4  CLOCK HANDLER

----------------------------------------------------------------------

| Messages sent to the clock handler direct it  to  perform  actions |

| after time intervals which you also specify in the message.        |

|                                                                    |

| Message Format:                                                    |

|                CLKMSG, ZBLOCK 3                                    |

|                        COMMND+CLASS+TSKNUM                         |

|                        TIMEHI                                      |

|                        TIMELO                                      |

|                        EXTRA1                                      |

|                        EXTRA 2                                     |

|                                                                    |

| where:                                                             |

|       CLKMSG    is a label identifying  the  start  of  the  clock |

|                 message  (the  Executive uses this location for an |

|                 event flag).                                       |

|       COMMND    is one of the commands listed in Table 4-1.        |

|       CLASS     is an  optional  specification  which  allows  the |

|                 clock CANCEL command to remove groups of requests; |

|                 you may  add  a  CLASS  designator  to  all  clock |

|                 requests  other  than  a  CANCEL  request to label |

|                 those requests for possible future cancelation.    |

|       TSKNUM    is  the  task  to  receive  message  beginning  at |

|                 CLKMSG,  TSKNUM=0  means that the task sending the |

|                 message is also the receiving task.                |

|       TIMEHI/LO specify a time interval from the present  time  in |

|                 terms  of "system ticks" (TIMEHI is the high-order |

|                 word and TIMELO is the low-order word of a double- |

|                 precision number).                                 |

|       EXTRA1    and EXTRA2 are words used by some of the  commands |

|                 listed in Table 4-1.                               |

----------------------------------------------------------------------

The Clock Handler accepts RTS/8 messages and inserts the entries  into

an  internal  clock  queue.   As  the  entries become due, the Handler

removes them from the queue;  the Handler then  decodes  and  executes

the request.

------------------- NOTE -------------------

|                                          |

| You should be careful that  you  do  not |

| exceed   the   queue   length  that  was |

| specified  when  the  RTS/8  system  was |

| generated.  If the handler receives more |

| messages than its queues can store,  the |

| system   behaves   unpredictably.    The |

| default and minimum queue length  is  16 |

| (decimal) entries.                       |

--------------------------------------------

In the following discussion of the Clock Handler,  "system  ticks"  is

defined  as  the number of hardware ticks that occur in one second and

4-11

RTS/8 SYSTEM TASKS

is machine dependent.  This system ticks value  is  called  HERTZ.   A

second  value,  called  SHERTZ, is the number of system ticks that the

Clock Handler will receive before it generates a software interrupt.

The maximum interval that you  can  enter  in  TIMEHI  and  TIMELO  is

(2**24)/SHERTZ.   If SHERTZ is 60, then this interval is approximately

three days.

The Clock Handler maintains the current TIME-OF-DAY (in  system  ticks

until  midnight) in symbolic locations TODH (high-order) and TODL (low

order) in page 0 of field 0.  When TIME-OF-DAY reaches zero (i.e.,  at

midnight),  it  resets the location to -(SHERTZ*86400) (24 hours until

midnight).  The Handler then increments by  one  day  the  OS/8-format

date  word  in  symbolic location DATE in page 0 of field 0.  If HERTZ

and SHERTZ are different, the Handler places a  value  (computed  when

the system was generated) into location TODX.  This value is the two's

complement of the number of hardware ticks HERTZ remaining  until  the

Clock Handler generates a software interrupt.

Note  that  SHERTZ  must  be  less   than   192   for   the   quantity

-(SHERTZ*86400) to be contained in 24 bits.

COMMND, in the above Message Format statement, is the type of  request

and has the meanings listed in Table 4-3.

Table 4-3

Clock Handler Commands

----------------------------------------------------------------------

| Octal  |      Symbolic       |            Description              |

|--------|---------------------|-------------------------------------|

| 0000   |   MARKTIme          |  Post the event flag  CLKMSG  after |

|        |                     |  the  specified  interval  elapses. |

|        |                     |  The   Handler   ignores    TSKNUM, |

|        |                     |  EXTRA1, and EXTRA2.                |

|        |                     |                                     |

| 1000   |   SCHEDUle          |  Post   the   event   flag   CLKMSG |

|        |                     |  immediately.     Execute   a   RUN |

|        |                     |  Executive Request on  task  TSKNUM |

|        |                     |  after   the   specified   interval |

|        |                     |  elapses.   The   Handler   ignores |

|        |                     |  EXTRA1 and EXTRA2.                 |

|        |                     |                                     |

| 2000   |   TIMOUT            |  Post CLKMSG  immediately.   DERAIL |

|        |                     |  task   TSKNUM  into  a  subroutine |

|        |                     |  whose address is in  EXTRA1  after |

|        |                     |  the  specified  interval  elapses. |

|        |                     |  The Handler ignores EXTRA2.        |

----------------------------------------------------------------------

(continued on next page)

4-12

RTS/8 SYSTEM TASKS

Table 4-3 (Cont.)

Clock Handler Commands

----------------------------------------------------------------------

| Octal  |      Symbolic       |            Description              |

|--------|---------------------|-------------------------------------|

| 3000   |   PERIODically      |  Post CLKMSG immediately.   Execute |

|        |                     |  a  RUN  Executive  Request on task |

|        |                     |  TSKNUM   after    the    specified |

|        |                     |  interval   elapses,   and  requeue |

|        |                     |  this  command  with   the   values |

|        |                     |  EXTRA1/EXTRA2    in    place    of |

|        |                     |  TIMEHI/TIMELO.    This   has   the |

|        |                     |  effect   of   running   the   task |

|        |                     |  periodically with the  new  period |

|        |                     |  EXTRA1/EXTRA2.                     |

|        |                     |                                     |

| 4000   |   REMOTE MARKTIme   |  Post   the   event   flag   CLKMSG |

|        |                     |  immediately.   After the specified |

|        |                     |  interval  elapses,  post   another |

|        |                     |  event  flag.   You  place a CDF to |

|        |                     |  the flag's  field  in  EXTRA1  and |

|        |                     |  its address in EXTRA2.             |

|        |                     |                                     |

| 7000   |   CANCEL            |  Cancel  clock  requests  for  task |

|        |                     |  TSKNUM     and     post     CLKMSG |

|        |                     |  immediately.  The Handler  ignores |

|        |                     |  TIMEHI,    TIMELO,   EXTRA1,   and |

|        |                     |  EXTRA2.  If CLASS  is  zero,  this |

|        |                     |  command  cancels all requests.  If |

|        |                     |  CLASS is a nonzero number  between |

|        |                     |  1  and 3, the Handler cancels only |

|        |                     |  those  requests  with   the   same |

|        |                     |  CLASS   number.   Note  that  only |

|        |                     |  another  clock  request  can   set |

|        |                     |  these  bits.   (You  set  them  by |

|        |                     |  indicating  a  CLASS  bit  in  any |

|        |                     |  clock message except a CANCEL.)    |

----------------------------------------------------------------------

In all clock message formats, you  must  include  all  indicated  word

positions,  even  if  the  command  ignores  the word.  For example, a

MARKTIME request might be:

CLKMSG, ZBLOCK 3

MARKTIME

0

170

ZBLOCK 2

Figure 4-6 illustrates bit positions in the Clock Handler Command Word

Format.

4-13

RTS/8 SYSTEM TASKS

---------------------------------------------------

| 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |

---------------------------------------------------

\_________/ \_____/ \___________________________/

|         |                 |

Command: _____________|         |                 |

0 MARKTIME                      |                 |

1 SCHEDULE                      |                 |

2 TIMOUT                        |                 |

3 SCHEDULE PERIODICALLY         |                 |

4 REMOTE MARKTIME               |                 |

7 CANCEL                        |                 |

|                 |

Class Number ___________________|                 |

|

Task Number ______________________________________|

Figure 4-6 Clock Handler Command Word Format

In some cases, you may require an immediate response to a  clock  tick

(if  this  feature  exists  in  your RTS/8 system).  The clock handler

allows you to supply a subroutine to perform this response.  Note that

the  Clock  Handler  calls  this  routine  at  the interrupt level and

interrupts remain disabled.   Also,  the  subroutine  returns  to  the

normal clock interrupt processing with a

CDF CIF RTS8    /RETURN TO EXECUTING FIELD

JMP I SUBN      /   OF CALLER

To enable the linkage of the subroutine, first  place  the  subroutine

address  in  the  global  ALARML  then place a CIF to the field of the

subroutine in ALARMH.  Note that the order of placement  is  critical.

(Setting  them  in  the  reverse  order  causes  unpredictable  system

actions.) To disable this function, simply zero ALARMH.

Should the  immediate  response  subroutine  wish  to  post  an  event

variable,  it  may  do  so  with the POSTDS interrupt release.  If the

routine issues a POSTDS, the Executive does not clear the clock  flag;

consequently,  the  interrupt  occurs  again.   The immediate response

subroutine should remember that it posted the event, so that it avoids

an infinite loop.

4-14

RTS/8 SYSTEM TASKS

------------------ NOTES -------------------

|                                          |

| 1.  You use this  routine  at  your  own |

|     risk.                                |

|                                          |

| 2.  You must not perform  any  Executive |

|     Requests  as  you  are  at interrupt |

|     level.                               |

|                                          |

| 3.  Immediate    Response     Subroutine |

|     processing  time should be kept to a |

|     minimum  (approximately  50   micro- |

|     seconds) as its time is added to the |

|     regular clock  interrupt  processing |

|     time.                                |

--------------------------------------------

Examples:

CAL            /WITH A 60HZ SYSTEM TICK RATE

SENDW          /   THIS CAUSES THE CURRENT TASK

CLOCK          /   TO "GO TO SLEEP" FOR 2 SEC.

SLEEPM

.

.

.

SLEEPM,   ZBLOCK 3        /MESSAGE HEADER

0               /SET EVENT FLAG AFTER INTERVAL

0; 170          /INTERVAL IS 120 (DECIMAL)

/   SYSTEM TICKS

If you change the value 170 in the  above  example  to  the  assembler

expression  2^SHERTZ,  the  preceding  sequence becomes configuration-

independent.

CAL             /RUN THE TASK REPORT ONCE

SEND            /   EVERY HOUR, INDEFINITELY,

CLOCK           /   ASSUMING A 60HZ SYSTEM

RUNMSG          /   TICK RATE

.

.

.

RUNMSG,   ZBLOCK 3        /MESSAGE HEADER

PERIODICALLY SCHEDULE

/RUN REPORT AFTER SPECIFIED

/   INTERVAL AND PERIODICALLY

/   THEREAFTER

0; 1            /FIRST RUN IS ALMOST IMMEDIATELY

/   (1/60 SECOND)

64; 5654        /PERIOD BETWEEN RUNS IS 216000

/   (DECIMAL) SYSTEM TICKS =

/   3600 SECONDS = 1 HOUR

4-15

RTS/8 SYSTEM TASKS

4.5  CONSOLE AND NONCONSOLE TERMINAL HANDLERS

----------------------------------------------------------------------

| Use the Console and non-Console Terminal Handlers to make  use  of |

| terminals.                                                         |

|                                                                    |

| Message Format 1:                                                  |

|                  ZBLOCK 3                                          |

|                  COMMND+LENGTH                                     |

|                  INBUF                                             |

|                  OUTTXT                                            |

|                                                                    |

| Message Format 2:                                                  |

|                  ZBLOCK 3                                          |

|                  ASSGN+TSKNUM                                      |

|                                                                    |

| Message Format 3:                                                  |

|                  ZBLOCK 3                                          |

|                  -2                                                |

|                  CDF OUTBUF                                        |

|                  COMMND+LENGTH                                     |

|                  INBUF                                             |

|                  OUTTXT                                            |

|                                                                    |

|where:                                                              |

|      COMMND     is one of the commands listed in Table 4-4.        |

|      LENGTH     specifies the maximum size of the input buffer  or |

|                 the number of characters to input.                 |

|      INBUF      is a pointer to the input buffer or is a  zero  if |

|                 no  input  (the  input  buffer must be in the same |

|                 field as the output buffer).                       |

|      OUTTXT     is the first word of the output text or a  pointer |

|                 to it.                                             |

|      ASSGN      is an RTS/8 mnemonic having a value  of  200  that |

|                 causes  the  Handler  to be assigned to a specific |

|                 task (or deassigned if TSKNUM=0).                  |

|      OUTBUF     is the field of the output buffer.                 |

----------------------------------------------------------------------

The RTS/8 Terminal Handler operates upon a terminal in either line  or

character  mode.   You  specify  line  mode  if  you wish the Terminal

Handler to input or output one line of text at a time.  You  terminate

input in line mode with a carriage return or an ESCAPE character.  You

may also edit the line using the DELETE (or RUBOUT) and CONTROL/U (^U)

characters.   (DELETE  deletes the last valid character you type and U

deletes the  entire  line.)  Character  deletion  and  other  terminal

characteristics are determined when the system was generated.

The character mode specifies how many characters the Terminal  Handler

processes.   The Handler does not echo character mode input;  instead,

it terminates this mode when the character count becomes greater  than

the number of characters that you specify.

You use FORMAT 1 when you write text and optionally read text.

4-16

RTS/8 SYSTEM TASKS

You use FORMAT 2 when you assign the Terminal Handler  to  a  specific

task.   To  deassign  the  terminal,  the task possessing the terminal

sends a separate message specifying that TSKNUM equals zero.

You use FORMAT 3 when the output buffer is in a different  field  from

that of the message.

At the time the system is generated, a Terminal Handler is made either

console  (i.e.,  it  talks  to  the MCR) or nonconsole.  (Note that an

RTS/8 system can have only one Console Terminal Handler.) The  Console

Terminal  Handler  invokes the MCR whenever you type a CONTROL/C (^C);

nonconsole terminals treat ^C the same as any  other  character.   For

the  Console  Handler,  ^C wakes up the MCR by posting the MCR's event

flag.

When an RTS/8 system is generated, a Terminal Handler can be  made  to

have  a read timeout (if the system has a Clock Handler).  If there is

no read timeout, the terminal can wait indefinitely  (except  for  EMT

messages,  see  Section  4.5.1)  for a read operation to complete.  If

read timeout is specified, the Terminal Handler may abort the  request

which  is  "hanging"  the  terminal.   First, the Handler checks if it

received any characters.  If the task  is  receiving  characters,  the

Handler  continues the read operation;  if not, the Handler checks its

request queue.  If there are no additional requests for  the  Handler,

the  "hung"  operation  retains  control.  Otherwise, RTS/8 aborts the

read operation and the Handler gives control to another  task  in  the

system.   However, if a task owns a terminal because of an ASSGN, only

a request that it issues can cause a timeout.  If, in this  case,  the

task times out, the Handler does not post the aborted request.

------------------- NOTE -------------------

|                                          |

| If the Handler aborts the input request, |

| it  will  print  a ^U.  It also places a |

| 4000 after the last  character  success- |

| fully input into the input buffer.       |

--------------------------------------------

Legal commands, which you may combine, are listed in Table 4-4.

4-17

RTS/8 SYSTEM TASKS

Table 4-4

Terminal Handler Commands

----------------------------------------------------------------------

| Octal  |Symbolic |       Action if       |     Action if not       |

| Command| Command |       Specified       |     Specified           |

|--------|---------|-----------------------|-------------------------|

| 4000   | NOPACK  | Output text is in     | Output text is in       |

|        |         | unpacked ASCII, one   | packed 6-bit, two       |

|        |         | character per word,   | characters per word     |

|        |         | terminated by a 0000. | terminated by a 00.     |

|        |         |                       |                         |

| 2000   | NOCRLF  | Do not type a CR/LF   | Type a CR/LF after      |

|        |         | after the message.    | typing the message.     |

|        |         |                       |                         |

| 1000   | IND     | OUTTXT points to the  | OUTTXT is the first     |

|        |         | first word of the     | word of the output      |

|        |         | output text.          | text.                   |

|        |         |                       |                         |

| 0400   | NOLINE  | Input is in character | Input is in line mode;  |

|        |         | mode;  terminated     | terminated by a CR or   |

|        |         | after LENGTH input    | ESCAPE.  The length is  |

|        |         | characters read.      | still tested.           |

----------------------------------------------------------------------

The following is a discussion of other aspects of the Terminal Handler

format.

LENGTH         is a 7-bit field which specifies the maximum  size

of the input buffer (if input is in line mode), or

the number of characters to input (if input is  in

character mode).  If the input is in line mode and

there are LENGTH-1 characters in the input buffer,

every character that you enter overwrites the last

character in the buffer.  The  Handler  posts  the

message  event  flag only when you type a carriage

return or an ESCAPE.

INBUF          is a pointer to the input buffer.  If this word is

zero,  the  Handler  does not read data.  If it is

non-zero, the Handler fills the input buffer  with

characters  that  it  packs  one per word and also

forces the parity bit (bit 4) on.  If input is  in

line  mode,  the  last  character  of  the line is

followed by a zero  word  (if  a  carriage  return

terminates  the  line)  or a -1 (7777) word (if an

ESCAPE  character  terminates  the  line).   If  a

timeout   occurs  or  the  EMT  aborts  the  input

message, the Handler places a 4000 after the  last

word successfully input.

ASSGN = 200    assigns  the  Terminal  Handler  to  task  TSKNUM,

causing  the  Terminal  Handler to accept messages

only from the task that you specify.   If  another

task  tries  to  SEND  a  message  to the Terminal

4-18

RTS/8 SYSTEM TASKS

Handler while it is assigned, the  Handler  places

the  message  in  the  Terminal  Handler's Message

Input  Queue;   RTS/8  does  not  remove  it   for

processing  by  the  Terminal  Handler  until task

TSKNUM releases it.  The task to which you  assign

the Terminal Handler can release the assignment by

sending a message assigning the  Terminal  Handler

to  task  number  0.  This assignment message does

not perform any I/O.

TSKNUM         is a 7-bit field used with the  ASSGN  command  to

specify  the  task number of the task to which you

are assigning the  terminal.   If  this  field  is

zero,  the  Handler  deassigns  the  terminal. The

zero assignment allows the terminal task to accept

commands from any task.

OUTBUF         is the field of the output  buffer.   You  specify

this  value (in Message Format 3) if the buffer is

in a field different  from  that  of  the  message

call.

RTS/8 permits you to use a single terminal for both the foreground and

background  (see  Chapter  2  of  the RTS/8 System Generation Manual).                                           ______________________________

When  you  start-up  the  system,  the  terminal  "belongs"   to   the

foreground.    Typing   a  CONTROL/B  (^B)  switches  control  to  the

background.  Conversely, typing a CONTROL/F (^F) while the  background

"owns" the terminal switches it to the foreground.

The change to the  background  occurs  only  at  a  message  boundary.

Consequently,  you must terminate any input characters with a carriage

return or ESCAPE  to  allow  the  ^B  to  take  effect.   However,  ^F

transfers control immediately to the foreground.

If you type a ^F while  in  the  foreground  or  a  ^B  while  in  the

background, the Handler treats the commands as no-ops.

The following two figures illustrate bit  positions  in  the  Terminal

Handler.

4-19

RTS/8 SYSTEM TASKS

---------------------------------------------------

| 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |

---------------------------------------------------

^   ^   ^   ^   ^  \___________________________/

|   |   |   |   |___            |

Command (bits 0-4)   |   |   |   |_____ |            |     __________________

0:  Packed ASCII   \_|   |   |_______ | |            |

1:  Unpacked ASCII /     |_________ | | |            |

| | | |            |

0:  CR/LF at end of message    \__| | | |            |

1:  No CR/LF at end of message /    | | |            |

| | |            |

0:  OUTTXT is the first word    \___| | |            |

1:  OUTTXT points to first word /     | |            |

| |            |

0:  Input in line mode      \_________| |            |

1:  Input in character mode /           |            |

|            |

Bit 4 must be a 0 ______________________|            |

|

Length (bits 5-11)                                   |     __________________

If bit 3=1, no. of characters to input   \___________|

If bit 3=0, maximum size of input buffer /

Figure 4-7 Terminal Handler I/O Mode Command and Length Word Format

---------------------------------------------------

| 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |

---------------------------------------------------

\_____________/  |  \___________________________/

|         |               |

Unused _____________|         |               |

|               |

Bit 4 must be a 1 ____________|               |

|

Task Number __________________________________|

Figure 4-8 Terminal Handler ASSGN Mode Command Word Format

Examples:

HIYA,     ZBLOCK 3        /MESSAGE HEADER

0               /PACKED TEXT, END WITH CR/LF

0               /NO INPUT

TEXT "HELLO"    /TEXT TO BE OUTPUT

Sending the above message to the Terminal Handler prints HELLO on  the

terminal.

QUEST,    ZBLOCK 3       /MESSAGE HEADER

NOCRLF+60      /PACKED TEXT, NO CR/LF

/   48-CHARACTER INPUT LIMIT

ANSWER         /POINTER TO INPUT BUFFER

TEXT "TYPE THE ANSWER"

4-20

RTS/8 SYSTEM TASKS

Sending the above message to the Terminal Handler prints

TYPE THE ANSWER:

on the terminal  and  inputs  a  reply  without  first  returning  the

carriage.   You could print the answer to this message on the terminal

by sending the following message:

TYPANS,    ZBLOCK 3       /MESSAGE HEADER

NOPACK+IND     /UNPACKED TEXT, INDIRECT, WITH CR/LF

0              /NO INPUT

ANSWER         /POINTER TO OUTPUT TEXT

4.5.1  EMT Task

----------------------------------------------------------------------

| Use the EMT (Emergency Message Task) to  send  emergency  messages |

| which originate from a task not in control of the terminal.        |

|                                                                    |

| Message Format 1:                                                  |

|                  ZBLOCK 3                                          |

|                  COMMND+LENGTH                                     |

|                  INBUF                                             |

|                  OUTTXT                                            |

|                                                                    |

| Message Format 2:                                                  |

|                  ZBLOCK 3                                          |

|                  -2                                                |

|                  CDF OUTBUF                                        |

|                  COMMND+LENGTH                                     |

|                  INBUF                                             |

|                  OUTTXT                                            |

----------------------------------------------------------------------

If, while one task is using the terminal, another task wishes to  send

an  emergency  message to the terminal, the sending task can interrupt

the task presently in control of the terminal by sending  the  message

to  the  EMT.   An emergency message overrides input waits, background

ownership, terminal assignments, and CONTROL/S commands.

The EMT sends messages to one and  only  one  terminal  in  any  RTS/8

system.   If  the system has more than one terminal, the EMT sends the

message to the terminal specified when the system was generated.

The emergency message has the same  format  as  an  ordinary  terminal

message.   However,  a  task sends the message to EMT rather than TTY.

After a  task sends  the  message,  the  EMT  seizes  control  of  the

terminal.  After  it  prints  the  message,  it  restores the Terminal

Handler to its original state.

4-21

RTS/8 SYSTEM TASKS

------------------ NOTE --------------------

|                                          |

| If the terminal was waiting  for  input, |

| the  EMT  aborts the input request.  EMT |

| signals that it aborted input by placing |

| a word containing 4000 at the end of any |

| input already received.  It also  prints |

| a ^U on the terminal.  The input is then |

| returned to the task as if it were done. |

| It  is the responsibility of the calling |

| task to retry the input.                 |

--------------------------------------------

Because the EMT breaks into the middle of the current  I/O  operation,

the  message  may  be  unreadable unless you format it with separators

such as carriage  returns  or  line  feeds.   You  may  also  find  it

desirable  to  output  the  bell  character (^G) as a means of drawing

attention to the message.

As additional precautions, you may wish to 1) have the EMT continually

output  the  emergency  message  until  it  receives   an  appropriate

response, or 2) require a confirmation before further I/O can proceed.

You  would  want to do this because the message could scroll off a CRT

screen if another task transmits data to the CRT.

4-22

RTS/8 SYSTEM TASKS

4.6  EXIT TASK

----------------------------------------------------------------------

| Use the exit task to perform special processing before  making  an |

| exit from RTS/8.                                                   |

----------------------------------------------------------------------

RTS/8 operations do not require the EXIT task.  However, if this  task

is  in  your  system,  it  is run by the MCR EXIT command (see Section

5.3.7).  Note that the EXIT task performs the same functions as  those

performed  by the MCR EXIT command;  that is, it waits for any pending

operations to be completed then turns off interrupts  and  returns  to

the OS/8 operating system.

In addition  to  normal  exit  processing,  the  EXIT  task  allows  a

user-task  to request additional special exit processing just prior to

shutting down RTS/8 by having the user-task send a message to the EXIT

task.   This  message contains a single word, namely, the address of a

routine (residing in the same field as  the  message)  that  the  task

calls (with a JMS) at the time of the exit.

When you type a MCR  EXIT  command,  these  routines  are  called  and

executed in the priority order of calling tasks.

------------------- NOTE -------------------

|                                          |

| Any message sent to the EXIT  task  will |

| not  get  posted.   Also, if you use the |

| MCR REQUEST  command  to  run  the  EXIT |

| task, the results are unpredictable.     |

--------------------------------------------

4-23

RTS/8 SYSTEM TASKS

4.7  FLOPPY DISK HANDLER

----------------------------------------------------------------------

| Use the Floppy Disk Handler to perform I/O on a floppy.            |

|                                                                    |

| Message Format 1:                                                  |

|                  ZBLOCK 3                                          |

|                  CODE+DEL+MODE+UNIT                                |

|                  RW+PAGES+FIELD                                    |

|                  BUFADD                                            |

|                  BLOKNO                                            |

|                  STATUS                                            |

|                                                                    |

| Message Format 2:                                                  |

|                  ZBLOCK 3                                          |

|                  1000+CODE+DEL+MODE+UNIT                           |

|                  RW+PAGES+FIELD                                    |

|                  BUFADD                                            |

|                  BLOKNO                                            |

|                  STATUS                                            |

----------------------------------------------------------------------

The format for this Handler is similar to other mass storage handlers;

however,  the  fourth  word  is  different  (see Section 4.10 for more

information).  When sending or receiving messages,  you  do  not  make

distinctions between single and double density floppies as the handler

has routines which determine the packing density.

Message Format 2 exists to  maintain  compatibility  with  other  mass

storage calls.  You would use it only if your task originally called a

device using that format.

The parameters of the above format statements are as follows:

CODE  = 0        is  the  regular  condition.   The  Floppy  Disk

Handler  interprets  BLOKNO  as  an OS/8 logical

record number, and interprets pages in the  OS/8

sense  to  mean  the  number of PAGES of data to

transfer.  The Handler ignores the DEL bit.

= 4000     is the special physical sector  condition.   The

Handler  ignores  PAGES  but  does  transfer one

segment,  namely,  the  segment   specified   by

BLOKNO.    The   Handler  interprets  BLOKNO  as

TTTTTTTSSSSS.  That is, the high order 7 bits of

this  word  represent  the physical track number

which must be in the range 0-76  decimal  (0-114

octal).    The   low-order   5  bits  of  BLOKNO

represent the sector number on that track.  This

number  must  be in the range 1-26 decimal (1-32

octal).

DEL   = 0        ignores deleted data marks.

= 2000     treats deleted marks (if CODE=4000) as follows:

1.  If writing a sector, write deleted data

4-24

RTS/8 SYSTEM TASKS

indication.   However, the Handler does

not indicate this fact  in  the  STATUS

word.

2.  If reading a sector, set bit 5  of  the

STATUS  word  to  a  1  if  it  reads a

deleted  data  indicator.   In  such  a

case,  the  STATUS  word may be nonzero

even though no physical error occurs.

MODE  = 0        specifies transfer in 12-bit mode, which is  the

OS/8 format.  In 12-bit mode, the Handler places

the 64 (decimal) 12-bit words that  comprise  an

OS/8  floppy  sector  into the first 96 bytes of

the sector, while  the  last  32  bytes  contain

random  bit  patterns.  In 12-bit mode, a sector

contains 64 (decimal) 12-bit words of data.

= 100      specifies transfer  in  8-bit  mode.   In  8-bit

mode,   an   8-bit   byte  on  the  floppy  disk

corresponds to the low order 8 bits of a  12-bit

word  in  memory.  The Handler does not transfer

data in the high order  4  bits  of  a  word  in

memory  to  the floppy.  In 8-bit mode, a sector

contains 128 (decimal) 8-bit bytes of data.

UNIT               specifies the drive unit number.  It may be 0 or

1000.   The  number  0 refers to the unit on the

left of a dual drive.

RW      = 0        reads data from floppy.

= 4000     writes data to floppy.

PAGES              specifies the number of pages to transfer (times

100  octal).   PAGES  =  0 transfers 40 pages (a

full field).  This value takes  the  range  0-37

(octal)  in  bits 1-5 of this word.  The Handler

ignores  PAGES  if  CODE=4000.   For  a   single

density floppy, if CODE equals 4000, the Handler

transfers either 100 (octal) 12-bit words or 200

8-bit  bytes (from 200 words) depending on MODE.

For  a  double  density  floppy,   the   Handler

transfers twice as much information.

FIELD              is the standard RTS/8 mass storage FIELD  format

(see Section 4.10).

BUFADD             specifies the address of the first word  of  the

buffer  containing  data.  FIELD  determines the

field of the buffer.  You specify the length  of

the  buffer  by  PAGES  if  CODE=0 or by MODE if

CODE=4000.

BLOKNO             represents  the  first  logical  OS/8  block  to

transfer if CODE=0.  Each OS/8 block consists of

four sectors.  The Handler ignores track  0  and

4-25

RTS/8 SYSTEM TASKS

it  employs  a  2  to  1  interleave scheme.  If

CODE=4000, this word contains the physical track

and sector numbers in the format TTTTTTTSSSSS.

STATUS             receives the status of the  operation  upon  its

completion.  If it is negative, a hardware error

occurred.  If it is  zero,  no  error  occurred.

This word may be a positive, nonzero number only

if DEL=2000.  See Table 4-5 for descriptions  of

the STATUS bits.

Table 4-5

Floppy Status Bit Descriptions

----------------------------------------------------------------------

|     |                       Meaning if 1                           |

|     |--------------------------------------------------------------|

| Bit |       Single Density        |        Double Density          |

|-----|--------------------------------------------------------------|

| O   | Hard error                  | Hard error                     |

| 1-3 | Not used by controller      | Not used by controller         |

| 4   | Not used by RTS/8           | Selected Drive Ready           |

| 5   | Deleted data indication     | Deleted data indication        |

| 6,7 | Not used by controller      | Not used by controller         |

| 8   | Reserved for future use     | RX02 Drive                     |

| 9   | INIT done (can occur after  | INIT done (can occur after     |

|     | temporary power failure to  | temporary power failure to     |

|     | controller)                 | controller)                    |

| 10  | Parity error                | Side 1 Ready                   |

| 11  | CRC (Cyclic Redundancy      | CRC (Cyclic Redundancy Check)  |

|     | Check) error                | error                          |

----------------------------------------------------------------------

The following two figures illustrate bit positions and their meanings.

---------------------------------------------------

| 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |

---------------------------------------------------

^   ^   ^           ^                         ^

|   |   |________   |                         |

|   |_________  |   |                         |

|__________  |  |   |                         |

0:  Regular condition       \__|  |  |   |                         |

1:  Special physical sector /     |  |   |                         |

condition                     |  |   |                         |

|  |   |                         |

0:  Do not handle deleted marks \_|  |   |                         |

1:  Handle deleted marks        /    |   |                         |

|   |                         |

RL01 Mode ___________________________|   |                         |

|                         |

0:  Transfer in 12-bit mode \____________|                         |

1:  Transfer in 8-bit mode  /                                      |

|

0:  Left unit of dual drive  \_____________________________________|

1:  Right unit of dual drive /

4-26

RTS/8 SYSTEM TASKS

CODE = 4000 (bit 0 set to 1) transfers one sector specified by BLOCKNO

as follows:

---------------------------------------------------

| 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |

---------------------------------------------------

\_________________________/ \___________________/

|                       |

Physical track no. ______________|                       |

(0-114 octal)                                            |

|

Sector no. on track _____________________________________|

(1-32 octal)

Figure 4-9 Floppy Disk Handler Unit Word Format

---------------------------------------------------

| 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |

---------------------------------------------------

^                   ^               ^    ^    ^

|                   |               |    |    |

Hard error _______|                   |               |    |    |

|               |    |    |

Deleted data _________________________|               |    |    |

|    |    |

INIT done ____________________________________________|    |    |

|    |

Parity error ______________________________________________|    |

|

CRC error ______________________________________________________|

Figure 4-10 Floppy Disk Handler Status Word Format

The largest legal OS/8 block number on a single density floppy disk is

755  octal.  For a double density floppy, the largest legal OS/8 block

number is 1733.  If you reference any block number  greater  than  755

for  a  single density floppy or 1733 for a double density floppy, the

Handler returns in STATUS the negative of the number of BLOKNO on  the

device.

Note that no hardware indication exists of a write  operation  to  the

wrong  density media.  The Handler precedes most write operations by a

directory  read  that  signals  the  device's   density.    For   more

information  on  the  relationship  between  single and double density

floppies, see the appropriate hardware and software manuals.

Table 4-6 lists the standard OS/8 interleave  scheme  for  single  and

double density floppies.

4-27

RTS/8 SYSTEM TASKS

Table 4-6

OS/8 Floppy Disk Interleave Scheme

----------------------------------------------------------------------

|        OS/8   |     Floppy Sectors (track/sector--decimal)         |

| Logical Block |----------------------------------------------------|

|    (octal)    |       Single Density           |  Double Density   |

|---------------|----------------------------------------------------|

|      0        | 1/1     1/3     1/5     1/7    |  1/1     1/4      |

|               |                                |                   |

|      1        | 1/9     1/11    1/13    1/15   |  1/7     1/10     |

|               |                                |                   |

|      2        | 1/17    1/19    1/21    1/23   |  1/13    1/16     |

|               |                                |                   |

|      3        | 1/25    1/2     1/4     1/6    |  1/19    1/22     |

|               |                                |                   |

|      4        | 1/8     1/10    1/12    1/14   |  1/25    1/2      |

|               |                                |                   |

|      5        | 1/16    1/18    1/20    1/22   |  1/5     1/8      |

|               |                                |                   |

|      6        | 1/24    1/26    2/1     2/3    |  1/11    1/14     |

|               |                                |                   |

|      7        | 2/5     2/7     2/9     2/11   |  1/17    1/20     |

|               |                                |                   |

|      10       | 2/13    2/15    2/17    2/19   |  1/23    1/26     |

|               |                                |                   |

|      11       | 2/21    2/23    2/25    2/2    |  1/3     1/6      |

|               |                                |                   |

|      12       | 2/4     2/6     2/8     2/10   |  1/9     1/12     |

|               |                                |                   |

|      13       | 2/12    2/14    2/16    2/18   |  1/15    1/18     |

|               |                                |                   |

|      14       | 2/20    2/22    2/24    2/26   |  1/21    1/24     |

|               |                                |                   |

|      15       | 3/1     3/3     3/5     3/7    |  2/1     2/4      |

|               |                                |                   |

|      .        |                                |                   |

|      .        |                                |                   |

|      .        |                                |                   |

----------------------------------------------------------------------

OS/8 does not use Track O and you cannot access it in the 12-bit mode.

4-28

RTS/8 SYSTEM TASKS

4.8  LINCTAPE HANDLER

----------------------------------------------------------------------

| Use the LINCtape Handler to perform I/O on LINCtape which  is  in  |

| either OS/8 or DIAL format.                                        |

|                                                                    |

| Message Format 1:                                                  |

|                  ZBLOCK 3                                          |

|                  MODE+UNIT                                         |

|                  RW+PAGES+FIELD                                    |

|                  BUFADD                                            |

|                  BLOKNO                                            |

|                  STATUS                                            |

|                                                                    |

| Message Format 2:                                                  |

|                  ZBLOCK 3                                          |

|                  1000+MODE+UNIT                                    |

|                  RW+PAGES+FIELD                                    |

|                  BUFADD                                            |

|                  BLOKNO                                            |

|                  STATUS                                            |

----------------------------------------------------------------------

You normally use Message Format 1.  The second  format  (the  absolute

block format) is for compatibility with tasks originally developed for

devices using that format.

The parameters of the above format statement are as follows:

MODE  = 0       specifies OS/8.  An OS/8 LINCtape contains 200 or

201 (octal) words per physical block.

= 4000    specifies DIAL  mode.   A  DIAL  LINCtape  should

contain 400 (octal) words per physical block.

------------------- NOTE --------------------

|                                           |

| The LINCtape Handler does  not  check  to |

| determine  if  the  LINCtape  is properly |

| formatted for the specific mode.  Using a |

| LINCtape with an improper physical format |

| produces unpredictable results.           |

---------------------------------------------

UNIT            specifies the LINCtape unit number in the range 0

to 7.

RW    = 0       reads data from LINCtape.

= 4000    writes data to LINCtape.

PAGES           specifies  the  number  of  128-word   pages   to

transfer   (times   100   octal).   For  example,

PAGES=2000  transfers  20  octal  pages  or  2048

words;   if  PAGES=0,  the  Handler  transfers 40

octal pages or 4096 words.

4-29

RTS/8 SYSTEM TASKS

FIELD           specifies the PDP-8 field in which  the  transfer

takes  place  (times  10  octal).   The format of

FIELD is in the standard OS/8 mass storage  field

format.     (See   Section   4.10   for   further

information.)

BUFADD          is the starting  address  of  the  buffer  to  be

transferred.

BLOKNO          is the block number on the device from which  the

transfer begins.  The Handler assumes all devices

have 256-word blocks.  On an OS/8  LINCtape,  two

consecutive  physical  blocks  comprise  one OS/8

logical block.  Note  that  only  the  first  128

words  in  each physical block contain meaningful

data.

When running in DIAL mode,  BLOKNO  represents  a

physical  LINCtape  block  number.  In this case,

PAGES must be even because the Handler  transfers

an   even   number   of   pages.    If  PAGES  is

(incorrectly) odd, the Handler does not  transfer

the  last  page.   The  exception to this is when

PAGES=1;  this value  instructs  the  Handler  to

transfer one block (2 pages).

STATUS          is the one's complement of tape check (checksum).

The  value 0 means no error.  Note that STATUS is

always 0 on a write operation.  When the  handler

performs   any   operation,   it  attempts  three

software retries on a checksum read error.  Also,

the  hardware  performs  an  infinite  number  of

retries on most errors (write-lock-out, tape  not

mounted,  bad  spot  on tape) and does not return

control to RTS/8 until successful.

------------------- NOTE -------------------

|                                          |

| In   the   OS/8   mode,   the    Handler |

| temporarily  destroys the word following |

| the end of the buffer while  a  LINCtape |

| operation  is  in progress.  The Handler |

| restores the location when it  completes |

| the  operation.  However, since RTS/8 is |

| a  real-time   system,   code   may   be |

| executing while the tape operation is in |

| progress.  Consequently, you must ensure |

| that  no  program  ever  references this |

| word while the LINCtape is  being  used. |

| Under  no  circumstances should the word |

| following the end of the  buffer  belong |

| to another task.                         |

--------------------------------------------

4-30

RTS/8 SYSTEM TASKS

---------------------------------------------------

| 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |

---------------------------------------------------

^       ^                          \___________/

|       |                                |

0:  OS/8 mode \___|       |                                |

1:  DIAL mode /           |                                |

|                                |

RLO1 Mode ________________|                                |

|

Unit number _______________________________________________|

Figure 4-11 LINCtape Handler Unit Word Format

4-31

RTS/8 SYSTEM TASKS

4.9  LINE PRINTER HANDLER

----------------------------------------------------------------------

| Use the Line Printer Handler  to  output  information  to  a  line |

| printer.                                                           |

|                                                                    |

| Message Format 1:                                                  |

|                  ZBLOCK 3                                          |

|                  COMMND+LENGTH                                     |

|                  ZBLOCK 1                                          |

|                  OUTTXT                                            |

|                                                                    |

| Message Format 2:                                                  |

|                  ZBLOCK 3                                          |

|                  ASSGN+TSKNUM                                      |

|                                                                    |

| Message Format 3:                                                  |

|                  ZBLOCK 3                                          |

|                  -2                                                |

|                  CDF OUTBUF                                        |

|                  COMMND+LENGTH                                     |

|                  ZBLOCK 1                                          |

|                  OUTTXT                                            |

|                                                                    |

| where:                                                             |

|       COMMND    is  one  of  the  commands  listed  in  Table  4-4 |

|                 (Console Terminal Commands).                       |

|       LENGTH    is the number of characters in the line.           |

|       OUTTXT    is the first word of the output text or a  pointer |

|                 to it.                                             |

|       ASSGN     is an RTS/8 mnemonic having a value  of  200  that |

|                 causes  the  Handler  to be assigned to a specific |

|                 task (or deassigned if TSKNUM=0).                  |

|       OUTBUF    is the field of the output buffer.                 |

----------------------------------------------------------------------

You use FORMAT 1 to output text.

You use FORMAT 2 to assign the Line  Printer  Handler  to  a  specific

task.  To deassign the Handler, specify TSKNUM=0.

You use FORMAT 3 when the output buffer is in a field  different  from

that of the message.

The RTS/8 Line Printer Handler outputs to an LS8, LS8E,  LP8,  or  LV8

line  printer.   The format of messages to the Line Printer Handler is

identical to the format of messages to the Terminal Handler,  but  the

Handler  ignores  the  INBUF word of the Terminal Handler (represented

above by ZBLOCK 1) and the LINE/NOLINE bit.   (The  INBUF  word  must,

however, be present in the message.)

The following figures  illustrate  the  bits  that  the  Line  Printer

Handler uses.

4-32

RTS/8 SYSTEM TASKS

---------------------------------------------------

| 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |

---------------------------------------------------

^   ^   ^   ^   ^   \__________________________/

|   |   |   |   |____           |

Command (bits 0-4)   |   |   |   |______ |           |     __________________

0:  Packed ASCII   \_|   |   |________ | |           |

1:  Unpacked ASCII /     |__________ | | |           |

| | | |           |

0:  CR/LF at end of message    \___| | | |           |

1:  No CR/LF at end of message /     | | |           |

| | |           |

0:  OUTTXT is the first word    \____| | |           |

1:  OUTTXT points to first word /      | |           |

| |           |

0:  Input in line mode      \__________| |           |

1:  Input in character mode /            |           |

|           |

Bit 4 must be a 0 _______________________|           |

|

Length (bits 5-11)                                   |     __________________

If bit 3=1 no. of characters to input   \____________|

If bit 3=0 maximum size of input buffer /

Figure 4-12  Line Printer Handler I/O Mode Command and

Length Word Format

---------------------------------------------------

| 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |

---------------------------------------------------

\_____________/  ^   ^  \_______________________/

|         |   |             |

Unused _________________|         |   |             |

|   |             |

Bit 4 must be a 1 ________________|   |             |

|             |

Unused _______________________________|             |

|

Task Number ________________________________________|

Figure 4-13 Line Printer Handler ASSGN Mode Command Word Format

4-33

RTS/8 SYSTEM TASKS

4.10  MASS STORAGE HANDLERS

----------------------------------------------------------------------

| Use the Mass Storage Handlers to perform  I/O  on  DF32  and  RF08 |

| fixed head disks, RK8 and RK8E moving-head disks, and TC08 DECtape |

| units.                                                             |

|                                                                    |

| Message Format 1:                                                  |

|                  ZBLOCK 3                                          |

|                  UNIT                                              |

|                  RW+PAGES+FIELD                                    |

|                  BUFADD                                            |

|                  BLOKNO                                            |

|                  STATUS                                            |

|                                                                    |

| Message Format 2:                                                  |

|                  ZBLOCK 3                                          |

|                  1000+UNIT                                         |

|                  RW+PAGES+FIELD                                    |

|                  BUFADD                                            |

|                  BLOKLO                                            |

|                  STATUS                                            |

|                  BLOKHI                                            |

----------------------------------------------------------------------

------------------ NOTE --------------------

|                                          |

| Floppy Disk and  LINCtape  mass  storage |

| usages  are  discussed elsewhere in this |

| manual.                                  |

--------------------------------------------

You normally use Message Format 1 for all devices that have less  than

4096  blocks.   This  message  format is compatible with the OS/8 file

structure.  For devices which have more than 4096 blocks (such  as  an

RK8E  or  an RL01), the handler divides the physical unit into several

logical units.

Message Format 2 supports 24 bit device addresses (as opposed  to  the

12  bit  address of Format 1).  However, Format 2 is incompatible with

OS/8 for devices which have more than 4096  blocks.   Note  that  this

message format has one logical device for each physical device.

If your device has less than 4096 blocks,  Format  2  gives  the  same

results as Format 1.

The parameters of the above format statement are as follows:

UNIT      is  the  number  of  the  logical  unit  on  which  the

operation  is  to  be  performed.   DF32 and RF08 disks

consist of only one unit.   TC08  DECtape  has  logical

units  0-7  corresponding  to their physical units 0-7.

RK8 disks have logical units 0-3 corresponding to their

physical  units  0-3.  RK8E disk has logical units 0-7.

4-34

RTS/8 SYSTEM TASKS

In this case, units 0-3 correspond to the outer  (lower

track number) half of physical units 0-3, and units 4-7

correspond to the inner (higher track number)  half  of

physical units 0-3, respectively.

In  Format  1,  the  RL01  physical  unit/logical  unit

correspondence is as follows:

Physical   Logical

Unit       Unit

0      0,  1,  2

1      4,  5,  6

2     10, 11, 12

3     14, 15, 16

------------------- NOTE -------------------

|                                          |

| In Format 1,  a  physical  unit  may  be |

| divided into more than one logical unit; |

| for example, an  RK05  disk  is  divided |

| into  RKA0  and  RKB0.  In Format 2, the |

| absolute  block  format,  each  physical |

| unit is a logical unit.  Here, you would |

| specify the  block  number  by  the  two |

| words  BLOKHI  and  BLOKLO.   This means |

| that Format 2  is  not  compatible  with |

| OS/8 for an RK05 or an RL01.             |

--------------------------------------------

RW        is 0  for  a  read  operation  and  4000  for  a  write

operation.

PAGES     specifies the number of (128-word)  pages  to  transfer

(times  100  octal).  For example, PAGES=2000 specifies

the transfer of 20 (octal) pages  of  2048  words;   if

PAGES=0, the Handler transfers 40 (octal) pages or 4096

words.

FIELD     is the PDP-8 field in which the  transfer  takes  place

(times  10  octal).   For  example,  if  FIELD=30,  the

transfer takes place in field 3.

All FIELD specifications for Mass Storage Handlers  are

in  bits  6-10.   Of the five field bits, the low order

three bits are in positions 6-8 (not 8-10).   The  high

order  bit is in bit 10 and the remaining bit is in bit

9.

If you think of the normal bit order  as  being  ABCDE,

then  the  order into which you place these bits in all

Mass Storage Handlers is CDEBA.

4-35

RTS/8 SYSTEM TASKS

Note  that  the  MACREL  directive  XEDF  automatically

places the bits into this order.

BUFADD    is  the  starting  address  of   the   buffer   to   be

transferred.

BLOKNO    is the block  number  on  the  device  from  which  the

transfer  begins.  The Handler assumes 256-word blocks.

On DECtape, the first 128 words of each  even/odd  pair

of 129-word DECtape records are defined as a block.

STATUS    is a word that the Handler sets on  completion  of  the

operation.   It  contains  a  zero  if the operation is

successful, otherwise it contains  a  nonzero  quantity

that  is  the  contents  of the device status register.

Tasks that use the Mass Storage  Handlers  should  test

this word after it completes an I/O operation (that is,

after the event flag has been posted) to  determine  if

any  errors   occurred  during the transfer.  All RTS/8

Mass Storage Handlers retry operations three  times  if

they encounter errors before setting the STATUS word to

a nonzero number.  For the meanings of the STATUS word,

see the appropriate device manual.

BLOKHI   is the high order part of the block address.

BLOKLO   is the low order part of the block address.

The middle three words of a message to the RTS/8 Mass Storage Handlers

are  identical  to  the arguments to an OS/8 Handler when they perform

the same operation.

The following figures illustrate bit positions which the Handler uses.

---------------------------------------------------

| 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |

---------------------------------------------------

^        ^  \_________________/ \_______________/

Call Format _____|        |           |                 |

|           |                 |

RL01 Mode ________________|           |                 |

|                 |

Reserved for task use ________________|                 |

|

Unit ___________________________________________________|

Figure 4-14 Mass Storage Handlers Unit Word Format

4-36

RTS/8 SYSTEM TASKS

---------------------------------------------------

| 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |

---------------------------------------------------

^  \_________________/ \__________________/   ^

|           |                   |             |

|_____      |                   |             |

|      |                   |             |

0:  Read operation  \__|      |                   |             |

1:  Write operation /         |                   |             |

|                   |             |

No. of pages to transfer _____|                   |             |

|             |

0-37:  Field of transfer _________________________|             |

|

Reserved for task use __________________________________________|

Figure 4-15 Mass Storage Handlers Function Word Format

Examples:

CAL

SENDW

DTA             /SEND A MESSAGE TO THE DECTAPE

/ HANDLER

DTAMSG          /WAIT FOR COMPLETION

TAD STATUS      /CHECK THE STATUS OF THE OPERATION

SZA CLA

JMP ERR         /BAD--GO TO ERROR ROUTINE

.               /OK--CONTINUE PROCESSING

.

.

DTAMSG,     ZBLOCK 3        /MESSAGE HEADER

4               /DECTAPE UNIT 4

4210            /WRITE 256 WORDS FROM FIELD 1

BUFFER          /ADDRESS OF BUFFER

55              /INTO BLOCK 55 (RECORDS 132 & 133)

STATUS,     0               /STATUS OF OPERATION STORED HERE

4-37

RTS/8 SYSTEM TASKS

4.11  OS/8 FILE SUPPORT TASK

----------------------------------------------------------------------

| Use the OS/8 file support task to  look  up,  create,  and  delete |

| files in OS/8 directories from a foreground task.                  |

|                                                                    |

| Message Format:                                                    |

|                ZBLOCK 3                                            |

|                DEVHND 10+UNIT+FUNCT                                |

|                FILPTR                                              |

|                STATUS                                              |

|                BLOKNO                                              |

|                LENGTH                                              |

----------------------------------------------------------------------

This task allows you to look up,  create, and  delete  files  in  OS/8

directories  in  the  same manner as if you were using OS/8.  You need

not have the OS/8 support task in your system to be able to  use  this

task.

If both OS8F and the OS/8 Support Task are present  in  a  system,  an

interlock  is  set  up  to  prevent simultaneous updating of directory

blocks by both tasks.  Note that because OS/8 tends to leave directory

blocks  in  memory  for  long  periods  of time, this interlock scheme

causes lengthy delays for the OS8F task.

The parameters of the above format statement are as follows:

DEVHND          is the task number of the Handler for the desired

device.

UNIT            is the unit number on which the operation  is  to

be performed.

FUNCT  = 0      looks up the filename and  returns  its  starting

block  number in BLOKNO, and its length in LENGTH

(as a two's complement number).

= 2000   enters the specified filename into the first

empty space (on the device) whose length is equal

to or exceeds the value in LENGTH.  It returns

the starting block number of the new file in

BLOKNO.  If a file of the same name previously

existed on the device, the support task deletes

the file. However, it does not change the value

of LENGTH.

= 4000   deletes the specified filename.

FILPTR            is a pointer to a 4-word  filename  in  the  same

field  as  the message.   You  can use the MACREL

pseudo-op FILENAME to generate these filenames.

STATUS            describes the final status of  the  operation  as

follows:

4-38

RTS/8 SYSTEM TASKS

0    Operation successful.

1    File not found on a lookup or delete

operation.

2    No room for file on enter operation.

>2   I/O error.  The value is the hardware error

status of the device.

-1   Invalid directory on device.

---------------------------------------------------

| 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |

---------------------------------------------------

\_____/ \_________________________/ \___________/

|                 |                    |

Function: __________|                 |                    |

O Lookup                              |                    |

1 Enter                               |                    |

2 Delete                              |                    |

3 Unused                              |                    |

|                    |

Task number __________________________|                    |

|

Unit ______________________________________________________|

Figure 4-16 OS8F Call Function Word

Before it performs a delete or enter operation, OS8F waits until  OS/8

is in a state in which:

1.  There  is  no  active  temporary  file  on  the  OS/8  device

corresponding to devhnd and unit.

2.  OS/8 loads the Keyboard Monitor, Command Decoder, or USR into

core.

RTS/8 does not interlock look up operations since they do  not  modify

the directory.

4-39

RTS/8 SYSTEM TASKS

4.12  OS/8 SUPPORT TASK

----------------------------------------------------------------------

| The OS/8 support task supports the execution of an OS/8 operating  |

| system in the background of a foreground/background system.        |

----------------------------------------------------------------------

The OS/8 Support Task executes the OS/8 operating  system  as  a  task

under RTS/8.  Note that the OS/8 File Support Task is not necessary to

run this task.  OS/8 is usually run in the  top  two  or  more  memory

fields  of  a  system.   However,  if  the  system  has a KT8-A memory

controller, the OS/8 fields, although  still  contiguous,  may  reside

elsewhere.

A user incorporates the OS/8 Support Task into a  system  when  it  is

generated.    When   RTS/8   initializes   OS/8,   it   establishes  a

correspondence between OS/8 devices and  RTS/8  Handler  tasks.   Note

that  terminal input and output from OS/8 are ring-buffered by several

characters to minimize input loss due to the stealing of  the  CPU  by

tasks  of higher priority.  Because of the large number of trapped CDF

instructions in OS/8 and its utilities, response time is  slower  than

that  of  a  stand-alone  OS/8  system;   however,  it  is still quite

reasonable.

KT8A systems execute background CDFs, RDFs, and RIFs in the  hardware.

Consequently,  background processing runs at nearly the same speed as

that of OS/8 running alone.

Because a foreground task and the background OS/8 operating system may

share  the  same  terminal,  you may interactively specify whether the

terminal belongs to the foreground or the background.  If you  type  a

CONTROL/B  (^B)  and  a  carriage  return  or  an ESCAPE, the Terminal

Handler gives control  of  the  terminal  to  OS/8.   If  you  type  a

CONTROL/F  (^F)  while  OS/8  has  control,  the Terminal Handler will

return to RTS/8 foreground operation.  For additional information, see

Section 4.5.

If you type a B while the terminal belongs to the background or  a  ^F

while it belongs to the foreground, the terminal treats the command as

a no-opt

The OS/8 system that runs under this  task  runs  all  OS/8  utilities

except BUILD, BOOT, PIP10, RXCOPY, INDUSTRIAL BASIC, and the BASIC and

FORTRAN laboratory runtime functions.  If the console terminal and the

OS/8  terminal are not the same, the Handler diverts all references to

the appropriate device.  In the same manner, the Handler also  diverts

references  in OS/8 to the LEO, LS8E, LP8, or LV8 line printers to the

RTS/8 Line  Printer  Handler  if  the  RTS/8  system  has  this  task;

otherwise,   the  support  task  executes  them.   RTS/8  diverts  the

following OS/8 device  name  references  to  the  corresponding  RTS/8

Handler (if that Handler exists in the RTS/8 system):

DTA0-DTA7

LTA0-LTA7

RKA0-RKA3

4-40

RTS/8 SYSTEM TASKS

RKB0-RKB3

RLOA-RL6A

RXA0-RXA7

In addition, RTS/8 diverts the  OS/8  SYS  and  DSK  Handlers  to  the

handlers specified when the system was generated.  Other references to

I/O under the OS/8 system may cause the OS/8 support task to hang in a

loop.  References to a handler called RTS8 are diverted to OS8COM (see

Section 4.12.1).

------------------ NOTE --------------------

|                                          |

| If neither the support task nor OS/8 are |

| configured  for the handlers you wish to |

| use, then you should run OS/8  alone  on |

| the  system and use BUILD to incorporate |

| the  handlers  into  OS/8.   After   you |

| configure   OS/8,  you  may  run  RTS/8. |

--------------------------------------------

If the background OS/8 task is in a loop or hangs up, you  may  reboot

it without disturbing the foreground by typing a CONTROL/Y ( Y).  This

reads the OS/8 system header blocks off the  OS/8  system  device  and

restarts the OS/8 background.

RTS/8 prints

FATAL BKG ERROR

(fatal background error)

or

BOOT ERROR

if it cannot start or restart OS/8 or exit from RTS/8 to OS/8.   These

error  messages  appear  on  the  background  terminal. These messages

indicate that the background OS/8 task cannot run.  However, all other

RTS/8   tasks   still  execute.   These  messages  may  occur  if  you

inadvertently attempt to run OS/8 from a non-system disk.

Recovery from this error is not possible while running RTS/8.  To  use

OS/8  you  must  abort  RTS/8  and  then restart the system, this time

having the RTS/8.SV image on the OS/8 system disk.

4.12.1  OS/8-RTS/8 Communication (OS8COM)

an RTS/8 task.  To perform this communication, the system must include

the RTS8 Handler.  This Handler can be a dummy;  that is, it need  not

do  anything.   (In  fact,  it  can be some other Handler to which you

assign the name RTS8 with an OS/8 ASSIGN command or through  the  OS/8

BUILD program.)

The OS/8 Support Task traps all calls to this Handler.  The  arguments

that an OS/8 program attempts to pass to the RTS8 Handler will instead

4-41

RTS/8 SYSTEM TASKS

be passed to an RTS8 task called OS8COM.

The OS8COM task performs an RTS/8  RECEIVE  ER.   The  task  can  then

receive a message any time an OS/8 program reads or writes to the RTS8

Handler.  This message looks like any other message to a mass  storage

device.   However, OS8SUP makes one change to the arguments.  Bits 6-8

of the function word originally contain the field of the  buffer,  and

this  is  the field where OS/8 expects the buffer to be located.  When

OS8COM gets  control,  these  bits  identify  the  actual  field  that

contains  the  buffer.  OS8COM can then return information to the OS/8

through these arguments.

An OS/8 program that runs an RTS/8 task  is  shown  in  the  following

example:

USER=7700       /LOCATION OF OS/8 USER SERVICE ROUTINE

JMS PRINT       /PRINT MESSAGE "WHAT TASK WOULD YOU

/   LIKE TO RUN?" ON THE OS/8 TERMINAL

JMS READ        /READ RESULTS FROM OS/8 KEYBOARD

/   RETURNS TASK NUMBER IN RANGE 1-177

/   (OCTAL) IN AC

DCA TASKNO      /STORE IT AWAY

CIF 10

JMS I (USR      /CALL USR

1               /TO DO A FETCH

DEVICE RTS8     /OF DEVICE "RTS8"

ENTRY,  ADDR            /DUMMY ADDRESS (HANDLER WILL ALREADY

/   BE RESIDENT)

HLT             /ERROR (HANDLER NOT FOUND)

/NOTE:  THIS CODE IS NOT REUSABLE.  ALSO, LOCATION "ENTRY"

/IS SET TO THE ENTRY POINT FOR THIS HANDLER

CIF 0

JMS I ENTRY     /CALL HANDLER

0               /DUMMY READ

TASKNO, 0               /TASK NUMBER

ZBLOCK 2        /DUMMY

JMP I (7605     /RETURN TO OS/8

You should note that the task passes TASKNO  as  the  second  argument

instead of the first because OS8SUP automatically modifies bits 6-8 of

the first argument, presuming that it will find a mapped field  number

there.

OS8COM expects three arguments after the Handler call  plus  an  error

return.  You must supply these arguments.

After writing the OS/8 portion of the program, you must also write the

OS8COM  task  that handles the RTS/8.  You write OS8COM like any other

RTS/8 user task, as the following example illustrates:

4-42

RTS/8 SYSTEM TASKS

.TASK OS8COM            /TASK NUMBER IS OS8COM

/IN THE PARAMETER FILE

/NAMETAB GETS 'OS8COM'

/IT IS RUNNABLE BY DEFAULT

.RSECT OS8SCT,LEVEL=0   /ARBITRARY SECTION NAME

START,     CAL

RECEIVE         /IMMEDIATELY GO INTO RECEIVE WAIT

MADDR,     0               /ADDRESS OF MESSAGE LEFT HERE

DCA MSGFLD      /CDF TO MESSAGE FIELD LEFT IN AC

MSGFLD,    HLT

ISZ MADDR       /POINT TO FUNCTION WORD

ISZ MADDR       /POINT TO BUFFER ADDRESS

/   (SECOND OS/8 ARGUMENT)

TAD I MADDR     /GET TASK NUMBER

CAL

RUN             /RUN THIS TASK

/OS8COM WANTS TO BE HIGHER PRIORITY

/   THAN TASK IT IS RUNNING

TAD MSGFLD

DCA EFCDF

TAD (-5)

TAD MADDR       /GET ADDRESS OF EVENT FLAG MESSAGE

CAL

POST            /POST MESSAGE

EFCDF,     HLT

JMP START       /GET ANOTHER MESSAGE

In this example, the task number was put in the second argument of the

OS/8  call.   However,  it  became the third word of the RTS/8 message

because OS8SUP always adds a word--namely the unit number--to the mass

storage  call  argument  list.   For a description of the OS/8 Handler

call format, see Section 4.1 of the OS/8 Software Support Manual.  For                                         ____________________________

a  description of the message format for mass storage devices, see the

appropriate sections in this manual.

4.12.2  Other Techniques

Other techniques which you may employ are:

1.  If the RTS/8 Handler STATUS word  (word  5)  of  the  message

which  OS8COM  posts  is nonzero, then the handler returns to

OS/8 at the error return of the Handler call.

2.  You may pass arguments back  to  OS/8  through  the  argument

list.

3.  If you pass more than three words  of  data  to  OS8COM  from

OS/8,  you  can  pass a CDF and the address of the area where

the data resides.  If the CDF occurs as the first argument to

the  Handler  call,  the  Handler  automatically relocates it

before passing it to OS8COM.  Note that  the  field  of  data

must be in the locations that OS/8 uses and not the locations

which RTS/8 can use.

4-43

RTS/8 SYSTEM TASKS

4.13  PDP-8A NULL TASK

----------------------------------------------------------------------

| Use the PDP-8A null task  to  count  in  decimal  on  the  console |

| accumulator LED display.                                           |

----------------------------------------------------------------------

The PDP-8A null  task  counts  from  1  to  9999  in  decimal  in  the

accumulator  display,  and  counts  from  1 to 7777 in octal in the MQ

display.

4-44

RTS/8 SYSTEM TASKS

4.14  POWER FAIL TASK

Use this task to provide for an orderly shutdown when  power  is  lost

and to allow a programmed restart when power returns.

The Power Fail  Task  provides  the  mechanism  by  which  the  system

recovers  from  a  power failure.  You use this task only with systems

that have core memory.

If a "power low" condition occurs, the power fail hardware  interrupts

the  running  task.   The  power  fail  interrupt  code then saves the

processor state and halts the  processor.   When  power  returns,  the

power  fail  interrupt  code restores the processor state and posts an

event flag which wakes up the Power Fail Task.  The  Power  Fail  Task

restores  the clock (if it is present) and also performs an action for

each task in the system based on the contents of  an  internal  table.

Each task has a 1-word entry in this table, which contains:

0         if nothing  should  be  done  for  this  task  (default

value).

-1        if the EFWT (event flag wait) bit should be cleared  in

the Task Flags Table entry for this task.

ADDR      if the task should be derailed to location ADDR in  the

field  in which it is executing as well as clearing its

EFWT bit.

The Power Fail Task uses the DERAIL Executive Request to  provide  for

power fail recovery.

Each task in the system may alter its entry in the Power  Fail  Task's

table  by sending a message to the Power Fail Task.  The format of the

message is:

PWRMSG,    ZBLOCK 3

WORD

where:

WORD       is the new contents of the  Power  Fail  Task's  table

entry  for the sending task (that is, this task should

be taken out of Event Flag Wait).

The Handler call is as follows:

CAL

SEND

PWRF

PWRMSG

.

.

.

PWRMSG, ZBLOCK 3

VALUE

4-45

RTS/8 SYSTEM TASKS

4.15  UDC/ICS HANDLER

The UDC/ICS Handler gives you the capability to  control  the  various

types  of  UDC/ICS functional devices.  The Handler performs two types

of  action:   immediate  and  associated.  Immediate  actions  include

reading  and  sending analog and digital values to appropriate UDC/ICS

functional devices.  You may also link associated actions to specified

events  within  the  UDC/ICS  (counters  overflowing,  switches  being

thrown, etc.).  The associated actions can do the following:

1.  Run a specified task when the event occurs.

2.  Set the Event Flag when the event occurs.

3.  DERAIL a specified task when the event occurs.

The number of associated requests that can be  pending  simultaneously

is  established  when you generate an RTS/8 system.  (See Chapter 2 of

the RTS/8 System Generation Manual for more information.)         ______________________________

The UDC/ICS Handler permits the following operations:

1.  Analog Input        - accepts input from analog subchannel

2.  Analog Output       - sends a 10-bit value to an analog

channel

3. Change of State      - finds the current Change-of-State

value for a contact channel

4. Digital Input        - reads a digital signal

5. Digital Output       - sends a 12-bit value to a digital

channel

6. Disable Contacts     - ignores interrupts from a contact

channel

7. Disable Counter      - disables interrupts from a counter

channel

8. Enable Contacts      - permits interrupts from a contact

channel

9. Enable Counter       - permits interrupts from a counter

channel

10. Get Generic Code    - determines the generic code for a

specified channel

11. Read Counter        - reads the channel value of the counter

channel

4-46

RTS/8 SYSTEM TASKS

Each of these operations  is  discussed  in  the  following  sections,

including the format of the message for specifying the operation.  The

RTS/8 Executive uses the first three words of a message;   the  fourth

word  specifies  one  of the eleven UDC/ICS operations listed in Table

4-7.

Table 4-7

UDC/ICS Operations

-------------------------

|  UDC/ICS   |    Value |

|------------|----------|

|    AO      |       0  |

|    DO      |       1  |

|    DI      |       2  |

|    GC      |       3  |

|    EC      |       4  |

|    RC      |       5  |

|    DC      |       6  |

|    ECT     |       7  |

|    CS      |      10  |

|    DCT     |      11  |

|    AI      |      12  |

-------------------------

Word 5 of a UDC/ICS message designates the channel the device uses for

the operation;  furthermore, certain operations may also require words

5 through 8 for their complete specification.  Word  9  specifies  the

value  read  or  the  value returned.  Finally, word 10 of all UDC/ICS

messages contains the error state (status).

The general format for a UDC/ICS is:

ZBLOCK 3

operation

channel

opword 1

opword 2

opword 3

value

status

The UDC/ICS Handler does not change the value of the  status  word  in

some operations unless an error occurs.  Consequently, you must always

clear this location before performing an operation.  As with all other

handler  status  words,  you  should  check  this location after every

operation to insure that the operation executes correctly.

4-47

RTS/8 SYSTEM TASKS

4.15.1  AI (Analog Input)

----------------------------------------------------------------------

| Use the AI operation to accept input from an analog channel.       |

|                                                                    |

| Message Format:                                                    |

|                ZBLOCK 3                                            |

|                AI                                                  |

|                CHANNL                                              |

|                SUBCHN & GAIN                                       |

|                ANSWER                                              |

|                ZBLOCK 2                                            |

|                STATUS                                              |

|                                                                    |

| where:                                                             |

|       AI        is an RTS/8 mnemonic for Analog Input.             |

|       CHANNL    is the Analog Input Channel,                       |

|       SUBCHN    (in bits 9-11 for UDC and 5-11  for  ICS)  is  the |

|                 subchannel.                                        |

|       GAIN      (in bits 1-3) is the signal amplitude.             |

|       ANSWER    is the value to be received.                       |

|       STATUS    is the error status.                               |

----------------------------------------------------------------------

The Handler automatically sets bit 0 (enable conversion) and reads the

control  register  (UDC  bit 8, ICS bit 4).  The ICS analog converters

must have addresses that are less than 20 (octal) since all  converter

modules  must be located in the first 16 slots of the ICS unit.  After

conversion, the Handler places the digitized value in the answer word.

---------------------------------------------------

| 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |

---------------------------------------------------

\_____/ \_______________________________________/

Subchannel _________|                       |

|

Value ______________________________________|

Figure 4-17 UDC/ICS Handler Subchannel and Gain Word Format

An example of a message for an input channel is:

AIEX,   ZBLOCK 3

AI                 /ANALOG INPUT

17                 /CHANNEL 17

3                  /SUBCHANNEL 3, GAIN 1

AIANS,  0                  /RESULT HERE

ZBLOCK 2

AIERR, 0                   /ERROR INDICATOR

You should ensure that for each major channel there is sufficient time

(approximately  250 microseconds for UDC;  5 milliseconds for ICS) for

each  subchannel  conversion  to  be  completed  before   another   is

indicated.  In  general,  it  may be helpful if one task initiates all

analog-to-digital conversions for a major channel.

4-48

RTS/8 SYSTEM TASKS

4.15.2  AO (Analog Output)

----------------------------------------------------------------------

| Use an AO operation to send a 10-bit value to an analog channel.   |

|                                                                    |

| Message Format:                                                    |

|                ZBLOCK 3                                            |

|                AO                                                  |

|                CHANNL                                              |

|                SUBCHN & VALUE                                      |

|                ZBLOCK 3                                            |

|                STATUS                                              |

|                                                                    |

| where:                                                             |

|       AO        is an RTS/8 mnemonic for Analog Output.            |

|       CHANNL    is the Analog Output channel.                      |

|       SUBCHN    (bits 0-1) is the subchannel.                      |

|       VALUE     (bits 2-11) is the value to be transmitted.        |

|       STATUS    is the error status.                               |

----------------------------------------------------------------------

The Analog Output operation  takes  a  value  which  you  specify  and

transforms  it  into  a  form acceptable to an analog device.  It then

writes this value, whose range is from 0 to 1023, to  the  appropriate

subchannel.

The format of the subchannel and value word is shown in Figure 4-18.

---------------------------------------------------

| 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |

---------------------------------------------------

^  \_________/  ^  \___________________________/

|__     |       |               |

Enable conversion __|     |       |               |

|       |               |

Gain _____________________|       |               |

|               |

ICS read control register; _______|               |

(for UDC, bit 8)                                  |

|

ICS subchannel; __________________________________|

(for UDC, bits 9-11)

Figure 4-18  UDC/ICS Handler Subchannel and Value Word Format

Example:

AOEX,   ZBLOCK 3

AO                /ANALOG OUTPUT

23                /CHANNEL 23

4614              /SUBCHANNEL 2, VALUE 614

ZBLOCK 3

AOERR,  0                 /ERROR INDICATOR

4-49

RTS/8 SYSTEM TASKS

4.15.3  CS (Change of State)

----------------------------------------------------------------------

| Use a CS operation to find the current change of state value for a |

| contact channel.                                                   |

|                                                                    |

| Message Format:                                                    |

|                ZBLOCK 3                                            |

|                CS                                                  |

|                CHANNL                                              |

|                RESULT                                              |

|                ZBLOCK 3                                            |

|                STATUS                                              |

|                                                                    |

| where:                                                             |

|       CS        is an RTS/8 mnemonic for Change of State.          |

|       CHANNL    is the contact channel  whose  current  change  of |

|                 state value is to be placed in RESULT.             |

|       STATUS    is the error status.                               |

----------------------------------------------------------------------

You use the CS operation to discover if the  "state"  (defined  to  be

either a logical 0 or 1) of a peripheral device changes.  If the state

changes, then the device is performing a different operation  than  it

performed  previously.   Note  that  a  user  task  must keep track of

previous states.

The function of this operation is to mask those bits that differ  from

their  previous  state  during a given interrupt.  Because the handler

does this operation, you need not  perform  an  exclusive  OR  at  the

interrupt level.

Example:

COSEX,  ZBLOCK 3

CS                  /READ COS

1                   /CHANNEL 1

COSANS, 0                   /RESULT HERE

ZBLOCK 3

COSERR, 0                   /ERROR INDICATOR

4-50

RTS/8 SYSTEM TASKS

4.15.4  DI (Digital Input)

----------------------------------------------------------------------

| Use a DI operation to read a digital signal.                       |

|                                                                    |

| Message Format:                                                    |

|                ZBLOCK 3                                            |

|                DI                                                  |

|                CHANNL                                              |

|                RESULT                                              |

|                ZBLOCK 3                                            |

|                STATUS                                              |

|                                                                    |

| where:                                                             |

|       DI        an RTS/8 mnemonic for Digital Input.               |

|       CHANNL    is the legal digital input channel.                |

|       RESULT    contains the value of the channel when read.       |

|       STATUS    is the error status.                               |

----------------------------------------------------------------------

The DI operation allows  you  to  receive  a  digital  signal  from  a

peripheral  device  and  to  store  it into a location in memory.  The

Handler reads this number, which may be  either  signed  or  unsigned,

from  the indicated channel and places it in the sixth block of the DI

message.

Example:

DIEX,   ZBLOCK 3

DI                /DIGITAL INPUT

27                /CHANNEL 27

DIANS,  0                 /VALUE OF CHANNEL 27 WILL BE HERE

ZBLOCK 3

DIERR,  0                 /ERROR INDICATOR

4.15.5  DO (Digital Output)

----------------------------------------------------------------------

| Use a DO operation to send a 12-bit value to a digital channel.    |

|                                                                    |

| Message Format:                                                    |

|                ZBLOCK 3                                            |

|                DO                                                  |

|                CHANNL                                              |

|                VALUE                                               |

|                ZBLOCK 3                                            |

|                STATUS                                              |

|                                                                    |

| where:                                                             |

|       DO        is an RTS/8 mnemonic for Digital Output.           |

|       CHANNL    is a legal digital output channel.                 |

|       VALUE     is the number to be written.                       |

|       STATUS    is the error status.                               |

----------------------------------------------------------------------

4-51

RTS/8 SYSTEM TASKS

The Digital Output operation allows you to send a computed  or  stored

value  to  a  digital  peripheral  device.   This number may be either

signed or unsigned.  As not all digital devices  can  receive  a  full

12-bit word, you must ensure that the value transmitted is appropriate

for the device.

Example:

DOEX,   ZBLOCK 3

DO                 /DIGITAL OUTPUT

20                 /CHANNEL 20

7777               /VALUE = 7777

ZBLOCK 3

DOERR,  0                  /ERROR INDICATOR

4.15.6  DCT (Disable Contacts)

----------------------------------------------------------------------

| Use a DCT operation to ignore interrupts from a contact channel.   |

|                                                                    |

| Message Format:                                                    |

|                ZBLOCK 3                                            |

|                DCT                                                 |

|                BIT & CHANNL                                        |

|                ZBLOCK 4                                            |

|                STATUS                                              |

|                                                                    |

| where:                                                             |

|       DCT       in an RTS/8 mnemonic for Disable Contacts.         |

|       BIT       (bits 0-3, value 0-13 octal) specifies the bit  on |

|                 the   contact   channel   from  which  to  disable |

|                 interrupts.                                        |

|       CHANNL    (bits 4-11) is the contact channel.                |

|       STATUS    is the error status.                               |

----------------------------------------------------------------------

The DCT operation disables interrupts from a contact channel.  Because

many  analog  devices  transmit  a  continuous stream of values, these

values can easily fill up system queues and values may be lost.

If you use this operation in conjunction with the Clock  Handler,  you

may  schedule  periodic  intervals  in  which you will receive (or not

receive) information from the channel.

Example:

DCTEX,  ZBLOCK 3

DCT                /DISABLE CONTACTS

5401               /FROM CHANNEL 1, BIT 13 (OCTAL)

ZBLOCK 4

DCTERR, 0                  /ERROR INDICATOR

4-52

RTS/8 SYSTEM TASKS

4.15.7  DC (Disable Counter)

----------------------------------------------------------------------

| Use a DC operation to disable interrupts from a counter channel.   |

|                                                                    |

| Message Format:                                                    |

|                ZBLOCK 3                                            |

|                DC                                                  |

|                CHANNL                                              |

|                ZBLOCK 4                                            |

|                STATUS                                              |

|                                                                    |

| where:                                                             |

|       DC        is an RTS/8 mnemonic for Disable Channel.          |

|       CHANNL    is the counter channel from which the  Handler  is |

|                 to ignore interrupts.                              |

|       STATUS    is the error status.                               |

----------------------------------------------------------------------

You use the DC operation  to  prevent  the  processor  from  receiving

interrupts  from  a  counter device.  Since a counter device may often

send more interrupts than the  handler  can  service,  this  operation

disables  the  interrupts so that it can properly service a portion of

them.

Example:

DCEX,   ZBLOCK 3

DC                 /DISABLE COUNTER

6                  /CHANNEL 6

ZBLOCK 4

DCERR,  0                  /ERROR INDICATOR

4-53

RTS/8 SYSTEM TASKS

4.15.8  ECT (Enable Contacts)

----------------------------------------------------------------------

| Use an ECT operation to permit interrupts from a contact channel.  |

|                                                                    |

| Message Format:                                                    |

|                ZBLOCK 3                                            |

|                ECT                                                 |

|                BIT & CHANNL                                        |

|                ACTION                                              |

|                ADDR                                                |

|                ZBLOCK 4                                            |

|                STATUS                                              |

|                                                                    |

| where:                                                             |

|       ECT       is an RTS/8 mnemonic for Enable Contacts.          |

|       BIT       (bits 0-3, value 0-13 octal) is the contact bit.   |

|       CHANNL    (bits 4-11) is the contact channel from  which  to |

|                 enable interrupts.                                 |

|       ACTION    specifies what happens when the interrupt occurs.  |

|       ADDR      is an optional address of the task  that  services |

|                 the interrupt.                                     |

|       STATUS    is the error status.                               |

----------------------------------------------------------------------

This operation specifies the action that the computer takes  after  an

interrupt  from  a  contact  channel  occurs.   You  must specify this

operation before the handler ever receives an interrupt  and  after  a

Disable Contact operation if you wish to process additional interrupts.

Because the channel has twelve contacts, the handler  requires  twelve

messages to enable the entire channel.

ACTION  and  ADDR  specify  what  happens  when  the  contact  channel

interrupts.   You  can indicate three mutually exclusive possibilities

by setting the appropriate bit in the event ACTION word as follows:

Bit 0 = 1  -   Set  Event  Wait  Flag  of  this  job;    continue

execution  of this job when the event occurs.  The

Handler does not use the address word.

Bit 1 = 1  -   Run a  task  that  sent  the  message;   run  task

specified by bits 4-11 of event ACTION word.

Bit 2 = 1  -   DERAIL  the  task  that  sent  the  message;   the

address  is  used only by the DERAIL operation and

specifies the address of  the  DERAIL  subroutine.

The  subroutine  must  be in the same field as the

calling task.

Bit 3 = 1  -   Do action just once.  If bit 3 =  0,  the  Handler

performs  the  action after each interrupt.  Bit 3

indicates whether the action is to occur  once  or

repeatedly.

4-54

RTS/8 SYSTEM TASKS

Examples:

ECTEX1, ZBLOCK 3

ECT                /ENABLE CONTACTS

5401               /FROM BIT 13 (OCTAL) OF CHANNEL 1

2013               /RUN TASK 13 AFTER AN EVENT OCCURS

ZBLOCK 3

ECTER1, 0                  /ERROR INDICATOR

ECTEX2, ZBLOCK 3

ECT                /ENABLE CONTACT

1001               /FROM BIT 2 OF CHANNEL 1

4000               /ON 1ST OCCURRENCE OF EVENT, POST

/   THE EVENT FLAG

ZBLOCK 3

ECTER2, 0                  /ERROR INDICATOR

4.15.9  EC (Enable Counter)

----------------------------------------------------------------------

| Use an EC operation to permit interrupts from a counter channel.   |

|                                                                    |

| Message Format:                                                    |

|                ZBLOCK 3                                            |

|                EC                                                  |

|                CHANNL                                              |

|                INIT                                                |

|                RELOAD                                              |

|                ACTION                                              |

|                ADDR                                                |

|                STATUS                                              |

|                                                                    |

| where:                                                             |

|       EC        is an RTS/8 mnemonic for Enable Counter.           |

|       CHANNL    is the counter channel to be enabled.              |

|       INIT      is the  initial  value  to  be  loaded  into  that |

|                 channel.                                           |

|       RELOAD    is the value with  which  to  reload  the  channel |

|                 after every event.                                 |

|       ACTION    specifies what action to take after the  interrupt |

|                 occurs (see Enable Contact for explanation).       |

|       ADDR      is the address of a DERAIL subroutine.             |

|       STATUS    is the error status.                               |

----------------------------------------------------------------------

The EC operation specifies the action  that  the  computer  will  take

after  an  interrupt  from a counter channel occurs.  You must specify

this operation before the handler receives any interrupts and after  a

Disable Counter operation if you wish to process additional interrupts.

Examples:

4-55

RTS/8 SYSTEM TASKS

EXEX1,  ZBLOCK 3

EC                 /ENABLE COUNTER

4                  /CHANNEL 4

7700               /INITIAL VALUE OF 7700

7710               /RESET TO 7710 AFTER EACH EVENT

4000               /POST EVENT FLAG ON EVENT EVERY TIME

/   IT OCCURS

ZBLOCK 1

ECERR1, 0                  /ERROR INDICATOR

ECEX2,  ZBLOCK 3

EC                 /ENABLE COUNTER

4                  /CHANNEL 4

1205               /INITIAL VALUE OF 1205

0                  /DON'T RESET

2016               /RUN TASK 16 ON EVENT EVERY TIME IT

/   OCCURS

ZBLOCK 1

EXERR2, 0                  /ERROR INDICATOR

EXEC3,  ZBLOCK 3

EC                 /ENABLE COUNTER

4                  /CHANNEL 4

10                 /INITIAL VALUE OF 10

7700               /RESET TO 7700

1015               /DERAIL TO TASK 15 EVERY TIME IT

5620               / OCCURS AT LOCATION 5620

ECERR3, 0                  /ERROR INDICATOR

4.15.10  GC (Generic Code)

----------------------------------------------------------------------

| Use a GC operation to determine the generic code for a specified   |

| channel.                                                           |

|                                                                    |

| Message Format:                                                    |

|                ZBLOCK 3                                            |

|                GC                                                  |

|                CHANNL                                              |

|                RESULT                                              |

|                ZBLOCK 3                                            |

|                STATUS                                              |

|                                                                    |

| where:                                                             |

|       GC        is an RTS/8 mnemonic for Generic Code.             |

|       CHANNL    is the channel.                                    |

|       RESULT    is one of the values listed in Table 4-8.          |

|       STATUS    is the error status.                               |

----------------------------------------------------------------------

The Generic Code operation classifies interrupts so that  the  handler

can  know  how  to  process them.  This operation is similar to a skip

chain, but the operation occurs at task level  (that  is,  during  the

UDC/ICS task) rather than at interrupt level.

4-56

RTS/8 SYSTEM TASKS

You may also use this operation to insure that the correct  module  is

attached.   For  example,  when  you  reconfigure your system, you may

accidentally invert  a  counter  and  analog-to-digital  module.   The

generic  code  operation  then  returns a value, in this case 4 rather

than the expected 7.

Table 4-8

Generic Code Values

-----------------------------------------------

| Generic Codes  |           Meaning          |

|   Value        |                            |

|----------------|----------------------------|

|     0          |  No interrupt              |

|     1          |  Controller Error          |

|    2,3         |  Contact Interrupt Modules |

|     4          |  Counter Module            |

|    5,6         |  Unused                    |

|     7          |  A/D converter             |

-----------------------------------------------

4.15.11  RC (Read Counter)

----------------------------------------------------------------------

| Use a RC operation to read the channel value of the counter        |

| channel.                                                           |

|                                                                    |

| Message Format:                                                    |

|                ZBLOCK 3                                            |

|                RC                                                  |

|                CHANNL                                              |

|                RESULT                                              |

|                ZBLOCK 3                                            |

|                STATUS                                              |

|                                                                    |

| where:                                                             |

|       RC        is an RTS/8 mnemonic for Read Counter.             |

|       CHANNL    is the counter channel whose current value  is  to |

|                 be read.                                           |

|       RESULT    location where the value read is placed.           |

|       STATUS    is the error status.                               |

----------------------------------------------------------------------

In many operations, you set a counter to a value and then  count  down

to zero.  When the counter reaches zero, you perform an interrupt.  In

some cases, however, you may wish to check the value  of  the  counter

before  it  reaches  zero  to  determine  how  far  an  operation  has

progressed.  This is the function of the Read Counter operation.

Example:

RCEX,   ZBLOCK 3

RC                 /READ COUNTER

6                  /CHANNEL 6

RCANS,  0                  /VALUE OF CHANNEL 6 PUT HERE

ZBLOCK 3

RCERR,  0                  /ERROR INDICATOR

4-57

RTS/8 SYSTEM TASKS

4.15.12  UDC/ICS Status/Error Conditions

To indicate error conditions, the UDC/ICS Handler places  a  value  in

the  tenth  word  (the status word) of the task's message.  The values

and meanings are listed in Table 4-9.

Table 4-9

UDC/ICS Messages

----------------------------------------------------------------------

|  Value  |                         Meaning                          |

|--------------------------------------------------------------------|

|    0    |  No error                                                |

|         |                                                          |

|    1    |  Illegal  generic  code  for  specified  channel  and    |

|         |  operation                                               |

|         |                                                          |

|    2    |  Channel or subchannel value not valid                   |

|         |                                                          |

|    3    |  Illegal function code                                   |

|         |                                                          |

|    4    |  Not used                                                |

|         |                                                          |

|    5    |  UDC/ICS  control  not  responding  (power  off  or      |

|         |  hardware error)                                         |

----------------------------------------------------------------------

4-58

CHAPTER 5

MONITOR CONSOLE ROUTINE

5.1  MCR FUNCTIONS

The Monitor Console Routine (MCR) receives interactive commands  which

control, inspect, and, to some extent, debug an RTS/8 system.  It also

allows you to schedule  and  execute  tasks  at  specified  intervals,

suspend task execution, and print system status information.

An MCR command consists of a command word followed  by  arguments  and

terminated  by  either a carriage return or an ESCAPE.  Only the first

two characters of the command are significant,  except  for  the  EXIT

command, for which you must enter all four letters.

Commands can be a maximum of 60 characters long.

If a carriage return terminates the command line, the MCR  returns  to

the  terminal  for  another  command  when  it finishes processing the

current command.  If you type an ESCAPE as the final  line  character,

the  MCR  puts  itself in a wait state when it finishes processing the

command.

You wake the MCR from the wait state by typing CONTROL/C (^C)  on  the

console terminal (if the terminal is in foreground mode).

The MCR indicates that it is active and ready to  accept  commands  by

printing  the  prompting  character  > on the system console terminal.

When the MCR prompts with its > and is waiting for input, only the MCR

task  can  use  the  terminal  (subject  to  TIMEOUT  restrictions and

emergency messages sent by the EMT task).  Therefore, if you are using

the  terminal  for something other than an exclusive MCR terminal, you

should type an ESCAPE in response to  the  prompting  >  to  free  the

terminal,  and  to put MCR into a wait state.  This procedure prevents

the MCR from tying up the terminal. '

If the MCR terminal handler  has  been  configured  with  the  TIMEOUT

facility  when  the system was generated, a timed out input causes the

handler to go into the same wait state as does typing an ESCAPE.

5.2  MCR COMMAND SYNTAX

You must use certain syntactic constructions as arguments  to  several

MCR commands.  The definitions of these arguments are:

<sp> or ,      You may use a  single  comma  or  a  single  space

interchangeably to separate MCR arguments.

TASK-ID        is either an octal number or a name.  If it  is  a

number,  it  represents the 1- to 3-digit internal

RTS/8 task number.  In  RTS/8,  this  number  also

5-1

MONITOR CONSOLE ROUTINE

signifies the priority of the task:  the lower the

number, the higher the priority.  If TASK-ID is  a

name, the MCR looks up the first six characters in

its Task Name Table to locate a task number.

TIME-OF-DAY    is of the form HH:MM:SS where HH represents  hours

past  midnight,  MM  represents  minutes  past the

hour, and SS represents seconds past the minute.

ADDRESS        is an octal number  from  one  to  six  digits  in

length that represents a PDP-8 memory address.  If

ADDRESS is less than six digits long, the MCR pads

the number on the left with 0s.

WORD           is an octal number from one to four digits long.

5.3  MCR COMMANDS

In the following command descriptions, the significant portion of  the

MCR  command  word  is  printed  in  capital  letters.  If the command

accepts an optional argument or arguments, these arguments are  placed

within  square brackets ([ ]).  If there is a choice of arguments, the

choices are placed within parentheses and separated by an  exclamation

point (!).  If a command is preceded by an asterisk (*), it is present

only in a system with a clock.

Table 5-1

MCR Command Summary

----------------------------------------------------------------------

|     *Cancel       Cancels clock queue entries                      |

|                                                                    |

|     *DAte         Sets or lists current system date                |

|                                                                    |

|     DEposit       Places words into memory                         |

|                                                                    |

|     DIsable       Sets the ENBWT waitbit                           |

|                                                                    |

|     ENable        Clears the ENBWT waitbit                         |

|                                                                    |

|     EXamine       Opens a word for examination                     |

|                                                                    |

|     EXIT          Runs the EXIT task (if present) and exits to     |

|                   OS/monitor                                       |

|                                                                    |

|     NAme          Changes the name by which a task is known to     |

|                   system                                           |

|                                                                    |

|     OPen          Displays a location on the terminal              |

|                                                                    |

|     POst          Posts an event flag                              |

|                                                                    |

|     REquest       Clears a task's RUNWT bit                        |

----------------------------------------------------------------------

(continued on next page)

5-2

MONITOR CONSOLE ROUTINE

Table 5-1 (Cont.)

MCR Command Summary

----------------------------------------------------------------------

|     STop          Sets a task's RUNWT bit                          |

|                                                                    |

|     SYstat        Displays system status information               |

|                                                                    |

|     *TIme         Sets or lists the current system time            |

----------------------------------------------------------------------

5.3.1  CANCEL

----------------------

|  * CAncel task-id  |

----------------------

The CAncel TASK-ID command cancels any clock queue  entries  involving

task TASK-ID.  This includes:

1.  Entries  made  by  the  MCR  (from  previous  timed   REquest

commands)

2.  Entries involving the task made by other tasks (for  example,

a timed derail)

3.  Entries made by the task involving  itself  (for  example,  a

timed post)

------------------- NOTE -------------------

|                                          |

| If the queue entry is a post, the  event |

| flag will not be posted and the task may |

| hang up forever waiting for it.          |

--------------------------------------------

5.3.2  DATE

--------------------------------------

|  * DAte [DD-MON-YY[,TIME-OF-DAY]]  |

--------------------------------------

The date, DD-MON-YY, if you  specify  it,  becomes  the  system  date.

RTS/8  automatically increments the system date at midnight.  However,

it also assumes that all months have 31 days.

If you specify the second argument, it sets the system TIME-OF-DAY.

If you do not specify an argument to this command, the MCR prints  the

current system date on the console terminal.

The RTS/8 date word is the same date word as the  one  used  by  OS/8.

Consequently,  if  you  specify  the  date  with  OS/8,  you  need not

5-3

MONITOR CONSOLE ROUTINE

respecify it when you are using RTS/8 unless the day changes  and  the

month does not have 31 days.

Example:

>DATE 31-JUL-78

>DATE

31-JUL-78

5.3.3  DEPOSIT

-----------------------------------------------

|  DEposit ADDRESS,WORD1[,WORD2][,WORD3]....  |

-----------------------------------------------

The DEposit address command deposits its second through nth arguments.

For example:

>DEPOSIT 26132,7700,7200,50

deposits 7700 into location 26132, 7200 into location 26133, and  0050

into location 26134.

If you have KT8-A extended memory hardware, you may specify a 6 digit

address.

The MCR does not allow insertions to wrap around memory.  It  prevents

this  at  two  places:   32K  in  systems  without a KT8-A and 128K in

systems with a KT8-A.  For example, if you specify:

>DE 77777 0123,4567

the second number to be placed in memory  could  go  into  location  0

(77777 + 1).   The  MCR  prevents  this  from occurring and prints the

error message BAD NUMBER.  Note, however,  that  MCR  places  0123  in

location 77777.

This command (as well as all other MCR commands) can contain up to  60

characters;  thus, all characters beyond this limit are lost.  The MCR

prints an error message if you enter more than 60 (octal)  characters,

but it will deposit the first 60 characters of the command.  This loss

is not a serious problem, but you should check to see  that  the  last

word  deposited  was  correct (because this command might deposit that

part of a word that exceeds sixty characters).

5.3.4  DISABLE

---------------------

|  DIsable TASK-ID  |

---------------------

The  DIsable  TASK-ID  command  disables  further  execution  of   the

5-4

MONITOR CONSOLE ROUTINE

specified task by setting the ENABWT bit in the Task Flags Table entry

for task TASK-ID.

5.3.5  ENABLE

--------------------

|  ENable TASK-ID  |

--------------------

The ENable TASK-ID command clears the ENABWT bit  in  the  Task  Flags

Table  entry  for  task  TASK-ID;   this permits the task to run if no

other WAITBIT is set.  However, if other bits are  set,  the  task  is

still  not  runnable.  If the ENABWT bit is not set, this command is a

no-op.

5.3.6  EXAMINE

------------------------------

|  EXamine ADDRESS [,COUNT]  |

------------------------------

The EXamine command is identical to the OPen command.

5.3.7  EXIT

----------

|  exit  |

----------

The EXIT command causes the MCR to  terminate  RTS/8  execution (after

pausing approximately one second for pending I/O to complete) and then

returns control to the OS/8 monitor at location 07600.

------------------- NOTE -------------------

|                                          |

| You   must   type   out   this   command |

| completely  or it will be interpreted as |

| an EXamine command.                      |

--------------------------------------------

If the exit task is included in the system,  this  command  runs  that

task before going to location 07600.

5.3.8  NAME

--------------------------

|  NAme TASK-ID,NEWNAME  |

--------------------------

The character string NEWNAME becomes the new name of the task and  the

5-5

MONITOR CONSOLE ROUTINE

old  name  of  the  task (if existing) is lost.  NEWNAME can be of any

length, but the MCR stores only the first six characters.  If  NEWNAME

is  the  name  of  any  other  task, the MCR prints the BAD NAME error

message.

Examples:

>NAME 7 SUPERVISOR

MCR gives the name SUPERV to task number 7

>NA SUPERVISOR,EXECUTIVE

Task number 7, which is internally known as SUPERV, is  now  known  as

EXECUT.

5.3.9  OPEN

---------------------------

|  OPen ADDRESS [,COUNT]  |

---------------------------

The OPen ADDRESS command displays the number of locations you  specify

in  COUNT  beginning  at  ADDRESS.   The  form  in  which MCR displays

information is:

address/contents

The range of locations MCR displays may cross a  field  boundary.   If

you do not specify count, MCR assumes it to be 1.

The MCR prevents the address from wrapping around at two  points:  32K

in  systems  without  a  KT8-A and 128K for systems with a KT8-A.  For

example, if you type

>OP 77775,3

MCR will display  the  contents  of  locations  77775  through  77777.

However, if you type:

>OP 77775,4

MCR displays locations 77775 through 77777 and  then  prints  the  BAD

NUMBER error message.

5.3.10  POST

------------------

|  POst ADDRESS  |

------------------

The POst ADDRESS command posts the  event  flag  at  the  address  you

5-6

MONITOR CONSOLE ROUTINE

specify.  This command is useful when you are debugging a system since

it can simulate the effects of nonexistent tasks or devices.

------------------- NOTE -------------------

|                                          |

| You should be careful in the way you use |

| this  command  because  these  simulated |

| postings indicate actions which  do  not |

| occur.                                   |

--------------------------------------------

5.3.11  REQUEST

-------------------------------------------------------------

|  REquest TASK-ID[,(@TIME-OF-DAY ! INTERVAL)[,INTERVAL] ]  |

-------------------------------------------------------------

The REquest TASK-ID command requests a task to run:

o  Immediately (if you specify only TASK-ID),

o  At a given TIME-OF-DAY,

o  After a given INTERVAL, or

o  At a given INTERVAL.

INTERVAL is of the form:

nH    n hours

nM    n minutes

nS    n seconds

nT    n system ticks

Requesting a task clears the RUNWT bit in the task's Task Flags  Table

entry.  (This command is equivalent to the RUN Executive Request.) The

interval, given as the third argument, specifies the period  at  which

the  task  is to be rerun.  If you do not have a clock in your system,

the  MCR  ignores  any  timing  arguments  and   executes   the   task

immediately.

The following examples illustrate  three  different  formats  for  the

REquest  command;   note,  however, that only the first two characters

are significant.

Examples:

>REQUEST X

runs task X immediately.

>RE NEW,@2:00

5-7

MONITOR CONSOLE ROUTINE

runs task NEW at 2:00 am (if it is after 2 am, NEW runs tomorrow at  2

am)

>RE 5,10M,5M

runs task 5 ten minutes from now, and every five minutes thereafter.

>REQ HIPR,1T,6T

On a machine with a 60 Hz clock,  this  command  runs  the  task  HIPR

immediately  (that  is, .016 seconds from now, and 10 times per second

thereafter).

------------------- NOTE -------------------

|                                          |

| If, at the time  the  MCR  executes  the |

| REquest  command  (which  may be several |

| hours after you enter it), the task does |

| not  have  the RUNWT bit set in its Task |

| Flags  Table  entry,  then  the  REquest |

| command   is  a  no-op  (no-operation).  |

|                                          |

| Similarly, the task will  not  run  upon |

| execution  of  the REquest command if it |

| has other  WAITBITS  set  beside  RUNWT; |

| the   task  runs  only  when  the  other |

| blocking bits are cleared.               |

--------------------------------------------

5.3.12  STOP

------------------

|  STop TASK-ID  |

------------------

The STop TASK-ID command suspends execution of the task you specify by

turning  on the RUNWT bit in the Task Flags Table entry for that task.

(This is identical to the SUSPND ER.) You may restart a task  that  is

stopped  by  using  the REquest MCR command.  (In this instance, it is

easier to think of the command as a REsume MCR command.)

5.3.13  SYSTAT

----------------------

|  SYstat [TASK-ID]  |

----------------------

The SYstat command, depending on  whether  you  specify  an  argument,

prints either a general system status report or a more detailed status

report on a single task.  If you do not type an argument,  the  SYstat

command  prints  a  system  status  report.   Each  line of the report

describes a task in the  system.   For  each  task,  furthermore,  the

5-8

MONITOR CONSOLE ROUTINE

report prints the task number/priority, task name (if it has one), and

which blocking bits are set in the Task Flags Table  entry.   The  MCR

prints  each blocking bit as a 1-letter code, preceded by a space.  If

more than one blocking bit is set, the MCR prints one letter for  each

bit.  The 1-letter codes and their meanings are:

D     Disabled

E     Waiting for event flag

M     Waiting for a message

N     Nonresident Wait

O     Waiting for an event flag or a message

R     Waiting to be REquested or RUN

S     Waiting to be swapped in

U     USERWT bit set

X     Task does not exist

In addition, an asterisk (*) printed at the end of the line means  the

task has messages waiting in its input queue.

You obtain more detailed information about a task  by  specifying  the

TASK-ID  as  an argument to the command.  The detailed report contains

all the information in the general status report, followed  by  either

five or six octal words:

WORD 1    The location of the Task State Table  entry  containing

words 2-5;  this word is followed by a colon.

WORD 2    Without KT8-A:

Task Link in sign bit,  Instruction  Field  in  bits

6-8,  Data field in bits 9-11 (PDP-8/E and 8/A Flags

register)

With KT8-A:

Task Link in sign bit, the high-order  bits  of  the

Instruction  Field  in  bits 1 and 2, the high-order

bits of the Data Field in bits 3  and  4,  the  user

mode in bit 5, the low-order bits of the Instruction

Field in bits 6 through 8, and the low-order bits of

the Data Field in bits 9 through 11.

WORD 3    Task PC

WORD 4    Task AC

WORD 5    Task MQ

5-9

MONITOR CONSOLE ROUTINE

WORD 6    Task EAE.  The MCR prints this word only if the system

has EAE hardware and RTS/8 saves the EAE registers.

With KT8-A:

This word is the Step Count + 4000 (if the system is

in  B  mode)  + 2000 (if the GTF flag is set).  Note

that not all machines have a B mode.

Without KT8-A:

This word is the Step Count + 4000 (if the system is

in B mode).

Examples:

A general SYSTAT command might produce  the  following  sample  output

line:

13 CARD E S *

This line means task number 13, named CARD, is in Event Flag Wait,  is

waiting  to  be  swapped  into memory, and has input messages pending.

The command

>SYSTAT CARD

might produce the single line:

13 CARD E S * 1320: 0021 1741 0000 2525

This line indicates, in addition to the information specified  in  the

general SYSTAT command, that task CARD is stopped at location 21741 in

data field 1, with its accumulator and Link set at zero, and  2525  in

its MQ.

5.3.14  TIME

--------------------------

|  * TIme [TIME-OF-DAY]  |

--------------------------

If you specify a TIME command, the argument to  this  command  becomes

the  system  TIME-OF-DAY.   If  you  enter  this  command  without  an

argument, the MCR prints the current system TIME-OF-DAY  in  the  form

hh:mm:ss.

Example:

>TIME 14:00

>TIME

14:00:04

5.3.15  MCR Errors

5-10

MONITOR CONSOLE ROUTINE

The MCR checks each command line for errors  and,  if  it  finds  one,

issues  one  of  the following messages.  It will then reprompt on the

terminal for the corrected command.

----------------------------------------------------------------------

|     Error      |                                                   |

|    Message     |                    Meaning                        |

|----------------|---------------------------------------------------|

|   BAD CHAR     |  Character was not expected where it occurred     |

|                |                                                   |

|   BAD NUMBER   |  Number is out of legal range or is missing       |

|                |                                                   |

|   BAD DELIM    |  No space or comma where one was expected         |

|                |                                                   |

|   BAD NAME     |  Task name is not in name table or is a duplicate |

----------------------------------------------------------------------

5-11

CHAPTER 6

NONRESIDENT TASKS

6.1  NONRESIDENT TASK DESCRIPTION

A nonresident task is a task or a portion of a task that resides on  a

mass  storage  device  during the time the task is not runnable.  (The

mass storage device is called the swap device.) It  can  be  any  mass

storage  medium;   for example, an RL01 disk or a floppy disk.  When a

nonresident task becomes executable, the Executive posts  a  residency

request,  and  then  runs a special swapper task.  This task loads the

nonresident portion of the task into memory.  (The loading process  is

called  a  "swap.")  The  memory  area  into which or out of which the

Executive swaps  the  nonresident  task  is  called  a  partition.   A

partition  is simply a contiguous block of memory that a task uses for

its execution;  the partition is readable in  a  single  mass  storage

call.

The use of nonresident tasks permits several tasks to share  the  same

areas of memory, thereby optimizing the use of available memory.

Tasks can be either totally or partially  nonresident.  However,  very

few   tasks  are  totally  nonresident;   in  most  applications,  the

nonresident portion  includes  all,  or  nearly  all,  of  the  active

locations.   (Active  locations  are  those  that  contain  executable

instructions or data that other tasks do  not  access.)  The  resident

portion  (called  the root segment) of a task includes messages, event

flags, buffers, and similar passive registers  that  other  tasks  may

need.

Note that the process that swaps the nonresident  portion  of  a  task

into memory is similar to the overlay capability found in OS/8 FORTRAN

IV and related programs.

RTS/8 requires information concerning two aspects of the code.  If the

Executive  can  interrupt  and swap the task out of memory without its

consent, it is "checkpointable." If the task can  be  modified  during

its  execution,  it  must  be written to the swapping device.  This is

called "modified" code.  (Note that if the task is not modified during

execution, it can be overwritten because all that need be saved is the

processor state.)

6-1

NONRESIDENT TASKS

PDP-8 MEMORY

-----------------------------                  SWAP DEVICE

0K |                           |             (Disk, Floppy, etc.)

| REAL TIME PROGRAM AREA    |                  ***********

|                           |                *  Contains   *

| Contains:                 |               *  nonresident  *

|   1 Resident tasks        |               * portion of a  *

|   2 Resident portions of  |                *    task     *

|     non resident tasks    |                  ***********

pK |---------------------------|                       ^

|      PARTITION AREA       |                       |

|                           |                       v

| 1. Contains non resident  |            ------------------------

|    portion of nonresident |            |    SWAPPER TASK      |

|    task when that task is |            |    (SWAP.PA)         |

|    being executed         |            |                      |

|                           |<---------->| 1. Loads nonresident |

| 2. Allows multiple tasks  |            | portion of task into |

|    of same relative size  |            |    memory.           |

|    to run in the same     |            | 2. Swaps out non-    |

|    memory area.           |            |    resident task     |

|                           |            |    onto Swap Device. |

| 3. The characteristic of  |            | 3. Also loads and    |

|    a partition is as      |            |    swaps writeable   |

|    follows:               |            |    and checkpoint-   |

|                           |            |    able tasks.       |

| a) has a starting address |            ------------------------

|                           |

| b) Resides completely     |

|    within a single memory |

|    field                  |

|                           |

| c) Length has to be an    |

|    integral number of     |

|    pages                  |

|                           |

| d) Cannot overlap another |

|    partition              |

nK -----------------------------

Figure 6-1 Nonresident Task Implementation

6.1.1  Checkpointable Tasks

A task is made checkpointable if the Executive  may  swap  it  out  of

memory  automatically,  without its consent, to make room for a higher

priority task.  The Executive swaps back a task that  it  checkpointed

and  swapped  out  of  memory  as  soon  as  all higher priority tasks

relinquish processor control and  when  the  partition  is  available.

Execution  then continues at the point where the Executive interrupted

it;  the task is not aware that it was interrupted.

Checkpointing is ideally suited for  long-running  tasks  because  the

system  can run short tasks in the same area, swap them out of memory,

and swap the long running task back again.

6-2

NONRESIDENT TASKS

6.1.2  Modified Tasks

A modifiable task is any task that is self-modifying;   that  is,  the

task's  code  is changeable during task execution.  It must be written

to the swapping device because the code in  memory  differs  from  the

copy  of  the  code  on  the  swapping  device.   The modified feature

guarantees that the nonresident portion of a task is always up to date

by  refreshing  the  swap  device  image  of  the  nonresident portion

whenever the Executive swaps the task out of memory.

6.1.3  Interaction Between Tasks

Resident tasks can interact with the root segments of any  nonresident

task.   Also,  two  nonresident  tasks occupying the same or different

memory regions  can  interact  with  each  other  through  their  root

segments.   For  example,  assume that a nonresident task is executing

and it sends a message.  Because of some  reason  unconnected  to  the

message, the Executive checkpoints the task.  The message receiver can

acknowledge the sender's message even though the sender is not totally

resident.  However, the message sent must be resident.

6.2  MEMORY PARTITIONS

The swappable portion of  a  nonresident  task  resides  in  a  memory

partition.    This  partition  is  simply a contiguous block of memory

locations assigned by the LINK program.  LINK insures that a partition

has the following characteristics:

1.  It is wholly contained in one memory field.  (This means that

any nonresident task must fit into one memory field.)

2.  It has a starting address.

3.  It has a length (size) that must divide evenly by 200 (octal)

since  OS/8  directory-structured  devices  read and write in

one-half block (l-page) increments.

4.  It always begins at an address which is a multiple of 400.

Note  that  the  partitions  and  the  tasks  residing  in  them  were

established during the System Generation.

Partitions are mutually  distinct;   that  is,  one  partition  cannot

overlap  another.   In  RTS/8,  you  may  have  up to 7 partitions;  a

maximum of 16 tasks can use the same partition.   The  partitions  are

numbered from 1 to 7. (Partition 0 contains only resident tasks.) Note

that tasks are assigned to partitions when the system  was  generated.

Partitions do not have to be adjacent to one another.

Only one task can occupy a partition at any time.  The occupying  task

owns  the  partition until the task executes a FREE command (described

in the next section), or (if the task is checkpointable)  as  long  as

higher priority tasks that share the partition remain nonexecutable.

6-3

NONRESIDENT TASKS

6.2.1  FREE Command

----------------------------------------------------------------------

| Use a FREE ER to signal the Executive that a task  can  relinquish |

| control of a partition.                                            |

|                                                                    |

| Format:                                                            |

|        CAL                                                         |

|        COMMND+FREE                                                 |

|                                                                    |

| where:                                                             |

|       CAL       is an RTS/8 mnemonic for JMS 20.                   |

|       COMMND    is an RTS/8 Executive Request.                     |

|       FREE      is an Executive Request mnemonic.                  |

----------------------------------------------------------------------

The FREE command states that the Executive can swap the  issuing  task

out  of  memory if some other task becomes executable and requires the

partition before the freeing task cancels the FREE request.

You typically append a  FREE  request  to  the  function  argument  of

relevant Executive Requests (for example, SEND, RECEIVE, etc.)

------------------- NOTE -------------------

|                                          |

| You may place a FREE request  after  any |

| Executive   Request.   However,  if  the |

| Executive  Request  does  not  put   the |

| nonresident  task into a wait state, the |

| Executive ignores the FREE.              |

--------------------------------------------

If you enter the sequence

CAL

FREE

the Executive interprets it as:

CAL

SEND+FREE

Only nonresident tasks may issue FREE requests;  the Executive ignores

FREE  requests  from  permanently resident tasks.  Although a task may

free its own partition, it can never free  the  partition  of  another

task.

A task normally frees its partition whenever it waits  for  an  event.

If  any  other  task  has  a  PENDING  request  for the partition, the

Executive swaps it into the free partition immediately.  If  there  is

no  PENDING  request  for the partition, the freeing task continues to

wait in the free partition until some other task requests residency of

the  same  partition.   In  either case, once the freeing task becomes

executable again, it must compete for the  partition  along  with  any

6-4

NONRESIDENT TASKS

other executable occupants.

The freeing task may become executable before  another  task  requests

residency in the partition.  If this occurs, the Executive cancels the

FREE  request,  and  the  freeing  task  retains  possession  of   the

partition.  No read or write operation is necessary to effect the swap

in this case.  By freeing the partition whenever  the  occupying  task

(which  may  or  may  not be modified) must wait for an event, you are

assured that the partition contains a  running  (i.e.,  a  nonwaiting)

task  whenever  possible.   If  there  are  no  modified  tasks in the

partition, no swap device I/O is involved in freeing the partition.

6.2.2  Writing Nonresident Tasks

A nonresident task must be written with the following  conventions  so

that it links and initializes correctly.

1.  No SECT may contain both resident and nonresident code.

2.  SECTs of the root segment (if any) must  contain  LEVEL=0  in

their SECT definition to define the resident portion.

3.  Although you may include a LEVEL parameter in the nonresident

SECT,  DIGITAL  strongly  advises  you  not to do so unless a

special need justifies its inclusion.

4.  One SECT of every task must include  a  .NONRES  macro.   The

format  of  this  macro, included in the RTS/8 system when it

was generated, is

NONRES CHECKPOINTABLE[,WRITEABLE]

where CHECKPOINTABLE is a 0 or 1 value  with  0  meaning  the

task is not checkpointable and 1 meaning it is.  WRITEABLE is

an optional argument which also may have a 0 or 1 value.   If

this value equals zero, then the task is not writeable.

If you inadvertently include more than one .NONRES  macro  in

an RTS/8 task, MACREL prints an error message.  If this error

occurs, you must eliminate .NONRES from  one  of  the  SECTs.

You  then  resubmit  the RTS/8 system generation MACREL batch

file.

5.  Normally,  you  do  not  include  FIELD   statements   in   a

nonresident  SECT.   Note,  however, that because the message

pointer in an RTS/8 Executive Request CAL  must  point  to  a

location  in  the same field as the CAL, the nonresident SECT

of a task must reside in the same field as the resident SECT.

In  this  case,  and  only in this case where the nonresident

SECT uses  an  Executive  Request  to  communicate  with  the

resident  SECT,  should  you include a FIELD statement in the

nonresident SECT.  In all  other  cases,  you  should  create

field-independent SECTs.

6-5

NONRESIDENT TASKS

You may find it convenient to debug a nonresident task as  a  resident

task.  If you do this, you should build an RTS/8 system with a swapper

task.  (This means that you include the DIGITAL swap task in the RTS/8

system  even  though you may not have any nonresident tasks.) However,

do not assign your user- task to a partition.  Your task will then  be

fully  resident. When the task is debugged, you may either rebuild the

entire system or modify the linker command  string  (included  in  the

LINK  BATCH  file  produced by the RTS/8 System Generation) to include

your task in an overlay.  Also, the swap device must be on the  system

device when you run RTS/8 from OS/8.

------------------- NOTE -------------------

|                                          |

| The  operations  that  LINK   views   as |

| overlays  are converted to partitions by |

| the swapper initialization code.         |

--------------------------------------------

The save image of the RTS/8 system contains the initial image  of  the

nonresident tasks.  In order for LINK to correctly pass information to

RTS/8, this save image must be run from SYS:. As the RTS/8  system  is

executed,  modifiable  tasks  cannot be written back over the original

image.  The /X switch (provided at link time), causes LINK to allocate

additional  space  in the save image.  This space contains the working

image of the nonresident task.  Consequently, you  can  run  the  same

image more than once.  These procedures are transparent to the user.

6-6

Chapter 7

PROGRAMMING TECHNIQUES

7.1  OVERVIEW

This chapter describes several aspects of writing  tasks  under  RTS/8

that  rely  heavily  on the user understanding the nature of the RTS/8

system.  You should only use the techniques described in the following

sections  after  you  become familiar with RTS/8.  The topics discused

are:

1.  Writing tasks where more than one task shares the information

in some locations.

2.  Writing tasks that must wait for more than one event flag.

3.  Writing tasks that modify system tables.

4.  Programming the KL8A.

7.2  PERFORMING A RESCHUDLE

7.2.1  Writing Delicate Code

Frequently, a task needs to manipulate data  in  another  task  or  in

shared  locations.   Since  tasks  are  running concurrently, problems

arise if two tasks want to access the same  data  at  the  same  time.

Consequently, delicate code should run with interrupts disabled when a

task accesses data which another task can also  access.   You  disable

the interrupts so that task switching does not occur.

------------------- NOTE -------------------

|					     |

| You may disable  interrupts  temporarily |

| using  either  an  IOF/ION  pair  or, on |

| machines with memory management,  a  CIF |

| instruction  (which  inhibits interrupts |

| until the execution of the next  JMP  or |

| JMS instruction).			     |

--------------------------------------------

For example, suppose Task A increments location COUNT occasionally and

Task B decrements location COUNT from time to time  during  execution.

The code might look like the following:

7-1

PROGRAMMING TECHNIQUES

/		     TASK A

LOOPA,  .

.

.

X,	     ISZ COUNT

.

.

.

JMP LOOPA

COUNT,  0

/		     TASK B

LOOPB,  .

.

.

STA

Y,      TAD COUNT

Z,      DCA COUNT

.

.

.

JMP LOOPB

If Task A increments COUNT the  same  number  of  times  that  Task  B

decrements COUNT, you might assume that COUNT would be 0 at the end of

the program.  However,  this  is  not  necessarily  so  since  a  race

condition can occur.

Suppose that Task A has a higher priority than Task B, and Task  A  is

waiting  for  an  event  to occur with COUNT currently containing a 6.

Task B is ready to decrement  COUNT.   However,  an  interrupt  occurs

after the processor executes location Y.  The accumulator contains a 5

and Task B is ready to store a  5  back  into  COUNT.   The  interrupt

service  routine,  noting  that the event for which Task A was waiting

has just occurred, now suspends Task B and resumes Task A.  Task A now

bumps  COUNT  from  6  to 7, and then goes back to sleep.  Task B then

resumes with the accumulator containing a 5 and stores a 5 into  COUNT

which is incorrect for proper program execution.

You can prevent this situation from arising  by  disabling  interrupts

around  the  delicate code.  Either of the following two solutions are

workable:

Solution 2 (only  usable  on  machines  with  memory  extension)  uses

the CIF instruction since it temporarily inhibits interrupts until the

processor executes the next JMP or JMS instruction.

SOLUTION 1			    SOLUTION 2

/TASK B				 /TASK B

LOOPB,  .			 LOOPB,  .

.				 .

.				 .

7-2

PROGRAMMING TECHNIQUES

.				 .

STA				 STA

IOF				 CIF .FLD

Y,      TAD COUNT		 Y,      TAD COUNT

Z,      DCA COUNT		 Z,      DCA COUNT

ION				 .

.				 .

.				 .

.				 .

JMP LOOPB			 JMP LOOPB

7.2.2  Inhibiting Task Switching

Although you can use the procedures in the previous section, they  can

be  very inefficient.  If the processor performs a considerable amount

of  manipulation  on  data  accessible  by  other  tasks,  it  may  be

inappropriate  to turn off interrupts. Moreover, inhibiting interrupts

for long periods of time could affect other  portions  of  the  system

where timed events are very important.  Also, an interrupt can be lost

(for example, clock interrupts) if the processor turns off  interrupts

for a substantial amount of time.

For this case, another solution is possible.  A task  can  inform  the

RTS/8  Executive that it wants to run, and that while it is executing,

no other task should run even if a task  of  higher  priority  becomes

runnable.  This process is known as inhibiting task switching.

You should inhibit task switching only under unusual circumstances and

perform  this  action  with  care.  While task switching is inhibited,

interrupts may still occur and an interrupt service routine  will  get

control.   Note  that  if  you  inhibit  task switching, the interrupt

service routine always returns control to the interrupted  task  after

the task services the interrupt, even if higher priority tasks are now

runnable.

------------------- NOTE -------------------

|					     |

| If you wish to manipulate data which  is |

| accessible    by    an   interrupt-level |

| routine, interrupts  must  be  inhibited |

| since  inhibiting  task  switching alone |

| will not be sufficient in this case.     |

--------------------------------------------

There are two methods of inhibiting task switching:

1.  Task switching is automatically inhibited whenever  a  task's

program  counter  is  less  than  100.  Thus,  you  may place

delicate code in the bottom of page 0  of  any  field  except

field 0.

7-3

PROGRAMMING TECHNIQUES

------------------- NOTE -------------------

|					     |

| You  should  not  use  locations   20-27 |

| because   the   RTS/8  Executive  stores |

| information in these locations.	     |

--------------------------------------------

Also, you should not use an ASECT in  field  0  which  places

information  below location 1000.  This is the area where the

RTS/8 Executive resides.

2.  A task may inhibit task switching by  zeroing  GLOBAL  TSWFLG

(the task switching flag).

In either case, after the task is through with its delicate  code,  it

may  not  be  sufficient  for the task to reset TSWFLG to its original

value (which is  1)  because  possibly  another  higher-priority  task

should  have  run  but  did  not  due  to  task  switching having been

inhibited.  You can determine this by interrogating  location  TSWFLG.

If  another  task  became runnable while task switching was inhibited,

the RTS/8 Executive sets this flag to -1.

When a task is ready to allow task switching again, it should  examine

this  flag  before  resetting  it to 1.  If it was -1, the task should

return control to the RTS/8 scheduler.  A task performs this by  using

the RESCHD Executive Request as follows:

CAL

RESCHD

This ER causes RTS/8 to perform a reschedule that allows the  runnable

task  of  highest  priority  to  be executed.  If you do not perform a

RESCHD after re-enabling task switching, then a higher  priority  task

that is entitled to run might not run until the next interrupt occurs.

However, the interrupt may never occur or, if it does, it may  be  too

late for proper program execution.

The preferred code for inhibiting task switching for Task B (which was

described previously) is:

/		     TASK B

LOOPB,  .

.

.

CLA

CDF TSWFLG

DCA I (TSWFLG)	     /INHIBIT TASK SWITCHING

STA

Y,      TAD COUNT

Z,      DCA COUNT

ISZ I (TSWFLG)

JMP .+3		     /SHOULD WE RESCHEDULE

CAL		     /YES

RESCHD

7-4

PROGRAMMING TECHNIQUES

CDF .FLD		     /NO

.

.

.

JMP LOOPB

------------------- NOTE -------------------

|					     |

| Interrupt level routines should not look |

| at or set the TSWFLG.		     |

--------------------------------------------

A summary of TSWFLG states is shown in Table 7-1.

Table 7-1

Task Switch Flag (TSWFLG) States

---------------------------------------------------

|	   TSWFLG State		    |   Value    |

|-------------------------------------------------|

| Task switching allowed		    |	  1	 |

|				    |		 |

| Task switching inhibited	    |	  0	 |

|				    |		 |

| Task switching inhibited;	    |	  -1	 |

| reschedule as soon as possible	    |		 |

---------------------------------------------------

7.3  MULTIPLE EVENT FLAG WAITS

Occasionally  you  may  find  it  desirable  to  wait  for  a  logical

combination  (AND  or OR) of event flags.  Waiting for the logical AND

of two event flags is quite simple.  The sequence

CAL

WAITE		 /WAIT FOR EVENT FLAG A

A

CAL

WAITE		 /WAIT FOR EVENT FLAG B

B

waits until both A and B have been posted.

Waiting for the logical OR of several event flags  is  more  difficult

since there is a possible race condition between the various tests and

the  interrupts  (or  task  executions)  that  post  the  event  flags

involved.   The key to waiting successfully for an OR of several event

flags is not to allow any interrupts to occur between the  testing  of

the  first  event  flag and the placing of the task in a wait state if

none of the flags were posted.

You accomplish this by using a special sequence of instructions and  a

7-5

PROGRAMMING TECHNIQUES

special  RTS/8  call  named  WAITM (defined as JMS I 25).  A task must

execute this instruction with interrupts off,  the  instruction  field

set  to  0,  and  the  data field set to the current field.  Also, the

instruction must be followed by a word containing the blocking  bit(s)

to be set in the Task Flags Table.

The action of WAITM is equivalent to  that  of  the  RTS/8  BLKARG  ER

except  that  a  fast  path  through  the RTS/8 Executive is taken and

interrupts remain off until the blocking bits are on in the Task Flags

Table.

As an example of the use of WAITM, assume that a task  TASK  wants  to

test  two  event flags, A and B.  If A is posted, control should go to

location ADONE;  if B is posted, control should go to location BDONE.

If neither is posted, the task must wait until one of them is  posted.

The code to perform this function is:

TESTAB,    IOF	     /INTERRUPTS OFF--DELICATE CODE

TAD A

SNA CLA

JMP ADONE	     /ADONE MUST TURN INTERRUPTS ON

TAD (4000+TASK) /SET A TO "WAITING" STATE INDICATING

/ THAT THIS TASK IS WAITING ON IT

DCA A

TAD B

SNA CLA

JMP BDONE       /BDONE MUST TURN INTERRUPTS ON

TAD (4000+TASK) /SET B TO "WAITING" STATE INDICATING

/ THAT THIS TASK IS WAITING ON IT

DCA B

CIF RTS8        /RTS8 IS THE FIELD OF RTS8 EXECUTIVE

CDF .FLD

WAITM

EFWT	     /BLOCK TASK ON EVENT FLAG WAIT

JMP TESTAB      /WE'RE BACK--ONE OF THE TWO EVENT

/ FLAGS HAS BEEN POSTED.  GO

/ BACK TO FIND OUT WHICH ONE.

7.4  EXECUTIVE INTERNAL TASK TABLES

The Executive uses the following  five  internal  tables  to  maintain

information about the tasks in the system:

Table				  Description

Task State Table (TSTABL)	     Contains  information  such   as   the

contents of the task's program counter

Link, and accumulator at the time  the

task stopped running.

Task Flags Table (TFTABL)	     Contains  information  about  why  the

task  is  not  running;   that  is, it

7-6

PROGRAMMING TECHNIQUES

indicates the conditions (blocking  of

WAITBITS)   for   which  the  task  is

waiting.

Task Input Message Queue	     Contains messages that have been  sent

Header Table (MSGTBL)	     to  this task.  This table is referred

to simply as the Message Table.

Residency Table (RESTBL)	     Used for nonresident tasks, this table

specifies  where the task is to reside

in memory and where it resides on  the

swap device.

Partition Table (PARTBL)	     Used for nonresident tasks, this table

contains information about each memory

partition.   Nonresident  tasks  share

memory partitions.

The Executive uses each task"s task number as an index into the  first

four  tables  to  retrieve  and update information for that task.  The

Executive uses the partition number as an index into the fifth table.

You may reference a table by using its GLOBAL name.   (Note  that  all

the tables reside in the field of the RTS/8 Executive.)

7.4.1  Task State Table (TSTABL)

The Task State table contains 4-word entries (five  if  the  Executive

saves  the  EAE  registers) which hold the most recent contents of the

CPU registers for each task as follows:

WORD 1  without KT8-A:

-link (bit 0)

-Greater Than Flag (bit  1)  if  flag  exists  on

machine

-User Mode Flag (bit 5) if flag exists on machine

-Instruction Field (bits 6-8)

-Data Field (bits 9-11)

with KT8-A:

-link (bit 0)

-high-order bits of the Instruction  Field  (bits

1-2)

-high-order bits of the Data Field (bits 3-4)

-user mode (bit 5)

-low-order bits of the  Instruction  Field  (bits

6-8)

-low-order bits of the Data Field (bits 9-11)

WORD 2  contains the contents of the Program Counter

WORD 3  contains the contents of the Accumulator

WORD 4  contains the contents of  the  Multiplier  Quotient  (MQ)

register if the system has been assembled to save the MQ.

7-7

PROGRAMMING TECHNIQUES

WORD 5  contains EAE information as follows:

-  If the  system  does  not  have  a  KT8-A,  this  word

contains  the Step Count + 4000 (if the system is in B

mode).

-  If the system has a KT8-A, this word contains the Step

Count  +  4000 (if the system is in B mode) + 2000 (if

the GTF flag is set).  Some machines do not have  a  B

mode.

Whenever the Executive executes a task, it loads the contents  of  the

task's  Task State Table entries into the corresponding CPU registers.

Also, whenever a task stops executing, the  Executive  sets  its  Task

State Table entries to the new contents of these registers.

Example:

Consider the following TSTABL entries for a task.

4012

3376

1234

0211

The Executive interrupted the task just as it was about to execute the

instruction  at  location 3376 (entry 2) of field 1 (bits 6-8 of entry

1).  At this point, the Executive  enters  the  contents  of  the  CPU

registers for this task in the TSTABL.  The accumulator is 1234 (entry

3), and the MQ is 0211 (entry 4).  The link is set (bit 0 of entry  1)

and the data field is 2 (bits 9-11 of entry 1).

7.4.2  Task Flags Table (TFTABL)

The Task Flags table contains a 1-word  entry  holding  various  flags

(bits)  for  each  task.   The  Executive uses this table to determine

whether the task is runnable.  A task is runnable  only  if  its  Task

Flags  Table entry is zero.  Each flag (bit) which is set in a nonzero

state indicates a reason why  the  task  cannot  run.   The  currently

defined flags and their meanings are listed in Table 7-2.

Table 7-2

Waitbit Descriptions

----------------------------------------------------------------------

| Symbolic Name  |  Value  |		 Meaning		  |

|----------------|---------|-----------------------------------------|

|    NONRWT      |   4000	|  Nonresident Wait--This task cannot run |

|		      |		|  because it is not in memory.		  |

|		      |		|					  |

|    EFWT        |   2000	|  Event Flag Wait--This task is  waiting |

|		      |		|  for  an  event  flag (which contains a |

|		      |		|  WAITING value  corresponding  to  this |

|		      |		|  task) to be POSTed.			  |

----------------------------------------------------------------------

(Continued on the next page)

7-8

PROGRAMMING TECHNIQUES

Table 7-2 (Cont.)

Waitbit Descriptions

----------------------------------------------------------------------

| Symbolic Name  |  Value  |		 Meaning		  |

|----------------|---------|-----------------------------------------|

|    RUNWT       |   1000	|  Run Wait--This task is waiting  for  a |

|		      |		|  RUN  ER to be executed with its number |

|		      |		|  in the AC, or for the operator to type |

|		      |		|  a   REQUEST  command  to  the  Monitor |

|		      |		|  Console Routine (see Chapter 5).	  |

|		      |		|					  |

|    SWPWT       |   0400	|  Swap  Wait--This   task   cannot   run |

|		      |		|  because  it is in the process of being |

|		      |		|  brought into memory.			  |

|		      |		|					  |

|    EORMWT      |   0200	|  Event or Message  Wait--This  task  is |

|		      |		|  waiting for an event flag to be POSTed |

|		      |		|  or  a  message  to  arrive,  whichever |

|		      |		|  happens first.			  |

|		      |		|					  |

|    USERWT      |   0100	|  User Wait--This bit  is  reserved  for |

|		      |		|  use by user-written tasks.  RTS/8 does |

|		      |		|  not use this bit.			  |

|		      |		|					  |

|    ENABWT      |   0040	|  Enable Wait--This task is  waiting  to |

|		      |		|  be   enabled.   Use  of  this  bit  is |

|		      |		|  restricted  to  the  Monitor   Console |

|		      |		|  Routine.    (See   Chapter  5  for  an |

|		      |		|  explanation of the ENABLE and  DISABLE |

|		      |		|  commands.)				  |

|		      |		|					  |

|    MSGWT       |   0020	|  Message Wait--This task is waiting  to |

|		      |		|  be sent a message.			  |

|		      |		|					  |

|    DNEWT       |   0001	|  Task does not exist.  This bit  should |

|		      |		|  never  be  set  or  cleared  by a user |

|		      |		|  task.				  |

|		      |		|					  |

|		      |   0000	|  Task is runnable			  |

----------------------------------------------------------------------

Examples:

If the TFTABL entry for a task is

0020

the task is waiting for a message.

If the TFTABL entry for a task is

0440

7-9

PROGRAMMING TECHNIQUES

the operator disabled the task from running at the MCR terminal  while

the  nonresident  portion  of  the  task is waiting to be swapped into

memory.

If the TFTABL entry for a task is:

0000

the task is runnable.  However, this task may not be running if a task

of higher priority controls the processor.

7.4.3  Task Input Message Queue Header Table (MSGTBL)

This table contains a 2-word entry that represents the "head"  (start)

of the input message queue for each task, as follows:

WORD 1    if zero, there  are  no  messages  in  the  queue;   if

nonzero,  the  word  is a CDF to the field of the first

message in the queue.

WORD 2    if word 1 was nonzero, this word is a  pointer  to  the

address of the first message in the queue.

Example:

Consider the following MSGTBL entries for a task:

6211

2044

Since the first entry is nonzero, there  are  messages  in  the  input

queue waiting for this task to receive them.  The first entry is a CDF

instruction to field 1.  The second entry is a pointer indicating that

the first message begins at location 2044 in field 1.

7.4.4  Residency Table (RESTBL)

The Residency Table contains a 2-word entry for each nonresident task.

WORD 1    contains a pointer to the task's Partition Table  entry

in   bits   0-9.    Bit   10   is  set  if  a  task  is

checkpointable, and bit 11 is set if the processor  can

modify  a task's code during execution.  Checkpointable

and modifiable tasks are defined in Section 6.1.

WORD 2    contains the absolute block address of the task's  core

image on the swap device.

Example:

Consider the following RESTBL entries for a task:

7-10

PROGRAMMING TECHNIQUES

1611

1124

This task has a nonresident portion.  The 4-word partition table entry

that  this  task  uses  begins  at  location  1610.   The  task is not

checkpointable (bit 10 of entry 1 is  a  0),  but  the  processor  can

modify it (bit 11 of entry 1 is a 1).

The disk-resident portion of this task begins at  block  1124  on  the

swap device.

7.4.5  Partition Table (PARTBL)

The Partition Table contains a 4-word entry for each partition.  It is

accessed through a partition number.

WORD 1    contains the length (size) in bits 1-5 and field  (bits

6-8)  argument  of  the mass storage device driver call

that reads an occupant  into  the  partition  with  the

WRITE  bit  set.   Bit 11 of this word is the partition

busy flag.

WORD 2    contains the memory address of the partition.

WORD 3    contains a pointer to word 1 of the  occupant's  RESTBL

entry.

WORD 4    Reserved for future expansion.

Example:

Consider the following PARTBL entries for a particular partition.

5421

1400

1553

0000

This partition is currently in use (bit 11 of entry 1 is  a  1)  by  a

task whose 2-word RESTBL entry begins at location 1553 (entry 3).  The

partition begins at location 1400 (entry 2) of field 2  (bits  6-8  of

entry  1).   The partition is 14 (octal) pages long (bits 1-5 of entry

1).

7.4.6  Task Table Structure

Figure 7-1  summarizes  the  internal  task  table  structure  of  the

Executive.   The  Residency  Table and Partition Table are optional in

that the Executive uses them only when nonresident tasks exist in  the

system.

7-11

PROGRAMMING TECHNIQUES

TASK STATE			     TASK FLAGS

TABLE (TSTABL)		   TABLE (TFTABL)

Word 1   Without KT8-A     ______

-----------------------------     -----------------------------

|				|     | Bits determine if task is |

|	 Link GT UM  IF   DF	|     | runnable		  |

|				|     |				  |

|	 0    1  5  6-8	 9-11	|     |				  |

|				|     |				  |

-----------------------------     -----------------------------

With KT8-A

-----------------------------     Task runnable if entry

|				|     contains zero

| Link High High UM Low Low |

|				|

|   0  1-2  3-4  5  6-8 9-11|

|				|

|---------------------------|

Word 2 |   Contents of Program     |     ______

|	Counter (PC)		|

|---------------------------|

Word 3 |  Contents of Accumulator  |     ______

|  (AC)			|

|---------------------------|

Word 4 |  Contents of Multiplier	|     ______

|  Quotient (MQ)		|

-----------------------------

Word 5   (Only if EAE)     ______

With KT8-A

-----------------------------

| 4000 + 2000 (GT) + Step	|

| Count (if B mode)		|

-----------------------------

Without KT8-A

-----------------------------

| 4000 + Step Count		|

| (if B mode)		|

-----------------------------

Upon task execution, TSTABL

loaded into corresponding

CPU registers

When the task terminates,

TSTABL entry set to new

contents of CPU registers

Figure 7-1 Executive Internal Task Table Structure

7-12

PROGRAMMING TECHNIQUES

TASK INPUT MESSAGE		     RESIDENCY

QUEUE HEADER (MSGTBL)		   TABLE (RESTBL)

---------------------------     -----------------------------

Word 1 | If zero, no messages in |	    | Contains pointer to task  |

| queue. If nonzero, word |	    | PARTBL entry in bits 0-9; |

| is CDF to the field of  |	    | bit 10 - ckeckpointable;  |

| first message	      |	    | bit 11 - writeable	|

|-------------------------|     -----------------------------

Word 2 | If nonzero, this word   |     | Block address plus 1 of	|

| is pointer to the first |	    | the task's core image on  |

| message in the queue    |     | swap device		|

---------------------------     -----------------------------

Represents the "head" of        bit 10 of word 1 is set if

the input message queue	    task checkpointable

for each task

bit 11 of word 1 is set if

task modified

PARTITION

TABLE (PARTBL)

--------------------------------------

Word 1 | Contains the length (size) and     |

| Field argument (bits 1-8) of the   |

| mass storage device driver call    |

| that reads occupant into partition |

| with 'write' bit set (bit 0).      |

|------------------------------------|

Word 2 | Contains starting address of the	 |

| partition			         |

|------------------------------------|

Word 3 | Contains pointer to word 1 of the  |

| occupant's RESTBL entry		 |

|------------------------------------|

Word 4 | Unused				 |

|					 |

--------------------------------------

Bit 0 is always set (write)

Bit 11 of word 1 is the partition

memory flag

Figure 7-1 Executive Internal Task Table Structure (cont.)

7-13

PROGRAMMING TECHNIQUES

7.4.7  Direct References to System Tables

A task may directly  interrogate  locations  in  the  RTS/8  Executive

tables  to  obtain information about itself or any other task, as long

as you observe the following two restrictions:

1.  Due to the interrupt-driven  nature  of  RTS/8,  these  table

entries  may  change at any time;  therefore, the task should

inhibit interrupts, if necessary, between the time  it  tests

these  entries and the time that the processing which depends

on the  testing  is  completed.   For  example,  testing  the

Message  Queue Header for a task may show no messages, but an

I/O interrupt occurring  immediately  after  the  test  might

allow  a higher priority task to run.  This task might send a

message, invalidating the result of  the  test.   To  prevent

this, the task might turn off interrupts during and after the

test.

2.  Although you can alter  the  system  table  entries,  DIGITAL

recommends  that  you  do  so with extreme care and only when

absolutely necessary.

RTS/8  defines  symbols  during  the  sysgen  that   permit   symbolic

references  to  be  made.   The  symbolic  expressions which yield the

address of the system table entries for task N are:

N+TFTABL	Task Flags Table entry for task N;  if the contents

are zero, task N is runnable.

N^2+MSGTBL	Input Message Queue Header  for  task  N;   if  the

contents  are  zero,  task N has no messages in its

queue.

N^4+TSTABL	First Word of Task State Table Entry  for  Task  N.

(The address is N^5+TSTABL if you save the EAE.)

N^2+RESTBL	First Word of Residency Table entry for nonresident

Task N.

For example, for a task to determine whether it has  messages  in  its

input queue without issuing a RECEIVE Executive Request, the code is:

CDF MSGTBL		  /EXEC TABLES IN FIELD 0

IOF			  /TURN INTERRUPTS OFF

TAD I (TASK 2+MSGTBL)

SNA CLA		  /ANY MESSAGES

JMP NONE		  /NO

ION			  /YES

(The code at NONE must eventually turn interrupts back on.)

7-14

PROGRAMMING TECHNIQUES

7.5  KL8-A SUPPORT

The KL8-A support in the Executive allows you to program the RL8-A  in

a manner similar to the KL8-J.

First, a task must insert the KL8-A into the interrupt skip chain  and

provide  a  keyboard and printer interrupt routine to service the line

if necessary.  For example,

.EXTERNAL KL8ACT   /DEFINE SYMBOL AS EXTERNAL

/TO MODULE

CDF .FLD

CIF RTS8	     /FIELD OF RTS/8 EXECUTIVE

TAD (LINE^4)

JMS I ( KL8ACT )

KEYBD INTERRUPT ROUTINE

PRINTER INTERRUPT ROUTINE

where LINE is the line number of the KL8-A.  KL8-A  line  numbers  are

consecutive,  begin  at 0, and may span across KL8-As.  The KL8-A line

number is actually of the form 4A+B where A is the number of the KL8-A

(0,  1, 2,...) and B is the physical line number of the specific KL8-A

(0-3).

Second, you must define  the  instruction  corresponding  to  the  TLS

instruction that you will use when writing to the KL8-A line.

For example, if the device code for the KL8-A is 40,  then  you  would

probably define an instruction such as:

TLSX=6406

in the task.

Normally, a task contains the following code to write a character:

TAD char

TLS

When using a KL8-A, the task would first connect up the KL8-A  support

by  calling  KL8ACT.   Then,  to output a character, you would use the

following code:

TAD line ^400

TAD char

TLSX

The TLSX instruction does not clear the AC.

7-15

APPENDIX A

RTS/8 DISTRIBUTED SOURCE FILES AND MNEMONICS

The following is a list of DIGITAL supplied tasks and handlers.

----------------------------------------------------------------------

| Task Name | File Name |	Task Function				  |

|-----------|-----------|---------------------------------------------

| -	 | RTS8.MA   |  RTS/8 Executive				  |

| CLOCK     | CLOCK.MA  |  Clock Handler Task			  |

| CSA       | CSA.MA    |  Cassette Driver Task			  |

| CSAF      | CSAF.MA   |  Cassette File Support Task		  |

| DTA       | DTA.MA    |  TC08 DECtape Driver Task		  |

| EXIT      | EXIT.MA   |  Exit Task				  |

| LPT       | LPT.MA    |  Line Printer Driver Task		  |

| LTA       | LTA.MA    |  LINCtape Driver Task			  |

| MCR       | MCR.MA    |  Monitor Console Routine			  |

| null task | MCR.MA    |  Null Task				  |

| NULL8A    | NULL8A.MA |  Null Task for PDP-8A			  |

| OS8       | OS8.MA    |  OS/8 Support Task			  |

| OS8F      | OS8F.MA   |  OS/8 File Support Task			  |

| PWRF      | PWRF.MA   |  Power Fail Task				  |

| RF08/DF32 | RF08.MA   |  RF08/DF32 Fixed-Head Driver Task	  |

| RK8       | RK8.MA    |  RK8 Disk Driver Task			  |

| RK8E      | RK8E.MA   |  RK8, Disk Driver Task			  |

| RL01      | RL01.MA   |  RL01 Disk Driver Task			  |

| RX8A      | RX.MA     |  Floppy Disk Handler (1st Controller)	  |

| RX8B      | RX.MA     |  Floppy Disk Handler (2nd Controller)	  |

| RX8C      | RX.MA     |  Floppy Disk Handler (3rd Controller)	  |

| RX8D      | RX.MA     |  Floppy Disk Handler (4th Controller)	  |

| SWAP      | SWAP.MA   |  Nonresident Task Swapper		  |

| TTY       | TTY.MA    |  Terminal Driver Task			  |

| TTY1      | TTY.MA    |  Terminal Driver Task			  |

| TTY2      | TTY. MA   |  Terminal Driver Task			  |

| TTY3      | TTY.MA    |  Terminal Driver Task			  |

| TTY4      | TTY.MA    |  Terminal Driver Task			  |

| UDC/ICS   | UDCICS.MA |  Universal Digital Controller/		  |

|           |           |  Industrial Controller Subsystem		  |

|           |           |  Handler Task				  |

----------------------------------------------------------------------

The following is a list of  task  mnemonics  which  the  RTS/8  System

Generation Program establishes.

CLOCK		 Clock Handler

CSA		 Cassette Driver Task

CSAF		 Cassette File Support Task

DF32		 DF32 Disk Driver Task

DKC8A		 Auxiliary DKC8A Handler

DTA		 DECtape Driver Task

EXIT		 Exit Task

LPT		 Line Printer Driver Task

LTA		 LINCtape Driver Task

A-1

RTS/8 DISTRIBUTED SOURCE FILES AND MNEMONICS

MCR		 Monitor Console Routine

NULL8A	 Null Job for PDP-8/A

OS8		 OS/8 Support Task

OS8F		 OS/8 File Support Task

PWRF		 Power Fail Handling Task

RF08		 RF08 Disk Driver Task

RK8		 RK8 or RK8E Driver Task

RX8A		 First Floppy Controller

RX8B		 Second Floppy Controller

RX8C		 Third Floppy Controller

RX8D		 Fourth Floppy Controller

SWAP		 Nonresident Task Swapper Task

TTY		 Teletype Driver Task

UDC		 Universal Digital Controller Task

The following is a list of system mnemonics you may use when  creating

tasks.   For  the mnemonics RTS/8 uses for system task parameters, see

the RTS/8 System Generation Manual.	 ______________________________

NUMBERS

AC7776	CLL STA RAL

AC7775	CLL STA RTL

AC4000	CLA STL RAR

AC3777	CLL STA RAR

AC2000	CLA STL RTR

AC0002	CLA STL RTL

AC5777	CLL STA RTR

RTS/8 EXECUTIVE

CAL		JMS 20

POSTDS	JMP I 24

WAITM		JMS I 25

SEND		0

RECEIV	1

WAITE		2

RUN		3

SUSPND	4

POST		5

SKPINS	6

DERAIL	7

BLKARG	10

SENDW		11

UNBARG	12

RESCHD	13

WAITX		14

FREE		4000

WAITBITS

NONRWT	4000

EFWT		2000

RUNWT		1000

A-2

RTS/8 DISTRIBUTED SOURCE FILES AND MNEMONICS

SWPWT		0400

EORMWT	0200

USERWT	0100

ENABWT	0040

MSGWT		0020

NETWT		0010

DNEWT		0001

CLOCK

MARKTIME	0

SCHEDULE	1000

TIMEOUT	2000

PERIODICALLY	3000

CANCEL	7000

TTY and LPT

NOPACK	4000

NOCRLF	2000

IND		1000

NOLINE	400

ASSGN		200

KL8ALINE	100

UDC/ICS

AO		0

DO		0

DI		2

GC		3

EC		4

RC		5

DC		6

ECT		7

CS		10

DCT		11

AI		12

A-3

APPENDIX B

RTS/8 FLOWCHARTS

This appendix contains RTS/8 flowcharts that  graphically  show  RTS/8

system operation.

B-1

RTS/8 FLOWCHARTS

***

*      *

*	 EXEC  *			Calling Sequence:

*      *

***				p-1	TAD VAL

|				p	CAL

|				p+1	FUNCTION CODE

V				p+2	ARG1

----------------			p+3	AGR2 (or return)

|  SAVE AC IN  |			p+4	RETURN

|    ACARG     |

----------------			CAL = JMS 20

|				Thru Current Field:

|				20	0

|				21	CDF CUR

V				22	CIF 0

////\\\\				23	CMP CALIOF (field 0)

/	       \ 			24	DSPOST

/	 TASK 	\  TSWFLG = -1		25	XWAITM

---/  SWITCHING \----  No:

|  \   ALLOWED  /   |  Rescan

0 |   \	   ?    /    |  ASAP

No |    \        /     |

|     \\\\//// 1    |

|	  |    Yes   |

-------->|    	     |

V	     |

------------------  |

|  INHIBIT TASK  |  |

|  SWITCHING.	  |  |

|  CLEAR RESCAN  |  |

|  FLAG	  |  |

------------------  |

|	     |

|<----------

V

------------------

|      ION	  |

------------------

|

V

------------------

|  SAVE CALLING  |

|	FIELD	  |

------------------

|

V

------------------

|  SAVE RETURN   |

|   ADDRESS	  |

------------------

|

V

To page B-3

B-2

RTS/8 FLOWCHARTS

From page B-2

|

------------------

|  GET FUNCTION  |

|	CODE	  |

------------------

|

V			SW designates an operation that

------------------		is performed only when using

|      SAVE	  |		nonresident operations.

|    COMMAND	  |

------------------		EAE designates an operation that

SW	  |			is performed only when the Extended

|			Arithmetic Element is used.

---------

|

V

------------------

|  EXECUTE	  |

|  DISPATCH JUMP |

|  THRU COMMAND  |

|  TABLE. IGNORE |

|  FREE BIT	  |

------------------

|

Function          |

Code	          V				     ****

|	 ------------------			   *      *

V	 | JMP I .+FUNC+1 |<----------------------* CMDJMP *

------------------			   *      *

0	 |     XSEND	  |    Pg. B-5		     ****

------------------

1	 |     XRECEIV	  |    Pg. B-17

------------------

2	 |     XWAITE	  |    Pg. B-10

------------------

3	 |     XRUN	  |    Pg. B-28

------------------

4	 |     XSUSPND	  |    Pg. B-28

------------------

5	 |     XPOST	  |    Pg. B-20

------------------

6	 |     XSKPINS	  |    Pg. B-21

------------------

7	 |     XDERAL	  |    Pg. B-22

------------------

10	 |     XBLKARG	  |    Pg. B-19

------------------

11	 |     XSENDW	  |    Pg. B-5

------------------

12	 |     XUNBARG	  |    Pg. B-18

------------------

13	 |     XTSTOP	  |    Pg. B-13

------------------

14	 |     XWAITX	  |    Pg. B-10

------------------

B-3

RTS/8 FLOWCHARTS

*****

*       *

*  EXRET  *

*       *

*****

|

|

V

-----------------

|     SET UP    |

|  RETURN FIELD |

-----------------

|

|

|

|

V

////// \\\\\\	TSWFLG = -1 ---------------

TSWFLG=1 	/	      \   Yes	    |  KEEP TASK  |

------------   	       / TASKSWITCHING \	    |  SWITCHING  |

|	Cannot	|<-------------\  NEEDS RESCAN /----------->|  INHIBITED  |

|  Happen  |		\      ?      /		    ---------------

------------		 \\\\\\ //////			    |

|			    V

| TSWFLG = 0		  *****

| No			*       *

|		       *  TSTOP  *

V			*	*

-----------------		  *****

|  ALLOW TASK-  |	        (Pg. B-13)

|  SWITCHING    |

|  JMP I 20     |

|  RETURN TO    |

|  CALLING TASK |

-----------------

B-4

RTS/8 FLOWCHARTS

---------							  ---------

| SEND  |							  | SEND  |

| SENDW |							  | SENDW |

---------							  ---------

****		  ***

*      *		*     *

* XSENDW *	       * XSEND *

*      *		*     *

****		  ***

|		   |

V		   V

---------------     ---------------	Calling Sequence

|     SET     |     |    CLEAR    |	p+1  SEND (W)

|  WAIT FLAG  |	    |  WAIT FLAG  |	p+2  TASK # (Sending to)

---------------	    ---------------	p+3  MSGADR

|		   |		P+4  RETURN

---------------------

|

|

V

-----------------

|  GET TASK     |

|  NUMBER TO    |

|  SEND MSG TO  |

-----------------

|

|

V

-----------------

|  GET MESSAGE  |

|  ADDRFSS	 |

-----------------

|

|

V

To page B-6

Initial Values   |  Becomes

-------------------------|--------------------

| p   |	 MSGADR,  0000	 |  Sending Task #   |

-------------------------|--------------------

| p+1 |		  0000   |  CDF to next      |

|     |			 |  msg, if any      |

-------------------------|--------------------

| p+2 |		  0000   |  Addr of next     |

|     |		 	 |  msg (0 if none)  |

-------------------------|--------------------

| p+3 |          Start of Message	     |

----------------------------------------------

B-5

RTS/8 FLOWCHARTS

From page B-5

|

V

//////////\\\\\\\\\\

/		       \

/      MESSAGE         \

Yes   /   ALREADY IN QUEUE     \

---------------\ (FIRST WORD OF MESSAGE /

|	         \	 =0)  ?	        /

|		  \		       /

|                 \\\\\\\\\\//////////

|			    |

|			    | No

V	        	    |		   ****

-------------------		    |	 	 *	*

|  BACKUP RETURN  |		    |<----------* MSGFRE *

|  TO CAL	  |		    |		 *      *

|  ADDRESS	  |		    |	   	   ****

-------------------		    V

|		   -------------------

****		 |		   |  BUMP RETURN    |

*      *		 |		   |  	  TO  	     |

* MSEFWT *--------->|		   |      CAL+4	     |

*      *		 |		   -------------------

****		 |			    |

V			    |

-------------------		    V

|     AC=	  |	   -------------------

|     MESSAGE	  |	   |  FIND RECEIVING |

|     ADDRESS	  |	   |  TASK #'s	     |

-------------------	   |  ENTRY IN	     |

|		   |  MSGTBL	     |

|		   -------------------

V		 	    |

***			    |

*	    *			    V

* WAITS *		   -------------------

*     *		   |    GET DATA     |

***		   |    FIELD OF     |

(Pg. B-10)		   |    MESSAGE	     |

-------------------

|

|

V

****

*	*

* ADDTOQ *

*      *

****

(Pg. B-7)

B-6

RTS/8 FLOWCHARTS

****

*      *

* ADDTOQ *

*      *

****

|

|

V

------------------

|  GET FIRST     |

|  QUEUE WORD    |

|  (CDF TO NEXT) |

------------------

|

|

////\\\\

Yes	  /	   \

-----------------------/  EMPTY   \

|			 \  (=0) ?  /

|			  \	   /

|			   \\\\////

|			      |

|			      | No

|			      V

|		      ------------------

|	 	      |    CDF INTO    |

|		      |    NEXT	       |

|		      |    MESSAGE     |

|		      ------------------

|			      |

|			      |

|			      V

|		      /////////\\\\\\\\\\

|		     /                   \

|		    /	      IS          \

|		   /	 SENDING TASK      \  No    ---------------

|		  /  NUMBER LESS THAN TASK  \------>|  FIND NEXT  |

|		  \	NUMBER IN THIS      /	    |  MESSAGE IN |

|		   \	  QUEUE MSG        /	    |  QUEUE      |

|		    \	      ?           /	    ---------------

|		     \			 /		   |

****		      \\\\\\\\\//////////		   |

*      * 			      |				   V

* ENDOFQ *---------------------->| Yes			  ****

*	     *			      |				*      *

****			      |			       * ADDTOQ *

|				*      *

V				  ****

To Page B-8			(Pg. B-7)

B-7

RTS/8 FLOWCHARTS

From Page B-7

|

V

------------------

|  PUT CDF TO    |

|  MSG AND PC OF |

|  MSG INTO      |

|  QUEUE WORDS   |

------------------

|

|

V

------------------

|      IOF	|

------------------

|

|

V

------------------

|   GET TASK #	|

|   TO SEND	|

|   MESSAGE TO 	|

------------------

|

|

V

------------------

|  FREE MSG WAIT |

|  AND/OR EVENT	|   Via call to FREEJ

|  WAIT BITS IN	|

|  STATUS TABLE 	|

------------------

|

|

V

--------------------

|  L=1 IF REVEIV-  |

|  ING TASK IS     |   This will be tested if

|  RUNNABLE & OF   |   this CAL was a SEND

|  HIGHER PRIORITY |   and not a SENDW.

|  THAN SENDER;	 |

|  L=0 IF NOT	 |

--------------------

|

|

V

To Page B-9

B-8

RTS/8 FLOWCHARTS

From Page B-8

|

D		    ****	       |

O		  *      *	       |

* MSGCDF *----------->|

N		  *      *	       |

O              ****	       V

T			      -------------------

|  STORE SENDING  |

T			      |  TASK NUMBER    |

O			      |  IN MSG HEADER  |

U			      -------------------

C				       |

H    Link indicates 	       |

whether a new		       V

L    task should be      -------------------

I    run or not.	      |  STORE CDF TO	|

N			      |  NEXT MSG IN	|   =0 if none

K    The link status     |  MSGHDR		|

will be checked     -------------------

I    later on in		       |

N    POSTEX.		       |

V

T			      -------------------

H			      |   STORE PC OF   |

I			      |   NEXT MSG IN   |   =0 if none

S			      |	  MSGHDR	|

-------------------	  Message is now

C				       |		  linked into the

O				       |		  message queue

D				       |

E				       V

////\\\\		   ****

!				  /        \  No	 *	*

!				 /  CAL WAS \---------->* POSTEX *

!				 \  SENDW   /		 *	*

!				  \   ?    /		   ****

!				   \\\\////		 (Pg. B-18)

|

|  Yes

V

****

*      *

* MSEFWT *

*      *

****

(Pg. B-6)

B-9

RTS/8 FLOWCHARTS

---------							  ---------

| WAITE |							  | WAITE |

| WAITX |							  | WAITX |

---------							  ---------

****		      ****

*      *		    *      *

* XWAITE *		   * XWAITX *

*      *		    *      *

****		      ****

|		       |

|		       |

V		       V	   Calling Sequence:

-------------	---------------

|   CLEAR   |	|  SET ACARG  |    p   CAL

|   ACARG   |	|    TO -1    |    p+1 WAITE

-------------	---------------    p+2 EVENT FLAG ADDR

|		       |	   p+3 RETURN

-------------------->|

|

V

-------------------

Enter Here From	      |   GET ADDRESS   |

SEND or SENDW          |  OF EVENT FLAG  |

(Pg. B-6)	      -------------------

****	              	       |

*      *	               |

* WAITS  *-------------------->|

*      *	               |

****			       V

-------------------

ACARG /= -1	      |  SAVE ADDRESS   |

|  OF EVENT FLAG  |

-------------------

|

|

V

-------------------

|       IOF	|

-------------------

|

|

V

////\\\\		    ****

/	    \  Yes	  *      *

/  EVENT   \---------->* EXRET  *

\ FLAG = 0 /		  *      *

\	    /		    ****

\\\\////

|    No		Restart caller if

|		TASKSW allows

V		(Pg. B-4)

To Page B-11

B-10

RTS/8 FLOWCHARTS

From Page B-10

|

V

-------------------

|     EVTFLG =    |   Give it waiting

|  4000 + TASK #  |   status

-------------------

|

V

////\\\\

Yes   /	    \

------------------/ WAS THIS \

|		  \ A WAITX  /

|		   \    ?   /

|		    \\\\////

V	               |    No

-----------------              |

|  PUSH PC BACK |              V

|  TO POINT TO  |	   ---------------   Store Event Flag

|      CAL	|	   |  AC = EFWT  |   Wait in Flag Table

-----------------	   ---------------

|			  |

|			  |

V			  V

-----------------		 ****	     Jump with link clear

|  AC = EORMWT  |	       *      *	     (tested at TSTOP)

-----------------	      * TSWAIT *     (Pg. B-12)

|		       *      *

|		 	 ****

V

****

*	   *

* TSWAIT *

*	   *

****

(Pg. B-12)

B-11

RTS/8 FLOWCHARTS

(Pg. B-17)

****

*	  *         Wait for mesasge.

* WAITM3 *        Enter here from RECEIVE.

*	  *

****

|

|

V

-----------------------

|  BACKUP CALLING PC  |

|  TO DO CALX AGAIN   |

-----------------------

|

V

-------------------

|     SET LINK	|   Message Wait Status

|    AC = MSGWT	|

-------------------

****			       |

*      *		       |

* TSWAIT *-------------------->|

*      *		       |

****			       V

-------------------

| SAVE AC IN MASK |

(Pg. B-19)	      -------------------

****			       |

*      *		       |

* TSWATX *-------------------->|

*      *		       |

****			       V

-----------------------

|  STORE IN TASK #'s  |

|  FLAG TABLE ENTRY   |

-----------------------

|

V

-------------------   Interrupts were off

|       ION	|   here if we came from

-------------------   XWAITE.

|

V

To Page B-13

B-12

RTS/8 FLOWCHARTS

From Page B-12

|

V

///\\\

/	   \   Yes

/  LINK  \------------

\  SET ? /		|

\	   /		|

\\\///		|

|   No		|

V		|

-------------------	|

|  CLEAR TASK AC  |	|

-------------------	|

****			       |		|

*      *		       |		|

* TSTOP	 *-------------------->|		|

*      *		       |		|

****			       V		|

----------------------	|

|  SAVE AC IN ACARG  |	|

----------------------	|

|		|

|<----------------

V

-----------------------------

|  GET TASK NUMBER'S ENTRY  |

|      IN STATUS TABLE      |

-----------------------------

|

V

-------------------

|   SAVE RESTART  |

|  FLAGS, PC, AC  |

-------------------

|

V

-------------------

|     SAVE MQ	|

-------------------

EAE      |

V

---------------------------

|  FREE PARTITION IF FREE |

|  BIT WAS SET IN COMMAND |

---------------------------

SW       |		    ****

|		  *	 *

|<----------------* FINDJ  *

|		  *	 *

|		    ****

V		 RTS/8 Scheduler

To Page B-14            (Pg. B-30)

B-13

RTS/8 FLOWCHARTS

From Page B-13

|

V

Schedule next	      -------------------

runnable task	      |    TASK # = 1   |

(start scan 	      -------------------

from top)			       |

V

-------------------

|      ENABLE 	|

|  TASKSWITCHING  |

-------------------

|

V

-------------------------

|  SET 'MACHINE STATUS  |

|  UNIMPORTANT' FLAG    |

-------------------------

****			       |

*      *		       |

* FINDJL *-------------------->|

*      *  ^		       |

****	   |		       V

|	    -----------------------

|	    |  GET TASK #'s ENTRY |

|	    |    IN FLAG TABLE    |

|	    -----------------------

|		       |

|		       |	    Interrupts temporarily

|		       |	    inhibited here.

|		       |

|		       V

|		   /////\\\\\		    ****

|		  /	     \  Yes	  *      *

|		 / FLAGS = 0  \--------->* STARTJ *

|		 \ (RUNNABLE) /		  *      *

|		  \     ?    /		    ****

|		   \\\\\/////		 (Pg. B-15)

|	               |    No

|		       |

|		       |	    Note no end of table

|		       |	    check - there must be

|		       |	    a runnable task.

|		       |	    (Null task is always

|		       |	    runnable.)

|		       V

|	     ----------------------

|	     |  BUMP TASK NUMBER  |

|	     ----------------------

|		       |

|		       |

---------------------

B-14

RTS/8 FLOWCHARTS

****

*	  *	    Here from FINDJ

* STARTJ *	    (Scheduler)

*	  *	    Found a runnable task

****

|

V

-------------------

|       ION	|

-------------------

|

V

-----------------------

|  GET TASK #'s ENTRY |

|   IN STATUS TABLE   |

-----------------------

|

V

-------------------

|       IOF	|

-------------------

|

V

-----------------------

|  SAVE OLD STATUS IN |

|    TEMP LOCS OF	  |

|  INTERRUPT ROUTINE  |

-----------------------

|

Interrupt			       |

Dismiss code		       |

|

|

****			       |

*      *		       |

* DISMIS *-------------------->|

*      *		       |

****			       |

V

/////\\\\\		    ****

/	     \  Yes	  *      *

This is tested on		 /  PENDING   \--------->* USERSK *

an 8/E or 8/a only		 \ INTERRUPTS /		  *      *

\     ?    /		    ****

\\\\\/////		 (Pg. B-23)

|    No

V

---------------------------

|  SWEEP OLD STATUS INTO  |

|  DF, IF, AC, PC, ETC.   |

---------------------------

|

V

To Page B-16

B-15

RTS/8 FLOWCHARTS

From Page B-15

|

V

-------------------

|    RESTORE MQ	|

-------------------

EAE      |

V

-------------------

|       ION	|

-------------------

|

V

---------------------------

|  	    JMP I 0	    |

|  RETURN FROM INTERRUPT  |

---------------------------

B-16

RTS/8 FLOWCHARTS

----------							 ----------

| RECEIV |							 | RECEIV |

----------							 ----------

*****	    Calling Sequence:

*	   *

* XRECEIV *	    p-1  TAD ARG

*	   *	    p    CAL

*****	    p+1  RECEIV

|	    p+2  MADDR, 0

|	    p+3  RETURN

V

-------------------

|   GET ENTRY IN	|

|  MESSAGE QUEUE	|

-------------------

|

V

-------------------

|  GET FIRST WORD |

|  OF QUEUE ENTRY |

-------------------

|

V

/////\\\\\		    ****

/	     \  Yes	  *      *

------------------->/   EMPTY    \--------->* WAITM3 *

|			 \ (NO MSGS)  /		  *      *

|			  \    ?     /		    ****

|	   		   \\\\\/////		 (Pg. B-12)

|	   	               |    No

|	   		       V

|			   /////\\\\\		 -----------------

|			  /	     \  No	 |  UNLINK THIS	 |

|			 /  ARG = 0   \--------->|  MESSAGE FROM |

|			 \     ?      /	    ^	 |   THE QUEUE 	 |

|			  \          /	    |	 -----------------

|	   		   \\\\\/////	    |		 V

|	   	               |    Yes	    |	 -----------------

|	   		       V	    |	 |  STORE ADDR   |

|			   /////\\\\\	    |	 |  OF MESSAGE   |

-----------------	  	  /	     \      |    |   IN CAL+2	 |

|  ADVANCE TO   |	     No	 /  SENDING   \ Yes |    -----------------

|  NEXT MESSAGE |<---------/ TASK OF MSG  \----|		 V

|  ON THE QUEUE |		\    = ARG     /	 -----------------

-----------------		 \	      /		 |  AC = CDF TO  |

\     ?    /		 |    MESSAGE	 |

\\\\\/////		 -----------------

V

****

*      *

* EXRET  *

*      *

****

(Pg. B-4)

B-17

RTS/8 FLOWCHARTS

----------							 ----------

| UNBARG |							 | UNBARG |

----------							 ----------

*****

*	   *

* XUNBARG *

*	   *

*****

|

V

-------------------

|  GET BIT MASK	|

|  FROM ARG LIST  |

-------------------

|		     ****

|		   *	  *

|<-----------------* UNBLOK *

|		   *	  *

V		     ****

-------------------

|	      IOF	|

-------------------

|	    Call FREEJ

V

---------------------  L=1 if clearing mask

|  CLEAR MASK BITS  |  bits makes task

|   IN FLAG TABLE   |  runnable and it is

---------------------  higher priority;

|	    L=0 otherwise.

V

-------------------

|	      ION	|

(Pg. B-9)	      -------------------

****			       |

*      *		       |

* POSTEX *-------------------->|

*      *		       |

****			       V

/////\\\\\		    ****

/	     \  No	  *      *

/  RUN NEW   \--------->* EXRET  *

\    TASK    /	 L = 0	  *      *

\     ?    /		    ****

\\\\\/////		 (Pg. B-4)

|    Yes

L = 1  |

V

****

*	   *

* TSTOP  *

*	   *

****

(Pg. B-13)

B-18

RTS/8 FLOWCHARTS

----------							 ----------

| BLKARG |							 | BLKARG |

----------							 ----------

*****	    Calling Sequence:

*	   *

* XBLKARG *	    p-1  TAD TASK #

*	   *	    p    CAL

*****	    p+1  BLKARG

|	    p+2  WAITBITS

|	    p+3  RETURN

V

-------------------

|  GET WAIT BITS  |

|  FROM ARG. LIST |

(Pg. B-28)	      -------------------

****			       |

*      *		       |

*  BLOK  *-------------------->|

*      *		       |

****			       V

/////\\\\\

/	     \		    ****

/  CALLING   \  Yes	  *      *

/  TASK TO BE  \-------->* TSWATX *

\    BLOCKED   /	  *      *

\ (TASK# = 0)/		    ****

\    ?     /		 (Pg. B-12)

\\\\\/////

|    No

V

-----------------------

|  SET MASK BITS IN   |

|  FLAG TABLE ENTRY   |

|  OF SPECIFIED TASK  |

-----------------------

|

V

*****

*	   *

*  EXRET  *

*	   *

*****

(Pg. B-4)

B-19

RTS/8 FLOWCHARTS

----------							 ----------

|  POST  |							 |  POST  |

----------							 ----------

*****

*	   *

*  XPOST  *

*	   *

*****

|	    Calling Sequence:

V

-------------------   p-1  TAD EFPTR

|  SAVE ORIGINAL  |   p    CAL

|   EVENT FLAG    |   p+1  POST

-------------------   p+2  ECDF, CDF EFFLD

|	    p+3	 RETURN

V

----------------------

|  CLEAR EVENT FLAG  |

----------------------

|

V

/////////\\\\\\\\\

/			 \		    ****

/    WAS SOMEONE     \  No		  *	 *

/  WAITING FOR THIS TO \------------>* EXRET  *

\	HAPPEN (OLD FLAG   / 	  	  *	 *

\	    NEGATIVE)     /  L=0	    ****

\	       ?         /	 	 (Pg. B-4)

\\\\\\\\\/////////

L=1  |    Yes

V

-------------------------

|  GET WAITING TASK'S #  |

|  FROM LOW ORDER BITS   |

|  OF THE OLD FLAG       |

--------------------------

|

V

---------------------------

|  CLEAR BOTH EVENT FLAG  |

|    WAIT AND EVENT OR    |

|         MESG WAIT	    |

---------------------------

|

V

*****

*	   *

* UNBLOK  *

*	   *

*****

(Pg. B-18)

B-20

RTS/8 FLOWCHARTS

----------							 ----------

| SKPINS |							 | SKPINS |

----------							 ----------

*****	    Calling Sequence:

*	   *

* XSKPINS *	    p    CAL

*	   *	    p+1  SKPINS

*****	    p+2  MODULE ADDR

|	    p+3  RETURN

|

V

-------------------

|    GET FIRST    |

|   MODULE NAME   |

-------------------

|

V

/////\\\\\

/	     \  No      -------------------

/  IS IT 0   \-------->|  ALREADY IN THE |

\ 	?     /		|  SKIP CHAIN !!! |

\          /		-------------------

\\\\\/////			  |

|    Yes			  V

V			*****

-----------------------	      *	      *

|  PUT DISMIS ROUTINE |	     *  EXRET  *

|   ADDR IN 1st WORD  |	      *	      *

-----------------------		*****

|		     (Pg. B-4)

V

--- -------------------

|   PUT CDF, CIF OF   |

|  DISMIS IN 2nd WORD |

-----------------------

|

V

-------------------------

|  PUT MODULE ADDRESS   |

|  AND CDF, CIF IN LAST |

|  MODULE OF INTERRUPT  |

|      SKIP CHAIN       |

-------------------------

|

V

*****

*	   *

*  EXRET  *

*	   *

*****

(Pg. B-4)

B-21

RTS/8 FLOWCHARTS

----------							 ----------

| DERAIL |							 | DERAIL |

----------							 ----------

****

*	  *

* XDERAL *

*	  *

****

|

V

-------------------   Calling Sequence:

|   GET TASK #'s  |

|   STATUS TABLE  |   p-1  TAD TASK #

|     POINTER     |   p    CAL

-------------------   p+1  DERAIL

|            p+2  SUBROUTINE

V	    p+3  RETURN

-------------------

|  STORE RESTART  |

|   PC IN SUBR    |

-------------------

|

V

-------------------

|  STORE SUBR+1   |

|  IN RESTART PC  |

-------------------

|

V

*****

*	   *

*  EXRET  *

*	   *

*****

(Pg. B-4)

B-22

RTS/8 FLOWCHARTS

*********

*	     *

*  INTERRUPT  *

*   ROUTINE   *

*           *

*********

|

|

V

/////\\\\\

/	     \		    *******

/    TIME    \  Yes	  *  GO TO  *

/    SHARING   \--------->*   OS/8    *

\   INTERRUPT  /          * SUPPORT *

\      ?     /		    *******

\          /		  if present

\\\\\/////

|    No

|

V

-------------------

|     SAVE THE    |

|  MACHINE STATUS |

(Pg. B-15)	      -------------------

****			       |

*      *		       |

* USERSK *-------------------->|

*      *		       |

****			       V

---------------------

|   USER-INSERTED   |

|   SKIPS FOR ANY   |

|  CRUCIAL DEVICES  |

---------------------

|

|

V

*****		   /////\\\\\

*	    *		  /	     \

*    GO TO    *	    Yes	 /    KL8A    \

* APPROPRIATE *<---------\  INTERRUPT /

*  INTERRUPT  *		  \     ?    /

* MODULE  *  		   \\\\\/////

*****	               |    No

|

V

To Page B-24

B-23

RTS/8 FLOWCHARTS

From Page B-23

|

V

/////\\\\\	        -------------------

/	     \  Yes     |  SET UP LOC 0   |

/   POWER    \-------->|  FOR POWER FAIL |

\  FAILURE   /		|     RESTART     |

\     ?    /		-------------------

\\\\\/////			 |

|    No			 V

V		     ********

/////\\\\\ 		    *	     *

*******		  /	     \		    *  HALT  *

*  GO TO  *	    Yes	 /   CLOCK    \		    *	     *

*   CLOCK   *<-----------\ 	      /		     ********

* HANDLER *		  \     ?    /

*******	   	   \\\\\/////		 as CPU restarts

if present	               |    No		   ************

V		  * 	       *

*********		  *  POWER-UP  *

*             *	  *  RESTART   *

*  GO TO FIRST  *	  * 	       *

*  USER INTER-  *	   ************

*  RUPT MODULE  *		 |

*             *		 V

*********		-------------------

|  AC = POWER-UP  |

|   EVENT FLAG    |

|     ADDRESS     |

-------------------

|

V

****

*	    *

* DSPOST *

*	    *

****

(Pg. B-25)

B-24

RTS/8 FLOWCHARTS

----------							 ----------

| POSTDS |							 | POSTDS |

----------							 ----------

(Pg. B-24)

*****

*	   *

*  DSPOST *

*	   *

*****

|	      Come here from user

V	      issusing POSTDS

/////\\\\\

****			  /	     \ 	      p-3  CDF EFFLD

*      *	    Yes	 /   AC = 0   \	      p-2  TAD EFPTR

* DISMIS *<--------------\ 	?     /	      p-1  COF 0

*      *		  \          /	      p    POSTDS

****	   		   \\\\\/////

(Pg. B-15)	               |    No

V

----------------------

|    SAVE THE OLD    |

|  EVENT FLAG VALUE  |

----------------------

|

V

---------------------

|  ZERO EVENT FLAG  |

---------------------

|

V

////////\\\\\\\\

/                \		    ****

/       WAS A      \  Yes	          *      *

/    TASK WAITING    \------------->* DISMIS *

\ ON THIS FLAG? (OLD /	          *	 *

\ VALUE NEGATIVE)  /		    ****

\       ?        /	         (Pg. B-15)

\\\\\\\\////////

|    No

V

--------------------------

|  CLEAR EVENT FLAG AND  |

|    EVENT OR MESSAGE    |   Call FREEJ

|   WAIT BITS IN TABLE   |

|   ENTRY OF WAIT TASK   |

--------------------------

|

V

To Page B-26

B-25

RTS/8 FLOWCHARTS

From Page B-25

|

V

/////\\\\\

Run the waiting task if	  /	     \		     ****

its flag word is now 	 /    RUN     \  No	   *      *

zero and it is higher	/   WAITING    \--------->* DISMIS *

priority than the 		\    TASK      /	   *      *

current user.		 \     ?      /		     ****

\          /		  (Pg. B-15)

\\\\\/////

|    Yes

V

/////\\\\\

/	     \  	  -----------------

/    TASK    \  Yes      |  SET RESCAN   |

/   SWITCHING  \--------->|     FLAG      |

\   INHIBITED  /  	  |  CTSFWLG = -1 |

\  	?     /		  -----------------

\          /			  |

\\\\\/////			  V

|    No			****

V		      *	     *

/////\\\\\		     * DISMIS *

/	     \ 		      *	     *

No	 /  IS SAVE   \			****

-----------------\ STATE FLAG /		     (Pg. B-15)

|		  \   -1     /

|   		   \\\\\/////

|   	               |    Yes

|   		       V

|	    ------------------------

|	    |  SAVE CURRENT TASK'S |

|	    |     PC, AC IN ITS    |

|	    |  STATUS TABLE ENTRY  |

|	    ------------------------

|		       |

|		       V

|	      -------------------

|	      |  SAVE TASK'S MQ |

|	      -------------------

|	      EAE      |

|--------------------->|

V

-------------------

|       ION	|

-------------------

|

V

To Page B-27

B-26

RTS/8 FLOWCHARTS

From Page B-27

|

V

-------------------

|    PREPARE TO   |

|  START NEW TASK |

-------------------

|

V

/////\\\\\			    ****

/	     \  No		  *      *

/   IS IT    \----------------->*  FINDJ *

\  RESIDENT  /			  *      *

\     ?    /			    ****

SW   \\\\\/////			 (Pg. B-30)

|    Yes

V

****

*	  *

* STARTJ *

*	  *

****

(Pg. B-15)

B-27

RTS/8 FLOWCHARTS

---------							  ---------

|  RUN  |							  |  RUN  |

---------							  ---------

*****

*	   *

*  XRUN   *

*	   *

*****

|		Calling Sequence:

V

-------------------	p-1  TAD TASK #

|   AC = RUNWT    |	p    CAL

-------------------	P+1  RUN

| Run Wait	p+2  RETURN

V

*****

*	   *

* UNBLOK  *		Go and set the

*	   *		flag bits.

*****

(Pg. B-18)

----------							 ----------

| SUSPND |							 | SUSPND |

----------							 ----------

*****

*	   *

* XSUSPND *

*	   *

*****

|		Calling Sequence:

V

-------------------	p-1  TAD TASK #

|   AC = RUNWT	|	p    CAL

-------------------	p+1  SUSPND

|		p+2  RETURN

V

*****

*	   *

*  BLOK   *

*	   *

*****

(Pg. B-19)

B-28

RTS/8 FLOWCHARTS

---------							  ---------

| WAITM |							  | WAITM |

---------							  ---------

*****

*	   *

*  XWAITM  *

*	   *

*****

|

V

-------------------

|    SAVE AC IN   |

|    'COMMAND'  	|

-------------------

SW       |

V

-------------------

|   CLEAR AC, L   |

-------------------

|

V

-------------------

|   FAKE A CAL	|

-------------------

|

V

-------------------

|  DISABLE TASK   |

|   SWITCHING	|

-------------------

|

V

-------------------

|     PUT WAIT	|

|    BITS IN AC	|

-------------------

|

V

*****

*	   *

*  TSWAIT *

*	   *

*****

(Pg. B-12)

B-29

RTS/8 FLOWCHARTS

*****

*	   *		RTS/8 SCHEDULER

*  FINDJ  *		FIND A TASK TO RUN

*	   *

*****

|

V

-------------------

|   ENABLE TASK	|

|    SWITCHING	|

-------------------

|

V

-----------------------

|      GET SET TO     |

|  EXAMINE FIRST TASK |

-----------------------

|

V

-------------------

|    ZERO SAVE	|

|    STATE FLAG	|

-------------------

|

---------------------->|

|		       V

|	      -------------------

|	      |       ION	|

|	      -------------------

|		       |

|		       V

|	      -------------------

|	      |     BUMP TO 	|

|	      |     NEXT TASK	|

|	      -------------------

|		       |

|		       V

|	      -------------------

|	      |       IOF	|

|	      -------------------

|		       |

|		       V

|	      -------------------

|	      |  GET TASK FLAGS |

|	      -------------------

|		       |

^		       V

From Page B-31		  To Page B-31

B-30

RTS/8 FLOWCHARTS

To Page B-30		 From Page B-30

^		       |

|		       V

|		  //////\\\\\\

|		 /            \

|		/     ANY      \

|	  Yes  /    BLOCKING    \

|--------------\BITS OTHER THAN /

\     NRWT     /

\     ?      /

\\\\\\//////

|     No

V

/////\\\\\			    ****

/	     \  Yes		  *      *

/  IS TASK   \----------------->* STARTS *

\  RESIDENT  /			  *      *

\     ?    /			    ****

\\\\\/////			 Start task

SW   |    No			 (Pg. B-15)

V

-------------------

|    PUT TASK	|

|    INTO SWPWT	|

-------------------

SW       |

V

-------------------

|   TAKE SWAPPER	|

|   OUT OF RUNWT	|

-------------------

SW       |

V

*****

*	   *

*  FINDJ  *

*	   *

*****

(Pg. B-30)

B-31

APPENDIX C

GLOSSARY

ACCUMULATOR

A part of the arithmetic unit of a computer.  It is a register that

receives  the results of a computation or holds one of the operands

in a computation.

ADDRESS

1. A name, label, or number which identifies a register, a location

in storage, or any other data source or destination.

2. The part of an instruction that specifies  the  location  of  an

operand of that instruction.

ANALOG

Signals, and other such quantities, which  can  occur  anywhere  on

some continuous scale.

ANALOG-TO-DIGITAL CONVERTER (ADC)

A circuit which converts analog signals to digital data.

ARGUMENT

1. A variable or constant which is given in a function  call  or  a

subroutine as information to it.

2. The known reference factor necessary to find an item in a  table

or array (i.e., the index).

ASSEMBLER

A program which  translates  assembly  language  instructions  into

machine  language  instructions  and  assigns  memory  locations to

variables and constants.

ASYNCHRONOUS

The scheduling of operations by "ready" and "done" signals rather

than by time intervals.

AUTO-INDEX REGISTER

Whenever one of the absolute locations from 0010  through  0017  in

any  memory  field  is  addressed  indirectly, the contents of that

location is incremented by one, rewritten in the same location, and

used  as the effective address of the current instruction.  Because

these registers increment by one, the programmer can  use  them  in

automatically indexing tables.

AUTORESTART

The ability to start up the CPU automatically on power up.

BACKGROUND PROCESSING

The automatic execution of a low  priority  computer  program  when

higher-priority  programs  are  not  using  the system's resources.

RTS/8  protects  the  foreground  from  the  background   but   the

background is not protected from the foreground.

C-1

GLOSSARY

BAUD

A unit  of  signalling  speed  equal  to  the  number  of  discrete

conditions  or  signal  events  per  second.  For example, one baud

equals one-half dot cycle per second in  Morse Code,  one  bit  per

second in a train of binary signals, and one 3-bit value per second

in a train of signals, each  of  which  can  assume  one  of  eight

different values.

BIT (BINARY DIGIT)

In the binary notation, either of the characters 0 or 1.

BLKARG

An RTS/8 Executive Request which stands for Block Argument.

BLOCK

1. A set  of  consecutive  machine  words,  characters,  or  digits

handled  as  a  unit, particularly in reference to I/O.  An OS/8

block is 400 (octal) contiguous words.

2. To inhibit a process from continuing.

BLOCK GAP

The blank space between blocks on a recording medium.

BLOCKING BITS

Bits in an RTS/8 monitor table which specify why a  given  task  is

blocked.

BYTE

A group of bits considered as a unit;  usually a subdivision  of  a

word.  Bytes on the PDP-8 are six bits in length.

CAL

An RTS/8 mnemonic which indicates JMS 20.

CASSETTE

A small case containing a length of magnetic tape--the tape remains

in the case for convenient use.

CENTRAL PROCESSING UNIT (CPU)

The part of the computer  containing  the  arithmetic  and  logical

units,  the  instruction control unit, and other circuits necessary

for controlling the interpretation and execution of instructions.

CHECKPOINTABLE TASK

A task that may be swapped out of memory automatically, without its

consent, to make room for higher-priority tasks.

CHECKSUM

A value representing the sum of all bytes in a program.   When  the

program  is  loaded,  the sum of the bytes can be compared with the

checksum to make sure that the entire program has been loaded.

CLEAR

1. To erase the contents of a storage  location  by  replacing  the

contents with zeros or spaces.

2. In binary code, to set to zero.

C-2

GLOSSARY

CLOCK

A device that generates periodic signals for synchronization.

COMMUNICATION REGION

Locations 20-27 of every field, used to simplify passing  Executive

Requests across field boundaries.

COMPUTE BOUND

Requiring extensive (or total) use of the  CPU  relative  to  other

hardware elements (such as I/O devices).

CONFIGURATION

1. A particular selection of computer, peripherals, and interfacing

equipment that function together.

2. A list of the devices and computers of a computer system.

CONTIGUOUS

Code which resides in memory immediately adjacent to other sections

of code.

CONTROLLER

The device or portion of a device which controls the  operation  of

connected  devices.   Some  controllers  can  initiate simultaneous

positioning commands to some of its devices and can then perform  a

transfer for one of its devices.

CORE IMAGE FILE

A file in core image format (i.e.,  a  'picture'  of  core);   also

known as a SAVE file.

CORE MEMORY

The main high-speed storage of a computer in which binary  data  is

represented  by  the  switching  polarity  of magnetic cores.  (See

also, MOS memory.)

COUNTER CHANNEL

A UDC/ICS functional device.

CPU REGISTERS

High-speed  circuitry  used  to  store  information  affecting  the

operation of the CPU (for example, PC, IF, DF).

DATA

A general term used to denote any or all facts,  numbers,  letters,

and symbols.

DATA FIELD (DF)

A register which determines the memory field  from  which  operands

are taken in indirectly addressed instructions.

DEBUG

To detect, locate, and correct mistakes in a program.

DEFERRED ACTIONS

Actions which are considered low priority  and  are  not  performed

C-3

GLOSSARY

until higher-priority actions are serviced.

DEFERRED REQUESTS

Requests which are considered low priority and are  serviced  after

high priority requests.

DERAIL

To  transfer  control  or  execution  of  a  specified  task  to  a

subroutine.

DEVICE CODES

Numbers assigned to each device in the system, used in the computer

instructions for those devices.

DEVICE STATUS REGISTER

A register which contains the current status of a device.

DIGITAL

Representing information by discrete units  instead  of  continuous

gradations.  (Note:  "digital" is not synonymous with "binary".)

DIRECTORY-STRUCTURED DEVICE

A storage volume with a true volume directory at its beginning that

contains information (file name, file type, length, etc.) about all

the files on the volume.  Such volumes include all disks, diskettes

(floppies), and DECtapes.

DRIVER, see Handler.

DYNAMIC

Pertaining to a quantity that is affected by some  condition  (such

as  time) and is therefore relative to that condition;  also refers

to features at system run-time.

ENTRY POINT

The location in a routine to which control can be  transferred  and

execution begun.

EVENT

A change in the processor status or an indication of the occurrence

of some activity that concerns an individual process.

EVENT FLAG

A bit that can be set or cleared to indicate the occurrence of  the

event   associated  with  that  flag.   Event  flags  are  used  to

synchronize activities in a process or among processes.

EXECUTIVE

The central component of the operating system.  It co-ordinates and

controls all other tasks in the system.

EXECUTIVE REQUESTS

Means of communication between tasks and the RTS/8 Executive.

C-4

GLOSSARY

FIELD

A division of memory on a PDP-8 computer referring to a 4K  section

of memory.

FILE GAP

An area on a storage medium, such as tape, used to indicate the end

of a file.

FLAG

A variable or register used to record  the  status  of  a  task  or

device. In the latter case, it is sometimes called a device flag.

FREE

An RTS/8 Executive Request that tells the  Executive  it  can  swap

another task into a partition.

GAIN

Amplification.

HANDLER

A routine which is designed to control the operation of a device.

HEADER

The control information prefixed in a message text;   for  example,

source or destination code, priority, or message type.

HERTZ

A unit of frequency equal to one  cycle  per  second.   Cycles  are

referred  to  as Hertz in honor of the experimenter Heinrich Hertz.

Abbreviated Hz.

HIBERNATION

A state in which a task is inactive, but  known  to  the  Executive

with  all  of  its  current  status.  A hibernating process becomes

active again when a wake request is issued (such  as  clearing  its

RUNWT bit).  A task can schedule a wake request before hibernating,

or another process can issue the wake request.

I/O BOUND

A condition in which a process is performing  much  I/O  but  using

very little CPU time.

IDLE TIME

That part of CPU time in which no job can run because all jobs  are

halted or are waiting for some external action such as I/O.

INDIRECT ADDRESS

An address in a computer instruction which indicates a location  in

memory where the address of the referenced operand is found.

INITIALIZATION CODE

Code which sets counters, switches, and addresses to zero or  other

starting  values  at  the beginning of or at prescribed points in a

computer routine.

C-5

GLOSSARY

INPUT BUFFER

A section of memory used for storage of input data.

INSTRUCTION FIELD (IF)

A register which holds the  contents  that  determines  from  which

field  the  operand  of  a directly addressed instruction should be

taken.

INTERACTIVE

Pertaining to the exchange of information  and  control  between  a

user and a computer process or processes.

INTERFACE

A shared  boundary,  for  example,  the  wires  and  perhaps  other

electronics connecting two subsystems.

INTERLOCK SCHEME

Arranging the  control  of  devices  so  that  their  operation  is

interdependent.

INTERRUPT

1. To break the normal operation of the routine being executed  and

pass  control  to  a specific location--generally accompanied by

saving the state of the current  routine  so  that  control  can

return later.

2. Also, the signal which causes the interrupt.

INTERRUPT PROCESSING ROUTINE

INTERRUPT SERVICE ROUTINE

The routine entered when a device interrupt occurs.

INTERVAL QUEUE

A list of actions to perform, each accompanied by the  interval  of

time  which  is  to  elapse from the previous action to the current

one.

LINE FREQUENCY CLOCK

A clock whose ticking occurs  at  a  multiple  of  the  power  line

frequency.

LINK

1. A 1-bit register in the PDP-8.

2. An address pointer generated automatically by the  assembler  to

indirectly address an off-page symbol.

LOGICAL OR

A logical function of two or more inputs  which  is  true  whenever

either input is true.

LOOP

A portion of a program that is  constructed  so  that  the  central

processor  repeats  the same sequence of instructions over and over

again until a predefined condition occurs.

C-6

GLOSSARY

MASS STORAGE

A device such as disk or DECtape which stores large amounts of data

readily accessible to the CPU.

MEMORY ADDRESS

A register which holds the address specified by a memory  reference

instruction.

MEMORY REFERENCE INSTRUCTION

A computer instruction that accesses the computer memory during its

execution, as opposed to a register instruction which only accesses

registers in the CPU and I/O instructions  which  are  commands  to

peripheral devices.

MESSAGE

A contiguous area of memory containing information that is used  in

intertask communication or information to be used by an I/O device.

MESSAGE DRIVEN

A task is message  driven  if  it  executes  only  in  response  to

messages received from other tasks.

MNEMONIC

A symbolic representation of an  operation  code  or  address  (for

example, ATL for Atlanta, JMP for jump, etc.).

MODULE

A routine which handles a particular function.

MONITOR CONSOLE ROUTINE (MCR)

The routine providing the user with functions which  allow  him  to

control, inspect, and debug a system.

MOS (METAL-OXIDE SEMICONDUCTORY) MEMORY

A type of memory that stores data by using a  charge  to  represent

the  value of a bit.  This memory is volatile in that a charge must

continually be applied to  it;   consequently,  if  such  a  system

crashes,  it  no longer retains the information stored in it.  Thus

power fail hardware cannot be used to restore it if a power failure

occurs.

MULTIPLIER QUOTIENT (MQ)

A 12-bit register used with the accumulator to perform mathematical

operations.

MULTIPROGRAMMING

A technique that allows scheduling such that more than one job at a

time  is  in  an executable state.  Thus, even with one CPU, it can

appear that more than one program is running simultaneously because

the  CPU  is  giving  small  bits  of  its  time to each executable

program.

NONRESIDENT TASK

A task or a portion of a task that is swapped into memory  when  it

becomes executable.

C-7

GLOSSARY

NO-OP

An instruction which specifically  instructs  the  computer  to  do

nothing for one cycle and then to get the next instruction.

NULL CHARACTERS

Characters with ASCII code of 000.

OVERFLOW

A condition occurring when a mathematical operation yields a result

whose magnitude is larger than that which the program is capable of

handling.

OVERLAY

The technique of repeatedly using the same area  of  memory  during

different  stages  of  a  program.   When  one routine is no longer

needed in memory, another routine can replace it.

PACK

1. To compress data in memory or on a peripheral storage device  by

taking  advantage  of  known characteristics of the data so that

the original data can be recovered.

2. A disk pack (that is, a removable set  of  disks  mounted  on  a

common shaft).

PAGE

1. That portion of a text file delimited by  form  feed  characters

and generally 50-60 lines long.

2. A 128-word section of memory, beginning at an address which is a

multiple of 128.

PARITY BIT

A binary digit appended to an array of binary digits  to  make  the

sum of all bits in a word or byte always odd or always even.

POINTER

1. A location or register containing an address rather  than  data.

A pointer may be used in indirect addressing or in indexing.

2. An instruction indicating the address, position, and length of a

byte of information (for example, a byte pointer).

POST

To set an event flag to a FINISHED state  via  an  RTS/8  Executive

Request.

POSTDS

An RTS/8 system mnemonic which  allows  posting  at  the  interrupt

level.

POSTED

One of the states of an event flag, indicating that  the  event  is

complete;  same as FINISHED.

POWER FAIL

Logic circuits that protect  an  operating  program  in  the  event

computer  primary  power  dies.   The  circuits automatically store

C-8

GLOSSARY

current operating parameters, restore them, and  resume  processing

when power returns.

PRIORITY SCHEME

A scheme by which certain operations or the execution of a  set  of

instructions is given preference over other operations.

PROGRAM COUNTER

A register in the  CPU  that  holds  the  address  of  the  current

instruction being executed plus one word;  in other words, it holds

the address of the next instruction unless the current  instruction

causes a jump.

PROMPTING CHARACTER

A character which prints on the console terminal and cues the  user

to perform some action.

QUEUE

A waiting list (for  example,  a  queue  of  programs  waiting  for

processing time).

READY FLAG

A bit which a device controller sets when it  is  ready  to  accept

commands from the CPU.

REAL-TIME SYSTEM

A system in which computation is performed while a related physical

process  is occurring so that the results of the computation can be

used in guiding or measuring the physical process.

RECEIV

An RTS/8 Executive Request which informs the Executive that a  task

wishes to receive a message.

RECEIVER

The task which has received a message from another task (sender).

RECORD

A collection of related items  of  data  treated  as  a  unit,  for

example,  a  line  of  source  code,  a person's name, rank, serial

number, etc.

RECORD HEADER AREA

An area of at least 40  (octal)  words  in  length  which  contains

information necessary to perform cassette operations.

RESCHD

An RTS/8 Executive Request which instructs the Executive to execute

the highest executable task.

RESPONSE TIME

The time between the generation of an inquiry or  request  and  the

receipt  of  the  response  or  the accomplishment of the requested

action.

C-9

GLOSSARY

RING BUFFER

A storage area for data accessed on a  first-in,  first-out  basis.

Similar to QUEUE, but usually involves storage of character codes.

ROOT SEGMENT

The portion of a nonresident task that is always in memory.

RUN

An RTS/8 Executive Request which clears a task's RUN WAIT bit.

SCHEDULING

The operation of sharing resources among computing tasks.

SEMAPHORE

A common word used to  control  the  exchange  of  signals  between

concurrent processes.

SEND

An RTS/8 Executive Request which transmits a message  to  one  task

from another.

SENDER

The task which has sent a message to another task (receiver).

SENDW

An RTS/8 Executive Request which transmits a message to  one  task,

then  places  the  task  into Message Wait until the receiving task

acknowledges receipt of the message.

SIGN BIT

In complementary arithmetic, the bit directly to the  left  of  the

number  indicates  the  sign (+ or -) of the number.  A 1 indicates

negative numbers while a 0 indicates positive.

SIMULATE

To represent the function of a  device,  system,  or  program  with

another device, system, or program.

SKIP CHAIN

An instruction sequence which determines the source of an interrupt

request;   contains  one or more tests for each possible (hardware)

interrupt condition.

SKPINS

An RTS/8 Executive Request that links interrupt processing routines

into a skip chain.

STATE

A complete description of the condition of a piece of  hardware  or

of a task.

SUSPEND

1. To temporarily halt execution of a task while  another  task  of

higher priority runs.

2. An RTS/8 Executive Request action which sets a waitbit.

C-10

GLOSSARY

SWAPPING

The movement by the Executive of a task between memory and  a  mass

storage  device.   This  allows  more than one task to use the same

memory space.

SWAPPING DEVICE

A mass storage device from which swapping occurs.  It must  be  the

system device.

SYNCHRONIZATION

A means of coordinating tasks (through event  flags)  so  that  one

task executes while others wait.

SYSTEM SOFTWARE

DIGITAL-supplied  programs  which  come  in  the   basic   software

packages.

SYSTEM TICKS

An RTS/8 convention designed to obviate the  tasks  in  the  system

from knowing the frequency of the clock.

TABLE

A collection of data sorted for ease of  reference;   generally  an

array.

TASK

A routine which performs a  specific  function  and  which  may  be

resident or nonresident.  A resident task is permanently located in

memory.  A nonresident task is loaded into memory as it  is  needed

and can be overlaid after its completion.

TASK FLAGS TABLE

A table of 1-word entries in the  RTS/8  Executive  whose  contents

determine whether a task is runnable.

TASK INPUT MESSAGE QUEUE HEADER TABLE

A table of 2-word entries which represent the  head  of  the  input

message queue for each task.

TASK NUMBER

A unique number between 1 and 127 (decimal) assigned to  each  task

in an RTS/8 system.

TASK STATE TABLE

A table of 4- or 5-word entries  which  contains  the  most  recent

contents of CPU registers for each task.

TASK SWITCHING

The act of stopping execution of one task and continuing  execution

of another from the point that it was last stopped.

TERMINAL

A device that permits data entry into or data exit from a  computer

system  or  computer  network.   Terminals  may accommodate data in

human or machine readable form.

C-11

GLOSSARY

TIME OUT

A specified amount of time that the system waits  for  a  response.

If there is no response within the specified time, an error occurs.

UNBARG

An RTS/8 Executive Request which clears a bit (or bits) in a task's

Task State Table.

UTILITIES

Routines to perform non-Monitor related functions.

WAITBITS

Bits which the RTS/8 Executive sets or clears.  These bits indicate

conditions that prevent a task from running.

WAITE

An RTS/8 Executive Request telling the Executive that a task wishes

to wait until another task posts an event flag.

WAITM

An RTS/8 mnemonic telling the  Executive  to  wait  until  multiple

event flags have been posted.

WAITX

An RTS/8 Executive Request telling the Executive that a task wishes

to wait until another task posts a particular event flag.

WAKE

To activate a hibernating process.  A hibernating  process  can  be

awakened  by  another  process  or by the clock, if the hibernating

process scheduled a wake-up call.

WORD

In the PDP-8, a 12-bit unit of data which  may  be  stored  in  one

addressable location.

C-12

INDEX

> (MCR prompt), 5-1		  Concurrent information

manipulation,

see Delicate code

Console handler,

ALARMH, 4-14			      see Terminal handler

ALARML, 4-14

Analog input UDC/ICS operation,

4-48

Analog output UDC/ICS operation,     DAte, MCR command, 5-3

4-49				  DECcassettes, 4-3

Delicate code, 7-1

DEposit, MCR command, 5-4

DERAIL, 3-7

Background terminal,		  DF32 handler,

see Terminal driver		    see Mass storage handler

BLKARG, 3-4			  Digital input UDC/ICS operation,

Blocking,				      4-51

see Executive requests,		  Digital output UDC/ICS

BLKARG				      operation, 4-51

Disable contacts UDC/ICS

operation, 4-52

Disable counter UDC/ICS

CAL, 3-1				      operation, 4-53

CAncel, MCR command, 5-3		  DIsable, MCR command, 5-4

Cassette file support handler,       Diskette,

4-8				    see Floppy disk handler

header formats, 4-9		  DNEWT, 3-5

message format, 4-9

message words, 4-8

operations, 4-8

Cassette handler, 4-3		  EFWT, 3-5

message format, 4-3		  Emergency breakthrough task,

message words, 4-4		      4-21

record size, 4-3			    message format, 4-21

Cassette utility function		  Emergency conditions,

handler, 4-6			    see Executive requests, DERAIL;

message format, 4-6		      Emergency breakthrough task

message words, 4-7		  ENABWT, 3-5

Change of state UDC/ICS		  Enable, MCR command, 5-5

operation, 4-50		  Enable contacts UDC/ICS

Checkpointable tasks, 6-2		      operation, 4-54

Clock handler, 4-11		  Enable counter UDC/ICS operation,

CLASS specification, 4-11	      4-55

commands, 4-12			  EORMWT, 3-5

immediate response subroutine,	  ER,

4-14				  see Executive requests

HERTZ, 4-11			  Event flags, 2-2, 3-16

maximum interval, 4-12		  multiple event waits, 7-5

message format, 4-11		    setting, 3-11

SHERTZ, 4-11			    states, 2-2, 2-3

Index-1

INDEX (CONT.)

Event flags (Cont.)		  Interrupts, 2-6

waiting for any, 3-26		  Intertask communication,

waiting for specific, 3-28	    see Task execution

EXamine, MCR command, 5-5

Executive, 1-1

location, 3-2

Executive requests, 3-1		  KL8-A, 7-15

BLKARG, 3-4			  KT8-A, 5-9, 7-7

DERAIL, 3-7, 4-45

dangers, 3-8

restrictions, 3-7, 3-8

FREE, 3-10, 6-4			  LEVEL definition, 6-5

POST, 2-2, 3-11			  LINCtape handler, 4-29

task control transfer, 3-11	    message format, 4-29

RECEIV, 3-12			    message words, 4-29

RESCHD, 3-14, 7-4

RUN, 3-15

SEND, 3-16			  Line printer handler, 4-32

task control transfer, 3-16	    see also, Terminal handler

SENDW, 3-17			      message format, 4-32

SKPINS, 3-18

summary, 3-2

UNBARG 3-24			  Mass storage handler, 4-34

wait states, 3-31		    message format, 4-34

WAITE, 2-3, 3-9, 3-26		    message word, 4-34

WAITX, 3-9, 3-26, 3-27		    RL01 format, 4-35

EXIT, MCR command, 5-5		    RL01 physical unit/logical

Exit task, 4-23			      unit correspondence, 4-35

MCR,

CAncel command, 5-3

command length, 5-1

Floppy disk handler, 4-24		    command summary, 5-2

interleave scheme, 4-28		    DAte command, 5-3

message format, 4-24		    DEposit command, 5-4

message words, 4-24		    DIsable command, 5-4

status bits, 4-26		    ENable command, 5-5

Foreground operation,		    error messages, 5-10

see Terminal driver		    EXamine command, 5-5

FREE, 3-10, 6-4			    EXIT command, 5-5

invoking, 4-17

NAme command, 5-5

OPen command, 5-6

Generic code UDC/ICS operation,	    POst command, 5-6

4-53				    prompt, 5-1

REquest command, 5-7

STop command, 5-8

syntax, 5-1, 5-2

Immediate response subroutine,	    SYstat command, 5-8

see Clock handler		    TIme, 5-10

Interrupt processing routines,       Message queue header table

3-18				      (MSGTBL), 7-10

execution time, 3-19		    location, 7-14

restrictions, 3-19		  Message queues, 2-4, 3-12

skip chain, 3-18

ordering, 3-18

Index-2

INDEX (CONT.)

Messages,				  Power fail task, 4-45

see Task communication,		  Priority

messages			    see Task priority

Mnemonics, A-1

Modifiable nonresident tasks,

6-3

MSGTBL, 7-10			  Read counter UDC/ICS operation,

MSGWT, 2-4				    4-57

Monitor console routine,		  Real-time operation, 1-3

see MCR				  RECEIV, 3-12

REquest, MCR command, 5-7

RESCHD, 3-14, 7-4

Residency table (RESTBL), 7-10

NAme, MCR command, 5-5		    location, 7-14

Nonconsole handler,		  RESTBL, 7-10

see Terminal handler		  RF08 handler,

NONRES macro, 6-5			    see Mass storage handler

Nonresident tasks, 3-10, 6-1	  RK8 handler,

checkpointable tasks, 6-2	    see Mass storage handler

debugging, 6-5			  RK8E handler,

FIELD definition, 6-5		    see Mass storage handler

LEVEL definition, 6-5		  RL01 format,

modifiable tasks, 6-3		    see Mass storage handler

NONRES macro, 6-5		  RL01 handler,

root segment, 6-1		    see Mass storage handler

SECT definition, 6-5		  RTS/8, described, 1-1, 1-3

NONRWT, 2-5			  RTS/8 Executive,

Null task, 4-44			    see Executive

RTS/8 mnemonics, A-1

RTS/8-OS/8 communication, 4-41

RTS/8 system execution, 2-8

OPen, MCR command, 5-6		  RUN, 3-15

OS/8 background terminal,		  RUNWT, 3-5, 3-15

see Terminal driver

OS/8 file support task, 4-38

message format, 4-38

message words, 4-38		  Scheduler,

OS/8-RTS/8 communication, 4-41	    see Task scheduling

OS/8 support task, 4-40		  SEND, 3-16

rebooting, 4-41			  SENDW, 3-17

SKPINS, 3-18

STop, MCR command, 5-8

SUSPND, 3-22

Swap device, 6-1

PARTBL, 7-11			  SWPWT, 3-5

Partition table, 7-11		  Synchronization,

Partitions,			    see Task communication

see Task partitions		  SYstat, MCR command, 5-8

PDP 8-A null task, 4-44		  System execution, 2-8

POST, executive request, 2-2,	  System status report, MCR

3-11				      command, 5-8

POST, MCR command, 5-6		  System tasks, 1-2

POSTDS, 3-19, 4-14			    summary, 4-1

Posting

see Task communication

Index-3

INDEX (CONT.)

Task communication, 2-2		  Terminal handler, 4-16

messages, 2-4, 2-5		    assigning tasks to, 4-18

acknowledging, 3-11		    background ownership, 4-17,

event completion signalling,	      4-40

3-11				    character mode, 4-18

formats, 2-4			    emergency breakthrough task,

message queues, 2-4, 2-5,	      see Emergency breakthrough

3-12					task

not waiting for, 3-13		    foreground ownership, 4-17

posting, 2-4			    line mode, 4-18

queues, 2-4, 2-5, 3-12		    MCR invocation, 4-17

receiving, 3-12		    message format, 4-16

send, 3-16			    message words, 4-16

send and wait, 3-17		    timeout, 4-17

waiting for, 3-12		  TFTABL, 7-8

with Executive, 3-1		  TIme, MCR command, 5-10

nonresident tasks, 6-1		  Timeout,

synchronization, 2-2		    see Terminal handler

Task creation, 2-6			  TSTABL, 7-7

Task execution, 2-2

changing, 3-7

resumption, 3-15

suspension of, 3-4, 3-22		  UDC/ICS handler, 4-46

Task flag's table (TFTBL), 7-8	    analog input, 4-48

location, 7-14			    analog output, 4-49

Task flag's word, 3-4, 3-24, 5-8	    change of state, 4-50

TASK macro, 2-7			    digital input, 4-51

Task partition, 3-10, 6-1, 6-3	    digital output, 4-51

competition for, 6-4		    disable contacts, 4-52

freeing, 6-4			    disable counter, 4-53

number, 6-3			    error messages, 4-58

Task priority, 2-1, 2-5		    enable contacts, 4-54

function, 2-1			    enable counter, 4-55

Task scheduling, 2-6, 3-14, 7-4	    generic code, 4-56

Task state table (TSTABL), 7-7	    operation summary, 4-47

location, 7-14			    read counter, 4-57

Task switching, inhibition,	  UNBARG, 3-24

7-3, 7-5			  USERWT, 3-5

Task tables, 7-6

locations, 47-12

message queue header table,

7-10				  WAITBITS, 3-4, 3-24, 5-8

partition table, 7-11		    symbolic names, 3-5

residency table, 7-10		  WAITE, 2-3, 3-9, 3-26

summary, 7-6			  WAITM, 7-6

task flag's table, 7-8		  WAITX, 3-9, 3-26, 3-27

task state table, 7-7

Tasks, 1-3

Index-4

RTS/8

User's Manual

AA-0724D-TA

READER'S COMMENTS

NOTE:  This form is for document comments only. DIGITAL will

use comments submitted on this form at the company's

discretion. If you require a written reply and are

eligible to receive one under Software Performance

Report (SPR) service, submit your comments on an SPR

form.

Did you find this manual understandable, usable, and well-organized?

Please make suggestions for improvement.

----------------------------------------------------------------------

----------------------------------------------------------------------

----------------------------------------------------------------------

Did you find errors in this manual? If so, specify the error and

the page number.

----------------------------------------------------------------------

----------------------------------------------------------------------

----------------------------------------------------------------------

Please indicate the type of reader that you most nearly represent.

o Assembly language programmer

o Higher-level language programmer

o Occasional programmer (experienced)

o User with little programming experience

o Student programmer

o Other (please specify)

Name_______________________________________ Date______________________

Organization__________________________________________________________

Street _______________________________________________________________

City_____________________________ State________ Zip Code _____________

or

Country

-------------------------------Fold Here------------------------------

-----------------Do Not Tear - Fold Here and Staple-------------------

+---------------+

|  FIRST CLASS  |

| PERMlT NO. 33 |

| MAYNARD. MASS.|

+---------------+

BUSINESS REPLY MAIL					   ========

NO POSTAGE STAMP NECESSARY IF MAILED IN THE UNITED STATES  ========

======================================================================

Postage will be paid by:				   ========

========

+-+-+-+-+-+-+-+			   ========

|d|i|g|i|t|a|l|			   ========

+-+-+-+-+-+-+-+			   ========

========

Software Communications                      ========

146 Main Street  ML5-5/E39		   ========

Maynard, Massachusetts  01754		   ========