RTS8V3RN.DOC

RTS/8 Release Notes

DISCLAIMER

This document file was created by scanning  the

original  document and then editing the scanned

text.  As much as possible, the  original  text

format  was restored.  Some format changes were

made to insure this  document  would  print  on

current laser printers using 60 lines per page.

The  original  spelling  and  grammar  has been

preserved.

1-Nov-1996

RTS/8

Release Notes

Order No. AA-5158B-TA

February 1979

ABSTRACT

This document describes the differences between Ver-

sion 2 and Version 3 of RTS/8, restrictions inherent in

the RTS/8 system, additional user notes not contained

in other RTS/8 manuals, and optional patches.

SUPERSESSION INFORMATION:   This document supersedes RTS/8 Release

Notes, Order No. AA-5158A-BA.

VERSION:			   RTS/8 V3 and RTS/8 System Generation

V1.

----------------------------------------------------------------------

| To order additional copies of this document, contact the Software  |

| Distribution Center, Digital Equipment Corporation. Maynard, 	  |

| Massachusetts 01754						  |

----------------------------------------------------------------------

digital equipment corporation  maynard, massachusetts

First Printing, February 1977

Revised:	      February 1979

The information in this document is subject to change without notice

and should not be construed as a commitment by Digital Equipment

Corporation. Digital Equipment Corporation assumes no responsibility

for any errors that may appear in this document.

The software described in this document is furnished under a license

and may only be used or copied in accordance with the terms of such

license.

No responsibility is assumed for the use or reliability of software on

equipment that is not supplied by DIGITAL or its affiliated companies,

Copyright (c) 1977, 1979 by Digital Equipment Corporation

The postage-prepaid READER'S COMMENTS form on the last page of this

document requests the user's critical evaluation to assist us in pre-

paring future documentation.

The following are trademarks of Digital Equipment Corporation:

DIGITAL		  DECsystem-10	      MASSBUS

DEC		  DECtape	      OMNIBUS

PDP		  DIBOL		      OS/8

DECUS		  EDUSYSTEM	      PHA

UNIBUS		  FLIP CHIP	      RSTS

COMPUTER LABS	  FOCAL		      RSX

COMTEX		  INDAC		      TYPESET-8

DDT		  LAB-8		      TYPESET-11

DECCOMM		  DECSYSTEM-20	      TMS-11

ASSIST-11		  RTS-8		      ITPS-10

VAX		  VMS		      SBI

DECnet		  IAS		      PDT

DATATRIEVE	  TRAX

CONTENTS

Page

1.0      OVERVIEW						    1

2.0      RTS/8 VERSION 2 AND VERSION 3 DIFFERENCES		    1

2.1	System Generation Procedures			    1

2.2	New or Substantially Rewritten Tasks		    2

2.2.1	128K Support					    2

2.2.2	Console and Non-console Terminal Handler	    2

2.2.3	Floppy Handler					    2

2.2.4	RL01 Handler					    2

2.2.5	Macros						    2

2.2.6	/X LINK Switch					    3

2.3	Enhancements					    3

2.3.1	Clock Handler					    3

2.3.2	LINCtape Handler				    3

2.3.3	Line Printer Handler				    3

2.3.4	Mass Storage Handlers				    3

2.3.5	RTS/8 Executive					    3

3.0      NOTES						    4

3.1	System Generation Form				    4

3.2	Off-Line Line Printer				    4

3.3	Non-standard Clock				    4

3.4	Parameter Sources				    4

3.5	/Z MACREL Switch				    4

3.6	/M LINK Switch					    5

3.7	/H LINK Optional Switch				    5

3.8	Task Source Programs				    5

4.0      RESTRICTIONS					    5

4.1	RTS/8 Device					    5

4.2	OS/8 SQUISH Command				    5

4.3	128K Support					    5

4.4	OS/8 Background KT8-A Support			    6

4.5	RUBOUT or DELETE				    6

4.6	Terminal Handler				    6

4.7	OS/8 File Support Task				    6

4.8	OS/8 Background Task				    6

4.9	KT8-A						    6

4.10	Line Printer Handler				    7

4.11	CONTROL/S and CONTROL/Q				    7

5.0      OPTIONAL PATCHES					    7

5.1	ESCAPE Transmission Terminator			    7

5.2	No Line Printer in System			    9

5.3	Non-system DSK Device				    9

5.4	Option Board Line Printer Support		    11

iii

1.0  OVERVIEW

This document describes

o the differences between Version 2 and Version 3 of RTS/8

o restrictions inherent in the RTS/8 system

o additional user notes not described in the RTS/8 User's Manual 						       ___________________

or the RTS/8 System Generation Manual 		   ______________________________

o optional patches

If you are a  new  user  of  RTS/8,  the  information  describing  the

differences  between  Version  2  and  Version 3 is not germane;  this

information is presented so that previous  users  of  RTS/8  may  make

Version 2 RTS/8 systems compatible with Version 3.

2.0  RTS/8 VERSION 2 AND VERSION 3 DIFFERENCES

This section describes functional differences between version 2 and  3

of  RTS/8.   RTS/8  V2  users,  after reading this section, can direct

their attention to those sections of  the  RTS/8  User's  Manual  that 						_____________________

discuss these differences.

The following categories of differences exist.

o the System Generation procedures

o new or substantially rewritten DIGITAL-supplied tasks

o enhancements to previously existing DIGITAL-supplied tasks

2.1  System Generation Procedures

The System Generation (or SYSGEN) is  a  new  procedure  for  creating

RTS/8  systems.   By  asking questions and recording your answers, the

SYSGEN automatically creates a parameter file  (which  you  previously

had  to  create  manually).   The SYSGEN also creates two batch files:

the first assembles the source modules that comprise the RTS/8  system

and  the  second  links the modules into one unit.  You may chain from

the SYSGEN to BATCH automatically and thus have BATCH create the RTS/8

system.  You may also chain from BATCH to the RTS/8 system.

You must now create RTS/8 systems using  MACREL/LINK  V2  rather  than

PAL8 and ABSLDR.  The manual linking and loading of an RTS/8 system to

create partitions for nonresident tasks is automatically performed  by

LINK.

As real-time systems are often tailored to specific applications,  you

can  also  modify the SYSGEN procedure so that it more precisely meets

your needs.

1

System Generation procedures are fully described in the  RTS/8  System 							      _____________

Generation Manual.      _________________

2.2  New or Substantially Rewritten Tasks

2.2.1  128K Support - RTS/8 now supports 128K  systems  that  use  the

KT8-A.

2.2.2  Console  and  Non-console  Terminal  Handler   -   Four   major

alterations were made in the terminal source module (TTY. MA).

1.  RTS/8 now  allows  you  to  use  the  same  terminal  as  the

foreground  console  terminal  (for example, the terminal for

MCR) and as the background OS/8 terminal.  (See Section  4.12

and 4.5 of the User's Manual. )

2.  You may interrupt a task that controls a terminal by  sending

a  message  to  the  Emergency Breakthrough Task (EMT).  This

task is, in actuality, a  segment  of  the  terminal  handler

code.  (See Section 4.5.1 of the User's Manual. )

3.  The output buffer can be in a different field  than  that  of

the message.

4.  You may optionally allow an unanswered input  request  to  be

aborted by other terminal requests on a timeout.

2.2.3  Floppy Handler - Substantial revisions have been made  so  that

RTS/8  can  use  the  same  handler for both single and double density

floppies.  The changes in this handler are transparent to users.

The handler also accepts messages in RL01 format.

2.2.4  RL01 Handler - RTS/8 now supports RL01 disks.  Because the RL01

has a greater storage capacity than other storage media used by RTS/8,

it has an additional mass storage call format with  a  two-word  block

number.   (All  mass  storage  handlers  were revised so that they may

optionally use the additional RL01 format.  This allows you to  freely

change media. )

2.2.5  Macros - You must include the RTS/8 TASK macro  in  all  tasks.

Nonresident  tasks  must  include the NONRES macro in one SECT.  These

macros are part of the parameter file and are  automatically  part  of

all RTS/8 systems.

2

2.2.6  /X LINK Switch - The /X option has been added to LINK  so  that

special  RTS/8  requirements  may  be  met.   The SYSGEN automatically

includes this switch when it creates the LINK Batch file.

2.3  Enhancements

2.3.1  Clock Handler - The following are changes in the clock handler:

1.  You may now specify a CLASS word in an RTS/8 message.   CLASS

is  an  optional  specification  that allows the clock CANCEL

command to remove groups of requests:  you may  add  a  CLASS

designator  to  all clock requests other than CANCEL requests

to label those requests for possible future cancellation.

2.  The  mnemonic  PERIODICALLY  SCHEDULE  has  been  changed  to

PERIODICALLY.   In  Version  2,  the PAL8 assembler logically

ORed the two words together and placed the  value  3000  into

the symbol table.  PERIODICALLY now has a value of 3000

3.  An interrupt linkage has been  added  that  allows  RTS/8  to

respond immediately to clock ticks.

2.3.2  LINCtape Handler - The LINCtape Handler now accepts messages in

RL01 format.

2.3.3  Line Printer Handler - The Line Printer Handler now permits the

output buffer to be in a field different from that of the message.

2.3.4  Mass Storage Handlers - DF32, RF08, RK8, RK8E, and TC08 DECtape

Handlers now accept messages in RL01 Format.

2.3.5  RTS/8 Executive - The Executive can reside  in  any  field  the

user  chooses.   Note that the automatic starting of RTS/8 is disabled

if RTS/8 does not reside in field 0.

3

3.0  NOTES

3.1  System Generation Form

The checklist SYSGEN form contained in the SYSGEN manual exists  as  a

source in the distribution kit.  It is called FORM. TX.

3.2  Off-Line Line Printer

The SYSGEN program allows  you  to  list  tasks  by  priority  and  by

partition.   These  listings normally go to the line printer.  If your

system does not have a line printer, SYSGEN diverts  the  listings  to

your  terminal.  If your system has a line printer and it is off-line,

OS/8 hangs up in a loop until the line printer is on-line.

3.3   Non-standard Clock

When you specify a clock that is not standard for a target machine, no

direct way exists to change the hardware tick rate.  Consequently, you

should use a text editor to change the equate  HERTZ=  in  the  output

parameter  file  before  assembling  the  RTS/8  system.  (If you must

modify HERTZ, do not chain to BATCH from SYSGEN. )

3.4  Parameter Sources

The parameter file that SYSGEN produces exists in source  form  within

the file SCRIPT. MA.

3.5  /Z MACREL Switch

The MACREL batch file that SYSGEN produces has /Z switches appended to

each  .MAC assembly command.  The /Z tells MACREL to move BATCH out of

memory while it is running.  This is necessary to allow assembly in  a

16K  OS/8  system.  If you do not want BATCH to be swapped from memory

while a particular system is being created, you may remove the  /Z  by

using  a  text  editor  on  the assembly batch file.  If MACREL should

never swap out BATCH, then you may  remove  the  /Z  switch  from  the

SCRIPT.MA  source.   If you remove the switch from SCRIPT.MA, you must

re-assemble and re-link SCRIPT.MA.

The /Z always diverts the MACREL error status print-out to your

terminal.

4

3.6  /M LINK Switch

The LINK Batch File that SYSGEN produces contains a /M=3 switch.  This

switch  prevents  LINK  from using the top page of fields 0 through 2:

however, LINK may still use the top pages of all other fields.

3.7  /H LINK Optional Switch

The LINK has a /H switch that  fills  unused  locations  with  a  halt

instruction.    This   makes  debugging  somewhat  easier  because  an

undebugged  program  tends  to  stop  executing  sooner.    (This   is

especially true for nonresident systems.)  You may enter the /H switch

into SCRIPT.MA  using  a  text  editor.   You  append  the  /H  switch

to the LINK batch file skeleton.

3.8  Task Source Programs

The SYSGEN procedure assumes that source  programs  contain  a  single

task.   If one source contains two tasks (each containing its own TASK

macro call), you must type both task names when the SYSGEN asks you to

enter  user  tasks.   You  must also create a dummy file of the second

name.  If you are using a task-less source, you must add the  task  to

the MACREL and LINK batch files with a text editor.

4.0  RESTRICTIONS

4.1  RTS/8 Device

If the RTS/8 system contains nonresident tasks or  possesses  an  OS/8

background, you must execute the system from SYS.

4.2  OS/8 SQUISH Command

Do not use the OS/8 SQUISH command in the background of RTS/8.  If you

do, the results are unpredictable.

4.3  128K Support

RTS/8 does not support 128K RK08, LINCtape, or UDC/ICS operation.

5

4.4  OS/8 Background KT8-A Support

An OS/8 background under KT8-A with 3 cycle devices may  not  cross  a

physical 32K boundary.

4.5  RUBOUT or DELETE

RUB-OUTs to the terminal do not echo correctly for  carriage  returns,

line feeds, and tabs.  However, the input buffer is correct.

4.6  Terminal Handler

When the terminal handler input buffer is full, characters continue to

echo.

4.7  OS/8 File Support Task

A lookup operation sent to the OS/8 File Support Task returns the file

size as a negative rather than positive number.

The OS/8 File  Support  Task  may  not  function  correctly  when  the

directory is almost out of room for additional files.

4.8  OS/8 Background Task

The OS/8 Background Task  does  not  print  the  accumulator  when  an

illegal IOT is encountered.

The OS/8 Background Task prints an absolute  address  for  KT8-A,  and

virtual address for KM8-E when an illegal IOT is encountered.

4.9  KT8-A

The KT8-A has a bug  that  you  may  encounter  if  you run  the  OS/8

background  with  the  KT8-A  disabled.   SYSGEN  cannot  run  in this

environment.  However, the rest of OS/8  can  with  the  exception  of

FOTP.   Modify FOTP by swapping the two instructions in FOTP which are

found as:

14703/ 1355

14704/ 6214

6

The KT8-A malfunctions  when  User  Mode  is  enabled,  the  KT8-A  is

disabled,  the accumulator has been loaded on the previous instruction

with a value 62x1-62x3, and an RDF is executed.

4.10  Line Printer Handler

The line printer handler contains an error handling problem.  When  an

error  is encountered, the line printer interrupts are disabled.  When

the line printer becomes ready again, its flag is  raised  but  cannot

interrupt.   In  a  quiescent  system, it may be necessary to cause an

interrupt later  in  the  skip  chain  to  wake-up  the  line-printer.

Typically this can be done by striking any key on the terminal.

4.11  CONTROL/S and CONTROL/Q

RTS/8 does not handle CONTROL/S and CONTROL/Q for either a  shared  or

non-shared OS/8 background.

5.0  OPTIONAL PATCHES

Different  users  of  the  system  will  have  different  needs.   The

following  patches  allow  the system to be tailored to change the way

RTS/8 normally operates.

The following patches are source patches.  Consequently, to alter  the

task,  you must alter the sources.  You reference the location to make

the changes by using a  text  editor  search  command  to  locate  the

appropriate  label.   The lines where changes occur are indicated with

asterisks.

5.1  ESCAPE Transmission Terminator

The  Terminal  Handler  treats  an   ESCAPE   (or   ALTMODE)   as   an

end-of-transmission  character  for  line-mode  input.  If you wish to

enter keypad data from a VT-52 or  VT-100,  you  must  disable  ESCAPE

recognition.  In this case, carriage return becomes the only line-mode

terminator.  The following patch disables ESCAPE recognition:

7

TAD I   REPLY    /GET IT AGAIN FOR ECHO

ISZ     REPLY

TAD     L7600    /SUBTRACT OFF 200,TTOUTR WANTS ONLY 7 BITS

JMS     TTOUTR   /RETURN AT INTERRUPT LEVEL !!

IJOIN,   TAD     HOLDER   /AN INTERRUPT HAPPENED WHILE WE WERE HERE

SNA	       /SKIP IF IT DID

JMP     NOMORE   /NO MORE CHAR'S

DCA     INCH     /HOLD EXTRA CHARACTER

DCA     HOLDER   /CLEAR FOR NEXT ONE

JMS     RELES    /KICK BACK TO MAINSTREAM

/

/  **    LOOK AT THE CHARACTER **

/

BYPASS,  TAD     INCH     /WHAT WAS THE CHAR

IFZERO  STRIP    <

SNA

JMP     EOIN     /SIGNAL END OF CHARACTER TYPE INPUT

>

TAD     (-15

SNA

JMP     CARRET

TAD     (-25+15  /CHECKING FOR CONTROL U

SNA

JMP     XCNTLU

TAD     (+25-33  /START CHECKING FOR VARIOUS ESCAPES

/	      SZA	       /ANY ESCAPE COLLECTS IN SZA CHAIN	 **

TAD     (+33-176

LITSZA,  SMA SZA	       /LITERAL TO ALWAYS SKIP ON ZERO AC

JMP     RUBOUT

/	      SZA							 **

IAC

/	      SNA							 **

/	      JMP     ALTMOD						 **

ISZ     INPCNT

JMP     RPLYLP

STA

RPLY1,   DCA     INPCNT

NOECH,   IOF

JMP     IJOIN

/

/  **    ESCAPE (ALTMODE) AND CARRIAGE RETURN TERMINATE INPUT  **

/

ALTMOD,  TAD     (44+HB   /ECHO A DOLLAR SIGN

JMS     OUT

STA	       /-1 TO BUFFER

CARRET,  JMS     SETBUF   /POINT TO BUFFER FIELD

DCA I   REPLY    /PLACE -1 FOR ESCAPE AND 0 FOR <CR>,

/AND 400 FOR ABORTED

JMP     EOIN     /END OF INPUT

8

5.2  No Line Printer in System

The OS/8 background  contains  a  small  module to  operate  the  line

printer  when  RTS/8  does  not have a line printer handler.  When the

OS/8 background operates BATCH on a system that has  no  line  printer

hardware,  BATCH hangs up.  The following patch allows BATCH to run on

a system with no line printer.

IFNDEF  LPT      <

XLLS,    /ISZ    LPFST						 **

/SKP							 **

/JMP    . +3						 **

/PSKF							 **

/JMP    . -1     /OH, HOW CRUDE!				 **

TAD     AC

PSLS

/DCA    LPFST    /CLEAR FIRST-TIME FLAG			 **

JMP I   (XNOP

LPFST,   /-1

XLSF,    PSKF	       /IF THE FLAG IS NOT UP,

JMP I   (XNOP    /BE A NOP

JMP I   (XSKP    /OTHERWISE BE A SKIP

>

5.3  Non-system DSK Device

The OS/8 Background Task assumes that the OS/8 name 'DSK' is the  same

as  'SYS'.  If you wish to name some other device 'DSK', the following

table gives the correct value of the unit number for 'DSK'.  Make  the

following patch:

/ **     DEVICE CORRESPONDENCE TABLE

/

HTBL,    DEVICE  SYS

OSSYD40

DEVICE  DSK

OSSYSD40

/REPLACE THIS WITH SIMILAR LINE FOR

/YOUR DEVICE, E.G., IF RKA1 USE LINE WITH

/COMMENTED STARS

IFDEF   DF32	<

DEVICE  RF

DF3240

>

IFDEF   RF08	<

DEVICE  RF

RF0840

>

9

IFDEF   DTA	<

DEVICE  DTA0

DTA40+0

DEVICE  DTA1

DTA40+1

DEVICE  DTA2

DTA40+2

DEVICE  DTA3

DTA40+3

DEVICE  DTA4

DTA40+4

DEVICE  DTA5

DTA40+5

DEVICE  DTA6

DTA40+6

DEVICE  DTA7

DTA40+7

>

IFDEF   RK8	<

DEVICE  RKA0

RK840+0

DEVICE  RKB0

RK840+4

DEVICE  RKA1

RK840+1

DEVICE  RKB1

RK840+5

DEVICE  RKA2

RK840+2

DEVICE  RKB2

RK840+6

DEVICE  RKA3

RK840+3

DEVICE  RKB3

RK840+7

>

IFDEF   RK8E	<

DEVICE  RKA0

RK8E40+0

DEVICE  RKB0

RK8E40+4

DEVICE  RKA1

RK8E40+1		/****

DEVICE  RKB1

RK8E40+5

DEVICE  RKA2

RK8E40+2

DEVICE  RKB2

RK8E40+6

DEVICE  RKA3

RK8E40+3

DEVICE  RKB3

RK8E40+7

>

10

5.4  Option Board Line Printer Support

RTS/8 has no support for the PDP 8/A line  printer  attached  with  an

options  board.  It is, however, possible to patch the OS/8 Background

Task so that it can access this printer.  When  OS/8  is  run  in  the

foreground,  assign  LP8A  to be the printer:  when OS/8 is run in the

background, assign the regular  line  printer.   In  the  OS/8  source

replace:

IFNDEF  LPT      <

XLLS,    ISZ     LPFST

SKP

JMP     .+3

PSKF	       /REPLACE WITH 6570			 **

JMP     .-1      /OH, HOW CRUDE!

TAD     AC

PSLS	       /REPLACE WITH CMA;6574			 **

DCA     LPFST    /CLEAR FIRST-TIME FLAG

JMP I   (XNOP

LPFST,   -1

XLSF,    PSKF	       /REPLACE WITH 6570			 **

/IF THE FLAG IS NOT UP,

JMP I   (XNOP    /BE A NOP

JMP I   (XSKP    /OTHERWISE BE A SKIP

11

Release Notes

AA-5158B-TA

READER'S COMMENTS

NOTE:  This form is for document comments only.  DIGITAL will

use comments submitted on this form at the company's

discretion.  If you require a written reply and are

eligible to receive one under Software Performance

Report (SPR) service, submit your comments on an SPR

form.

Did you find this manual understandable, usable, and well-organized?

Please make suggestions for improvement.

______________________________________________________________________

______________________________________________________________________

______________________________________________________________________

______________________________________________________________________

______________________________________________________________________

______________________________________________________________________

______________________________________________________________________

______________________________________________________________________

______________________________________________________________________

Did you find errors in this manual?  If so, specify the error and the

page number.

______________________________________________________________________

______________________________________________________________________

______________________________________________________________________

______________________________________________________________________

______________________________________________________________________

______________________________________________________________________

______________________________________________________________________

______________________________________________________________________

Please indicate the type of user/reader that you most nearly represent.

[]  Assembly language programmer

[]  Higher-level language programmer

[]  Occasional programmer (experienced)

[]  User with little programming experience

[]  Student programmer

[]  Other (please specify) _______________________________________

Name______________________________________Date________________________

Organization__________________________________________________________

Street________________________________________________________________

City__________________________State_____________Zip Code______________

or

Country

-------------------------------Fold Here------------------------------

-----------------Do Not Tear - Fold Here and Staple-------------------

+---------------+

|  FIRST CLASS  |

| PERMlT NO. 33 |

| MAYNARD. MASS.|

+---------------+

BUSINESS REPLY MAIL					   ========

NO POSTAGE STAMP NECESSARY IF MAILED IN THE UNITED STATES  ========

======================================================================

Postage will be paid by:				   ========

========

+-+-+-+-+-+-+-+			   ========

|d|i|g|i|t|a|l|			   ========

+-+-+-+-+-+-+-+			   ========

========

Software Communications			   ========

146 Main Street  ML5-5/E39		   ========

Maynard, Massachusetts  01754		   ========