SYSGEN.DOC

DISCLAIMER

This document file was created by scanning  the

original  document and then editing the scanned

text.  As much as possible, the  original  text

format  was restored.  Some format changes were

made to insure this  document  would  print  on

current laser printers using 60 lines per page.

The  original  spelling  and  grammar  has been

preserved.

1-Sep-1996

RTS/8

System Generation Manual

Order No. AA-D943A-TA

February 1979

ABSTRACT

This manual describes the procedure by which a user generates an RTS/8

realtime operating system. After answering System Generation

questions, the program can automatically build, using OS/8 BATCH and

MACREL/LINK, an RTS/8 system with little or no user intervention.

The System Generation procedure is a flexible one that presents the

user with the minimum number of questions needed to build a system. As

user systems may vary significantly, the RTS/8 System Generation

permits modifications in its interactive dialogue so that the System

Generation is unique to a user's needs.

SUPERSESSION INFORMATION: This is a new manual.

VERSION: V01

------------------------------------------------------------

| To order additional copies of this document, contact the |

|      Software Distribution Center, Digital Equipment     |

|	  Corporation, Maynard, Massachusetts 01754	    |

------------------------------------------------------------

digital equipment corporation - maynard. massachusetts

First Printing, February 1979

The information in this document is subject to change without notice

(and should not be construed as a commitment by Digital Equipment

Corporation.  Digital Equipment Corporation assumes no responsibility

for any errors that may appear in this document.

The software described in this document is furnished under a license

and may only be used or copied in accordance with the terms of such

license.

No responsibility is assumed for the use or reliability of software on

equipment that is not supplied by DIGITAL or its affiliated companies.

Copyright  (c)  1979 by Digital Equipment Corporation

The postage-prepaid READER'S COMMENTS form on the last page of this

document requests the user's critical evaluation to assist us in

preparing future documentation.

The following are trademarks of Digital Equipment Corporation:

DIGITAL		  DECsystem-10	      MASSBUS

DEC		  DECtape	      OMNIBUS

PDP		  DIBOL		      OS/8

DECUS		  EDUSYSTEM	      PHA

UNIBUS		  FLIP CHIP	      RSTS

COMPUTER LABS	  FOCAL		      RSX

COMTEX		  INDAC		      TYPESET-8

DDT		  LAB-8		      TYPESET-11

DECCOMM		  DECSYSTEM-20	      TMS-11

ASSIST-11		  RTS-8		      ITPS-10

VAX		  VMS		      SBI

DECnet		  IAS		      PDT

DATATRIEVE	  TRAX

CONTENTS

Page

PREFACE							   v

CHAPTER  1	INTRODUCTION					   1-1

CHAPTER  2	THE SYSGEN DIALOGUE				   2-1

2.1   OVERVIEW					   2-1

2.2   THE DIALOGUE				   2-2

2.3   PASS 2 AND BATCH				   2-46

2.4   RTS/8 SYSGEN DOCUMENT NUMBERS		   2-51

CHAPTER  3 TWO EXAMPLE SYSGENS				   3-1

3.1   OVERVIEW					   3-1

3.2   A DEFAULT SYSGEN				   3-1

3.3   A NORMAL SYSGEN				   3-4

CHAPTER  4 MODIFYING THE SYSGEN				   4-1

4.1   OVERVIEW					   4-1

4.2   THE STRUCTURE OF SCRIPT.MA			   4-1

4.2.1   Record Directory				   4-2

4.2.2   Script Records				   4-3

4.2.3   Parameter Records				   4-10

4.2.4   Skeleton Records				   4-12

4.2.5   Error Message Records			   4-12

4.2.6   LINK and MACREL BATCH Records		   4-12

4.2.7   Limit Records				   4-12

4.3   EXAMPLES					   4-13

4.3.1   Deleting Questions			   4-13

4.3.2   Adding Questions				   4-17

4.3.3   Changing the Default			   4-31

4.3.4   Multiple TTY Systems			   4-34

APPENDIX A	    RTS/8 SYSTEM GENERATION SOURCE FILES	   A-1

APPENDIX B     DEC/X8 SPECIFICATION SHEET			   B-1

APPENDIX C     SYSGEN CHECK LIST				   C-1

APPENDIX D	    RTS/8 PARAMETER LISTING			   D-1

APPENDIX E	    ERROR MESSAGES				   E-1

INDEX						       Index-1

iii

PREFACE

This manual describes the RTS/8 system generation  procedure.   System

generation,  or  SYSGEN,  is  an  interactive  procedure  you use when

building RTS/8 operating systems from source  modules.   These  source

modules are tasks that DIGITAL supplies and tasks that you create.  In

addition,  the  design  of  the RTS/8 SYSGEN script allows you to make

changes in the interactive  dialogue  so  that  the  SYSGEN  procedure

itself more precisely meets your needs.

The manner in which you program an RTS/8 operating system after it  is

running  is  described  in  the  RTS/8 User's Manual, V3.  This SYSGEN

manual assumes that you are familiar with the  DIGITAL-supplied  tasks

as  they  are  described in the User's Manual.  For example, the RTS/8

Monitor Console Routine is not described.  You are  expected  to  have

read the sections of the User's Manual which describe it.

The RTS/8 SYSGEN, among other functions,  allows  you  to  incorporate

various  device handlers into your RTS/8 system.  However, this manual

does not attempt to describe the devices themselves.  It  assumes  you

know  which  devices  are  in  your  system  and are familiar with the

information contained in the hardware manual that  you  received  when

you purchased the device.  For example, no attempt is made to describe

an RLO1 disk.

The first three chapters of this  manual  introduce  and  explain  the

SYSGEN.   Chapter 1 is an overview of the SYSGEN process.  The portion

of the SYSGEN that is most  visible  to  a  user  is  the  interactive

dialogue.   Chapter 2 discusses this dialogue.  Any SYSGEN you perform

uses portions of this dialogue.  As Chapter 2 presents the dialogue in

its entirety, you may find it difficult to see the continuity and flow

inherent in the information the SYSGEN prints.  Consequently,  Chapter

3 is a listing of two example SYSGENs.

The advanced user may wish to  change  the  SYSGEN  so  that  it  more

precisely  conforms  to  the kinds of systems that will be built.  The

final chapter details how you may tailor the SYSGEN procedure to  your

needs.

v

CHAPTER 1

INTRODUCTION

RTS/8 SYSGEN is the procedure you use to build a  real-time  operating

system.   Unless you decide to reconfigure the SYSGEN (see Chapter 4),

most of the operations that the SYSGEN performs are invisible to  you;

that  is, the operations the SYSGEN performs are not seen and, in most

cases, are not important to what you are doing.

The minimum memory size that you will need to run the SYSGEN  is  12K.

Also,  the  SYSGEN  program,  SYSGEN.SV,  must  reside  on SYS and the

dialogue must reside on a mass storage device.  If you chain to BATCH,

you  will need a 16K system.  In addition, you conceivably may require

more memory than 16K if you are creating a large RTS/8 system.

The SYSGEN program produces three batch files that OS/8 BATCH uses  to

create  the  RTS/8  system.  The SYSGEN may also create TTY and floppy

disk control files.  These files contain information that is  specific

to  a  particular  TTY  or floppy device.  When MACREL creates the .RB

(Relocatable Binary) file, it uses the equates in these control  files

to create a task that is specific to your configuration.

The SYSGEN is an interactive dialogue;  that is, it asks you questions

and  you  supply  answers.  After you answer the questions, the SYSGEN

builds a real-time operating system that consists of an Executive (the

routines  that  control  task execution) and other tasks.  These other

tasks are either DIGITAL-supplied tasks or those that you create.

You may also build an  RTS/8  system  that  consists  of  a  real-time

operating system foreground and an OS/8 background.  In such a system,

the real-time tasks have primary control of the computer.  When  these

tasks  do  not  need to use the computer, OS/8 can run;  that is, OS/8

runs in the "background" of the RTS/8 operating system.

You begin executing the SYSGEN, called SYSGEN.SV on  its  distribution

medium, by typing

R SYSGEN

in response to the OS/8 prompt.  (The SYSGEN runs only under OS/8  and

SYSGEN.SV must be on SYS.) If SYSGEN.SV is not on SYS, you must use an

OS/8 COPY command to bring it to SYS.  The program then asks you which

device  contains  the  interactive  dialogue.   The  dialogue,  called

SCRIPT.SV on the distribution medium, can either reside on SYS  or  on

any  other  file-structured  device  that  OS/8  recognizes.   If  the

dialogue is on SYS, you type a carriage return.  Otherwise,  type  the

OS/8 device name.

1-1

INTRODUCTION

After you respond to this question, the SYSGEN  dialogue  begins.   In

general,  each  question is composed of three parts.  The first is the

question itself.  The second is a string within  square  brackets  [].

This string indicates the type of response you must type;  for example

[D] means decimal, [Y/N] means YES/NO and so on.  The  third  portion,

within parentheses, indicates the default answer.  For example:

DO YOU WANT ASSEMBLY LISTINGS [Y/N] (N):

In this question, N (meaning no) is the default answer that you select

by typing a carriage return.

Many of the questions pertain to DIGITAL supplied tasks.  For some  of

the  DIGITAL  tasks, you need only state that you wish to include that

task in the RTS/8 system.  Other DIGITAL tasks may require  more  than

one  response.   For example, if you include the RTS/8 Monitor Console

Routine, the SYSGEN eventually asks if you wish to include the  SYSTAT

command.

The first series of SYSGEN questions ask  you  to  supply  information

about  the  computer  on which you will run RTS/8.  A second series of

questions ask where the SYSGEN should  store  output  files,  what  it

should  do  with the output files, and other similar information.  For

example, you may  create  an  RTS/8  system  that  provides  you  with

assembly listings, linker load maps, and comes up running.

The SYSGEN then asks which DIGITAL tasks you wish to  include  in  the

system.   It  then  asks  questions about these tasks (if applicable).

The last questions of the dialogue ask if you  wish  to  include  user

tasks  and in which partitions you wish them to reside (if the task is

nonresident), if you wish  to  change  task  priorities,  and  if  the

partition allocation is correct.

During any one session, the SYSGEN does not print all the questions it

is capable of asking because questions may be mutually exclusive.  For

example, if you state that the computer upon which RTS/8 will  execute

does  not include EAE hardware, the SYSGEN does not ask if you wish to

save the contents of the EAE registers. Consequently, you see only the

questions that are necessary to build the system.

While typing answers, you may edit your response by using  the  delete

key  or  erase the entire answer with a CTRL/U.  If, however, you type

an incorrect response, the SYSGEN prints an  error  message  and  then

reprints  the  question.  A list of error messages appears in Appendix

E.

In general, you must provide four types of information to the SYSGEN.

o  You must tell the SYSGEN what actions it is to perform when it

completes  the  interactive  dialogue.   These actions include

writing output files, creating listings, and chaining  to  the

RTS/8 system.

1-2

INTRODUCTION

o  You must provide  information  that  tells  the  SYSGEN  which

machine  you are creating an RTS/8 system for and the kinds of

hardware (not peripherals) that exist in the configuration.

o  You must specify which of the DIGITAL tasks you  will  include

in  the  system.   The  DIGITAL  tasks, for the most part, are

handlers for peripheral devices.  Other DIGITAL tasks are  the

Monitor  Console Routine and the Power Fail Task.  For some of

the  DIGITAL  tasks,  you  must  provide   specific   hardware

information such as the peripheral's device code.

o  You can place tasks that you write into the RTS/8 system.  You

may  also  make  any user task nonresident and place it into a

partition.

When you begin a SYSGEN, you should have all the information you  will

need  with  you  at the terminal.  Appendix C is a checklist of SYSGEN

questions.  Using a copy of this form, you can fill in your  responses

before you sit down at the terminal.  In addition, this form documents

the systems that you create.

The RTS/8 SYSGEN also allows you to create a unique  SYSGEN  procedure

as  well  as  a  unique  operating  system.  By customizing the SYSGEN

procedure, you create an interactive dialogue  and  presentation  that

best  fits  your needs.  For example, the SYSGEN contains questions on

RF02 and DF32 fixed head disks and LINCtapes.  If your system does not

contain these devices, you may make a simple change in SCRIPT.MA  (the

source  program  for  SCRIPT.SV) using any text editor.  These changes

prevent the SYSGEN from asking these questions.

The following two chapters  present  the  SYSGEN  dialogue.   For  the

novice  user  of  the SYSGEN, these are the only chapters that need be

read as they discuss the SYSGEN that DIGITAL distributes.

The user who wishes to  make  modifications  in  the  SYSGEN  dialogue

should  read  Chapter 4, which discusses how you can delete questions,

add questions, and change the default system.

1-3

CHAPTER 2

THE SYSGEN DIALOGUE

2.1  OVERVIEW

This chapter  describes  RTS/8  SYSGEN  questions,  expected  answers,

SYSGEN  remarks, and SYSGEN run paths.  Because the SYSGEN chooses the

questions it prints (basing its choices on the  answers  you  give  to

other  questions), some of the questions in this chapter may appear to

be out of order.  In addition, other questions were purposely  printed

here  in  a  different order so that you may more fully understand the

material that the SYSGEN offers.

Chapter 3 is closely related to  this  chapter  in  that  it  consists

entirely  of  two  example  dialogues.   (These  examples  present the

dialogue in the order they would appear on your terminal.)

The following section contains four types of information.

o  The first is the question  that  the  SYSGEN  prints.   SYSGEN

questions  appear  in  capital  letters  and begin at the left

margin.

o  The second is the remark, also  printed  in  capital  letters,

that  the  SYSGEN  prints  if  you ask it to.  The remarks are

indented from the left margin.  (The SYSGEN  does  not  indent

these remarks when it prints them.)

o  The third type of  information  is  remarks  that  the  SYSGEN

always  prints.   These  remarks appear in capital letters and

also begin at the left margin.  To distinguish  these  remarks

from  questions,  each  line  of the remark is followed by the

word "remark" in this chapter.

o  The fourth kind of information is a further explanation of the

question.   This  explanation (in upper and lower case) is not

part of the SYSGEN.

Some of the upper and lower case explanations state that  if  you  are

unsure  if  something exists in your system or if you do not have some

information, such as a device code, you should check with your  system

manager  or  your  sales  representative.  You can also check the form

that DIGITAL provided when you  purchased  your  system.   This  form,

reproduced  as  Appendix  B,  lists the device codes.  In this sample,

note that the devices you purchased are listed in  column  1  and  the

device codes in column 2.

2-1

THE SYSGEN DIALOGUE

2.2  THE DIALOGUE

Before running the SYSGEN, you should type an OS/8

SET TTY NO PAUSE

command because page pauses are  unnecessary  (and  distracting)  when

running the SYSGEN program.

The machine you use to create the RTS/8 system  must  contain  12K  of

memory.   If  you will chain to BATCH, as will be discussed below, you

must  have  sufficient  memory  to  assemble  and  link   the   system

(ordinarily, 16K).

You begin the SYSGEN by running the SYSGEN.SV program.   This  program

runs only under OS/8.  If SYSGEN is not on the system device, you must

copy it to SYS.  After the OS/8 monitor prompt, type

R SYSGEN

After the SYSGEN is running, it prints the question:

ON WHAT DEVICE IS SCRIPT.SV LOCATED ? :

SCRIPT.SV is the file that  contains  the  interactive  dialogue.   If

SCRIPT.SV is on SYS, type a carriage return.  Otherwise, type the OS/8

name of the device upon which the file SCRIPT.SV resides.  The  SYSGEN

program then begins printing the interactive dialogue.

RTS/8 V03 SYSTEM BUILDER PROGRAM V01.			     remark

remark

THIS PROGRAM IS A CONVERSATIONAL				     remark

QUESTION AND ANSWER SYSTEM FOR				     remark

BUILDING RTS/8 REAL TIME SYSTEMS.				     remark

remark

THE QUESTIONS AND EXPLANATIONS IN THIS			     remark

DIALOGUE ARE BRIEF. CHAPTER 2 IN THE			     remark

SYSTEM GENERATION MANUAL INCLUDES AD-			     remark

DITIONAL INFORMATION.					     remark

DO YOU WANT PROGRAM OPERATION

INFORMATION? (YOU MAY TYPE Y FOR YES OR N

FOR NO FOLLOWED BY A CARRIAGE RETURN):

If you answer "Y" to this question, you receive an explanation of  how

you  interact with the SYSGEN dialogue.  If you are using this program

2-2

THE SYSGEN DIALOGUE

for the first time, you  should  read  these  instructions  carefully.

(The following paragraphs also explain how to use the SYSGEN program.)

The standard SYSGEN question has the following format

NUMBER    QUESTION    ANSWER-TYPE    DEFAULT

Each of these elements is discussed in the following  paragraphs.   An

example of a question that the SYSGEN prints is

003 DO YOU WISH TO BUILD AN RTS/8 SYSTEM WITHOUT

SPECIFYING PARAMETERS? [Y/N] (N):

The SYSGEN discussion begins with a description  of  the  answer-types

that SYSGEN expects to a question.

YES OR NO ANSWERS ARE INDICATED

BY A [Y/N] NOTATION. TYPE A 'Y' FOR YES

OR AN 'N' FOR NO, FOLLOWED BY A CARRIAGE

RETURN, TO ANSWER THE QUESTION.

Answer a YES/NO question with a Y or N or with a YES or NO.  Note that

the  SYSGEN does not record your response to a question until you type

a carriage return.

QUESTIONS REQUIRING A NUMERICAL RESPONSE

ARE INDICATED BY A [D] FOR DECIMAL OR AN [O]

FOR OCTAL.  ENTER THE NUMBER FOLLOWED BY

A CARRIAGE RETURN.  (ALWAYS INTEGER)

If your response to a question is a number, type the number  and  then

type  a  carriage  return.  The SYSGEN checks to ensure that numerical

responses do not contain non-numeric characters.  SYSGEN always checks

to  determine  if numerical answers are within acceptable limits.  For

example, if your answer to a question was 100 (octal) and the  largest

answer SYSGEN allows for the question is 77 (octal), the SYSGEN prints

an error message.  Also, if your answer is  greater  than  six  digits

long, SYSGEN prints an error message.  (Some teletype questions accept

only five character or digit answers.) If the expected response is  an

octal  number,  SYSGEN  rejects any number that contains an 8 or 9 and

prints an error message.

Chapter 4 discusses how SYSGEN evaluates numerical answers and how you

may change the limits.

2-3

THE SYSGEN DIALOGUE

ARE YOU READY TO CONTINUE? [Y/N] (Y):

When you finish reading the above instructions, type a response to the

ARE YOU READY question.  You may type a Y or, because Y is the default

answer, a carriage return.  (See below for an explanation  of  default

answers.)  If  you  type  an  N, the SYSGEN repeats the question.  Any

other response produces an error message.

This question exists  so  that  the  SYSGEN  does  not  print  further

instructions  until  you  tell  it  that  you  are ready.  (If you are

displaying the information on a hard copy terminal, you  may  wish  to

remove this question.  See Chapter 4.)

After you answer this question, SYSGEN types additional information.

QUESTIONS REQUIRING AN ALPHANUMERIC

ANSWER ARE INDICATED BY [A].  RESPOND

BY TYPING THE ALPHANUMERIC DATA FOLLOWED

BY A CARRIAGE RETURN.

The error-checking that the SYSGEN performs  on  alphanumeric  answers

determines  if you type too many characters and if the answer contains

characters other than the upper case  letters  A  through  Z  and  the

numbers  0  through 9.  Because many alphanumeric responses are device

specifications, you must be especially careful that  you  enter  these

responses correctly.  If you do not, OS/8 BATCH or the generated RTS/8

system cannot execute.

IF YOU WOULD LIKE AN EXPLANATION

OF A QUESTION, TYPE A ? FOLLOWED BY A CARRIAGE

RETURN.  THE PROGRAM THEN PRINTS AN EXPLANA

TION AND ALSO REPRINTS THE QUESTION.

Most questions have explanations.  The purpose of  these  explanations

is  to  jog  your  memory about what the question is asking.  For more

information, you should read this  manual,  the  appropriate  hardware

manual, or the RTS/8 User's Manual.		    ___________________

ON THE LINE FOLLOWING MOST REMARKS IS A

DOCUMENTATION NUMBER OF THE FORM

(DOC. NO NNN). YOU USE THIS NUMBER TO LOCATE

ADDITIONAL INFORMATION IN THE SYSTEM GENERATION

MANUAL.

The documentation number gives you a means to find information in this

chapter while you are generating a system.

2-4

THE SYSGEN DIALOGUE

The last page of this chapter is a table that lists all  the  document

numbers  that  the SYSGEN references.  Across from the document number

is the page in this manual where you can find additional  information.

(These  numbers are the same as the MACREL tag that the SYSGEN uses to

locate the remark.)

EACH QUESTION HAS A NUMBER ASSOCIATED

WITH IT.  IF YOU NOTICE A MISTAKE DURING AN

RTS/8 QUESTION SESSION, TYPING #NNN

CAUSES QUESTION NUMBER NNN TO BE REPEATED.

YOU CAN THEN CHANGE THE ANSWER TO NNN.

THE PROGRAM THEN RESUMES AT THIS NEW

QUESTION. IT ALSO RENUMBERS QUESTIONS

BEGINNING AT THIS NUMBER.  (NOTE THAT

AFTER A TASK PRIORITY CHANGE, THIS FEATURE IS NOT

SERVICED.)

For example, if you are on question 47 and you  wish  to  change  your

answer  to  question  44, you type #44;  the SYSGEN then reprints that

question.

You may only branch backwards.  When the SYSGEN branches,  it  removes

the  answers  to  questions  it  branches  over.   Therefore, you must

reanswer questions that the SYSGEN branches over.

Because the SYSGEN dynamically numbers questions, it is possible  that

the  question  you  branched  from will have a different number if you

reencounter that question.  In  the  above  example,  if  question  44

branches  in  a different manner than it previously did, then the next

time question 47 appears, it will have a different number.   Moreover,

if  your  answer  to  question  44 means that question 47 is no longer

relevant, the SYSGEN does not repeat it.

After you add your own tasks to the system in  response  to  a  SYSGEN

question (this occurs near the end of the SYSGEN), it asks if you wish

to change the priority of any of the tasks in the system.  After  this

question,  you  cannot  use  the  # branching feature to jump to other

questions.  See DOC. NO 312 for information on task priority changes.

IF YOU TYPE AN INVALID ANSWER, THE PROGRAM

PRINTS AN ERROR MESSAGE, AND THEN REPRINTS

THE QUESTION.

Appendix E lists the error messages and explains  the  responses  that

cause  the  SYSGEN  to print a message.  For all non-fatal errors, the

SYSGEN retypes the question to which you gave an  incorrect  response.

If  the  error  was fatal, control returns to the OS/8 monitor.  Fatal

errors occur only if

2-5

THE SYSGEN DIALOGUE

1.  BATCH, TTY, and other similar utilities do not exist  in  the

system upon which you are generating the RTS/8 system.

2.  You enter an incorrect device specification  and  the  SYSGEN

cannot locate the devices.

3.  You make errors in modifying  the  SYSGEN  (as  described  in

Chapter 4).

4.  The device to which you send SYSGEN output files do not  have

sufficient space to contain the files.

All other errors are non-fatal.

CONTROL/C RETURNS YOU TO THE OS/8 KEY-

BOARD MONITOR AND ABORTS THE CURRENT SYSGEN.

If you type a CONTROL/C, all the information you enter is lost and the

SYSGEN  does  not  create  output  files.  There is no recovery if you

accidently type CONTROL/C.

FOR FILE STORAGE SPECIFICATIONS SUCH

AS DTA0, THE PROGRAM EXPECTS STANDARD DEVICE

NAMES.  NOTE THAT YOU DO NOT APPEND A COLON

TO THE DEVICE NAME.

The SYSGEN accepts any string you type that  contains  the  upper-case

letters  A  through  Z and the numbers 0 through 9.  Consequently, you

must make sure that the file specification is a character string  that

OS/8  recognizes.   You  may, of course, use an OS/8 ASSIGN command to

equate a user file specification with a system file specification.

NEXT TO THE BRACKETS [] INDICATING THE

EXPECTED RESPONSE ARE PARENTHESES () WHICH

INDICATE THE DEFAULT ANSWER.  YOU SELECT THE

DEFAULT ANSWER BY JUST TYPING A CARRIAGE RETURN.

A default answer is a predetermined answer that satisfies the majority

of  the  users  of RTS/8.  However, it may not be appropriate for your

particular system.

See Chapter 4 for information on changing defaults.

If a question does not have  a  default,  the  parentheses  contain  a

question mark, that is, (?).

2-6

THE SYSGEN DIALOGUE

The following questions ask you to state which computer of  the  PDP/8

family  you wish to install the RTS/8 system on. If the processor does

not match the  machine  type,  it  represents  a  special  case.   For

example,  the  8/A  620 series has an 8/E processor in an 8/A cabinet.

In such a case, you use the processor designation to specify the clock

task  and  the  powerfail  task.   Here, the target machine has an 8/A

clock and 8/E powerfail hardware.  For example, if you do not  have  a

clock  or  powerfail  hardware,  you may state that you have an 8/A or

8/E.  If the target machine has a clock, call it an 8/A.   If  it  has

powerfail  hardware,  call it an 8/E.  If it has both, call it an 8/E;

when you answer the clock question, specify a non-standard 8/A clock.

IS THE TARGET MACHINE FOR WHICH YOU ARE

BUILDING THE RTS/8 SYSTEM A PDP 8/A? [Y/N] (Y):

THE TARGET MACHINE IS THE MACHINE FOR

WHICH THE RTS/8 SYSTEM IS BEING BUILT.  THE

MACHINE RUNNING THIS PROGRAM AND THE TARGET MACHINE

DO NOT HAVE TO BE THE SAME.

(DOC. NO 042)

For example, you may build a system for a PDP 8/A that  contains  128K

of  memory  and many I/O peripherals while running the SYSGEN on a PDP

12 with 12K of memory whose only peripheral is a LINCtape drive.

IS THE TARGET MACHINE A PDP 8/E? [Y/N] (N):

THE TARGET MACHINE IS THE MACHINE FOR

WHICH THE RTS/8 SYSTEM IS BEING BUILT.  THE

MACHINE RUNNING THIS PROGRAM AND THE TARGET MACHINE

DO NOT HAVE TO BE THE SAME.

(DOC. NO 042)

IS THE TARGET MACHINE A PDP-12? [Y/N] (N):

THE TARGET MACHINE IS THE MACHINE FOR

WHICH THE RTS/8 SYSTEM IS BEING BUILT.  THE

MACHINE RUNNING THIS PROGRAM AND THE TARGET MACHINE

DO NOT HAVE TO BE THE SAME.

(DOC. NO 042)

IS THE TARGET MACHINE A VT78 SYSTEM? [Y/N] (N):

THE TARGET MACHINE IS THE MACHINE FOR

WHICH THE RTS/8 SYSTEM IS BEING BUILT.  THE

MACHINE RUNNING THIS PROGRAM AND THE TARGET MACHINE

DO NOT HAVE TO BE THE SAME.

(DOC. NO 042)

2-7

THE SYSGEN DIALOGUE

If your answers were "no" to the questions asking which machine  RTS/8

will run on, the SYSGEN assumes it is a PDP 8/L, a PDP-8, or a LINC-8.

THE TARGET MACHINE MUST BE A PDP 8/I,			     remark

A PDP 8/L, A PDP-8, OR A LINC-8 BECAUSE IT IS		     remark

NOT ONE OF THE ABOVE.					     remark

(DOC. NO 046)

The SYSGEN requires you to specify the actions it  will  perform  upon

completion  of  this  dialogue.   Depending  upon  your answer, it can

return to the monitor, chain to OS/8 BATCH to  execute  the  files  it

creates,  and/or  continue  through  the  BATCH procedure to the RTS/8

system.

The SYSGEN also requires you to specify  where  it  is  to  write  its

output  files,  what  it  should  name  them,  and  other such similar

information.

TO WHICH DEVICE SHOULD SYSGEN WRITE THE OUTPUT

FILES? [A] (SYS):

THIS PROGRAM PRODUCES THREE MAJOR OUTPUT

FILES:  (1) A PARAMETER FILE, (2) A MACREL BATCH

FILE, AND (3) A LINKER BATCH FILE.  OS/8 BATCH

MUST KNOW THE DEVICE UPON WHICH THESE FILES

RESIDE IN ORDER TO GENERATE AN RTS/8 SYSTEM.

(DOC. NO 022)

The device that you specify must be active in the system that you  are

using to create the RTS/8 system.  If you specify a device that is not

in the system, the  SYSGEN  produces  an  error  message  and  returns

execution  control  to the OS/8 Keyboard Monitor, and all your answers

are lost.

The parameter file is a series of statements  that  MACREL  uses  when

assembling  source  files.   These statements are equates that, if the

SYSGEN defines them, MACREL  uses  when  assembling  another  program.

(See Chapter 4 for a discussion of the parameter file.  Also, Appendix

D is an abbreviated listing of the parameter file.)

The MACREL batch  file  is  the  list  of  source  files  that  MACREL

assembles  to  create  the  .RB files that the LINK program uses.  The

SYSGEN enters the source file name for each task you indicate  in  the

SYSGEN.   (The task name must be the name of the source file.) It also

appends switches depending on answers you give to  questions  in  this

section.

The LINK batch file contains the file specifications that  LINK  needs

to create the .SV image of the RTS/8 system.

2-8

THE SYSGEN DIALOGUE

The MACREL and LINK batch files must reside on SYS  (or  a  coresident

device  such as RKB0 or RL0B) when BATCH executes.  If they are not on

SYS or a coresident device, BATCH cannot execute.  (This  is  an  OS/8

restriction.)

The SYSGEN appends switches to the  MACREL  and  LINK  batch  file  in

response to your answers to questions.

For further information on MACREL and LINK, see the MACREL/LINK User's							 __________________

Manual.     ______

DO YOU WANT TO CHAIN IMMEDIATELY TO THE

RTS/8 SYSTEM WHICH BATCH CAN PRODUCE? [Y/N] (Y):

THIS PROGRAM CAN INSERT A REQUEST TO RUN THE

RTS/8 SYSTEM IT PRODUCES WHEN BATCH FINISHES

RUNNING.  NOTE THAT INPUT SOURCE FILES MUST BE

ON ONE MEDIUM TO PROVIDE FOR THIS FEATURE TO

WORK CORRECTLY.

(DOC. NO 024)

If you wish the RTS/8 system to begin running  after  BATCH  generates

the  RTS/8  system,  answer yes to this question.  When BATCH finishes

executing, it chains to RTS/8 automatically if the RTS/8 save image is

on  SYS.   (If  the  save  image  is not on SYS, BATCH still chains to

RTS/8. However, the save image is not executable.)

WHAT FIVE CHARACTER NAME WOULD YOU LIKE

TO CALL THE FILES PRODUCED? [A] (DEFLT):

THE RTS/8 SYSGEN PROGRAM NAMES THE OUTPUT FILES.

EXAMPLE:  MDEFLT.BI IS THE INPUT SOURCE BATCH

FILE FOR THE MACREL ASSEMBLIES; LDEFLT.BI IS

THE INPUT SOURCE BATCH FILE FOR THE LINKER;

PDEFLT.MA IS THE PARAMETER FILE PRODUCED.

(DOC. NO 026)

The name you select for the output  file  can  be  from  one  to  five

characters  long.   If  your answer is six characters long, the SYSGEN

truncates it to five characters.  (If it is more than  six  characters

long, the SYSGEN prints an error message.)

The above remark assumes you chose the default  file  name  of  DEFLT.

However,  if  you  chose  the name MYOWN, the SYSGEN creates the files

MMYOWN.BI, LMYOWN.BI, and PMYOWN.MA.

2-9

THE SYSGEN DIALOGUE

The first character of the three major output files* are always M  for

MACREL,  L  for  LINK,  and  P  for  parameter.   See  Chapter 4 for a

discussion of these files.  See also DOC. NO  022  for  a  summary  of

these files.

DO YOU WANT THE SYSGEN PROGRAM TO REQUEST

BATCH TO RUN BEFORE TERMINATING? [Y/N] (Y):

THE RTS/8 SYSGEN PROGRAM CAN CHAIN TO BATCH

AFTER COMPLETING THIS QUESTION AND ANSWER SESSION.

THE PROGRAM BUILDS A BATCH FILE BASED ON

THE DEVICE SPECIFICATIONS YOU PROVIDE IN YOUR

ANSWERS.  BATCH ALSO RESUBMITS ITSELF IF ALL

SOURCE FILES ARE NOT ON THE SAME MEDIUM.

BATCH THEN CREATES THE RTS/8 SAVE IMAGE.

(DOC. NO 030)

Answering yes to this question instructs the SYSGEN to chain to BATCH.

BATCH then executes the MACREL and LINK batch files.

If all of the source MACREL files for the RTS/8 tasks are not  on  the

same  medium, which you will indicate in another question (see DOC. NO

340), the SYSGEN inserts instructions that  allow  you  to  mount  the

media  when  BATCH  needs  them.   If  the sources are not on the same

medium, then the MACREL batch file does not automatically chain to the

LINK  batch  file.  After BATCH causes MACREL to assemble all sources,

you must interrupt BATCH using a C, then submit the LINK batch file.

If you also answered yes to a previous question (DOC. NO  024),  RTS/8

also comes up running after the conclusion of the SYSGEN.

ON WHICH INPUT DEVICE DO THE SOURCE

FILES RESIDE? [A] (SYS):

FOR EXAMPLE, THE DEVICE COULD BE RXA0 OR DTA0.

(NOTE THAT A ':' IS NOT A LEGAL CHARACTER).

THE SOURCE FILES MUST RESIDE ON ONE TYPE OF

MEDIA AND THE .RB FILES MUST ALL RESIDE ON ONE

MEDIUM. IF RTS/8 IS TO RUN AUTOMATICALLY, BATCH

MUST WRITE THE .SV FILE ONTO SYS.

(DOC. NO 033)

The source files this question references are the source files for any

tasks that you include in the system and the source files that DIGITAL

provides.

--------------

*  The SYSGEN may also create control files for the  TTY  drivers  and

floppy controllers.

2-10

THE SYSGEN DIALOGUE

Note that the colon terminating this  question  is  not  part  of  the

device name;  it is part of the SYSGEN question.

Although the sources must be on one type of device, they do  not  have

to  be  on the same physical device.  For example, some of the sources

could be on one floppy and the remainder on another.  However, you can

not have some of the sources on one medium such as a floppy and others

on another medium such as a DECtape.

If, for example, the source files are on two floppies, you  answer  no

to  this  question.   When BATCH submits the MACREL batch file, MACREL

assembles all the source files on the first floppy.  BATCH then prints

a  message  that  tells  you to insert the next floppy into the drive.

After you  signal  that  you  have  performed  this  action,  assembly

continues.   This  process continues until MACREL assembles all source

files.  You must then type a C to abort  BATCH  because  BATCH  cannot

determine when MACREL has assembled all of the files.  (BATCH keeps on

displaying the message to insert the new medium.)  You  then  manually

submit the LINK batch file.

ON WHICH DEVICE SHOULD BATCH STORE THE .RB

FILES? [A] (SYS):

FOR EXAMPLE, THE DEVICE COULD BE RXA0 OR DTA0.

(NOTE THAT A ': ' IS NOT A LEGAL CHARACTER).

THE SOURCE FILES MUST RESIDE ON THE TYPE OF

MEDIA AND THE .RB FILES MUST ALL RESIDE ON ONE

MEDIUM. IF RTS/8 IS TO RUN AUTOMATICALLY, BATCH

MUST WRITE THE .SV FILE ONTO SYS.

(DOC. NO 033)

The SYSGEN uses this remark, DOC. NO 033, for several questions.

The .RB files are the assembled source files  that  are  ready  to  be

linked.

The device referenced in this question must  be  in  the  system  when

BATCH  executes  the MACREL batch file.  If there is insufficient room

on the device for storing the  .RB  files,  BATCH  displays  an  error

message.  If this occurs, you must either delete files from the medium

or place another device in that drive (if possible) and then  resubmit

the MACREL batch file.

ON WHICH OUTPUT DEVICE SHOULD BATCH STORE

THE .SV FILE? [A] (SYS):

FOR EXAMPLE, THE DEVICE COULD BE RXA0 OR DTA0.

(NOTE THAT A ': ' IS NOT A LEGAL CHARACTER).

THE SOURCE FILES MUST RESIDE ON ONE TYPE OF

MEDIA AND THE .RB FILES MUST ALL RESIDE ON ONE

MEDIUM. IF RTS/8 IS TO RUN AUTOMATICALLY, BATCH

2-11

THE SYSGEN DIALOGUE

MUST WRITE THE .SV FILE ONTO SYS.

(DOC. NO 033)

You may write the .SV image to any device in the system.  However,  if

this  device  is  not  SYS,  BATCH cannot chain to an executable RTS/8

system.  If you wish to write the .SV image to  a  non-system  device,

you  must copy the RTS/8 save file onto SYS before executing the RTS/8

image.

DO YOU WANT ASSEMBLY LISTINGS? [Y/N] (N):

YOU MAY ELECT TO OBTAIN ASSEMBLY LISTINGS

OF SYSTEM AND USER TASKS AS MACREL ASSEMBLES

THEM DURING THE BATCH STREAM.

(DOC. NO 334)

If you wish assembly listings, you receive them for all tasks  in  the

RTS/8  system.   Obtaining  assembly listings is useful if you will be

changing the sources.  For the  form,  organization,  and  content  of

assembly listings, see the MACREL/LINK User's Manual.				_________________________

TO WHICH OUTPUT DEVICE SHOULD BATCH

WRITE THE ASSEMBLY LISTINGS? [A] (LPT):

FOR EXAMPLE, THE DEVICE COULD BE RXA0 OR DTA0.

(NOTE THAT A ':' IS NOT A LEGAL CHARACTER).

THE SOURCE FILES MUST RESIDE ON ONE TYPE OF

MEDIA AND THE .RB FILES MUST ALL RESIDE ON ONE

MEDIUM. IF RTS/8 IS TO RUN AUTOMATICALLY, BATCH

MUST WRITE THE .SV FILE ONTO SYS.

(DOC. NO 033)

Normally, the SYSGEN directs MACREL to print the assembly listings  on

the  line  printer.   You may, however, instruct MACREL to write these

listings to files on any device.  If MACREL  writes  the  listings  to

device  files,  you must make sure that the device has sufficient room

to contain them.

If the SYSGEN writes the assembly listings to a mass  storage  device,

the   listings   have   an   .LS   extension   and  the  corresponding

cross-reference listing extension will be .LT.

DO YOU WANT LINKER LOAD MAP LISTINGS?

[Y/N] (N):

THE LINKER LOAD MAP DESCRIBES THE MEMORY LAYOUT

OF THE RTS/8 SYSTEM.

(DOC. NO 341)

2-12

THE SYSGEN DIALOGUE

You request a load map if you must know the locations in memory  where

tasks reside.  See the MACREL/LINK User's Manual for an explanation of			    _________________________

the form and contents of the load map.

ON WHICH OUTPUT DEVICE SHOULD BATCH

WRITE THE MEMORY MAP LISTINGS? [A] (LPT):

FOR EXAMPLE, THE DEVICE COULD BE RXA0 OR DTA0.

(NOTE THAT A :  IS NOT A LEGAL CHARACTER).

THE SOURCE FILES MUST RESIDE ON ONE TYPE OF

MEDIA AND THE .RB FILES MUST ALL RESIDE ON ONE

MEDIUM. IF RTS/8 IS TO RUN AUTOMATICALLY, BATCH

MUST WRITE THE .SV FILE ONTO SYS.

(DOC. NO 033)

Normally, the SYSGEN directs LINK to print the memory maps on the line

printer.   A  non-default answer tells LINK which device to write this

file to.

The following question asks if you want to  build  a  default  system.

This system is a small RTS/8 system and includes only the tasks listed

in DOC. NO 017.  A default system does not include user tasks.

DO YOU WISH TO GENERATE AN RTS/8 SYSTEM WITHOUT

SPECIFYING PARAMETERS? [Y/N] (N):

IF YOU ANSWER YES TO THIS QUESTION, THE

RTS/8 SYSGEN PROGRAM GOES TO PASS 2 AND

GENERATES A SYSTEM WHICH INCLUDES A TTY TASK,

A MONITOR CONSOLE ROUTINE, A TEST ROUTINE,

A NULL ROUTINE, A CLOCK ROUTINE, AND AN EXIT

TASK.

(DOC. NO 017)

For an explanation of the function and purpose of these tasks, see the

RTS/8 User's Manual.  However, the Test Routine is explained below.     ___________________

If you answer yes to this question, the  SYSGEN  immediately  goes  to

Pass  2.   Pass  2  is  the  part of SYSGEN that builds the Parameter,

MACREL, and LINK batch files.  (Section 2.3 explains Pass 2 actions.)

ARE ALL SOURCE FILES ON THE SAME MEDIUM? [Y/N] (Y):

IF, FOR EXAMPLE, THE SOURCE FILES EXIST ON

SEVERAL FLOPPIES, THEN THE BATCH FILE WILL BE

CONSTRUCTED TO RESUBMIT ITSELF.  ON EACH

PASS, THE SOURCES FROM ANOTHER

FLOPPY CAN BE ASSEMBLED.

(DOC. NO 340)

2-13

THE SYSGEN DIALOGUE

If all of the sources are not on the same medium,  OS/8  BATCH  pauses

and  prints  a  message  that  allows you to place the medium into the

drive unit.  (If they are not on the same medium, the SYSGEN inserts a

BATCH  /E  switch.   This switch specifies that when BATCH cannot find

the file on the medium, it should  not  abort.   This  gives  you  the

opportunity to place the medium with the file into the device drive.)

Note that you cannot have sources on two different kinds  of  devices,

for example, DECtape and floppies.

The SYSGEN now asks you to enter information on  the  target  system's

hardware.   (The SYSGEN does not ask these and the following questions

if you select the default system;  see DOC. NO 017.)

DOES THE TARGET SYSTEM HAVE EAE HARDWARE? [Y/N] (N):

EAE HARDWARE IS AN OPTION WHICH MAY BE

PRESENT IN THE TARGET CONFIGURATION.  EAE

HARDWARE ALLOWS OPERATIONS SUCH AS MULTIPLY AND

DIVIDE TO BE PERFORMED IN THE HARDWARE.

(DOC. NO 050)

The EAE (Extended Arithmetic Element) allows the central processor  to

perform  arithmetic  operations  at high speed.  Most users employ the

EAE with a 23-bit Floating-Point Processor to provide  fast  floating-

point  arithmetic, mathematical and trigonometric function evaluation,

and formatted floating-point I/O.

Check with the system manager or your sales representative if  you  do

not know if the system has EAE hardware.

DO YOU WISH TO SAVE THE CONTENTS OF THE EAE

REGISTERS WHEN TASK SWITCHING OCCURS? [Y/N] (Y):

IF MORE THAN ONE TASK USES THE EAE

REGISTERS, YOU SHOULD SAVE ITS CONTENTS

TO AVOID LOSING DATA WHEN TASK SWITCHING OCCURS.

ASSUME THAT BOTH TASK A AND TASK B

USE THE EAE.  IF TASK B STARTS EXECUTING

BEFORE TASK A FINISHES, THEN THE CONTENTS OF THE

EAE MAY BE DIFFERENT WHEN TASK A RESUMES BECAUSE

OF TASK B'S USE OF THE EAE.

(DOC. NO 052)

Although it is usually desirable to  save  the  contents  of  the  EAE

registers,  you  do  not  have  to  save  them.   Considerably greater

processor overhead is needed if you save the  registers;   you  should

save  them  only if the tasks that you add to the system use them.  If

you save the EAE registers, the RTS/8 Executive includes an additional

location  for  each  task;   for example, if you save the EAE and your

2-14

THE SYSGEN DIALOGUE

system has 30  tasks,  then  the  RTS/8  Executive  contains  30  more

locations than it would if you did not save the EAE registers.

DOES THE TARGET SYSTEM HAVE A HARDWARE

MQ REGISTER? [Y/N] (Y):

SOME TARGET SYSTEMS MAY NOT.

THE MQ REGISTER IS OPTIONAL ON THE PDP 8/I,

8/L, LINC 8, AND THE 8.

(DOC. NO 054)

The MQ (Multiplier Quotient Register) is a 12-bit bidirectional  shift

register  that acts as an extension of the accumulator during extended

arithmetic operations.  You may also use this register as a  temporary

storage location.

If you indicate that RTS/8 should save the EAE, then it also saves the

MQ register contents.  However, if you indicate that the target system

does not contain an EAE or that you do not wish to save the  EAE,  you

still may wish to save the MQ.

DO YOU WANT TO SAVE THE CONTENTS OF THE

MQ REGISTER WHEN TASK SWITCHING OCCURS? [Y/N] (Y):

IF MORE THAN ONE TASK USES THE MQ

REGISTER, YOU SHOULD SAVE ITS CONTENTS

TO AVOID LOSING DATA WHEN TASK SWITCHING OCCURS.

ASSUME THAT BOTH TASK A AND TASK B

USE THE MQ.  IF TASK B STARTS EXECUTING

BEFORE TASK A FINISHES, THEN THE CONTENTS OF THE

MQ MAY BE DIFFERENT WHEN TASK A RESUMES

BECAUSE OF TASK B'S USE OF THE EAE.

(DOC. NO 056)

You should save the contents of the MQ only if the tasks  you  install

in  the  RTS/8  system  use  the  MQ.   Saving the MQ requires greater

processor overhead.

IN WHICH FIELD DOES RTS8 RESIDE? [D] (0):

UNLESS YOU ARE REALLY SURE, RTS8 SHOULD

RESIDE IN FIELD 0.

(DOC. NO 353)

The answer to this question must be from 0 to 31 (decimal).

2-15

THE SYSGEN DIALOGUE

DIGITAL strongly recommends that you always place RTS/8  in  field  0.

(You  might  wish  to  place  RTS/8  in  a  different field if you are

generating a symbiont system.)

For example, if the target machine contains 16K of memory and you wish

to  place  RTS/8 in the highest possible memory locations, answer 4 to

the number of fields and then place RTS/8 in field 3.

If you place RTS/8 in a field other than 0, then you must control LINK

to  ensure  that  other RTS/8 tasks do not execute in fields below the

RTS/8 field.

HOW MANY FIELDS OF MEMORY EXIST IN THE

TARGET MACHINE? [D] (4):

IF YOU HAVE 16K OF MEMORY, THE ANSWER

IS 4.  ENTER THE NUMBER IN DECIMAL.

(DOC. NO 060)

Your answer to this question must be between 2 and 32 (decimal).

Consider carefully your answer  to  this  question.   If  you  do  not

specify  sufficient memory, there may not be enough room for the RTS/8

save image to execute  properly.   Consequently,  unless  you  have  a

reason  for  limiting  memory,  your answer should specify the maximum

amount of memory available in the target system.  (The minimum  amount

of  memory is determined by the amount of storage LINK needs to create

the system.)

If you are unsure how much memory the target system  has,  check  with

the system manager or your sales representative.  If the target system

has OS/8, you may also determine the number of fields  by  typing  the

OS/8  Keyboard  Monitor  MEMORY  command.   See  the OS/8 Handbook for								  _____________

further information.

DOES YOUR TARGET SYSTEM HAVE POWER FAIL HARDWARE? [Y/N] (N):

POWER FAIL HARDWARE IS AN OPTION

AVAILABLE ON ALL MACHINES (EXCEPT THE VT-78)

WHICH HAVE CORE MEMORY.

(DOC. NO 062)

Power fail hardware causes an interrupt to occur when the power  falls

below  a  predetermined  threshold.  When power returns, this hardware

causes the computer to restart.

The power fail hardware is useful only with machines  that  have  core

memory (which is non-volatile) If the target machine has semiconductor

memory, it ordinarily does not possess power fail hardware.

2-16

THE SYSGEN DIALOGUE

If you do not know if the system has this hardware,  check  with  your

system manager or your sales representative.

DO YOU WANT THE POWER FAIL HANDLER? [Y/N] (N):

EACH DEC TASK WILL BE LISTED ONE AT A

TIME.  IF YOU WISH TO INCLUDE THE TASK IN YOUR

SYSTEM, TYPE Y.  IF NOT, TYPE N.  IF THE

SYSGEN PROGRAM REQUIRES MORE INFORMATION

ABOUT A TASK, IT WILL ASK THE QUESTIONS

IN ANOTHER SECTION.

(DOC. NO 073)

Answer yes to this question if you wish  to  provide  for  an  orderly

shutdown  of  the  computer  when it loses power.  This task saves the

contents of all volatile registers.  When sufficient power exists, the

task  restores  the  contents  of  the  volatile registers and returns

control to RTS/8.  If you answered no to the question  that  asked  if

you  have power fail hardware (DOC. NO 062), the SYSGEN does not print

this question.

----------------- CAUTION ------------------

| If  you  erroneously  answer  that   the |

| target  system  has  power fail hardware |

| and you state that you wish to have  the |

| power  fail  task,  then  the  resulting |

| RTS/8 system  will  execute  improperly. |

| This  is an error that the SYSGEN cannot |

| check for because the target machine and |

| the   machine   upon   which   you   are |

| generating the RTS/8 system may  not  be |

| the same.				    |

--------------------------------------------

DOES THE TARGET SYSTEM HAVE KL8-A

HARDWARE? [Y/N] (N):

THE KL8-A IS A 4-CHANNEL ASYNCHRONOUS

INTERFACE FOR THE PDP-8 FAMILY.  IT PROVIDES

FULL MODEM CONTROL ON 1 CHANNEL AND PARTIAL MODEM

CONTROL ON THE OTHER THREE CHANNELS.  IF

YOU HAVE ANY KL8-A'S IN YOUR TARGET SYSTEM,

ANSWER YES TO THE QUESTION.

(DOC. NO 064)

HOW MANY KL8-A'S ARE IN THE TARGET SYSTEM? [D] (1):

THERE ARE 4 CHANNELS PER KL8-A.  IF, FOR

EXAMPLE, YOU HAVE 8 CHANNELS THE ANSWER IS 2.

(DOC. NO 066)

2-17

THE SYSGEN DIALOGUE

Your answer must be between 1 and 3.

If you do not know if the target system has a KL8-A or you do not know

how  many  KL8-As  exist  in  the  target  system, consult your system

manager or your sales representative.

THIS SECTION PRESENTS THE PARAMETERS			     remark

REQUIRED TO SPECIFY THE KL8-A TASK WHICH			     remark

YOU HAVE SELECTED.						     remark

(DOC. NO 274)						     remark

If you select the KL8-A, the SYSGEN prints the above remark  after  it

presents all DIGITAL tasks.

WHAT IS THE DEVICE CODE FOR THE FIRST KL8-A

DEVICE? [O] (40):

MULTIPLE KL8-A HARDWARE MUST HAVE

CONSECUTIVE DEVICE CODES.  THE ANSWER TO THIS

QUESTION IS THE DEVICE CODE FOR THE FIRST KL8-A.

(DOC. NO 276)

Your answer must be between 1 and 77 (octal).

If you do not know the device code, check with the system  manager  or

your sales representative.

DO YOU HAVE KT8A HARDWARE IN THE TARGET

SYSTEM? [Y/N] (N):

THE KT8A IS A MEMORY MANAGEMENT OPTION

WHICH ALLOWS YOU TO HAVE UP TO 128K OF

MEMORY ON SEVERAL MACHINES OF THE PDP-8 FAMILY.

(DOC. NO 070)

If the target machine has more than 32K of  memory,  it  has  a  KT8A.

(However,  a  machine  with  less  than  32K  may  also  possess  KT8A

hardware.) You can determine if you have more than 32K  of  memory  by

using  the  OS/8  Keyboard Monitor MEMORY command (provided the target

system has OS/8). If you are unsure if the target  system  contains  a

KT8A, check with your system manager.

DO YOU WANT ANY OF THE DEC- PROVIDED

SYSTEM TASKS IN YOUR RTS/8 SYSTEM? [Y/N] (Y):

SEVERAL TASKS ARE PROVIDED BY DEC IN

SOURCE FORM WHICH YOU CAN INCLUDE IN YOUR

2-18

THE SYSGEN DIALOGUE

RTS/8 SYSTEM (FOR EXAMPLE, THE TTY TASK).

(DOC. NO 072)

If you answer yes  to  the  above  question,  the  SYSGEN  prints  the

following remark.

EACH DEC TASK WILL BE LISTED ONE AT A			     remark

TIME.  IF YOU WISH TO INCLUDE THE TASK IN YOUR		     remark

SYSTEM, TYPE Y.  IF NOT, TYPE N.  IF THE			     remark

SYSGEN PROGRAM REQUIRES MORE INFORMATION			     remark

ABOUT A TASK, IT WILL ASK THE QUESTIONS			     remark

IN ANOTHER SECTION.					     remark

(DOC. NO 073)						     remark

This remark is also the  response  you  receive  when  you  type  a  ?

response  to  many  of  the  SYSGEN  questions that ask if you wish to

include a DIGITAL-supplied task.  For more information on any of these

questions,  see  the  appropriate  hardware manual or the RTS/8 User's							       ____________

Manual.     ______

If a task you select requires more than a yes/no response, the  SYSGEN

asks  these  questions  after  it asks all questions that allow you to

select DIGITAL tasks.  However, here they  are  grouped  together  for

your convenience.

CLOCK HANDLER? [Y/N] (N):

EACH DEC TASK WILL BE LISTED ONE AT A

TIME.  IF YOU WISH TO INCLUDE THE TASK IN YOUR

SYSTEM, TYPE Y.  IF NOT, TYPE N.  IF THE

SYSGEN PROGRAM REQUIRES MORE INFORMATION

ABOUT A TASK, IT WILL ASK THE QUESTIONS

IN ANOTHER SECTION.

(DOC. NO 073)

Answer yes to this question only if the target  system  has  a  clock.

(If the target system does not have a clock and you answer yes to this

question, RTS/8  ignores  the  clock  handler  task  that  the  SYSGEN

installs.) If you are unsure if the target system has a clock, consult

the system manager or your sales representative.

THIS SECTION PRESENTS THE PARAMETERS			     remark

REQUIRED TO SPECIFY THE CLOCK TASK WHICH			     remark

YOU HAVE SELECTED.						     remark

(DOC. NO 216)						     remark

If you state that the target system has a clock, the SYSGEN prints the

2-19

THE SYSGEN DIALOGUE

above remark and the following questions after it presents all DIGITAL

tasks.

DOES YOUR TARGET SYSTEM HAVE A

STANDARD CLOCK? [Y/N] (Y):

STANDARD CLOCKS:

CPU	 CLOCK

---	      -----

PDP 8/A     PDP8A

PDP 8/E     DK8EA/DK8EC

PDP-12      KW12

VT-78       PDP8A

OTHER       DK8EA/DK8E

IF YOUR TARGET SYSTEM HAS A CLOCK OTHER

THAN THOSE INDICATED IN THE TABLE,

ANSWER 'N' TO THIS QUESTION.

(DOC. NO 220)

If the target system contains the clock that DIGITAL normally provides

with  your  PDP  8  system,  answer  yes to this question.  If you are

unsure if you have a standard clock, consult  the  system  manager  or

your sales representative.

If the processor does not match the  machine  type,  it  represents  a

special case.  For example, the 8/A 620 series has an 8/E processor in

an 8/A cabinet.  For  information  on  answering  this  and  the  next

question, see the paragraph preceding DOC. NO 42.

WHAT IS THE DEC DESIGNATION FOR THE NON-

STANDARD CLOCK? [A] (?):

THE NON-STANDARD CLOCKS SUPPORTED ARE THE

DK8EP AND ALL THOSE WITH INSTRUCTION SETS

COMPATIBLE WITH ONE OF THE ABOVE.

(DOC. NO 222)

If you have a non-standard clock, you must check its  instruction  set

against those that this handler supports.

If the clock is incompatible with those listed above, you must provide

your own clock handler.

Note that this answer does not have a default.

HOW MANY ENTRY SLOTS IN THE CLOCK QUEUE

DO YOU WANT? [D] (16):

2-20

THE SYSGEN DIALOGUE

THE LEGAL RANGE IS 16-256.  EACH SLOT REQUIRES

6 MEMORY LOCATIONS.  (DOC. NO 224)

The clock handler uses 6 locations in memory to store a clock request.

(The  six  contiguous  locations  are  called an entry slot.) You must

ensure that the clock queue has sufficient slots to store requests; if

you specify an insufficient number, your system may malfunction.

HOW MANY SOFTWARE TICKS PER SECOND? [D] (10):

RTS8 ALLOWS THE HARDWARE CLOCK INTERRUPT RATE

TO BE SUBDIVIDED SO THAT THE SOFTWARE SEES ONLY

EVERY NTH INTERRUPT.  THE SOFTWARE INTERRUPT RATE

SHERTZ MUST DIVIDE EVENLY INTO THE HARDWARE RATE

HERTZ.  IF NOT, ASSEMBLY ERRORS RESULT.

(DOC. NO 347)

Your answer must be in the range 1 through 192 (decimal).

To answer this question, you must know the hardware tick  rate.   This

tick  rate  is usually 50 or 60 times a second.  If, for example, your

hardware clock can produce an interrupt every 1/60 of a  second,  then

you  could  answer,  among  other  answers,  1,  6,  12,  20,  and 30.

Respectively, the RTS/8 software receives interrupts every 1/60, 1/10,

1/5, 1/3, and 1/2 seconds.

DO YOU WANT THE SPECIAL CLOCK INTERRUPT LINKAGE? [Y/N] (N):

THIS FEATURE, TO BE USED WITH EXTREME CARE,

ALLOWS A USER-SUPPLIED SUBROUTINE TO BE CALLED

FROM CLOCK INTERRUPT LEVEL.

(DOC. NO 351)

In some cases, a task may require an action to be taken at a  time  as

soon  as  possible  after  the clock hardware interrupt.  This linkage

provides this close timing.  (Note that it is normally  sufficient  to

have  the clock schedule a task to carry out an action rather than use

this linkage.)

For more information, see Section 4.4  of  the  RTS/8  User's  Manual.						     _____________________

DIGITAL  recommends  that you incorporate the linkage into your system

only if a task  must  respond  immediately  after  a  clock  interrupt

occurs.

UNIVERSAL DIGITAL CONTROLLER TASK? [Y/N] (N):

EACH DEC TASK WILL BE LISTED ONE AT A

TIME.  IF YOU WISH TO INCLUDE THE TASK IN YOUR

2-21

THE SYSGEN DIALOGUE

SYSTEM, TYPE Y.  IF NOT, TYPE N.  IF THE

SYSGEN PROGRAM REQUIRES MORE INFORMATION

ABOUT A TASK, IT WILL ASK THE QUESTIONS

IN ANOTHER SECTION.

(DOC. NO 073)

The UDC/ICS is an analog and  digital  I/O  device  that  permits  the

computer   to  interrogate  external  devices  in  a  process  control

environment.  This handler is the means by which a  task  communicates

with  the  UDC/ICS  functional devices.  See Section 4.15 of the RTS/8								      _____

User's Manual for a description of this handler.     _____________

THIS SECTION PRESENTS THOSE PARAMETERS			     remark

WHICH RTS/8 REQUIRES TO SPECIFY THE UDC			     remark

TASK WHICH YOU HAVE SELECTED.				     remark

(DOC. NO 225)						     remark

If you state that the target system contains a UDC, the SYSGEN  prints

the above remark after it presents all DIGITAL tasks.

HOW MANY INTERRUPTS CAN BE STORED AT

ONE TIME? [O] (12):

YOU SET THE NUMBER OF SLOTS RESERVED FOR

INTERRUPTS PENDING SERVICE BY THIS QUESTION

(DOC. NO 227)

Your answer must be in the range 1 through 50 (octal).

This number represents the  maximum  number  of  interrupts  that  the

handler  can store.  The handler stores interrupts so that it does not

have to interrupt other processing until it  can  service  the  latest

interrupt.   If  you do not specify sufficient slots, the RTS/8 system

may either not receive an interrupt or it may stop executing.

HOW MANY COUNTER MODULES IN THE TARGET

SYSTEM'S UDC? [O] (1):

ENTER THE NUMBER OF MODULE BOARDS WHICH

WILL BE IN THE UDC OF THE TARGET SYSTEM.

(DOC. NO 231)

Your answer must be in the range 1-20 (octal).

Consult the system manager to determine how many counter modules exist

in the target system.

2-22

THE SYSGEN DIALOGUE

HOW MANY CONTACT MODULES IN THE UDC? [O] (2):

ENTER THE NUMBER OF MODULE BOARDS WHICH

WILL BE IN THE UDC OF THE TARGET SYSTEM.

(DOC. NO 231)

Your answer must be in the range 1-20 (octal).

Consult the system manager to determine how many contact modules exist

in the target system.

HOW MANY ANALOG INPUT CONVERTER MODULES? [O] (2):

ENTER THE NUMBER OF MODULE BOARDS WHICH

WILL BE IN THE UDC OF THE TARGET SYSTEM.

(DOC. NO 231)

Your answer must be in the range 1-20 (octal).

Consult  the  system  manager  to  determine  how  many  analog  input

converter modules exist in the target system.

WHAT IS THE ADDRESS OF THE FIRST COUNTER

MODULE? [O] (?):

ENTER THE ADDRESS OF THE FIRST MODULE.

THE ADDRESSES MUST BE CONTIGUOUS.

(DOC. NO 235)

Your answer must be  in  the  range  0  through  3777  (octal).   This

question has no default answer.

Consult the system manager  to  determine  the  first  counter  module

address.

WHAT IS THE ADDRESS OF THE FIRST CONTACT

INTERRUPT MODULE? [O] (?):

ENTER THE ADDRESS OF THE FIRST MODULE.

THE ADDRESSES MUST BE CONTIGUOUS.

(DOC. NO 235)

Your answer must be in the range 0 through 3777  (octal).   Note  that

there is no default answer to this question.

Consult the system manager  to  determine  the  first  contact  module

address.

2-23

THE SYSGEN DIALOGUE

WHAT IS THE ADDRESS OF THE FIRST A/D

CONVERTER MODULE? [O] (?):

ENTER THE ADDRESS OF THE FIRST MODULE.

THE ADDRESSES MUST BE CONTIGUOUS.

(DOC. NO 235)

Your answer must be from 0 to  3777  (octal).   There  is  no  default

answer to this question.

Consult the system manager to determine the first  analog  to  digital

converter address.

RF08 DISK DRIVER? [Y/N] (N):

EACH DEC TASK WILL BE LISTED ONE AT A

TIME.  IF YOU WISH TO INCLUDE THE TASK IN YOUR

SYSTEM, TYPE Y.  IF NOT, TYPE N.  IF THE

SYSGEN PROGRAM REQUIRES MORE INFORMATION

ABOUT A TASK, IT WILL ASK THE QUESTIONS

IN ANOTHER SECTION.

(DOC. NO 073)

Answer yes to this question only if the  target  system  has  an  RF08

fixed head disk drive and you wish to perform I/O using that device.

If you will be supplying your own handler or driver, answer no to this

question.

DF32 DISK DRIVER? [Y/N] (N):

EACH DEC TASK WILL BE LISTED ONE AT A

TIME.  IF YOU WISH TO INCLUDE THE TASK IN YOUR

SYSTEM, TYPE Y.  IF NOT, TYPE N.  IF THE

SYSGEN PROGRAM REQUIRES MORE INFORMATION

ABOUT A TASK, IT WILL ASK THE QUESTIONS

IN ANOTHER SECTION.

(DOC. NO 073)

Answer yes to this question if the target system has a DF32 fixed head

disk drive and you wish to perform I/O operations using the device.

RK8E DISK DRIVER? [Y/N] (N):

EACH DEC TASK WILL BE LISTED ONE AT A

TIME.  IF YOU WISH TO INCLUDE THE TASK IN YOUR

SYSTEM, TYPE Y.  IF NOT, TYPE N.  IF THE

SYSGEN PROGRAM REQUIRES MORE INFORMATION

2-24

THE SYSGEN DIALOGUE

ABOUT A TASK, IT WILL ASK THE QUESTIONS

IN ANOTHER SECTION.

(DOC. NO 073)

Answer yes to this question if the target system has an RK8E cartridge

disk drive and you will use it for I/O.

RK8 DISK DRIVER? [Y/N] (N):

EACH DEC TASK WILL BE LISTED ONE AT A

TIME.  IF YOU WISH TO INCLUDE THE TASK IN YOUR

SYSTEM, TYPE Y.  IF NOT, TYPE N.  IF THE

SYSGEN PROGRAM REQUIRES MORE INFORMATION

ABOUT A TASK, IT WILL ASK THE QUESTIONS

IN ANOTHER SECTION.

(DOC. NO 073)

Answer yes to this question if the target system has an RK8  cartridge

disk drive.

RL01 DISK DRIVER? [Y/N] (N):

EACH DEC TASK WILL BE LISTED ONE AT A

TIME.  IF YOU WISH TO INCLUDE THE TASK IN YOUR

SYSTEM, TYPE Y.  IF NOT, TYPE N.  IF THE

SYSGEN PROGRAM REQUIRES MORE INFORMATION

ABOUT A TASK, IT WILL ASK THE QUESTIONS

IN ANOTHER SECTION.

(DOC. NO 073)

Answer yes to this question if the target  system  has  an  RL01  mass

storage disk drive.

DECTAPE DRIVER? [Y/N] (N):

EACH DEC TASK WILL BE LISTED ONE AT A

TIME.  IF YOU WISH TO INCLUDE THE TASK IN YOUR

SYSTEM, TYPE Y.  IF NOT, TYPE N.  IF THE

SYSGEN PROGRAM REQUIRES MORE INFORMATION

ABOUT A TASK, IT WILL ASK THE QUESTIONS

IN ANOTHER SECTION.

(DOC. NO 073)

Answer yes to this question if the  system  contains  a  TC08  DECtape

drive.  Note that RTS/8 does not support TD8E DECtape drives.

2-25

THE SYSGEN DIALOGUE

LINCTAPE DRIVER? [Y/N] (N):

EACH DEC TASK WILL BE LISTED ONE AT A

TIME.  IF YOU WISH TO INCLUDE THE TASK IN YOUR

SYSTEM, TYPE Y.  IF NOT, TYPE N.  IF THE

SYSGEN PROGRAM REQUIRES MORE INFORMATION

ABOUT A TASK, IT WILL ASK THE QUESTIONS

IN ANOTHER SECTION.

(DOC. NO 073)

The SYSGEN asks you this question only  if  you  previously  specified

that the target machine is a PDP 12.  If you wish to use the LINCtape,

answer yes to this question.

The SYSGEN now asks if you want floppy disk controllers and  how  many

you wish to have.

FIRST FLOPPY CONTROLLER? [Y/N] (N):

EACH DEC TASK WILL BE LISTED ONE AT A

TIME.  IF YOU WISH TO INCLUDE THE TASK IN YOUR

SYSTEM, TYPE Y.  IF NOT, TYPE N.  IF THE

SYSGEN PROGRAM REQUIRES MORE INFORMATION

ABOUT A TASK, IT WILL ASK THE QUESTIONS

IN ANOTHER SECTION.

(DOC. NO 073)

SECOND FLOPPY CONTROLLER? [Y/N] (N):

EACH DEC TASK WILL BE LISTED ONE AT A

TIME.  IF YOU WISH TO INCLUDE THE TASK IN YOUR

SYSTEM, TYPE Y.  IF NOT, TYPE N.  IF THE

SYSGEN PROGRAM REQUIRES MORE INFORMATION

ABOUT A TASK, IT WILL ASK THE QUESTIONS

IN ANOTHER SECTION.

(DOC. NO 073)

THIRD FLOPPY CONTROLLER? [Y/N] (N):

EACH DEC TASK WILL BE LISTED ONE AT A

TIME.  IF YOU WISH TO INCLUDE THE TASK IN YOUR

SYSTEM, TYPE Y.  IF NOT, TYPE N.  IF THE

SYSGEN PROGRAM REQUIRES MORE INFORMATION

ABOUT A TASK, IT WILL ASK THE QUESTIONS

IN ANOTHER SECTION.

(DOC. NO 073)

FOURTH FLOPPY CONTROLLER? [Y/N] (N):

EACH DEC TASK WILL BE LISTED ONE AT A

TIME.  IF YOU WISH TO INCLUDE THE TASK IN YOUR

SYSTEM, TYPE Y.  IF NOT, TYPE N.  IF THE

2-26

THE SYSGEN DIALOGUE

SYSGEN PROGRAM REQUIRES MORE INFORMATION

ABOUT A TASK, IT WILL ASK THE QUESTIONS

IN ANOTHER SECTION.

(DOC. NO 073)

Each floppy controller performs I/O upon two floppies;  that  is,  one

floppy  unit  contains two drives.  Thus, if your system allows you to

use six floppies simultaneously, you include three floppy  controllers

in the RTS/8 system.

Answer yes to these questions if the  target  system  contains  either

single  or  double  density floppies.  Since the floppy controller can

distinguish between single and double  density  floppies,  the  SYSGEN

does not ask which kind the target system contains.

The SYSGEN allows you to include up to four floppy controllers  in  an

RTS/8  system.  (The SYSGEN also builds a control file for each floppy

controller you wish to include in the system.  A  control  file  is  a

file  that  MACREL combines with a source file.  This allows SYSGEN to

use a single source file to create more than one task.)

The SYSGEN asks if you want a second controller only  if  you  respond

affirmatively  to its question asking if you want a floppy controller.

Likewise, it asks if you want a third controller only if you said  you

wanted a second controller, and so on.

THIS SECTION PRESENTS THE HARDWARE DEVICE			     remark

CODE SPECIFICATION(S) FOR THE FLOPPY CONTROLLER(S)		     remark

WHICH YOU HAVE SELECTED.					     remark

(DOC. NO 335)

If you state that the target system contains floppy drives, the SYSGEN

prints the above remark after it finishes asking which tasks it should

include in the system.

WHAT IS THE DEVICE NUMBER FOR THE FIRST

DEVICE CONTROLLER? [O] (75):

THE RTS/8 SYSGEN PROGRAM PRODUCES A

PARAMETER CONTROL FILE FOR EACH FLOPPY

CONTROLLER YOU SPECIFY.  ENTER THE ANSWER IN

OCTAL.

(DOC. NO 136)

WHAT IS THE DEVICE NUMBER FOR THE SECOND

DEVICE CONTROLLER? [O] (70):

2-27

THE SYSGEN DIALOGUE

THE RTS/8 SYSGEN PROGRAM PRODUCES A

PARAMETER CONTROL FILE FOR EACH FLOPPY

CONTROLLER YOU SPECIFY.  ENTER THE ANSWER IN

OCTAL.

(DOC. NO 136)

WHAT IS THE DEVICE NUMBER FOR THE THIRD

DEVICE CONTROLLER? [O] (71):

THE RTS/8 SYSGEN PROGRAM PRODUCES A

PARAMETER CONTROL FILE FOR EACH FLOPPY

CONTROLLER YOU SPECIFY.  ENTER THE ANSWER IN

OCTAL.

(DOC. NO 136)

WHAT IS THE DEVICE NUMBER FOR THE FOURTH

DEVICE CONTROLLER? [O] (72):

THE RTS/8 SYSGEN PROGRAM PRODUCES A

PARAMETER CONTROL FILE FOR EACH FLOPPY

CONTROLLER YOU SPECIFY.  ENTER THE ANSWER IN

OCTAL.

(DOC. NO 136)

Your answers to these questions must be between 1 and 77 (octal).

If you do not know the device codes  for  the  floppies,  consult  the

system  manager or your sales representative.  DIGITAL also provides a

specification sheet that lists the devices codes.  See Appendix B  for

a sample sheet.

LINE PRINTER DRIVER? [Y/N] (N):

EACH DEC TASK WILL BE LISTED ONE AT A

TIME.  IF YOU WISH TO INCLUDE THE TASK IN YOUR

SYSTEM, TYPE Y.  IF NOT, TYPE N.  IF THE

SYSGEN PROGRAM REQUIRES MORE INFORMATION

ABOUT A TASK, IT WILL ASK THE QUESTIONS

IN ANOTHER SECTION.

(DOC. NO 073)

Answer yes to this question if you wish to include  the  line  printer

handler.

EMERGENCY BREAKTHROUGH TASK? [Y/N] (N):

THE EMERGENCY BREAKTHROUGH TASK ALLOWS A TASK

TO BREAK THROUGH TO A TTY (SELECTED LATER) FOR URGENT

MESSAGES.

(DOC. NO 325)

2-28

THE SYSGEN DIALOGUE

The emergency breakthrough task interrupts a task  that  controls  the

terminal  so  that  an  emergency message can be sent to the terminal.

(Using the standard RTS/8 message format, you can  send  an  emergency

message  from  one  task to this task;  the emergency task then prints

the message on the terminal.)  This  procedure  is  necessary  because

other  tasks  cannot  print messages while another task has control of

the terminal.  For more information, see Chapter 4 of the RTS/8 User's							       ____________

Manual.     ______

WHAT IS THE TASK NAME OF THE DEVICE HANDLER TO BE

USED FOR EMERGENCY BREAKTHROUGH MESSAGES? [A] (TTY):

YOU HAVE SELECTED THE EMERGENCY BREAKTHROUGH TASK.

SELECT WHICH TTY TASK IS TO BE PRE-EMPTED FOR

EMERGENCY MESSAGES.

(DOC. NO 331)

If you select the emergency breakthrough task,  state  which  terminal

receives emergency messages.

If you select the default answer, you must answer YES to the following

question.

TERMINAL DRIVER? [Y/N] (Y):

EACH DEC TASK WILL BE LISTED ONE AT A

TIME.  IF YOU WISH TO INCLUDE THE TASK IN YOUR

SYSTEM, TYPE Y.  IF NOT, TYPE N.  IF THE

SYSGEN PROGRAM REQUIRES MORE INFORMATION

ABOUT A TASK, IT WILL ASK THE QUESTIONS

IN ANOTHER SECTION.

(DOC. NO 073)

Answer yes to this question if your target system includes  terminals.

If  the  target  system  uses the MCR task or the OS/8 background task

(and they both share the same terminal), you must include  this  task.

You  may, of course, provide your own TTY handler provided you name it

TTY.MA.

HOW MANY TTY TASKS WILL BE IN THE

TARGET SYSTEM? [D] (1):

THE RTS/8 SYSGEN PROCEDURE AUTOMATICALLY

GENERATES UP TO FIVE TTY TASKS FOR YOUR

RTS/8 TARGET SYSTEM; 1 FOR EACH OF UP

TO FIVE TTY PERIPHERALS.  A CONTROL FILE WILL

BE BUILT FOR ANY IN EXCESS OF 1 TTY TASK,

NOT EXCEEDING A TOTAL OF FIVE.

(DOC. NO 317)

2-29

THE SYSGEN DIALOGUE

The SYSGEN program allows you to include up to five TTY tasks  in  the

target system.  If your system requires more than five, see Chapter 4.

For each TTY task other than the first, the SYSGEN creates  a  control

file  that contains equates necessary to generate the TTY .RB image or

images.  (A control file is a file that MACREL combines with a  source

file.   This  allows SYSGEN to use a single source file to create more

than one task.) The last line of each control file is an .INCLUDE that

tells  MACREL  to  use the principal TTY task with the equates for the

specific TTY task you generate.

The following TTY questions differ from other SYSGEN questions in that

the answer you type must be five or fewer characters.

IN THE FOLLOWING QUESTIONS, YOU CAN ENTER			     remark

UP TO FIVE ANSWERS, SEPARATED BY COMMAS.			     remark

THE ANSWERS INDICATE YOUR CHOICE FOR THE GIVEN		     remark

PARAMETER FOR EACH OF THE TTY TASKS THAT YOU HAVE		     remark

SELECTED.  EXAMPLE:					     remark

IF YOU TYPE TO THE DEVICE CODE QUESTION:			     remark

04,31,33,35,37 (CARRIAGE RETURN) YOU SELECT		     remark

DEVICE CODE CODE 04 FOR TTY TASK, 31 FOR TTY1		     remark

ETC.							     remark

(DOC. NO 252)						     remark

For the next eight questions, you  must  specify  either  the  default

answer  or  you  must specify the answer for each TTY task.  (When you

are running the SYSGEN, it prints a message that tells you that  there

are  no  more  questions  that  require more than one answer.) If, for

example, the target system contains five TTY tasks, you must supply an

answer for each.  You cannot type two answers and then have the SYSGEN

supply the default answer for the remaining three.

WHAT IS THE KEYBOARD DEVICE CODE FOR THE

TTY TASK? [O] (03):

THE DEVICE CODE FOR THE TTY TASK HAS A

DEFAULT OF 4 BUT YOU MAY CHANGE IT TO

MATCH THE TARGET SYSTEM'S HARDWARE CONFIGURATION.

THE KEYBOARD DEFAULT IS (03).

(DOC. NO 254)

The keyboard device code specifies the part of the terminal from which

the  computer  receives  information.   If  you do not know the device

code, check with your system manager  or  your  sales  representative.

The  default  shown  for  this  question is valid if the target system

contains only one TTY and it is correct for your system.

If your terminal is connected to a KL8-A, the device code is 100  plus

2-30

THE SYSGEN DIALOGUE

the  line  number for the keyboard (KB) and the teleprinter (TT).  For

additional information, see DOC. NO 174.

WHAT IS THE TTY DEVICE CODE FOR THE

TTY TASK? [O] (04):

THE DEVICE CODE FOR THE TTY TASK HAS A

DEFAULT OF 4 BUT YOU MAY CHANGE IT TO

MATCH THE TARGET SYSTEM'S HARDWARE CONFIGURATION.

THE KEYBOARD DEFAULT IS (03).

(DOC. NO 254)

This device code specification is the code the computer uses to signal

the  terminal  that  it is to receive information.  If you do not know

the device  code,  check  with  your  system  manager  or  your  sales

representative.   The default for this question is valid if the target

system contains one TTY and  the  device  code  is  correct  for  your

system.

DO YOU WANT CNTRL/S AND CNTRL/Q FEATURES

FOR YOUR TTY TASK? [Y/N] (Y):

WHEN YOU SET THIS PARAMETER, CNTRL/S STOPS

TTY PRINTOUT AND CNTRL/Q RESUMES THE PRINTOUT.

(DOC. NO 257)

DO YOU WANT RUBOUT SUPPORT FOR THE TTY

SCREEN? [Y/N] (Y):

IF YOU SET THIS PARAMETER, TYPING RUBOUT

CAUSES THE CURSOR TO MOVE LEFT 1 POSITION AND

REMOVE THE LAST CHARACTER YOU TYPE FROM SCREEN.

(DOC. NO 261)

You normally include these features if you have a CRT.  However, it is

not  necessary for you to include these features.  (On many terminals,

the RUBOUT key is labeled DELETE.)

HOW MANY NULLS AFTER A LINE FEED FOR THE TTY

DEVICE? [O] (0):

NULLS ALLOW A LINE FEED TO TAKE PLACE

BEFORE THE NEXT CHARACTER IS TRANSMITTED.

(DOC. NO 263)

Some terminals require the transmission of null  characters  in  order

that  the TTY device can complete the carriage return/line feed action

before it prints another character.

2-31

THE SYSGEN DIALOGUE

HOW MANY CHARACTERS DO YOU WANT ON A LINE

FOR THE TTY DEVICE? [D] (80):

THIS QUESTION ASKS HOW MANY CHARACTERS FIT

ON A LINE.  0 MEANS THAT NO POSITION COMPUTATIONS

ARE DONE AT ALL.  NOTE THAT THE WIDTH MUST BE A

MULTIPLE OF 8.

(DOC. NO 265)

The maximum on most CRT screens is 80.  The maximum on most  hard-copy

terminals  is 128.  (A 132-character line requires that you answer 128

to this question, because 128 is the largest number that is  divisible

by 8.) If your answer is not a multiple of 8, the teletype handler may

execute improperly.

DOES THE HARDWARE SUPPORT TABS ON THE

TTY DEVICE? [Y/N] (Y):

IF YOU ANSWER NO, THE SOFTWARE

SIMULATES TABS BY INSERTING SPACES.

(DOC. NO 273)

If you are unsure if a terminal supports  hardware  tabs,  check  with

your system manager.

HOW MANY SECONDS BEFORE A TTY READ TIMEOUT? [D] (0):

A ZERO ANSWER MEANS NO TIMEOUT EXISTS.

TIMEOUT MEANS THAT IF THE TASK HAS A PENDING

REQUEST AND YOU DO NOT TYPE A RESPONSE WITHIN THE

TIMEOUT PERIOD, THE TTY HANDLER ABORTS THE

CURRENT REQUEST.

(DOC. NO 267)

The TTY read timeout function tells RTS/8 to abort a read  request  if

the  receiving  task  does  not receive a prompt reply.  This function

allows the  terminal  to  be  freed  without  resorting  to  emergency

messages.

SWAPPER TASK? [Y/N] (N):

THE SWAPPER TASK SWAPS NONRESIDENT TASKS

INTO AND OUT OF MEMORY.  YOU MUST INCLUDE THIS

TASK IF THE RTS/8 SYSTEM WILL HAVE ANY NON-

RESIDENT TASKS.

(DOC. NO 342)

2-32

THE SYSGEN DIALOGUE

Answer yes  to  this  question  if  the  target  system  will  contain

nonresident  tasks.   If  you answer 'no' to this question, the SYSGEN

does not ask any questions about nonresident tasks  or  allow  you  to

change partitions at a later time.

You may wish to include the swapper  even  if  no  task  is  initially

non-resident.   Specifically, if you are debugging a nonresident task,

DIGITAL recommends that  you  debug  it  when  it  is  resident.   For

additional information, see Chapter 6 of the RTS/8 User's Manual.						  ___________________

THIS SECTION PRESENTS THOSE PARAMETERS			     remark

REQUIRED TO SPECIFY THE SWAPPER TASK WHICH			     remark

YOU HAVE SELECTED.						     remark

(DOC. NO 240)						     remark

If you stated that the target system contains the  swapper  task,  the

SYSGEN prints this remark after it presents all DIGITAL tasks.

WHICH RTS/8 DEVICE TASK WILL THE SWAPPER TASK

USE TO MAKE TRANSFERS? [A] (RK8E):

SWAPPING MUST OCCUR FROM THE DEVICE THAT WAS

SYS WHEN THE RTS/8 SYSTEM WAS ORIGINALLY RUN FROM OS/8.

TYPE THE RTS/8 DEVICE HANDLER FOR THAT DEVICE.

(DOC. NO 246)

This question asks you to type the name of the device  that  the  SWAP

task uses when it reads and writes task object code.  This device must

be the OS/8 system from which RTS/8 was run.

WHICH PHYSICAL UNIT WILL THE SWAPPER USE

FOR MAKING TRANSFERS? [A] (0):

TYPE THE DEVICE NUMBER OF THE DEVICE

CORRESPONDING TO SYS. TYPE 0 FOR RKA0.

(DOC. NO 250)

As some mass storage devices have more than one physical  device,  you

must  also specify which unit of the device swapper makes transfers to

or from.  Typically, this is unit 0;  however it can be other units.

CASSETTE DRIVER? [Y/N] (N):

EACH DEC TASK WILL BE LISTED ONE AT A

TIME.  IF YOU WISH TO INCLUDE THE TASK IN YOUR

SYSTEM, TYPE Y.  IF NOT, TYPE N.  IF THE

SYSGEN PROGRAM REQUIRES MORE INFORMATION

2-33

THE SYSGEN DIALOGUE

ABOUT A TASK, IT WILL ASK THE QUESTIONS

IN ANOTHER SECTION.

(DOC. NO 073)

Answer yes to this question if the target system  performs  I/O  on  a

tape cassette.

DO YOU WANT THE CASSETTE FILE SUPPORT

TASK? [Y/N] (N):

EACH DEC TASK WILL BE LISTED ONE AT A

TIME.  IF YOU WISH TO INCLUDE THE TASK IN YOUR

SYSTEM, TYPE Y.  IF NOT, TYPE N.  IF THE

SYSGEN PROGRAM REQUIRES MORE INFORMATION

ABOUT A TASK, IT WILL ASK THE QUESTIONS

IN ANOTHER SECTION.

(DOC. NO 073)

This task performs auxiliary operations such as rewind,  backspace  to

file gap, write file gap, backspace to a block gap, and skip a file on

a CAPS-8 formatted cassette.

OS/8 FILE SUPPORT TASK? [Y/N] (N):

THE OS/8 FILE SUPPORT TASK ALLOWS OTHER

TASKS TO LOOKUP, CREATE, AND DELETE FILES IN

OS/8 DIRECTORIES.  A SYSTEM MAY HAVE THIS TASK

WITHOUT HAVING OS/8 IN THE SYSTEM.

(DOC. NO 153)

The file support task allows foreground jobs to  manipulate  files  on

file-structured media.

MONITOR CONSOLE ROUTINE? [Y/N] (Y):

EACH DEC TASK WILL BE LISTED ONE AT A

TIME.  IF YOU WISH TO INCLUDE THE TASK IN YOUR

SYSTEM, TYPE Y.  IF NOT, TYPE N.  IF THE

SYSGEN PROGRAM REQUIRES MORE INFORMATION

ABOUT A TASK, IT WILL ASK THE QUESTIONS

IN ANOTHER SECTION.

(DOC. NO 073)

The MCR accepts a series of interactive commands  that  allow  you  to

control,  inspect,  and,  to  some extent, debug the RTS/8 system.  It

also allows you to schedule and execute tasks at specified  intervals,

suspend task execution, and print system status information.

2-34

THE SYSGEN DIALOGUE

THIS SECTION PRESENTS THE PARAMETERS			     remark

REQUIRED TO SPECIFY THE MCR TASK				     remark

WHICH YOU HAVE SELECTED.					     remark

(DOC. NO 205)						     remark

The SYSGEN prints this remark and the following questions if you  have

indicated  that  the  target  system  contains an MCR.  For additional

information, see Chapter 5 of the RTS/8 User's Manual.				       ___________________

DO YOU WANT TIME-OF-DAY FACILITY IN

YOUR MCR TASK? [Y/N] (Y):

THE MCR TIME COMMAND ALLOWS MCR

TO PRINT OR CHANGE THE TIME-OF-DAY.

DOC. NO 207)

The SYSGEN prints this question  if  you  specified  that  the  target

system contains a clock.  If you include this facility, you may either

change or display the system time of day.

DO YOU WANT THE MCR SYSTEM STATUS

REPORT CAPABILITY? [Y/N] (Y):

THE SYSTAT COMMAND OF THE MCR TASK

PRINTS THE STATUS OF A SINGLE TASK OR OF THE

ENTIRE SYSTEM.  INDICATED ARE FLAGS/MESSAGES, RUN

STATUS, AND SOME CURRENT REGISTER CONTENTS.

(DOC. NO 211)

See Section 5.3.13 of the RTS/8 User's Manual for a description of the					       ___________________

SYSTAT facility.

WHICH TERMINAL WILL THE MCR TASK

USE? [A] (TTY):

ENTER THE TASK NAME OF THE TASK WHICH

RTS/8 WILL USE AS THE MCR CONSOLE DEVICE.

(DOC. NO 215)

If you specify the default answer, you must include at least  one  TTY

task in the system (see DOC. NO 317).  If you do not, the RTS/8 system

you are creating will execute improperly.  (You may  also  select  the

default answer if you include your own TTY task provided that you name

this task TTY.MA.)

2-35

THE SYSGEN DIALOGUE

RTS/8 TEST TASK? [Y/N] (N):

EACH DEC TASK WILL BE LISTED ONE AT A

TIME.  IF YOU WISH TO INCLUDE THE TASK IN YOUR

SYSTEM, TYPE Y.  IF NOT, TYPE N.  IF THE

SYSGEN PROGRAM REQUIRES MORE INFORMATION

ABOUT A TASK, IT WILL ASK THE QUESTIONS

IN ANOTHER SECTION.

(DOC. NO 073)

The Test Task is a demonstration program that prints a string on  your

terminal.   As DIGITAL distributes it, the task is in a run state when

the RTS/8 system begins executing.   If  the  target  system  has  the

Monitor  Console Routine, it releases control of the terminal, and the

test task sends a message that it wishes to print  a   string  on  the

terminal.  This string consists of the characters you type in response

to the Test Task prompt.  After you type the  string,  the  Test  Task

prints it.

If you type an ESCAPE to terminate the string, the Test Task  suspends

itself.  You may reactivate the Test Task by requesting it to run from

the MCR.

The MACREL source code for the test task shows the use  of  the  .TASK

and .NONRES macros.  Thus, you can make the Test Task nonresident.

EXIT TASK? [Y/N] (N):

EACH DEC TASK WILL BE LISTED ONE AT A

TIME.  IF YOU WISH TO INCLUDE THE TASK IN YOUR

SYSTEM, TYPE Y.  IF NOT, TYPE N.  IF THE

SYSGEN PROGRAM REQUIRES MORE INFORMATION

ABOUT A TASK, IT WILL ASK THE QUESTIONS

IN ANOTHER SECTION.

(DOC. NO 073)

You include this task so that the target system  can  perform  special

processing  before  the  RTS/8  system  returns  control  to  the OS/8

Monitor. See Section 4.6 of the RTS/8 User's Manual.				     ___________________

OS/8 SUPPORT TASK? [Y/N] (N):

EACH DEC TASK WILL BE LISTED ONE AT A

TIME.  IF YOU WISH TO INCLUDE THE TASK IN YOUR

SYSTEM, TYPE Y.  IF NOT, TYPE N.  IF THE

SYSGEN PROGRAM REQUIRES MORE INFORMATION

ABOUT A TASK, IT WILL ASK THE QUESTIONS

IN ANOTHER SECTION.

(DOC. NO 073)

2-36

THE SYSGEN DIALOGUE

This task allows the RTS/8 system to support the execution of an  OS/8

operating system in the background of a foreground/background system.

OS/8 SHARED BACKGROUND TERMINAL SUPPORT TASK? [Y/N] (Y):

THE OS/8 BACKGROUND TERMINAL SUPPORT TASK

ALLOWS A TERMINAL TO BE SHARED BY FOREGROUND

TASKS AND AN OS/8 BACKGROUND.

(DOC. NO 323)

Answer yes to this question if the OS/8 terminal  background  terminal

is the same as the RTS/8 foreground.

THIS SECTION PRESENTS THE QUESTIONS			     remark

WHICH REQUIRE MULTIPLE PARAMETERS TO SPECIFY		     remark

THE OS/8 BACKGROUND TASK WHICH YOU HAVE SELECTED.		     remark

(DOC. NO 166)						     remark

If you selected the OS/8 task, the SYSGEN prints the above remark.

HOW MANY FIELDS DO YOU WISH TO ALLOCATE

FOR THE OS/8 BACKGROUND? [D] (2):

OS/8 REQUIRES A MINIMUM OF 2 FIELDS (8K).

IF YOU WISH TO USE BATCH, YOU NEED A

MINIMUM OF 3 FIELDS (12K).

(DOC. NO 170)

OS/8 requires a minimum of two memory fields to operate.  If you  will

use  the  OS/8  BATCH  utility  when RTS/8 is running, OS/8 requires a

minimum of three fields.  You may, however, specify up to 8 fields  of

memory if the target system has a KT8A.

WHAT IS THE OS/8 KEYBOARD DEVICE

CODE? [O] (03):

TYPE THE DEVICE CODE FOR THE KEY-

BOARD DEVICE YOU HAVE SELECTED FOR THE

OS/8 TASK.

(DOC. NO 174)

Your answer must be between 1 and 113.

The keyboard device code is the code for the terminal keyboard  (which

is  different  than  the  teleprinter described in the next question).

See also DOC. NO 254.

2-37

THE SYSGEN DIALOGUE

SYSGEN uses device codes in the range 100 to 113 to indicate that  the

terminal  is  attached  to  a  KL8-A.   Note  that  the  keyboard  and

teleprinter codes are the same for a KL8-A  terminal.   Thus,  100-103

are  the  four lines on the first KL8-A;  104-107 for the second;  and

110-113 for the third.

WHAT IS THE OS/8 TELEPRINTER DEVICE

CODE? [O] (04):

TYPE THE DEVICE CODE FOR THE

TERMINAL YOU HAVE SELECTED FOR THE OS/8 TASK.

(DOC. NO 172)

Your answer must be in the range 1 through 113.

The teleprinter device code is the code  for  the  logical  unit  that

prints  information  from  the  PDP/8.   If you do not know the device

code,  check  with  the  system  manager   or   your   DIGITAL   sales

representative.

WHAT IS THE RTS/8 HANDLER TASK FOR OS/8? [A] (RK8E):

YOU SPECIFY THE SYSTEM DEVICE FOR THE OS/8

TASK HERE.  TO ANSWER THIS QUESTION, TYPE THE SYSTEM

DRIVER TASK (UP TO 6 CHARACTERS).  THIS DEVICE

MUST BE 'SYS' AT THE TIME YOU RUN THE TARGET

SYSTEM.

(DOC. NO 176)

This question asks you  to  enter  the  device  name  upon  which  SYS

resides.  This device is also that which the swap task uses.

DO YOU WANT THE OS/8 BACKGROUND TASK TO RUN IN

THE HIGHEST AVAILABLE FIELDS? [Y/N] (Y):

THE OS/8 SUPPORT TASK RUNS THE OS/8 BACKGROUND

IN CONTIGUOUS FIELDS (TYPICALLY THE HIGHEST FIELDS

OF THE TARGET MACHINE). THE KT8A ALLOWS THE OS/8

BACKGROUND FIELDS TO BE LOCATED IN CONTIGUOUS FIELDS

OTHER THAN THE HIGHEST FIELDS. ENTER THE DECIMAL

NUMBER OF THE LOWEST FIELD IN WHICH OS/8 WILL RUN.

FIELD 0 IS RESERVED FOR RTS/8.

(DOC. NO 202)

If the target system possesses KT8A memory  management  hardware,  the

OS/8 support task can reside wherever it is most convenient for you to

place it.  (If you do not have the KT8A, the SYSGEN does not ask  this

2-38

THE SYSGEN DIALOGUE

question.  Instead, it automatically places OS/8 in the highest memory

fields.)

You must make sure that OS/8 does not reside in the same  field  where

RTS/8  resides  (see  DOC. NO 353).  Normally you would place RTS/8 in

field 0 and OS/8 in the upper memory  fields.   If  you  inadvertently

place the two in the same field of memory, RTS/8 does not assemble.

WHAT IS THE LOWEST FIELD WHICH RTS/8 WILL

USE FOR THE OS/8 TASK? [D] (?):

THE OS/8 SUPPORT TASK RUNS THE OS/8 BACKGROUND

IN CONTIGUOUS FIELDS (TYPICALLY THE HIGHEST FIELDS

OF THE TARGET MACHINE). THE KT8A ALLOWS THE OS/8

BACKGROUND FIELDS TO BE LOCATED IN CONTIGUOUS FIELDS

OTHER THAN THE HIGHEST FIELDS.  ENTER THE DECIMAL

NUMBER OF THE LOWEST FIELD IN WHICH OS/8 WILL RUN.

FIELD 0 IS RESERVED FOR RTS/8.

(DOC. NO 202)

Your answer must be in the range 1 through 30 (decimal).

Only if you answer no to the previous question does the  SYSGEN  print

this  question.   If you do not wish OS/8 to run in the highest fields

of memory, enter the lowest field in which it will run.  (See also the

comment for DOC. NO 202 above.)

HOW MANY NULL CHARACTERS ARE REQUIRED

AFTER A LINE FEED UNDER OS/8? [D] (0):

TO ALLOW AN I/O DEVICE TIME TO COMPLETE

A LINE FEED BEFORE TRANSMITTING THE NEXT

CHARACTER, ZERO CHARACTERS (NULLS) ARE

PADDED INTO THE TRANSMISSION FOLLOWING THE

LINE FEED CHARACTER.  YOU MUST DETERMINE

FROM THE CONFIGURATION OF THE TARGET SYSTEM

HOW MANY ARE REQUIRED.  (EXAMPLE: A 2400

BAUD VT-50 REQUIRES 4).

(DOC. NO 204)

Your answer must be in the range 0 through 10 (decimal).

Some terminals require null characters as fill  to  allow  a  carriage

return/line feed to complete its mechanical motion before transmitting

additional characters.  If you  are  unsure,  check  with  the  system

manager or your sales representative.

If  you  have  a  KL8-JA  interface,  it  automatically  provides  pad

characters.

2-39

THE SYSGEN DIALOGUE

WHAT IS THE TASK NAME FOR THE OS/8 BACKGROUND

TERMINAL SUPPORT TASK? [A] (TTY):

TYPE THE TASK NAME OF THE TERMINAL HANDLER

WHICH THE FOREGROUND WILL SHARE WITH THE OS/8

SUPPORT TASK.

(DOC. NO 327)

If you select the default to  this  question,  you  must  have  stated

previously  that  the target system has at least one TTY task.  If you

do not include a TTY task, the RTS/8 system you create cannot  execute

correctly unless you include an alternate task.

NULL JOB? [Y/N] (N):

EACH DEC TASK WILL BE LISTED ONE AT A

TIME.  IF YOU WISH TO INCLUDE THE TASK IN YOUR

SYSTEM, TYPE Y.  IF NOT, TYPE N.  IF THE

SYSGEN PROGRAM REQUIRES MORE INFORMATION

ABOUT A TASK, IT WILL ASK THE QUESTIONS

IN ANOTHER SECTION.

(DOC. NO 073)

SYSGEN prints this question only if you have a PDP 8/A.  The  function

of  this  task is to display a sequential series of numbers on the LED

accumulator.  If you answer no to this question or if you do not  have

a  PDP  8/A,  the  Executive  uses a null task that is part of the MCR

task.

If your system does not have an MCR, the RTS/8 Executive uses a  small

null routine included within its code.

DO YOU WANT TO INCLUDE ANY USER TASKS

IN THIS RTS/8 SYSTEM? [Y/N] (N):

YOU MAY SPECIFY USER TASKS DURING THIS

PORTION OF THE QUESTION AND ANSWER PROGRAM.

ANSWER YES IF YOU HAVE ANY TASKS TO INCLUDE

OTHER THAN THE SYSTEM TASKS SELECTED ABOVE.

(DOC. NO 300)

A user task is a task that you write which is to be part of the  RTS/8

system.   If  you  answer  yes to this question, the SYSGEN prints the

following remark.

2-40

THE SYSGEN DIALOGUE

ENTER THE NAMES OF YOUR TASKS FOLLOWED BY			     remark

A CARRIAGE RETURN.  EXAMPLE:				     remark

remark

TASK 1:  NAME-1,M						     remark

TASK 2:  NAME-2,M						     remark

.						     remark

.						     remark

.						     remark

TASK N:  NAME-N,M						     remark

remark

M IS THE PARTITION WHERE THE TASK WILL RESIDE		     remark

(ENTER ZERO OR NO ENTRY FOR THE MAIN LEVEL).		     remark

YOU SHOULD ENTER THE TASK IN THE ORDER OF			     remark

DECREASING PRIORITY (THE FIRST ENTERED HAS THE HIGH-	     remark

EST PRIORITY).  TERMINATE ENTRY BY TYPING A CARRIAGE	     remark

RETURN (ONLY).						     remark

(DOC. NO 301)						     remark

TASK 001 ?:

An RTS/8 system may contain up to 127 tasks.  However, the  number  of

tasks that you may include at this point is dependent on the number of

DIGITAL tasks you specify;  that is, the total number of tasks is 127,

not the number of user tasks.

The task name must be the name of the source file;  for example,  task

MYOWN must have the source file MYOWN.MA.

You should enter the tasks in order of decreasing priority.   You  may

not enter an extension to any user task as the task name that you type

must have an .MA extension.  (Note that the period you type before  an

extension  is  an  illegal  character  in  the  SYSGEN.) If you make a

mistake in assigning priorities (or partitions), the SYSGEN gives  you

an  opportunity  to  make  corrections in the following two questions.

However, if you enter a task name incorrectly, you  must  repeat  this

question  beginning  at  task  1.  You repeat the question by typing a

carriage return to leave the question, then type a # followed  by  the

question number of this question.

If you attempt to enter more than 127  tasks,  the  SYSGEN  prints  an

error  message  and then prints the following question.  This question

is also the next question when you type a carriage return in  response

to the TASK prompt.

DO YOU WANT TO CHANGE ANY OF THE TASK'S

PRIORITIES? [Y/N] (N):

YOU MAY CHANGE THE PRIORITY OF ANY OF THE

TASKS HERE.  DIGITAL RECOMMENDS THAT YOU LEAVE

THE PRIORITY OF THE SYSTEM TASKS AS IS.  FOR

YOUR CONVENIENCE, THE NAMES OF SYSTEM AND

2-41

THE SYSGEN DIALOGUE

USER TASKS WHICH YOU HAVE SPECIFIED CAN BE

LISTED HERE.

(DOC. NO 307)

If you answer YES to this question, the SYSGEN  prints  the  following

instructions.

THE WAY IN WHICH YOU CHANGE DEVICE PRIORITIES		     remark

IS TO NAME THAT TASK WHICH WILL BE HIGHER IN		     remark

PRIORITY THAN THE NEXT NAMED TASK, TAKEN A PAIR AT A	     remark

TIME, AND DELIMITED BY A COMMA.				     remark

remark

EXAMPLE:							     remark

remark

PRIORITY CHANGE:  TASK-3, TASK-1				     remark

PLACES TASK-3 HIGHER IN PRIORITY THAN			     remark

TASK-1.  THE TASK ORDER WILL NOW BE TASK-3,		     remark

TASK-1, TASK-2.  WHEN YOU FINISH CHANGING			     remark

PRIORITIES, A CARRIAGE RETURN (ONLY) TERMINATES THE ENTRY.      remark

remark

TYPE ? AND A CARRIAGE RETURN TO OBTAIN A LIST-		     remark

ING OF THE CURRENT TASK PRIORITIES AFTER ANY		     remark

PRIORITY PROMPT DURING THE RESPONSE TO THIS QUESTION	     remark

(DOC. NO 312)

PRIORITY CHANGE:

The above line is the system prompt for you to begin.

If you are unsure of the task priority order, type a question mark  in

response  to  the  PRIORITY  CHANGE prompt.  The SYSGEN then lists all

tasks in the system and their priority in octal.  The following  is  a

sample worksheet.

RTS8 BUILD PROGRAM

TASK PRIORITY ASSIGNMENT

TASK    PRIORITY

PWRF   = 001

CLOCK  = 002

RL01   = 003

RX8A   = 004

RX8B   = 005

LPT    = 006

TTY    = 007

MCR    = 010

0S8    = 011

2-42

THE SYSGEN DIALOGUE

If, in response to PRIORITY CHANGE prompts, you type

RX8A,RL01

RX8B,RL01

and then type a question mark, you receive the following worksheet.

RTS8 BUILD PROGRAM

TASK PRIORITY ASSIGNMENT

TASK    PRIORITY

PWRF   = 001

CLOCK  = 002

RX8A   = 003

RX8B   = 004

RL01   = 005

LPT    = 006

TTY    = 007

MCR    = 010

0S8    = 011

In this second listing, the two floppy controller  tasks  now  have  a

higher  priority  than the RL01 task.  The remainder of the listing is

the same.

If your system does not have a line printer, the SYSGEN  automatically

displays  the  information  on  your terminal.  If you wish to display

this information on your terminal and you have a line printer, use  an

OS/8 ASSIGN command to assign the line printer to TTY before beginning

the SYSGEN.  If you assign the line printer to your terminal  and  you

state  in  other questions that the assembler and map listings will go

to the LPT, then you must

o  Not chain to BATCH,

o  Use the OS/8 DEASSIGN command, and

o  Execute the Pass 2 output files by running BATCH.

If your system includes nonresident tasks, the SWAP task must  have  a

higher priority than any nonresident user task.

After answering this question,  you  may  not  branch  to  a  previous

question using the # feature.

DO YOU WANT TO CHANGE ANY TASK'S PARTITION

ASSIGNMENT? [Y/N] (N):

2-43

THE SYSGEN DIALOGUE

IF YOU WISH TO CHANGE THE PARTITION

ASSIGNMENT OF ANY TASK (SYSTEM OR USER), ANSWER

YES TO THE QUESTION. MOST SYSTEM TASKS MUST BE

MEMORY RESIDENT, AND SHOULD NOT BE ASSIGNED TO A

PARTITION. NO TASK WITH A PRIORITY HIGHER THAN

THE SWAP TASK MAY BE SWAPPED.

(DOC. NO 242)

If you answer YES to this question, the SYSGEN  prints  the  following

instructions.

AT THIS POINT, YOU SPECIFY THE NAMES OF THE		     remark

TASKS WHICH YOU WISH TO PLACE IN DIFFERENT			     remark

PARTITIONS.  EXAMPLE:					     remark

remark

PARTITION :  TASK-1,M					     remark

CAUSES TASK-1 TO BE PLACED IN PARTITION M.			     remark

TYPE A ? AND A CARRIAGE RETURN TO OBTAIN A CURRENT		     remark

LISTING OF PARTITION ALLOCATION.				     remark

A CARRIAGE RETURN (ONLY) TERMINATES THE ENTRY.		     remark

(DOC. NO 314)

PARTITION :

This question changes only the partition assignments.

If  you  wish  to  obtain  a  worksheet  of  the   present   partition

assignments, type a question mark in response to the PARTITION prompt.

A worksheet for a small system might look as follows.

RTS8 BUILD PROGRAM

TASK PARTITION ALLOCATION

TASK    PARTITION

PWRF     0

CLOCK    0

RL01     0

RX8A     0

RX8B     0

LPT      0

TTY      0

MCR      0

OS8      0

SWAP     0

A	   1

B	   1

C	   2

D	   3

E	   4

2-44

THE SYSGEN DIALOGUE

The following responses to PARTITION prompts

MCR,6

B,5

cause the following worksheet to appear  in  response  to  a  question

mark.

RTS8 BUILD PROGRAM

TASK PARTITION ALLOCATION

TASK    PARTITION

PWRF     0

CLOCK    0

RL01     0

RX8A     0

RX8B     0

LPT      0

TTY      0

OS8      0

SWAP     0

A	   1

C	   2

D	   3

E	   4

B	   5

MCR      6

This second worksheet differs from the first in  that  it  shows  that

task  B  now executes in partition 5 and the MCR executes in partition

6.

------------------- NOTE -------------------

| MCR is the  only  DIGITAL-supplied  task |

| that    you    can   successfully   make |

| nonresident.  However, the  SYSGEN  does |

| not  know  that  a  task  cannot be made |

| nonresident.  Thus, you  can  place  any |

| task  into  any  partition.   To prevent |

| DIGITAL tasks from becoming nonresident, |

| each  task  except the MCR has code that |

| overrides your input and does not  allow |

| LINK to place the task into a partition. |

--------------------------------------------

If you do not have a line  printer  to  display  the  partitions,  the

SYSGEN  displays the information on your terminal.  If you have a line

printer and wish to display the information on your terminal,  use  an

2-45

THE SYSGEN DIALOGUE

OS/8  ASSIGN command before beginning the SYSGEN.  (See the comment to

DOC. NO 301 for additional information.)

The above question is the last question of the SYSGEN.

If you took the default path, the SYSGEN prints

THE QUESTION AND ANSWER SESSION IS				     remark

COMPLETED.  A DEFAULT SYSTEM IS NOW BEING			     remark

PRODUCED.							     remark

If you do not take the default path, the SYSGEN prints

THE QUESTION AND ANSWER SESSION IS COMPLETED.		     remark

THE PROGRAM WILL CONTINUE ON TO				     remark

PASS 2 (FILE BUILDING).					     remark

For both default and non-default, the SYSGEN prints the  following  if

you do not chain to BATCH.

PASS 2 COMPLETED.  RETURNING TO MONITOR

If you chain to BATCH, the BATCH program prints

PASS 2 COMPLETED.  CHAINING TO BATCH AS REQUESTED

$JOB-MACREL TASK ASSEMBLY

$MSG MOUNT TASK SOURCES ON SYS: AND STRIKE ANY KEY TO CONTINUE

2.3  PASS 2 AND BATCH

As stated in the previous section, the output of the SYSGEN is

1.  A Parameter file

2.  A MACREL batch file

3.  A LINK batch file

4.  From 0 to 4 TTY control files (The source  file  TTY.MA  does

not  need  an  external  control  file.  Instead, the control

information for this file is part of the parameter file.)

2-46

THE SYSGEN DIALOGUE

5.  From 0 to 4 floppy control files

The function of Pass 2 is to create these files and write them to  the

device  you  specify.   The Pass 2 actions always occur and the SYSGEN

always writes these files.

Chapter 3 presents these files for a major RTS/8 system.   Ordinarily,

you  do  not  need  to  understand  the information contained in these

files.  You may, of course, use OS/8  to  list  these  files  on  your

terminal or line printer.

The parameter file that the SYSGEN creates consists  of  a  series  of

equates.  These equates define symbols that the RTS/8 task sources use

when they assemble.  The equates may  define  task  priority,  set  up

conditional  assemblies,  and  provide  other  such  information.  See

Chapter 4 for additional information on the Parameter File.

The switches that appear  in  the  MACREL  and  LINK  batch  file  are

dependent upon the answers you give to SYSGEN questions.

The MACREL batch file is a series of commands, one for  each  task  in

the  RTS/8  system,  that  instruct  MACREL  to assemble the tasks.  A

sample statement is:

.MAC SYS: TTY<SYS: PMYOWN,SYS: TTY/B/C/Z

This statement tells MACREL to assemble the DIGITAL-supplied TTY task,

using  the  equates  contained in the parameter file PMYOWN.MA.  (This

sample line assumes that you choose to give  the  name  MYOWN  to  the

output  files.)  The  /B option tells MACREL to print a header, the /C

option tells MACREL to create a cross-reference listing and  /Z  tells

MACREL  to  swap  BATCH  out of memory.  (The /C is a no-op unless you

request listings.)

------------------- NOTE -------------------

| These option switches appear only if you |

| require listings.			     |

--------------------------------------------

The last line of the MACREL batch file is an instruction  that  either

tells  BATCH  to  submit  the  LINK batch file and/or resubmit itself.

Note that BATCH prints a log on the OS/8 terminal.

The LINK batch file contains  the  file  specifications  necessary  to

generate the .SV image of the RTS/8 system.  The switches in this file

are:

/K set virtual core

/M load the top page of fields

/C close current overlay and open next overlay

2-47

THE SYSGEN DIALOGUE

/O close current overlay level and open next level

/X tells LINK that this is an RTS/8 system generation

If you have specified that BATCH should chain to RTS/8, the last  line

of  this file is a RUN dev: Sname command.  This command tells OS/8 to

begin executing RTS/8.

When BATCH executes, the MACREL and LINK files must reside on  SYS  or

on  a  device  coresident  with SYS such as RKB0 or RL01.  If they are

not, BATCH cannot execute.

The SYSGEN gives you four ways to create an RTS/8 system.

1.  Automatic--SYSGEN chains to BATCH and  BATCH  chains  to  the

RTS/8  system.   The  automatic  path  requires  the  minimum

intervention.  If the source files are all on one device, you

answer yes to questions that:

o  tell SYSGEN to chain to BATCH

o  tell BATCH to  execute  the  MACREL  batch  file  that

assembles  the RTS/8 sources and submit the LINK batch

file that creates the RTS/8 system

o  chain from BATCH to the RTS/8 system

Because RTS/8 executes within a batch  file,  you  return  to

BATCH  when  you  leave RTS/8.  However, since the command to

execute the RTS/8 system was  the  last  instruction  in  the

batch file, control immediately passes to the OS/8 Monitor.

If your sources are on the same kind of device but are not on

the  same  physical device, BATCH prints a message that tells

you to insert the physical  device  into  the  device  drive.

(You must then resubmit the batch file.)

2.  Semi-automatic--SYSGEN chains to BATCH  and  creates  a  save

image  of  the RTS/8 system.  You may then run this system by

using an OS/8 R command when you need to execute  RTS/8.   If

the  RTS/8  system  is not on SYS, you must either copy it to

SYS or, if the device where the save image of  RTS/8  resides

is a system device, boot the RTS/8 save image device.

This differs from the Automatic only in the last step:  BATCH

does not chain to the RTS/8 system.

3.  Manual--SYSGEN returns to the OS/8  Keyboard  Monitor.   When

you  wish  to create the RTS/8 system, you execute OS/8 BATCH

as follows

.R BATCH

*MNAME/T

or

2-48

THE SYSGEN DIALOGUE

.SUBMIT MNAME/T

where MNAME is the name of the MACREL batch file.

If the source files are not on the same medium, then you must

also  use  the /E option so that BATCH does not abort when it

does not find a file.  If the files are not all on  the  same

medium,  the  BATCH  file does not automatically chain to the

LINK BATCH file.  Instead, you must type a  ^C  and  manually

submit the LINK BATCH file.

You use the /T option so that the BATCH log  prints  on  your

terminal rather than the line printer.

After BATCH processes this file, it automatically submits the

LINK  file.   When  BATCH concludes, you have a save image of

RTS/8.  (You may then run the system as specified in 2.)

4.  Chained Manual--the same as  the  Manual  except  that  BATCH

chains to the RTS/8 system it creates.

2-49

THE SYSGEN DIALOGUE

The following block diagram illustrates the four paths.

Automatic      Semi-automatic       Manual       Chained-Manual

----------       ----------       ----------       ----------

|	   |       |	    |       |	     |       |	      |

|  OS/8  |       |  OS/8  |       |  OS/8  |       | OS/8   |

|	   |       |	    |       |	     |       |	      |

----------       ----------       ----------       ----------

|		       |		|		 |

v		       v		v		 v

----------       ----------       ----------       ----------

|	   |       |	    |       |	     |       |	      |

| SYSGEN |       | SYSGEN |       | SYSGEN |       | SYSGEN |

|	   |       |	    |       |	     |       |	      |

----------       ----------       ----------       ----------

|		       |		|		 |

v		       v		v		 v

----------       ----------       ----------       ----------

|	   |       |	    |       |	     |       |	      |

|  BATCH |       |  BATCH |       |  OS/8  |       |  OS/8  |

|	   |       |	    |       |	     |       |	      |

----------       ----------       ----------       ----------

|		       |		|		 |

v		       v		v		 v

----------       ----------       ----------       ----------

|	   |       |	    |       |	     |       |	      |

|  RTS/8 |       |  OS/8  |       |  BATCH |       |  BATCH |

|	   |       |	    |       |	     |       |	      |

----------       ----------       ----------       ----------

|		|		 |

v		v		 v

----------       ----------       ----------

|	    |       |	     |       |	      |

|  RTS/8 |       |  OS/8  |       |  RTS/8 |

|	    |       |	     |       |	      |

----------       ----------       ----------

|

v

----------

|	     |

|  RTS/8 |

|	     |

----------

The intervening OS/8 steps in this diagram  allow  you  to  switch  to

different machines or defer executing the RTS/8 system.

The RTS/8 system that BATCH creates has the name of the form  SNAME.SV

(where  NAME  is  the  character string you specified in response to a

SYSGEN question).  You execute  this  program  by  typing  an  OS/8  R

command (if you do not chain to RTS/8).  For more information, see the

RTS/8 User's Manual.     ___________________

2-50

THE SYSGEN DIALOGUE

The MACREL and LINK batch files are always run as one unit.   This  is

intentional.   The  SYSGEN assembles each source file with a parameter

file.  Changes in the parameter file due to different SYSGEN paths can

cause  great  differences in parameter file equates.  Consequently, if

the MACREL and LINK files are not run together, the RTS/8  system  may

assemble  incorrectly.   Although  it  is  tempting  to  use .RB files

created in other SYSGENS, you should not use this procedure unless the

parameter  files  each  .RB  file  was assembled with are identical or

differences will not affect the system.

2.4  RTS/8 SYSGEN DOCUMENT NUMBERS

The following table specifies the page number of  this  chapter  where

you may find additional information for RTS/8 questions.

DOC NO		PAGE	    DOC NO	       PAGE

______		____	    ______	       ____

17 . . . . . . . 2-12	     225 . . . . . . . 2-19

22 . . . . . . . 2-7	     227 . . . . . . . 2-19

24 . . . . . . . 2-8	     231 . . . . . . . 2-19

26 . . . . . . . 2-8	     235 . . . . . . . 2-20

30 . . . . . . . 2-9	     240 . . . . . . . 2-28

33 . . . . . . . 2-9	     242 . . . . . . . 2-38

42 . . . . . . . 2-6	     246 . . . . . . . 2-29

46 . . . . . . . 2-7	     250 . . . . . . . 2-29

50 . . . . . . . 2-12	     252 . . . . . . . 2-26

52 . . . . . . . 2-13	     254 . . . . . . . 2-26

54 . . . . . . . 2-13	     257 . . . . . . . 2-27

56 . . . . . . . 2-13	     261 . . . . . . . 2-27

60 . . . . . . . 2-14	     263 . . . . . . . 2-27

62 . . . . . . . 2-14	     265 . . . . . . . 2-27

64 . . . . . . . 2-15	     267 . . . . . . . 2-28

66 . . . . . . . 2-15	     273 . . . . . . . 2-28

70 . . . . . . . 2-16	     274 . . . . . . . 2-16

72 . . . . . . . 2-16	     276 . . . . . . . 2-16

73 . . . . . . . 2-16	     300 . . . . . . . 2-35

136 . . . . . . . 2-24	     301 . . . . . . . 2-35

153 . . . . . . . 2-30	     307 . . . . . . . 2-36

166 . . . . . . . 2-32	     312 . . . . . . . 2-36

170 . . . . . . . 2-32	     314 . . . . . . . 2-38

172 . . . . . . . 2-33	     317 . . . . . . . 2-26

174 . . . . . . . 2-32	     323 . . . . . . . 2-32

176 . . . . . . . 2-33	     325 . . . . . . . 2-25

202 . . . . . . . 2-33	     327 . . . . . . . 2-34

204 . . . . . . . 2-34	     331 . . . . . . . 2-25

205 . . . . . . . 2-30	     334 . . . . . . . 2-10

207 . . . . . . . 2-30	     335 . . . . . . . 2-24

211 . . . . . . . 2-30	     340 . . . . . . . 2-12

215 . . . . . . . 2-31	     341 . . . . . . . 2-11

216 . . . . . . . 2-17	     342 . . . . . . . 2-28

220 . . . . . . . 2-17	     347 . . . . . . . 2-18

222 . . . . . . . 2-18	     351 . . . . . . . 2-18

224 . . . . . . . 2-18	     353 . . . . . . . 2-14

2-51

CHAPTER 3

TWO EXAMPLE SYSGENS

3.1  OVERVIEW

This chapter is a listing of two example SYSGENS.  The  first  example

is the terminal dialogue you receive when you create a default system.

This example (which shows the dialogue, the execution  of  the  MACREL

and  LINK  batch files, and a segment of MCR prompts from an executing

RTS/8 system) does not attempt to show many of  the  features  of  the

SYSGEN.

The second example is the creation of a  larger  RTS/8  system.   This

example  shows  many of the SYSGEN features and the majority of SYSGEN

questions.  Some error messages were included within this  example  so

that  you may see how they print.  This example also lists portions of

the output files which the SYSGEN creates.

As the previous chapter discussed all elements of the SYSGEN dialogue,

the   examples  in  this  chapter  are  presented  without  additional

explanatory comment.  where necessary, comments explain which file  is

produced and other similar information.

3.2  A DEFAULT SYSGEN

.R SYSGEN

ON WHAT DEVICE IS SCRIPT.SV LOCATED ? :

RTS/8 V03 SYSTEM BUILDER PROGRAM V01.

THIS PROGRAM IS A CONVERSATIONAL

QUESTION AND ANSWER SYSTEM FOR

BUILDING AN RTS/8 REAL TIME SYSTEMS.

THE QUESTIONS AND EXPLANATIONS IN THIS

DIALOGUE ARE BRIEF.  CHAPTER 2 IN THE

SYSTEM GENERATION MANUAL INCLUDES AD-

DITIONAL INFORMATION.

001 DO YOU WANT PROGRAM OPERATION

INFORMATION? (YOU MAY TYPE Y FOR YES OR N

FOR NO FOLLOWED BY A CARRIAGE RETURN):N

002 IS THE TARGET MACHINE FOR WHICH YOU ARE

BUILDING THE RTS/8 SYSTEM A PDP 8/A? [Y/N] (Y):N

3-1

TWO EXAMPLE SYSGENS

003 IS THE TARGET MACHINE A PDP 8/E? [Y/N] (N):Y

004 TO WHICH DEVICE SHOULD SYSGEN WRITE THE OUTPUT

FILES? [A] (SYS):

005 DO YOU WANT TO CHAIN IMMEDIATELY TO THE

RTS/8 SYSTEM WHICH BATCH CAN PRODUCE? [Y/N] (Y):

006 WHAT FIVE CHARACTER NAME WOULD YOU LIKE

TO CALL THE FILES PRODUCED? [A] (DEFLT):

007 DO YOU WANT THE SYSGEN PROGRAM TO REQUEST

BATCH TO RUN BEFORE TERMINATING [Y/N] (Y):

008 ON WHICH INPUT DEVICE DO THE SOURCE

FILES RESIDE? [A] (SYS):RKB0

009 ON WHICH DEVICE SHOULD BATCH STORE

THE .RB FILES? [A] (SYS):

010 ON WHICH OUTPUT DEVICE SHOULD BATCH STORE

THE .SV FILE?  [A] (SYS):

011 DO YOU WANT ASSEMBLY LISTINGS? [Y/N] (N):

012 DO YOU WANT LINKER LOAD MAP LISTINGS? [Y/N] (N):

013 DO YOU WISH TO BUILD AN RTS/8 SYSTEM WITHOUT

SPECIFYING ANY PARAMETERS?  [Y/N] (N):Y

THE QUESTION AND ANSWER SESSION IS

COMPLETED.  A DEFAULT SYSTEM IS NOW BEING

PRODUCED.

PASS2 COMPLETED.  CHAINING TO BATCH AS REQUESTED.

The following is a listing of the batch log for the  creation  of  the

default  system created in the dialogue.  This is immediately followed

by statements from the RTS/8 MCR.

3-2

TWO EXAMPLE SYSGENS

$JOB-MACREL TASK ASSEMBLY

$MSG MOUNT TASK SOURCES ON RKB0:AND STRIKE ANY KEY TO CONTINUE

.MAC SYS:RTS8  <SYS:PDEFLT,RKB0:RTS8

NO ERRORS

.MAC SYS:TTY   <SYS:PDEFLT,RKB0:TTY

NO ERRORS

.MAC SYS:MCR   <SYS:PDEFLT,RKB0:MCR

NO ERRORS

.MAC SYS:CLOCK <SYS:PDEFLT,RKB0:CLOCK

NO ERRORS

.MAC SYS:EXIT  <SYS:PDEFLT,RKB0:EXIT

NO ERRORS

.SU SYS:LDEFLT.BI/T

$JOB RTS8 MODULE LINKAGE

.R LINK

*SYS:SDEFLT</K=01/M/X

*SYS:RTS8

*SYS:TTY

*SYS:MCR

*SYS:EXIT

*SYS:CLOCK

*$

.RU SYS:SDEFLT

>SY

01  CLOCK  O

02  TTY    M

03  MCR

04  EXIT   R

>SYSTAT TTY

02  TTY    M   0051: 0000  2224  0000  0062

>DATE  27-NOV-78

>DATE

27-NOV-78

>TIME

00:00:35

>TIME 15:04

>TIME

15:04:01

>EXIT

$END

$END BATCH

.

3-3

TWO EXAMPLE SYSGENS

3.3  A NORMAL SYSGEN

.R SYSGEN

ON WHAT DEVICE IS SCRIPT.SV LOCATED ? :  SYS

RTS/8 V03 SYSTEM BUILDER PROGRAM V01.

THIS PROGRAM IS A CONVERSATIONAL

QUESTION AND ANSWER SYSTEM FOR

BUILDING AN RTS/8 REAL TIME SYSTEMS.

THE QUESTIONS AND EXPLANATIONS IN THIS

DIALOGUE ARE BRIEF.  CHAPTER 2 IN THE

SYSTEM GENERATION MANUAL INCLUDES AD-

DITIONAL INFORMATION.

001 DO YOU WANT PROGRAM OPERATION

INFORMATION? (YOU MAY TYPE Y FOR YES OR N

FOR NO FOLLOWED BY A CARRIAGE RETURN):

YES OR NO ANSWERS ARE INDICATED

BY A [Y/N] NOTATION.  TYPE A 'Y' FOR YES

OR AN 'N' FOR NO FOLLOWED BY A CARRIAGE

RETURN TO ANSWER THE QUESTION.

QUESTIONS REQUIRING A NUMERICAL RESPONSE

ARE INDICATED BY A [D] FOR DECIMAL OR AN [O]

FOR OCTAL.  ENTER THE NUMBER FOLLOWED BY

A CARRIAGE RETURN.  (ALWAYS INTEGER)

QUESTIONS REQUIRING AN ALPHANUMERIC

ANSWER ARE INDICATED BY [A].  RESPOND

BY TYPING THE ALPHANUMERIC DATA FOLLOWED

BY A CARRIAGE RETURN.

002 ARE YOU READY TO CONTINUE? [Y/N](Y):

IF YOU WOULD LIKE AN EXPLANATION

OF A QUESTION, TYPE A ? FOLLOWED BY A CARRIAGE

RETURN.  THE PROGRAM THEN PRINTS AN EXPLANA-

TION AND ALSO REPRINTS THE QUESTION.

ON THE LINE FOLLOWING MOST REMARKS IS A

DOCUMENTATION NUMBER OF THE FORM

(DOC. NO NNN). YOU USE THIS NUMBER TO LOCATE

ADDITIONAL INFORMATION IN THE SYSTEM GENERATION

MANUAL.

3-4

TWO EXAMPLE SYSGENS

EACH QUESTION HAS A NUMBER ASSOCIATED

WITH IT.  IF YOU NOTICE A MISTAKE DURING AN

RTS/8 QUESTION SESSION, TYPING #NNN

CAUSES QUESTION NUMBER NNN TO BE REPEATED.

YOU CAN THEN CHANGE THE ANSWER TO NNN.

THE PROGRAM THEN RESUMES AT THIS NEW

QUESTION.  IT ALSO RENUMBERS QUESTIONS

BEGINNING AT THIS NUMBER.  (NOTE THAT

AFTER A TASK PRIORITY CHANGE, THIS FEATURE IS NOT

SERVICED.)

003 ARE YOU READY TO CONTINUE? [Y/N](Y):

IF YOU TYPE AN INVALID ANSWER, THE PROGRAM

PRINTS AN ERROR MESSAGE, AND THEN REPRINTS

THE QUESTION.

CONTROL C RETURNS YOU TO THE OS/8 KEY-

BOARD MONITOR AND ABORTS THE CURRENT SYSGEN.

FOR FILE STORAGE SPECIFICATIONS SUCH

AS DTA0, THE PROGRAM EXPECTS STANDARD DEVICE

NAMES.  NOTE THAT YOU DO NOT APPEND A COLON

TO THE DEVICE NAME.

NEXT TO THE BRACKETS [] INDICATING THE

EXPECTED RESPONSE ARE PARENTHESES () WHICH

INDICATE THE DEFAULT ANSWER.  YOU SELECT THE

DEFAULT ANSWER BY JUST TYPING A CARRIAGE RETURN.

004 IS THE TARGET MACHINE FOR WHICH YOU ARE

BUILDING THE RTS/8 SYSTEM A PDP 8/A? [Y/N] (Y):NO

005 IS THE TARGET MACHINE A PDP 8/E? [Y/N] (N):MAYBE

ERROR 021 NOT A YES OR NO ANSWER OR BAD DEFAULT FOR Y/N RECORD

005 IS THE TARGET MACHINE A PDP 8/E? [Y/N] (N):

006 IS THE TARGET MACHINE A PDP-12? [Y/N] (N):

007 IS THE TARGET MACHINE A VT78 SYSTEM? [Y/N] (N):

THE TARGET MACHINE MUST BE A PDP 8/I,

A PDP 8/L, A PDP-8, OR A LINC-8 BECAUSE IT IS

NOT ONE OF THE ABOVE.

(DOC.  NO 046)

3-5

TWO EXAMPLE SYSGENS

008 TO WHICH DEVICE SHOULD SYSGEN WRITE THE OUTPUT

FILES? [A] (SYS):RXA0

009 DO YOU WANT TO CHAIN IMMEDIATELY TO THE

RTS/8 SYSTEM WHICH BATCH CAN PRODUCE?  [Y/N] (Y):#8

008 TO WHICH DEVICE SHOULD SYSGEN WRITE THE OUTPUT

FILES? [A] (SYS):

009 DO YOU WANT TO CHAIN IMMEDIATELY TO THE

RTS/8 SYSTEM WHICH BATCH CAN PRODUCE?  [Y/N:] (Y):NO

010 WHAT FIVE CHARACTER NAME  WOULD YOU LIKE

TO CALL THE  FILES PRODUCED?  [A] (DEFLT):MY^U

MINE

011 DO YOU WANT THE SYSGEN PROGRAM TO REQUEST

BATCH TO RUN BEFORE TERMINATING [Y/N] (Y):

012 ON WHICH INPUT DEVICE DO THE SOURCE

FILES RESIDE?  [A] (SYS):RKB0

013 ON WHICH DEVICE SHOULD BATCH STORE THE .RB

FILES? [A] (SYS):RKA0

014 ON WHICH OUTPUT DEVICE SHOULD BATCH STORE

THE .SV FILE?  [A] (SYS):RXA0

015 DO YOU WANT ASSEMBLY LISTINGS? [Y/N] (N):Y

016 TO WHICH OUTPUT DEVICE SHOULD BATCH

WRITE THE ASSEMBLY LISTINGS?  [A] (LPT):

017 DO YOU WANT LINKER LOAD MAP LISTINGS?

[Y/N] (N):Y

018 ON WHICH OUTPUT DEVICE SHOULD BATCH

WRITE THE MEMORY MAP LISTINGS?  [A] (LPT):

019 DO YOU WISH TO BUILD AN RTS/8 SYSTEM WITHOUT

SPECIFYING ANY PARAMETERS? [Y/N] (N):

3-6

TWO EXAMPLE SYSGENS

020 ARE ALL SOURCE FILES ON THE SAME MEDIUM?  [Y/N] (Y):Y

021 DOES THE TARGET SYSTEM HAVE EAE HARDWARE?[Y/N] (N):?

EAE HARDWARE IS AN OPTION WHICH MAY BE

PRESENT IN THE TARGET CONFIGURATION.  EAE

HARDWARE ALLOWS OPERATIONS SUCH AS MULTIPLY AND

DIVIDE TO BE PERFORMED IN THE HARDWARE.

(DOC. NO 050)

021 DOES THE TARGET SYSTEM HAVE EAE HARDWARE?[Y/N] (N):Y

022 DO YOU WISH TO SAVE THE CONTENTS OF THE EAE

REGISTERS WHEN TASK SWITCHING OCCURS? [Y/N] (Y):

023 HOW MANY FIELDS OF MEMORY EXIST IN THE

TARGET MACHINE? [D] (4):#21

021 DOES THE TARGET SYSTEM HAVE EAE HARDWARE?[Y/N] (N):

022 DOES THE TARGET SYSTEM HAVE A HARDWARE

MQ REGISTER?  [Y/N] (Y):

023 DO YOU WANT TO SAVE THE CONTENTS OF THE

MQ REGISTER WHEN TASK SWITCHING OCCURS? [Y/N] (Y):

024 HOW MANY FIELDS OF MEMORY EXIST IN THE

TARGET MACHINE? [D] (4):32

025 IN WHICH FIELD DOES RTS8 RESIDE?  [D] (0):32

ERROR 202 NUMERICAL ANSWER NOT WITHIN SPECIFIED LIMITS

025 IN WHICH FIELD DOES RTS8 RESIDE?  [D] (0):29

026 DOES YOUR TARGET SYSTEM HAVE POWER FAIL

HARDWARE?  [Y/N] (N):Y

027 DOES THE TARGET SYSTEM HAVE KL8A

HARDWARE?  [Y/N] (N):Y

028 HOW MANY KL8A'S ARE IN THE TARGET SYSTEM? [D] (1):3

3-7

TWO EXAMPLE SYSGENS

029 DO YOU HAVE KT8A HARDWARE IN THE TARGET

SYSTEM?  [Y/N] (N):Y

030 DO YOU WANT ANY OF THE DEC- PROVIDED

SYSTEM TASKS IN YOUR RTS/8 SYSTEM? [Y/N] (Y):

EACH DEC TASK WILL BE LISTED ONE AT A

TIME.  IF YOU WISH TO INCLUDE THE TASK IN YOUR

SYSTEM, TYPE Y.  IF NOT, TYPE N.  IF THE

SYSGEN PROGRAM REQUIRES MORE INFORMATION

ABOUT A TASK, IT WILL ASK THE QUESTIONS

IN ANOTHER SECTION .

(DOC. NO 073)

031 DO YOU WANT THE POWER FAIL HANDLER?  [Y/N] (N) :Y

032 CLOCK HANDLER?  [Y/N] (N):Y

033 UNIVERSAL DIGITAL CONTROLLER TASK?  [Y/N] (N):Y

034 RF08 DISK DRIVER?  [Y/N] (N):Y

035 DF32 DISK DRIVER?  [Y/N] (N):Y

036 RK8E DISK DRIVER?  [Y/N] (N):Y

037 RK8 DISK DRIVER?  [Y/N] (N):Y

038 RL01 DISK DRIVER?  [Y/N] (N):Y

039 DECTAFE DRIVER?  [Y/N] (N):Y

040 FIRST FLOPPY CONTROLLER?  [Y/N] (N):Y

041 SECOND FLOPPY CONTROLLER?  [Y/N] (N):Y

042 THIRD FLOPPY CONTROLLER?  [Y/N] (N):Y

043 FOURTH FLOPPY CONTROLLER?  [Y/N] (N):Y

3-8

TWO EXAMPLE SYSGENS

044 LINE PRINTER DRIVER?  [Y/N] (N):Y

045 TELETYPE DRIVER?  [Y/N] (Y):Y

046 EMERGENCY BREAKTHROUGH TASK?  [Y/N] (N):Y

047 SWAPPER TASK?  [Y/N] (N):Y

048 CASSETTE DRIVER?  [Y/N] (N):Y

049 DO YOU WANT THE CASSETTE FILE SUPPORT

TASK?  [Y/N] (N):Y

050 OS/8 FILE SUPPORT TASK?  [Y/N] (N):Y

051 MONITOR CONSOLE ROUTINE?  [Y/N] (Y):

052 RTS/8 TEST TASK?  [Y/N] (N):N

053 EXIT TASK?  [Y/N] (N):Y

054 OS/8 SUPPORT TASK?  [Y/N] (N):Y

055 OS/8 BACKGROUND TERMINAL SUPPORT TASK? [Y/N] (Y):Y

THIS SECTION PRESENTS THE QUESTIONS WHICH REQUIRE

MULTIPLE PARAMETERS TO SPECIFY

THE OS/8 TASK WHICH YOU HAVE SELECTED.

(DOC. NO 166)

056 HOW MANY FIELDS DO YOU  WISH TO ALLOCATE

FOR THE OS/8 BACKGROUND?  [D] (2):?

OS/8 REQUIRES A MINIMUM OF 2 FIELDS (8K).

IF YOU WISH TO USE BATCH. YOU NEED A

MINIMUM OF 3 FIELDS (12K).

(DOC. NO 170)

056 HOW MANY FIELDS DO YOU WISH TO ALLOCATE

FOR THE OS/8 BACKGROUND?  [D] (2):3

3-9

TWO EXAMPLE SYSGENS

057 WHAT IS THE OS/8 KEYBOARD DEVICE

CODE?  [O] (03):9

ERROR 030 RESPONSE NOT OCTAL/DECIMAL AS EXPECTED

057 WHAT IS THE OS/8 KEYBOARD DEVICE

CODE?  [O] (03):

058 WHAT IS THE OS/8 TELEPRINTER DEVICE

CODE?  [O] (04):

059 WHAT IS THE RTS/8 HANDLER TASK FOR

OS/8?  [A] (RK8E):

060 HOW MANY NULL CHARACTERS ARE REQUIRED

AFTER A LINE FEED UNDER OS/8?  [D] (O):

061 DO YOU WANT THE OS/8 BACKGROUND TASK TO RUN IN

THE HIGHEST AVAILABLE FIELDS? [Y/N] (Y):?

THE OS/8 SUPPORT TASK RUNS THE OS/8 BACKGROUND

IN CONTIGUOUS FIELDS (TYPICALLY THE HIGHEST FIELDS

OF THE TARGET MACHINE). THE KT8A ALLOWS THE OS/8

BACKGROUND FIELDS TO BE LOCATED IN CONTIGUOUS FIELDS

OTHER THAN THE HIGHEST FIELDS. ENTER THE DECIMAL

NUMBER OF THE LOWEST FIELD IN WHICH OS/8 WILL RUN.

FIELD 0 IS RESERVED FOR RTS/8.

(DOC. NO 202)

061 DO YOU WANT THE OS/8 BACKGROUND TASK TO RUN IN

THE HIGHEST AVAILABLE FIELDS? [Y/N] (Y):30

ERROR 021 NOT A YES OR NO ANSWER OR BAD DEFAULT FOR Y/N RECORD

061 DO YOU WANT THE OS/8 BACKGROUND TASK TO RUN IN

THE HIGHEST AVAILABLE FIELDS? [Y/N] (Y):N

062 WHAT IS THE LOWEST FIELD WHICH RTS/8 WILL

USE FOR THE OS/8 TASK? [D] (?):30

THIS SECTION PRESENTS THE PARAMETERS

REQUIRED TO SPECIFY THE MCR TASK

WHICH YOU HAVE SELECTED.

(DOC. NO 205)

3-10

TWO EXAMPLE SYSGENS

063 DO YOU WANT TIME-OF-DAY FACILITY IN

YOUR MCR TASK?  [Y/N] (Y):

064 DO YOU WANT THE MCR SYSTEM STATUS

REPORT CAPABILITY?  [Y/N] (Y):

065 WHICH TERMINAL WILL THE MCR TASK

USE?   [A] (TTY):

THIS SECTION PRESENTS THE PARAMETERS

REQUIRED TO SPECIFY THE CLOCK TASK WHICH

YOU HAVE SELECTED.

(DOC. NO 216)

066 DOES YOUR TARGET SYSTEM HAVE A

STANDARD CLOCK?  [Y/N] (Y):

067 HOW MANY SOFTWARE TICKS PER SECOND?  [D] (10):#66

066 DOES YOUR TARGET SYSTEM HAVE A

STANDARD CLOCK?  [Y/N] (Y):?

STANDARD CLOCKS:

CPU	 CLOCK

---	 -----

PDP 8/A     PDP8A

PDP 8/E     DK8EA/DK8EC

PDP-12      KW12

VT-78       PDP8A

OTHER       DK8EA/DK8E

IF YOUR TARGET SYSTEM HAS A CLOCK OTHER

THOSE INDICATED IN THE TABLE,

ANSWER 'N' TO THIS QUESTION

(DOC. NO 20)

066 DOES YOUR TARGET SYSTEM HAVE A

STANDARD CLOCK?  [Y/N] (Y):N

067 WHAT IS THE DEC DESIGNATION FOR THE NON-

STANDARD CLOCK?  [A] (?):PDP8A

068 HOW MANY SOFTWARE TICKS PER SECOND?  [D] (10):30

069 HOW MANY ENTRY SLOTS IN THE CLOCK QUEUE

DO YOU WANT?  [D] (16):128

3-11

TWO EXAMPLE SYSGENS

070 DO YOU WANT THE SPECIAL CLOCK INTERRUPT LINKAGE?  [Y/N] (N):Y

THIS SECTION PRESENTS THOSE PARAMETERS

WHICH RTS/8 REQUIRES TO SPECIFY THE UDC

TASK WHICH YOU HAVE SELECTED.

(DOC. NO 225)

071 HOW MANY INTERRUPTS CAN BE STORED AT

ONE TIME?  [O] (12):

072 HOW MANY COUNTER MODULES IN THE TARGET

SYSTEM UDC?  [O] (1):

073 HOW MANY CONTACT MODULES IN THE UDC? [O] (2):

074 HOW MANY ANALOG INPUT CONVERTER MODULES? [O] (2):

075 WHAT IS THE ADDRESS OF THE FIRST COUNTER

MODULE?  [O] (?):

ERROR 23 NO DEFAULT ALLOWED FOR THIS QUESTION

075 WHAT IS THE ADDRESS OF THE FIRST COUNTER

MODULE?  [O] (?):1000

076 WHAT IS THE ADDRESS OF THE FIRST CONTACT

INTERRUPT MODULE?  [0] (O):1040

077 WHAT IS THE ADDRESS OF THE FIRST A/D

CONVERTER MODULE?  [O] (?):1100

THIS SECTION PRESENTS THOSE PARAMETERS

REQUIRED TO SPECIFY THE SWAPPER TASK WHICH

YOU HAVE SELECTED

(DOC. NO 240)

078 WHICH RTS/8 DEVICE TASK WILL THE SWAPPER TASK

USE TO MAKE TRANSFERS? [A] (RKBE):

079 WHICH PHYSICAL UNIT WILL THE SWAPPER USE

FOR MAKING TRANSFERS? [A] (0).

3-12

TWO EXAMPLE SYSGENS

080 HOW MANY TTY TASKS WILL BE IN THE TARGET

SYSTEM? [D] (1):

081 WHAT IS THE KEYBOARD DEVICE CODE FOR THE

TTY TASK?  [O] (03):

082 WHAT IS THE TELEPRINTER DEVICE CODE FOR THE

TTY TASK?  [O] (04):

083 DO YOU WANT CNTRL/S AND CNTRL/Q FEATURES FOR

YOUR TTY TASK?  [Y/N] (Y):

084 DO YOU WANT RUBOUT SUPPORT FOR THE TTY

SCREEN? [Y/N] (Y):#80

080 HOW MANY TTY TASKS WILL BE IN THE

TARGET SYSTEM?  [D] (1):5

IN THE FOLLOWING QUESTIONS.  YOU CAN ENTER

UP TO FIVE ANSWERS, SEPARATED BY COMMAS.

THE ANSWERS INDICATE YOUR CHOICE FOR THE GIVEN

PARAMETER FOR EACH OF THE TTY TASKS THAT YOU HAVE

SELECTED.  EXAMPLE:

IF YOU TYPE TO THE DEVICE CODE QUESTION:

04,31,33,35,37 (CARRIAGE RETURN) YOU SELECT

DEVICE CODE CODE 04 FOR TTY TASK, 31 FOR TTY1

ETC.

081 WHAT IS THE KEYBOARD DEVICE CODE FOR THE

TTY TASK?  [O] (03):3,5,7,11

ERROR 100 DEVICE CODE MUST BE SPECIFIED FOR EACH TTY TASK

081 WHAT IS THE KEYBOARD DEVICE CODE FOR THE

TTY TASK?  [O] (03):3,5,7,11,13

082 WHAT IS THE TELEPRINTER DEVICE CODE FOR THE

TTY TASK?  [O] (04):4,6,10,12,14

083 DO YOU WANT CNTRL/S AND CNTRL/Q FEATURES

FOR YOUR TTY TASK?  [Y/N] (Y):Y,N,Y,N,A

ERROR 021 NOT A YES OR NO ANSWER  OR BAD DEFAULT FOR Y/N RECORD

3-13

TWO EXAMPLE SYSGENS

083 DO YOU WANT CNTRL/S AND CNTRL/Q FEATURES

FOR YOUR TTY TASK?  [Y/N] (Y):Y,N,Y,N,Y

084 DO YOU WANT RUBOUT SUPPORT FOR THE

TTY SCREEN? [Y/N] (Y):Y,N,Y,N,Y

085 HOW MANY NULLS AFTER A LINE FEED FOR THE TTY

DEVICE?  [O] (0):0,10,0,10,0

086 HOW MANY CHARACTERS DO YOU WANT ON A LINE

FOR THE TTY DEVICE?  [D] (8O):80,128,80,128,80

087 DOES THE HARDWARE SUPPORT TABS ON THE

TTY DEVICE?  [Y/N] (Y):Y,N,Y,N,Y

088 HOW MANY SECONDS BEFORE A TTY READ TIMEOUT? [D] (0):10,0,0,10,0

END OF MULTIPLE ANSWER SECTION.  THE FOLLOWING

QUESTIONS REQUIRE SINGLE ANSWERS.

(DOC. NO 270)

089 WHAT IS THE TASK NAME FOR THE OS/8 BACKGROUND

TERMINAL SUPPORT TASK? [A] (TTY):

090 WHAT IS THE TASK NAME OF THE DEVICE HANDLER TO BE

USED FOR EMERGENCY BREAKTHROUGH MESSAGES? [A] (TTY):

THIS SECTION PRESENTS THE PARAMETERS

REQUIRED TO SPECIFY THE KL8A TASK WHICH

YOU HAVE SELECTED.

(DOC. NO 274)

091 WHAT IS THE DEVICE CODE FOR THE FIRST KL8A

DEVICE?  [O] (40):

THIS SECTION PRESENTS THE HARDWARE DEVICE

CODE SPECIFICATION(S) FOR THE FLOPPY CONTROLLER(S)

WHICH YOU HAVE SELECTED

(DOC. NO 335)

092 WHAT IS THE DEVICE NUMBER FOR THE FIRST

DEVICE CONTROLLER? [O] (75):

3-14

TWO EXAMPLE SYSGENS

093 WHAT IS THE DEVICE NUMBER FOR THE SECOND

DEVICE CONTROLLER? [O] (70):

094 WHAT IS THE DEVICE NUMBER FOR THE THIRD

DEVICE CONTROLLER? [O] (71):

095 WHAT IS THE DEVICE NUMBER FOR THE FOURTH

DEVICE CONTROLLER? [0] (72):

096 DO YOU WANT TO INCLUDE ANY USER TASKS IN THIS

RTS/8 SYSTEM?  [Y/N] (N):Y

ENTER THE NAMES OF YOUR TASKS FOLLOWED BY A

CARRIAGE RETURN.  EXAMPLE:

TASK 1:  NAME-1,M

TASK 2:  NAME-2,M

.

.

.

TASK N:  NAME-N,M

M IS THE PARTITION WHERE THE TASK WILL RESIDE

(ENTER ZERO OR NO ENTRY FOR THE MAIN LEVEL).

YOU SHOULD ENTER THE TASKS TN THE ORDER OF

DECREASING PRIORITY (THE FIRST ENTERED HAS THE HIGH-

EST PRIORITY).  TERMINATE ENTRY BY TYPING A CARRIAGE

RETURN (ONLY).

(DOC. NO 301)

TASK 001 ?:A,1

TASK 002 ?:B,1

TASK 003 ?:C,1

TASK 004 ?:D,1

TASK 005 ?:E,1

TASK 006 ?:F,1

.

.

.

TASK 099 ?:LH,5

3-15

TWO EXAMPLE SYSGENS

TASK 100 ?:LI,6

ERROR 120 SUM OF SYSTEM PLUS USER TASKS ALREADY 127 DECIMAL

097 DO YOU WANT TO CHANGE ANY OF THE TASK'S

PRIORITIES?  [Y/N] (N):Y

THE WAY IN WHICH YOU CHANGE DEVICE PRIORITIES

IS TO NAME THAT TASK WHICH WILL BE HIGHER IN

PRIORITY THAN THE NEXT NAMED TASK, TAKEN A PAIR AT A

TIME, AND DELIMITED BY A COMMA.

EXAMPLE:

PRIORITY CHANGE:  TASK-3, TASK-1

PLACES TASK-3 HIGHER IN PRIORITY THAN

TASK-1.  THE TASK ORDER WILL NOW BE TASK-3,

TASK-1, TASK-2.  WHEN YOU FINISH CHANGING

PRIORITIES, A CARRIAGE RETURN (ONLY) TERMINATES THE ENTRY.

(DOC. NO 312)

TYPE ? AND A CARRIAGE RETURN TO OBTAIN A LIST-

ING OF THE CURRENT TASK PRIORITIES AFTER ANY

PRIORITY PROMPT DURING THE RESPONSE TO THIS QUESTION

(DOC. NO 244)

PRIORITY CHANGE:?

The following is a segment of the task priority list that was  printed

on a line printer in response to a question mark.

RTS8 BUILD PROGRAM

TASK PRIORITY ASSIGNMENT

TASK     PRIORITY

PWRF    = 001

CLOCK   = 002

DTA     = 003

CSA     = 004

UDC     = 005

RL01    = 006

RF08    = 007

DF32    = 010

RK8     = 011

RK8E    = 012

RX8A    = 013

RX8B    = 014

RX8C    = 015

RX8D    = 016

LPT     = 017

3-16

TWO EXAMPLE SYSGENS

EMT     = 020

TTY     = 021

TTY1    = 022

TTY2    = 023

TTY3    = 024

TTY4    = 025

SWAP    = 026

CSAF    = 027

OS8F    = 030

MCR     = 031

A       = 032

B       = 033

C       = 034

.

.

.

SD      = 164

LA      = 165

LB      = 166

LC      = 167

LD      = 170

LE      = 171

LF      = 172

LG      = 173

LH      = 174

EXIT    = 175

BKT     = 176

OS8     = 177

PRIORITY CHANGE:AA,TTY

PRIORITY CHANGE:AB,TTY

PRIORITY CHANGE:AC,TTY

PRIORITY CHANGE:AD,TTY

PRIORITY CHANGE:LA,TTY

PRIORITY CHANGE:?

PRIORITY CHANGE:

After several changes were made, another worksheet  was  obtained.   A

segment of this listing is:

3-17

TWO EXAMPLE SYSGENS

LPT     = 017

EMT     = 020

AA      = 021

AB      = 022

AC      = 023

AD      = 024

LA      = 025

TTY     = 026

TTY1    = 027

098 DO YOU WANT TO CHANGE ANY TASK'S PARTITION

ASSIGNMENT? [Y/N] (N):Y

AT THIS POINT, YOU SPECIFY THE NAMES OF THE

TASKS WHICH YOU WISH TO PLACE IN DIFFERENT

PARTITIONS.  EXAMPLE:

PARTITION :  TASK-1,M

CAUSES TASK-1 TO BE PLACED IN PARTITION M.

TYPE A ? AND A CARRIAGE RETURN TO OBTAIN A CURRENT

LISTING OF PARTITION ALLOCATION

A CARRIAGE RETURN (ONLY) TERMINATES THE ENTRY.

(DO. NO 314)

PARTITION :?

In response to a question mark, the SYSGEN printed a  listing  of  the

partitions  in which tasks reside.  The following is a segment of that

list.

RTS8 BUILD PROGRAM

TASK PARTITION ALLOCATION

TASK     PARTITION

PWRF      0

CLOCK     0

DTA       0

CSA       0

UDC       0

RL01      0

RF08      0

DF32      0

RK8       0

RK8E      0

RX8A      0

RX8B      0

RX8C      0

RX8D      0

LPT       0

EMT       0

TTY       0

3-18

TWO EXAMPLE SYSGENS

TTY1      0

TTY2      0

TTY3      0

TTY4      0

SWAP      0

CSAF      0

OS8F      0

MCR       0

Q	       0

R	       0

S	       0

EXIT      0

BKT       0

OS8       0

A	       1

B	       1

C	       1

D	       1

.

.

.

PC	       7

PD	       7

PE	       7

PF	       7

PG	       7

PH	       7

PI	       7

PARTITION :MCR,6

PARTITION :AA,3

ERROR 203 NON-RESIDENT TASKS MAY NOT HAVE PRIORITY

HIGHER THAN THE SWAP TASK.

PARTITION :A,3

PARTITION :B,4

PARTITION :C,5

PARTITION :D,6

PARTITION :E,7

ERROR 206 NO MORE THAN 16 ENTRIES PER PARTITION.

3-19

TWO EXAMPLE SYSGENS

PARTITION :E,6

PARTITION :

THE QUESTION AND ANSWER SESSION IS COMPLETED.

THE PROGRAM WILL CONTINUE ON TO

PASS 2 (FILE BUILDING).

PASS2 COMPLETED.  CHAINING TO BATCH AS REQUESTED.

$JOB-MACREL TASK ASSEMBLY

$MSG MOUNT TASK SOURCES ON RKB0:AND STRIKE ANY KEY TO CONTINUE

The following is a partial listing of the parameter file  that  SYSGEN

produces.

PWRF=001

CLOCK=002

DTA=003

CSA=004

UDC=005

RL01=00

RF08=007

DF32=010

RK8=011

RK8E=012

RX8A=013

RX8B=014

RX8C=015

RX8D=016

LPT=017

EMT=020

AA=021

AB=022

AC=023

AD=024

LA=025

TTY=026

TTY1=027

TTY2=030

TTY3=031

TTY4=032

SWAP=033

CSAF=034

OS8F=035

MCR=036

A=037

B=040

.

3-20

TWO EXAMPLE SYSGENS

.

.

LB=166

LC=167

LD=170

LE=171

LF=172

LG=173

LH=174

EXIT=175

BKT=176

OS8=177

NTASKS=177

MQ=1

MQSV=1

NFLDS=32.

RTFLD=29.

PWRFAL=1

KL8A=3.

KT8A=1

OSFLDS=3.

OSTTDV=04

OSKBDV=03

OSSYSD=RKBE

OSFILL=0.

OSBASE=30.

BKTCDV=TTY

MCRCLK=1

MCRSYS=1

MCRCDV=TTY

CLKTYP=PDP8A

SHERTZ=30.

CLKQLN=128.

CLKLRM=1

RINGBF=12

NCNTR=1

NCNTC=2

NAD=2

FCTR=1000

FCT=1040

FAD=1100

SYS=RK8E

KBDEV=3

TTDEV=4

VT50=1

SCOPE=1

FILL=0

WIDTH=80.

TAB=1

EMTCDV=TTY

TTTIM=10.

KL8ADV=40

AC7776= CLL STA RAL

AC7775= CLL STA RTL

3-21

TWO EXAMPLE SYSGENS

AC4000= CLA STL RAR

AC3777= CLL STA RAR

AC2000= CLA STL RTR

AC0002= CLA STL RTL

AC5777= CLL STA RTR

CAL=    JMS 20

POSTDS=JMP I 24

WAITM=  JMS l 25

SEND=   0

RECEIV=l

WAITE=  2

RUN=    3

SUSPND=4

POST=   5

SKPINS=6

DERAIL=7

BLKARG=10

SENDW=  11

UNBARG=12

RESCHD=13

WAITX=  14

FREE= 4000

lFDEF UDC  <

AO=0

DO=0

DI=2

GC=3

EC=4

RC=5

DC=6

ECT=7

CS=l0

DCT=11

AI=12>

NONRWT=4000

EFWT=   2000

RUNWT=  1000

SWPWT=  0400

EORMWT=0200

USERWT=0100

ENABWT=0040

MSGWT=  0020

NETWT=  0010

DNEWT=  0001

NOPACK=4000

NOCRLF=2000

IND=1000

NOLINE=400

ASSGN=200

KL8ALINE=100

IFDEF CLOCK <

MARKTIME=0

SCHEDULE=1000

TIMOUT=2000

3-22

TWO EXAMPLE SYSGENS

PERIODICALLY=2000

CANCEL=7000>

IFNDEF OS8 <

SLASHK=NFLDS-1>

IFDEF OS8 <

IFNDEF OSBASE <OSBASE=NFLDS-OSFLDS>

IFZERO NFLDS-OSBASE-OSFLDS <SLASHK=NFLDS-OSFLDS>>

IFNDEF BKTCDV <BKTCDV=0>

IFNDEF  SUNIT <SUNIT= 0>

HGHFLD=NFLDS-1

HGHFLD=<<HGHFLD&7[10>+<HGHFLD&10%2>+<HGHFLD&20[4>>

IFNDEF  PDP8E   <PDP8E=0>

IFNDEF PDP8A    <PDP8A=0>

IFNDEF  PDP12   <PDP12=0>

IFNDEF VT78     <VT78=0>

OMNI=PDP8E+PDP8A+VT78

IFNDEF  EAESV   <EAESV=0>

IFNDEF  EAE     <EAE=0>

IFNDEF  MQSV    <MQSV=0>

IFNDEF  MQ      <MQ=0>

IFNDEF  PWRFAL  <PWRFAL=0>

IFNDEF  KL8A    <KL8A=0>

IFNDEF KT8A     <KT8A=0>

IFNZRO EAESV    <MQSV=1>

IFNZRO EAE      <MQ=1>

IFZERO EAE      <EAESV=0>

IFZERO MQ       <MQSV=0>

IFDEF CLOCK     <

IFNDEF CLKQLN   <CLKQLN=20>

IFNDEF SHERTZ   <SHERTZ=12>

IFNDEF CLKLRM   <CLKLRM=0>

IFNDEF CLKTYP   <

CLKTYP=3

IFNZRO PDP8A    <CLKTYP=2>

IFNZRO PDP12    <CLKTYP=0>

IFNZRO PDP8E    <CLKTYP=0>

>

IFNZRO CLKTYP&1 <HERTZ=1750>

IFZERO CLKTYP   <HERTZ=170>

IFZERO CLKTYP-2 <HERTZ=144>

IFNZRO HERTZ%SHERTZ[SHERTZ-HERTZ <_ERROR>

IFZERO SHERTZ-301&4000 <_ERROR_>

>

IFNDEF RTFLD    <RTFLD=0>

IFNZRO KT8A     <

KTFLDS=40

RACA=6175

RACB=6176

RACC=6177

>

EJECT

3-23

TWO EXAMPLE SYSGENS

The SYSGEN also produced  four  floppy  control  files  and  four  TTY

control files.

TASK=RX8A

RXDVCD=75

.INCLUDE RKB0:RX.MA

TASK=RX8B

RXDVCD=70

.INCLUDE RKB0:RX.MA

TASK=RX8C

RXDVCD=71

.INCLUDE RKB0:RX.MA

TASK=RX8D

RXDVCD=72

.INCLUDE RKB0:RX.MA

TASK=TTY1

TTDEV=6

KBDEV=5

FILL=10

WIDTH=126.

TTTIM=O.

.INCLUDE RKB0:TTY.MA

TASK=TTY2

TTDEV=10

KBDEV=7

VT50=1

SCOPE=1

FILL=0

WIDTH=80

TAB= 1

TTTIM=O.

.INCLUDE RKB0:TTY.MA

TASK=TTY3

TTDEV=12

KBDEV=11

FILL=10

WIDTH=128.

TTTIM=10.

.INCLUDE RKB0:TTY.MA

3-24

TWO EXAMPLE SYSGENS

TASK=TTY4

TTDEV=14

KBDEV=13

VT50=1

SCOPE=1

FILL=0

WIDTH=80.

TAB=1

TTTIM=O.

.INCLUDE RKB0:TTY.MA

The following are excerpts from the MACREL and LINK batch  files.   In

the  LINK  batch  file,  note the placement of the /O switches.  It is

these switches that actually cause the  tasks  to  be  placed  into  a

partition.   Note that the parameter file does not contain information

on partition allocation.  This information exists only in this file.

$JOB-MACREL TASK ASSEMBLY

$MSG MOUNT TASK SOURCES ON RKB0:AND STRIKE ANY KEY TO CONTINUE

.MAC RKA0:RTS8  ,LPT:RTS8 <SYS:PMINE,RKB0:RTS8  /B/C

.MAC RKA0:PWRF,LPT:PWRF<SYS:PMINE,RKB0:PWRF/B/C

.MAC RKA0:CLOCK,LPT:CLOCK<SYS:PMINE,RKB0:CLOCK/B/C

.MAC RKA0:DTA,LPT:DTA<SYS:PMINE,RKB0:DTA/B/C

.MAC RKA0:CSA,LPT:CSA<SYS:PMINE,RKB0:CSA/B/C

.MAC RKA0:UDC,LPT:UDC<SYS:PMINE,RKB0:UDC/B/C

.MAC RKA0:RL01,LPT:RL01<SYS:PMINE,RKB0:RL01/B/C

.MAC RKA0:RF08,LPT:RF08<SYS:PMINE,RKB0:RF08/B/C

.MAC RKA0:DF32,LPT:DF32<SYS:PMINE,RKB0:DF32/B/C

.MAC RKA0:RK8,LPT:RK8<SYS:PMINE,RKB0:RK8/B/C

.MAC RKA0:RK8E,LPT:RK8E<SYS:PMINE,RKB0:RK8E/B/C

.MAC RKA0:RX8A,LPT:RX8A<SYS:PMINE,RKB0:RX8A/B/C

.

.

.

.MAC RKA0:LE,LPT:LE<SYS:PMINE,RKB0:LE/B/C

.MAC RKA0:LF,LPT:LF<SYS:PMINE,RKB0:LF/B/C

.MAC RKA0:LG,LPT:LG<SYS:PMINE,RKB0:LG/B/C

.MAC RKA0:LH,LPT:LH<SYS:PMINE,RKB0:LH/B/C

.MAC RKA0:EXIT,LPT:EXIT<SYS:PMINE,RKB0:EXIT/B/C

.MAC RKA0:OS8,LPT:OS8<SYS:PMINE,RKB0:OS8/B/C

.SU SYS:LMINE.BI/T

$END

$JOB RTS8 MODULE LINKAGE

.R LINK

*RXA0:SMINE,LPT:SMINE</K=37/M/X

*RKA0:RTS8

*RKA0:PWRF

*RKA0:CLOCK

3-25

TWO EXAMPLE SYSGENS

*RKA0:DTA

*RKA0:CSA

*RKA0:UDC

*RKA0:RL01

*RKA0:RF08

*RKA0:DF32

*RKA0:RK8

*RKA0:RK8E

*RKA0:RX8A

*RKA0:RX8B

*RKA0:RX8C

*RKA0:RX8D

*RKA0:LPT

*RKA0:TTY

*RKA0:TTY1

*RKA0:TTY2

*RKA0:TTY3

*RKA0:TTY4

*RKA0:SWAP

*RKA0:CSAF

*RKA0:OS8F

*RKA0:Q

*RKA0:R

*RKA0:S

*RKA0:EXIT

*RKA0:OS8

*/O

*RKA0:F/C

*RKA0:G/C

*RKA0:H/C

*RKA0:I/C

*RKA0:J/C

*RKA0:K/C

*RKA0:L/C

*RKA0:M/C

*RKA0:N/C

*RKA0:O/C

*RKA0:P/C

*/O

*RKA0:AA/C

*RKA0:T/C

.

.

.

*RKA0:ZB/C

*RKA0:LE/C

*/O

*RKA0:Y/C

*RKA0:AF/C

*RKA0:AM/C

*RKA0:KF/C

*RKA0:BE/C

*RKA0:BF/C

*RKA0:BG/C

3-26

TWO EXAMPLE SYSGENS

*RKA0:PA/C

*RKA0:PB/C

*RKA0:PC/C

*RKA0:PD/C

*RKA0:PE/C

*RKA0:PF/C

*RKA0:PG/C

*RKA0:PH/C

*RKA0:PL/C

*$

$END

3-27

CHAPTER 4

MODIFYING THE SYSGEN

4.1  OVERVIEW

This chapter describes how you make changes to the SYSGEN so  that  it

can  more  precisely  meet  your  needs.   Although you may change the

sources to the SYSGEN.SV program, DIGITAL does not recommend that  you

proceed  in  that  manner.   Instead, DIGITAL recommends that you make

changes in SCRIPT.MA.   (SCRIPT.MA  is  the  MACREL  source  file  for

SCRIPT.SV, the file upon which SYSGEN operates.)

The reason you modify the script is to reduce the potential for errors

that can occur when a person unfamiliar with  the  SYSGEN  creates  an

RTS/8 system.

4.2  THE STRUCTURE OF SCRIPT.MA

SCRIPT.MA contains the following sections.

1.  DATA FORMAT INFORMATION--comments that describe the SCRIPT.MA

record format.

2.  RECORD DIRECTORY--a table  that  SYSGEN  uses  when  locating

records.

3.  SCRIPT RECORDS--the script that  you  see  when  creating  an

RTS/8  system  as  well as conditional statements that SYSGEN

uses when branching over questions.

4.  PARAMETER RECORDS--the equates from which the SYSGEN  creates

the Parameter file.

5.  DEFAULT PARAMETERS--the  parameters  necessary  to  create  a

default RTS/8 system.

6.  SYSTEM PARAMETERS--a list of RTS/8 system equates  and  other

related information.

7.  TTY SKELETON CONTROL FILES--the framework  that  SYSGEN  uses

when creating TTY tasks.

8.  FLOPPY CONTROL SKELETON FILES--the framework that SYSGEN uses

when creating FLOPPY tasks.

9.  ERROR RECORDS--the records SYSGEN references when it  outputs

an error message.

4-1

MODIFYING THE SYSGEN

10.  PASS2  MESSAGES--the  messages  SYSGEN  prints  when  it   is

creating the output files.

11.  PASS2 MACREL BATCH RECORDS--the skeletal forms of the  MACREL

BATCH file.

12.  PASS2 LINKER BATCH MESSAGES--the skeletal forms of the LINKER

BATCH file.

13.  LIMIT CHECKING RECORDS--the numbers  that  the  SYSGEN  limit

checking  routine  uses  when  verifying  numerical  entries.

The following sections discuss these SCRIPT.MA elements.

4.2.1  Record Directory

This section describes the entry format for incorporating records into

the  SCRIPT.MA  record  directory.   (Most of the records in SCRIPT.MA

have a three-statement entry in the record directory.)

----------------------------------------------------------------------

| The format of a record directory entry is:			  |

|									  |

|      N								  |

|      MN								  |

|      EDF     MN							  |

|									  |

| where:								  |

|       N       is the record number.				  |

|       MN      is the corresponding message number.		  |

----------------------------------------------------------------------

The order of the entries in the record directory  corresponds  to  the

order  in  which the SYSGEN asks questions or accesses records.  It is

also the order in which SYSGEN accesses records when  it  creates  the

PASS2  output  files.   (This order does not necessarily correspond to

the order in which records exist in the script section of SCRIPT.MA.)

Each record in SCRIPT.MA has a unique, fixed tag.  This  tag,  besides

identifying  the  record and allowing SYSGEN to access the record, has

the following meanings to the SYSGEN program.

RECORD NUMBER	= script record

RECORD NUMBER + 2000 = parameter   record   (corresponding   to    the

question)

RECORD NUMBER + 3000 = limit record (limits on answers to questions if

any)

The reason this relationship  exists  between  record  numbers  is  to

facilitate  the  addition of new entries.  For example, if you were to

add a question with the number 521,  you  would  add  the  appropriate

parameter  at  2521,  and  the  limit  record  at  3521.  Because this

4-2

MODIFYING THE SYSGEN

relationship exists, the SYSGEN requires  that every  record  you  add

follow this scheme.

The above record numbers  are  the  ones  you  use  when  manipulating

SCRIPT.MA.   Other  record  numbers  also  have meaning to the system.

However, the  relationship  between  the  script  question  and  other

records are not as distinct.  Ordinarily, you should not add or delete

information that has the following numbers.

1000 = error messages and special tags

4000 = reserved for future use

5000 = PASS2 messages and control file records

6000 = LINKER parameter (used for building LINKER batch file)

7000 = MACREL parameter (used for building MACREL batch file)

The SYSGEN generally uses Records n777 where  n  equals  1,2,...,7  to

delimit  the  different script sections and you should not use them as

record numbers.

4.2.2  Script Records

The Script Records section contains  all  questions  that  the  SYSGEN

prints.   These questions are in approximately the order in which they

appear during a SYSGEN.  However, it is not important where you  place

them  within  this section because the SYSGEN references them by their

tag.

----------------------------------------------------------------------

| The general format of a script question is:			  |

|									  |

| MN,     TEXT    "WYYY SCRIPT [X] (z)"				  |

|									  |

| where:								  |

|       N      is the record number.				  |

|       MN     is the corresponding message number			  |

|       W      is the record type and may be			  |

|			 A:  ASCII response				  |

|			 D:  a decimal response				  |

|			 N:  a null record (ignored)			  |

|			 O:  an octal response				  |

|			 Y:  a Yes/No response				  |

|       YYY    is a user question number (initially equal to N;      |

|		    overwritten by a dynamically generated number when    |

|		    printed on a terminal).				  |

|       X      is the expected answer type.			  |

|       Z      is the default answer.				  |

|       SCRIPT is the text of the question.			  |

----------------------------------------------------------------------

4-3

MODIFYING THE SYSGEN

The above general format is for one line questions.  If  the  question

requires more than one line, the format is

MN,     TEXT    "WYYY SCRIPT"<215><212>

TEXT    "SCRIPT"<215><212>

.

.

TEXT    "SCRIPT [X] (z)"

In this format, the <215><212> string tells MACREL to print a carriage

return/line  feed  when  it  prints a line of text.  If you omit these

characters, SYSGEN will not print any lines of  the  record  following

the  omission.   Also,  if  the  last  line  of  a  record  contains a

<215><212>, the SYSGEN cannot find the  end  of  the  text.   If  this

occurs,  the  next  record  becomes  part  of  the text of the current

record.

The maximum length of any script record text is 384 characters.

When you are creating a record, you only have to use the  record  type

indicator  to tell the SYSGEN which of its internal routines it should

use when processing an answer.  For example, if you  create  a  record

that begins

M521,   TEXT    "0521...

The SYSGEN uses its octal routines.  Likewise

M521,   TEXT    "A521...

tells the SYSGEN to process an ASCII answer.

You must be careful that the number YYY that you select is  unique  in

the  system.   You select this number, which must be in the range 0 to

577, from a cross-reference listing of the  assembled  SCRIPT.MA.   If

the  number is not in the cross-reference listing, it is available for

your use. (This number is the octal tag that MACREL references when it

assembles  SCRIPT.   It is also the means by which the SYSGEN accesses

individual records.) Generally,  the  numbers  from  360  to  577  are

available.   (Other  record numbers can become available if you delete

existing script records.   See  Section  4.3.1  for  a  discussion  of

deleting records.)

In addition to the record types described above, several other  record

types also exist.  A second major record type is the remark or R type.

A remark is the text that the SYSGEN prints when you type  a  question

mark as the answer to a question.

The SYSGEN requires that the record following a question be a  remark.

(Following  means  that it is the next record pointed to in the record

4-4

MODIFYING THE SYSGEN

directory.  It does not mean the next sequential record in the  script

section.) The SYSGEN automatically skips over the record unless a user

types a question mark in response to a SYSGEN question.  When the user

types a question mark, the SYSGEN accesses this record.

----------------------------------------------------------------------

| The format of an R record is:					  |

|									  |

| MN,   TEXT    "RYYY SCRIPT"<215><212>				  |

|       TEXT    "SCRIPT"<215><212>					  |

|       .								  |

|       .								  |

|       .								  |

|       TEXT    "SCRIPT"						  |

|									  |

| where:								  |

|       R      indicates that the record is a remark.		  |

|       SCRIPT is the text of the remark.				  |

----------------------------------------------------------------------

A remark can contain up to 384 (decimal) characters.  Note that  every

line  except  the  last  in  a  record  contains  the character string

<215><212>.  This string tells MACREL to type a  carriage  return/line

feed  at that point.  If you omit this string on an intermediate line,

the SYSGEN does not print all lines past the omission.  If you include

a <215><212> on the last line of a record's script, the SYSGEN assumes

(erroneously) that the next record is also part of the text.

For example, if you add a question that asks if the SYSGEN user wishes

to  include  a  task  named  MYOWN, you might add the following to the

script:

M400, TEXT "Y400 DO YOU WANT TASK 'MYOWN'? [Y/N] (Y):"

As  stated  above,  SYSGEN  requires  that  all   questions   have   a

corresponding remark.  The remark to this question might be:

M401,   TEXT    "R401 DO NOT INCLUDE THIS TASK IN THE"<215><212>

"SYSTEM UNLESS YOU USE DECTAPES."

If the SYSGEN should print this question before the question that asks

if  the user wants the LINCtape driver, you should physically enter it

there.  (Entering questions and remarks in the  order  in  which  they

appear  is  good  programming practice as it aids debugging.  However,

the SYSGEN does not require you to  enter  questions  and  remarks  in

order.)  You  then  enter the record number into the record directory.

The segment of the record directory would look as follows:

4-5

MODIFYING THE SYSGEN

400	     /ADDED QUESTION

M400

EDF    M400

401	     /ADDED REMARK

M401

EDF    M401

133	     /LINCTAPE QUESTION

M133

EDF    M133

(See the discussion of the Parameter File  for  information  regarding

how the SYSGEN records the answer to a question.)

The only complication that can occur when adding questions is that the

SYSGEN  branches  over  questions.   This  means  that if you insert a

question in a section of SCRIPT that the SYSGEN branches over, it  may

not  print  the  questions  you  add to the script.  (See below, for a

discussion of conditional records.) Section 4.3.2 presents  the  exact

procedure that you should follow in inserting questions.

Always include the default answer, enclosed within parentheses,  in  a

script  question.   Omitting  the  default  answer is a fatal run-time

error.

When  you  select  the  default  answer,  the  SYSGEN   examines   the

information  between  the  parentheses.  It then uses that information

as the answer to the question.  If you wish to change the  default  of

any  question,  you  need  to  change only the information between the

parentheses.  For example, if, when  creating  an  RTS/8  system,  you

normally want assembly listings, you would change the question

M333,   TEXT    "Y333 DO YOU WANT ASSEMBLY LISTINGS? [Y/N] (N):"

|

|

to								 |

|

v

M333,   TEXT    "Y333 DO YOU WANT ASSEMBLY LISTINGS? [Y/N] (Y):"

You do not have to include the SYSGEN answer  type  prompts  that  are

enclosed  within  square  brackets.  However, it is good documentation

practice to include them.

When you are entering questions, you may find it desirable  to  branch

around  questions  that  are  irrelevant because of a previous answer.

Thus, you may include  conditional  records  that  operate  upon  your

previous  answers  to  A, O, D, or Y (alphanumeric, octal, decimal, or

yes/no, respectively) records.

4-6

MODIFYING THE SYSGEN

----------------------------------------------------------------------

| The format for conditionals is:					  |

|									  |

| MN,   TEXT    "IA,N1,N2,ARG"					  |

|									  |

| where:								  |

|       I      designates the record as a conditional		  |

|       A      is the type of conditional such that		  |

|			 Z:  answer equals zero				  |

|			 P:  answer not equal to zero			  |

|			 E:  answer equals ARG				  |

|			 N:  answer not equal to ARG			  |

|			 G:  answer greater than ARG			  |

|			 L:  answer less than ARG			  |

|			 S:  answer true,  set  no  source  (there  is  a |

|			     priority assignment but no source code)      |

|       N1     is the record whose answer is being tested		  |

|       N2     is the record number to which the program will branch |

|		    if the conditional test is true.			  |

|       ARG    is any octal number (from  0-7777)  and  is  required |

|		    after E, N, G, and L				  |

----------------------------------------------------------------------

In this format, I is the record type and A is the subtype.

If N1 equals N2 the program branches to the  previous  record  (normal

branching is toward the end of the script table).

A yes/no answer has a value of 1 and 0 respectively.

The SYSGEN allows you to  use  record  numbers  600  through  776  for

conditionals  and  remarks  because  these  record  types do not write

entries into SYSGEN tables.  (Note that the examples in  this  chapter

do  not  follow  this convention.  This is because the SYSGEN does not

require you to use  the  600  through  776  numbers  for  remarks  and

conditionals.)

If you enter a question with a 600  through  677  record  number,  the

SYSGEN  executes  the  question  correctly.  However, DIGITAL reserves

these numbers for future expansion;  if you use them, future  releases

of the SYSGEN may require you to redo your script.

Record numbers 700 through 777 are reserved for listing  switches  and

control  file  entries.   The  SYSGEN uses these numbers to store your

answers to TTY and floppy questions.  If you enter questions which use

these  numbers  and  your target system uses more than one terminal or

has floppies, you will be using the same  numbers  for  two  different

entries.   If  this  occurs,  the  RTS/8  system  you  create executes

improperly.

Conditionals and remarks can use these numbers  because  they  do  not

store answers.

4-7

MODIFYING THE SYSGEN

In the example presented above, the remark suggested  that  you  would

not  include  the  task  MYOWN  in  the  system  if you did not have a

DECtape.  By using a conditional, you could direct the SYSGEN to  skip

this  question  if  you  answered  NO  to  the  DECtape question.  The

conditional you would add, after checking the cross-reference  listing

for an available number, might be

I402,   TEXT    "IZ133,134"     /133 IS DECTAPE, 134 IS LINCTAPE

It is also  permissible  to  branch  to  a  second  conditional.   For

example,  if you also did not want to ask the LINCtape question if the

target machine was not a PDP 12 (as is done in the script)  you  could

instead enter:

I402,   TEXT    "IZ133,345"

Statement 345 is

I345,   TEXT    "IZ044,134"     /044 REFERS TO PDP-12 WHICH IS

/THE MACHINE THAT USES LINCTAPE

The following is how  the  record  directory  would  look  after  this

change.

132	     /THIS IS THE DECTAPE QUESTION

M132

EDF     M132

073	     /THIS IS THE DECTAPE REMARK

M073

EDF     M073

402	     /THIS IS CONDITIONAL BRANCH

I402	     /OVER ADDED QUESTION

EDF     I402

400	     /THIS IS ADDED TASK QUESTION

M400

EDF     M400

401	     /THIS IS ADDED TASK REMARK

M401

EDF     M401

345	     /THIS IS CONDITIONAL AROUND

I345	     /LINCTAPE QUESTION

EDF     I345

4-8

MODIFYING THE SYSGEN

You must place a three-word entry into the record directory  for  each

record you incorporate into SCRIPT.MA.

Because the SYSGEN can branch over records, you must insure  that  you

do  not  place  your  questions where the SYSGEN will access them only

when a user specifies a certain task or  combination  of  tasks.   The

following are three places where this problem does not arise:

o  Before all DIGITAL single statement task questions

o  After all DIGITAL single statement task questions

o  Before the question asking if you wish to include user tasks

If you place records at other positions, you may inadvertently  branch

over your question unless you change branch targets.

------------------- NOTE -------------------

|					     |

| One  other  record   type   exists:    T |

| records.   These T records direct SYSGEN |

| to  execute   special   DIGITAL-supplied |

| processing  subroutines  at  that point. |

--------------------------------------------

Removing questions that  you  do  not  need  is  simpler  than  adding

questions.   Because the SYSGEN uses only records listed in the record

directory,  you  can  comment  out  the  question's  record  directory

entries.  (You comment out a line by placing a slash before the line.)

There is no need even to remove the question or comment it out in  the

script.   If  a record is not in the record directory, the SYSGEN will

not print it.  (DIGITAL recommends that you  leave  questions  in  the

script  rather  than remove them in case your requirements change.) In

addition, if you remove a question, you must also  remove  the  remark

associated with the question.

You must perform one other action when you delete  questions.   It  is

possible  that  a conditional record uses the record you are deleting.

To locate these conditional records, use a  text  editor  with  search

capabilities (such as TECO or EDIT) to look for other instances of the

record number.  The conditional record can use a record number in  two

ways.

1.  If the record number indicates an answer that the SYSGEN must

compare to the branch type, you should remove the conditional

from the system.  In most cases,  if  the  conditional  skips

over a record, you probably do not need the record or records

being skipped over.

2.  If the record indicates the target of a  conditional  branch,

you  must  change the target.  Normally, the target should be

the next sequential record in the record directory (excluding

remarks).

4-9

MODIFYING THE SYSGEN

4.2.3  Parameter Records

The configuration, system tasks,  multiple  parameters  for  a  single

task, and symbol table equates reside in the Parameter Records section

of SCRIPT.MA.

----------------------------------------------------------------------

| Each statement in this section is of the form:			  |

|									  |

| M2N,  TEXT    "EQUATE"						  |

|									  |

| where:								  |

|       N      is the message number.				  |

|       EQUATE is the symbol that MACREL uses  when  assembling  the |

|		    RTS/8 system.					  |

----------------------------------------------------------------------

This section of SCRIPT.MA contains the following subsections.

1.  Hardware configuration parameters

2.  System tasks in priority order

3.  Multiple parameter equates

4.  Default parameters

5.  System parameters

6.  Task and nonresident macros

The numbers in this section and the numbers  you  use  in  the  script

section  correspond  directly;   that  is,  the message number in this

section is the message number in the script + 2000.  For example,  the

task  equate  you  would add for the question in the previous examples

would be

M2400,  TEXT    "MYOWN="

(You would place this equate into Section 2 of  the  parameter  file.)

When the SYSGEN processes Section 2 of the parameter file, it sets the

equates and writes it to the output file.  If the equate  is  not  set

(that  is,  the answer is false), the SYSGEN ignores the record.  This

number is the task's priority.

The order in which you enter a task's equate in this  section  of  the

parameter  file  helps  determine  a  task's  priority.   (The  SYSGEN

determines a system task's priority by its initial  placement  in  the

parameter file.)

4-10

MODIFYING THE SYSGEN

If you change the task's priority (in response to a SYSGEN  question),

the placement of this equate is in the task order and not the order in

which you place it in the parameter file.

When you add a question such as the one described above to the SYSGEN,

the  task  is  now a system task.  In other words, the SYSGEN formally

differentiates user tasks and system tasks.  That is, a user task is a

task  that becomes part of the RTS/8 system because of a user response

to the question that asks which additional tasks you wish  to  include

in  the  system.   A system task, on the other hand, is a task that is

part of the configuration of the system and the SYSGEN asks  questions

about the particular task.

When you receive the SYSGEN, it included only  DIGITAL  system  tasks.

However,  when  you add questions that ask about other tasks, you have

added system tasks.

In  the  case  of  the  configuration  parameters  and  the   multiple

parameters,  the  SYSGEN writes an equate to the output parameter file

only if it places information after the equals sign.  If it  does  not

place information after the equals sign, it ignores the equate.

Section 5 of the parameter  list  consists  of  general  RTS/8  system

equates.   These equates become part of every RTS/8 system unless they

are within a conditional assembly.  For example, the  RTS/8  Executive

Request  mnemonics  are always part of the RTS/8 system.  However, the

SYSGEN includes the UDC equates only if a user selects the UDC.

You do not need to include record numbers for equates you add in  this

section  because  the SYSGEN automatically uses all records from label

P1001 through label D1001.

The examples in the previous sections illustrated how you add a single

task  to  the  system.   In these examples, you only needed to add the

equate MYOWN= to  the  parameter  file  task  section  and  place  the

equate's  tag in the record directory.  In some cases, you may wish to

add equates that direct MACREL to  assemble  portions  of  the  source

code.    Section  4.3.2  illustrates  how  you  add  such  equates  to

SCRIPT.MA.

Section 4 of the parameter list is a  conditional  assembly  that  the

SYSGEN  uses  for  a  default  system.   You  may make changes in this

section to alter the default system.  Specifically, you may delete the

tasks listed here and/or add DIGITAL and user tasks.  If the tasks you

create have multiple  equates,  you  may  also  add  them  within  the

conditional  assembly.   You  do not need to include record numbers in

this section because the SYSGEN automatically includes everything from

label M1000 to label P1001.

If you change the default, you must also alter the default MACREL  and

LINKER BATCH files so that the SYSGEN properly creates the new default

system.  The default MACREL file begins at record 7004.   The  default

LINKER  file  begins  at  record 6006.  Section 4.3.3 is an example of

changing this default.

4-11

MODIFYING THE SYSGEN

The last part of this section consists of the sources for the TASK and

NONRES  macros  that all tasks use.  You should not alter these macros

in any way.

4.2.4  Skeleton Records

The next two sections consists of  the  skeleton  files  for  the  TTY

drivers  and floppy controllers.  You should not make modifications to

these files.  If you do, the MACREL assembler will create  tasks  that

execute improperly.

4.2.5  Error Message Records

The SYSGEN references all error messages by their record number.   The

record  number  preceding  the  error  message is not coordinated with

specific questions.  Instead, SYSGEN routines return the error message

record number.  Other SYSGEN routines then locate the record number in

the error directory.  Unless you incorporate your  own  error-handling

routines, you should not make alterations in this section.

4.2.6  LINK and MACREL BATCH Records

These sections are each divided  into  two  parts.   The  first  is  a

skeleton  for  default  systems.   If you change the default, you must

change the default LINK and MACREL BATCH  records.   You  change  this

section  only to explicitly name tasks that you wish to include in the

default or delete a task that you no longer wish to  be  part  of  the

default.  See Section 4.3.3 for more information.

The second is a more generalized  skeleton  for  non-default  systems.

You must not make modifications in these records because the number of

characters in each record is important.  The SYSGEN creates the  BATCH

file  by  substituting characters into the skeletons.  It also assumes

that certain characters exist in certain places in the  skeleton.   If

you  change  the  displacement into the skeleton, SYSGEN creates BATCH

files that execute indeterminately.

The only user modification that is normally feasible in these sections

is  adding  additional  switches.  If you add switches, you must place

them at the end of the record.

4.2.7  Limit Records

Every question the SYSGEN asks that requires a numerical  answer  must

have  a limit record.  The SYSGEN checks this record to insure that an

answer is neither less than nor greater than certain values.

4-12

MODIFYING THE SYSGEN

----------------------------------------------------------------------

|									  |

| The format for a limit record is:				  |

|									  |

| M3N,  TEXT    "L3N LOW,HIGH"					  |

|									  |

| where:								  |

|       M3N    is the record tag.					  |

|       N      is the record number.				  |

|       L      is the designator for a limit record.		  |

|       LOW    is the lowest value that the  question's  answer  can |

|		    be.							  |

|       HIGH   is the highest value that the question's  answer  can |

|		    be.							  |

----------------------------------------------------------------------

In the limit records, you type the limits in  the  same  base  as  the

answer.   For  example, a decimal question has the limits expressed in

decimal and an octal question in octal.  The SYSGEN automatically uses

an  octal  or  decimal  processing  routine;  the record type (D or O)

tells SYSGEN which routines to use.

You must include the limit record's  record  number  into  the  record

directory.

If you wish to change  the  limits  on  any  question  for  a  DIGITAL

supplied task, you can alter the number in this section.

See Section 4.3.2 for examples of incorporating limit records.

4.3  EXAMPLES

This section demonstrates four possible changes you may wish  to  make

in the SYSGEN.  The changes are:

o  Deleting questions

o  Adding questions

o  Changing the default

o  Adding additional TTY handlers

4.3.1  Deleting Questions

Because DIGITAL must provide a system  generation  procedure  that  is

useful  to a wide range of users, the SYSGEN asks questions concerning

a large number of devices.  This  example  shows  how  to  change  the

SYSGEN  so  that  it does not ask questions about the following tasks:

power fail, clock, UDC, RF08 disk,  DF32  disk,  RK8  disk,  LINCtape,

4-13

MODIFYING THE SYSGEN

DECtape,  and  cassettes.   It  also shows how to eliminate the PDP 12

question.

To eliminate these questions, look up the record number in the  script

portion  of  the  directory.   The  following list presents the record

numbers and the associated system remark.

Question	      Remark

Number	      Number			      ______	      ______

Power Fail		074		073

Clock		075		073

UDC		125		073

RF08		126		073

DF32		127		073

RK8		130		073

DECtape		132		073

LINCtape		133		073

Cassette		150		073

Cass. File Sup.	151		073

Test		160		073

Exit		161		073

PDP 12		044		042

The first action you take is to  comment  out  these  records  in  the

record  directory.  The second step is to use the question number in a

TECO or EDIT search either to eliminate or  change  conditionals  that

may  use  the  answers  to  these  deleted  questions.   The following

conditionals were found.

604

705

713

720

727

736

740

741

742

772

Because this system does not include the Power Fail Task, it  is  also

no  longer  necessary  to  ask  if  the  target  system has power fail

hardware. The power fail question is number  061  and  its  remark  is

062.   When  searching  through  the script for related numbers, it is

noted that you must remove all clock and UDC questions.  These records

are

4-14

MODIFYING THE SYSGEN

Clock	     216

217     220

221     222

346     347

223     224

350     351

UDC	     225

226     227

230     231

232     231

233     231

234     235

236     235

237     235

Other records to omit are

MCR	     206     207

PDP 12	     343

When these changes were made in SCRIPT.MA, these record  numbers  were

commented  out in the record directory.  The questions themselves were

left in the script.

The following is an example of  portions  of  the  dialogue  that  the

SYSGEN now prints.

023 DO YOU WANT ANY OF THE DEC- PROVIDED

SYSTEM TASKS IN YOUR RTS/8 SYSTEM? [Y/N] (Y):

EACH DEC TASK WILL BE:  LISTED ONE AT A

TIME.  IF YOU WISH TO INCLUDE THE TASK IN YOUR

SYSTEM, TYPE Y.  IF NOT, TYPE N.  IF THE

SYSTEM PROGRAM REQUIRES MORE INFORMATION

ABOUT A TASK, IT WILL ASK THE QUESTIONS

IN ANOTHER SECTION.

(DOC NO 073)

024 RK8E DISK DRIVER? [Y/N] (N):

025  RL01 DISK DRIVER? [Y/N] (N):

026 FIRST FLOPPY CONTROLLER?  [Y/N] (N):Y

027 SECOND FLOPPY CONTROLLER?  [Y/N] (N):

4-15

MODIFYING THE SYSGEN

028 LINE PRINTER DRIVER?  [Y/N] (N):

029 TELETYPE DRIVER?  [Y/N] (Y):

030 EMERGENCY BREAKTHROUGH TASK? [Y/N] (N):

031 SWAPPER TASK? [Y/N]  (N):

032 OS/8 FILE SUPPORT TASK?  [Y/N] (N):

033 MONITOR CONSOLE ROUTINE?  [Y/N] (Y):

034 OS/8 SUPPORT TASK?  [Y/N] (N):

THIS SECTION PRESENTS THE PARAMETERS

REQUIRED TO SPECIFY THE MCR TASK

WHICH YOU HAVE SELECTED.

(DOC NO 205)

035 DO YOU WANT THE MCR SYSTEM STATUS

REPORT CAPABILITY?  [Y/N] (Y):

036 WHICH TERMINAL WILL THE MCR TASK

USE?  [A] (TTY):

This example shows a partial listing of a system generation.  In  this

listing,  you  should  compare  the  tasks that the SYSGEN prints with

those that the SYSGEN prints if you  had  not  removed  the  question.

(See Chapter 3 for a listing of the order in which the SYSGEN normally

prints questions.)

When deciding whether or not to remove questions, you must  weigh  the

convenience  of  leaving  the SYSGEN as DIGITAL distributes it against

the possible errors that can occur if a user inadvertently includes a

task that should not be part of the RTS/8 system.

In general, the SYSGEN dialogue is sufficiently fast that the  time  a

user  saves  by  seeing fewer questions should not be a factor in your

decision.  The only factor should be a potential for fewer errors when

creating systems.

4-16

MODIFYING THE SYSGEN

4.3.2  Adding Questions

The preceding example demonstrated how you delete  questions.   Before

you  add  questions, you should ask yourself a similar question;  that

is, do you believe that adding  questions  to  incorporate  new  tasks

improves  the  SYSGEN  process? It is usually simpler to supply a task

name in response to a SYSGEN question than to add questions   so  that

the SYSGEN explicitly asks whether a user wishes to include a task.

When adding questions you must do the following:

1.  Write the question and the associated remark.

2.  Consult a cross-reference listing of SCRIPT.MA  to  determine

which record numbers are available.

3.  Assign the record numbers to specific records.

4.  Write the conditional records (if any) that the  question  or

questions require.

5.  Write  the  task  equates  and  task  parameters  that   your

insertion requires.  The record number you use is the same as

you used in 3) except you preface it with a 2.

6.  Write the limit records.  The record number is  the  same  as

you  used in 3) except that you add 1000 to the number.  Note

that only octal and decimal records have limit records.

7.  Determine where the question or questions  should  appear  in

the SYSGEN.

8.  Examine the record directory at the point where you  wish  to

insert   the   records.    Carefully   check to  insure  that

conditional records, prefaced with  the  letter  "I"  in  the

directory,  do  not  branch  over  the  question.  (Using the

letter "I"  is  a  convention  that  allows  you  to  quickly

identify  branch  records.   It  is  not mandatory to do so.)

Because the SYSGEN branches over questions it does  not  need

to  ask,  you  must  be careful that you do not branch over a

question you add because of an answer to a question  that  is

unimportant to your system.

9.  Insert the record numbers  in the  record  directory.   These

insertions  occur in three places:  where the question should

be asked, where the parameter file record exists,  and  where

the  limit  record section exists.  If you omit the parameter

or limit records, the SYSGEN may detect a fatal error.  If it

does,  it  prints  an  error  message and returns to the OS/8

Monitor.

10.  Type the records themselves.  It is usually wisest  to  place

them where they logically belong due to normal question flow.

However, the SYSGEN only requires that you place them in  the

4-17

MODIFYING THE SYSGEN

proper  section of SCRIPT because it references them by their

tag, not their physical placement.

11.  Use MACREL to create the .RB image.

12.  Use LINK to create the .SV image.  The name you give to  this

file must be SCRIPT.SV.  Your command string should be of the

form

.R LINK

*SCRIPT[,filespec]<SCRIPT/M/K=37$

where:

filespec is an optional listing device or file.

Carefully examine the load map against a  load  map  for  the

most recent version of SCRIPT.  Make sure that your questions

do not  cause  an  overflow  in  one  of  the  DSECTS.   (The

following  example  includes  a  listing  of the load map for

SCRIPT.SV as DIGITAL distributes it and  a  second  load  map

for the version of SCRIPT created for the following example.)

13.  You may now run using the new script in place of the old.

14.  Examine the Parameter, MACREL, and LINK files  built  by  the

new SYSGEN to make sure that they are correct.

The following example illustrates  the  insertion  of  a  sequence  of

questions.   For  the  purposes of this example, it was assumed that a

user created a transaction processing task.  The function of this task

is to receive messages from terminals in the system, process them in a

way that is not material to this example, and add the information to a

data base.  Optional parts of the task specify where information is to

be written in case of overflow, how often  the  data  base  should  be

backed up, and other similar information.

A script was written and appropriate branch records included.

After looking in a cross-reference listing, it was found that all  400

record numbers were available.

.DSECT SCRT05

FIELD 5

M400,  TEXT   "Y400 DO YOU WANT THE TRANSACTION PROCESSING"<215><212>

TEXT   "TASK? [Y/N] (Y):"

M401,  TEXT   "R401 YOU INCLUDE THIS TASK IF YOU WANT REAL-"<2l5><2l2>

TEXT   "TIME UPDATING OF YOUR DATA BASE."

I402,  TEXT   "IZ400,735"	     /GO TO MCR IF NO TO M400

4-18

MODIFYING THE SYSGEN

M403,  TEXT   "D403 HOW MANY TERMINALS WILL BE USING THE"<215><212>

TEXT   "TRANSACTION TASK? [D] (2):"

M404,  TEXT   "R404 THE ANSWER YOU SPECIFY HERE SHOULD BE"<215><212>

TEXT   "LESS THAN OR EQUAL TO THE NUMBER OF TERMINALS"<215><212>

TEXT   "YOU SPECIFY IN THE TELETYPE DRIVER QUESTION."

M405,  TEXT   "A405 WHAT IS THE I/O DEVICE WHICH THE TRANS-"<215><212>

TEXT   "ACTION TASK USES? [A] (RL0B):"

M406,  TEXT   "R406 THIS DEVICE SHOULD HAVE A LARGE AMOUNT OF"

<215><212>

TEXT   "STORAGE.  IN MOST CASES, A FLOPPY IS UNACCEP-"<215><212>

TEXT   "TABLE AND YOU SHOULD USE AN RK8E OR RL01."

M407,  TEXT   "Y407 IS THERE AN OVERFLOW DEVICE? [Y/N] (N):"

M410,  TEXT   "R410 IF THE SYSTEM WILL PROCESS MORE THAN 300"<215><212>

TEXT   "TRANSACTIONS AN HOUR, YOU SHOULD PROVIDE A "<215><212>

TEXT   "DEVICE SO THAT TRANSACTIONS ARE NOT LOST."

I411,  TEXT   "IZ407,414"	           /SKIP OVERFLOW DEV OUES

M412,  TEXT   "A412 WHAT IS THE OVERFLOW DEVICE? [A] (RL0C):"

M413,  TEXT   "R413 THE SPACE FOR OVERFLOW CAN BE MUCH SMALLER"

<215><2l2>

TEXT   "THAT THE PRIMARY DEVICE, YOU MAY USE A FLOPPY"<215><212>

TEXT   "FOR THE OVERFLOW."

M414,  TEXT   "Y414 DO YOU WANT TO BACKUP THE TRANSACTION STORAGE"

<215><212>

TEXT   "DEVICE? [Y/N] (Y):"

M415,  TEXT   "R415 YOU BACK UP THE DEVICE SO THAT IF THERE IS A"

<215><212>

TEXT   "SYSTEM FAILURE, YOU HAVE THE MAJORITY OF RECENT"

<2l5><212>

TEXT   "TRANSACTIONS ON FILE."

I416,  TEXT   "IZ414,735"	     /IF NO, GO TO MCR OUES

M417,  TEXT   "A417 WHAT IS THE BACKUP STORAGE DEVICE [A] (RL1A):"

M420,  TEXT   "R420 THIS DEVICE SHOULD HAVE THE SAME STORAGE"<215><212>

TEXT   "CAPACITY AS THE DEVICE UPON WHICH YOU STORE THE"

<215><212>

TEXT   "TRANSACTION FILE."

I421,  TEXT   "I7407,427"	           /IF NO OVERFLOW, GO TO HOW

/OFTEN DO YOU BACKUP

M422,  TEXT   "Y422 DO YOU WANT OVERFLOW STORAGE FOR THE BACKUP"

<215><212>

TEXT   "DEVICE? [Y/N] (Y):"

4-19

MODIFYING THE SYSGEN

M423,  TEXT   "R423 YOU SHOULD PROVIDE OVERFLOW STORAGE IF THE"

<215><2l2>

TEXT   "TRANSACTION DEVICE HAS OVERFLOW."

I424,  TEXT   "IZ422,427"	           /IF NO, GO TO HOW OFTEN BACK UP

/DO YOU BACKUP

M425,  TEXT   "A425 WHAT IS THE BACK UP OVERFLOW DEVICE? [A] (RL1C):"

M426,  TEXT   "R426 YOU SHOULD INCLUDE BACKUP STORAGE THAT HAS"

<2l5><212

TEXT   "CAPACTITY AS THE TRANSACTION BACKUP DEVICE."

M427,  TEXT   "Y427 DO YOU WANT TO BACK UP THE TRANSACTION DEVICE"

<215><212

TEXT   "AFTER A FIXED NUMBER OF TRANSACTIONS? [Y/N] (Y):"

M430,  TEXT   "R430 IF YOU BACK UP THE DEVICE AFTER A FIXED NUMBER"

<215><212>

TEXT   "OF TRANSACTIONS, THIS IS THE MAXIMUM NUMBER OF TRANS "

<215><212

TEXT   "THAT CAN BE LOST IF THERE IS A SYSTEM FAILURE."

I431,  TEXT   "IZ427,435"	           /GO TO BACK UP BY TIME

M432,  TEXT   "D432 AFTER HOW MANY TRANSACTIONS SHOULD THE BACKUP"

<215><212>

TEXT   "ROUTINE BEGIN WRITING THE INFORMATION ON THE TRANS-"

<215><212>

TEXT   "ACTION DEVICE TO AUXILLIARY STORAGE? [D] (50):"

M433,  TEXT   "R433 THE RANGE FOR THIS ANSWER IS 16 THROUGH 1023."

I434,  TEXT   "IZ427,440"	           /SKIP NEXT IF ANSWER TO PREVIOUS

M435,  TEXT   "R435 SINCE YOU DID NOT WANT BACKUP TO PROCEED AFTER"

<215><212>

TEXT   "FIXED NUMBER OF TRANSACTIONS, YOU MUST SPECIFY THE"

<215><2l2>

TEXT   "INTERVAL AT WHICH BACK UP WILL OCCUR."

M436,  TEXT   "D436 THE INTERVAL IS? [D] (600):"

M437,  TEXT   "R377 THE LIMITS ON YOUR ANSWER ARE 32 TO 1023,"

<215><212>

TEXT   "WHEN YOU CHOSE THIS OPTION, YOU ARE USUALLY"<215~<212>

TEXT   "ASKING FOR BACKUP TO OCCUR MORE OFTEN THAN IT"<215><212>

TEXT   "OCCURS BY THE NUMBER OF TRANSACTIONS."

M440,  TEXT   "Y440 DO YOU WANT TO STORE TRANSACTIONS THAT"<215><212>

TEXT   "OCCUR WHILE BACKING UP IS GOING ON? [Y/N] (N):"

4-20

MODIFYING THE SYSGEN

M441,  TEXT   "R441 THIS OPTION PROVIDES FOR MAXIMUM RECOVERY"

<215><212>

TEXT   "IF THERE IS A SYSTEM FAILURE.  HOWEVER, THIS"<215><212>

TEXT   "PROCEDURE WILL STEAL TIME AND THUS SLOW DOWN"<215><212>

TEXT   "RESPONSE TIME AT USER TERMINALS."

I442,  TEXT   "IZ440,735"

M443,  TEXT   "A443 WHAT IS THE DEVICE TO WHICH THESE TRANSACTIONS"

<215><212>

TEXT   "SHOULD BE WRITTEN? [A] (RL1B):"

M444,  TEXT   "R444 THIS SHOULD BE A VERY FAST I/O DEVICE TO MIN-"

<215><212>

TEXT   "THE TIME NECESSARY TO STORE THE INFORMATION."

The task and other equates were written.  The equates were:

M2403,  TEXT    "BACKUP="      /TASK EQUATES

M2405,  TEXT    "TRANS="

M2412,  TEXT    "TRNTTY="      /PARAMETERS TO

M2417,  TEXT    "TRNDEV="      /PASS TO SOURCE

M2425,  TEXT    "BKDEV="       /FILES

M2432,  TEXT    "BKTRNS="

M2436,  TEXT    "BKTIME="

M2443,  TEXT    "BKINTR="

The following limit records were written.

M3412,  TEXT    "L3412 001,005"

M3432,  TEXT    "L3432 016,1023"

M3436,  TEXT    "L3436 032,1023"

The point where this task was to be located was immediately before the

multiple MCR questions.

After  examining  SCRIPT.MA,  it  was   found   that   the   following

conditionals had to be changed.

I733,   TEXT    "IZ162,735"

I755,   TEXT    "IZ321,735"

The text was typed.

SCRIPT.MA was assembled and linked.  The following is the load map for

the DIGITAL-supplied SCRIPT.MA.

4-21

MODIFYING THE SYSGEN

OS/8   LINKER V 2A   LOAD MAP      16-OCT-78

SECTION  ADDR    SIZE  ENTRY    ADDR    ENTRY    ADDR

SCRT01   10000   3106

SCRT02   20000   7350

SCRT03   30000   7312

SCRT04   40000   6320

SCRT05   50000   4706

SCRT06   60000   7464

SCRT07   70000   5770

SCRT10  100000   2244

SCRT11  110000   0170

SCRT12  120000   0662

SCRT13  130000   0246

SCRT14  140000   0442

PROGRAM ENTRY:   00200

JOB STATUS WORD: 0000

The following is the new load map.  (Note that DSECT 5 was split  into

two parts, DSECT 5 and DSECT 15.)

.R LINK

*SCRIPT,TTY:<SCRP2/M/K=37

*$

OS/8   LINKER V 2A   LOAD MAP      16-OCT-78

SECTION  ADDR    SIZE  ENTRY    ADDR    ENTRY    ADDR

SCRT01   10000   3334

SCRT02   20000   7350

SCRT03   30000   7312

SCRT04   40000   6310

SCRT05   50000   3720

SCRT06   60000   7464

SCRT07   70000   6064

SCRT10  100000   2244

SCRT11  110000   0170

SCRT12  120000   0662

SCRT13  130000   0246

SCRT14  140000   0500

SCRT15  150000   4716

PROGRAM ENTRY:   00200

JOB STATUS WORD: 0000

4-22

MODIFYING THE SYSGEN

The SYSGEN was run.  The following text shows these questions as typed

on  a  terminal.   Note  the two displays.  The first checks out every

branch that the SYSGEN can take in a question.  Note  also  that  each

remark  is  printed.  DIGITAL recommends that when you add changes you

do them a segment at a time so that you can expeditiously verify  that

the modified script is working.

The second version is one pass through the dialogue  to  determine  if

that works as hoped for.

042 OS/8 SUPPORT TASK? [Y/N] (N):

043 NULL JOB? [Y/N] (N):

044 DO YOU WANT THE TRANSACTION PROCESSING

TASK? [Y/N] (Y):

045 HOW MANY TERMINALS WILL BE USING THE

TRANSACTION TASK? [D] (2):51

ERROR 202 NUMERICAL ANSWER NOT WITHIN SPECIFIED LIMITS

045 HOW MANY TERMINALS WILL BE USING THE

TRANSACTION TASK? [D] (2):?

THE ANSWER YOU SPECIFY HERE SHOULD BE

LESS THAN OR EQUAL TO THE NUMBER OF TERMINALS

YOU SPECIFY IN THE TELETYPE DRIVER QUESTION.

045 HOW MANY TERMINALS WILL BE USING THE

TRANSACTION TASK? [D] (2):#44

044 DO YOU WANT THE TRANSACTION PROCESSING

TASK? [Y/N] (Y):N

THIS SECTION PRESENTS THE PARAMETERS

REQUIRED TO SPECIFY THE MCR TASK

WHICH YOU HAVE SELECTED.

(DOC. NO 205)

045 DO YOU WANT THE MCR SYSTEM STATUS

REPORT CAPABILITY? [Y/N] (Y):#44

4-23

MODIFYING THE SYSGEN

044 DO YOU WANT THE TRANSACTION PROCESSING

TASK? [Y/N] (Y):?

YOU INCLUDE THIS TASK IF YOU WANT REAL-

TIME UPDATING OF YOUR DATA BASE.

044 DO YOU WANT THE TRANSACTION PROCESSING

TASK? [Y/N] (Y):

045 HOW MANY TERMINALS WILL BE USING THE

TRANSACTION TASK? [D] (2):4

THE ANSWER YOU SPECIFY HERE SHOULD BE

LESS THAN OR EQUAL TO THE NUMBER OF TERMINALS

YOU SPECIFY IN THE TELETYPE DRIVER QUESTION.

045 HOW MANY TERMINALS WILL BE USING THE

TRANSACTION TASK? [D] (2):4

046 WHAT IS THE I/O DEVICE WHICH THE TRANS-

ACTION TASK USES? [A] (RL0B):?

THIS DEVICE SHOULD HAVE A LARGE AMOUNT OF

STORAGE.  IN MOST CASES, A FLOPPY IS UNACCEP-

TABLE AND YOU SHOULD USE AN RK8E OR RL01.

046 WHAT IS THE I/0 DEVICE WHICH THE TRANS-

ACTION TASK USES? [A] (RL0B):

047 IS THERE AN OVERFLOW DEVICE? [Y/N] (N):N

048 DO YOU WANT TO BACKUP THE TRANSACTION STORAGE

DEVICE? [Y/N] (Y):#47

047 IS THERE AN OVERFLOW DEVICE? [Y/N] (N):?

IF THE SYSTEM WILL PROCESS MORE THAN 300

TRANSACTIONS AN HOUR, YOU SHOULD PROVIDE AN

DEVICE SO THAT TRANSACTIONS ARE NOT LOST.

047 IS THERE AN OVERFLOW DEVICE? [Y/N] (N):Y

048 WHAT IS THE OVERFLOW DEVICE? [A] (RL0C):?

4-24

MODIFYING THE SYSGEN

THE SPACE FOR OVERFLOW CAN BE MUCH SMALLER

THAN THE PRIMARY DEVICE.  YOU MAY USE A FLOPPY

FOR THE OVERFLOW.

048 WHAT IS THE OVERFLOW DEVICE? [A] (RL0C):

049 DO YOU WANT TO BACKUP THE TRANSACTION STORAGE

DEVICE? [Y/N] (Y):?

YOU BACK UP THE DEVICE SO THAT IF THERE IS A

SYSTEM FAILURE, YOU HAVE THE MAJORITY OF RECENT

TRANSACTIONS ON FILE.

049 DO YOU WANT TO BACKUP THE TRANSACTION STORAGE

DEVICE? [Y/N] (Y):N

THIS SECTION PRESENTS THE PARAMETERS

REQUIRED TO SPECIFY THE MCR TASK

WHICH YOU HAVE SPECIFIED.

(DOC. NO 205)

050 DO YOU WANT THE MCR SYSTEM STATUS

REPORT CAPABILITY? [Y/N] (Y):#49

049 DO YOU WANT TO BACKUP THE TRANSACTION STORAGE

DEVICE? [Y/N] (Y):N

050 WHAT IS THE BACKUP STORAGE DEVICE [A] (RL1A):?

THIS DEVICE SHOULD HAVE THE SAME STORAGE

CAPACITY AS THE DEVICE UPON WHICH YOU STORE THE

TRANSACTION FILE.

050 WHAT IS THE BACKUP STORAGE DEVICE [A] (RL1A):

051 DO YOU WANT OVERFLOW STORAGE FOR THE BACKUP

DEVICE? [Y/N] (Y):?

YOU SHOULD PROVIDE OVERFLOW STORAGE IF THE

TRANSACTION DEVICE HAS OVERFLOW.

051 DO YOU WANT OVERFLOW STORAGE FOR THE BACKUP

DEVICE? [Y/N] (Y):N

4-25

MODIFYING THE SYSGEN

052 DO YOU WANT TO BACKUP THE TRANSACTION DEVICE

AFTER A FIXED NUMBER OF TRANSACTIONS? [Y/N] (Y):#51

051 DO YOU WANT OVERFLOW STORAGE FOR THE BACKUP

DEVICE? [Y/N] (Y):

052 WHAT IS THE BACKUP OVERFLOW DEVICE? [A] (RL1C):?

YOU SHOULD INCLUDE BACKUP STORAGE THAT HAS

CAPACTITY AS THE TRANSACTION BACKUP DEVICE.

052 WHAT IS THE BACKUP OVERFLOW DEVICE? [A] (RL1C):

053 DO YOU WANT TO BACKUP THE TRANSACTION DEVICE

AFTER A FIXED NUMBER OF TRANSACTIONS? [Y/N] (Y):?

IF YOU BACKUP THE DEVICE AFTER A FIXED NUMBER

OF TRANSACTIONS, THIS IS THE MAXIMUM NUMBER OF TRANS-

THAT CAN BE LOST IF THERE IS A SYSTEM FAILURE.

053 DO YOU WANT TO BACKUP THE TRANSACTION DEVICE

AFTER A FIXED NUMBER OF TRANSACTIONS? [Y/N] (Y):

054 AFTER HOW MANY TRANSACTIONS SHOULD THE BACKUP

ROUTINE BEGIN WRITING THE INFORMATION ON THE TRANS-

ACTION DEVICE TO AUXILLIARY STORAGE? [D] (50):?

THE RANGE FOR THIS ANSWER IS 16 THROUGH 1023.

054 AFTER HOW MANY TRANSACTIONS SHOULD THE BACKUP

ROUTINE BEGIN WRITING THE INFORMATION ON THE TRANS-

ACTION DEVICE TO AUXILLIARY STORAGE? [D] (50):300

055 DO YOU WANT TO STORE TRANSACTIONS THAT

OCCUR WHILE BACKING UP IS GOING ON? [Y/N] (N):#53

053 DO YOU WANT TO BACKUP THE TRANSACTION DEVICE AFTER

FIXED NUMBER OF TRANSACTIONS? [Y/N] (Y):N

SINCE YOU DID NOT WANT BACKUP TO PROCEED AFTER

FIXED NUMBER OF TRANSACTIONS, YOU MUST SPECIFY THE

INTERVAL AT WHICH BACKUP WILL  OCCUR.

054 THE INTERVAL IS? [D] (600):?

4-26

MODIFYING THE SYSGEN

THE LIMIT ON YOUR ANSWER ARE 32 TO 1023.

WHEN YOU CHOSE THIS OPTION, YOU ARE USUALLY

ASKING FOR BACKUP TO OCCUR MORE OFTEN THAN IT

OCCURS BY THE NUMBER OF TRANSACTIONS.

054 THE INTERVAL IS? [D] (600):1000

055 DO YOU WANT TO STORE TRANSACTIONS THAT

OCCUR WHILE BACKING UP IS GOING ON? [Y/N] (N):?

THIS OPTION PROVIDES FOR MAXIMUM RECOVERY

IF THERE IS A SYSTEM FAILURE.  HOWEVER, THIS

PROCEDURE WILL STEAL TIME AND THUS SLOW DOWN

RESPONSE TIME AT USER TERMINALS.

055 DO YOU WANT TO STORE TRANSACTIONS THAT

OCCUR WHILE BACKING UP IS GOING ON? [Y/N] (N):N

THIS SECTION PRESENTS THE PARAMETERS

REQUIRED TO SPECIFY THE MCR TASK

WHICH YOU HAVE SELECTED

(DOC. NO 205)

056 DO YOU WANT THE MCR SYSTEM STATUS

REPORT CAPABILITY?  [Y/N] (Y):#55

054 DO YOU WANT THE OS/8 BACKGROUND TASK TO RUN IN

THE HIGHEST AVAILABLE FIELDS? [Y/N] (Y):

055 DO YOU WANT THE TRANSACTION PROCESSING

TASK? [Y/N] (Y):

056 HOW MANY TERMINALS WILL BE USING THE

TRANSACTION TASK? [D] (2):

057 WHAT IS THE I/O DEVICE WHICH THE

TRANSACTION TASK USES? [A] (RL0B):

058 IS THERE AN OVERFLOW DEVICE? [Y/N] (N):

059 DO YOU WANT TO BACKUP THE TRANSACTION STORAGE

DEVICE? [Y/N] (Y):

060 WHAT IS THE BACKUP STORAGE DEVICE [A] (RL1A):

4-27

MODIFYING THE SYSGEN

061 DO YOU WANT TO BACKUP THE TRANSACTION DEVICE

AFTER A FIXED NUMBER OF TRANSACTIONS? [Y/N] (Y):N

SINCE YOU DID NOT WANT BACKUP TO PROCEED AFTER

FIXED NUMBER OF TRANSACTIONS,  YOU MUST SPECIFY THE

INTERVAL AT WHICH BACKUP WILL OCCUR.

062 THE INTERVAL IS? [D] (600):1000

063 DO YOU WANT TO STORE TRANSACTIONS THAT

OCCUR WHILE BACKING UP IS GOING ON? [Y/N] (N):Y

064 WHAT IS THE DEVICE TO WHICH THESE TRANSACTIONS

SHOULD BE WRITTEN? [A] (RL1B):

THIS SECTION PRESENTS THE PARAMETERS

REQUIRED TO SPECIFY THE MCR TASK

WHICH YOU HAVE SELECTED.

(DOC. NO 205)

065 DO YOU WANT TIME-OF-DAY FACILITY IN

YOUR MCR TASK? [Y/N] (Y):

The last verification step is to check the parameter, MACREL, and LINK

files to insure that the sysgen is recording information properly.

------------------- NOTE -------------------

|					     |

| Because this transaction task  does  not |

| really exist, no attempt was made to run |

| the MACREL  and  LINK  files  so  as  to |

| actually create an RTS/8 system.	     |

--------------------------------------------

PWRF=001

CLOCK=002

DTA=003

CSA=004

UDC=005

BACKUP=006

TRANS=007

RL01=010

RF08=011

DF32=012

RK8=013

RK8E=014

RX8A=015

4-28

MODIFYING THE SYSGEN

RX8B=016

RX8C=017

RX8D=020

LPT=021

EMT=022

TTY=023

TTY1=024

TTY2=025

TTY3=026

SWAP=027

CSAF=030

OS8F=031

MCR=032

TEST=033

EXIT=034

BKT=035

OS8=036

NULL8A=037

NTASKS=037

PDP8A=1

EAE=1

EAESV=1

NFLDS=4.

RTFLD=0.

PWRFAL=1

KL8A=2.

KT8A=1

OSFLDS=2.

OSTTDV=04

OSKBDV=03

OSSYSD=RK8E

OSFILL=0.

BKTCDV=TTY

MCRCLK=1

MCRSYS=1

MCRCDV=TTY

SHERTZ=10.

CLKQLN=16.

RINGBF=12

NCNTR=1

NCNTC=2

NAD=2

FCTR=1

FCT=1

FAD=1

SYS=RK8E

KBDEV=3

TTDEV=4

VT50=1

SCOPE=1

FILL=0

WIDTH=80.

TAB=1

4-29

MODIFYING THE SYSGEN

EMTCDV=TTY

TTTIM=0.

TRNTTY=4.

TRNDEV=RL0B

BKDEV=RL1A

BKTIME=1000.

BKINTR=RL1B

KL8ADV=40

$JOB-MACREL TASK ASSEMBLY

$MSG MOUNT TASK SOURCES ON SYS:AND STRIKE ANY KEY TO CONTINUE

.MAC SYS:RTS8 <SYS:PDEFLT,SYS:RTS8

.MAC SYS:PWRF<SYS:PDEFLT,SYS:PWRF

.MAC SYS:CLOCK<SYS:PDEFLT,SYS:CLOCK

.MAC SYS:DTA<SYS:PDEFLT,SYS:DTA

.MAC SYS:CSA<SYS:PDEFLT,SYS:CSA

.MAC SYS:UDC<SYS:PDEFLT,SYS:UDC

.MAC SYS:BACKUP<SYS:PDEFLT,SYS:BACKUP

.MAC SYS:TRANS<SYS:PDEFLT,SYS:TRANS

.MAC SYS:RL01<SYS:PDEFLT,SYS:RL01

.MAC SYS:RF08<SYS:PDEFLT,SYS:RF08

.MAC SYS:DF32<SYS:PDEFLT,SYS:DF32

.MAC SYS:RK8<SYS:PDEFLT,SYS:RK8

.MAC SYS:RK8E<SYS:PDEFLT,SYS:RK8E

.MAC SYS:RX8A<SYS:PDEFLT,SYS:RX8A

.MAC SYS:RX8B<SYS:PDEFLT,SYS:RX8B

.MAC SYS:RX8C<SYS:PDEFLT,SYS:RX8C

.MAC SYS:RX8D<SYS:PDEFLT,SYS:RX8D

.MAC SYS:LPT<SYS:PDEFLT,SYS:LPT

.MAC SYS:TTY<SYS:PDEFLT,SYS:TTY

.MAC SYS:TTYl<SYS:PDEFLT,SYS:TTY1

.MAC SYS:TTY2<SYS:PDEFLT,SYS:TTY2

.MAC SYS:TTY3<SYS:PDEFLT,SYS:TTY3

.MAC SYS:SWAP<SYS:PDEFLT,SYS:SWAP

.MAC SYS:CSAF<SYS:PDEFLT,SYS:CSAF

.MAC SYS:OS8F<SYS:PDEFLT,SYS:OS8F

.MAC SYS:MCR<SYS:PDEFLT,SYS:MCR

.MAC SYS:TEST<SYS:PDEFLT,SYS:TEST

.MAC SYS:EXIT<SYS:PDEFLT,SYS:EXIT

.MAC SYS:OS8<SYS:PDEFLT,SYS:OS8

.MAC SYS:NULL8A<SYS:PDEFLT,SYS:NULL8A

.SU SYS:LDEFLT.BI/T

$END

$JOB RTS8 MODULE LINKAGE

.R LINK

$SYS:SDEFLT</K=02/M/X

*SYS:RTS8

*SYS:PWRF

*SYS:CLOCK

*SYS:DTA

*SYS:CSA

*SYS:UDC

*SYS:BACKUP

4-30

MODIFYING THE SYSGEN

*SYS:TRANS

*SYS:RL01

*SYS:RF08

*SYS:DF32

*SYS:RK8

*SYS:RK8E

*SYS:RX8A

*SYS:RX8B

*SYS:RX8C

*SYS:RX8D

*SYS:LPT

*SYS:TTY

*SYS:TTYl

*SYS:TTY2

*SYS:TTY3

*SYS:SWAP

*SYS:CSAF

*SYS:OS8F

*SYS:MCR

*SYS:TEST

*SYS:EXIT

*SYS:OS8

*SYS:NULL8A

*/O

*/O

*/O

*/O

*/O

*/O

*/O

*$

.RU SYS:SDEFLT

$END

You may achieve  many  of  the  same  results  more  simply  by  using

conditional  assembly  statements.   For example, you could ask if the

system will include the transaction task.  If the system will  include

this  task,  your  next  question could be for the user to specify the

kind of transaction task.  The answer could be a number that specifies

the  equates necessary for the transaction task.  By using conditional

assembly statements, you could check for each numerical value.   Using

this  procedure,  you could create the above transaction task with two

questions rather than the number used here.

4.3.3  Changing the Default

The RTS/8 default procedure allows a user to create a system with  the

minimum  amount   of effort.  When creating a default system, the user

has only to specify where the SYSGEN is to write files,  where  source

files  reside,  and  other similar information.  For a default system,

4-31

MODIFYING THE SYSGEN

the SYSGEN uses prespecified  equates  and  explicitly  listed  output

files.

The procedure for creating a new default system is as follows

1.  Change the default equate  listing  in  the  parameter  file.

This  listing  begins  at  record  number M1000.  The records

following this number do not have record  numbers.   Instead,

the  SYSGEN  writes  all  information  within the conditional

assembly statement to the output parameter  file.   You  may,

therefore, insert any equate you wish within the conditional.

Then, if a user selects the default, the  SYSGEN  writes  the

entire conditional to the output file.

Note that the equate NTASK (meaning "number  of  tasks")  has

been  set  to  10 (octal).  This means that you may insert an

additional two tasks before you change this number.   If  you

insert  more than two, you must make this number greater than

or equal to the number of tasks in the system.  Also, you may

need  to  increase  the NFLD (number of fields) equate if the

default is a large system.

You may, of course, delete any  of  the  equates  that  exist

within the conditional in the DIGITAL-supplied SCRIPT.MA.

2.  If the system has tasks  other  than  those  in  the  DIGITAL

default,  you must explicitly name these files for the MACREL

batch file.  Beginning at record number M7004 is  a  list  of

the tasks that are part of the default.

The records you add must have the same  physical  spacing  as

the  records  in  the  DIGITAL  default.   The "X" characters

represent null characters that the SYSGEN replaces with  your

answer  to  a  question.   (Each "X" represents one character

position that SYSGEN assumes  will  exist.)  Also,  you  must

enter  a  six-character task name.  A shorter name is a fatal

error.  If your task name contains less than six  characters,

you  must  pad  it with trailing spaces so that the task name

occupies six characters.  Each task name occurs  three  times

on each line.

If you add a new record to the default, you must include  the

record number in the record directory.

3.  You must create a LINK batch file in much the same way as the

MACREL batch file.  The records for this file begin at record

M6006.  All records you enter must have the same form as that

of  the  DIGITAL  default  (including  the  leading "X" place

holder for the device specification.) For these records,  the

name need not be six characters long.

If you add new records, you must include the record number in

the record directory.

4-32

MODIFYING THE SYSGEN

After you make the changes discussed in the preceding list, reassemble

and  relink SCRIPT.MA.  The file that LINK produces must have the name

SCRIPT.SV or the SYSGEN cannot run.

The following example is a small modification of the  DIGITAL  default

to  incorporate  the  transaction  task  that  was  the example in the

previous section.

The changes that were made to SCRIPT.MA are

1.  Remark number M017 was updated to reflect  the  new  default.

This  change  is not necessary but should be made so that the

remark best reflects the current SYSGEN.

2.  The TEST task was deleted from the default and TRANS (for the

transaction  task)  was  added.   (The  task  priority number

immediately following the task name  was  not  changed.   You

may,  of  course,  make any changes you like to this priority

number.)

3.  The equate TRNDEV (for the transaction device) was included.

4.  The TEST task was removed from the MACREL batch file and  the

transaction task substituted.

5.  The TEST task was removed from the LINK batch  file  and  the

transaction task substituted.

The following listings show a segment of the new SYSGEN dialogue  that

incorporates  the  changed  dialogue,  the  section  of  the SCRIPT.MA

parameter file that shows the new default conditional, the new  MACREL

batch file, and the new LINK batch file.

012 DO YOU WISH TO BUILD AN RTS/8 SYSTEM WITHOUT

SPECIFYING ANY PARAMETERS? [Y/N] (N):?

IF YOU ANSWER YES TO THIS QUESTION THE

RTS/8 BUILD PROGRAM GOES TO PASS 2 AND

CREATES A SYSTEM THAT INCLUDES A TTY TASK,

MONITOR CONSOLE ROUTINE, TRANSACTION,

TASK, A CLOCK ROUTINE, AND AN EXIT

TASK.

(DOC. NO 017)

012 DO YOU WISH TO BUILD AN RTS/S SYSTEM WITHOUT

SPECIFYING ANY PARAMETERS? [Y/N] (N):Y

THE QUESTION AND ANSWER SESSION IS

COMPLETED.  A DEFAULT SYSTEM IS NOW BEING

PRODUCED.

PASS2 COMPLETED.  CHAINING TO BATCH AS REQUESTED.

4-33

MODIFYING THE SYSGEN

DFLT=1

IFDEF   DFLT<

TTY=2

MCR=3

TRANS=4

EXIT=6

CLOCK=1

NTASKS=10

KL8A=0

NFLDS=2

TRNDEV=RL1B

MQ=0

MQSV=0>

PDP8A=1

AC7776= CLL STA RAL

AC7775= CLL STA RTL

$JOB-MACREL TASK ASSEMBLY

$MSG MOUNT TASK SOURCES ON SYS:AND STRIKE ANY KEY TO CONTINUE

.MAC SYS:RTS8  <SYS:PDEFLT,SYS:RTS8

.MAC SYS:TTY   <SYS:PDEFLT,SYS:TTY

.MAC SYS:MCR   <SYS:PDEFLT,SYS:MCR

.MAC SYS:TRANS <SYS:PDEFLT,SYS:TRANS

.MAC SYS:CLOCK <SYS:PDEFLT,SYS:CLOCK

.MAC SYS:EXIT  <SYS:PDEFLT,SYS:EXIT

.SU SYS:LDEFLT.BI/T

$END

$JOB RTS8 MODULE LINKAGE

.R LINK

*SYS:SDEFLT</K=0l/M/X

*SYS:RTS8

*SYS:TTY

$SYS:MCR

*SYS:TRANS

*SYS:EXIT

*SYS:CLOCK

*$

.RU SYS:SDEFLT

$END

4.3.4  Multiple TTY Systems

As DIGITAL distributes the SYSGEN, you can create only five TTY tasks.

If  your  system  requires  more  than five, you can use the following

procedure to include as many TTY tasks as your memory can contain.

When you request the SYSGEN to create  more  than  one  TTY  task,  it

writes a control file for each task other than the first.  The control

file skeleton is as follows:

4-34

MODIFYING THE SYSGEN

TASK=TTYn       /WHERE n REPRESENTS THE TTY FILE NAME

TTDEV=	     /PRINTER DEVICE CODE

KBDEV=	     /KEYBOARD DEVICE CODE

VT50=	     /1 ENABLE CTRL/S AND CTRL/Q

SCOPE=	     /1 MEANS TTY CAN DO A BACKSPACE

FILL=	     /NUMBER OF FILL CHARACTERS

WIDTH	     /TTY LINE WIDTH

TAB=	     /1 IF TTY HAS HARDWARE TABS

TTIM=	     /TTY READ TIMEOUT PARAMETER

.INCLUDE dev:TTY.MA

/dev: IS WHERE TTY.MA RESIDES

In this skeleton,

TASK=TTYn

relates a specific task name to the parameter file.  Consequently, you

may  include  as  many  of  these files as are necessary on the device

where source files reside.  Then, when you  are  running  the  SYSGEN,

include these files as user tasks.

You can create these files in two ways.  The  first  procedure  is  to

create  them  using  a text editor.  Add the appropriate values to the

right side of each equate, delete equates that  are  not  relevant  to

your  system,  and  enter  the  device name where TTY.MA resides.  You

would then give this file a name such as  TTY6.MA.   By  passing  this

file through a text editor, you may create as many TTY source files as

your system requires.  When you  create  the  files  in  this  manner,

however,  the  contents of the TTY control files are static;  that is,

they are not created during the SYSGEN process.

The second procedure is to create the TTY  control  files  during  the

SYSGEN.   This  procedure  requires  that  you run the SYSGEN at least

twice. The first times that you run the SYSGEN, you  create  an  RTS/8

system that uses only TTY tasks.  In this way, you can create four TTY

control files each time you pass through the SYSGEN.  For example,  if

your  target  system uses twelve terminals, you would pass through the

SYSGEN three times.  The first pass creates  four  files,  the  second

three, and the third five.

After each SYSGEN pass, run each TTY task through a text editor.   For

example,  if  your target system uses six terminals, you would run the

SYSGEN, creating two TTY tasks (the first is TTY.MA and  is  the  same

for  each  pass).   If your text editor were TECO, you would issue the

following instructions

TECO outdev:TTY5.MA<indev:TTYl.MA

*FSTTY1$TTY5$		     !REPLACE TTY1 WITH TTY5!

EX$$			     !RETURN TO MONITOR!

4-35

MODIFYING THE SYSGEN

You then run the SYSGEN creating the RTS/8 system.   When  the  SYSGEN

asks if you wish to include any user tasks, you include TTY6.

If you ask for a task priority listing in the next question  that  the

SYSGEN  prints,  you  will  find that it prints the five TTY tasks you

entered, possibly followed by several DIGITAL tasks.  These tasks  can

be  SWAP,  CSAF,  OS8F,  and the MCR.  Following the MCR are your user

tasks. Consequently, you may wish to change the priority  of  TTY5  so

that it follows TTY4.

The following example illustrates adding four TTY tasks  to  an  RTS/8

system.   The  first  path through the SYSGEN created four TTY control

files, named TTY1.MA, TTY2.MA, TTY3.MA, and  TTY4.MA.   Using  a  text

editor,  as  discussed above, the appropriate changes were made.  When

TECO wrote the  output  files,  their  names  were  TTY5.MA,  TTY6.MA,

TTY7.MA, and TTY8.MA.

The following example shows

1.  The dialogue  for  adding  files  that  will  become  TTY.MA,

TTY1.MA, TTY2.MA, TTY3.MA, and TTY4.MA to the system.

2.  The dialogue for entering the additional TTY tasks  into  the

system.

3.  Changing the priorities so that the TTY tasks are adjacent to

one another in the priority listing.

4.  A listing from the executing RTS/8 system.

044 HOW MANY TTY TASKS WILL BE IN THE

TARGET SYSTEM? [D] (1):5

IN THE FOLLOWING QUESTIONS, YOU CAN ENTER

UP TO FIVE ANSWERS, SEPARATED BY COMMAS.

THE ANSWERS INDICATE YOUR CHOICE FOR THE GIVEN

PARAMETER FOR EACH OF THE TTY TASKS THAT YOU HAVE

SELECTED.  EXAMPLE:

IF YOU TYPE TO THE DEVICE CODE QUESTION:

04,31,33,35,37 (CARRIAGE RETURN) YOU SELECT

DEVICE CODE CODE 04 FOR TTY TASK, 31 FOR TTY1

ETC.

045 WHAT IS THE KEYBOARD, DEVICE CODE FOR THE

TTY TASK? [O] (03):3,41,43,45,47

046 WHAT IS THE TELEPRINTER DEVICE CODE FOR THE

TTY TASK? [O] (04):4,42,44,46,50

4-36

MODIFYING THE SYSGEN

047 DO YOU WANT CNTRL/S AND CNTRL/Q FEATURES

FOR YOUR TTY TASK? [Y/N] (Y):

048 DO YOU WANT RUBOUT SUPPORT FOR THE TTY

SCREEN'? [Y/N] (Y):

049 HOW MANY NULLS AFTER A LINE FEED FOR THE TTY

DEVICE? [O] (0):

050 HOW MANY CHARACTERS DO YOU WANT ON A LINE

FOR THE TTY DEVICE? [D] (80):

051 DOES THE HARDWARE SUPPORT TABS ON THE

TTY DEVICE [Y/N] (Y):

052 HOW MANY SECONDS BEFORE A TTY READ TIMEOUT? [D] (0):

END OF MULTIPLE ANSWER SECTION. THE FOLLOWING

QUESTIONS REQUIRE SINGLE ANSWERS.

(DOC. NO 270)

053 DO YOU WANT TO INCLUDE ANY USER TASKS

IN THIS RTS/8 SYSTEM?  [Y/N] (N):Y

ENTER THE NAMES OF YOUR TASKS FOLLOWED BY

A CARRIAGE RETURN. EXAMPLE:

TASK 1:  NAME-1,M

TASK 2:  NAME-2,M

.

.

.

TASK N:  NAME-N,M

M IS THE PARTITION WHERE THE TASK WILL RESIDE

(ENTER ZERO OR NO ENTRY FOR THE MAIN LEVEL).

YOU SHOULD ENTER THE TASKS IN THE ORDER OF

DECREASING PRIORITY (THE FIRST ENTERED HAS THE HIGH-

EST PRIORITY).  TERMINATE ENTRY BY TYPING A CARRIAGE

RETURN (ONLY).

(DOC. NO 301)

TASK 001 ?:TTY5

TASK 002 ?:TTY6

4-37

MODIFYING THE SYSGEN

TASK 003 ?:TTY7

TASK 004 ?:TTY8

TASK 005 ?:

054 DO YOU WANT TO CHANGE ANY OF THE TASK'S

PRIORITIES? [Y/N] (N):Y

THE WAY IN WHICH YOU CHANGE DEVICE PRIORITIES

IS TO NAME THAT TASK WHICH WILL BE HIGHER IN

PRIORITY THAN THE NEXT NAMED TASK, TAKEN A PAIR AT A

TIME, AND DELIMINTED BY A COMMA.

EXAMPLE:

PRIORITY CHANGE:  TASK-3, TASK-1

PLACES TASK-3 HIGHER IN PRIORITY THAN

TASK-1.  THE TASK ORDER WILL NOW BE TASK-3,

TASK-1, TASK-2.  WHEN YOU FINISH CHANGING

PRIORITIES, A CARRIAGE RETURN (ONLY) TERMINATES THE ENTRY.

(DOC. NO 312)

TYPE ? AND A CARRIAGE RETURN TO OBTAIN A LIST-

ING OF THE CURRENT TASK PRIORITIES AFTER ANY

PRIORITY PROMPT DURING THE  RESPONSE TO THIS QUESTION

(DOC. NO 244)

PRIORITY CHANGE:?

PRIORITY CHANGE:TTY5,MCR

PRIORITY CHANGE:TTY6,MCR

PRIORITY CHANGE:TTY7,MCR

PRIORITY CHANGE:TTY8, MCR

PRIORITY CHANGE:

THE QUESTION AND ANSWER SESSION IS COMPLETED.

THE PROGRAM WILL CONTINUE ON TO

PASS 2 (FILE BUILDING).

4-38

MODIFYING THE SYSGEN

PASS2 COMPLETED.  CHAINING TO BATCH AS REQUESTED.

RTS8 BUILD PROGRAM

TASK PRIORITY ASSIGNMENT

TASK     PRIORITY

CLOCK   = 001

RL01    = 002

RK8E    = 003

LPT     = 004

EMT     = 005

TTY     = 006

TTY1    = 007

TTY2    = 010

TTY3    = 011

TTY4    = 012

MCR     = 013

TTY5    = 014

TTY6    = 015

TTY7    = 016

TTY8    = 017

EXIT    = 020

BKT     = 021

OS8     = 022

RTS8 BUILD PROGRAM

TASK PRIORITY ASSIGNMENT

TASK     PRIORITY

CLOCK   = 001

RL01    = 002

RK8E    = 003

LPT     = 004

EMT     = 005

TTY     = 006

TTY1    = 007

TTY2    = 010

TTY3    = 011

TTY4    = 012

TTY5    = 013

TTY6    = 014

TTY7    = 015

TTY8    = 016

MCR     = 017

EXIT    = 020

BKT     = 021

OS8     = 022

4-39

MODIFYING THE SYSGEN

.RU SYS:S9TTY

>SYSAT

01 CLOCK   O

02 RL01    M

03 RK8E    M

04 LPT     M

05 EMT     M

06 TTY     M

07 TTTASK  M

10 TTTASK  M

11 TTTASK  M

12 TTTASK  M

13 TTTASK  M

14 TTTASK  M

15 TTTASK  M

16 TTTASK  M

17 MCR

20 EXIT    R *

21 BKT     O

22 OS8     E

>TIME

00:00:20

>DATE 16-OCT-78

>DATE

16-OCT-78

>TIME

00:00:46

>SYSTAT CLOCK

01  CLOCK  O  2600: 0000  3403  0000  0026

>$

^B

.DIR/F=6/M

<EMPTY>    <EMPTY>    <EMPTY>    <EMPTY>

1083 FREE: BLOCKS

4-40

APPENDIX A

RTS/8 SYSTEM GENERATION SOURCE FILES

TASK SOURCES

----------------------------------------------------------------------

|	Task Name    File Name			 Task Function		  |

|--------------------------------------------------------------------|

|									  |

|	-	     RTS8.MA	   RTS/8 Executive			  |

|	CLOCK	     CLOCK.MA	   Clock Handler Task			  |

|	CSA	     CSA.MA	   Cassette Driver Task			  |

|	CSAF	     CSAF.MA	   Cassette File Support Task		  |

|	DTA	     DTA.MA	   TC08 DECtape Driver Task		  |

|	EXIT	     EXIT.MA	   Exit Task				  |

|	LPT	     LPT.MA	   Line Printer Driver Task		  |

|	LTA	     LTA.MA	   LINCtape Driver Task			  |

|	MCR	     MCR.MA	   Monitor Console Routine		  |

|	NULL8A	     NULL8A.MA	   Null Task for PDP-8A			  |

|	OS8	     OS8.MA	   OS/8 Support Task			  |

|	OS8F	     OS8F.MA	   OS/8 File Support Task		  |

|	PWRF	     PWRF.MA	   Power Fail Task			  |

|	RF08/DF32    RF08.MA	   RF08/DF32 Fixed-Head Driver Task	  |

|	RK8	     RK8.MA	   RK8 Disk Driver Task			  |

|	RK8E	     RK8E.MA	   RK8E Disk Driver Task		  |

|	RL01	     RL01.MA	   RL01 Disk Driver Task		  |

|	RX8A	     RX.MA	   Floppy Disk Handler (1st Controller)   |

|	RX8B	     RX.MA	   Floppy Disk Handler (2nd Controller)	  |

|	RX8C	     RX.MA	   Floppy Disk Handler (3rd Controller)	  |

|	RX8D	     RX.MA	   Floppy Disk Handler (4th Controller)	  |

|	SWAP	     SWAP.MA	   Nonresident Task Swapper		  |

|	TTY	     TTY.MA	   Terminal Driver Task			  |

|	TTY1	     TTY.MA	   Terminal Driver Task			  |

|	TTY2	     TTY.MA	   Terminal Driver Task			  |

|	TTY3	     TTY.MA	   Terminal Driver Task			  |

|	TTY4	     TTY.MA	   Terminal Driver Task			  |

|	UDC/ICS	     UDCICS.MA	   Universal Digital Controller/	  |

|				   Industrial Controller Subsystem	  |

|				   Handler task				  |

|	SYSGEN	     SYSGEN.SV	   The SYSGEN save image		  |

|	SCRIPT	     SCRIPT.SV	   The SYSGEN script, assembled		  |

|	SCRIPT	     SCRIPT.MA	   The SYSGEN MACREL source file	  |

----------------------------------------------------------------------

A-1

APPENDIX B

DEC/X8 SPECIFICATION SHEET

Date_________ Manufacturing Facility__________________________________

Customer_______________ DEC#____________ System Serial #______________

Processor PDP-8______ Memory__________K    EAE?_________yes_______no

Power Fail?__yes__no   Memory Parity?__yes__no   Time Share?__yes__no

If DEC/X8 builder Input other than paper tape, specify device

If DEC/X8 builder Output other than paper tape, specify device

Filename______________.SV

List devices in system with information as indicated below:

----------------------------------------------------------------------

|		|    IF INCLUDED IN	| IF NOT INCLUDED

|		|   CUSTOM EXERCISER    | IN CUSTOM EXER-

|		| DEC/X8		| CISER (REASON):

|    DEVICE	| MODULE  DEC/X8   JOB  | MEMORY  NO DEC/X8

DEVICE	|    CODE(S)	|  NAME  PRIORITY  SLOT |  LIMIT   MODULE

----------------------------------------------------------------------

|		|	|        |	|	 |

|		|	|        |	|	 |

|		|	|        |	|	 |

|		|	|        |	|	 |

|		|	|        |	|	 |

|		|	|        |	|	 |

|		|	|        |	|	 |

|		|	|        |	|	 |

|		|	|        |	|	 |

|		|	|        |	|	 |

|		|	|        |	|	 |

|		|	|        |	|	 |

|		|	|        |	|	 |

|		|	|        |	|	 |

|		|	|        |	|	 |

|		|	|        |	|	 |

|		|	|        |	|	 |

|		|	|        |	|	 |

|		|	|        |	|	 |

|		|	|        |	|	 |

|		|	|        |	|	 |

|		|	|        |	|	 |

|		|	|        |	|	 |

|		|	|        |	|	 |

----------------------------------------------------------------------

B-1

APPENDIX C

SYSGEN CHECK LIST

This worksheet list overviews the questions that the SYSGEN  asks.  In

the worksheet's spaces you can specify the answers you wish to give to

questions before you sit at the terminal. For some of  the  questions,

you  may  refer  to  the  Chapter 2 comments by their document numbers

(listed in Section 2.4).

C-1

SYSGEN CHECK LIST

----------------------------------------------------------------------

|	QUESTION		   | ANSWER | DEFLT |	COMMENT    | DOC. |

|				   |	    |	    |		   | NO.  |

|====================================================================|

|	PDP 8/A			   | y    n | Y	    |		   | 042  |

|--------------------------------------------------------------------|

|	PDP 8/E			   | Y 	  n | N	    |		   | 042  |

|--------------------------------------------------------------------|

|	PDP-12			   | y	  n | N	    |		   | 042  |

|--------------------------------------------------------------------|

|	VT78			   | y	  n | N	    |		   | 042  |

|--------------------------------------------------------------------|

|	EAE HARDWARE		   | y 	  n | N	    |		   | 050  |

|				   |--------|-------|		   |      |

|	  Save EAE		   | y	  n | Y	    |		   | 052  |

|--------------------------------------------------------------------|

|	MQ HARDWARE		   | y	  n | Y	    |		   | 054  |

|				   |--------|-------|		   |	  |

|	  Save MQ		   | y	  n | Y	    |		   | 056  |

|--------------------------------------------------------------------|

|	MEMORY FIELDS		   |	    | 4     | 1-32 decimal | 060  |

|--------------------------------------------------------------------|

|	RTS/8 FIELD		   |	    | 0     | 0-32 decimal | 353  |

|--------------------------------------------------------------------|

|	POWER FAIL HARDWARE	   | y	  n | N	    |		   | 062  |

|--------------------------------------------------------------------|

|	KL8A HARDWARE		   | y	  n | N	    |		   | 064  |

|				   |--------|-------|		   |      |

|	  How many in system	   |	    | 1     | 1-8 decimal  | 066  |

|				   |--------|-------|		   |	  |

|	  Device Code		   |	    | 40    | 1-77 octal   | 276  |

|--------------------------------------------------------------------|

|	KT8A HARDWARE		   | y	  n | N	    |		   | 070  |

|--------------------------------------------------------------------|

|	SYSGEN OUTPUT TO	   |	    | SYS   | device name  | 022  |

|--------------------------------------------------------------------|

|	CHAIN TO RTS/8 FROM BATCH  | y	  n |	    |		   | 024  |

|--------------------------------------------------------------------|

|	OUTPUT FILES NAME	   |        | DEFLT | 1 to 5 char. | 026  |

|--------------------------------------------------------------------|

|	CHAIN FROM SYSGEN TO BATCH | y    n | Y     |		   | 030  |

|--------------------------------------------------------------------|

|	WHERE ARE SOURCES	   |	    | SYS   | device name  | 340  |

|--------------------------------------------------------------------|

|	WHERE TO STORE .RB FILES   |	    | SYS   | device name  |	  |

|--------------------------------------------------------------------|

|	ASSEMBLY LISTINGS	   | y	  n | N	    |		   | 334  |

|				   |--------|-------|		   |	  |

|	Listing Device		   |	    | LPT   | device name  |	  |

|--------------------------------------------------------------------|

|	LINKER LOAD MAPS	   | y	  n | N	    |		   | 341  |

|				   |--------|-------|		   |	  |

|	Listing Device		   |	    | LPT   | device name  |	  |

----------------------------------------------------------------------

C-2

SYSGEN CHECK LIST

----------------------------------------------------------------------

|	QUESTION		   | ANSWER | DEFLT |	COMMENT    | DOC. |

|				   |	    |	    |		   | NO.  |

|====================================================================|

|	ARE SOURCES ON SAME MEDIA  | y 	  n |	    |		   | 340  |

|--------------------------------------------------------------------|

|	DEFAULT SYSTEM		   | y	  n | N	    |		   | 017  |

|--------------------------------------------------------------------|

|	POWER FAIL HANDLER	   | y	  n | N	    |		   |	  |

|--------------------------------------------------------------------|

|	CLOCK HANDLER		   | y	  n | N	    |		   |	  |

|				   |--------|-------|		   |	  |

|	  Standard Clock	   | y	  n |	    |		   | 220  |

|				   |--------|-------|		   |	  |

|	    Designation		   |	    | ?	    |		   | 222  |

|				   |--------|-------|		   |	  |

|	  Software Ticks	   |	    | 10    | 1-192 decimal| 347  |

|				   |--------|-------|		   |	  |

|	  # of Entry Slots	   |	    | 16    |16-256 decimal| 224  |

|				   |--------|-------|		   |	  |

|	  Interrupt Linkage	   | y	  n | N	    |		   | 351  |

|--------------------------------------------------------------------|

|	UDC			   | y	  n | N	    |		   |	  |

|				   |--------|-------|		   |	  |

|	  Interrupts Stored	   |	    | 12    | 1-50 octal   | 227  |

|				   |--------|-------|		   |	  |

|	  # of Counter Modules	   |	    | 1	    | 1-20 octal   | 231  |

|				   |--------|-------|		   |	  |

|	  # of Contact Modules	   |	    | 2	    | 1-20 octal   | 231  |

|				   |--------|-------|		   |	  |

|	  # of Analog Inputs	   |	    | 2	    | 1-20 octal   | 231  |

|				   |--------|-------|		   |	  |

|	  Address of Counter	   |	    | ?	    | 0-3777 octal | 235  |

|				   |--------|-------|		   |	  |

|	  Address of Analog	   |	    | ?	    | 0-3777 octal | 235  |

|--------------------------------------------------------------------|

|	RF08 DISK		   | y	  n | N	    |		   |	  |

|--------------------------------------------------------------------|

|	DF32 DISK		   | y	  n | N	    |		   |	  |

|--------------------------------------------------------------------|

|	RK8E DISK		   | y	  n | N	    |		   |	  |

|--------------------------------------------------------------------|

|	RL01 DISK		   | y	  n | N	    |		   |	  |

|--------------------------------------------------------------------|

|	DECTAPE			   | y	  n | N	    |		   |	  |

|--------------------------------------------------------------------|

|	LINCTAPE		   | y	  n | N	    |		   |	  |

----------------------------------------------------------------------

C-3

SYSGEN CHECK LIST

----------------------------------------------------------------------

|	QUESTION		   | ANSWER | DEFLT |	COMMENT    | DOC. |

|				   |	    |	    |		   | NO.  |

|====================================================================|

|	FIRST FLOPPY		   | y	  n | N	    |		   |	  |

|				   |--------|-------|		   |	  |

|	Device Number		   |	    | 75    | 1-77 octal   | 136  |

|--------------------------------------------------------------------|

|	SECOND FLOPPY		   | y	  n | N	    |		   |	  |

|				   |--------|-------|		   |	  |

|	Device Number		   |	    | 70    | 1-77 octal   | 136  |

|--------------------------------------------------------------------|

|	THIRD FLOPPY		   | y	  n | N	    |		   |	  |

|				   |--------|-------|		   |	  |

|	Device Number		   | 	    | 71    | 1-77 octal   | 136  |

|--------------------------------------------------------------------|

|	FOURTH FLOPPY		   | y	  n | N	    |		   |	  |

|				   |--------|-------|		   |	  |

|	Device Number		   |	    | N	    | 1-77 octal   | 136  |

|--------------------------------------------------------------------|

|	LINE PRINTER		   | y	  n | N	    |		   |	  |

|--------------------------------------------------------------------|

|	TERMINAL		   | y	  n | Y	    |		   |	  |

|				   |--------|-------|		   |	  |

|	  # of TTYs in sys.	   |	    | 1     | 1-5	   | 317  |

|				   |--------|-------|		   |	  |

|	  Keyboard Dev. Code	   |	    | 03    | 1-113 octal  | 254  |

|				   |--------|-------|		   |	  |

|	  Teleprinter Dev. Code    |	    | 04    | 1-113 octal  | 254  |

|				   |--------|-------|		   |	  |

|	  CTRL/S, CTRL/Q	   | y	  n | Y	    |		   | 257  |

|				   |--------|-------|		   |	  |

|	  Rubout		   | y	  n | Y	    |		   | 261  |

|				   |--------|-------|		   |	  |

|	  Nulls after <LF>	   |	    | 0     | 0-10 octal   | 263  |

|				   |--------|-------|		   |	  |

|	  Characters on line	   |	    | 80    | multiple of 8| 265  |

|				   |--------|-------|		   |	  |

|	  Tab Support		   | y	  n | Y	    |		   | 273  |

|				   |--------|-------|		   |	  |

|	  Timeout		   |	    | 0	    | 0-10 decimal | 267  |

|--------------------------------------------------------------------|

|	EMERGENCY BREAKTHROUGH	   | y	  n | N	    |		   | 325  |

|				   |--------|-------|		   |	  |

|	  Device to Use	TTY	   |	    | TTY   | name	   | 331  |

|--------------------------------------------------------------------|

|	SWAPPER			   | y	  n | N	    |		   | 342  |

|				   |--------|-------|		   |	  |

|	  Swap Device		   |	    | RK8E  | device name  | 246  |

|				   |--------|-------|		   |	  |

|	  Physical Unit		   |	    | 0	    | alphanumeric | 250  |

|--------------------------------------------------------------------|

|	CASSETTE		   | y	  n | N     |		   |	  |

----------------------------------------------------------------------

C-4

SYSGEN CHECK LIST

----------------------------------------------------------------------

|	QUESTION		   | ANSWER | DEFLT |	COMMENT    | DOC. |

|				   |	    |	    |		   | NO.  |

|====================================================================|

|	CASSETTE FILE SUPPORT	   | y	  n | N     |		   |	  |

|--------------------------------------------------------------------|

|	OS/8 FILE SUPPORT	   | y	  n | N	    |		   | 153  |

|--------------------------------------------------------------------|

|	MCR			   | y	  n | Y	    |		   |	  |

|				   |--------|-------|		   |	  |

|	  Time of Day		   | y	  n | Y	    |		   | 207  |

|				   |--------|-------|		   |	  |

|	  Systat		   | y	  n | Y	    |		   | 211  |

|				   |--------|-------|		   |	  |

|	  MCR terminal		   | 	    | TTY   | TTY name 	   | 215  |

|--------------------------------------------------------------------|

|	TEST TASK		   | y	  n | N     |		   |	  |

|--------------------------------------------------------------------|

|	EXIT TASK		   | y	  n | N     |		   |	  |

|--------------------------------------------------------------------|

|	OS/8 SUPPORT		   | y    n | N	    |		   |	  |

|				   |--------|-------|		   |	  |

|	  # of fields for OS/8	   |	    | 2	    | 1-7 decimal  | 170  |

|				   |--------|-------|		   |	  |

|	  OS/8 Keyboard Device	   |	    | 03    | device code  | 174  |

|				   |--------|-------|		   |	  |

|	  OS/8 Teleprinter Dev.    |	    | 04    | device code  | 172  |

|				   |--------|-------|		   |	  |

|	  RTS/8 Handler for OS/8   |	    | RK8E  | device name  | 176  |

|				   |--------|-------|		   |	  |

|	  OS/8 in top fields	   | y	  n | Y     |		   | 202  |

|				   |--------|-------|		   |	  |

|	    Which Field then	   |	    | ?     | 2-30 decimal | 202  |

|				   |--------|-------|		   |	  |

|	  Nulls after line feed    |	    | 0	    | 0-10 decimal | 204  |

|--------------------------------------------------------------------|

|	OS/8 BACKGROUND		   | y	  n | Y	    |		   | 323  |

|				   |--------|-------|		   |	  |

|	  Background Terminal	   |	    | TTY   | TTY name 	   | 327  |

|--------------------------------------------------------------------|

|	NULL TASK		   | y	  n | N	    | PDP 8/A only |	  |

|--------------------------------------------------------------------|

|	ENTER USER TASKS	   | y	  n | N	    |		   | 300  |

----------------------------------------------------------------------

C-5

SYSGEN CHECK LIST

---------------------------------------------

|	     TASK ENTRY WORKSHEET	     |

|===========================================|

|					     |

|   Task Name________________  Part._____   |

|					     |

|   Task Name________________  Part._____   |

|					     |

|   Task Name________________  Part._____   |

|					     |

|   Task Name________________  Part._____   |

|					     |

|   Task Name________________  Part._____   |

|					     |

|   Task Name________________  Part._____   |

|					     |

|   Task Name________________  Part._____   |

|					     |

|   Task Name________________  Part._____   |

|					     |

|   Task Name________________  Part._____   |

|					     |

|   Task Name________________  Part._____   |

|					     |

|   Task Name________________  Part._____   |

|					     |

|   Task Name________________  Part._____   |

|					     |

|   Task Name________________  Part._____   |

|					     |

|   Task Name________________  Part._____   |

|					     |

|   Task Name________________  Part._____   |

|					     |

|   Task Name________________  Part._____   |

|					     |

|   Task Name________________  Part._____   |

|					     |

|   Task Name________________  Part._____   |

|					     |

|   Task Name________________  Part._____   |

|					     |

|   Task Name________________  Part._____   |

|					     |

|   Task Name________________  Part._____   |

|					     |

|   Task Name________________  Part._____   |

|					     |

|   Task Name________________  Part._____   |

|					     |

|   Task Name________________  Part._____   |

---------------------------------------------

C-6

APPENDIX D

RTS/8 PARAMETER LISTING

This appendix is an abbreviated listing of the RTS/8  Parameter  file.

It  differs  from  the parameter file in SCRIPT.MA in that information

that is redundant or useful only to the SYSGEN is not included.  Also,

where necessary, additional comments have been added.

.SBTTL PARAMETER RECORDS

/2.9 PARAMETERS FOR RTS/8 TASKS 	     V3

/

/

/COPYRIGHT  (C)  1974,1975,1976,1977,1978,1979

/		      BY DIGITAL EQUIPMENT CORPORATION

/THE INFORMATION IN THIS DOCUMENT IS SUBJECT TO CHANGE WITHOUT NOTICE

/AND SHOULD NOT BE CONSTRUED AS A COMMITMENT BY DIGITAL EQUIPMENT

/CORPORATION. DIGITAL EQUIPMENT CORPORATION ASSUMES NO RESPONSIBILITY

/FOR ANY ERRORS THAT MAY APPEAR IN THIS DOCUMENT.

/

/THE SOFTWARE DESCRIBED IN THIS DOCUMENT IS FURNISHED TO THE PURCHASER

/UNDER A LICENSE FOR USE ON A SINGLE COMPUTER SYSTEM AND CAN BE COPIED

/(WITH INCLUSION OF DIGITAL'S COPYRIGHT NOTICE) ONLY FOR USE IN SUCH

/SYSTEM, EXCEPT AS MAY OTHERWISE BE PROVIDED IN WRITING BY DIGITAL.

/

/DIGITAL EQUIPMENT CORPORATION ASSUMES NO RESPONSIBILITY FOR THE USE

/OR RELIABILITY OF ITS SOFTWARE ON EQUIPMENT THAT IS NOT SUPPLIED BY

/DIGITAL.

/RTS/8 V3 - PARAMETER FILE

/INDEX:

/SECTION 1-CONFIGURATION PARAMETERS

/SECTION 2-SYSTEM TASK TABLE

/SECTION 3-MULTIPLE PARAMETER TASK LISTS

/SECTION 4-DEFAULT PARAMETERS (NORMALLY FIXED)

/SECTION 5-SYSTEM PARAMETERS (NORMALLY FIXED)

/SECTION 6-TTY SKELETON CONTROL FILES

/SECTION 7-RX FLOPPY SKELETON CONTROL FILES

D-1

RTS/8 PARAMETER LISTING

/SECTION 1-CONFIGURATION PARAMETERS

M1777,  TEXT    "IFNDEF RTS8    <XLIST>"

M2016,  TEXT    "DFLT="	     /1 IF USER SPECIFIES DEFAULT SYSTEM

/   DURING SYSGEN

M2041,  TEXT    "PDP8A="	     /1 IF PDP 8/A, ELSE 0

M2043,  TEXT    "PDP8E="	     /1 IF PDP 8/E, ELSE 0

M2044,  TEXT    "PDP12="	     /1 IF PDP-12, 0 OR UNDEFINED

/   IF NOT PDP-12

M2047,  TEXT    "EAE="	     /1 IF SYSTEM HAS EAE HARDWARE

M2051,  TEXT    "EAESV="	     /1 IF CONTENTS OF EAE REGISTERS SAVED

/   WHEN CONTEXT SWITCHING OCCURS

M2053,  TEXT    "MQ="	     /1 IF SYSTEM HAS HARDWARE MQ REGISTER

M2055,  TEXT    "MQSV="	     /1 IF CONTENTS OF MQ REGISTER SAVED

/   WHEN CONTEXT SWITCHING OCCURS

M2057,  TEXT    "NFLDS="	     /NUMBER OF FIELDS, RIGHT JUSTIFIED

M2352,  TEXT    "RTFLD="	     /FIELD IN WHICH RTS8 RESIDES

M2061,  TEXT    "PWRFAL="	     /1 IF POWER FAIL/RESTART ENABLED IN

M2065,  TEXT    "KL8A="	     /   HARDWARE NUMBER OF KL8A'S IN USE

/I.E. 1 IF ONE KL8A (UP TO 4 LINES)

M2067,  TEXT    "KT8A="	     /1 IF SYSTEM HAS EXTENDED MEMORY

/   HARDWARE

M2045,  TEXT    "VT78="	     /1 IF SYSTEM IS A VT78 SYSTEM

/SECTION 2 - SYSTEM TASK TABLE

/COMMON TASKS--THE SYSGEN ONLY PLACES THE TASKS YOU SELECT

/IN THE OUTPUT PARAMETER FILE

/TASKS ARE LISTED IN DECREASING ORDER OF PRIORITY (DEFAULT-TASK

/NOT SELECTED, AND PRIORITY NOT SPECIFIED)

M2074,  TEXT    "PWRF="	     /POWER FAIL HANDLING TASK

M2075,  TEXT    "CLOCK="	     /CLOCK HANDLER - SHOULD BE HIGH

/   PRIORITY. MULTIPLE PARAMETER

/   TASK-SEE LISTING BELOW

M2132,  TEXT    "DTA="	     /DECTAPE DRIVER TASK

M2150,  TEXT    "CSA="	     /CASSETTE DRIVER TASK

D-2

RTS/8 PARAMETER LISTING

M2125,  TEXT    "UDC="	     /UNIVERSAL DIGITAL CONTROLLER TASK

/MULTIPLE PARAMETER TASK-SEE

/   LISTING BELOW

M2336,  TEXT    "RL01="	     /RL01 DISK DRIVER TASK

M2126,  TEXT    "RF08="	     /RF08 DISK DRIVER TASK

M2127,  TEXT    "DF32="	     /DF32 DISK DRIVER TASK

M2130,  TEXT    "RK8="	     /RK8 OR RK8E DISK DRIVER TASK

M2131,  TEXT    "RK8E="	     /RK8E DISK DRIVER TASK

M2133,  TEXT    "LTA="	     /LINCTAPE DRIVER TASK

M2134,  TEXT    "RX8A="	     /FIRST FLOPPY CONTROLLER

M2137,  TEXT    "RX8B="	     /SECOND FLOPPY CONTROLLER

M2141,  TEXT    "RX8C="	     /THIRD FLOPPY CONTROLLER

M2143,  TEXT    "RX8D="	     /FOURTH FLOPPY CONTROLLER

M2145,  TEXT    "LPT="	     /LINE PRINTER DRIVER TASK

M2324,  TEXT    "EMT="	     /EMERGENCY BREAKTHROUGH MESSAGE TASK

M2146,  TEXT    "TTY="          /TELETYPE DRIVER TASK

/MULTIPLE PARAMETER TASK - SEE

/   LISTING BELOW

M2701,  TEXT    "TTY1="	     /FIRST TTY CONTROL FILE TASK

M2716,  TEXT    "TTY2="	     /SECOND

M2731,  TEXT    "TTY3="	     /THIRD

M2746,  TEXT    "TTY4="	     /FOURTH

M2147,  TEXT    "SWAP="	     /NONRESIDENT TASK SWAPPER TASK

/    MULTIPLE PARAMETER TASK-SEE

/    LISTING BELOW

M2151,  TEXT    "CSAF="	     /CASSETTE FILE SUPPORT TASK

M2152,  TEXT    "OS8F="	     /OS/8 FILE SUPPORT

M2156,  TEXT    "MCR="	     /MONITOR CONSOLE ROUTINE

/MULTIPLE PARAMETER TASK-SEE

/   LISTING BELOW

/SYSGEN INSERTS USER TASKS HERE

M2160,  TEXT    "TEST="	     /RTS/8 TEST TASK; COMES UP RUNNING

/WHEN DEFAULT SYSTEM IS SPECIFIED

M2161,  TEXT    "EXIT="	     /EXIT TASK

D-3

RTS/8 PARAMETER LISTING

M2322,  TEXT    "BKT="	     /OS/8 TERMINAL SUPPORT TASK

M2162,  TEXT    "OS8="	     /OS/8 SUPPORT - NORMALLY LOWEST

/   PRIORITY MULTIPLE PARAMETER

/   TASK-SEE LISTING BELOW

M2163,  TEXT    "NULL8A="	     /NULL JOB FOR PDP-8/A

M1776,  TEXT    "NTASKS="	     /NUMBER OF TASKS IN SYSTEM

/   (SYSTEM+USER TASKS)

/SECTION 3 - MULTIPLE PARAMETER TASK LISTS

/INCLUDE WHEN OS-8 TASK IS SPECIFIED

M2167,  TEXT    "OSFLDS="	     /NUMBER OF FIELDS TO ALLOCATE TO

/   OS/8 (KG 2)

M2171,  TEXT    "OSTTDV="	     /OS/8 TELEPRINTER DEVICE CODE (KG 04)

M2173,  TEXT    "OSKBDV="	     /OS/8 KEYBOARD DEVICE CODE (KG 03 IS

/   CONSOLE)

M2175,  TEXT    "OSSYSD="	     /OS/8 SYSTEM DEVICE DRIVER TASK

/   (EG RK8E)

M2203,  TEXT    "OSFILL="	     /FILLS NEEDED AFTER LINE FEED

/   (EG 4 FOR 2400 BAUD VT05)

M2201,  TEXT    "OSBASE="	     /IF KT8A HARDWARE PRESENT; LOWEST

/   FIELD IN WHICH OS/8 TASK RUNS

M2326,  TEXT    "BKTCDV="	     /TASK NAME OF TTY DEVICE TO SHARE

/   WITH OS/8

/INCLUDE WHEN MCR TASK IS SPECIFIED

M2206,  TEXT    "MCRCLK="	     /0 IF MCR TIME OF DAY FACILITIES TO

/   BE OMITTED (DEFAULT IS 1)

M2210,  TEXT    "MCRSYS="	     /0 IF MCR SYSTAT FACILITY NOT DESIRED

/1 IF DESIRED (DEFAULT)

M2214,  TEXT    "MCRCDV="	     /CONSOLE TO BE USED BY MCR, E.G. TTY

/DEFAULT IS TTY

/INCLUDE WHEN CLOCK TASK IS SPECIFIED

M2221,  TEXT    "CLKTYP="	     /0 (DK8EA/DK8EC) OR l(KW12) OR 2

/   (PDP8/A) OR 3 (DK8EP)

M2346,  TEXT    "SHERTZ="	     /SOFTWARE CLOCK TICK RATE (DEFAULT

/   IS 16 (DECIMAL)

M2223,  TEXT    "CLKQLN="	     /MINIMUM NUMBER OF ENTRY SLOTS IN

/   THE CLOCK QUEUE

/MAY BE CHANGED BY USER

D-4

RTS/8 PARAMETER LISTING

M2350,  TEXT    "CLKLRM="	     /ENABLE INTERRUPT LINKAGE FEATURE

/   1 IS ENABLE, 0 IS DEFAULT

/INCLUDE WHEN UDC TASK IS SPECIFIED

M2226,  TEXT    "RINGBF="	     /NUMBER OF INTERRUPTS THAT CAN BE

/   STORED

M2230,  TEXT    "NCNTR="	     /NUMBER OF COUNTER MODULES

M2232,  TEXT    "NCNTC="	     /NUMBER OF CONTACT MODULES

M2233,  TEXT    "NAD="	     /NUMBER OF ANALOG INPUT CONVERTER

/   MODULES

M2234,  TEXT    "FCTR="	     /ADDRESS OF THE FIRST COUNTER MODULE

M2236,  TEXT    "FCT="	     /ADDRESS OF THE FIRST CONTACT

/   INTERRUPT MODULE

M2237,  TEXT    "FAD="	     /ADDRESS OF THE FIRST A/D CONVERTER

/   MODULE

/INCLUDE WHEN SWAPPER TASK IS SPECIFIED

M2245,  TEXT    "SYS="	     /SWAP DEVICE DRIVER TASK (KG RK8E)

M2250,  TEXT    "SUNIT=" 	     /SWAP DEVICE PHYSICAL DRIVE UNIT

/   (EG 0 FOR RKA0)

/INCLUDE IF TTY TASK IS SPECIFIED

/THIS IS ALSO THE STRUCTURE FOR TTY

/CONTROL FILES

M2253,  TEXT    "KBDEV="	     /KEYBOARD DEVICE CODE - DEFAULT IS 3

M2255,  TEXT    "TTDEV="	     /PRINTER DEVICE CODE - DEFAULT IS 4

M2256,  TEXT    "VT50="	     /1 ENABLES CTRL/S AND CTRL/Q

M2260,  TEXT    "SCOPE="	     /1 MEANS TTY CAN DO A BACKSPACE

M2262,  TEXT    "FILL="	     /NUMBER OF FILL CHARACTERS, I.E. 4

M2264,  TEXT    "WIDTH="	     /TTY LINE WIDTH (0 MEANS INFINITE),

/   DEFAULT IS 120

M2272,  TEXT    "TAB="	     /1 IF TTY HAS HARDWARE TABS

M2330,  TEXT    "EMTCDV="	     /NAME OF TTY DEVICE FOR EMERGENCY

/   MESSAGE BREAKTHROUGH

M2266,  TEXT    "TTTIM="	     /TTY READ TIMEOUT PARAMETER.

/INCLUDE IF KL8A TASK IS SPECIFIED

M2275,  TEXT    "KL8ADV="       /KL8A DEVICE CODE - DEFAULT IS 40

D-5

RTS/8 PARAMETER LISTING

.SBTTL DEFAULT PARAMETERS

/SECTION 4 - DEFAULT PARAMETERS (NORMALLY FIXED)

/CONFIGURATION DEFAULTS (DEFAULT TASKS INCLUDED)

M1000,  TEXT    "IFDEF DFLT<"

TEXT    "TTY=2"	     /DEFAULT TASK ASSIGNMENT

TEXT    "MCR=3"

TEXT    "TEST=4"

TEXT    "EXIT=6"

TEXT    "CLOCK=1"	     /END;DEFAULT ASSIGNMENT

TEXT    "NTASKS=10"     /DEFAULT TOTAL NUMBER OF TASKS

TEXT    "KL8A=0"

TEXT    "NFLDS=2"

TEXT    "MQ=0"

TEXT    "MQSV=0>"

.SBTTL SYSTEM PARAMETERS

/SECTION 5 - SYSTEM PARAMETERS (NORMALLY FIXED)

/EQUIVALENCES:

P1001,  TEXT    "AC7776=	     CLL STA RAL"

TEXT    "AC7775=	     CLL STA RTL"

TEXT    "AC4000=	     CLA STL RAR"

TEXT    "AC3777=	     CLL STA RAR"

TEXT    "AC2000=	     CLA STL RTR"

TEXT    "AC0002=	     CLA STL RTL"

TEXT    "AC5777=	     CLL STA RTR"

/MONITOR CALL VALUES:

TEXT    "CAL= JMS 20"          /CALL THE EXECUTIVE

TEXT    "POSTDS=JMP I 24"      /DISMISS AN INTERRUPT

TEXT    "WAITM= JMS I 25"	    /WAIT FOR MULTIPLE EVENTS

/NOTE: "**" MEANS CRITICAL VALUE MAY NOT

/BE CHANGED WITHOUT MODIFYING SYSTEM CODE!!

TEXT    "SEND= 0"	     /SEND MESSAGE

TEXT    "RECEIV=1"	     /RECEIVE MESSAGE

TEXT    "WAITE= 2"	     /WAIT FOR EVENT FLAG

TEXT    "RUN= 3"	     /CONTINUE TASK EXECUTION

TEXT    "SUSPND=4"	     /SUSPEND TASK EXECUTION

TEXT    "POST= 5"	     /POST AN EVENT FLAG

D-6

RTS/8 PARAMETER LISTING

TEXT    "SKPINS=6"	     /INSERT CODE INTO INTERRUPT

/    SKIP CHAIN

TEXT    "DERAIL=7"	     /INITIATE SOFTWARE INTERRUPT

TEXT    "BLKARG=10"     /BLOCK TASK FOR REASON SPECIFIED

/   IN ARG

TEXT    "UNBARG=12"     /UNBLOCK TASK FOR REASON SPECIFIED

/   IN ARG

TEXT    "RESCHD=13"     /FORCE A RESCHEDULE

TEXT    "WAITX= 14"     /WAIT FOR EXACTLY THIS EVENT FLAG

TEXT    "FREE= 4000"    /**FREE PARTITION

/TASK STATUS FLAGS:

TEXT    "IFDEF UDC	<"

TEXT    "AO=0"

TEXT    "DO=0"

TEXT    "DI=2"

TEXT    "GC=3"

TEXT    "EC=4"

TEXT    "RC=5"

TEXT    "DC=6"

TEXT    "ECT=7"

TEXT    "CS=10"

TEXT    "DCT=11"

TEXT    "AI=12>"

TEXT    "NONRWT=4000"	    /**NONRESIDENT TASK WAIT

TEXT    "EFWT= 2000"	    /EVENT FLAG WAIT

TEXT    "RUNWT= 1000"	    /SCHEDULE WAIT

TEXT    "SWPWT= 0400"	    /**SWAPPER WAIT

TEXT    "EORMWT=0200"	    /EVENT FLAG OR MESSAGE WAIT

TEXT    "USERWT=0100"	    /USER SPECIFIED WAIT

TEXT    "ENABWT=0040"	    /ENABLE WAIT

TEXT    "MSGWT= 0020"	    /MESSAGE WAIT

TEXT    "NETWT= 0010"	    /NETWORK WAIT

/   (RESERVED FOR FUTURE USE)

TEXT    "DNEWT= 0001"	    /**DOES NOT EXIST WAIT

/SOME USEFUL EQUATES FOR TTY AND LPT MESSAGES:

TEXT    "NOPACK=4000"   /TEXT IS NOT PACKED IN 6-BIT

TEXT    "NOCRLF=2000"   /OUTPUT SHOULD NOT BE FOLLOWED

/   BY CR/LF

D-7

RTS/8 PARAMETER LISTING

TEXT    "IND=1000"      /OUTTXT PTS TO FIRST WORD OF TEXT

TEXT    "NOLINE=400"    /INPUT IS IN CHARACTER MODE

TEXT    "ASSGN=200"     /ASSIGNS DEVICE

TEXT    "KL8ALINE=100"  /USED TO SPECIFY A LINE OF A KL8A

/SOME USEFUL EQUATES FOR STANDARD CLOCK MESSAGES:

TEXT    "IFDEF CLOCK    <"

TEXT    "MARKTIME=0"    /POST EVENT FLAG AFTER SPECIFIED

/   INTERVAL

TEXT    "SCHEDULE=1000" /RUN TASK AFTER SPECIFIED INTERVAL

TEXT    "TIMOUT=2000"   /DERAIL TASK AFTER SPECIFIED INTERVAL

TEXT    "PERIODICALLY=3000"

/USED AS MODIFIER TO 'SCHEDULE'

/   RE-QUEUES RUN REQUREST AFTER

/   SPECIFIED INTERVAL, E.G.,

/   'SCHEDULE FOO PERIODICALLY'

TEXT    "REMOTE=4000" /POST CLOCK MESSAGE IMMEDIATELY.

/   AFTER SPECIFIED INTERVAL REPOST

TEXT    "CANCEL=7000>" /DELETE ALL REQUESTS FROM SPECIFIED

/   TASK FROM QUEUE

TEXT    "IFNDEF OS8     <"

TEXT    "SLASHK=NFLDS-1>"

/SLASHK CONTROLS GENERATION OF EXEC

/   LINKAGE, 20-26 EACH FIELD

TEXT    "IFDEF OS8      <"

TEXT    "IFNDEF OSBASE  <OSBASE=NFLDS-OSFLDS>"

TEXT    "IFZERO NFLDS-OSBASE-OSFLDS <SLASHK=NFLDS-OSFLDS>>"

TEXT    "IFNDEF BKTCDV  <BKTCDV=0>"

TEXT    "IFNDEF SUNIT   <SUNIT= 0>"

/DEFAULT SWAP UNIT IS 0

TEXT    "HGHFLD=NFLDS-1" 	     /FOR HISTORICAL REASONS

/   CALCULATES CDF TO

/ HIGHEST LEGAL FIELD

TEXT    "HGHFLD=<<HGHFLD&710>+<HGHFLD&10%2>+<HGHFLD&204>>"

TEXT    "IFNDEF PDP8E   <PDP8E=0>"	    /FORCE UNDEFINED TO

TEXT    "IFNDEF PDP8A   <PDP8A=0>"	    / EQUAL ZERO

TEXT    "IFNDEF PDP12   <PDP12=0>"

TEXT    "IFNDEF VT78    <VT78=0>"

TEXT    "OMNI=PDP8E+PDP8A+VT78"	    /OMNI BUS FOR TTY

/   SETUP

/BSW

D-8

RTS/8 PARAMETER LISTING

TEXT    "IFNDEF EAESV   <EAESV=0>"	    /MORE UNDEFINED

TEXT    "IFNDEF EAE     <EAE=0>"	    /   EQUATES SET TO

TEXT    "IFNDEF MQSV    <MQSV=0>"	    /   ZERO

TEXT    "IFNDEF MQ      <MQ=0>"

TEXT    "IFNDEF PWRFAL  <PWRFAL=0>"

TEXT    "IFNDEF KL8A    <KL8A=0>"

TEXT    "IFNDEF KT8A    <KT8A=0>"

TEXT    "IFNZRO EAESV   <MQSV=1>"	    /IF EAE SAVED, MQ IS

/   ALSO SAVED

TEXT    "IFNZRO EAE     <MQ=1>" 	    /IF THERE IS EAE,

/   SYSTEM

/   ALSO HAS MQ

TEXT    "IFZERO EAE     <EAESV=0>"     /CAN'T SAVE IF THERE

TEXT    "IFZERO MQ	     <MQSV=0>"      /   ISN'T ONE

TEXT    "IFDEF CLOCK    <"

TEXT    "IFNDEF CLKQLN  <CLKQLN=20"    /DEFAULT

TEXT    "IFNDEF SHERTZ  <SHERTZ=12>"   /CLOCK

TEXT    "IFNDEF CLKLRM  <CLKLRM=0>"    /SETTINGS

TEXT    "IFNDEF CLKTYP  <"

TEXT    "CLKTYP=3"

TEXT    "IFNZRO PDP8A   <CLKTYP=2>"    /DEFAULT CLOCK FOR

TEXT    "IFNZRO PDP12   <CLKTYP=1>"    /   EACH KIND OF

TEXT    "IFNZRO PDP8E   <CLKTYP=0>"    /   MACHINE

TEXT    ">"

TEXT    "IFNZRO CLKTYP&1 <HERTZ=1750>" /DEFAULT TICK RATE

TEXT    "IFZERO CLKTYP  <HERTZ=170>"   /   FOR EACH KIND

TEXT    "IFZERO CLKTYP-2 <HERTZ=144>"  /      OF CLOCK

TEXT    "IFNZRO HERTZ%SHERTZ SHERTZ-HERTZ <ERROR>"

/IS HERTZ A MULTIPLE

/   OF SHERTZ?

TEXT    "IFZERO SHERTZ-301&4000 <ERROR>"

TEXT    ">"

TEXT    "IFNDEF RTFLD   <RTFLD=0>"	    /EXECUTIVE DEFAULT

TEXT    "IFNZRO KT8A    <" 	    /   IS FIELD ZERO

TEXT    "KTFLDS=40	     " 		    /KT, IF ON, FOR 128K

TEXT    "RACA=6175	     " 		    /KT8A

TEXT    "RACB=6176	     " 		    /BIT

TEXT    "RACC=6177	     "  	    /INSTRUCTIONS

TEXT    ">"

/SECTION 6 - RX FLOPPY SKELETON CONTROL FILES

/RX CONTROL FILE

M1770,  TEXT    "TASK=RX8A"     /SPECIFY RX8A, FIRST FLOPPY

/   CONTROL FILE

A1770,  TEXT    "RXDVCD="	     /DEVICE CODE

D-9

RTS/8 PARAMETER LISTING

.SBTTL PASS2 MESSAGES

/THESE MESSAGES INDICATE THE STATUS OF PASS3 TO THE USER.

/ALSO INCLUDE RECORDS FOR CONTROL FILES ARE ENTERED HERE.

M5700,  TEXT    "R  PASS2 COMPLETED.  RETURNING TO MONITOR"

M5701,  TEXT    "R  PASS2 COMPLETED.  CHAINING TO BATCH AS REQUESTED."

M5702,  TEXT    ".INCLUDE XXXXXX:TTY.MA"

M5703,  TEXT    ".INCLUDE XXXXXX:RX.MA"

M5777,  TEXT    "DONE MARKER"

.SBTTL PASS2 MACREL BATCH RECORDS

/THIS SECTION OF THE SCRIPT CONTAINS THOSE RECORDS

/REQUIRED TO CREATE THE MACREL BATCH FILE USED TO

/ASSEMBLE USER SPECIFIED SYSTEM AND USER TASKS.

/ 	     For this listing, the following section has been

/       split into two lines. It does not appear this

/       way in the parameter file

M7004, TEXT ".MAC XXXXXX:RTS8  ,XXXXXX:RTS8  <

XXXXXX:XXXXXX,XXXXXX:RTS8  /B/C"

M7001, TEXT ".MAC XXXXXX: TTY  ,XXXXXX:TTY   <

XXXXXX:XXXXXX,XXXXXX:TTY   /B/C"

M7002, TEXT ".MAC XXXXXX:MCR   ,XXXXXX:MCR   <

XXXXXX:XXXXXX,XXXXXX:MCR   /B/C"

M7003, TEXT ".MAC XXXXXX:TEST  ,XXXXXX:TEST  <

XXXXXX:XXXXXX,XXXXXX:TEST /B/C"

M7006, TEXT ".MAC XXXXXX:EXIT  ,XXXXXX: EXIT <

XXXXXX:XXXXXX,XXXXXX:EXIT  /B/C"

M7005, TEXT ".MAC XXXXXX:CLOCK ,XXXXXX: CLOCK <

XXXXXX:XXXXXX,XXXXXX:CLOCK /B/C"

/RECORD   NUMBERS M7001 THROUGH M7050 ARE RESERVED FOR THE DEFAULT

/TASK SECTION, DELIMITED BY RECORD 7776.  DO NOT INSERT OTHER THAN

/DEFAULT TASK EQUATES BEFORE RECORD 7776.

M7776,  TEXT    "DONE DEFAULT TASK MARKER"

D-10

RTS/8 PARAMETER LISTING

/FOLLOWING MESSAGES ARE USED BY MBLD, A SERVICE ROUTINE UNDER

/MAIN3, TO BUILD THE MACREL BATCH FILE.  USE CAUTION TO AVOID

/DIFFICULTY IN THAT THESE RECORDS ARE PARSED ON A DISPLACEMENT

/BASIS TO INSERT THE VARIABLE DATA.

M7110,  TEXT    "$JOB-MACREL TASK ASSEMBLY"

M7111,  TEXT    "$END"

M7112,  TEXT    "$MSG MOUNT TASK SOURCES ON XXXXXX:

AND STRIKE ANY KEY TO CONTINUE:'

M7114,  TEXT    ".MAC XXXXXX:XXXXXX,XXXXXX:XXXXXX<

XXXXXX:XXXXXX,XXXXXX:XXXXXX/B/C"

M7116,  TEXT    ".SU XXXXXX:XXXXXX.BI/E/T"

M7117,  TEXT    ".SU XXXXXX:XXXXXX.BI/T"

M7777,  TEXT    "DONE MARKER"

.SBTTL PASS2 LINKER BATCH MESSAGES

/THESE SCRIPT MESSAGES ARE USED TO BUILD THE LINKER BATCH FILE.

/THEY ARE OPERATED UPON BY PASS2 ROUTINE LBLD.MA. CAUTION SHOULD

/BE EXERCISED IN MODIFYING THEM, SINCE THEY ARE DISPLACEMENT

/SENSITIVE.

/

/

/DEFAULT SECTION:

M6006,  TEXT    "*XXXXXX:RTS8"

M6001,  TEXT    "*XXXXXX:TTY"

M6002,  TEXT    "*XXXXXX:MCR"

M6003,  TEXT    "*XXXXXX:TEST"

M6004,  TEXT    "*XXXXXX:EXIT"

M6005,  TEXT    "*XXXXXX:CLOCK"

M6776,  TEXT    "DONEMARKER"

/END OF DEFAULT TASKS

M6100,  TEXT    ".R LINK"

M6101,  TEXT    "*XXXXXX:XXXXXX,XXXXXX:XXXXXX</K=01/M/X"

D-11

RTS/8 PARAMETER LISTING

/MORE OPTIONS MAY BE ADDED TO END OF M6101 WITHOUT DISRUPTING

/PROGRAM OPERATION.

M6102,  TEXT    "*XXXXXX:XXXXXX"

M6103,  TEXT    "*XXXXXX:XXXXXX/C"

M6104,  TEXT    "*/O"

M6105,  TEXT    "*$"

/DELIMITS LINKER COMMAND STRING

M6107,  TEXT    ".RU XXXXXX:SXXXXX"

/CHAIN TO RTS8 AFTER BUILD IF USER SELECTS

M6110,  TEXT    "$END"

M6111,  TEXT    "$JOB RTS8 MODULE LINKAGE"

M6777,  TEXT    "DONE MARKER"

.SBTTL LIMIT CHECKING RECORDS

/FOR EACH OCTAL/DECIMAL QUESTION IN THE INTERACTIVE SCRIPT RECORDS,

/AN ENTRY IS MADE IN THIS LIST OF THE LOWER,UPPER VALUES WHICH THE

/CORRESPONDING ANSWER MAY ASSUME. EACH PASS THROUGH OCTDEC THE

/LIMIT CHECKING ROUTINE IS CALLED TO VERIFY THE ENTRY.

M3057,  TEXT    "L3057 002,032"

M3352,  TEXT    "L3352 000,031"

M3065,  TEXT    "L3065 001,003"

M3167,  TEXT    "L3167 002,008"

M3171,  TEXT    "L3171 001,113"

M3173,  TEXT    "L3173 001,113"

M3201,  TEXT    "L3201 002,030"

M3203,  TEXT    "L3203 000,010"

M3346,  TEXT    "L3346 001,192"

M3223,  TEXT    "L3223 016,256"

M3226,  TEXT    "L3226 001,050"

M3230,  TEXT    "L3230 001,020"

D-12

RTS/8 PARAMETER LISTING

M3232,  TEXT    "L3232 001,020"

M3233,  TEXT    "L3233 001,020"

M3234,  TEXT    "L3234 000,3777"

M3236,  TEXT    "L3236 000,3777"

M3135,  TEXT    "L3135 001,077"

M3140,  TEXT    "L3140 001,077"

M3142,  TEXT    "L3142 001,077"

M3144,  TEXT    "L3144 001,077"

M3237,  TEXT    "L3237 000,3777"

M3243,  TEXT    "L3243 001,007"

M3251,  TEXT    "L3251 001,005"

M3253,  TEXT    "L3253 001,113"

M3255,  TEXT    "L3255 001,113"

M3262,  TEXT    "L3262 000,010"

M3264,  TEXT    "L3264 000,160"

M3266,  TEXT    "L3266 000,3001"

M3275,  TEXT    "L3275 001,007"

D-13

APPENDIX E

ERROR MESSAGES

This appendix  lists  all  of  the  RTS/8  System  Generation  Errors.

Immediately  following  each  error  message  are two paragraphs.  The

first paragraph  further  explains  the  error  message.   The  second

describes the action the SYSGEN takes when it encounters the error and

what you should do to prevent the reoccurrence of the error.

ERROR 005 BAD RECORD DIRECTORY READ FROM MASS STORAGE (FATAL)

The SYSGEN was unable to access a record in SCRIPT.SV.

You may have a  bad  image  of  SCRIPT.SV  or  a  faulty  device.

Execution  control returns to the OS/8 Monitor.  You should check

your device and/or relink SCRIPT.SV.

ERROR 012 BAD SCRIPT RECORD READ (FATAL)

The SYSGEN was unable to locate a record.

This usually  indicates  a  problem  with  the  device  on  which

SCRIPT.SV resides but it may indicate a bad SCRIPT.SV image.  You

should check the device upon which SCRIPT resides  and/or  relink

SCRIPT.SV.  Execution control returns to the OS/8 Monitor.

ERROR 013 FEATURE INCORRECTLY USED (NUMBER NOT FOUND)

You attempted to branch forward.   The  SYSGEN  can  only  branch

backwards.   This  error  may also indicate that you have typed a

non-numeric character after the # symbol.

The SYSGEN reprints the question that it printed before you  made

the bad branch.  You can now branch to a question already printed

or answer the question.

ERROR 016 SUBSEQUENT RECORD DIRECTORY READS UNSUCCESSFUL (FATAL)

The SYSGEN was unable to locate a record in SCRIPT.SV.

This error indicates that you are having device problems  or  you

have  a  bad SCRIPT.SV image.  You should examine the device upon

which SCRIPT.SV resides and/or relink SCRIPT.SV.  Control returns

to the OS/8 Monitor.

E-1

ERROR MESSAGES

ERROR 017 RESPONSE IS TOO LARGE

You typed more than  six  characters  (five  characters  for  TTY

questions) in an answer.

After the SYSGEN reprints the question, type the correct  answer.

This  error  normally  occurs  when  typing an answer to a device

specification question.  The longest character string  that  OS/8

recognizes is six characters.

ERROR 021 NOT A YES OR NO ANSWER OR BAD DEFAULT FOR Y/N RECORD

A Yes/No question requires an answer that begins with the  letter

'Y'  or  letter  'N'.   This  error can indicate that the default

listed within parentheses is not a Y or N.  (This may occur if  a

user   made   a   typographical  error  when  making  changes  in

SCRIPT.MA.)

After the SYSGEN reprints the question, type a Y or  N.   If  the

problem  was  that  the default was not a Y or N, you must type a

^C,  use  a  text  editor  to  change  the  default,   reassemble

SCRIPT.MA, and then redo the system generation.

ERROR 022 SCRIPT RECORD WITH NO DEFAULT (FATAL)

The question does not have a default answer  within  parentheses.

This  occurs  only  when  a  user changes the text and forgets to

include a default in the question.

Control returns to the OS/8 Monitor.  You must use a text  editor

to enter a default answer and then reassemble the question.

ERROR 023 NO DEFAULT ALLOWED FOR THIS QUESTION

Some SYSGEN questions have a question mark as their default.  For

these  answers  you  must  explicitly enter an answer rather than

type a carriage return.

After the SYSGEN reprints the question, type the answer.

ERROR 030 RESPONSE NOT OCTAL/DECIMAL AS EXPECTED

Your answer contained characters other than the numbers 0 through

9 (or 0 through 7 for octal answers).

After the SYSGEN reprints the question, retype the answer.

E-2

ERROR MESSAGES

ERROR 050 ILLEGAL BRANCH RECORD (FATAL)

A branch record could not find its target.  This  message  occurs

when  an  error  was  made in modifying the script.  The cause is

that a record was deleted from SCRIPT and a branch left in or, in

the  case  of  adding  records, a branch was added and the branch

target does not exist.

Control returns to the OS/8 Monitor.  You must  now  examine  the

portion  of  the record directory following the last question you

answered to locate the branch record  that  caused  the  problem.

After  you  find the branch record, either change it or eliminate

it.  After the correction is made, reassemble SCRIPT.MA.

ERROR 055 ASCII ANSWER CONTAINS NON ALPHANUMERIC DATA

Your answer  contained  characters  other  than  the  upper  case

letters A through Z and the numbers 0 through 9.

After the SYSGEN reprints the question, retype the answer.

ERROR 060 T SUBROUTINE REQUESTED DOES NOT EXIST (FATAL)

The SYSGEN could not find a T  subroutine.   (These  are  special

processing  subroutines  included in the SYSGEN.  A user may also

write his own.)

If the T subroutine exists, no modification was made to the RTS/8

system  program  TYPET.MA.  If the subroutine does not exist, the

SYSGEN was linked improperly.

ERROR 070 MORE THAN FIVE TTY TASKS SPECIFIED

Your answer was greater than five.

After the SYSGEN reprints the question, type a number from  1  to

5.

ERROR 100 DEVICE CODE MUST BE SPECIFIED FOR EACH TTY TASK

You typed a carriage return or typed fewer device codes than  the

number of terminals in the target system.

After the SYSGEN reprints the question, type a  device  code  for

each device.

E-3

ERROR MESSAGES

ERROR 110 INCORRECT NUMBER OF ENTRIES FOR NUMBER OF TTY TASKS

SPECIFIED

You typed a carriage return  or  typed  fewer  entries  than  the

number of terminals in the system.

After the SYSGEN reprints the question, type an  entry  for  each

device.

ERROR 120 SUM OF SYSTEM PLUS USER TASKS ALREADY 127 DECIMAL

The SYSGEN prints this error message when you  attempt  to  enter

the 128th task into the system.

The SYSGEN prints the next question (which asks if  you  wish  to

change  the priorities of any tasks).  If you wish to include the

128th task, you must branch back to the question  asking  if  you

want  to include any user tasks.  You then reenter all the tasks,

eliminating one of the tasks you previously entered.  If you wish

to  retain  all  the  user tasks, you must type a ^C and redo the

SYSGEN eliminating  one  of  the  DIGITAL  tasks  you  previously

specified.    Note  that  127 tasks is the maximum number you can

have in an RTS/8 system.

ERROR 132 CANNOT OPEN, CLOSE, OR WRITE TO OUTPUT FILE (FATAL)

The device is write locked  or  some  other  similar  problem  is

preventing  the  SYSGEN  from writing information to the file you

specified in other questions.

Control returns to the OS/8 Monitor.  You must redo  the  SYSGEN,

making sure that the output device is working properly and is not

write locked.  (This message does not indicate that the device is

not configured in OS/8.)

ERROR 134 INSUFFICIENT ROOM FOR OUTPUT FILE (FATAL)

The output device does not have enough free blocks for the SYSGEN

output files.

Control returns to the OS/8 Monitor.  You must delete blocks from

the output device, place another device in that drive, or specify

another output device in response to SYSGEN questions.

E-4

ERROR MESSAGES

ERROR 135 INCORRECT DEVICE SPEC.  FOR BUILD OUTPUT FILES (FATAL)

The device specification you gave for the SYSGEN  output  is  not

active in the OS/8 operating system.

Execution control returns to the  OS/8  Monitor.   Use  the  OS/8

RESORC  command  to  determine if the device is active.  If it is

active, you entered the device name incorrectly.  If it  is  not,

use  the BUILD program to install the device.  You must then redo

the SYSGEN.

ERROR 137 DEVICE SPEC FOR OUTPUT FILES NOT MASS STORAGE (FATAL)

The output device for  SYSGEN  files  is  not  a  file-structured

device (such as a disk or a DECtape).  You accidently specified a

device such as a line printer or paper tape punch.

Execution control returns to the OS/8 Monitor.  You must redo the

SYSGEN,  this  time  making  sure  you  specify a file-structured

device.

ERROR 141 BATCH.SV NOT FOUND (FATAL)

The BATCH.SV OS/8 utility program is not on your  system  device.

The SYSGEN program cannot automatically create an RTS/8 system if

you do not have the BATCH program.

After copying BATCH to SYS, you must resubmit  the  MACREL  BATCH

file.

ERROR 144 I/O ERROR ON TASK LISTING DEVICE

The I/O device is inoperative.  (If your system does not  have  a

line printer, the SYSGEN automatically prints information on your

terminal display.)

If the task listing device (a line printer)  is  active  in  your

OS/8 system, check to see if it is turned on.  If it is turned on

and its name is LPT (or another name attributed to it by an  OS/8

ASSIGN  command),  you are having problems with your line printer

handler.  The quickest solution is assign LPT to TTY so that  the

listing appears on your terminal.  However, you should attempt to

rebuild and/or reboot your OS/8 system.

ERROR 145 I/O ERROR ON TASK LISTING DEVICE

The I/O device is inactive.

See the comment to ERROR 144.

E-5

ERROR MESSAGES

ERROR 150 INCORRECT FORMAT FOR TASK PRIORITY CHANGE

Your answer to a task priority change prompt was incorrect.

After the SYSGEN reprints the prompt, retype your response.   The

format  for  specifying  a  change  is  to  type  two  task names

separated by commas.  The first task is  the  task  you  wish  to

move;   the  second  is  the  task  which   the first task should

immediatly precede.

ERROR 160 PRIORITY TASK AS SPECIFIED NOT FOUND

The task you specified has not yet been entered in the system or,

if it was entered, you made a typographical error.

If your response contained a typographical error, you can  retype

the  task name.  If the task name you typed was correct, obtain a

listing of tasks in the system by typing a question  mark  and  a

carriage return.  This listing will tell you the tasks entered in

the system.  If you made a typographical error in entering a task

name, type a ^C and redo the SYSGEN.

ERROR 200 ENCOUNTERED NON-EXISTENT RECORD TYPE (FATAL)

When the SYSGEN encountered a record, its first  letter  was  not

one  of those it recognizes.  The types the SYSGEN recognizes are

A, D, I, L, N, R, O, T, and Y.

Control returns to  the  OS/8  Monitor.   You  should  examine  a

listing  of  SCRIPT to determine where the error occurred.  After

you locate the error, make the correction with a text editor  and

reassemble script.  You must also redo the entire SYSGEN.

ERROR 201 # FEATURE USAGE NOT OK AFTER PRIORITY CHANGES

After the question that asks if you wish to  change  any  of  the

task's  priorities,  the SYSGEN disables the function that allows

it to branch back to a previous question.

The SYSGEN reprints the question to which you gave a #  response.

If  there was an error to a previous question, you must type a ^C

and redo the SYSGEN.  Otherwise, you may continue forward.

E-6

ERROR MESSAGES

ERROR 202 NUMERICAL ANSWER NOT WITHIN SPECIFIED LIMITS

The answer you specified was either greater than or less than the

answer that is allowed for a question.

After the SYSGEN retypes the question,  type  a  number  that  is

within  the  question's  limits.  You may determine the limits by

examining the limit records in a listing of SCRIPT.MA or  looking

in the parameter file listing in Appendix D.

ERROR 203 NON-RESIDENT TASKS MAY NOT HAVE PRIORITY HIGHER THAN THE

SWAP TASK

When making changes in task priorities, you attempted to  give  a

higher  priority to a nonresident task than the priority that the

SWAP task has.

After the SYSGEN reprompts, change the priority of  the  task  to

one  that  is  lower  than the swap task.  If you do not know the

task priorities, type a question mark and a carriage return.  The

SYSGEN then prints a task priority listing.

ERROR 204 PARTITION ENTRY LIMITED FROM 0 (MAIN) TO 7

You attempted to place a nonresident task in  a  partition  other

than  those  in  the  range  0  through 7.  This message also may

indicate a typographical error in entering the partition number.

After the SYSGEN retypes the prompt, retype  the  entry  using  a

partition number in the range 0 through 7.

ERROR 205 INCORRECT FORMAT FOR PARTITION CHANGE

The SYSGEN requires that you type the task name, a comma, and the

partition  into  which  you  wish  to place the task.  This error

indicates that the text you typed was not in this format.

After the SYSGEN reprompts,  retype  the  entry  in  the  correct

format.

ERROR 206 NO MORE THAN 16 ENTRIES PER PARTITION

You attempted to place  a  seventeenth  task  into  partitions  1

through  7.   (You may place as many as you like into "partition"

0.)

If you receive this message  while  entering  tasks,  you  should

place the task that caused this message into another partition.

E-7

ERROR MESSAGES

ERROR 207 TASK HAS DUPLICATE NAME

The task whose name you just entered has already been entered.

After the SYSGEN reprompts, continue entering tasks.

E-8

INDEX

Alphanumeric questions, 2-4	  Decimal questions, 2-3

see also SCRIPT.MA record	      see also SCRIPT.MA record

type indicator		        type indicator

Analog to digital converter,	  DECtape driver, 2-25

see UDC			  Default answers, 2-4

Answers, maximum length, 2-3	    changing, 4-6

Assembly listings, 2-12		  Default systems, 2-13

device, 2-12			    adding tasks to, 4-31, 4-32

example, 4-32

creating, 4-11

example, 4-32

Background operation,		  Default systems (Cont.),

see OS/8 support task		    example, 3-1

BATCH, 2-9, 2-11, 2-12	            LINK batch file, 4-11, 4-32

Branching, 2-4			      changing, 4-32

renumbering, 2-5			      listing, D-11

MACREL batch file, 4-11, 4-32

changing, 4-32

listing, D-10

Cassette driver, 2-33		    parameter record listing, D-6

Cassette file support task, 2-34     Deleting questions, example,

Chaining,                                4-13

to BATCH, 2-10, 2-48               DF32 disk driver, 2-24

to RTS/8, 2-9, 2-48		  Diskette,

Changing limits,			      see Floppy disk controller

see Limit records		  DOC. NO,

Clock handler, 2-19		      see Document numbers

HERTZ (hardware tick rate),	  Document numbers, 2-4

2-21				    listing, 2-51

interrupt linkage, 2-21		  Drivers,

nonstandard clocks, 2-20		      see under Device name

queue entry slots, 2-20

SHERTZ, 2-21

standard clocks, 2-20

tick rate, 2-21			  EAE, 2-14

Conditionals, 4-6			    saving registers of, 2-14

format, 4-7			  Emergency breakthrough task,

record numbers (preferred),	      2-28

4-7				    terminal name, 2-29

type, 4-7			  EMT,

Controllers,			      see Emergency breakthrough

see under Device name                  task

Control files, 2-10, 2-27, 2-30      Equates, 2-47, 4-11

Error message records, 4-12

Error messages, 2-5

INDEX-1

INDEX (Cont.)

Errors, 2-5			  LINK load map, 2-12, 4-18

fatal, 2-5                           device, 2-13

SYSGEN responses, 2-6                example, 4-22

Example SYSGENs, 3-1, 3-4              listing, 2-12

Execution,                           Load map,

see SYSGEN                           see LINK load map

Exit task, 2-36

MACREL batch file, 2-8, 2-47,

File specifications, 2-6                 4-12, 4-32

Floppy disk controller, 2-26           default system, 4-11

control files, 2-27, 3-24              listing, 3-25, D-10

device numbers, 2-27               MACREL source files, 2-10, A-1

number in system, 2-27               assembly listings, 2-12

Foreground operation, 1-1, 2-37          device, 2-12

multiple media, 2-10

.RB output files, 2-10, 2-11

source devices, 2-13

Handlers,                              source media, 2-10

see under Device name            MCR, 2-30

HERTZ,                                 system status report, 2-30

see Clock handler                  terminal, 2-35

time-of-day command, 2-30

Memory fields,

KT8-A, 2-18

Immediate response clock		    number in system, 2-16

subroutine,			    OS/8, 2-37

see Clock handler		      placement in, 2-38

Interrupt clock linkage,               RTS/8, 2-16

see Clock handler                Minimum configuration, 1-1

Monitor Console Routine,

see MCR

MQ, 2-15

KL8-A, 2-17			    saving registers, 2-15

device codes, 2-18

number in system, 2-17

KT8-A, 2-18, 2-38

Nonresident tasks, 2-32, 2-42,

2-44

Null task, 2-40

Limit records, 4-12

changing, 4-13

format, 4-13

listing, D-12		        Octal questions, 2-3

LINC-8, 2-8			    see also SCRIPT.MA record

LINCtape driver, 2-26		      type indicators

Line printer driver, 2-28		Operating information, 2-2

LINK batch file, 2-8, 2-47,	OS/8 file support task, 2-30

4-12, 4-32			OS/8 support task, 2-36

default system, 4-11		  background operation, 2-37

listing, 3-25, D-11              keyboard device code, 2-37

LINK, 2-11, 2-47                     line feed, nulls after, 2-39

INDEX-2

INDEX (Cont.)

OS/8 support task, (Cont.)           .RB files, 2-10

memory fields, 2-37                Record directory,

placement in, 2-38                   see SCRIPT.MA

RTS/8 handler task, 2-38           Remarks, 2-4, 4-4

teleprinter device code, 2-38          see also SCRIPT.MA record

terminal driver, 2-40                    type indicator

Output files,                          adding, 4-5

see MACREL batch file;               example, 4-5, 4-17

LINK batch file; Parameter       RF08 disk driver, 2-24

file; Control files              RK8 disk driver, 2-25

RK8E disk driver, 2-24

RLO1 disk driver, 2-25

RTS/8 field, 2-16

Parameter file, 2-8, 2-47,           RTS/8 .SV image, 2-11, 2-47

3-20, 4-10                       RTS/8 system equates, 4-11

see also Parameter records         listing, D-2

Parameter records, 4-10              RTS/8 tasks, number permitted,

default system listing, D-6            2-41

format, 4-10

listing, D-1

records, 4-10

system task listing, D-2           Save image file storage,

Partitions,                              see RTS/8 .SV image

see Task partitions              Script records,

PDP 8, 2-7                               see SCRIPT.MA

PDP 8/A, 2-7                         SCRIPT.MA, 4-1

null task, 2-40                      accessing records, 4-2

PDP 8/E, 2-7                           cross-reference listing, 4-17

PDP 8/I, 2-7                           delimiting tags, 4-3

PDP 8/L, 2-7                           parameter records,

PDP-12, 2-7                              see Parameter records

Power fail handler, 2-17               questions,

Power fail hardware, 2-16                adding, 4-5

Priority,                                deleting, 4-9

see Task priority                      restrictions, 4-9

example, 4-17

record directory, 4-2

format, 4-2

Questions,                             record length, 4-4, 4-5

adding, 4-5                          record numbers, 4-2

alternate procedure, 4-32          record tags, 4-2

example, 4-17                      record type indicators, 4-3,

guidelines for, 4-17                 4-4

automatic numbering, 2-5             remark records, 4-4

branching, 2-5                         adding, 4-5

default answers, 2-4                 script records, 4-3

deleting, 4-9                          format, 4-3

example, 4-14                        type indicator, 4-3

SCRIPT.SV, 1-1

devices, 2-2

SHERTZ,

see Clock handler

INDEX-3

INDEX (Cont.)

Source files,			  Terminal driver, (Cont.)

see MACREL source files	    multiple terminal system, 4-34

.SV file,				      example, 4-36

see RTS/8 .SV image                number in system, 2-30

Swap task, 2-32                        OS/8, 2-39

swapping device, 2-33                  keyboard device code, 2-37

SYSGEN,				      line feed, nulls after, 2-39

defined, 1-1			      teleprinter device code, 2-38

error responses, 2-5		    RUBOUT, 2-31

SYSGEN (Cont.),                        tabs (hardware), 2-32

examples, 3-1, 3-4                   teleprinter device code, 2-31

minimum configuration for, 1-1       timeout, 2-32

operating information, 2-2         Test task, 2-36

output files, 2-8

overview, 1-1, 1-2, 2-1

running, 1-1, 2-2

SYSGEN.SV, 1-1                       UDC/ICS, 2-21

System equates, 4-11                   analog input converter, 2-23

listing, D-6                           address, 2-24

System generation,                   UDC/ICS (Cont.),

see SYSGEN                           contact module, 2-23

System tasks, 4-11                       address, 2-23

initial priority, 4-11               counter module, 2-22

parameter record listing, D-2          address, 2-23

source files, A-1                    interrupts stored, 2-22

User responses,

information required, 1-2

to questions, 1-2

Target computer,                     User tasks, 2-40, 4-11

see Target machine                   see also Task partitions;

Target machine, 2-7                        Task priorities

Task partitions, 2-41, 2-44            names, 2-41

changing, 2-44                       number permitted, 2-41

listing, 2-44

Task priority, 2-41

changing, 2-41

listing, 2-42                      VT78, 2-7

Tasks, number permitted, 2-41

Terminal driver, 2-29

character deletion, 2-31

characters per line, 2-32          Yes/no questions, 2-3

control files, 2-30, 3-24              see also SCRIPT.MA record

CTRL/Q, 2-31                             type indicators

CTRL/S, 2-31

emergency breakthrough task,

see Emergency breakthrough

task

keyboard device code, 2-30

line feed, nulls after, 2-31

OS/8, 2-39

line length, 2-32

MCR terminal, 2-35

INDEX-4

RTS/8

User's Manual

AA-0724D-TA

READER'S COMMENTS

NOTE:  This form is for document comments only. DIGITAL will

use comments submitted on this form at the company's

discretion. If you require a written reply and are

eligible to receive one under Software Performance

Report (SPR) service, submit your comments on an SPR

form.

Did you find this manual understandable, usable, and well-organized?

Please make suggestions for improvement.

----------------------------------------------------------------------

----------------------------------------------------------------------

----------------------------------------------------------------------

Did you find errors in this manual? If so, specify the error and

the page number.

----------------------------------------------------------------------

----------------------------------------------------------------------

----------------------------------------------------------------------

Please indicate the type of reader that you most nearly represent.

o Assembly language programmer

o Higher-level language programmer

o Occasional programmer (experienced)

o User with little programming experience

o Student programmer

o Other (please specify)

Name_______________________________________ Date______________________

Organization__________________________________________________________

Street _______________________________________________________________

City_____________________________ State________ Zip Code _____________

or

Country

-------------------------------Fold Here------------------------------

-----------------Do Not Tear - Fold Here and Staple-------------------

+---------------+

|  FIRST CLASS  |

| PERMlT NO. 33 |

| MAYNARD. MASS.|

+---------------+

BUSINESS REPLY MAIL					   ========

NO POSTAGE STAMP NECESSARY IF MAILED IN THE UNITED STATES  ========

======================================================================

Postage will be paid by:				   ========

========

+-+-+-+-+-+-+-+			   ========

|d|i|g|i|t|a|l|			   ========

+-+-+-+-+-+-+-+			   ========

========

Software Communications                      ========

146 Main Street  ML5-5/E39		   ========

Maynard, Massachusetts  01754		   ========