OS/8 SOFTWARE SUPPORT MANUAL

(Version 3)

DISCLAIMER

This document file was created by scanning  the

original  document and then editing the scanned

text.  As much as possible, the	 original  text

format	was restored.  Some format changes were

made to insure this  document  would  print  on

current laser printers using 60 lines per page.

The  original  spelling	 and  grammar  has been

preserved.

1-Sep-1996

OS/8 SOFTWARE SUPPORT MANUAL

(Version 3)

DEC-S8-OSSMB-A-D

-------------------------------------------------------

| Order additional copies as directed on the Software |

| Information page at the back of this document.      |

-------------------------------------------------------

digital equipment corporation * maynard, massachusetts

First Printing, January 1973

Revised, June 1974

The information in this document is subject to change without notice

and should not be construed as a commitment by Digital Equipment

Corporation.  Digital Equipment Corporation assumes no responsibility

for any errors that may appear in this manual.

The software described in this document is furnished to the purchaser

under a license for use on a single computer system and can be copied

(with inclusion of DIGITAL's copyright notice) only for use in such

system, except as may otherwise be provided in writing by DIGITAL.

Digital Equipment Corporation assumes no responsibility for the use

or reliability of its software on equipment that is not supplied by

DIGITAL.

Copyright (C) 1973, 1974 by Digital Equipment Corporation

The HOW TO OBTAIN SOFTWARE INFORMATION page, located at the back of

this document, explains the various services available to DIGITAL

software users.

The postage prepaid READER'S COMMENTS form on the last page of this

document requests the user's critical evaluation to assist us in

preparing future documentation.

The following are trademarks of Digital Equipment Corporation:

CDP              DIGITAL          INDAC            PS/8

COMPUTER LAB     DNC              KA10             QUICKPOINT

COMSYST          EDGRIN           LAB-8            RAD-8

COMTEX           EDUSYSTEM        LAB-8/e          RSTS

DDT              FLIP CHIP        LAB-K            RSX

DEC              FOCAL            OMNIBUS          RTM

DECCOMM          GLC-8            05/8             RT-ll

DECTAPE          IDAC             PDP              SABR

DIBOL            IDACS            PHA              TYPESET 8

UNIBUS

PREFACE

The 8K Operating  System  (OS/8)  is  an  extremely  powerful  program

development  system.   OS/8 greatly expands the capabilities of any 8K

PDP-8, 8/I, 8/L, 8/E, or PDP-12 computer having the necessary disk	 or

DECtape  storage.	 Use  of  OS/8,	 is described in detail in the OS/8

HANDBOOK (DEC-S8-OSHBA-A-D).

This manual covers a wide range of advanced topics	 pertinent  to	the

experienced  user.	 In Chapter 1 the various basic system concepts are

described and terms are defined.  Chapter 2 explains  the	process	 by

which  user  programs  call  upon	the  system  for the performance of

important operations including loading device  handlers,  opening	and

closing  files,  and chaining to other programs.  Chapter 3 covers the

functions of the Command Decoder and  the	means  by  which  the  user

program  can  employ  its	services.   Chapter  4 explains the use and

operation of the device handlers in  detail.   Chapter  5	covers	the

details  of  "custom  tailoring"  a  system,  including how to write a

device handler for a non-standard device.

Technical information, intended to enhance the information in the OS/8

HANDBOOK,	as  well  as  this  manual, can be found in the Appendices.

Appendix A details the OS/8 directory  structure  and  gives  standard

file  format.  Appendix B describes the system data base and gives the

layouts of the system areas.  Appendix C  gives  a	 complete  list	 of

system  error  messages.	Appendix D illustrates some useful advanced

techniques and programming "tricks" for  use  with	 the  OS/8  system.

Appendix  E  is  a complete list of the standard ASCII character codes

meaningful to OS/8. Finally, Appendix F describes a set of generalized

I/O routines for use under the OS/8 system.

iii

CONTENTS

Page

CHAPTER 1	       OS/8 CONCEPTS AND TERMINOLOGY		       1-1

1.1       SOFTWARE COMPONENTS OF OS/8		       1-1

1.2       FILES					       1-2

1.2.1     File Names and Extensions		       1-2

1.2.2     File Structured Devices			       1-2

1.2.3     File Types				       1-3

1.2.4     File Directories and Additional

Information Words			       1-3

1.3       CORE CONTROL BLOCK			       1-4

1.3.1     Program Starting Address			       1-5

1.3.2     Job Status Word				       1-5

1.3.3     Software Core Size			       1-6

1.4       DEVICE NAMES AND DEVICE NUMBERS		       1-6

1.5       THE DEVICE AND FILENAME PSEUDO-OPS	       1-7

CHAPTER 2	       USER SERVICE ROUTINE			       2-1

2.1       CALLING THE USR				       2-1

2.1.1     Standard USR Call			       2-1

2.1.2     Direct and Indirect Calling Sequence	       2-2

2.2       SUMMARY OF USR FUNCTIONS			       2-3

2.2.1     FETCH Device Handler			       2-4

2.2.2     LOOKUP Permanent File			       2-6

2.2.3     ENTER Output (Tentative) File		       2-7

2.2.4     The CLOSE Function			       2-8

2.2.5     Call Command Decoder (DECODE)		       2-10

2.2.6     CHAIN Function				       2-10

2.2.7     Signal User Error			       2-12

2.2.8     Lock USR in Core (USRIN)			       2-13

2.2.9     Dismiss USR from Core (USROUT)		       2-13

2.2.10    Ascertain Device Information (INQUIRE)	       2-14

2.2.11    RESET System Tables			       2-15

CHAPTER 3	       THE COMMAND DECODER			       3-1

3.1       COMMAND DECODER CONVENTION		       3-1

3.2       COMMAND DECODER ERROR MESSAGES		       3-3

3.3       CALLING THE COMMAND DECODER		       3-3

3.4       COMMAND DECODER TABLES			       3-4

3.4.1     Output Files				       3-5

3.4.2     Input Files				       3-5

3.4.3     Command Decoder Option Table		       3-6

3.4.4     Example					       3-7

iv

Page

3.5       SPECIAL MODE OF THE COMMAND DECODER	       3-10

3.5.1     Calling the Command Decoder Special Mode	       3-10

3.5.2     Operation of the Command Decoder in Special

Mode					       3-10

3.6       CCL AND THE COMMAND DECODER		       3-11

3.7       USEFUL LOCATIONS IN BATCH		       3-11

3.8       CCL TABLES				       3-12

CHAPTER 4	       USING DEVICE HANDLERS			       4-1

4.1       CALLING DEVICE HANDLERS			       4-1

4.2       DEVICE DEPENDENT OPERATIONS		       4-4

4.2.1     Teletype (TTY)				       4-4

4.2.2     High-Speed Paper Tape Reader (PTR)	       4-4

4.2.3     High-Speed Paper Tape Punch (PTP)	       4-5

4.2.4     Line Printer (LPT)			       4-5

4.2.5     Cassettes				       4-7

4.2.6     Card Reader (CDR)			       4-8

4.2.7     TM8E Handler				       4-8

4.2.8     File Structured Devices			       4-12

4.2.9     TD8E DECtape				       4-12

4.2.10    KL8E Teletype Handler			       4-13

CHAPTER 5	       RECONFIGURING THE OS/8 SYSTEM		       5-1

5.1       WRITING DEVICE HANDLERS			       5-1

5.2       INSERTING DEVICE HANDLERS INTO OS/8	       5-5

APPENDIX A	       OS/8 FILE STRUCTURES			       A-1

A.1       FILE DIRECTORIES				       A-1

A.1.1     Directory Entries			       A-2

A.1.2     Number and Size of 05/8 Files		       A-3

A.1.3     Sample Directory				       A-3

A.2       FILE FORMATS				       A-4

A.2.1     ASCII and Binary Files			       A-4

A.2.2     Core Image (.SV format) Files		       A-5

A.2.3     Relocatable FORTRAN Library File		       A-8

APPENDIX B	       DETAILED LAYOUT OF THE SYSTEM		       B-1

B.1       LAYOUT OF THE SYSTEM DEVICE		       B-1

B.2       LAYOUT OF THE OS/8 RESIDENT PROGRAM	       B-3

B.3       SYSTEM DEVICE TABLES			       B-5

B.3.1     Permanent Device Name Table		       B-5

v

Page

B.3.2     User Device Name Table			       B-6

B.3.3     Device Handler Residency Table		       B-6

B.3.4     Device Handler Information Table		       B-6

B.3.5     Device Control Word Table		       B-7

B.3.6     Device Length Table			       B-8

APPENDIX C	       SYSTEM ERROR CONDITIONS AND MESSAGES	       C-1

C.1       SYSTEM HALTS				       C-1

C.2       USR ERRORS				       C-2

C.3       KEYBOARD MONITOR ERRORS			       C-3

C.4       CCL ERROR MESSAGES			       C-4

C.5       COMMAND DECODER ERRORS			       C-7

APPENDIX D	       PROGRAMMING NOTES			       D-1

D.1       THE DEFAULT FILE STORAGE DEVICE, DSK	       D-1

D.2       MODIFICATION TO CARD READER HANDLER	       D-2

D.3       SUPPRESSION OF CARRIAGE RETURN/LINE FEED

IN FORTRAN				       D-4

D.4       ACCESSING THE SYSTEM DATE IN A FORTRAN

PROGRAM					       D-4

D.5       DETERMINING CORE SIZE ON PDP-8 FAMILY

COMPUTERS				       D-5

D.6       USING PRTC12-F TO CONVERT OS/8 DECTAPES

TO OS/12 LINCTAPES			       D-6

D.7       NOTES ON LOADING DEVICE HANDLERS		       D-7

D.7.1     Problem with Multiple Input Files	       D-7

D.7.2     Dynamically Loading Device Handlers	       D-8

D.8       AVAILABLE LOCATIONS IN THE USR AREA	       D-9

D.9       ACCESSING ADDITIONAL INFORMATION WORDS

IN OS/8					       D-9

D.9.1     After a LOOKUP or ENTER			       D-9

D.9.2     After a CLOSE				       D-10

D.9.3     Rewriting the Current Directory Segment	       D-10

D.10      SABR PROGRAMMING NOTES			       D-11

D.10.1    Optimizing SABR Code			       D-11

D.10.2    Calling the USR and Device Handlers from

SABR Code				       D-13

vi

Page

APPENDIX E	       CHARACTER CODES AND CONVENTIONS		       E-1

APPENDIX F	       OS/8 INPUT/OUTPUT ROUTINES		       F-1

F.1       GENERAL DESCRIPTION			       F-1

F.2       SUBROUTINE FUNCTIONS			       F-1

F.3       SUBROUTINE PARAMETERS			       F-3

F.3.1     Example					       F-4

F.3.2     Subroutine Listing			       F-5

INDEX							       I-1

vii

CHAPTER 1

OS/8 CONCEPTS AND TERMINOLOGY

Before examining the details of the OS/8  system,	the  reader  should

first  be	familiar  with the simpler techniques and terms used within

the framework of the OS/8 system.	The material in this chapter, along

with that contained in the OS/8 HANDBOOK, provides the tools needed to

pursue the later chapters.

1.1  SOFTWARE COMPONENTS OF OS/8

There are four main components of the OS/8 system:

1.  The Keyboard Monitor performs commands specified by the  user

at  the keyboard console.	 The nine Keyboard Monitor commands

(ASSIGN, DEASSIGN, GET, SAVE, ODT, RUN, R, START,	 and  DATE)

are explained in Chapter 1 of the OS/8 HANDBOOK.

User programs can exit to the Keyboard Monitor by executing a

JMP  to  location	 7600  in field 0. All JMPs to 7600 must be

made with the  DATA  FIELD  set  to  zero.   This	 saves	the

contents	of  locations 0000 to 1777 in field 0 and loads the

Keyboard Monitor which could be called by a JMP  to  location

7605 in field 0. In this latter case the contents of core are

not saved, which conserves some time.

Existing system programs, device handlers,  and  the  Command

Decoder  test  for the CTRL/C character in the terminal input

buffer and, on finding  this  character,	abort  the  current

operation	 and perform a JMP to 7600 in field 0. Thus, typing

CTRL/C is the conventional method	 of  calling  the  Keyboard

Monitor from the console.

2.  Device handlers, which are  subroutines  for  performing	all

device-oriented  input/output  operations, can be utilized by

any  program.   These  subroutines  have	 standard   calling

sequences	 and  "mask"  from  the	 user  program	the special

characteristics of the  I/O  device.   In	 this  way,  device

independent  I/O	is  achieved.	A  detailed  description of

device handlers is found in Chapter 4.

3.  The User Service Routine (USR)  is  to  a	 program  what	the

Keyboard	Monitor	 is to the user.  For example, programs can

request the  USR	to  fetch  device  handlers,  perform  file

operations  on  any device, chain to another program, or call

the Command Decoder.  A full description of the USR functions

is found in Chapter 2.

1-1

OS/8 CONCEPTS AND TERMINOLOGY

4.  The Command Decoder interprets a command line  typed  by	the

user  to indicate input and output files and various options.

The command line format is described in detail in	 Chapter  1

of the OS/8 HANDBOOK.  The Command Decoder removes the burden

of this repetitive operation from the user's program.  A full

description  of  the  Command  Decoder's function is found in

Chapter 3.

5.  Two other components, ABSLDR and ODT, are not logically  part

of  the  OS/8  system.  However, in the sources and listings,

ABSLDR is combined with the Keyboard Monitor and USR.  ODT is

combined with the command decoder.

1.2 FILES

Files are basic units of the OS/8 system, and a thorough understanding

of	 file  structure is required for its use.  A file is any collection

of data to be  treated  as	 a  unit.   The	 format	 of  this  data	 is

unimportant;   for	 example,  OS/8	 can  manipulate  several  standard

formats, including ASCII files, binary files, and	core  image  files.

The  important  consideration  is	that  the  data forms a single unit

within the system.

1.2.1 File Names and Extensions

An individual file is identified by its file name and extension.	The

file  name	 consists  of up to six alphanumeric characters, optionally

followed by a two character extension.  The extension is often used to

clarify the format of the data within the file.  For example, an ASCII

file used as input to PAL8 might be given a PA extension, while a core

image file has a SV extension.

1.2.2 File Structured Devices

Devices that can be logically divided into a number of 256-word blocks

and  have	the  ability  to  read and write from any desired block are

called  file  structured  devices.	  Disks	 and  DECtapes	 are   file

structured devices while a paper tape reader or terminal is not.

Cassettes and magnetic tapes form an intermediate case.  They  may	 be

treated  directly as non-file structured devices, or the program MCPIP

may appear to be file structured.

The system device (SYS) in any OS/8 system is always file	structured,

as is the default storage device, DSK.

All OS/8 file structured devices must be logically divided into  these

256-word  blocks.	 Hence,	 256  words is considered the standard OS/8

1-2

OS/8 CONCEPTS AND TERMINOLOGY

block size.  Some	devices,  like	RK8,  DECtape,	and  LINCtape,	are

physically	 divided  into blocks.	These physical blocks should not be

confused with the logical 256-word blocks.	 For example, DECtapes must

be	 formatted  with standard 129-word physical blocks.  A logical OS/8

block consists of the first 128  words  of	 two  consecutive  physical

DECtape  blocks.  The 129th word of every DECtape block is not used by

OS/8. Similarly, LINCtapes are formatted with 129 (or 128)	 words	per

block but never 256, as this format is unacceptable to OS/8.

A given OS/8 file consists of one or more	sequential  blocks  of	256

words  each  (consecutively numbered). A minimum of one block per file

is required, although a single file could occupy all of the blocks	 on

a device.

1.2.3  File Types

Three different types of files exist in the OS/8 system:

1.  An "empty file" is a contiguous area of unused blocks.  Empty

files are created when permanent files are deleted.

2.  A "tentative file" is a file that is open	 to  accept  output

and  has not yet been closed.  Only one tentative file can be

open on any single device at one time.

3.  A "permanent file" is a file that has been given a fixed size

and  is  no  longer  expandable.	 A  tentative  file becomes

permanent when it is closed.

To further understand file types, consider what occurs when a file	 is

created.	Normally, the User Service Routine, in creating a tentative

file, first locates the largest empty file	 available  and	 creates  a

tentative file in that space.  This establishes the maximum space into

which the file can expand.	 The user program then writes data into the

tentative	file.  At the end of the data, the program calls the USR to

close the tentative file, making it a permanent file.  The USR does so

and  allocates whatever space remains on the end of the tentative file

to a new, smaller, empty file.

1.2.4  File Directories and Additional Information Words

To maintain records of the files on a device, OS/8 allocates blocks  1

through  6	 of  each  file	 structured  device  as the file directory.

Entries in this directory inform the system of  the  name,	 size,	and

location  of  each	 file,	including all empty files and the tentative

file, if one exists.  For a detailed description of the entries in the

file directory, see Appendix A.

Each entry in a directory can, optionally, have  extra  storage  words

called   Additional  Information  Words.	The  number  of	 Additional

1-3

OS/8 CONCEPTS AND TERMINOLOGY

Information Words is determined at the time the directory is initially

created  (normally by using the /S or /Z features of PIP;	see Chapter

1 of the OS/8 HANDBOOK.

Whenever Additional Information Words are used, the first one for each

file  entry  is used to store the value of the System Date Word at the

time the file was created.	 OS/8 automatically uses one extra word per

entry  for	 the  date.   This value is set by executing a DATE command

(see Chapter 1 of the OS/8 HANDBOOK) which codes the current date into

memory location 07666 in the following format:

0	3 4	   8 9	     11

+---------+----------+----------+

|	 |	    |	       |

+---------+----------+----------+

MONTH	     DAY      YEAR-1970

(1-14(8))  (1-37(8))	(0-7)

A date word of 0 implies that no DATE command has been executed  since

the system initialization.

The values of  Additional	Information  Words  beyond  the	 first	are

user-defined.   See  Appendix  D for further information on Additional

Information Words.

1.3  CORE CONTROL BLOCK

Associated with each core image file (SV file)  is	 a  block  of  data

called  the  Core Control Block.  The Core Control Block is a table of

information containing the program's starting address, areas  of  core

into  which  the program is loaded, and the program's Job Status Word.

The Core Control Block is created at the time the program is loaded by

ABSLDR  or	 other	means  and  is written onto the SV file by the SAVE

operation.	 More information on the Core Control Block can be found in

the  description  of  core	 image	files  in  section A.2.2. Note that

specifying arguments to the SAVE command as described in Chapter 1	 of

the OS/8 HANDBOOK, can alter the contents of the target program's Core

Control Block.

When a program is loaded, the starting address and Job Status Word are

read  from the Core Control Block and saved in core.  The Core Control

Block itself is saved in the last 200 (octal) words of block 37 on the

system  device  unless  the program was loaded with the R (rather than

GET or RUN) command.

1-4

OS/8 CONCEPTS AND TERMINOLOGY

1.3.1  Program Starting Address

The current starting address (used by the START command) is stored	 in

two words at locations 07744 and 07745. The format of these words is:

LOCATION	 CONTENTS	 NOTES		 ________	 ________	 _____

07744	   62n3	      n is the field in

which to start.

07745	   nnnn	      Starting address of

the program.

1.3.2  Job Status Word

The  Job  Status  Word  contains  certain	flags  that   affect   OS/8

operations,  such	as  whether  to	 save  core when loading the USR or

Command Decoder.  The Job Status Word for	the  program  currently	 in

core   is	 saved	 at  location  07746  and  contains  the  following

information:

Bit Condition	   Meaning	       _____________	   _______

Bit 0 = 1	   File does not load into locations 00000

to 01777.

Bit 1 = 1	   File does not load into locations 00000

to 01777.

Bit 2 = 1	   Program must be reloaded before it can

be restarted.

Bit 3 = 1	   Program does not destroy the contents of

the highest existing memory field, an

optimization for the Batch system.

Bits 4 thru 9	   Reserved for future use.

Bit 10 = 1	   Locations 00000 to 01777 need not be

saved when calling the Command Decoder.

Bit 11 = 1	   Locations 10000 to 11777 need not be

saved when calling the USR.

When bit 2 of the Job Status Word is 1, any attempt to perform a START

(without an explicit address) results in a

NO!!

error message being printed.  As this bit is always zero in  the  Core

Control Block, the user program is expected to internally set this bit

(in location 7746) if a program is not  restartable.   This  could	 be

1-5

OS/8 CONCEPTS AND TERMINOLOGY

done as follows:

CDF 0

TAD I (7746	    /LOAD JOB STATUS WORD

AND (6777

TAD (1000

DCA I (7746	    /JOB IS NOT RESTARTABLE

The Job Status Word can be updated from the user's program or with the

ABSLDR /P	option,	 thus providing optimization of tape (disk) motion.

More information on the  Core  Control  Block  can	 be  found  in	the

description of Core Image (SV) files found in Appendix A.

Bit 3 of the JSW (Job Status Word) is used as an optimization for	the

Batch  operating  system.	 If  a	program can never cause the highest

existing memory field to be altered, this	bit  should  be	 set.	For

example,  EDIT,  PIP,  FORT,  and	SABR can never use memory above 8K.

Thus, they should set bit 3 of the	 JSW.	Programs  such	as  ABSLDR,

LOADER,  PAL8 and CREF can alter all of core.  They should perhaps not

have bit 3 on.  Note that the more core that exists, the more unlikely

it	 is  that a program will destroy upper core.  Thus, on 28K systems,

only the largest FORTRAN programs can alter field 6 and,  in  general,

bit 3 should be set.

1.3.3  Software Core Size

Location 07777 contains the software  core	 size  in  bits	 6-8.  This

represents the highest memory field available for use by OS/8. If bits

6-8 contain 0, all of the available memory is used.  Most	OS/8  cusps

interrogate  this word to determine how much memory is available.	The

other bits of this location are reserved for use by BATCH	and  should

not be touched by user programs.

1.4  DEVICE NAMES AND DEVICE NUMBERS

The OS/8 system can accommodate up to 15 separate devices.	 In Chapter

1	of  the	 OS/8  HANDBOOK	 the reader is introduced to the concept of

device names.  Briefly, each device on the	 system	 is  recognized	 by

either  a permanent device name (such as PTR or DTAl) which is created

when the system is built, or a user-defined device name determined	 by

an	 ASSIGN	 command.   The system insures that the user-defined device

name takes precedence.  For example,

.ASSIGN DSK DTA4

causes all future references to DTA4 to address the device DSK.

In calling the User Service Routine, a  device  can  be  alternatively

recognized by a device number.  Each device on the system has a unique

predefined number in the range 1 to 17 (octal) assigned  at  the  time

1-6

OS/8 CONCEPTS AND TERMINOLOGY

the  system  is  generated.   Thus,  user	programs have the choice of

referring to a device by either name or number.  Referencing a  device

by	 name  is  preferable,	as it maintains device independence for the

program.

Accessing devices by number should be done only when  the	appropriate

number  has been obtained from a USR INQUIRE CALL.	 Except for SYS and

DSK, the 05/8 peripherals do not have fixed numbers;  instead,  device

numbers  vary  whenever BUILD is used to modify a system.	Thus, it is

suggested that reference by name be used whenever possible.

To determine whether a  device  name  is  recognized  in  the  system,

attempt  to  ASSIGN  that	device.	  For example, to determine whether

LINCtape handlers are called LTA or DTA, perform:

.DEASSIGN

.AS LTA0

If the system responds with a dot (.), LTA0 does indeed exist.  If the

system responds with:

LTA0 NOT AVAILABLE

no device named LTA0 is present.

1.5  THE DEVICE AND FILENAME PSEUDO-OPS

Several of the USR functions  take	 device	 names	or  file  names	 as

arguments.	  To  simplify	the task of generating these arguments, the

DEVICE and FILENAME pseudo-ops have been added to the PAL8 Assembler.

A	device	name  consists	of  a  two  word  block,  containing   four

alphanumeric  characters  in  six-bit  ASCII  format.  A block in this

format can be created by the DEVICE pseudo-op as follows:

DEVICE DTA1

generates the following two words:

0424

0161

similarly, the FILENAME pseudo-op creates a four word block, the first

three  words  of  which  contain  the file name and the fourth word of

which contains the file extension.	 For example:

FILENAME PIP.SV

generates the following four words:

1-7

OS/8 CONCEPTS AND TERMINOLOGY

2011

2000

0000

2326

Note that positions for characters 4 through 6 are filled with zeros.

The DEVICE and  FILENAME  pseudo-ops  are	used  in  examples  in	the

following chapters.

1-8

CHAPTER 2

USER SERVICE ROUTINE

The User Service Routine, or USR, is a collection of subroutines which

perform  the  operations  of opening and closing files, loading device

handlers, program chaining, and calling the Command Decoder.  The	USR

provides  these  functions not only for the system itself, but for all

programs running under the OS/8 system.

2.1  CALLING THE USR

Performing any USR function is as simple as giving a JMS  followed	 by

the  proper  arguments.   Calls to the USR take a standardized calling

sequence.	This standard call should be studied before progressing	 to

the operation of the various USR functions.

2.1.1  Standard USR Call

In the remainder of this chapter, the following  calling  sequence	 is

referenced:

TAD VAL		   The contents of the AC is applicable	 in

some cases only.

CDF N		   Where N is  the  value  of  the  current

program field multiplied by 10 (octal).

CIF 10

JMS I (USR	   Where USR is either 7700 or	0200,  (see

section 2.1.2).

FUNCTION		   This word contains an integer from 1	 to

13	 (octal)   indicating	which	USR

operation is to be performed.

ARG(l)		   The number and meaning of these argument

words  varies  with	the  particular USR

function to be performed.

error return	   When	 applicable,  this  is	the  return

address for all errors.

normal return	   The operation was successful.  The AC is

cleared  and	 the  data  field is set to

current field.

2-1

USER SERVICE ROUTINE

This calling sequence can	change	from  function	to  function.	For

example,  some functions take no value in the AC and others have fewer

or	 greater  numbers  of  arguments.   Nonetheless,  this	format	 is

generally followed.

The value	of  the	 data  field  preceding	 the  JMS  to  the  USR	 is

exceedingly  important.   The  data  field	 MUST be set to the current

field, and the instruction field MUST be set to 1. Note that a CDF	 is

not  explicitly required if the data field is already correct.  When a

doubt exists as to the data field setting, an explicit CDF	 should	 be

executed.

There are three other restrictions which apply to all  USR	 calls,	 as

follows:

1.  The USR can never be called from any  address  between  10000

and 11777. Attempting to do so results in the:

MONITOR ERROR 4 AT xxxxx  (ILLEGAL USR CALL)

message and termination of program execution.  The  value	 of

xxxxx  is	 the  address  of the calling sequence (in all such

MONITOR ERROR messages).

2.  Several USR calls take address pointers as arguments.   These

pointers always refer to data in the same memory field as the

call.

3.  When calling the USR from field  1,  these  address  pointers

must  never  refer  to  data  that  lies in the area 10000 to

11777.

2.1.2  Direct and Indirect Calling Sequence

A user program can call the USR in two ways.  First, by  performing  a

JMS to location 17700 In this case, locations 10000 to 11777 are saved

on a special area on the system device, and the  USR  is  then  loaded

into  10000  to  11777. When the USR operation is completed, locations

10000 to 11777 are restored to their previous values.

NOTE

By setting bit 11 of the Job Status Word

to	a  1, the user can avoid this saving

and restoring of  core  when  preserving

core is unnecessary.

Alternatively, a program can opt to keep the USR permanently  resident

in	 core  at locations 10000 to 11777 by using the USRIN function (see

section 2.2.8). Once the USR has been brought into core,  a  USR  call

can  be  made  by performing a JMS to location 10200. This is the most

efficient way of calling the  USR.	  When	USR  operations	 have  been

2-2

USER SERVICE ROUTINE

completed,	 the  program  restores	 locations  10000 to 11777 to their

initial state by executing the  USROUT  function,	if  necessary  (see

section 2.2.9).

2.2  Summary of USR Functions

Function

Code	 Name			Operation	  ________	 ____			_________

1		 FETCH	   Loads  a  device  handler   into   core.

Returns   the   entry   address  of	the

handler.

2		 LOOKUP	   Searches  the  file	directory  on	any

device  to  locate a specified permanent

file.

3		 ENTER	   Creates and opens for output a tentative

file on a specified device.

4		 CLOSE	   Closes the currently open tentative file

on  the  specified  device and becomes a

permanent  file.   Also,  any   previous

permanent  file  with the same file name

and extension is deleted.

5		 DECODE	   Calls the Command Decoder.  The function

of  the  Command Decoder is described in

Chapter 3.

6		 CHAIN	   Loads a specified core image	 file  from

the system device and starts it.

7		 ERROR	   Prints an error message of the form USER

ERROR n AT LOCATION xxxxx.

10		 USRIN	   Loads the  USR  into	 core.	 Subsequent

calls to the USR are by an effective JMS

to location 10200.

11		 USROUT	   Dismisses the USR from core and restores

the previous contents of locations 10000

to 11777.

12		 INQUIRE   Ascertains whether a given device exists

and,	 if  so,  whether its handler is in

core.

13		 RESET	   Resets system tables	 to  their  initial

cleared state.

14-17		   Not	currently   used,   these   request

numbers are reserved for future use.

2-3

USER SERVICE ROUTINE

An attempt to call the USR with a code greater than  13  (octal)  will

currently	cause  a  Monitor  Error  4  message  to be printed and the

program to be aborted.

2.2.1  FETCH Device Handler	  Function Code = 1

Device handlers must be loaded into core so as to be available to	the

USR  and  user  program  for  I/O	operations  on that device.  Before

performing a LOOKUP, ENTER, or  CLOSE  function  on  any  device,	the

handler for that device must be loaded by FETCH.

The FETCH function takes two distinct forms:

1.  Load a device handler corresponding to a given device name.

2.  Load a device handler corresponding to a given device number.

First, the following is an example of loading a handler by	 name  from

memory field 0:

CLA	      /AC MUST BE CLEAR

CDF 0	      /DF = CURRENT FIELD

CIF 10	      /IF = 1

JMS I (USR

1	      /FUNCTION CODE = 1

DEVICE DTA3    /GENERATES TWO WORDS: ARG(1)

/AND ARG(2)

6001	      /ARG(3)

JMP ERR	      /ERROR RETURN

.	      /NORMAL RETURN

.

.

ARG(1) and ARG(2) contain the device name in standard format.  If	the

normal  return  is	 taken,	 ARG(2)	 is  changed  to  the device number

corresponding to the device loaded.   ARG(3)  contains  the  following

information:

Bits 0 to 4 contain the page number into which the handler is

loaded.

Bit 11 is 0 if the user program  can  only  accept  a  1-page

handler.

Bit 11 is 1 if there is room for a 2-page handler.

Notice that in the example above, the handler for DTA3 is to be loaded

into locations 6000 to 6177. If necessary, a two page handler could be

loaded;  the second page would be placed in locations  6200  to  6377.

After a normal return, ARG(3) is changed to contain the entry point of

the handler.

2-4

USER SERVICE ROUTINE

A different set of arguments is used to  fetch  a	device	handler	 by

number.  The following is an example of this form:

TAD VAL	      /AC IS NOT ZERO

CDF 0	      /DF = CURRENT FIELD

CIF 10	      /IF = 1

JMS I (USR

1	      /FUNCTION CODE = 1

6001	      /ARG(1)

JMP ERR	      /ERROR RETURN

.	      /NORMAL RETURN

.

.

On entry to the USR, the AC contains the device number in bits 8 to 11

(bit  0  to  7 are ignored). The format for ARG(1) is the same as that

for ARG(3) in the previous example.  Following a normal return  ARG(1)

is replaced with the entry point of the handler.

The conditions that can cause an error return to occur in	both  cases

are as follows:

1.  There is no device corresponding to the given device name	 or

device number, or

2.  An attempt was made to load a two page handler into one page.

If  this	is  an	attempt	 to  load  the handler by name, the

contents of ARG(2) have been changed already to the  internal

device number.

In addition, one of the  following	 Monitor  errors  can  be  printed,

followed by a return to the Keyboard Monitor:

Error Message			  Meaning	       _____________			  _______

MONITOR ERROR 4 AT xxxxx	Results if bits 8 to 11 of the AC

(ILLEGAL USR CALL)		are zero  (and	bits  0	 to  7	are

non-zero).

MONITOR ERROR 5 AT xxxxx	Results if a read error occurs

(I/O ERROR ON SYS)		while loading the device handler.

The FETCH function checks to see if the handler is in core, and if	 it

is	 not,  then  the  handler  and all co-resident handlers are loaded.

While the FETCH operation is essentially a simple one, the user should

be aware of the following points:

1.  Device handlers are always loaded into memory field 0.

2.  The entry point that is returned	may  not  be  on  the  page

desired.	 This  would  happen  if  the  handler were already

resident.

3.  Never attempt to load a handler into the 7600  page  or  into

page 0. Never load a two page handler into the 7400 page.

2-5

USER SERVICE ROUTINE

For more information on using device handlers, see Chapter 4.

NOTE

Two	 or   more   device   handlers	 are

"co-resident"   when   they	  are	both

included in the same  one  or  two	core

pages.   For  example,  the	 paper	tape

reader   and    punch    routines	 are

co-resident,  as  are  the eight DECtape

handler routines.

2.2.2  LOOKUP Permanent file	   Function Code = 2

This request locates a permanent file entry on a given device, if	one

exists.  An example of a typical LOOKUP would be:

TAD VAL	   /LOAD DEVICE NUMBER

CDF 0	   /DF=CURRENT FIELD

CIF 10	   /IF = 1

JMS I (USR

2	   /FUNCTION CODE = 2

NAME	   /ARG(1), POINTS TO FILE NAME

O	   /ARG(2)

JMP ERR	   /ERROR RETURN

.	   /NORMAL RETURN

NAME,     FILENAME PROG.PA

This request looks up a permanent file entry with	the  name  PROG.PA.

The  device number on which the lookup is to be performed is in AC bit

8 to 11. ARG(1) contains a pointer to the file name.   Note  that	the

file name block must be in the same memory field as the call, and that

it cannot be in locations 10000 to 11777. The device handler must have

been  previously  loaded  into  core.   If the normal return is taken,

ARG(l) is changed to  the	starting  block	 of  the  file	and  ARG(2)

contains  the  file  length  in  blocks  as a negative number.  If the

device specified  is  a  readable,	 non-file  structured  device  (for

example,  the  papertape  reader),	 then ARG(1) and ARG(2) contain the

file length in blocks as a negative number.  If the  device  specified

is a readable, non-file structured device (for example, the paper tape

reader), then ARG(1) and ARG(2) are both set to zero.

If the error return is taken, ARG(1) and ARG(2)  are  unchanged.	The

following conditions cause an error return:

1.  The device specified is a write-only device.

2.  The file specified was not found.

In addition,  specifying  illegal	arguments  can	cause  one  of	the

following	monitor	 errors,  followed  by	a  return  to  the Keyboard

Monitor:

2-6

USER SERVICE ROUTINE

Error Message			  Meaning	       _____________			  _______

MONITOR ERROR 2 AT xxxxx	Results if an I/O error occurred

(DIRECTORY I/O ERROR)		while reading the device directory.

MONITOR ERROR 3 AT xxxxx	Results if the device  handler	for

(DEVICE HANDLER NOT IN CORE)	the  specified	device	is  not	 in

core.

MONITOR ERROR 4 AT xxxxx	Results if bits 8 to 11 of  the	 AC

(ILLEGAL USR CALL)		are zero.

The LOOKUP function is the standard method of opening a permanent file

for input.

2.2.3  ENTER Output (Tentative) File	   Function Code = 3

The ENTER function is used to create a tentative file entry to be used

for output.  An example of a typical ENTER function is as follows:

TAD VAL		   /AC IS NOT ZERO

CDF 0		   /DF = CURRENT FIELD

CIF 10		   /IF = 1

JMS I (USR

3		   /FUNCTION CODE = 3

NAME		   /ARG(1) POINTS TO FILE NAME

0		   /ARG(2)

JMP ERROR	   /ERROR RETURN

.		   /NORMAL RETURN

.

.

NAME,     FILENAME PROG.LS

Bits 8 to 11 of the AC contain  the  device  number  of  the  selected

device;   the  device handler for this device must be loaded into core

before performing an ENTER function.  If bits 0 to 7  of  the  AC	are

non-zero,	this value is considered to be a declaration of the maximum

length of the file.  The ENTER function searches  the  file  directory

for the smallest empty file that contains at least the declared number

of blocks.	 If bits 0 to 7 of the AC  are	zero,  the  ENTER  function

locates the largest available empty file.

On a normal return, the contents  of  ARG(1)  are	replaced  with	the

starting  block  of the file.  The 2's complement of the actual length

of the created tentative file in blocks (which  can  be  equal  to	 or

greater  than  the	 requested  length)  replaces  ARG(2).	If the file

directory contains any Additional Information Words, the  system  DATE

(location	17666)	is written as the first Additional Information Word

of the newly created tentative file at this time.

2-7

USER SERVICE ROUTINE

NOTE

If	the  selected  device  is  not	file

structured but permits output operations

(e.g., the high speed punch), the  ENTER

operation	always	succeeds.   In	this

case, ARG(l) and ARG(2) are both  zeroed

on return.

If the error return is taken, ARG(1) and ARG(2)  are  unchanged.	The

following conditions cause an error return:

1.  The device specified by bits 8 to 11 of the AC is a read only

device.

2.  No empty file exists which  satisfies  the  requested  length

requirement.

3.  Another tentative file is already active on this device (only

one output file can be active at any given time).

4.  The first word of the file name was 0 (an illegal file name).

In addition, one of the following monitor errors can  occur,  followed

by a return to the Keyboard Monitor:

Error Message			   Meaning	       _____________			   _______

MONITOR ERROR 2 AT xxxxx	 Result if an I/O error occurred

(DIRECTORY I/O ERROR)		 while reading or writing the device

directory.

MONITOR ERROR 3 AT xxxxx	 Results if the device handler for

(DEVICE HANDLER NOT IN CORE)	 the  specified	 device	 is  not  in

core.

MONITOR ERROR 4 AT xxxxx	 Results if AC bits 8 to 11 are

(ILLEGAL USR CALL)		 zero.

MONITOR ERROR 5 AT xxxxx	 Read error on the system device

(I/O ERROR ON SYS)		 while bringing in the overlay	code

for the ENTER function.

MONITOR ERROR 6 AT xxxxx	 Results if a directory overflow

(DIRECTORY OVERFLOW)		 occurred  (no	room  for  tentative

file entry in directory).

2.2.4 The CLOSE Function		Function Code = 4

The CLOSE function has a dual purpose:  first, it is used to close the

current  active  tentative	 file, making it a permanent file.  Second,

when a tentative file becomes permanent it is necessary to remove	any

2-8

USER SERVICE ROUTINE

permanent file having the same name;  this operation is also performed

by the CLOSE function.  An example of CLOSE usage follows:

TAD VAL		   /GET DEVICE NUMBER

CDF 0		   /DF=CURRENT FIELD

CIF 10		   /IF=1

JMS I (USR

4		   /FUNCTION CODE = 4

NAME		   /ARG(1)

15		   /ARG(2)

JMP ERR		   /ERROR RETURN

.		   /NORMAL RETURN

.

NAME,     FILENAME PROG.LS

The device number is contained in AC bits 8 to  11	 when  calling	the

USR.   ARG(1)  is	a pointer to the name of the file to be deleted and

ARG(2) contains the number of blocks to be used for the new  permanent

file.

The normal sequence of operations on an output file is:

1.  FETCH the device handler for the output device.

2.  ENTER the tentative file on the output  device,  getting	the

starting block and the maximum number of blocks available for

the file.

3.  Perform the actual output using the device  handler,  keeping

track  of how many blocks are written, and checking to insure

that the file does not exceed the available space.

4.  CLOSE the tentative file, making	it  permanent.	 The  CLOSE

operation	 would	always	use the same file name as the ENTER

performed in step 2. The closing file length would have  been

computed in step 3.

After a normal  return  from  CLOSE,  the	active	tentative  file	 is

permanent	and  any  permanent  file  having  the	specified file name

already stored on the device is deleted.  If the specified device is a

non-file  structured device that permits output (the paper tape punch,

for example) the CLOSE function will always succeed.

NOTE

The user must be careful to specify	 the

same  file	names  to  the ENTER and the

CLOSE functions.  Failure to do  so	 can

cause   several   permanent	 files	with

identical  names  to   appear   in	 the

directory.	If CLOSE is intended only to

be used to delete  some  existing  file,

then the number of blocks, ARG(2) should

be zero.

2-9

USER SERVICE ROUTINE

The following conditions cause the error return to be taken:

1.  The device specified by bits 8 to 11 of the AC is a read only

device.

2.  There is neither an active tentative file to be made  into  a

permanent	 file, nor a permanent file with the specified name

to be deleted.

In addition, one of the following Monitor errors can occur:

Error Message			  Meaning	       _____________			  _______

MONITOR ERROR 1 AT xxxxx	Results if the length specified by

(CLOSE,ERROR)			ARG(2) exceeded the allotted space.

MONITOR ERROR 2 AT xxxxx	Results	 if  an	 I/O error occurred

(DIRECTORY I/O ERROR)		while reading or writing the device

directory.

MONITOR ERROR 3 AT xxxxx	Results if the device handler for

(DEVICE HANDLER NOT IN CORE)	the  specified	 device	 is  not  in

core.

MONITOR ERROR 4 AT xxxxx	Results if AC bits 8 to 11 are

(ILLEGAL USR CALL)		zero.

2.2.5  Call Command Decoder (DECODE)	Function Code = 5

The DECODE function causes the USR to load	 and  execute  the  Command

Decoder.	The  Command  Decoder accepts (from the Teletype) a list of

input and output devices and files, along with various  options.	The

Command  Decoder  performs	 a LOOKUP  on  all  input  files,  sets up

necessary tables in the top page of field 1, and returns to  the  user

program.  These operations are described in detail in Chapter 3, which

should be read before attempting to use the DECODE function.

A typical call to the Command Decoder looks as follows:

CDF 0		   /DF=CURRENT FIELD

CIF 10		   /IF=1

JMS I (USR

5		   /FUNCTION CODE = 5

2001		   /ARG(1), ASSUMED INPUT EXTENSION

0		   /ARG(2), ZERO TO PRESERVE ALL

/TENTATIVE FILES

.		   /NORMAL RETURN

.

.

ARG(1) is the assumed input extension, in the preceding example it	 is

2-10

USER SERVICE ROUTINE

".PA".  On	 return	 from the Command Decoder, information is stored in

tables located in the top page of memory field 1. The DECODE  function

also  resets  all	system	tables	as in the RESET function (see RESET

function,	section	 2.2.11)  if  ARG(2)  is  0  all  currently  active

tentative	files  remain  open;   if  ARG(2) is non-zero all tentative

files are deleted and the	normal	return	is  to	ARG(2)	instead	 of

ARG(2)+1.

The DECODE	 function  has	no  error  return  (Command  Decoder  error

messages are given in Chapter 3). However, the following Monitor error

can occur:

Error Message			  Meaning	       _____________			  _______

MONITOR ERROR 5 AT xxxxx	I/O error occurred while reading or

(I/O ERROR ON SYS)		writing on the system device.

2.2.6  CHAIN Function	  Function Code = 6

The CHAIN function permits a program to load and start another program

with  the restriction that the program chained to must be a core image

(.SV) file located on the system device.  A typical implementation	 of

the CHAIN function looks as follows:

CDF 0		   /DF=CURRENT FIELD

CIF 10		   /IF=1

JMS I (USR

6		   /FUNCTION CODE = 6

BLOCK		   /ARG(1), STARTING BLOCK NUMBER

There is no normal or error return from CHAIN.  However, the following

monitor error can occur:

Error Message			  Meaning	       _____________			  _______

MONITOR ERROR 5 AT xxxxx	I/O error occurred while reading or

(I/O ERROR ON SYS)		writing on the system device.

CHAIN ERR			If an attempt is made to CHAIN to a

file  which  is	 not  a	 core image

(.SV) file.  Control returns to the

keyboard monitor.

The CHAIN function loads a core  image  file  located  on	the  system

device  beginning	at  the	 block number specified as ARG(1) (which is

normally determined by performing a LOOKUP on the desired file  name).

Once loaded, the program is started at an address one greater than the

starting address specified by the program's Core Control Block.

CHAIN automatically performs a USROUT function (see section 2.2.9)	 to

dismiss the USR from core, and a RESET to clear all system tables see

section 2.2.11), but CHAIN does not delete tentative files.

2-11

USER SERVICE ROUTINE

The areas of core altered by the CHAIN function are determined by	the

contents  of  the	Core Control Block of the core image file loaded by

CHAIN.  The Core Control Block for the file is set up by other  ABSLDR

or	 LOADER	 programs.  It can be modified by performing a SAVE command

with specific arguments.  Every page of core in  which  at	 least	one

location  was  saved  is  loaded.	 If  the  page	is  one of the "odd

numbered" pages (pages 1, 3, etc.; locations 0200	to  0377,  0600	 to

0777,  etc.),  the previous page is always loaded. In  addition, CHAIN

always alters the contents of locations 07200 to 07577.

NOTE

CHAIN destroys a necessary part  of the

ODT resident  breakpoint routine.  Thus

an	ODT  breakpoint should	 never	 be

maintained across a CHAIN.

With the above exceptions, programs can pass data back  and  forth	 in

core while chaining.  For example, FORTRAN programs normally leave the

COMMON area in memory field 1 unchanged.  This COMMON area can then be

accessed by the program activated by the CHAIN.

2.2.7  Signal User ERROR	      Function Code = 7

The USR can be called to print a user error  message  for	a  program.

The following is a possible ERROR call:

CDF 0		   /DF = CURRENT FIELD

CIF 10		   /IF = 1

JMS I (USR

7		   /FUNCTION CODE = 7

2		   /ARG(1), ERROR NUMBER

The ERROR function causes a message of the form:

USER ERROR n AT xxxxx

to be printed.  Here n is the error number given as ARG(1); n must	 be

between  0	 and  11  (octal),  and	 xxxxx is the address of ARG(1). If

ARG(1) in the sample call above were at location 500 in field  0,	the

message:

USER ERROR 2 AT 00500

would be printed.	Following the message, the USR returns	control	 to

the Keyboard Monitor, preserving the user program intact.

The error number is arbitrary.  Two numbers  have	currently  assigned

meanings:

2-12

USER SERVICE ROUTINE

Error Message			  Meaning	       _____________			  _______

USER ERROR 0 AT xxxxx		During a RUN, GET,  or	R  command,

this  error  message indicates that

an error occurred while loading the

core image.

USER ERROR 1 AT xxxxx		While executing a FORTRAN or  SABR

program, this error indicates that

a call was  made to  a	subroutine

that was not loaded.

2.2.8  Lock USR in Core (USRIN)	   Function Code = 10

When making a number of calls to the USR  it  is  advantageous  for  a

program  to avoid reloading the USR each time a USR call is made.	The

USR can be brought into core and kept there for subsequent use by	the

USRIN  function.  The calling sequence for the USRIN function looks as

follows:

CDF 0		   /DF = CURRENT FIELD

CIF 10		   /IF = 1

JMS I (7700

10		   /FUNCTION CODE = 10

.		   /NORMAL RETURN

.

.

THE USRIN function saves the contents of locations 10000 to  11777	 on

the  system  scratch  blocks,  provided the calling program loads into

this area as indicated by the current JSW, and  loads  the	 USR,  then

returns control to the user program.

NOTE

If bit 11 of the current Job Status Word

is	a  one,	 the USRIN function will not

save the  contents	of  locations  10000

thru 11777.

2.2.9  Dismiss USR from Core (USROUT)	 Function Code = 11

When a program has loaded the USR into core with  the  USRIN  function

and  no  longer wants or needs the USR in core, the USROUT function is

used to restore the original contents of locations 10000 to 11777. The

calling sequence for the USROUT function is as follows:

2-13

USER SERVICE ROUTINE

CDF 0		   /DF = CURRENT FIELD

CIF 10		   /IF = 1

JMS I (200	   /DO NOT JMS TO 17700!!

11		   /FUNCTION CODE = 11

.		   /NORMAL RETURN

.

.

The  USROUT  function  and	 the  USRIN  function	are   complementary

operations.   Subsequent calls to the USR must be made by performing a

JMS to location 7700 in field 1.

NOTE

If bit 11 of the current Job Status Word

is	a  1,  the  contents of core are not

changed by the USROUT function.  In this

case  USROUT  is  a	 redundant operation

since core	was  not  preserved  by	 the

USRIN function.

2.2.10  Ascertain Device Information (INQUIRE) Function Code = 12

On some occasions a user may wish to determine  what  internal  device

number  corresponds  to  a	 given	device	name  or whether the device

handler for a specified device is in core, without actually performing

a	FETCH  operation.   INQUIRE performs these operations for the user.

The function call for INQUIRE  closely  resembles	the  FETCH  handler

call.

INQUIRE, like FETCH, has two distinct forms:

1.  Obtain the device number corresponding to a given device name

and  determine  if  the  handler	for  that device is in core

(example shown below).

2.  Determine if the handler	corresponding  to  a  given  device

number is in core.

An example of the INQUIRE call is shown below:

CLA		   /AC MUST BE CLEAR

CDF 0		   /DF = CURRENT FIELD

CIF 10		   /IF = 1

JMS I (USR

12		   /FUNCTION CODE = 12

DEVICE DTA3	   /GENERATES TWO WORDS:

/ARG(1) AND ARG(2)

0		   /ARG(3)

JMP ERR		   /ERROR RETURN

.		   /NORMAL RETURN

.

.

2-14

USER SERVICE ROUTINE

ARG(1) and ARG(2) contain the device name in  standard  format.   When

the  normal  return  is  taken  ARG(2) is changed to the device number

corresponding to the given name, and ARG(3) contains either the  entry

point  of	the device handler if it is already in core, or zero if the

corresponding device handler has not yet been loaded.

A slightly different set of arguments  is	used  to  inquire  about  a

device by its device number:

TAD VAL		   /AC IS NON-ZERO

CDF 0		   /DF = CURRENT FIELD

CIF 10		   /IF = 1

JMS I (USR

12		   /FUNCTION CODE = 12

0		   /ARG(1)

JMP ERR		   /ERROR RETURN

.		   /NORMAL RETURN

.

.

On entry to INQUIRE, AC bits 8 to 11 contain the device number.

NOTE

If AC bits O to 7 are non-zero, and bits

8  to  11  are  zero  (an illegal device

number) a:

MONITOR ERROR 4 AT xxxxx

message is printed and program execution

is terminated.

On normal return ARG(1) is set  to	 the  entry  point  of	the  device

handler  if it is already in core, or zero if the corresponding device

handler has not yet been loaded.  The error return in  both  cases	 is

taken  only  if there is no device corresponding to the device name or

number specified.

2.2.11  RESET System Tables	 Function Code = 13

There are certain occasions when it is desired  to	 reset	the  system

tables,  effectively removing from core all device handlers except the

system handler.  An example of the RESET function is shown below:

CDF 0		   /DF = CURRENT FIELD

CIF 10		   /IF = 1

JMS I (USR

13		   /FUNCTION CODE = 13

0		   /0 PRESERVES TENTATIVE FILES

.		   /NORMAL RETURN

.

.

2-15

USER SERVICE ROUTINE

RESET zeros all entries except the one for the system  device  in	the

Device  Handler  Residency	 Table	(see  section  B.3.3,  removing all

device handlers, other than that for the  system  device,	from  core.

This  should be done anytime a user program modifies any page in which

a device handler was loaded.

RESET has the additional function of  deleting  all  currently  active

tentative files (files that have been entered but not closed). This is

accomplished by zeroing bits 9 through 11 of every entry in the Device

Control Word Table (see section B.3.5).

If RESET is to be used in this last  fashion,  to	delete	all  active

tentative files, then ARG(1) must be non-zero and the normal return is

to ARG(1) rather than to ARG(1)+1. For  example,  the  following  call

would serve this purpose

CDF 0		   /DF:CURRENT FIELD

CIF 10		   /IF = 1

JMS I (USR

13		   /FUNCTION CODE = 13

CLA CMA		   /NON-ZERO:

The normal return would execute the CLA CMA and all  active  tentative

files on all devices would be deleted.  The Keyboard Monitor currently

does not reset the Monitor tables.	 If user programs which do not call

the  Command  Decoder  are	 used,	it  is wise to do a RESET operation

before loading device handlers.  The RESET will ensure that the proper

handler will be loaded into core.

2-16

CHAPTER 3

THE COMMAND DECODER

OS/8 provides a powerful subroutine called the Command Decoder for use

by all system programs.  The Command Decoder is normally called when a

program starts running.  When called, the Command Decoder prints an  *

and  then	accepts	 a  command  line  from	 the  console Teletype that

includes a list  of  I/O  devices,	 file  names,  and  various  option

specifications.   The  Command  Decoder validates the command line for

accuracy, performs a LOOKUP on all input files, and  sets	up  various

tables for the calling program.

The operations performed by the Command Decoder greatly  simplify	the

initialization  routines  of  all	OS/8 programs.	Also, since command

lines all have a standard basic structure, the Command  Decoder  makes

learning to use OS/8 much easier.

3.1  COMMAND DECODER CONVENTIONS

Chapter 1 of the OS/8 HANDBOOK describes the syntax  for  the  command

line  in  detail.	 A brief synopsis is given here only to clarify the

later discussion in this chapter.

The command line has the following general form:

*output files < input files/ (options)

There can be 0 to 3 output files and 0 to 9 input files specified.

Output File Format			       Meaning	  __________________			       _______

EXPLE.EX			Output to a file named EXPLE.EX	 on

device	 DSK   (the   default  file

storage device).

LPT:				Output to  the	LPT.   This  format

generally   specifies	a  non-file

structured device.

DTA2:EXPLE.EX			Output to a file named EXPLE.EX	 on

device DTA2.

DTA2:EXPLE.EX[99]		Output to a file named EXPLE.EX	 on

device	DTA2. A maximum output file

size of 99 blocks is specified.

null				No output specified.

3-1

THE COMMAND DECODER

An input file specification has one of the following forms:

Input File Format			     Meaning	  _________________			     _______

DTA2:INPUT			Input from a file named INPUT.df on

device	DTA2.  "df"  is the assumed

input file extension  specified	 in

the Command Decoder.

DTA2:INPUT.EX			Input from a file named INPUT.EX on

device	DTA2.	In  this  case	.EX

overrides the  assumed	input  file

extension.

INPUT.EX			Input from a file named INPUT.EX.

If there is no previously specified

input device, input is from  device

DSK,   the   default  file  storage

device;	   otherwise,	the   input

device	is  the	 same  as  the last

specified input device.

PTR:				Input from  device  PTR;   no  file

name   is   needed   for   non-file

structured devices.

DTA2:				Input from device DTA2 treated as a

non-file structured device, as, for

example, in the PIP command line:

*TTY:/L<DTA2:

In both of the last two formats, no

LOOKUP operation is performed since

the  device  is	  assumed   to	 be

non-file structured.

null				Repeats	 input	from  the  previous

device specified (must not be first

in input list, and must refer to  a

non-file  structured  device).	For

example:

* <PTR:,,

(two  null  files)  indicates  that

three paper tapes are to be loaded.

NOTE

Whenever a file extension is left off an

input  file	 specification,	 the Command

Decoder first performs a LOOKUP for	 the

given name appending a specified assumed

extension.	 If  the  LOOKUP  fails,   a

3-2

THE COMMAND DECODER

second  LOOKUP  is	made  for  the	file

appending a null (zero) extension.

The Command Decoder verifies that the  specified  device  names,  file

names, and extensions consist only of the characters A through Z and 0

through 9. If not, a syntax error is generated and the command line is

considered to be valid.

There are	two  kinds  of	options	 that  can  be	specified:   first,

alphanumeric  option  switches  are  denoted  by a single alphanumeric

character preceded by a slash (/) or a string of  characters  enclosed

in	 parentheses;	secondly,  a  numeric option can be specified as an

octal number from 1 to 37777777 preceded by an equal sign	(=).  These

options are passed to the user program and are interpreted differently

by each program.

Finally, the Command Decoder permits the command line to be terminated

by either the RETURN or ALT MODE key.  This information is also passed

to the user program.

3.2  COMMAND DECODER ERROR MESSAGES

If an error in the command line is detected by  the  Command  Decoder,

one  of  the  following  error  messages  is printed.  After the error

message, the Command Decoder starts a new line, prints an *, and waits

for another command line.	The erroneous command is ignored.

Error Message			 Meaning	       _____________			 _______

ILLEGAL SYNTAX		The  command  line   is	  formatted

incorrectly.

TOO MANY FILES		More than  three  output  files	 or

nine  input  files  were specified.

(Or in special mode,  more  than  1

output	file  or  more than 5 input

files.)

device DOES NOT EXIST		The specified device name does	not

correspond  to any permanent device

name or any  user  assigned  device

name.

name NOT FOUND		The specified input file  name	was

not found on the selected device.

3.3 CALLING THE COMMAND DECODER

The Command Decoder is initiated by the DECODE function  of  the  USR.

DECODE  causes  the  contents  of locations 0 to 1777 of field 0 to be

3-3

THE COMMAND DECODER

saved on the system scratch blocks, and Command Decoder to be  brought

into  that	 area  of core and started.  When the command line has been

entered and properly interpreted, the Command  Decoder  exits  to	the

USR,  which restores the original contents of 0 to 1777 and returns to

the calling program.

NOTE

By setting bit 10 of the Job Status Word

to	a  1  the user can avoid this saving

and restoring of core for programs	that

do not occupy locations 0 to 1777.

The DECODE call can reside in the area between 0000 to 1777 and  still

function correctly.  A typical call would appear as follows:

CDF 0	   /SET DATA FIELD TO CURRENT FIELD

CIF 10	   /INSTRUCTION FIELD MUST BE 1

JMS I (USR /USR=7700 IF USR IS NOT IN CORE

/OR USR=0200 IF USRIN WAS PERFORMED

5	   /DECODE FUNCTION = 5

2001	   /ARG(1),ASSUMED INPUT EXTENSION

0	   /ARG(2),ZERO TO PRESERVE

/ALL TENTATIVE FILES

.	   /NORMAL RETURN

.

.

ARG(1) is the assumed input extension.  If an input file name is given

with  no  specified  extension,  the  Command Decoder first performs a

LOOKUP for a file having the given name with  the	assumed	 extension.

If	 the LOOKUP fails, the Command Decoder performs a second LOOKUP for

a file having the given name and a null  (zero)  extension.   In  this

example, the assumed input extension is ".PA".

DECODE performs an automatic RESET operation (see section	2.2.11)	 to

remove  from  core	 all device handlers except those equivalent to the

system device.  As in the	RESET  function,  if  ARG(2)  is  zero	all

currently	 active	 tentative  files  are	preserved.   If	 ARG(2)	 is

non-zero, all tentative files are deleted and DECODE returns to ARG(2)

instead of ARG(2)+1.

As the Command Decoder normally handles all of its own  errors,  there

is no error return from the DECODE operation.

3.4  COMMAND DECODER TABLES

The Command decoder sets up various tables in the top page of field  1

that describe the command line typed to the user program.

3-4

THE COMMAND DECODER

3.4.1  Output Files

There is room for three entries in the output file table  that  begins

at location 17600. Each entry is five words long and has the following

format:

0  1	2  3  4	 5  6  7  8  9	10 11

+-------------------------------------+

WORD 1	| USER SPECIFIED	|4 BIT-DEVICE |

| FILE LENGTH		|    NUMBER   |

+-----------------+-----+-------------+

WORD 2	| FILE NAME	  |   FILE NAME	      |\

| CHARACTER 1	  |   CHARACTER 2     | |

+-----------------+-------------------+ |

WORD 3	| FILE NAME	  |   FILE NAME	      | \ OUTPUT FILE NAME

| CHARACTER 3	  |   CHARACTER 4     | / 6 CHARACTERS

+-----------------+-------------------+ |

WORD 4	| FILE NAME	  |   FILE NAME	      | |

| CHARACTER 5	  |   CHARACTER 6     |/

+-----------------+-------------------+

WORD 5	| FILE EXTENSION  |   FILE EXTENSION  |\ FILE EXTENSION

| CHARACTER 1	  |   CHARACTER 2     |/ 2 CHARACTERS

+-----------------+-------------------+

Bits 0 to 7 of word 1 in each entry contain the file  length,  if	the

file  length was specified with the square bracket construction in the

command line.  Otherwise, those bits are zero.

The entry for the first output file is in locations  17600	 to  17604,

the  second  is  in  locations  17605  to	17611,	and the third is in

locations 17612 to 17616.	If  word  1  of	 any  entry  is	 zero,	the

corresponding  output  file was not specified.  A zero in word 2 means

that no file name was specified.

Also, if word 5 of any entry is zero no file extension  was  specified

for  the  corresponding  file.  It is left to the user program to take

the proper action in these cases.

These entries are	in  a  format  that  is	 acceptable  to	 the  ENTER

function.

3.4.2  Input Files

There is room for nine entries in the input file table that begins	 at

location  17617.  Each  entry  is two words long and has the following

format:

3-5

THE COMMAND DECODER

0  1	 2  3  4  5  6	7  8  9	 10 11

+-------------------------+--------------+

WORD 1  | MINUS FILE		 | 4-BIT DEVICE |

| LENGTH			 |  NUMBER	|

+-------------------------+--------------+

WORD 2  | STARTING BLOCK OF FILE			|

+----------------------------------------+

Bits 0 to 7 of word 1 contain the file length as  a  negative  number.

Thus,  377 (octal) in these bits is a length of one block, 376 (octal)

is a length of two blocks,	 etc.	If  bits  0  to	 7  are	 zero,	the

specified	file  has a length greater than or equal to 256 blocks or a

non-file structured device was specified.

NOTE

This restriction to 255 blocks of actual

specified  size  can cause some problems

if the program has no way  of  detecting

end-of-file	 conditions.   For  example,

PIP cannot copy in image mode  any	file

on	a  file	 structured  device  that is

greater than 255 blocks  long,  although

it can handle in /A or/B modes (ASCII or

Binary) files of unlimited size.  In  /A

or/B  modes	 PIP  will detect the CTRL/Z

marking the end-of-file.

If this is liable to be a problem, it is

suggested  that  the user program employ

the special mode of the Command  Decoder

described in section 3.5 and perform its

own LOOKUP on the input files to  obtain

the exact file length.

The two-word  input  file	list  entries  beginning  at  odd  numbered

locations	from  17617 to 17637 inclusive.	 If location 17617 is zero,

no input files were indicated in the command line.	 If less than  nine

input  files were specified, the unused entries in the input file list

are zeroed (location  17641  is  always  set  to  zero  to	 provide  a

terminator even when no files are specified).

3.4.3  Command Decoder Option Table

Five words are reserved beginning	at  location  17642  to	 store	the

various  options  specified  in the command line.	The format of these

five words is as follows:

3-6

THE COMMAND DECODER

0	1  2  3	 4  5  6  7  8	9  10 11

+-----------------------------------+

17642 |	    HIGH ORDER 11 BITS OF	|

|		  = N OPTIONS		|

+--+--+--+--+--+--+--+--+--+--+--+--+

17643 |A |B |C |D |E |F |G |H |I |J |K |L |

|  |  |  |	|  |  |	 |  |  |  |  |	|

17644 |M |N |O |P |Q |R |S |T |U |V |W |X |

|  |  |  |	|  |  |	 |  |  |  |  |	|

17645 |Y |Z |0 |1 |2 |3 |4 |5 |6 |7 |8 |9 |

+--+--+--+--+--+--+--+--+--+--+--+--+

17646 | LOW ORDER 12 BITS OF = > OPTIONS	|

+-----------------------------------+

Each of these bits corresponds to one  of	the  possible  alphanumeric

option  switches.	 The  corresponding  bit  is  1	 if  the switch was

specified, 0 otherwise.

NOTE

If no  =  n	 option	 is  specified,	 the

Command  Decoder zeroes 17646 and bits 1

to 11  of  17642.  Thus,  typing  =0  is

meaningless	  since	  the  user  program

cannot  tell   that	  any	option	 was

specified.

Bit 0 of location  17642  is  0  if	 the

command   line   was   terminated  by  a

carriage return, 1 if it was  terminated

by an ALT MODE.

3.4.4  Example

To clarify some of the preceding, consider the interpretation  of	the

following command line:

*BIN[10]<PTR:,,DTA2:PARA,MAIN /L=14200$

If this command line is typed to PAL8, it would cause  assembly  of  a

Program  consisting of four separate parts:  two paper tapes, one file

named PARA.PA (or just PARA) on DTA2, and one file named  MAIN.PA	(or

just  MAIN)  also on DTA2. The binary output is placed on a file named

BIN.BN on device DSK, for which only 10 blocks need be allocated.	 No

listing  is  generated.   In addition, automatic loading of the binary

output is specified by the /L option, with the starting address  given

as 4200 in field 1. Finally, the line is terminated by the ALTMODE key

(which echoes as $) causing a return to the Keyboard Monitor after the

program is loaded.

In the case of this example, the Command Decoder returns to PAL8  with

the following values in the system tables:

3-7

THE COMMAND DECODER

NOTE

The entries for PTR (where no input file

name is specified) have a starting block

number and file size of zero.   This  is

always  true  of  the  input table for a

non-file structured device,	 or  a	file

structured	device on which no file name

is given.

3-8

THE COMMAND DECODER

+-------+

17600|  0242 |--	 DSK:IS DEVICE NUMBER 2

|	    |

|  0211 |\

|	    | |

|  1600 |  > FILE NAME IS BIN

|	    | |

|  0000 |/

|	    |

17604|  0000 |--	 NULL EXTENSION

|-------|

17605|	    |\

|	    | |	 REMAINING ENTRIES

~	    ~  > IN OUTPUT TABLES

|	    | |	 ARE ZERO

17616|_______|/

17617|	    |\

|  0016 | |

|	    |  > FIRST PTR INPUT

|  0000 | |

17620|_______|/

17621|	    |\

|  0016 | |

|	    |  > SECOND PTR INPUT

|  0000 | |

17622|_______|/

17623|  7667 |\

|	    | |	 DTA2: PARA PA IS 5 BLOCKS LONG,

|  0100 | |	 BEGINNING AT 100(8)

17624|_______|/

17625|	    |\

|  0007 | |	 DTA2: MAIN PA IS 256(10) OR MORE

|	    |  > BLOCKS LONG, BEGINNING AT BLOCK 105(8)

|  0105 | |

17626|_______|/

17627|	    |\

|	    | |	 REMAINING ENTRIES

~	    ~  > IN INPUT TABLES

|	    | |	 ARE ZERO.

17641|	    |/

|	    |

17642|  4001 |--	 LINE WAS TERMINATED BY ALT MODE

|	    |

17643|  0001 |\

|	    | |

17644|  0000 | \	 /L WAS ONLY OPTION SWITCH

|	    | /	 SPECIFIED

|	    | |

17645|  0000 |/

|	    |

17646|  4200 |--	 =14200 WAS SPECIFIED

+-------+

3-9

THE COMMAND DECODER

3.5  SPECIAL MODE OF THE COMMAND DECODER

Occasionally the user program does not want  the  Command	Decoder	 to

perform  the  LOOKUP  on  input files, leaving this option to the user

program itself.  Programs such as	format	conversion  routines  which

access non-standard file structures could use this special format.	 If

the input files  were  not	 OS/8  format,	a  command  decoder  LOOKUP

operation	would fail.  The capability to handle this case is provided

in the OS/8 Command Decoder.  This capability is generally referred to

as the "special mode" of the Command Decoder.

3.5.1  Calling the Command Decoder Special Mode

The special mode call to the  Command  Decoder  is	 identical  to	the

standard  DECODE  call  except  that the assumed input file extension,

specified by ARG(1), is equal to 5200. The value 5200  corresponds	 to

an	 assumed  extension  of	 ".*",	which  is  illegal.  Therefore, the

special mode of the Command Decoder  in  no  way  conflicts  with	the

normal mode.

3.5.2  Operation of the Command Decoder in Special Mode

In special mode the Command Decoder is loaded  and	 inputs	 a  command

line  as  usual.  The appearance of the command line is altered by the

special mode in these respects:

1.  Only one output file can be specified.

2.  No more than five input files can be specified,  rather  than

the nine acceptable in normal mode.

3.  The characters asterisk (*) and question mark (?)	 are  legal

in  file	names  and  extensions,	 both in input files and on

output files.  It is strongly suggested that these characters

be  tested  by the user program and treated either as special

options or as illegal file names.	 The user program  must	 be

careful  not  to	ENTER  an  output  file with an asterisk or

question mark in its name as such a  file	 cannot	 easily	 be

manipulated or deleted by the standard system programs.

The output and option table set up	 by  the  Command  Decoder  is	not

altered  in  special  mode.  Entries in the input table are changed to

the following format:

3-10

THE COMMAND DECODER

0  1	2  3  4	 5  6  7  8  9	10 11

+-----------------------+--------------+

WORD 1	|			| 4-BIT DEVICE |   BITS 0-7 ARE

|			| NUMBER       |   ALWAYS 0

+-----------------+-----+--------------+

WORD 2	| FILE NAME	  |   FILE NAME	       |\

| CHARACTER 1	  |   CHARACTER 2      | |

+-----------------+--------------------+ |

WORD 3	| FILE NAME	  |   FILE NAME	       | \ INPUT FILE NAME

| CHARACTER 3	  |   CHARACTER 4      | / 6 CHARACTER

+-----------------+--------------------+ |

WORD 4	| FILE NAME	  |   FILE NAME	       | |

| CHARACTER 5	  |   CHARACTER 6      |/

+-----------------+--------------------+

WORD 5	| FILE EXTENSION  |   FILE EXTENSION   |\  FILE EXTENSION

| CHARACTER 1	  |   CHARACTER 1      |/  2 CHARACTERS

+-----------------+--------------------+

The table entry for the first input file  is  in  locations  17605	 to

17611;  the second in locations 17612 to 17616; the third in locations

17617 to 17623; the fourth in locations 17624 to 17630; and the  fifth

in	 locations  17631 to 17635. A zero in word 1 terminates the list of

input files.  If word 2 of an entry is zero, no input  file  name	was

specified.

The OS/8 batch generating system  will  allow  calls  to  the  command

decoder in special mode.

3.6 CCL AND THE COMMAND DECODER

CCL uses its own copy of the  Command  Decoder  instead  of  the  copy

available from the monitor.  Thus, the CCL Command Decoder has several

options not available via standard	 USR  calls  to	 the  OS/8  Command

Decoder, e.g., multiple default extensions.

3.7 USEFUL LOCATIONS IN BATCH

BATCH will run whenever bit 0 of location 07777 is a 1. The  user	may

wish to access the following useful locations in BATCH.  The locations

are in the highest memory field available to OS/8:

BATERR = 7000     JMP here to abort BATCH.

BATOUT = 7400     JMS here to print character

in AC in BATCH log.

BATSPL = 7200     JMS here to permit spooling

with default extension in AC.

3-11

THE COMMAND DECODER

3.8 CCL TABLES

A description of all tables used by CCL is included in the file CCL.PA

supplied to all users of OS/8 version 3.

3-12

CHAPTER 4

USING DEVICE HANDLERS

A device handler is a system subroutine that is used by all  parts	 of

the  OS/8	system	and  by all standard system programs to perform I/O

transfers.	 All device handlers are called in the same  way  and  they

all  perform the same basic operation:  reading or writing a specified

number of 128 word records beginning at a selected core address.

These subroutines	effectively  mask  the	unique	characteristics	 of

different	I/O  devices from the calling program;	thus, programs that

use device handlers properly  are	effectively  "device  independent".

Changing devices involves merely changing the device handlers used for

I/O.

OS/8 device handlers have another important feature.  They are able to

transfer  a number of records as a single operation.  On a device like

DECtape this permits many blocks of data  to  be  transferred  without

stopping  the  tape  motion.   On	a  disk,  a  single operation could

transfer an entire	 track	or  more.   This  capability  significantly

increases	the  speed of operation of OS/8 programs, such as PIP, that

have large buffer areas.

NOTE

The word "record" is defined to mean 128

words  of  data;   thus,  an  OS/8 block

consists of two 128 word records.

4.1  CALLING DEVICE HANDLERS

Device handlers are loaded into a user selected area in memory field 0

by the FETCH function.  FETCH returns in ARG(1) the entry point of the

handler loaded.  The handler is called by	performing  a  JMS  to	the

specified entry point address.  It has the following format:

CDF N		 /WHERE N IS THE VALUE OF THE CURRENT

/PROGRAM INSTRUCTION FIELD TIMES 10 (OCTAL)

CIF 0		 /DEVICE HANDLER ALWAYS IN FIELD 0

JMS I ENTRY

ARG(1)	 /FUNCTION CONTROL WORD

ARG(2)	 /BUFFER ADDRESS

ARG(3)	 /STARTING BLOCK NUMBER

JMP ERR	 /ERROR RETURN

.		 /NORMAL RETURN (I/O TRANSFER COMPLETE)

.

ENTRY 0		 /ENTRY CONTAINS THE ENTRY POINT OF THE

/HANDLER, DETERMINED WHEN LOADED BY FETCH

4-1

USING DEVICE HANDLERS

As with calls to the USR, it is important that the data field  is	set

to	 the  current program field before teldevice handler is called.	 On

exit from the device handler, the data field will remain  set  to	the

current program field.

ARG(1) is the  function  control  word,  and  contains  the  following

information:

Bits			    Contents

Bit 0		      0 for an input operation,	 1  for	 an  output

operation.

Bits 1 to 5	      The  number  of  128  word  records   to	 be

transferred.   If	 bits  1-5 are zero and the

device is	 non-file  structured  (i.e.,  TTY,

LPT, etc.) the operation is device dependent.

If  the  device  is  file	 structured   (SYS,

DECtape,	disk,  etc.),  a  read/write  of 40

(octal) pages is performed.

Bits 6 to 8	      The memory field in which the transfer is	 to

be performed.

Bits 9 to 11	      Device dependent	bits,  can  be	left  zero.

Currently	 only  bit  11 is used;	 on DECtape

bit 11 determines the direction in which	the

tape  is	started.   If bit 11 is 0, the tape

starts in reverse.  If bit 11 is 1, the  tape

starts  forward.	 All  other handlers ignore

these bits at present (except TM8E and TA8E).

NOTE

Starting forward saves time as  long  as

the block number, ARG(3), is about seven

or more blocks greater than	 the  number

of	the  block  at	which  the  tape  is

currently positioned.

ARG(2) is the starting location of the transfer buffer.

ARG(3) is the number of the block on which the transfer is	 to  begin.

The  user	program	 initially  determines	this  value by performing a

LOOKUP or ENTER operation.	  After	 each  transfer	 the  user  program

should  itself  add  to  the current block number the actual number of

blocks transferred, equal to one-half the number of 128  word  records

specified, rounded up if the number of records was odd.

There are two kinds of error returns:  fatal and non-fatal.   When	 an

error return occurs and the contents of the AC are negative, the error

is fatal.	A fatal error can be caused by a parity error on  input,  a

write  lock  error	 on  output,  or an attempt to write on a read-only

device (or vice versa). The meaning can vary from	device	to  device,

4-2

USING DEVICE HANDLERS

but  in  all  cases  it  is  serious  enough to indicate that the data

transferred, if any, is invalid.

When an error return occurs and the contents of  the  AC  are  greater

than  or  equal  to  zero, a non-fatal error has occurred.	 This error

always indicates detection of the logical end-of-file.   For  example,

when  the paper tape reader handler detects the end of a paper tape it

inserts a CTRL/Z code in the buffer and takes the error exit with	the

AC	 equal	to  zero.   While all non-file structured input devices can

detect the end-of-file  condition,	 no  file  structured  device  can;

furthermore,  no  device handler takes the non-fatal error return when

doing output.

The following restrictions apply to the use of device handlers:

1.  If bits 1 to 5 of the  function  control	word,  ARG(1),	are

zero,  a	transfer  of  40  (octal) pages or an entire memory

field is indicated.  Care must be used  to  ensure  that	the

handler  is  not overlaid in this call.  This only applies to

file-structured handlers.

2.  The user program must never specify an input  into  locations

07600  to	 07777,	 17600	to  17777,  or	27600-27777, or the

page(s) in which	the  device  handler  itself  resides.	 In

general, 7600-7777 in every memory field are reserved for use

by system software.  Those areas should be used with caution.

3.  Note that the amount of data transferred is given as a number

of  128  word  records,  exactly	one  half of an OS/8 block.

Attempting to output an odd number of records can change	the

contents	of  the	 last  128 words of the last block written.

For example, outputting 128 words to a block on the RK8  disk

causes  the  last	 128  words  of the block to be filled with

zeroes.

4.  The specified buffer address does not have to  begin  at	the

start of a page.	The specified buffer cannot overlap fields,

rather the address will "wrap around" memory.  For example, a

write  of	 2  pages  starting  at	 location 07600 would cause

locations 07600-07777  and  00000-00177  of  field  0  to	 be

written.

5.  If bits 1-5 of the function control word ARG(1) are  zero,  a

device-dependent operation occurs.  Users should not expect a

40-page (full field) transfer of data.  The  CLOSE  operation

of  the  USR  calls the handler with bits 1-5 and 9-11 of the

function control word 0. This condition  means  'perform	any

special	close	operations  desired'.  Non-file	 structured

handlers which need no special handling on the conclusion	 of

data  transfers should treat this case as a NOP.	Examples of

usage of such special codes:

LPT - perform a form feed

CSAn, MTAn - write two file marks

4-3

USING DEVICE HANDLERS

4.2 DEVICE DEPENDENT OPERATIONS

This section describes briefly the operation of certain standard  OS/8

device    handlers,    including	normal	 operation,   any   special

initialization operations for block  0,  terminating  conditions,	and

response  to  control  characters	typed  at  the	keyboard.   Further

information on device handlers can be found in Chapter 5.

4.2.1  1-Page Terminal (TTY) (AS33)

1.  Normal Operation

This handler inputs characters from the terminal keyboard and

packs  them  into	 the  buffer or unpacks characters from the

buffer and outputs them to the console terminal.

On input, characters are echoed as they are typed.  Following

a carriage return, a line feed character is inserted into the

input buffer and printed on the terminal.

2.  Initialization for Block 0

None.

3.  Terminating Conditions

On input, detection of a CTRL/Z causes a CTRL/Z  (octal  code

232) to be placed in the input buffer, the remaining words of

the buffer to be filled with zeros, and a non-fatal error	 to

be  returned.   On output, detection of a CTRL/Z character in

the output buffer causes output  to  be  terminated  and	the

normal  return  to  be  taken.   There  are  no  fatal errors

associated with the 1-page terminal handler.

4.  Terminal Interaction

CTRL/C forces a  return  to  the	Keyboard  Monitor.   CTRL/Z

forces  an  end-of-file  on  input (see 3). CTRL/O terminates

printing of the contents of the current buffer on output.

4.2.2  High-Speed Paper Tape Reader (PTR)

1.  Normal Operation

This handler inputs characters from the high-speed paper tape

reader and packs them into the buffer.

2.  Initialization for Block 0

The handler prints an up-arrow (^) on the terminal and  waits

4-4

USING DEVICE HANDLERS

for  the	user  to load the paper tape reader.  By typing any

single character (except CTRL/C) the user	 initiates  reading

of the paper tape.

NOTE

On some terminals, up-arrow is  replaced

by the circumflex (^) character.

3.  Terminating Conditions

Detection of  an	end-of-tape  condition,	 indicated  by	the

failure  to  get	a  character in a specified period of time,

causes a CTRL/Z to be entered in the  buffer,  the  remaining

words  of the buffer to be filled with zeros, and a non-fatal

error to be returned.  Attempting output to  the	paper  tape

reader causes a fatal error to be returned.

4.  Terminal Interaction

Typing CTRL/C forces a return to the Keyboard Monitor.

4.2.3  High-Speed Paper Tape Punch (PTP)

1.  Normal Operation

This handler unpacks characters from the	output	buffer	and

punches them on the paper tape punch.

2.  Initialization for Block 0

None.

3.  Terminating Conditions

Attempting to input from the paper tape punch causes a  fatal

error   to  be  returned.	  There	 are  no  non-fatal  errors

associated with this handler.

4.  Terminal Interaction

Typing CTRL/C forces a return to the  Keyboard  Monitor,	but

only when actual punching has begun, or if ^C is typed before

punching commences.  If the punch is off	line,  ^C  is  only

effective immediately before punching would begin.

4.2.4  Line Printer (LPT)	(LPSV)

1.  Normal Operation

4-5

USING DEVICE HANDLERS

This handler unpacks characters from the	buffer	and  prints

them  on	the  line  printer.   The characters horizontal tab

(ASCII 211)  causes  sufficient  spaces  to  be  inserted	 to

position	the  next  character  at a "tab stop" (every eighth

column, by definition). The  character  vertical	tab  (ASCII

213)  causes  a  skip to the next paper position for vertical

tabulation  if  the  line	 printer  hardware  provides   that

feature.	 The  character form feed (ASCII 214) causes a skip

to the top of the next page.  Finally, the handler  maintains

a	 record	 of  the current print column and starts a new line

after 80 or 128 columns  have  been  printed.   This  handler

functions	 properly  only on ASCII data.	The handler for the

LS8E line printer handler	 utilizes  the	expanded  character

capability  of  the  printer.   If  a  216 (CTRL/N) character

appears anywhere in a  line  of  text,  the  entire  line	 is

printed in the expanded character mode.  The 216 must be used

on a line-by-line basis.

2.  Initialization for Block 0

Before printing begins, the line	printer	 handler  issues  a

form feed to space to the top of the next page.

3.  Terminating Condition

On detection of a CTRL/Z character in the	 buffer,  the  line

printer  handler issues a form feed and immediately takes the

normal return.  Attempting to input  from	 the  line  printer

forces  a	 fatal error to be returned.  A fatal error is also

returned if the line printer error flag is set.  There are no

non-fatal errors associated with the line printer handler.

4.  Terminal Interaction

Typing CTRL/C forces a return to the Keyboard Monitor.

5.  Patching the LPSV Handler

The following patches are available for the LPSV line printer

handler.

rel. loc 0: set to -printer width-l (i.e. set to	-121(octal)

for  an  80  column  line

printer)

rel. loc 1: set to 4		  for	 the	LV8E   line

printers

set to 14				  for  the  LP08  and  LS8E

printers

rel. loc 2: set to -40		  to convert lower case	 to

upper case

set to 0				  if your printer can print

lower case

4-6

USING DEVICE HANDLERS

4.2.5 Cassettes

1. Normal Operation

This handler performs character I/O between the cassettes and

the  buffer.   It	 treats	 cassettes as a non-file structured

device.  Data appears on cassette in 192-byte records.

2.  Initialization for Block 0

On input the cassette is rewound.	 On output the cassette	 is

rewound and a file gap is written.

3.  Terminating Condition

An end-of-file on input is a software error.

4.  Terminal Interaction

Typing CTRL/C forces a return to the Keyboard monitor.

5.  Special Codes (device dependent features)

If the handler is called with bits 1-5 of the  function  word

=0, then bits 10-11 are examined.	 The meaning of these codes

are as follows:

0	   write a file gap

1	   rewind also, then write a file gap if bit 0=1

2	   space backwards one record

3	   skip one file (direction depends on bit 0)

NOTE

The handler	 neither  reads	 nor  writes

standard  files.   It  is merely a paper

tape replacement.  It  writes  raw	data

(organized	into  192-byte records) onto

the cassettes starting at the beginning;

and then later reads it back.

The source is already in OS/8 BUILD format.  The handler has only	two

entry  points (for drives A and B of a controller). The decision as to

which controller it uses is made at  assembly  time  by  changing	the

symbol code.  The result is as follows:

CODE     DEVICE NAME   HANDLER	DEVICE CODE	    ____     ___________   _______	___________

0		TA8A	   A:CSA0	    70

B:CSA1

1		TA8B	   A:CSA2	    71

B:CSA3

4-7

USING DEVICE HANDLERS

2		TA8C	   A:CSA4	    72

B:CSA5

3		TA8D	   A:CSA6	    73

B:CSA7

The handler has the internal device code of  27  (see  Table  2-12	 in

Chapter 2 of the OS/8 HANDBOOK.  The handler is two pages long.

4.2.6  Card Reader (CDR)

1.  Normal Operation

This handler reads characters from the card reader and  packs

them  into the input buffer.  Trailing spaces (blank columns)

on a card are deleted from input.	  The  handler	can  accept

only  alphanumeric  format data on cards (the DEC029 standard

card codes are used).

2. Initialization for Block 0

None.

3. Terminating Conditions

A card which contains an underline character in column 1	(an

0-8-5   punch)   with  the  remaining  columns  blank  is	 an

end-of-file card.	 In addition, after reading each  card	the

handler  checks to see if a CTRL/Z was typed at the keyboard.

After either an end-of-file card or a CTRL/Z being  typed,  a

CTRL/Z  is inserted in the buffer, the remaining words of the

input buffer are filled with zeros, and a non-fatal error	 is

returned.	  Attempting  to output to the card reader causes a

fatal error to be returned.

4. Terminal Interaction

Typing CTRL/C forces a return to the Keyboard Monitor.

Typing CTRL/Z forces an end-of-file to occur (see 3.).

4.2.7 TM8-E Handler

1. Normal Operation:

When the handler is used	in  its	 normal	 mode,	single-file

mode, magtapes may consist of exactly one file.  It starts at

the beginning of the tape and consists of consecutive records

until an end-of-file mark (EOF) is reached.  In this sense, a

magtape is similar to one big paper tape.	 This is  the  same

way that OS/8 currently treats cassettes.

4-8

USING DEVICE HANDLERS

Since the capacity of magtapes is	 so  big,  provisions  have

been  made  for  storing	multiple files per tape.  In such a

structure, several files may exist on one magtape.  They	are

unlabeled	 and are separated from each other by a single file

mark.  The last one is followed  by  two	file  marks.   Each

'file'  looks  like  a  paper  tape.   It	 is referenced in a

non-file structured manner.   The	 magtape  handler  must	 be

altered first to work in file mode.  Then the magtape must be

positioned to exactly the correct	 spot  where  the  read	 or

write  operation	will  commence.	  This may be done with any

program using  the  auxiliary  capabilities  of  the  magtape

handler  (described  below), or the positioner program, CAMP.

To read a file, the handler must be positioned to just before

the first data record of that file.  To write file #1, rewind

the tape (i.e., be at BOT).  To  write  file  #n,	 (n>1)	the

handler  should  be positioned after the (n-1)st file mark on

the tape.	 Previous file n and all files past it then  become

unreadable (non-existent).

A. Device-Dependent Capabilities:

The TM8-E handler has several	 auxiliary  features  which

may  be invoked by a user program which calls the handler

in a device-dependent manner.	 These features all rely on

the  contents of bits 9-11 of the function word (argument

1 of the handler call) and some  require  argument  3	 in

addition.

These features are brought to life whenever  the  handler

is  called  with  a  page  count  of	0  (bits 1-5 of the

function word).  Call bits 9-11 of the function word, the

Special Function Register (SFR) for short, and also refer

to bit 0 of the function word as the direction  bit.	 If

the  page  count  is	not  0, the contents of the SFR are

ignored.

If the page count is 0, then the SFR	together  with	the

direction  bit  (and	possibly argument 3) determine what

special function to perform, as follows:

SFR		     OPERATION		  ___		     _________

0.  CLOSE.  Write two EOF's.

1.  Rewind.

2.  Space  forward/reverse  records.	 The  direction	 to

space  is	 determined  by	 the direction bit (0 means

space forward, 1 means space reverse).  The  negative

(two's  complement) of the number of records to space

over is given by argument 3 of the handler call.	(-1

means  space  past one record, 0 means 4096 records.)

The error return is taken if either a  file  mark	 or

BOT is encountered.  In such cases, you would be left

4-9

USING DEVICE HANDLERS

positioned at the beginning of a file.

3.  Space forward/reverse files.  The direction to  space

is  determined  by  the  direction bit (0 means space

forward, 1 means space reverse). The negative of	the

number  of  file	marks  to  space  past	is given by

argument 3 (-1 means space past  one  file  mark;	  0

means  4096 file marks). In reverse mode, the tape is

left positioned at the end of a file;   an  error	 is

given  if	 BOT  is encountered.  In forward mode, the

tape is left positioned at the beginning of  a  file.

If EOD is reached, the handler automatically performs

a backspace record to leave you between the two  file

marks;  no error is given.

4.  Rewind the unit and put drive off-line.

5.  Write a single EOF.

6.  Special read/write function.  The direction  bit	(as

usual)  determines  read	or  write  (0 means read, 1

means  write).  The  specified   I/O   operation	 is

performed	  between   the	 user's	 buffer	 (start	 is

specified by argument 2) and the	very  next  magtape

record.	Only  one  record  is  transferred  and the

user's buffer must be large  enough  to  contain	it.

The  record  length  is  specified by the negative of

argument 3 (in words).  0 means a	 record	 length	 of

4096.

7.  Unused.  Reserved for future use.	 If  specified,	 it

currently acts as a NO-OP.

In each case, the unit affected is determined  by	the  handler  entry

point.

B. Other Common Operations:

(a) To backspace n files, use Special Code 3 to pass over

n+1  file marks backwards, then use Special code 2 to

advance (forward) over one record (EOF) ignoring	the

EOF error.

(b) To advance to EOD, first perform a backspace  of	one

record  (or  perform  a rewind to play safe) then use

special code 3 to advance	 over  4096  files  in	the

forward direction (argument 3=0).

2.  Special Handling for Block 0

If the handler is called to read or write block  0,  it  will

first  perform  a rewind.	 This feature can be patched out if

4-10

USING DEVICE HANDLERS

desired by altering relative location 1 from a 0 to a 1. This

altered  handler	should	be  operating  in  file	 mode.	The

original handler should be operating in single-file mode.

A.  Special Handling for CLOSE:

A close operation is signaled to the handler	by  calling

it with a function word which has a page Count of 0 (bits

1-5) and which has bits 9-11 all zeroes.  This is how the

USR  CLOSE  operation	 calls	the handler (OS/8 V3 only).

This causes the handler  to  write  two  Successive  file

marks	 on the tape.  Two Successive EOF's is the software

indication of end-of-data (EOD).

B.  Restrictions:

In single-file mode,	should	not  have  more	 than  4095

blocks  because  on trying to write the 4096th block, the

handler will think it's writing block	 0  and	 perform  a

rewind.   This  restriction does not apply when using the

handler in file-mode;	 but beware, some  CUSPs,  Such	 as

PIP,	are  suspected to behave strangely on block 4096 of

non-file-structured devices.

3.  Terminating Conditions

None.

4.  Keyboard Interaction:

Typing ^C on the keyboard while the handler is  in  operation

causes  the  handler to abort and return to the OS/8 keyboard

monitor via location 7600. Such action is	 ill-advised  since

it leaves the magtape without an end-of-file indicator.

5.  Error Conditions:

On a hard error, the handler takes the error return  (with  a

negative	AC)  and  the  AC contains the Contents of the main

status register, as follows:

Bit on	      Meaning		   ______	      _______

0		   Error flag

1		   Tape rewinding

2		   BOT

3		   Select error

4		   Parity error (vertical, longitudinal, or

CRC)

5		   EOF

6		   Record length incorrect

7		   Data request late

8		   EOT

9		   File protect

10		   Read compare error

11		   Illegal function

4-11

USING DEVICE HANDLERS

4.2.8  File-Structured Devices

1.  Normal Operation

(DECtape, LINCtape, TD8E DECtape, DF32, RF08, and RK8, RK8E)

These handlers transfer data directly between the device	and

the buffer.

2.  Initialization for Block 0

None.

3.  Terminating Conditions

A fatal error is returned whenever the transfer causes one of

the error flags in the device status register to be set.	For

example, a  fatal	 error	would  result  if  a  parity  error

occurred	on input, or a write lock error occurred on output.

The device handlers generally try three times to perform	the

operation	 before	 giving	 up  and  returning  a fatal error.

There are no non-fatal errors associated with file structured

devices.

4.  Terminal Interaction

Typing CTRL/C forces a return to the Keyboard Monitor.

NOTE

The	 system	 device	 handler  does	 NOT

respond to a typed CTRL/C.

4.2.9  TD8E DECtape

TD8E DECtape is the new accumulator transfer DECtape.  Since OS/8 is a

noninterrupt  driven  system,  TD8E  DECtape  has	data transfer rates

equivalent to those for TC08 DECtape;  however, the  interrupt  should

never  be	used  with  the	 TD8E.	Device handlers for TD8 DECtape are

supplied as a standard part of OS/8.  Each pair of drives (0 and 1,  2

and  3,  etc.)  requires  a 2-page	 device handler.  Thus, to have all

eight TD8E drives in the system at one time will require four separate

handlers.	 Thus for TD8E, it is wise to restrict usage to those units

that physically exist.  Also, the tape drives are hardwired to  select

one  of  two  possible  unit  numbers;  thus, the first pair of drives

installed must be called units 0 to 1. Any others numbers will cause a

SELECT  error.   In  this	case,  the computer hangs until the correct

drive is selected.

4-12

USING DEVICE HANDLERS

4.2.10 KL8E Terminal Handler

Listed  are  the  features	 of  the  KL8E	handler.   Those  that	are

conditional are marked by an asterisk:

1.  It reads a line at a time.  Whenever the user  types  CR,	 it

enters  CR,  LF into the buffer;	it echoes CR, LF;  and then

pads the remainder of the buffer with nulls  and	returns	 to

the calling program.  The characters get put into the buffer,

one character per word.  Thus every third character is a null

as far as OS/8 is concerned.

2.  RUBOUT deletes the previous character.  It echoes as either a

back  slash  (\) or as the character rubbed out, depending on

assembly parameters.  RUBOUT at the beginning of a line  acts

as ^U.

3.  CTRL/U echoes as ^U and erases the current line, allowing the

user  to	retype	it.   (It  also	 echoes CR, LF.) The buffer

pointer is reset to the beginning of the buffer.

4.  CTRL/Z  echoes  as  ^Z  (followed	 by  CR,  LF)  and  signals

end-of-input.   The ^Z enters the buffer and the remainder of

the buffer is padded with nulls.	The error return  is  taken

with a positive AC (non-fatal error).

5.  Nulls are ignored.

*6.  The altmode characters (octal 175 and 176) are  converted	 to

escapes (octal 33).

*7.  Lower-case characters typed may be automatically converted to

upper case.

8.  CTRL/C echoes as ^C and  returns	control	 to  the  keyboard

monitor via location 07600.

On output:	 (either normal output or when echoing input)

1.  CTRL/C on keyboard echoes as ^C and returns  control  to	the

keyboard monitor via location 7600.

2.  CTRL/O on keyboard stops further echoing.	 All echoing ceases

(through possibly many buffer loads) until either the handler

is reloaded into core or the user	 types	a  character  other

than to on the keyboard.	Not operative during input.

3.  ^S causes the handler to stop sending to	the  terminal.	 No

characters  are  lost  and  outputting  resumes  when a ^Q is

typed.   ^S  and	^Q  do	not  echo.   These  characters	are

operative	 only upon output.  On input, they are treated like

any other input characters.  This	 is  very  useful  on  high

speed CRT displays.

4-13

USING DEVICE HANDLERS

4.  Nulls are ignored.

*5.  Lower case characters may be optionally printed as upper case

and flagged with an apostrophe.

*6.  Tabs may be handled in one of three manners:

a.  Output as actual tabs,

b.  Output as actual tab followed by padding of two rubouts,

c.  Output as the correct number of spaces to bring the  text

to the start of the next tab stop.

7.  Whenever the output line reaches the end of the physical line

(length  set  at	assembly  time),  the handler automatically

performs a carriage-return line-feed.

*8.  The escape character (octal 33) prints as a dollar sign.

*9.  The handler may be set to delay about 16 ms after typing	any

character	 (specified  at	 assembly  time), for example, line

feed.

*10.  Control characters are printed as their corresponding  letter

preceded by an up-arrow.	Thus CTRL/K prints as ^K.

4-14

CHAPTER 5

RECONFIGURING THE OS/8 SYSTEM

It is sometimes necessary to construct an OS/8 system from scratch, or

to make a new peripheral device available to OS/8. Both of these tasks

are a part of reconfiguring the OS/8 system.   OS/8  BUILD,  which	 is

described in detail in Chapter 2 of the OS/8 HANDBOOK, allows the user

quickly and easily to build a  new	 system	 or  to	 alter	the  device

complement of an existing system.

5.1  WRITING DEVICE HANDLERS

A device handler is a page-independent subroutine	one  or	 two  pages

long.   The device handler must run properly in any single page or two

contiguous pages in field 0 (except 0000 to 0177 or 7600 to 7777). All

device handlers have the same calling sequence:

CDF N		/N IS CURRENT FIELD TIMES 10 (OCTAL)

CIF 0		/DEVICE HANDLER LOCATED IN FIELD 0

JMS I ENTRY	/ENTRY IS DETERMINED BY SR "FETCH"

FUNCTION	/FUNCTION IS BROKEN DOWN AS FOLLOWS:

/BIT 0 = 0 FOR READ

/BIT 0 = 1 FOR WRITE

/BITS 1 TO 5 = NUMBER OF PGS TO TRANSFER

/BITS 6 TO 8 = FIELD FOR TRANSFER

/BITS 9 TO 11 = DEVICE DEPENDENT BITS

BUFFER	/CORE ADDRESS OF TRANSFER BUFFER

BLOCK		/BLOCK NUMBER TO START TRANSFER

ERROR		/ERROR RETURN, AC>=0 MEANS END-OF-FILE

AC<0 MEANS FATAL ERROR

NORMAL	/NORMAL RETURN

The device handler reads or  writes  a  number  of	 128  word  records

beginning	at  the selected block.	 In general, device handlers should

conform to the following standards:

1.  On normal return from a device handler the AC is zero and the

DATA FIELD is always restored to its original entry value.

2.  Although the starting block number has true significance only

for file structured devices, handlers for non-file structured

devices can check the block number and perform initialization

if  the  block number is zero.  For example, the line printer

handler  outputs	a  form	 feed  before  printing	 when	the

specified block number is zero.

5-1

RECONFIGURING THE OS/8 SYSTEM

3.  Handlers should be written to be	as  foolproof  as  possible

checking	for  the most common errors in the calling program.

Examples of typical user errors  are:  calling  handler  with

non-zero	AC (always perform a CLA in the handler); trying to

read on a write only device, or trying to	 write	on  a  read

only  device (gives a fatal error return); specifying 0 pages

to be transferred (accept as meaning no actual transfer is to

take  place);  or	 attempting  to	 access a nonexistent block

(gives a fatal error return).

4.  Device handlers normally check to see if a CTRL/C (ASCII 203)

has  been	 typed	at  the system console by the user.  If one

has, the handler aborts I/O and JMP's  to	 location  7600	 in

field  0.	 The seven low order bits of the keyboard should be

checked for a 3 so as to allow parity terminals.	KRS  should

be  used	over  KRB so that any paper tape in the reader will

not be advanced if its character is not ^C.

5.  Device handlers should  be  able	to  detect  standard  error

conditions   like	  checksum   or	 parity	 errors.   Whenever

possible, several attempts to perform the transfer should	 be

made  before  aborting  I/O  and	taking	the error exit.	 In

addition, when operator intervention is required, the handler

would  normally  wait for the action rather than take a fatal

error exit.  For example, if the	paper  tape  punch  is	not

turned  on,  the PTP handler waits for the punch to be turned

on.

6.  By convention, in any handler for	 a  device  (like  DECtape)

that  can	 search either forward or backward for a block, Bit

11 of the function word (one of  the  device-dependent  bits)

controls the starting direction of the search.  Bit 11 is a 1

if the starting direction	 is  forward  and  a  0	 if  it	 is

reverse.	 The  other  two  device  dependent  bits  are	not

assigned any significance at the present time.

7.  Remember that the user specifies a multiple of 128  words	 to

transfer,	 whereas  the transfer starts at the beginning of a

128 or 256 word block.  This  means  that	 the  handler  must

provide  that capability of reading or writing the first half

of a block.  Writing the first half of the block	causes	the

contents	of the second half of the block to be altered.	For

example, writing 128 words to the RK8 disk (256 word  blocks)

causes the second half of the block to be filled with zeroes.

This is usually done by the hardware controller.

8.  The entry point to a two page device handler must be  in	the

first page.

9.  A number of handlers (maximum of 15 decimal) can be  included

in the one or two pages of code.	Where more than one handler

is included in a single handler subroutine, the handlers	are

called  co-resident.  Co-resident handlers are always brought

5-2

RECONFIGURING THE OS/8 SYSTEM

into core together.  For example, all eight DECtape  handlers

fit   into   one	page;	hence,	the  DECtape  handlers	are

co-resident.  One restriction on co-resident handlers is that

if  they	are  two pages long all entry points must be in the

first page.

10.  The USR, while doing file operations, maintains in  core	the

last  directory  block  read in order to reduce the number of

directory reads necessary.  The proper  functioning  of  this

feature  depends	on  the	 fact  that  every  handler  for  a

file-structured device on a single system has a unique  entry

point relative to the beginning of the handler.  The relative

entry points currently assigned for file structured  handlers

are:

Device Handlers		  Relative Entry Points		 _______________		  _____________________

System Device Handler		     7

DECtape, LINCtape or TD8E DECtape	     10 to 17

RKA0				     20

RKA1				     21

RKA2				     22

RKA3				     23

RK08 or DF32			     24

Reserved for user			     40 to 67

11.  If the device is block oriented (such as	DECtape,  LINCtape,

or  Disk),  then	the handler transfers data directly with no

alteration.  However, if the  device  is	character  oriented

(such as a paper tape reader, Teletype, or line printer), the

handler is required to pack characters  into  the	 buffer	 on

input and unpack them on output.	The standard OS/8 character

packing format puts three 8-bit characters into two words	 as

follows:

+---------------+------------------+

WORD 1 |CHARACTER 3    |			 |

|	  BITS 0-3    |	  CHARACTER 1	 |

+---------------+------------------+

WORD 2 |CHARACTER 3    |			 |

|	  BITS 4-7    |	  CHARACTER 2	 |

+---------------+------------------+

0	    3  4	       11

For example, the 3 characters 'ABC' would be  packed  into  2

words as follows:

Word 1: 6301

Word 2: 1702

When packing characters on input, the character CTRL/Z (octal

232)  is inserted at the logical end-of-file (for example, at

the end of the  tape  in	the  paper  tape  reader  handler).

Following	 CTRL/Z,  the  remaining  words of the input buffer

should be zeroed.

5-3

RECONFIGURING THE OS/8 SYSTEM

12.  A close operation should be performed by non-file	 structured

handlers if bits 1-5 and 9-11 of the function word are 0.

The device handler, whether one or two pages long, must be	 completely

page  independent:	  it must be capable of executing in any page(s) in

field 0, except page 0 and 7600 to 7777.  Page  independent  code	can

have  no  address	constants.  Writing one page handlers is relatively

easy, since the addressing structure of the PDP-8 is essentially  page

independent.  Writing page relocatable code for two pages, however, is

considerably more difficult, as the two pages must	 communicate.	The

usual  technique  utilized	 in writing two page handlers is to include

some initialization code which includes a	JMS.   This  replaces  that

location  by  an address on the page the handler was loaded on.  Using

this, the handler can then determine where the relevant pieces of code

are in core.

As an example, the following is the initialization procedure performed

by	 the  TD8E DECtape routine.  This is by no means the only technique

that is possible, but it is a workable solution.

*200

/EXECUTED CODE

JINIT,	    JMP INIT	 /START INITIALIZATION

.

.

.

INIT,	    JMS.	 /FOUND OUT WHERE WE ARE.

BASE,	    TAD CRDQAD	 /INIT GETS ADDRESS OF BASE

SPA		 /NEGATIVE TERMINATES LIST

JMP NXINIT	 /INITIALIZE SECOND PAGE

TAD INIT	 /NOW UPDATE THE LIST OF

DCA CRDQAD	 /ADDRESS DEPENDENT LOCATIONS

ISZ .-1	 /POINT TO NEXT ELEMENT

ISZ BASE	 /NEXT INPUT VALUE

JMP BASE	 /LOOP OVER INPUT TABLE.

.

.

.

.

CRDQAD,	    R4LINE-BASE	 /THESE ARE ALL POSITIVE DIFFERENCES,

CINIT2,	    INIT2-BASE	 /SINCE THE ROUTINES INDICATED ARE

CSELCT,	    SELECT-BASE	 /IN THE SECOND PAGE. AFTER

CXUNIT,	    XUNIT-BASE	 /INITIALIZATION, CRDQAD POINTS TO

BUFF,	    4000	 /THE ACTUAL ADDRESS OF R4LINE, ETC.

.	 /THE 4000 IN BUFF TERMINATES

.	 /THE FIRST INITIALIZATION.

.	 /MORE PAGE INDEPENDENT CODE

NXINIT,	    JMS I CINIT2 /INITIALIZE SECOND PAGE

BASE2,	    DCA JINIT	 /CLEAR OUT JINIT. NO MORE

JMP JINIT	 /RELOCATING IS NEEDED UNTIL THE

/HANDLER IS LOADED INTO CORE AGAIN.

5-4

RECONFIGURING THE OS/8 SYSTEM

*400	 /SECOND PAGE OF HANDLER

INIT2,	    0		 /ADDRESS OF BASE2 GOES HERE

INIT3,	    TAD CTRY3

SNA		 /A 0 TERMINATES THIS LIST

JMP I INIT2

TAD INIT2	 /ADD VALUE OF BASE2 TO LIST

DCA CTRY3	 /PUT BACK INTO LIST

ISZ .-1	 /NEXT LOC. TABLE

ISZ INIT3

JMP INIT3

.

.

.

.

CTRY3,	    TRY3-BASE2	 /THIS LIST GETS VALUE OF BASE2

CRWCOM,	    TRWCOM-BSSE2 /ADDED IN TO POINT TO THE REAL

XBUFF,	    0		 /ROUTINE.

Writing 2 page independent code can be  expensive	in  terms  of  core

required.	 The routines should be set up in such a way as to minimize

communication between the two pages.  Some other  points  to  keep	 in

mind are:

1.  Relocation code is once-only code.  It is done once when	the

handler  is  loaded  and	need  never be done again until the

handler is  re-loaded  from  the	system	device.	  For  this

reason, the relocation code can be placed in a buffer area or

setup in temporary scratch locations which are later used	 as

temporary storage.

2.  A useful hint is that a JMP into the next page of code is not

required.	  The code can just as easily fall through 377 into

400. This may save a few locations of relocation code.

3.  Useful techniques for writing 2-page handlers can be found in

the source of the KL8E handler.

5.2  INSERTING DEVICE HANDLERS INTO OS/8

After the handler has  been  written  and	thoroughly  debugged  as  a

stand-alone routine, it can be integrated into the OS/8 Monitor, where

it	 will  become  a  resident  device  handler.   To  accomplish	the

integration, use OS/8 BUILD, described thoroughly in the BUILD section

in Chapter 2 of the OS/8 HANDBOOK.

Notes for writing system handlers system handlers	may  be	 integrated

into BUILD just like non-system handlers with the following additional

notes:

5-5

RECONFIGURING THE OS/8 SYSTEM

1.  Body of system handler should be origined	 to  200  but  must

start  with  a  2	 BLOCK	7.  Entry point must be at relative

location 7 (corresponds to location 7607).

2.  Name of system handler must be SYS.

3.  Each  handler  entry  point  has	an  8-word  handler   block

associated with it.  The following additions apply:

a.  word 5:  bits 9-11 should normally be 0.

If the device can have  multiple  platters  (like  RF08)

then	 this  field  specifies	 maximum number of platters

allowed.  Each platter above first  bumps  internal	DCB

code by 1.

word 6:  bit 0=1 means system device is two pages  long.

The	second	page  is  origined  into 400 but resides in

field 2 location 7600. Bit 1=1 if entry  point  is  SYS.

Bit 2=1 if entry point is coresident with SYS.

word 7:  must be 0

word	 10: number  of	 blocks	 in  device.	Immediately

following  the  header  records  is	the  code  for	the

device's bootstrap.	 This  is  preceded  by	 minus	the

number of words in the bootstrap.  No origins may appear

in this code.   It must be less than 47 locations long.

5-6

APPENDIX A

OS/8 FILE STRUCTURES

A.1  FILE DIRECTORIES

Blocks 1 through 6 on all file structured devices are reserved for the

file  directory  of  that	device.	  Six  blocks are always allocated,

though all are not necessarily active at any given time.  To  minimize

the  number  of  directory	 reads and writes necessary, OS/8 fills one

directory block completely before overflowing  onto  a  second  block.

Thus  the user with only a few files can perform directory LOOKUPs and

ENTERs faster than one with many files.

The directory blocks are each structured according	 to  the  following

format:

ENTRY

+--------------------------------+

0  | MINUS THE NUMBER OF ENTRIES	   |

| IN THIS SEGMENT		   |

+--------------------------------+

1  | THE STARTING BLOCK NUMBER	   |

| OF THE FIRST FILE IN THIS	   |

| SEGMENT			   |

+--------------------------------+

2  | LINK TO NEXT SEGMENT-ZERO	   |

| IF NO NEXT SEGMENT		   |

+--------------------------------+

3  | FLAG WORD-POINTS TO LAST WORD  |

| OF TENTATIVE FILE ENTRY IN	   |

| THIS SEGMENT		   |

| DIRECTORY SEGMENTS ARE ALWAYS  |

| LOADED INTO LOCATIONS 11400	   |

| TO 117777 BY THE USR; THIS	   |

| POINTER IS EITHER 0 OR BETWEEN |

| 1400 TO 1777.		   |

+--------------------------------+

4  | MINUS THE NUMBER OF		   |

| ADDITIONAL INFORMATION WORDS.  |

| THE NUMBER OF ADDITIONAL	   |

| INFORMATION WORDS SPECIFIED	   |

| MUST BE THE SAME IN ALL	   |

| DIRECTORY SEGMENTS		   |

+--------------------------------+

5  | BEGINNING OF FILE ENTRIES	   |

+--------------------------------+

.  |				   |

.  ~				   ~

.  |				   |

+--------------------------------+

377| END OF DIRECTORY BLOCK	   |

(8)+--------------------------------+

A-1

OS/8 FILE STRUCTURES

Locations 0 through 4 of each directory block are called  the  segment

header.

A.1.1  Directory Entries

There are three types of file directory entries.  They  are  PERMANENT

FILE  ENTRY,  EMPTY FILE ENTRY, and TENTATIVE FILE ENTRY.	A permanent

file entry appears as follows:

Location	  Contents		      Notes

+-------------------------------+

0 | FILE NAME	  FILE NAME	 |\

| CHARACTER 1	  CHARACTER 2	 | |

+-------------------------------+ |

1 | FILE NAME	  FILE NAME	 | |

| CHARACTER 3	  CHARACTER 4	 | |  THE FILE NAME AND EXTENSION

+-------------------------------+  > ARE PACKED IN SIXBIT ASCII

2 | FILE NAME	  FILE NAME	 | |  (i.e., "A" would be 01)

| CHARACTER 5	  CHARACTER 6	 | |

+-------------------------------+ |

3 | FILE EXTENSION FILE EXTENSION | |

| CHARACTER 1	  CHARACTER 2	 |/

+-------------------------------+

|				 |\

|				 | |  N, THE NUMBER OF ADDITIONAL

+-------------------------------+ |  INFORMATION WORDS, IS GIVEN

|	ADDITIONAL		 | |  BY WORD 4 OF THE DIRECTORY

~	INFORMATION		 ~  > HEADER.  WORD 4 OF THE ENTRY

|	WORDS			 | |  IS EITHER 0 OR THE CREATION

+-------------------------------+ |  DATE OF THE FILE.

|				 | |

N+3 |				 |/

+-------------------------------+

|				 |

N+4 | MINUS FILE LENGTH IN BLOCKS	 |

+-------------------------------+

NOTE

If word 3 is zero, the given file has  a

null extension.

An empty file entry appears as follows:

Location	   Contents

+-----------------------------+

0 | ENTRY IS ALWAYS 0000	 |

+-----------------------------+

1 | MINUS THE NUMBER OF BLOCKS	 |

| IN THIS EMPTY FILE		 |

+-----------------------------+

A-2

OS/8 FILE STRUCTURES

A tentative file entry appears as a permanent file entry with a length

of	 zero.	 It  is always immediately followed by an empty file entry.

When the tentative file is entered in a directory, location 3  in	the

segment  header  becomes  a pointer to this entry.	 The CLOSE function

inserts the length word of the  tentative	file  entry,  making  it  a

permanent	file,  and  adjusts  the length of the following empty file

entry (deleting that entry if the length becomes zero).

Whether or not there is  a	 tentative  file  open	on  any	 device	 is

determined by examination of bits 9 to 11 of the system Device Control

Word Table (see section B.3.5) not the contents of location 3  in	the

segment  header.	Zeroing these bits in the Device Control Word Table

makes the active tentative file on the device inactive.  The next time

that  the	system	has  to	 write	the directory segment, the inactive

tentative file entry is removed.  The distinction between	active	and

inactive  tentative  files is made so that OS/8 can avoid spending the

time required to perform  an  extra  read	and  write  of	the  device

directory.

A.1.2  Number and Size of OS/8 Files

All files on an OS/8 device must occupy a contiguous group	 of  blocks

on	 the  device.  The length of any file is indicated in its directory

entry, and the starting  block  of	 the  file  is	deduced	 by  adding

together  word  1	of  the segment header and the lengths of all files

whose entries precede it in the directory segment.

Each directory segment must have enough unused words  at  the  end	 to

accommodate  a  permanent file entry (N+5 words, where N is the number

of Additional  Information	 Words).  Thus,	 if  N	is  the	 number	 of

Additional	 Information  Words  the  maximum  number of permanent file

entries in any one segment is:

256-7 - (N+5)	  244-N

MIN = [-------------] =	 [-----]

N+7		   N+7

with N=1, MAX=40, and MIN=30.  Since there are  six  segments  in	the

directory,	 the  maximum  number of files possible (with N=1) would be

240.

Finally, OS/8 devices are limited to 4095 blocks, each block being 256

words long.  Thus, the maximum size of any single OS/8 file structured

device is 1,048,320 words.	 Blocks 0  through  6  of  the	device	are

unavailable for file storage;  therefore, the largest possible file is

4088 blocks long, or 1,046,528 words.

A.1.3  Sample Directory

The initial directory written when the OS/8 system is built  looks	 as

follows:

A-3

OS/8 FILE STRUCTURES

Location       Contents			   Notes

+----------+

/ 0  |   7776   |	TWO ENTRIES

| 1  |   0070   |	FILE STORAGE STARTS AT BLOCK 70(8)*

HEADER <  2  |   0000   |	NO ADDITIONAL DIRECTORY SEGMENTS

| 3  |   0000   |	NO TENTATIVE FILES.

\ 4  |   7777   |	ONE ADDITIONAL INFORMATION WORD

+----------+

/  5  |   0102   |  \

|  6  |   2314   |   >  FILE NAME IS "ABSLDR"

PERMANENT /  7  |   0422   |  /

FILE  \ 10  |   2326   |	FILE EXTENSION IS SV

ENTRY  | 11  |   5370   |	DATE IS 10/31/70

\ 12  |   7773   |	LENGTH IS FIVE BLOCKS

+----------+

EMPTY  / 13  |   0000   |	EMPTY FILE

FILE <      |		|

ENTRY  \ 14  |   6534   |	LENGTH IS 1244(8) (676(10))BLOCKS -

+----------+	THIS IS DEPENDENT ON THE SYSTEM

|		|	DEVICE USED.  676 IS THE VALUE

~		~	FOR A DECTAPE SYSTEM

|		|

377(8) +----------+

*This leaves room for the OS/8 System

Areas

A.2  FILE FORMATS

There are three different standard	 file  formats	used  by  OS/8	and

associated system programs:

1. ASCII and Binary files.

2,  Core Image files (.SV format).

3.  Relocatable  FORTRAN  library  files  (LIB8.RL  is  the  only

current example of this format).

NOTE

Binary files can contain either absolute

binary  data (i.e., output from PAL8) or

relocatable binary	data  (i.e.,  output

from SABR).

A.2.1  ASCII and Binary Files

ASCII and Binary files are packed three characters into two words,	 as

follows:

A-4

OS/8 FILE STRUCTURES

+---------------+------------------------+

WORD 1	|  CHARACTER 3	|	 CHARACTER 1	 |

|   BITS 0-3	|			 |

+---------------+------------------------+

WORD 2	|  CHARACTER 3	|	 CHARACTER 2	 |

|   BITS 4-7	|			 |

+---------------+------------------------+

0	       3 4		       11

The following conventions are used by OS/8 system programs:

1.  In ASCII files the  character  NULL  (ASCII  000)	 is  always

ignored.	Most programs only examine the low-order 7 bits, in

ASCII files.  The parity bit  is	usually	 ignored;   do	not

assume  that  this  bit  is  set	or that data transfers will

preserve it (image mode transfers, always preserve it).

2.  In Binary files the binary data must be preceded	by  one	 or

more  frames  of	leader/trailer	code  (ASCII 200 code). The

first character of binary data must  be  either  100  to	177

(octal) (an origin setting for absolute binary files), 240 to

257 (octal)  (a  COMMON  declaration  frame  for	relocatable

binary  files),  or  300 (octal), which is an origin setting.

The end of binary data is indicated by one or more frames	 of

leader/trailer code.

3.  ASCII and Binary files are terminated by a CTRL/Z code (ASCII

232).  In	 binary	 files,	 a  CTRL/Z  code  data	rather than

end-of-file.

A.2.2  Core Image (.SV Format) Files

A core image file consists of a header by the actual core image.	The

header block is called the Core Control Block.  The Core Control Block

consists of the first 128 words of the 256	 word  block  reserved	for

that  purpose.   The  second  128	words are unused.  The Core Control

Block is formatted as follows:

A-5

OS/8 FILE STRUCTURES

Location	       Contents			   Notes

CORE CONTROL BLOCK

+------------------------------------+

0 |	MINUS THE NUMBER OF CORE SEGMENTS |

+------------------------------------+   62N3 WHERE N IS THE

1 |	CDF CIF (STARTING FIELD)	  |<--STARTING FIELD

+------------------------------------+

2 |	STARTING ADDRESS		  |

+------------------------------------+

3 |	JOB STATUS WORD			  |

+------------------------------------+

4 |					  |\  CORE SEGMENT

+------------------------------------+ | CONTROL DOUBLE WORDS

~					  ~  >

+------------------------------------+ | (K IS THE NUMBER OF

2K +3 |					  |/  CORE SEGMENTS)

+------------------------------------+

377(8) ~					  ~   REMAINDER OF BLOCK

+------------------------------------+   IS UNUSED

The format of the Job Status Word is as follows:

Bit Condition			Meaning     _____________			_______

Bit 0 = 1	      File does not load into locations 0  to  1777

in field 0.

Bit 1 = 1	      File does not load into locations 0  to  1777

in field 1.

Bit 2 = 1	      Program must be reloaded	before	it  can	 be

restarted.

Bit 3 = 1	      Program never uses above	8K.  This  is  used

when Batch processing is active.

Bit 10 = 1	      Locations 0 to 1777 in field 0  need  not	 be

preserved when the Command Decoder is called.

Bit 11 = 1	      Locations 0 to 1777 in field 1  need  not	 be

preserved when the USR is called.

A-6

OS/8 FILE STRUCTURES

The Core Segment Doublewords control the reading and  writing  of	the

associated areas of core.	The format of each entry is as follows:

Location		      Contents		     Notes

+---------------------------------------------+

1 |		      CORE ORIGIN		| MULTIPLE OF 400(8)

+---+------------------+-----------+----------+

2 |   |NUMBER OF PAGES	 |   FIELD   |		| BITS 0 AND 9-11

|   |	  TO LOAD	 |  TO LOAD  |		| ARE ZERO

+---+------------------+-----------+----------+

0   1		5 6	    8 9	      11

The core origin must be a multiple of 400 (octal).	 The  Core  Segment

Control  Doublewords  are	sorted	within the header block in order of

decreasing field and increasing origin within the same  field.   There

can  be  no more than 32 (decimal) Core Segment Control Doublewords in

any Core Control Block.

The Core Control Block for the program at the time it is  loaded  into

core is always saved in words 200 (octal) through 377 (octal) of block

37 (octal)	 (one of the system scratch blocks) on	the  system  device.

It	 is  placed there by the GET and RUN operations or by the ABSLDR or

LOADER programs.  This Core Control Block is used	when  performing  a

SAVE without arguments.

NOTE

The	 R  command  differs  from  the	 RUN

command   in  that	the  program's	Core

Control Block is not  written  onto	 the

scratch  area  when using the R command.

In order to SAVE a program that has been

loaded  by	the  R	command	 all  of the

arguments of the SAVE  command  must  be

explicitly stated.

A-7

OS/8 FILE STRUCTURES

A.2.3  Relocatable FORTRAN Library File

A relocatable FORTRAN library consists of a  library  directory  block

followed  by relocatable binary segments.	The directory block has the

following format:

Location	    Contents			      Notes	  ________	    ________			      _____

+--------------+---------------+

0	  |   CH1	 |    CH2	 |\

+--------------+---------------+ |  NAME OF ENTRY IN

1	  |   CH3	 |    CH4	 |  > SIXBIT ASCII PADDED

+--------------+---------------+ |  WITH TRAILING BLANKS

2	  |   CH5	 |    CH6	 |/

+--------------+---------------+

3 +-| LOAD POINTER |		 |

| +--------------+---------------+

4 | | ADDITIONAL ENTRIES		 |

| ~		  .		 ~

5 | |		  .		 |

| +------------------------------+

| |		  0		 |\

| +------------------------------+ |

| |		  0		 | |

| +------------------------------+  > DENOTES END OF

| |		  0		 | |  NAME ENTRIES

| +------------------------------+ |

| |		  0		 |/

| +------------------------------+

/  ~				 ~

|	  +------------------------------+

\->|				 |-> LOADER CONTROL WORD(S)

+------------------------------+

|		  0		 |-> END OF LOADER CONTROL

+------------------------------+   WORDS FOR THIS ENTRY

~				 ~

+------------------------------+

377(8)	  |				 |

+------------------------------+

The Load Pointer is a number between 0 and 377  (octal)  which  points

(relative to the beginning of the block) to an array of Loader Control

Words.  The Loader Control Words have the following information:

+--------------+--------------------+

|		 |		      |

+--------------+--------------------+

0		4 5		    11

NUMBER OF PAGES OCCUPIED	      (STARTING BLOCK OF RELO-

BY THIS SEGMENT AFTER LOADING       CATABLE BINARY DATA)

(DIRECTORY BLOCK #)-1

A-8

OS/8 FILE STRUCTURES

There can be one or more Loader Control Words  for	 each  entry.	The

Loader  Control  Words  for an entry are terminated by a word of zero.

The following is a simple directory block.

Location	       Contents

+---------------+

0   |		 1117 |	  NAME OF ENTRY IS "IOH___"

+---------------+

1   |		 1040 |

+---------------+

2   |		 4040 |

+---------------+

3   |		 0376 |	  LOAD POINTER FOR "IOH"

+---------------+

4   |		 0530 |

+---------------+

5   |		 1124 |	  NAME OF ENTRY IS "EXIT__"

+---------------+

6   |		 4040 |

+---------------+

7   |		 0373 |	  LOAD POINTER FOR "EXIT"

+---------------+

10   |		 0000 |

+---------------+

11   |		 0000 |	  MARKS END OF ENTRIES

+---------------+

12   |		 0000 |

+---------------+

13   |		 0000 |

+---------------+

~		      ~

+---------------+

LOADER	373   |		 0207 |	  (RELATIVE BLOCK 10(8))(ONE

CONTROL	      +---------------+	  PAGE LONG)

WORDS FOR 374   |		 0411 |	  (RELATIVE BLOCK 12(8))(TWO

"EXIT"	      +---------------+	  PAGES LONG)

375   |		 0000 |

+---------------+

LOADER	376   |		 2400 |	  (RELATIVE BLOCK 1)(12(8) PAGES

CONTROL	      +---------------+	  LONG)

WORDS FOR 377   |		 0000 |

"IOH"	      +---------------+

A-9

APPENDIX B

DETAILED LAYOUT OF THE SYSTEM

This appendix covers three topics:	 the reserved areas on	the  system

device, the resident portion of OS/8, and the various system tables.

B.1  LAYOUT OF THE SYSTEM DEVICE

The first 70 octal	 blocks	 (14K  words)  on  the	system	device	are

reserved by the OS/8 system.  These blocks are used as follows:

Block(s) in Octal	      Contents     _________________	      ________

0		      System Bootstrap Routine

1-6		      Device Directory

7-12		      Keyboard Monitor

13-15		      User Service Routine

16-25		      Device Handlers

26		      ENTER Processor for USR

27-50		      System Scratch Blocks

51-53		      Command Decoder

54-55		      SAVE and DATE Overlays

56		      Monitor Error Routine

57		      CHAIN Processor for USR

60-63		      SYSTEM ODT

64		      Reserved for System Expansion

65		      CCL Reminiscences

66		      12K TD8E Resident code

67		      CCL Overlay

File storage begins with block 70 (octal).

The system scratch blocks are used for preserving the contents of core

when  the	Keyboard  Monitor, USR, Command Decoder, or ODT are loaded.

In addition, various system  programs  use	 the  scratch  area.   Most

importantly,  the	SAVE  command  expects the Core Control Block to be

loaded in words 200 (octal) to 377 (octal) of block  37  (octal).	The

Core  Control  Block  is  stored  at those locations by the GET or RUN

command or by the ABSLDR or LOADER program.

B-1

DETAILED LAYOUT OF THE SYSTEM

A detailed breakdown of system scratch block usage follows:

Block(s) in Octal		 Contents       _________________		 ________

27-32		      The contents of locations 10000 to 11777	are

saved in this area when the USR is loaded.

33-36		      The contents of locations 0 to 1777 are saved

in   this	 area  when  the  Command  Decoder,

Keyboard Monitor, or ODT is loaded.

37		      Words 200 (octal)	 to  377  (octal)  of  this

block  contain the Core Control Block for the

last  program  loaded  by	 the  GET  or	RUN

command, or the ABSLDR or LOADER program.

40-47		      Used as scratch storage  by  the	ABSLDR	and

LOADER programs.

50		      Reserved for future expansion.

B-2

DETAILED LAYOUT OF THE SYSTEM

B.2 LAYOUT OF THE OS/8 RESIDENT PROGRAM

The top core pages in fields 0, 1, and 2  are  used  by  the  resident

portion  of  OS/8	and  are  not accessible by the user.  As a general

rule, system and user programs should never destroy  the  contents	 of

locations 7600 to 7777 of any field.

The resident portion of OS/8 is structured as follows:

Location	 Contents		     Notes	  ________	 ________		     _____

+-------------------------+

7600  | WRITE OPERATION	     |<- NON-DESTRUCTIVE

+-------------------------+	 ENTRY TO OS/8

7605  | JMP TO FIELD 1 FOR READ |<- DESTRUCTIVE

+-------------------------+	 ENTRY TO OS/8

7607  |			     |<- ENTRY TO SYSTEM

~ SYSTEM DEVICE HANDLER   ~	 DEVICE HANDLER

7743  |			     |

+-------------------------+

7744  |			     |

7745  | CURRENT STARTING ADDRESS|

+-------------------------+

7746  | JOB STATUS WORD	     |

+-------------------------+

7747  |		0	     |<- MUST ALWAYS BE ZERO

+-------------------------+

7750  |			     |

| RESERVED FOR DATA BREAK |

| LOCATIONS		     |

7755  |			     |

+-------------------------+

7756  |			     |<- THE KEYBOARD MONITOR

~ PROGRAM SETUP AREA	     ~	 AND ODT MODIFY THIS

7777  |			     |	 AREA

+-------------------------+

B-3

DETAILED LAYOUT OF THE SYSTEM

TOP PAGE OF FIELD 1

Location		 Contents		     Notes

+--------------------------------+		     \

7600    ~    OUTPUT FILE LIST (3 ENTRIES)~		      |

7616    +--------------------------------+		      |

7617    ~    INPUT FILE LIST	      ~   0 MARKS END |

|    (MAXIMUM 9 ENTRIES)	    <---  OF LIST     | COMMAND

7641    +--------------------------------+		       >DECODER

7642    |    *  HIGH 11 BITS OF =N	      |<- * BIT 0=1   | AREA

+--------------------------------+   IF COMMAND  |

7643    |    SPECIFIED OPTIONS	      |   LINE TERMI- |

7645    +--------------------------------+   NATED BY    |

7646    |    LOW 12 BITS OF =N	      |   ALTMODE     |

+--------------------------------+		      |

7647    |    DEVICE HANDLER	      |		     /

7665    ~    RESIDENCY TABLE	      ~

+--------------------------------+

7666    |    SYSTEM DATE WORD	      |

+--------------------------------+

7667    ~    READ OPERATION	      ~

7677    |    (LOAD KEYBOARD MONITOR)     |

+--------------------------------+

7700    ~    USR CALL AND RETURN AREA    ~<- ENTRY TO USR

7740    +--------------------------------+	      NOTE						      ____

7741    ~    USER DEVICE		      ~   SYSTEM ODT DESTROYS

7757    |    NAME TABLE		      |   CONTENTS OF THIS TABLE

+--------------------------------+   WHEN SETTING BREAKPOINTS

7760    ~    DEVICE CONTROL WORD TABLE   ~

7776    +--------------------------------+

7777    |    UNUSED		      |   RESERVED FOR FUTURE USE

+--------------------------------+

Systems built around TD8E DECtape without the Read-Only-Memory  option

use 7600 in field 2 as an extension of the system device handler.

TOP PAGE OF FIELD 2.

7600   +------------------+

|			 |

|			 |   Used   only   for	 12K   TD8E

|			 |   systems.	 Part	of   system

|			 |   handler resides here  in  that

|			 |   case.

7773   |__________________|

7774   |			 |

|			 |   Four words reserved for  BATCH

|			 |   use  if  machine  has  exactly

|			 |   12K.  Contains  pointers  into

|			 |   input file.

|			 |

7777   +------------------+

B-4

DETAILED LAYOUT OF THE SYSTEM

If the machine has more than 12K, the top 4 locations  (7774-7777)	 of

the last field are reserved for use by BATCH.

If a ROM (Read-Only-Memory) is being used	with  an  8K  TD8E  system,

locations	7400-7777 of  field  7 are inaccessible to the user.  That

core is used for system handler functions.

B.3 SYSTEM DEVICE TABLES

Each device is described to the  system  by  entries  in  five  system

tables.   Each of these tables is fifteen words long, where the device

number is the index into the table.  The  five  tables  are  described

below.

B.3.1 Permanent Device Name Table

Entries in this table specify the permanent name of each device.	The

entries are computed by encoding the actual four-character device name

in a single word as follows:

1.  The device name is expressed as two  words  in  the  standard

DEVICE  format.	For  example, if the device name were "PTR"

the two words would be:

WORD 1: 2024

WORD 2: 2200

Note that when the device name is left  justified;   0's	are

inserted to fill four characters.

2.  A single word is created by adding together these two words.

3.  If word 2 is non-zero, bit 0 of the resulting word is  forced

to be a one.  For example, the table entry for "PTR" would be

4224.

An entry of zero means that there is no device for	 the  corresponding

device number.

NOTE

Conventionally,  device  names   consist

only  of  the characters A to Z and 0 to

9.	The first character  of	 the  device

name  should  be alphabetic.  The coding

used makes all  one	 and  two  character

device  names unique;  however, names of

more than two characters are not unique.

For	 example,  "PTR"  and "RTP" have the

same encoding.

B-5

DETAILED LAYOUT OF THE SYSTEM

The Permanent Device Name table is fifteen locations long;	 it resides

in	 the USR.  When the USR is in core the beginning of the table is in

field 1 at a location the address of which is  contained  in  location

10036.

B.3.2 User Device Name Table

Entries are made in this table whenever the user  performs	 an  ASSIGN

and  are  restored to zero by a DEASSIGN.	These entries have the same

format as those in the Permanent Device Name Table.

The User Device Name Table resides in locations 17741 through 17757.

B.3.3  Device Handler Residency Table

When a device handler is loaded by the USR, the entry  in	this  table

for  the device loaded (and entries for all devices whose handlers are

co-resident, if any) is set to contain the entry point for the  device

handler.	Entries other than those that contain an address above 7600

(thus referring to the system handler) are restored to 0 when a RESET,

DECODE  or	 CHAIN	function  is executed.	When a program exits to the

Keyboard Monitor this table is  not  cleared.   The  Keyboard  Monitor

Commands GET, RUN, R, SAVE, and START (with no explicit address) clear

this table.

NOTE

Since  the	system	device	handler	  is

always  resident the first entry (SYS is

always device number 1)  in	 the  Device

Handler  Residency	Table is always 7607

(the entry point of	 the  system  device

handler).

The Device Handler Residency Table resides in locations 17647  through

17665.

B.3.4  Device Handler Information Table

Each entry in this table contains all the information  needed  by	the

USR to load the corresponding handler.  The format of these entries is

as follows:

B-6

DETAILED LAYOUT OF THE SYSTEM

Bit Condition				  Meaning	  _____________				  _______

Bits 0 = 1		   If this is a two page device handler.

Bits 1 to 4		   Contain the relative block  location	 of

the	device handler record on the system

device.  This is computed by subtracting

15  (octal)	(one  less  than  the first

device handler block)  from	the  actual

block number.

Bits 5 to 11		   Contain the offset of the handler  entry

point  from	the  beginning of the page.

Note	 that  the  entry  points  to	all

handlers must be in the first page.

If an entry is 0 the corresponding device handler is not saved in	any

of	 the  device handler storage blocks.  This is always true of device

number 1 (the system device) and for all device numbers that  are	not

used  in  a given configuration.  The Device Handler Information Table

is 15 locations long and resides in the USR.  When the USR is in  core

the  beginning of the table is in field 1 at a location the address of

which is contained in location 10037.

B.3.5 Device Control Word Table

Entries  in  this	table  specify	special	  device   characteristics,

including the physical device type.  The entry format is as follows:

Bit Condition				  Meaning	  _____________				  _______

Bit 0 = 1		   If the device is file-structured.

Bit 1 = 1		   If the device is read-only.

Bit 2 = 1		   If the device is write-only.

Bits 3 to 8		   Contain the physical	 device	 type  code

(described below),

Bits 9 to 11		   For file structured devices, these  bits

contain  the	 directory  block number of

the currently active tentative file.	 If

bits	 9  to	11  are	 zero,	there is no

active tentative  file  on  the  device.

For	non-file structured devices, bits 9

to 11 are always zero.  Bits 9 to 11 are

reset  to zero by the commands GET, RUN,

R,  SAVE,  START   (with   no   explicit

address)  and  optionally   by  the	USR

functions RESET and DECODE.

B-7

DETAILED LAYOUT OF THE SYSTEM

The device type is a number between 0  and	 77  (octal),  of  which  0

through  20  (octal)  are	currently  assigned to existing devices, as

follows:

Device Code		Device	       ___________		______

0	      Teletype

1	      High-speed paper tape reader

2	      High-speed paper tape punch

3	      Card Reader

4	      Line Printer

5	      RK8 Disk

6	      256K Disk (RF08)

7	      512K Disk (RF08 + RS08)

10	      768K Disk (RF08 + 2 RS08's)

11	      1024K disk (RF08 + 3 RS08's)

12	      32K Disk (DF32)

13	      64K Disk (DF32 + DS32)

14	      96K Disk (DF32 + 2 DS32's)

15	      128K Disk (DF32 + 3 DS32's)

16	      DECtape

17	      LINCtape (PDP-12 only)

20	      TM8E Magnetic Tape

21	      TD8E DECtape

22	      BATCH handler

23	      RK8E Disk

24	      NULL

25-26     reserved for future disks

27	      TA8E Cassette

30	      VR12 Scope

31-37     reserved for future use by DEC

40-57     reserved for use by users

The Device Control Word  Table  resides  in  locations  17760  through

17776.

B.3.6  Device Length Table

There is a sixth table that is not normally  considered  part  of	the

system  tables.   This  is the Device Length Table and is used only by

PIP to perform the	 /Z  (zero  directory)	and  /S	 (compress  device)

options.  This table is 64 locations long, one entry for each possible

device type.  In this table an entry of 0 means that the corresponding

device  is	 non-file  structured;	 otherwise  the	 entry contains the

negative of the number of available 256-word blocks on the device.

For example, the entry for a 256K disk would be  6000  (octal)  (minus

2000 (octal), or 1024 (decimal), 256-word blocks).

B-8

DETAILED LAYOUT OF THE SYSTEM

The Device Length Table resides in PIP.  When PIP is brought into core

the  Device  Length  Table	 is  in	 locations 13600 to 13677. When new

device types are added to the system this table should be patched with

ODT to reflect the device length of the new device.

A similar table occurs in RESORC which the user may wish to patch.	 It

is	 located  in  field 0 locations 2000-2377 and contains 64 four-word

entries;  one entry for each device type.	Words 1 and 2 of  an  entry

are  the names of the device (in sixbit) and word 3 is the negative of

the number.  Word 4 of the entry should be 0 for non-standard devices.

B-9

APPENDIX C

SYSTEM ERROR CONDITIONS AND MESSAGES

This is a summary of all error messages that are a	 result	 of  system

errors.   These  errors are also described in the relevant sections of

this manual and in the OS/8 HANDBOOK.

C.1  SYSTEM HALTS

Errors that occur as a result of a major I/O  failure  on	the  system

device can cause a system halt to occur.  These are as follows:

Value of PC			     Meaning	___________			     _______

00601		      A read error  occurred  while  attempting	 to

load  ODT.  Return to the Keyboard Monitor by

restarting at 07605.

07461		      An error occurred	 while	reading	 a  program

into  core  during  a  CHAIN.   Return to the

Keyboard Monitor by restarting at 07605.

07605		      An error occurred while attempting  to  write

the  Keyboard  Monitor  area  onto the system

scratch  blocks.	 Verify	 that  the   system

device  is  not  WRITE  LOCKed and restart at

location 07600 to try again.

07702		      A user program has performed a JMS to 7700 in

field  0.	 This is a result of trying to call

the USR without first performing a CIF 10. As

location	07700  has been destroyed, the user

must re-bootstrap the system.

07764		      A	 read  error  occurred	while	loading	  a

program.	 Return	 to the Keyboard Monitor by

restarting at 07605.

07772		      A read error occurred on the  system  scratch

area  while loading a program.  Return to the

Keyboard Monitor by restarting at 07605.

10066		      An input error occurred while  attempting	 to

restore  the  USR.   Return  to  the Keyboard

Monitor by restarting at 07605.

10256		      A read error  occurred  while  attempting	 to

load the Monitor by restarting at 07605.

C-1

SYSTEM ERROR CONDITIONS AND MESSAGES

17676		      An error occurred while  attempting  to  read

the  Keyboard Monitor from the system device.

Try again by restarting at location 07605. DO

NOT PRESS CONTINUE.

17721		      An error occurred while saving the USR  area.

Verify  that  the	 system device is not WRITE

LOCKed, and press CONTINUE to try again.

17727		      An error occurred while  attempting  to  read

the  USR	from  the system device.  Return to

the Keyboard Monitor by restarting at 07605.

17736		      An error occurred while reading  the  scratch

blocks  to  restore  the USR area.  Return to

the Keyboard Monitor by restarting at 07605.

Also, there is one halt in the LOADER program:

00005		      A parity error occurred  when  attempting	 to

overlay  the  LOADER  from the system scratch

blocks.  Return to the  Keyboard	Monitor	 by

restarting at 07605, and try again.

After retrying the operation which caused the failure,  if	 the  error

persists, it is the result of a hardware malfunction or a parity error

in the system area.  Run the appropriate diagnostic program  to  check

the device and rebuild the system.

C.2  USR ERRORS

Fatal errors that occur during operation of the USR cause the message:

MONITOR ERROR n AT xxxxx

to be printed.  In these cases, the value "n" describes the error	and

"xxxxx"  is  the address of the call to the USR that caused the error.

The six Monitor errors are:

Message				  Meaning	       _______				  _______

MONITOR ERROR 1 AT xxxxx	File length in	CLOSE  function	 is

[CLOSE ERROR]			too large.

MONITOR ERROR 2 AT xxxxx	An I/O	error  occurred	 while	at-

[DIRECTORY I/O ERROR]		tempting  to  read   or	  write	  a

directory block.  This is generally

caused by the  device  being  WRITE

LOCKed.

MONITOR ERROR 3 AT xxxxx	The  device  handler required for a

[DEVICE HANDLER NOT IN CORE]	file  operation	  (LOOKUP,   ENTER,

CLOSE) is not in core.

C-2

SYSTEM ERROR CONDITIONS AND MESSAGES

MONITOR ERROR 4 AT xxxxx	Illegal call to the USR;  either an

[ILLEGAL USR CALL]		attempt	 has  been made to call the

USR from locations 10000  to  11777

or  a  device  number  of  zero was

specified.

MONITOR ERROR 5 AT xxxxx	I/O error occurred while reading or

[I/O ERROR ON SYS]		writing	 on  the   system   device.

Verify	that  the  system device is

not WRITE LOCKed.

MONITOR ERROR 6 AT xxxxx	Directory  overflow  occurred  (see

[DIRECTORY OVERFLOW]		section A.1.2  for  limitations	 on

number of directory entries).

In addition to the MONITOR ERROR messages, system	and  user  programs

can use the USR to print:

USER ERROR n AT xxxxx

by using the ERROR function.   In	this  case  the	 value	of  "n"	 is

user-defined and "xxxxx" is the address of the call to the USR.

Currently, two USER ERROR numbers have been assigned:

Message				  Meaning	       _______				  _______

USER ERROR 0 AT xxxxx		An   I/O   error   occurred   while

attempting  to	load a program with

the GET, RUN, or R command.

USER ERROR 1 AT xxxxx		While running  a  FORTRAN  or  SABR

program,  an  attempt  was  made to

call a subroutine that had not been

loaded.

If an I/O error is made during the monitor CHAIN function the message

CHAIN ERR

is generated, and control returns to the keyboard.

Following either a MONITOR ERROR message or a USER ERROR  message	the

USR  exits	 to the keyboard Monitor;  the current contents of core are

preserved and bit 2 of the Job Status Word is set to a  1	to  prevent

continuing from the error.

C.3  KEYBOARD MONITOR ERRORS

In addition to the USR  errors  described	previously,  the  following

errors can occur after a command is given to the Keyboard Monitor:

C-3

SYSTEM ERROR CONDITIONS AND MESSAGES

Message				  Meaning	  _______				  _______

aaaa?			   The Keyboard	 Monitor  cannot  interpret

the	command "aaaa".	 For example if the

user types HELLO the system will respond

HELLO?

BAD ARGS		   Arguments  to   a   SAVE   command	are

inconsistent, or illegal.

BAD CORE IMAGE	   The file requested with an  R,  RUN,	 or

GET command is not a core image file.

BAD DATE		   Improper syntax in a DATE command.

device NOT AVAILABLE	   The permanent device name  specified	 in

an  ASSIGN,	SAVE,  RUN,  or GET command

does not exist.

ILLEGAL ARG		   The	SAVE  command  was  not	  expressed

correctly.

name NOT FOUND	   The file name specified was not  located

on the device indicated.  This error can

also be caused by trying to RUN  or	GET

from an output only device.

NO!!			   A  START  command   (with   no   address

specified)  is  prohibited when bit 2 of

the Job Status Word (location 07746)	 is

a 1.

NO CCL!		   Command was	a  valid  CCL  command	but

CCL.SV is not on the system.

SAVE ERROR		   An I/O error occurred while	saving	the

program.   The  contents  of core remain

intact.

SYSTEM ERR		   An error occurred while doing I/O to the

system device.

TOO FEW ARGS		   An argument	has  been  omitted  from  a

command.

C.4  CCL ERROR MESSAGES

Message				  Meaning	  _______				  _______

BAD DEVICE		   The device specified in a CCL command is

not	 of   the   correct   form,  (e.g.,

DTA0.PA:).

C-4

SYSTEM ERROR CONDITIONS AND MESSAGES

BAD EXTENSION		   Either  an	extension   was	  specified

without  a file name (e.g., DTA1:.PA) or

two	extensions  were  specified  (e.g.,

DTA1:FILE.PA.BN).

BAD MONITOR		   The	version	 of  the  Keyboard  Monitor

being  used	is not compatible with CCL.

A new version of  the  monitor  must	 be

obtained  from Digital before CCL can be

used.

BAD NUMBER		   A  CCL  command   which   uses   the	  #

construction	 does  not  have  the  full

16-digit specification that is required.

BAD RECOLLECTION	   An attempt was made to use a	 previously

remembered argument when no argument was

saved.   This  error	 occurs	  when	 no

argument  was  previously  saved or when

the DATE command has been used since the

argument was saved.

BAD SWITCH OPTION	   The character used with a slash  (/)	 to

indicate   an  option  is  not  a  legal

option.

CANNOT CHANGE CORE	   A CORE  command  was	 issued	 while	the

CAPACITY WHILE RUNNING   BATCH program was running.

BATCH

%CANT REMEMBER	   The argument specified in a CCL  command

line	 is too long to be remembered or an

I/O error occurred.

CCL 3X OVERLAY &	   The	version	 of  CCL  being used is not

MONITOR INCOMPATIBLE	   compatible  with  the  Keyboard  Monitor

present  on	the  system.  Type R CCL to

retry.

COMMAND LINE OVERFLOW	   The command line specified  with  the  @

construction is more than 512 characters

in length.

COMMAND TOO LONG	   The length of a text argument in a  MUNG

command is too long.

CONTRADICTORY SWITCHES   Either two CCL processor  switches  were

specified   in  the	same  command  line

(e.g., FILE-PA-FT) or the file extension

and	the  processor	switch do not agree

(e.g., FILE.FT-BA).

name DOES NOT EXIST	   The device with the name  given  is	not

present on the OS/8 system.

C-5

SYSTEM ERROR CONDITIONS AND MESSAGES

ERROR IN COMMAND	   A command not entered directly from	the

console  terminal  is  not  a  legal CCL

command.  This  error  occurs  when	the

argument  of a UA, UB, or UC command was

not a legal command.

ILLEGAL * OR ?	   An * or ? was used in a CCL command that

does	   not	 accept	  the	wild   card

construction.  Only	CCL  commands  that

run	FOTP  or DIRECT allow the wild card

construction.

ILLEGAL SYNTAX	   The	CCL  command  line  was	  formatted

incorrectly.

INPUT ERROR READING	   CCL cannot read the file specified  with

INDIRECT FILE		   the @ construction.

I/O ERROR ON SYS:	   An error occurred while doing I/O to the

system  device.   The  system  must	 be

restarted   at   7600   or	7605.	 Do

not press  CONT.  as	 that  will  surely

cause further errors.

I/O ERROR TRYING TO	   An I/O  error  occurred  while  CCL	was

RECALL		   trying to remember an argument.

NO CCL!		   CCL.SV is  not  present  on	the  system

device.

NOT ENOUGH CORE	   The number specified in a  CORE  command

is  larger  than  the  number of 4K core

banks on the system.

name NOT FOUND	   The file with  the  name  given  is	not

present  on the specified device, or the

user tried to input from an	output-only

device.

%SUPERCEDED		   The file specified  in  a  MAKE  command

already   exists.   This  is	 a  warning

message  indicating	that  the  file	 is

being replaced.

SWITCH NOT ALLOWED HERE  Either a CCL option was specified on the

left	 side of the < or was used when not

allowed.  For example:  COMPARE FILE-NB.

TOO MANY FILES	   Too many files were included	 in  a	CCL

command.

C-6

SYSTEM ERROR CONDITIONS AND MESSAGES

C.5  COMMAND DECODER ERRORS

The following errors are printed by the Command  Decoder.	 After	the

error  message,  the Command Decoder starts a new line, prints a * and

waits for another command line.  The erroneous command is ignored.

Message				  Meaning

ILLEGAL SYNTAX	   The	 command    line    is	  formatted

incorrectly.

TOO MANY FILES	   More than three  output  files  or  nine

input   files  were	specified  (regular

mode)  or  >	 1  output  or	>  5  input

(special mode).

name NOT FOUND	   The specified input file  name  was	not

found on the device indicated.

C-7

APPENDIX D

PROGRAMMING NOTES

This appendix is a potpourri of ideas and techniques that have  proven

useful in programming the PDP-8. OS/8 users may find some use in their

own programs for the techniques mentioned here.

D.1  The Default File Storage Device, DSK

D.2  Modification to Card Reader Handler

D.3  Suppression of Carriage Return/Line Feed in FORTRAN I/O

D.4  Accessing the System Date in a FORTRAN Program

D.5  Determining Core Size on PDP-8 Family Computers

D.6  Using PRTC12-F to Convert OS/8 DECtapes to OS/12 LINCtapes

D.7  Notes on Loading Device Handlers

D.8  Available Locations in the USR Area

D.9  Accessing Additional Information Words in OS/8

D.10 SABR Programming Notes

D.1  THE DEFAULT FILE STORAGE DEVICE, DSK

The Command Decoder, as noted earlier, makes certain assumptions about

the I/O device where none is explicitly stated.  Namely, on all output

files where no device name is given, the device DSK  is  assumed.	 On

the  first	 input	file where no device name is given, DSK is assumed.

Subsequent input files assume the same device as  the  previous  input

file.  This convention was adopted to simplify typing command lines.

The permanent device name DSK is assigned when the	 system	 is  built.

On all standard systems, DSK is equivalent to SYS.	 A useful technique

is	 to  use  the  ASSIGN  command	to  redefine  the  meaning  of	DSK

temporarily.   For example, where device DTA0 is equivalent to DSK and

it becomes desirable to change DSK to DTA1, the following command can

be given:

.ASSIGN DTA1 DSK

D-1

PROGRAMMING NOTES

DTA1 remains the default file storage device until it  is	assigned  a

new  name	or  a  DEASSIGN	 command  is  executed.	  This technique is

considerably easier to use than rebuilding the entire system.

If 'DSK' has not been assigned via	 the  ASSIGN  command,	then  'DSK'

always  exists and has internal device number 2. User programs wishing

to use DSK should do an  INQUIRE  to  find	 its  number  in  case	the

operator has re-assigned it.

D.2  MODIFICATION TO CARD READER HANDLER

The standard card reader handler for OS/8	uses  the  DEC029  standard

card  codes.   Some  installations	 may prefer to use the DEC026 codes

instead.  This can be done by changing the card conversion codes  with

the BUILD command ALTER.

1.  Call OS/8 BUILD by typing:

RUN SYS:BUILD

in response to the dot printed by the Keyboard Monitor.

2.  Load the card reader handler as described on page 2-42 of the

OS/8 HANDBOOK.

3.  Use the ALTER command (see page  2-49 of the  OS/8  HANDBOOK)

to make the following changes:

CHANGE RELATIVE LOCATION	FROM	  TO	      ________________________	____	  __

104		3203	  7735

105		4007	  4076

106		3502	  0774

114		7514	  3314

115		0577	  1002

116		3637	  0305

124		0104	  3204

125		1211	  1273

126		3374	  3606

127		0641	  1341

134		7316	  3716

135		3410	  1175

136		1376	  3401

The new system will have modified card codes.

Note that this procedure does not affect FORTRAN run time	card  input

with  READ	 (3,n).	 The  conversion  table for FORTRAN is UTILTY.SB on

source DECtape #2.	 (DEC-S8-OSYSB-A-UA2)

D-2

PROGRAMMING NOTES

026 PUNCH CARD CODES

Octal 8-bit   DEC026		   Octal 8-bit	  DEC026

CODE	    CODE       CHARACTER     CODE	   CODE	  CHARACTER

240	   BLANK	  SPACE	       300	   8-4	      @

241	   12-8-7	  !	       301	   12-1	      A

242	   0-8-5	  "	       302	   12-2	      B

243	   0-8-6	  #	       303	   12-3	      C

244	   11-8-3	  $	       304	   12-4	      D

245	   0-8-7	  %	       305	   12-5	      E

246	   11-8-7	  &	       306	   12-6	      F

247	   8-6		  '	       307	   12-7	      G

250	   0-8-4	  (	       310	   12-8	      H

251	   12-8-4	  )	       311	   12-9	      I

252	   11-8-4	  *	       312	   11-1	      J

253	   12		  +	       313	   11-2	      K

254	   0-8-3	  '	       314	   11-3	      L

255	   11		  -	       315	   11-4	      M

256	   12-8-3	  .	       316	   11-5	      N

257	   0-1		  /	       317	   11-6	      O

260	   0		  0	       320	   11-7	      P

261	   1		  1	       321	   11-8	      Q

262	   2		  2	       322	   11-9	      R

263	   3		  3	       323	   0-2	      S

264	   4		  4	       324	   0-3	      T

265	   5		  5	       325	   0-4	      U

266	   6		  6	       326	   0-5	      V

267	   7		  7	       327	   0-6	      W

270	   8		  8	       330	   0-7	      X

271	   9		  9	       331	   0-8	      Y

272	   11-8-2	  :	       332	   0-9	      Z

273	   0-8-2	  ;	       333	   11-8-5     [

274	   12-8-6	  <	       334	   8-7	      \

275	   8-3		  =	       335	   12-8-5     ]

276	   11-8-6	  >	       336	   8-5	      ^

277	   12-8-2	  ?	       337	   8-2

NOTE

On some IBM 026 Keyboards this character

is graphically represented as a [].

A card containing an  8-2  in  column  1

with  all  remaining columns blank is an

end-of-file card.

D-3

PROGRAMMING NOTES

D.3  SUPPRESSION OF CARRIAGE RETURN/LINE FEED IN FORTRAN

It is often desirable to suppress the automatic  carriage	return/line

feed  (CR/LF)  following FORTRAN WRITE statements to achieve an easily

readable text.  The following three methods in  OS/8  FORTRAN  can	 be

used to achieve this result:

1.  Follow the I/O list of a WRITE statement with a comma.  Thus,

the following statements:

WRITE (1,100) N,

100	  FORMAT (1X,15HTHE VALUE OF A(,I2,5H) IS )

READ (1,101)A(N)

101	  FORMAT (F8.4)

result in the following single line (assume N has a value	 of

12 and a value of 147.83 is being input):

THE VALUE OF A(12) IS 147.83

2.  Use of an empty field print statement enables a  text  to	 be

printed  without	a following CR/LF when there is no variable

to be printed.  For example:

WRITE (1,102)IDUMMY,

102	  FORMAT ('DESIRED TEXT',I0)

3.  READ statement using break character, as follows:

READ (1,101) IA,IB,IC

101	  FORMAT ('A=',I1,'B=',I1,'C=',I1)

results in no CR/LF after each phrase is printed.	  That	is,

the output is all printed on a single line.

D.4  ACCESSING THE SYSTEM DATE IN A FORTRAN PROGRAM

The availability of the system Date word in location 17666	 is  useful

to	 many OS/8 programs.  The following FORTRAN program illustrates how

the Date can be accessed in SABR code:

C       PROGRAM PRINTS THE CURRENT DATE

C

S       DUMMY DATE

S       TAD I DATE

S       DCA TEMP

S       TAD TEMP

S       AND (7

S       DCA \IYR

S       TAD TEMP

S       RAR;RTR

S       AND (37

D-4

PROGRAMMING NOTES

S       DCA \IDAY

S       TAD TEMP

S       CLL RAL;RTL;RTL

S       AND (17

S       DCA \IMO

WRITE (1,100) IMO,IDAY,IYR

100     FORMAT (/'DATE:	'I2'-'I2'-197'I1/)

CALL EXIT

S       CPAGE 2

SDATE,  6211

S       7666

STEMP,  0

D.5  DETERMINING CORE SIZE ON PDP-8 FAMILY COMPUTERS

Many times system programs	 need  to  determine  the  amount  of  core

available	to them at run time.  For example, the OS/8 system programs

LOADER,  PAL8,  and  CREF	perform	 this  calculation.    Because	 of

differences  in the extended memory control of PDP-8 family computers,

subroutines that work on one machine might not work on another.

The following three conditions cause the most difficulty:

1.  On a  PDP-8  with	 an  extended  memory  control,	 addressing

nonexistent   memory   from  field  0  causes  the  following

instruction  to  be  skipped  and	  the	contents   of	the

corresponding field 0 location to be executed.  For example:

CDF 70		/NONEXISTENT FIELD

TAD I(X)		/EXECUTED LOCATION X

HLT		/THIS INSTRUCTION SKIPPED

.

.

.

X,  CLA CLL CML RAR	/LOAD 4000

The preceding code causes 4000 to be loaded into the  AC	and

the HLT instruction to be skipped when executed on a PDP-8.

2.  On a PDP-12 with an odd number of 4K banks (12K,	20K,  28K),

all  reads  in the first nonexistent field load zeros.  Reads

to higher fields, as well as all reads to nonexistent  memory

on  a  machine  with  an even number of 4K banks load all one

bits.

3.  The PDP-8/L normally treats all CDF's to fields 2	 through  7

as  NOP's.  (It  tests  bits  6  to  7  of  all  CDF  and CIF

instructions for 0's  before  executing  the  IOT.)  However,

there  is a special 12K option for the PDP-8/L called a BM08.

With this option a CDF to field 2 is valid, but  a  CDF's	 to

fields 4 through 7 remain NOP's.

D-5

PROGRAMMING NOTES

For those who are interested, the following subroutine has been tested

on	 the  PDP-8, 8/S, 8/L, 8/I, 8/E, PDP-12, and LINC-8 computers.	For

the purpose of this example, it is assembled at 00200.

/SUBROUTINE TO DETERMINE CORE SIZE.

/THIS SUBROUTINE WORKS ON ANY PDP-8 FAMILY

/COMPUTER. THE VALUE, FROM 1 TO 10 (OCTAL),

/OF THE FIRST NON-EXISTENT MEMORY FIELD IS

/RETURNED IN THE AC.

/NOTE - THIS ROUTINE MUST BE PLACED IN FIELD 0

0200 0000	 CORE,	 0

0201 7300		 CLA CLL

0202 6201	 COR0,	 CDF 0		 /(NEEDED FOR PDP-8L)

0203 1237		 TAD	 CORSIZ	 /GET FIELD TO TEST

0204 7006		 RTL

0205 7004		 RAL

0206 0217		 AND	 COR70	 /MASK USEFUL BITS

0207 1232		 TAD	 COREX

0210 3211		 DCA	 .+1	 /SET UP CDF TO FIELD

0211 6201	 COR1,	 CDF	 \N	 /N IS FIELD TO TEST.

0201 1635		 TAD I	 CORLOC	 /SAVE CURRENT CONTENTS

0213 7000	 COR2,	 NOP		 /(HACK FOR PDP-8!)

0214 3211		 DCA	 COR1

0215 1213		 TAD	 COR2	 /7000 IS A "GOOD" PATTERN

0216 3635		 DCA I	 CORLOC

0217 0070	 COR70,	 70		 /(HACK FOR PDP-8.,NO-OP)

0020 1635		 TAD I	 CORLOC	 /TRY TO READ BACK 7000

0221 7400	 CORX,	 7400		 /(HACK FOR PDP-8,.NO-OP)

0022 1221		 TAD	 CORX	 /GUARD AGAINST "WRAP AROUND"

0223 1236		 TAD	 CORV	 /TAD (1400)

0224 7640		 SZA CLA

0225 5232		 JMP	 COREX	 /NON-EXISTENT FIELD EXIT

0226 1211		 TAD	 COR1	 /RESTORE CONTENTS DESTROYED

0227 3635		 DCA I	 CORLOC

0230 2237		 ISZ	 CORSIZ	 /TRY NEXT HIGHER FIELD

0231 5202		 JMP	 COR0

0232 6201	 COREX,	 CDF0		 /LEAVE WITH DATA FIELD 0

0233 1237		 TAD	 CORSIZ	 /1ST NON-EXISTENT FIELD

0234 5600		 JMP I	 CORE

0235 0221	 CORLOC, CORX		 /ADDRESS TO TEST IN EACH FIELD

0236 1400	 CORV,	 1400		 /7000+7400+1400=0

0237 0001	 CORSIZ, 1		 /CURRENT FIELD TO TEST

D.6  USING PRTC12-F TO CONVERT OS/8 DECTAPES TO OS/12 LINCTAPES

Many users of OS/8 on the PDP-12 will be interested in the fact  that,

since  OS/8  uses	an  identical  file structure on all devices, PDP-8

DECtape in OS/8 format may be directly copied to OS/8 LINCtapes by the

PRTC12-F program.

D-6

PROGRAMMING NOTES

The PRTC12-F program uses the PDP-12 TC12-F hardware  option  to  read

DECtapes and convert these tapes to LINCtape.  This hardware option is

required to read DECtapes on the PDP-12

The PRTC12-F program is described in the document DEC-12-YIYA-D.  This

document  describes  the program operation in detail, and must be read

before attempting to use PRTC12-F.	 The operations that  convert  OS/8

format DECtapes are as follows:

1.  Mount the OS/8 DECtape  on  unit	1  and	a  PDP-12  LINCtape

formatted with 129 words per block on unit 2.

2.  When the READ questionnaire is displayed, respond as  follows

(responses  are  underlined;   the  character  }	 stands for

carriage return and  ~   stands for line feed):

READ 1777  }	 BLOCKS

TAPE FORMAT A  }   UNIT 1 }

STARTING WITH BLOCK 0 } ~

etc.

3.  When  the	 WRITE	questionnaire  is  displayed,  respond	 as

follows:

WRITE THE RESULT

IN TAPE FORMAT B }	ON UNIT 2 }

STARTING AT BLOCK 0 } ~

etc.

D.7  NOTES ON LOADING DEVICE HANDLERS

D.7.1  Problem With Multiple Input Files

There is a problem associated with reusing	 Device	 Handler  areas	 in

OS/8.  This problem is best illustrated by an example:

Assume a program  has  reserved  locations	 1000-1377  for	 its  input

handler  and  locations  7400-7577	 for  its  output  handler.  If the

program gives a USR FETCH command to load the DTA1 handler as an input

device handler, all 8 DECtape handlers will load into 1000-1377, since

they are all co-resident.	If another FETCH is issued to load the DTA2

handler  as an output device handler, that handler will not be loaded,

because it shares space with the DTA1 handler currently in core.  This

is fine - however, if the user now switches input devices and FETCHes

the paper tape reader handler as  an  input  device  handler  it  will

destroy the DTA2 handler and the next attempt to output using the DTA2

handler will produce errors.  There are two ways to  get  around  this

problem.

D-7

PROGRAMMING NOTES

1.  Always assign the handler which you expect to  stay  in  core

the  longest  first.  Most programs can process more than one

input file per program step (e.g., an assembly  pass  is	one

program  step)  but  only	 one  output file;  therefore, they

assign the output handler before any of the  input  handlers.

In  the above example, the problem would be eliminated if the

DTA2 handler were assigned first.

2.  Always give a USR RESET call before each	FETCH.	 Obviously,

this  call  should  not  delete  any open output files.  This

means that the USR will always load the new handler, even	 if

another  copy  is	 in  core.   The user must FETCH the output

handler again before issuing the USR  CLOSE  call,  otherwise

the USR will determine that the output handler is not in core

and give a MONITOR ERROR 3 message.

8K FORTRAN uses this second method for device-independent I/O  at	run

time.

D.7.2  Dynamically Loading Device Handlers

Some programs which use dynamic core allocations will want to use OS/8

Device  handlers  but  cannot afford to always allocate the maximum of

two pages per handler.  The following is a subroutine  which  loads  a

device handler dynamically, returning its entry in the AC.	 It assumes

that the name of the handler is in locations NAME1 and  NAME2,  and  a

subroutine	 GETPAG	 exists	 which	gets  a	 page  from  the  bottom of

available field 0 of storage and returns its address in the AC.   This

example  subroutine  runs in field 1 and can only be called from field

1, but can be rewritten for any other possibility.

ASSIGN, 0

TAD NAME1

DCA N1

TAD NAME2

DCA N2       /MOVE DEVICE NAME INTO "INQUIRE" COMMAND

CDF CIF 10

JMS I (7700

10	       /USRIN - FORCE USR INTO CORE

JMS I (200

12	       /INQUIRE

N1,	  0

N2,	  0

LOC1,	  0

JMP ASSERR   /NO SUCH DEVICE - QUIT

TAD LOC1

SZA	       /IS THE HANDLER ALREADY IN CORE?

JMP I ASSIGN /YES - RETURN ITS ENTRY POINT

JMS GETPAG   /GET A PAGE DYNAMICALLY

DCA LOC2

ASSTRY, TAD N2       /LOAD DEVICE NUMBER

D-8

PROGRAMMING NOTES

JMS I (200

1	       /FETCH

LOC2,	  0	       /PAGE TO FETCH INTO

JMP TWOPAG   /FAILED - MUST BE A TWO-PAGE HANDLER

TAD LOC2

JMP I ASSIGN /RETURN ENTRY POINT

TWOPAG, JMS GETPAG   /GET ANOTHER PAGE

ISZ LOC2     /SET "TWO PAGE HANDLER ALLOWED" BIT

CLA

JMP ASSTRY   /FETCH WILL SUCCEED THIS TIME

ASSERR, .....	       /ERROR ROUTINE

D.8 AVAILABLE LOCATIONS IN THE USR AREA

A few programs may need additional storage space in field 1  when	the

USR  is  in  core.	  A  number  of locations in the USR area (10000 to

11777) are available and may be used whenever the USR is in core.	The

locations are as follows:

1.  Locations 10000 to 10006 are available  for  scratch  storage

and/or ODT breakpoint usage, without restriction.

2.  All auto-index registers (locations 10010 to  10017)  may	 be

used, but these locations are destroyed by USR operations.

3.  Location 10020 to 10037 may be used as scratch  storage  with

no restrictions.

4.  Locations 11400 to 11777 are used by the USR to preserve	the

last directory segment read while performing a LOOKUP, ENTER,

or CLOSE operation.  Location 10007 contains a key specifying

which segment of which device is currently in core.

Any user program may use locations 11400 to 11777 as  scratch

storage  as  long	 as  location  10007 is set to 0 before the

first  use.   Of	course,	 the  LOOKUP,  ENTER,	and   CLOSE

operations  will read a directory segment into 11400 to 11777

and set 100007 to a non-zero value again.

D.9   ACCESSING ADDITIONAL INFORMATION WORDS IN OS/8

In all of these cases, the USR must have been previously brought  into

core with the USRIN function.

D.9.1   After a LOOKUP or ENTER

After a LOOKUP or ENTER, location 10017 points to the length  word	 of

the  file entry.  To get a pointer to the first Additional Information

Word, a program would execute the following code:

D-9

PROGRAMMING NOTES

CDF 10

TAD I (1404	  /GET # OF ADDITIONAL INFORMATION WORDS

/FROM DIRECTORY

SNA

JMP NONE	  /NO ADDITIONAL INFORMATION WORDS

TAD I (0017

DCA POINTER

.

.

.

"POINTER" now points to the first Additional Information Word.

D.9.2   After a CLOSE

Because CLOSE is a legal operation even if no output file is  present,

it	 is  not  suggested  that  Additional Information Words be modified

following a CLOSE.	 To alter the Additional  Information  Words  of  a

permanent	file,  do  a LOOKUP to get the directory segment into core,

then alter the words and rewrite the directory segment.

D.9.3   Rewriting the Current Directory Segment

Whenever a user program changes the Additional Information Words of  a

file, it must rewrite the directory segment containing that file entry

in order to make sure the changes are permanently recorded.

The following code, which must be in field 1, will rewrite the current

directory segment:

CDF 10	  /CODE IS IN FIELD 1

TAD 7		  /GET DIRECTORY KEY WORD

AND (7	  /EXTRACT SEGMENT NUMBER

DCA SEGNO

CIF 0

JMS I 51	  /LOC 51 POINTS TO THE DEVICE HANDLER

4210		  /WRITE OPERATION

1400		  /DIRECTORY SEGMENT CORE ADDRESS

SEGNO,  0

JMP ERROR	  /ERROR REWRITING DIRECTORY

Location 10051 will always point to the  device  handler  entry  point

used  to  read  in	 the  last directory segment, following a LOOKUP or

ENTER operation.

D-10

PROGRAMMING NOTES

D.10 SABR PROGRAMMING NOTES

D.10.1 Optimizing SABR Code

There are two types of users who will be using the	 SABR  assembler  -

those  who	 like the convenience of page-boundary-independent code and

are willing to pay the price for it, and those who need a	relocatable

assembler	but  are  still	 very location conscious.  These optimizing

hints are directed to the latter user.

One way to beat the  high	cost  of  non-paged  code  is  to  Page	 It

Yourself.	 This  is  done	 by  using the LAP (Leave Automatic Paging)

pseudo-op and the PAGE pseudo-op to force paging where  needed.   This

saves  2  to  4  instructions  per	 page  from elimination of the page

escape.  In addition, the fact  that  the	program	 must  be  properly

segmented may save a considerable amount.

Wasted core  may  be  reduced  by	eliminating  the  ever-present	CDF

instructions which SABR inserts into a program.  This is done by using

n fake indirects". Define the following op codes:

OPDEF  ANDI 0400

OPDEF  TADI 1400

OPDEF  ISZI 2400

OPDEF  DCAI 3400

.

.

.

These codes correspond to the PDP-8 memory reference instructions	but

they  include an indirect bit.  The difference can best be appreciated

by an example:

If X is off-page, the sequence

LABEL,  SZA

DCA X

is assembled by SABR into

LABEL,  SZA

JMS 45

SKP

DCA I (X)

or four instructions and one literal.

The sequence

D-11

PROGRAMMING NOTES

PX,     X

.

.

.

LABEL,  SZA

DCAI PX

assembles into three instructions for a saving of 40  percent.   Note,

however,  that  the  user	must  be  sure	that the data field will be

correct when the code at LABEL is encountered.   Also  note  that	the

SABR  assumes  that  the  Data Field is equal to the Instruction Field

after a JMS instruction, so subroutine returns should not use the JMPI

op code.

The standard method to fetch a scalar integer argument of a subroutine

in SABR is:

Code

IARG,   DUMMY X

0

SUBR,   BLOCK 2

TAD I SUBR

DCA X

INC SUBR#

TAD I SUBR

DCA X#

INC SUBR#

TAD I X

DCA IARG

.

.

.

X,      BLOCK 2

This code	requires  19  words  of	 core  and  takes  several  hundred

microseconds to execute.  The following sequence:

Code

IARG,   0

SUBR,   BLOCK 2

TAD I SUBR

DCA X

INC SUBR#

TADI SUBR#

DCA IARG

INC SUBR#

X,      HLT	 /THIS IS A CDF

TAD I IARG

DCA IARG

.

.

.

takes only 14 words and executes in approximately 1/3 the time.

D-12

PROGRAMMING NOTES

D.10.2  Calling the USR and Device Handlers from SABR Code

One important thing to remember is that any code which calls  the	USR

must  not	reside	in  locations  10000 to 11777.	Therefore, any SABR

routine which calls the USR must be loaded into a field other  than  1

or	 above	location 2000 in field 1. To call the USR from SABR use the

sequence:

CPAGE n	  /N=7+(# OF ARGUMENTS)

6212		  /CIF 10

JMS 7700	  /OR 200 IF USR IN CORE

REQUEST

ARGUMENTS	  /OPTIONAL DEPENDING ON REQUEST

ERROR RETURN	  /OPTIONAL DEPENDING ON REQUEST

To call a device handler from SABR use the sequence:

CPAGE12	  /10 IF "HAND" IN PAGE 0

6202		  /CIF 0

JMS I HAND	  /DO NOT USE JMSI

FUNCT

ADDR

BLOCK

ERROR RETURN

SKP

HAND,	  0		  /"HAND" MUST BE ON SAME PAGE

/AS CALL, OR IN PAGE 0!!

D-13

APPENDIX E

CHARACTER CODES AND CONVENTIONS

Table E-1 contains a list of the control characters used by  OS/8	and

associated	 system	 programs.   Table  E-2 contains the OS/8 character

set, which is a subset of the complete ASCII code, the unlisted  codes

are  generally  not  used	by  OS/8  or the system programs.  Note the

following:

1.  On some terminals, the character back-arrow (_)  is  replaced

by  an  underline	 (_)   character,  and	the up-arrow (^) is

replaced by circumflex (^).

2.  Some terminals use  parity  codes	 rather	 than  forcing	the

leading  bit  of the 8-bit character code to be a 1. To avoid

problems, OS/8 system programs always ignore the	parity	bit

during ASCII input.

3.  OS/8 does not handle lower case characters (octal	 codes	341

through  372).  The  exceptions to this are the editors, EDIT

and TECO.	 The KL8E and LPSV  handlers  can  be  modified	 to

handle lower case.

Table E-1

OS/8 Control Characters

+---------------------------------------------------------------------+

|	  Octal								   |

|	  8-bit	      Character						   |

|	  Code		Name			  Remarks		   |

+---------------------------------------------------------------------+

|	  000	    null		Ignored in ASCII input.		   |

|									   |

|	  200	    leader/trailer	Leader/trailer	code  precedes	and|

|					follows	 the data portion of binary|

|					files.				   |

|									   |

|	  203	    CTRL/C		OS/8 break character, forces return|

|					to Keyboard Monitor, echoed as ^C. |

|									   |

|	  207	    BELL		CTRL/G.				   |

|									   |

|	  211	    TAB			CTRL/I, horizontal tabulation.	   |

|									   |

|	  212	    LINE FEED		Used as a control character by	the|

|					Command Decoder and ODT.	   |

|									   |

|	  213	    VT			CTRL/K, vertical tabulation.	   |

+---------------------------------------------------------------------+

E-1

CHARACTER CODES AND CONVENTIONS

Table E-1 (Cont.)

OS/8 Control Characters

+---------------------------------------------------------------------+

|	  Octal								   |

|	  8-bit	      Character						   |

|	  Code		Name			  Remarks		   |

+---------------------------------------------------------------------+

|	  214	    FORM		CTRL/L, form feed.		   |

|									   |

|	  215	    RETURN		Carriage return,  generally  echoed|

|					as  carriage  return  followed by a|

|					line feed.			   |

|									   |

|	  216				Used only  on  LS8E  line  printer.|

|					Puts  current  line  into  expanded|

|					character mode.			   |

|									   |

|	  217	    CTRL/O		Break Character, used  to  suppress|

|					Teletype output, echoed as ^O.	   |

|									   |

|	  225	    CTRL/U		Delete current input  line,  echoed|

|					as ^U.				   |

|									   |

|	  232	    CTRL/Z		End-of-File character for all ASCII|

|					and  binary  files  (in relocatable|

|					binary	files  CTRL/Z  is   not	  a|

|					terminator  if it occurs before the|

|					trailer code).			   |

|									   |

|	  233	    ESC			ESCape	replaces  ALTMODE  on  some|

|					terminals.   Considered	 equivalent|

|					to ALTMODE.			   |

|									   |

|	  375	    ALTMODE		Special	  break	   character	for|

|					Teletype input.			   |

|									   |

|	  376	    PREFIX		PREFIX	replaces  ALTMODE  on  some|

|					terminals.   Considered	 equivalent|

|					to ALTMODE.			   |

|									   |

|	  377	    RUBOUT		Key  is	 labeled  DELETE  on   some|

|					terminals.   Deletes  the  previous|

|					character typed.		   |

+---------------------------------------------------------------------+

E-2

CHARACTER CODES AND CONVENTIONS

Table E-2

ASCII Character Codes

+-------+--------+----------+--------------+--------------------------+

| Octal |	      |	 Punched |  Character	|			   |

| 8-bit |	6-bit |	 Card	 |  Representa- |			   |

| Code  |	Code  |	 Code	 |  tion	|  Remarks		   |

+-------+--------+----------+--------------+--------------------------+

|	     |	      |		 |		|			   |

| 240   |	  40  |	 blank	 |		|  space (non-printing)	   |

| 241   |	  41  |	 11-8-2	 |     !	|  exclamation point	   |

| 242   |	  42  |	 8-7	 |     "	|  quotation marks	   |

| 243   |	  43  |	 8-3	 |     #	|  number sign		   |

| 244   |	  44  |	 11-8-3	 |     $	|  dollar sign		   |

| 245   |	  45  |	 0-8-3	 |     %	|  percent		   |

| 246   |	  46  |	 12	 |     &	|  ampersand		   |

| 247   |	  47  |	 8-5	 |     '	|  apostrophe or acute	   |

|	     |	      |		 |		|  accent		   |

|	     |	      |		 |		|			   |

| 250   |	  50  |	 12-8-5	 |     (	|  opening parenthesis	   |

| 251   |	  51  |	 11-8-5	 |     )	|  closing parenthesis	   |

| 252   |	  52  |	 11-8-4	 |     *	|  asterisk		   |

| 253   |	  53  |	 12-8-6	 |     +	|  plus			   |

| 254   |	  54  |	 0-8-3	 |     ,	|  comma		   |

| 255   |	  55  |	 11	 |     _	|  minus sign or hyphen	   |

| 256   |	  56  |	 12-8-3	 |     .	|  period or decimal point |

| 257   |	  57  |	 0-1	 |     /	|  slash		   |

|	     |	      |		 |		|			   |

| 260   |	  60  |	 0	 |     0	|			   |

| 261   |	  61  |	 1	 |     1	|			   |

| 262   |	  62  |	 2	 |     2	|			   |

| 263   |	  63  |	 3	 |     3	|			   |

| 264   |	  64  |	 4	 |     4	|			   |

| 265   |	  65  |	 5	 |     5	|			   |

| 266   |	  66  |	 6	 |     6	|			   |

| 267   |	  67  |	 7	 |     7	|			   |

|	     |	      |		 |		|			   |

| 270   |	  70  |	 8	 |     8	|			   |

| 271   |	  71  |	 9	 |     9	|			   |

| 272   |	  72  |	 8-2	 |     :	|  colon		   |

| 273   |	  73  |	 11-8-6	 |     ;	|  semicolon		   |

| 274   |	  74  |	 12-8-4	 |     <	|  less than		   |

| 275   |	  75  |	 8-6	 |     =	|  equals		   |

| 276   |	  76  |	 0-8-6	 |     >	|  greater than		   |

| 277   |	  77  |	 0-8-7	 |     ?	|  question mark	   |

|	     |	      |		 |		|			   |

| 300   |	  00  |	 8-4	 |     @	|  at sign		   |

| 301   |	  01  |	 12-1	 |     A	|			   |

| 302   |	  02  |	 12-2	 |     B	|			   |

| 303   |	  03  |	 12-3	 |     C	|			   |

| 304   |	  04  |	 12-4	 |     D	|			   |

| 305   |	  05  |	 12-5	 |     E	|			   |

| 306   |	  06  |	 12-6	 |     F	|			   |

| 307   |	  07  |	 12-7	 |     G	|			   |

+-------+--------+----------+--------------+--------------------------+

E-3

CHARACTER CODES AND CONVENTIONS

Table E-2 (Cont.)

ASCII Character Codes

+-------+--------+----------+--------------+--------------------------+

| Octal |	      |	 Punched |  Character	|			   |

| 8-bit |	6-bit |	 Card	 |  Representa- |			   |

| Code  |	Code  |	 Code	 |  tion	|  Remarks		   |

+-------+--------+----------+--------------+--------------------------+

|	     |	      |		 |		|			   |

| 310   |	  10  |	 12-8	 |     H	|			   |

| 311   |	  11  |	 12-9	 |     I	|			   |

| 312   |	  12  |	 11-1	 |     J	|			   |

| 313   |	  13  |	 11-2	 |     K	|			   |

| 314   |	  14  |	 11-3	 |     L	|			   |

| 315   |	  15  |	 11-4	 |     M	|			   |

| 316   |	  16  |	 11-5	 |     N	|			   |

| 317   |	  17  |	 11-6	 |     O	|			   |

|	     |	      |		 |		|			   |

| 320   |	  20  |	 11-7	 |     P	|			   |

| 321   |	  21  |	 11-8	 |     Q	|			   |

| 322   |	  22  |	 11-9	 |     R	|			   |

| 323   |	  23  |	 0-2	 |     S	|			   |

| 324   |	  24  |	 0-3	 |     T	|			   |

| 325   |	  25  |	 0-4	 |     U	|			   |

| 326   |	  26  |	 0-5	 |     V	|			   |

| 327   |	  27  |	 0-6	 |     W	|			   |

|	     |	      |		 |		|			   |

| 330   |	  30  |	 0-7	 |     X	|			   |

| 331   |	  31  |	 0-8	 |     Y	|			   |

| 332   |	  32  |	 0-9	 |     Z	|			   |

| 333   |	  33  |	 12-8-2	 |     [	|  opening bracket,SHIFT/K |

| 334   |	  34  |	 11-8-7	 |     \	|  backslash, SHIFT/L	   |

| 335   |	  35  |	 0-8-2	 |     ]	|  closing bracket, SHIFT/M|

| 336   |	  36  |	 12-8-7	 |     ^	|  circumflex		   |

| 337   |	  37  |	 0-8-5	 |     _	|  underline - EOF signal  |

+-------+--------+----------+--------------+--------------------------+

NOTES

1.  These are the DEC029 standard card codes.

2.  On most DEC Teletypes circumflex is replaced by up-arrow (^).

3.  A card containing	 0-8-5	in  column  1  with  all  remaining

columns blank is an end-of-file card.

4.  On most DEC Teletypes underline  is  replaced  by	 back-arrow

(_).

5.  On some IBM 029 keyboards 1 is graphically represented  as  a

cent sign (c#/).

6.  On some IBM 029 keyboards \  is  graphically  represented	 as

logical NOT (-,).

E-4

CHARACTER CODES AND CONVENTIONS

7.  On some IBM 029 keyboards	 ^  is graphically  represented	 as

vertical bar (|).

8.  On a very few LP08 line printers, the character diamond  (<>)

is printed instead of backslash.

9.  On a very few LP08 line printers, the character heart (V)	 is

printed instead of underline.

10.  The character number sign on some terminals  is  replaced	 by

pound sign (L).

E-5

APPENDIX F

OS/8 INPUT/OUTPUT ROUTINES

Appendix F describes a set of generalized I/O routines for	 use  under

the OS/8 system.  The routines presented here are used in all the OS/8

CUSPs (Commonly Used System Programs)  in	more  or  less	this  form.

Variations	 are  made  depending on the particular application and how

errors are to be handled.	The routines, as indicated,  will  work	 as

presented.	  The  routines work most efficiently in field 1, since CIF

10's are not necessary when addressing the Monitor,  and  the  Command

Decoder tables are similarly available.  Obviously the routines can be

modified to run in any memory field or core locations.

F.1  GENERAL DESCRIPTION

These subroutines assume that the Command Decoder tables have been set

up	 to  indicate  the  proper I/O devices.	 The routines handle device

handler assignment without user interference.   All  I/O  is  done	 by

simple  subroutine	 calls.	  The user program never needs to interface

with the Monitor or  device  handlers.   All  buffering  and  internal

bookkeeping  are  performed by the routines.  In these routines, it is

assumed that only one output device is  used  at  a  time,	 i.e.,	the

output  routine  does  not	 automatically	set  up for the next output

device.  This modification can be made if desired.	 As  many  as  nine

inputs  are  handled  automatically.   When  input	 from one device is

exhausted, the input  routine  will  automatically	 utilize  the  next

device specified in the Command Decoder list of inputs.

F.2  SUBROUTINES FUNCTIONS

Following is a brief list of the subroutines and their functions.

ICHAR - Character input routine.

Call sequence:

JMS ICHAR

ERROR RETURN

NORMAL RETURN

Error:

If AC>0, an EOF on input has occurred.  No  more

input is available.	If AC<0, a device error has

occurred.

F-1

OS/8 INPUT/OUTPUT ROUTINES

Normal:

8 bit character is in the AC.

OCHAR - Character output routine.

Call:

TAD CHAR 	/8 BIT CHARACTER

JMS OCHAR

ERROR RETURN

NORMAL RETURN

Error:

AC<0 implies a fatal error.

AC > or = 0 implies that the hole  allotted	for

output was exceeded.

Normal:

AC=0. The character has been put into the device

output buffer.

IOPEN - Input initialize routine.

Call:

JMS IOPEN

RETURN

Return:

Input pointers reset.  The next  call  to  ICHAR

will	 read  from the first device in the Command

Decoder input list.

OOPEN - Output initialize routine.

Call:

JMS OOPEN

ERROR RETURN

NORMAL RETURN

Error:

If AC> or = 0, no output device was	specified.

If AC<0, an error occurred opening the file.

Normal:

An output file has been opened.   No	 action	 if

the output was a non-file structured device.

F-2

OS/8 INPUT/OUTPUT ROUTINES

OCLOSE - Output close routine.

Call:

JMS OCLOSE

ERROR RETURN

NORMAL RETURN

Error:

Either the closing length is too large  for	the

space allotted or an output error has occurred.

Normal:

The output file is now a permanent file  on	the

output device.

F.3  SUBROUTINE PARAMETERS

These subroutines handle  device  assignment  and	internal  buffering

automatically.  To accomplish this, certain parameters must be defined

at assembly time.	These parameters specify  all  details	of  handler

location, and buffer size for the routines.

Parameter			Definition	  _________			__________

INBUF =	 Address of input buffer.

INCTL =	 Input buffer control word.   See  the	section	 on

using device handlers for details of the control

word format.

OUBUF =	 Output buffer address.

OUCTL =	 Output	 buffer	 control  word.	  This	must  be  a

negative number to indicate a write operation.

INRECS =	 Number of input records in input buffer.  INRECS =

INCTL/256 (DECIMAL).

INDEVH =	 Address of input device handler.

OUDEVH =	 Address of output device handler.

The parameters can either be a part of the actual	subroutine  source,

or	 they can be contained in a separate parameter file to be assembled

with the  subroutine  file.   The	latter	approach  provides  greater

flexibility in using the routines.

F-3

OS/8 INPUT/OUTPUT ROUTINES

F.3.1  Example

Following is a sample of the use  of  the	subroutines.   The  program

simply  calls  the Command Decoder, and transfers input from the input

devices to the output file, closes the output, and exits.

FIELD 1

*2000

JMS I (7700		/LOCK MONITOR INTO CORE.

10

CALLCD,   JMS I (200		/CALL COMMAND DECODER

5			/TO PICK OPTIONS.

0

JMS I (IOPEN		/SETUP TO START LOOKING AT

/CD INPUT FILE.

JMS I (OOPEN		/OPEN UP AN OUTPUT FILE.

SMA CLA			/IF AC<0, WE HAD A FATAL

JMP OK			/TYPE ERROR. AC>0 IS O.K.

JMS TERR			/ERROR.

TEXT /OPEN FAILED/

OK,       JMS I (ICHAR

JMP TSTEOF		/EITHER ERROR OR EOF.

/SAVE IT.

JMS I (OCHAR		/TRANSFER THE CHARACTER

JMP OUTERR		/OUTPUT ERROR

JMP OK			/TRANSFER UNTIL EOF FOUND.

TSTEOF,   SMA CLA			/IF NEG., FATAL

JMP CLOSE		/EOF. CLOSE OUTPUT

JMS TERR

TEXT /READ ERROR/

CLOSE,    JMS I (OCLOSE		/CLOSE OUTPUT FILE.

JMP CLERR		/CLOSE ERROR

JMP CALLCD		/NEXT.

OUTERR,   JMS TERR

TEXT /OUTPUT ERROR/

CLERR,    JMS TERR

TEXT /CLOSE ERROR/

TERR,     0

TAD I TERR

RTR;RTR;RTR

JMS TYPIT

TAD I TERR

JMS TYPIT

ISZ TERR

JMP TERR+1

TYPIT,    0

AND (77

SNA

JMP CRLF

TAD (300

JMS TTYOUT

JMP I TYPIT

F-4

OS/8 INPUT/OUTPUT ROUTINES

CRLF,     TAD (215

JMS TTYOUT

TAD (212

JMS TTYOUT

JMP CALLCD

TTYOUT,   0

TLS

TSF

JMP.-1

CLA

JMP I TTYOUT

F.3.2  Subroutine Listing

A listing of the routines follows.	 The parameters are set up in  such

a	way  as	 to  allow  them  to  be put into a separate file.  Another

parameter, ORIGIN, determines the location of the routines.

F-5

OS/8 INPUT/OUTPUT ROUTINES

/OS/8 GENERAL CHARACTER I/O ROUTINES FO   PAL8-VC0  05-OCT-96 PAGE 1

/OS/8 GENERAL CHARACTER I/O ROUTINES FOR BLEEP

/CALLED AS FOLLOWS:

/JMS I (IOPEN           INITIALIZES THE INPUT ROUTINE

/JMS I (ICHAR           READS A CHARACTER

/ERROR RETURN           AC>0 IF END OF FILE, AC<0 IF READ

/                       ERROR

/JMS I (OOPEN           INITIALIZES THE OUTPUT ROUTINE

/ERROR RETURN           AC>0 IF NO OUTPUT DEVICE/FILE, AC<0

/                       IF ERROR

/JMS I (OCHAR           OUTPUTS A CHARACTER

/ERROR RETURN           OUTPUT ERROR OR TOO MUCH OUTPUT

/JMS I (OCLOSE          CLOSES THE OUTPUT FILE

/ERROR RETURN           FILE TOO LARGE TO BE CLOSED OR OUTPUT

/                       ERROR

/JMS I (OTYPE           RETURNS DCB WORD OF OUTPUT DEVICE IN

/                       AC

/PARAMETERS NEEDED:

/INBUF=         ADDRESS OF INPUT BUFFER

/INCTL=         INPUT BUFFER CONTROL WORD

/OUBUF=         ADDRESS OF OUTPUT BUFFER

/OUCTL=         OUTPUT BUFFER CONTROL WORD (MUST BE NEGATIVE)

/INRECS=        [INCTL/256]

/INDEVH=        ADDRESS OF PAGE FOR INPUT HANDLER

/OUDEVH=        ADDRESS OF PAGE FOR OUTPUT HANDLER

/IORFLD=        /FIELD OF THE SUBROUTINES.

/ORIGIN=        /THE SUBROUTINES RESIDE AT 16600.

/ASSUMES I/O MONITOR IS RESIDENT IN CORE.

/CAN BE CALLED FROM ANY FIELD WITH BUFFERS IN ANY FIELD.

F-6

OS/8 INPUT/OUTPUT ROUTINES

/OS/8 GENERAL CHARACTER I/O ROUTINES FO   PAL8-VC0  05-OCT-96 PAGE 2

/PARAMETER DEFINITION:

5000          OUBUF=  5000    /OUTPUT BUFFER STARTS AT 05000

4200          OUCTL=  4200    /AND IS 2 PAGES LONG.

6600          OUDEVH= 6600    /OUTPUT HANDLER GETS LOADED AT 6600.

/WE ALLOW TWO PAGE HANDLERS.

5400          INBUF=  5400    /INPUT BUFFER STARTS AT 05400

0200          INCTL=  0200    /ALSO TWO PAGES LONG.

0001          INRECS= 1       /2 PAGES = 1 RECORD

7000          INDEVH= 7000    /ALLOW 2 PAGE INPUT HANDLER AT 7000.

0001          IORFLD= 1       /FIELD OF THE SUBROUTINES.

6600          ORIGIN= 6600    /THE SUBROUTINES RESIDE AT 16600.

0001          FIELD    IORFLD

6600          *ORIGIN

0000          INFLD=  INCTL&70        /INPUT BUFFER FIELD

0000          OUFLD=  OUCTL&70        /OUTPUT BUFFER FIELD

7760          DCB=    7760            /DEVICE CONTROL TABLE

F-7

OS/8 INPUT/OUTPUT ROUTINES

/OS/8 GENERAL CHARACTER I/O ROUTINES FO   PAL8-VC0  05-OCT-96 PAGE 3

016600  0000  ICHAR,  0                       /INPUT A CHARACTER

016601  7600  IN7600, 7600

016602  6214          RDF                     /SAVE CALLING FIELD FOR RETURN

016603  1232          TAD     INCDIF

016604  3316          DCA     INRTRN

016605  6201  INCHAR, CDF     INFLD           /DATA FIELD TO FIELD OF BUFFER

016606  2265          ISZ     INJMP           /3 - WAY UNPACKING SWITCH

016607  2325          ISZ     INCHCT          /INPUT BUFFER EXHAUSTED?

016610  5265  INJMPP, JMP     INJMP           /NO..UNPACK THE NEXT CHAR.

016611  1321          TAD     INEOF           /DID LAST READ GIVE EOF ON

016612  7650          SNA CLA                 / THIS DEVICE?

016613  5216          JMP     INGBUF          /NO. CONTINUE READING.

016614  4325  GETNEW, JMS     INNEWF          /YES..GET NEXT INPUT IF IT

/ EXISTS.

016615  5264          JMP     EOFERR          /TAKE EOF EXIT FROM ICHAR.

016616  1324  INGBUF, TAD     INCTR           /INCTR HOLDS THE CURRENT LENGTH

/ OF THE INPUT FILE.  WHEN THE

/ AMOUNT REMAINING TO READ IS

/ LESS THAN THE SIZE OF THE

/ INPUT BUFFER, AN EOF IS

/ SIGNALED.

016617  7100          CLL

016620  1377          TAD     (INRECS

016621  7420          SNL

016622  3324          DCA     INCTR           /UPDATE REMAINING LENGTH

016623  7430          SZL

016624  2321          ISZ     INEOF           /AND SIGNAL EOF FOR NEXT READ.

016625  7172          CLL CML CMA RTR         /CONSTRUCT A CONTROL WORD FOR

016626  7012          RTR                     / THIS READ FROM THE OVERFLOW,

016627  7012          RTR                     / IF ANY, AND THE STANDARD

016630  1376          TAD     (INCTL+1        / CONTROL WORD.

016631  3235          DCA     INCTLW

016632  6203  INCDIF, CIF CDF 0               /NOW CALL THE INPUT HANDLER

016633  6211          CDF     10              /HANDLER IS IN FIELD 0

016634  4736          JMS I   INHNDL

016635  0000  INCTLW, 0                       /INPUT CONTROL WORLD

016636  5400  INBUFP, INBUF                   /INPUT BUFFER ADDRESS

016637  0000  INREC,  0                       /POINTER TO INPUT RECORD

016640  5260          JMP     INERRX

016641  1237  INBREC, TAD     INREC

016642  1377          TAD     (INRECS         /UPDATE POINTER INTO FILE

016643  3237          DCA     INREC

016644  1235          TAD     INCTLW          /NOW COMPUTE THE NUMBER OF

016645  0201          AND     IN7600          / CHARACTERS IN THIS INPUT

016646  7104          CLL RAL                 / BUFFER

016647  1235          TAD     INCTLW

016650  0201          AND     IN7600

016651  7040          CMA

016652  3325          DCA     INCHCT          /NEW NUMBER OF CHARACTERS

016653  1210          TAD     INJMPP          /RESET 3 WAY SWITCH

016654  3265          DCA     INJMP

F-8

OS/8 INPUT/OUTPUT ROUTINES

/OS/8 GENERAL CHARACTER I/O ROUTINES FO   PAL8-VC0  05-OCT-96 PAGE 3-1

016655  1236          TAD     INBUFP

016656  3323          DCA     INPTR           /AND BUFFER POINTER

016657  5205          JMP     INCHAR          /NOW READ THE BUFFER

016660  2321  INERRX, ISZ     INEOF           /SET EOF JUST IN CASE

016661  7700          SMA CLA                 /IF <0, A PHYSICAL ERROR

016662  5241          JMP     INBREC          /EOF ON INPUT

016663  7330  INERR,  CLA CLL CML RAR         /FATAL

016664  5316  EOFERR, JMP     INRTRN          /GET OUT

016665  7402  INJMP,  HLT                     /3 WAY UNPACK SWITCH

016666  5307          JMP     ICHAR1          /GET 1ST OF 3

016667  5303          JMP     ICHAR2          /SECOND

016670  1210  ICHAR3, TAD     INJMPP

016671  3265          DCA     INJMP           /SET FOR FIRST CHAR. NEXT

016672  1723          TAD I   INPTR           /THE THIRD WORD IS MADE OF THE

016673  0320  IN200,  AND     IN7400          / HIGH ORDER FOUR BITS OF THE

016674  7112          CLL RTR                 / FIRST TWO.

016675  7012          RTR

016676  1235          TAD     INCTLW

016677  7012          RTR

016700  7012          RTR

016701  2323          ISZ     INPTR           /POINT TO NEXT WORD

016702  5310          JMP     INCOMN          /GET OUT WITH CHAR IN AC

016703  1723  ICHAR2, TAD I   INPTR

016704  0320          AND     IN7400          /SAVE HIGH ORDER FOR THIRD

016705  3235          DCA     INCTLW          / WORD

016706  2323          ISZ     INPTR

016707  1723  ICHAR1, TAD I   INPTR

016710  0375  INCOMN, AND     (377

016711  1374          TAD     (-232           /IS IT A ^Z (EOF)?

016712  7450          SNA

016713  5214          JMP     GETNEW          /YES..LOOK AT NEXT INPUT

016714  1373          TAD     (232

016715  2200          ISZ     ICHAR           /TAKE NORMAL RETURN

016716  0000  INRTRN, 0                       /CIF CDF N.

016717  5600          JMP I   ICHAR

016720  7400  IN7400, 7400

016721  0000  INEOF,  0                       /END OF FILE FLAG

016722  0000  INFPTR, 0                       /POINTER TO INPUT FILE NAMES

016723  0000  INPTR,  0                       /INPUT BUFFER POINTER

016724  0000  INCTR,  0                       /LENGTH OF CURRENT INPUT FILE

F-9

OS/8 INPUT/OUTPUT ROUTINES

/OS/8 GENERAL CHARACTER I/O ROUTINES FO   PAL8-VC0  05-OCT-96 PAGE 4

016725  7777  INNEWF, -1                      /OPEN A NEW INPUT FILE

6725  INCHCT= INNEWF

016726  6211          CDF     10

016727  1372          TAD     (INDEVH+1       /INITIALIZE JUST IN CASE

016730  3336          DCA     INHNDL

016731  1722          TAD I   INFPTR          /MORE INPUT?

016732  7450          SNA

016733  5725          JMP I   INNEWF          /NOPE

016734  4673          JMS I   IN200           /CALL MONITOR TO GET HANDLER

016735  0001          1

016736  0000  INHNDL, 0

016737  7402          HLT                     /VERY BAD!

016740  1722          TAD I   INFPTR

016741  0371          AND     (7760           /GET INPUT FILE LENGTH

016742  7440          SZA

016743  1370          TAD     (17

016744  7132          CLL CML RTR

016745  7012          RTR                     /NEGATIVE OF FILE LENGTH

016746  3324          DCA     INCTR

016747  2322          ISZ     INFPTR          /POINT TO STARTING BLOCK

016750  1722          TAD I   INFPTR

016751  3237          DCA     INREC           /STORE IN HANDLER CALL

016752  2322          ISZ     INFPTR          /NEXT INPUT.

016753  3321          DCA     INEOF           /CLEAR EOF FLAG.

016754  2325          ISZ     INNEWF

016755  5725          JMP I   INNEWF

016770  0017

016771  7760

016772  7001

016773  0232

016774  7546

016775  0377

016776  0201

016777  0001

7000          PAGE

F-10

OS/8 INPUT/OUTPUT ROUTINES

/OS/8 GENERAL CHARACTER I/O ROUTINES FO   PAL8-VC0  05-OCT-96 PAGE 5

017000  0000  OOPEN,  0                       /OPEN OUTPUT FILE

017001  7600  OU7600, 7600

017002  1347          TAD     OU7601          /POINT TO OUTPUT FILE NAME IN

017003  3221          DCA     OUBLK           / CD AREA

017004  1377          TAD     (OUDEVH+1

017005  3214          DCA     OUHNDL          /INITIALIZE OUTPUT HANDLER

017006  1601          TAD I   OU7600          /PICK UP OUTPUT DEVICE NUMBER

017007  0376          AND     (17

017010  7450          SNA                     /IS THERE ONE?

017011  5244          JMP     ONOFIL          /NO..INHIBIT OUTPUT

017012  4775          JMS I   (200            /FETCH OUTPUT HANDLER

017013  0001          1

017014  0000  OUHNDL, 0

017015  7402          HLT                     /BAD THING

017016  1601  OUENTR, TAD I   OU7600

017017  4775          JMS I   (200            /ENTER THE OUTPUT FILE

017020  0003          3

017021  7601  OUBLK,  7601                    /GETS STARTING BLOCK OF HOLE

017022  0000  OUELEN, 0                       /GETS SIZE OF HOLE AVAILABLE

017023  5232          JMP     OEFAIL          /FAILURE. SEE WHAT WE DID.

017024  3350          DCA     OUCCNT          /CLEAR CLOSING LENGTH

017025  3774          DCA I   (OUTINH         /CLEAR OUTPUT INHIBIT

017026  4773          JMS I   (OUSETP         /SET UP POINTERS

017027  2200          ISZ     OOPEN

017030  6213  OORETN, CDF CIF 10

017031  5600          JMP I   OOPEN           /RETURN O.K.

017032  1601  OEFAIL, TAD I   OU7600          /IF LENGTH=0, GIVE OPEN ERROR

017033  0372          AND     (7760           /IF NOT, MAKE IT 0 AND TRY

017034  7650          SNA CLA                 / AGAIN

017035  5242          JMP     ONTERR          /WAS 0. FAILED

017036  1601          TAD I   OU7600

017037  0376          AND     (17             /MAKE IT 0

017040  3601          DCA I   OU7600

017041  5216          JMP     OUENTR          /AND TRY AGAIN

017042  7330  ONTERR, CLA CLL CML RAR

017043  5230          JMP     OORETN

017044  2774  ONOFIL, ISZ I   (OUTINH         /INHIBIT OUTPUT

017045  5230          JMP     OORETN

F-11

OS/8 INPUT/OUTPUT ROUTINES

/OS/8 GENERAL CHARACTER I/O ROUTINES FO   PAL8-VC0  05-OCT-96 PAGE 6

017046  0000  OUTDMP, 0                       /DUMP OUTPUT BUFFER

017047  3300          DCA     OUCTLW          /STORE CONTROL WORD

017050  6211          CDF     10

017051  1774          TAD I   (OUTINH         /IS OUTPUT INHIBITED?

017052  7640          SZA CLA

017053  5304          JMP     OUNOWR          /YEP.

017054  1350          TAD     OUCCNT          /IF THIS IS FIRST WRITE, START

017055  7450          SNA                     / SEARCH FORWARD ON DECTAPE

017056  2300          ISZ     OUCTLW

017057  1221          TAD     OUBLK           /GET STARTING BLOCK OF THIS

017060  3302          DCA     OUREC           / TRANSFER

017061  1300          TAD     OUCTLW

017062  7106          CLL RTL

017063  7006          RTL

017064  7006          RTL                     /COMPUTE # OF RECORDS TO

017065  0376          AND     (17             / OUTPUT

017066  1350          TAD     OUCCNT          /UPDATE CLOSING LENGTH

017067  3350          DCA     OUCCNT

017070  1350          TAD     OUCCNT          /SEE IF CLOSING LENGTH WILL

017071  7120          CLL CML                 / BE BIGGER THAN OUTPUT HOLE

017072  1222          TAD     OUELEN

017073  7660          SNL SZA CLA

017074  5646          JMP I   OUTDMP          /WILL BE TOO BIG

017075  6203  OUCDIV, CIF CDF 0

017076  6211          CDF     10

017077  4614          JMS I   OUHNDL          /DO THE WRITE

017100  0000  OUCTLW, 0

017101  5000          OUBUF

017102  0000  OUREC,  0

017103  7410          SKP                     /ERROR

017104  2246  OUNOWR, ISZ     OUTDMP          /TAKE NORMAL RETURN

017105  5646          JMP I   OUTDMP

0020  PTP=    0020

F-12

OS/8 INPUT/OUTPUT ROUTINES

/OS/8 GENERAL CHARACTER I/O ROUTINES FO   PAL8-VC0  05-OCT-96 PAGE 7

017106  0000  OCLOSE, 0                       /CLOSE OUTPUT FILE

017107  6211          CDF     10

017110  1774          TAD I   (OUTINH         /IF OUTPUT INHIBITED, CLOSE

/ IS A NOP.

017111  7640          SZA CLA

017112  5352          JMP     OCISZ           /A NOP

017113  4771          JMS I   (OTYPE          /DETERMINE IF OUTPUT IS TO PTP

017114  0370          AND     (770            /IF IT IS, DON'T OUTPUT A ^Z.

017115  1372          TAD     (-PTP

017116  7640          SZA CLA

017117  1367          TAD     (232            /NOT PTP. OUTPUT ^Z AS EOF

017120  4766          JMS I   (OCHAR

017121  5353          JMP     OCRET           /ERROR RETURN

017122  4766          JMS I   (OCHAR          /FILL WITH 0 CHARACTERS

017123  5353          JMP     OCRET

017124  4766  FILLIP, JMS I   (OCHAR          /FILL TO BOUNDARY WITH 0

017125  5353          JMP     OCRET

017126  4771          JMS I   (OTYPE          /IF OUTPUT IS DIRECTORY DEVICE,

017127  7710          SPA CLA                 / FILL WHOLE RECORD, ELSE HALF

017130  1365          TAD     (100            / RECORD

017131  1364          TAD     (77)

017132  0763          AND I   (OUDWCT         /ARE WE UP TO BOUNDARY YET?

017133  7640          SZA CLA

017134  5324          JMP     FILLIP          /NO

017135  1763          TAD I   (OUDWCT

017136  1375          TAD     (OUCTL&3700     /IS THERE A FULL WRITE LEFT?

017137  7450          SNA

017140  5344          JMP     NODUMP          /YES. BUT DON'T DO IT, AS ^Z

/ IS OUT.

017141  1362          TAD     (4000+OUFLD

017142  4246          JMS     OUTDMP          /DUMP LAST BUFFER

017143  5353          JMP     OCRET

017144  1601  NODUMP, TAD I   OU7600          /GET DEVICE NUMBER

017145  4775          JMS I   (200            /CLOSE THE OUTPUT FILE

017146  0004          4

017147  7601  OU7601, 7601                    /POINTER TO FILE NAME

017150  0000  OUCCNT, 0                       /CLOSING FILE LENGTH HERE

017151  7410          SKP                     /ERROR

017152  2306  OCISZ,  ISZ     OCLOSE          /NORMAL RETURN

017153  6213  OCRET,  CDF CIF 10              /RESTORE CALLING FIELDS

017154  5706          JMP I   OCLOSE

017162  4000

017163  7272

017164  0077

017165  0100

017166  7211

017167  0232

017170  0770

017171  7274

017172  7760

F-13

OS/8 INPUT/OUTPUT ROUTINES

/OS/8 GENERAL CHARACTER I/O ROUTINES FO   PAL8-VC0  05-OCT-96 PAGE 7-1

017173  7200

017174  7273

017175  0200

017176  0017

017177  6601

7200          PAGE

F-14

OS/8 INPUT/OUTPUT ROUTINES

/OS/8 GENERAL CHARACTER I/O ROUTINES FO   PAL8-VC0  05-OCT-96 PAGE 8

017200  0000  OUSETP, 0                       /INITIALIZE OUTPUT POINTERS

017201  1377          TAD     (OUCTL&3700

017202  7041          CIA

017203  3272          DCA     OUDWCT          /DOUBLE WORD OUTPUT COUNT

017204  1376          TAD     (OUBUF          /INITIALIZE WORD POINTER

017205  3270          DCA     OUPTR

017206  1271          TAD     OUJMPE

017207  3224          DCA     OUJMP           /3 WAY UNPACK SWITCH

017210  5600          JMP I   OUSETP

F-15

OS/8 INPUT/OUTPUT ROUTINES

/OS/8 GENERAL CHARACTER I/O ROUTINES FO   PAL8-VC0  05-OCT-96 PAGE 9

017211  0000  OCHAR,  0                       /OUTPUT CHARACTER ROUTINE

017212  0375          AND     (377            /ISOLATE EIGHT BITS

017213  3266          DCA     OUTEMP

017214  6214          RDF                     /GET FIELD WE WERE CALLED

017215  1374          TAD     (CIF CDF 0      / FROM

017216  3264          DCA     OUCRET

017217  1273          TAD     OUTINH          /OUTPUT INHIBITED

017220  7640          SZA CLA

017221  5263          JMP     OUCOMN          /YES. NOP.

017222  6201  OUCHAR, CDF     OUFLD           /GO TO DATA FIELD OF BUFFER

017223  2224          ISZ     OUJMP           /BUMP CHARACTER SWITCH

017224  7402  OUJMP,  HLT                     /GETS JMP.,JMP.+1,ETC.

017225  5261          JMP     OCHAR1

017226  5256          JMP     OCHAR2

017227  1266  OCHAR3, TAD     OUTEMP          /THIRD CHAR

017230  7106          CLL RTL                 /HIGH ORDER BITS GO INTO THE

017231  7006          RTL                     /HIGH ORDER 4 BITS OF THE

017232  0373          AND     (7400           /FIRST OF TWO WORDS

017233  1667          TAD I   OUPOLD

017234  3667          DCA I   OUPOLD

017235  1266          TAD     OUTEMP          /THE SECOND DOUBLE WORD GETS

017236  7112          CLL RTR                 /THE LOW ORDER BITS OF

017237  7012          RTR                     /THE THIRD CHAR

017240  7010          RAR

017241  0373          AND     (7400

017242  1670          TAD I   OUPTR

017243  3670          DCA I   OUPTR

017244  1271          TAD     OUJMPE          /RESET CHARACTER SWITCH

017245  3224          DCA     OUJMP

017246  2270          ISZ     OUPTR           /POINT TO NEXT BUFFER WORD

017247  2272          ISZ     OUDWCT          /BUMP DOUBLE COUNT AFTER

/3 CHARS.

017250  5263          JMP     OUCOMN          /GET OUT

017251  1372          TAD     (OUCTL          /READY TO OUTPUT A BUFFER

017252  4771          JMS I   (OUTDMP         /OUTPUT IT

017253  5264          JMP     OUCRET          /AN ERROR

017254  4200          JMS     OUSETP          /RESET OUTPUT POINTERS

017255  5263          JMP     OUCOMN

017256  1270  OCHAR2, TAD     OUPTR           /POINT TO FIRST DOUBLE WORD

017257  3267          DCA     OUPOLD

017260  2270          ISZ     OUPTR           /POINT OUPTR TO SECOND

017261  1266  OCHAR1, TAD     OUTEMP

017262  3670          DCA I   OUPTR

017263  2211  OUCOMN, ISZ     OCHAR           /NORMAL EXIT

017264  7402  OUCRET, HLT

017265  5611          JMP I   OCHAR

017266  0000  OUTEMP, 0

017267  0000  OUPOLD, 0

017270  0000  OUPTR,  0

017271  5224  OUJMPE, JMP     OUJMP

017272  0000  OUDWCT, 0

F-16

OS/8 INPUT/OUTPUT ROUTINES

/OS/8 GENERAL CHARACTER I/O ROUTINES FO   PAL8-VC0  05-OCT-96 PAGE 9-1

017273  0000  OUTINH, 0

017274  0000  OTYPE,  0                       /OTYPE LOOKS AT THE OUTPUT

017275  6214          RDF                     / DEVICE #.

017276  1374          TAD     (CIF CDF 0

017277  3306          DCA     OTRTN

017300  6211          CDF     10

017301  1770          TAD I   (7600           /AND LOOKS UP THE DCB WORD

017302  0367          AND     (17             / FOR THAT DEVICE.

017303  1366          TAD     (DCB-1

017304  3266          DCA     OUTEMP

017305  1666          TAD I   OUTEMP          /GET DCB ENTRY

017306  7402  OTRTN,  HLT

017307  5674          JMP I   OTYPE

F-17

OS/8 INPUT/OUTPUT ROUTINES

/OS/8 GENERAL CHARACTER I/O ROUTINES FO   PAL8-VC0  05-OCT-96 PAGE 10

017310  0000  IOPEN,  0                       /INITIALIZE THE INPUT ROUTINE

017311  7240          CLA CMA                 /SET TO FORCE READ FROM NEW

017312  3765          DCA I   (INCHCT         / DEVICE

017313  2764          ISZ I   (INEOF          /FORCE A NEW INPUT FILE.

017314  1363          TAD     (7617           /POINT TO CD INPUT LIST.

017315  3762          DCA I   (INFPTR

017316  6214          RDF

017317  1761          TAD I   (INCDIF

017320  3321          DCA .+1

017321  7402          HLT

017322  5710          JMP I   IOPEN

017361  6632

017362  6722

017363  7617

017364  6721

017365  6725

017366  7757

017367  0017

017370  7600

017371  7046

017372  4200

017373  7400

017374  6203

017375  0377

017376  5000

017377  0200

7400          PAGE

F-18

OS/8 INPUT/OUTPUT ROUTINES

/OS/8 GENERAL CHARACTER I/O ROUTINES FO   PAL8-VC0  05-OCT-96 PAGE 11

/

/       EXAMPLE PROGRAM TO COPY A ASCII FILE

/

0001          FIELD 1

2000          *2000

012000  4777          JMS I   (7700           /LOCK MONITOR INTO CORE

012001  0010          10

012002  4776  CALLCD, JMS I   (200            /CALL THE COMMAND DECODER

012003  0005          5

012004  0000          0

012005  4775          JMS I   (IOPEN          /SETUP INPUT POINTERS

012006  4774          JMS I   (OOPEN          /OPEN OUTPUT FILE

012007  7700          SMA CLA                 /ERROR. IF AC<0, IT WAS FATAL

012010  5220          JMP OK                  /NON FILE STRUCTURED OUTPUT

012011  4263          JMS TERR

012012  1720          TEXT /OPEN FAILED/

012013  0516

012014  4006

012015  0111

012016  1405

012017  0400

012020  4773  OK,     JMS I   (ICHAR          /READ A CHARACTER

012021  5227          JMP     TSTEOF          /ERROR. SEE IF EOF.

012022  7450          SNA                     /IGNORE BLANKS

012023  5220          JMP     OK

012024  4772          JMS I   (OCHAR          /AND OUTPUT THE CHARACTER

012025  5243          JMP     OUTERR

012026  5220          JMP     OK              /CONTINUE UNTIL EOF.

012027  7700  TSTEOF, SMA CLA                 /WAS IT FATAL?

012030  5240          JMP     CLOSE           /NO..EOF. CLOSE OUTPUT

012031  4263          JMS TERR

012032  2205          TEXT /READ ERROR/

012033  0104

012034  4005

012035  2222

012036  1722

012037  0000

012040  4771  CLOSE,  JMS I   (OCLOSE

012041  5253          JMP     CLERR           /FILE CLOSE FAILED

012042  5202          JMP     CALLCD          /NEW INPUT.

012043  4263  OUTERR, JMS     TERR

012044  1725          TEXT /OUTPUT ERROR/

012045  2420

012046  2524

012047  4005

012050  2222

012051  1722

012052  0000

F-19

OS/8 INPUT/OUTPUT ROUTINES

/OS/8 GENERAL CHARACTER I/O ROUTINES FO   PAL8-VC0  05-OCT-96 PAGE 11-1

012053  4263  CLERR,  JMS     TERR            /CLOSE FAILURE

012054  0314          TEXT /CLOSE FAILED/

012055  1723

012056  0540

012057  0601

012060  1114

012061  0504

012062  0000

012063  0000  TERR,     0                     /ROUTINE TO PRINT ERROR

012064  1663          TAD I   TERR            / MESSAGES

012065  7012          RTR;RTR;RTR

012066  7012

012067  7012

012070  4275          JMS     TYPIT           /TYPE THE CHARACTER

012071  1663          TAD I   TERR

012072  4275          JMS     TYPIT

012073  2263          ISZ     TERR

012074  5264          JMP     TERR+1

012075  0000  TYPIT,  0

012076  0370          AND     (77             /ISOLATE THE CHARACTER

012077  7450          SNA

012100  5304          JMP     CRLF            /0 TERMINATES IT

012101  1367          TAD     (300

012102  4311          JMS     TTYOUT

012103  5675          JMP I   TYPIT

012104  1366  CRLF,   TAD     (215

012105  4311          JMS     TTYOUT

012106  1365          TAD     (212

012107  4311          JMS     TTYOUT

012110  5202          JMP     CALLCD

012111  0000  TTYOUT, 0

012112  6046          TLS

012113  6041          TSF

012114  5313          JMP       .-1

012115  7200          CLA

012116  5711          JMP I     TTYOUT

012165  0212

012166  0215

012167  0300

012170  0077

012171  7106

012172  7211

012173  6600

012174  7000

012175  7310

012176  0200

012177  7700

$$$$$$$$$$

F-20

OS/8 INPUT/OUTPUT ROUTINES

/OS/8 GENERAL CHARACTER I/O ROUTINES FO   PAL8-VC0  05-OCT-96 PAGE 12

CALLCD 2002      OORETN 7030

CLERR  2053      ORIGIN 6600

CLOSE  2040      OTRTN  7306

CRLF   2104      OTYPE  7274

DCB    7760      OUBLK  7021

EOFERR 6664      OUBUF  5000

FILLIP 7124      OUCCNT 7150

GETNEW 6614      OUCDIV 7075

ICHAR  6600      OUCHAR 7222

ICHAR1 6707      OUCOMN 7263

ICHAR2 6703      OUCRET 7264

ICHAR3 6670      OUCTL  4200

INBREC 6641      OUCTLW 7100

INBUF  5400      OUDEVH 6600

INBUFP 6636      OUDWCT 7272

INCDIF 6632      OUELEN 7022

INCHAR 6605      OUENTR 7016

INCHCT 6725      OUFLD  0000

INCOMN 6710      OUHNDL 7014

INCTL  0200      OUJMP  7224

INCTLW 6635      OUJMPE 7271

INCTR  6724      OUNOWR 7104

INDEVH 7000      OUPOLD 7267

INEOF  6721      OUPTR  7270

INERR  6663      OUREC  7102

INERRX 6660      OUSETP 7200

INFLD  0000      OUTDMP 7046

INFPTR 6722      OUTEMP 7266

INGBUF 6616      OUTERR 2043

INHNDL 6736      OUTINH 7273

INJMP  6665      OU7600 7001

INJMPP 6610      OU7601 7147

INNEWF 6725      PTP    0020

INPTR  6723      TERR   2063

INREC  6637      TSTEOF 2027

INRECS 0001      TTYOUT 2111

INRTRN 6716      TYPIT  2075

IN200  6673

IN7400 6720

IN7600 6601

IOPEN  7310

IORFLD 0001

NODUMP 7144

OCHAR  7211

OCHAR1 7261

OCHAR2 7256

OCHAR3 7227

OCISZ  7152

OCLOSE 7106

OCRET  7153

OEFAIL 7032

OK     2020

F-21

OS/8 INPUT/OUTPUT ROUTINES

ONOFIL 7044

ONTERR 7042

OOPEN  7000

ERRORS DETECTED: 0

LINKS GENERATED: 0

F-22

INDEX

Additional information words,	  CLOSE function, 2-8, D-10

1-3, 2-7, D-10		 	  Code, non-paged, D-11

Alphanumeric option, Command	  Command Decoder

Decoder, 3-3			  calling, 3-3

switches, 3-6			    conventions, 3-1

ALTMODE key, 3-3			    error messages, 3-3

Ascertain device information	    errors summary, C-7

(INQUIRE function), 2-14	    example, 3-7

ASCII character codes, E-3, E-4	    options, 3-3

ASCII files format, A-4		    output files, 3-5

ASSIGN command, 1-6		    special mode, 3-10

Asterisk, caution in using, 3-9	    tables, 3-4

COMMON area, 2-12

Control characters, OS/8, E-1

Batch mode, 3-11			  Conventions

Batch operating system, 1-6	    Command Decoder, 3-1

Binary file format, A-4		    OS/8, E-1

Block, core control, 1-4		  Core control block, 1-4, 2-11, A-5

Blocks				  Core image files (.SV format), A-5

directory, A-1			  Core origin, A-7

system scratch, B-1		  Core segment doublewords, A-7

Blocks of words, 1-2, 1-3		  Core size, PDP-8 computers, D-5

BUILD program, 5-1			    software, 1-6

Co-resident device handlers, 2-6

Creating files, 1-3

Call Command Decoder (DECODE

function), 2-10, 3-3

in Special Mode, 3-9		  Data exchange in core, 2-12

Calling device handlers, 4-1	  Data field value, 2-2

Calling USR and device handlers	  Data transfer, 4-1, 4-2

from SABR code, D-13		  DATE command, 1-4, 2-7

Card codes, DEC029, 4-8		  DECODE function, 2-10

Card Reader (CDR) operation, 4-8	  DECtape operation, 4-1

Card Reader handler modification,	  Default file storage device, DSK,

D-2				      D-1

Carriage return/line feed sup-	  Deleting tentative files, 2-16

pression in FORTRAN, D-4	  Device control word table, B-7

Cassettes operation, 4-7		  Device dependent operations, 4-4

CCL, 3-10				  Device handler

CCL error messages, C-4		    entry point, 2-15, 5-8

CHAIN function, 2-11		    information table, B-6

Character codes			    residency table, 2-15, B-6

ASCII, E-3, E-4			  Device handlers

OS/8, E-1			    device dependent operations, 4-4

Character mode, expanded, 4-6	      Card Reader (CDR), 4-8

Character packing format, 5-8	      Cassettes operation, 4-7

Characters, lower case, E-1	      High-Speed Paper Tape Punch

Circumflex (^) character, 4-5		(PTP), 4-5

INDEX 1

INDEX

Device handlers (Cont.)		  FETCH device handler function,

High-Speed Paper Tape Reader	      2-4, 4-1

(PTR), 4-4			  File creation, 1-3

file structured devices oper-	  File directories, A-1

ation, 4-12			  block format, A-1

TD8E DECtape, 4-12		    entries, A-2

TM8E Magtape, 4-8		    example, A-4

Device handlers, 1-1		    formats, A-4

calling, 4-1			    number, A-3

co-resident, 2-6			    size, A-3

inserting into OS/8, 5-5		  File extension, omission of, 3-2

loading dynamically, D-8		  File length restriction, Command

notes on loading, D-7		      Decoder, 3-6

writing, 5-1			  FILENAME pseudo-op, 1-7

Device length table, B-8		  Files, 1-2

Device names and numbers, 1-6	    additional information words, 1-3

DEVICE pseudo-op, 1-7		    devices, 1-2

Devices, file structured, 1-2	    directories, 1-3

DF32 disk operation, 4-12		    names, 1-2

Direct calling sequence, USR, 2-2	  types, 1-3

Directories, file, 1-3		  File structured devices operation,

Directory block structure format,	      4-12

A-1				  Formats for

Directory entries, A-2		    character packing, 5-3

Directory example, A-4		    command line, 3-1

Directory segment, rewriting,	    directory block structure, A-1

D-10				    files, A-4

Directory, system, 5-2		    FORTRAN Library File, A-8

Dismiss USR from core, 2-13	    input file, 3-2

Dot (.) used as system response,	    Job Status Word, A-6

1-6				  Form feed, 4-6

Doublewords, core segment, A-7       FORTRAN Library File Format, A-8

Functions, USR see USR functions

Empty file entry, A-2

Empty files, 1-3			  High-Speed Paper Tape Punch (PTP)

End-of-file card, 4-8		      operation, 4-5

End-of-file Condition, 4-2, 4-4	  High-Speed Paper Tape Reader (PTR)

ENTER output (tentative) file	      operation, 4-4

function, 2-7, D-9		  Horizontal tab, 4-6

Entry points for device handlers,

5-2

ERROR function, 2-12		  Indirect calling sequence, USR,

Error messages, Command Decoder,	      2-2

3-3				  Information words, additional, 1-3,

Error messages summary, C-1	      2-7

Error returns, device handler,	    accessing, D-10

4-2				  Input file format, 3-2

Exit to Keyboard Monitor, 1-1	  Input files, Command Decoder, 3-5

Expanded character mode, 4-6	  Input/output routines, F-1

Extensions of file names, 1-2	  Input table, Command Decoder, 3-9

INQUIRE function, 2-14

Inserting device handlers into

OS/8, 5-5

INDEX-2

INDEX

Job Status Word, 1-5, A-6		  Records (definition), 4-1

outputting odd number of, 4-3

RESET system table, 2-15

Keyboard Monitor, 1-1		  Resident program layout, B-3

error summary, C-3		  RESORC, B-9

KL8E terminal handler, 4-13	  Restrictions to USR calls, 2-2

RETURN key, 3-3

RF08 disk

LAP pseudo-op, D-11		    operation, 4-12

Layouts for OS/8 system, B-1	  RK8E handlers, 4-12

LINCtape operation, 4-12		  ROM (Read-Only-Memory), B-4

Line Printer (LPT) operation,

4-5, 4-6

Load and start subprogram, 2-11	  SABR programming notes, D-11

Loading device handlers dynam-	  SAVE command, 1-4, 2-12

ically, D-8			  Scratch blocks, B-1, B-2

Lock USR in core (USRIN), 2-13	  Signal user ERROR function, 2-12

Logical blocks, 1-2		  Size of OS files, A-3

LOOKUP permanent file function,	  Software components, 1-1

2-6, D-9			  Software core size, 1-6

Lower case characters, E-1		  Special mode of Command Decoder,

3-10

calling, 3-9

Names of devices, 1-6		    operation, 3-9

Names of files, 1-2		  Standard USR call, 2-1

Number and size of OS files, A-3	    restrictions, 2-2

Numbers of devices, 1-6		  START command, 1-5

Numeric option, Command Decoder,	  Starting address of program, 1-5

3-3				  Storage space, additional, D-9

Storage words, 1-3

Subroutine

ODT breakpoint, 2-12		    examples, F-4

Operations, device dependent, 4-4	    functions, F-1, F-2

Options				    listing, F-5

Command decoder, 3-3, 3-6	    parameters, F-3

Origin of core, A-7		  Summary of USR functions, see USR

Output files, Command Decoder,	      functions

3-5				  .SV file, 1-4

System DATE, 2-7

System device layout, B-1

PAGE pseudo-op, D-11		  System devices, 1-6

Permanent device name table, B-5     System device table, B-5

Permanent file, 1-3		  System halts error messages

deletion, 2-8, 2-9		      summary, C-1

entry, A-2			  System table values, Command

Physical blocks, 1-2		      Decoder, 3-7

PIP, 3-6

Program, see specific subject

Programming notes, D-1		  Tables

PRTC12-F used to convert DECtapes	    Command Decoder, 3-4

to LINCtapes, D-6		    device control word, B-7

Pseudo-ops				    device handler information, B-6

DEVICE, 1-7			    device handler residency, B-6

FILENAME, 1-7			    device length, B-8

Punch card codes, DEC026, D-3

INDEX-3

INDEX

Tables (Cont.)

permanent device name, B-5

system device, B-5

user device name, B-6

TD8E DECtape

operation, 4-12

Teletype operation, 4-4

Tentative files, 1-3

closing, 2-8

deletion, 2-8

entry, A-3

Terminal handlers

1-page, 4-4

2-page, 4-13

up arrow (^) character, 4-5

User device name table, B-6

User Service Routine (USR), 1-1,

2-1

available location in area,

D-9

calling, 1-6, 2-1

calling sequences, 2-2

errors summary, C-2

restrictions on standard call,

2-2

USR functions

CHAIN, 2-11

CLOSE, 2-8

DECODE, 2-10

ENTER, 2-7

ERROR, 2-12

FETCH, 2-4

INQUIRE, 2-14

LOOKUP, 2-6

RESET, 2-15

summary, 2-3

USRIN, 2-13

USROUT, 2-13

Vertical tab, 4-6

Wrap around memory, 4-3

Writing device handlers, 5-1

Word blocks, 1-2

INDEX 4

HOW TO OBTAIN SOFTWARE INFORMATION

SOFTWARE NEWSLETTERS, MAILING LIST    __________________________________

The Software Communications Group, located at corporate headquarters in

Maynard, publishes newsletters and Software Performance Summaries (SPS)

for the various Digital products.  Newsletters are published monthly,				       ___________

and contain announcements of new and revised software, programming

notes, software problems and solutions, and documentation corrections.

Software Performance Summaries are a collection of existing problems    ______________________________

and solutions for a given software system, and are published periodi-

cally.  For information on the distribution of these documents and how

to get on the software newsletter mailing list, write to:

Software Communications

P. O. Box F

Maynard, Massachusetts  01754

SOFTWARE PROBLEMS    _________________

Questions or problems relating to Digital's software should be reported

to a Software Support Specialist.  A specialist is located in each

Digital Sales Office in the United States.  In Europe, software problem

reporting centers are in the following cities.

Reading, England	   Milan, Italy

Paris, France	   Solna, Sweden

The Hague, Holland      Geneva, Switzerland

Tel Aviv, Israel	   Munich, West Germany

Software Problem Report (SPR) forms are available from the specialists

or from the Software Distribution Centers cited below.

PROGRAMS AND MANUALS    ____________________

Software and manuals should be ordered by title and order number.  In

the United States, send orders to the nearest distribution center.

Digital Equipment Corporation     Digital Equipment Corporation

Software Distribution Center      Software Distribution Center

146 Main Street		   1400 Terra Bella

Maynard, Massachusetts  01754     Mountain View, California  94043

Outside of the United States, orders should be directed to the nearest

Digital Field Sales Office or representative.

USERS SOCIETY    _____________

DECUS, Digital Equipment Computer Users Society, maintains a user ex-

change center for user-written programs and technical application in-

formation.  A catalog of existing programs is available.  The society

publishes a periodical, DECUSCOPE, and holds technical seminars in the

United States, Canada, Europet and Australia.  For information on the

society and membership application forms, write to:

DECUS				   DECUS

Digital Equipment Corporation     Digital Equipment, S.A.

146 Main Street		   81 Route de l'Aire

Maynard, Massachusetts 01754      1211 Geneva 26

Switzerland

OS/8 Software Support Manual

DEC-S8-OSSMB-A-D

READER'S COMMENTS

NOTE:  This form is for document comments only.  Problems

with software should be reported on a Software

Problem Report (SPR) form (see the E10A' TO OBTAIN

SOFTWARE INFORMATION page).

Did you find errors in this manual?  If so, specify by page.

______________________________________________________________________

______________________________________________________________________

______________________________________________________________________

______________________________________________________________________

______________________________________________________________________

Did you find this manual understandable, usable, and well-organized?

Please make suggestions for improvement.

______________________________________________________________________

______________________________________________________________________

______________________________________________________________________

______________________________________________________________________

______________________________________________________________________

Is there sufficient documentation on associated system programs

required for use of the software described in this manual?  If not,

what material is missing and where should it be placed?

______________________________________________________________________

______________________________________________________________________

______________________________________________________________________

______________________________________________________________________

______________________________________________________________________

Please indicate the type of user/reader that you most nearly represent.

[]  Assembly language programmer

[]  Higher-level language programmer

[]  Occasional programmer (experienced)

[]  User with little programming experience

[]  Student programmer

[]  Non-programmer interested in computer concepts and capabilities

Name______________________________________Date________________________

Organization__________________________________________________________

Street________________________________________________________________

City__________________________State_____________Zip Code______________

or

Country

If you do not require a written reply, please check here.  []

-------------------------------Fold Here------------------------------

-----------------Do Not Tear - Fold Here and Staple-------------------

+---------------+

|  FIRST CLASS  |

| PERMlT NO. 33 |

| MAYNARD. MASS.|

+---------------+

BUSINESS REPLY MAIL					   ========

NO POSTAGE STAMP NECESSARY IF MAILED IN THE UNITED STATES  ========

======================================================================

Postage will be paid by:				   ========

========

+-+-+-+-+-+-+-+			   ========

|d|i|g|i|t|a|l|			   ========

+-+-+-+-+-+-+-+			   ========

========

Software Communications			   ========

P. O. Box F				   ========

Maynard, Massachusetts  01754		   ========